| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0356 |  |
|  |  |  | English only |  |
| WG(s): 1, 2, 6 |  | 3rd FG IPTV meeting: Mountain View, USA, 22-26 January 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Internet Streaming Media Alliance |  |  |  |
| Title: | Requirement on Fast Channel Change |  |  |  |

Introduction

The output Document, FG-IPTV-DOC-0040 mentions that the IPTV solution should support efficient navigation between different channels. This contribution looks at some of the delays in ‘tune-in’ acquisition of live streams and proposes some techniques of general applicability to minimize the delay perceived by the user.

Motivation

It is necessary to clarify the demand that each element of the IPTV system should meet to satisfy the requirement for the user's channel change response. We analyzed the cause of the delay generally generated by the VOD system. Next, we should clarify the requirement for system architecture and each subsystem.

Analysis

The delay between the initiation of tune-in and initiation of rendering consists of:

- RTSP negotiation (if requesting streams or stream data)

- IGMP session joining (if tuning to a multicast session)

- Network latency

- Video and audio random access points (RAP) acquisition

- Client stream buffering (to de-jitter reception, to allow time for re-transmission, or to handle video packet re-ordering)

- Synchronization between streams (RTCP sender report)

- Encryption key acquisition

The dominant causes of delay are usually (a) de-jitter buffering, especially when traffic smoothing is used and (b) acquisition of a video random access point (RAP).Some of these delays can be mitigated, while others, such as network latency, cannot. It should be emphasized that many of the problems are not specific to RTP but arise from coding or other problems. The following sections discuss these delays in more detail.

## RTSP negotiation

RTSP is a protocol that can be used to initiate tune-in to a multicast stream or to request a unicast stream from a server. When it is used, it can take several packet round-trips of request/response before the first media packet is sent by the RTSP server.  In some situations, this can be significant.

## IGMP session joining

If the content is multicast on the network, then there are typically no processes operating at the RTP level between the source and the client.  Multicast is an efficient distribution protocol for live streams as each network link needs carry only one copy of each stream.  Routers in the network replicate streams from inputs onto the output links which have one or more clients.  Clients indicate their interest in the multicast by sending a special control packet (IGMP), which is intercepted by the router.  The routers in turn refer to each other to find, and forward, the packet stream.

This find-and-forward process (called multicast ‘join’) can, in some circumstances, take some time.

## RAP acquisition

Video is classically ‘differentially coded’.  Very few coded frames contain all the information needed to create a complete set of decoded pixels.  Instead, frames are coded as differences from one or more other frames.

In order to handle both recovery from loss and tune-in to live streams, ‘independently decodable’ frames or I-frames can be sent periodically.  Decoder refresh frames or IDR-frames are I-frames that have the additional property that they mark a division of the sequence; frames displayed after the decoder refresh do not depend on frames before it.  They are true Random Access Points (RAPs).

Because these frames are so much larger (in bytes) than differentially coded frames, they are sent rarely in order to keep the bit-rate low. A delay occurs when tuning into a new stream, since the decoder must wait for a RAP before it can start displaying video.  In addition, their size often means that traffic smoothing causes adjustment of their send time and that of adjacent packets.

## Client buffering

If there is deviation in the arrival time of packets from the relative timing that they would have if they were to arrive just-in-time to be played, then a de-jitter buffer is needed if we are to avoid under-run (starvation).  This jitter has two causes: deliberately introduced jitter from the source, for traffic smoothing, and network-introduced jitter (e.g. caused by cross-traffic in network equipment).  The source-introduced jitter tends to occur most, or even exclusively, in video, where the variation in coded frame size (in bytes) can be large (e.g. I-frames can be many times as large as B frames).

A buffer is also needed if there is to be time to perform re-transmissions.

A delay occurs during the time the client fills its buffer before starting to render.

## Synchronization between streams (RTCP sender report)

RTP streams have ‘free floating’ timestamps – they have arbitrary origins (and indeed, usually different streams have different tick-rates).  In order to synchronize audio and video, their time stamps have to be related.  Associated with each stream are periodic RTCP sender reports, which associate the RTP timestamps in the stream with a common clock at the transmitter (usually the time-of-day clock, but actually any clock is permitted as long as it is common to all streams).

Until at least one RTCP sender report has been acquired for each stream, the streams cannot be played in synchronization.  The only exception to this is that the RTSP control protocol has provision for sending the initial synchronization information at the beginning of a play interval.  However, RTSP is not used in all situations.

Thus, the frequency and timing of RTCP reports often contribute to the delay before audio and video are rendered, not just to their synchronization, because many clients will not render anything before synchronization has been established.

## Encryption keys acquisition

If the content is encrypted, then the decryption keys must be acquired.  Generally it is not desirable to acquire keys too far ahead of need (either for the future of the current stream, or for other potential streams).  Since this problem is both independent of RTP, and dependent on the key-exchange method used, it is not discussed here.

Requirement

The delay time from user action to tune-in and display must be less than xxx seconds.

To achieve the delay time defined above, requirements of each delay element are shown below.

RTP negotiation response time must be less than xxx seconds.

IGMP Session joining delay time must be less than xxx seconds.

RAP acquisition time must be less than xxx seconds.

Client buffering capacity must be less than xxx seconds.

The error margin of RTP Time stamp must be within xxx seconds.

The frequency of RTCP report must be more than xxx /second.

Encryption keys acquisition time must be less than xxx seconds.

Proposal

We should define requirements shown above as soon as possible.

Then, to satisfy these requirements, we should consider improvement techniquies at the delay time. For example:

## In-band Synchronization

The fact that RTP synchronization is usually provided (and must be provided) in RTCP does not preclude its provision by other, additional, means.  Indeed, one has already been noted – its provision in the RTSP protocol.

Using the header extension defined in http://www.ietf.org/internet-drafts/draft-ietf-avt-rtp-hdrext-05.txt, it is possible to provide the time-mapping in selected RTP packets, in-band.  This would take the form of four extra 32-bit words in those packets.  (One word indicates that the header extension is in use and the length of all the extensions, then there is a one-byte tag and length for this extension, 8 bytes of NTP timestamp, and, alas, 3 bytes of padding.)

This would typically only need to be sent on those packets where the decoding can start – RAP frames in video – or their temporally aligned packets in other streams.  These packets are already quite large, as noted, and this overhead is small in comparison.

Implementing this scheme eliminates the delay caused by waiting for the RTCP sender reports.

## Server buffering of content

If a streaming server is being used for the final distribution of content, then it could buffer the stream so that it is able to supply to each client, as it tunes in, a decoder refresh point and its temporally aligned audio packet.  Servers doing packet reflection already ‘pace’ their output packet stream, not sending N copies of each incoming packet to their N clients simultaneously, but spreading them out in time.  If they buffer each decoder refresh interval, providing the beginning of the most recent one, to each client as they tune-in, this will have a similar stochastic spreading effect.

In order to do this, the packet stream is ideally marked in a codec-independent fashion. Indeed, it has long been an awkward nature of RTP that random-access points – for tune-in or error-recovery –are not marked at the RTP level.  The general header extension mechanism can again be used, marking packets as RAPs, or as aligned with the RAP in a synchronized stream (such as a parallel audio stream).  Such marking can also label frames which are independently decidable even if not RAPs, or as ‘leaves’ in the dependency graph (i.e. no other frame depends on them).

If buffering entire streams is unacceptable, for memory reasons, then the server could choose to buffer only video frames that have dependencies, or indeed only I frames.

There are two ways to use this buffering.  The client may use the unicast content from the server only during the tune-in period;  once it has acquired the multicast, and the packet sequence numbers overlap between the unicast and multicast feeds, then the unicast can be dropped (the streams splice perfectly).  Alternatively, it may be desirable to use client-server unicast for the duration of the stream, as this enables retransmission and client-server buffer management.

If the server is performing application-level replication (as opposed to IP multicast, which is network-level) then it needs to be placed ‘suitably close’ to the point where the physical transmissions diverge, as it has N copies of the input traffic for N clients, on its output link.  This is not necessarily co-located with the router, for example, that supplied the CPE.  There are deployment cost trade-offs between many smaller servers located close to the routers, and a larger one deeper in the network, with suitable bandwidth out to the routers that feed the CPE.

The load on those routers varies, of course.  If they are replicating multicast, only one packet is received for N client packets transmitted, whereas if an upstream  server is replicating, then N unicast packets are received.  Unicast routing is typically cheaper than multicast routing, but this does represent a change of load.

The tagging is done using the header extension in RTP, referred to above. If the tags for video are present in an audio stream, then they provide information about the temporally aligned video.  The tag has the name tv.isma.rtp.frametags.video/092006.

The video tag is a one-byte field with the following values.  This tag may be extended in future with extra bytes, which may be ignored if not recognized.  It has the following definition:

unsigned int(1) reserved = 0;
unsigned int(1) RAP;
unsigned int(2) sample_depends_on;
unsigned int(2) sample_is_depended_on;
unsigned int(2) sample_has_redundancy;

RAP takes one of the following two values:

0:  not known to be a random access point

1:  Random access point

sample_depends_on takes one of the following four values:

0:  the dependency of this sample is unknown;

1:  this sample does depend on others (not an I picture);

2:  this sample does not depend on others (I picture);

3:  reserved

sample_is_depended_on takes one of the following four values:

0:  the dependency of other samples on this sample is unknown;

1:  other samples depend on this one (not disposable);

2:  no other sample depends on this one (disposable);

3:  reserved

sample_has_redundancy takes one of the following four values:

0:  it is unknown whether there is redundant coding in this sample;

1:  there is redundant coding in this sample;

2:  there is no redundant coding in this sample;

3:  reserved

## Multicast Configuration

We discuss two methods to reduce delays associated with joining a multicast session.

Single-source multicast enables the routers to follow normal unicast routing towards the multicast source. This does not require special multicast configuration and routing tables, and can be faster.

Also, on a managed network it is possible to setup the multicasts so that they normally ‘flood’ the network down to the last switching node on the network (for instance the DSLAM). The IGMP join is thus limited to the last mile and session joining is faster.

## Predictive Tuning (improvement)

One of the simplest techniques for reducing channel switch times is to predict what channel the user will next ask for, and tune that in early.  If there is enough bandwidth, holding open channels that this user often watches, or the ‘next’ and ‘previous’ channels (for the channel surfer).

However, this consumes the bandwidth for multiple streams for each client, and this may use  excessive bandwidth.  It also does not optimize all stream switches, and so is at best a partial solution.

For deployments based on VDSL (25-50 Mbps) and FTTH there may be enough bandwidth available and the system is designed to handle several streams to each household anyway. In any case, predictive tuning is a technique that can be implemented at the client, most probably without the need for additional protocols.

## RTSP Switching

The following  3GPP SA4 contribution, http://ietfreport.isoc.org/idref/draft-einarsson-mmusic-rtsp-macuri/, suggests techniques for reducing delays associated with RTSP handshaking.  This proposes that if the channels concerned are indeed channels in a family, then they can presumably share codecs and codec settings, screen size, audio channel count etc., and then the usual three round trips to tear-down and setup a channel over RTSP can be reduced.

Clearly round-trip delays by themselves can be optimized (e.g. the SETUPs can be pipelined, or a new RTSP command used to switch video and audio streams on existing ports).

## De-jitter buffering

Unless the content is encoded at a constant bit-rate, or its peak bit-rate is below the channel rate – whereupon no source-side traffic smoothing need occur – there is going to be jitter in the packet arrival times, caused by traffic smoothing.  In a managed network the network-induced jitter can be minimized.

One technique for the client is to notice the first I-frame to arrive, and to display it, even before ‘full playback’ has started.  This presents a picture ‘early’ on the screen.

A more systematic approach is to use a protocol for managing re-transmissions, bandwidth usage, and client-side buffer space, such as the one defined by 3GPP.  If the content bandwidth is lower than the network bandwidth, these protocols can build the client-side buffer faster than real-time, so a buffer that takes 3 seconds to play out can be built in a second or less.

In addition, even if the ‘ideal’ buffer is large, it may be worth risking underflow for a short period, deliberately playing the content slightly slowly so that the buffer builds towards a safer value.

In 3GPP 26.234 <http://www.3gpp.org/ftp/Specs/html-info/26234.htm> (currently version 6.8), section 6.2.3.3 defines how to perform RTP retransmission, and section 10 details a client-server buffer management and link-rate adaptation protocol.  Used together, a streaming server and client can both build the de-jitter at faster than the link-rate, and then use that buffered period as a window in which re-transmissions can be requested.

## Low resolution stream

In order to limit the number of equipments on the network, a second low resolution stream, conveying mainly I frames or with a small GOP (IPP for instance) can be sent in parallel when the user changes channel.

This stream will have a small bandwidth, but will deliver an I frame within at most 200ms. This stream will be up-scaled to fit in the targeted resolution and used until an I frame arrives on the full resolution / long GOP stream. When the full I frame has arrived, the low resolution stream is stopped.

The solution is scalable as no new servers are required on the network; it just needs other encoders to generate this low resolution stream at the headend. And this stream can also be already present on the network to be used for PiP, portable terminals, or for a selection mosaic or channel-guide preview..

_____________