| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0376 |  |
|  |  |  | English only |  |
| WG(s): 6 |  | 3rd FG IPTV meeting: Mountain View, USA, 22-26 January 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | China Telecom |  |  |  |
| Title: | Proposal on Metadata Sets for typical IPTV Services and Metadata encoding |  |  |  |

Abstract

In IPTV service, each type of content has relevant metadata that describes its related information. Working document of WG6,FG IPTV-DOC-0051,has accepted a series of requirements for metadata.

This contribution made reference on the C-0214 and proposes that according to the metadata using scenario, metadata sets should be classified into two types. One is independent of presentation but necessary to instruct services provider to control IPTV services and anther inversely is only dependent on presentation in EPG. This contribution also list the metadata sets for typical IPTV services from this perspective.

Discussion 1

The following propose basic metadata sets for typical IPTV service.  It should be noted that the following is very basic, metadata configuration can be extended depending on the application requirements.

1) Broadcasting TV (BTV)

Metadata sets what are independent of presentation:

| Element name | Description |
| --- | --- |
| CPId | Content provider identification |
| SPId | Services provider identification |
| Action | to identify what this metadata set is used for. |
| ContentId | Content series number |
| ContentType | to identify the type of contents described by this metadata set or metadata file. |
| Grade | Content Grade info |
| Country | Country identification |
| state | state identification |
| city | city identification |
| language | spoken language identification |
| caption | To identify the caption language |
| VideoType | Content video format |
| audioType | Content audio format |
| StreamType | to identify the type of stream format or protocol |
| bitrate | to identify the bitrate info of per stream. |
| resolution | Video resolution identification |
| TimeShiftedTag | Tag for identifying if this channel supports timeshifted function. |
| TimeShiftDuration | To identify the TimeShifted function duration |
| ChannelMultiCastIP | Info of MultiCast IP address |
| ChannelMultiCastPort | Info of MultiCast IP port |

Metadata sets what are dependent of EPG presentation

| Element name | Description |
| --- | --- |
| contentName | contentName or title |
| description | Content description in text |
| channellogo | Channel Logo info, graphic or textual |
| PosterInfo | URL information of poster |
| programscheduleinfo | URL information of program schedule file. |

2) Scheduled Time-shifted TV

Metadata sets what are independent of presentation:

They are same with those of  Broadcasting TV

Metadata sets what are dependent of EPG presentation:

| Element name | Description |
| --- | --- |
| ChannelName | Channel name displayed |
| ProgramName | contentName or title displayed |
| channellogo | Channel Logo info, graphic or textual |
| ProgramDate | Date schedule of separate program |
| ProgramStartTime | Starting time of separate program |
| ProgramEndTime | End time of separate program |
| programscheduleinfo | URL information of program schedule file. |

3) Video on Demand (VOD)

Metadata sets what are independent of presentation:

| Element name | Description |
| --- | --- |
| CPId | Content provider identification, for multiple CPs-single SP |
| SPId | Services provider identification, for single CP- multiple SPs. |
| Action | to identify what this metadata set is used for. |
| ContentId | Content series number |
| ContentType | to identify the type of contents described by this metadata set or metadata file. |
| Grade | Content Grade info |
| Country | Country identification |
| state | state identification |
| city | city identification |
| language | spoken language identification |
| caption | To identify the caption language |
| VideoType | Content video format |
| AudioType | Content audio format |
| StreamType | to identify the type of stream format or protocol |
| bitrate | to identify the bitrate info of per stream. |
| resolution | Video resolution identification |
| TimeDuration | To identify the program duration |
| ContentUrl | download URL Info of content file |
| ContentPrevUrl | download URL Info of content preview file |
| PosterInfo | URL information of poster |
| StartingTime | Starting time point when content valids from |
| ExpiredTime | end time point when content valids till |
| searchKey | Key words for searching |

Metadata sets what are dependent of EPG presentation:

| Element name | Description |
| --- | --- |
| contentName | contentName or title |
| description | Content description in text |
| Poster | URL information of poster |
| programscheduleinfo | URL information of program schedule file. |
| TimeDuration | To identify the program duration |
| episodeIndex | Episode Index |
| episodeNum | Episode Number totally |
| director | Director of this content |
| actor | Actor(s) of this content |
| producer | Producer of this content |
| playWriter | Writer of this content |
| caption | To identify the caption language |

Discussion 2

Regarding to the metadata encoding mechanism, there have been many organizations that have long work in metadata and they usually suggested that metadata should be represented in XML format for the purpose of interoperability.  For example, TV-Anytime Forum has adopted XML as the common representation format for metadata.

For IPTV, metadata encoding mechanism shall be extensible and consider consistency with those existing ones.

Proposal

1.  It is proposed that metadata elements listed above for typical IPTV services should be referenced as start point for further working on metadata in part A of working document of WG6.

2.  It is also proposed that the metadata sets should be encoded using XML.

Reference

http://www.tv-anytime.org

- Contributor List

| Name | Company | Tel. | Email |
| --- | --- | --- | --- |
| Siqing Luo | China Telecom | +86 21 58755452 | luosq@sttri.com.cn |
| Haining Yan | China Telecom | +86 21 28970207 | yanhn@sttri.com.cn |
| Jianting Meng | China Telecom | +86 21 58755057 | mengjt@sttri.com.cn |
| Liding Jia | China Telecom | +86 21 28970089 | jiald@sttri.com.cn |

____________