| IPTV Reference Model INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0388 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 3rd FG IPTV meeting: Mountain View, USA, 22– 26 January 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | BT, UK (Simon T Jones) |  |  |  |
| Title: | Reference Model and Definitions for IPTV |  |  |  |
| Doc: | FG IPTV- INTERIM -DOC-002 Rev2, Section 7 also FG IPTV-C-0261 and FG IPTV-DOC-056 |  |  |  |

## IPTV Service Types

## IPTV Definitions

## Operational Model

To support the decomposition of the IPTV Services into an Architecture, a generic model for the operation of an IPTV Service is required.

The following sequence represents one of a number of possible operational models for IPTV. It assumes that all provisioning and initial connection activities have been completed.

- Customer’s network gateway obtains an  IP Address and configuration details, via DHCP or PPP, from network

- IPTV Terminal obtains IP Address and configuration details, via DHCP, from the Gateway

- IPTV Terminal runs IPTV Application and obtains a logical URL for the content that the customer wishes to receive

- IPTV Terminal resolves the logical URL of content into the physical URL of content using the IPTV server

  - IPTV server determines the URL of the best server to deliver the content by passing IPTV Terminal’s IP address and content reference to the Content Location service

    - Content Location service uses the IPTV Terminal’s IP address to determine its geographical location

    - Content Location service uses information about the distribution of content in the network to find a server that has the requested content and free resources to deliver it

    - Content Location service uses sever and terminal information to reserve delivery capacity on physical server

    - Content Location service returns URL of physical server and content to the IPTV server

  - IPTV server sets up network resources for path from server to network core

  - IPTV server sets up network resources for path from network core to client

  - IPTV server returns URL of physical server and content to IPTV Terminal

- IPTV Terminal uses its Media Client to connect to and control the delivery of the Content

- Media Client receives Content