| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV C-0425 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 3rd FG IPTV meeting: Mountain View, USA, 22-26 January 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Korea (Republic of) |  |  |  |
| Title: | Some changes on IPTV Services Requirements |  |  |  |

This contribution proposes some changes on the working document of IPTV Service Requirements which is the results of the electric meeting at 18 – 21 December 2006.

In the following texts, the sentences by using Italic characters starting with <comment> are not the main part of texts. It just gives some comments.

______________

CONTENTS

1.	Scope	4

2.	References	4

3.	Definitions	4

4.	Abbreviations	5

5.	IPTV Service Requirements	5

5.1       Requirements for Architecture and Services	5

5.1.1	Scenarios and drivers	6

5.1.2    Accounting and charging Requirements	6

5.1.3 Architecture	7

5.1.4 Relationship with other services and networks	12

5.2 Requirements for QoS and Performance Aspects	14

5.2.1 QoE	14

5.2.2 Performance	15

5.2.3 Traffic Management	15

5.2.4 Bandwidth requirements and constraints	15

5.3	Requirements for Service Security and Contents Protection Aspects	15

5.3.1 Digital Rights Management	15

5.3.2	Content protection	15

5.3.3	Security (e.g. conditional access)	16

5.3.4 Authentication	17

5.3.5 Authorization	17

5.4	Requirements for Network and Control Aspects	17

5.4.1 Network Requirements	20

5.4.2 Control protocol & signalling	22

5.4.3 Naming, addressing, and identification aspects	23

5.4.4 Routing and multicast session control	23

5.4.5 Content distribution	24

5.4.6 Home Network	24

5.5 Requirements for End Systems and Interoperability aspects	27

5.5.1 End-user requirements	28

5.5.2 Implementation scenarios and applications	35

5.5.3 Terminals	35

5.5.4 End-user domain (home & extensions)	38

5.5.5 Remote management	39

5.6 Requirements for Middleware, Application and Content Platforms	39

5.6.1 Middleware	39

5.6.2 Enhanced EPG, Channel and Menu Processing	40

5.6.3 Content Coding	41

5.6.4 Metadata	41

5.6.5 Content Discovery	43

5.6.6 Content Delivery	43

5.7	Public interest requirements	45

5.7.1 Accessibility Requirements	45

5.7.2 Emergency telecommunications	46

IPTV SERVICERS REQUIREMENTS

# Scope

This document describes requirements for the design, the deployment and the operations of IPTV services.

Editor’s Note: material is required for the scope

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this working document. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this working document are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this working document does not give it, as a stand-alone document, the status of a Recommendation.

[H.Sup.1] 	ITU-T Recommendation H.Sup.1 (05/99), Application profile - Sign language and lip-reading real-time conversation using low bit-rate video communication

# Definitions

This Working Document uses or defines the following terms:

3.1	IPTV: IPTV is defined as multimedia services such as television/video/ audio/text/graphics/data delivered over IP based networks managed to provide the required level of QoS/QoE, security, interactivity and reliability.

Note: This definition has been approved by FG IPTV plenary and is not subject to change or modification

(Editor’s Note:  The following terms will be defined and captured in the yet-to-be-published glossary output document, as agreed during the Busan meeting)

Linear/Broadcast TV (audio, video and data)

TBD

Linear Broadcast TV with Trick Modes

TBD

Multi-angle service

TBD

Time-shift TV

TBD

Pay Per View (PPV)

TBD

Personal Video Recorder (PVR)

TBD

Personal Digital Recorder (PDR)

TBD

PDR can be viewed as an instance of PVR, which assumes digital storage capabilities. PVR would then be seen as a recording capability that can be solicited and operated by end users to record and store video and audio data

Device: Any of the following apparatus: TV set, Set-Top Box, Remote control, home gateway, etc.

Editor’s Note: elaborate on the kind of device whenever possible in this document based on the title of the section, a device can be any of the following apparatus: TV set, Set Top Box, Remote control, home gateway, etc.

Customer: A customer is a legal representative who has the (legal) ability to subscribe to a service offering.

End-user: An end user is an entity (a human being or a process, from a general perspective) who has been named by a subscriber and appropriately identified by a service provider, so that he might benefit from a service according to his associated rights and duties. An end user can be confused with a customer in the case of residential services.

Editor’s Note: the following text to be used to define different types of VoDs

Solicited VoD Content versus Unsolicited Content: Distinction has to be made between solicited VoD content (requested by the end-user) and unsolicited content (pushed by the operator without user request, e.g. advertisement, new movie in line with user preferences)

# Abbreviations

This Working Document uses the following abbreviations:

# IPTV Service Requirements

## 5.1       Requirements for Architecture and Services

Specific functions should be considered in the IPTV NGN or non-NGN architecture:

- navigation for the user, e.g. EPG function

IPTV_ARC_001: IPTV provides diverse services for the user, such as entertainment service, information service and even convergence service, so the navigation function is needed to facilitate the use of the end user.

- content distribution and delivery

IPTV_ARC_002: For IPTV, a large number of users will access a large number of media/content at the same time.

Content locating

IPTV_ARC_003: IPTV service essentially provides all kinds of media/content to users. So how the suitable media/content is discovered and located dynamically should be resolved by architecture.

- content management  function

IPTV_ARC_004: A content management function will be required to manage the content.

- service management function

IPTV_ARC_005: The service provider may integrate different content into a content bundle, or arrange schedule for the program, etc. So the service management function is very important and should be supported.

- security aspects

IPTV_ARC_006: supporting service authentication and authorization

IPTV_ARC_007: supporting service/content protection

IPTV_ARC_008: The IPTV Architecture should allow the delivery of IPTV services over different access networks (e.g. cable, optical, xDSL, wireless)

IPTV_ARC_009:  The IPTV Architecture should allow the delivery of IPTV services to different end devices, including mobile terminals that can be accessed through wireless networks (e.g. terminal devices such as mobile terminal (cell phone), TV set, PC).

IPTV_ARC_010: IPTV services should be delivered to any kind of IPTV-enabled terminal devices, whatever the underlying technologies and access network capabilities involved, and whether the end user is in motion or not.

IPTV_ARC_011: When operating over a mobile network, the IPTV service should be able to dynamically adapt to possible change in wireless network characteristics.

<comment> It is still  not clear since there are many options to consider; 1. cope with handover interruption time, 2. adjust video quality between SD and HD, 3. dynamically adapt screen size quality of mobile terminal

Editor’s note: The plenary could not come to an agreement to the crossed out requirement. Contributions are requested.

<comment> handle at 5.1.4.1

### 5.1.1	Scenarios and drivers

<comment> check  the relevance of section 5.1.1. scenarios and drivers compared with section 5.1.4.1. Otherwise, scenario can be classified into various criteria. For example, It can be classified by service categories such as T-channel, T-VoD, T-interactive, T-commerce, and T-communication. Also, scenario can be classified by the domain concept and/or business models of content provider, service provider, network provider, and customer. In addition, the requirements contained at this section are tightly relating to the other document on service and scenario for IPTV.

IPTV_ARC_013: The IPTV service should support various encapsulation types

<comment> The encapsulation type should be clarified whether it is based on layer concept such as IP-in-IP tunnelling, otherwise encapsulating different video coding and transfer format by using same IP frame. In addition, it should check whether the multicast tunnelling between a number of multicast sources and destinations are also considered since there are a number of multicast  tunnelling mechanisms.

### 5.1.2    Accounting and charging Requirements

IPTV_ARC_014: The IPTV Architecture should support appropriate billing models for end-user originated content. [IIF.ARCH.SERVICE.26]

IPTV_ARC_015: The IPTV Architecture shall not preclude mechanisms for the service provider to restrict the end-user shared content per business rules. [IIF.ARCH.SERVICE.27]

IPTV_ARC_016: The IPTV Architecture may support DRM of end-user shared content. [IIF.ARCH.SERVICE.28]

IPTV_ARC_017: The IPTV Architecture should support appropriate billing models for games. [IIF.ARCH.SERVICE.31]

IPTV_ARC_018: The IPTV Architecture shall support IPTV service charging through various techniques (such as prepay, post pay, advice of charge, and third party charging). [IIF.ARCH.OPERATOR.20]

IPTV_ARC_019: The IPTV specification should provide means by which service providers can integrate IPTV Service Accounting and Billing functions with a system that can be used across multiple NGN services and applications. [IIF.ARCH.NETWORK.16]

IPTV_ARC_020: CONDITIONAL MANDATORY:  If the integration described in IIF.ARCH.NETWORK.16 is implemented, it shall be implemented following the Charging Architecture for ITU-T NGN and ATIS TMOC. [IIF.ARCH.NETWORK.17]

IPTV_ARC_021: The IPTV Architecture shall not preclude the use of NGN Charging Architectures for all IPTV Service Accounting and Billing functions. [IIF.ARCH.NETWORK.18]

IPTV_ARC_022: The IPTV Architecture shall support mechanisms for the collection of data for accounting and reporting purposes, partner settlements, and reconciliation of end-user usage -- such as Service subscriptions, purchases, and transactions. [IIF.ARCH.CONTEXT.22]

IPTV_ARC_023: The IPTV Architecture shall provide for the use of mechanisms for accounting and charging for packet transport and/or service usage. [IIF.ARCH.NETWORK.10]

#### 5.1.2.1 Subscription

IPTV_ARC_024: TBD

#### 5.1.2.2 Pay per View

IPTV_ARC_025: The IPTV Architecture shall support the acquisition of appropriate VOD accounting data, to fulfil licensing agreements. [IIF.ARCH.SERVICE.15]

#### 5.1.2.3 Revenue Collection

<comment> duplicate with ARC_027

IPTV_ARC_027: Call Detail Record

IPTV_ARC_028: The Mobile TV is deployed in some areas and countries. There are some implementation ways for the Mobile TV. It is expected that Mobile TV is realized through IP Mobility and wireless access technologies.

### 5.1.3 Architecture

IPTV_ARC_029: The IPTV Architecture shall support the decomposition into high-level and sub-level domains. [IIF.ARCH.CONTEXT.01]

IPTV_ARC_030: The IPTV Architecture shall support the decomposition of the network into geographical sub-domains.[ IIF.ARCH.CONTEXT.02]

IPTV_ARC_031: The IPTV Architecture shall provide a framework that identifies the key components and measurement points for Quality of Service Measurement (QoSM). [IIF.ARCH.CONTEXT.03]

IPTV_ARC_032: The IPTV Architecture sub-domains shall be easily mapped to other key standards such as NGN. [ IIF.ARCH.CONTEXT.04]

IPTV_ARC_033: The IPTV Architecture shall define the interfaces into the Service Provider Management sub-domain, not the functions within it.[ IIF.ARCH.CONTEXT.05]

IPTV_ARC_034: The IPTV Architecture shall define the key service delivery components of IPTV services and their interfaces into the other domains.[ IIF.ARCH.CONTEXT.06]

IPTV_ARC_035: The IPTV Architecture specifications may further break up the Device, User, and Subscriber concepts into a set or a sub-hierarchy of their own.[ IIF.ARCH.CONTEXT.07]

IPTV_ARC_036: The IPTV Architecture specifications shall support the data models which allow for the representation of the defined end-user domain hierarchy.[ IIF.ARCH.CONTEXT.08]

IPTV_ARC_037: The IPTV Architecture shall allow for two-way communications between End-user Network and the Service Provider. [IIF.ARCH.CONTEXT.09]

IPTV_ARC_038: The IPTV Architecture shall support the individual addressability of devices acting as ITFs within the End-user Network. [IIF.ARCH.CONTEXT.10]

IPTV_ARC_039: The IPTV Architecture may support the delivery of multiple services over the common IP transport with a controllable IP Quality of Service (QoS); services maybe delivered from multiple service providers or from a Single provider. [IIF.ARCH.CONTEXT.11]

IPTV_ARC_040: The IPTV Architecture shall allow the delivery of IPTV services with a defined Quality of Experience (QoE) for the IPTV end-user. [IIF.ARCH.CONTEXT.12]

IPTV_ARC_041: The IPTV Architecture shall meet the security requirements of the Content Providers, Service Providers, Network Providers, and End-users.[ IIF.ARCH.CONTEXT.17]

IPTV_ARC_042: The IPTV Architecture shall allow for the utilization of Internet protocols and standards. [IIF.ARCH.CONTEXT.20]

IPTV_ARC_043: The IPTV Architecture shall provide the ability to remotely configure and administer Network and Service elements. [IIF.ARCH.CONTEXT.21]

IPTV_ARC_044: The IPTV Architecture shall support mechanisms for the delivery and rendering of various profiles of Linear/Broadcast TV content, including both High Definition (HD) and Standard Definition (SD) profiles.[ IIF.ARCH.SERVICE.01]

IPTV_ARC_045: The IPTV Architecture shall support mechanisms for the receipt of content in the Super Head End (SHE) via a variety of interfaces -- e.g., satellite, dedicated IP connections.[ IIF.ARCH.SERVICE.02]

IPTV_ARC_046: The IPTV Architecture shall provide mechanisms to support PPV services. [IIF.ARCH.SERVICE.05]

IPTV_ARC_047: The IPTV Architecture shall provide mechanisms to support On Demand services. [IIF.ARCH.SERVICE.06]

IPTV_ARC_048: The IPTV Architecture shall support multiple service models for VOD such as Subscription VOD (SVOD) and Free VOD (FVOD). [IIF.ARCH.SERVICE.08]

IPTV_ARC_049: The IPTV Architecture shall allow the delivery of multiple VOD content profiles -- e.g., HD, SD, multi-channel audio. [IIF.ARCH.SERVICE.10]

IPTV_ARC_050: The IPTV Architecture shall make it possible for the user to restrict purchases and access to services and content through the use of appropriate controls -- e.g., PIN number, login. [IIF.ARCH.SERVICE.11]

IPTV_ARC_051: The IPTV Architecture shall allow for the insertion of content into VOD content. [IIF.ARCH.SERVICE.12]

IPTV_ARC_052: The IPTV Architecture may provide mechanisms to capture and utilize user profiles and preferences to target/restrict content items. [IIF.ARCH.SERVICE.14]

IPTV_ARC_053: The IPTV Architecture shall support interactive services such as educational and entertainment applications, communications services (such as mail, chat, and messaging), and information services (such as stock and weather services). [IIF.ARCH.SERVICE.16]

IPTV_ARC_054: The IPTV Architecture shall provide mechanisms to support pre-positioned content services -- e.g., Push VOD. [IIF.ARCH.SERVICE.21]

IPTV_ARC_055: The IPTV Architecture shall support Audio Content (e.g., Music, Books) in the same manner that video content is supported (e.g., DRM, Encoding, Encapsulation, Delivery). [IIF.ARCH.SERVICE.29]

IPTV_ARC_056: The IPTV Architecture shall include the ability for games (and other applications) to be both associated with specific video services (i.e., bound to the playout of a specific video service) and unassociated with a specific video service (i.e., unbound to the playout of a specific video service). [IIF.ARCH.SERVICE.30]

IPTV_ARC_057: The IPTV Architecture shall allow for the search, download, storage, and presentation of digital pictures. [IIF.ARCH.SERVICE.34]

IPTV_ARC_058: The IPTV Architecture may provide mechanisms to convert digital pictures from its stored format into one that is compatible with that required to be displayed on the ITF supporting device. [IIF.ARCH.SERVICE.35]

Editor’s Note: for the following requirement, if we should give an example about national regulations, it should be not country-specific, we can use an ITU-T reference >

IPTV_ARC_059: The IPTV Architecture shall support Emergency Alert Service per the relevant national regulations. [IIF.ARCH.SERVICE.36]

IPTV_ARC_060: The IPTV Architecture shall provide appropriate interfaces to support the ingestion of suitably authorized Emergency Alert information. [IIF.ARCH.SERVICE.37]

IPTV_ARC_061: The IPTV Architecture shall support the ability for the ITF to decode and display emergency alert information. [IIF.ARCH.SERVICE.38]

IPTV_ARC_062: The IPTV Architecture shall support converged services [IIF.ARCH.SERVICE.52]

<comment> need some definition on converged services. It seems too high level requirements. It can be classified by media types, coding format, network capabilities, processing capabilities and application types, etc.

IPTV_ARC_063: The IPTV Architecture shall support the input feeds necessary to support the non-video components of the IPTV services. [IIF.ARCH.CONTENT.07]

IPTV_ARC_064: The IPTV Architecture shall provide mechanisms to block transmission of content to specified zones whenever blackout requirements are applicable. [IIF.ARCH.OPERATOR.03]

IPTV_ARC_065: The IPTV Architecture shall provide a mechanism that allows service signalling messages to be routed based on the capabilities of the end-user device and/or the application servers available to provide services. [IIF.ARCH.NETWORK.07]

IPTV_ARC_066: In an IPTV service environment, there is a need for signalling a set of capabilities, access conditions, end-user profile, etc. so that the service can be designed, deployed and operated with the relevant levels of quality and security.

IPTV_ARC_067: The IPTV Architecture shall provide a mechanism that allows for service-based transport QoS to be managed across multiple network domains. This function must include capabilities for transferring settlement information between transport providers. [IIF.ARCH.NETWORK.08]

IPTV_ARC_068: The IPTV Architecture shall provide a mechanism for NAT traversal. [IIF.ARCH.NETWORK.11]

IPTV_ARC_069: The IPTV Architecture shall provide mechanisms that allow private entities (e.g., residential users) to act as content-providers for the purpose of sharing their own content. This mechanism may need to deal with issues related to dynamic IP-address assignment, non DNS registered hosts, servers located on private networks behind NAT and FWs. [IIF.ARCH.NETWORK.12]

IPTV_ARC_070: The IPTV Architecture should provide for the establishment of sessions as a means of maintaining associations between users and the applications they use. [IIF.ARCH.NETWORK.13]

IPTV_ARC_071: Implementation of the mechanisms described in IIF.ARCH.NETWORK.07 – 13 shall be by means of the functions specified in NGN or extensions thereof. [IIF.ARCH.NETWORK.14]

IPTV_ARC_072: The IPTV Architecture shall not preclude the use of IMS mechanisms for all applicable IPTV functions. [IIF.ARCH.NETWORK.15]

#### 5.1.4.1 Segmentation by role

##### 5.1.4.1.1 Content Provider

IPTV_ARC_073: The content management should be provided to content providers, include uploading content, deleting content or modifying the relevant attributes of content.

IPTV_ARC_074: The statistic data of using frequency of content should be provided to content providers.

##### 5.1.4.1.2 Service Provider

IPTV_ARC_075: The system should be able to manage various services to ensure that the overall service can normally operate.

IPTV_ARC_076: The system should provide complete subscriber management functions such as opening an account or cancelling an account.

IPTV_ARC_077: The system should provide the functions for service usage and the prevention of the service abuse.

IPTV_ARC_078: The IPTV Architecture shall provide mechanisms for the IPTV Service Provider to Operate, Administer, Maintain, and Provision (OAMP) IPTV equipment.[ IIF.ARCH.CONTEXT.13]

IPTV_ARC_079: The IPTV Architecture shall facilitate the ability of the Service Provider to manage the IPTV service with regards to Fault, Configuration, Accounting, Performance, Security (FCAPS).[ IIF.ARCH.CONTEXT.14]

IPTV_ARC_080: The IPTV Architecture shall provide the capability for the Service provider to actively monitor the Service elements (e.g., ITF) to report alarms in the event of faults.[ IIF.ARCH.CONTEXT.16]

IPTV_ARC_081: The IPTV Architecture shall enable the service provider to configure the packages and package types available to the end-user. [IIF.ARCH.SERVICE.09]

IPTV_ARC_082: The IPTV Architecture shall provide a means for the service provider to provide an integrated presentation of content received at the ITF via out-of-band (to IP) delivery methods and via IP networks -- i.e., hybrid methods. [IIF.ARCH.SERVICE.63]

IPTV_ARC_083: The IPTV Architecture shall support the ability for service providers' appropriate service attachment parameters to be provided to the ITF. [IIF.ARCH.OPERATOR.16]

IPTV_ARC_084: The IPTV Architecture shall provide a mechanism that allows for service operators to allow IPTV service delivery from a 3rd party provider. This function should include capabilities for exchanging settlement information between the service operator and the 3rd party provider. [IIF.ARCH.NETWORK.09]

IPTV_ARC_085: Provide facilities for conversion of digital/analogue television signals onto an IPTV network stream

Editor’s note: Support for peer-to-peer services needs to be discussed in the following requirement.

IPTV_ARC_086: P2P mechanism in IPTV should not preclude the use of client server mechanisms in IPTV.

IPTV_ARC_087: The IPTV system shall provide navigation capability for appropriate IPTV content

IPTV_ARC_088: The service provider shall be able to authenticate the subscriber.

IPTV_ARC_089: The service provider shall be able to authenticate the end-user.

IPTV_ARC_090: The service provider shall be able to authenticate the end-user device.

IPTV_ARC_091: The service provider shall be able to distinguish between end-user and subscriber.

Service provider requirements for Remote Management

IPTV_ARC_092: The service provider shall be able to update the end-user device remotely.

IPTV_ARC_093: The service provider shall be able to upgrade the end-user device remotely.

IPTV_ARC_094: The service provider shall be able to get QoS information from the end-user device.

IPTV_ARC_095: The service provider shall be able to get the end-user device characteristics (e.g.: codecs, access limitations, etc).

Service provider requirements for Push Content

IPTV_ARC_096: The service provider shall be able to push content on the end-user device (content requested or not by the end-user).

IPTV_ARC_097: Service provider requirements for Content played from PVR.

IPTV_ARC_098: The service provider shall be able to insert contents in content played from the end-user PVR.

##### 5.1.4.1.3 Network Provider Requirements

IPTV_ARC_099: The networks should possess high usability and high reliability,

IPTV_ARC_100: The network is administrable and controllable.

IPTV_ARC_101: IPTV services must be delivered over networks that participate in providing the relevant level of quality associated to the range of IPTV services the end users may access and subscribe to. This level of quality may have (or should be) been negotiated then contractually defined between the customer and the service provider, by means of an IPTV service-specific Service Level Specification (SLS) that would include IPTV network-specific parameters, such as the one-way transit delay, the packet loss rate, etc.

IPTV_ARC_102: The IPTV Architecture shall provide the capability for the Network provider to actively monitor the network elements (e.g., IP Routers) to report alarms in the event of faults.[ IIF.ARCH.CONTEXT.15]

##### 5.1.4.1.4 Customer

User Requirements

IPTV_ARC_103: The IPTV should provide user the interactive experience of video-audio content

IPTV_ARC_104: The IPTV should provide long-term steady QoS

IPTV_ARC_105: The interactive operation for the user should be easy and convenient.

IPTV_ARC_106: The IPTV Architecture should support user’s privacy.

Multi-lingual Requirements

IPTV_ARC_107: The content provider may define a set of language options for its content.

IPTV_ARC_108: The end-user may choose a preferred language option (audio, subtitle, captioning, and descriptive audio track) from various languages that the content provider pre-defined and the service provider delivered.

IPTV_ARC_109: The end-user may alter his or her preferred language options at anytime.

IPTV_ARC_110: The content may have multiple language audio tracks, multiple language subtitles, multiple languages captioning, and multiple language descriptive audio track. One audio track, one subtitle, one captioning and multiple language descriptive audio track should be defined as a default language.

IPTV_ARC_111: The end-user may watch the program with the preferred audio, subtitle, captioning and descriptive audio track.

IPTV_ARC_112: If the end-user’s option can not match with the content languages, the end device should playback the content with default audio, default subtitle, default captioning, and default descriptive audio track.

IPTV_ARC_113: The end-user may switch audio tracks, subtitles, captioning and descriptive audio track back and forth when the user is watching the program without having to change his or her preferred language settings.

IPTV_ARC_114: The descriptive audio track, if present, shall be decoded if required.

IPTV_ARC_115: The end-user shall be able to turn on and off the audio, the subtitle, captioning, and descriptive audio track at anytime without altering any of the default setting options.

### 5.1.4 Relationship with other services and networks

IPTV_ARC_116: The IPTV system should provide capabilities for the interoperability and user mobility between different IPTV networks. For instance, the IPTV service could be accessed by the customer wherever he might be and whether he's in motion or not, possibly depending on the roaming agreements that may have been defined between participating IPTV network operators

IPTV_ARC_117: The IPTV Architecture shall support 3rd party content providers. [IIF.ARCH.SERVICE.13]

IPTV_ARC_118: The IPTV Architecture shall support end devices which constantly listen for EAN messages. [IIF.ARCH.SERVICE.41]

IPTV_ARC_119: The IPTV Architecture should allow network operators to leverage their existing network attachment capabilities. [IIF.ARCH.OPERATOR.12]

IPTV_ARC_120: The IPTV Architecture should provide a means by which service/network operators can integrate IPTV subscriber management functions with a subscriber management system that can be used across multiple NGN services, including IMS-based services. [IIF.ARCH.NETWORK.01]

IPTV_ARC_121: The integration of IPTV services in IMS-based environments should rely upon any relevant IMS-based NGN-specific capabilities, including (but not necessarily limited to) HSS/HLR and GUP [IIF.ARCH.NETWORK.02]

IPTV_ARC_122: The IPTV Architecture shall not preclude the related NGN functional components for all IPTV Subscriber Management functions. [IIF.ARCH.NETWORK.03]

<comment> it is not just for HSS, SLR.

(Editors Note, further clarification is required)

IPTV_ARC_123: Requirements of the interaction of IPTV and other NGN services

The network should provide the capability to associate the ongoing IPTV session with the ongoing NGN communication session of a user, and the network should realize the interaction between the two sessions.

IPTV_ARC_124: The IPTV server should interact with the Message server, or integrate the message capability, to provide some message related services

IPTV Interworking Requirements

<comment> The subsystem capability is a part of NGN. It is not relevant at this section. Each subsystem Messaging capability is duplicated with other requirement IPTV_ARC_053.

#### 5.1.4.1 Telecommunications

IPTV_ARC_126: TBD

#### 5.1.4.2 Service transparency (from network)

<comment> this section requests to move session 5.1.3.

Service Integration

IPTV_ARC_127: IPTV service should be integrated with the existing services such as POTS telephone, video conference etc. All these services may be provided through a unified service platform.

<comment> The IPTV service architecture should have the evolutional features in nature. It should adopt all the existing services. Then, this requirement move to architecture session 5.1.3.

IPTV_ARC_128: The IPTV architecture must allow the seamless design, deployment and operation of IPTV services across distinct operators' domains

<comment> It should consider whether this requirement should be moved at section 5.1.4.1 (segment by role). Also, it may be not relevant since this requirement is too general.

## 5.2 Requirements for QoS and Performance Aspects

<comment> It is not requirement. It requests the requirements on QoS and  performance parameters.

IPTV_QoS_010: The IPTV Architecture should provide a mechanism by which network operators can integrate IPTV transport QoS management functions into a common framework with other NGN services and applications. [IIF.ARCH.NETWORK.04]

IPTV_QoS_011: The integration of IPTV services into IMS-based environments should rely upon any relevant QoS capabilities, including (but not necessarily limited to) RACF and DiffServ capabilities. [IIF.ARCH.NETWORK.05]

IPTV_QoS_012: The IPTV Architecture shall not preclude the use of the emerging NGN-RACF for all IPTV Transport Policy and overall QoS functions. [IIF.ARCH.NETWORK.06]

IPTV_QoS_013: Video should be transmitted with sufficient quality for sign language perception if there is sign language in the contents. This requires at least 25 frames per second and sufficient spatial resolution to reproduce details of the signing person’s hands, face, lips, eyes and body.

IPTV_QoS_014: Video should be transmitted with sufficient quality for lip reading perception. This requires at least 25 frames per second and a sufficient spatial resolution to reproduce details of the speaking person’s lips and face.

IPTV_QoS_015: Video and audio should be transmitted and displayed with sufficient synchronism for lip-reading.

IPTV_QoS_016: Any on-Screen Display facilities for control of the terminal and the programs should be easy to read and not difficult to understand.

### 5.2.1 QoE

IPTV_QoS_017: The IPTV Architecture shall support an appropriate QoE for end-users eligible for uploading content to the service provider’s network. [IIF.ARCH.SERVICE.25]

IPTV_QoS_018: The IPTV Architecture shall provide mechanisms for supporting appropriate resiliency in the service provider infrastructure to maintain a high QoE for video services. [IIF.ARCH.OPERATOR.10]

IPTV_QoS_019: The IPTV service channel switch time should be short enough to satisfy the customer’s QoE requirements.

Editor’s Note: The following requirement is arguable… what about replacing zapping time instead of switch time? It is proposed that the channel switch time is about 2 seconds.

IPTV_QoS_020: Requirement on efficient operation of channel navigation

IPTV_QoS_021: Should support efficient navigation between different channels

<comment>  These QoE requirements_020 and _021 are too general. They does not give any specific QoE requirements. It should be modified to introduce specific requirements by considering navigation mechanism, filtering option, preference option, and human behaviour analysis, etc.

IPTV_QoS_022: The IPTV service should provide the capability to monitor video quality.

### 5.2.2 Performance

IPTV_QoS_023: The IPTV service shall satisfy IP QoS requirements specified in Y.1541.

IPTV_QoS_xxx: The IPTV service shall satisfy availability performance requirements.

IPTV_QoS_xxx: The IPTV service shall satisfy performance requirements on service interrupt time to support seamless connectivity during mobile communication environment.

### 5.2.3 Traffic Management

IPTV_QoS_024: The IPTV Architecture shall provide mechanisms to monitor/track age of transmitted packets. [IIF.ARCH.OPERATOR.26]

IPTV_QoS_025: The IPTV Architecture shall support a mechanism for assigning packet priority. [IIF.ARCH.OPERATOR.27]

IPTV_QoS_026: The IPTV Architecture shall support the relevant mechanisms for IPTV traffic identification, classification and marking, policing and conditioning, scheduling and discarding.

IPTV_QoS_027: The IPTV architecture shall provide mechanisms for dynamic IPTV traffic load balancing so as to dynamically accommodate with the network load and congestion conditions at any given time, so as to deliver the set of IPTV services to the end users with the relevant level of quality.

### 5.2.4 Bandwidth requirements and constraints

IPTV_QoS_028: The IPTV Architecture shall provide the capability for management of capacity on the services and network elements. [IIF.ARCH.SERVICE.07]

## Requirements for Service Security and Contents Protection Aspects

### 5.3.1 Digital Rights Management

IPTV_SEC_001: The IPTV Architecture shall provide appropriate security mechanisms with regards to gaming players’ identities and DRM for the games. [IIF.ARCH.SERVICE.32]

### 5.3.2	Content protection

IPTV_SEC_002: IPTV Architecture specification shall provide a mechanism for securing content. [IIF.ARCH.OPERATOR.11]

<comment> It is not relevant to indicate other De facto standards. It is enough to include as a reference. The specific requirements from ATIS-0800001 could  be included.

IPTV_SEC_004: The IPTV Architecture may include the ability for applications to interact with and be managed by the content management and protection capabilities. [IIF.ARCH.OPERATOR.32]

<comment> This requirement is not clear. It should be modified to indicate specific interaction requirements.

<comment>It requests the detail texts.

IPTV_SEC_006: A content provider will want to prevent a competitor from substituting elements of their content with competing content

<comment> It requests on relevant definitions whether the content provider can include broadcaster. This requirement should be checked with data integrity and data confidentiality at X.805 and Y.2701 (NGN security requirements) document. Some re-wording may be required.

IPTV_SEC_007: Content providers should be able to provide information about content in a way that can be identified by the end-user as to its source and protected from alteration by others in the value chain.

<comment> It requests on relevant definitions whether the content provider can include broadcaster, content producer and 3rd party metadata provider.

IPTV_SEC_008: An advertiser who has paid a premium price for a spot in a pod will wish to ensure they are the only people capable of changing it.

### 5.3.3	Security (e.g. conditional access)

IPTV_SEC_009: The IPTV Architecture shall provide one or more mechanisms to establish authorization before service delivery. [IIF.ARCH.OPERATOR.39]

IPTV_SEC_010: The IPTV Architecture shall provide mechanisms to support secure storage for the pre-positioned content. [IIF.ARCH.OPERATOR.40]

IPTV_SEC_011: The IPTV Architecture shall support the secure provisioning of equipment in the end-user network. [IIF.ARCH.OPERATOR.41]

IPTV_SEC_012: The IPTV Architecture shall provide mechanisms to enable the application of appropriate DRM on content. [IIF.ARCH.OPERATOR.08]

<comment> It is not clear. The specific DRM requirements are needed.

IPTV_SEC_013: Middleware SHOULD support security management function, such as user authentication, network security, software download authentication, and etc, by interoperating with the IPTV system.

<comment> The definition of middleware in views of security should be defined in alignment with X.805 and Y.2701. If the middleware security is a part of application security, this requirement should be modified. The security functions at User-/Control-/Management-plane should be clarified at this text..

Channel Access Control

IPTV_SEC_014: Fully allowed: customer can access the channel anytime

IPTV_SEC_015: Preview allowed: customer can have preview on the channel

IPTV_SEC_016: Not allowed: customer can’t access the channel

IPTV_SEC_017: Consider security capability at IP access network for IPTV service

High level Security requirement

IPTV_SEC_018: Enable security capabilities to protect end-to-end IPTV services across multiple network domains.

Editor’s note: Needs definition of end-to-end

Editor’s note: Consider Content security requirements

<comment> If the requirements are developed in detail, it should be aligned with X.805 and Y.2701. Is the content security a part of application security ?

IPTV_SEC_019: Consider Service security requirements

<comment> If the requirements are developed in detail, it should be aligned with X.805 and Y.2701

IPTV_SEC_020: The IPTV system should support capabilities for flagging content types. For instance this can be used for Parental control usage.

IPTV_SEC_021: IPTV applications shall support copy protection and authentication.

<comment> This is a part of IPTV_SEC_018 and IPTV_SEC_019.

### 5.3.4 Authentication

IPTV_SEC_022: The IPTV Architecture shall provide a mechanism to allow the service provider to force users of the service to participate in an authentication and authorization procedure with the network, before granting access to the service. [IIF.ARCH.OPERATOR.13]

IPTV_SEC_023: The IPTV Architecture shall provide mechanisms to control unauthorized access by unsubscribed end-users to the service. It may redirect unsubscribed end-users to a mechanism where they may subscribe. [IIF.ARCH.CONTEXT.18]

<comment> It duplicates with IPTV_SEC_012.

### 5.3.5 Authorization

IPTV_SEC_025: Channel authorization

## Requirements for Network and Control Aspects

IPTV_NET_001: Support multicasting and unicasting communication configurations together with interactive control

IPTV_NET_002: Support bound and unbound multicast group

<comment> it needs some definitions on bound/unbound service and group.

IPTV_NET_003: Support reliable data transmission

<comment>This is too general. It request to change specifically in order to identify the specific requirements. Otherwise, it request to move section 5.2 to develop the relevant QoS requirement.

IPTV_NET_004: Support QoS (Quality of Service) mechanisms to support IPTV QoS requirements.

<comment>This is too general. It does not indicate any specific requirements.

IPTV_NET_005: Support means for error handling

<comment>This is too general. Depending on IPTV service categories such as T-channel, T-interactive, T-VoD, etc., various error handling mechanisms such as FEC and ARQ could be applicable to meet the QoS requirements including japing time requirements.

IPTV_NET_006: Provide architectural requirements for p2p services of IPTV

<comment>It requests to decompose into specific requirements to support p2p service features. A number of p2p configurations could be modelled.

<comment> It is not clear at the current stage. What conditions require the distributed lookup ? What is the differences between distributed lookup and centralized lookup ?  Could the reliability requirements be divided into transport reliability and service reliability ?

<comment> It is not clear. It is not relevant at the current stage.

IPTV_NET_009: IPTV service provider should provide device or connection point identification to the user and provider for IPTV service

<comment> Generally, identification requirements for IPTV services can be classified into source aspects (e.g., provider side) and destination aspects (e.g., client side).  It should be developed according to layered concept of network identification and service identification. But, unfortunately, the service identification concept is not clear at the current IP technology since TCP/UDP port no. or URL information are not enough to identify IPTV services.

Also, there are a number of identifications such as end user identification, service identification, physical/logical connection identification. The end user could be identified into viewpoints of end user interface, end user device/system, and human itself.  In addition, a number of grouping and clustering could be identified into their own mechanism. Also, the channel identifier applicable at T-channel services could be  uniquely identified at specific local area and administrative domain. Then, this requirement should be modified into the relevant classification of identification mechanism.

In addition, the applications of identification can be used to identify EAN events and location identification as in IPTV_NET_017. It may request some collaborations of IDM (Identification Management) Focus Group.

It should be checked whether the relevant requirements are needed at section 5.4.3.

Editor’s note: need more clarification

<comment> It is not clear to indicate any specific distribution and delivery models of IPTV services.

<comment> It is not clear to assume any specific management requirements.

<comment> duplicate with IPTV_NET_013.

IPTV_NET_013: Shall support bi-directional communications to many terminals to provide interactive control

IPTV_NET_014: Consider providing functional requirements for p2p service in IPTV for:

Traffic Statistic and efficient data delivery

<comment> The same comment on IPTV_NET_006 is applied. The specific p2p requirements are needed in views of traffic statistics and data delivery.

IPTV_NET_015: The IPTV Architecture shall support mechanisms, inhibitors, and interfaces for the ITF to control the streaming of video content -- i.e., trick modes. [IIF.ARCH.SERVICE.03]

IPTV_NET_016: The IPTV Architecture may support a PVR/DVR function residing in either the end-user or service provider networks which supports content streaming control. [IIF.ARCH.SERVICE.04]

IPTV_NET_017: The IPTV Architecture shall provide mechanisms for the ITF to decode EAN messages which communicate the following information [IIF.ARCH.SERVICE.42]:

- A unique tag identifying the EAN event. This allows end devices to determine whether the event is new or a duplicate.

<comment> see the comment at IPTV_NET_009

- Originator of the EAN message.

- EAN Priority (National, State, or local).

- EAN Text for graphic overlay on current user programming

- Network address (assumed to be IP multicast address) of Emergency Alert Video and associated audio.

- Target Location ID -- this is the geographic area that is compelled to handle the EAN it consists of State and county code. If the priority is National, then all areas are compelled to process the EAN.

<comment> see the comment at IPTV_NET_009

- Exempted Channels -- the ID of content streams that are empted from the alert because they are direct processors of the EAN such that their content stream will contain the EAN without any intervention from the service provider.

- An option to set a trigger for the start time and either duration or end time of the EAN.

<comment> duplicate with IPTV_NET_024

IPTV_NET_019: The IPTV Architecture shall support the acquisition of content encoded with content advisories per ATSC A/65. [IIF.ARCH.SERVICE.46]

IPTV_NET_020: The IPTV Architecture shall provide mechanisms to support the users' ability to screen content based on content advisories. [IIF.ARCH.SERVICE.47]

IPTV_NET_021: The IPTV Architecture shall have the capability to support insertion of appropriately scoped advertisements -- e.g., geographically or demographically. [IIF.ARCH.SERVICE.48]

IPTV_NET_022: The IPTV Architecture shall support mechanisms for targeted advertising. [IIF.ARCH.SERVICE.49]

<comment> It needs some definition on targeted advertising.

IPTV_NET_023: The IPTV Architecture should support mechanisms for insertion of advertising content in non-video services. [IIF.ARCH.SERVICE.50]

IPTV_NET_024: The IPTV Architecture shall provide mechanisms to support the enforcement of parental controls in a manner consistent with ratings defined by recognized content advisory boards. [IIF.ARCH.SERVICE.59]

IPTV_NET_025: The IPTV Architecture shall provide mechanisms for the service provider to support the enforcement of parental controls on a user profile basis or a policy basis -- e.g., time limit. [IIF.ARCH.SERVICE.60]

IPTV_NET_026: The IPTV Architecture shall provide mechanisms to address notification messages to all, multiple, or a single user. [IIF.ARCH.SERVICE.61]

IPTV_NET_027: The IPTV Architecture shall have a mechanism to listen for notification messages, and display notifications to the user based on pre-set preferences -- e.g., do not disturb, service operator override. [IIF.ARCH.SERVICE.62]

IPTV_NET_028: IPTV network can support a service selection mechanism [DVB].

<comment> do not need to indicate the detail technology. It may be depending on service identification and discovery mechanisms.

IPTV_NET_029: IPTV network shall provide network time services for logging.

IPTV_NET_030: IPTV network can support for mobile terminal continuously IPTV service.

<comment> This requirement should be moved to section 5.2 QoS requirement with some modification to provide seamless handover.

IPTV_NET_031: IPTV network shall provide congestion avoidance mechanism.

### 5.4.1 Network Requirements

IPTV_NET_032: The IPTV Architecture shall support the management and enforcement of the service providers' transport policies by the network provider. [IIF.ARCH.OPERATOR.17]

IPTV_NET_033: The IPTV Architecture solution should include interfaces that allow service elements within the architecture to obtain information about the location of the end-user's end-point. [IIF.ARCH.OPERATOR.18]

<comment> refer to the comment on IPTV_NET_009. It requests to bind the location information with interface point.

IPTV_NET_034: If the IPTV Architecture includes mechanisms for accessing end-user location information, those mechanisms should comply with emerging standards for such information -- e.g., emerging ITU-T NGN standards for Network Attachment Control Function (NACF) interfaces. [IIF.ARCH.OPERATOR.19]

IPTV_NET_035: The IPTV Architecture shall define mechanism to appropriately distinguish different forms of traffic -- e.g., data and voice. [IIF.ARCH.OPERATOR.21]

IPTV_NET_036: The IPTV Architecture shall provide a mechanism for the service provider to authenticate the source of content. [IIF.ARCH.OPERATOR.22]

comment> This requirement should be moved to section 5.3 security requirement.

IPTV_NET_037: The IPTV Architecture shall support a mechanism for building associations between the non-video services and other stream types such as video and audio programs. [IIF.ARCH.OPERATOR.23]

IPTV_NET_038: The IPTV Architecture shall support a mechanism for identifying the classification of the non-video service (Independent, Real Time, and Synchronized). [IIF.ARCH.OPERATOR.24]

IPTV_NET_039: The IPTV Architecture shall provide a mechanism for the ITF to synchronize between different content streams. [IIF.ARCH.OPERATOR.28]

IPTV_NET_040: The IPTV Architecture shall support the following triggers: [IIF.ARCH.OPERATOR.29]

- Time In – Time used to trigger the consumption of the non-video service.

- Time-out – The time to stop the use of the object.

- Time-in and Time-out can also be used to indicate the window of relevance for the non-video service.  For instance, IPG data would use this to indicate that the entries cover a specific time window.

IPTV_NET_041: The IPTV Architecture shall support the following flags: [IIF.ARCH.OPERATOR.30]

- Mandatory – The non-video service can not be ignored.

- Discretionary –The ITF may choose to ignore the non video service.

- Advertisement – Declares the non-video service to be an advertisement.

- Unsolicited – The data is not in response to a user request.

- Disposition – This indicates what should be done with the non-video service once it has been consumed.  The disposition flag has several values including:

- Acknowledge – Send a message to the originator indicating it has been consumed.

- Persist - Stay resident for subsequent use, includes an expiration time (which may be a value indicating never).

IPTV_NET_042: The IPTV Architecture shall be robust in the presence of Denial of Service (DoS) attacks on or initiated by other devices in the home network. [IIF.ARCH.OPERATOR.36]

IPTV_NET_043: The IPTV Architecture shall be resistant to DoS attacks targeting the IPTV service infrastructure. [IIF.ARCH.OPERATOR.37]

IPTV_NET_044: The IPTV Architecture shall be hardened against attacks on any multicast VPN group management capabilities. [IIF.ARCH.OPERATOR.38]

<comment> It assumes that the multicast capability is a kind of VPN group.

IPTV_NET_045: The IPTV Architecture shall provide the capability for the ITF device to re-establish service without end-user intervention in the event of network outages.  [IIF.ARCH.OPERATOR.42]

IPTV_NET_046: For infrastructure components with device failure rates that are not insignificant, the IPTV service architecture shall provide redundancy and failover mechanisms. [IIF.ARCH.OPERATOR.43]

IPTV_NET_047: The IPTV Architecture shall support an option to provide a frequency reference from the Network Provider to the Service Provider that is traceable to national frequency standards. [IIF.ARCH.OPERATOR.44]

IPTV_NET_048: The IPTV Architecture should support an option to provide a time reference from the network provider to the service provider that is traceable to national time standards. [IIF.ARCH.OPERATOR.45]

<comment> Is it mandatory to provide time reference ?

IPTV_NET_049: Where the network provider transports multiple latency sensitive traffic types (e.g., the IPTV service and frequency distribution protocols, or time distribution protocols), these protocols and the network infrastructure should be configured such that they do not impact the latency requirements of each service. [IIF.ARCH.NETWORK.19]

<comment> It is mainly implementation issues to protect the IPTV service. It can move to security section if it is modified.

#### 5.4.1.1 Core & Metro Networks

IPTV_NET_051: TBD

#### 5.4.1.2 Access Network

IPTV_NET_052: TBD

#### 5.4.1.3 Management

IPTV_NET_053: TBD

### 5.4.2 Control protocol & signalling

IPTV_NET_054: The IPTV Architecture shall provide a means of signaling end users as to the occurrence of an Emergency Alert Notification (EAN) regardless of the services currently being consumed by the end user. [IIF.ARCH.SERVICE.39]

Requirements of transport protocol

<comment> It man duplicate with IPTV_QoS_022 for monitoring video traffic.

<comment> It may duplicate with IPTV_QoS_022 for monitoring video traffic.

IPTV_NET_057: Transport protocol should support physical server aware client

<comment> Should IPTV clients be aware of physical server ?

IPTV_NET_058: Transport protocol should support packet filtering

<comment> IPTV transport protocol have to support session-based or non-session based.

#### 5.4.2.1 Roaming and Service Handover for mobility

IPTV_NET_059: Support nomadism of End System

#### 5.4.2.2 Service transparency (network abstraction)

Unified service architecture

IPTV_NET_060: The IPTV architecture shall support unicast, broadcast and may optionally support multicast.

<Comment> The network model for unicast, multicast and broadcast is not the requirements. It requests to move to section 5.4.3.

IPTV_NET_061: Provide network transparency

<Comment> It needs a definition compared with service transparency. It assumes the encapsulation mechanisms independent of heterogeneous access and core transport networks.

IPTV_NET_062: Should not preclude service continuity over different network

<Comment> Is this text a requirement ? It looks like an implementation guideline.

### 5.4.3 Naming, addressing, and identification aspects

IPTV_NET_064: The IPTV Architecture shall support a mechanism for assigning IP-addresses and subnet masks to an attaching DNG. [IIF.ARCH.OPERATOR.14]

IPTV_NET_065: The IPTV Architecture should specify the use of existing standards and/or protocols to assign IP-addresses and subnet masks to an attaching user’s device -- e.g., DHCP. [IIF.ARCH.OPERATOR.15]

IPTV_NET_0xx: The IPTV architecture shall support existing naming, addressing and numbering schemes (including multicast).

<comment> It should be a separate requirement.

IPTV_NET_066: Provide NAT traversal

### 5.4.4 Routing and multicast session control

IPTV_NET_067: The IPTV Architecture shall support multicast means of communication to all end users. [IIF.ARCH.SERVICE.40]

IPTV_NET_068: The IPTV Architecture shall allow the service provider to utilize the network providers' multicast delivery capabilities. [IIF.ARCH.OPERATOR.25]

Requirement for the Multicast Feature of IPTV Bearer Network

IPTV_NET_069: The Multicast Feature of IPTV Bearer Network should construct the multicast service distribution tree in the backbone network

IPTV_NET_070: The Multicast Feature of IPTV Bearer Network should be adaptive to the changes of backbone network unicast routing

Requirements of stream session control

IPTV_NET_071: Stream session control protocol should support cross server session

IPTV_NET_072: Stream session control protocol should support representation of position in frame

IPTV_NET_073: Stream session control protocol should support carrying authentication/authorization token

IPTV_NET_074: Stream session control protocol should support carrying message signature

### 5.4.5 Content distribution

#### 5.4.5.1 Multicast and Broadcast distribution

IPTV_NET_075: IPTV receiver should select at least one version amongst several versions of the IPTV content following its own decision as well as condition.

<comment> It may be duplicate with section 5.4.3. Then, it requests to move to section 5.4.3. Otherwise, if it indicates access filtering capability on specific content by using a specific  multicast group, it should move to section 5.3 on  security requirements.

IPTV_NET_077: One multicast group address representing a IPTV channel

IPTV_NET_078: The multicast address of IPTV stream should interconnect some broadcasting service sources across multiple multicast bearer domains

IPTV_NET_079: The multicast address of IPTV stream coming from other domains should be optionally changed before entering local domain

IPTV_NET_080: The multicast address of IPTV stream should arrange the inquiry and registration on the PORTAL of carrier-provided Multicast Address and Service Control Platform

<comment> It is not a requirement.

### 5.4.6 Home Network

<comment> The definition of home network should be well defined. Is home network is a part of IPTV service domains ? If yes, change the domain or role boundaries of Content Provider, Service Provider, Network Provider and Customer at section 5.1.4.1. Otherwise, if home network is only considered within geographical architecture for IPTV service, the existing understanding of Customer Premises Network should be changed and compared. What is the real meaning of home network? It may have only geographical location points of view. It should not be confused with In-door/Out-door network and private/public carrier network. What is the difference with office/company/organization/enterprise networks ? Is there any meaning of home network in views of customer business model and consumer equipments inside home area ?

IPTV_NET_082: The Home Network (HN) shall support at least one In-door interface with sufficient bandwidth and QoS to support the IPTV service. This shall include the simultaneous transport of multiple video streams, in addition to voice and data traffic. [IIF.ARCH.HOME.09]

<comment> Would the IPI-4 interface be a standard interface within ITU-T ?

<comment> These are not the IPTV requirement. It is a kind of general principle.

IPTV_NET_085: The IPI-4c Interface, if supported, shall support the transport of digital information, in packet format, over RF carriers over coax. [IIF.ARCH.HOME.12]

IPTV_NET_086: The RF carrier(s) utilized to carry the digital information over the IPI-4c Interface shall not interfere with the standard CATV RF plan within the residence. To clarify further, the new RF carrier(s) used for the IPTV service shall either operate below the CATV-defined RF spectrum or above it. [IIF.ARCH.HOME.13]

IPTV_NET_087: The IPTV Architecture shall specify one or more fixed wireless instantiations of the IPI-4d interface. [IIF.ARCH.HOME.14]

IPTV_NET_088: The IPTV Architecture shall support at least one IPI-4 interface with sufficient bandwidth and QoS to support the IPTV service. [IIF.ARCH.HOME.15]

IPTV_NET_089: The IPTV Architecture shall support the ability to simultaneously transport multiple video streams and voice and data over an IPI-4 interface.  [IIF.ARCH.HOME.16]

IPTV_NET_090: The IPTV Architecture shall support the ability for the DNGF to implement standard IP routing functions, per established IETF specifications. [IIF.ARCH.HOME.17]

IPTV_NET_091: The IPTV Architecture shall support the ability for the DNGF to support routing functions per IPv4 specifications. [IIF.ARCH.HOME.18]

IPTV_NET_092: The IPTV Architecture should support the ability for the DNGF to support routing functions per IPv6 specifications. [IIF.ARCH.HOME.19]

IPTV_NET_093: The IPTV Architecture shall support the ability for the DNGF to support the routing of IP packets between all interfaces toward the access/core network (WAN side of the HN), and toward the ITF (LAN side of the HN). Specifically, this means: [IIF.ARCH.HOME.20]

- Routing shall be supported from DN to HNS (downstream flow).

- Routing shall be supported from HNS to DN (upstream flow).

- Routing shall be supported from any HNS to any other HNS (bidirectional flows).

IPTV_NET_094: The IPTV Architecture shall define a means for the DNG, upon installation and power up, to attach to the network at the IP layer, and obtain an IP address from the network provider. [IIF.ARCH.HOME.21]

IPTV_NET_095: The IPTV Architecture shall define mechanisms to support the authentication procedures required by the network and service providers for home network elements. [IIF.ARCH.HOME.22]

IPTV_NET_096: The IPTV Architecture shall support the ability for the DNGF to support multiple logical IP interfaces (multiple attachment points at the IP layer) on any particular physical DN interface. [IIF.ARCH.HOME.23]

IPTV_NET_097: The IPTV Architecture shall define traffic management mechanisms for the differential treatment of IPTV traffic. [IIF.ARCH.HOME.24]

IPTV_NET_098: The IPTV Architecture shall support the ability for the DNGF to assign IP addresses to devices in the Home Network. [IIF.ARCH.HOME.25]

IPTV_NET_099: The IPTV Architecture shall specify a means for learning detailed information (e.g., ITF’s capabilities and manufacturer) from the ITFs. [IIF.ARCH.HOME.26]

IPTV_NET_100: The IPTV Architecture shall support the ability for the DNGF to present the entire home network through one or more WAN (public) IP addresses, which are used for all external communication. [IIF.ARCH.HOME.27]

IPTV_NET_101: The IPTV Architecture shall support the ability for the DNGF to support NAT/NAPT capability mapping IP addresses and port numbers between the public WAN and the LAN(s). [IIF.ARCH.HOME.28]

IPTV_NET_102: To protect the home network from malicious or unauthorized access, the IPTV architecture shall support the ability for the DNGF to establish a firewall, with multiple levels of security and appropriate application level gateways. [IIF.ARCH.HOME.29]

IPTV_NET_103: The Home Network shall support at least one HNS IPI-1 interface with sufficient bandwidth to support the IPTV service. [IIF.ARCH.HOME.41]

IPTV_NET_104: If IP transmission over power lines is supported in the Home Network, the IPTV Architecture shall support one or more instantiations of the IPI-1a interface. [IIF.ARCH.HOME.42]

IPTV_NET_105: If Ethernet over point-to-point CAT5 copper interface is supported in the home network, the IPI-1b Interface shall comply with standard IEEE 802.3 10/100/1000BaseT specifications. Multiple instances (multiple lines) could exist between the DNGF and the different devices implementing the ITFs. [IIF.ARCH.HOME.43]

IPTV_NET_106: If Ethernet over copper telephony wiring is supported in the home network, the IPI-1c Interface shall comply with standard HomePNA over copper specifications, per ITU-T G.9954. IIF.ARCH.HOME.44

IPTV_NET_107: If coaxial HNS networking is supported in the home network, the IPI-1d interface shall comply with coaxial transport specifications listed below: [IIF.ARCH.HOME.45]

I)	The first is based on HomePNA 3.0, per ITU-T G.9954.

II)	The second is based on a recommendation offered by the Multimedia over Coax Alliance (MoCA), which is a consortium of companies and not a formal standards organization. More information can be found at < http://www.mocalliance.org >.

III)	The third is based on TVnet technology offered by Coaxsys, which argues for a simple solution that does not require software to operate.

NOTE - There are at least three competing technologies for the transport of digital data over coax.

IPTV_NET_108: If wireless HNS interface is supported in the home network, the IPI-1e Interface shall comply with standard WiFi specifications, per 802.11 a/g/n. [IIF.ARCH.HOME.46]

The IPTV Architecture shall support the ability for the ITF to support channel change functionality.  [IIF.ARCH.HOME.54]

IPTV_NET_109: The IPTV Architecture shall support the ability for the DNGF to support channel change functions between the ITF and service provider. [IIF.ARCH.HOME.55]

IPTV_NET_110: The IPTV Architecture shall support the ability to provision the DNG and ITF remotely. [IIF.ARCH.HOME.56]

IPTV_NET_111: The IPTV Architecture shall support the ability to collect status information from the DNG and ITF remotely. [IIF.ARCH.HOME.57]

IPTV_NET_112: The IPTV Architecture shall provide mechanisms for transport of time of day to the Home Network. [IIF.ARCH.HOME.58]

IPTV_NET_113: The IPTV Architecture shall identify circumstances under which a frequency distribution protocol may be required to support aspects of IPTV services. [IIF.ARCH.HOME.59]

IPTV_NET_114: Where the Home Network transports multiple latency sensitive traffic types (e.g., the IPTV service and frequency distribution protocols, or time distribution protocols), these protocols and the Home Network infrastructure should be configured such that they do not impact the latency requirements of each service. [IIF.ARCH.HOME.60]

QoS

IPTV_NET_115: The IPTV Architecture should support the ability for the DNGF to perform the bandwidth management of all HNS networks attached to the DNG. [IIF.ARCH.HOME.30]

IPTV_NET_116: The IPTV Architecture shall support the ability for the DNGF to be the arbitrator for local traffic traversing from one HNS network to another HNS network, attached to the DNG. [IIF.ARCH.HOME.31]

IPTV_NET_117: The IPTV Architecture should support the ability for the DNGF to perform call admission functions to protect the home network from excessive and harmful traffic on any HNS, between two HNSs attached to the DNG, and between any HNS and any DN. [IIF.ARCH.HOME.32]

IPTV_NET_118: The IPTV Architecture should support the ability for the DNGF to perform policing functions on incoming traffic and drop offending traffic to protect the home network. [IIF.ARCH.HOME.33]

IPTV_NET_119: The IPTV Architecture shall support the ability for the DNGF to enforce proper queuing, scheduling, and prioritization mechanisms to guarantee the processing of higher priority flows and to meet SLA on traffic from the end-user network to the delivery network. [IIF.ARCH.HOME.34]

IPTV_NET_120: The IPTV Architecture shall support the ability for the DNGF to route IP traffic based on provision-able/manageable mechanisms to guarantee QoS for different service classes. [IIF.ARCH.HOME.35]

IPTV_NET_121: The IPTV Architecture shall support the ability for the DNGF to support IP filtering functions to prevent selected local multicast traffic on the HNS from appearing on the DN. [IIF.ARCH.HOME.36]

IPTV_NET_122: The IPTV Architecture shall support the ability for the DNGF to map downstream traffic to corresponding local flows to provide QoS for the different services. This includes L3 to L2 mapping, based on the local HNS. [IIF.ARCH.HOME.37]

IPTV_NET_123: The IPTV Architecture shall support the ability for the DNGF to map upstream traffic generated by the end-devices to corresponding outgoing flows to provide QoS for the different services. This includes L2 to L3 mapping. [IIF.ARCH.HOME.38]

IPTV_NET_124: The IPTV Architecture shall support the ability to provision the DNGF with QoS rules that govern traffic mapping (upstream or downstream) for the different services. [IIF.ARCH.HOME.39]

IPTV_NET_125: The IPTV Architecture shall support the ability of the DNGF to classify traffic based on factors such as: [IIF.ARCH.HOME.40]

## 5.5 Requirements for End Systems and Interoperability aspects

IPTV_ESI_001: Provide the device transparency and interoperability

IPTV_ESI_002: IPTV should allow the user to connect to multiple service providers

IPTV_ESI_003: Provide Terminal Management requirement for IPTV

IPTV_ESI_004: Requirement # 1: The IPTV content should be delivered in several yet optional versions to be selected according to the capabilities of the IPTV receiver (e.g. access rate, resolution, supported formats, etc.).

IPTV_ESI_005: Requirement # 2 (initial proposal): The IPTV content may have a means to be selected according to the bandwidth restrictions of the access network of the IPTV receiver.

IPTV_ESI_006: Requirement # 2.1 (reworded based on Req 2)The IPTV system may support signalling capabilities for transmitting bandwidth related information

IPTV_ESI_007: Requirement # 2.2 (reworded based on Req 2) The IPTV system shall be able to identify the available bandwidth to the IPTV terminal

IPTV_ESI_008: Requirement # 2.3 (reworded based on Req 2) The IPTV system may use the bandwidth related information to determine the appropriate content coding means to deliver the content.

IPTV_ESI_009: Requirement # 2.4 (reworded based on Req 2) The IPTV terminal may be able to indicate to the IPTV system the available bandwidth

IPTV_ESI_010: The IPTV system may use the bandwidth related information to determine the appropriate content coding means to deliver the content.

IPTV_ESI_011: The IPTV receiver may be able to indicate to the IPTV system the available bandwidth information.

(Editors Note: the requirements 2.1, 2.2, 2.3 and 2.4 need further elaboration contribution # 147)

IPTV_ESI_012: Should have the ability to view the captions in a separate window and vary its presentation, different colours for different speakers, larger font etc. (Middleware).

IPTV_ESI_013: IPTV middleware shall be capable of performing self-diagnostics information such as power status, boot status, memory allocation, software version, middleware version, network address, and network status.

IPTV_ESI_014: IPTV application can support device management information to the service provider or the network provider.

IPTV_ESI_015: IPTV application can support the download of software based on transport protocols and security systems.

IPTV_ESI_016: The IPTV Architecture shall support the ability for the ITF to decode video and audio and present the video content to standard end-user electronics interfaces. [IIF.ARCH.HOME.49]

IPTV_ESI_017: The IPTV Architecture shall provide the capability of receiving and correctly processing the metadata for content coming from the content providers. [IIF.ARCH.CONTENT.02]

5.5.1 End-user requirements

#### 5.5.1.1 End-user requirements for Linear/Broadcast TV (audio, video and data)

IPTV_ESI_018: The IPTV architecture shall provide the end-user ability to end-user watch all the TV Channels offered for free or from the IPTV Service Provider if subscribed.

IPTV_ESI_019: The IPTV architecture shall provide the end-user ability to switch from any authorized channel to another in an easy way and as fast as possible using remote control keys (e.g. channel up, channel down, channel number).

IPTV_ESI_020: The IPTV architecture shall provide the end-user ability to watch different TV channels on different end-user terminals if the delivery network allows the simultaneous delivery of several TV channels to the end-user premises.

IPTV_ESI_021: The IPTV architecture shall provide the end-user ability to end-user switch between audio flows, if more than one is available.

IPTV_ESI_022: When several audio flows are available with a TV channel, the end-user terminal will render the preferred audio flow.

IPTV_ESI_023: The IPTV architecture shall provide the end-user ability to discover and display any additional information delivered with the TV channel.

IPTV_ESI_024: The IPTV architecture shall provide the end-user ability to discover and display the closed caption flow delivered with the TV channel.

IPTV_ESI_025: When several closed caption flows are available with a TV channel, the end-user terminal will render the preferred closed caption flow.

#### 5.5.1.2 End-user Requirements for Linear/Broadcast TV (audio, video and data) with trick mode

IPTV_ESI_026: The IPTV architecture shall provide the end-user the ability to pause and later resume watching a TV channel.

IPTV_ESI_027: The IPTV architecture shall provide the end-user the ability to replay what he has just been watched on a TV channel.

IPTV_ESI_028: The IPTV architecture shall provide the end-user the ability to pause and later play in fast forward mode to catch up with the normal delivery time.

#### 5.5.1.3 End-user Requirements for Time-shift TV (Near VoD broadcasting)

IPTV_ESI_029: The IPTV architecture shall provide the end-user the ability to easily identify the TV channels delivering the selected content and the relative progress position in the delivery of this content.

IPTV_ESI_030: The IPTV architecture shall provide the end-user the ability to end-user switch easily between the TV channels which are delivering or about to deliver the same content but at different delivery time using the remote control keys (e.g. channel up, channel down, etc.)

#### 5.5.1.4 End-user Requirements for VoD

IPTV_ESI_031: The IPTV architecture shall provide the end-user the ability to discover and browse content catalogue(s).

IPTV_ESI_032: The IPTV architecture shall provide the end-user the ability to select contents using a single criterion or a combination of criteria such as title, reference, genre, keyword, director, actor, etc.)

IPTV_ESI_033: The IPTV architecture shall provide the end-user the ability to get the selected content streamed after authorization and maybe payment.

IPTV_ESI_034: The IPTV architecture shall provide the end-user the ability to pause and later resume watching the selected content.

IPTV_ESI_035: The IPTV architecture shall provide the end-user the ability to play a VoD content.

IPTV_ESI_036: The IPTV architecture shall provide the end-user the ability to jump backward or forward in a VoD content and to play it from that point.

IPTV_ESI_037: The IPTV architecture shall provide the end-user the ability to watch a VoD content in fast forward mode and to stop whenever desired.

End user Requirements for Push VoD

IPTV_ESI_038: The IPTV architecture shall provide the end-user the ability to receive a push VoD content without end-user(s) being disturbed (e.g. when watching a TV channel, recording a content on a PVR, downloading from the internet, having a videoconference, etc.) and without disturbing the devices engaged in providing a service (e.g. recording a content on a PVR, etc.).

IPTV_ESI_039: The IPTV architecture shall provide the end-user the ability to receive unsolicited push VoD content with no impact on the storage allocated to the user.

IPTV_ESI_040: The IPTV architecture shall provide the end-user the ability to be informed after push VoD Content reception is completed if not otherwise stated.

IPTV_ESI_041: The IPTV architecture shall provide the end-user the ability to receive content without being disturbed. (e.g. when watching a TV channel, recording a content on a PVR, downloading from the internet, having a videoconference, etc.) and without disturbing the devices engaged in providing a service (e.g. a recorder recording a content on a PVR, etc.).

IPTV_ESI_042: The IPTV architecture shall provide the end-user the ability to receive a push VoD content without impacting the storage allocated for the user contents.

#### 5.5.1.5 End-user Requirements for Client PVR service

IPTV_ESI_043: Requirements from TV Anytime specification TS 102 822 Part 01

IPTV_ESI_044: The IPTV architecture shall provide the end-user ability to capture and play back content on a Personal Digital Recorder (PDR).

IPTV_ESI_045: The IPTV architecture shall provide the end-user ability to pause live incoming content on a PDR so that they can "resume" later and continue to watch the content in time shift mode.

IPTV_ESI_046: The IPTV architecture shall provide the end-user ability to view an on-screen menu of content already captured.

IPTV_ESI_047: The IPTV architecture shall provide the end-user ability to view a schedule of forthcoming items so they can choose content to record.

IPTV_ESI_048: The IPTV architecture shall provide the end-user ability to choose whether a new recording of content replaces existing content that is out of date or is added alongside old content on the PDR.

IPTV_ESI_048: The IPTV architecture shall provide the end-user ability to decide whether to capture single or multiple episodes of a series or other programme groupings.

IPTV_ESI_049: The IPTV architecture shall provide the end-user ability to amend the list of items "cued" for capture.

IPTV_ESI_050: The IPTV architecture shall provide the end-user ability to record the content of the buffer and continue recording so the entire content is captured if the device is already tuned to a particular source and has been buffering that content in memory. If the device was not turned to that output there may be a desire to indicate it at the next availability

IPTV_ESI_051: The IPTV architecture shall provide the end-user ability to set up and manage multiple personal profiles on their PDR associated with one or more service providers.

IPTV_ESI_052:  The IPTV architecture shall provide the end-user ability to manage the storage space on their PDR system or give an appropriate provider(s) permission to do so e.g. items to be deleted next, permanently stored, etc.

IPTV_ESI_053: The IPTV architecture shall provide the end-user ability to allow the PDR to automatically capture content based on consumer viewing behaviour (profiling).

IPTV_ESI_054: The IPTV architecture shall provide the end-user ability to allow capturing of consumer profile, so it can be aggregated and analysed for targeting purposes.

IPTV_ESI_055: The IPTV architecture shall provide the end-user ability to allow insertion of pre-captured advertisements or promotions into live/broadcast content based on the consumer viewing profile.

IPTV_ESI_056: The IPTV architecture shall provide the end-user ability to allow the insertion of pre-captured advertisements or promotions into content being played back based on the consumer viewing profile.

IPTV_ESI_057: The IPTV architecture shall provide the end-user ability to allow a service provider to remotely control the functionality of the consumer PDR system (e.g. to capture settings, profile settings, etc.).

IPTV_ESI_058: The IPTV architecture shall provide the end-user ability to select segments of programmes for recording based on information provided by the service or content provider.

IPTV_ESI_059: The IPTV architecture shall provide the end-user ability to view content stored on a PDR system in a similar way to viewing content on a DVD - e.g. with index points and a play list enabling "passive" highlight or other playback modes.

IPTV_ESI_060: The IPTV architecture shall provide the end-user ability to navigate and explore content segments using provider indexes (e.g. step through, short/long form, etc.).

IPTV_ESI_061: The IPTV architecture shall provide the end-user ability to request PDR system to create single personalized programmes from individual "personally linked" segments.

IPTV_ESI_062: The IPTV architecture shall provide the end-user ability to create separate profiles for each member of the household (separate recorded content menus, profiling, parental control, etc.).

IPTV_ESI_063: The IPTV architecture shall provide the end-user ability to capture high value premium content. Service/content providers will require flexible usage rules (limited viewing windows for example) so that end-users can view their content on the PDR system.

IPTV_ESI_064: The IPTV architecture shall provide the end-user ability to capture high value premium "content on demand". Service/content providers will require flexible pricing information so that end-users can select the content of their choice within a selection of commercial offers.

IPTV_ESI_065: The IPTV architecture shall provide the end-user ability to store, on a bi-directional PDR system, "personal" content on network storage devices. (e.g. if the end-user disk is full).

IPTV_ESI_066:  The IPTV architecture shall provide the end-user ability to move personal profiles to different PDRs or PDR systems in other physical locations. (e.g. when upgrading the devices or while viewing in a hotel when on holiday).

Editor’s Note: These two bullets are not requirements per se. Contributors are encouraged to elaborate on such "requirements", provide rewording accordingly and relocating them in the appropriate section.

- 3rd parties or service/content providers can provide recommendations, content referencing and resolution of content potentially from many other providers.

- Service/content providers can force download "premium/PPV" content to the PDR system (i.e. Local VOD).

Editor’s note: the following requirement to be relocated in the appropriate section

IPTV_ESI_067: The IPTV architecture shall provide the end-user ability to capture content onto the portable device over a wired or wireless network and transfer that content with associated metadata to their home device and other mobile devices

IPTV_ESI_068: The IPTV architecture shall provide the end-user ability to make available personal information to another user within the home network or to a friend through an external transmission path, or any removable exchange medium available.

IPTV_ESI_069: The IPTV architecture shall provide the end-user ability to capture, through PDR, a complete interactive package such as interactive TV that contains applications, data, text, graphics, video and audio files and links to other online content.

IPTV_ESI_070: The IPTV architecture shall provide the end-user ability to move between a free to air (FTA) and pay per view (PPV) environment when a broadcaster transmits both FTA and PPV content that are linked.

IPTV_ESI_071: The IPTV architecture shall provide the end-user ability to record, through PDR, elements of content (such as games) from trusted content providers and be told the best order in which to consume this content

IPTV_ESI_072: The IPTV architecture shall provide the end-user ability to request, by setting manual preferences, that all his mobile PDR content contain mostly audio or television programmes containing audio description

IPTV_ESI_073: The IPTV architecture shall provide the end-user travelling abroad ability to set preferences that allows capturing content relevant to the home location preferences or personal preferences

IPTV_ESI_074: The IPTV architecture shall provide the end-user ability to have available personal preferences when using rented TV-Anytime equipment, and that equipment is capable of will recording content of interest, regardless of location.

IPTV_ESI_075: The IPTV architecture shall provide the end-user ability to capture enhancing content in advance of a broadcasted event so that when the main element is broadcast; those enhanced features are available on the TV-Anytime device for synchronous playback

IPTV_ESI_076: The IPTV architecture shall provide the end-user ability to play complex games that include pre-recorded content and links to broadcast programming.

IPTV_ESI_077: The IPTV architecture shall provide the end-user ability to have PDR seamlessly and automatically update time sensitive content (such as news and advertising) recorded on the device.

IPTV_ESI_078: The IPTV architecture shall provide the end-user ability to pre-record programme associated content such as subtitles, captions and additional textual information so that during a subsequent live broadcast programme PDR can offer alternative, synchronised subtitles, captions and background textual information.

IPTV_ESI_079: The IPTV architecture shall provide the end-user ability to capture a multi-stream offering in such a way that when playing the content at a later date the different streams remain synchronous.

IPTV_ESI_080: The IPTV architecture shall provide the end-user ability to set preference between using content off the PDR devices in ‘subtitle text’ mode rather than audio streams.

IPTV_ESI_081: The IPTV architecture shall provide the end-user ability be certain that any content searched for is playable on the device

IPTV_ESI_082: The IPTV architecture shall provide the end-user who subscribes to content service providers, certainty of suitability of the content with what has been subscribed to is appropriate for each or all of his devices.

IPTV_ESI_083: The IPTV architecture shall provide the end-user ability to acquire content while on the move, and for his TV-Anytime service to know the terminal capabilities before delivering content

Editor’s Note: The following requirement doesn't seem to be strictly relevant to IPTV service design or operation requirements considerations, but rather suggests content owner/provider-specific issues.

IPTV_ESI_084: An advertiser will want to ensure that any replacement of advertising spots complies with national, regional advertising rules and regulations

Editor’s Note: Discuss the relevancy of such wording in a requirements document

IPTV_ESI_085: Charging and billing capabilities of an IPTV service architecture should make sure that end users will be charged for content once the free viewing period (to be possibly proposed and configured by the content owner or provider) has expired.

IPTV_ESI_086: The IPTV architecture shall provide the end-user ability to make purchases after seeing promotional or advertising features downloaded to the PDR

IPTV_ESI_087: The IPTV architecture shall provide the end-user ability to choose what to record using 3rd party metadata services. I should be possible to update the PDR recordings.

IPTV_ESI_088: The IPTV architecture shall provide the end-user ability to programme the PDR and query it remotely from a mobile device or device connected to the internet

IPTV_ESI_089: The IPTV architecture shall provide the end-user ability to set preferences on the device that allow it to capture subtitles or audio description in his native language when programmes are transmitted in a foreign language

IPTV_ESI_090: The IPTV architecture shall provide the end-user ability to receive from a broadcaster generic metadata to the PDR where the content is not yet fully established (e.g. a series of sporting events with a flexible timing schedule). The device should be able to record the whole event, and then discard unwanted elements when the broadcaster updates the metadata

IPTV_ESI_091: The IPTV architecture shall provide the end-user ability to select content to record depending on its broadcast characteristics such as broadcast quality, aspect ratio etc.

IPTV_ESI_092: The IPTV architecture shall provide the end-user ability to capture more streams than the PDR is capable of recording. In this instance it is desired to be able to “book” a recording on a service provided outside PDR and on a “Network Digital Recorder” which can then be transferred to PDR when it is free to receive it

IPTV_ESI_093: The IPTV architecture shall provide the end-user ability to see relevant, timely interstitials (commercials and promos) when time-shifts viewing of content. Where such content is out of time, the content should be replaced with more appropriate material.

IPTV_ESI_094: The IPTV architecture shall provide the end-user to pause what is being watched, , view the additional content, and then resume the original material from where he has paused when the broadcaster or advertiser provides additional material (a telescope advert for instance).

IPTV_ESI_095: The IPTV architecture shall provide the end-user ability to choose from a variety of payment models when viewing content with different amounts of interstitial content and depending on the cost structure.

Editor’s Note: the two following requirements don't seem to be strictly IPTV service-specific and/or should deserve some elaboration (second "requirement" suggests the notion of "targeting rules", which are yet-to-be defined): Once reword it, have to be relocated accordingly.

IPTV_ESI_096: An advertiser will want to ensure that commercials for similar brands are not shown close to their own spots.

IPTV_ESI_097: Where national compliance rules for the showing of commercials apply, these rules should over-ride any other targeting rules

IPTV_ESI_098: The IPTV architecture shall provide the end-user ability to see a limit on commercial advertisements. To prevent negative brand impressions, advertisers may therefore set an upper limit for each advertisement in a program.

IPTV_ESI_099: The IPTV architecture shall provide the end-user ability to set a preference to see a specific product or range of products depending on the purchasing needs or habits or interests.

IPTV_ESI_100: The IPTV architecture shall provide the end-user ability to set a preference to block specific products or range of products based on the purchasing needs, habits of lifestyles.

IPTV_ESI_101: The IPTV architecture shall provide the end-user ability to pay a premium to replace advertising pods in certain events (such as live sport) with relevant editorial programme content

IPTV_ESI_102: The IPTV architecture shall provide the end-user ability to print text, images and other associated assets that broadcasters make available as enhancements to their programmes

IPTV_ESI_103: The IPTV architecture shall provide the end-user ability toget the content selected to be recorded, even if there is a schedule change recoverable by the PDR (delayed broadcast, no overlapping with other programmed recording, etc.) without any specific customer action.

5.5.1.6 End-uses Requirements for Regulatory Information services

IPTV_ESI_104: The IPTV architecture shall provide the end-user ability to be notified and receive regulatory information services (unassociated with TV channels) whenever his terminal is active (e.g. displaying an EPG or a TV channel).

IPTV_ESI_105: The reception of regulatory information services shall trigger mechanisms to draw the customer attention.

#### 5.5.1.7 End-users Requirements for Service Information

IPTV_ESI_106: The IPTV architecture should provide the end-user capability to watch a list of the TV channels authorized to watch (free or subscribed channels).

IPTV_ESI_107: The IPTV architecture should provide the end-user capability to tune to a TV channel from the displayed list of authorized TV channels.

IPTV_ESI_108: The IPTV architecture should provide the end-user capability to configure or watch a short list of the TV channels among the channels he is authorized to watch (free or subscribed channels).

IPTV_ESI_109: The IPTV architecture should provide the end-user capability to tune to a TV channel from the displayed list of authorized TV channels.

IPTV_ESI_110: The IPTV architecture should provide the end-user capability to get information about the available TV channels (name, logos, owner, web site, etc.)

IPTV_ESI_111: The IPTV architecture should provide the end-user capability to get information about the TV channel programmes (event title, summary, start time, end time, genre, etc.) for the authorized channels.

IPTV_ESI_112: The IPTV architecture should provide the end-user capability to see TV channel programmes, filtered by time (now, this evening, Sunday afternoon, etc).

IPTV_ESI_113: The customer shall be able to see TV channel programmes, filtered by user preferences (list of selected channels, genre, director, etc)

End-users Requirements for unassociated Information Services

IPTV_ESI_114: The Regulatory Information Services are specific unassociated Information Services. Examples of unassociated information services: news highlights, stock exchange, traffic conditions, sport results, etc.

IPTV_ESI_115: The customer shall be able to receive information about broadcast information services unassociated to VOD contents or TV channels such as news highlights, stock exchange, traffic conditions, sport results, etc.

IPTV_ESI_116: The customer shall be able to select one or more unassociated information service(s) for scrolled display while watching TV or VoD.

ER for associated Information Services

IPTV_ESI_117: Examples of associated information services: links to other web sites, betting, voting, etc.

IPTV_ESI_118: The customer shall be warned when information associated to the video he is watching is available over than the metadata related to the content, e.g.: a link towards an on-line shopping site, a link in a commercial towards a call center or a company web site, a link towards a web site for further information

IPTV_ESI_119: The customer shall be able to turn off the warning for availability of additional information linked to watched contents.

IPTV_ESI_120: The customer shall be given the possibility to see the associated information (e.g. Pressing a specific remote control key) and to act upon it (e.g. to follow the link, to make a choice, to bet, to return, etc.).

### 5.5.2 Implementation scenarios and applications

IPTV_ESI_121: TBD

#### 5.5.2.1 Device Transparency

IPTV_ESI_122: TBD

### 5.5.3 Terminals

IPTV_ESI_123: The IPTV Architecture may support the ability to store, cache, update, and run applications on devices that implement the ITF. [IIF.ARCH.SERVICE.18]

IPTV_ESI_124: The IPTV Architecture may support the ability for applications to be delivered to ITF implementing devices with a video service (i.e., streamed with the video), by user request over the 2-way network connection, and by the service operator at the service operator’s direction. [IIF.ARCH.SERVICE.19]

IPTV_ESI_125: The IPTV Architecture may support the ability of ITF implementing devices to serve as a master monitor or switcher application that can be updated by the service operator and that can manage the execution of other applications [IIF.ARCH.SERVICE.20]

IPTV_ESI_126: The IPTV Architecture shall provide an extensible method for the service provider to query for the ITF's capabilities and status. [IIF.ARCH.OPERATOR.33]

IPTV_ESI_127: The IPTV Architecture shall be capable of handling values represented in the response to the device inquiry, which may have no rigidly defined order or structure. [IIF.ARCH.OPERATOR.34]

IPTV_ESI_128: The IPTV Architecture shall be capable of verifying the digitally signed response to the discovery query from the ITF device so as to prevent spoofing. [IIF.ARCH.OPERATOR.35]

IPTV_ESI_129: The IPTV Architecture shall identify mechanisms for the service provider to manage the ITF and associated peripheral devices (e.g., display or storage devices) and the physical devices that implement the ITF. [IIF.ARCH.HOME.47]

IPTV_ESI_130: The IPTV Architecture may support the ability for service provider-managed applications to be stored and executed in the devices implementing the ITF. [IIF.ARCH.HOME.50]

IPTV_ESI_131: The IPTV Architecture may support the ability for video service-associated applications to be stored and executed in the devices implementing the ITF. [IIF.ARCH.HOME.51]

IPTV_ESI_132: The IPTV Architecture may support user-requested applications to be stored and executed in the devices implementing the ITF. [IIF.ARCH.HOME.52]

IPTV_ESI_133: The hybrid broadcast/broadband terminal devices shall have interoperability between both interfaces for contents guide which provides service lists and other information to the user

Device security requirements

IPTV_ESI_134: Content protection

IPTV_ESI_135: Content key protection

IPTV_ESI_136: Device secret protection

IPTV_ESI_137: User information protection

IPTV_ESI_138: Software protection

IPTV_ESI_139: Hardware tamper resistance

IPTV_ESI_140: Providing secure communication method and authenticationProtection from malicious code

IPTV_ESI_141: Interoperability and renewability support for content protection system.

Input Requirements of the IPTV End System

IPTV_ESI_142: The IPTV end system should support a text input module for easier typing information. For example: a QWERTY layout keyboard.

IPTV_ESI_143: The IPTV end system should support an analog input module for easier moving cursor.  For example: a track ball, a thumb sticks, etc.

IPTV_ESI_144: The IPTV end system should support special RCs. For example, an easy-using RC for children could automatic limit the access in the proper categories of content.

IPTV_ESI_145: The IPTV end system should support a radio wireless link between the RC and the STB for faster and steady interaction.  For example, Bluetooth, Wi-Fi, etc.

Editor’s Note All the following requirements under C-186 were copied from the ATIS and J.193 recommendations. Which requirement is from the ATIS and which one from J.193? the ATIS requirements shall be removed since they are already in the document.

IPTV Terminal Requirements

IPTV_ESI_146: The IPTV terminal should support an IP-based 2-way communication channel.

IPTV_ESI_147: The IPTV terminal should support embedded and/or distributed Personal Video Recording functionality (PVR).

IPTV_ESI_148: The IPTV terminal should communicate with the network servers to figure out what services are available.

IPTV_ESI_149: The IPTV terminal should be able to change channel to switch between channels for the multi-channel IPTV service.

Security Requirements

IPTV_ESI_150: The IPTV terminal should have a renewable security system to perform the functions of Conditional Access, including decryption, authorization, authentication, entitlement, and key generation.

IPTV_ESI_151: The IPTV terminal should include copy protection and redistribution control.

IPTV_ESI_152: The IPTV terminal should implement a secure software download mechanism from the network.

IPTV_ESI_153: The IPTV terminal should secure the communications used to support billing.

Video requirements

IPTV_ESI_154: The IPTV terminal is required to handle all digital transport streams according to the following requirements:

IPTV_ESI_155: The IPTV terminal should support interpreting one of the following codec.

IPTV_ESI_156: The IPTV terminal should support a set of relevant decoding capabilities such as, but necessarily limited, to SD and HD.

IPTV_ESI_157: The IPTV terminal should process emergency messages if they are available in the transmission infrastructure.

Audio Requirements

IPTV_ESI_158: The IPTV terminal should support music channel services.

IPTV_ESI_159: The IPTV terminal should support mono, stereo and multichannel decode.

Requirements for interfaces

IPTV_ESI_160: The IPTV terminal should provide a human control interface (for example an infrared remote-control).

Provisioning requirements

IPTV_ESI_161: The IPTV terminal should be easy to install and configure for operation.

IPTV_ESI_162: The IPTV terminal should support self- and remote provisioning of services, including network configuration and device-specific service enabling tasks.

IPTV_ESI_163: The IPTV terminal should provide secure provisioning and configuration mechanisms.

IPTV_ESI_164: The IPTV terminal should be protected from unauthorized configuration.

PVR Requirements

IPTV_ESI_165: The IPTV terminal should support both internal and/or external control of PVR functionality.

IPTV_ESI_166: If the IPTV terminal implements PVR functionality, it should preserve the digital rights associated with the content, and should appropriately manage content usage in accordance with any digital rights associated with the content.

#### 5.5.2.1 Mobile Terminals

IPTV_ESI_167: TBD

#### 5.5.2.2 Fixed Terminals

IPTV_ESI_168: TBD

### 5.5.4 End-user domain (home & extensions)

IPTV_ESI_169: The IPTV Architecture shall provide a mechanism through which end-users can make content they produced/created available to other end-users. [IIF.ARCH.SERVICE.22]

IPTV_ESI_170: The IPTV Architecture shall provide mechanisms for users to control who is able to view end-user originated content; everyone versus a specific subset of people. [IIF.ARCH.SERVICE.23]

IPTV_ESI_171: The IPTV Architecture shall have some mechanisms for authenticating and authorizing capable end-users for content sharing services -- i.e., end-users with appropriate service subscriptions and network resources. [IIF.ARCH.SERVICE.24]

IPTV_ESI_172: The IPTV Architecture shall support an IPTV service for the whole End-user Domain, which may consist of multiple Home Networks and stationary or mobile end-devices with direct wireless interfaces with the network provider domain – i.e., the IPTV architecture must allow for the whole End-user Domain to belong to the same service domain. [IIF.ARCH.HOME.01]

IPTV_ESI_173: The IPTV Architecture should allow multiple network providers for a single Home Network – i.e., the IPTV architecture should allow a single Home Network to belong to more than one transport domain. [IIF.ARCH.HOME.02]

IPTV_ESI_174: The IPTV Architecture shall allow different providers for Transport and Service strata – i.e., the IPTV architecture must allow a single Home Network to belong to separate transport and service domains. [IIF.ARCH.HOME.03]

IPTV_ESI_175: The IPTV Architecture shall allow for multiple providers for different or similar services – i.e., the IPTV architecture must allow a single Home Network to belong to multiple service domains. [IIF.ARCH.HOME.04]

IPTV_ESI_176: The IPTV Architecture shall define the means for offline subscribers’ correlation between Transport and Service Strata – e.g., for example, for billing and management purposes. [IIF.ARCH.HOME.05]

IPTV_ESI_177: The IPTV Specifications shall define the minimal set of functions that constitute a DNGF. [IIF.ARCH.HOME.06]

IPTV_ESI_178: The IPTV Specifications shall define the minimal set of functions that constitute an ITF. [IIF.ARCH.HOME.07]

IPTV_ESI_179: The IPTV Architecture shall specify one or more mappings of the identified DNGF and ITF onto physical devices. [IIF.ARCH.HOME.08]

IPTV_ESI_180: The IPTV system may accommodate content sharing between devices in the home network.

### 5.5.5 Remote management

IPTV_ESI_181: Middleware SHOULD support software version upgrade and configuration data download by connecting to download server.

## 5.6 Requirements for Middleware, Application and Content Platforms

### 5.6.1 Middleware

IPTV_MID_001: Middleware MAY provide the API to access metadata information to generate program guide.

IPTV_MID_002: Middleware SHALL provide API for stopping and starting Video and Audio.

IPTV_MID_003: Middleware MAY provide API for recording programs locally.

IPTV_MID_004: Middleware MAY provide capabilities to run value-added applications.

IPTV_MID_005: Middleware SHALL provide API to and be able to communicate with IPTV service providers to implement media transmission and media control function

IPTV_MID_006: Middleware shall provide API for DRM function to communicate with DRM system

IPTV_MID_007: Middleware SHALL support system startup and initialization function

IPTV_MID_008: Terminal SHALL support play, pause and stop

IPTV_MID_009: IPTV terminal SHOULD support skip forward and skip backward (e.g. by time period, interval, location in content, etc.)

IPTV_MID_010: Terminal SHALL be able to display the service logo.

IPTV_MID_011: The IPTV system must be able to display the access term (e.g. duration the viewer is entitled to consume the content, PPV, etc.) and price of the selected program.

IPTV_MID_012: Terminals may have the ability to select from a range of languages, (May) speed and verbosity for captioning.(to be confirmed with ETSI)(trick mode for captioning)select range of language.

IPTV_MID_013: Terminals SHOULD provide captioning.

IPTV_MID_014: The IPTV system may have the ability to arrange display of the two video sources in different layouts.

IPTV_MID_015: Middleware SHALL provide API to and be able to communicate with IPTV service providers to implement media transmission and media control function

IPTV_MID_016: Provide negotiation mechanism between clients and servers at any time of service session.

Editor’s Note: the following requirement should be elaborated

IPTV_MID_017: Should provide the middleware transparency for IPTV service

Editor’s Note: Reword the following requirement.

IPTV_MID_018: Provide support of re-transmission of digital terrestrial (DTT) or satellite television for IPTV

IPTV_MID_019: The IPTV Architecture should provide a UI for a smooth transition between gaming and other device functionality, to promote a good gaming experience. [IIF.ARCH.SERVICE.33]

IPTV_MID_020: Any On-Screen Display facilities should also be available via external interfaces.

IPTV_MID_021: The ITU-T accessibility checklist should be applied to the work on IPTV.

Editor’s Note: the following requirement deserves some elaboration. It will be reworded to cover the architecture requirement and Security end-systems as well.

IPTV_MID_022: Consider in home content sharing remotely

IPTV_MID_023: The content protection mechanisms should be provided to content providers to integrate the digital copyright management with content.

IPTV_MID_024: The statistic data of using frequency of content should be provided to content providers.

IPTV_MID_025: The IPTV Architecture shall provide a means to present the ITF with the time of day reference. [IIF.ARCH.SERVICE.57]

Content Formats

Internationalization Requirements of the IPTV service

IPTV_MID_026: The IPTV service should support various content rating standards.

IPTV_MID_027: The IPTV service should support various content resolutions.

IPTV_MID_028: The IPTV service should support various content aspect ratios.  For example, 4:3, 16:9.

IPTV_MID_029: The IPTV service should support various languages.

IPTV_MID_030: The content provider may support the various audio coding schemes

IPTV_MID_031: The EPG should be displayed in the language that the end-user chooses.

Video- Encoding resolution and output standards

IPTV_MID_032: TBD

Audio - Encoding and output standards

IPTV_MID_033: TBD

MetaData

IPTV_MID_034: IPTV middleware can manage the information about nature of the applications [ATSP].

IPTV_MID_035: IPTV middleware shall decompress and decode multimedia contents.

IPTV_MID_036: IPTV middleware shall support executing of multiple simultaneous applications [DVB-MHP].

IPTV_MID_037: IPTV middleware shall provide the network technology independence [OMA].

#### 5.6.1.1 Application and UI transparency

IPTV_MID_038: The IPTV Architecture shall support mechanisms for the end-user to log advertisement information of interest. [IIF.ARCH.SERVICE.51]

### 5.6.2 Enhanced EPG, Channel and Menu Processing

IPTV_MID_039: IPTV system must support channel selection through EPG. (the program guide application)

IPTV_MID_040: The IPTV Architecture shall support the ability of the ITF to display and allow user selection of program, content, and service descriptions, such as Interactive Program Guides (IPG) and VOD service guides. [IIF.ARCH.SERVICE.53]

IPTV_MID_041: Channel Preview Capability

IPTV_MID_042: Maximum duration for each preview

IPTV_MID_043: Maximum number of previews

IPTV_MID_044: Blackout duration after each preview

IPTV_MID_045: Reset period of channel preview system

IPTV_MID_046: Recognition Time of channel preview system

IPTV_MID_047: UI Skin Requirement of IPTV Service

IPTV_MID_048: The skins should contain properties of the graphical entities of the EPG such as the fonts, the icons, the pointer, the background pictures, and the sounds effect, etc.

IPTV_MID_049: The CP/SP or service should provide various certified skins on the EPG.

IPTV_MID_050: The end user should purchase the skins on the EPG as content.

IPTV_MID_051: The end system should download the skin on boot process, or store it in storage.

IPTV_MID_052: The end system should render the UI with the user preferred skin.

High-level requirements for Enhanced-EPG

IPTV_MID_053: Content information viewing: EPG should show useful information used by user to choose the channel.

IPTV_MID_054: Contents selection: EPG should allow user to select desired contents.

IPTV_MID_055: Contents searching: EPG should support content searching in order to provide easy contents discovery because of a number of IPTV channel or contents provider

IPTV_MID_056: Contents list composition: EPG should allow user-customized contents list based on the given channel list

IPTV_MID_057: Up to date information: EPG should contain always up to date information about contents.

IPTV_MID_058: Description format independency: Contents metadata format should be compatible among multiple providers or transformable with each others

IPTV_MID_059: Device adaptation: EPG should be presentable without any consideration on the type of terminal and should be able to adapt on it.

IPTV_MID_060: Flexible layout: EPG should allow users to change or rearrange EPG layout for visibility problem due to the display size limitation.

IPTV_MID_061: UCC support: EPG should provide some interface to make UCC

### 5.6.3 Content Coding

IPTV_MID_062: The IPTV Architecture should facilitate the acquisition of content -- e.g., VOD, Advertising. [IIF.ARCH.CONTENT.05]

IPTV_MID_063: Partitioning should be codec format aware. E.g., it should support codec format like AVS，MPEG-2, MPEG-4, VC-1 and H.264.

IPTV_MID_064: If video is received in IP packets, over any physical medium (wireless, copper, or coaxial), the IPTV Architecture shall support the ability for the ITF to extract the video streams. [IIF.ARCH.HOME.48]

### 5.6.4 Metadata

IPTV_MID_065: The IPTV Architecture shall provide a means to avoid sending to an ITF content that is unable to be consumed. [IIF.ARCH.CONTENT.01]

IPTV_MID_066: The IPTV Architecture should be able to provide the service provider with the capability of creating or amending the metadata associated with a particular content.. [IIF.ARCH.CONTENT.03]

IPTV_MID_067: The IPTV Architecture may support the ingestion of multiple metadata formats such as those standards defined by the DVB, ETSI, CableLabs®, ICE, and MPEG. [IIF.ARCH.CONTENT.04]

IPTV_MID_068: IPTV Architecture specification shall define an extensible schema for asset metadata and format. [IIF.ARCH.OPERATOR.09]

IPTV_MID_069: The IPTV Architecture shall provide a network based capability for the end-user to select the content to be delivered. [IIF.ARCH.CONTEXT.25]

IPTV_MID_070: The IPTV Architecture shall define a linkage mechanism for all services (e.g., channels) to locate their respective stream details (e.g., source, addressability). [IIF.ARCH.SERVICE.54]

IPTV_MID_071: The IPTV Architecture shall specify a standard metadata format between service providers and the ITF for the IPG. [IIF.ARCH.SERVICE.58]

IPTV_MID_072: Terminals should be able to declare Media-providing entities their usage environments description.

Media-providing entities MAY declare their usage environments description to terminals.

IPTV_MID_073: Media-providing entities and Media terminals MAY be able to negotiate/coordinate appropriate Codecs/Profiles/Encoding configurations for specific usage environment: to adapt to heterogeneous usage environments

IPTV_MID_074: Metadata SHOULD have a means to declare and specify the service logo.

Metadata SHOULD allow describing multi-audio content.

IPTV_MID_075: Metadata SHOULD be able to describe user preferences for personal favourite channels.

IPTV_MID_076: Metadata MUST be able to describe content that need to be qualified. (e.g. parental control)

IPTV_MID_077: Metadata SHOULD be able to describe advertisement content (e.g. commercial video).

IPTV_MID_078: Metadata SHOULD be able to describe service.

IPTV_MID_079: Metadata SHOULD signal whether the content is encrypted or not.

IPTV_MID_080: The metadata SHOULD be flexible and extensible for the future advanced IPTV services, and SHOULD be interoperable as much as possible with the metadata in traditional broadcasting services.

IPTV_MID_081: The IPTV metadata SHOULD be effectively represented in XML.

Requirements for metadata set for EPG service:

IPTV_MID_082: The IPTV metadata schema for EPG service shall support to provide general information about a piece of content that does not change regardless of how the content is published or broadcasted. For example, title, synopsis, actor and actress, etc. (ProgramInformation)

IPTV_MID_083: The IPTV metadata schema for EPG service shall support to represent a group of programs. It is more efficient for user’s choice and purchase than a particular program. Metadata for a group of programs need to support various types of group such as series, and show. (GroupInformation)

IPTV_MID_084: The IPTV metadata schema for EPG service shall support to describe a service provider to provide contents. For example, name, owner, logo, etc. (ServiceInformation)

IPTV_MID_085: The IPTV metadata schema for EPG service shall support to describe a group of consecutive schedule events together, which span a given time period on a single service. (Schedule)

IPTV_MID_086: The IPTV metadata schema for EPG service shall support to describe "publication event" that can be acquired on demand (as opposed to broadcast). For example, start time of which contents is avail, or end time of which contents is avail, etc. (OnDemandProgram)

IPTV_MID_087: The IPTV metadata schema for EPG service shall support to describe contents with its price and conditions of availability like the number of plays in duration, as well as an access to a pricing server. (PurchaseInformation)

Requirements for package metadata for package service:

IPTV_MID_088: The IPTV metadata schema for package service shall support to express the options for consumption depending on usage environment (device, network and etc.).

IPTV_MID_089: The IPTV metadata schema for package shall support to express the options for consumption depending on user preference.

IPTV_MID_090: The IPTV metadata schema for package service shall support to provide targeting service which is defined as automatically matching and delivering relevant content to profiled end-users

IPTV_MID_091: The IPTV metadata schema for package service shall support to describe synchronization (temporal) and spatial information between contents to allow contents to be consumed as the service provider intended.

IPTV_MID_092: The IPTV metadata schema for package service need to describe package with its price and conditions of availability like the number of plays in duration, as well as an access to a pricing server.

### 5.6.5 Content Discovery

IPTV_MID_093: The IPTV Architecture shall support the ability for a user to interact with service operator-controlled server-side services through the 2-way network. [IIF.ARCH.SERVICE.17]

IPTV_MID_094: Metadata should provide navigation capability across IPTV contents.

IPTV_MID_095: The IPTV Architecture shall provide the ability to search for available content. [IIF.ARCH.CONTEXT.23]

IPTV_MID_096: The IPTV Architecture shall have the capability to provide information about the content available to the end user.[ IIF.ARCH.CONTEXT.24]

IPTV_MID_097: The IPTV system MAY provide the search for the specific program. The user can input some information about the program to implement the search.

IPTV_MID_098: The search function MAY support the following functions: The inquiry according to the initials of the keyword (such as title, director, the leading actor, etc.）

IPTV_MID_099: Any elements of content metadata MAY be searchable

### 5.6.6 Content Delivery

IPTV_MID_100 The IPTV Architecture shall define a standard reference point for the start of the IPTV Service which must include standard representation of content, identification, and metadata. [IIF.ARCH.OPERATOR.04]

IPTV_MID_101: The IPTV Architecture shall support the integration and interoperability of all platform and processing equipment necessary for the acquisition and processing of content during the ingestion phase -- e.g., applying DRM, insertion of commercials, encoding, editing. [IIF.ARCH.OPERATOR.05]

IPTV_MID_102: The IPTV Architecture shall provide mechanisms to support the acquisition of digital forms of content. [IIF.ARCH.OPERATOR.06]

IPTV_MID_103: The IPTV Architecture shall provide a mechanism for the service provider to ingest and store VOD assets. [IIF.ARCH.OPERATOR.07]

IPTV_MID_104: IPTV service SHOULD support fast forward and fast rewind according to multiple-rate（e.g. 2/4/8/16/32X)

IPTV_MID_105: Both a pull and a push method of content delivery to terminals SHALL be supported.

IPTV_MID_106: Metadata MAY be provided in conjunction with the audiovisual contents of the IPTV program.

IPTV_MID_107: The metadata multicasting in conjunction with the audiovisual contents of the IPTV programs MAY be possible.

IPTV_MID_108: The metadata multicasting without the audiovisual contents of the IPTV programs MAY be possible.

IPTV_MID_109: Shall have the ability to select and receive two audio sources. (e.g. one with audio description)

Editor’s Note: In the following requirement, why wouldn't an advertised want to expand its scope in regions where there is precisely a need to advertise its products, or where he wants to sell its products?

IPTV_MID_110: An advertiser will want to ensure their commercials are only seen in areas where their products or services are available.

IPTV_MID_111: Advertisers and broadcasters will, on replay of recorded content, want to replace interstitial material with more relevant interstitials based on parameters such as time of viewing, environmental triggers (such as weather, season etc) and number of times seen.

IPTV_MID_112: Broadcasters, advertisers and service operators will want demographic data (un-attributable, or, with permission, attributable) to be returned from the end-users device.

IPTV_MID_113: An advertiser will want to replace expired content with the latest content without having to keep track of the content items that have already been distributed.

IPTV_MID_114: Advertisers will want to ensure their commercials are only seen by the appropriate audience. Where, for instance, a minor is replaying content, a commercial for an alcoholic beverage should be skipped.

IPTV_MID_115: A broadcaster will want to sell the interstitial placement in their content based on live or recorded viewing, replacing interstitials based on flexible business rules.

IPTV_MID_116: The content management should be provided to content providers, include uploading content, deleting content or modifying the relevant attributes of content.

IPTV_MID_117: The IPTV Architecture shall provide the mechanism to make available the description of the programming events on each channel. [IIF.ARCH.SERVICE.55]

IPTV_MID_118: The IPTV Architecture shall provide the mechanism to make available a detailed description of specific programming events. [IIF.ARCH.SERVICE.56]

IPTV_MID_119: The IPTV Architecture shall support both the push and pull model for the delivery of the IPG service. [IIF.ARCH.OPERATOR.01]

IPTV_MID_120: The IPTV Architecture should allow the service provider to opportunistically push the video content onto a subscriber’s ITF implementing device without an explicit request. [IIF.ARCH.CONTENT.06]

#### 5.6.7.1 Distribution

IPTV_MID_121: The IPTV Architecture shall provide mechanisms for the unicast delivery of contents.

IPTV_MID_122 The IPTV Architecture shall provide mechanisms for the multicast delivery of contents.

IPTV_MID_123: The IPTV Architecture shall provide mechanisms for content downloading.

IPTV_MID_124: The IPTV Architecture shall provide mechanisms for the service provider to push contents to the end-user receiver.

## 5.7	Public interest requirements

### 5.7.1 Accessibility Requirements

Provide accessibility for people with disabilities and people with special needs and meet the minimum regulatory requirements.

Additionally, these access requirements will provide accessibility for people with temporary environmental dysfunctions to include people with varied or different languages, to those with learning difficulties and to those whose capabilities are diminished by age and degeneration By 2015, it is estimated that 50% of the population will be over the age 50* and require accessibility features, therefore making such features mainstream requirements.  It is also noted that some of these accessibility features are presently mainstream in their use and have been included in other clauses in this document. Appropriate references to those other clauses for such accessibility features are included in the list below as an aide for the implementer, as national regulation may apply.

(Note: Some of the following requirements below may have regulatory implications in some countries and may not be required for all video IPTV applications, this may change over time. National regulations may place specific additional requirements that shall be honoured).

As the high level requirements, the architecture and services shall provide:

IPTV_PUB_001: Captioning: Provision for captioning.

The IPTV Architecture shall provide mechanisms to support closed captioning. [IIF.ARCH.SERVICE.43]

The IPTV Architecture shall support the ability of the ITF to decode and display closed captioning information. [IIF.ARCH.SERVICE.44]

IPTV_PUB_002: Captions in own window:  The ability to view the captions and vary its presentation in a separate window. Reference needed

IPTV_PUB_003: At least two video sources:  The ability to select and receive two (related) video sources. (e.g. one with sign language translation ) Reference needed

IPTV_PUB_004: At least two audio sources: The ability to select and receive two audio sources. (e.g. one with audio description ) Reference needed

IPTV_PUB_005: Good audio quality: The audio transmission and reproduction should be of good quality to make it possible for people to perceive the sound well. Reference need

IPTV_PUB_006: Good video quality for sign language: Video should be transmitted with sufficient quality for sign language perception if there is sign language in the contents [1] Referenced needed

IPTV_PUB_007: Good video quality for lip reading: Video should be transmitted with sufficient quality for lip reading perception. Reference needed

IPTV_PUB_008: Use Accessibility checklist: The ITU-T accessibility checklist should be applied to the work on IPTV [2]. Reference will change after AAP process for SG16 for version 5

IPTV_PUB_009: Universal Design* Applications and equipment shall be designed using principles of Universal Design so that a wider population with users of varying capabilities can access such applications and equipment (note *reference needs to be replaced)

IPTV_PUB_010: Meta-data: Meta-data shall contain information about the provision of Accessibility Features.

IPTV_PUB_011: Recording: Any recordings that the equipment makes shall also include the appropriate Accessibility Features. The recording of Accessibility Features should be done in such a way that, when watching the recording, they can be both switched on and switched off. Therefore, it is a good idea to record all Accessibility Features streams and link them to the service as a whole.

### 5.7.2 Emergency telecommunications

IPTV_PUB_012: The IPTV Architecture shall support Emergency Alert Service per the relevant national regulations -- e.g., ANSI J-STD-042-2002. [IIF.ARCH.SERVICE.36]

IPTV_PUB_013: The IPTV Architecture shall provide appropriate interfaces to support the ingestion of suitably authorized Emergency Alert information. [IIF.ARCH.SERVICE.37]

IPTV_PUB_014: The IPTV Architecture shall support the ability for the ITF to decode and display emergency alert information. [IIF.ARCH.SERVICE.38]

IPTV_PUB_015: Meta-data shall contain information about the provision of Accessibility Features.

- IPTV_PUB_016: Any recordings that the equipment makes shall also include the appropriate Accessibility Features. The recording of the Accessibility Features should be done in such a way that, when watching the recording, they can be both switched on and switched off. Therefore, it is a good idea to record all relevant Access Service streams and link them to the service as a whole.

- IPTV_PUB_017: Applications and equipment shall be designed using principles of Universal Design* and cater for the needs of the users of such applications and equipment.

- 5.7.3 Provider Selection and Portability Requirements

- IPTV_PUB_018: The IPTV architecture should provide the mechanism for customers to select IPTV network providers, IPTV service providers, and IPTV content providers with their preference.

- IPTV_PUB_019: IPTV architecture should provide number portability.

- 5.7.4 Lawful Interception and User Identification Requirements

- IPTV_PUB_020: The IPTV architecture should provide lawful interception.

- IPTV_PUB_021: IPTV architecture should provide malicious communication identification.

- IPTV_PUB_022: The IPTV architecture shall provide user identifier presentation and privacy.

- IPTV_PUB_023: The IPTV architecture shall provide the mechanism to prevent unwanted IPTV services such as unwanted advertisement and spam, etc.

From ATIS

Figure 1: IPTV Logical Domains

Editor Note: some discussions took place; we postponed the discussion to proceed first by working on section 5

______________________________

*	 J M Gill, Ms S A Perera: Accessible Universal Design of Interactive Digital Television;  http://www.tiresias.org/reports/brighton.htm