| Abstract INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0564 |  |
|  |  |  | English only |  |
| WG(s): 5 |  | 4th FG IPTV meeting: Bled, Slovenia, 7-11 May 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Royal National Institute of the Blind (RNIB, UK) |  |  |  |
| Title: | Proposed additions to sub-clause 5.5 of IPTV-DOC-0060 |  |  |  |

This contribution proposes changes to sub-clause 5.5 of FG IPTV-DOC-0060, working document on IPTV services requirements.

# Background

At the most recent FG IPTV meeting, i.e. 22-26 January 2007, significant progress was made and a better document was produced.

# Discussion

This contribution proposes some additions intended to improve some users’ experience when using IPTV Terminals.

# Reference

FG IPTV-DOC-0060 (22-26 January 2006), working document: IPTV services requirements

# Proposal

It is proposed to make the following additions to section 5.5, in the subsection entitled IPTV Terminal Device Requirements:

IPTV_ESI_nn1: The IPTV Terminal should provide a user setting to select an opaque background for any text that is overlayed onto video, thereby making it easier to read.

IPTV_ESI_nn2: The IPTV Terminal should provide a user setting to change the default colours of text and background for on-screen displays. As a minimum, a combination of white text on a black background should be offered.

IPTV_ESI_nn3: The IPTV Terminal may provide a user setting to change the default size of text for on-screen displays.

IPTV_ESI_nn4: The IPTV Terminal should be able to determine when it is waiting for an action to complete (such as downloading content) and inform the user to wait.

IPTV_ESI_nn5: The IPTV Terminal User Interface shall not rely on colour alone to convey information.

IPTV_ESI_nn6: The IPTV Terminal should display Arabic numerals (1, 2, 3, 4) rather than Roman numerals (I, II, III, IV) where applicable to the language used.

IPTV_ESI_nn7: The IPTV Architecture must have the capability to display captions in any character set of the content provider’s choosing. Note: This can be achieved by using bitmapped captions as defined by DVB in ETSI EN 300 743 Digital Video Broadcasting (DVB);Subtitling systems

IPTV_ESI_nn9: The IPTV Terminal should maintain the user’s selected captions, supplementary audio content and supplementary video content as far as possible when switching between services.