| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On IPTV |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  | FG IPTV-C-0590 |
|  |  | Original: English |
| WG(s): | 5 | 4th FG IPTV meeting: Bled, Slovenia, 7-11 May 2007 |
| CONTRIBUTION |  |  |
| Source: | China Netcom Group (P.R. China) |  |
| Title: | Proposal to addition of IPTV terminal device Security Module (SM) functionality |  |

# Introduction

According to the working document “FG IPTV-DOC-0069” in the third FG IPTV meeting, we have the following information:

- TD-SM: An interface between IPTV Terminal Device and removable security functions such as IC card.

The followings are the specification for TD-SM.

- An IPTV Terminal Device should be equipped with one interface for TD-SM.

- The interface for TD-SM should be a smart card interface defined in ISO7816-1 [ISO7816-1], ISO 7816-2 [ISO7816-2] and ISO7816-3 [ISO7816-3].

# Proposal

The smart card is a key mode to IPTV service and terminal operation. Besides security function, we propose the smart card (Security Module) should have the following functions:

- The IPTV smart card should follow a common uniform format similar to SIM.

- The IPTV smart card should have the ability to support prepay and postpay mode by enough information.

- To support CCITT Recommendation E.118, CCITT Recommendation E.164, CCITT Recommendation T. 50, according to charging and numbering needs.

# Reference

CCITT Recommendation E.118: The international telecommunication charge card.

CCITT Recommendation E.164: Numbering plan for the ISDN era.

CCITT Recommendation T.50: International Alphabet No. 5.