| Abstract INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0667 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 5th FG IPTV meeting: Geneva, 23 – 31 July, 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | NTT Corporation |  |  |  |
| Title: | Proposed modifications on IPTV service requirements document- 6.7 Public-related Requirements |  |  |  |

This contribution proposes changing some public related requirements in section 6.7 of the IPTV service requirements document, which was developed in the 4th FG-IPTV meeting as FG-IPTV-doc-0083, to improve its level of comprehensiveness.

# 1. Introduction

In the last FG-IPTV meeting in May, the output document of IPTV service requirements was developed in WG1 as FG-IPTV-doc-0083.  This document contains public-related requirements in the subsection 6.7, and they have been improved by several contributions. However, some of them are still unclear.  This contribution proposes some modifications of the public-related requirements to improve its level of comprehensiveness.

# 2. Proposal

This contribution proposes the specific modifications listed in the following table.

| # | IPTV Number | Proposed changes | Proposal and reason |
| --- | --- | --- | --- |
| 1 | IPTV_MID_013: | [deleted] | This requirement is overlapped with IPTV_PUB_001a and should be deleted. |
| 2 | IPTV_PUB_012: | The IPTV Architecture is required to support Emergency Alert Service where required by regulation or law of the country in their application area -- e.g., ANSI J-STD-042-2002 [IIF.ARCH.SERVICE.36]. | The case of countries without relevant regulation is ambiguous. It should be changed. |
| 3 | IPTV_PUB_013: | The IPTV Architecture is required to support appropriate interfaces to support the ingestion of suitably authorized Emergency Alert information where Emergency Alert Service is provided [IIF.ARCH.SERVICE.37]. | Condition should be added such as “where Emergency Alert Service is provided”. |
| 4 | IPTV_PUB_014: | The IPTV Architecture is required to support the ability for the ITF to decode and display emergency alert information where Emergency Alert Service is provided [IIF.ARCH.SERVICE.38]. | Condition should be added such as “where Emergency Alert Service is provided”. |
| 5 | IPTV_PUB_ ISSUE _01: | Editor’s note: we need to define a notion of public domain in the requirement ISSUE_01 The IPTV Architecture is required to support an open and standardized Interface to public emergency domains where Emergency Alert Service is provided. | Condition should be added such as “where Emergency Alert Service is provided”. |
| 6 | IPTV_PUB_ ISSUE _02: | The IPTV Architecture is recommended to support the capability of logging and forwarding Emergency Alert Notifications when used as the emergency messages distribution system where Emergency Alert Service is provided. [Contributor’s note: Note should be added to clarify relation and difference between EAS and EAN.] | Condition should be added such as “where Emergency Alert Service is provided”. Note should be added to clarify relation and difference between EAS and EAN |
| 7 | IPTV_PUB_ ISSUE _03: | The IPTV Architecture is recommended to support the delivery of the specific emergency messages to the corresponding user groups based on pre-defined policies (e.g. geographic location) where Emergency Alert Service is provided. [Contributor’s note: Note should be added to clarify relation and difference among similar terminologies such as EAS, EAN, Emergency Alert information, emergency message, and alerting message.] | Condition should be added such as “where Emergency Alert Service is provided”. What is the “specific emergency message”? Relations and differences among EAS, EAN, Emergency Alert information and emergency message should be made clear. Note should be added. |
| 8 | IPTV_PUB_REPORT _1: | The IPTV Architecture is recommended to support identification of the alerting messages sources and to support the classification of such messages upon pre-defined policies where Emergency Alert Notification is provided. [Contributor’s note: Note should be added to clarify relation and difference among similar terminologies such as EAS, EAN, Emergency Alert information, emergency message, and alerting message.] | Condition should be added such as “where Emergency Alert Service is provided”. What is the “alerting message”? Is it the same as Emergency Alert information? Note should be added to make it clear. |

_____________