| 1. Introduction INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0727 |  |
|  |  |  | English only |  |
| WG(s): 6 |  | 5th FG IPTV meeting : Geneva, 23-31 July 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | ICU |  |  |  |
| Title: | Discussion issues about Web-based IPTV Portal service scenario with WG6 |  |  |  |

At the last meeting, we suggested web-based IPTV portal service scenario and it is adopted in the IPTV service scenarios document by WG1. At this meeting, we suggested update scenario. Web-based IPTV portal service means that the existing Terrestrial/Cable/Satellite TV contents can easily deliver through the IP network by integration of web and IP data services.

This contribution proposes detailed considerations to support suggested scenario with WG6.

2. Considerations for the alignments with WG6 activities to support suggested scenario

- Metadata and Service Discovery

For the purposes of searching, finding, and acquiring programs, as well as for advertising and promoting content, the IPTV service must support metadata in general and digital program guides, in particular. This involves specifications for IPTV service discovery and identification of content and locations, and others. The metadata specifications such as TV-Anytime, DVB, and others, including service information in digital broadcasting for simple electronic program guide will be considered.

- Content Formats

It is necessary to recommend standardized formats and encoding methods of content. In the current broadcasting, MPEG-2 is generally used, but it is expected that other formats, such as H.264, will be useable in IPTV environment.  For maximum interoperability, careful review of the relevant technologies is necessary.

- Channel searching

Web-based IPTV channel is just a spreading way of content through the web, in this case through the pages that have a television in. The difference between an web-based IPTV channel and a normal TV channel is that the internet potential is higher nowadays and its features as well. Web-based IPTV channel offers content, previously scheduled and known previously. Web-based IPTV channel offers this feature as well as instant selections of the on demand users or ask for TV on live. Web-based IPTV is a fact, and with it come a great number of television channels. The user can select between several channels or create his own channel to watch it from web.

3. Proposal

We hope that considerations should be discussed by WG6 to support suggested scenario.

_______________