| Summary: INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0787 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 5th FG IPTV meeting: Geneva, 23-31 July 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | Proposal to revise parental control requirements IPTV_NET_025 in FG-IPTV-0083 |  |  |  |

The document proposes to revise requirements IPTV_NET_025 in order to address the use case that mobile terminals remotely control IPTV including EPG proceeding on Home Network. The proposal takes the mobility which was mentioned as a compulsory feature to be supported in requirement IPTV_ESI_172 of working document IPTV Services Requirements into consideration. With such revision two correlated requirements are becoming more consistent.

Discussion:

Below is a use case which takes consideration of mobile terminals:

Parents who are in business trip could subscribe, control and monitor IPTV programs including EPG/ECG which is fit for teenagers over mobile terminals, in order to prevent impacts on children from violence or adult programs and confine amusement programs at appreciate moment.  Home Network Terminal Devices only show the subscribed IPTV programs including ECG/EPG, people in Home Network will not be able to see unsubscribed programs.

Parental control requirements are presented in FG-IPTV-0083.  It is said as below: IPTV_NET_025: The IPTV Architecture is required to support mechanisms for the service provider to support the enforcement of parental controls on a user profile basis or a policy basis – e.g. time limit [IIF.ARCH.SERVICE.60].

In the requirement, parental controls mechanism could not support the above use case.  The parental control mechanism should be able to change policies dynamically by remote mobile stations.  Moreover, parental controls mechanism may support to monitor proceeding on Home Network. In this case, network may replicate multi-media stream to mobile stations associated with Home Network.

Proposal:

IPTV_NET_025: The IPTV Architecture is required to support mechanisms for the service provider to support the enforcement of parental controls which allow associated users to control and monitor IPTV programs including EPG/ECG from fixed or mobile terminals.  The mechanism asks for network to store and forward the under-controlled TV programs to associated terminals. The mechanism is also required to be on the basis of a user profile and policies which may be changed by associated user from other terminals.  The policies are varieties include time limit [IIF.ARCH.SERVICE.60], the programme classes and etc.

_____________