| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-1094 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 7th FG IPTV meeting: Qawra, St Paul’s Bay, Malta, 11-18 December 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | Proposal on modification of Application Functions |  |  |  |

Abstract

This contribution proposes modifications to the working document: “IPTV Architecture” FG IPTV-DOC-0148 produced at the 6th FG IPTV meeting in October.

Discussion

In section 9.6, we can see that a block “Application Provisioning” exist in Application Functions.

## Description of functional entities common to all three architectures

### Application Functions

The following clauses provide a breakdown of the Application functions of the IPTV Architecture. Functional blocks identified below the horizontal dotted line are as described in the clause where the high-level architecture is explained.

- Figure 9-5: Detailed Application Functional Architecture

But in the high level architecture of IPTV, there is no such block.

## High Level Architecture of IPTV

- Figure 9-1 Functional Architecture for IPTV

In section 9.6.4.1, we can see that the main function of Application Provisioning is to manage the life-cycle of IPTV applications. But it can be done by Application Management Functions and Application Profile Functions. So it is not necessary to add the block.

Reference

FG IPTV-DOC-0148 (15-19 October 2007), Working Document: IPTV Architecture

Proposal

From the discussion, we can see that Application Provisioning is not necessary, so it is proposed

to delete it as follows. Additionally, the title of the block is “Service Discovery & Selection A”. But in section 9.6.1.1.1, the title of this block is “Service and Application Discovery & Selection Application”, which is more accurate. So the name must be unified. The modification can be seen in the figure below.

## Description of functional entities common to all three architectures

### Application Functions

The following clauses provide a breakdown of the Application functions of the IPTV Architecture. Functional blocks identified below the horizontal dotted line are as described in the clause where the high-level architecture is explained.

- Figure 9-5: Detailed Application Functional Architecture

#### 9.6.1.4 Application Provisioning

Application Provisioning manages the life-cycle of IPTV applications

___________________