| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-1126 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 7th FG IPTV meeting: Qawra, St Paul's Bay, Malta, 11-18 December 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | Proposal for additional text to clarify admission control |  |  |  |

1, Document Scope

- This document proposes additional text in working document FG IPTV-DOC-0152 “Traffic Management Mechanisms for the Support of IPTV Services” to clarify admission control scenario.

2, Discussion

Admission control for IPTV services has been addressed in sub-clause 6.1.2 in working document FG IPTV-DOC-0152 “Traffic Management Mechanisms for the Support of IPTV Services”. However, there are two points which need to be clarified:

- In this sub-clause, only admission control for on-demand services is mentioned. This doesn’t align with the title of the sub clause “Admission control for IPTV services”.

- In this sub-clause, the network provider can be used to apply admission control to IPTV services. However, the signalling exchange between the user equipment and network provider are not very clear here. There is a need to clarify some details, e.g., who sends IPTV services to whom.

3, Proposal

We propose WG2 to consider additional text in working document FG IPTV-DOC-0152 “Traffic Management Mechanisms for the Support of IPTV Services” to clarify admission control for IPTV services to read as follows:

#### 6.1.2 Admission control for IPTV services

In the current environment, i.e. cable or satellite TV, support of basic linear TV service is managed on subscription basis. Once subscribed, there is an expectation by the end-user that the linear TV service will continue without interruption with any further need for admission every time the TV set is turned on. This behaviour must be allowed for IPTV to be able to compete with Cable and Satellite. Therefore, applying admission control to regular linear TV service is not recommended and management of the service must continue to be on subscription basis. A service provider ensures that the network is properly engineered to accommodate the number of subscribers that match its business model.

There are scenarios where multiple set-top boxes may exist in a single home and reception of multiple video streams may be restricted due to the access bandwidth. These scenarios may require the service provider to limit the number of streams admitted for a single subscriber.

There are other scenarios where reception of some video stream programs may be forbidden due to no appropriate subscriber privileges. These scenarios may require the network provider to limit the number of streams admitted for a single subscriber.

Other IPTV services may be subject to admission control.

For example a service provider may apply admission control to IPTV services. The request for IPTV services involves the exchange of signalling between user equipment (e.g. set-top box) and the service provider video server. The service provider may reject the IPTV services request based on network resources, user status, or the current use of a specific program across all subscribers. Criteria such as user standing and the current use of a given program are related to policies set by the service provider and do not change very frequently.

As another example, a network provider may apply admission control to IPTV services. Request for IPTV services involves exchanging of signalling between user equipment and network provider, e.g., Digital Subscriber Line Access Multiplexer (DSLAM) equipment maintains subscriber privileges in a local database and may receive multicast services request. Based on subscriber privileges,DSLAM may reject this request or forward it to multicast router transparently and then allow media stream to be transferred from multicast router to subscriber.

The effect of an IPTV services request on the network resources is a dynamic parameter and it varies based on the program requested. For instance requesting high definition content will impose more demand on the network compared to the demand generated when low definition content is requested. The role of the admission function in this case is to accept or reject the IPTV services request based on the traffic demand of the new request and the current utilization of network resources.

____________