| 1 Scope 2 INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-DOC-0067 |  |
|  |  |  | English only |  |
| WG(s): 3 |  | 3rd FG IPTV meeting: Mountain View, USA, 22-26 January 2007 |  |  |
| OUTPUT DOCUMENT |  |  |  |  |
| Source: | Editor |  |  |  |
| Title: | Working document: IPTV Security Aspects |  |  |  |

2	References	2

3	Definitions	2

4	Abbreviations	3

5	Security Threats	3

5.1	Content Security Threat	4

5.1.1	Sources of Content Security Threats	5

5.2	Service Security Threat	7

5.3	Network Security Threat	8

5.4	Terminal Device Security Threat	9

5.5	Subscriber Security Threat	10

6	Security Requirements	11

6.1	Content Security Requirement	11

6.2	Service Security Requirement	14

6.3	Network Security Requirement	15

6.4	Terminal Device Security Requirement	15

6.5	Subscriber Security Requirement	16

6.5.1	Subscriber Personal Data Requirements	16

6.5.2	Protection of Legal Minors	17

7	Security Architecture	18

7.1	Security Trust Model	18

7.2	Security General Framework	18

7.2.1	Content Protection Architecture	18

7.3	Security Function Modules	19

7.4	Security Interfaces	19

8	Security Mechanisms	19

8.1	Content Security Mechanisms	19

8.1.1	Content Protection	19

8.1.2	Conditional Access	19

8.1.3	Digital Rights Management	19

8.2	Service Security Mechanisms	19

8.2.1	Service Authentication	19

8.2.2	Service Authorization	19

8.3	Network Security Mechanisms	20

8.4	Terminal Device Security Mechanisms	20

8.4.1	Terminal Device Authentication	20

8.4.2	Terminal Device Authorization	20

8.5	Subscriber Security Mechanisms	20

8.5.1	Subscriber Authentication	20

8.5.2	Subscriber Authorization	21

# Scope

This working document addresses threats, requirements, architecture, and mechanisms that pertain to security and content protection aspects of IPTV content, services, networks, terminal devices, and subscribers.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this working document. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this working document are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation Y.2012 (September 2006), “Functional requirements and architecture of the NGN of Release 1”

Additional References

The following additional references are informational in nature, and do not constitute provisions of this working document.

[2]	ETSI TS 102 822-1-1 (January 2006), “Broadcast and On-line Services: Search, select, and rightful use of content on personal storage systems ("TV-Anytime"); Part 1: Benchmark Features”

# Definitions

This Document defines the following terms:

[TBD]

# Abbreviations

This Document uses the following abbreviations.

AAA	Authentication, Authorization, and Accounting

CAS	Conditional Access System

CDN	Content Delivery Network

CPE	Customer Premises Equipment

DOS	Denial of Service

DRM	Digital Rights Management

DRM-B	DRM Bridge

DRM-EE	DRM End-to-End

DRM-IX	DRM Interchange

DVI-D	Digital Visual Interface – Digital Only

HDD	Hard Disk Drive

HDMI	High Definition Multimedia Interface

HGW	Home Gateway

IPTV	Internet Protocol Television

PC	Personal Computer

PKI	Public Key Infrastructure

PPV	Pay Per View

PVR	Personal Video Recorder

REL	Rights Expression Language

VOD	Video on Demand

# Security Threats

This section describes a set of identified security threats that some requirement or mechanism of this document addresses.

Security Objectives

- Only legitimate actors can access and operate on the assets of IPTV services after be authorized.

- The IPTV service should be protected against unauthorized deletion, insertion, modification or replay attack.

The following conceptual threat model provides a classification of the related threats to the IPTV system. This threat model requires significantly more refinement to define the complete list of known risks and mitigations. This model does not include network security threats which exist in all IP networks and assumes that such threats have been mitigated.

Content Theft

Service Theft

Information

Theft

Service Disruption

System Control

Compromise IPTV Solution

Figure 5-1 - Compromise IPTV Solution Attack Tree

## Content Security Threat

Service Chain

The service chain of an IPTV service can be divided into four parts: Content Source, Service Integration & Operations, Content Transport and Content Consumption.

- Content Source/Content provider: The entities that own or are licensed to sell content to IPTV service providers.

- Service Integration & Operations/Service provider: the entity providing a service to the end-user.

- Content Transport/Delivery network: The delivery network usually is composed of access networks and core or backbone networks, using a variety of network technologies.

- Content Consumption/Consumer: The domain where the IPTV services are consumed.

Assets to be protected in IPTV service

In the context of this document, an asset that needs protection in IPTV service is:

Broadcast TV content, VOD content, Push VOD content and PVR content.

Threats to the digital content may include:

- Interception: a breach of confidentiality of the digit content by illegal monitoring the service networks.

- Unauthorized viewing.

- Unauthorized copying or moving.

Table 5-1 - Content Assets, Risks, and Threats

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Compressed, plaintext content work | Unauthorized copy obtained from network, network device or end system | High if the work is within the release window or deemed highly valuable by the provider; Medium to Low depending on the work. | Cracker | High |
|  |  |  | Professional |  |
|  |  |  | Insider |  |
|  |  |  | Consumer | Medium to Low |
| Compressed, encrypted content work | Unauthorized access | Low | Cracker | Low unless the key is obtained and High otherwise |
|  |  |  | Professional |  |
|  |  |  | Insider |  |
|  |  |  | Consumer | Low |
| Any content work | Denial of service attack | High | Cracker | Low |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |

### Sources of Content Security Threats

Security threats can be classified into the following types: content security threats, service security threats, network security threats, terminal device security threats, and user security threats. The security threats model below, Figure 5-2 - Threat Attack Points, shows the relationships between each of these threats. Furthermore, the model shows each of the security threats.

Figure 5-2 - Threat Attack Points

Content security threats are expected to come mainly from the network and from terminals.

Content security threats from Network

The main content security threats from the network are wiretapping, modification, and spoofing. These threats can be prevented by considering the points below in relation to content distribution.

- Wiretapping: content encryption

- Modification and spoofing: digital signature

Content security threats from Devices

Content security threats from home networks can be broadly divided into two kinds—threats that exploit vulnerabilities of the interfaces between individual devices, and methods that destroy a part of the inside of a device. Security design guidelines to protect against the former kind of threats can be established easily, since the attack points are clear. However, it is difficult to analyze the latter kind of security threats because the technical sophistication of attackers, the attack methods, and the attack points all vary widely.

Interface attack

- Home gateway – terminal (equipment on premises of users)

  - As a typical example, we can consider an attempt to discover a vulnerability in the DRM/CAS protocol in order to obtain the key information necessary to decode the encrypted content captured from the home network (e.g. Ethernet).

- Terminal – CAS

  - Threats can occur in cases where the hardware configuration of the CAS is detachable from the terminal. In the case of video distribution by multicast, the same kind of threats as those of ordinary broadcasting can be expected. Since CAS is widely used in broadcasting, its technical maturity is high. One advantage is that when a vulnerability is found in the CAS itself, the CAS can be replaced relatively easily.

- Terminal – PCs

  - It is conceivable that content inside a terminal is wiretapped or unauthorized accessed via the home network. A particular danger is the risk of unauthorized access to content by spoofing as a terminal in the case when content or licenses can be transferred between terminals.

- Terminal – monitor display

  - This threat is conceivable when the TV monitor and terminal are separate. It is extremely difficult to eliminate this threat completely when output is passed through an analog output. An analog video coping is not serious problem because the image is deteriorated. There is no degradation of video when using digital outputs, such as HDMI and DVI-D, it is necessary to prevent this threat totally.

- Terminal – removable media (memory device)

  - Writing content to highly portable devices such as removable media is extremely appealing, since it allows users to use content anytime and anywhere. To enable this, it is necessary to copy or move the encrypted content and encryption keys securely from the terminal to the removable media, but this can present a security threat.

  - Note that a fundamental issue here is that even if it is assumed that there is perfect protection against security threats when the device is connected to the system, if there is no user identification function included with the device it is impossible to prevent items such as books and rental videos being shared by users. Thus, it is difficult to estimate the level of security threat presented by removable media.

- DRM/CAS in the Terminal

  - This threat needs to be considered particularly when the CAS and the terminal are integrated in the terminal. If implemented in software, it is impossible to make the DRM/CAS completely tamper-resistant, so it is necessary to determine the expected security attack level, and consider an implementation method that can stand up to such attacks. Also, if the DRM/CAS is integrated in the terminal, it is necessary to take into account the fact that it is extremely difficult to replace the DRM/CAS in the event of piracy.

- HDD in the Terminal

  - It is necessary to ensure that content cannot be read irregularly from a terminal containing a built-in hard disk. In particular, when reading from a terminal connected to a network is allowed, careful attention must be paid to preventing unauthorized reading of content.

- Memory device

  - In the case of highly portable media, many of the devices themselves have content protection mechanisms. When using such media, it is essentially impossible to share the media itself, so this should be taken into account to achieve a security balance.

## Service Security Threat

Service integration & Operation

- Impingement copyrights of the programs which IPTV service platform provided to the subscribers.

- Masquerading / spoofing IPTV service provider.

- Malicious threats aiming at the IPTV servers (AAA servers, media servers, etc.):  may include the hacking aiming at security leaks in application software or communication protocol, denial of service attack, etc.

- Theft (often use malicious programs, such as Trojan horse) of the subscribers’ information (e.g. identity information, billing information, subscription information).

Table 5-2 - Service Assets, Risks, and Threats

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Domain Name Server | Denial of service attack | Medium | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Electronic Program Guide | Denial of service attack | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Media Servers and multiplexers | Denial of service attack | High | Cracker | High |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access | High | Cracker | High |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| CAS and Subscriber Management | Denial of service attack | High | Cracker | High |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized use | High | Cracker | High |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |

## Network Security Threat

- Intentional threats aiming at the network equipments or resources (bandwidth): malicious attacks such as denial of service to the bearer network.

- Security threats to multicast technical used in IPTV bearer network: such as masquerading / spoofing multicast TV sources, or illegitimate multicast group members.

- Malicious attacks (like DOS, hacking) on nodes in Content Distribution Network.

The following table lists assets, risks and threats for the Core Network.

Table 5-3 - Network Assets, Risks, and Threats

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Network bandwidth | Denial of service attack | Medium | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access or use | High | Cracker | Medium |
|  |  |  | Professional | High |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Network Messages | Unauthorized access | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized modification | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | High |
|  |  |  | Consumer | Low |
|  | Replay | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Routers, intermediate systems, access servers and system hosts | Denial of service attack | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized use | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |

## Terminal Device Security Threat

Some security threats for IPTV terminal devices are listed as follows.

- Illegally accessing clear content by tampering device hardware or software. For example, clear contents can be copied by bus data interception or DRM software cracking.

- Illegally accessing keys or other secret information in devices using software cracking or hardware tampering. Attackers can tamper the device memory or analyze the data flow to obtain the keys and other secrets. Content key exposure results in content leakage and device key leakage leads to device impersonation.

- Device malfunctioning by hardware methods, such as control of the device clock system to disable the functions of the DRM/CAS systems, or by software methods, such as installation of viruses to exhaust the device resources.

- Unauthenticated terminal devices connecting to the home network.

- Unauthorized use by subscribers

Table 5-4 - Terminal Device Assets, Risks, and Threats

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Locality | Fixed-location device that is outside residence is given service | Medium | Cracker | Low |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Medium |
|  | Mobile device owned by non-subscriber is given service | Medium | Cracker | Low |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Medium |
| Processor and Disk | Infection by malicious software | High | Cracker | Low unless executables downloaded from the network |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized use | High | Cracker | Low |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Interface | Subversion of content protection controls (e.g. HDCP, HDMI) | Medium | Cracker | Low |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |

## Subscriber Security Threat

Content consumption

- The failure of the terminal equipments (hardware and software) caused by malicious codes/viruses from the network.

- Remote theft of the subscribers’ important information (e.g. login ID and password) by malicious programs, such as Trojan horse.

- Unauthorized data deletion, insertion, modification, re-ordering, replay or delay.

- Unauthorized data (i.e. the data come from the illegal IPTV program sources) transmission.

- Masquerade IPTV service nodes in the carrier network.

- Security threats to CDN.

Table 5-5 - Subscriber Assets, Risks, and Threats

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Subscriber information | Unauthorized access: Disclosure to unauthorized party | High | Cracker | Medium |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access: Modification by unauthorized party | High | Cracker | Medium |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Transaction information Credit card info | Unauthorized access: Disclosure to unauthorized party | High | Cracker | Medium |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access: Modification by unauthorized party | High | Cracker | Medium |
|  |  |  | Professional | Medium |
|  |  |  | Insider | High |
|  |  |  | Consumer | Low |

# Security Requirements

Requirements for realizing the division of roles in the business model

IPTV services are realized not just by the content provider who serves as the principal agent of the service, but rather through the coordinated efforts of multiple business players, including also the content rights holder, the network provider, and the CPE (Consumer Premises Equipment) manufacturer. Security functions need to be clear about the division of these separate roles and the realization of these various operations. It is vital to strike a balance between the distribution of content and the distribution of the money paid for it, in order to enable a backwards flow of profitability, in line with the rights that each player possesses and the costs related to service provision. The requirements for this are listed below.

Functional requirements

There are requirements other than those relating to items 1.1 and 1.2 above. IPTV services for the home use the same CPE as current broadcasting services, so it is necessary to consider the questions of sharing specifications and usability with broadcasting services. These functional requirements are enumerated below.

  - Copyright of content and related data

  - Content protection, e.g. connection of external devices to CPEs, recording controls

  - Billing controls for paid content

  - Ensuring access rights to CPEs

  - Protection against unauthorized access to servers

  - Protection of personal data, etc. relating to CPEs

## Content Security Requirement

IPTV content security requirements may include:

- Content Access Control: Only the authorized subscribers can use the digital video content. The rights granted may include: the receiving device (one or a group), time period for viewing, watching times, output format etc. Unauthorized subscribers cannot receive the video stream, and even if they received it, they cannot decrypt it.

- Copy Protection: Protect the digit content transmitted or stored in the IPTV service network from unauthorized duplication.

- Integrity and confidentiality of the digital content: IPTV service should be able to guarantee the integrity and confidentiality of the digital content stored or delivered in the service network.

- Tracing / Non-repudiation: Digital content owner should be able to trace the illegal usage of the content by technical methods, for example forensic digital watermarking.

- Consider requirements for the labeling of content for filtering purposes.

Key Management System and License Issuer should be defined as two separated parts and the following requirement should be considered:

  - Key Management System Requirement

  - License Issuer Requirement

  - Interface Requirement between Key management system and license issuer

Interoperability

The following areas represent key interoperability elements that are required in the DRM-EE, DRM-B and DRM-IX modes.

Authentication of devices, users and DRM systems

Before content can be exchanged between entities, the identity of the receiving device and possibly its user(s) must be confidently established.  Also, since content providers may not trust specific DRM systems, it is important that it be possible to authenticate the receiving DRM system(s) or implementation levels before exchanging content.  This authentication should have a sound cryptographic basis and may employ various well-known digital signature techniques.  Public Key cryptography in particular provides a sound mechanism for digital signatures in authentication protocols.

Rights Expression Exchange

Different DRM systems use different rights expression languages or license formats.  For DRM-B and DRM-IX modes to function, some means for a common rights expression is required.  This could take the form of a common rights expression language (REL) or a rights expression translator.  Another possible rights expression exchange mechanism is license negotiation.

Common encryption algorithms for content exchange

For content to pass securely from the control of one DRM system to another or within the same DRM system but on different physical devices, content encryption is required.  This renders the content unusable except for entities that possess the appropriate key or keys necessary for decryption to occur.  There are many different types of encryption algorithms (e.g. block ciphers, stream ciphers, public-key-based, etc.) but generally those that use symmetric keys tend to be best suited for high-speed content exchange.  For interoperability purposes, some small number of commonly agreed algorithms must be chosen.  Ideally, one default algorithm would also be specified.

Key management and/or exchange for the common encryption algorithms

Before secure content exchange can take place, keys to be used in specific instances need to be exchanged or commonly generated by the authenticated entities.   Key management is usually the most difficult part of a security system to implement.  Techniques such as Public Key cryptography have simplified device key distribution but require a public key infrastructure (PKI) to establish and maintain the validity of these keys.  Such an infrastructure could be sanctioned and maintained by a license authority which has responsibility for content protection (as opposed to general network security).

Secure download of DRM client

Ideally, any receiving device would be able to exchange content obtained (legitimately) through other devices and/or using any DRM or CAS system according to the granted rights (i.e., the DRM-IX mode).  However, it is not practical to pre-load at manufacture time every receiving device with every CAS and DRM system that market forces will demand.   Thus, a secure mechanism for downloading and executing a selected CAS or DRM system onto a receiving device is needed.  Elements such as secure bootloaders and secure download protocols play a part in this area of interoperability.

- DRM/CAS technology for content security should support the interoperability.

- We should investigate in the following elements for interoperability and for other advantages.

- Authentication of devices, users and DRM systems

- Rights Expression Exchange

- Common encryption algorithms for content exchange

- Key management and/or exchange for the common encryption algorithms

- Secure download of DRM client

- IPTV devices should have a trusted architecture to support interoperability of content security.

Specific Requirements

The following table contains specific requirements that pertain (primarily) to content security aspects:

Table 6-6 - Content Security Requirements

| Number | Requirement | Source |
| --- | --- | --- |
| IPTV_SEC_C_101 | The IPTV security solution shall be scalable to support protection of content that is distributed simultaneously to a very high number of subscribers. | IIF.DRM.General.0100 |
| IPTV_SEC_C_102 | The IPTV security solution shall support protection of content that is transferred over multicast channels and/or over unicast streams. | IIF.DRM.General.0200 |
| IPTV_SEC_C_103 | The IPTV security solution shall make possible rights specifications and protect content for multiple common existing content formats (e.g., MP-3, MPEG-4, etc.) and/or common computing platforms. | IIF.DRM.General.0300 |
| IPTV_SEC_C_104 | For certain content types (such as movies that are stored on a PVR) the IPTV security solution may provide stronger cryptographic measures to protect against theft. | IIF.DRM.General.0400 |
| IPTV_SEC_C_105 | The IPTV Security Solution shall support the securing of DVR content. | IIF.DRM.General.1600 |
| IPTV_SEC_C_106 | The IPTV security solution shall enable the operator to apply robust content tracing to content in real-time (e.g., broadcast content). | IIF.DRM.Operator.1000 |
| IPTV_SEC_C_107 | The IPTV security solution shall enable the operator to apply robust content tracing to content in an offline manner (e.g., VOD Content). | IIF.DRM.Operator.1100 |
| IPTV_SEC_C_108 | The entropy of the key used with the IPTV common scrambling algorithm shall be sufficiently large to protect the content fully from plausible threats (e.g., at least 128 bits for the AES symmetric algorithms). | IIF.DRM.Scrambling.0100-0400 |
| IPTV_SEC_C_109 | The IPTV common scrambling algorithm shall be built using publicly-available cryptographic algorithms that have been standardized by other appropriate organizations and that have been openly vetted by competent cryptographic experts e.g., the Advanced Encryption Standard (AES). | IIF.DRM.Scrambling.0100-0500 |
| IPTV_SEC_C_110 | The specification of the IPTV common scrambling algorithm shall be publicly available. | IIF.DRM.Scrambling.0100-0600 |
| IPTV_SEC_C_111 | The IPTV scrambling algorithm shall not preclude support for either Simulcrypt or Multicrypt approaches. The purpose is to enable multiple DRM vendors within a single network platform when required by the content distributor. | IIF.DRM.Scrambling.0100-0700 |
| IPTV_SEC_C_112 | The scrambling algorithm shall be designed to last a minimum of 10 years with suitable, variable key lengths defined by the appropriate experts. The technical solution shall allow configurable, variable parameters (such as key length, crypto periods, etc.) allowing for scalable and future proof levels of security in the same stream. Given the current rate of technology advances in processor, storage, etc., the designers of a secure scrambling algorithm must consider the capabilities of offline descramblers to eventually decrypt the content within a reasonable amount of time, using “brute force” attacks. The rationale for the scrambling algorithm designers must be a cost/benefit analysis of the cost of implementing a solution for today that will still be valid at least 10 years out in time. | IIF.DRM.Scrambling.0400-0200 |
| IPTV_SEC_C_113 | The IPTV security solution shall pass through and not interfere with the distribution of any content tracing technology encoded within the content. Such content tracing technology may be designed to uniquely identify the supplier of content. | IIF.DRM.Tracing.0100 |
| IPTV_SEC_C_114 | The IPTV security solution shall enable the application of content tracing technology within the output of an IPTV Receiving Device for the purpose of uniquely identifying a session (e.g., channel, time/date), IPTV Receiving Device and/or network operator. Examples of such content tracing technology may include visible and invisible information. | IIF.DRM.Tracing.0200 |
| IPTV_SEC_C_115 | The IPTV security solution shall allow the retrieval of all the content tracing information from the content. | IIF.DRM.Tracing.0400 |
| IPTV_SEC_C_116 | The content tracing information may contain the operator ID, content owner ID, IPTV Receiving Device ID, and other information. | IIF.DRM.Tracing.0500-0100 |

## Service Security Requirement

Specific Requirements

The following table contains specific requirements that pertain (primarily) to service security aspects:

Table 6-7 - Service Security Requirements

| Number | Requirement | Source |
| --- | --- | --- |
| IPTV_SEC_S_101 | The IPTV security solution shall provide a mechanism to signal the IPTV Receiving Device to utilize a specified scrambling algorithm based on a standardized framework. | IIF.DRM.General.0600-0300 |
| IPTV_SEC_S_102 | The IPTV security solution shall have the ability to use standard key management systems (e.g., MIKE, EMM/ECM), to the extent that this is required for interoperability. | IIF.DRM.General.0600-0400 |
| IPTV_SEC_S_103 | The server side DRM interfaces shall provide the capability to update and query the DRM System concerning the IPTV Scrambling Algorithm and any other operator-selected scrambling algorithms. | IIF.DRM.General.0700-0600 |
| IPTV_SEC_S_104 | The IPTV Security Solution shall support rights management that is independent of specific content formats or standards. | IIF.DRM.General.1400 |
| IPTV_SEC_S_105 | The IPTV solution shall provide a mechanism for secure delivery of entitlements to the IPTV Receiving Devices. | IIF.DRM.General.1400-0100 |
| IPTV_SEC_S_106 | The IPTV Security Solution shall support content usage control (e.g., re-play entitlements). | IIF.DRM.General.1700 |
| IPTV_SEC_S_107 | The IPTV Security Solution shall support different modes of re-play entitlements, e.g., limit on number of plays, time limit on plays, restriction of Fast Forward or Rewind, etc. | IIF.DRM.General.1700-0100 |
| IPTV_SEC_S_108 | The IPTV Security Solution shall support impulse transactions. | IIF.DRM.General.1700-0200 |
| IPTV_SEC_S_109 | The IPTV security solution shall support a mechanism for transmitting signaling messages between the DRM Server and the IPTV Receiving Device DRM Component. | IIF.DRM.General.2100 |
| IPTV_SEC_S_110 | The IPTV security solution shall support a mechanism to allow for the confidentiality of signaling messages. | IIF.DRM.General.2100-0100 |
| IPTV_SEC_S_111 | The IPTV security solution shall support a mechanism to allow for the authenticity of signaling messages. | IIF.DRM.General.2100-0200 |
| IPTV_SEC_S_112 | The IPTV security solution shall support a mechanism to allow for the integrity of signaling messages. | IIF.DRM.General.2100-0300 |
| IPTV_SEC_S_113 | In order to provide content confidentiality, the headend may support multiple scrambling algorithms; in that case, the IPTV operator shall be able to select and/or enable/disable any of the supported scrambling algorithms at the headend. | IIF.DRM.Operator.0100 |
| IPTV_SEC_S_114 | The IPTV security solution should provide a “blackout mechanism”: a mechanism for limiting viewing-rights of certain programs to certain groups of subscribers e.g., block viewing by residents of a specific area (for example, this may be useful for sporting events). | IIF.DRM.Operator.0200 |
| IPTV_SEC_S_115 | The IPTV security solution shall provide a mechanism to allow the IPTV operator to securely retrieve the parameters (e.g., configuration, status) of an IPTV Receiving Device DRM Component. | IIF.DRM.Operator.0500 |
| IPTV_SEC_S_116 | The IPTV security solution shall provide a mechanism to allow IPTV operator to securely update the parameters (e.g., configuration) of IPTV Receiving Device DRM Components. | IIF.DRM.Operator.0600 |
| IPTV_SEC_S_117 | The IPTV security solution shall enable the operator to turn on and off content tracing function with flexibility (e.g., based on time, an Event, Asset, or Channel). | IIF.DRM.Operator.0900 |

## Network Security Requirement

Specific Requirements

The following table contains specific requirements that pertain (primarily) to network security aspects:

Table 6-8 - Network Security Requirements

| Number | Requirement | Source |
| --- | --- | --- |
| IPTV_SEC_N_101 | The IPTV security solution shall work well, even in the presence of the following: NAT/NAPT, Firewalls, DHCP. | IIF.DRM.General.2800 |

## Terminal Device Security Requirement

Specific Requirements

The following table contains specific requirements that pertain (primarily) to terminal security aspects:

Table 6-9 - Terminal Security Requirements

| Number | Requirement | Source |
| --- | --- | --- |
| IPTV_SEC_T_101 | The IPTV Security Solution shall support a method to authenticate IPTV Receiving Devices. | IIF.DRM.General.1000 |
| IPTV_SEC_T_102 | The IPTV Security Solution shall support physical tamper-resistance. | IIF.DRM.General.1800 |
| IPTV_SEC_T_103 | The IPTV Security Solution shall support physical tamper-detection. | IIF.DRM.General.1900 |
| IPTV_SEC_T_104 | The IPTV Security Solution shall support secure download and install of the DRM operating code to IPTV Receiving Devices. | IIF.DRM.General.2300 |
| IPTV_SEC_T_105 | The IPTV scrambling algorithm shall be efficiently implementable for both hardware and software implementations. | IIF.DRM.Scrambling.0200-0100 |

## Subscriber Security Requirement

Specific Requirements

The following table contains specific requirements that pertain (primarily) to subscriber (end-user) security aspects:

Table 6-10 - Subscriber Security Requirements

| Number | Requirement | Source |
| --- | --- | --- |
| IPTV_SEC_E_101 | As rights allow, the IPTV security solution should allow the end user to change an IPTV Receiving Device without inherently affecting his/her rights to consume content. | IIF.DRM.General.0500 |
| IPTV_SEC_E_102 | The IPTV security solution shall support a mechanism to allow an IPTV Receiving Device DRM Component to authenticate the DRM Servers. | IIF.DRM.General.2000 |
| IPTV_SEC_E_103 | The IPTV security solution should not significantly degrade the performance of the IPTV services. | IIF.DRM.General.2700 |
| IPTV_SEC_E_104 | The IPTV security solution shall provide a mechanism for restricting access to programs/system by allowing the subscriber to set an access code (password). | IIF.DRM.Subscriber.0100 |
| IPTV_SEC_E_105 | The IPTV security solution shall provide a mechanism for parental rating i.e., facilities for rating programs according to content. | IIF.DRM.Subscriber.0200 |
| IPTV_SEC_E_106 | The IPTV security solution shall provide subscribers flexible usage controls to limit access to content by content type (such as VOD, TV, WWW, etc.). | IIF.DRM.Subscriber.0300 |
| IPTV_SEC_E_107 | The IPTV security solution shall be capable of indicating why access to content has been denied to the user. | IIF.DRM.Subscriber.0700 |
| IPTV_SEC_E_108 | The IPTV security solution shall allow for a mechanism for a Subscriber to obtain extensions (e.g., more plays, more play-time) to digital rights associated with specific content instances. | IIF.DRM.Subscriber.0800 |

### Subscriber Personal Data Requirements

When implementing IPTV services to general user, it is necessary to give sufficient consideration to the subscriber’s personal data of subscriber. This subscriber’s personal data includes basic data such as address, name, age, occupation, and telephone number, as well as various other kinds of information, such as family composition, hobbies, and household income.

- It is desirable to handle the minimum subscriber’s personal data necessary for delivering the IPTV services.

- It is desirable to explain the intended use of the subscriber’s personal data and obtain the consent for this from the subscriber before collecting the information necessary to deliver the IPTV services.

- It is desirable to appropriately destroy subscriber’s personal data that becomes unnecessary for the continuation of IPTV services.

- When the service provider administers the subscriber’s personal data, it is desirable to store the collected data under strict security

There are three possible routes by which subscriber’s personal data can leak. These are leaking from the service company, leaking from the network, and leaking from the home, e.g. through receiver devices. Here we present methods for protecting the subscriber’s personal data for each of these leakage routes.

#### Subscriber Personal Data Leakage from Service Provider

In order to prevent leakage of subscriber’s personal data, the IPTV service operator must pay careful attention to the points below.

- Classifying subscriber’s personal data into that which needs to be controlled and that which does not

- Securely administering the subscriber’s personal data that needs to be controlled

- Ensuring that subscriber’s personal data that needs to be controlled is not used for any purpose other than that intended

#### Subscriber Personal Data Leakage from Network Provider

IPTV service companies must pay careful attention to the points below in relation to services/transactions that involve handling of subscriber’s personal data.

- Classifying subscriber’s personal data into that which needs to be controlled and that which does not

- Using encrypted communication channels for transmission of subscriber’s personal data that needs to be controlled

#### Subscriber Personal Data Leakage from Terminal Device

IPTV service companies sometimes store subscriber’s personal data in terminals, in order to enhance service efficiency. In such cases, they must pay careful attention to the points below. Furthermore, it is necessary to consider security when transferring terminals.

- Ensuring that no third party can easily read out subscriber’s personal data stored inside terminals

- It is desirable to control access to subscriber’s personal data stored in terminals

- Ensuring that subscriber’s personal data stored in terminals can be completely deleted by subscriber or service providers

- It is desirable that terminals are protected against infection by computer viruses in the near future.

### Protection of Legal Minors

In a typical usage pattern, a receiver for IPTV services is shared in a home by multiple people of a wide age range.

- Ensuring that parental ratings can be set for content as necessary

- Ensuring that terminals can be operated in accordance to parental ratings

- Ensuring that terminals are capable of changing parental rating settings of the terminals

- Ensuring that terminals are capable of password-based controls, so that only the guardians of legal minors can change parental ratings

Note that it is necessary to consider the conditions of each country in relation to third-party organizations for eliminating harmful content, since this is related to content censorship. When considering simultaneous retransmission of broadcasting, we can assume that the original content creator gives sufficient consideration to this at the time of producing the content, so it is necessary to give sufficient consideration to transmission delays and distribution cost increases.

#### Content Labeling

Detailed and at the same time sufficiently general information labelling content or service for possibly offensive material is needed. As an example of such information, ETSI TS 102 822 [2] defines a schema for “Content Alert” metadata, a set of elements that alert people about content they might not want to see/hear (or they might not want their children to see/hear). The metadata is detailed enough to indicate content with labels for, for examples, “Scenes of explicit sexual behaviour suitable for adults only”; “Occasional use of very strong language”; “Scenes with extreme horror effects”; “Strobing that could impact on those suffering from Photosensitive epilepsy”.

# Security Architecture

The assets that need to be protected in IPTV service are:

- Content asset: which reside on transit networks controlled by IPTV businesses along the distribution chain, on networks owned by the IPTV consumer and on the IPTV terminal device.

- Service assets: servers such as media servers, AAA servers, DRM servers, ..., and operation information such as service logs and billing information.

- Network assets: equipments (e.g. routers, switches), bandwidth, multicast services, and CDN.

- Terminal assets, which the residential consumer uses to process and store content works and other information related to the IPTV service.

- Subscriber assets: which consist of information about the subscriber, the subscriber household, and their IPTV transactions that are processed at multiple points along the IPTV delivery chain.

## Security Trust Model

[TBD]

## Security General Framework

[TBD]

### Content Protection Architecture

[TBD]

## Security Function Modules

[TBD]

## Security Interfaces

[TBD]

# Security Mechanisms

[TBD]

## Content Security Mechanisms

[TBD]

### Content Protection

[TBD]

### Conditional Access

[TBD]

### Digital Rights Management

[TBD]

## Service Security Mechanisms

[TBD]

### Service Authentication

[TBD]

### Service Authorization

Based on the layered NGN architecture in Y.2012 [1], a basic mechanism to authorize the Linear TV services is as follows:

- The IPTV application functions send the rights information related with a particular user to the transport functions to which terminal is connected. The transport functions store the received rights information.

- When receiving a user request, the transport functions perform the service authorization procedures according to the stored rights to determine whether to grant access to the user

The procedure is depicted below in Figure 8-3 - Service Authorization Procedure defined as follows:

- User Subscribes or requests a channel via Service Control Functions.

- The IPTV application gets the rights related with the subscribed channel.

- IPTV application  sends the rights information to the transport functions via transport control functions.

- The transport functions store the rights information.

- The transport functions notify the IPTV application of successfully receiving the rights information.

- The IPTV application sends a response back to the terminal.

- The user requests a TV channel by sending a message to the transport functions.

- The transport functions perform service authorization according to the stored rights information.

- Based on the authorization, the transport functions grant access or deny the request

Figure 8-3 - Service Authorization Procedure

## Network Security Mechanisms

[TBD]

## Terminal Device Security Mechanisms

[TBD]

### Terminal Device Authentication

[TBD]

### Terminal Device Authorization

[TBD]

## Subscriber Security Mechanisms

[TBD]

### Subscriber Authentication

[TBD]

### Subscriber Authorization

[TBD]