| The following information has been agreed by WG2 to be under further consideration by WG2 prior to inclusion in formal output documents. INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-DOC-0106 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 4th FG IPTV meeting: Bled, Slovenia, 7-11 May 2007 |  |  |
| OUTPUT DOCUMENT |  |  |  |  |
| Source: | Editor |  |  |  |
| Title: | Living List: Application layer reliability solutions for IPTV |  |  |  |

### Item 1 (Busan Meeting): ARQ

Source: FG IPTV-C-0220, ETRI, “Existing standards/activities for ARQ-based application layer reliability solutions”

### Item 2 (Mountain View Meeting):  ATIS IIF IPTV Packet Loss Issue Report

Source: FG IPTV-IL-0040, ATIS IIF,  “ATIS IIF Liaison to ITU-T Focus Group on IPTV”

Enclosed in this incoming liaison from ATIS IIF is the ATIS IIF IPTV Packet Loss Issue Report (ATIS-0800005). The IPTV Packet Loss Issue report is a technical report that explores the range of potential solutions and makes recommendations regarding their applicability for an IPTV service.

ATIS IIF offers to the ITU-T Focus Group on IPTV that if wishes to use text from these documents for the purposes of developing IPTV standards and requirements to contact the liaison contact person.

It is was decided that the ATIS IIF IPTV Packet Loss Issue Report (ATIS-0800005) is kept in this document for easy access of information relevant to the work on Application layer reliability solutions for IPTV in the ITU-T Focus Group on IPTV.

### Item 3 (Mountain View Meeting): Application-Layer FEC

Sources:

- FG IPTV-IL-0031, DVB Project,  “Response to "FG IPTV-OD-0008 Rev.1: Liaison to DVB Project on QoS and Performance work for IPTV" on Application Layer FEC in DVB-IPI”

- FG IPTV-C-0405, Digital Fountain, “Application layer Forward Error Correction for IPTV services”

Incoming Liaison FG IPTV-IL-0031 from the Digital Video Broadcasting Project (DVB) Technical Module group on IP Infrastructure (TM-IPI) informed the Focus Group on IPTV (FG IPTV), specifically the FG IPTV WG2 (QoS and Performance Aspects), on DVB’s work on Application-layer Forward Error Correction (AL-FEC).

DVB TM-IPI provided updated information on the work on AL-FEC: The DVB Technical Module (TM) approved the specification of the AL-FEC to be published as an Annex to the DVB-IP phase 1.3 specification (TS 102 034) on November 23, 2006. However, publication of this specification still requires

- the inclusion of the Service Discovery and Setup (SD&S) into the main body of DVB-IP phase 1.3 specification and therefore the approval of the main body at the next DVB TM meeting in January,

- the formal approval of DVB Commercial Module (CM) in January 2007, and

- the formal approval of DVB Steering Board in February 2007.

Nevertheless, with the new achievements with respect to Application Layer FEC in DVB it seems quite appropriate to inform ITU IPTV FG on the status of the work. However, due to the public document policy of ITU FG it was not possible to send the not yet published specification to the ITU IPTV FG meeting in January 2007. However, a powerpoint presentation which contains relevant background on DVB’s work on AL-FEC was attached to the letter. Digital Fountain as a member of DVB and one of the leaders in the definition of the DVB-IPI specification on AL-FEC presented the slide set at the third ITU IPTV FG meeting.

The presentation is included into this living list.

FG IPTV-C-0405 proposed, among others, to recommends and references the solution from DVB-IPI once it is available. Furthermore, it proposed to assesses the DVB-IPI solution in all application environments, which are of interest for ITU IPTV FG.

Based on the these two documents, it is decided to

- send a reply Liaison Statement to the DVB project kindly asking for:

  - the published specification once it is available or the status of the work in DVB.

  - further information on the evaluation process which lead to the specification.

  - Additional information on the usage of the specification in different environments, e.g. implementation guidelines, etc.

- study the adoption of DVB-IPI AL-FEC solution for ITU FG IPTV AL-FEC. The study will comprise, among others,

  - the assessment of the applicability of the DVB-IPI AL-FEC specification to ITU FG IPTV architecture.

  - Performance evaluation for ITU FG IPTV relevant use cases.

### Item 3 continued (Bled Meeting): Application-Layer FEC

Sources:

- FG IPTV-IL-0064, Digital Video Broadcasting Project; Technical Module subgroup on IP Infrastructure (DVB TM-IPI),  “Response to FG IPTV-OL-0020 on Application Layer FEC in DVB-IPI”

In incoming Liaison Statement FG IPTV-IL-0064 the Digital Video Broadcasting Project (DVB) Technical Module group on IP Infrastructure (TM-IPI) stated the appreciation of the efforts of the ITU-T FG IPTV WG2 to study the adoption of DVB-IP AL-FEC solution for ITU-T FG IPTV AL-FEC.

In this document, the DVB project provided ITU-T FG IPTV WG2 with the latest information on the AL-FEC work for your upcoming meeting. Specifically, the following recently published documents have been provided:

| - A086rev4 - Transport of MPEG 2 Transport Stream (TS) Based DVB Services over IP Based Networks (DVB-IPTV Phase 1.3 - Draft TS 102 034 1.3.1) as a bluebook which contains in Annex E the specification of the DVB-IPI AL-FEC. |  |
| --- | --- |
| - A115 - Blue Book on AL-FEC Evaluations also as a bluebook which is intended to provide background on the evaluation process behind the choice of the hybrid approach in the DVB application layer forward error correction (AL-FEC) specification as included in the DVB-IP Phase 1 Handbook. |  |

A115 document consists of two documents that were created by DVB as part of the AL-FEC evaluation process. Document A presents the evaluation criterion that were agreed before the selection process started and Document B is the report on the evaluation process. Document B also gives the rationale for the choice of the hybrid approach used in the DVB AL-FEC specification.

Both documents can also be downloaded from http://www.dvb.org/technology/bluebooks/index.xml.

- [DVB-A086] 			DVB Blue Book A086rev4 (2007) - Transport of MPEG 2 Transport Stream (TS) Based DVB Services over IP Based Networks (DVB-IPTV Phase 1.3 - Draft TS 102 034 1.3.1), see: http://www.dvb.org/technology/bluebooks/a086r4.tm3727r1.dTS102034.V1.3.1.DVB-IP1.3.pdf-xml.zip

- [DVB-A115]			DVB Blue Book A0115 (2007) -  DVB Application Layer FEC Evaluations, see http://www.dvb.org/technology/bluebooks/a115.tm3783.AL-FEC_Evaluation.pdf.

Based on this document and two additional input contributions, the following conclusions were drawn:

- In the case that a network can not fulfil the requested IPTV service requirements tbd, the use of the DVB-IPI AL-FEC solution [Annex E, DVB-A086] is provisionally endorsed as the ITU-T AL-FEC solution.

  - The details on what parts of the DVB-IPI AL-FEC solution are required, and what is recommended, or what can optionally be used, is tbd.

  - The relation of the AL-FEC solution to the QoS classes in Y.1541 [ITU-Y.1541], especially recommended parameters settings for different QoS classes and different services is tbd.

Provision to the endorsement was made due to the following two reasons:

- The specification [Annex E, DVB-A086] and the accompanying evaluation document [DVB-A0115] was made available to the ITU-T IPTV FG only 3 days prior to the 4th meeting of the ITU-T FG on IPTV. Therefore, some more thorough checking of the details is necessary for some ITU-T IPTV FG participants.

- As DVB is not among the Organizations Qualified for Including References in ITU-T Recommendations under Rec. A.5 Procedures, it formally not possible to add this endorsement to the Working document. The group agreed that an individual company submission at the next meeting with copy&paste of the solution specified in [Annex E, DVB-A086] or to await ETSI publication of the DVB specification will be able to solve this formal issue.

____________

The specification [Annex E, DVB-A086] and the accompanying evaluation document [DVB-A0115] was made available to the ITU-T IPTV FG only 3 days prior to the 4th meeting of the ITU-T FG on IPTV. Therefore, some more thorough checking of the details is necessary for some ITU-T IPTV FG participants.