| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-DOC-0119 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 5th FG IPTV meeting: Geneva, Switzerland, 23-31 July 2007 |  |  |
| OUTPUT DOCUMENT |  |  |  |  |
| Source: | Acting Editor |  |  |  |
| Title: | Working document: Traffic Management Mechanisms for the Support of IPTV Services |  |  |  |

Summary

TBD

Keywords

TBD

Introduction

TBD

Contents

1	Scope	3

2	References	3

3	Definitions	3

4	Abbreviations and acronyms	3

5	Conventions	3

6	Traffic management mechanisms	3

6.1	Control plane	3

6.1.1	Admission control	3

6.1.4	QoS routing	3

6.1.5	Resource management	3

6.2	Data plane	3

6.2.1	Traffic conditioning	3

6.2.2	Transmission Schedulers	3

6.2.3	Buffer management	3

6.3	Multicast	3

6.3.1	Multicast replication point	3

7	IPTV service mapping to IP performance classes	3

7.1	Network QoS classes	3

7.2	Association of IPTV services with Y.1541 classes	3

7.3	Mapping of IPTV services with QoS classes	3

8	Video coding and transmission over IP networks	3

9	Cross-layer interaction for IPTV services	3

10	Traffic management capabilities: core networks	3

10.1	IP network traffic management capabilities	3

10.2	IP traffic management capabilities applicable to IPTV	3

11	Traffic management capabilities: access networks	3

11.1	Overview of access network technologies	3

11.2	Access traffic management capabilities applicable to IPTV	3

12	Traffic management capabilities: home networks	3

Traffic Management Mechanisms for the Support of IPTV Services

# Scope

This document describes a set of traffic management mechanisms which needs to be implemented at the transport network for the efficient support of IPTV services. The network supporting IPTV services will span a number of network segments, with the possibility of being established by different providers and may differ in their traffic management capabilities. Therefore, it is expected that the network provider (s) will implement a subset of these mechanisms to ensure IPTV service objectives are satisfied efficiently.

Traffic management mechanisms will be discussed for the home, access, and core networks.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this working document. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this working document are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this working document does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.264]		ITU-T Recommendation H.264 (2005), Advanced video coding for generic 					audiovisual services

[ITU-T Y.1541]		ITU-T Recommendation Y.1541 (2006), Network performance objectives for 				IP-based services

[ITU-T Y.1291]		ITU-T Recommendation Y.1291(2006), An architectural framework for 					support of Quality of Service in packet networks

[IETF RFC2250]	IETF RFC 2550 (1998), RTP Payload Format for MPEG1 and MPEG2 Video

[IETF RFC3984]	IETF RFC 3984 (2005), RTP Payload Format for H.264 Video

[IETF RFC 2753]	IETF RFC 2753 (2000), A Framework for Policy Based Admission Control

[IETF RFC 2309]	IETF RFC 2309 (1998), Recommendations on Queue Management and 					Congestion Avoidance in the Internet

[IETF RFC 3247]	IETF RFC 3246 (2002), An Expedited Forwarding PHB (Per Hop Behavior)

[IETF RFC 2597]	IETF RFC 2597 (1999), Assured Forwarding PHB Group

[IETF RFC 2386]	IETF RFC 2386 (1998), A Framework for QoS-based Routing in the Internet

[IETF RFC 3272]	IETF RFC 3272 (2002), Overview and Principles of Internet Traffic Engineering

[HGI]				Home Gateway Initiatives, Home Gateway Technical Requirements, Release 1

# Definitions

This Working Document uses or defines the following terms:

# Abbreviations and acronyms

This Working Document uses the following abbreviations and acronyms:

AR		    Access Router

BAS		    Broadband Access Server

BRAS		    Broadband Remote Access Server

- BE	Best Effort

- DSLAM	Digital Subscriber Line Access Multiplexer

- DSCP	Differentiated Services Codepoint

- IGMP	Internet Group Management Protocol

IP	Internet Protocol

IPDV	IP packet Delay Variation

IPER	IP packet Error Ratio

IPLR	IP packet Loss Ratio

IPRR	IP Packet Reordering Ratio

IPTD	IP Packet Transfer Delay

LAN	Local Area Network

- LL	Low Loss

- MPEG	Moving Picture Experts Group

NAL	Network Adaptation Layer

NALU	Network Adaptation Layer Unit

OSPF	Open Shortest Path First

PDU	Protocol Data Unit

- PPV	Pay Per View

- PVR	Private Video Recorder

QoS	Quality of Service

RP	Replication Point

RSVP	Resource Reservation Protocol

- RTI	Real-Time Interactive

- RTMU	Real-Time Multicast & Unicast

RTP	Real Time Protocol

TCP	Transport Control Protocol

VCL	Video Coding Layer

VoD	Video on Demand

VoIP	Voice over IP

VTC	Video Teleconference

HG	Home Gateway

# Conventions

TBD

# Traffic management mechanisms

This section provides an overview of the basic traffic management capabilities that are commonly deployed in a managed network. A provider may choose to deploy a subset of these capabilities depending on the performance objectives of applications supported by the network.

Traffic management is a set of generic network mechanisms for controlling the network service response to a service request, which can be specific to a network element, or for signalling between network elements, or for controlling and administering traffic across a network, mainly including bandwidth allocation, admission control, packet classification/marking, congestion management, congestion avoidance, traffic policing, and traffic shaping and line rate constraint, etc. Its basic processing sequence is shown in Figure 6-1.

Transmission

Token bucket

Queue1

Queue2

QueueN

Queue0

Token

Classification/Marking

Traffic policing Traffic Shaping

Discarding

Discarding

Out queuing

CAR

GTS

RED

WRED

In queuing

Other

Processing

Origin address

Destination address

Destination port

Origin port

Protocol type

TOS

Congestion Avoidance

Queue

Packet should

be transmitted

by this interface

FIFO

PQ

CQ

WFQ

CBWF

Figure 6-1:	The basic processing sequence of traffic management

The traffic management framework contains mechanisms dealing with the user traffic directly, the pathways through which user traffic travels, and the operation, administration and management aspects of the network. As depicted in Figure 6-2, its building blocks are organized into three planes: control plane, data plane, and management plane.

Buffer

Management

Congestion

Avoidance

Packet

Making

Queuing&

Scheduling

Buffer

Shaping

Traffic

Policing

Traffic

Classification

Data Plane

Admission

Control

QoS

Routing

Resource

Management

Control Plane

Management Plane

Metering

Policy

Service Level

Agreement

Figure 6-2: Architectural framework for QoS support

## Control plane

### Admission control

This mechanism controls the traffic to be admitted into the network. Normally the admission criteria are policy driven [IETF RFC 2753]. Whether traffic is admitted depends on an a priori service level agreement. In addition, the decision can depend on whether adequate network resources are available so that newly admitted traffic does not overload the network and degrade service to ongoing traffic. For a service provider, maximal traffic should be admitted while the same level of QoS (including transaction performance as well as service reliability/availability expectations) is maintained for the existing traffic.

Admission control meets requirements for IPTV multicast/unicast services as negotiated in the SLA. Specifically, the desired IPTV services reliability/availability can be requested as a priority level for admission control. Admission control mechanisms can efficiently support QoS assurance for IPTV services on a managed network. IPTV Services supported by admission control will meet unicast and multicast service demand for subscribers. Unicast admission control ensures IPTV unicast services, such as VOD, Time Shifted TV etc. Multicast admission control satisfies multicast services, such as BTV, NVOD etc. The policy of multicast admission control is triggered through subscribers joining multicast groups via IP multicast protocols (e.g., IGMP).

#### Admission control for IPTV services

In the current environment, i.e. Cable or Satellite TV, support of basic broadcast TV service is managed on subscription basis. Once subscribed, there is an expectation by the end customer that the broadcast TV service will continue without interruption with any further need for admission every time the TV set is turned on. This behaviour must be allowed for IPTV to be able to compete with Cable and Satellite. Therefore, applying admission control to regular broadcast TV service is not recommended and management of the service must continue to be on subscription basis. A service provider ensures that the network is properly engineered to accommodate the number of subscribers that match his business model.

There are scenarios where multiple set top boxes may exist in a single home and reception of multiple video streams may be restricted due to the access bandwidth. These scenarios may require the service provider to limit the number of streams admitted for a single subscriber.

There are other scenarios where reception of some video stream programs may be forbidden due to no appropriate subscriber privileges. These scenarios may require the network provider to limit the number of streams admitted for a single subscriber.

Other IPTV services may be subject to admission control.

For example a service provider may apply admission control to on-demand services. The request for on-demand service involves the exchange of signalling between user equipment (e.g. set-top box) and the service provider video server. The service provider may reject the on-demand request based on network resources, user status, or the current use of a specific program across all subscribers. Criteria such as user standing and the current use of a given program are related to policies set by the service provider and do not change very frequently.

As another example, a network provider may apply admission control to on-demand services. Request for on-demand services involves exchanging of signalling between user equipment and network provider, e.g., DSLAM equipment that maintains subscriber privileges in a local database. Upon receiving this request, the network provider may reject this request based on subscriber privileges or allow media stream from media source behind service provider to subscriber.

The effect of an on-demand request on the network resources is a dynamic parameter and it varies based on the program requested. For instance requesting high definition content will impose more demand on the network compared to the demand generated when low definition content is requested. The role of the admission function in this case is to accept or reject the on-demand request based on the traffic demand of the new request and the current utilization of network resources.

#### Admission control example

Figure 6-3 shows an example of admission control for on-demand service in an IPTV service scenario using IMS.

On-demand

Request

Figure 6-3:	Admission Control Example

The admission sequence involves the generation of the request by customer equipment using session level protocol. The request is received by the core control function, e.g. the core IMS in this case, that maintains records of the transport resource utilization and the traffic requirements of the current request. It also consults with service support functions to ensure, for instance, availability of the requested program. Based on this information the incoming request may be accepted or rejected and the result of the admission process is reported back to the customer.

### QoS routing

This text is based on a modification of ITU-T Rec. Y.1291[ITU-T Y.1291], Section 7.2.

This mechanism is used to control network congestion and improve network utilization by selecting an appropriate path. The path selected is most likely not the traditional shortest path.  Depending on the specifics and the number of QoS metrics involved, computation required for path selection can become prohibitively expensive as the network size grows.  Hence practical QoS routing schemes consider mainly cases for a single QoS metric (e.g., bandwidth or delay) or for dual QoS metrics (e.g., cost-delay, cost-bandwidth, and bandwidth-delay). To further reduce the complexity of path computation, various routing strategies exist. According to how the state information is maintained and how the search of feasible paths is carried out, there are strategies such as source routing, hierarchical routing, and distributed routing. And according to QoS granularity and model expansibility, there are strategies such as intserv, diffserv and class based intserv in which the network is divided into several QoS autonomous systems (i.e., QoS domains) and each QoS domain may adopt its own resource allocation scheme so that end to end QoS can be achieved based on resources allocated in a single QoS domain or multiple QoS domains. In addition, according to how multiple QoS metrics are handled, there are strategies such as metric ordering and sequential filtering, which may trade global optimality with reduced computational complexity [IETF RFC 2386].

The path selection process involves the knowledge of the flow’s QoS requirements and characteristics and (frequently changing) information on the availability of network resources (expressed in terms of standard metrics such as available bandwidth and delay).  The knowledge is typically obtained and distributed with the aid of signalling protocols.  For example, RSVP [IETF RFC 2205] can be used for conveying a flow’s requirements and characteristics and OSPF extensions as defined in IETF RFC 2676 for resource availability.  Compared with shortest-path routing that selects optimal routes based on a relatively constant metric (i.e., hop count or cost), QoS routing tends to entail more frequent and complex path computation and more signalling traffic.

It is important to note that QoS routing provides a means to determine only a path that can likely accommodate the requested performance.  To guarantee performance on a selected path, QoS routing needs to be used in conjunction with resource reservation to reserve necessary network resources along the path.

QoS routing can also be generalized to apply to traffic engineering. (Concerning slowly-changing traffic patterns over a long time scale and a coarse granularity of traffic flows, traffic engineering encompasses traffic management, capacity management, traffic measurement and modelling, network modelling, and performance analysis.)  To this end, routing selection often take into account a variety of constraints such as traffic attributes, network constraints, and policy constraints [IETF RFC 3272].  Such generalized QoS routing is also called constraint-based routing.

### Resource management

The network resource management system acquires network traffic information (e.g. QoS requests) from a network resource manager of a bearer control layer, and carries out network traffic management according to the acquired network information by means of data transmission between the network resource manager and the network nodes (e.g. DSLAM, BRAS, switches, routers, gateways etc.). A network management interface needs to be configured on the resource manager for exchanging the network information with the network management system, and the network information is acquired from the resource manager via the configured network management interface, and is added, modified and deleted in the network management system. By employing the network resource manager of the bearer control layer, high-quality IPTV services are able to be provided in the IPTV system.

## Data plane

### Traffic conditioning

The following are required in access network for supporting multimedia service such as IPTV.

- QoS provisioning per subscriber and per service

- Traffic classification based on multiple header fields (5-tuple, 6-tuple, and etc.)

- Minimum bandwidth guarantee

- Traffic shaping/policing

- Advanced packet queuing and scheduling

- Traffic marking (DSCP, 802.1p, and etc.)

### Transmission Schedulers

This mechanism controls which packets to select for transmission on an outgoing link. Incoming traffic is held in a queuing system, which is made of, typically, multiple queues and a scheduler. Governing the queuing system is the queuing and scheduling discipline it employs. There are several key approaches:

– First-in, first-out scheduler: Packets are placed into a single queue and served in the same order as they arrive in the queue.

– Round-Robin Scheduler: Packets are classified based on some criteria. One possible criterion is the classification based on flow identity. A less granular and consequently more scalable is to classify packets based on application or forwarding classes. Each class is then assigned a separate queue, and queues are serviced in round-robin fashion. .

– Priority scheduler: Packets are first classified and then placed into different priority queues. Packets are scheduled from the head of a given queue only if all queues of higher priority are empty. Within each of the priority queues, packets are scheduled in first-in, first-out order.

– Weighted Round-Robin scheduler: Packets are classified into flows and assigned to queues dedicated to respective flows. A queue is assigned a percentage of output bandwidth according to the bandwidth need of the corresponding flow. By distinguishing variable-length packets, this approach also prevents flows with larger packets from being allocated more bandwidth than those with smaller packets.

– Class-based scheduler: Packets are classified into various service classes and then assigned to queues assigned to the service classes, respectively. Each queue can be assigned a different percentage of the output bandwidth and is serviced in round robin. Empty queues are skipped.

– Hierarchical scheduling:  to be defined.

### Buffer management

Queue or buffer management deals with which packets, awaiting transmission, to store or drop. An important goal of buffer management is to minimize the steady-state buffer size while not underutilizing link as well as avoiding the monopolization of the available buffer space by a single flow [IETF RFC 2309]. Schemes for buffer management differ mainly in the allocation of the buffer space and the criteria for dropping packets. .

A common criterion for dropping packets is the buffer reaching its maximum size. Packets are dropped when the buffer is full. Determining which packet to drop depends on the drop disciplines, for example: :

– "Tail drop" drops the newly arriving packet. This is the most common strategy since it is easy to implement. .

– "Front drop" A buffer space for the newly arriving packet is made available by dropping the packet at the front of the queue, provided that the length of the newly arriving packet is less than or equal to the dropped packet. .

– ""Random drop" A buffer space for the newly arriving packet is made available by randomly choosing a packet to drop from the queue, provided that the length of the newly arriving packet is less or equal to the dropped packet.

Active queue management (AQM) is a scheme for dropping packet before the buffer reaches its full size. Random Early Drop (RED) is an example of theses schemes. AQM algorithms are developed in the context of TCP congestion avoidance scheme with the objective to improve network throughput. AQM may not be applicable to IPTV traffic since TCP is not likely to be used as the transmission protocol for these applications.

## Multicast

IPTV network supports transportation of multicast traffic and multicast admission control policy for QoS of IPTV services. The multicast admission control system based on users’ admission control policy allocates, reserves resource and controls IPTV multicast transportation for subscribers. The multicast replication function forwards the IPTV multicast stream to subscribers according to their successful resources allocation in the network control system. So quality of service for IPTV multicast services will be able to be guaranteed by multicast admission control.

### Multicast replication point

Replication of video streams is one of the essential functions for multicast and selection of applicable replication points (RPs) within the IPTV network is important because it may affect bandwidth utilization and traffic management complexity.

The network equipment serving as the multicast RP is required to have good functionality and processing capability. Network elements including Broadband Access server, Access Router and L2 equipment can perform multicast replication.

There is a trade-off relationship when we select RPs within the network. The nearer the multicast replication points approach to users, the less bandwidth the network needs, while the management becomes more complex.

When selected as the multicast RP, the BAS/AR implements the user-oriented replication without requiring the access network to support multicast. This solution supports simplicity in management. On the other hand, the BAS/AR poses higher bandwidth requirement for the access network. Moreover, the BAS/AR demands more on its port density and processing capability.

If L2 equipment nearest to the user (such as DSLAM or LAN switch) is selected as the multicast replication point, the bandwidth of the access network can be saved at the most. This solution supports distributed replication, so it has a relatively weak requirement for processing capability at the replication point. Hence, it is applicable for large-scale service deployment. However, the access network must support multicast, and the terminal equipment is required to support IGMP snooping. And also this solution had the complexity in management.

# IPTV service mapping to IP performance classes

## Network QoS classes

- ITU-T Recommendation Y.1541 defines classes of network Quality of Service (QoS) with objectives for Internet Protocol performance parameters. Each QoS class defines a specific combination of limits on the performance values. The following Table 1 shows IP network QoS definitions, network performance objectives and guidance for applicability. Class 6 and 7 are intended to support the performance requirements of high bit rate user applications that have more stringent loss/error requirements than those supported by classes 0 through 4.

- Table 1 – IP network QoS class definitions and network performance objectives/Applications

| - QoS class | - IPTD | - IPDV | - IPLR | - IPER | - IPRR | - Applications (examples) |
| --- | --- | --- | --- | --- | --- | --- |
| - 0 | - 100 ms | - 50 ms | - 1 x 10-3 | - 1 x 10-4 | - - | - Real-time, jitter sensitive, - high interaction (VoIP, VTC) |
| - 1 | - 400 ms | - 50 ms | - 1 x 10-3 | - 1 x 10-4 | - - | - Real-time, jitter sensitive, - Interactive |
| - 2 | - 100 ms | - U | - 1 x 10-3 | - 1 x 10-4 | - - | - Transaction data, - highly interactive (Signalling) |
| - 3 | - 400 ms | - U | - 1 x 10-3 | - 1 x 10-4 | - - | - Transaction data, interactive |
| - 4 | - 1 s | - U | - 1 x 10-3 | - 1 x 10-4 | - - | - Low loss only (short transaction, bulk data, video streaming) |
| - 5 | - U | - U | - U | - U | - - | - Traditional applications of - default IP network |
| - 6 | - 100ms | - 50 ms | - 1 x 10-5 | - 1 x 10-6 | - 1 x 10-6 | - High bit rate, strictly low loss/error (TV broadcast on IP) |
| - 7 | - 400ms | - 50 ms | - 1 x 10-5 | - 1 x 10-6 | - 1 x 10-6 | - High bit rate, strictly low loss/error |

- Editor Note: further clarification is needed on the IPLR values for classes 6 and 7.

## Association of IPTV services with Y.1541 classes

- This section presents classified sets of IPTV services by network QoS requirements and associates them with IP QoS classes defined in [ITU-T Y.1541]. We classify IPTV services into 5 categories. The first set is named Best Effort (BE) service. In this set, services are regarded as traditional default IP network applications. They do not need to be real-time, and errors can be recovered using transmission control protocol on the best effort network. So this equals to the QoS class 5 of Y.1541. The second set is Low Loss (LL) service set. This set is equal to the QoS class 4, because of its features of short transaction and bulk data. The next sets are Interactive (I) service, Real-Time Interactive (RTI) service, and Real-Time Multicast & Unicast (RTMU) service. And they are corresponding to QoS class 2/3, QoS class 0/1 and QoS class 6/7, respectively. And Table 2 also shows the IPTV service examples of each category.

- Table 2 - Association of IPTV services with Y.1541 QoS classes

| - IPTV service category | - IP QoS class | - IPTV service examples |
| --- | --- | --- |
| - Best Effort (BE) service | - QoS class 5 | - content download service |
|  |  | - regulatory information service |
|  |  | - T-information |
|  |  | - e-mail |
| - Low Loss (LL) service | - QoS class 4 | - Download VOD, MOD |
|  |  | - T-commerce |
|  |  | - T-learning based on VOD |
| - Interactive (I) service | - QoS class 2/3 | - messenger |
|  |  | - T-learning based on interactive |
| - Real-Time Interactive (RTI) service | - QoS class 0/1 | - VOIP |
|  | - QoS class 0 | - multi player game, video phone |
| - Real-Time Multicast & Unicast (RTMU) service | - QoS class 6/7 | - linear/broadcast |
|  |  | - multi angle |
|  |  | - pay per view (PPV) |
|  |  | - networked personal video record (PVR, time-shift) |

Editor Notes:

- The above table is just a starting point.

- Each QoS class shows one of the candidate requirements.

- The consistency with Y.1541 needs to be considered.

- In determining the required value for a QoS class, considering the error correction functions, such as FEC, and their performance is necessary.

- It is possible that a standard definition Download VOD application (to hard drive) with play-out <4 Mb/s could be satisfied with the loss ratio of Class 2, or Class 3 with <200 ms delay commitment under the assumption that TCP will be used as Appendix IX/Y.1541.

## Mapping of IPTV services with QoS classes

- This section defines the mapping of Y.1541 QoS classes on IPTV services currently identified..

- Editor Note: The list and definition of IPTV services still needs to be clarified further , according to the WG1 work progress on defining IPTV services. Thus, this table should be updated after the service list and each IPTV service scenarios are clearly defined. The identified services are examples pending WG 1 outcome.

- Table 3 - Mapping of IPTV services with Y.1541 QoS classes

| IPTV services | IPTV service category/QoS class |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BE | LL | I |  | RTI |  | RTMU |  |
|  | 5 | 4 | 3 | 2 | 1 | 0 | 7 | 6 |
| Linear/broadcast TV (audio, video and data) |  |  |  |  |  |  |  |  |
| Multi-angle service |  |  |  |  |  |  |  |  |
| Time-shift TV (network PVR) |  |  |  |  |  |  |  |  |
| Pay Per View (PPV) (Near Vod) |  |  |  |  |  |  |  |  |
| Real VoD |  |  |  |  |  |  |  |  |
| Downloaded based video content distribution (Push VoD) |  |  |  |  |  |  |  |  |
| Content download service |  |  |  |  |  |  |  |  |
| Consumer originated content (video, etc. and applications) |  |  |  |  |  |  |  |  |
| Consumer originated broadcast |  |  |  |  |  |  |  |  |
| Linear broadcast audio |  |  |  |  |  |  |  |  |
| Music on demand including audio book |  |  |  |  |  |  |  |  |
| Pictures |  |  |  |  |  |  |  |  |
| T-learning based on VoD |  |  |  |  |  |  |  |  |
| T-learning based on interactive |  |  |  |  |  |  |  |  |
| Single player games (game on demand) |  |  |  |  |  |  |  |  |
| Multiplayer games |  |  |  |  |  |  |  |  |
| T-information (news, weather, transportation, etc.) |  |  |  |  |  |  |  |  |
| T-commerce (banking, stock, shopping, ticketing, auction, delivery, event etc.) |  |  |  |  |  |  |  |  |
| T-communication data (e-mail, SMS, web surfing, etc.) |  |  |  |  |  |  |  |  |
| T-communication interactive (messenger, channel chatting) |  |  |  |  |  |  |  |  |
| T-communication voice (VoIP, multiple video conference, video phone, etc.) |  |  |  |  |  |  |  |  |
| T-entertainment data (photo album, lottery, blog, etc.) |  |  |  |  |  |  |  |  |
| T-entertainment VOD (karaoke) |  |  |  |  |  |  |  |  |
| Presence service |  |  |  |  |  |  |  |  |
| Communication messaging |  |  |  |  |  |  |  |  |

Editor Notes:

- The above table is just a starting point.

- Each QoS class shows one of the candidate requirements.

- The consistency with Y.1541 needs to be considered.

- In determining the required value for a QoS class, considering the error correction functions, such as FEC, and their performance is necessary.

# Video coding and transmission over IP networks

Editor’s Note:  Appropriate text will be included based upon output from WG4 (relating to Streaming Protocols) and WG6 (relating to Codec definition).

# Cross-layer interaction for IPTV services

Figure 9-1 shows generic layer architecture for offering of the video services. At the application layer video codec generates video elementary streams as a result of the encoding process. Service layer is responsible for packaging video streams into services such as broadcast video, video on demand (VoD), etc. The function of the transport layer is to transport video streams from source to the destination through different network segments.

Application Layer

Service Layer

Transport Layer

H.264

MPEG-2

VoD

Broadcast Video

IP

RTP

Ethernet

802.11

Figure 9-1:	Layer Architecture for Video Transport

Cross layer interaction is the ability of one layer adapting its performance based on information received from other layers. For instance in a wireless transport environment it may be beneficial for the application to adjust its rate based on the status of the rates that can be supported by the transport layer. Cross layer interaction may be implemented in a top-down or down-top fashion.

Of particular interest is the ability to make use of application layer priority to optimize transport layer performance. Video codec generates video elementary streams with different priorities based on the picture type (i.e. I, P or B). I type pictures are essential for the codec operation and should always be given transport priority over the P and B frames. MPEG-2 transport stream and H.264 network adaptation layer (NAL) generates transport video stream packets with relative priority indication (one bit transport priority in the MPEG-2 transport stream packet and 2 bits NAL reference id (NRI) in H.264)

# Traffic management capabilities: core networks

## IP network traffic management capabilities

This sub-clause describes an overview of the IP traffic management capabilities including those related to integrated services and differentiated service.

IP networks allow both reservation as well as differentiated models for QoS support and traffic management. The IP reservation model is that based on the IP Integrated Services (IntServ) and allows the use of RSVP signalling protocol for bandwidth reservation. Two service classes are defined in the context of IP IntServ:

- Guaranteed Service (GS) defines a service type where all packets belonging to the same session are delivered within a deterministic delay bound. Routers along the path of the session must employ appropriate scheduling algorithm and reserve resources to ensure that the delay bound is always satisfied.

- The Controlled Load Service (CLS) provides a service equivalent to a “lightly” loaded best effort network.

IP networks are also capable of supporting a differentiated model with the IP differentiated service (DiffServ). The IP differentiated model defines a set of edge functions and a number of per-hop behaviour (PHB). Edge functions are those related to classification, metering, marking, dropping, and shaping. A PHB defines an externally observable treatment applied at a network node to a DiffServ behaviour aggregate. A number of PHBs are defined by the IETF in the context of IP DiffServ:

- Expedited Forwarding (EF): The intent of the EF behaviour is to provide the building block for the creation of low-loss, low delay, and low delay variation services. The EF PHB definition mandates that the EF packets should ideally be served at a rate R or faster and bounds the deviation of the actual departure time for each packet by the relationship [IETF RFC 3246]:

dj≤fj+Eafj=maxaj,mindj−1,fj−1+ℓjR

where dj is the actual departure time of the jth packet, fj is the target departure time of the jth packet, and Ea is an error term bounding the deviation of the actual departure time from the target departure time. Eais a function of the nodal scheduler implementation. ℓj is the length of the jth packet. The above relationship paves the way for the specification of a delay bound as long as the aggregate EF traffic adheres to some traffic pattern (e.g. as specified by a leaky bucket).

- Assured Forwarding (AF): The AF PHB group defines four AF classes. For each class three discard precedence levels are defined (AFij, i=1, 2, 3 or 4 and j=1, 2 or 3). No priority order is defined among the four AF classes. [IETF RFC 2597] requires that each node in the networks to allocate certain amount of forwarding resources for each AF class to assure forwarding of the IP packets. However, no delay or loss bounds are defined.

- Class Selector (CS): The CS PHB group is defined for DiffServ to be backward compatible with legacy routers that support the type of service (ToS) bits in the IP packet header. CS PHB group defines 8 forward treatments where packets belonging to CSi are served before packets marked with CSj as long as i>j. It effectively defines a straight priority discipline with CS1 being reserved for the default forwarding (DF) used for the traditional IP best effort (BE) service.

IP DiffServ is scalable compared to IP IntServ since it doesn’t require per flow reservation, scheduling, and buffering. In IP DiffServ flow information is available at the network edge while nodal behaviour is applied on an aggregate basis.

IP DiffServ paradigm doesn’t include the definition of service classes. However service classes could be realized by proper specification of edge rules and consistent application of PHB by the different nodes in the network.

## IP traffic management capabilities applicable to IPTV

The core IP network transporting IPTV services is expected to be well-engineered and shared between a number of applications including data and voice support. Since IP Diffser allows the core IP network to scale to a large number of flows it is also expected that the network will be equipped with DiffServ capabilities in terms of scheduling implementation that supports all or some of the DiffServ PHBs. It can be shown that if the EF traffic arriving at an interface is bounded by a leaky bucket with parameters (B, R) then the delay of any packet departing the interface is bounded by:

D≤BR+Ea

IPTV services that require stringent loss and delay constraints may be supported using EF PHB. The use of PHB will ensure that IPTV packets are transported within a certain delay bound with almost no loss as long as the aggregate burstiness of the offered EF traffic is within some bound. The EF PHB must be configured with enough resources to satisfy the delay requirements of video packets. Since performance of video application is sensitive to packet loss, sufficient buffer needs to be allocated to ensure that it accommodates a burst size equal to B.

It is also possible to support IPTV service using AF PHB group as long as the AF class or classes designated for IPTV service are configured with the amount of resources needed to assure the service QoS parameters. Unlike the EF PHB there is no delay constraint associated with the definition of the AF PHB. However the use of the AF offers the added benefit of selectively discard video packets based on their relative importance. This will require the relative importance of video information to be inherited by the IP layer.

An IP node supporting the CS PHB group implies an absolute priority scheduler is in place. IPTV shall be supported at a priority level high enough to maintain its quality assurances. Starvation of classes supporting IPTV services must always be avoided

# Traffic management capabilities: access networks

## Overview of access network technologies

Access networks should have means to control the following functions for traffic management:

- per packet/flow and per class basis CAC (Connection Admission Control)

- per user and per service QoS provisioning

- per packet/flow Mapping between DSCP and 802.1p

## Access traffic management capabilities applicable to IPTV

Generic mechanisms are required for traffic management of upstream and downstream directions in an access network.

- Upstream

To support quality of service in access networks including broadcasting traffic, network nodes should have the following requirements in the upstream direction.

- Guarantee of minimum bandwidth

- Guarantee of bandwidth quality per service (e.g. VoIP, Video conference, and IPTV).

To address the above 2 requirements, the following functions should be supported:

- 5-tuple based Classification & Policing

- Priority Queuing

- Traffic shaping

- Downstream

To support quality of service in access networks including broadcasting traffic, network nodes should have the following requirements in the downstream direction.

- Guarantee of minimum bandwidth per service

To support the above requirement access networks should have the following functions in downstream side.

- per packet/flow and per class basis CAC (Connection Admission Control) for broadcast channels

- Traffic shaping/policing per subscriber

- Support of CoS (Class of Service) based DSCP

      - 5-tuple classification

      - DSCP marking

Mapping between DSCP and 802.1p

# Traffic management capabilities: home networks

This clause describes home network environment and the need for traffic management for IPTV applications. A possible architecture for the home network is shown in Figure 12-1 [HGI].

Figure 12-1: Home Network Architecture

As shown in Figure 12-1 home network employs multiple LAN technologies. Prominent among those technologies are the wireless LAN (WLAN) based on IEEE 802.11 and wireline Ethernet and bridges based on IEEE 802.3 and IEEE 802.1. The home gateway (HG) is the interface of the home network to provider access network. HG receives and sends packets from and to the WAN. It also supports transit (LAN-to-LAN) traffic. The main function of the HG is the classification of the packets as they traverse the HG to and from the WAN and setting the appropriate priority level using the IP DSCP or the Ethernet user priority bits. Possible classification criteria include type of LAN, MAC addressing, and VLAN Tag. HG requirements include the support of multiple queues managed using strict priority or weighted round robin at the WAN egress ports. It requires the support buffer management mechanisms.

Home Ethernet switches may support the existence of the user priority bits (a.k.a. p-bits). User priority bits define up to 8 priority levels that can be used to identify traffic classes such as voice, video, best effort, etc.

The current IEEE 802.11 standards support rates up to 54 Mbps. Emerging IEEE 802.11 standards are expected to support rates up to 600 Mbps based on IEEE 802.11n. Traffic management is supported on a WLAN using IEEE 802.11e which defines 4 access categories that effectively supports 4 priority levels.

____________

The bulk of Figure 1 is reproduced from FG IPTV-DOC-0062 on IPTV Architecture