| The following information has been agreed by WG2 to be under further consideration by WG2 prior to inclusion in formal output documents. INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-DOC-0138 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 5th FG IPTV meeting: Geneva, 23-31 July 2007 |  |  |
| OUTPUT DOCUMENT |  |  |  |  |
| Source: | Editor |  |  |  |
| Title: | Living List: Application layer error recovery mechanisms for IPTV |  |  |  |

# Item 1 (added at 2nd Meeting of ITU-T FG IPTV, Busan): ARQ

Source: FG IPTV-C-0220, ETRI, “Existing standards/activities for ARQ-based application layer reliability solutions”

Status: closed.

# Item 2 (added at 3rd Meeting of ITU-T FG IPTV, Mountain View):  ATIS IIF IPTV Packet Loss Issue Report

Source: FG IPTV-IL-0040, ATIS IIF,  “ATIS IIF Liaison to ITU-T Focus Group on IPTV”

Enclosed in this incoming liaison from ATIS IIF is the ATIS IIF IPTV Packet Loss Issue Report (ATIS-0800005). The IPTV Packet Loss Issue report is a technical report that explores the range of potential solutions and makes recommendations regarding their applicability for an IPTV service.

ATIS IIF offers to the ITU-T Focus Group on IPTV that if wishes to use text from these documents for the purposes of developing IPTV standards and requirements to contact the liaison contact person.

### Status: It was decided that the ATIS IIF IPTV Packet Loss Issue Report (ATIS-0800005) is kept in this document for easy access of information relevant to the work on Application layer reliability solutions for IPTV in the ITU-T Focus Group on IPTV.

# Item 3 (added at 3rf Meeting of ITU-T FG IPTV, Mountain View): Background Information on DVB Application-Layer FEC

During the 3rd meeting of the ITU-T FG IPTV, a high-level information on the DVB AL-FEC solution was received. Additional details were received during the 4th meeting of the ITU-T FG IPTV.

Sources:

- FG IPTV-IL-0064, Digital Video Broadcasting Project; Technical Module subgroup on IP Infrastructure (DVB TM-IPI),  “Response to FG IPTV-OL-0020 on Application Layer FEC in DVB-IPI”

In incoming Liaison Statement FG IPTV-IL-0064 the Digital Video Broadcasting Project (DVB) Technical Module group on IP Infrastructure (TM-IPI) stated the appreciation of the efforts of the ITU-T FG IPTV WG2 to study the adoption of DVB-IP AL-FEC solution for ITU-T FG IPTV AL-FEC.

In this document, the DVB project provided ITU-T FG IPTV WG2 with the latest information on the AL-FEC work for your upcoming meeting. Specifically, the following recently published documents have been provided:

| - A086rev4 - Transport of MPEG 2 Transport Stream (TS) Based DVB Services over IP Based Networks (DVB-IPTV Phase 1.3 - Draft TS 102 034 1.3.1) as a bluebook which contains in Annex E the specification of the DVB-IPI AL-FEC. |  |
| --- | --- |
| - A115 - Blue Book on AL-FEC Evaluations also as a bluebook which is intended to provide background on the evaluation process behind the choice of the hybrid approach in the DVB application layer forward error correction (AL-FEC) specification as included in the DVB-IP Phase 1 Handbook. |  |

A115 document consists of two documents that were created by DVB as part of the AL-FEC evaluation process. Document A presents the evaluation criterion that were agreed before the selection process started and Document B is the report on the evaluation process. Document B also gives the rationale for the choice of the hybrid approach used in the DVB AL-FEC specification.

Status: It was decided to keep this background information in the living list as the documents may contain relevant information on the ITU-T AL-FEC solution.

# Item 4 (added 4th Meeting of ITU-T FG IPTV, Bled): DVB AL-FEC as ITU-T AL-FEC

During the 4th Meeting of ITU-T FG IPTV a provisional endorsement was added to the living list. The remaining issues were resolved during the 5th Meeting of ITU-T FG IPTV.

The only remaining aspect not integrated in the WD on application layer reliability solutions are the details on what parts of the DVB-IPI AL-FEC solution are required, and what is recommended, or what can optionally be used. Clarification information was provided in Input Contribution FG IPTV-C-0776, Digital Fountain, “Usage of DVB-IP Application Layer FEC Specification”.

Specifically section 3 of this document provides clarification of this subject:

The ITU-T FG IPTV does not yet define a full set of content delivery protocols, i.e. the control and transport protocols yet. However, it quite likely that the media will be transported either through MPEG-2 TS over RTP/UDP or by generic audio/video streams. Therefore, DVB AL-FEC solution likely covers most of the expected transport protocols.

With respect to control protocols, the details can not be specified at this point, but it is essential to understand what control information needs to be transported for the AL-FEC, once the content delivery protocol more clearly defined.

For the transport protocols it is assumed that the same protocols for the ITU-T FG IPTV AL-FEC as for the DVB-IP AL-FEC solution as specified in section E.6 are used.

For the control protocols, it is assumed that the same semantics for the ITU-T FG IPTV AL-FEC as for the DVB-IP AL-FEC solution as specified in section E.6 are used. The details on the syntax need to be defined.

For the decoder, it is proposed that if application layer FEC in ITU-T FG IPTV is supported, then it is

- required to support the minimum decoder performance as specified in section E.5.1.1 in case MPEG-2 Transport Stream over RTP is used,

- recommend to support the enhanced decoder performance as specified in section E.5.1.2 in case MPEG-2 Transport Stream over RTP is used,

- required to support a decoder performance for which the minimum performance is determined by only using step 2 in section E.5.2.1, i.e. only Raptor decoding performed, or equivalently, the performance of the reference decoding algorithm as provided in [1], Annex C, is fulfilled, in case arbitrary packet flows are used according to the definition in section E.6.3 and E.6.4.

[1] 	3GPP TS 26.346 – 3rd Generation Partnership Project; Technical Specification GroupServices and System Aspects; Multimedia Broadcast/Multicast Services (MBMS); Protocols and codec’s (Release 6).

Status: It was decided to keep this information in the LL as the information may be relevant to integrate the recommended AL-FEC solution into the IPTV architecture and to provide background information on the recommended AL-FEC solution.

# Item 5 (added 5th Meeting of ITU-T FG IPTV, Geneva): Other AL-FEC Methods

Source: FG IPTV-C-0684, NEC Corporation, “Proposal for Modifications to FG IPTV-DOC-0088”

Status: open.

____________