| Performance Monitoring for IPTV INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-DOC-0154 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 6th FG IPTV meeting: Tokyo, Japan, 15–19 October 2007 |  |  |
| OUTPUT DOCUMENT |  |  |  |  |
| Source: | Editor |  |  |  |
| Title: | Working Document: Performance monitoring for IPTV |  |  |  |

(October 2007)

Summary

TBD

Keywords

TBD

Introduction

TBD

Contents

1	Scope	5

2	References	5

3	Definitions	6

4	Abbreviations and acronyms	6

5	Conventions	7

6	Monitor points	8

6.1	Monitoring Point Definitions	9

7	Monitoring parameters	10

7.1	Physical Layer Parameters	12

7.2	IP Layer/Network Parameters	12

7.3	Transport Stream parameters	15

7.4	Service Line-Up parameters	16

7.5	Channel Attribute parameters	18

7.6	VOD Parameters	19

7.7	Other Parameters	19

8	Monitoring methods	19

8.1	Generalized monitoring method for multi-media data based on transmission packet loss	20

8.2	RF Integrity Monitoring	21

8.3	Network performance monitoring	22

8.4	IPTV service attribute monitoring	22

8.5	Video quality monitoring	23

8.6	Audio quality monitoring	31

8.7	Ancillary attribute monitoring	33

9	Bibliography	33

Annex A	34

Considerations for Network Performance Monitoring when using Overlay Networks	34

1	Overlay Measurement	34

2	Measurement Unit	34

List of Figures

Figure ‎6-1 - IPTV Domains	8

Figure ‎6-2 - Monitoring Points	9

Figure ‎7-1 - Television Service Hierarchy	10

Figure ‎7-2 - Television Service Quality	16

Figure ‎8-1 - Performance Monitoring Point Deployment	20

Figure ‎8-2 - Typical Index Structure	21

Figure ‎8-3 - Direct Observation Monitoring Model for Domain E	22

Figure ‎8-4 - A Full-Reference Model	25

Figure ‎8-5 - A Reduced-Reference Model	25

Figure ‎8-6 - Back Channel for Quality Reporting	26

Figure ‎8-7 - A streaming server which optimizes codec parameters based on the perceptual quality	26

Figure ‎8-8 - A network provider which uses perceptual quality information	27

Figure ‎8-9 - A block diagram of video quality monitoring using an RR model	27

Figure ‎8-10 - A method for a transmitter to monitor video quality at a receiver using transmission	28

Figure ‎8-11 - Relationship between packet-layer model and bit stream layer model Hybrid perceptual/bit-stream models	29

Figure ‎8-12 - Input requirements for FR hybrid perceptual/bit-stream models	29

Figure ‎8-13 - Input requirements for RR hybrid perceptual/bit-stream models	29

Figure ‎8-14 - Input requirements for NR hybrid perceptual/bit-stream models	30

Figure ‎8-15 - Generating and transmitting video quality scores of compressed data	30

Figure ‎8-16 - Input requirements for a FR hybrid perceptual/bit-stream model which uses the video quality scores of transmitted compressed video data	31

Figure ‎8-17 - Input requirements for a RR hybrid perceptual/bit-stream model which uses the video quality scores of transmitted compressed video data	31

Figure ‎8-18 - Input requirements for a NR hybrid perceptual/bit-stream model which uses the video quality scores of transmitted compressed video data	31

List of Tables

Table ‎7-1 - Monitoring points and parameters	10

Table ‎7-2 - RF standards and their essential measurements	12

Table ‎8-1 - Video Quality Measurements in Various Domains	25

Table ‎8-2 - Message Descriptions	28

Table ‎8-3 - Audio Quality and Impairment Scales	31

Table ‎8-4 - Audio Comparison Scale	32

Performance Monitoring for IPTV

(October 2007)

# Scope

This document defines performance monitoring for IPTV. Monitoring parameters, monitoring points and monitoring methods are defined that allow the service provider/network operator to monitor the performance of the service delivery to the end user.

Successful deployment of IPTV services requires performance to be monitored at the customer premise (e.g. set-top-box), key aggregation points such as DSLAM (DSL Access Multiplexer) or CMTS (cable modem termination system) and at interconnect points between disparate network domains. Performance monitoring can help:

- Find errors in an end-to-end system (system debugging)

- Check the resource utilization and the work load of system components

- Compare values (metrics) regarding performance of different system deployments

- Provide a base for modelling the system

- Find system bottlenecks

- Optimize IPTV network deployment

- Ensure that system performance does not degrade with time.

The goal of performance monitoring for IPTV is to provide higher QoS/QoE to customers by identifying, localizing and quantifying service and network issues.

IPTV performance monitoring can be software based, hardware based, or hybrid of software/hardware based.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this working document. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this working document are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this working document does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T Y.1540]	ITU-T Recommendation Y.1540 (2002), Internet protocol data communication service – IP packet transfer and availability performance parameters

[ITU-R BT.500]	ITU-R Recommendation ITU-R BT.500-11(2002), Methodology for the subjective assessment of the quality of television pictures

[ITU-T E.800] 	ITU-T Recommendation E.800 (1994), Terms and Definitions Related to Quality of Service and Network Performance Including Dependability

[ITU-T O.211]	ITU-T Recommendation O.211 (2006), Test and Measurement Equipment to Perform Tests at the IP Layer

[ITU-T P.800]	ITU-T Recommendation P.800 (1996), Methods for subjective determination of transmission quality

[ITU-T P.862]	ITU-T Recommendation P.862 (2001), Perceptual evaluation of speech quality (PESQ): An objective method for end-to-end speech quality assessment of narrow-band telephone networks and speech codecs

[ITU-T Y.1541]	ITU-T Recommendation Y.1541 (2006), Network Performance Objectives for IP-based Services

[ITU-T Y.1543]	ITU-T Recommendation Y.1543 (2007), Measurements in IP networks for inter-domain performance assessment

[ITU-R BS.1387]	ITU-R Recommendation BS.1387 (2001), Method for objective measurements of perceived audio quality

[CEA CEA-608D]	Consumer Electronics Association Standard CEA-608D (May 2007), Line 21 Data Services

[CEA CEA-708C]	Consumer Electronics Association Standard CEA-708C (July 2006), Digital Television (DTV) Closed Captioning

[DSL TR-126]	DSL Forum Technical Report TR-126 (2006), Triple-Play Services Quality of Experience (QoE) Requirements

[ETSI TR 101-290]	ETSI Technical Report 101-290 (2000), Digital Video Broadcasting (DVB); Measurement guidelines for DVB systems

[ETSI EN 300 743]	ETSI EN 300 743 V1.2.1 (2002-10) Digital Video Broadcasting (DVB); Subtitling systems

[IETF RFC2550]	IETF RFC 2550 (1998), RTP Payload Format for MPEG 1 and MPEG 2 Video

[IETF RFC3393]	IETF RFC 3393 (2002), IP Packet Delay Variation Metric for IP Performance Metrics (IPPM)

[IETF RFC3611]	IETF RFC3611 RTP Control Protocol Extended Reports (RTCP XR)

[IETF RFC3984] 	IETF RFC 3984 (2005), RTP Payload Format for H.264 Video

# Definitions

This working document uses or defines the following terms:

TBDError: Reference source not found

# Abbreviations and acronyms

This working document uses the following abbreviations and acronyms:

AAA	Authentication, Authorization, and Accounting

CAT	Conditional Access Table

DSCS	Double Stimulus Comparison Scale

DSCQS	Double Stimulus Continuous Quality Scale

EPG	Electronic Program Guide

FR	Full Reference

HTTP	Hyper Text Transfer Protocol

IGMP	Internet Group Management Protocol

IPDV	IP Packet Delay Variation

IPER	IP Error Rate

IPLR	IP Packet Loss Rate

IPRR	IP Reordering Ratio

IPTD	IP Packet Transfer Delay

KOD	Karaoke On Demand

MAE	Mean Absolute Error

MPTS	Multi-Program Transport Stream

MSE	Mean Square Error

NMS	Network Management System

MOS	Mean Opinion Score

NR	No Reference

OSS	Operations Support System

PID	Program Identifier

PSNR	Peak Signal to Noise Ratio

PVS	Processed Video Sequence

QoE	Quality of Experience

RED	Random Early Discard

RF	Radio Frequency

RR	Reduced Reference

SI	System Information

SPTS	Single Program Transport Stream

SRC	Source Video Sequence

SSCQE	Single Stimulus Continuous Quality Evaluation

STB	Set-Top Box

TS	Transport Stream

VoIP	Voice over IP

XML	Extensible Markup Language

# Conventions

TBD

# Monitor points

The entire content delivery chain can be divided into multiple domains. Operators at domain borders have the option to perform monitoring which, when taken together, forms an end-to-end monitoring topology.

This domain approach is independent of any specific monitoring method (e.g. RR).

Monitored performance characteristics, across a single domain or multiple domains, can be integrated with existing or new operations support systems (OSS) and/or network management system (NMS) systems.

Figure ‎6-1 - IPTV Domains

The exact topology and domains will vary from one operator to another; however, monitoring can be applied at each domain boundary. An example topology with generalized domain boundaries is shown in Figure ‎6-1. These domains are further divided into specific monitoring domains in Figure ‎6-2. Within each domain, different aspects can be monitored at each domain boundary as outlined below.

Error: Reference source not found

Figure ‎6-2 - Monitoring Points

A whole performance monitoring system is recommended to include a performance monitoring management platform.

The management platform entity manages monitoring tasks for all points and collects parameters from all points, performs performance analysis, and generates reports. The range of the management platform covers all five domains as shown in Figure ‎6-2.

### Monitoring Point Definitions

#### Point 1 – PT1

This point demarcates the domain border between content provision and IPTV control. It should aim for source video quality monitoring, source audio quality monitoring, and metadata verification.

#### Point 2 – PT2

This point demarcates the domain border between Service Provider and Network Provider. It should aim for original streaming quality monitoring, such as audio-visual quality monitoring, IPTV Service Attribute Monitoring, and metadata verification.

#### Point 3 – PT3

This point demarcates the IP Core and IP Edge networks where monitoring of IP-related performance parameters, such as Network Monitoring, Network Performance Monitoring are important.

#### Point 4 – PT4

This point is closest to the user where monitoring the quality of streaming, audio-visual quality, and IPTV service attribute monitoring are important.

#### Point 5 – PT5

This point is at the final end point and directly relates to end user QoE. Monitoring audiovisual quality and IPTV service attribute monitoring are important.

# Monitoring parameters

This clause describes the parameters which need to be monitored at different stages of an end-to-end IPTV delivery chain. With reference to Figure ‎6-2 which identifies monitoring points and domains, specific parameters are applicable to only few domains, but all parameters listed are equally significant for monitoring the true performance of IPTV delivery.

There is a hierarchical relationship within an IP television service, where a physical interface (e.g. Ethernet) can carry one or many IP flows, those IP flows in turn carry single or multiple program transport streams, each channel/program within, having multiple attributes defining the content.

Figure ‎7-3 - Television Service Hierarchy

Table ‎7-1 identifies the monitoring points which are shown in Figure ‎6-2. The word “yes” in each cell of Table ‎7-1 indicates applicability of a monitoring parameter. These monitoring parameters are defined in this clause.

Table ‎7-1 - Monitoring points and parameters

| Monitoring Parameters | (PT1) | PT2 | PT3 | PT4 | \ PT5 |
| --- | --- | --- | --- | --- | --- |
| Physical Layer Parameters |  |  |  |  |  |
| RF Integrity | Yes |  |  |  |  |
| Available Bandwidth | Yes | Yes | Yes | Yes | Yes |
| IP Layer Parameters |  |  |  |  |  |
| Packet Loss Metrics | Yes (Only for content over IP contribution) |  | Yes | Yes | Yes |
| Jitter | Yes (Only for content over IP contribution) |  | Yes | Yes | Yes |
| IGMP join/ leave time |  |  |  | Yes | Yes |
| IP Flow List | Yes | Yes | Yes | Yes | Yes |
| Transport Layer Parameters |  |  |  |  |  |
| Packet Loss Metrics | Yes |  | Yes | Yes | Yes |
| Jitter | Yes |  | Yes | Yes | Yes |
| TR 101 290 | Yes Priority 1, 2 & 3 | Yes Priority 1 and 2 | Yes Priority 1 | Yes Priority 1 | Yes Priority 1 |
| Service Line-up Parameters |  |  |  |  |  |
| Meta Data Verification (parental control/ EPG/ Subtitles) | Yes | Yes |  |  | Yes |
| Metadata Validity | Yes | Yes |  |  | Yes |
| Metadata Integrity | Yes | Yes |  |  | Yes |
| Channel Zap time |  |  |  |  | Yes |
| Channel Line Up Verification |  | Yes |  |  | Yes |
| Buffering Delay |  |  |  |  | Yes |
| Decoding Delay |  |  |  |  | Yes |
| Channel Attribute Parameters |  |  |  |  |  |
| Video/Audio Bandwidth | Yes | Yes | Yes | Yes | Yes |
| Video Quality | Yes | Yes |  |  | Yes |
| Audio Quality | Yes | Yes |  |  | Yes |
| Blackout Detection | Yes | Yes |  |  | Yes |
| Freeze Frame Detection | Yes | Yes |  |  | Yes |
| Audio Loss/ Presence | Yes | Yes |  |  | Yes |
| VOD Parameters |  |  |  |  |  |
| VOD Request Performance |  |  |  |  | Yes |
| VOD Request Accuracy (Correctness Rate) |  |  |  |  | Yes |
| Buffering Delay |  |  |  |  | Yes |
| Decoding Delay |  |  |  |  | Yes |
| Connection Success Rate |  | Yes | No | Yes |  |
| Connection Time |  | Yes | No | Yes | Yes |
| Other Parameters |  |  |  |  |  |
| AAA Success Rate |  | Yes |  |  | Yes |

### Physical Layer Parameters

RF Integrity presents a list of some commonly used RF standards for the delivery of broadcast content over terrestrial, satellite and cable mediums. While this list is not exhaustive and other standards or vendor proprietary solutions may exist, the measurements below can be used to characterize the state of the RF signal. An unacceptable result in any of the measurements can result in the delivery of poor service down the transmission chain and throughout the rest of the Domains.

Table ‎7-2 - RF standards and their essential measurements

| Monitoring Parameters | Terrestrial |  | Satellite |  | Cable |
| --- | --- | --- | --- | --- | --- |
| RF Signal Level | DVB-T ETSI EN 300 744 | 8VSB ATSC A/53B | DVB-S ETSI EN 300 421 | DVB-S2 ETSI EN 302 307 | DVB-C ETSI EN 300 429 |
| SNR Signal to Noise Ratio |  |  |  |  |  |
| MER Modulation Error Ratio |  |  |  |  |  |
| BER Bit Error Rate |  |  |  |  |  |

### IP Layer/Network Parameters

[ITU-T Y.1540] defines parameters for network performance and RFC 3357 [b_RFC 3357] defines loss distance, loss period, loss noticeable rate, loss period length, and inter loss period length.

In addition, impairments may occur in a number of burst patterns. Sparse bursts are periods of high packet loss, analogous to severely errored seconds, and is a period that begins and ends with a lost (or discarded) packet during which some constraint is satisfied. Sparse burst are often due to network congestion, Random Early Discard (RED) and related effects.

Continuous bursts are periods during which every packet is lost.  Contiguous losses may occur due to packetization (i.e. packing several transport packets within an IP packet), to link failures within an IP network or other phenomena.

Finally, isolated lost packets are also possible and typically occur due to bit errors in transmission or excessive collisions on local area networks.

#### Link IP Layer Used Bandwidth.

The link IP layer used bandwidth is defined as the sum of the IP layer bandwidth for all IP packet flows within in a link.

#### Link IP Layer Available Bandwidth.

The link IP layer available bandwidth is defined as the maximum IP layer bandwidth which the link can provide without influencing other existing flows (background flows) in the link.

NOTE 1 – For a given link, the "Link IP Layer Available Bandwidth" plus the "Link IP Layer Used Bandwidth" is equal to the "Link IP Layer Bandwidth".

NOTE 2 – With the knowledge of the values of the above two parameters the network providers can determine the bandwidth utilization ratio of a link.

#### End-to-End IP Layer Bandwidth.

The end-to-end IP layer bandwidth is defined as the maximum IP layer bandwidth an end-to-end path can provide given no background flows exist along that end-to-end path. It can be also understood as equal to the lowest Link IP Layer Bandwidth along that end-to-end path, and hence the bottleneck along that path.

#### End-to-End IP Layer Available Bandwidth.

End-to-end IP layer available bandwidth is defined as the maximum IP layer bandwidth which an end-to-end path can provide without influencing other existing flows (background flows) along that path.

#### Loss run length distribution

Loss run length distribution describes the distribution of run lengths of consecutive lost IP packets.

Let li, i = 1, 2, … , n-1 denote the number of loss bursts of length i, where n-1 is the longest loss burst. L denotes the number of total lost packets (L > 0). The loss run length distribution can be calculated as li/L, i = 1, 2, … , n-1.

#### Error-free interval distribution

Error-free interval distribution describes the distribution of run lengths of consecutive correctly received IP packets.

Let fi, i = 1, 2, … , n-1 denote the number of error-free intervals having length i, where n-1 is the longest error-free interval. F denotes the number of total received packets (F > 0). The error-free interval distribution can be calculated as fi/F, i = 1, 2, … , n-1.

#### Packet Loss Metrics & Models

- Sparse bursts

Sparse bursts  and extended report (XR) packet type for the RTP control protocol (RTCP) [IETF RFC3611] are periods of high packet loss, analogous to severely errored seconds.  These may be modelled using multi-state Markov Models [Reference?] or Gilbert-Elliott models (see [b_BSTJ Elliot] and [b_BSTJ Gilbert]).  A sparse burst is a period that begins and ends with a lost (or discarded) packet during which some constraint is satisfied.  In RFC3611 [IETF RFC3611] the defined constraint is that within a burst there must be less than Gmin consecutively received packets.  Gmin is selected such that the minimum effective loss rate within a burst corresponds to the lowest packet loss rate at which some noticeable distortion occurs within the decoded media stream.  Sparse burst are often due to network congestion, Random Early Discard (RED) and related effects.

- Continuous bursts

Continuous bursts are periods during which every packet is lost.  Contiguous losses may occur due to packetization (i.e. packing several transport packets within an IP packet), to link failures within an IP network or other phenomena.

- Isolated losses

Isolated lost packets typically occur due to bit errors in transmission or excessive collisions on local area networks.

#### Streaming Jitter (SJ)

Streaming Jitter (SJ) represents the maximum and minimal bit rate of the streaming server output. SJ is from the network’s viewpoint and it is a vital metric when monitoring the performance.

#### Packet Loss Rate

Proportion of IP or RTP packets lost in the network or discarded by Layer 1 or 2, or lost after correction by FEC or retransmission, expressed as a binary fraction of the Total Packets Lost divided by Total Packets Expected for a given Measurement Interval.

#### Out of Order Packet Rate

Proportion of (IP or RTP) packets arriving out of order, expressed as a binary fraction of the Total Packets Out of Order divided by the Total Packets Expected for a given Measurement Interval.

Indicates proportion of packets that arrive out of order. Indicates network condition and some video decoders discard out of order packets.

#### Burst Loss Rate

Proportion of IP or RTP packets lost within a (sparse) burst expressed as a binary fraction represented by the Burst Loss divided by Burst Length.

Indicates the severity of transient problems affecting video quality. Useful for configuration of FEC and retransmission.

#### Gap Loss Rate

Proportion of IP or RTP packets lost within gap periods represented by the Gap Loss divided by Total Packets Expected in the gap period.

Loss distribution before error correction. Indicates loss conditions during good quality periods.

#### Mean Gap Length

The mean length of gaps between bursts of IP or RTP packets.

Indicates loss conditions during good quality periods.

#### Mean RTP Burst Length

Mean length of (sparse) burst periods or IP or RTP packets.

Indicates the severity of transient problems affecting video quality.

#### Loss Period Count

The count of the number of times the loss period exceeded the Loss Period Threshold.

Indicates the frequency of very severe loss conditions where the video screen will go blank. Also the period or hole is significant where the in regards to recovery mechanism.

#### IP Maximum Loss Period

Maximum number of IP packets in an excess loss event (max hole size). This metric is used for the watching of a program. It will report the maximum time a screen will freeze or go blank.

Useful for setting error correction or retransmission parameters

#### Retransmissions

Number of retransmitted RTP/UDP packets in a Measurement Interval.

This metric will help viewing reliability where retransmissions indicate a lost packet(s). Indicates error conditions and bandwidth usage.

### Transport Stream parameters

The MPEG-2 Transport Stream protocol facilitates the exchange of audio/video services with associated System Information tables between compatible equipment. This mechanism is commonly used in broadcast environments.

IPTV operators might plan to deploy networks that do not use the Transport Stream protocol. This approach compromises the interoperability between networks and equipment. Such deployments have to rely on proprietary equipment. This also ties the operator with a particular equipment vendor or product family.

The following paragraphs assume that the Transport Stream packet headers are not encrypted. Some encryption schemes might involve the scrambling of all data after the IP header. In such cases, no monitoring of Transport Stream parameters is possible. Only IP related statistics would be relevant.

ETSI TR 101 290 [ETSI TR 101-290] defines the essential TS parameters to be monitored. These are categorized according to severity and are listed as Priorities 1, 2 and 3.

- Priority 1 is defined as those “necessary for de-codability (basic monitoring)”,

- Priority 2 is “recommended for continuous or periodic monitoring”, and

- Priority 3 is defined as “application dependant monitoring”

Scenario 1: DVB Compliant MPEG-2 Transport streams delivery of IPTV

Monitoring all the parameters listed in Priority 1,2 and 3 of the [ETSI TR 101 290] specification are ESSENTIAL in this scenario.

Scenario 2: IPTV MPEG-2 Transport Streams without DVB System Information Tables

Operators adopt multiple approaches to deliver IPTV. The use of DVB System Information (SI) tables is optional, as operators might utilize other mechanisms to describe events and associated data. In the absence of DVB SI, certain Transport Stream tests described in the TR 101 290 specification are still applicable.

All the tests in Priority 1 are relevant. These tests relate to basic MPEG-2 Transport Stream parameters, regardless of whether DVB SI is used.

Monitoring the Conditional Access Table (CAT) in Priority 2 is not required. The encryption algorithms used in IPTV systems might not conform to conditional access signalling protocol. All other Priority 2 parameters should be monitored.

Tests in Priority 3 are specific to DVB SI and are thus not required. IPTV operators typically utilize proprietary means to specify EPG and other data services.

Transport stream parameters– End to End IPTV monitoring

With reference to the monitoring points identified in Figure 1, the applicable transport stream parameters across different domains are marked as “Yes” in Table 2, below.

Table 2 - Transport Stream Monitoring Parameters

| TR 101 290 | Domain A | Domain B (PT1 to PT2) | Domain C (PT2 to PT3) | Domain D (PT3-PT4) | Domain E (PT4- PT5) |
| --- | --- | --- | --- | --- | --- |
| Priority 1 | Yes | Yes | Yes | Yes | Yes |
| Priority 2 | Yes (excluding CAT_error test) | Yes (excluding CAT_error test) |  |  |  |
| Priority 3 | Yes |  |  |  |  |

### Service Line-Up parameters

Quality Television Service

Video

Audio

Meta Data

Parental control

EPG

Subtitles

Figure ‎7-4 - Television Service Quality

#### Channel line up

It is essential to monitor the parameters applicable to the entire channels/programs offering of the IPTV Service. Monitoring these parameters is essential as it details the presence and correctness of channels intended to be delivered over the service.

Some of the parameters that need to be monitored are:

- Number of channels on the service

- Channel Identity (ID) Number

- The names of the services

- Source of content for the services (content provider/ aggregator)

- Number of interactive channels

- Number of free to air/ pay channels

- Genre of channel content and number in each genre

#### Service meta-data

Service meta-data carries additional information aside from the video, audio, and subtitles comprising the main components of the television service.  Incorrect or non-existent meta-data might severely affect overall service quality or result in legal liablities for the operator.

A non-exhaustive list of meta-data parameters includes:

- Parental or age rating

- Source provider of meta-data

- Language sets correctness

- EPG correlation to actual content

- Correctness of subtitling

- Size of meta-data

- Availability of meta-data

Metadata Validity (MV)

Metadata Validity is for checking whether the metadata files from content provision meet or comply with the language used, syntax and semantics which has been specified in advance. For example, the IPTV control domain should check if the file used XML as the description language if XML is the normative description language for IPTV metadata file.

Metadata Integrity (MI)

Metadata Integrity is for checking whether the metadata files from content provision has contain all the necessary information and comply with the metadata specification. For example, the IPTV control domain should check all the attributes such as information about director, actor and so on when it received a metadata file of a movie for VOD purpose.

#### Channel zap time

The measure of channel zap time is the time it takes for the channel to change from the moment the subscriber presses the button on the remote till the video leaves the display.

Channel zap time is typically measured in milliseconds (ms).

#### Correctness Rate (CR)

The correctness rate is the number of times the “correct” content expected by the user is played as a percentage of total number of requests attempted by users in a pre-defined time interval. The CR can be treated as an important parameter to measure the overall users’ experience of the IPTV service.

##### Connection Success Rate (CSR)

The Connection Success Rate is the number of connections which were successfully established with the streaming server as a percentage of total attempted connections in a unit time. The CSR is a good viewpoint from users’ experience when using IPTV service.

##### Connection Time (CT)

Connect time measures the amount of time elapsed between the initial request by the media player or STB and the start of buffering. This time may include Domain Name System (DNS) lookup and resolution, metafile actions, RTSP handshakes and the transport of the first byte of data to the player. The CT is also a good viewpoint from IPTV users’ experience.

### Channel Attribute parameters

As described in Figure ‎7-4, each channel has specific attributes attached to it. They are embedded as part of the transport stream carrying the channels. The PID (program ID) defines the identity of the program being carried by the transport stream (TS).

#### Channel attributes

Multiple attributes of each channel have to be monitored to ensure the performance of the service.

#### PID (Program ID)

#### Type: Audio/ Video/ Data

#### Aspect Ratio

-   Conditional Access Enabled/ Disabled

-   Age rating

#### Encoding Format of the channel

#### Bit-rate of the channel

#### Bit-rate of each component of the channel in case it carries more than one type of information (eg. Audio and Video)

#### Video quality

Video quality assessment represents the video quality grade by assessing according to the quality assessment parameter sets, subjective assessment, or objective assessment.

The user perception of video quality can only be approximated by using objective estimation models and represented as an MOS (Mean Opinion Score).

#### Audio quality

Audio quality assessment represents the audio quality grade by assessing according to the quality assessment parameter sets, subjective assessment, or objective assessment.

The user perception of audio quality can only be approximated by using objective estimation model and represented as an MOS (Mean Opinion Score).

#### Ancillary channel associated attributes

##### Subtitles

Subtitles (also known as Closed Captions) can be defined as textual versions of the dialog in films and television programs, usually displayed at the bottom of the screen. Subtitling information can be “burned in” to the video itself, known as “hard subtitles” or passed as metadata to the program content and superimposed on the video within the set top box.

Subtitles passed as meta-data within the transmission stream are sent using either proprietary formats, or according standard methods such as those defined in [CEA CEA-608D], [CEA CEA-708C], and [ETSI EN 300 743].

The parameters to be monitored for subtitling are:

- Language: The language of subtitling.

- Type: Type and formatting such as plain text, HTML or XML

- Timing Precision: How accurate is the timing precision of the inserted subtitles. The metric is milliseconds. Timing precision is also dependent on bit stream and frame encoding.

##### Descriptive audio

##### Conditional access/scrambling considerations

The MPEG-2 transport stream packet contains the MPEG-2 payload along with the header information. This section assumes the transport stream is in the clear and the conditional access section can be accessed.

The parameters that need to be monitored are part of the conditional access section of the MPEG-2 payload.

The monitoring sub-system must be able to decode the conditional access tables in the MPEG-2 transport stream. The Presence/ Absence of the conditional access can also be derived from the table decode of the MPEG-2 Transport stream.

Multiple proprietary flavours of conditional access are available, each applying the encryption services at different layers of the protocol stack.

### VOD Parameters

#### Content on Demand (VOD/KOD) request performance

Similar to channel zap time the performance of on-demand content can be measured as the time elapsed between the user pressing the remote control button to request content till the time the video first leaves on the screen.

### Other Parameters

#### AAA Success Rate (AAASR)

The success rate about service related request from end users should reflect the users’ experience. So authentication, authorization, and accounting success rate should be monitored.

#### STB Booting Time

Turning on the STB includes the process of a user clicking on an ‘on’ button. The time it takes for the STB to start up and be ready for service consumption (i.e. STB boot up time) is of interest to the consumer.

# Monitoring methods

Network performance monitoring can be performed by either a network element device or dedicated measurement device can perform network performance monitoring. Methods of network performance monitoring are divided largely into three categories: active monitoring, passive monitoring and active and passive hybrid monitoring.

In the active monitoring, a measurement device injects test packets into the network, and the same device or a separate device at other points within the network measures test packets. This method makes the network suffers extra traffic during active monitoring time. On the other hand, it provides control of traffic generation based on variant scenario.

In passive monitoring, a measurement device just observes characteristics of packets on a network link. The observed characteristics can be used for flow analysis. This method does not generate any extra traffic, so it can measure real network status. However, there are limitations in observing characteristics of all packets and in estimating trouble scenarios.

In active and passive hybrid monitoring, passive devices can recognize the sent active probe packets and measure network level metrics such as delay and jitter from the customer’s terminal to the passive measurement point. For this method to be possible, IPPMS (IP Performance Measurement Specification) which is defined in O.211 [ITU-T O.211] to uniquely identify the active probe packet across multiple administration domains is used. O.211 defines controller ID and flow ID to make absolute identification across multiple administration domains. The passive measurement with IPPMS capability captures the desired packets for various measurement operations. The main advantage of this method is to reduce a large number of active probes in the middle of the managed networks.

### Generalized monitoring method for multi-media data based on transmission packet loss

In this method, monitoring points are composed of some sampling points and a reference point. At the reference point, the whole copies of data sent by the sender must be obtained, and the sampling points can be located wherever service monitoring is requested. A bidirectional channel between the reference point and the sampling points is essential, see Figure ‎8-5.

Figure ‎8-5 - Performance Monitoring Point Deployment

There are three steps required to execute this method:

Step 1. After the reference point get the whole copies of sender’s data, the relations between transmission packet number and the characteristics are established and saved.

One typical index structure is shown in Figure ‎8-6.

Figure ‎8-6 - Typical Index Structure

A typical index structure includes three parts: Packet No., Packet Type, and Index Data. Packet No.: specifies the received multi-media packet number, which is used to retrieve the multi-media data.

Packet Type: specifies the type of current multi-media packet, which can be video type or audio type.

Index Data: specifies the multi-media characteristics corresponding to the packet number.

For video type, the index data should include Video Parameters, video frame number, Macro-block Information and Residual Information, where Macro-block Information and Residual Information are optional multi-media characteristics for index construction.

Video Parameters: specifies the parameters of video sequences, such as image width, image height, video frame rate, video format, etc.

Video Frame number: specifies the video frame index corresponding to the multi-media video packet.

Macro-block Information: specifies the feature data that is used to reconstruct damaged video frames, including block type, block address, motion vector, reference picture index, etc.

Residual Information: specifies the difference between a prediction of a sample and its decoded value for each pixel.

For audio type, the index data should include Audio Parameters, Audio Frame No.

Audio Parameters: specifies the parameters of audio.

Audio Frame No.: specifies the video frame index corresponding to the multi-media audio packet.

Step 2. At the sampling point, received packets are sorted according to packet numbers. We can find out which packets are lost by checking the integrity of packets then feed back to the reference point.

Step 3. At the reference point, according to loss packet numbers from the sampling points, we can retrieve loss characteristics according to the stored relationship between the transmitted packet number and their characteristics. Consequently we can judge which grade current service belongs to. The graded means and details are not discussed in this proposal.

### RF Integrity Monitoring

Several processes are involved in the demodulation of RF signals. Generally, they are tuning and demodulation. Demodulation comprises of the following major blocks: analog to digital converter, frequency control and forward error correction.

The RF signal level can be extracted from the tuning stage, while the rest of the measurements can be taken from the demodulator module. The unit of measure for RF signal level is dBm.

The Signal to Noise Ratio (SNR) and the Modulation Error Ratio (MER) can be derived from the constellation of the RF signal, or by other means. The unit of measure for both the SNR and BER is dB.

The Bit Error Rate (BER) can be derived from the forward error correction stage. Depending on the RF modulation, different forward error correction schemes are used.

### Network performance monitoring

Network resource and operational status monitoring shall be conducted during network operation and should include link and bandwidth characteristics, flow direction, inter-node time delay, jitter, and packet loss rate. When a monitored network parameter exceeds a threshold value defined by the operator the network operator should be notified.

#### Network Performance Monitoring Procedure

[ed. Further input and description described here, perhaps reference new recommendation Y.1543  [ITU-T Y.1543] on Network Performance Monitoring?]

### IPTV service attribute monitoring

A direct observation approach to monitoring via the use of a dedicated measurement devices or functionality built into a device deployed at appropriate monitoring points in the IPTV delivery network presents a true metric of service performance.

Figure ‎8-7 shows a deployment scenario that involves monitoring the direct user experience before and after the STB. This setup is applicable for Domain E and monitoring points 4 and 5.

Figure ‎8-7 - Direct Observation Monitoring Model for Domain E

#### Channel line up validation

All the channels in the service should be tested. This has to take into account the various service plans and also region specific channels.

8.4.1.1 Channel Line Up Validation for Domain B

Channel line up validation at Domain B involves checking the various unicast and multicast IP flows prior to delivery from the headend. All the outgoing IP flows should be checked. Their unicast and multicast addresses should be correlated with the channel line up. This will ensure that the IP flows’ addresses are configured correctly.

8.4.1.2 Channel Line Up Validation for Domain E

Channel line up validation at Domain E involves examining the IP traffic entering the STB as well as the audio/video output of the STB.

Examining the STB’s incoming IP traffic requires checking the unicast or multicast address. Each channel is typically assigned a multicast address. A remote control should be used to invoke channel change on the STB. A correlation should be made between the remote control’s channel change parameter and the multicast address of the IP traffic. This channel – multicast address correspondence is specific to an operator’s service. This test also ascertains whether an STB has received the correct channel – multicast address mapping information.

The presence and status of the audio/video output of the STB should be checked whenever a new channel change command is issued by the remote control. Presence indicates whether any audio/video content is produced. Status indicates whether the video is frozen or is in the blackout condition. For the case of audio, silence and tone should be checked. This test is applicable to all the available audio/video outputs of the STB.

#### Service meta-data validation

Meta-Data is typically delivered via tables in the Transport Stream, or by other means, which might comprise of XML tables, HTTP traffic, encrypted data and out-of-band channel delivery. The method in which meta-data is represented and delivered is usually operator and STB specific. This implies a potentially proprietary approach to decoding and examining the meta-data.

8.4.2.1 Service Meta-Data Validation at Domains A and B

Meta-data that are generated and subsequently embedded in the IP flows or delivered by other means should be decoded and validated against the operator’s service offering. Multiple meta-data sets can be checked simultaneously.

8.4.2.2 Service Meta-Data Validation at Domain E

Meta-data is usually presented to the user in the form of an Electronic Program Guide. The STB’s incoming IP traffic should be decoded appropriately at monitoring point 4, according to the method of delivering the meta-data. This meta-data should be checked against the operator’s service offering. Only one meta-data set can be checked at any one time.

#### Channel zap time

Channel zap time can be measured by taking the time difference between the channel change commands from the remote control and the time the new channel’s video and audio are presented on the display. This measurement method takes into account the STB’s internal data buffering and also the audio/video decoding time.

A device can be used to detect the presence of the new channel’s video and audio content. This entails connecting the audio/video outputs of the STB to a display device. An appropriate method must be employed to determine whether audio/video content relating to the new channel is actually received.

### Video quality monitoring

Video quality assessment can be carried out in two ways:

- Subjective assessment

- Objective assessment

Objective assessment can be further subdivided into the following:

- Methods based on perceptual models of the Human Visual System, and

- Methods based on parametric models

Subjective and objective assessment may be used appropriately. Since the performance monitoring for IPTV should be performed automatically at the monitoring points in Figure ‎6-2, objective assessment may be used in real IPTV services. Subjective assessment is still useful for verifying the performance of objective assessment methods.

#### Subjective quality monitoring

There are a number of subjective video quality measurement methods suggested in ITU-R BT. 500 [ITU-R BT.500]	. Depending on the impairment being evaluated, the selection of a method requires careful considerations of sequence duration, transmission condition, ecological environment and other factors. The output of the subjective tests is often an average of the quality ratings called Mean Opinion Score (MOS).

ITU-R BT.500-7 recommends a five-grade impairment scale:

5 – imperceptible

4 – perceptible, but not annoying

3 – slightly annoying

2 – annoying

1 – very annoying

The grading of video quality can also be expressed as a percent from 0 to 100, with 0 indicating no distortion. The higher the value is, the greater the distortion occurs.

ITU-R BT.500-11 includes specifications on how to perform different types of subjective test. The double stimulus continuous quality scale (DSCQS) and double stimulus comparison scale (DSCS) are both double stimulus where viewers rate the quality or change in quality of two video streams (reference and impaired). The single stimulus continuous quality evaluation (SSCQE) assesses only one impaired video stream. The DSCQS is claimed to be less sensitive to the context, while SSCQE is claimed to produce more representative estimates for quality monitoring.

In addition, ITU-T P.910 includes specifications on how to perform a type of subjective test for video. The absolute category rating with hidden reference removal (ACR-HRR) is a single stimulus continuous quality evaluation. The ACR-HRR is adopted in the VQEG Multimedia / RR-NR / HDTV test models.

#### Objective quality monitoring

Many advances have been made in objective video quality monitoring techniques. Although not as accurate as subjective quality measurement, objective assessment can provide quick support to fine-tuning network variables. Due to the complexity and time consuming of subjective video monitoring, there has been significant development of perceptual video quality measurement (PVQM) algorithms in recent years in an attempt to replicate the scores given by subjects in an objective tool. Objective quality monitoring can be classified into following categories:

- Techniques based on models of human video perception

- Techniques based on video signal parameters

Human video perception models attempt to emulate the characteristics of the human vision system to obtain video quality scores. A good model should yield high correlation to the ratings the actual viewers would provide. Three popular approaches can be used:

- Full reference (FR) – both the original transmitted and received video are available to determine the video quality (Figure ‎8-8).

- Reduced reference (RF) – partial information about transmitted video and full information about received video are available to determine the video quality (Figure ‎8-9).

- No reference (NF) – only the received video is available to determine the video quality.

By comparison between the transmitted and received video, various parameters can be obtained which are valuable in quality monitoring. Mean square error (MSE) and mean absolute error (MAE) calculate the frame difference between corresponding streams. The difference can also be assessed by the peak signal to noise ratio (PSNR) which is the log of the ratio of the peak signal squared to the MSE in a similar fashion to analogue system.

Figure ‎8-8 - A Full-Reference Model

Figure ‎8-9 - A Reduced-Reference Model

The three objective models of measuring video quality can be applied to different parts of the IPTV delivery chain. Table ‎8-3 shows the relevance of the three models in the five domains.

Table ‎8-3 - Video Quality Measurements in Various Domains

|  | Domain A | Domain B | Domain C | Domain D | Domain E |
| --- | --- | --- | --- | --- | --- |
| Full Reference | Yes | Yes |  |  |  |
| Reduced Reference | Yes | Yes |  |  | Yes |
| No Reference | Yes | Yes |  |  | Yes |

##### Back channel requirements

IPTV terminals may have video quality evaluation function and reporting function of the video quality scores to IPTV servers (or service managing servers) in order to gather video quality reports from all or some of IPTV terminals.

When this approach is employed, an appropriate transmission protocol to send video quality scores and other information related with end-user quality is required. Furthermore, an appropriate transmission protocol to send feature information is required, when a reduced reference method is employed for video quality evaluation.

This approach is shown in Figure ‎8-10.

Figure ‎8-10 - Back Channel for Quality Reporting

Quality measurements can be made at various measuring points including a number of reference points and terminals and can be used for various purposes. For example, they can be used for quality-based billing. Quality monitoring can be also used to optimize codec parameters at streaming servers. Furthermore, it can be used for network management. Therefore, depending on applications, the unit in which quality measurements are performed should send the quality scores to the corresponding destination. If the quality scores are used for quality-based billing, the measuring unit should send the information to the service provider (Figure 7). When quality scores are used to optimize the codec parameters, the measuring unit needs to send the information to a streaming server (Figure ‎8-11). If they are to be used for network management, the measurement results need to be sent to the network provider (Figure ‎8-12).

Figure ‎8-11 - A streaming server which optimizes codec parameters based on the perceptual quality

Figure ‎8-12 - A network provider which uses perceptual quality information

##### Full-Reference Models

A full-reference model requires that both source video sequences and processed video sequences should be available at the measurement point.

ITU-T J.144 specifies different types of objective test methods for monitoring objective video quality as a full reference model. They are based on human video perception.

##### Reduced-Reference Models

A reduced-reference method can be employed to monitor the perceptual video quality ata measurement point. If a reduced-reference method is used at the receiver, the transmitter needs to transmit feature data in addition to video data (Figure ‎8-13). When a reduced-reference is to be employed in IPTV applications, a transmission protocol to send the feature data should be specified. It is desirable that the feature data should be available at the receiver in a timely manner. It is preferred that the feature data is sent with a time advance. Furthermore, some error handling mechanisms should be employed for sending the feature data.

Figure ‎8-13 - A block diagram of video quality monitoring using an RR model

##### No-Reference Models

When a no-reference method is used, there may be no special requirements for the system.

Although the performance of no-reference methods is inferior to that of full-reference and reduced-reference methods, it can be improved by developing NR methods which also use video stream data.

##### Reconstructing the received video using transmission error information

In digital communications, transmission errors include packet loss and packet delay and their effects can be exactly identified when video data is transmitted using packets. Furthermore, if there is no transmission error, the video quality at the receiver will be identical to the video quality of the video sent by the transmitter. Therefore, if the receiver sends transmission error information which includes information on packet loss and delay in packetized video transmission, the service provider can exactly reconstruct the received video seen at the receiver (Figure ‎8-14). Finally, the service provide may use a FR or RR method to evaluate the video quality of the received video seen at the receiver. Table ‎8-4 describes messages for sending transmission error information to the service provider.

Figure ‎8-14 - A method for a transmitter to monitor video quality at a receiver using transmission

Table ‎8-4 - Message Descriptions

| Type of transmission errors | Message descriptions |
| --- | --- |
| Information on the receiver | A model identification message |
| Source Identifier | A source identification message |
| Information on a lost packet | A lost packet index |
| Information on lost packets | A starting packet index and an ending packet index of the lost packets |
| Information on a delayed frame | A delayed frame index and the amount of delayed time |
| Information on a skipped frame | A skipped frame index |
| Information on skipped frames | A starting frame index and an ending frame index of the skipped frames |

##### Packet-layer model

The packet-layer model can estimate video quality using IP information that excludes video-related payload information, as shown in Figure ‎8-15. This model cannot take into account the effects of video content on the subjective quality. Its computational load is very light because it does not use video-related payload information.

##### Bit stream layer model

It is possible that an objective model may have access to bit stream data from which the model can obtain additional information on transmission errors (e.g., delay, packet loss), codec parameters (e.g., type, bit-rates, frame rates, codec parameters), etc. This kind of information is easily available from bit-stream data at the receiver. It is expected that such models may provide improved performance in terms of accuracy and speed compared to objective video quality models which use only processed video sequences.

The bit stream layer model can estimate the quality of each video sequence using IP information that includes video-related payload information, as shown in Figure ‎8-15. That is, it can take into account the effects of video contents.

ES

TS Header

RTP Header

UDP Header

IP Header

Packet Layer Model

Bit Stream Layer Model

Product Information

(information about the Codec, Client, etc.)

Figure ‎8-15 - Relationship between packet-layer model and bit stream layer model Hybrid perceptual/bit-stream models

If an IPTV terminal or monitoring equipment is to use such a hybrid perceptual/bit-stream model for quality monitoring, it should provide bit-stream data to the model. The input requirements for hybrid perceptual/bit-stream models are shown in Figure ‎8-16, Figure ‎8-17, and Figure ‎8-18.

Figure ‎8-16 - Input requirements for FR hybrid perceptual/bit-stream models

Figure ‎8-17 - Input requirements for RR hybrid perceptual/bit-stream models

Figure ‎8-18 - Input requirements for NR hybrid perceptual/bit-stream models

In order to improve the accuracy of objective models, it is also possible to transmit video quality scores of the compressed video data which are transmitted (Figure ‎8-19). Periodic video quality measurements should be made such that sufficient information is available on the video quality of the processed video quality sequence. If there are no transmission errors, the video quality at the receiver would be the same as that of the transmitted video sequence. If transmission errors occur, the received video sequence suffers from both compression impairments and transmission error impairments. With video quality scores available, an objective model which measures the video quality of the received video sequence may be improved, particularly for NR models. The video quality scores can be transmitted as meta-data. Alternatively, they can be transmitted using watermark techniques.

Figure ‎8-19 - Generating and transmitting video quality scores of compressed data

If an IPTV terminal or monitoring equipment are to use these objective models for quality monitoring, the input requirements are shown in Figure ‎8-20 through Figure ‎8-22.

Figure ‎8-20 - Input requirements for a FR hybrid perceptual/bit-stream model which uses the video quality scores of transmitted compressed video data

Figure ‎8-21 - Input requirements for a RR hybrid perceptual/bit-stream model which uses the video quality scores of transmitted compressed video data

Figure ‎8-22 - Input requirements for a NR hybrid perceptual/bit-stream model which uses the video quality scores of transmitted compressed video data

### Audio quality monitoring

Audio quality can be carried out in two ways:

- Subjective assessment

- Objective assessment

#### Subjective quality monitoring

There are a number of subjective audio quality measurement methods suggested in ITU-R BS.1283-1 and ITU-T Recommendation P.800 [ITU-T P.800]. The methods used for the subjective assessment of sound quality itself and of the performance of audio systems depend to some extent on the intended purpose of the assessment. The output of the subjective tests is often an average of the quality ratings called Mean Opinion Score (MOS).

The following five-grade scales may be used for the subjective assessment of sound quality or impairment as shown in Table ‎8-5

Table ‎8-5 - Audio Quality and Impairment Scales

| Quality |  | Impairment |  |
| --- | --- | --- | --- |
| 5 | Excellent | 5 | Imperceptible |
| 4 | Good | 4 | Perceptible, but not annoying |
| 3 | Fair | 3 | Slightly annoying |
| 2 | Poor | 2 | Annoying |
| 1 | Bad | 1 | Very annoying |

For comparison tests, either a method based on the seven-grade comparison scale shown in Table ‎8-6 or one based on numerical differences using the above five-grade scales may be used. In general, these are not equivalent and may not give the same results. It is essential that the intended direction of the comparison be clearly indicated.

Table ‎8-6 - Audio Comparison Scale

| Comparison |  |
| --- | --- |
| 3 | Much better |
| 2 | Better |
| 1 | Slightly better |
| 0 | The same |
| –1 | Slightly worse |
| –2 | Worse |
| –3 | Much worse |

ITU-R BS.1283-1 shows the list of Recommendations on subjective methods for audio quality assessment. Clearly, some areas of application overlap. The applicability of one or more of the Recommendations should be decided, based on the purpose of the tests, before the tests are designed.

#### Objective quality monitoring

Since subjective quality assessments are both time consuming and expensive, it is desirable to develop an objective measurement method in order to produce an estimate of the audio quality. A number of methods for making objective perceptual measurements of perceived audio quality have been introduced during the last decade, including Perceptual Estimation Audio Quality (PEAQ), the full reference audio quality estimation method in the ITU-R BS.1387 [ITU-R BS.1387] and Perceptual Estimation of Speech Quality (PESQ) as defined in ITU-T Recommendation P.862 [ITU-T P.862].

One must, however, keep in mind that objective measurement methods are not generally a substitute for formal subjective listening tests.

### Ancillary attribute monitoring

#### Subtitles

#### Closed captions

#### Descriptive audio

# Bibliography

[b_IETF RFC3357]	IETF RFC 3357 (August 2002), One-way Loss Pattern Sample Metrics.

[b_BSTJ Elliot]	E. O. Elliot. Estimates of error rates for codes on burst-noise channels. Bell System Technical Journal, 42:1977-1997, September 1963

[b_BSTJ Gilbert]	E. N. Gilbert, Capacity of a burst-noise channel. Bell System Technical Journal, 39:1253-1265, September 1960

Annex A

Considerations for Network Performance Monitoring when using Overlay Networks

(This annex forms an integral part of this document)

- Overlay Measurement

In applications such as IPTV overlay multicast, the overlay path is constructed between overlay nodes in the application layer to perform overlay functions. In order to ensure high performance of the overlay network, overlay measurement needs to be carried out to measure the IP network performance, e.g. bandwidth, packet loss between overlay nodes such that the appropriate overlay path is selected.

On the other hand, overlay measurement can also be used by the service providers to monitor the IP network performance of the delivered applications to help ensure QoS to the end users. For example, if the IP network performance of an overlay path is poor, another overlay path may be constructed to guarantee good service quality to the end users.

# Measurement Unit

A network measurement unit for overlay measurement is recommended to be deployed in the IP network layer. The reason is that the network measurement unit could reside on, or correspond with one or more existing network monitoring points (defined in Section 6 – “Monitor Points” in WD – “Performance Monitoring for IPTV”) to utilize the capability of performance monitoring of these monitoring points.

overlay network

IP network

commands receiving

measurement

monitoring points

results forwarding

Figure A.1 - Network Measurement Unit

Hence, a network measurement unit could include a receiving function, measurement function and forwarding function, as shown in Figure A.1. The receiving function receives overlay measurement commands from the overlay network and forwards the commands to the measurement function. The measurement function calls the network monitoring points to measure the performance between network devices and forwards the results to the forwarding function. The forwarding function sends the network measurement results to the overlay network.