| The following information has been agreed by WG2 to be under further consideration by WG2 prior to inclusion in formal output documents. INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-DOC-0173 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 6th FG IPTV meeting: Tokyo, Japan, 15-19 October 2007 |  |  |
| OUTPUT DOCUMENT |  |  |  |  |
| Source: | Editor |  |  |  |
| Title: | Living List: Application layer reliability solutions for IPTV |  |  |  |

# Item 1 (added at 2nd Meeting of ITU-T FG IPTV, Busan): ARQ

Source: FG IPTV-C-0220, ETRI, “Existing standards/activities for ARQ-based application layer reliability solutions”

Status: closed.

# Item 2 (added at 3rd Meeting of ITU-T FG IPTV, Mountain View):  ATIS IIF IPTV Packet Loss Issue Report

Source: FG IPTV-IL-0040, ATIS IIF,  “ATIS IIF Liaison to ITU-T Focus Group on IPTV”

Enclosed in this incoming liaison from ATIS IIF is the ATIS IIF IPTV Packet Loss Issue Report (ATIS-0800005). The IPTV Packet Loss Issue report is a technical report that explores the range of potential solutions and makes recommendations regarding their applicability for an IPTV service.

ATIS IIF offers to the ITU-T Focus Group on IPTV that if wishes to use text from these documents for the purposes of developing IPTV standards and requirements to contact the liaison contact person.

Status: It was decided that the ATIS IIF IPTV Packet Loss Issue Report (ATIS-0800005) is kept in this document for easy access of information relevant to the work on Application layer reliability solutions for IPTV in the ITU-T Focus Group on IPTV.

# Item 3 (added at 3rd Meeting of ITU-T FG IPTV, Mountain View): Background Information on DVB Application-Layer FEC

During the 3rd meeting of the ITU-T FG IPTV, a high-level information on the DVB AL-FEC solution was received. Additional details were received during the 4th meeting of the ITU-T FG IPTV.

Sources:

- FG IPTV-IL-0064, Digital Video Broadcasting Project; Technical Module subgroup on IP Infrastructure (DVB TM-IPI),  “Response to FG IPTV-OL-0020 on Application Layer FEC in DVB-IPI”

In incoming Liaison Statement FG IPTV-IL-0064 the Digital Video Broadcasting Project (DVB) Technical Module group on IP Infrastructure (TM-IPI) stated the appreciation of the efforts of the ITU-T FG IPTV WG2 to study the adoption of DVB-IP AL-FEC solution for ITU-T FG IPTV AL-FEC.

In this document, the DVB project provided ITU-T FG IPTV WG2 with the latest information on the AL-FEC work for your upcoming meeting. Specifically, the following recently published documents have been provided:

| - A086rev4 - Transport of MPEG 2 Transport Stream (TS) Based DVB Services over IP Based Networks (DVB-IPTV Phase 1.3 - Draft TS 102 034 1.3.1) as a bluebook which contains in Annex E the specification of the DVB-IPI AL-FEC. |  |
| --- | --- |
| - A115 - Blue Book on AL-FEC Evaluations also as a bluebook which is intended to provide background on the evaluation process behind the choice of the hybrid approach in the DVB application layer forward error correction (AL-FEC) specification as included in the DVB-IP Phase 1 Handbook. |  |

A115 document consists of two documents that were created by DVB as part of the AL-FEC evaluation process. Document A presents the evaluation criterion that were agreed before the selection process started and Document B is the report on the evaluation process. Document B also gives the rationale for the choice of the hybrid approach used in the DVB AL-FEC specification.

Status: It was decided to keep this background information in the living list as the documents may contain relevant information on the ITU-T AL-FEC solution.

# Item 4 (added 4th Meeting of ITU-T FG IPTV, Bled): DVB AL-FEC as ITU-T AL-FEC

During the 4th Meeting of ITU-T FG IPTV a provisional endorsement was added to the living list. The remaining issues were resolved during the 5th Meeting of ITU-T FG IPTV.

The only remaining aspect not integrated in the WD on application layer reliability solutions are the details on what parts of the DVB-IPI AL-FEC solution are required, and what is recommended, or what can optionally be used. Clarification information was provided in Input Contribution FG IPTV-C-0776, Digital Fountain, “Usage of DVB-IP Application Layer FEC Specification”.

Specifically section 3 of this document provides clarification of this subject:

The ITU-T FG IPTV does not yet define a full set of content delivery protocols, i.e. the control and transport protocols yet. However, it quite likely that the media will be transported either through MPEG-2 TS over RTP/UDP or by generic audio/video streams. Therefore, DVB AL-FEC solution likely covers most of the expected transport protocols.

With respect to control protocols, the details can not be specified at this point, but it is essential to understand what control information needs to be transported for the AL-FEC, once the content delivery protocol more clearly defined.

For the transport protocols it is assumed that the same protocols for the ITU-T FG IPTV AL-FEC as for the DVB-IP AL-FEC solution as specified in section E.6 are used.

For the control protocols, it is assumed that the same semantics for the ITU-T FG IPTV AL-FEC as for the DVB-IP AL-FEC solution as specified in section E.6 are used. The details on the syntax need to be defined.

For the decoder, it is proposed that if application layer FEC in ITU-T FG IPTV is supported, then it is

- required to support the minimum decoder performance as specified in section E.5.1.1 in case MPEG-2 Transport Stream over RTP is used,

- recommend to support the enhanced decoder performance as specified in section E.5.1.2 in case MPEG-2 Transport Stream over RTP is used,

- required to support a decoder performance for which the minimum performance is determined by only using step 2 in section E.5.2.1, i.e. only Raptor decoding performed, or equivalently, the performance of the reference decoding algorithm as provided in [1], Annex C, is fulfilled, in case arbitrary packet flows are used according to the definition in section E.6.3 and E.6.4.

[1] 	3GPP TS 26.346 – 3rd Generation Partnership Project; Technical Specification GroupServices and System Aspects; Multimedia Broadcast/Multicast Services (MBMS); Protocols and codec’s (Release 6).

Status: It was decided to keep this information in the LL as the information may be relevant to integrate the recommended AL-FEC solution into the IPTV architecture and to provide background information on the recommended AL-FEC solution.

# Item 5 (added 5th Meeting of ITU-T FG IPTV, Geneva): Other AL-FEC Methods

Source: FG IPTV-C-0684, NEC Corporation, “Proposal for Modifications to FG IPTV-DOC-0088”

Status: open.

# Item 6 (added 6th Meeting of ITU-T FG IPTV, Tokyo): Application Layer Definition and Other Application Layer Error Recovery Mechanisms

Source: FG IPTV-C-0885, ZTE Corporation, “Supplement for application layer error recovery mechanisms for IPTV”

In this contribution,

- application layer is described

- other application layer error recovery mechanisms are introduced, including interleaved mode, video error resilience techniques, etc.

Status: A definition of the application layer in the working document is considered helpful, but the proposal in the document was not considered to be mature enough for the working document. Further input on this subject is encouraged. On the subject of other application layer error recovery mechanisms, further clarification was requested.

# Item 7 (added 6th Meeting of ITU-T FG IPTV, Tokyo): DVB work on retransmission and content download services

Source: FG IPTV-IL-0105, Digital Video Broadcasting Project, “Response to FG IPTV-OL-0058 on “Update on Application Layer Error Recovery Mechanisms””

In this contribution, among others, the ITU FG on IPTV was informed on ongoing work in DVB on retransmission and content download services (CDS). Brief background on scope of the work was provided.

The scope of the retransmission work in DVB is to describe how a Home Network End Device (HNED) can provide immediate feedback (FB) towards the network with respect to the reception of a MPEG2TS/RTP/UDP DVB compliant stream and through which missing packets can be re-quested for retransmission.  This mechanism is optional for an HNED and may be used instead of the optional DVB AL-FEC mechanism or in combination with the DVB AL-FEC mechanism, to provide for protection against packet loss of RTP streams. It is envisioned to be applicable for both unicast services (Content-on-Demand and Broadcast TV service with trick mode) as well as for multicast (Live Broadcast TV service), with the constraint that these services are delivered over RTP.

The scope of the CDS work in DVB is to specify a content download mechanism in an end-to-end IPTV system. This content download mechanism is for downloading of IPTV content to a local store in order to allow for IPTV delivery in areas where a real-time channel is not possible or prone to errors, for delivery of multiple simultaneous services and/or for reduced cost offers as the band-width consumption may be limited compared to live services.

For both work items, RET and CDS, Commercial Requirements have been completed and technical specification work is ongoing.

Status: It was decided to keep this information in the LL as the information may be relevant to further activities in the ITU-T FG on IPTV.

# Item 8 (added 6th Meeting of ITU-T FG IPTV, Tokyo): Application Layer FEC for Hybrid Combination of FEC with Retransmission and Feedback

Source: FG IPTV-C-1003, Digital Fountain, “Proposed Updates for Working Document on “Application Layer Error Recovery Mechanisms”, Section 8.3. “Hybrid””

The DVB AL-FEC mechanism [ETSI TS 102 034], Annex E, with the availability of fountain prop-erty of the Raptor code provides all the properties to deploy such combinations of FEC and receiver feedback in an efficient and backward-compatible manner.

Status: Further clarification on this subject was requested.

# Item 9 (added 6th Meeting of ITU-T FG IPTV, Tokyo): Signalling and Control Information for DVB-IP AL-FEC mechanism

Source: FG IPTV-C-1000, Digital Fountain, “Signalling and Control Information for DVB-IP AL-FEC mechanism”

The ITU-T FG IPTV does not yet define a full set of content delivery protocols, i.e. the control and transport protocols are still under review. However, it quite likely that the media will be transported either through MPEG-2 TS over RTP/UDP or by generic audio/video streams. Therefore, DVB AL-FEC solution likely covers the expected transport protocols.

With respect to control protocols, the details to support the AL-FEC mechanism need to be specified at this point, but it is essential to understand what control information needs to be transported for the AL-FEC, once the content delivery protocol more clearly defined.

For the control protocols, it is proposed to use the same semantics for the ITU-T FG IPTV AL-FEC as for the DVB-IP AL-FEC solution as specified in section E.6. The details on the syntax still need to be defined.

The following information may be necessary to be conveyed for the AL-FEC (see also [ETSI TS 102 034]):

## FEC Base Address

In the multicast case, this option may be included in messages from server to client. It indicates the IP Multicast address on which the base AL-FEC layer may be found. If not included then the base AL-FEC layer is sent on the same multicast address as the source data.

## FEC Base Port

This option may be included in messages from server to client and from client to server. It indicates the UDP destination port for the base AL-FEC layer. When included in a message from client to server, it indicates that AL-FEC is supported by the client and specifies the destination UDP port that should be used for the AL-FEC base layer. When included in a message from server to client, it indicates the UDP destination port that the server will use for the AL-FEC base layer. In the multicast case, if this option is specified but the FEC Base Address is not, then the AL-FEC layer is assumed to be available on the same multicast address as the main stream. If this header is not specified then AL-FEC is not provided and the FEC Base Address shall not be present. This header shall be present if the FEC Enhance Port header is present.

## FEC Enhance Address

In the multicast case, this option may be included in messages from server to client. It indicates the IP Multicast address on which an enhancement AL-FEC layer may be found. This option may be repeated to specify multiple enhancement layers.

## FEC Enhance Port

This option may be included in messages from server to client and from client to server. It indicates the UDP destination port for the enhancement AL-FEC layer. When included in a message from client to server, it indicates that the AL-FEC enhancement layer is supported by the client and specifies the destination UDP port that should be used for the AL-FEC enhancement layer. When included in a message from server to client, it indicates the UDP destination port that the server will use for the AL-FEC enhancement layer. In the multicast case, if this option is specified but the FEC Enhance Address is not, then the AL-FEC enhancement layer is assumed to be available on the same multicast address as the main stream. If this header is not specified then AL-FEC enhancement is not provided and the FEC Enhance Address shall not be present. This header shall only be present if the FEC Base Port Layer header is present.

## FEC MaxBlockSize

This indicates the maximum number of stream source packets that will occur between the first packet of a source block (which is included) and the last packet for that source block (source or repair).

## FEC MaxBlockTime

This indicates the maximum sending duration of any AL-FEC block in milliseconds.

## FEC OTI

This indicates the FEC Object Transmission Information for the Raptor AL-FEC layer(s).

A specific syntax for transporting this information can be found in [ETSI TS 102 034]

Status: This information was considered to be relevant and was also discussed in WG4 and WG5. It was agreed that this information shall be tracked and will likely be integrated in a new Working Document under control of WG4 entitled “Working Document on IPTV-related Protocols”. Details on integration are expected to be resolved during the 7th meeting of the ITU-T FG on IPTV once this WD has evolved.

____________