| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-DOC-0178 |  |
|  |  |  | English only |  |
| WG(s): 5 |  | 6th FG IPTV meeting: Tokyo, Japan, 15-19 October 2007 |  |  |
| OUTPUT DOCUMENT |  |  |  |  |
| Source: | Editor |  |  |  |
| Title: | Living List: Aspects of Home Network supporting IPTV Services |  |  |  |

This document is the updated Issues List for WG5.

# Issues List Structure

What is sometimes referred to as the “Terms of Reference” and the “Living List” is combined into a single table, the ‘Issues List’, consisting of 4 columns as shown below:

| - Item Number | - Status | - Item Description | - Reference(s) |
| --- | --- | --- | --- |
| - 1 | - Open | - Identifying HN specific issues in DHCP operation. | - C526 |
| - 2 | - Close | - QoS mechanism based on HGI |  |
| - 3 | - Close | - Access Gateway based on HGI |  |
| - 4 | - Close | - Remote management based on HGI |  |
| - 5 | - Open | - Home Network requirements | - Annex C |
| - 6 | - Open | - Home Network QoS operation | - Annex D |
| - 7 | - Open | - DHCP operation | - Annex E |

- The ‘Item Number’ uniquely identifies an issue for quick reference.

- The ‘Status’ of an issue shall be either ‘Open (date)’, ‘Agreed (resolution date)’ or 'Closed (resolution date)'.

- An issue may become partially resolved at some point in time. In this case, it shall be split apart. The unresolved part of the issue shall keep the original item number and Open status. The resolved part of the issue shall use the same number with an added numerical suffix (e.g., “9.1”) and shall have the status ‘Agreed (resolution date)’.

- The ‘Item Description’ delineates an issue as follows:

- ‘Open’ issues, which generally address what the issue is, shall be formatted as questions.

Example: “Should the IPTV Architecture allow for the utilization of Internet protocols and standards?”

- ‘Agreed (resolution date)’ issues, which generally address how the issue has been resolved, can be categorized as general ‘goals’ or ‘requirements’, or as ‘specific actions’.

- Goals shall be identified by using the words ‘that ... should ...’ in the delineation of an issue.

- Example: “that the IPTV Architecture should allow for the utilization of Internet protocols and standards”.

- Requirements shall be identified by using the words ‘that ... shall ...’ in the delineation of an issue.

- Example: “that the IPTV Architecture shall allow for the utilization of Internet protocols and standards”.

- Specific actions shall be identified by using the words ‘to adopt ... ’ in the delineation of an issue. These are generally for adopting referenced text.

- Examples:	“to adopt (doc #) as the initial working text for the IPTV Architecture

- document”

- “to adopt the text of (doc #) as the text for section y.y of the IPTV

- Architecture document”.

- 'Closed (resolution date)' issues are those that will no longer be considered, either because it has been explicitly agreed to no longer consider them, or because they have been superseded by other agreements and are no longer pertinent.

- The ‘Reference(s)’ is a list of meeting contribution numbers that address a particular issue.

For contributions that have been submitted “for information only”:

- references to existing open issues are shown in parentheses

- references to agreed issues are not shown

- OPEN issues will automatically be CLOSED after 3 meetings of inactivity. CLOSED issues will be deleted from the Issues List after one meeting cycle.

Annex C

Remote management based on HGI

# 9	Remote management

# 9.1	Management Architecture

Note1 – Baseline material: Clause 5.4 (HGI)

Note2 – The management architecture including NW side entities such as RMS and ACS should be aligned with WG1 and WG4.

Note3 – Figure 11 and 12(HGI) should be revised.

# 9.1.1	Device Management

Note1 – Baseline material: Clause 5.4.1 (HGI)

Note2 – Figure 13, 14, 15 and 16 should be revised.

Note3 – Table 2 shows some categories such as D, U, CD and so on for terminal device from the viewpoint of remote management capability. The relevant category for IPTV-TD should be discussed.

# 9.1.2	QoS Management

Note – Baseline material: Clause 5.4.2 (HGI)

# 9.1.3	Security Management

Note1 – Baseline material: Clause 5.4.3 (HGI)

Note2 – Security management should be aligned with WG3.

# 9.1.4	Software Management and Upgrade

Note – Baseline material: Clause 5.4.4 (HGI)

# 9.1.5	Performance Monitoring and Diagnostics & Troubleshooting

Note – Baseline material: Clause 5.4.5 (HGI)

# 9.1.6	Local Management Application

Note1 – Baseline material: Clause 5.4.6 (HGI)

Note2 – Figure 17 should be revised.

## 9.2	Management requirement

The IPTV TD shall support an IP-based bi-directional communication including a software updating mechanism for remote management.

Editor’s Note – The above sentence is a original text contained in clause 9.1 of FG IPTV-DOC-0069.

Note – Baseline material: Clause 6.5(HGI)

## 9.2.1	Northbound Interfaces

Note1 – Baseline material: Clause 6.5.1(HGI)

Note2 – R214 and R215 are defined as mandatory items in HGI document.  Further study is needed.

Note3 – R214: Further study is needed on the interface for HG/ACS.  Alignment with WG1 and WG4 should be discussed.

R214: “The HG/ACS communication interface MUST comply with all the mandatory requirements of the CWMP protocol v1 as defined in TR-069 [1].”

## 9.2.2	RMS Requirements

Note1 – Baseline material: Clause 6.5.2(HGI)

Note2 – R216, R217, R220, R221, R223 and R223 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.3	General HG configuration and management

Note1 – Baseline material: Clause 6.5.3(HGI)

Note2 – R225, R226, R227, R229, R230. R231, R232, R233 and R234 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.4	Definition of the data model supported

Note1 – Baseline material: Clause 6.5.4(HGI)

Note2 – R235, R236, R237 and R238 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.5	Diagnostics, notifications and alarms

Note – Baseline material: Clause 6.5.5(HGI)

## 9.2.5.1	Notifications and logging

Note – Baseline material: Clause 6.5.5.1(HGI)

## 9.2.5.1.1	Mechanisms

Note1 – Baseline material: Clause 6.5.5.1.1(HGI)

Note2 – R239, R240 and R241 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.5.1.2	Mechanisms

Note1 – Baseline material: Clause 6.5.5.1.2(HGI)

Note2 – R242, R243, R244 and R245 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.5.2	Statistics and Diagnostics

Note – Baseline material: Clause 6.5.5.2(HGI)

## 9.2.5.2.1	Mechanisms

Note1 – Baseline material: Clause 6.5.5.2.1(HGI)

Note2 – R246 and R247 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.5.2.2	Defined diagnostics and statistics

Note1 – Baseline material: Clause 6.5.5.2.2(HGI)

Note2 – R248, R250, R251 and R252 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.6	End device management

Note – Baseline material: Clause 6.5.6(HGI)

## 9.2.6.1	Device Identification

Note1 – Baseline material: Clause 6.5.6.1(HGI)

Note2 – R254, R257, R258, R259, R260, R261, R262, R263, R264, R266, R267, R268, R269, R270, R271, R273, R274, R275 and R276 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.6.2	Activity discovery and Database management

Note1 – Baseline material: Clause 6.5.6.2(HGI)

Note2 – R277, R278, R279, R280, R281, R282, R283, R284, R285, R286, R287, R288, R289, R290,and R291 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.7	Local HG management user interface

Note – Baseline material: Clause 6.5.7(HGI)

## 9.2.7.1	General

Note1 – Baseline material: Clause 6.5.7.1(HGI)

Note2 – R293, R294, R296, R297 and R299 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.7.2	Design guidelines

Note1 – Baseline material: Clause 6.5.7.2(HGI)

Note2 – R300 and R302 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.7.3	Operator Portal User Interface link

Note1 – Baseline material: Clause 6.5.7.3(HGI)

Note2 – R305 and R306 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.7.4	Contents and functionality

Note1 – Baseline material: Clause 6.5.7.4(HGI)

Note2 – R308, R309, R310 and R311 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.8	Firmware management and updates

Note1 – Baseline material: Clause 6.5.8(HGI)

Note2 – R312 is defined as mandatory items in HGI document.  Further study is needed.

## 9.2.8.1	General

Note – Baseline material: Clause 6.5.8.1(HGI)

Note2 – R313, R314 and R315 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.8.2	Software upgrades

Note1 – Baseline material: Clause 6.5.8.2(HGI)

Note2 – R316, R317, R319, R320, R321 and R322 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.8.3	Configuration

Note1 – Baseline material: Clause 6.5.8.3(HGI)

Note2 – R323, R324, R325, R326 and R327 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.8.4	User Interaction

Note1 – Baseline material: Clause 6.5.8.4(HGI)

Note2 – R329 is defined as mandatory items in HGI document.  Further study is needed.

## 9.2.9	Security management

Note – Baseline material: Clause 6.5.9(HGI)

## 9.2.9.1	Firewall management

Note1 – Baseline material: Clause 6.5.9.1(HGI)

Note2 – R330, R331, R332, R333, R334, R335 and R336 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.9.2	Application Layer Gateway Management

Note1 – Baseline material: Clause 6.5.9.2(HGI)

Note2 – R337, R338 and R339 are defined as mandatory items in HGI document.  Further study is needed.

## 9.2.10	End-device recommendations

Note1 – Baseline material: Clause 6.5.10(HGI)

Note2 – R340, R341, R342, R343, R344, R345 and R346 are defined as mandatory items in HGI document.  Further study is needed.

Annex C

Home Network requirements

# 6.2.2.1.1	Requirements for Classification of packets received upon the WAN ingress

| N° | Requirement |
| --- | --- |
| R103 -> LL | Access GW MUST be able to classify packets based upon IP destination address. |
| R104 -> LL | Access GW MUST provide a configurable IP destination subnet mask, so that classification is performed only upon the network prefix bits of the IP destination address. |
| R105 -> LL | Access GW MUST be able to classify packets based upon IP source address. |
| R106 -> LL | Access GW MUST provide a configurable IP source subnet mask, so that classification is performed only upon the network prefix bits of the IP source address. |
| R107 -> LL | Access GW MUST be able to classify packets based upon the DSCP field. |
| R108 -> LL | Access GW MUST be able to classify packets based upon the Protocol field in the IP header (e.g. ICMP, IGMP, TCP, UDP, …). |
| R109 -> LL | Access GW MUST be able to classify packets based upon source TCP/UDP port number or range of port numbers. Port numbers may be either statically configured or determined dynamically within Access GW. This specification does not specify methods for dynamic determination of port number. This could be done, for example, by use of application layer logic. |
| R110 -> LL | Access GW MUST be able to classify packets based upon destination TCP/UDP port number or range of port numbers. Port numbers may be either statically configured or determined dynamically within Access GW. This specification does not specify methods for dynamic determination of port number. This could be done, for example, by use of application layer logic. |

# 6.2.2.1.3	Requirements for Multi-field Classification packets received on the LAN ingress ports

| N° | Requirement |
| --- | --- |
| R128 -> LL | Access GW MUST be able to classify packets based upon the LAN type (Ethernet, Wi-Fi, etc.) |
| R129 -> LL | Access GW SHOULD be able to classify packets based upon physical port. |
| R130 -> LL | Access GW MUST be able to classify packets based upon MAC source address |
| R131 -> LL | Access GW MUST have a configurable MAC source address mask, so that classification is performed only upon bit fields within the MAC source address determined by this source address mask. |
| R132 -> LL | Access GW MUST be able to classify packets based upon Wi-Fi SSID |
| R133 -> LL | Access GW MUST be able to classify packets based upon MAC destination address |
| R134 -> LL | Access GW MUST have a configurable MAC destination address mask, so that classification is performed only upon bit fields within the MAC destination address determined by this destination address mask. |
| R135 -> LL | Access GW MUST be able to classify packets based upon IP destination address |
| R136 -> LL | Access GW MUST provide a configurable IP destination subnet mask, so that classification is performed only upon the network bit field within the IP destination address as determined by this destination subnet mask. |
| R137 -> LL | Access GW MUST be able to classify packets based upon IP source address |
| R138 -> LL | Access GW MUST provide a configurable IP source subnet mask, so that classification is performed only upon the network bit field within the IP source address as determined by this source subnet mask. |
| R139 -> LL | Access GW MUST be able to classify packets based upon DSCP. |
| R140 -> LL | Access GW MUST be able to classify packets based upon the Protocol field in the IP Header (e.g. ICMP, IGMP, TCP, UDP,…). |
| R141 -> LL | Access GW MUST be able to classify packets based upon source TCP/UDP port number or range of port numbers. Port numbers may be either statically configured or determined dynamically Within Access GW. This specification does not specify methods for dynamic determination of port number. This could be done, for example, by use of an application layer logic. |
| R142 -> LL | Access GW MUST be able to classify packets based upon destination TCP/UDP port number or range of port numbers. Port numbers may be either statically configured or determined dynamically within Access GW. This specification does not specify methods for dynamic determination of port number. This could be done, for example, by use of an application layer logic. |

# 6.2.2.1.3.1Requirements for Classification of packets received on the LAN ingress using information determined by DHCP Options 60, 61, and 77

| N° | Requirement |
| --- | --- |
| R146 -> LL | Access GW MUST be able to interpret DHCP option 60 messages received on LAN ports and associate a vendor class ID with a source IP address so that the source IP address can be used as a classification parameter. |
| R147 -> LL | Access GW MUST be able to interpret DHCP option 60 messages received on LAN ports and associate a vendor class ID with a source MAC address so that the source MAC address can be used as a classification parameter. |
| R148 -> LL | Access GW MUST be able to interpret DHCP option 61 messages received on LAN ports and associate a client identifier with a source IP address so that the source IP address can be used as a classification parameter. |
| R149 -> LL | Access GW MUST be able to interpret DHCP option 61 messages received on LAN ports and associate a client identifier with a source MAC address so that the source MAC address can be used as a classification parameter. |
| R150 -> LL | Access GW MUST be able to interpret DHCP option 77 messages received on LAN ports and associate a user class ID with a source IP address so that the source IP address can be used as a classification parameter. |
| R151 -> LL | Access GW MUST be able to interpret DHCP option 77 messages received on LAN ports and associate a user class ID with a source MAC address so that the source MAC address can be used as a classification parameter. |

# 6.2.2.3.3	Requirements for Classification Rule Sets

| N° | Requirement |
| --- | --- |
| R167 -> LL | Access GW MUST support a minimum of 32 concurrent rules for WAN_Rule_Set_1 |
| R168 -> LL | Access GW MUST support a minimum of 32 concurrent rules for LAN_Rule_Set_1 |
| R169 -> LL | Access GW MUST support a minimum of 16 concurrent rules for LAN_Rule_Set_2 |
| R170 -> LL | Access GW MUST support a minimum of 16 concurrent rules for LAN_Rule_Set_3 |

# 6.2.2.6	Dropping/Congestion Management (-> LL)

The congestion manager monitors the buffer resource consumption by tracking the depth of each class queue for each out-going port.  The congestion manager will either permit or deny the packet from being enqueued into the class queue based on the Random Early Discard algorithm.

The following requirements pertain to congestion management functions in Access GW.

| N° | Requirement |
| --- | --- |
| R179 | Access GW MUST support Random Early Discard (RED) for the upstream queues and the LAN egress queues |
| R180 | The operation of the RED function in either direction MUST be configurable from the ACS for each queue |
| R181 | Access GW MUST support the RED function to be disabled in either direction. |
| R182 | Access GW MUST allow independent configuration of the maximum threshold parameter for RED in each direction |
| R183 | Access GW MUST allow independent configuration of the minimum threshold parameter for RED in each direction |
| R184 | Access GW MUST allow independent configuration of the w_q (weighting factor) parameter for RED in each direction |
| R185 | Access GW MUST allow independent configuration of the maximum probability parameter for RED in each direction |

Editor’s Note – The above text is imported from clause 6.4.6 of HGI document.

# 6.6.1.2	Access Gateway Functionalities: block diagram

Figure 6-7 Access GW functional blocks ( -> LL)

Annex D

QoS operation in Home Network

# 6.2.2.5.1	Overall Mappings (Appendix)

# 6.2.2.5.1.1	Access GW Egress Markings (Appendix)

Table 6-1shows the correspondence between HGI service classes and the default DSCP or layer 2 markings applied by Access GW to packets as they are emitted onto the LAN side.

Table 6-1 Correspondence between Service Classes and HG Egress Markings

Editor’s Note – The above text is imported from clause 6.4.5.1.1 of HGI document.

# 6.2.2.5.2	Integrated Access Devices

The following requirements pertain to the use of priority markings for integrated wireless or powerline access devices within Access GW.

| N° | Requirement |
| --- | --- |
| R178 | If both layer 2 and layer 3 egress markings are used, the marking action associated with the classification rules SHOULD be configured so that layer 2 and layer 3 markings follow the correspondence with service classes as shown in Table 6-2. |

Editor’s Note – The above text is imported from clause 6.4.5.2 of HGI document.

# 6.2.2.7.3	Example of Queuing Configuration (Appendix)

Table 6-2 provides an example of a queuing configuration that may be configured in Access GW.

| Direction | Purpose | Scheduling into Port |
| --- | --- | --- |
| Upstream | Voice | Strict Priority (highest) |
| Upstream | Video | Strict Priority (next) |
| Upstream | Temporary Voice | W1 Weighted Round Robin |
| Upstream | Premium Data, GPRS Data, Game Data | W2 Weighted Round Robin |
| Upstream | Best Effort Data | W3 Weighted Round Robin |
| Downstream | Managed Services | Strict Priority (highest) |
| Transit | LAN streaming | Strict Priority (next) |
| Downstream | Best Effort Data | W2 Weighted Round Robin |

Table 6-2 Example of Queuing Configuration

Editor’s Note – The above text is imported from clause 6.4.7.3 of HGI document.

# 6.2.2.9	Bridging devices within IP-HN-S and IP-HN-P

Table 6-5 Correspondence between Service Classes and Infrastructure Device Markings ( -> Appendix)

Editor’s Note – The above text is imported from clause 6.4.9.2 of HGI document.

# 6.2.2.9.1	End devices (-> Appendix)

Table 6-6 shows the correspondence between service classes and the default DSCP and layer 2 markings applied to packets by end devices (IPTV-TD and HN-TD) within IP-HN-P and IP-HN-S.

Table 6-6 Correspondence between Service Classes and End Device Markings

Editor’s Note – The above text is imported from clause 6.4.9.3 of HGI document.

# 6.2.2.9.3.1	Informative notes on IPTV-TD and VoIP Client Devices (-> Appendix)

The following notes pertain to IPTV-TD deployed within the home network and not integrated within Access GW.

- The outgoing packets from IPTV-TD should have their DSCP bits configurable to conform to the “Upstream Video” category in.

- The default DSCP markings for outgoing packets from the IPTV-TD should conform to the “Upstream Video” category in Table x.

The following notes pertain to VoIP client devices deployed within the home network and not integrated within Access GW.

- The outgoing packets from the VoIP Client Device should have their DSCP bits configurable.

The default DSCP marking for outgoing voice packets from the VoIP client device should conform to the  “Upstream Voice” category in Table 6-6.

Editor’s Note – The above text is imported from clause 6.4.9.3.1 of HGI document.

Annex E

DHCP operation

## 3.9 Issue of 6.6.4.1	Support of routed model and extensions WAN side

This clause corresponds to clause 6.3.1(HGI)

Comment on R68, R69 and R70 – These requirements specify the DHCP options supported by a DHCP client residing in the Access Gateway (Home Gateway).  These options are summarised below.  Some of these options may not be necessary for IPTV service. Further study is needed.

Table-2 – A list of DHCP options specified for DHCP client in Access Gateway by HGI document

| Option code | Requirement (MUST/SHOULD/MAY) | Note/Comment |
| --- | --- | --- |
| 0 Padding | MAY | Not specific to IPTV |
| 1 Subnet Mask | MUST | Not specific to IPTV |
| 2 Timer Offset | MAY | Not specific to IPTV |
| 3 Default Gateway | MUST | Not specific to IPTV |
| 6 Domain Server | MUST | Not specific to IPTV |
| 7 SysLog Servers | SHOULD | Related to NW side function |
| 12 Host Name | MAY | Not specific to IPTV |
| 15 Domain Name | MAY | Not specific to IPTV |
| 33 Classful Static Route | MAY | Not specific to IPTV |
| 42 NTP Server | MAY | Related to NW side function. The distribution of time information to Access Gateway needs to be studied. |
| 43 Vendor Specific Information | MAY | Clarification on the use of this code is needed. |
| 50 Requested IP Address | MUST | Not specific to IPTV |
| 51 Lease Time | MUST | Not specific to IPTV |
| 53 DHCP Message Type | MAY | Not specific to IPTV |
| 54 DHCP Server Identifier | MAY | Not specific to IPTV |
| 55 Parameter Request List | MAY | Not specific to IPTV |
| 56 DHCP Notification Message | MAY | Not specific to IPTV |
| 58 Renewal time | MAY | Not specific to IPTV |
| 59 rebind time | MAY | Not specific to IPTV |
| 60 Vendor Class Identifier | MUST | Clarification on the use of this code is needed. |
| 61 Client Identifier | MUST | Clarification on the use of this code is needed. |
| 66 TFTP Server Name | MAY | Related to NW side function |
| 67 Boot Filename | MAY | Related to NW side function |
| 77 User Class | SHOULD | Not specific to IPTV |
| 121 Classless Static Route | MAY | Not specific to IPTV |
| 125 Vendor Specific Information | MUST | Clarification on the use of this code is needed. |
| 255 End Option | MAY | Not specific to IPTV |

## 3.11 Issue of 6.6.4.2.3	LAN-side DHCP requirements

This clause corresponds to clause 6.3.2.3(HGI)

Comment on R81 and R82 – These requirements state a local DHCP server at Access Gateway as mandatory.  The relationship NW side functionality needs to be studied.

R81: “A LAN side DHCP server MUST be available on the HG.”

R82: “The HG MUST support a means to remotely configure its own DHCP server”

Comment on R83 and R84 – These requirements specify the DHCP options supported by a DHCP server residing in the Access Gateway (Home Gateway).  These options are summarised below.  Some of these options may not be necessary for IPTV service. Further study is needed.

Table-3 – A list of DHCP options specified for local DHCP server in Access Gateway by HGI document

| Option code | Requirement (MUST/SHOULD/MAY) | Note/Comment |
| --- | --- | --- |
| 0 Padding | MUST | Not specific to IPTV |
| 1 Subnet Mask | MUST | Not specific to IPTV |
| 3 Default Gateway | MUST | Not specific to IPTV |
| 6 Domain Server | MUST | Not specific to IPTV |
| 12 Host Name | MUST | Not specific to IPTV |
| 15 Domain Name | MUST | Not specific to IPTV |
| 42 NTP Server | MUST | Related to NW side function. The distribution of time information to Access Gateway needs to be studied. |
| 43 Vendor Specific Information | MUST | Clarification on the use of this code is needed. |
| 50 Requested IP Address | MUST | Not specific to IPTV |
| 51 Lease Time | MUST | Not specific to IPTV |
| 53 DHCP Message Type | MUST | Not specific to IPTV |
| 54 DHCP Server Identifier | MUST | Not specific to IPTV |
| 55 Parameter Request List | MUST | Not specific to IPTV |
| 56 DHCP Notification Message | MUST | Not specific to IPTV |
| 58 Renewal time | MUST | Not specific to IPTV |
| 59 rebind time | MUST | Not specific to IPTV |
| 60 Vendor Class Identifier | MUST | Used for QoS management. (See R146 and R147) |
| 61 Client Identifier | MUST | Used for QoS management. (See R148 and R149) |
| 66 TFTP Server Name | SHOULD | Related to NW side function |
| 77 User Class | MUST | Used for QoS management. (See R150 and R151) |
| 125 Vendor Specific Information | MUST | Clarification on the use of this code is needed. |
| 255 End Option | MUST | Not specific to IPTV |

________________

This specification does not specify methods for dynamic determination of port number. This could be done, for example, by use of an ALG.