| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-DOC-0189 |  |
|  |  |  | English only |  |
| WG(s): 4 |  | 7th FG IPTV meeting: Qawra, St Paul’s Bay, Malta, 11-18 December 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Editors |  |  |  |
| Title: | IPTV Network Control Aspects |  |  |  |

Summary

This document describes various aspects of IPTV network control. It provides a list of detailed requirements to address control and signalling matters related to authentication and authorization, content delivery, consumer domain attachment and initialization, quality of service (QoS), quality of experience (QoE) and security.

Keywords

IPTV, overlay service, overlay network, QoS, QoE

Current status

After carefully reviewing and extensive editing works, this document contains consensus of information from all the contributions addressed to the working group. It mainly addresses the detailed functional requirements and implementation guidelines of the following network control aspects:

- Network control and signalling

- Content delivery network

- IPTV consumer domain attachment and initialization

- Identification

- IPTV inter-working

- Overlay networks

In terms of IPTV network control and signalling inter-working, this document provides a list of items for further study. The list serves as a starting point and is not complete. Further study is needed in the following areas:

- Identifying the complete list of inter-working aspects in the context of IPTV network control

- Providing detailed guidelines for inter-working aspects related to  IPTV network control

Dependency or relationship to other FG IPTV documents:

- IPTV services requirements

- IPTV architecture

- IPTV multicast frameworks

- Aspects of  IPTV end system – terminal device

- Aspects of home network supporting IPTV services

- IPTV Middleware, Applications, and Content Platforms

CONTENTS

1	Scope	5

2	References	5

3	Definitions	6

3.1	Terms defined elsewhere	6

3.2	Terms defined in this document	6

4	Abbreviations and Acronyms	6

5	Conventions	7

6	Framework of IPTV Network Control Aspects	7

7	Control and Signalling Aspects	8

7.1	Network Control	8

7.1.1	Unicast Network Control	8

7.1.2	Multicast Network Control	8

7.1.2.1	Multicast IP Address Management	9

7.1.2.2	Multicast User Management	9

7.1.2.3	Multicast Session Identifier Management	9

7.1.2.4	Source Specific Multicast	9

7.1.3	Connection Admission Control	9

7.2	IPTV Network High Availability	10

7.2.1	IPTV Network Service Failure Recovery	10

7.2.1.1	Redundant System Architecture and Topology	10

7.2.1.2	Robust Network against Attacks	10

7.3	Linear TV Control	10

7.4	Parental Control	11

7.5	Network Traffic Identification	11

7.6	Session Control	11

7.7	Stream Control	11

8	Content Delivery Network Aspects	11

8.1	Content Delivery Network Topology	12

8.2	CDN-based IPTV Media Delivery Mechanism	12

8.3	Distributed Content Storage/Cache and Content Serving	13

8.4	Centralized Content Location Management	13

8.5	Distributed Content Delivery	14

8.5.1	Distributed Edge Server Networks	14

8.6	Content Segmentation	14

8.7	Statistical Performance of Content Delivery Network	15

8.8	Content Delivery Protocols	15

9	IPTV Consumer Domain Attachment and Initialization	15

10	Identification Aspects	16

11	IPTV Inter-working	17

12	Overlay Network Aspects	17

12.1	Control Function in IPTV Overlay Network	18

12.2	Multicast Function in IPTV Overlay Network	18

12.3	Session Manager in IPTV Overlay Network	18

12.4	Manageable Overlay Network	18

13	Other Aspects	19

Appendix I: Example of IPTV Consumer Domain Attachment and Initialization	20

Appendix II: Example of Multicast VPN	31

Appendix III: Mapping between IPTV Overlay Network Capability and IPTV functional architecture	33

Appendix IV: ISP-manageable P2P Network Structure	36

IPTV network control aspects

# Scope

This document describes various aspects of IPTV network control. It provides a list of detailed requirements to address control and signalling matters related to authentication and authorization, content delivery, consumer domain attachment and initialization, quality of service (QoS), quality of experience (QoE) and security.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this working document. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this working document are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this working document does not give it, as a stand-alone document, the status of a Recommendation.

[FG IPTV-DOC-0147] 	ITU-T FG IPTV document IPTV Services Requirements

[FG IPTV-DOC-0181] 	ITU-T FG IPTV document IPTV Architecture

[FG IPTV-DOC-0190] 	ITU-T FG IPTV document IPTV Multicast Frameworks

[ITU-T Y.2111]	ITU-T Recommendation Y.2111 (2006), Resource and Admission Control Functions in Next Generation Networks

[IETF RFC 2236] 		IETF RFC 2236(1997), Internet Group Management Protocol, Version 2

[IETF RFC 2710] 		IETF RFC 2710 (1999), Multicast Listener Discovery (MLD) for IPv6

[IETF RFC 3376] 		IETF RFC 3376(2002), Internet Group Management Protocol, Version 3

[IETF RFC 3810] 		IETF RFC 3810(2004), Multicast Listener Discovery Version 2 (MLDv2) for Ipv6

[IETF RFC 4541] 		IETF RFC 4541(2006), Considerations for Internet Group Management Protocol (IGMP) and Multicast Listener Discovery (MLD) Snooping Switches.

[IETF RFC 4601] 		IETF RFC 4601(2006), Protocol Independent Multicast – Sparse Mode (PIM-SM)

[IETF RFC 4604] 		IETF RFC 4604(2006), Using Internet Group Management Protocol Version 3 (IGMPv3) and Multicast Listener Discovery Protocol Version 2 (MLDv2) for Source-Specific Multicast.

[IETF RFC 4605] 		IETF RFC 4605(2006), Group Management Protocol (IGMP) / Multicast Listener Discovery (MLD)-Based Multicast Forwarding (“IGMP/MLD Proxying”).

# Definitions

## 3.1	Terms defined elsewhere

This document uses the following terms defined elsewhere:

    - Channel zapping: The act of fast changing from one channel to another.

    - Connection admission control

The mechanism that controls the traffic to be admitted into the network.

    - Electronic program guide: A structured set of data, intended to provide information on available content that may be accessed by end-users.

    - End-user [ITU-T J.112]: A human being, organization, or telecommunications system that accesses the network in order to communicate via the services provided by the network

    - IPTV terminal function: The functionality that is responsible for processing the content conveyed by the IP transport.

    - Linear TV: A television service in which a continuous stream flows in real time from the service provider to the terminal device and where the user cannot control the temporal order in which contents are viewed.

    - Video on demand: A service in which the subscriber can view video content whenever desired. The operating assumption is that the content is stored on the provider’s VoD server. Subscriber accesses the movie from a library directory which may include search engine that accesses movie description and rating.  Subscribers typically have the ability to pause, play, rewind, fast forward the content, or even stop viewing it and return to it at a later time when using this service.

## 3.2	Terms defined in this document

This document defines the following terms:

    - Overlay network: A virtual network of nodes and logical links that is built on top of the underlying network infrastructure with the purpose of implement a network service that is not available in the underlying network infrastructure.

    - Overlay multicast network: One type of overlay network that provides multicast services to end users on top of the general network infrastructure.

    - Parental control: Parental Control provides a mechanism for deciding the suitability of particular content for a minor on his/her age.

# Abbreviations and Acronyms

This document uses the following abbreviations.

ASM					Any Source Multicast

CAC					Connection Admission Control

CDN 					Content delivery network

CRID		Content Reference Identifier

DHT		Distributed Hash Table

DoS					Denial of Service

EPG					Electronic Program Guide

IGMP		Internet Group Management Protocol

ISP   		Internet Service Provider

ITF		IPTV Terminating Function

P2P		Peer-to-Peer

QoE		Quality of Experience

QoS		Quality of Service

SNI		Service Node Interface

SSM		Source Specific Multicast

VoD		Video on Demand

VPN		Virtual Private Network

# Conventions

In this document:

The keywords “is required to” indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

The keywords “is prohibited from” indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

The keywords “is recommended” indicate a requirement which is recommended but which is not absolutely required.  Thus this requirement need not be present to claim conformance.

The keywords “is not recommended” indicate a requirement which is not recommended but which is not specifically prohibited.  Thus, conformance with this specification can still be claimed even if this requirement is present.

The keywords “can optionally” indicate an optional requirement which is permissible, without implying any sense of being recommended.  This term is not intended to imply that the vendor’s implementation must provide the option and the feature can be optionally enabled by the network operator/service provider.  Rather, it means the vendor may optionally provide the feature and still claim conformance with the specification.

# Framework of IPTV Network Control Aspects

Note: The terminology and the architecture assumed in this document needs to be checked for alignment with IPTV architecture document.

This clause provides a general overview of IPTV network control aspects, as shown in Figure 6-1. There are many views of addressing control aspects of IPTV networks. This overall framework contains functional components, such as Service Control Functions, Content Delivery Functions and Network Functions, related to IPTV network control aspects and is aligned with IPTV Functional Architecture Framework.

Figure 6-1: Overview of IPTV network control

# Control and Signalling Aspects

## Network Control

### Unicast Network Control

For Further Study

### Multicast Network Control

Multicast network transport is one of the major drivers to promote IPTV service in telecommunications networks. IP multicast communication can improve the efficiency of data transmission and make efficient use of network bandwidth resources when delivering broadcast content. Therefore, multicast technology and its control functions are important in the network to deliver IPTV service.

The IPTV architecture is recommended to support alternative multicast schemes such as content delivery network (CDN), overlay multicast, peer-to-peer (P2P) etc., with control of each multicast scheme. It is required to have a compatible capability of managing different multicast schemes.

The IPTV architecture is recommended to support static configuration of the IP multicast distribution tree, and to support well designed dynamic IP multicast protocol to reduce traffic for IPTV service.

IPTV multicast network control is required to support the multicast control function and multicast replication function which provide IPTV multicast services for users. Multicast control function builds a privilege table for multicast users according to the multicast group address, so that when users receive IPTV multicast services, multicast network control deals with IPTV multicast services according to users' multicast privilege. The multicast replication function forwards multicast media content to users which have the privilege of IPTV multicast services. That is to say, IPTV multicast network control will forward the IPTV service contents to a user only if the user has multicast privilege.

The IPTV architecture is recommended to support multicast in access network and multicast protocol proxy in access nodes or other network nodes where multicast traffic is replicated to users.

#### Multicast IP Address Management

- The IPTV architecture is recommended to support mechanisms to manage multicast IP address to ensure the deployment of IPTV service successfully.

- The IPTV architecture is recommended to support Multicast IP Address Transition for Cross-domain IPTV Service.

- The IPTV architecture is recommended to support the control of end-users’ multicast address for user multicasting.

#### Multicast User Management

- The IPTV architecture is recommended to support multicast user authentication function for multicast services of IPTV.

- The IPTV architecture is recommended to support configuring users’ corresponding multicast privileges according to the multicast group address.

- The IPTV architecture is recommended to support configuring the relationship between users and their multicast privilege.

#### Multicast Session Identifier Management

Because there can be various multicast schemes, it is required to describe multicast session uniquely. For example, a multicast session using pure IP multicast can be described by IP multicast address but a multicast session using CDN can be described by URL. IPTV is recommended to manage identifier to describe a specific multicast session uniquely.

#### Source Specific Multicast

Source Specific Multicast (SSM) reduces the overall complexity within IPTV. SSM is also optimized for One-to-Many deployment scenarios which are typically used within IPTV. Any Source Multicast (ASM) and SSM can be used in parallel (deploying different address ranges) if necessary. The use of SSM eases the deployment of IPTV and reduces the complexity, allowing more flexible IPTV services.

### Connection Admission Control

- It is required that Connection Admission Control (CAC) is supported in the access network based on available resources. When the end user subscribes to a multicast stream, the access network will perform CAC to check if the current available resources are enough for the new service subscription. The resources can be bandwidth, connection number and user service privilege profile.

For the reason that some IPTV service streams are mixed in the access network,

- The IPTV architecture is recommended to support the QoS class queues for each IPTV service stream.

- The IPTV architecture is recommended to support queue control functions to guarantee quality for IPTV service streams in the access network.

- The IPTV architecture is recommended to support bandwidth allocation function for each IPTV service stream in the access network. For reference, bandwidth allocation function  can be operated based on the User ID, Service Stream ID, and Bandwidth which includes the whole bandwidth of the access line, minimum guaranteed bandwidth and maximum guaranteed bandwidth.

## IPTV Network High Availability

For general network high availability service requirements, please refer to [FG IPTV-DOC-0147] sub-clause 6.4.

### IPTV Network Service Failure Recovery

- The IPTV network is recommended to provide the capability for ensuring sufficient availability of IPTV services.

#### Redundant System Architecture and Topology

- The IPTV network is recommended to be a fully redundant system architecture and topology such that a single point of failure is recommended to not affect the whole network.

#### Robust Network against Attacks

- The IPTV network is recommended to be designed to be robust such that it can tolerate unexpected attacks such as denial of service (DoS) attack.

## Linear TV Control

Channel zapping protocol is the interactions for and between the Service Stratum and the Transport Stratum in terms of channel selection.

Multicast control of Linear TV is recommended to include but not limit to the followings:

- Channel Access Control: Channel Access Control of Linear TV is recommended to support some access right statuses of the entitlement to each of the broadcast channels for each user, such as fully allowed, preview allowed, not allowed.

- Channel Preview Capability: Channel Preview Capability of Linear TV is recommended to support some preview control functions of the Linear TV channel, such as Maximum duration for each preview, Maximum times of previews, Blackout duration after each preview, Reset period of channel preview, Recognition Time of channel preview, etc.

- Call Detail Record: Call Detail Record is recommended to report the entitlement status of the channel being access, and records the channel access activities of each customer should be generated automatically.

- Priority of Linear and other traffic: Linear TV is recommended to support capabilities for classifying channel priorities such as based on bandwidth and multicast resource reserved for programmes, etc. Channel audience rating statistics could be obtained by policy server based on end-user multicast behaviour information collected from access nodes and used to determine and provision subscriber channels priority in the access node for channels difference disposal. The end-user multicast behaviour information can be optionally to include programme item number, start viewing time, stop viewing time. In order to guarantee Linear TV’s quality and service provisioning, Linear TV is recommended to be of strict priority than other traffic and no impact from data traffic during the transmission.

- Service Management System: Service management system is recommended to support some service related management functions of Linear TV channel, including user management, policy management, SP/CP management, accounting management, service customization management, security management, performance management, portal management, STB management.

## Parental Control

Parental controls have been typically included in digital television services, video games etc. that allow parents to monitor or limit what their children can see or do. For the IPTV architecture, parental control mechanism can be used to restrict IPTV contents that children can receive. Policy control and management for parental control is recommended to support the network capability to store and forward the requested IPTV services and its rating to specified terminals (e.g., monitoring terminal for parent) based on preconfigured control policy and user profiles. The policies are varieties include time limiting, content rating, etc.

## Network Traffic Identification

Identification mechanism of Network traffic can optionally be required over the transport network to check traffic status when necessary. Traffic flow traversing IPTV transport network could be satisfied the demand of IPTV services. The use of identification mechanism makes it possible to manage properly according to operating policies. It can also be used to prevent the network intrusion so that to protect IPTV services.

## Session Control

- The stream session control protocol is recommended to support carrying authentication/authorization token, carrying message signature and cross server session.

## Stream Control

- The stream transport protocol is recommended to support representation of position in frame, frame awareness, frame awareness for trick modes, physical server aware client.

# Content Delivery Network Aspects

- IPTV content delivery networks are required to support non-real time and real-time IPTV services such as Linear TV, Video on Demand (VoD) and Electronic Program Guide (EPG) content delivery, system information, ad-insertion (e.g. SCTE 35); to provide IPTV capabilities such as content protection, closed captioning, content transport leveraging and content segmentation; and to define CDN components to record and provide performance statistic. Content Delivery Network related functions are indicated in Figure 8-1 and aligned with IPTV Functional Architecture Framework.

- Figure 8-1: IPTV Content Delivery Functions

## Content Delivery Network Topology

- The IPTV content delivery network is recommended to be layered. Each layer may have several serving nodes that flexibly construct a content delivery network. The serving node may be constructed by a node controller and several streaming servers. The node controller provides the load balancing among the streaming servers in the serving node, and adjusts the deployment of the contents between serving nodes. The streaming server is required to provide streaming service in real-time to users under the control of the node controller.

- A distributed structure is recommended to be adopted in the content delivery network layer. The serving nodes in the content delivery network layer are optionally required to communicate with each other to exchange the contents.

## CDN-based IPTV Media Delivery Mechanism

- In order to ensure manageability and interoperability of CDN-based IPTV content delivery network, the CDN-based IPTV media delivery mechanism is recommended to include: Media Delivery Manager, Media Delivery Agent and Streaming media server.

The Media Delivery Manager, which is responsible for global load balancing and interfacing with other subsystems, supports serving control function, content control function and operation maintenance function.

The Media Delivery Agent, which is responsible for local load balancing, is attached to every streaming media node and supports serving control function, content control function and operation maintenance function for the corresponding node.

The Streaming Media Server, which is responsible for storing content and streaming media, supports streaming function, content storage function, content delivery function, and operation maintenance function.

Server-based IPTV media delivery mechanism and CDN-based IPTV media delivery mechanism are described in [FG IPTV-DOC-0190]. In server-based IPTV media delivery mechanism, ISP puts a media server or server pool somewhere; each member connects media server or server pool; then the media server sends data to every receiver. In CDN-based IPTV media delivery mechanism, ISP pre-installs CDN servers in an appropriate place; each receiver finds and then connects the nearest CDN server. Then multimedia streams from source are distributed to each receiver along the data delivery path of CDN servers.

## Distributed Content Storage/Cache and Content Serving

  - Distributed Content Storage/Cache

The content delivery network can store, cache, distribute and deliver IPTV media contents according to a distributed content delivery mechanism. The distributed content delivery mechanism enables service providers to store IPTV media contents on storage/cache nodes. The content storage/cache nodes are optionally required to be deployed according to their physical network location and their subscriber coverage. The distributed content storage/cache network enables service providers to dynamically distribute IPTV media contents according to pre-defined delivery policies.

  - Distributed Content Serving

Content serving in distributed content delivery network is implemented by independent content serving nodes. The content delivery control node can arrange an appropriate content serving node to provide IPTV streaming service for each end user according to the load of each content serving node and the state of content distributed in each content storage/cache node.

## Centralized Content Location Management

- A centralized content location management module is recommended to be responsible for keeping track of the locations of all the content in the IPTV network. It maintains a record of network topology, a content location, and a content delivery session.

The network topology describes the topology of the media servers and the network topology of the media servers. Media content is copied from one location to another by creating content delivery sessions between two media servers in the both locations. These sessions are created and deleted as the copy process starts and finishes respectively. This information is used by the content location management module to keep track of the bandwidth usage among the media servers and the central site.

- The characteristics of the centralized content location management are as follows.

- Tracking each content program location in a distributed media servers.

- Tracking the delivery sessions among media servers.

- Keeping statistical and historic data of content delivery and copy sessions.

## Distributed Content Delivery

- The IPTV architecture is recommended to support a distributed architecture among content edge servers.

- Co-located edge media servers are recommended to have the ability to be organized in a distributed structure to share the load, to improve the reliability and to reduce the total cost of the edge servers.

- According to the method of the distributed structure, original files or content can be stored evenly on the edge servers. User’s requests can be redirected to the most suitable edge server automatically without the help of a central device.

### Distributed Edge Server Networks

Distributed architecture is used to organize all local edge server networks to form distributed edge server networks. That is, distributed architecture among all local edge server networks is recommended to be supportable such that the burden of both serving users and content storage can be evenly distributed on all local edge server networks.

- Content Locating

Some automatic content locating approach is used in the local edge server network to find a suitable edge server for the user. If not found, the user’s request will be routed among all local edge server networks (by using automatic content locating method again) to locate a suitable local edge server network containing edge server with desired content.

- Server functions

Each edge server in local edge server network is recommended to support automatic content locating methods for distributed structure. Each local edge server network is recommended to support automatic content locating methods for distributed structure. Automatic content locating capability is recommended to be supported by some agent server in each local edge server network, i.e. the agent server acts as the content locating agent of the corresponding local edge server network.

In Distributed Hash Table (DHT), edge server calculates and distributes content ID and content information to edge server in its local edge server network. Agent server calculates and distributes content ID and content information to another agent server which directly distributes content ID and content information to edge server in that edge server network. Content information is searched among edge servers in requestor’s local network. If not found, some agent server is found to be responsible for the content information. That agent server directly searches content information in that edge server network by locating some edge server storing the required content information.

## Content Segmentation

- Content Segmentation can optionally be required in some IPTV deployments. IPTV Content Segmentation functions are recommended to generate segment at codec frame boundaries; to realize codec format (e.g. MPEG-2, MPEG-4, AVS, H.264 and VC-1); to apply to both clear-text and encrypted contents; and to support the quick identification of any frame within the segment and the entire program file.

## Statistical Performance of Content Delivery Network

Statistical performance of Content Delivery network mainly includes the content storage server, the content serving server and the content delivery control function.

  - The content storage server - manages content stored in both memory and persistent storage according to policy, i.e. content swapping, content deletion etc.

The content storage server records the number of requests satisfied by pushed contents, requests satisfied by pulled contents, requests satisfied by pulling remote contents, and requests satisfied by local contents. The content storage also records requests for remote contents, content push sessions and content pull sessions. The ratio between number of requests satisfied by pushed contents and the number of content push sessions indicates the effectiveness of content push. The ratio between number of requests satisfied by pulled contents and the number of content push sessions indicates the effectiveness of content pull.

  - The content serving server - transmits requested content to end system in designated forms, or redirect request to other functionalities.

The content serving server records the number of received requests for contents, served bytes and unsatisfied requests. The number of received requests for contents indicates the amount of load on the node. The number of served bytes indicates the number of bytes actually consumed by users – the effective bytes. Sum of bytes for pushed contents and bytes for pulled contents indicates the number of bytes made available to users. The ratio between the effective bytes and the available bytes indicates effectiveness of content push and pull.

  - Content delivery control function - triggers and controls content transmission among content storage server, designate content, source, destination, speed of the transmission.

## Content Delivery Protocols

- The typical initial content delivery is through content pushing.  The content is pushed from central media server to edge media servers according to the policy, schedule and delivery topology provided by centralized content location management.

- The content delivery protocols are recommended to support the following.

- Flexible policies for content delivery

- Scheduled pushing of content to delivery servers

- Quickly identifying a source of the content and starting to copy

- The dynamic pulling of content by the delivery servers

- Multiple sources for content pulling.

# IPTV Consumer Domain Attachment and Initialization

The IPTV device attachment and initialization process can be broken down into the following steps: Setup & Configuration, Network Attachment, Service Provider Discovery, and Services discovery & Service attachment.

  - Setup & Configuration

Setup & Configuration activities relate to acquiring and configuring a physical IPTV Terminating Function (ITF) device. This step establishes the identity and authentication credentials for the ITF, and service provider identity and any additional information for one or more service providers.

  - Network Attachment

Network Attachment refers to the activities associated with the ITF establishing Layer-3 connectivity to an IP-network and potentially obtaining additional network configuration data. During the network attachment phase, some information, such as IPv4 address, network mask, default route, DNS, local network domain name, SIP proxies, others…, may be obtained.

  - Service Provider Discovery

Service Provider discovery is the process by which an ITF becomes aware of the available IPTV Service Providers, learns the location of their Service Discovery (SD) Servers, and learns the means for attaching to each SD server.  As a result, by contacting the discovered SD Server(s), an ITF can perform the subsequent Services Discovery and Service Attachment procedures.

  - Service Discovery & Service Attachment

Services Discovery is the process by which an ITF receives the necessary signalling information which prepares it to learn about and access the available IPTV Services.  The ITF interacts with one or more Service Provider Servers (discovered earlier) in order to acquire information about specific services.

The Services Discovery function includes both the ‘services discovery authorization’ means and the ‘services description’. After authorized by a Service Provider SD Server, the consumer needs to be able to navigate through the variety of service offerings and then attach to a specific service.

For implementation examples, please refer to Appendix I.

# Identification Aspects

It is necessary to provide identification mechanism to identify IPTV users, customers and subscribers to support IPTV services, which could be used in various players such as customer, service providers, network providers and content providers.

The IPTV network controls, manages and makes provision to optimize the service for various types of resources.  In order to realize service optimization, the IPTV network can optionally utilize IPTV Identifiers such as Content ID, Content Provider ID, Service Provider ID, Network provider ID and User ID.

- Content ID (CID) - unique identifier of Content.

- Content Provider ID (CPID) - unique identifier of Content Provider.

- Service Provider ID (SPID) - unique identifier of Service Provider.

- Network Provider ID (NPID) – unique identifier of Network Provider.

- User ID (UID) – unique identifier of User or Subscriber.

- The IPTV network uses content identifier to specify content with different granularity, and some locators to locate content among CDN to optimize the storage and serving. The IPTV network can optionally utilize the following IDs and Locators to specify content.

- Persistent Content ID – specifies content with persistent and globally interoperable identifier.  It is used for transaction between service provider and content provider, or between service providers.

- Content Referencing ID – specifies content which is provisioned for service, and it is issued by a service provider.  It may be not persistent and may be valid only in the service provider domain.  It is used for transaction between a service provider and a user/subscriber.

- Logical Locator – specifies content in the context of service, time, place and resource. It is used for requesting a service to the IPTV network.  It may include the protocol and service specific information.

- Physical Locator – specifies content in the context of physical IP network resource.  It is used for establishing and routing of a delivery connection to the IPTV network. It may include the protocol, IP address and port number.

- Internal Locator – specifies content in the context of internal delivery platform resource of a service provider.  It is used for allocation and management of internal server resources of delivery platform.  Especially, when servers share a single IP address, it is used for load balancing.

# IPTV Inter-working

Notes: The following IPTV inter-working aspects need further study. Identified aspects are based on the contributions to this working group. The complete list of IPTV inter-working items needs further study also.

Inter-working among Non-NGN, NGN, IMS-based NGN

- Network level inter-working and service-level inter-working

- Inter-working between homogeneous IPTV service providers or between heterogeneous IPTV service providers, etc. with views of administrative domain.

More technically even though all these aspects are in question,

- Inter-working between unicast channel and multicast channel

- Inter-working among protocol, signalling, routing, QoS, addressing, naming, and etc.

- Inter-working between different routing algorithms for IPTV services

# Overlay Network Aspects

IPTV overlay network is responsible for forwarding and handling of IPTV application data in ways that are different form. And overlay network is operated in organized and coherent way by the third party service provider or network provider to provide IPTV services.

IPTV overlay network consists of virtual network topologies on top of the physical network, as shown in figure 16-1, which directly interfaces to users. With the rapid advancement of internet and computing technology, much more aggregate information and computing resources are available from clients or peers than from a limited number of centralized servers.

Figure 12-1: Overlay Service and Network for IPTV

## Control Function in IPTV Overlay Network

In order to provide overlay session control in overlay networks for IPTV service, IPTV service control function is necessary to perform session control and management for IPTV overlay network, to establish and to maintain the network and system resources.

## Multicast Function in IPTV Overlay Network

The overlay IPTV multicast can use diverse mechanisms for constructing different multicast trees depending on IPTV application parameters or application classes. The overlay IPTV multicast supports efficient routing and resource usage by overlay multicast control.

In order to provide scalability for multicast function, hierarchical structure for overlay multicast will be introduced in IPTV overlay networks.

## Session Manager in IPTV Overlay Network

In the IPTV Overlay Network, session manager is the process of keeping track of session configuration and maintenance for IPTV service, and provide session initiation, release and management.

## Manageable Overlay Network

In P2P-based IPTV service delivery scheme, each peer can be both a media producer as well as a media consumer. Although peer can exchange media between themselves and the size of peers group is not logically bounded, P2P model does not want to have ISP’s censorship.

In order to achieve manageable P2P network, ISP is recommended to involve in some management functions of P2P. With these management functions, ISP can tightly manage the P2P network.

The advantages of manageable P2P model are:

  - ISP can tightly manage content delivery network by involving in the management of P2P.

  - The number of peers is not bounded by the capability of media server.

Manageable P2P network architecture principles:

    - Peers can be classified into some groups.

    - Each group has a Management Node (MN) deployed by ISP to manage peers in this group.

    - Peers exchange IPTV media contents under the management of MN.

    - Group information (e.g. meta-data of media) could be exchanged among groups.

Please refer to Appendix IV for manageable P2P examples.

Notes: Mapping manageable overlay network to IPTV functional architecture needs for further study.

# Other Aspects

Multicast VPN

IPTV multicast VPN service may accelerate IPTV applications as the worldwide demand for VPN services grows. In the meantime, IPTV multicast VPN also may enrich IPTV applications, such as classified IPTV service features according to geographical groups and customers’ demand, differentiated IPTV service features in security and QoS for communities or groups, classified IPTV group service features, and personalized IPTV service capabilities on IPTV service provider’s network.

Please refer to Appendix II for Multicast VPN examples.

Various home, access and core transport scenarios for multicasting

- The DSL technology is one of the access networks as the underlying transport network to support IPTV services. The DSL technology is concerned with the networking architecture(s) superimposed over DSL transport to provide IPTV services.

- Appendix I: Example of IPTV Consumer Domain Attachment and Initialization

（This appendix does not form an integral part of this document）

I.1	 IPTV Device Initialization and Attachment Flow

Figure I-1 is an example of an IPTV device Initialization and Attachment based on DHCP.

Figure I-1: IPTV Device Initialization and Attachment flow based on DHCP

Figure I-2 is an example of an IPTV device Initialization and Attachment based on PPPOE.

Figure I-2: IPTV Device Initialization and Attachment flow based on PPPOE

The following two sub-sections attached here only for reference purpose.

I.1.1 	Network Attachment

I.1.1.1 	Delivery Network Gateway (DNG)

The term ‘Network Attachment’ refers to the activities associated with the DNG establishing Layer-3 connectivity to an IP-network. Once the network attachment activities have been completed, the DNG will be capable of transmitting and receiving IP packets, and establishing a management session with a remote configuration server. Subsequently the remote configuration server, such as an auto-configuration server (ACS) defined in TR-69, can be used to provision the DNG with access-network specific parameters.

I.1.1.1.1	 DHCP Approach1

The most prevalent automatic method currently used to attach to the network is the DHCP protocol. Using this protocol, the DNG acquires its WAN IP address, and can also learn the address of the remote configuration server.

Figure I-3 below shows the sequence of events that take place when the DNG is powered up or initialized.

Figure I-3: DNG Network Attachment Approach 1

Specifically, the following procedure takes place:

1) The DNG establishes L1 and L2 connectivity with the network. The specific steps depend on the type of the access network (DSL, PON, HFC, Wireless, etc) and are beyond the scope of this document.

2) The DNG issues a DHCP DISCOVER message toward the network that must include all the mandatory DHCP options, per RFC 2131. If the DNG device were to support the Container option, it would request it in option 55 (parameter request list). This would be performed in step 2 in the figure above.

DHCP DISCOVERY message can optionally include Rapid Commit Option per RFC4039 to obtain IP address and configuration information using a 2-message exchange rather than the usual 4-message exchange, expediting the TD configuration.

3) The network provider DHCP server replies to the DNG with a DHCP OFFER message that must include all the DHCP mandatory options, per RFC 2131. The DHCP server must provide the following information to the DNG:

a) DNG WAN IP address

b) DNG network mask

c) DNS Server IP address (es)

d) The Container option requested in step 2 above, if configured, to convey other options to the TD.

e) The remote configuration server addressing information (see possibilities below), if configured.

f) If the DHCP DISCOVERY message contains a Rapid Commit Option, the DHCP server responds with a DHCP ACK message including IP address and configuration information not sending DHCP OFFER.

There are three possibilities related to the siaddr at this point, reflected in the content of the DHCP OFFER message:

i) If the DHCP server is provisioned with the remote configuration server IP address, then it must include it in the siaddr field of the OFFER message. This is shown as step 3-a in the figure above.

Ⅱ) If the DHCP server is provisioned with the remote configuration server domain name but not its IP address, then it must set siaddr = 0.0.0.0 and provide the remote configuration server domain name to the DNG. The DNG must interact with the DNS to resolve the remote configuration server IP address. This is shown as step 3-b in the figure above,

Ⅱi) If the DHCP server is not provisioned with the remote configuration server addressing information (IP address or domain name) and therefore does not provide the remote configuration server IP address or its domain name, then there must be a mechanism in place to allow the end user to manually configure the DNG. This is shown as step 3-c in the figure above.

4) The DHCP protocol sequence completes with the standard DHCP REQUEST and DHCP ACK messages.

After acquiring the IP address of the remote configuration server, the DNG interacts with the remote configuration server to acquire needed configuration parameters.

I.1.1.1.2	 DHCP Approach2

Another approach of network attachment is supported by DHCP as shown in Figure I-4.

Figure I-4: DNG Network Attachment Approach 2

The flow for IPTV DNG using DHCP to attach the network is following:

  - When DNG is powered on, DNG check whether are there user name and password for application layer in it’s store device, and the user name and password are wanted if they are not stored in it’s store device;

  - DNG encapsulates the user name and password in options 60 of DHCP request and broadcasts the request message;

  - The DCHP server platform including DHCP server and authentication server authenticate the validity of the user name and password when the DHCP request message with options 60 received. If the authentication passes, then the DHCP OFFER message with IP address assigned by the DHCP server is delivered to DNG; and if the authentication fails, then the DCHP request message will be dropped.

  - After receiving the DCHP OFFER message, DNG issues a DHCP request message (including the random number R for encryption, the time tamp TS created recently) again with encapsulation the user name and password to the  DCHP server using the IP address assigned in (3);

(5)  The DCHP platform echo ACK to DNG. Then the process of DNG acquire IP address finishes.

I.1.1.1.3 	PPPOE

PPPoE is a prevalent method to attach the access network. The DNG can attach the access network using the PPPoE. After the process of the PPPoE is complete, the DNG can get the IP address assigned by the ACCESS PLATFORM, and also can acquire the IP address of the remote management server.

Figure I-5 below shows the sequence of events that take place when the DNG is powered up or initialized.

Figure I-5: DNG Network Attachment through PPPoE

The flow for DNG using PPPoE to attach the network is following:

- The DNG establishes L1 and L2 connectivity with the network. The specific steps depend on the type of the access network (DSL, PON, HFC, Wireless, etc) and are beyond the scope of this document.

- The DNG broadcasts a PADI message toward the network;

- The network ACCESS PLATFORM replies to the DNG with a PADO message:

Above is the discovery part of the PPPoE;

- The DNG issues a PADR message with the user name and the password to a ACCESS PLATFORM to get the PPP session, this is the LCP process;

- The ACCESS PLATFORM authenticate the user’s validity

- The ACCESS PLATFORM echo the DNG with the assigned IP address and DNS address if the authentication successful.

After acquiring the IP address of the remote configuration server, the DNG interacts with the remote configuration server to acquire needed configuration parameters.

I.1.1.2 	IPTV Terminal Device (TD)

During the network attachment phase the following information may be obtained or otherwise established by the TD:

1) IPv4 address, network mask, default route

2) DNS, local network domain name, SIP proxies, others…

Once network attachment activities have been completed, the TD will be capable of transmitting and receiving IP packets and establishing a management session with a remote configuration server. Among existing mechanisms for implementing the Network Attachment function and relevant are:

1) The Dynamic Host Configuration Protocol (DHCP)

2) NGN NACF

The profiles and the details for each of the mechanisms as applicable for various IPTV use cases are described in the following sections.I.1.1.2.1 DHCP

There are two cases to consider. The first case is when the DNG supports DHCP server capability and thus acts as the network DHCP server for the TDs on the home network. The second case is when the DNG passes DHCP messages through as a DHCP relay agent.

Note that while the second case can still be found in today’s deployments, home networks are moving towards self-contained IP domains, with the network DHCP server located in the DNG.

I.1.1.2.1.1 	Local DHCP Server on the DNG

In this case it is assumed that the DNG has already attached to the network and acquired its L3 networking information. The following sequence takes place when an TD device is powered up or initialized:

1) The TD establishes L1 and L2 connectivity with the DNG across the home network, which could be over a copper network, a coaxial network, fibre/plastic network, or wireless network.

2) The TD issues a DHCP DISCOVER message toward the DNG that must include all the mandatory DHCP options, per RFC 2131. The TD must include option 60 to allow it to identify itself to the DNG.  If the TD were to support the option(s) carried in the Container option, it would request those options in option 55 (parameter request list).

3) The DNG, as a DHCP server, replies to the TD with a DHCP OFFER message that must include all the DHCP mandatory options, per RFC 2131. The DNG must provide the following information to the TD:

a) TD device local IP address

b) TD device local network mask

c) DNS Server IP address(es)

d) The remote configuration server IP address (same as the one assigned to the DNG).

e) Default gateway IP address

f) If configured, the option(s) requested in step 2 above (carried in the Container option)  to convey specific information to the ITF, such as Service Provider Discovery information.

4) The TD continues the DHCP message exchange with the standard DHCP REQUEST message.

5) The DNG completes the DHCP message exchange with the standard DHCP ACK message

The flow is shown in Figure I-6 below:

Figure I-6: TD Network Attachment with a Local DHCP Server in the DNG

After acquiring the IP address of the remote configuration server, the TD interacts with the remote configuration server to acquire needed configuration parameters.

I.1.1.2.1.2 	Remote DHCP Server across the network

In this case the DNG passes all DHCP messages transparently to the network. Figure I-7 shows the sequence of events that take place when the TD is powered up or initialized.

Figure I-7: TD Network Attachment with a Remote DHCP Server

Specifically, the following takes place:

1) The TD establishes L1 and L2 connectivity with the DNG across the home network, which could be a copper network, a coaxial network, fibre/plastic network, or wireless network (not shown in the figure above).

2) The TD issues a DHCP DISCOVER message toward the DNG that must include all the mandatory DHCP options, per RFC 2131. The TD must also include option 60 to allow it to identify itself to the network. The DNG passes this DISCOVER message to the WAN network transparently.

The network provider DHCP server replies to the TD with a DHCP OFFER message (or DHCP ACK with the Rapid Commit option) that must include all the DHCP mandatory options, per RFC 2131.

3) The network provider DHCP server replies to the TD with a DHCP OFFER message that must include all the DHCP mandatory options, per RFC 2131. The DHCP server must provide the following information to the TD:

a) TD IP address

b) TD network mask

c) DNS server IP address(es)

d) If available, the remote configuration server IP address

e) Default gateway IP address

The DNG must pass the DHCP OFFER message to the TD transparently.

There are three possibilities at this point reflected in the content of the DHCP OFFER message:

i) If the DHCP server is provisioned with the remote configuration server IP address, then it must include it in the siaddr field of the OFFER message. This is shown as step 3-a in the figure above.

Ⅱ) If the DHCP server is provisioned with the remote configuration server domain name but not its IP address, then it must set siaddr = 0.0.0.0 and provide the remote configuration server domain name to the TD. The TD must interact with the DNS to resolve the remote configuration server IP address. This is shown as step 3-b in the figure above.

Ⅱi) If the DHCP server is not provisioned with the remote configuration server addressing information (IP address or domain name) and therefore does not provide the remote configuration server IP address or its domain name, then there must be a mechanism in place to allow the end user to manually configure the TD or the TD device may obtain that information from its Service Provider.  This is shown as step 3-c in the figure above.

4) The DHCP protocol sequence completes with the standard DHCP REQUEST and DHCP ACK messages.

After acquiring the IP address of the remote configuration server, the TD interacts with the remote configuration server to acquire needed configuration parameters.

I.1.1.2.2	 PPPOE

The Terminal device can use DHCP protocol to get it’s address. In the case, the DNG should have a dhcp server. When DNG get the network parameters through PPPoE, then TD can acquires the IP address and some network parameters form DNG through DHCP options. If the TD behind the NAT of DNG, TD sends it’s IP and ports request to DNG.

Figure I-8 below shows the sequence of events that take place when the DNG and TD is powered up or initialized.

Figure I-8: TD Network Attachment based on PPPOE

I.1.2 	Service Provider Discovery

When the ITF attaches to the network, it has to know from where to get the description of the IPTV Service Providers available to it. This is what is called an entry point.

An IPTV Service Provider Description entry point can be a multicast or a unicast address.

There are different ways for the ITF to get IPTV Service Provider Description entry points:

- A well known multicast address registered with IANA,

- Starting with an entry point address provisioned in the Terminal Device Configuration Data

- Starting with a domain name in the Terminal Device Configuration Data and using DNS to acquire entry point addresses

- At network connection, with DHCP, the Terminal Device might be provided with domain names to be used to acquire entry point addresses via DNS.

In ETSI TS 102034: Digital Video Broadcasting (DVB); Transport of MPEG-2 Based DVB Services over IP Based Networks, several ways to discover entry point addresses for non-IMS Terminal Device are described.

- Appendix II: Example of Multicast VPN

（This appendix does not form an integral part of this document）

II.1 	Requirement on Multicast VPN in IPTV Network Control Aspect

The IPTV multicast VPN operates with the security, management and Quality of Service (QoS) policies of a private network. Figure 1 depicts an example of basic IPTV multicast VPN services. In Figure B.1, the main office offers diverse IPTV application services which are sent from IPTV contents server, to home office, remote office and business partner with IPTV connected on VPN.

Figure II-1: Example of Basic Configuration in IPTV Multicast VPN

In order to provide IPTV multicast VPN, The IPTV access node will provide secured and encrypted multiple connections between main office and remote office. So, the IPTV multicast VPN will have multiple benefits as follows:

- Extend geographic connectivity

- Improve security

- Reduce operational costs

- Reduce transit time and transportation costs for remote users

II.2 	IPTV Multicast VPN Group Management

The IPTV multicast VPN group management is to achieve privacy and integrity in IPTV network. According to features of IPTV service, geographical position of IPTV users or both, IPTV multicast VPN groups can be formed diversely into same IPTV VPN or different IPTV VPN sites. Each IPTV multicast VPN group is assigned according to different security grade and QoS.

In order to support additionally secure group management, IPTV multicast VPN group will be applied to provide the following:

- Limiting of IPTV Multicast VPN Group Member:

The member limitation of IPTV multicast VPN group needs to optimize the resource usage of IPTV network.

- User Privacy:

The identity of the IPTV multicast VPN group members is kept secret from other IPTV VPN group. If needed, it is kept secret from other members of the same IPTV group.

- Data integrity:

The data received by the IPTV multicast VPN group members is coming from an authorized sender and has not been modified during transit.

- Appendix III: Mapping between IPTV Overlay Network Capability and IPTV functional architecture

（This appendix does not form an integral part of this document）

The function of IPTV overlay networking is performed under the control of Content Delivery Control  Function of IPTV functional reference architecture, and service control function for IPTV overlay networking initiate and manage IPTV overlay sessions to configure service capability of IPTV overlay networking. The control of Content Delivery Control Function is performed with associated operation among overlay nodes, and the service control function for IPTV overlay networking is performed with Session Manager.

IPTV Functional Architecture indicates the additional functional entities for IPTV overlay network in Functional Reference Architecture.

    - Session Manager in IPTV overlay networking is mapped to IPTV Service Control Function, and provides the functions to request service requirements/release to RACF for IPTV overlay networking.

    - Association among overlay nodes in IPTV overlay networking provides Content Delivery Control Function to control the location of requested contents, and to distribute control functions to the contents servers. Thus, the overlay nodes can be mapped with Content Delivery Control Functions, Location Control Function and Distribution Control Function, functionally.

- Overlay nodes will perform the function to search the location of IPTV contents sources with dynamic communication capability in overlay networking. Thus, Location Control Function for overlay networking is performed by the search function among multiple overlay nodes in IPTV Overlay Networking.

- Overlay nodes will perform the function to exchange control information among the IPTV contents servers. Thus, the association of overlay nodes in IPTV Overlay Networking will provide the function of Distribution Control Function.

The procedural functions and interfaces for IPTV Overlay Networking are shown in Figure III-1, which provides the functional model to perform overlay networking capability on IPTV Functional Architecture.

Figure III-1: Overlay Mapping for IPTV Functions

As shown in Figure III-1, Session Manager, Location Control Function and Distribution Control Function for IPTV overlay networking perform the functions for IPTV services features and contents control. And the interface function between SCF and RACF will be request and release for resource reservation and release in accordance with IPTV service requirements. The RACF sends confirmation of resource reservation to Session Manager when the resource has been reserved. Thus, Session Manager performs following functions to function of IPTV Service Control Function.

    - Service Resource Request & Release to create and release overlay session according to IPTV service requirements. In order to provide this function, Session Manager configures and manages overlay sessions topology for IPTV service requirements, and requests its resource reservation to RACF.

    - Service Access Control to allow service to authenticate IPTV users according to their access rights (e.g. subscriber information, PPV, age limit). And Service Access Control manages the session associated with the content.

    - Provision of mapping function between virtual address of overlay node and physical address of transport network to control service features on session topology.

Location Control Function for IPTV services will be provided through association among overlay nodes to provide the following functions in Content Delivery Control Function.

    - Identify the location of IPTV content servers for user request with cooperated operation among IPTV overlay nodes (e.g., by DHT: Distributed Hashing Table, content topology table may be created per IPTV channel basis).

    - Mapping location Information, e.g., mapping between logical and physical address, for IPTV contents locations Also, Location Control Function translates a logical content reference into a reference to the physical server.

    - Maintain the interface between Service Control Function and Content Delivery Control Function to exchange contents control information.

Distribution Control Function for IPTV service will be performed with exchanges the distribution control information among IPTV overlay nodes.

    - Establish optimal distribution policy for distribution content for IPTV overlay networking service features. When overlay node receives response message which other overlay nodes have the requested channel, it let Session Manager know about destination information after applying policy of each contents.

    - Use & maintains the distribution information about how the contents is distributed among Content Delivery & Storage Functions. It may be obtained through the request and response among IPTV overlay nodes for distribution control function.

- Appendix IV: ISP-manageable P2P Network Structure

（This appendix does not form an integral part of this document）

ISP Plane

User Plane

peer

peer

peer

peer

peer

peer

peer

peer

MN

MN

MN

MN

MN

MN

MN

MN

MN

Signalling

Content

MN domain

MN domain

MN domain

MN interdomain

Figure IV-1:  ISP-manageable P2P Network Structure

As shown in Figure IV-1, peers are classified into several groups with each group having a management node (MN) deployed by ISP. Peer classification can be done based on either content sharing criterion or physical location criterion. In content sharing criterion, peers with common interest on certain kind of content (e.g. movie, file) are classified into one group. In physical location criterion, peers with short distance (e.g. network distance, geography distant) are classified into one group.

MN performs peer management functions in each group, such as peer registration, maintenance, AAA, etc., as well as P2P signalling proxy including content publish, search, etc. MN may also perform media signalling proxy for peers when media is transferred among peers.

Several MNs are organized in MN domain where MNs share content information using P2P signalling such as content put, get, update, etc. Several MN domains can be further organized in MN interdomain to share the content information among MN domains. Content exchange always happens between peers such that the size of peers will not be bounded.

Management Functions of MN

In peer group, MN implements peers management functions including:

    - Peer registration. Peer register with MN about its capabilities, e.g. CPU, storage, bandwidth, etc.

    - Peer maintenance. MN periodically collects status, e.g. link, capabilities etc. from peers.

    - Peer AAA. MN can authenticate and authorize peer when it joins the group, and account for peer during service provisioning.

Media Signalling Proxy Function of MN

In order to get media status in peer group, media signalling proxy may be performed by MN. MN implements media signalling proxy function by transferring media connection signalling between peers and obtaining media session status of peers.

P2P Functions of MN

In peer group, MN performs P2P proxy for peers including:

    - Peer content publication.

As shown in Figure IV-2, peer A publishes its content ID to MN1, together with its contact address. As the P2P proxy of peer A, MN1 publishes the content ID and peer A address to responsible MN (MN3 in Figure 2) in the MN domain which is responsible for the content ID, according to some P2P content publish mechanism such as Chord, Pastry, etc.

peer A

MN1

MN3

MN2

MN Domain

content ID, peer A address

content ID, peer A address

Figure IV-2:  peer content publish

    - Peer content search in MN domain.

As shown in Figure IV-3, peer B sends content request (e.g. content ID) to MN4. As the P2P proxy of peer B, MN4 finds the responsible MN (MN3 in Figure 3) in the MN domain which stores the content ID, according to some P2P content search mechanism such as Chord, Pastry, etc. MN3 then returns the corresponding peer A address to peer B.

MN4

MN3

MN2

MN Domain

content ID

content ID

peer B

peer A address

MN1

Figure IV-3: peer content search

    - Peer content search in MN interdomain.

If the requested content ID can not be found in MN domain, MN could launch a search procedure in MN interdomain. As shown in Figure IV-4, when MN3 does not contain the requested content ID, it could forward the content request to MN5 in another MN domain through the MN interdomain. MN5 then finds the responsible MN (MN6 in Figure 4) in its MN domain which stores the content ID, according to some P2P content search mechanism such as Chord, Pastry, etc. MN6 finally returns the corresponding peer C address to peer B.

MN4

MN3

MN2

MN Domain

content ID

content ID

peer B

MN5

MN6

MN Domain

peer C address

MN InterDomain

MN1

Figure IV- 4:  peer content search across MN domain

__________