|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-OL-0093 |  |  |
|  |  |  |  | English only |  |  |
| WG(s): PL |  |  | 7th FG IPTV meeting: Qawra, St Paul’s Bay, Malta, 11-18 December 2007 |  |  |  |
| OUTGOING LIAISON STATEMENT |  |  |  |  |  |  |
| Source: |  | Chairman, ITU-T FG IPTV |  |  |  |  |
| Title: |  | Response to the liaison statements from ISO/IEC JTC 1/SC 25/WG1 on residential gateway |  |  |  |  |
| To: |  | ISO/IEC JTC 1/SC 25/WG1 |  |  |  |  |
| Approval: |  | FG IPTV Plenary |  |  |  |  |
| For: |  | Action/information |  |  |  |  |
| Deadline: |  | 10 January 2008 |  |  |  |  |
| Contact: |  | Mr. Ghassem Koleyni Nortel Networks (Canada) Canada |  |  | Tel: +1 (613) 763 4154 Email: ghassem@nortel.com |  |

The Focus Group on IPTV (FG IPTV) is pleased to inform you that we made significant progress at our 7th meeting in Malta, 11-18 December 2007.

At the meeting, your liaison statement on FCD ISO/IEC 15045-2 and the published standard ISO/IEC15045-1 were reviewed. We provide the following comments for your consideration.

In clause 7.1 of FCD ISO/IEC 15045-2, the protocol stack at the residential gateway is described.  IPTV traffic requires specific considerations such as QoS and multicast.  We would like to ensure that content delivery and interactivity of IPTV are not impaired by the mechanisms of the residential gateway.

Quoting clause 7.1 of FCD ISO/IEC 15045-2,

The GIWF translates the Network 1 application language into the GL and then sends it downward through the GL meta-protocol stack to the HES-gateway intermediate bus. The data are transferred by the intermediate bus to the Network 2 HES-gateway module where they are passed upward through its GL stack to the GIWF for translation into the application language of Network 2. The data are then passed down the specific Network 2 stack to Network 2 — and then on to the final destination on Network 2.

This text implies that the packets received at the gateway propagate through the application layer. In the case of IPTV, the gateway device is typically expected to work more like a network device, for example a router.  By this understanding, it is likely that the residential gateway described in this FCD would work as a terminal device rather than a DNG in our document.  Considering Figure 1 and accompanying explanations in your document, this is a kind of hybrid device in which, for example in case of IPTV, the DNG and IPTV TD are combined.

We think this is an interesting topic for IPTV and a good study item.  We hope the ISO/IEC JTC 1/SC 25/WG1 will collaborate with the continuing activity of IPTV, namely, the IPTV-GSI (see below), on this topic.

Please note that FG-IPTV produced an “IPTV related protocol” [FG IPTV-DOC- 0191].  This document may help you to understand and compare the protocol structure of your FCD, specifically clause 7.1 and 7.2, and our described IPTV service.

FG IPTV would also like to remind you that all its documents (Inputs, Outputs, Reports and Liaison Statements) are publicly available.

Our recent meeting produced a number of deliverables which may be of interest to your members. All FG IPTV documents can be obtained from

http://www.itu.int/md/T05-FG.IPTV-071211/sum/en

The FG IPTV sincerely thanks the ISO/IEC JTC 1/SC 25/WG1 for your cooperation during the period of its activities and is glad to inform you that the ITU-T standardization work on IPTV following on from the results of the FG IPTV will continue in the ITU-T study groups under the auspices of the IPTV-GSI (Global standards initiative). The first event of IPTV-GSI is to take place in Seoul, Republic of Korea from 15 to 22 January 2008. More information on IPTV-GSI can be obtained from:

http://www.itu.int/ITU-T/gsi/iptv/

We will welcome the continued involvement of the ISO/IEC JTC 1/SC 25/WG1 in the ongoing work which will be coordinated by an IPTV Joint Coordination Activity established by ITU-T Study Group 13. Any future correspondence or questions related to the ITU-T IPTV-GSI studies may continue to be addressed to tsbiptv@itu.int with a copy to Mr. Ghassem Koleyni (ghassem@nortel.com)

___________________