|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 19 – C 22 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | April 2007 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 2/19 |  |  |  |
| STUDY GROUP 19 – CONTRIBUTION 22 |  |  |  |  |  |
| Source: |  | Korea (Republic of) |  |  |  |
| Title: |  | Proposed text for the revision of LMF version 0.3 |  |  |  |

TSB note: this text has also been published in Study Group 13 as COM 13-C 296

1. Introduction

This contribution proposes a revision of the draft Recommendation Q.LMF version 0.3 which was the latest output document produced in the 2006 October meeting of ITU-T NGN-GSI. This proposed revision includes structural changes, editorial updates, and editor’s notes.

2. Proposal

This is submitted for discussion to the ITU-T SG19 and NGN-GSI MM group 2007 April meeting, and it is proposed that this text be used as a base document so as to produce LMF version 0.4.

Draft New ITU-T Recommendation LMF

Framework of Location Management for NGN

Summary

TBD

Keywords

NGN, IMT, Location Management, Framework

Table of Contents

1	Scope	4

2	References	4

3	Definitions	5

4	Abbreviations	5

5	Design Considerations	6

5.1  Basic Functions	7

5.2  Design Considerations	8

6 	Reference Functional Architecture	9

6.1  Basic Functional Architecture	9

6.3  Detailed Functional Architectures	10

6.3  Relationship with NGN-FRA	10

7	Functional Information Flows	11

7.1  Location Management for Inter-CN (MM1)	11

7.2  Location Management for Inter-AN (MM2)	12

7.3  Location Management for Intra-AN (MM3)	12

A.	Usage Models	16

A.1  SIP based Location Management	16

A.2  MIP based Location Management	17

B.	Functional compatibility considerations for Recommendation LMF	19

B.1 Functional concept for compatibility with NGN	19

B.2 Compatibility of location management functions among various mobile technologies	20

# 1	Scope

This Recommendation is purposed to provide the framework of location management for NGN. To design the framework of location management for NGN, this Recommendation describes the followings:

- Design considerations and basic functions for location management;

- Reference functional architecture for location management;

- Functional information flows of location management; and

- Application of candidate protocols for location management.

This work on Location Management Framework (LMF) has been made based on the ITU-T Recommendations: Mobility Management Requirements (MMR) and Mobility Management Framework (MMF). LMF also shall be well harmonized with Handover  Management Framework (HMF).

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation

[1]	ITU-T Supplement Q.sup52 (2004), Technical Report on Mobility Management Requirements for Systems beyond IMT-2000

[2]	ITU-T Draft New Recommendation MMR (working in progress), Mobility Management Requirements for Fixed and Mobile Convergence Networks

[3]	ITU-T Draft New Recommendation MMF (working in progress), Generic Framework of Mobility Management for Fixed and Mobile Convergence Networks

[4] 	NGN Focus Group draft deliverable, “Mobility Management Capability Requirements for NGN”

[5]	ITU-T Recommendation Q.1701 (1999), Signalling Requirements and Protocols for IMT-2000

[6]	ITU-T Recommendation Q.1702 (2002), Long-term Vision of Network Aspects for Systems beyond IMT-2000

[7]	ITU-T Recommendation Q.1711 (1999), Network Functional Model for IMT-2000

[8] 	ITU-T Recommendation Q.1721 (2000), Information Flows for IMT-2000 Capability Set 1

[9]	ITU-T Recommendation Q.1741.3 (2003), IMT-2000 References to Release 5 of GSM evolved UMTS Core Network

[10]	ITU-T Recommendation Q.1742.3 (2004), IMT-2000 References (approved as of 30 June 2003) to ANSI-41 Evolved Core Network with cdma2000 Access Network

[11]	ITU-T Recommendation Q.1761 (2004), Principles and requirements for convergence of fixed and existing IMT-2000 systems

[12]	ITU-R Recommendation M.687-2 (1997), International Mobile Telecommunications-2000

[13]	ITU-R Recommendation M.1034-1 (1997), Requirements for the radio interface(s) for (IMT-2000)

[14]	ITU-R Recommendation M.1168 (1995), Framework of IMT-2000

[15]	ITU-R Recommendation M.1224 (1997), Vocabulary of terms for IMT-2000

# 3	Definitions

This Recommendation uses the definitions and terms that have been defined in the draft Recommendations MMR and MMF.

In addition, the following terms are used in this Recommendation:

[Editor’s Note] definitions section will be filled as the work progresses.

| Location Management | A process that enables the network to determine a mobile’s current location, i.e., the mobile’s current network attachment point where the mobile can receive traffic from the network. |
| --- | --- |

# 4	Abbreviations

[Editor’s Note] This section will be updated as the work progresses

AAA		Authentication, Authorization and Accounting

AN		Access Network

BU		Binding Update

CCoA		Co-located Care of Address

CH		Correspondent Host

CN		Core Network

CoA		Care of Address

DHCP		Dynamic Host Configuration

FA		Foreign Agent

FRA		Functional Requirements and Architecture

GLM		Gateway Location Manager

HA		Home Agent

HLM		Home Location Manager

HoA		Home Address

ID		Identification

IETF 		Internet Engineering Task Force

IP		Internet Protocol

IPCP		IP Control Protocol

LLM		Local Location Manager

LM		Location Management

LMF		Location Management Framework

MIP		Mobile IP

MIP4		Mobile IPv4

MIP6		Mobile IPv6

MM		Mobility Management

MMR		Mobility Management Requirement

MT		Mobile Terminal

NGN		Next Generation Network

DHCP		Dynamic Host Configuration

QoS		Quality of Service

SIP		Session Initiation Protocol

SCTP		Stream Control Transmission Protocol

TCP		Transmission Control Protocol

UA		User Agent

UDP		User Datagram Protocol

URI		Uniform Resource Identifier

[Editor’s Note] To clarify the scope of MMF/LMF/HMF, new introduction section may be necessary.

# 5	Design Considerations

This Recommendation describes the Location Management Framework (LMF) for NGN, which is based on the associated ITU-T Recommendations: Mobility Management Requirement (MMR) in terms of the requirements for MM in the NGN and Mobility Management Framework (MMF) in terms of the generic framework of MM in the NGN. Also the contents of this recommendation will be harmonized with a family recommendation, HMF.

The goal of the location management functionality is to keep track of the mobile users/terminals in the network, to identify where the mobile user is in the network, and to maintain the location information of the mobile user as it continues to move around in the network so that a caller can deliver packet to the mobile users/terminals. For this purpose, the location information of a mobile user/terminal needs to be identified, registered, and maintained at the corresponding location manager. The location information will be referred to, whenever the connection request for the concerned mobile user arrives from the correspondent user.

As described in the ITU-T Recommendation MMF, the basic functional flows for location management can be briefly classified into the functions for MTs and for caller, which can be mobile or fixed. The functions for MTs is to register current MTs’ location in network and includes the interworking with AAA for user/terminal identification, configuration of location ID (e.g. new IP address), location registration and location update function. The functions for caller are to track current MT’s location and delivery packet to the location. It includes location query using user/terminal ID, location reply and packet delivery.

This Recommendation describes the basic functions of location management. This Recommendation will also describe the reference functional architecture and procedures of location management. Based on the LMF designed in this Recommendation, a couple of possible existing location management protocolswill be reviewed how their components are map to FEs of LMF for location management in the NGN.

## 5.1  Basic Functions

In this Recommendation, the LMF has following basic functions to support location management:

A. Terminal/User Identification

The MM protocols for NGN must specify how users/terminals are to be authenticated, authorized accounted, and secured for services using standard AAA and security mechanisms.

The result of the AAA functionality will be a yes/no decision on the service request made by a user. As a next step, the access network configuration will be adapted to the mobile/nomadic user such that it satisfies the particular Quality of Service (QoS) level and security association for the requested service. These mechanisms should be based on the user’s subscription profile and the technical resource constraints of the respective access networks.

While it is envisaged that many users will have a terminal for their exclusive personal use, there will also be many cases where terminals must be shared among multiple users (e.g., public telephones) and cases where a particular user, for good and valid reasons, may need to borrow a terminal from another user who normally has exclusive use of his terminal but is willing to let it be used by someone else for good and valid reasons (e.g., an emergency call when the terminal has exhausted its battery.) For these and other circumstances, it is considered essential that the mechanisms for identifying users and terminals be separate, that is, that a user’s identify is distinct from a terminal’s identity, and that there is a non-permanent association of a user with a particular terminal, which may be transitory or long lasting, depending on the circumstances. Users may also have multiple terminals and will want to be able to transfer from one terminal to another according to their needs. As the overall context is mobility, it is also important that geographic information be separate from the user identity and the terminal identity. The mechanisms to handle all of these will require careful investigation since it will also be essential that they are interoperable with existing naming, numbering, addressing and routing mechanisms.

B. Configuration of location ID

It is noted that the user ID for an MT is given by the home operator or service provider. The user ID may typically contain the domain name of the home operator, as we can see in the example of “user@domain”.

When an MT enters a new network region, it must first get its current location ID (e.g., a routable IP address). For this purpose, the MT may use the Dynamic Host Configuration Protocol (DHCP), IP Control Protocol (IPCP), or IPv6 stateless IP address auto-configuration. The location ID will be typically an IP address or an identifier to be converted in the unique IP address, which may include the information on the local network (or local network operator).

In the configuration of a location ID, the MT may be enforced to contact with the AAA server for authentication and authorization. In this Recommendation, the MMF assumes that the MT could get an IP address using the relevant IP address configuration and AAA scheme. The detailed procedures of the address configuration and interaction with the AAA servers are outside the scope of this recommendation.

C. Location Registration/Update

Each MT should register its current location ID (e.g. IP address) with its associated location manager, when it awares that not in Home network in order to be tracked by the network.   Also it should update its locatin ID whenever it moves into another new region and gets a new location ID. Each location manager will keep and maintain the information of mapping between User ID and Location ID for the concerned MTs.

D. Packet Delivery to an MT

When another external or internal (fixed or mobile) terminal, named the ‘caller,’ wants to communicate with the MT, first of all, the caller terminal should know the current location ID of the called MT. In this case, the location information managed by LM will be used. The caller terminal will first query the home LM about the current location ID (IP address) of the called MT. After getting the current location of the MT, now the caller terminal will begin the call/session establishment and/or the data transport process with the destination MT.

It is noted that the location query and reply procedures could be differently applied to the services or applications, depending on whether or not they need out-of-band signaling for call/session setup.

E. Paging for Idle Terminals

The paging capability for supporting idle mode terminals is essential in large-scale networks because it provides power saving in mobile terminals as well as signalling reduction in networks, which improve the scalability of NGN. In particular, paging support needs to be provided together with location management.

When an MT goes into the idle (or dormant) state, it registers with a Paging Manager. The MT will also update its paging area, whenever it changes the paging area. The paging management could be implemented in the access link layer and/or IP-based network layer. In the LMF, the IP-based paging may be concerned, since the paging schemes in the underlying access link layer are subject to a specific access technology.

## 5.2  Design Considerations

In this Recommendation, the LMF is designed with the following considerations:

A. Various mapping scenarios between Local Location Manager (LLM) and Access Networks (ANs)

It is noted that the location managers are local entities rather than physical entities. Considering how to map them into the practical access networks, there could be various mappings between LLM and ANs according to operators’ policies such as capacity of LLM. Therefore the structure and procedure of LMF should be designed to cover these various mappings.

[Editor’s Note] There might be more necessary design condierations. Further contibutions are requested.

6 	Reference Functional Architecture

## 6.1  Basic Functional Architecture

The figure 1 shows general model to support the consistency of conceptual function between LMF and NGN architecture.

Figure 1. Conceptual model for location management

The separation between control plane and transport plane is a basic assumption for location management (LM). LM are hierarchically composed into Local Location Manager (LLM) and Home Location Manager (HLM). These location managers also could be distributed between a visited network and a home network.

UE, User equipment, also need a LLM to support location management procedure, for example, sending location update(LU) message and receiving LU-ACK message.

LM may be located at transport control function or service control function. LM is likely to be LLM for various access network and HLM for core network. Especially HLM supports the capability to provide the control connection with other networks.

Based on this generalized conceptual model for location management, other location managers for personal/service mobility management also could be added.

## 6.3  Detailed Functional Architectures

[Editor’s Note] The description on detaild functional architecture is necessary. Contributions are solicited.

[Editor’s Note: This section should include the contents regarding follows issues

- Detailed Functional architecture, which includes LM and AAA, DHCP, CSCF, etc

- Reference Configuration for MM1/2/3 Scenarios

- Text that shows the linkage with NGN-FRA]

[Editor’s Note: Further consideration on the usage of “Home” in location manager and home network is requested]

## 6.3  Relationship with NGN-FRA

[Editor’s Note] It was agreed that it is very necessary to clarify how to map LMF functinal entities into NGN-FRA functional entities. This section will include the text realted to that issue.

# 7	Functional Information Flows

## 7.1  Location Management for Inter-CN (MM1)

Following figure shows inter-CN location management architecture for NGN. The mobile terminal’s home network and the visited network have their own home LM and local LMs. A gateway location manager (GLM) is deployed at the edge of the visited network to perform location management functions for the visiting mobile terminal at the visited network. As far as the physical location is concerned, the GLM of the visited network can be collocated with home LM of the visited network.

Figure 2. Inter-CN Location Management Architecture.

As the MT moves into the visited network, the MT performs location registration with a local LM in the visited network. When the local LM receives the location registration message from the visiting MT, the local LM will forward the location registration message to the GLM, not to the home LM of the visited network. As the GLM receives the location registration message, the GLM downloads the subscriber profile information of the visiting MT from the home LM of the mobile’s home network. The GLM reports to the home LM of the mobile’s home network that the MT is currently located in the visited network. From the view point of the home LM in the home network, the GLM can be considered as another local LM of the home network. The GLM treats the subsequent location update messages received while the MT is in the visited network as if it were the MT’s home LM. The GLM handles the location update procedure locally as the MT is moving within the visited network; therefore, there is no need for inter-CN signaling for location management until the MT leaves the visited network.

As far as the signaling traffic for location management is concerned, the relationship between the home LM of the home network and the GLM is the same as that between the home LM and the local LM in the home network, and the relationship between the GLM and the local LM of the visited network is the same as that between the home LM and the local LM in the visited network. From the viewpoint of the home LM at the home network, the GLM is considered as a local LM. From the viewpoint of the local LM of the visited network, the GLM is considered as if it were the home LM at the visited network.

Following figure shows the signaling flow of the mobile’s first location registration at the visited network. The semantics used for the location registration signaling between the local LM and the gateway LM is the same as that between a local LM and a typical home LM, and the semantics used between the gateway LM and the home LM is the same as that between a typical local LM and a home LM except for the subscriber profile information download procedure which may possibly include the mobile’s Inter-CN roaming agreement verification.

Figure: Inter-CN Location Registration Signaling Flow.

## 7.2  Location Management for Inter-AN (MM2)

[Editor’s Note] This section will describe the information flow in case of Inter-AN MM. Contributions are invited.

## 7.3  Location Management for Intra-AN (MM3)

[Editor’s Note] This section will describe the information flow in case of Intra-AN MM. Contributions are invited.

Appendix

# A.	Usage Models

[Editor’s Note] this section will describe how some of the candidate IP-based protocols could be used for location management and what are needed for it, based on the LMF identified in this Recommendation. Some reference texts are given below. Further contributions are invited.

## A.1  SIP based Location Management

[Editor’s Note] In NGN, SIP is likely to be used in the form of IMS. In this case, this section may be changed into “IMS for Location Management”. Further contributions are invited

The Session Initiation Protocol (SIP) has been specified in the IETF for supporting the control of IP-based multimedia sessions as a signaling protocol. The SIP is an application-layer control protocol that can establish, modify, and terminate multimedia sessions. The SIP functional entities include UA, Proxy Server, Redirect Server, Registrar and the location DB. It operates independently of the underlying transport layer protocols such as Transmission Control Protocol (TCP), User Datagram Protocol (UDP) and Stream Control Transmission Protocol (SCTP). More details on SIP are given in IETF RFC 3261.

A.1.1  Location Update

SIP provides the location registration and update functionality using the SIP Registrar. When an MT moves into a new network region, it registers its current location ID (SIP URI) with the location database via a SIP Registrar.

Figure A2-1 shows the functional information flows for the SIP-based location registration and update operations.

Figure A-1. SIP-based Location Update

As shown in the figure, each time an MT gets a new location ID (IP address) from the new AR region, it will update its location with the SIP Registrar server by sending the SIP REGISTER request message. In response to the REGISTER message, the Registrar will send an SIP status message (e.g., 200 OK) to the MT.

A.1.2  Location Query and Session Establishment

SIP also provides the location query functionality for call/session establishment using the SIP Proxy or Redirect servers. It is noted in SIP that the session establishment could be done at the same time with the location query operations, in particular, in case of using SIP proxy server instead of SIP redirect server.

The following figure illustrates an example flow of the SIP-based location query with the call/session establishment. The SIP INVITE message is used for the session establishment, in which the location query operation is performed at the same time.

Figure A-2. SIP-based Location Query with Session Establishment

A.1.3	Relationship with LMF FEs

[Editor’s Note] This section will shows how the entities of candidate protocol could be mapped into LLM FEs, like as shown in Table A.1 in Appendix. Contributions are requested.

## A.2  MIP based Location Management

The Mobile IP (MIP) is an MM protocol that has been specified in the IETF. Mobile IP may be divided into Mobile IPv4 (MIPv4) and Mobile IPv6 (MIPv6). Details of MIPv4 and MIPv6 are described in IETF RFC 3344 and RFC 3775, respectively.

These two protocols basically provide similar functionality with some exceptions in the detailed operation mechanisms. In this section, the Mobile IPv4 is reviewed in terms of the LMF.

A.2.1  Location Update

When a MT moves into a new subnet, apart from the Home Network, it will get a CoA of FA (IP address of FA) in the new network. The MT may use a Co-located CoA (CCoA), which will be obtained by Dynamic Host Configuration Protocol (DHCP). In the process of obtaining the CoA address, the MT may be required for performing the appropriate authentication procedure via AAA server.

After realizing that the MT is in the new network, it registers with the HA with a Care of Address (CoA). The MT must register its CoA with the HA whenever the MT changes its subnet. This is called ‘Binding Update’ procedure in the MIP. It is noted that the MIP HA is used as an LM. That is, the HA maintains the location information for the MT. In this case, the Home Address (HoA) of the MT is used as the User or Terminal ID, whereas the CoA of FA will be used as the Location ID (IP address).

In this way, the HA maintains and updates the current location ID for the MT, as shown in the figure below.

Figure 8. Location Update by MIP

A.2.2  Data Transport with Location Management

When an external Correspondent Host (CH) sends a data packet toward the MT (over the HoA of the MT), the HA will intercept and receive the packets destined for the MT from the CH, since the MT is roaming in a visited network.

The HA then forwards these packets to the MT using the CoA of MT. These data packets will be delivered to the MT through the Mobile IP tunnel. Basically, the MIP uses the standard IP routing scheme for the data transport.

When receiving the tunneled packets from the HA, the FA will de-capsulate the received packets and delivers the original packets to the MT, as shown in the figure below.

Figure 9. Data Transport by MIP

A.2.3	Realtionship with LMF FEs

[Editor’s Note] This section will shows how the entities of candidate protocol could be mapped into LLM FEs, like as shown in Table A.1 in Appendix. Contributions are requested.

# B.	Functional compatibility considerations for Recommendation LMF

[Editor’s Note: Regarding following architecture, some issues that should be addressed are raised as follows;

- Consider if Mobile Location Agent may not be collocated with the End User Function.

- Where is the delineation between Access and Core Network? We should make sure there is a coherency with FMC and NGN FRA.

- Should the LLM and HLM be also in Service Control Function?

[Editor’s Note: Following architecture is not agreed. However it is agreed that it includes many valuable information for future works of LMF and should be reflected later in main text. Further contributions are solicited for this]

## B.1 Functional concept for compatibility with NGN

Fig A.1 shows the location management functional architecture model for transport stratum. In this figure, we propose to consider the location management function in both access network and core network as defined in Q.MMF and Q.LMF. In the access network, local location management function is considered, while home location management is considered in the core network. Both location management functions are worked with other specific functions such as user profile management function, AAA function, and network access control function. The local location management function is connected with ‘mobile location agent’ as well as with home location management function. The mobile location agent is physically located in the part of an end user, and just works as the functional interface between an end user and an access network. Also, LLM / HLM functions are connected with service control function such as SIP and non-SIP based mobility management.

Fig. B.1. Location management functional architecture model for transport stratum

In the basis of Fig. A.1, Fig. A.2 illustrates the location management functional architecture for the compatibility with NGN. This figure is reasonable to align with NGN architecture.

Fig. A.2. Location management functional architecture for compatibility with NGN

## B.2 Compatibility of location management functions among various mobile technologies

Functional entities related to location management are defined in various mobile technologies, such as Mobile IP, GPRS, and 3GPP LTE (Long Term Evolution). For future convergence networks, we need to consider the functional compatibility among these standardized technologies due to the efficient management without any modification. Currently, similar functions are defined somewhat differently in various technologies. The functional architecture discussed in Fig.1 must provide the required location management functions by Mobile IP, GPRS, and 3GPP/3GPP2, especially for vertical handovers. In summary, table 1 shows a draft description how functions of HLM and LLM are provided by functional entities of other standard technologies. The reference of the table 1 is shown in the Appendix of this contribution.

Table A.1. Compatibility of Functional Entities among Various Technologies

| ITU-T Functional Entity Mobile Technology |  |  | HLM | LLM |
| --- | --- | --- | --- | --- |
| Provided by | MIPv4, MIPv6 |  | Home Agent | Foreign Agent |
|  | IEEE 802.xx family |  | To Be Determined | To Be Determined |
|  | SIP |  | To Be Contributed | To Be Contributed |
|  | GPRS (GSM, Circuit switched) |  | HLR SGSN | VLR |
|  | 3GPP LTE | Circuit switched service | HLR HSS | VLR 3GPP Anchor (for GPRS) MME/UPE |
|  |  | Packet switched service | To Be Contributed | SAE Anchor (for packet networks) |

- HLR: home location register, VLR: visited location register, SGSN: serving GPRS support node

- HSS: home subscriber server, MME: mobility management entity

- UPE: user plane entity, SAE: system architecture evolution

________________