| Genève, le 15 mars 2006 UNION INTERNATIONALE DES TÉLÉCOMMUNICATIONS Bureau de la normalisation des télécommunications |  |
| --- | --- |

| Réf: Tél.: Fax: E-mail: | Circulaire TSB 83 COM 5/JKK +41 22 730 5780 +41 22 730 5853 tsbsg5@itu.int | - Aux administrations des Etats Membres de l'Union |
| --- | --- | --- |
|  |  | Copie : - Aux Membres du Secteur UIT-T; - Aux Associés de l'UIT-T; - Aux Président et Vice-présidents de la Commission d'études 5; - Au Directeur du Bureau de développement des télécommunications; - Au Directeur du Bureau des radiocommunications |

| Objet: | Questionnaire sur l'environnement électromagnétique à domicile |
| --- | --- |

Madame, Monsieur,

1	Le déploiement rapide de services large bande dans le monde (avec quelque 100 millions de lignes à large bande en service fin 2005) offre la possibilité aux opérateurs de réseaux de télécommunication de créer de nouveaux revenus en fournissant des services simultanés voix, données et télévision (ce que l'on appelle "offre tri-services").

2	Pour cela, le service large bande devra être introduit dans un plus grand nombre de foyers qu'à l'heure actuelle vers un ensemble de dispositifs susceptibles de pouvoir fonctionner n'importe où. Par exemple, pour la fourniture de services de télévision, le service large bande devra être acheminé à un boîtier décodeur placé près d'un poste de télévision. Une gamme de technologies de distribution à domicile, filaires et hertziennes, est en cours d'examen.

3	Il est essentiel de bien comprendre l'environnement électromagnétique à domicile pour assurer le bon fonctionnement des installations de distribution domestiques. Nous avons un besoin urgent de connaissance dans le domaine afin de prévenir les brouillages au niveau de la couche physique des installations de distribution domestiques causés par l'environnement électromagnétique à domicile. Une connaissance est nécessaire aussi bien au niveau de l'environnement de conduction - qui a une influence sur les technologies de distribution filaires - et de l'environnement de rayonnement - qui a une influence sur les technologies de distribution hertziennes.

4	La Commission d'études 5 de l'UIT-T examine ces brouillages potentiels au niveau de l'acheminement des offres tri-services et fait appel à des contributions et informations sur le sujet. Nous prions donc les parties intéressées d'effectuer des mesures simples conformément à la méthode d'essai simple décrite dans l'Annexe 1 et de faire parvenir les réponses (voir Appendice A) au Rapporteur pour la Question 8/5 (avec une copie à tsbsg5@itu.int) avant le 1er mai 2006:

| Ryuichi Kobayashi 4-10-23, Higashi Gotanda, Shinagawa-ku, Tokyo, 141-0022, Japon | Tél.: +81-3-5739-3241 Fax: +81-3-6408-2914 E-mail: kobayasi@tasc.mod.east.ntt.co.jp |
| --- | --- |

5	Vous êtes invités à renvoyer les formulaires figurant à l'Appendice A par e-mail ou par télécopie. N'hésitez pas, le cas échéant, à utiliser des pages supplémentaires si l'espace prévu dans le questionnaire est insuffisant.

6	Nous comptons sur votre coopération pour que vous fassiez en sorte de formuler des réponses aussi précises que possible et de les communiquer au Rapporteur avant la date limite susmentionnée.

Veuillez agréer, Madame, Monsieur, l'assurance de ma haute considération.

H. Zhao
Directeur du Bureau de la
normalisation des télécommunications

Annexe: 1

Annex 1
(to TSB Circular 83)

Simplified measurement method for evaluating 
Electromagnetic Environment

# 1	General approach for evaluating EM-environment in the Home Network

## 1.1	General information

It takes a long time to precisely evaluate the EM-environment,. Therefore, a simplified method for evaluating the EM-environment is useful to grasp the real in home EM-environment.

The measurement investigation should be performed in two steps on:

- Conducted disturbances;

- Radiated disturbances.

The measurement of the conducted disturbances could be carried out on all the power and signal lines connected to the equipment of the home network. In some cases, it is difficult to measure all lines in the home. In this case, it is recommended that a sample of telecom and power lines should be measured.

The measurement of the radiated disturbances should be carried out in each room of the home environment. It is recommend that situation of the measurement should be noted on the measurement report.

As a minimum, the environment should be measured in the following steps

- With every electrical/electronic device within the home switched off.

- With every electrical/electronic device within the home switched on.

If possible, the following additional steps are recommended:

a) switch on one electrical/electronic device and measure the EM-environment;

b) Repeat (a) until all electrical/electronic devices are switched on.

The results to be reported in the questionnaire should indicate the minimum and the maximum measured values. It is recommended that visual results, i.e. a wave form, wave shape, frequency spectrum, and so on, would be noted on the measurement report.

## 1.2	Measurement report

After the measurement, measurement report should be made and it should include the following information, at least.

- Results of the measurement, such as level of the interferences, frequencies, and so on, should be recorded in the report. If you can, visual data would be attached to it.

- The equipment in each room should be listed in the report. Name of the manufacturer and model are important information.

- The consumer electronic and/or electrical devices, such as TV, refrigerator, or lighting system, should be also listed in the report, because these kinds of devices may be the cause of trouble in home, in some case.

- Diagrams showing equipment position, telecom/power cable arrangement and measurement position should be presented in the report.

- Type of the residence, i.e. apartment, semi-detached house, or detached house, should be note in the report.  Construction of the home, eg wood, brick and so on, and separation of home from nearest neighbour should be also recorded.

- Measurement time and date should be described in the report, because, in some case, an influence of the circumstances, such as interference from neighbour house (room), depends on these parameters.

# 2	Simplified method for measuring a conducted disturbance

Figure 1 shows simplified measurement method for a conducted common-mode current. Current probe is used to detect “common-mode current” on the telecom. and/or power line. Oscilloscope is used to measure the waveform of the current.

Figure 1 - Simplified measurement method for conducted common-mode current

Figure 2 shows simplified measurement method for a conducted common-mode voltage on the line.  It is important to isolate between Oscilloscope and the earth, because of reducing the influence from the earth and reducing risks of a power fault (making electrical short-circuit) or human safety for electrical shock when measuring power line.  Using Battery operated Oscilloscope is better way to measure a voltage of the conducted disturbances without avoiding these risks. Especially, when you measure a voltage on a power line, using jig as shown in fig.3 is also better way to reduce risk of safety.

It is recommended that measurement condition should be noted on the measurement report.

Figure 2 - Simplified measurement method for conducted common-mode voltage

Figure 3 - Example of “Jig” for measuring conducted interference voltage on a power line by voltage probe

# 3	Simplified method for measuring a radiated disturbance

It is sometimes difficult to measure a radiated disturbance in the home. To simplify the measurement, the following procedure is recommended.

1) Select a frequency range concerned.

Ex. Mid-wave radio: 500kHz ~ 2MHz, TV: over 80MHz to below 1GHz, WiFi: 2.4 GHz band or 5.4GHz band

2) Select appropriate antenna that can measure the frequency range.

If the frequency range is wide, it is recommended to use “Broadband antenna”, such as biconical antenna, loop antenna, and so on.

If the frequency range is narrow, it is recommended to use Dipole antenna (Tuned), horn antenna, tuned loop antenna, and so on.

3) Select measurement equipment.

If you measure a spectrum, it is recommended to use Spectrum analyser.

If you measure a spectrum and time variation, it is recommended to use Real-time spectrum analyser.

4) It is recommended that using “Auto configuration” function is easy way to set up measurement .equipment.

Figure 3 shows set up for measuring a radiated disturbance.

In reality, distance from the equipment (or devices), height of the antenna, and polarity of the antenna are key parameters to evaluate the measurement results. Following points are recommended to measure the radiated emission in real home environment.

a) Distance from the object: It is recommended 3 m between antenna and the objects. If in the case of distance is shorter or longer than 3m, or in the case of there are lot of devices in close proximity, then distance from the objects should be noted on the measurement report.

b) Antenna height: It is recommended that it should be greater than 1.5 m above the floor. The height should be noted on the measurement report.

c) Polarity of the antenna: It is recommend that 3-plorities (one vertical, and two orthogonal horizontals) should be measured to survey radiated emission. If it is difficult to measure the three directions, it must note the measured polarity on the measurement report.

Figure 4 - Simplified measurement method for radiated disturbances

Appendix A
(to TSB Circular 83)

Questionnaire on EM-environment

## 1. Company or organization name:

## 2. Country:

## 3. Contact person:

## 4. EMC trouble in home network (including telecommunication services)

4.1 Have you had an experience?

- Yes

- No

4.2 If “yes”, please fill the information in the table in Appendix A.

* Please feel free to add additional pages for additional information.

## 5. EM environment in home

5.1 Have you ever measured an EM environment in Home?

- Yes

- No

5.2 If “yes”, please fill the information in the table in Appendix A.

* Please feel free to add additional pages for additional information.

Sheet for collecting information on EM-environment

| Title |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sort of trouble (If experienced) |  |  |  |  | Acoustic noise, malfunction, disturbance, other |  |  |  |
|  |  |  |  |  | More detail : |  |  |  |
| Environment |  | Type |  |  | 1. Detached house, 2. Semi-detached house 3. User building (used by one user / used by multiple users) |  |  |  |
|  |  | Use |  |  | 1. Telecom. centre, 2. Data centre, 3. Office, 4. Residence, 5. Others ( ) |  |  |  |
| Situation, configuration, measured data, and so on (Please add some figures, if necessary) |  |  |  |  |  |  |  |  |
| Source of EM interference (If clarified) |  |  |  |  |  |  |  |  |
| Type of the interference |  |  |  | Characteristics of the interferences |  |  |  |  |
|  |  |  |  | Type |  | Frequency ( band) | Level | Others |
| Conducted | Voltage or Current |  |  | Continuous |  | Hz | [ ] |  |
|  |  |  |  | Impulsive |  | Hz | [ ] |  |
| Radiated | Electromagnetic wave(field) |  |  | Continuous |  | Hz | [ ] |  |
|  |  |  |  | Impulsive |  | Hz | [ ] |  |

___________________