| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On "From/In/To Cars Communication II" |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | FG CarCom-C-13 |
|  |  | English only Original: English |
|  |  | Geneva, 6 March 2009 |
| CONTRIBUTION |  |  |
| Source: | Technische Universität Braunschweig |  |
| Title: | Background Information to the Wideband Digital Interface |  |

Background Information to the Wideband Digital Interface

In this contribution we want to provide some background information to the Digital Interface Concept, section 8 of the current draft specification. In particular, the optimum choice of parameters for signal separation in the test case section 8.2.4 “Speech Distortion in Double Talk” is given. Additionally, for information, some example measurements of MOS-LQOW are given for two example systems under test.

Signal separation parameters (section 8.2.4)

As concerns the signal separation methodology in reference [3], we worked on the respective optimization for wideband signals. A document is attached as pdf, the relevant text portion is highlighted in yellow. The parameters window, window size, and frame shift are already updated and correct in the new draft recommendation. In addition, in Table 1 of the attached document, a consistently identical test result is found using MOS-LQOW using the signal separation technique (“black box” test), a white box test, or a subjective CMOS test. These results again justify the use of the proposed technique for system comparison using MOS-LQOW (Wideband PESQ).

The document should – in addition to the basic approach [3] – be referenced in the draft recommendation:

[3b]	Steinert, K., Suhadi, S., Fingscheidt, T., A Comparison of Instrumental Measures for Wideband Speech Quality Assessment of Hands-free Systems in Echoic Condition, DAGA 2009, Rotterdam, The Netherlands, March 2009.

Example MOS-LQOW Measurements

As concerns the concrete values of MOS-LQOW in the (optional) requirements in section 8.2.3 and 8.2.4 we performed a number of tests for two example wideband handsfree system implementations. They are briefly described in the DAGA paper attached above.

The curves show the MOS-LQOW results of the two systems at digital interface access point S1. The black curves are related to one way speech component (!) quality in single-talk (i.e., signal-to-echo ratio -> infinity), while the colour curves show speech component quality in double talk (blue: SER=10 dB down to red: SER = 0 dB).

It clearly turns out that both systems show an MOS-LQO at around 4.4 for clean conditions (no noise, no echo). Even in the noisy case (car noise) down to SNR = 0 dB both systems offer MOS-LQOW(S1) much larger than the current requirement of 3.6.

For the double talk case, even in heavy noise, both systems offer a speech component quality that is larger than 2,5 in MOS-LQOW(S1).

Considering the fact, that the requirements in section 8 only relate to the speech component (not to the enhanced signal in sending direction with residual echo), and given the fact that the requirements in section 8 are related to noise-free conditions only, and given the fact that the shown two example systems show a MOS-LQOW(S1) > 3.0 for all tested SER values, the requirement of 2,5 in double talk definitely does not seem too hard to reach.

Likewise the requirement of 3,6 in single talk does not seem to be too strong, since the shown two example systems exceed it by 0.8. In both cases, double talk and single talk, the margin of MOS-LQOW(S1) towards the requirements values should comfortably allow for the additional degradation introduced by the measurement at POI.