| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On "From/In/To Cars Communication II" |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | FG CarCom-C-19 |
|  |  | English only Original: English |
|  |  | Paris, 17-18 June 2009 |
| CONTRIBUTION |  |  |
| Source: | Chairman of the Focus Group |  |
| Title: | Input from Mats Forsen / H W Gierlich on “Ambient Background noise” |  |

Suggestion for requirements on background noise levels during measurements.

The following text is to be added in the specification.

-------

7.1.8	Environmental Conditions

Depending on tests, different requirement applies for background noise levels inside the car.

Unless specified otherwise the background noise level shall be less than -54 dBPa(A).

For specified  tests it is desirable to have a background noise level of less than -74 dBPa(A), but the minimum background noise level of -64dBPa(A) should never be exceeded.

Table xx Maximum background noise levels, SPL(dB)

<Levels to be verified>

| Frequency | Maximum upper limit unless specified otherwise | Maximum upper limit at specific tests | Recommended upper limit at specific tests |
| --- | --- | --- | --- |
| 100 Hz | 60 dB | 50 dB | 40 dB |
| 250 Hz | 50 dB | 40 dB | 30 dB |
| 500 Hz | 40 dB | 30 dB | 20 dB |
| 1 000 Hz | 40 dB | 30 dB | 20 dB |
| 2 000 Hz | 40 dB | 30 dB | 20 dB |
| 4 000 Hz | 45 dB | 35 dB | 25 dB |
| 8 000 Hz | 50 dB | 40 dB | 30 dB |
| 10 000 Hz | 50 dB | 40 dB | 30 dB |

Figure 1, Background noise level inside car.

------------

Some background

This requirement is mainly a requirement on the lab/chamber where the measurements are done. It seems that the “standard” way of measuring background noise levels inside an anechoic chambers is by using dB SPL,  ISO 3745.

HW Gierlich: Perhaps we also could have a look to the EBU std. EBU Tech. 3276 where we have noise criterions such as NR 10, NR 15.

One examples:

http://www.me.mtu.edu/courses/meem4704/project/papers/nc05_chamber.pdf