| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On Car Communication |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | FG CarCOM-C-8 |
|  |  | English only Original: English |
|  |  | Detroit, 26-27 August 2010 |
| CONTRIBUTION |  |  |
| Source: | Chairman of the Focus Group |  |
| Title: | Issues for clarification in P.1110 |  |

Issues  in the P.1110 Specification

|  | Diffuse-field Equalization | Exact Description should be : for headset measurement the Diffuse-field average equalization according to ITU-T Recommendation P.58 is valid. For Speakerphone the individual equalization should be used. |
| --- | --- | --- |
|  | Appendix I: TOSQA | Appendix I: TOSQA should be measured with “high quality handset” and “wideband”; but there is just the parameter “headset (wideband)”; parameter “high quality handset” describes the “narrowband handset”. |
|  | ### 11.7.2 Idle Channel Noise in Receive Direction | 11.7.2.2. item 2, last sentence; P.1110 idle channel noise should be carried out by DRP/ERP correction according to the specification – needs to be changed to DF correction. |
|  | ### 11.8.1 Discrimination against Out-of-Band Signals in Send Direction | 11.8.1.2 item 5, 3rd sentence: “The signal level is determined in the frequency range from 300 Hz to 3.4 kHz at the electrical reference point (POI).” 2 indications of measurements in-band leve; one was updated to 200 Hz … 7 kHz, the other one is still indicated as described in P.1100 NB (300 Hz … 3.4 kHz). |
|  | ### 11.8.2 Spurious Out-of-Band Signal in Receive Direction | 11.8.2 Spurious Out of band RCV: “The out-of-band signal with the in-band signal present shall be at least 10 dB below the signal level (reference signal level) in the same frequency range with no in-band signal present. “ It should be reverse: “The out-of-band signal with the in-band signal present shall be no more than 10 dB above the signal level (reference signal level) in the same frequency range with no in-band signal present. “ |
|  | ## 11.10 Distortion in Receive Direction | 1. sentence: The distortion is measured from POI to the artificial ear up to 15 khz. Measurements are carried out with a measurement microphone. |
|  | ## 11.11.4b Initial Convergence without BGN with Artificial Voice | The measurement does not show enough relevant results, as the level of the artificial voice signal varies so much that the tolerance scheme cannot be fulfilled. |
|  | ## 11.11.8.2 Activation in Receive Direction | “For headset hands-free terminals the sound pressure measured at the DRP and corrected to the ERP according to ITU-T Recommendation P.57 [10]. “ Fault for headset measurements, according to the specification it should be measured with DRP/ERP correction – to be changed to DF. |
|  | ## 11.13.2 BGN Transmission after Call Setup | Requirement: “should not cause higher excitation than 15 cp/cPa between 300 hz and 3.400 HZ” Probably wrong frequency range |

_______________