Annex X

Reference-free SNR Measurement

# Introduction

This annex describes an approach to measure the SNR of a speech signal distorted by car noise as close as possible to the reference SNR from ITU-T Recommendation P.56 [2], but without using any reference signals based on [1]. This “reference-free” nature allows for a wide flexibility; the proposed method just has to be applied to the noisy speech signal. Moreover, it offers low complexity, and can be applied both to narrowband and wideband signals.

To measure the SNR of the noisy speech signal in this approach, first the speech and noise power have to be estimated. Because of the additive nature of the noise, the speech power is approximated by the difference between the noisy speech power and the noise power. The noise power is estimated by a noise variance tracking algorithm. To estimate the P.56 active speech level only speech active frames are taken for the speech power estimation. Therefore, a voice activity detection (VAD) is needed. The power of the noise component is estimated by means of the noise tracking algorithm and—in order to ensure a robust estimation—a speech pause detection (SPD). Then, the measured SNR is the ratio of the speech and noise power estimates. Since the effectiveness of the employed algorithms (such as noise variance tracking, VAD, and SPD) decreases at low reference SNR values, the raw SNR measurement leads to systematical measurement errors in this region. Therefore, the resulting raw SNR values will be corrected by a mapping curve.

# Signal-to-Noise Ratio Measurement

The input signal y(n) of the measurement system is assumed to consist of the clean speech signal s(n) and an additive noise signal n(n), with n being the discrete time index. After segmentation, windowing, and the discrete Fourier transform (DFT), the input signal can be rewritten as Y(l, k) = S(l, k)+N(l, k) with Y, S, N, as well as l, and k being the short-time spectra of the noisy speech, the clean speech, the noise component, as well as the analysis frame index, and the frequency bin index, respectively. We define the signal-to-noise ratio (SNR) as	
	SNR=10log10PSPN [dB],	(1)

with log10(.), PS, PN being the logarithm to the base 10, the speech and the noise power, respectively.

Since PS and PN are not accessible in practice, they have to be estimated. Exploiting the additive nature of the noise, the numerator is estimated with contributions PSl,k=maxY(l,k)2-σN2(l,k), where Y(l,k)2 and σN2(l,k) are the power spectral density (PSD) of the noisy speech signal and the estimated noise variance (see Section 3), respectively. The maximum operator ensures a non-negative speech power estimate. Conforming to the definition of the active speech level [2], for the estimation of PS in (1) only speech active frames are taken as

PS=1Λ1KlϵΛ1kmaxY(l,k)2-σN2l,k,0,	(2)

with Λ1, Λ1, and K being the set of active speech frames detected by a VAD as a part of the noise tracking algorithm (see Section 3), the number of elements in Λ1, and the number of all frequency bins, respectively.

The precise estimation of the noise power PN is essential for the SNR estimation. However, the accuracy of the noise variance tracking algorithm is affected especially at low SNRs. In order to increase the robustness of the approach, for the estimation of PN in (1), the estimated noise variance σN2(l,k) is averaged over all frequency bins k and those frames l belonging to speech pause	
	
	PN=1Λ0KlϵΛ0kσN2l,k,	(3)

with Λ0 and Λ0 being the set of speech pause frames detected by a separate conservative SPD described in Section 4, and the number of elements in Λ0, respectively.

# Noise Variance Tracking and Voice Activity Detection

The estimation of the noise variance σN2(l,k) in (2) is performed utilizing a 3–state classifier specific to this estimator. Based on the smoothed periodogram of the noisy speech signal

Y(l,k)2=0.5Y(l-1,k)2+0.5Y(l,k)2	(4)

and a dynamic threshold Θ(l,k), one hypothesis H(l,k) out of three is chosen:

Hsp:	Speech presence is assumed if Y(l,k)2>2Θl,k.

Hsa:	Speech absence is assumed if Y(l,k)2≤2Θl,k ∧ Y(l,k)2<σN2l-1,k.

Hst:	Speech transition is assumed if Y(l,k)2≤2Θl,k ∧ Y(l,k)2≥σN2l-1,k.

The noise variance estimate σN2(l,k) is updated as

σN2(l,k)=ε(l,k)σN2l-1,k+1-ε(l,k)Y(l,k)2	(5)

where the initial value is σN2(l=0,k)=0 and the time-varying smoothing factor ε(l,k) depends on the hypothesis of the current frame H(l,k):

εl,k=10.50.875  ififif   Hl,k=Hsp,Hl,k=Hsa,Hl,k=Hst.	(6)

The update of the dynamic threshold Θ(l,k) is performed as follows

Θl,k=ΔΘ(l-1,k)Y(l-1,k)2Θl-1,k  ififelse,   Y(l,k)2>2Θl-1,k,Y(l,k)2<Θl-1,k, 	(7)

where the multiplication by the step-size constant Δ=1.07 for narrowband signals (Δ=1.2 for wideband signals) slowly but steadily decreases the probability of selecting the speech presence hypothesis. The unsmoothed update to the last periodogram value Y(l-1,k)2 allows for rapid threshold adaptation in speech pauses. The control parameter is initialized as Θl=0,k→∞.

The frame-wise VAD is based on the hypotheses of the noise variance tracking algorithm as introduced above. Therefore, it was integrated into the noise variance tracking algorithm. Frame l is detected as voice active and becomes element of set Λ1, if at least 90 % of its frequency bins from the range [500 Hz 2500 Hz] containing relevant speech information are classified as speech active Hsp or transient Hst by the 3–state classifier.

# Speech Pause Detection

This section describes the algorithm which is employed to detect frames with speech pause Λ0. Please note that different to the VAD, this is a separate algorithm which is independent from the noise power tracking. The SPD is based on the frame energy of the noisy speech spectrum calculated by means of (4) as follows

PY(l)=1Kk∈KY(l,k)2	(8)

with K and K being the set of frequency bins between 500 Hz and 2500 Hz and the number of elements in this set, respectively. This frequency range excludes low frequencies where most of the car noise power is concentrated. Based on an adaptive threshold Ξ(l), the SPD delivers a hypothesis HSPD(l) based on the following 3 states:

HSP:	Speech presence is assumed if PY(l)>Ξ(l).

HST:	Speech transition state follows every HSP decision with a duration of Ltrans frames, unless threshold Ξl has been exceeded again.

HSA:	Speech pause is assumed in all other situations.

Frame l is considered as speech absent and becomes element of Λ0, if HSPDl=HSA. The adaptive threshold Ξ(l) is calculated as

Ξl=5Φl-1+α,	(9)

with the additive term α. The SPD floor signal Φl is updated recursively as follows (cf. (5))

Φl=β(l)Φl-1+1-β(l)PY(l),	(10)

where the initial value is Φl=0=0 and the time-varying smoothing factor β(l) is defined as follows (cf. (6))

βl=0.8750.51  ififelse,   A ∧ PYl>Φl-1,A ∧ PYl≤Φl-1, 	(11)

with A=PYl≤2Υl-1 ∧ HSPDl≠HST ensuring a more conservative floor signal update in speech pauses than just conditioning on HSA.

The SPD control parameter Υl is increased slightly in speech presence and updated strongly during speech absence as (cf. (7))

Υl=δΥ(l-1)PYlΥ(l-1)  ififelse,   PYl>2Υl-1,PYl<Υl-1, 	(12)

with the control update constant δ. The control parameter is initialized as Υl=0→∞.

The SPD parameters which differ for the narrowband and the wideband implementation are summarized in Tab 1.

| Parameter | Ltrans | α | δ |
| --- | --- | --- | --- |
| fs=8 kHz | 6 | 107 | 1.085 |
| fs=16 kHz | 7 | 108 | 1.055 |

Tab 1 Parameters of the SPD

# Measurement Setup

At a sampling frequency of 8 kHz (16 kHz), the noisy speech signal y(n)  has to be segmented by a Hann window, an analysis frame length of 256 (512) samples, and an analysis frame shift of 50% (50%) to obtain Y(l, k). The raw SNR value can be calculated as follows

SNRraw=10log10PSPN [dB],	(13)

using (2), the noise variance tracker, and the VAD as described in Section 3, as well as (3), the noise variance tracker from Section 2, and the SPD as introduced in Section 4. This raw SNR value has to be corrected by the following mapping function in order to obtain an unbiased SNR measurement

SNR=maxj=010pjSNRraw-msj,-11 [dB],	(14)

with the scaling parameters m and s, as well as the polynomial coefficients pj as shown in Tab 2.

| fs | m | s | pj |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | p0 | p1 | p2 | p3 | p4 | p5 | p6 | p7 | p8 | p9 | p10 |
| 8 kHz | 16.043 | 11.252 | -2.1312 | 6.4129 | 2.0957 | -19.199 | 5.0992 | 19.709 | -7.7268 | -6.6348 | 2.0857 | 13.066 | 11.555 |
| 16 kHz | 15.461 | 11.798 | -0.8082 | 2.8537 | -0.3609 | -7.5337 | 4.3304 | 7.2828 | -5.0623 | -1.8734 | 0.8842 | 13.06 | 11.48 |

Tab 2 Scaling parameters m and s as well as the polynomial coefficients pj of the mapping function

The approach is summarized by means of a pseudocode in Tab 3.

... % parameter initialization for signal segmentation, FFT, VAD, SPD, etc. for l=1:L, % main loop over all frames ... % signal analysis: segmentation, windowing, FFT, absolute square (4), (7), (6), (5) % 3-state noise variance tracking, VAD (4), (8), (9), (12), (11), (10) % SPD end (2) % speech power estimation (3) % noise power estimation (13) % raw SNR (14) % corrected SNR

Tab 3 Overview of the approach by means of pseudocode

# References

[1]	Fodor, B.; Fingscheidt, T., “Reference-free SNR Measurement for Narrowband and Wideband Speech Signals in Car Noise”, In Proc. 10th ITG Conference on Speech Communication, Braunschweig, Germany, Sep. 26-28, 2012, available online at IEEE Xplore

[2]	ITU-T, “Objective Measurement of Active Speech Level”, ITU-T Recommendation P.56,        Mar. 1993