| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On Car Communication |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | FG CarCOM-C-45 |
|  |  | English only Original: English |
|  |  | Aachen, 12-13 November 2012 |
| CONTRIBUTION |  |  |
| Source: | FG Chairman |  |
| Title: | Draft 19 of FG.VSSR |  |

FG.VSSR – Draft 19

FG CarCom SubSystem Requirements for Automotive Speech Services

Summary

This Specification defines test methodologies for and standard behaviour of subsystems used in automotive speakerphone terminals. The purpose is to provide guidance on the design and optimization of subsystems as well as diagnostic capabilities needed to give a consistent and high Quality of Service of the overall speakerphone terminal for the users of such devices. This specification is intended to give guidance to all parties involved in the design and integration of speakerphone terminals. This specification covers both, narrowband and wideband systems.

Keywords

Hands-Free, Speakerphone, Motor Vehicles, Subsystem, Quality of Service, Performance, Performance

# Contents

Table of content

Contents	2

1)	Scope	9

2)	References	9

3)	Definitions	11

4)	Abbreviations	12

5)	Conventions	15

6)	How to Use the Specification	16

7)	Architectures	16

7.1	Distributed  speakerphone	17

7.2	System with integrated microphone	18

7.3	System with integrated speaker	18

7.4	System with integrated microphone and speaker	19

7.5	System Delay, Subsystem Delay and Buffering	19

8)	Subsystems	20

8.1	Test Interface description	20

8.1.1	Analog electrical interface	20

8.1.2	Digital electrical interface	20

8.1.3	Acoustic interface	20

8.1.4	Wireless interface	21

8.1.5	Test signal levels	21

8.2	Acoustic Subsystem	21

8.2.1	Microphone subsystem (send path)	22

8.2.2	Audio subsystem (receive path)	39

8.2.3	Coupling between acoustical subsystems (echo path)	54

8.3	Signal Enhancement Subsystem	55

8.3.1	Test set-up	55

8.3.2	Round-Trip Delay of  Signal Enhancement	57

8.3.3	Signal enhancement delay in send direction	58

8.3.4	Signal enhancement delay in receive direction	59

8.3.5	Signal enhancement send junction loudness ratings	59

8.3.6	Receive signal enhancement loudness ratings	60

8.3.7	Signal enhancement send sensitivity frequency response	61

8.3.8	Signal enhancement receive sensitivity frequency response	68

8.3.9	Signal Enhancement Send Speech Quality (MOS-LQO-SE)	74

8.3.10	Signal Enhancement Receive Speech Quality (MOS-LQO-RE)	75

8.3.11	Signal Enhancement Send Speech Quality Stability (MOS-LQO-STB-SE)	75

8.3.12	Signal Enhancement Receive Speech Quality Stability (MOS-LQO-STB-RE)	76

8.3.13	Signal Enhancement Send Intelligibility (SIE)	76

8.3.14	Signal Enhancement Receive Intelligibility (RIE)	77

8.3.15	Signal Enhancement Send Idle Channel Noise (SICNE)	77

8.3.16	Signal Enhancement Receive Idle Channel Noise (RICNE)	77

8.3.17	Discrimination against out-of-band signals after Send Enhancement (DOSE)	78

8.3.18	Spurious out-of-band signal after Receive Enhancement (SORE)	78

8.3.19	Distortion in Send after Enhancement (DSE)	79

8.3.20	Distortion in Receive after Enhancement (DRE)	79

8.3.21	Signal Enhancement Weighted Terminal Coupling Loss (TCLwE)	80

8.3.22	Signal Enhancement Echo Level Versus Time (ELVTE)	80

8.3.23	Signal Enhancement Spectral Echo Attenuation (SEAE)	81

8.3.24	Signal Enhancement Initial Convergence without background noise (ICE)	81

8.3.25	Signal Enhancement Initial Convergence With Background Noise (ICBNE)	82

8.3.26	Signal Enhancement Echo Performance with Time Variant Echo Path (TVEPE)	82

8.3.27	Signal Enahncement Echo Performance with Time Variant Echo Path and Artificial Voice (TVEPE-AV)	83

8.3.28	Signal Enhancement Send Activation (SAE)	83

8.3.29	Signal Enhancement Receive Activation (RAE)	84

8.3.30	Signal Enhancement Send Attenuation Range (SARE)	84

8.3.31	Signal Enhancement Receive Attenuation Range (RARE)	84

8.3.32	Signal Enhancement Attenuation Range in Send during Double-Talk (ASDTE)	85

8.3.33	Signal Enhancement Attenuation Range in Receive during Double-Talk (ARDTE)	85

8.3.34	Detection of Echo Components during Double-Talk after Enhancement (DECDTE)	86

8.3.35	Signal Enhancement Sent Speech Attenuation during Double-Talk (SSADTE)	86

8.3.36	Signal Enhancement Speech-to-Echo Ratio (SpERE)	87

8.3.37	SNR Improvement after Enhancement (SNRIE)	87

8.3.38	Signal Enhancement Background Noise Transmission After Call Set-up (BGNTACSE)	87

8.3.39	Enhanced Speech Quality in the Presence of Background Noise (SQPBGNE)	88

8.3.40	Enhanced Quality of Background Noise Transmission with Far End Speech (QBGNTFSE)	88

8.3.41	Enhanced Quality of Background Noise Transmission with Near End Speech (QBGNTNSE)	89

8.3.42	Comfort Noise Injection after Enhancement (CNIE)	89

8.3.43	Speech Recognition Accuracy Indicator after Enhancement (SRAIE)	90

8.3.44	Send path	90

8.3.45	Echo path	90

8.3.46	Receive path	90

8.4	Network Access Subsystem	90

8.4.1	Send path	90

8.4.2	Echo path	91

8.4.3	Receive path	91

8.4.4	Send-receive path coupling	91

9)	Bidirectional Signal Transport: Wired and Short range Wireless Transmission	91

9.1	Bidirectional transport delay in send direction	93

9.1.1	Parameter description	93

9.1.2	Test	93

9.1.3	Performance level classification based on values of this parameter	94

9.1.4	Design guidance and root-cause analysis	94

9.2	Bidirectional transport delay in receive direction	94

9.2.1	Parameter description	94

9.2.2	Test	95

9.2.3	Performance level classification based on values of this parameter	95

9.2.4	Design guidance and root-cause analysis	95

9.3	Bidirectional transport send junction loudness ratings	96

9.3.1	Parameter description	96

9.3.2	Test	96

9.3.3	Performance level classification based on values of this parameter	96

9.3.4	Design guidance and root-cause analysis	96

9.4	Bidirectional transport receive loudness ratings	97

9.4.1	Parameter description	97

9.4.2	Test	97

9.4.3	Performance level classification based on values of this parameter	97

9.4.4	Design guidance and root-cause analysis	98

9.5	Bidirectional transport linearity in send direction	98

9.5.1	Parameter description	98

9.5.2	Test	98

9.5.3	Performance level classification based on values of this parameter	98

9.5.4	Design guidance and root-cause analysis	99

9.6	Bidirectional transport linearity in receive direction	99

9.6.1	Parameter description	99

9.6.2	Test	99

9.6.3	Performance level classification based on values of this parameter	99

9.6.4	Design guidance and root-cause analysis	100

9.7	Bidirectional transport send sensitivity frequency response	100

9.7.1	Parameter description	100

9.7.2	Test	100

9.7.3	Performance level classification based on values of this parameter	100

9.7.4	Design guidance and root-cause analysis	102

9.8	Bidirectional transport receive sensitivity frequency response	103

9.8.1	Parameter description	103

9.8.2	Test	103

9.8.3	Performance level classification based on values of this parameter	103

9.8.4	Design guidance and root-cause analysis	105

9.9	Bidirectional transport idle channel noise in send	105

9.9.1	Parameter description	105

9.9.2	Test	105

9.9.3	Performance level classification based on values of this parameter	106

9.9.4	Design guidance and root-cause analysis	106

9.10	Bidirectional transport idle channel noise in receive	107

9.10.1	Parameter description	107

9.10.2	Test	107

9.10.3	Performance level classification based on values of this parameter	107

9.10.4	Design guidance and root-cause analysis	108

9.11	Bidirectional transport noise cancellation test in send direction	108

9.11.1	Parameter description	108

9.11.2	Test	108

9.11.3	Performance level classification based on values of this parameter	108

9.11.4	Design guidance and root-cause analysis	109

9.12	Bidirectional transport one-way speech quality in send direction	109

9.12.1	Parameter description	109

9.12.2	Test	109

9.12.3	Performance level classification based on values of this parameter	110

9.12.4	Design guidance and root-cause analysis	110

9.13	Bidirectional transport speech quality stability in send direction	110

9.13.1	Parameter description	110

9.13.2	Test	110

9.13.3	Performance level classification based on values of this parameter	112

9.13.4	Design guidance and root-cause analysis	112

9.14	Bidirectional transport one-way speech quality in receive direction	113

9.14.1	Parameter description	113

9.14.2	Test	113

9.14.3	Performance level classification based on values of this parameter	113

9.14.4	Design guidance and root-cause analysis	114

9.15	Bidirectional transport speech quality stability in receive direction	114

9.15.1	Parameter description	114

9.15.2	Test	114

9.15.3	Performance level classification based on values of this parameter	114

9.15.4	Design guidance and root-cause analysis	114

9.16	Verification of bidirectional transport disabled echo control	115

9.16.1	Parameter description	115

9.16.2	Test	115

9.16.3	Performance level classification based on values of this parameter	116

9.16.4	Design guidance and root-cause analysis	117

10)	Unidirectional Signal Transport: Wired and Short range Wireless Transmission	117

10.1	Test setup	117

10.2	Unidirectional transport delay 	118

10.2.1	Parameter description	118

10.2.2	Test	118

10.2.3	Performance level classification based on values of this parameter	118

10.2.4	Design guidance and root-cause analysis	119

10.3	Unidirectional transport junction loudness ratings	119

10.3.1	Parameter description	119

10.3.2	Test	119

10.3.3	Performance level classification based on values of this parameter	119

10.3.4	Design guidance and root-cause analysis	120

10.4	Unidirectional transport linearity	120

10.4.1	Parameter description	120

10.4.2	Test	120

10.4.3	Performance level classification based on values of this parameter	120

10.4.4	Design guidance and root-cause analysis	121

10.5	Unidirectional transport sensitivity frequency response	121

10.5.1	Parameter description	121

10.5.2	Test	121

10.5.3	Performance level classification based on values of this parameter	121

10.5.4	Design guidance and root-cause analysis	123

10.6	Unidirectional transport idle channel noise	124

10.6.1	Parameter description	124

10.6.2	Test	124

10.6.3	Performance level classification based on values of this parameter	124

10.6.4	Design guidance and root-cause analysis	125

10.7	Unidirectional transport noise cancellation test	125

10.7.1	Parameter description	125

10.7.2	Test	125

10.7.3	Performance level classification based on values of this parameter	125

10.7.4	Design guidance and root-cause analysis	126

10.8	Unidirectional transport speech quality	126

10.8.1	Parameter description	126

10.8.2	Test	126

10.8.3	Performance level classification based on values of this parameter	126

10.8.4	Design guidance and root-cause analysis	127

10.9	Unidirectional transport speech quality stability	127

10.9.1	Parameter description	127

10.9.2	Test	127

10.9.3	Performance level classification based on values of this parameter	128

10.9.4	Design guidance and root-cause analysis	129

10.10	System with integrated microphone and speaker	129

11)	Microphone sensitivity (in anechoic conditions)	130

11.1	Parameter description	130

11.2	Test	130

11.3	Performance level classification based on values of this parameter	130

11.4	Design guidance and root-cause analysis	131

12)	Microphone frequency response (in anechoic conditions)	131

12.1	Parameter description	131

12.2	Test	131

12.3	Performance level classification based on values of this parameter	131

12.4	Design guidance and root-cause analysis	134

13)	Microphone directional characteristics (in anechoic conditions)	134

13.1	Parameter description	134

13.2	Test	134

13.3	Performance level classification based on values of this parameter	134

13.4	Design guidance and root-cause analysis	135

14)	Microphone distortion (in anechoic conditions)	135

14.1	Parameter description	135

14.2	Test	135

14.3	Performance level classification based on values of this parameter	135

14.4	Design guidance and root-cause analysis	136

15)	Microphone maximum sound pressure level (in anechoic conditions)	136

15.1	Parameter description	136

15.2	Test	136

15.3	Performance level classification based on values of this parameter	136

15.4	Design guidance and root-cause analysis	137

16)	Microphone self-noise (in anechoic conditions)	137

16.1	Parameter description	137

16.2	Test	137

16.3	Performance level classification based on values of this parameter	137

16.4	Design guidance and root-cause analysis	138

1.	Introduction	145

2.	Signal-to-Noise Ratio Measurement	145

3.	Noise Variance Tracking and Voice Activity Detection	146

4.	Speech Pause Detection	147

5.	Measurement Setup	148

References	149

X.1	General	150

X.2	Frame interval	150

X.3	Delay	150

X.4	Arbitration between two subsystems having different frame intervals	151

Case 1)	Sender’s and receiver’s frame intervals are identical	151

Case 2)	Sender’s frame interval is a multiple of receiver’s frame interval	151

Case 3)	Receiver’s frame interval is a multiple of sender’s frame interval	153

Case 4)	Other cases	153

X.5	Example	154

# Scope

The aim of this Specification is the definition of test methods and requirements for subsystems of automotive narrowband and wideband speakerphone terminals and other speech services using the speakerphone acoustic interface.  The Specification covers

- Definition of subsystems based on different system architectures

- Performance requirements for subsystems

- Diagnostic information

- Guidance on component and subsystem optimization

- Coordination of subsystems

- Both, narrowband and wideband systems

- Acoustic interface requirements for other speech services such as speech recognition and application prompt playback

The methods, the analysis and the performance parameters described in this Specification are based on test signals and test procedures as defined in ITU-T Recommendations P.50 [10], P.501 [11], P.502 [12] and P.340 [18], P.380 [19], P.1100[xx], P.1110 [xx] and ETSI ES 202 739 [24] and ETSI ES 202 740 [25].

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation

[1]	Berger, J., Results of objective speech quality assessment including receiving terminals using the advanced TOSQA2001, ITU-T Contribution, Dec. 2000, COM 12-20-E

[2]	ETSI EG 202 396-1: Speech quality performance in the presence of background noise; Part 1: Background noise simulation technique and background noise database.

[3]	Fingscheidt, T., Suhadi, S.: Quality Assessment of Speech Enhancement Systems by Separation of Enhanced Speech, Noise, and Echo, INTERSPEECH 2007, Antwerp, Belgium, Aug. 2007.

[4]	Steinert, K., Suhadi, S., Fingscheidt, T., A Comparison of Instrumental Measures for Wideband Speech Quality Assessment of Hands-free Systems in Echoic Condition, DAGA 2009, Rotterdam, The Netherlands, March 2009.

[5]	IEC 61260: Electroacoustics - Octave-band and fractional-octave-band filters - Part 1: Specifications, 1995.

[6]	IEC 60268-4: Sound system equipment - Part 4: Microphones, 2004.

[7]	ITU-T Recommendation G.122: Influence of National Systems on Stability and Talker Echo in International Connections.

[8]	ITU-T Recommendation P.340: Transmission Characteristics and Speech Quality Parameters of Hands-free Telephones.

[9]	ITU-T Recommendation P.380: Electroacoustic Measurements on Headsets

[10]	ITU-T Recommendation P.50 (1993) : Artificial Voices.

[11]	ITU-T Recommendation P.501: Test Signals for Use in Telephonometry.

[12]	ITU-T Recommendation P.502: Objective Analysis Methods for Speech Communication Systems, Using Complex Test Signals.

[13]	ITU-T Recommendation P.56: Objective Measurement of Active Speech Level.

[14]	ITU-T Recommendation P.57 : Artificial Ears.

[15]	ITU-T Recommendation P.58: Head and Torso Simulators for Telephonometry.

[16]	ITU-T Recommendation P.581 (05/00): Use of Head and Torso Simulators (HATS) for Hands-free Terminal Testing.

[17]	ITU-T Recommendation P.79: Calculation of Loudness Rating for Telephone Sets.

[18]	ITU-T Recommendation P.800: Methods for Subjective Determination of Transmission Quality.

[19]	ITU-T Recommendation P.800.1: Mean Opinion Score (MOS) Terminology.

[20]	ITU-T Recommendation P. 862.2: Wideband extension to Recommendation P.862 for the assessment of wideband telephone networks and speech codecs.

[21] 	ITU-T Recommendation P. 862.1: Mapping Function for Transforming P.862 Raw Result Scores to MOS-LQO.

[22]	Sottek, R., Genuit, K.: Models of Signal Processing in Human Hearing, International Journal of Electronics and Communications, pp. 157-165, 2005.

[23]	Kettler, F., Gierlich, H.W.: Evaluation of Hands-Free Terminals, in “Topics in Speech and Audio Processing” edited by E. Hänsler, G. Schmidt, Springer, ISBN: 978-3-540-70601-4.

[24]	ETSI ES 202 739: Transmission requirements for wideband VoIP terminals (handset and headset) from a Performance perspective as perceived by the user

[25]	ETSI ES 202 740: Transmission requirements for wideband loudspeaking and hands-free terminals from a Performance perspective as perceived by the user

[26]	ETSI EG 202 396-3: Speech quality performance in the presence of background noise; Part 3: Background noise transmission - objective model.

[27]	ISO 3745: Acoustics -- Determination of sound power levels of noise sources using sound pressure -- Precision methods for anechoic and hemi-anechoic rooms

P.1100

P.1110

G. 100.1

# Definitions

Artificial ear: Device incorporating an acoustic coupler and a calibrated microphone for the measurement of the sound pressure and having an overall acoustic impedance similar to that of the median adult human ear over a given frequency band.

Codec: Combination of an analogue-to-digital encoder and a digital-to-analogue decoder operating in opposite directions of transmission in the same equipment.

Composite Source Signal (CSS): Signal composed in time by various signal elements.

Diffuse field equalization: Equalization of the HATS sound pick-up, equalization of the difference, in dB, between the spectrum level of the acoustic pressure at the ear Drum Reference Point (DRP) and the spectrum level of the acoustic pressure at the HATS Reference Point (HRP) in a diffuse sound field with the HATS absent using the reverse nominal curve given in Table 3 of ITU-T Recommendation P.58 [15].

Ear-Drum Reference Point (DRP): Point located at the end of the ear canal, corresponding to the ear-drum position.

Freefield reference point: Point located in the free sound field, at least in 1,5 m distance from a sound source radiating in free air (in case of a head and torso simulator [HATS] in the center of the artificial head with no artificial head present).

Freefield equalization: The transfer characteristic of the artificial head is equalized in such a way that for frontal sound incidence in anechoic conditions the frequency response of the artificial head is flat. This equalization is specific to the HATS used.

Hands-Free Reference Point (HFRP): A point located on the axis of the artificial mouth, at 50 cm from the outer plane of the lip ring, where the level calibration is made, under free-field conditions. It corresponds to the measurement point 11, as defined in ITU-T Rec. P.51.

Hands-free terminal: Telephone set that does not require the use of hands during the communications session; examples are headset, speakerphone and group-audio terminal.

Head And Torso Simulator (HATS) for telephonometry: Manikin extending downward from the top of the head to the waist, designed to simulate the sound pick-up characteristics and the acoustic diffraction produced by a median human adult and to reproduce the acoustic field generated by the human mouth.

Headset: Device which includes telephone receiver and transmitter which is typically secured to the head or the ear of the wearer.

MOS-LQO (Mean Opinion Score – Listening-only Quality Objective): The score is calculated by means of an objective model which aims at predicting the quality for a listening-only test situation. Objective measurements made using the model given in ITU-T Rec. P.862 give results in terms of MOS-LQO (for further information see Annex A).

MOS-TQO (Mean Opinion Score – Talking Quality Objective): The score is calculated by means of an objective model which aims at predicting the quality for a talking-only test situation. Methods generating a MOS-TQO are currently under development and not yet standardized.

Mouth Reference Point (MRP): The MRP is located on axis and 25 mm in front of the lip plane of a mouth simulator.

Nominal setting of the volume control: When a receive volume control is provided, the setting which is closest to the nominal RLR of 2 dB.

Receive loudness rating (RLR): The loudness loss between an electric interface in the network and the listening subscriber's ear. (The loudness loss is here defined as the weighted (dB) average of driving e.m.f. to measured sound pressure.)

Send loudness rating (SLR): The loudness loss between the speaking subscriber's mouth and an electric interface in the network. (The loudness loss is here defined as the weighted (dB) average of driving sound pressure to measured voltage.)

Wideband speech: Voice service with enhanced quality compared to PCM G.711 and allowing the transmission of a vocal frequency range of at least 150 Hz to 7 kHz

# Abbreviations

ACR			Absolute category rating

A/D			Analogue/Digital

AGC			Automatic Gain Control

AH,R			Attenuation Range in receive direction

AH,R,dt		Attenuation Range in receive direction during Double Talk

AH,S			Attenuation Range in Send direction

AH,S,dt			Attenuation Range in Send direction during Double Talk

BGN			BackGround Noise

BT			Bluetooth

BTR			Bluetooth Reference Point

COHREF-LSP		Coherence between the test points LSP and REF

CSS			Composite Source Signal

D/A			Digital/Analogue

D 			D-Value is computed directly from measurements of the difference Sm between the send sensitivities for diffuse and direct sound, Ssi (diff) and Ssi (direct), respectively.

Sm = Ssi (diff)  Ssi (direct)	(E-2)

D is computed as a weighted average of Sm

DELSM		DELSM is sometimes used for Sm. (see D-Value)

DRP			Drum Reference Point

DTX			Discontinuous Transmission

DUT			Device under Test

ECBD 			Bidirectional Transport  Echo Cancellation

EEB			Early Energy Balance

ERL			Echo Return Loss

FFT			Fast Fourier Transform

FRAS 			Frequency Response Audio Subsystem

FRBDR			Frequency Response Bidirectional Transport, Receive

FRBDS			Frequency Response Bidirectional Transport, Send

FRMS			Frequency Response Microphone, Send

FRSES			Frequency Response Signal Enhancement Subsystem, Send

FRSER			Frequency Response Signal Enhancement Subsystem, Receive

FRUD			Frequency Response Unidirectional Transport

HATS			Head And Torso Simulator

HATS-HFRP	Head And Torso Simulator – Hands Free Reference Point

HF System		Hands Free System

HFT			Hands Free Terminal

HVAC			Heating Ventilation Air Condition

JLR			Junction Loudness Rating

JLRBDR			Junction Loudness Rating Bidirectional Transport, Receive

JLRBDS			Junction Loudness Rating Bidirectional Transport, Send

JLR SER			Junction Loudness Rating Signal Enhancement, Receive

JLR SES			Junction Loudness Rating Signal Enhancement, Send

JLRUD			Junction Loudness Rating Unidirectional Transport

LS,min			minimum activation level (Send Direction)

LMS			Microphone Output Level, Send

LINMS			Microphone Input/Output Linearity, Send

LINREF-LSP		Audio Subsystem Linearity between REF and LSP

LQAS			Audio Subsystem Speech Quality

LQBDR 			Bidirectional Transport Speech Quality, Receive

LQBDS 			Bidirectional Transport Speech Quality, Send

LQMS			Microphone Speech Quality, Send

LQUD 			Unidirectional Transport Speech Quality

LQSBDR 		Bidirectional Transport Speech Quality Stability, Receive

LQSBDS 		Bidirectional Transport Speech Quality Stability, Send

LQSUD	 		Unidirectional Transport Speech Quality Stability

LQBGNMS		Microphone Speech Quality with Background Noise, Send

MOS			Mean Opinion Score

MRP			Mouth Reference Point

NAS			Audio Subsystem Idle Noise

NBDR			Bidirectional Transport Idle Noise, Receive

NBDS			Bidirectional Transport Idle Noise, Send

NMS			Microphone Idle Noise, Send

NUD			Unidirectional Transport Idle Noise

NC			Noise  Criterion

NCBDR			Bidirectional Transport Noise Cancellation, Receive

NCBDS			Bidirectional Transport Noise Cancellation, Send

NCUD			Unidirectional Transport Noise Cancellation

NR			Noise Reduction

OHC			Overhead Console

OLASmax		Audio Subsystem maximum output level

OVLMS			Microphone Overload Point

OVLAS			Audio Subsystem Overload Point

PCM			Pulse Code Modulation

RFIFAP		Reference Interface Access Point

POI			Point Of Interconnection

QoS			Quality of Service

REVMS 			Microphone Reverberation, Send

RLR			Receive Loudness Rating

SINR			Speech to idle noise ratio

SNR			Signal to Noise Ratio

SNRD			SNR Difference

SLR			Send Loudness Rating

SRW			Short Range Wireless Transmission

SRWAP		Short Range Wireless Reference Access Point

Test Point		Subsystem Access Point

Ssi(diff)			Diffuse field sensitivity

Ssi(direct)		Direct sound sensitivity

SNNR			Signal to Noise Ratio

SNRM			Microphone SNR

TAS			Audio Subsystem Delay

TBDR			Bidirectional Transport Delay, Receive

TBDS			Bidirectional Transport Delay, Send

TUD			Unidirectional Transport Delay

TCLw			weighted Terminal Coupling Loss

TREF-LSP		Audio Subsystem Delay between REF and LSP

Tr,R			built-up time (Receive Direction)

Tr,S			built-up time (Send Direction)

Ts			Send Delay hands-free terminal

TSES			Send Delay signal enhancement

Tr			Receive Delay hands-free terminal

TSER			Receive Delay signal enhancement

TSERTD			Round Trip Delay signal enhancement

Trtd-HF			Round Trip Delay hands-free terminal

# Conventions

dBm: absolute power level relative to 1 milliwatt, expressed in dB

dBm0: absolute power level in dBm referred to a point of zero relative level (0 dBr point)

dBm0p: weighted dBm0, according to ITU-T Recommendation O.41

dBm0(C): C weighted dBm0, according to ISO 1999

dBov: dB relative to the overload point of a digital system according to ITU-T Recommendation G.100.1dBPa: sound pressure level relative to 1 Pa, expressed in dB

dBPa(A): A-weighted sound pressure level relative to 1 Pa, expressed in dB

dBSPL: sound pressure level relative to 20µPa, expressed in dB; (94dBSPL=0dBPa)

dBV(P): P-weighted voltage relative to 1 V, expressed in dB, acc. to O.41

dBr: relative power level of a signal in a transmission path referred to the level at a reference point on the path (0 dBr point)

N: Newton

Vrms: Voltage – root mean square

cPa: Compressed Pascal, sound pressure at the output of the hearing model in the “Relative Approach” after nonlinear signal processing by the human ear

# How to Use the Specification

This specification uses the concept of performance classes for each parameter to be tested. It is the intention to identify the quality of the different subsystems by the classification based on the different parameters. The following general definition applies:

Performance class 1:

Performance class 1 characterizes an exceptionally good implementation of the relevant parameter. Performance class 1 can typically only be achieved with special constraints or effort in terms of design, cost and time.

Performance class 2:

Performance class 2 characterizes an implementation which most likely fulfils the requirements in ITU-T Recommendation P.1100 and P.1110. Performance class 2 can be expected to be representative for good implementations.

Performance class 3:

Performance class 3 characterizes some weakness for the parameter under test. If a parameter is fulfilling just Performance class 3 it may be acceptable only given that other parameters benefit from that or may compensate the weakness of this parameter and the complete system is still fulfilling the requirements of ITU-T Recommendation P.1100 and P.1110.

Performance class 4:

Performance class 4 characterizes an unacceptable weakness of the parameter under test.

# Architectures

Fig. 7.1 Abstract architecture – System Overview

Fig. 7.2: System Overview with detailed transport

Fig. 7.3: System overview with the example of typical devices

## Distributed  speakerphone

The subsystems are tested as described in the different sections of this specification. The test points for the different subsystems are shown in Fig. 7.2

Fig. 7.4 Abstract architecture and Testpoints

Fig. 7.5 System overview and test points, example of a typical implementation

## System with integrated microphone

The system is tested as described in ITU-T Rec. P.1100 (for narrowband systems) or in ITU-T Rec. P.1110 (for wideband systems). If no network access is provided by the system the system is connected to a reference interface replacing the network access system. The requirements as given in ITU-T Recs. P.1100 and P.1110 apply.

Fig. 7.6: System with integrated microphone

The loudspeaker subsystem is tested as described in section xx.

The network access subsystem is tested as described in section xx.

## System with integrated speaker

The system is tested as described in ITU-T Rec. P.1100 (for narrowband systems) or in ITU-T Rec. P.1110 (for wideband systems). If no network access is provided by the system the system is connected to a reference interface replacing the network access system. The requirements as given in ITU-T Recs. P.1100 and P.1110 apply.

Fig. 7.7: System with integrated speaker

The microphone subsystem is tested as described in section xx.

The network access subsystem is tested as described in section xx.

## System with integrated microphone and speaker

The system is tested as described in ITU-T Rec. P.1100 (for narrowband systems) or in ITU-T Rec. P.1110 (for wideband systems). If no network access is provided by the system the system is connected to a reference interface replacing the network access system. The requirements as given in ITU-T Recs. P.1100 and P.1110 apply.

Fig. 7.8: System with integrated microphone and speaker

## System Delay, Subsystem Delay and Buffering

The system delay is measured as described in ITU-T Recommendations P.1100 [xx] for narrowband systems and P.1110 [xx] for wideband systems. The subsystem delay as described and measured within this specification only includes the algorithmic delay. In case a subsystem is implemented in a hardware the computational delay is measured (see Annex G).

It is known that buffering needed in order to e.g. take care of different frame sizes in subsystems may cause a significant increase of the system delay. Therefore buffering and frame sizes should ideally be optimized to get a minimum system delay.

Annex G describes synchronization issues and the impact of different frame sizes and buffering in detail and gives guidance how to optimize subsystems to achieve a mostly low system delay.

# Subsystems

## Test Interface description

### Analog electrical interface

The analog interface of the test system should match the impedance and the sensitivity of the system under test.

### Digital electrical interface

The digital interface of the test system should provide a dynamic range of at least 16 bit equivalent and otherwise should match the interface specification of the system under test.

### Acoustic interface

#### Artificial mouth

The artificial mouth of the artificial head shall conform to ITU-T Recommendation P.58 [xx]. The artificial mouth is equalized at the MRP according to ITU-T Recommendation P.340 [xx].

The sound pressure level is calibrated at the HATS-HFRP so that the average level at HATS-HFRP is -28.7 dBPa. The sound pressure level at the MRP has to be corrected correspondingly. The detailed description for equalization at the MRP and level correction at the HATS-HFRP can be found in ITU-T Recommendation P.581 [xx].

When testing with vehicle noise, the output level of the mouth is increased to account for the “Lombard effect”. The Lombard effect refers to the change in speaking behaviour caused by acoustic noise. The level is increased by 3 dB for every 10 dB that the long-term A-weighted noise level exceeds 50 dB(A) [xx]. This relationship is shown in the following formula:

IN=0forN<500.3N−50for50≤N≤77

Where:

I = The dB increase in mouth output level due to noise level

N = The long-term A-weighted noise level measured near the driver’s head position

As an example, if the vehicle noise measures 70 dB(A), then the output of the mouth would be increased by 6 dB. No gain is applied for noise levels below 50 dB(A). The maximum amount of gain that can be applied is 8 dB. Vehicle noise levels are measured using a measurement microphone positioned near the driver’s head position.

#### Artificial ear

For speakerphone hands-free terminals the ear signals of both ears of the artificial head are used. The artificial head is free-field or diffuse-field equalized (see ITU-T Recs. P.1100 and P.1110), more detailed information can be found in ITU-T Recommendation P.581 [xx].

### Wireless interface

The interface of the test system should conform to the interface specification of the wireless system under test.

### Test signal levels

In general the test signal levels are individual to the system design and vary between implementations. However, if not stated otherwise as a general rule the following test signal levels shall be used:

For digital interfaces the nominal signal level is derived by measuring the signal level at the according digital input of the subsystem when applying a signal with nominal level either at the acoustical interface of the hands-free terminal or at the POI.

Typically the nominal test signal level should be -22 dBov at the digital interface for digital systems. (assuming a nominal signal level of -16dBm0 and an overload point of 3.14 dBm0 for digital transmission systems)

For analog interfaces a similar approach can be taken. The nominal signal level is derived by measuring the signal level at the according analog input of the subsystem when applying a signal with nominal level either at the acoustical interface of the hands-free terminal or at the POI.

## Acoustic Subsystem

The acoustic subsystem and related test points are shown in Fig. 8.1. This figure also shows that the acoustic subsystem can be further divided into the following subsystems:

- Microphone subsystem (send path)

- Audio subsystem (receive path)

- Coupling between transducers subsystems (echo path).

Fig. 8.1: Acoustic subsystem and test points

### Microphone subsystem (send path)

        - Send Speech-to-Noise Ratio (SpNR)

          - Is application dependent?

          - Soft & loud speakers

          - Wind buffeting noise

          - Belt microphone noise issues; Structure borne noise

        - Cover spectral and time variant components

        - Send Distortion + Noise (SDN) ?

        - Delay

        - For multi microphone solutions only:

          - Delay variation (over sensors and time)

          - Frequency response variation (over sensors and time)

          - Level variation (over sensors and time)

The microphone subsystem may consist of one or more microphones and may include a signal enhancement layer for the microphone signal as well.

The purpose of this section is to provide guidance for positioning microphones and optimize the microphone performance parameters in such a way that the microphone provides optimum input signals to the speech enhancement.

Fig. 8.2: Microphone subsystem and test points

The DUT is connected to the reference interfaces. The reference interfaces are calibrated as specified in clause 8.1.

This clause is applicable to single microphones but not to the output of microphone arrays.

#### Microphone measurements in the car

Positioning of hands-free microphones

The speech quality in hands-free communication is significantly affected by the positioning of the hands-free microphone. As the optimal microphone position can vary strongly depending on vehicle design as well as on specific requirements, there is no universally valid rule for the positioning of the microphone. However, there are some guidelines which should be considered. Nevertheless, in practice, this often means to find the best compromise, as not all requirements can be equally fulfilled.

–	The hands-free microphone should always be placed as close as possible to the speaker, as within the near field of a sound source (in a vehicle this is up to 80-100 cm) the speech level drops by 1/d². In practical applications, this typically means an analogous loss in signal-to-noise ratio. For this reason, a single microphone placed nearby might give a better performance than a microphone array, which is placed further away.

–	There has to be a direct path between the speaker's mouth and the microphone. If this is not given, this might result in a significant decrease in signal-to-noise ratio as well as in speech quality since the speech signal becomes reverberant.

–	The direction of the highest sensitivity of the microphone should point in the direction of the speaker's mouth. If different seating positions or several speakers are to be covered by one microphone, a compromise for the microphone position has to be found, as the direction of the highest sensitivity might not cover all. However, this often means a significantly reduced performance in comparison to an optimal alignment of the microphone for a single speaker. In this case, the application of additional microphones might be considered to achieve an optimal speech quality.

–	A direct airstream to the microphone, e.g., from the air conditioning, has to be avoided as the speech signal might be highly disturbed by wind buffeting.

–	Saturation of the microphone by loudspeakers nearby, e.g., by a centre-speaker, has to be avoided. If necessary, the levels of the affected loudspeakers have to be reduced.

Coupling of structure-borne sound to the microphone has to be avoided.

NOTE 1 – When the microphone is measured in the car, it is recommended to use the power supply provided by the car/car hands-free system.

NOTE 2 – If the microphone is integrated digitally in the car, it is recommended to measure the microphone at a digital access point, if available. Care should be taken in order to correctly calibrate the access point.

NOTE 3 – In case the car is not available it is recommended to integrate the microphone in a surrogate environment as close as possible to the production environment. In case wind noise may have a major influence on the microphone performance it is recommended to also simulate the wind noise.

##### Microphone overload point (in the car)

###### Parameter description

The microphone overload point OVLMS is measured from the test point (Mic) to the test point (S1a) or the test point (S2).

The microphone overload point is determined by the highest signal which is received by the microphone in the car without distorting. This maybe the speech signal of the near end speaker, the background noise picked up by the hands-free microphone or the signal received from the car audio system.

###### Test

1)	The maximum spectrum is recorded by a reference microphone close to the hands-free microphone (MIC) by generating, background noise, near end speech and the signal generated by the car audio system with maximum volume.

- The background noise conditions to be simulated are found in Annex D of ITU-T Rec. P.1100 and P.1110. The background noise producing the maximum level at the microphone is used.

- The maximum near end signal is generated by generating the British-English single talk sequence described in clause 7.3.2 of  ITU-T Recommendation P.501 [xx] with a level as described in 8.1.3.1.

- The maximum loudspeaker signal is generated by injecting the British-English single talk sequence described in clause 7.3.2 of  ITU-T Recommendation P.501 [xx] nominal signal level at the far end and setting the audio volume control to maximum.

2)	For the signal producing the maximum level (Levmax) the coherence between the reference microphone positioned at MIC and S2 is determined. The coherence is determined as an average of the coherence function in the frequency range from 100 Hz to 7 kHz.

3)	The measurement is repeated by reducing the signal level of the worst case signal in steps of 1 dB until the required coherence is achieved.

NOTE: The British-English single talk sequence  based tests are averaged over the complete speech sequence voice.

###### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the Microphone overload point has to meet the requirements for the applicable Performance classes for the Microphone overload point as defined in table xx:

Table xx –Limits for the Microphone overload point above 0.95 coherence

| Performance Class | OVL re Levmax |
| --- | --- |
| 1 | 0 dB |
| 2 | -3 dB |
| 3 | -6 dB |
| 4 | -9 dB |

###### Design guidance and root-cause analysis

The microphone is the first element in the transmission system of the hands-free system. Its dynamic range should be adapted to the type of car it is used in. Besides other factors this requires careful consideration of the maximum acoustical level it may be exposed to. Any amplitude distortion (clipping) introduced by the microphone may impact the speech sound quality, the transmission quality of the background noise, the performance of any noise canceller and the performance of echo cancellers.

##### Microphone frequency response (in the car)

###### Parameter description

The microphone frequency response in send FRMS is measured from the test point (Mic) to the test point (S1a) or the test point (S2).

###### Test

1)	The test arrangement is according to clause 8.2.

2)	The British-English single talk sequence described in clause 7.3.2of  ITU-T Recommendation P.501 [xx]   is used as the test signal. Alternatively, a periodic noise signal or CS signal according to [ITU-T P.501] can be used. The correct activation of the measurement object during the measurement has to be ensured by the test lab. The artificial mouth is equalized at the MRP, the test signal level shall be –4.7 dBPa at the MRP. The test signal level is the average level of the complete test signal. Finally, the level at the HATS-HFRP is adjusted to –28.7 dBPa.

3)	The measured power density spectrum at the MRP is used as the reference power density spectrum for determining the frequency response in send direction.

4)	The sensitivity frequency response is determined in third octave intervals as given by [IEC 61260] for frequencies of 100 Hz and 8 kHz, inclusive. For calculation, the average measured level is referred to the level of the reference signal in each frequency band averaged over the complete test sequence length.

5)	The sensitivity is determined in dBV/Pa.

###### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the frequency response has to meet the requirements for the applicable Performance classes for the microphone send sensitivity as defined in the tables below:

The masks are drawn by straight lines between the breaking points in Table 9-xx to 9-xx on a logarithmic (frequency) – linear (dB sensitivity) scale.

Table xx – Tolerance mask for the send sensitivity frequency response for Performance class 1

| Frequency | Upper limit | Lower limit |
| --- | --- | --- |
| 100 Hz | 2 dB | – dB |
| 300 Hz | 2 dB | –2dB |
| 500 Hz | 2 dB | -2 dB |
| 1 000 Hz | 2 dB | –2 dB |
| 2 500 Hz | 2 dB | -2 dB |
| 3 100 Hz | (Note) | –2 dB |
| 5 000 Hz | (Note) | -2 dB |
| 7 000 Hz | 3 dB | -2 dB* |
| 11 000 Hz | 3 dB | –2 dB* |
| 12 000 Hz | 3 dB | – dB* |
| NOTE 1 – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. * NOTE 2 – If the microphone is purely used for speech communication and not used for speech recognition or other services an upper limit of 7 kHz is sufficient. |  |  |

Table xx – Tolerance mask for the send sensitivity frequency response for Performance class 2

| Frequency | Upper limit | Lower limit |
| --- | --- | --- |
| 100 Hz | - | – dB |
| 300 Hz | 2 dB | –5 dB |
| 500 Hz | 2 dB | -4 dB |
| 1 000 Hz | 2 dB | –2 dB |
| 2 500Hz | 4 dB | -2 dB |
| 3 100 Hz | (Note) | –2 dB |
| 5 000 Hz | (Note) | – dB |
| 6 300 Hz | 5 dB | – dB |
| 8 000 Hz | - | – dB |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the send sensitivity frequency response for Performance class 3

| Frequency | Upper limit | Lower limit |
| --- | --- | --- |
| 100 Hz | - | – dB |
| 300 Hz | 4 dB | –10 dB |
| 500 Hz | 4 dB | -8 dB |
| 1 000 Hz | 4 dB | –4 dB |
| 2 500Hz | 8 dB | (Note) |
| 3 100 Hz | 8 dB | –4 dB |
| 5 000 Hz | 8 dB | – dB |
| 6 300 Hz | 8 dB | – dB |
| 8 000 Hz | - | – dB |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the send sensitivity frequency response for Performance class 4

| Frequency | Upper limit | Lower limit |
| --- | --- | --- |
| 100 Hz | N/A | N/A |
| 300 Hz | N/A | N/A |
| 500 Hz | N/A | N/A |
| 1 000 Hz | N/A | N/A |
| 2 500Hz | N/A | N/A |
| 3 100 Hz | N/A | N/A |
| 5 000 Hz | N/A | N/A |
| 6 300 Hz | N/A | N/A |
| 8 000 Hz | N/A | N/A |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

NOTE: Performance class 1 mask is intended to support high quality wideband with no need to equalize the response. Performance class 2 is intended to support high quality narrowband with no need to equalize the response. Performance class 3 is intended to support narrowband communication and some equalization may be required.

###### Design guidance and root-cause analysis

NOTE – Ideally, the response characteristics of the microphone should be flat in the frequency range of wideband transmission (100 Hz – 7 kHz). However, especially in the presence of background noise, a bandwidth limitation may be desirable. No explicit recommendation can be given here since such limitation would depend on the level and spectral content of the background noise and ideally should be adaptive. If, however, a bandwidth limitation is introduced, it should be made at both, the high and low frequencies.

##### Microphone  idle channel noise (in the car)

###### Parameter description

The microphone idle channel noise in send NMS is measured from the test point (Mic) to the test point (S1a) or the test point (S2).

###### Test

1)	The British-English single talk sequence described in clause 7.3.2of  ITU-T Recommendation P.501 [xx]  with an average signal level of -4.7 dBPa [ITU-T P.50] is applied at the MRP. The test signal level is the average level of the complete test signal. Finally, the level at the HATS-HFRP is adjusted to –28.7 dBPa. The output level at the hands-free microphone is measured using a speech level voltmeter according to ITU-T P.56. This level is the reference speech signal level.

2)	For the noise measurement, no test signal is used. However, all sources which potentially contribute to noise at the output of the microphone such as GSM noise, electrical noise introduced by the car must be considered. In order to ensure a reliable activation of active microphone arrangements, an activation signal is inserted before the actual measurement. The activation signal consists of a sequence of 4 composite source signals according to [ITU-T P.501]. The spectrum of the test at the MRP is calibrated under free-field conditions. The level of the activation signal is –28.7 dBPa measured at the HATS-HFRP.

3)	The test arrangement is according to clause 7.1.

The idle channel noise is measured at the output of the test circuit according to Figure 7-2 in the frequency range between 100 Hz and 8 kHz. The measurement requires a time window which starts exactly at the end of the activation signal. Any "ringing" of filters or receivers shall be taken into account, the time window must be shifted accordingly. The length of the time window is 1 second which is the averaging time for the idle channel noise. The test lab has to ensure the correct activation of the microphone/microphone arrangement during the measurement. If the microphone arrangement is deactivated during measurement, the measurement window has to be cut to the duration when the microphone remains activated.

The power density spectrum of the noise signal is determined using FFT (8 k samples/48 kHz sampling rate or equivalent). A Hann window is used.

If it is known that the microphone stays activated without any activation signal, no activation signal is required. In this case, a simple noise measurement is conducted.

4)	The idle channel noise is determined by A-weighting and referring to the reference speech signal level as determined with speech sequence. Spectral peaks are measured in the frequency domain. The average noise spectrum used for determining the spectral peak should be calculated as the arithmetic mean of the noise spectrum values when stated in dBVP.

###### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the speech to idle noise ratio SINR has to meet the requirements for the applicable Performance classes for the SINR  as defined in table xx:

Spectral peaks in the frequency domain shall not exceed the averaged spectrum by more than 10 dB.

Table xx –Limits for the idle channel noise re. reference speech signal level

| Performance Class | SINR | Interpretation |
| --- | --- | --- |
| 1 | > 40 dB |  |
| 2 | > 30 dB |  |
| 3 | > 25 dB |  |
| 4 | ≤ 20 dB |  |

###### Design guidance and root-cause analysis

Be sure that receive path is deactivated, Check shielding, check microphone preamplifier should not demodulate RF into NF,

##### Microphone SNR (in the car)

###### Parameter description

The SNR measurement is based on individual broadband estimations of the speech signal power and the noise signal power and is performed using the reference-free measurement method described in Annex G.

NOTE 1 – It is recognized that fan noise, which varies from car to car and depends upon the relative positioning of the microphone and fan, may contribute significantly to the noise perceived by the far end user. In order to determine the impact of the level and spectral content of this noise under different operating conditions, a noise test as described below may be used.

NOTE 2 – The method described requires stationary type of background noise and therefore should be applied for constant driving conditions or conditions where the background noise can be considered to be stationary.

The microphone signal to noise ration SNRM is measured at the test point (S1a) or the test point (S2). Test point (S2) should only be used if test point (S1a) is not accessible.

###### Test

- The test arrangement is according to clause 8.1.

- The speech test signal used is the British-English single talk sequence described in clause 7.3.2of  ITU-T Recommendation P.501 [xx]. To account for the Lombard effect, the test signal level is adjusted to the level as specified in clause 8.1, the test signal level is measured as "active speech level" according to [ITU-T P.56].

- For the measurement speech and background noise are recorded simultaneously while driving.

- The actual measurement is conducted with the hands-free microphone.

- The SNR is calculated according to Annex G.

NOTE – The method is applicable in the SNR range from –10dB to +30dB. Care should be taken that the resulting SNR in the measurement is within this SNR-range.

###### Performance level classification based on values of this parameter

Background noises for driving speed ≤ 80 km/h:

To claim compliance with a certain Performance class the SNRD  has to meet the requirements for the applicable Performance classes for the SNRD as defined in table xx:

Table xx –Limits for the SNRD

| Performance Class | SNRD |
| --- | --- |
| 1 | xx dB |
| 2 | TBD |
| 3 | TBD |
| 4 | TBD |

Background noises for driving speed ≤ 120 km/h:

To claim compliance with a certain Performance class the SNRD  has to meet the requirements for the applicable Performance classes for the SNRD as defined in table xx:

Table xx –Limits for the SNRD

| Performance Class | SNRD |
| --- | --- |
| 1 | xx dB |
| 2 | TBD |
| 3 | TBD |
| 4 | TBD |

###### Design guidance and root-cause analysis

Insufficient SNRD can be caused by an insufficient directivity of the microphone used, by the wrong orientation of the microphone or by the wrong microphone placement (e.g. too far away from the speaker), too high background noise at the microphone location.

#### New tests to be defined

##### Microphone Send ReVerberation (SRV)

###### Parameter description

The microphone reverberation in send REVMS is measured from the test point (MRP) to the test point (S1a) or the test point (S2).

A measure for determining the subjectively perceived reverberation is defined in ITU-T Recommendation P.340 [xx]. The Early Energy Balance is a measure which seems to correlate to the subjectively perceived reverberation when transmitting reverberant speech over hands-free terminals. The EEB is defined as:

###### Test

- The test arrangement is according to clause 8.1.

- The test signal should be composed of an activation signal (e.g. voiced sound of the CSS) and a maximal-length sequence of several periods as described in [ITU-T P.340] . The period length must be longer than the length at the impulse response of the system under test.

- The impulse response is calculated from the average of several periods, beginning with the second period.

- The delay introduced by the system under test and the test system has to be compensated for.

- From this the Early Energy Balance (EEB) is calculated.

###### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the EEB  has to meet the requirements for the applicable Performance classes for the EEB  as defined in table xx:

Table xx –Limits for the TBD

| Performance Class | EEB |
| --- | --- |
| 1 | x |
| 2 |  |
| 3 |  |
| 4 |  |

###### Design guidance and root-cause analysis

Too high reverberation may lead to hollow sounding speech. The reverberation may be caused by positioning of the microphone(s) too close to reflecting surfaces (typically windows) or by insufficient damping of reflecting surfaces in a car. Additional damping of surfaces or changing the microphone position may help to correct the problem.

##### Microphone Send Input/Output Linearity

###### Parameter description

The microphone Send Input/Output Linearity LINMS is measured from the test point (Mic) to the test point (S1a) or the test point (S2).

The microphone linearity is determined by the measurement of the A-weighted output level between input and output in the level range from -40 dB to +6 dB re. to the nominal input signal level and referring the measured level to the input signal level.

###### Test

- The test arrangement is according to clause 8.1. The spectrum at MIC is recorded by a reference microphone close to the hands-free microphone (MIC).

- The test signal used is the British-English single talk sequence described in clause 7.3.2of  ITU-T Recommendation P.501 [xx]. The correct activation of the measurement object during the measurement has to be ensured by the test lab. The artificial mouth is equalized at the MRP, the test signal level shall be -30 dB, -20 dB, -10 dB, 0 dB and +8 dB re to –4.7 dBPa at the MRP. The test signal level is the average level of the complete test signal.

- The A-weighted output level is determined at the test point S2 and referred to the input signal level.

- The measurement is repeated for all input signal levels.

###### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the measured level difference between input and output has to meet for all input signal levels the requirements for the applicable Performance classes as defined in table xx:

Table xx –Limits for the LINMS

| Performance Class | LINMS |
| --- | --- |
| 1 | < 0.2 dB |
| 2 | Xx |
| 3 | Xx |
| 4 | Xx |

###### Design guidance and root-cause analysis

##### Microphone Send speech quality

###### Parameter description

The microphone listening speech quality in send LQMS is measured from the test point (MRP) to the test point (S1a) or the test point (S2).

The test is intended to determine any impairment of the listening speech quality introduced by the microphone.

###### Test

The tests method to be used is ITU-T Recommendation P.863.

NOTE: ITU-T Recommendation P.863 [xx] does not provide a wideband mode. Wideband systems are evaluated on a superwideband scale. Therefore the MOS Requirements are given in MOS-LQOsw. See P.863 for more information.

1)	The test signals used are the English test sequences as specified in [ITU-T P.501, Annex B] (2 male speakers, 2 female speakers, two sentences each). The test signal level is the nominal signal level measured as "active speech level" according to [ITU-T P.56]. The speech activity should be between 30% and 70%.

The original speech signal is used as the reference signal for the determination of the speech quality.

2)	The test arrangement is according to clause 8.1. For wideband systems MOS-LQOsw is determined. For narrowband systems MOS-LQOn is determined

The calculation is made using the signal recorded at the test point (S2).

###### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the listening speech quality in send has to meet the requirements for the applicable Performance classes as given in the table below:

Table xx –Limits for the microphone listening speech quality in send

| Performance Class | MOS-LQOn | MOS-LQOsw |
| --- | --- | --- |
| 1 | > 4.2 | > 4.5 |
| 2 | > 3.8 | > 4.1 |
| 3 | > 3.4 | > 3.7 |
| 4 | ≤ 3.4 | ≤ 3.7 |

###### Design guidance and root-cause analysis

No impairment on listening speech quality should be introduced by the microphone. Any degradation of listening speech quality will deteriorate the overall listening speech quality and should be avoided. Therefore the listening speech quality under quiet conditions is checked. Potential degradations of the microphone listening speech quality in send may be caused by noise (e.g. circuit noise), non optimum frequency response characteristics, additional reverberance or impairments introduced by signal processing such as non linearities.

##### Microphone Send speech quality with background noise

###### Parameter description

The microphone speech quality in send with background noise LQBGNMS is measured at the test point (S1a) or the test point (S2) with the reference point (Mic) for the unprocessed speech + noise signal.

The speech quality in background noise is tested based on [b-ETSI EG 202 396-3]. The test method described leads to three MOS-LQO quality numbers:

N-MOS-LQO: 	Transmission quality of the background noise

S-MOS-LQO: 	Transmission quality of the speech

G-MOS-LQO:	Overall transmission quality

For this test, the speech level is adjusted at the MRP to take into account the Lombard effect. The level adjustment is calculated according to clause 8.1.

###### Test

- The test arrangement is given in clause 8.1.

2)	According to the specification of the manufacturer/test lab, the background noise is played back. The background noise should be applied for at least 5 s in case speech enhancement is used in order to adapt noise reduction algorithms in advance the test.

3)	The near end speech signal consists of 8 sentences of speech (2 male and 2 female talkers, 2 sentences each). Appropriate speech samples can be found in [ITU-T P.501]. The preferred language for wideband  is French since the objective method was validated with the French language. The preferred language in narrowband is English. The test signal level is the signal level according to clause 8.1 measured as "active speech level" according to [ITU-T P.56]. The speech activity should be between 30% and 70%.Three signals are required for the tests:

–	The clean speech signal is used as the undisturbed reference (see [b-ETSI EG 202 396-3]).

–	The speech plus undisturbed background noise signal is recorded at the microphone position (Mic) using an omnidirectional measurement microphone with a linear frequency response between 50 Hz and 12 kHz.

–	The send signal is recorded at the test point (S2).

N-MOS-LQO, S-MOS-LQO and G-MOS-LQO are calculated as described in [b-ETSI EG 202 396-3].

###### Performance level classification based on values of this parameter

Background noises for driving speed ≤ 80 km/h:

To claim compliance with a certain Performance class the listening speech quality with background noise in send has to meet the requirements for the applicable Performance classes as given in the table below:

Table xx –Limits for the listening speech quality with background noise in send: N-MOS

| Performance Class | N-MOS-LQOn | N-MOS-LQOw |
| --- | --- | --- |
| 1 | > 4.0 | > 4.0 |
| 2 | > 3.0 | > 3.0 |
| 3 | > 2.5 | > 2.5 |
| 4 | ≤ 2.5 | ≤ 2.5 |

Table xx –Limits for the listening speech quality with background noise in send: S-MOS

| Performance Class | S-MOS-LQOn | S-MOS-LQOw |
| --- | --- | --- |
| 1 | > 4.0 | > 4.0 |
| 2 | > 3.0 | > 3.0 |
| 3 | > 2.5 | > 2.5 |
| 4 | ≤ 2.5 | ≤ 2.5 |

Table xx –Limits for the listening speech quality with background noise in send: G-MOS

| Performance Class | G-MOS-LQOn | G-MOS-LQOw |
| --- | --- | --- |
| 1 | > 4.0 | > 4.0 |
| 2 | > 3.0 | > 3.0 |
| 3 | > 2.5 | > 2.5 |
| 4 | ≤ 2.5 | ≤ 2.5 |

Background noises for driving speed ≤ 120 km/h:

To claim compliance with a certain Performance class the listening speech quality with background noise in send has to meet the requirements for the applicable Performance classes as given in the table below:

Table xx –Limits for the listening speech quality with background noise in send: N-MOS

| Performance Class | N-MOS-LQOn | N-MOS-LQOw |
| --- | --- | --- |
| 1 | > 3.5 | > 3.5 |
| 2 | > 2.5 | > 2.5 |
| 3 | > 2.0 | > 2.0 |
| 4 | ≤ 2.0 | ≤ 2.0 |

Table xx –Limits for the listening speech quality with background noise in send: S-MOS

| Performance Class | S-MOS-LQOn | S-MOS-LQOw |
| --- | --- | --- |
| 1 | > 3.5 | > 3.5 |
| 2 | > 2.5 | > 2.5 |
| 3 | > 2.0 | > 2.0 |
| 4 | ≤ 2.0 | ≤ 2.0 |

Table xx –Limits for the listening speech quality with background noise in send: G-MOS

| Performance Class | G-MOS-LQOn | G-MOS-LQOw |
| --- | --- | --- |
| 1 | > 3.5 | > 3.5 |
| 2 | > 2.5 | > 2.5 |
| 3 | > 2.0 | > 2.0 |
| 4 | ≤ 2.0 | ≤ 2.0 |

###### Design guidance and root-cause analysis

The microphone send speech quality with background noise mainly depends on the SNR at the hands-free microphone. Other factors which may influence the speech quality in background noise are the microphone placement (which may help to improve the SNR) and the microphone frequency response characteristics. Further impact can be expected by wind buffeting caused e.g. by poorly designed outlets for air conditioning or heating systems.

A methodology for more detailed investigation of background noise modulations due to noise cancelling algorithms can be found in Annex E.

### Audio subsystem (receive path)

The audio subsystem and its related access points are shown in Fig. 8.xx. The audio subsystem may consist of one or more loudspeakers with associated amplifiers and an associated signal enhancement system. In difference to other subsystems the audio subsystem may provide an output signal (reference channel) for the echo canceller which is part of the signal enhancement subsystem (see 8.3).

Fig. 8.3: Audio subsystem and test points

Note – All measurements and requirements are made for stand still or low speed conditions. Any speed or noise dependant equalization, compression or other functionality is not covered by the chapter because this functionality is considerd to be very car specific and no general recommendation can be given.

        - Receive Conversational Gain (RCG)

        - Receive speech quality (MOS-LQO)

        - Receive Intelligibility (RI)

        - Cabin Noise (CN)

        - Receive Input/Output Linearity

#### Audio subsystem delay

##### Parameter description

The delay TAS is measured from the test point (R2) to the test point (DRP). The delay TAS should be minimized. The measurement is conducted at nominal volume control setting.

The system delay tSystem depends on the transmission method used and the delay of the reference interface. The delay tSystem must be known and deducted from the test result.

##### Test

1)	For the measurements, a composite source signal (CSS) according to [ITU-T P.501] is used. The pseudo random noise (pn)-part of the CSS has to be longer than the maximum expected delay. It is recommended to use a pn sequence of 16 k samples (with 48 kHz sampling rate). The test signal is inserted at test point (R2) at a level  according to clause 8.1.5.

The reference signal is the original signal (test signal).

The test set-up is according to clause 8.1.

2)	The delay is determined by cross-correlation analysis between the measured signal at the DRP and the original signal. The measurement is corrected by delays which are caused by the test equipment. The measurement is corrected by delays which are caused by the test equipment.

3)	The delay is measured in ms and the maximum of the cross-correlation function is used for the determination.

Tmeasure  = TAS + TSystem

##### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the delay in the send direction has to meet the requirements for the applicable Performance classes for the delay TAS as defined in table xx:

Table xx –Limits for the audio subsystem delay for speech communication services

| Performance Class | TAS |
| --- | --- |
| 1 | < 7 ms |
| 2 | < 12 ms |
| 3 | < 30 ms |
| 4 | ≥ 30 ms |

##### Design guidance and root-cause analysis

Low delay is essential for a seamless conversational performance especially for highly interactive conversations. Delay also contributes to echo perception: Higher delay low leads to higher impairment resulting even from low level echo components (see ITU-T Rec. G. 111 [xx]). Therefore any design providing as low delay in the connection as possible is preferable. Furthermore higher delays may impair the performance of the echo canceller in the signal enhancement subsystem.

#### Audio subsystem sensitivity frequency response

##### Parameter description

The frequency response FRAS is measured from the test point (R2) to the test point (DRP). The measurement is conducted at nominal volume control setting.

The sensitivity response on the audio subsystem should be mostly flat in the entire frequency range.

##### Test

1)	The test signal used for the measurements shall be the British-English single talk sequence described in clause 7.3.2 of  ITU-T Recommendation P.501 [22]. The test signal level is the nominal signal level, the level is averaged over the complete test signal.

Note: If the signal enhancement subsystem includes an equalization function for the receive path the equalization curve should be determined and applied to the test signal inserted. Such a pre-equalized signal is used for the measurements.

The measured power density spectrum at the test point (R2) interface is used as the reference power density spectrum for determining the audio subsystem sensitivity.

2)	The test arrangement is according to clause 8.1. The audio subsystem frequency response is determined in third octave intervals, as given by [IEC 61260] for frequencies of 100 Hz and 8 kHz, inclusive. In each third octave band, the level of the measured signal is referred to the level of the reference signal averaged over the complete test sequence length.

3)	The sensitivity is determined in dBPa/V.

##### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the frequency response has to meet the requirements for the applicable Performance classes for the unidirectional transport FRAS as defined in the tables below:

The masks are drawn by straight lines between the breaking points in Table 8-xx to 8-xx on a logarithmic (frequency) – linear (dB sensitivity) scale.

Table xx – Tolerance mask for the wideband audio subsystem sensitivity frequency response for Performance class 1

| 100 Hz | 3 dB | –6 dB |
| --- | --- | --- |
| 20ker response charcteristics.erformed in the audio subsystem as well as the loudspearacteristics of the audio subsystem,tions or0 Hz | 3 dB | -6 dB |
| 315 Hz | 3 dB | –3 dB |
| 6 300 Hz | 3 dB | –3 dB |
| 7 600 Hz | 3 dB | -6 dB |
| 8 000 Hz | 3 dB | x |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the wideband audio subsystem sensitivity frequency response for Performance class 2

| Frequency | Upper limit | Lower limit |
| --- | --- | --- |
| 100 Hz | 8 dB | – dB |
| 250 Hz | 8 dB | -9 dB |
| 315 Hz | 8 dB | –6 dB |
| 400 Hz | 6 dB | -6 dB |
| 5 000 Hz | 6 dB | -6 dB |
| 7 000 Hz | 6 dB | –9 dB |
| 8 000 Hz | 6 dB | – dB |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the wideband audio subsystem sensitivity frequency response for Performance class 3

| Frequency | Upper limit | Lower limit |
| --- | --- | --- |
| 100 Hz | 8 dB | – dB |
| 250 Hz | 8 dB | - dB |
| 315 Hz | 8 dB | –9 dB |
| 400 Hz | 6 dB | -6 dB |
| 5 000 Hz | 6 dB | -9 dB |
| 7 000 Hz | 6 dB | –12 dB |
| 8 000 Hz | 6 dB | – dB |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the wideband audio subsystem sensitivity frequency response for Performance class 4

| Frequency | Upper limit | Lower limit |
| --- | --- | --- |
| 100 Hz | N/A | N/A |
| 250 Hz | N/A | N/A |
| 315 Hz | N/A | N/A |
| 400 Hz | N/A | N/A |
| 5 000 Hz | N/A | N/A |
| 7 000 Hz | N/A | N/A |
| 8 000 Hz | N/A | N/A |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the audio subsystem sensitivity frequency response for Performance class 1

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 200 | 3 dB | -6 dB |
| 250 | 3 dB | (Note) |
| 315 | 3 dB | –3 dB |
| 400 | 3 dB | –3 dB |
| 3 100 | 3 dB | –3 dB |
| 3 400 | 3 dB | -6 dB |
| 4 000 | 3 dB | – dB |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the narrowband audio subsystem sensitivity frequency response for Performance class 2

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 200 | 6 dB | – dB |
| 250 | 6 dB | – dB |
| 315 | 6 dB | –9 dB |
| 400 | 6 dB | –6 dB |
| 3 100 | 6 dB | –6 dB |
| 4 000 | 6 dB | – dB |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the narrowband audio subsystem sensitivity frequency response for Performance class 3

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 200 | 6 dB | – dB |
| 250 | 6 dB | – dB |
| 315 | 6 dB | –12 dB |
| 400 | 6 dB | –8dB |
| 3 100 | 6 dB | –8 dB |
| 4 000 | 6 dB | – dB |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the narrowband audio subsystem sensitivity frequency response for Performance class 4

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 200 | N/A | N/A |
| 315 | N/A | N/A |
| 1 000 | N/A | N/A |
| 1 300 | N/A | N/A |
| 1 600 | N/A | N/A |
| 2 000 | N/A | N/A |
| 3 100 | N/A | N/A |
| 4 000 | N/A |  |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

T

##### Design guidance and root-cause analysis

Deviations from a flat frequency response characteristics may cause degradation of the listening speech quality in the receive direction or may result in insufficient listening speech quality. A flat frequency response mostly contributes to a good customer experience in the car.

#### Audio subsystem speech quality

##### Parameter description

The audio subsystem listening speech quality LQAS is measured from the test point (R2) to the test point (DRP). The measurement is conducted at nominal and at maximum volume control setting.

The test is intended to determine any impairment of the listening speech quality introduced by the unidirectional transport.

##### Test

The tests method to be used is ITU-T Recommendation P.863. NOTE: ITU-T Recommendation P.863 [xx] does not provide a wideband mode. Wideband systems are evaluated on a superwideband scale. Therefore the MOS Requirements are given in MOS-LQOsw. See P.863 for more information.

1)	The test signals used are the English test sequences as specified in [ITU-T P.501] (2 male speakers, 2 female speakers, two sentences each). The test signal level is the nominal signal level, measured at the test point (R2), the test signal level is measured as "active speech level" according to [ITU-T P.56]. The speech activity should be between 30% and 70%.

The original speech signal is used as the reference signal for the determination of the speech quality.

Note: If the signal enhancement subsystem includes an equalization function for the receive path the equalization curve should be determined and applied to the test signal inserted. Such a pre-equalized signal is used for the measurements.

2)	The test arrangement is according to clause 8.1. For wideband systems MOS-LQOsw is determined. For narrowband systems MOS-LQOn is determined

The calculation is made using the signal recorded at the test point (DRP).

##### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the listening speech quality has to meet the requirements for the applicable Performance classes as given in the table below:

Table xx –Limits for the audio subsystem listening speech quality at nominal volume control setting

| Performance Class | MOS-LQOn | MOS-LQOsw |
| --- | --- | --- |
| 1 | > 4.2 | > 4.5 |
| 2 | > 3.8 | > 4.1 |
| 3 | > 3.4 | > 3.7 |
| 4 | ≤ 3.4 | ≤ 3.7 |

Table xx –Limits for the audio subsystem listening speech quality at maximum volume control setting

| Performance Class | MOS-LQOn | MOS-LQOsw |
| --- | --- | --- |
| 1 | > 4.2 | > 4.5 |
| 2 | > 3.8 | > 4.1 |
| 3 | > 3.4 | > 3.7 |
| 4 | ≤ 3.4 | ≤ 3.7 |

##### Design guidance and root-cause analysis

No additional impairment on listening speech quality should be introduced by the audio subsystem. Any degradation of listening speech quality will deteriorate the overall listening speech quality for the user in the car and should be avoided. Therefore the listening speech quality under quiet conditions is checked.

#### Audio subsystem idle channel noise

##### Parameter description

The audio subsystem transport idle channel noise in NAS is measured from the test point (R2) to the test point (DRP). The measurement is conducted at nominal and at maximum volume control setting.

##### Test

1)	The British-English single talk sequence described in clause 7.3.2 of  ITU-T Recommendation P.501 [xx] with the nominal average signal level [ITU-T P.50] is applied at the test point (R2). The test signal level is the average level of the complete test signal. The output level is measured using a speech level voltmeter according to ITU-T P.56. This level is the reference speech signal level.

2)	For the noise measurement, no test signal is used. However, all sources which potentially contribute to noise at the output of the unidirectional transport as GSM noise, electrical noise introduced by the car must be considered. In order to ensure a reliable activation, an activation signal is inserted before the actual measurement. The activation signal consists of a sequence of 4 composite source signals according to [ITU-T P.501]. The level of the activation signal is –16 dBm0.

3)	The test arrangement is according to clause 8.1.

The idle channel noise is measured at the output in the frequency range between 100 Hz and 8 kHz. The measurement requires a time window which starts exactly at the end of the activation signal. Any "ringing" of filters or receivers shall be taken into account, the time window must be shifted accordingly. The length of the time window is 1 second which is the averaging time for the idle channel noise. The test lab has to ensure the correct activation of the audio subsystem during the measurement. If the audio subsystem is deactivated during measurement, the measurement window has to be cut to the duration when the Audio subsystem remains activated.

The power density spectrum of the noise signal is determined using FFT (8 k samples/48 kHz sampling rate or equivalent). A Hann window is used.

If it is known that the audio subsystem stays activated without any activation signal, no activation signal is required. In this case, a simple noise measurement is conducted.

4)	The idle channel noise is determined by A-weighting and referring to the reference speech signal level as determined with the British-English test sentence. Spectral peaks are measured in the frequency domain. The average noise spectrum used for determining the spectral peak should be calculated as the arithmetic mean of the noise spectrum values when stated in dBVP.

##### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the speech to idle noise ratio SINR has to meet the requirements for the applicable Performance classes for the SINR as defined in table xx:

Spectral peaks in the frequency domain shall not exceed the averaged spectrum by more than 10 dB.

Table xx –Limits for the idle channel noise re. reference speech signal level at nominal volume control setting

| Performance Class | SINR |
| --- | --- |
| 1 | > 70 dB |
| 2 | > 60 dB |
| 3 | > 40 dB |
| 4 | ≤ 40 dB |

Table xx –Limits for the idle channel noise re. reference speech signal level at maximum volume control setting

| Performance Class | SINR |
| --- | --- |
| 1 | > 70 dB |
| 2 | > 60 dB |
| 3 | > 40 dB |
| 4 | ≤ 40 dB |

##### Design guidance and root-cause analysis

Check shielding, connectors and cabling with respect to potentially induced interfering signals. Check quality of amplifiers used to insert signals and preamplifiers used to receive signals. Check power amplifiers. Verify that preamplifiers do not demodulate RF into NF, If upsampling is used check artefacts due to upsampling.

#### Audio subsystem output level at maximum level setting

##### Parameter description

The audio subsystem maximum output level OLASmax is measured from the test point (R2) to the test point (DRP). The measurement is conducted at maximum volume control setting.

The audio subsystem maximum output level is determined with nominal input signal level at R2  and maximum volume control setting.

##### Test

- The test setup is according to cause 8.1.

- The test signal used is the British-English single talk sequence described in clause 7.3.2of  ITU-T Recommendation P.501 [xx] with the nominal average signal level applied at the test point (R2). The test signal level is the average level of the complete test signal.

- The maximum spectrum generated by the car audio system with maximum volume is recorded at the DRP.

- The A-weighted sound pressure level is determined at the DRP in the frequency range from 100 Hz to 7 kHz.

##### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the audio subsystem maximum output level has to meet the requirements for the applicable Performance classes for the audio subsystem maximum output level as defined in table xx:

Table xx –Limits for the audio subsystem maximum output level

| Performance Class | OLASmax |
| --- | --- |
| 1 | > 84 dB(A) |
| 2 | >78 dB(A) |
| 3 | >72 dB(A) |
| 4 | <72 dB(A) |

##### Design guidance and root-cause analysis

Insufficient output level will lead to poor speech intelligibility for the driver in the car. This may be especially problematic with high background noise e.g. at high driving speeds. In general luxury cars or cars with good, low noise acoustical design are less affected by this problem compared to cars with less acoustical treatment. The audio system design should ensure good speech intelligibility/comprehension with low listening effort at typical driving conditions.

#### Audio subsystem overload point

##### Parameter description

The audio subsystem overload point OVLAS is measured from the test point (R2) to the test point (LSP). The measurement is conducted at level setting of 15 dB above nominal volume and maximum control setting.

The audio subsystem overload point is determined by the highest signal which is produced by the audio subsystem in the car without distorting.

##### Test

- The test setup is according to cause 8.1.

- The test signal used is the British-English single talk sequence described in clause 7.3.2 of  ITU-T Recommendation P.501 [xx] with the nominal average signal level applied at the test point (R2). The test signal level is the average level of the complete test signal.

- The maximum spectrum generated by the car audio system with maximum volume is recorded with a reference microphone at the LSP.

- The coherence between the signal measured at the LSP and the signal inserted at R2 is determined as an average of the coherence function in the frequency range from 100 Hz to 7 kHz for wideband and 300 Hz to 3.5 kHz for narrowband system.

Note: In case of multiple loudspeaker s it is recommended to position the reference microphone close to a midrange speaker (for narrowband systems) or find a position as close as possible between the midrange and the high-frequency speaker (for wideband systems).

##### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the audio subsystem overload point has to meet the requirements for the applicable Performance classes for the audio subsystem overload point as defined in table xx:

Table xx –Limits for the audio subsystem overload point above 0.95 coherence

| Performance Class | OVL re Levmax |
| --- | --- |
| 1 | 0 dB |
| 2 | -3 dB |
| 3 | -6 dB |
| 4 | -9 dB |

Note: This table is used for both volume control settings independently.

##### Design guidance and root-cause analysis

Distortion in the audio subsystem will mainly cause two problems: Audible distortion to the driver in the car and nonlinearities in the echopath. While the user may tolerate quite some distortion the impact on the echo canceller may be more severe. Any non-linearity in the echo path may impair the convergence performance of the echo canceller and may lead to intolerable echo or switching caused by the NLP in the echo canceller. In such case either the maximum output volume should be limited or a less distorting audio subsystem is required.

#### Audio subsystem delay between REF and LSP

##### Parameter description

The delay TREF-DRP is measured from the test point (REF) to the test point (LSP). The delay TREF-LSP should be minimized. The measurement is conducted at nominal volume control setting.

##### Test

1)	For the measurements, a composite source signal (CSS) according to [ITU-T P.501] is used. The pseudo random noise (pn)-part of the CSS has to be longer than the maximum expected delay. It is recommended to use a pn sequence of 16 k samples (with 48 kHz sampling rate). The test signal is inserted at test point (R2) at a level  according to clause 8.1.5.

The reference signal is the signal measured at REF.

The test set-up is according to clause 8.1.

2)	The delay is determined by cross-correlation analysis between the measured signal at the LSP and the signal measured at REF. The measurement is corrected by delays which are caused by the test equipment. The measurement is corrected by delays which are caused by the test equipment.

3)	The delay is measured in ms and the maximum of the cross-correlation function is used for the determination.

Tmeasure  = TREF-LSP + TSystem

##### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the delay in the send direction has to meet the requirements for the applicable Performance classes for the delay TREF-LSP as defined in table xx:

Table xx –Limits for the audio subsystem delay between REF and LSP

| Performance Class | TREF-LSP |
| --- | --- |
| 1 | < ±3 ms |
| 2 | < ±6 ms |
| 3 | < ±10 ms |
| 4 | ≥ ±10 ms |

##### Design guidance and root-cause analysis

Low delay is for a proper functionalyity of teh echo canceller in the signal enhancement layer. Any delay between LSP and REF may impair the performance of the echo canceller and therefore should be avoided or minimized.

#### Audio Subsystem Linearity between reference output and LSP

##### Parameter description

The Audio Subsystem Linearity LINREF-LSP is measured from the test point (REF) to the test point (LSP).

The microphone linearity is determined by the measurement of the A-weighted output level between at LSP in the level range from -40 dB to +6 dB re. to the nominal input signal level and referring the measured level to the signal level measured at REF.

The measurement is conducted at nominal and at maximum volume control setting.

##### Test

- The test arrangement is according to clause 8.1.

- The test signal is inserted at (R2). The test signal used is the British-English single talk sequence described in clause 7.3.2 of ITU-T Recommendation P.501 [xx]. The correct activation of the measurement object during the measurement has to be ensured by the test lab. The test signal level shall be -40 dB, -30 dB, -20 dB, -10 dB, 0 dB and +6 dB re. to the nominal signal level. The test signal level is the average level of the complete test signal.

- The volume control is set to nominal.

- The signals are recorded simultaneously at the test points (LSP) and REF).

- The A-weighted output level is determined at the test point S2 and referred to the input signal level.

- The measurement is repeated for all input signal levels.

- The measurement is repeated for maximum volume control setting.

##### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the measured level difference between REF and LSP has to meet for all input signal levels the requirements for the applicable Performance classes as defined in table xx:

Table xx –Limits for the LINREF-LSP

| Performance Class | LINREF-LSP |
| --- | --- |
| 1 | < 0.2 dB |
| 2 | Xx |
| 3 | Xx |
| 4 | Xx |

##### Design guidance and root-cause analysis

#### Coherence between reference output and LSP

##### Parameter description

The REF – LSP coherence is measured from the test point (REF) to the test point (LSP).

The COHREF-LSP is determined between (REF) and (LSP) output in the level range from -40 dB to +6 dB re. to the nominal input signal level.

The measurement is conducted at nominal and at maximum volume control setting.

##### Test

- The test arrangement is according to clause 8.1.

- The test signal is inserted at (R2). The test signal used is the British-English single talk sequence described in clause 7.3.2of  ITU-T Recommendation P.501 [xx]. The correct activation of the measurement object during the measurement has to be ensured by the test lab. The test signal level shall be -40 dB, -30 dB, -20 dB, -10 dB, 0 dB and +6 dB re. to the nominal signal level. The test signal level is the average level of the complete test signal.

- The volume control is set to nominal.

- The signals are recorded simultaneously at the test points (LSP) and REF). The coherence between the LSP and REF is determined. The coherence is determined as an average of the coherence function in the frequency range from 100 Hz to 7 kHz.

- The measurement is repeated for all input signal levels.

- The measurement is repeated for maximum volume control setting.

##### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the COHREF-LSP has to meet the requirements for the applicable Performance classes for the TBD  as defined in table xx:

Table xx –Limits for the COHREF-LSP

| Performance Class | COHREF-LSP |
| --- | --- |
| 1 | X |
| 2 |  |
| 3 |  |
| 4 |  |

##### Design guidance and root-cause analysis

### Coupling between acoustical subsystems (echo path)

The focus of this test is to measure the electroacoustic coupling between loudspeaker and microphone. The test is targeted to optimize, positioning, packaging and type of microphone/speakers used by the hands-free system. The access points should be chosen as close as possible to the electro-acoustical components.

If speech enhancement is provided in the microphone subsystem, the measurement is conducted at S2.

Speech-to-Echo Ratio (SpER)

        - Echo Tail Delay (ETD)

        - Echo Tail coherence (ETco)

        - Check time invariance of processing (e.g. fixed beamformer…)

## Signal Enhancement Subsystem

The Signal Enhancement subsystem attempts to improve signals captured and played at the acoustic interface. Network-side problems can also be reduced or eliminated. Figure xx shows how this subsystem fits into the overall telecommunications system. This subsystem could be physically located in either the terminal device or a network device. However, the terminal device is usually a better location for technical reasons. Common signal processing functions provided by this subsystem include:

- Send direction

  - Acoustic echo cancellation

  - Noise reduction

  - Equalization

  - Automatic Level Control

  - Fixed gain

  - Limiter

- Receive direction

  - Bandwidth Extension

  - Equalization

  - Automatic Level Control

  - Automatic Level Enhancement

  - Fixed gain

  - Limiter

The next sub-clause describes the test set-up for measuring performance of the Signal Enhancement subsystem. Subsequent sub-clauses provide information related to each of the measurement parameters. This information includes a parameter description, test method, requirements, design guidance, and root-cause analysis.

### Test set-up

The test set-up for measuring performance of the Signal Enhancement subsystem consists of:

- Speech, noise test signals and impulse response traces including the effects of other subsystems involved

- A setup that enables simulation-based testing by:

  - Passing test signals to and from the signal enhancement layer

  - Simulating the dynamic effects introduced by other subsystems (e.g., acoustic interface)

Speech and noise test signals are processed to capture the effects of other subsystems involved in the end-to-end connection (e.g., vehicle microphones, speech coders, etc.).

In general the simulation can be made either file-based (pre-recorded test signals covering the effects of other subsystems) or simulation based (simulate the effects of other subsystems by impulse response traces).

The Signal Enhancement subsystem shall be tested with the set of signals acquired or simulated for the individual car under test. In case of Signal Enhancement Subsystem comparison e.g. for the purpose of system qualification for different cars a predefined set of test signals representative for the cars under consideration can be defined and used. An example and 3 subset of test signals are given in Annex XX.

[There are 3 subsets of test signals. Each subset of test signals is representative of a particular vehicle platform. All Signal Enhancement subsystem requirements shall be tested with each subset. Algorithm parameters of the Signal Enhancement subsystem may be adjusted between each subset, but shall not be adjusted during testing within a subset. Test results shall be reported separately for each subset. More information about the speech and noise database can be found in Annex <X>].

The setup enables testing the signal enhancement layer in file-mode or by simulation. Test signals are passed to test points S2 and R3 which represent the inputs to the signal enhancement layer. The processed output from the signal enhancement layer is retrieved from test points S3 and R2a and analyzed.

The output at REF is also convolved with an impulse response simulating the acoustic echo path in the vehicle. The simulated acoustic echo is then mixed with the send test signal before being presented to test point S2. This allows any changes of the test signal in the receive direction (e.g., gain change due to the signal enhancement layer) to be dynamically reflected in the input test signal at S2—which is what happens in the real world.

If pre-recorded signals are used information about the implementation for a framework can be found in Annex <X>.

If simulated signals are used information about the implementation for a framework can be found in Annex <X>.

Fig. 8.2: Signal Enhancement subsystem and test points

Note 1: Some Signal Enhancement subsystems might not use test point R2b.

Note 2: Ideally the access to the signal enhancement subsystem is made using the S3/R3 interface. However, if these interfaces are not available the S4/R4 or S5/R5 interface can be used applying the same requirements.

### Round-Trip Delay of  Signal Enhancement

The round-trip delay of  signal enhancement TSERTD consists of the delay in send direction the TSES plus the delay in receive direction TSER.

#### Performance level classification based on values of this parameter (TSERTD)

To claim compliance with a certain Performance class the delay in the send direction has to meet the requirements for the applicable Performance classes for the signal enhancement delay TSERTD as defined in table xx:

Table xx –Limits for the round trip delay of signal enhancement for speech communication services

| Performance Class | TSERTD |
| --- | --- |
| 1 | < 40 ms |
| 2 | < 70 ms |
| 3 | < 100 ms |
| 4 | ≥ 100 ms |

#### Design guidance and root-cause analysis

Low delay is essential for a seamless conversational performance especially for highly interactive conversations. Delay also contributes to echo perception: Higher delay low leads to higher impairment resulting even from low level echo components (see ITU-T Rec. G. 111 [xx]). Therefore any design providing as low delay in the connection as possible is preferable.

### Signal enhancement delay in send direction (TSES)

#### Parameter description

The delay in send direction TSES is measured from the test point (S2) to the test point (S3). The delay TSES should be minimized.

The system delay tSystem depends on the delay of the reference interface used. The delay tSystem must be known and deducted from the test result.

#### Test

1)	For the measurements, a composite source signal (CSS) according to [ITU-T P.501] is used. The pseudo random noise (pn)-part of the CSS has to be longer than the maximum expected delay. It is recommended to use a pn sequence of 16 k samples (with 48 kHz sampling rate). The test signal is inserted at test point (S2) at a level  according to clause 8.1.5.

The reference signal is the original signal (test signal).

The test set-up is according to clause 8.3.

2)	The delay is determined by cross-correlation analysis between the measured signal at the electrical access point (S3) and the original signal. The measurement is corrected by delays which are caused by the test equipment.

3)	The delay is measured in ms and the maximum of the cross-correlation function is used for the determination.

Tmeasure  = TSES + TSystem

### Signal enhancement delay in receive direction (TSER)

#### Parameter description

The signal enhancement delay in receive direction TSER is measured from the test point (R3) to the test point (R2).

The system delay tSystem depends on the delay of the reference interface used. The delay tSystem must be known and deducted from the test result.

#### Test

1)	For the measurements, a composite source signal (CSS) according to [ITU-T P.501] is used. The pseudo random noise (pn)-part of the CSS has to be longer than the maximum expected delay. It is recommended to use a pn sequence of 16 k samples (with 48 kHz sampling rate). The test signal is applied at test point (R3) at a level according to clause 8.1.5.

The reference signal is the original signal (test signal).

2)	The test arrangement is according to clause 8.3.

3)	The delay is determined by cross-correlation analysis between the measured signal at the test point (R2) and the original signal. The measurement is corrected by delays which are caused by the test equipment.

4)	The delay is measured in ms and the maximum of the cross-correlation function is used for the determination.

Tmeasure  = TSER + TSystem

### Signal enhancement send junction loudness ratings (JLRSES)

#### Parameter description

The send junction loudness rating JLR SES is measured from the test point (S2) to the test point (S3).

The JLR describes any amplification or attenuation on the network transport layer. The send junction loudness rating JLRSES should be 0 dB.

#### Test

1)	The test signal used for the measurements shall be British-English single talk sequence described in clause 7.3.2 of  ITU-T Recommendation P.501 [xx], recorded at the output of the microphone subsystem S2. The test signal level is the nominal signal level, the level is averaged over the complete test signal.

The measured power density spectrum at the test point interface (S2)is used as the reference power-density spectrum for determining the send sensitivity.

2)	The test arrangement is according to clause 8.3. For wideband the send sensitivity is calculated from each band of the 20 frequencies given in Table A.2 of [ITU-T P.79], bands 1-20. For narrowband bands 4-17 are used for calculation.

For the calculation, the average measured level at the electrical reference point for each frequency band is referred to the average test signal level measured in each frequency band at the reference interface.

3)	The sensitivity is expressed in dBV/V, the junction loudness rating JLRSES shall be calculated according to [ITU-T P.79], Formula A-23d, bands 1-20, M = 0.175, and the weighting factors WJ for JLR according to Table A.2 of [ITU-T P.79]. For narrowband bands 4-17 are used for the calculation.

#### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the JLR in send has to meet the requirements for the applicable Performance classes for the signal enhancement JLRSES as defined in table xx:

Table xx –Limits for the signal enhancement JLR in send

| Performance Class | JLRSES |
| --- | --- |
| 1 | 0 ± 0.5 dB |
| 2 | 0 ± 3 dB |
| 3 | 0 ± 6 dB |
| 4 | > ± 6 dB |

#### Design guidance and root-cause analysis

Any deviation of the JLRSES from 0 dB may result either in insufficient or too high speech levels and thus may deteriorate the performance of the interconnected devices and should be avoided.

### Receive signal enhancement loudness ratings (JLRSER)

#### Parameter description

The receive junction loudness rating JLRSER is measured from the test point (R3) to the test point (R2).

The JLR describes any amplification or attenuation on the network transport layer. The receive junction loudness rating JLRSER should be 0 dB.

#### Test

1)	The test signal used for the measurements shall be the British-English single talk sequence described in clause 7.3.2 of ITU-T Recommendation P.501 [xx], recorded at the output of the network transport subsystem R3. The test signal is the nominal signal level.

The measured power density spectrum at the test point interface (R3) is used as the reference power-density spectrum for determining the send sensitivity.

2)	The test arrangement is according to clause 8.3. For the calculation, the averaged level at the test point (R2) is used. In wideband the test point receive sensitivity is determined by the bands 1-20 according to Table A.2 of [ITU-T P.79]. For narrowband systems bands 4-17 are used.

For the calculation, the average signal level of each frequency band is referred to the signal level of the reference signal measured in each frequency band.

3)	The sensitivity is expressed in dBV/V, the test pointjunction loudness rating in receive JLRBDRv shall be calculated according to [ITU-T P.79], Formula A-23d, bands 1-20, M = 0.175, and the weighting factors WJ for JLR according to Table A.2 of [ITU-T P.79]. For narrowband systems bands 4-17 are used for the calculation.

#### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the JLR in receive at nominal setting of the volume control has to meet the requirements for the applicable Performance classes for the signal enhancement JLRSER as defined in table xx:

Table xx –Limits for the signal enhancement JLR in receive

| Performance Class | JLRSER |
| --- | --- |
| 1 | 0 ± 0.5 dB |
| 2 | 0 ± 3 dB |
| 3 | 0 ± 6 dB |
| 4 | > ± 6 dB |

#### Design guidance and root-cause analysis

Any deviation of the JLRSER from 0 dB may result either in insufficient or too high speech levels and thus may deteriorate the performance of the interconnected devices and should be avoided.

### Signal enhancement send sensitivity frequency response (FRSES)

#### Parameter description

The send frequency response FRSES is measured from the test point (S2) to the test point (S3). This parameter measures the sensitivity frequency response of the signal enhancement subsystem in send.

#### Test

1)	The test signal used for the measurements shall be the British-English single talk sequence described in clause 7.3.2 of ITU-T Recommendation P.501 [xx], recorded at the output of the microphone subsystem S2. The test signal level is the nominal signal level. The level is averaged over the complete test signal.

The measured power density spectrum at the MRP interface is used as the reference power density spectrum for determining the signal enhancement subsystem send sensitivity frequency response.

2)	The test arrangement is according to clause 8.3. In wideband the send sensitivity frequency response is determined in third octave intervals, as given by [IEC 61260] for frequencies of 100 Hz and 8 kHz, inclusive.  In narrowband it is determined for frequencies from 200 Hz to 4kHz. In each third octave band, the level of the measured signal is referred to the level of the reference signal averaged over the complete test sequence length.

3)	The sensitivity is determined in dBV/Pa.

#### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the frequency response in send has to meet the requirements for the applicable Performance classes for the signal enhancement subsystem FRSES as defined in the tables below:

The masks are drawn by straight lines between the breaking points in Table 9-xx to 9-xx on a logarithmic (frequency) – linear (dB sensitivity) scale.

Table xx – Tolerance mask for the wideband signal enhancement subsystem send sensitivity frequency response for Performance class 1

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 4 dB | –10 dB |
| 125 | 4 dB | –5 dB |
| 200 | 3 dB | –3 dB |
| 1 000 | 3 dB | –3 dB |
| 6 300 | 5 dB | –-3 dB |
| 7 600 | 5 dB | -6 dB |
| 8 000 | 5 dB | x |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the wideband signal enhancement subsystem send senTable xx – Tolerance mask for the wideband signal enhancement subsystem send sensitivity frequency response for Performance class 2

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 4 dB | – dB |
| 125 | 4 dB | –10 dB |
| 200 | 4 dB | –4 dB |
| 1 000 | 4 dB | –4 dB |
| 5 000 | (Note) | –4 dB |
| 6 300 | 9 dB | –7 dB |
| 8 000 | 9 dB | – dB |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the wideband signal enhancement subsystem send sensitivity frequency response for Performance class 3

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 6 dB | X |
| 125 | 6 dB | –15 dB |
| 200 | 6 dB | –6 dB |
| 1 000 | 6 dB | –6 dB |
| 5 000 | (Note) | –6 dB |
| 6 300 | 9 dB | –8 dB |
| 8 000 | X | X |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the wideband signal enhancement subsystem send sensitivity frequency response for Performance class 4

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | N/A | N/A |
| 125 | N/A | N/A |
| 200 | N/A | N/A |
| 1 000 | N/A | N/A |
| 5 000 | N/A | N/A |
| 6 300 | N/A | N/A |
| 8 000 | N/A | N/A |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the narrowband signal enhancement subsystem send sensitivity frequency response for Performance class 1

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 200 | 3 dB | –10 dB |
| 315 | 3 dB | –3 dB |
| 1 000 | 3 dB | –3 dB |
| 1 300 | 5 dB | –3 dB |
| 1 600 | 6 dB | –3 dB |
| 2 000 | 6 dB | –3 dB |
| 3 100 | 6 dB | –3 dB |
| 3 400 | (Note) | -6 dB |
| 4 000 | 3 dB | – dB |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the narrowband signal enhancement subsystem send sensitivity frequency response for Performance class 2

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 200 | 4 dB | – dB |
| 315 | 4 dB | –10 dB |
| 1 000 | 4 dB | –4 dB |
| 1 300 | 6 dB | –4 dB |
| 1 600 | 7 dB | –4 dB |
| 2 000 | 8 dB | –4 dB |
| 3 100 | 8 dB | –4 dB |
| 4 000 | 4dB | – dB |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the narrowband signal enhancement subsystem send sensitivity frequency response for Performance class 3

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 200 | 6 dB | X |
| 315 | 6 dB | –10 dB |
| 1 000 | 6 dB | –6 dB |
| 1 300 | 6 dB | –6 dB |
| 1 600 | (Note) | –6 dB |
| 2 000 | 9 dB | –6 dB |
| 3 100 | 9 dB | –10 dB |
| 4 000 | X |  |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the narrowband signal enhancement subsystem send sensitivity frequency response for Performance class 4

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 200 | N/A | N/A |
| 315 | N/A | N/A |
| 1 000 | N/A | N/A |
| 1 300 | N/A | N/A |
| 1 600 | N/A | N/A |
| 2 000 | N/A | N/A |
| 3 100 | N/A | N/A |
| 4 000 | N/A |  |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

T

#### Design guidance and root-cause analysis

Deviations from the desired frequency response characteristic may cause degradation of the listening speech quality in the send direction or may result in insufficient listening speech quality in the presence of background noise which affects the listing speech quality perceived by the far end subscriber.

### Signal enhancement receive sensitivity frequency response (FRSER)

#### Parameter description

The receive frequency response FRSER is measured from the test point (R3) to the test point (R2). This parameter measures the sensitivity frequency response of the signal enhancement subsystem in receive.

#### Test

1)	The test signal used for the measurements shall be the British-English single talk sequence described in clause 7.3.2 of ITU-T Recommendation P.501 [xx], recorded at the output of the network transport subsystem R3. The test signal level is the nominal signal level. The level is averaged over the complete test signal.

The measured power density spectrum at the test point (R3) interface is used as the reference power density spectrum for determining the signal enhancement subsystem receive sensitivity frequency response.

2)	The test arrangement is according to clause 8.3. In wideband the send sensitivity frequency response is determined in third octave intervals, as given by [IEC 61260] for frequencies of 100 Hz and 8 kHz, inclusive.  In narrowband it is determined for frequencies from 200 Hz to 4kHz. In each third octave band, the level of the measured signal is referred to the level of the reference signal averaged over the complete test sequence length.

3)	The sensitivity is determined in dBV/V.

4)	The measured result is corrected by the measured sensitivity frequency response characteristics of the audio subsystem in order to take into account any equalization potentially performed in the audio subsystem as well as the loudspeaker response characteristics. Alternatively a simulated R2 to DRP simulation could be used. In such a case the simulated output signal would be referred to the measured power density spectrum at the test point (R3) interface.

#### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the frequency response in receive has to meet the requirements for the applicable Performance classes for the signal enhancement subsystem FRSER as defined in the tables below:

The masks are drawn by straight lines between the breaking points in Table 9-xx to 9-xx on a logarithmic (frequency) – linear (dB sensitivity) scale.

Table xx – Tolerance mask for the wideband receive enhancement subsystem receive sensitivity frequency response for Performance class 1

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 3 dB | -6 dB |
| 200 | 3 dB | –3 dB |
| 6 300 | 3 dB | –3 dB |
| 7 600 | 3 dB | -6 dB |
| 8 000 | 3 dB | – dB |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the wideband signal enhancement subsystem receive Table xx – Tolerance mask for the wideband signal enhancement subsystem send sensitivity frequency response for Performance class 2

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 125 | 8 dB | – dB |
| 200 | 8 dB | –12 dB |
| 250 | 8 dB | –9 dB |
| 315 | 7 dB | –6 dB |
| 400 | 6 dB | –6 dB |
| 5 000 | 6 dB | –6 dB |
| 6 300 | 6 dB | –9 dB |
| 8 000 | 6 dB | – dB |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the wideband signal enhancement subsystem receive sensitivity frequency response for Performance class 3

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 8 dB | X |
| 200 | 8 dB | X |
| 250 | 8 dB | X |
| 315 | 8 dB | –9 dB |
| 400 | 6 dB | –6 dB |
| 5 000 | 6 dB | –9 dB |
| 7 000 | 6 dB | –12 dB |
| 8 000 | 6 dB | x |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the wideband signal enhancement subsystem receive sensitivity frequency response for Performance class 4

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | N/A | N/A |
| 125 | N/A | N/A |
| 200 | N/A | N/A |
| 1 000 | N/A | N/A |
| 5 000 | N/A | N/A |
| 6 300 | N/A | N/A |
| 8 000 | N/A | N/A |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the narrowband signal enhancement subsystem receive sensitivity frequency response for Performance class 1

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 200 | 3 dB | -6 dB |
| 250 | 3 dB | (Note) |
| 315 | 3 dB | –3 dB |
| 400 | 3 dB | –3 dB |
| 3 100 | 3 dB | –3 dB |
| 3 400 | 3 dB | -6 dB |
| 4 000 | 3 dB | – dB |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the narrowband signal enhancement subsystem receive sensitivity frequency response for Performance class 2

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 200 | 6 dB | – dB |
| 250 | 6 dB | – dB |
| 315 | 6 dB | –9 dB |
| 400 | 6 dB | –6 dB |
| 3 100 | 6 dB | –6 dB |
| 4 000 | 6 dB | – dB |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the narrowband signal enhancement subsystem receive sensitivity frequency response for Performance class 3

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 200 | 6 dB | – dB |
| 250 | 6 dB | – dB |
| 315 | 6 dB | –12 dB |
| 400 | 6 dB | –8dB |
| 3 100 | 6 dB | –8 dB |
| 4 000 | 6 dB | – dB |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the narrowband signal enhancement subsystem receive sensitivity frequency response for Performance class 4

| Frequency [Hz] | Upper limit | Lower limit |
| --- | --- | --- |
| 200 | N/A | N/A |
| 315 | N/A | N/A |
| 1 000 | N/A | N/A |
| 1 300 | N/A | N/A |
| 1 600 | N/A | N/A |
| 2 000 | N/A | N/A |
| 3 100 | N/A | N/A |
| 4 000 | N/A |  |
| NOTE – The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) – logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

T

#### Design guidance and root-cause analysis

Deviations from the desired frequency response characteristic may cause degradation of the listening speech quality for the subscriber in the car.

Careful design taking into account:

- Not too strong equalization in order to avoid overload at certain frequencies

- No counteraction equalization between audio subsystem and signal enhancement

### Signal Enhancement Send Speech Quality (MOS-LQO-SE)

#### Parameter description

This parameter predicts speech quality under quiet conditions in the Send direction after the Signal Enhancement subsystem.

#### Test

<Adapt from ITU-T P.1100/1110. Need to decide on objective model (e.g., ETSI, P.863, etc.). Note caveats and more relaxed requirements because of using perceptual-based model to measure signal intended for input of speech encoder.>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Receive Speech Quality (MOS-LQO-RE)

#### Parameter description

This parameter predicts speech quality in the Receive direction after the Signal Enhancement subsystem.

#### Test

<Adapt from ITU-T P.1100/1110. Need to decide on objective model (e.g., ETSI, P.863, etc.). Note caveats and more relaxed requirements because of using perceptual-based model to measure signal intended for input of speech encoder.>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Send Speech Quality Stability (MOS-LQO-STB-SE)

#### Parameter description

This parameter measures the reliability (i.e., consistency of measured results) of the Enhanced Send Speech Quality (MOS-LQO-SE) values repeatedly measured over a 3 to 5 minute interval.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Receive Speech Quality Stability (MOS-LQO-STB-RE)

#### Parameter description

This parameter measures the reliability (i.e., consistency of measured results) of the Enhanced Receive Speech Quality (MOS-LQO-RE) values repeatedly measured over a 3 to 5 minute interval.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Send Intelligibility (SIE)

#### Parameter description

This parameter predicts speech intelligibility in the Send direction after the Signal Enhancement subsystem. A measure of intelligibility is desirable because some automotive speech services (e.g., eCall) should be optimized for speech intelligibility/comprehension.

#### Test method

<Should consider both approaches that use signal-based measures (e.g., STI, AI, SII, etc.) and ASR-based approaches (see previous FG CarCOM contributions on this topic).>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Receive Intelligibility (RIE)

#### Parameter description

This parameter predicts speech intelligibility in the Receive direction after the Signal Enhancement subsystem. A measure of intelligibility is desirable because some automotive speech services (e.g., eCall) should be optimized for speech intelligibility/comprehension.

#### Test

<Should consider both approaches that use signal-based measures (e.g., STI, AI, SII, etc.) and ASR-based approaches (see previous FG CarCOM contributions on this topic).>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Send Idle Channel Noise (SICNE)

#### Parameter description

This parameter measures the idle channel noise, when no significant acoustic noise is present in the vehicle, in the Send direction after the Signal Enhancement subsystem.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Receive Idle Channel Noise (RICNE)

#### Parameter description

This parameter measures the idle channel noise, when there is no significant noise coming from the network or far end terminal, in the Receive direction after the Signal Enhancement subsystem.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Discrimination against out-of-band signals after Send Enhancement (DOSE)

#### Parameter description

This parameter measures the degree to which speech and noise signal frequencies, which are outside the transmission channel passband at the input to the Signal Enhancement subsystem, cause noise in the Send direction at the output of the Signal Enhancement subsystem.

#### Test

<Adapt from ITU-T P.1100/1110. Note that initial versions of the software test program may not support different input sample rates.>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Spurious out-of-band signal after Receive Enhancement (SORE)

#### Parameter description

This parameter measures the degree to which speech signal frequencies within the transmission channel passband cause spurious noise outside the passband at the output of the Signal Enhancement subsystem in the Receive direction.

#### Test

<Adapt from ITU-T P.1100/1110. Note that initial versions of the software test program may not support different input sample rates.>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Distortion in Send after Enhancement (DSE)

#### Parameter description

This parameter measures speech signal distortion in the Send direction after the Signal Enhancement subsystem.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Distortion in Receive after Enhancement (DRE)

#### Parameter description

This parameter measures speech signal distortion in the Receive direction after the Signal Enhancement subsystem.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Weighted Terminal Coupling Loss (TCLwE)

#### Parameter description

This parameter measures acoustic echo loss during Receive single-talk. Echo loss is measured from the input to the Signal Enhancement subsystem in the Receive direction, to the output of the Signal Enhancement subsystem in the Send direction.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Echo Level Versus Time (ELVTE)

#### Parameter description

This parameter measures acoustic echo loss as a function of time during Receive single-talk. Echo loss is measured from the input to the Signal Enhancement subsystem in the Receive direction, to the output of the Signal Enhancement subsystem in the Send direction.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Spectral Echo Attenuation (SEAE)

#### Parameter description

This set of parameters measures the frequency response of the acoustic echo path during Receive single-talk. The echo path frequency response is measured from the input to the Signal Enhancement subsystem in the Receive direction, to the output of the Signal Enhancement subsystem in the Send direction.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Initial Convergence without background noise (ICE)

#### Parameter description

This parameter measures acoustic echo loss as a function of time immediately after call set-up when there is no background noise. Echo loss is measured from the input to the Signal Enhancement subsystem in the Receive direction, to the output of the Signal Enhancement subsystem in the Send direction.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Initial Convergence With Background Noise (ICBNE)

#### Parameter description

This parameter measures acoustic echo loss as a function of time immediately after call set-up when background noise is present. Echo loss is measured from the input to the Signal Enhancement subsystem in the Receive direction, to the output of the Signal Enhancement subsystem in the Send direction.

#### Test method

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Echo Performance with Time Variant Echo Path (TVEPE)

#### Parameter description

This parameter measures acoustic echo loss when the echo path changes during the measurement. Echo loss is measured from the input to the Signal Enhancement subsystem in the Receive direction, to the output of the Signal Enhancement subsystem in the Send direction.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enahncement Echo Performance with Time Variant Echo Path and Artificial Voice (TVEPE-AV)

#### Parameter description

This parameter measures acoustic echo loss when the echo path changes during the measurement and the ITU-T P.50 artificial voice is used as the test signal. Echo loss is measured from the input to the Signal Enhancement subsystem in the Receive direction, to the output of the Signal Enhancement subsystem in the Send direction.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Send Activation (SAE)

#### Parameter description

This parameter measures the minimum speech signal level required to activate the Send transmission path up to the output of the Signal Enhancement subsystem in the Send direction.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Receive Activation (RAE)

#### Parameter description

This parameter measures the minimum speech signal level required to activate the Receive transmission path up to the output of the Signal Enhancement subsystem in the Receive direction.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Send Attenuation Range (SARE)

#### Parameter description

This parameter measures the range of attenuation in the Send direction during single-talk due to the Signal Enhancement subsystem.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Receive Attenuation Range (RARE)

#### Parameter description

This parameter measures the range of attenuation in the Receive direction during single-talk due to the Signal Enhancement subsystem.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Attenuation Range in Send during Double-Talk (ASDTE)

#### Parameter description

This parameter measures the range of attenuation in the Send direction during double-talk due to the Signal Enhancement subsystem.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Attenuation Range in Receive during Double-Talk (ARDTE)

#### Parameter description

This parameter measures the range of attenuation in the Receive direction during double-talk due to the Signal Enhancement subsystem.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Detection of Echo Components during Double-Talk after Enhancement (DECDTE)

#### Parameter description

This parameter measures acoustic echo loss during double-talk. Echo loss is measured from the input to the Signal Enhancement subsystem in the Receive direction, to the output of the Signal Enhancement subsystem in the Send direction.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Sent Speech Attenuation during Double-Talk (SSADTE)

#### Parameter description

This parameter measures the range of attenuation in the Send direction immediately after the onset double-talk due to the Signal Enhancement subsystem. It is intended to complement the “Enhanced Attenuation Range in Send during Double-Talk” measurement parameter by verifying the attenuation range at the onset of double-talk is consistent with that measured during double-talk.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Speech-to-Echo Ratio (SpERE)

#### Parameter description

This parameter provides a measurement of the Send speech level relative to the echo level at the output of the Signal Enhancement subsystem.

#### Test

<Need to identify best test method. Could use temporally-weighted terminal coupling loss w/ varying echo path to calculate echo portion.>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### SNR Improvement after Enhancement (SNRIE)

#### Parameter description

This parameter provides a measurement of the Send speech level relative to the noise level at the output of the Signal Enhancement subsystem.

#### Test

<This measurement parameter is listed for further study in P.1100. Could potentially use measures from G.160 Appendix II.>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Signal Enhancement Background Noise Transmission After Call Set-up (BGNTACSE)

#### Parameter description

This parameter predicts the transmitted noise quality after call set-up at the output of the Signal Enhancement subsystem.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### 8.3.3.4 Design guidance and root-cause analysis

For further study.

### Enhanced Speech Quality in the Presence of Background Noise (SQPBGNE)

#### Parameter description

This parameter predicts speech quality under noisy conditions in the Send direction after the Signal Enhancement subsystem.

#### Test

<Adapt from ITU-T P.1100/1110. Need to decide on objective model (e.g., ETSI, P.863, etc.). Note caveats and more relaxed requirements because of using perceptual-based model to measure signal intended for input of speech encoder.>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Enhanced Quality of Background Noise Transmission with Far End Speech (QBGNTFSE)

#### Parameter description

This parameter measures the variation in noise level at the output of the Signal Enhancement subsystem in the Send direction due to the presence/absence of speech in the Receive direction.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Enhanced Quality of Background Noise Transmission with Near End Speech (QBGNTNSE)

#### Parameter description

This parameter measures the variation in noise level at the output of the Signal Enhancement subsystem in the Send direction due to the presence/absence of speech in the Send direction.

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Background Noise Distortion Measure (xxx )

#### Parameter description

The noise distortion caused by background noise reduction algorithms of the signal enhancement subsystem can be measured by an objective noise distortion measure, which can be found in Annex E. This method can be used in addition to measurement of the enhanced speech quality in the presence of background noise (see xx) for investigating the root causes of background noise impairments in more detail and optimizing noise reduction algorithms.

#### Test

Some text has to be added -

See Annex E

#### Performance level classification based on values of this parameter

The performance classes are defined in Annex E.

#### Design guidance and root-cause analysis

Background noise attenuation achieved by the signal enhancement subsystem will lead to enhanced speech-to-noise ratio. In addition to that, background noise distortion may occur due to different factors like applying different noise reduction algorithms and different parameterizations inside a noise reduction algorithm. An objective noise distortion measure can be found in Annex E, which objectively measures the background noise distortion based only on the input background noise and the filtered background noise after the noise reduction algorithm. Such a noise distortion measure can be applied to optimize the parameterization of the noise reduction algorithm under test in the sense of background noise quality.

### Comfort Noise Injection after Enhancement (CNIE)

#### Parameter description

This set of parameters measures the difference in noise spectrum due to comfort noise injection at the output of the Signal Enhancement subsystem

#### Test

<Adapt from ITU-T P.1100/1110>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Speech Recognition Accuracy Indicator after Enhancement (SRAIE)

#### Parameter description

This parameter predicts Speech Recognition (SR) accuracy at the output of the Signal Enhancement subsystem.

#### Test

<Need to identify test method. Both signal-based (e.g., weighted SNR, AI, etc.) and reference speech recognizer-based (e.g., % correct, % confusions, etc.) measures should be considered.>

#### Performance level classification based on values of this parameter

<To be determined based on test results>

#### Design guidance and root-cause analysis

For further study.

### Send path

        - Enhanced Send Speech Level (SSLE)

        - Enhanced Speech-to-Noise Ratio (SpNRE)

        - Equalized Send Frequency Response (SFRE)

        - Enhanced Send ReVerberation (SRVE)

        - Enhanced Send Distortion + Noise (SDNE)

### Echo path

        - Enhanced Speech-to-Echo Ratio (SpERE)

        - Enhancement layer Round-Trip Delay (RTDE)

### Receive path

        - Enhanced Receive Speech Level (RSLE)

        - Enhanced Receive speech quality (MOS-LQOE)

        - Enhanced Receive Intelligibility (RIE)

## Network Access Subsystem

### Send path

        - Enhanced Send Speech Level (SSLE)

        - Enhanced Speech-to-Noise Ratio (SpNRE)

        - Equalized Send Frequency Response (SFRE)

        - Enhanced Send ReVerberation (SRVE)

        - Enhanced Send Distortion + Noise (SDNE)

### Echo path

        - Enhanced Speech-to-Echo Ratio (SpERE)

        - Enhancement layer Round-Trip Delay (RTDE)

### Receive path

        - Enhanced Receive Speech Level (RSLE)

        - Enhanced Receive speech quality (MOS-LQOE)

        - Enhanced Receive Intelligibility (RIE)

### Send-receive path coupling

      - See section in P.1100

- Acoustic interface

  - For each measurement parameter:

    - Parameter description

    - Test method

    - Single Pass/Fail or multiple thresholds of degradation for measured value

- Speech enhancement layer

  - For each measurement parameter:

    - Parameter description

    - Test method

    - Performance level classification based on values of this parameter

    - Design guidance and root-cause analysis

- Local transport

  - For each measurement parameter:

    - Parameter description

    - Test method

    - Performance level classification based on values of this parameter

    - Design guidance and root-cause analysis

- Network transport

  - For each measurement parameter:

    - Parameter description

    - Test method

    - Performance level classification based on values of this parameter

    - Design guidance and root-cause analysis

# Bidirectional Signal Transport: Wired and Short range Wireless Transmission

The purpose of this section is to ensure linearity (as far as possible) and time invariance of a bidirectional signal  transport over a wired or a short range wireless transmission system. Any bidirectional signal transport system including the network transport system in conjunction with the devices it is integrated in should not introduce any additional signal processing. Any other function than pure speech data transmission should be avoided.

The DUT is connected to the reference interfaces. The reference interfaces are calibrated as specified in clause 8.1.

## Bidirectional transport delay in send direction

### Parameter description

The delay in send direction TBDS is measured from the test point (S3) to the test point (S4) or to the POI (input of the reference speech coder of the system simulators) test point(S5). The test point (S3) may be the SRWAP. The delay TBDS should be minimized.

The system delay tSystem depends on the transmission method used, the delay of the reference interface and the network simulator. The delay tSystem must be known and deducted from the test result.

### Test

1)	For the measurements, a composite source signal (CSS) according to [ITU-T P.501] is used. The pseudo random noise (pn)-part of the CSS has to be longer than the maximum expected delay. It is recommended to use a pn sequence of 16 k samples (with 48 kHz sampling rate). The test signal is inserted at test point (S3) at a level  according to clause 8.1.5.

The reference signal is the original signal (test signal).

The test set-up is according to clause 9.1.

2)	The delay is determined by cross-correlation analysis between the measured signal at the electrical access point and the original signal. The measurement is corrected by delays which are caused by the test equipment.

3)	The delay is measured in ms and the maximum of the cross-correlation function is used for the determination.

Tmeasure  = TBDS + TSystem

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the delay in the send direction has to meet the requirements for the applicable Performance classes for the bidirectional transport delay TBDS as defined in table xx:

Table xx –Limits for the bidirectional transport delay in send direction for speech communication services

| Performance Class | TBDS |
| --- | --- |
| 1 | < 2 ms |
| 2 | < 10 ms |
| 3 | < 30 ms |
| 4 | ≥ 30 ms |

### Design guidance and root-cause analysis

Low delay is essential for a seamless conversational performance especially for highly interactive conversations. Delay also contributes to echo perception: Higher delay low leads to higher impairment resulting even from low level echo components (see ITU-T Rec. G. 111 [xx]). Therefore any design providing as low delay in the connection as possible is preferable.

## Bidirectional transport delay in receive direction

### Parameter description

The delay in receive direction TBDR is measured from the test point (R4) or the POI (input of the reference speech coder of the system simulators) (R5) to the test point (R3) which may be  the SRWAP (R3).

The system delay tSystem depends on the transmission system, the delay of the reference interface and on the network simulator used. The delay tSystem must be known and deducted from the test result.

### Test

1)	For the measurements, a composite source signal (CSS) according to [ITU-T P.501] is used. The pseudo random noise (pn)-part of the CSS has to be longer than the maximum expected delay. It is recommended to use a pn sequence of 16 k samples (with 48 kHz sampling rate). The test signal is applied at test point (R4) or the test point (R5)  at a level according to clause 8.1.5.

The reference signal is the original signal (test signal).

2)	The test arrangement is according to clause 9.1.

3)	The delay is determined by cross-correlation analysis between the measured signal at the SRWR and the original signal. The measurement is corrected by delays which are caused by the test equipment.

4)	The delay is measured in ms and the maximum of the cross-correlation function is used for the determination.

Tmeasure  = TBDR + TSystem

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the delay in the receive direction has to meet the requirements for the applicable Performance classes for the bidirectional transport delay TBDR as defined in table xx:

Table xx –Limits for the bidirectional transport delay in receive direction for speech communication services

| Performance Class | TBDR |
| --- | --- |
| 1 | < 2 ms |
| 2 | < 10 ms |
| 3 | < 30 ms |
| 4 | ≥ 30 ms |

### Design guidance and root-cause analysis

Low delay is essential for a seamless conversational performance especially for highly interactive conversations. Delay also contributes to echo perception: Higher delay low leads to higher impairment resulting even from low level echo components (see ITU-T Rec. G. 111 [xx]). Therefore any design providing as low delay in the connection as possible is preferable.

## Bidirectional transport send junction loudness ratings

### Parameter description

The send junction loudness rating JLRBDS is measured from the test point (S3) to the test point (S4) or to the POI (input of the reference speech coder of the system simulators) (S5). The test point (S3) may be the SRWAP.

The JLR describes any amplification or attenuation on the network transport layer. The send junction loudness rating JLRBDS should be 0 dB.

### Test

1)	The test signal used for the measurements shall be CSS according to [ITU-T P.501]. The test signal level is the nominal signal level, the level is averaged over the complete test signal.

The measured power density spectrum at the test point interface (S3)is used as the reference power-density spectrum for determining the bidirectional transport send sensitivity.

2)	The test arrangement is according to clause 9.1. For wideband the bidirectional send sensitivity is calculated from each band of the 20 frequencies given in Table A.2 of [ITU-T P.79], bands 1-20. For narrowband bands 4-17 are used for calculation.

For the calculation, the average measured level at the electrical reference point for each frequency band is referred to the average test signal level measured in each frequency band at the reference interface.

3)	The sensitivity is expressed in dBV/V, the junction loudness rating JLRBDS shall be calculated according to [ITU-T P.79], Formula A-23d, bands 1-20, M = 0.175, and the weighting factors WJ for JLR according to Table A.2 of [ITU-T P.79]. For narrowband bands 4-17 are used for the calculation.

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the JLR in send has to meet the requirements for the applicable Performance classes for the bidirectional transport JLRBDS as defined in table xx:

Table xx –Limits for the bidirectional transport JLR in send

| Performance Class | JLRBDS |
| --- | --- |
| 1 | 0 ± 0.5 dB |
| 2 | 0 ± 3 dB |
| 3 | 0 ± 6 dB |
| 4 | > ± 6 dB |

### Design guidance and root-cause analysis

Any deviation of the JLRBDS from 0 dB may result either in insufficient or too high speech levels and thus may deteriorate the performance of the interconnected devices. and should be avoided.

However, it is recognized that different SRW implementations deviate significantly (±6 dB) from this level. In order to maintain a good system performance, appropriate corrections have to be integrated in the hands-free implementation to ensure a good system performance. Based on these tests, appropriate level corrections can be inserted.

## Bidirectional transport receive loudness ratings

### Parameter description

The receive junction loudness rating JLRBDR is measured from the test point (R4) or the POI (input of the reference speech coder of the system simulators) (R5) to the test point (R3). The test point (R3) may be the SRWAP.

The JLR describes any amplification or attenuation on the network transport layer. The send junction loudness rating JLRBDR should be 0 dB.

### Test

1)	The test signal used for the measurements shall be CSS according to [ITU-T P.501]. The test signal is the nominal signal level.

2)	The test arrangement is according to clause 9.1. For the calculation, the averaged level at the test point (R4) (alternatively R5) interface is used. In wideband the test point receive sensitivity is determined by the bands 1-20 according to Table A.2 of [ITU-T P.79]. For narrowband systems bands 4-17 are used.

For the calculation, the average signal level of each frequency band is referred to the signal level of the reference signal measured in each frequency band.

3)	The sensitivity is expressed in dBV/V, the test pointjunction loudness rating in receive JLRBDRv shall be calculated according to [ITU-T P.79], Formula A-23d, bands 1-20, M = 0.175, and the weighting factors WJ for JLR according to Table A.2 of [ITU-T P.79]. For narrowband systems bands 4-17 are used for the calculation.

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the JLR in receive has to meet the requirements for the applicable Performance classes for the bidirectional transport JLRBDR as defined in table xx:

Table xx –Limits for the bidirectional transport JLR in receive

| Performance Class | JLRBDR |
| --- | --- |
| 1 | 0 ± 0.5 dB |
| 2 | 0 ± 3 dB |
| 3 | 0 ± 6 dB |
| 4 | > ± 6 dB |

### Design guidance and root-cause analysis

Any deviation of the JLRBDR from 0 dB may result either in insufficient or too high speech levels and thus may deteriorate the performance of the interconnected devices. and should be avoided.

However, it is recognized that different SRW implementations deviate significantly (±6 dB) from this level. In order to maintain a good system performance, appropriate corrections have to be integrated in the hands-free implementation to ensure a good system performance. Based on these tests, appropriate level corrections can be inserted.

## Bidirectional transport linearity in send direction

### Parameter description

The linearity of the send junction loudness rating JLRBDS is measured from the test point (S3) to the test point (S4) or to the POI (input of the reference speech coder of the system simulators) (S5). The test point (S3) may be the SRWAP.

The test is aimed to detect any amplitude non-linearities including AGC or companding.

### Test

1)	The test signal used for the measurements shall be CSS according to [ITU-T P.501]. The test signals are in the range of –40 dBV to 5 dBV in steps of 5 dB relative to the nominal signal level, measured at the test point (S3). The test signal level is the average level of the complete test signal.

The measured power density spectrum at the test point (S3) is used as the reference power-density spectrum for determining the send sensitivity.

2)	The test arrangement is according to clause 9.1. In wideband the test point send sensitivity is calculated from each band of the 20 frequencies given in Table A.2 of [ITU-T P.79], bands 1-20. For narrowband systems bands 4-17 are used.

For the calculation, the average measured level at the electrical reference point for each frequency band is referred to the average test signal level measured in each frequency band at the test point (S3) reference interface.

3)	In wideband the sensitivity is expressed in dBV/V, the bidirectional junction loudness rating JLRBDS shall be calculated according to [ITU-T P.79], Formula A-23d, bands 1-20, M = 0.175, and the weighting factors in send direction according to Table A.2 of [ITU-T P.79]. For narrowband systems bands 4-17 are used for the calculation.

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the JLR in send has to meet the requirements for the applicable Performance classes as defined in table xx. The bidirectional transport JLRBDS deviation is measured relative to the measured JLRBDSnom measured with nominal signal level:

Table xx –Limits for the bidirectional transport linearity in the send direction

| Performance Class | Input signal level range | JLRBDS re JLRBDSnom |
| --- | --- | --- |
| 1 | –40 dB/+5 dB | 0 ± 0.5 dB |
| 2 | –40 dB/+5 dB | 0 ± 3 dB |
| 3 | –40 dB/+5 dB | 0 ± 6 dB |
| 4 | –40 dB/+5 dB | > ± 6 dB |

### Design guidance and root-cause analysis

Any AGC or companding on the network transport should be avoided. Additional non-linearities of this type may counteract algorithms implemented in the signal processing subsystem and deteriorate it’s performance.

## Bidirectional transport linearity in receive direction

### Parameter description

The linearity of receive junction loudness rating JLRBDR is measured from the test point (R4) or the POI (input of the reference speech coder of the system simulators) (R5) to the test point (R3). The test point (R3) may be the SRWAP.

The test is aimed to detect any amplitude non-linearities including AGC or companding.

### Test

1)	The test signal used for the measurements shall be CSS according to [ITU-T P.501]. The test signals are in the range of –40 dBV to 5 dBV in steps of 5 dB relative to the nominal signal level measured at  test point (R4) (alternatively R5) . The test signal level is the average level of the complete test signal.

The measured power density spectrum at the test point (R4) (alternatively R5) is used as the reference power-density spectrum for determining the receive sensitivity.

2)	In wideband the test arrangement is according to clause 9.1. The test point receive sensitivity is calculated from each band of the 20 frequencies given in Table A.2 of [ITU-T P.79], bands 1-20. For narrowband bands 4-17 are used.

For the calculation, the average measured level at the test point for each frequency band is referred to the average test signal level measured in each frequency band at the electrical reference interface.

3)	The sensitivity is expressed in dBV/V, the bidirectional transport junction loudness rating JLRBDS shall be calculated according to [ITU-T P.79], Formula A-23d, bands 1-20, M = 0.175, and the weighting factors in send direction according to Table A.2 of [ITU-T P.79]. For narrowband bands 4-17 are used for the calculation.

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the JLR in receive has to meet the requirements for the applicable Performance classes as defined in table xx. The bidirectional transport JLRBDR deviation is measured relative to the measured JLRBDRnom measured with nominal signal level:

Table xx –Limits for the bidirectional transport linearity in the receive direction

| Performance Class | Input signal level range | JLRBDR re JLRBDRnom |
| --- | --- | --- |
| 1 | –40 dB/+5 dB | 0 ± 0.5 dB |
| 2 | –40 dB/+5 dB | 0 ± 3 dB |
| 3 | –40 dB/+5 dB | 0 ± 6 dB |
| 4 | –40 dB/+5 dB | > ± 6 dB |

### Design guidance and root-cause analysis

Any AGC or companding on the network transport should be avoided. Additional non-linearities of this type may counteract algorithms implemented in the signal processing subsystem and deteriorate it’s performance.

## Bidirectional transport send sensitivity frequency response

### Parameter description

The send frequency response FRBDS is measured from the test point (S3) to the test point (S4) or to the POI (input of the reference speech coder of the system simulators) (S5). The test point (S3) may be the SRWAP.The send sensitivity response on the network transport should be mostly flat in the entire frequency range in order not to interfere with any (wanted) response characteristic in the signal processing subsystem or the acoustical frontend.

### Test

1)	The test signal used for the measurements shall be the British-English single talk sequence described in clause 7.3.2of  ITU-T Recommendation P.501 [xx]  . The test signal level is the nominal signal level , the level is averaged over the complete test signal.

The measured power density spectrum at the test point (S3) interface is used as the reference power density spectrum for determining the bidirectional transport send sensitivity.

2)	The test arrangement is according to clause 9.1. In wideband the bidirectional send sensitivity is determined in third octave intervals, as given by [IEC 61260] for frequencies of 100 Hz and 8 kHz, inclusive.  In narrowband it is determined for frequencies from 200 Hz to 4kHz. In each third octave band, the level of the measured signal is referred to the level of the reference signal averaged over the complete test sequence length.

3)	The sensitivity is determined in dBV/V.

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the frequency response in send has to meet the requirements for the applicable Performance classes for the bidirectional transport FRBDS as defined in the tables below:

The masks are drawn by straight lines between the breaking points in Table 9-xx to 9-xx on a logarithmic (frequency) – linear (dB sensitivity) scale.

Table xx – Tolerance mask for the bidirectional transport wideband send sensitivity frequency response for Performance class 1

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 50 | 0.5 | –0.5 |
| 6 200 | 0.5 | –0.5 |
| 7 000 | 0.5 | –3 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the bidirectional transport wideband send sensitivity frequency response for Performance class 2

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 2 | –2 |
| 6 200 | 2 | –2 |
| 7 000 | 2 | –5 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the bidirectional transport wideband send sensitivity frequency response for Performance class 3

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 4 | –4 |
| 6 200 | 4 | –4 |
| 7 000 | 4 | –9 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the bidirectional transport wideband send sensitivity frequency response for Performance class 4

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | NA | NA |
| 6 200 | NA | NA |
| 7 000 | NA | NA |
| NOTE – There is no frequency tolerance mask requirement for class 4. |  |  |

Table xx – Tolerance mask for the bidirectional transport narrowband send sensitivity frequency response for Performance class 1

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 50 | 0.5 | –0.5 |
| 3 100 | 0.5 | –0.5 |
| 3 500 | 0.5 | –3 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the bidirectional transport narrowband send sensitivity frequency response for Performance class 2

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 2 | –2 |
| 3 100 | 2 | –2 |
| 3 500 | 2 | –5 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the bidirectional transport narrowband send sensitivity frequency response for Performance class 3

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 4 | –4 |
| 3 100 | 4 | –4 |
| 3 500 | 4 | –9 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the bidirectional transport narrowband send sensitivity frequency response for Performance class 4

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | NA | NA |
| 3 100 | NA | NA |
| 3 500 | NA | NA |
| NOTE – There is no frequency tolerance mask requirement for class 4. |  |  |

### Design guidance and root-cause analysis

Deviations from a flat frequency response characteristics may cause degradation of the listening speech quality in the send direction or may result in insufficient listening speech quality in the presence of background noise which affects the listing speech quality perceived by the far end subscriber.

## Bidirectional transport receive sensitivity frequency response

### Parameter description

The receive frequency response FRBDR is measured from the test point (R4) or the POI (input of the reference speech coder of the system simulators) (R5) to the test point (R3). The test point (R3) may be the SRWAP.

### Test

1)	The test signal used for the measurements shall be the British-English single talk sequence described in clause 7.3.2of  ITU-T Recommendation P.501 [xx]  . The test signal is the nominal signal level, measured at the test point (R4) (alternatively R5) and averaged over the complete test signal sequence.

2)	The test arrangement is according to clause 9.1. For wideband the bidirectional transport receive sensitivity is determined in third octave intervals as given by [IEC 61260] for frequencies of 100 Hz and 8 kHz, inclusive. In narrowband it is determined for frequencies from 200 Hz to 4kHz. In each third octave band, the level of the measured signal is referred to the level of the reference signal, averaged over the complete test sequence length.

3)	The sensitivity is determined in dBV/V.

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the frequency response in send has to meet the requirements for the applicable Performance classes for the bidirectional transport FRBDR as defined in the tables below:

The masks are drawn by straight lines between the breaking points in Table 9-xx to 9-xx on a logarithmic (frequency) – linear (dB sensitivity) scale.

Table xx – Tolerance mask for the bidirectional transport wideband receive sensitivity frequency response for Performance class 1

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 50 | 0.5 | –0.5 |
| 6 200 | 0.5 | –0.5 |
| 7 000 | 0.5 | –3 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the bidirectional transport wideband receive sensitivity frequency response for Performance class 2

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 2 | –2 |
| 6 200 | 2 | –2 |
| 7 000 | 2 | –5 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the bidirectional transport wideband receive sensitivity frequency response for Performance class 3

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 4 | –4 |
| 6 200 | 4 | –4 |
| 7 000 | 4 | –9 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the bidirectional transport wideband receive sensitivity frequency response for Performance class 4

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | NA | NA |
| 6 200 | NA | NA |
| 7 000 | NA | NA |
| NOTE – There is no frequency tolerance mask requirement for class 4. |  |  |

Table xx – Tolerance mask for the bidirectional transport narrowband receive sensitivity frequency response for Performance class 1

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 50 | 0.5 | –0.5 |
| 3 100 | 0.5 | –0.5 |
| 3 500 | 0.5 | –3 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the bidirectional transport wideband receive sensitivity frequency response for Performance class 2

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 2 | –2 |
| 3 100 | 2 | –2 |
| 3 500 | 2 | –5 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the bidirectional transport wideband receive sensitivity frequency response for Performance class 3

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 4 | –4 |
| 3 100 | 4 | –4 |
| 3 500 | 4 | –9 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the bidirectional transport narrowband receive sensitivity frequency response for Performance class 4

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | NA | NA |
| 3 100 | NA | NA |
| 3 500 | NA | NA |
| NOTE – There is no frequency tolerance mask requirement for class 4. |  |  |

### Design guidance and root-cause analysis

Deviations from a flat frequency response characteristics may cause degradation of the listening speech quality in the receive direction or may result in insufficient listening speech quality in the presence of background noise which affects the listing speech quality perceived by the subscriber in the car.

## Bidirectional transport idle channel noise in send

### Parameter description

The bidirectional transport idle channel noise in send NBDS is measured from the test point (S3) to the test point (S4) or to the POI (input of the reference speech coder of the system simulators) (S5). The test point (S3) may be the SRWAP.

### Test

1)	The British-English single talk sequence described in clause 7.3.2of  ITU-T Recommendation P.501 [xx]Artificial voice with nominal signal level is applied at the test point (S3). The test signal level is the average level of the complete test signal. The output level is measured using a speech level voltmeter according to ITU-T P.56. This level is the reference speech signal level.

2)	For the noise measurement, no test signal is used. However, all sources which potentially contribute to noise at the output of the bidirectional transport as GSM noise, electrical noise introduced by the car must be considered. In order to ensure a reliable activation, an activation signal is inserted before the actual measurement. The activation signal consists of a sequence of 4 composite source signals according to [ITU-T P.501]. The level of the activation signal is –16 dBm0.

3)	The test arrangement is according to clause 9.1.

The idle channel noise is measured at the output in the frequency range between 100 Hz and 8 kHz. The measurement requires a time window which starts exactly at the end of the activation signal. Any "ringing" of filters or receivers shall be taken into account, the time window must be shifted accordingly. The length of the time window is 1 second which is the averaging time for the idle channel noise. The test lab has to ensure the correct activation of the bidirectional transport during the measurement. If the bidirectional transport is deactivated during measurement, the measurement window has to be cut to the duration when the transport system remains activated.

The power density spectrum of the noise signal is determined using FFT (8 k samples/48 kHz sampling rate or equivalent). A Hann window is used.

If it is known that the bidirectional transport stays activated without any activation signal, no activation signal is required. In this case, a simple noise measurement is conducted.

4)	The idle channel noise is determined by A-weighting and referring to the reference speech signal level as determined with speech sequence. Spectral peaks are measured in the frequency domain. The average noise spectrum used for determining the spectral peak should be calculated as the arithmetic mean of the noise spectrum values when stated in dBVP.

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the speech to idle noise ratio SINR has to meet the requirements for the applicable Performance classes for the SINR  as defined in table xx:

Spectral peaks in the frequency domain shall not exceed the averaged spectrum by more than 10 dB.

Table xx –Limits for the idle channel noise re. reference speech signal level

| Performance Class | SINR |
| --- | --- |
| 1 | > 70 dB |
| 2 | > 60 dB |
| 3 | > 40 dB |
| 4 | ≤ 40 dB |

### Design guidance and root-cause analysis

Check shielding, connectors and cabling with respect to potentially induced interfering signals. Check quality of amplifiers used to insert signals and preamplifiers used to receive signals. Verify that preamplifiers do not demodulate RF into NF,

## Bidirectional transport idle channel noise in receive

### Parameter description

The bidirectional transport idle channel noise in send NBDR is measured from the test point (R4) or the POI (input of the reference speech coder of the system simulators) (R5) to the test point (R3). The test point (R3) may be the SRWAP.

### Test

1)	The British-English single talk sequence described in clause 7.3.2of  ITU-T Recommendation P.501 [xx] with nominal signal level  is applied at the test point (R4). The test signal level is the average level of the complete test signal. The output level is measured using a speech level voltmeter according to ITU-T P.56. This level is the reference speech signal level.

2)	For the noise measurement, no test signal is used. However, all sources which potentially contribute to noise at the output of the bidirectional transport as GSM noise, electrical noise introduced by the car must be considered. In order to ensure a reliable activation, an activation signal is inserted before the actual measurement. The activation signal consists of a sequence of 4 composite source signals according to [ITU-T P.501]. The level of the activation signal is –16 dBm0.

3)	The test arrangement is according to clause 9.1.

The idle channel noise is measured at the output in the frequency range between 100 Hz and 8 kHz. The measurement requires a time window which starts exactly at the end of the activation signal. Any "ringing" of filters or receivers shall be taken into account, the time window must be shifted accordingly. The length of the time window is 1 second which is the averaging time for the idle channel noise. The test lab has to ensure the correct activation of the bidirectional transport during the measurement. If the bidirectional transport is deactivated during measurement, the measurement window has to be cut to the duration when the transport system remains activated.

The power density spectrum of the noise signal is determined using FFT (8 k samples/48 kHz sampling rate or equivalent). A Hann window is used.

If it is known that the bidirectional transport stays activated without any activation signal, no activation signal is required. In this case, a simple noise measurement is conducted.

4)	The idle channel noise is determined by A-weighting and referring to the reference speech signal level as determined with speech sequence. Spectral peaks are measured in the frequency domain. The average noise spectrum used for determining the spectral peak should be calculated as the arithmetic mean of the noise spectrum values when stated in dBVP.

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the speech to idle noise ratio SINR has to meet the requirements for the applicable Performance classes for the SINR  as defined in table xx:

Spectral peaks in the frequency domain shall not exceed the averaged spectrum by more than 10 dB.

Table xx –Limits for the idle channel noise re. reference speech signal level

| Performance Class | SINR |
| --- | --- |
| 1 | > 70 dB |
| 2 | > 60 dB |
| 3 | > 40 dB |
| 4 | ≤ 40 dB |

### Design guidance and root-cause analysis

Check shielding, connectors and cabling with respect to potentially induced interfering signals. Check quality of amplifiers used to insert signals and preamplifiers used to receive signals. Verify that preamplifiers do not demodulate RF into NF,

## Bidirectional transport noise cancellation test in send direction

### Parameter description

The send noise cancellation NCBDS is measured from the test point (S3) to the test point (S4) or to the POI (input of the reference speech coder of the system simulators) (S5). The test point (S3) may be the SRWAP.

The objective of this test is to check whether no noise cancellation is active in send direction of the bidirectional transport.

### Test

1)	The test arrangement is according to clause 9.1.

2)	The test signal used for the measurements shall be pink noise with a duration of 5 s followed by a pause of 3 s, both repeated 3 times. The test signal during the active parts of the signal isthe nominal signal level, the level is averaged over the active parts of the test signal.

3)	The level of the transmitted signal is measured at the test point(S4) (alternatively S5) . The measured signal level is referred to the test signal level measured at test point (S3) and displayed vs time. The levels are calculated from the time domain using an integration time of 5 ms.

4)	The attenuation vs time is determined for each pink noise section.

–

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the NC in send has to meet the requirements for the applicable Performance classes for the bidirectional transport NCBDS as defined in table xx. The attenuation of the simulated background noise test signal (BGN) shall not deviate more than the given value in the relevant Performance class for all periods of the test signal.

Table xx – Tolerance for the bidirectional transport background noise attenuation

| Performance Class | BGN attenuation |
| --- | --- |
| 1 | <± 1 dB |
| 2 | < ± 2 dB |
| 3 | < ± 6 dB |

### Design guidance and root-cause analysis

Any activated (tandemed) noise cancellation would interfere with the noise cancellation in the signal processing subsystem and will lead to significant listening speech degradation in the send direction.

If a non-linear or time variant behaviour of the is observed, the tests, as described in ITU-T P.1100 [xx] clause 11.13 (for narrowband systems) or  P.1110 [xx] clause 11.13 (for wideband systems), can be applied to determine the behaviour of the bidirectional transport in more detail. Instead of inserting the tests signals acoustically, they have to be inserted electrically.

## Bidirectional transport one-way speech quality in send direction

### Parameter description

The bidirectional transport listening speech quality in send LQBDS is measured from the test point (S3) to the test point (S4) or to the POI (input of the reference speech coder of the system simulators) (S5). The test point (S3) may be the SRWAP.

The test is intended to determine any impairment of the listening speech quality introduced by the bidirectional transport.

### Test

The tests method to be used is ITU-T Recommendation P.863. NOTE: ITU-T Recommendation P.863 [xx] does not provide a wideband mode. Wideband systems are evaluated on a superwideband scale. Therefore the MOS Requirements are given in MOS-LQOsw. See P.863 for more information.

1)	The test signals used are the English test sequences as specified in [ITU-T P.501] (2 male speakers, 2 female speakers, two sentences each). The test signal level is the nominal signal level, measured at thetest point (S3), the test signal level is measured as "active speech level" according to [ITU-T P.56]. The speech activity should be between 30% and 70%.

The original speech signal is used as the reference signal for the determination of the speech quality.

2)	The test arrangement is according to clause 9.1. For wideband systems MOS-LQOsw is determined. For narrowband systems MOS-LQOn is determined

The calculation is made using the signal recorded at the test point (S4) (alternatively test point S5).

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the listening speech quality in send has to meet the requirements for the applicable Performance classes as given in the table below:

Table xx –Limits for the bidirectional transport listening speech quality in send

| Performance Class | MOS-LQOn | MOS-LQOsw |
| --- | --- | --- |
| 1 | > 4.2 | > 4.5 |
| 2 | > 3.8 | > 4.1 |
| 3 | > 3.4 | > 3.7 |
| 4 | ≤ 3.4 | ≤ 3.7 |

### Design guidance and root-cause analysis

No additional impairment on listening speech quality should be introduced by the network transport. Any degradation of listening speech quality will deteriorate the overall listening speech quality and should be avoided. Therefore the listening speech quality under quiet conditions is checked.

## Bidirectional transport speech quality stability in send direction

### Parameter description

The bidirectional transport listening speech quality stability in send LQSBDS is measured from the test point (S3) to the test point (S4) or to the POI (input of the reference speech coder of the system simulators) (S5). The test point (S3) may be the SRWAP.

This test is intended to detect speech quality instabilities due to unreliable connection (speech frame loss....). This test should be repeated in different representative test configurations, e.g. positions of the phone within the vehicle, different SRW links active and different WIFI links active.

### Test

Several measurements of MOS-LQOsw (for wideband systems) and MOS-LQOn (for narrowband systems)  according to Section xx are performed in series within the same call or for different calls, within the test arrangement defined below. Typically, for the same call, a measurement each 20 s issufficient. The results are reported in terms of statistics.

The procedure is identical for narrowband and wideband systems. In narrowband systems MOS-LQOn ist used, in wideband systems MOS-LQOsw.

The assessment of Listening Speech Quality Stability is preformed in 5 steps:

- To measure the MOS-LQO periodically over the duration of one communication. N measurements provide N MOS-LQO values.

- For each MOS-LQO (i) value i from 2 to N, MOS-LQO_GAP(i) is calculated as the absolute difference with the previous value  MOS-LQO (i-1):

MOS-LQO_GAP (i) =| MOS-LQO (i)- MOS-LQO (i-1)|

- In order to take into account the subjective perception and measurement accuracy, MOS-LQO_GAP (i) is set to 0 when the difference is equal or lower to THRESHOLD1:

- if MOS-LQO_GAP (i)> (2*THRESHOLD1), then MOS-LQO_GAP (i)= MOS-LQO_GAP (i)

- if THRESHOLD1 < MOS-LQO_GAP (i) ≦ 2*THRESHOLD1, then MOS-LQO_GAP (i)= [MOS-LQO_GAP (i) * 2] – (2*THRESHOLD1)

- if MOS-LQO_GAP (i)≦ THRESHOLD1, then MOS-LQO_GAP (i)=0

- The instability (INS_MOS-LQO) associated to the MOS-LQO over the whole N measurements is defined by mean value of MOS-LQO_GAP (i).

INS_MOS-LQO=1/(N-1)Σ MOS-LQO_GAP (i) with  i=[2:N]

- A linear weighting function is applied in order to express Stability ST-MOS-LQO on a 0 to 100 scale.

This formulation used to determine the Listening Quality Stability (ST-MOS) as

ST-MOS=100-(250* INS_MOS)

and

ST-MOS= 0 if [100-(250* INS_MOS)] < 0

When ST-MOS is calculated within a single call, the call should be longer than 3 minutes (Recommended duration being between 3 and 5 minutes). The duration of each measurement depends on the length of the speech samples used for the test as described in xx

(As an example, if a sample is 15s length and the analysis is done every 20 s a minimum of 10 values will be measured).

### Performance level classification based on values of this parameter

For the stability indicator about Listening Speech Quality, THRESHOLD1 = 0.1 and the linear weighting function applies in order to express Stability (ST-MOS) on a 0 to 100 scale. By definition Stability equals 100 when no variations occur and Stability ST-MOS equals 0 when MOS-LQO variation is equal or more than 0.4.

To claim compliance with a certain Performance class the listening speech quality stability in send has to meet the requirements for the applicable Performance classes as given in the table below:

Table xx –Limits for the bidirectional transport listening speech quality stability in send

| Performance Class | ST-MOSn | ST-MOSsw |
| --- | --- | --- |
| 1 | > 95 | > 95 |
| 2 | > 80 | > 80 |
| 3 | > 50 | > 50 |
| 4 | ≤ 50 | ≤ 50 |

### Design guidance and root-cause analysis

Listening quality stability during a call (if the position or transmission characteristics change during the call (or during several different calls) takes into account degradations generated on the signal by the transmission link impairment and the phone position.

In case of systems using SRW transmission it is the purpose to verify the integration of SRW radio network by evaluating change of speech quality over time. This will help in detect problems with RF coverage inside the car cabin and verify the error concealment (packet loss, bit errors) caused by weak RF link or interference with other radio.

A guidance to proceed is

1 - Check RF coverage from SRW unit to possible mobile positions (Protocol analyzer)

2 – Identify weak and bad reception areas inside the vehicle. (Protocol analyzer)

Check bit error rate, rate of packet loss …

3 – Use the speech quality measurement to rate the quality of error concealment in the weak areas identified in step 2 above and compare with measurements from areas with good coverage.

4 – Identify possible issues from interference with parallel SRW links or other networks. This will check how the SRW can handle interference and change to undisturbed channels. Speech quality measurements can be used to see the performance of handling these problems.

It’s recommended to use to a known link as reference.

## Bidirectional transport one-way speech quality in receive direction

### Parameter description

The bidirectional transport listening speech quality in receive LQBDR is measured from the test point (R4) or the POI (input of the reference speech coder of the system simulators) (R5) to the test point (R3). The test point (R3) may be the SRWAP.

The test is intended to determine any impairment of the listening speech quality introduced by the bidirectional transport.

### Test

The tests method to be used is ITU-T Recommendation P.863.

NOTE: ITU-T Recommendation P.863 [xx] does not provide a wideband mode. Wideband systems are evaluated on a superwideband scale. Therefore the MOS Requirements are given in MOS-LQOsw. See P.863 for more information.

1)	The test signals used are the English test sequences as specified in [ITU-T P.501] (2 male speakers, 2 female speakers, two sentences each). The test signal level ist the nominal signal level measured at test point (R4), alternatively (R5), the test signal level is measured as "active speech level" according to [ITU-T P.56]. The speech activity should be between 30% and 70%.

The original speech signal is used as the reference signal for the determination of the speech quality.

2)	The test arrangement is according to clause 9.1. For wideband systems MOS-LQOsw is determined, for narrowband systems MOS-LQOn. The signal measured at the test point (R3) is used for the calculation

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the listening speech quality in send has to meet the requirements for the applicable Performance classes as given in the table below:

Table xx –Limits for the bidirectional transport listening speech quality in receive

| Performance Class | MOS-LQOn | MOS-LQOsw |
| --- | --- | --- |
| 1 | > 4.2 | > 4.5 |
| 2 | > 3.8 | > 4.1 |
| 3 | > 3.4 | > 3.7 |
| 4 | ≤ 3.4 | ≤ 3.7 |

### Design guidance and root-cause analysis

No additional impairment on listening speech quality should be introduced by the network transport. Any degradation of listening speech quality will deteriorate the overall listening speech quality and should be avoided. Therefore the listening speech quality under quiet conditions is checked.

## Bidirectional transport speech quality stability in receive direction

### Parameter description

The bidirectional transport listening speech quality in receive LQBDR is measured from the test point (R4) or the POI (input of the reference speech coder of the system simulators) (R5) to the test point (R3). The test point (R3) may be the SRWAP.

This test is intended to detect speech quality instabilities due to unreliable SRW connection (speech frame loss....). This test should be repeated in different representative positions of the phone within the vehicle, different SRW links active and different WIFI links active.

### Test

The test procedure is described in Sectionxx.

### Performance level classification based on values of this parameter

For the stability indicator about Listening Speech Quality, THRESHOLD1 = 0.1 and the linear weighting function applies in order to express Stability (ST-MOS) on a 0 to 100 scale. By definition Stability equals 100 when no variations occur and Stability ST-MOS equals 0 when MOS-LQO variation is equal or more than 0.4.

To claim compliance with a certain Performance class the listening speech quality stability in send has to meet the requirements for the applicable Performance classes as given in the table below:

Table xx –Limits for the bidirectional transport listening speech quality stability in receive

| Performance Class | ST-MOSn | ST-MOSsw |
| --- | --- | --- |
| 1 | > 95 | > 95 |
| 2 | > 80 | > 80 |
| 3 | > 50 | > 50 |
| 4 | ≤ 50 | ≤ 50 |

### Design guidance and root-cause analysis

Listening quality stability during a call (if the position or transmission characteristics change during the call (or during several different calls) takes into account degradations generated on the signal by the transmission link impairment and the phone position.

In case of systems using SRW transmission it is the purpose to verify the integration of SRW radio network by evaluating change of speech quality over time. This will help in detect problems with RF coverage inside the car cabin and verify the error concealment (packet loss, bit errors) caused by weak RF link or interference with other radio.

A guidance to proceed is

1 - Check RF coverage from SRW unit to possible mobile positions (Protocol analyzer)

2 – Identify weak and bad reception areas inside the vehicle. (Protocol analyzer)

Check bit error rate, rate of packet loss …

3 – Use the speech quality measurement to rate the quality of error concealment in the weak areas identified in step 2 above and compare with measurements from areas with good coverage.

4 – Identify possible issues from interference with parallel SRW links or other networks. This will check how the SRW can handle interference and change to undisturbed channels. Speech quality measurements can be used to see the performance of handling these problems.

It’s recommended to use to a known link as reference.

## Verification of bidirectional transport disabled echo control

### Parameter description

The bidirectional transport echo control ECBD is measured from the test point (R4) or the POI (input of the reference speech coder of the system simulators) (R5) to the test point (S4) or the POI (input of the reference speech coder of the system simulators) (S5).

No acoustic echo control shall be active in the mobile phone.

### Test

1)	For the test, an artificial echo path is inserted at thetest point (R4), alternatively (R5). The artificial echo path consists of an attenuation of 20 dB/40 dB and a delay of 20 ms is inserted at the test point (S3/R3) interface. The test set-up is shown in Figure xx:

Figure 12-3 – Test set-up with artificial echo loss

2)	The attenuation between the input of the test point (R4) to the output of the test point (R4) is measured using a speech-like test signal. Alternatively the measurement is conducted from the input of test point (R5) to the output of test point (S5).

3)	Before the actual measurement, a training sequence consisting of the British-English single talk sequence described in clause 7.3.2of  ITU-T Recommendation P.501 [xx] is inserted. The training sequence level is the nominal signal level.

4)	The test signal is a pn sequence, according to [ITU-T P.501], with a length of 4 096 points (48 kHz sampling rate) and a crest factor of 6 dB. The duration of the test signal is 250 ms, the test signal level is  -6 dBov as applicable to the individual interface. The low crest factor is achieved by random alternation of the phase between –180° and +180°.

5)	TCLW is calculated according to clause B.4 of [ITU-T G.122] (trapezoidal pseudo rule). For the calculation, the average measured echo level at each frequency band is referred to the average level of the test signal measured in each frequency band. For the measurement, a time window has to be applied which is adapted to the duration of the actual test signal (250 ms).

6)	The difference between the echo loss measured with 20 dB echo loss and 40 dB echo loss is determined.

### Performance level classification based on values of this parameter

The difference between the echo loss ELDBD measured with 20 dB echo loss and 40 dB echo loss is measured. To claim compliance with a certain Performance class the listening speech quality stability in send has to meet the requirements for the applicable Performance classes as given in the table below:

Table xx –Limits for the bidirectional transport echo loss difference

| Performance Class | ELDBDmin | ELDBDmax |
| --- | --- | --- |
| 1 | 19.8 dB | 20.2 dB |
| 2 | N.A. | N.A. |
| 3 | N.A. | N.A. |
| 4 | ≤ 19.8 dB | ≥ 20.2 dB |

### Design guidance and root-cause analysis

Any additional echo cancellation would impair the performance of the acoustic echo cancellation integrated in the signal processing subsystem. As a result additional impairments on speech quality and double talk performance will occur.

# Unidirectional Signal Transport: Wired and Short range Wireless Transmission

The purpose of this section is to ensure linearity (as far as possible) and time invariance of the signal transport over wired or a short range wireless transmission system. Any unidirectional transport system in conjunction with the devices it is integrated in should not introduce any additional signal processing. Any other functions than pure speech data transmission should be avoided.

## Test setup

The DUT is connected to the reference interfaces. The reference interfaces are calibrated as specified in clause 8.1.

## Unidirectional transport delay

### Parameter description

The delay TUD is measured from the test point (xx) to the test point (xy). The delay TUD should be minimized.

The system delay tSystem depends on the transmission method used, the delay of the reference interface and the network simulator. The delay tSystem must be known and deducted from the test result.

### Test

1)	For the measurements, a composite source signal (CSS) according to [ITU-T P.501] is used. The pseudo random noise (pn)-part of the CSS has to be longer than the maximum expected delay. It is recommended to use a pn sequence of 16 k samples (with 48 kHz sampling rate). The test signal is inserted at test point (S3) at a level  according to clause 8.1.5.

The reference signal is the original signal (test signal).

The test set-up is according to clause 9.1.

2)	The delay is determined by cross-correlation analysis between the measured signal at the electrical access point and the original signal. The measurement is corrected by delays which are caused by the test equipment.

3)	The delay is measured in ms and the maximum of the cross-correlation function is used for the determination.

Tmeasure  = TUD + TSystem

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the delay in the send direction has to meet the requirements for the applicable Performance classes for the bidirectional transport delay TUD as defined in table xx:

Table xx –Limits for the unidirectional transport delay for speech communication services

| Performance Class | TUD |
| --- | --- |
| 1 | < 2 ms |
| 2 | < 10 ms |
| 3 | < 30 ms |
| 4 | ≥ 30 ms |

### Design guidance and root-cause analysis

Low delay is essential for a seamless conversational performance especially for highly interactive conversations. Delay also contributes to echo perception: Higher delay low leads to higher impairment resulting even from low level echo components (see ITU-T Rec. G. 111 [xx]). Therefore any design providing as low delay in the connection as possible is preferable.

## Unidirectional transport junction loudness ratings

### Parameter description

The junction loudness rating JLRUD is measured from the test point (xx) to the test point (xy).

The JLR describes any amplification or attenuation on the transport layer. The junction loudness rating JLRUD should be 0 dB.

### Test

1)	The test signal used for the measurements shall be CSS according to [ITU-T P.501]. The test signal level is the nominal signal level, the level is averaged over the complete test signal.

The measured power density spectrum at the test point interface (S3)is used as the reference power-density spectrum for determining the unidirectional transport send sensitivity.

2)	The test arrangement is according to clause 9.1. For wideband the unidirectional sensitivity is calculated from each band of the 20 frequencies given in Table A.2 of [ITU-T P.79], bands 1-20. For narrowband bands 4-17 are used for calculation.

For the calculation, the average measured level at the electrical reference point for each frequency band is referred to the average test signal level measured in each frequency band at the reference interface.

3)	The sensitivity is expressed in dBV/V, the junction loudness rating JLRUD shall be calculated according to [ITU-T P.79], Formula A-23d, bands 1-20, M = 0.175, and the weighting factors WJ for JLR according to Table A.2 of [ITU-T P.79]. For narrowband bands 4-17 are used for the calculation.

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the JLR has to meet the requirements for the applicable Performance classes for the bidirectional transport JLRUD as defined in table xx:

Table xx –Limits for the unidirectional transport JLR

| Performance Class | JLRUD |
| --- | --- |
| 1 | 0 ± 0.5 dB |
| 2 | 0 ± 3 dB |
| 3 | 0 ± 6 dB |
| 4 | > ± 6 dB |

### Design guidance and root-cause analysis

Any deviation of the JLRBDS from 0 dB may result either in insufficient or too high speech levels and thus may deteriorate the performance of the interconnected devices. and should be avoided.

However, it is recognized that different SRW implementations deviate significantly (±6 dB) from this level. In order to maintain a good system performance, appropriate corrections have to be integrated in the hands-free implementation to ensure a good system performance. Based on these tests, appropriate level corrections can be inserted.

## Unidirectional transport linearity

### Parameter description

The linearity of the junction loudness rating JLRUD is measured from the test point (xx) to the test point (xy)

The test is aimed to detect any amplitude non-linearities including AGC or companding.

### Test

1)	The test signal used for the measurements shall be CSS according to [ITU-T P.501]. The test signals are in the range of –40 dBV to 5 dBV in steps of 5 dB relative to the nominal signal level, measured at the test point (xx). The test signal level is the average level of the complete test signal.

The measured power density spectrum at the test point (xx) is used as the reference power-density spectrum for determining the send sensitivity.

2)	The test arrangement is according to clause 9.1. In wideband the sensitivity is calculated from each band of the 20 frequencies given in Table A.2 of [ITU-T P.79], bands 1-20. For narrowband systems bands 4-17 are used.

For the calculation, the average measured level at the electrical reference point for each frequency band is referred to the average test signal level measured in each frequency band at the test point (xx) reference interface.

3)	In wideband the sensitivity is expressed in dBV/V, the bidirectional junction loudness rating JLRUD shall be calculated according to [ITU-T P.79], Formula A-23d, bands 1-20, M = 0.175, and the weighting factors in send direction according to Table A.2 of [ITU-T P.79]. For narrowband systems bands 4-17 are used for the calculation.

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the JLR has to meet the requirements for the applicable Performance classes as defined in table xx. The bidirectional transport JLRUD deviation is measured relative to the measured JLRUDnom measured with nominal signal level:

Table xx –Limits for the bidirectional transport linearity

| Performance Class | Input signal level range | JLRUD re JLRUDnom |
| --- | --- | --- |
| 1 | –40 dB/+5 dB | 0 ± 0.5 dB |
| 2 | –40 dB/+5 dB | 0 ± 3 dB |
| 3 | –40 dB/+5 dB | 0 ± 6 dB |
| 4 | –40 dB/+5 dB | > ± 6 dB |

### Design guidance and root-cause analysis

Any AGC or companding on the network transport should be avoided. Additional non-linearities of this type may counteract algorithms implemented in the signal processing subsystem and deteriorate its performance.

## Unidirectional transport sensitivity frequency response

### Parameter description

The frequency response FRUD is measured from the test point (xx) to the test point (xy)

The sensitivity response on the network transport should be mostly flat in the entire frequency range in order not to interfere with any (wanted) response characteristic in the signal processing subsystem or the acoustical frontend.

### Test

1)	The test signal used for the measurements shall be the British-English single talk sequence described in clause 7.3.2of  ITU-T Recommendation P.501 [xx] . The test signal level is the nominal signal level , the level is averaged over the complete test signal.

The measured power density spectrum at the test point (xx) interface is used as the reference power density spectrum for determining the unidirectional transport send sensitivity.

2)	The test arrangement is according to clause 9.1. In wideband the unidirectional send sensitivity is determined in third octave intervals, as given by [IEC 61260] for frequencies of 100 Hz and 8 kHz, inclusive.  In narrowband it is determined for frequencies from 200 Hz to 4kHz. In each third octave band, the level of the measured signal is referred to the level of the reference signal averaged over the complete test sequence length.

3)	The sensitivity is determined in dBV/V.

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the frequency response has to meet the requirements for the applicable Performance classes for the unidirectional transport FRUD as defined in the tables below:

The masks are drawn by straight lines between the breaking points in Table 9-xx to 9-xx on a logarithmic (frequency) – linear (dB sensitivity) scale.

Table xx – Tolerance mask for the unidirectional transport wideband sensitivity frequency response for Performance class 1

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 50 | 0.5 | –0.5 |
| 6 200 | 0.5 | –0.5 |
| 7 000 | 0.5 | –3 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the unidirectional transport wideband sensitivity frequency response for Performance class 2

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 2 | –2 |
| 6 200 | 2 | –2 |
| 7 000 | 2 | –5 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the unidirectional transport wideband sensitivity frequency response for Performance class 3

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 4 | –4 |
| 6 200 | 4 | –4 |
| 7 000 | 4 | –9 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the unidirectional transport wideband sensitivity frequency response for Performance class 4

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | NA | NA |
| 6 200 | NA | NA |
| 7 000 | NA | NA |
| NOTE – There is no frequency tolerance mask requirement for class 4. |  |  |

Table xx – Tolerance mask for the unidirectional transport narrowband sensitivity frequency response for Performance class 1

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 50 | 0.5 | –0.5 |
| 3 100 | 0.5 | –0.5 |
| 3 500 | 0.5 | –3 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the unidirectional transport narrowband sensitivity frequency response for Performance class 2

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 2 | –2 |
| 3 100 | 2 | –2 |
| 3 500 | 2 | –5 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the unidirectional transport narrowband sensitivity frequency response for Performance class 3

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | 4 | –4 |
| 3 100 | 4 | –4 |
| 3 500 | 4 | –9 |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table xx – Tolerance mask for the unidirectional transport narrowband sensitivity frequency response for Performance class 4

| Frequency (Hz) | Upper limit | Lower limit |
| --- | --- | --- |
| 100 | NA | NA |
| 3 100 | NA | NA |
| 3 500 | NA | NA |
| NOTE – There is no frequency tolerance mask requirement for class 4. |  |  |

### Design guidance and root-cause analysis

Deviations from a flat frequency response characteristics may cause degradation of the listening speech quality in the send direction or may result in insufficient listening speech quality in the presence of background noise which affects the listing speech quality perceived by the far end subscriber.

## Unidirectional transport idle channel noise

### Parameter description

The unidirectional transport idle channel noise in NUD is measured from the test point (Sxx) to the test point (Sxy).

### Test

1)	The British-English single talk sequence described in clause 7.3.2of  ITU-T Recommendation P.501 [xx] with nominal signal level  is applied at the test point (S3). The test signal level is the average level of the complete test signal. The output level is measured using a speech level voltmeter according to ITU-T P.56. This level is the reference speech signal level.

2)	For the noise measurement, no test signal is used. However, all sources which potentially contribute to noise at the output of the unidirectional transport as GSM noise, electrical noise introduced by the car must be considered. In order to ensure a reliable activation, an activation signal is inserted before the actual measurement. The activation signal consists of a sequence of 4 composite source signals according to [ITU-T P.501]. The level of the activation signal is –16 dBm0.

3)	The test arrangement is according to clause 9.1.

The idle channel noise is measured at the output in the frequency range between 100 Hz and 8 kHz. The measurement requires a time window which starts exactly at the end of the activation signal. Any "ringing" of filters or receivers shall be taken into account, the time window must be shifted accordingly. The length of the time window is 1 second which is the averaging time for the idle channel noise. The test lab has to ensure the correct activation of the unidirectional transport during the measurement. If the unidirectional transport is deactivated during measurement, the measurement window has to be cut to the duration when the transport system remains activated.

The power density spectrum of the noise signal is determined using FFT (8 k samples/48 kHz sampling rate or equivalent). A Hann window is used.

If it is known that the unidirectional transport stays activated without any activation signal, no activation signal is required. In this case, a simple noise measurement is conducted.

4)	The idle channel noise is determined by A-weighting and referring to the reference speech signal level as determined with speech sequence. Spectral peaks are measured in the frequency domain. The average noise spectrum used for determining the spectral peak should be calculated as the arithmetic mean of the noise spectrum values when stated in dBVP.

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the speech to idle noise ratio SINR has to meet the requirements for the applicable Performance classes for the SINR  as defined in table xx:

Spectral peaks in the frequency domain shall not exceed the averaged spectrum by more than 10 dB.

Table xx –Limits for the idle channel noise re. reference speech signal level

| Performance Class | SINR |
| --- | --- |
| 1 | > 70 dB |
| 2 | > 60 dB |
| 3 | > 40 dB |
| 4 | ≤ 40 dB |

### Design guidance and root-cause analysis

Check shielding, connectors and cabling with respect to potentially induced interfering signals. Check quality of amplifiers used to insert signals and preamplifiers used to receive signals. Verify that preamplifiers do not demodulate RF into NF,

## Unidirectional transport noise cancellation test

### Parameter description

The send noise cancellation NCUD is measured from the test point (xx) to the test point (xy)

The objective of this test is to check whether no noise cancellation is active in the unidirectional transport.

### Test

1)	The test arrangement is according to clause 9.1.

2)	The test signal used for the measurements shall be pink noise with a duration of 5 s followed by a pause of 3 s, both repeated 3 times. The test signal during the active parts of the signal isthe nominal signal level, the level is averaged over the active parts of the test signal.

3)	The level of the transmitted signal is measured at the test point(xy) . The measured signal level is referred to the test signal level measured at test point (xx) and displayed vs time. The levels are calculated from the time domain using an integration time of 5 ms.

4)	The attenuation vs time is determined for each pink noise section.

–

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the NC in send has to meet the requirements for the applicable Performance classes for the unidirectional transport NCUD as defined in table xx. The attenuation of the simulated background noise test signal (BGN) shall not deviate more than the given value in the relevant Performance class for all periods of the test signal.

Table xx – Tolerance for the unidirectional transport background noise attenuation

| Performance Class | BGN attenuation |
| --- | --- |
| 1 | <± 1 dB |
| 2 | < ± 2 dB |
| 3 | < ± 6 dB |

### Design guidance and root-cause analysis

Any activated (tandemed) noise cancellation would interfere with the noise cancellation in the signal processing subsystem and will lead to significant listening speech degradation in the send direction.

If a non-linear or time variant behaviour of the is observed, the tests, as described in ITU-T P.1100 [xx] clause 11.13 (for narrowband systems) or  P.1110 [xx] clause 11.13 (for wideband systems), can be applied to determine the behaviour of the unidirectional transport in more detail. Instead of inserting the tests signals acoustically, they have to be inserted electrically.

## Unidirectional transport speech quality

### Parameter description

The unirectional transport listening speech quality LQUD is measured from the test point (xx) to the test point (xy).

The test is intended to determine any impairment of the listening speech quality introduced by the unidirectional transport.

### Test

The tests method to be used is ITU-T Recommendation P.863. NOTE: ITU-T Recommendation P.863 [xx] does not provide a wideband mode. Wideband systems are evaluated on a superwideband scale. Therefore the MOS Requirements are given in MOS-LQOsw. See P.863 for more information.

1)	The test signals used are the English test sequences as specified in [ITU-T P.501] (2 male speakers, 2 female speakers, two sentences each). The test signal level is the nominal signal level, measured at thetest point (S3), the test signal level is measured as "active speech level" according to [ITU-T P.56]. The speech activity should be between 30% and 70%.

The original speech signal is used as the reference signal for the determination of the speech quality.

2)	The test arrangement is according to clause 9.1. For wideband systems MOS-LQOsw is determined. For narrowband systems MOS-LQOn is determined

The calculation is made using the signal recorded at the test point (xy).

### Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the listening speech quality has to meet the requirements for the applicable Performance classes as given in the table below:

Table xx –Limits for the unidirectional transport listening speech quality

| Performance Class | MOS-LQOn | MOS-LQOsw |
| --- | --- | --- |
| 1 | > 4.2 | > 4.5 |
| 2 | > 3.8 | > 4.1 |
| 3 | > 3.4 | > 3.7 |
| 4 | ≤ 3.4 | ≤ 3.7 |

### Design guidance and root-cause analysis

No additional impairment on listening speech quality should be introduced by the signal transport. Any degradation of listening speech quality will deteriorate the overall listening speech quality and should be avoided. Therefore the listening speech quality under quiet conditions is checked.

## Unidirectional transport speech quality stability

### Parameter description

The uidirectional transport listening speech quality stability LQSUD is measured from the test point (xx) to the test point (xy).

This test is intended to detect speech quality instabilities due to unreliable connection (speech frame loss....). This test should be repeated in different representative test configurations, e.g. positions of the phone within the vehicle, different SRW links active and different WIFI links active.

### Test

Several measurements of MOS-LQOsw (for wideband systems) and MOS-LQOn (for narrowband systems)  according to Section xx are performed in series within the same call or for different calls, within the test arrangement defined below. Typically, for the same call, a measurement each 20 s issufficient. The results are reported in terms of statistics.

The procedure is identical for narrowband and wideband systems. In narrowband systems MOS-LQOn ist used, in wideband systems MOS-LQOsw.

The assessment of Listening Speech Quality Stability is preformed in 5 steps:

- To measure the MOS-LQO periodically over the duration of one communication. N measurements provide N MOS-LQO values.

- For each MOS-LQO (i) value i from 2 to N, MOS-LQO_GAP(i) is calculated as the absolute difference with the previous value  MOS-LQO (i-1):

MOS-LQO_GAP (i) =| MOS-LQO (i)- MOS-LQO (i-1)|

- In order to take into account the subjective perception and measurement accuracy, MOS-LQO_GAP (i) is set to 0 when the difference is equal or lower to THRESHOLD1:

- if MOS-LQO_GAP (i)> (2*THRESHOLD1), then MOS-LQO_GAP (i)= MOS-LQO_GAP (i)

- if THRESHOLD1 < MOS-LQO_GAP (i) ≦ 2*THRESHOLD1, then MOS-LQO_GAP (i)= [MOS-LQO_GAP (i) * 2] – (2*THRESHOLD1)

- if MOS-LQO_GAP (i)≦ THRESHOLD1, then MOS-LQO_GAP (i)=0

- The instability (INS_MOS-LQO) associated to the MOS-LQO over the whole N measurements is defined by mean value of MOS-LQO_GAP (i).

INS_MOS-LQO=1/(N-1)Σ MOS-LQO_GAP (i) with  i=[2:N]

- A linear weighting function is applied in order to express Stability ST-MOS-LQO on a 0 to 100 scale.

This formulation used to determine the Listening Quality Stability (ST-MOS) as

ST-MOS=100-(250* INS_MOS)

and

ST-MOS= 0 if [100-(250* INS_MOS)] < 0

When ST-MOS is calculated within a single call, the call should be longer than 3 minutes (Recommended duration being between 3 and 5 minutes). The duration of each measurement depends on the length of the speech samples used for the test as described in xx

(As an example, if a sample is 15s length and the analysis is done every 20 s a minimum of 10 values will be measured).

### Performance level classification based on values of this parameter

For the stability indicator about Listening Speech Quality, THRESHOLD1 = 0.1 and the linear weighting function applies in order to express Stability (ST-MOS) on a 0 to 100 scale. By definition Stability equals 100 when no variations occur and Stability ST-MOS equals 0 when MOS-LQO variation is equal or more than 0.4.

To claim compliance with a certain Performance class the listening speech quality stability has to meet the requirements for the applicable Performance classes as given in the table below:

Table xx –Limits for the unidirectional transport listening speech quality stability

| Performance Class | ST-MOSn | ST-MOSsw |
| --- | --- | --- |
| 1 | > 95 | > 95 |
| 2 | > 80 | > 80 |
| 3 | > 50 | > 50 |
| 4 | ≤ 50 | ≤ 50 |

### Design guidance and root-cause analysis

Listening quality stability during a call (if the position or transmission characteristics change during the call (or during several different calls) takes into account degradations generated on the signal by the transmission link impairment and the phone position.

In case of systems using SRW transmission it is the purpose to verify the integration of SRW radio network by evaluating change of speech quality over time. This will help in detect problems with RF coverage inside the car cabin and verify the error concealment (packet loss, bit errors) caused by weak RF link or interference with other radio.

A guidance to proceed is

1 - Check RF coverage from SRW unit to possible mobile positions (Protocol analyzer)

2 – Identify weak and bad reception areas inside the vehicle. (Protocol analyzer)

Check bit error rate, rate of packet loss …

3 – Use the speech quality measurement to rate the quality of error concealment in the weak areas identified in step 2 above and compare with measurements from areas with good coverage.

4 – Identify possible issues from interference with parallel SRW links or other networks. This will check how the SRW can handle interference and change to undisturbed channels. Speech quality measurements can be used to see the performance of handling these problems.

It’s recommended to use to a known link as reference.

- Speech Recognition

- Prompts

- Appendix I-Design Guidance and Root-Cause Analysis

## System with integrated microphone and speaker

The system is tested as described in ITU-T Rec. P.1100 (for narrowband systems) or in ITU-T Rec. P.1110 (for wideband systems). If no network access is provided by the system the system is connected to a reference interface replacing the network access system. The requirements as given in ITU-T Recs. P.1100 and P.1110 apply.

Annex A

Microphone measurements in anechoic conditions

The scope of these measurements is the verification of microphone parameters in a defined acoustic environment. The influence of integration such as mounting, orientation and in car acoustics is quite important for the overall performance and therefore it is recommended to integrate the microphone in a surrogate environment as close as possible to the production environment. In case wind noise may have a major influence on the microphone performance it is recommended to also simulate the wind noise in the anechoic environment.

# Microphone sensitivity (in anechoic conditions)

## Parameter description

Microphone sensitivity has to be measured in the free sound field. The sensitivity refers to the sound pressure of the undisturbed free sound field (in the absence of the microphone). The sensitivity is measured at the output of the test circuit according to Figure 7-2.

Microphone sensitivity at 1 kHz shall be measured in the direction of its maximum sensitivity.

## Test

1)	The test signal is a sine wave of 1 kHz at a level of 0 dBPa at the microphone position in the undisturbed free sound field.

2)	The microphone is positioned in a distance of 1 m in the acoustic centre line of the loudspeaker.

3)	The microphone is oriented toward the loudspeaker with its direction of maximum sensitivity.

4)	The sensitivity is determined in mV/Pa.

Further information can be found in [b-IEC 60268-4].

## Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the Microphone Sensitivity has to meet the requirements for the applicable Performance classes for the Microphone Sensitivity as defined in table xx:

Table xx –Limits for the Microphone Sensitivity (in anechoic conditions)

| Performance Class | Sensitivity |
| --- | --- |
| 1 | 300 mV/Pa +/- 3 dB |
| 2 | TBD |
| 3 | TBD |
| 4 | NA |

## Design guidance and root-cause analysis

With a nominal sensitivity of 300 mV/Pa there is a good balance between detecting speech accurately without causing clipping due to high audio level in the car (providing a Voltage supply of 8V).

# Microphone frequency response (in anechoic conditions)

        - Send Frequency Response (SFR)

## Parameter description

The microphone frequency response has to be measured in the free sound field. The frequency response refers to the sound pressure of the undisturbed free sound field (in the absence of the microphone). The frequency response is measured at the output of the test circuit according to Figure 7-2.

## Test

1)	The test signals are sine waves at a level of 0 dBPa at the microphone position in the undisturbed free sound field covering at least the defined frequency range.

2)	The microphone is positioned in a distance of 1 m in the acoustic centre line of the loudspeaker.

3)	The microphone is oriented toward the loudspeaker with its direction of maximum sensitivity.

4)	The sensitivity for each frequency is determined in mV/Pa.

Further information can be found in [b-IEC 60268-4].

## Performance level classification based on values of this parameter

| To claim compliance with a certain Performance class the frequency response has to meet the requirements for the applicable Performance classes for the microphone send sensitivity as defined in the tables below: The masks are drawn by straight lines between the breaking points in Table 9-xx to 9-xx on a logarithmic (frequency) – linear (dB sensitivity) scale. Table xx – Tolerance mask for the microphone send sensitivity frequency response for Performance class 1 |  |  |
| --- | --- | --- |
| Frequency (Hz) | Upper limit | Lower limit |
| 100 | N |  |
| 125 | 0 |  |
| 200 | 0 | –14 |
| 315 | 0 | –13 |
| 400 | 0 | –12 |
| 500 | 0 | –11 |
| 630 | 0 | –10 |
| 1 000 | 0 | –8 |
| 1 300 | 2 | –8 |
| 1 600 | 3 | –8 |
| 2 000 | 4 | –8 |
| 3 100 | 4 | –8 |
| 4 000 | 4 | –8 |
| 8 000 | 4 |  |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

| Table xx – Tolerance mask for the microphone send sensitivity frequency response for Performance class 2 |  |  |
| --- | --- | --- |
| Frequency (Hz) | Upper limit | Lower limit |
| 100 |  |  |
| 125 |  |  |
| 200 |  |  |
| 315 |  |  |
| 400 |  |  |
| 500 |  |  |
| 630 |  |  |
| 1 000 |  |  |
| 1 300 |  |  |
| 1 600 |  |  |
| 2 000 |  |  |
| 3 100 |  |  |
| 4 000 |  |  |
| 8 000 |  |  |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

| Table xx – Tolerance mask for the microphone send sensitivity frequency response for Performance class 3 |  |  |
| --- | --- | --- |
| Frequency (Hz) | Upper limit | Lower limit |
| 100 |  |  |
| 125 |  |  |
| 200 |  |  |
| 315 |  |  |
| 400 |  |  |
| 500 |  |  |
| 630 |  |  |
| 1 000 |  |  |
| 1 300 |  |  |
| 1 600 |  |  |
| 2 000 |  |  |
| 3 100 |  |  |
| 4 000 |  |  |
| 8 000 |  |  |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

| Table xx – Tolerance mask for the microphone send sensitivity frequency response for Performance class 4 |  |  |
| --- | --- | --- |
| Frequency (Hz) | Upper limit | Lower limit |
| 100 | NA | NA |
| 125 | NA | NA |
| 200 | NA | NA |
| 315 | NA | NA |
| 400 | NA | NA |
| 500 | NA | NA |
| 630 | NA | NA |
| 1 000 | NA | NA |
| 1 300 | NA | NA |
| 1 600 | NA | NA |
| 2 000 | NA | NA |
| 3 100 | NA | NA |
| 4 000 | NA | NA |
| 8 000 | NA | NA |
| NOTE – All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

## Design guidance and root-cause analysis

NOTE 1 – Depending on customer demands, other tolerance schemes than those described in Table 10-1. may be applied and have to be defined in an equivalent format.

NOTE 2 – Ideally the response characteristics of the microphone should be flat in the frequency range of wideband transmission (100 Hz – 7 kHz). However, especially in the presence of background noise, a bandwidth limitation may be desirable. No explicit recommendation can be given here since such limitation would depend on level and spectral content of the background noise and ideally should be adaptive. If, however, a bandwidth limitation is introduced, it should be made at both, the high and low frequencies.

NOTE 3 – Table 10-1 applies wider tolerances than Table 10-2 to achieve the freedom of adapting the microphones frequency response to the needs in the car.

# Microphone directional characteristics (in anechoic conditions)

## Parameter description

The directional characteristic of a microphone is described by different sensitivities at different angels of sound incidence.

The front to back ratio is the ratio between the sensitivity in direction of highest sensitivity and the sensitivity at the angle of lowest sensitivity expressed in dB at 1 kHz. The front to back ratio is measured at the output of the test circuit according to Figure 7-2.

## Test

1)	The test signal is a sine wave of 1 kHz at a level of 0 dBPa at the microphone position in the undisturbed free sound field.

2)	The microphone is positioned in a distance of 1 m in the acoustic centre line of the loudspeaker.

3)	The first measurement is done with the microphone oriented to the loudspeaker with its direction of maximum sensitivity. The second measurement is done with the microphone oriented toward the loudspeaker with its direction of minimum sensitivity. If the direction of minimum sensitivity is not known, it has to be determined by rotating the microphone until the minimum is found.

4)	The front to back ratio is determined in dB.

Further information can be found in [b-IEC 60268-4].

## Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the Microphone directionality has to meet the requirements for the applicable Performance classes for the Microphone directional characteristics  as defined in table xx:

Table xx –Limits for the Microphone directional characteristics

| Performance Class | Front to Back Ratio |
| --- | --- |
| 1 | >= 10 dB |
| 2 | TBD |
| 3 | TBD |
| 4 | < TBD |

## Design guidance and root-cause analysis

To achieve appropriate noise reduction a high front to back ratio is desired.

NOTE – Depending on mounting and orientation, also lower front to back ratios can be an advantage.

# Microphone distortion (in anechoic conditions)

        - Send Distortion + Noise (SDN)

## Parameter description

The microphone distortion refers to the sound pressure of the undisturbed free field. The distortion is measured at the output of the test circuit according to Figure 7-2.

The harmonic distortion with a sound pressure level of 0 dBPa (94 dBSPL) at the position of the microphone shall be measured in the narrow-band frequency range.

## Test

1)	The test signal is a sine wave with a frequency of 300 Hz, 500 Hz, 1 kHz, 2 kHz at a level of 0 dBPa.

2)	The microphone is positioned in an acoustic centre line of the loudspeaker.

3)	The microphone is oriented toward the loudspeaker with its direction of maximum sensitivity.

4)	The total harmonic distortion is expressed in %.

Care has to be taken that the loudspeaker is able to produce the required sound pressure level with a lower distortion than the microphone under test.

Further information can be found in [b-IEC 60268-4].

## Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the Microphone distortion has to meet the requirements for the applicable Performance classes for the Microphone distortion as defined in table xx:

Table xx –Limits for the Microphone distortion

| Performance Class | SDN |
| --- | --- |
| 1 | < 1% |
| 2 | TBD |
| 3 | TBD |
| 4 | > TBD |

## Design guidance and root-cause analysis

TBD

# Microphone maximum sound pressure level (in anechoic conditions)

## Parameter description

The maximum sound pressure is defined by the sound pressure level where the total harmonic distortion of the microphone at 1 kHz is 3% in the narrow-band frequency range. The total harmonic distortion is measured at the output of the test circuit according to Figure 7-2.

## Test

1)	The test signal is a sine wave with a frequency of 1 kHz and an increasing level to determine the level of 3% total harmonic distortion.

2)	The microphone is positioned in an acoustic centre line of the loudspeaker.

3)	The microphone is oriented toward the loudspeaker with its direction of maximum sensitivity.

4)	The maximum sound pressure level is expressed in dBSPL or dBPa.

Care has to be taken that the loudspeaker is able to produce the required sound pressure level with a lower distortion as the microphone under test.

## Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the Microphone sound pressure level has to meet the requirements for the applicable Performance classes for the Microphone maximum sound pressure level as defined in table xx:

Table xx –Limits for the Microphone maximum sound pressure level

| Performance Class |  |
| --- | --- |
| 1 | >= 106 dB SPL |
| 2 |  |
| 3 |  |
| 4 |  |

## Design guidance and root-cause analysis

The maximum sound pressure level should be higher than 106 dBSPL for a microphone with a typical sensitivity of 300 mV/Pa.

NOTE – With a good microphone design, the maximum sound pressure level is electrically limited by the supply circuit referred to in Figure 7-2. A microphone with higher sensitivity will reach the electrical output limits at a lower sound pressure level compared to another microphone with lower sensitivity.

# Microphone self-noise (in anechoic conditions)

## Parameter description

The maximum self-noise shall be measured at the output of the test circuit according to Figure 7-2 in quiet conditions.

## Test

1)	For the measurement, no test signal is used.

2)	The microphone has to be powered with a low noise voltage supply.

3)	The self-noise is measured at the output of the test circuit according to Figure 7-2 in the frequency range between 100 Hz and 8 kHz, A-weighting has to applied.

4)	The self-noise is expressed in dBV(A).

Care has to be taken that the environmental noise is below the equivalent self-noise of the microphone.

## Performance level classification based on values of this parameter

To claim compliance with a certain Performance class the TBD  has to meet the requirements for the applicable Performance classes for the Microphone self-noise as defined in table xx:

Table xx –Limits for the Microphone self-noise

| Performance Class |  |
| --- | --- |
| 1 | < 72 dBV(A). |
| 2 | TBD |
| 3 | TBD |
| 4 | > TBD |

## Design guidance and root-cause analysis

TBD

Annex D

Standard set of user scenarios.

Table D.1: Standard set of user scenarios used to collect noise recordings.

|  |  | Vehicle settings |  |  |  |  |  | Environmental conditions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| User scenario | Description | Vehicle speed | HVAC settings | Windows | Wipers | Turn signal | Back-ground talkers | Road surface (see Note 3) | Wind speed | Precipitation | Temp. |
| 1 | Stationary vehicle with low HVAC noise | 0 km/h (at idle) | FAN=Lowest setting | Up | Off | Off | None | N/A | < 5 m/s (12mph) | None | > -20C and < 40C |
| 2 | City driving with high HVAC noise | 60 km/h (37mph) | FAN= Setting closest to 6 dB(A) above driving noise with FAN =lowest setting; AIRFLOW=Directed to windows | Up | Off | Off | None | Dry; Rough road | < 5 m/s (12mph) | None | > -20C and < 40C |
| 3 | Highway driving with low HVAC noise | 120 km/h (75mph) | FAN=Lowest setting | Up | Off | Off | None | Dry; Rough road | < 5 m/s (12mph) | None | > -20C and < 40C |
| 4 | Highway driving with high HVAC noise | 120 km/h (75mph) | FAN=Setting closest to 6 dB(A) above driving noise as in condition 3; AIRFLOW=Directed to windows | Up | Off | Off | None | Dry; Rough road | < 5 m/s (12mph) | None | > -20C and < 40C |
| 5 see note 1 | Highway driving with high HVAC noise | ≥160 km/h (≥100 mph) | FAN=Lowest Setting AIRFLOW=Directed to windows | Up | Off | Off | None | Dry; Rough road | < 5 m/s (12mph) | None | > -20C and < 40C |
| 6 see note 2 | Stationary vehicle with high HVAC noise | 0 km/h (at idle) | FAN=Highest setting, AIRFLOW= Directed to microphone if possible | Up | Off | Off | None | N/A | < 5 m/s (12mph) | None | > -20C and < 40C |
| 7 see note 3 | Highway driving with low HVAC noise; windows down and open roof (if possible) | 120 km/h (75mph) | FAN=Lowest settings | Up | Off | Off | None | Dry; Rough road | < 5 m/s (12mph) | None | > -20C and < 40C |

Note 1: Optional test: If the hands-free system is to be deployed in countries where higher driving speeds are allowed, then testing at 160 k.p.h. or more should be performed in addition to the standard set of user scenarios.

Note 2: Required test if the airflow can be directed towards the microphone. (This may result in wind buffeting.)

Note 3: Optional test: This test is applicable if the hands-free system is required to work with windows down, roof open or convertibles.

Note 4: Smooth road surfaces that generate very little tire noise shall not be used. Also, road surfaces with bumps that cause significant impulse noises shall not be used either. If available, concrete surfaces are preferred because they often result in worst case conditions that cause impairments not seen on other road surfaces.

Note 5: AIRFLOW refers to the HVAC mode settings related to how air is directed into the cabin. For example, in North American vehicles there is typically a “Defrost” setting that will direct the flow of air onto the windows.

Annex E

Testing for Noise Distortion

This Annex describes a test using an objective noise distortion measure specifically designed to quantify the amount of musical tones. The system under test is the noise reduction functionality in the signal enhancement subsystem as described in section 8.3. If comfort noise generation is part of the system, it should be switched off. The purpose of the measure described in this Annex is to help to optimize the signal enhancement subsystem during the development and optimization process. The described test is recommended, although not mandatory.

  - Parameter description

The noise distortion is measured in send direction with the weighted log-average kurtosis ratio WLAKR of the signals at test point (S2) and at test point (S3) of the signal enhancement subsystem as shown in Fig. <SignalEnhancementSubsystem_with_TP>. The weighted log-average kurtosis ratio WLAKR describes the amount of musical tones by measuring the average change of the weighted kurtosis of the unprocessed background noise signals and the processed background noise signals using the signal enhancement subsystem.

  - Test

In principle, the test could be carried out using noise signals acquired by the hands-free microphone in the car. A number of not less than 18 noise samples of length 8 seconds taken from typical stationary driving conditions should then be recorded and taken for the subsequent measurement. An even more relevant problem to solve may be to allow comparison to tests carried out for other signal enhancement subsystems in other labs. Therefore, as the recommended alternative, the noise data shall be taken from the inside-car background noise signals in section 8.2 (“Stereophonic Signals”) of [ETSI EG 202 396-1]. The left channel of all 9 signals including fullsize car1, midsize car1 and midsize car2 with 80 km/h, 100 km/h and 130 km/h are selected, respectively. Each signal is downsampled from 48 kHz to 16 kHz, the first 16 seconds are taken and segmented into 2 noise signals with a length of 8 seconds each. This leads to altogether 18 reference background noise signals. For the test of a wideband subsystem the ITU-T P.341 weighting filter [ITU-T P.341] for wideband signals (50 … 7000 Hz) is applied. For the test of a narrowband subsystem the modified IRS (MIRS) weighting filter according to ITU-T Recommendation P.830 [ITU-T P.830] (300 … 3400 Hz) is applied and subsequently downsampled to 8 kHz. The mean weighted log-average kurtosis ratio WLAKR is then computed from all 18 weighted log-average kurtosis ratios. For each of the 18 background noise signals, one weighted log-average kurtosis ratio WLAKR is calculated as follows:

- The long-term rms level of the 16 kHz/8 kHz sampled background noise signal of length 8 seconds is adjusted to -26 dBov according to ITU-T Recommendation P.56 [13]. Then the signal is digitally stored as the reference signal, and fed into test point (S2) as an input to the signal enhancement subsystem.

- The background noise signal processed by the signal enhancement subsystem is taken from test point (S3) and also digitally stored.

- Now the signals are transformed into the Fourier domain. To accomplish this, a square root Hann window of length 512 (or 256 for narrowband signals) is applied, followed by a discrete Fourier transform (DFT) of the same length and a frame shift of 50%. These operations are separately performed for the stored reference signal and the processed noise signal with  being the time index, respectively.

- Based on the DFT coefficients of the reference signal and the processed background noise signal, the weighted log-average kurtosis ratio WLAKR = is computed, where is the natural logarithm, and are the average weighted kurtosis of the reference signal and the processed noise signal, respectively. is calculated by averaging the instantaneous kurtosis of the weighted squared amplitude reference signal DFT coefficientsover all frames

(1)

with  and  being the frame index and frequency bin, respectively, and K being the DFT length. The weighting factor  is being calculated for each frequency bin as the inverse mean value of  across all L frames. The average weighted kurtosis  can be calculated in the same way by using  and  instead of  and  in equation (1), respectively. The noise distortion, i.e., the amount of musical tones produced by the signal enhancement subsystem, shall be evaluated in terms of the mean of the weighted log-average kurtosis ratio WLAKR. More details are described in [YU-KURTOSIS-WB] and [YU-KURTOSIS-NB].

  - Classification of QoS level based on values of this parameter

To claim compliance with a certain QoS class the WLAKR of the signal enhancement subsystem in send direction has to meet the requirement for the applicable QoS classes as defined in Table YYY for wideband subsystems, and in Table ZZZ for narrowband subsystems. The audibility of musical tones is rated with seven categories on a mean opinion score (MOS) scale: (1) intolerably audible, (2) loudly audible, (3) rather loudly audible, (4) moderately audible, (5) slightly audible, (6) just audible, and (7) inaudible.

Table YYY –Limits for the mean of the weighted log-average kurtosis ratio WLAKR of the signal enhancement subsystem in send direction (wideband)

| Performance Class | WLAKR | Interpretation |  |
| --- | --- | --- | --- |
|  |  | Mean Opinion Score (MOS) | Audibility of Musical Tones |
| 1 | WLAKR<0.72 | 5.5<MOS | Just audible or inaudible |
| 2 | 0.72≤WLAKR<1.16 | 4.5<MOS≤5.5 | Slightly audible |
| 3 | 1.16≤WLAKR<1.56 | 3.5<MOS≤4.5 | Moderately audible |
| 4 | 1.56≤WLAKR | MOS≤3.5 | Intolerably audible or loudly audible or rather loudly audible |

Table ZZZ –Limits for the mean of the weighted log-average kurtosis ratio WLAKR of the signal enhancement subsystem in send direction (narrowband)

| Performance Class | WLAKR | Interpretation |  |
| --- | --- | --- | --- |
|  |  | Mean Opinion Score (MOS) | Audibility of Musical Tones |
| 1 | WLAKR<0.30 | 5.5<MOS | Just audible or inaudible |
| 2 | 0.30≤WLAKR<0.67 | 4.5<MOS≤5.5 | Slightly audible |
| 3 | 0.67≤WLAKR<1.00 | 3.5<MOS≤4.5 | Moderately audible |
| 4 | 1.00≤WLAKR | MOS≤3.5 | Intolerably audible or loudly audible or rather loudly audible |

  - Design guidance and root-cause analysis

Noise distortion introduced by the signal enhancement subsystem may lead to a high degradation of the overall perceived speech quality. Therefore, the described noise distortion measure should be evaluated during the optimization of the signal enhancement subsystem, and judged along with speech signal-related quality measures. A high noise distortion may indicate a too low spectral gain floor in spectral noise reduction, or a too low noise overestimation factor in spectral noise power estimation, or a too low SNR floor and/or a too weak temporal smoothing in SNR estimation.

References:

[ETSI EG 202 396-1]: Speech Processing, Transmission and Quality Aspects (STQ), Speech Quality Performance in the Presence of Background Noise; Part 1: Background Noise Simulation technique and Background Noise Database, ETSI, 2008

[ITU-T P.341]: ITU-T Recommendation P.341, Characteristics of Wideband Terminals, ITU, Geneva, 1994.

[ITU-T P.830] ITU-T Recommendation P.830, Subjective Performance Assessment of Telephone-band and Wideband Digital Codecs, Annex D, ITU, Geneva, 1996

[YU-KURTOSIS-WB]: H. Yu and T. Fingscheidt, A Weighted Log Kurtosis Ratio Measure for Instrumental Musical Tones Assessment in Wideband Speech, 10th ITG Fachtagung Sprachkommunikation, accepted for publication, Braunschweig, Germany, Sep. 2012

[YU-KURTOSIS-NB]: H. Yu and T. Fingscheidt, Instrumental Musical Tones Measurement of Arbitrary Noise Reduction Systems, in Proc. of DAGA’12, pp. 255-256, Darmstadt, Germany, Mar. 2012

Annex F

Reference-free SNR Measurement

# Introduction

This annex describes an approach to measure the SNR of a speech signal distorted by car noise as close as possible to the reference SNR from ITU-T Recommendation P.56 [2], but without using any reference signals based on [1]. This “reference-free” nature allows for a wide flexibility; the proposed method just has to be applied to the noisy speech signal. Moreover, it offers low complexity, and can be applied both to narrowband and wideband signals.

To measure the SNR of the noisy speech signal in this approach, first the speech and noise power have to be estimated. Because of the additive nature of the noise, the speech power is approximated by the difference between the noisy speech power and the noise power. The noise power is estimated by a noise variance tracking algorithm. To estimate the P.56 active speech level only speech active frames are taken for the speech power estimation. Therefore, a voice activity detection (VAD) is needed. The power of the noise component is estimated by means of the noise tracking algorithm and—in order to ensure a robust estimation—a speech pause detection (SPD). Then, the measured SNR is the ratio of the speech and noise power estimates. Since the effectiveness of the employed algorithms (such as noise variance tracking, VAD, and SPD) decreases at low reference SNR values, the raw SNR measurement leads to systematical measurement errors in this region. Therefore, the resulting raw SNR values will be corrected by a mapping curve.

# Signal-to-Noise Ratio Measurement

The input signal  of the measurement system is assumed to consist of the clean speech signal  and an additive noise signal , with n being the discrete time index. After segmentation, windowing, and the discrete Fourier transform (DFT), the input signal can be rewritten as  with , , , as well as , and  being the short-time spectra of the noisy speech, the clean speech, the noise component, as well as the analysis frame index, and the frequency bin index, respectively. We define the signal-to-noise ratio (SNR) as	
		(1)

with , ,  being the logarithm to the base 10, the speech and the noise power, respectively.

Since  and  are not accessible in practice, they have to be estimated. Exploiting the additive nature of the noise, the numerator is estimated with contributions , where  and  are the power spectral density (PSD) of the noisy speech signal and the estimated noise variance (see Section 3), respectively. The maximum operator ensures a non-negative speech power estimate. Conforming to the definition of the active speech level [2], for the estimation of  in (1) only speech active frames are taken as

(2)

with , , and  being the set of active speech frames detected by a VAD as a part of the noise tracking algorithm (see Section 3), the number of elements in , and the number of all frequency bins, respectively.

The precise estimation of the noise power  is essential for the SNR estimation. However, the accuracy of the noise variance tracking algorithm is affected especially at low SNRs. In order to increase the robustness of the approach, for the estimation of  in (1), the estimated noise variance  is averaged over all frequency bins  and those frames  belonging to speech pause	
	
		(3)

with  and  being the set of speech pause frames detected by a separate conservative SPD described in Section 4, and the number of elements in , respectively.

# Noise Variance Tracking and Voice Activity Detection

The estimation of the noise variance  in (2) is performed utilizing a 3–state classifier specific to this estimator. Based on the smoothed periodogram of the noisy speech signal

(4)

and a dynamic threshold , one hypothesis  out of three is chosen:

:	Speech presence is assumed if

:	Speech absence is assumed if .

:	Speech transition is assumed if .

The noise variance estimate  is updated as

(5)

where the initial value is  and the time-varying smoothing factor  depends on the hypothesis of the current frame :

(6)

The update of the dynamic threshold  is performed as follows

(7)

where the multiplication by the step-size constant =1.07 for narrowband signals (=1.2 for wideband signals) slowly but steadily decreases the probability of selecting the speech presence hypothesis. The unsmoothed update to the last periodogram value  allows for rapid threshold adaptation in speech pauses. The control parameter is initialized as .

The frame-wise VAD is based on the hypotheses of the noise variance tracking algorithm as introduced above. Therefore, it was integrated into the noise variance tracking algorithm. Frame  is detected as voice active and becomes element of set , if at least 90 % of its frequency bins from the range [500 Hz 2500 Hz] containing relevant speech information are classified as speech active  or transient  by the 3–state classifier.

# Speech Pause Detection

This section describes the algorithm which is employed to detect frames with speech pause . Please note that different to the VAD, this is a separate algorithm which is independent from the noise power tracking. The SPD is based on the frame energy of the noisy speech spectrum calculated by means of (4) as follows

(8)

with  and  being the set of frequency bins between 500 Hz and 2500 Hz and the number of elements in this set, respectively. This frequency range excludes low frequencies where most of the car noise power is concentrated. Based on an adaptive threshold, the SPD delivers a hypothesis  based on the following 3 states:

:	Speech presence is assumed if

:	Speech transition state follows every  decision with a duration of  frames, unless threshold  has been exceeded again.

:	Speech pause is assumed in all other situations.

Frame  is considered as speech absent and becomes element of , if . The adaptive threshold  is calculated as

,	(9)

with the additive term . The SPD floor signal  is updated recursively as follows (cf. (5))

,	(10)

where the initial value is  and the time-varying smoothing factor  is defined as follows (cf. (6))

(11)

with  ensuring a more conservative floor signal update in speech pauses than just conditioning on .

The SPD control parameter  is increased slightly in speech presence and updated strongly during speech absence as (cf. (7))

(12)

with the control update constant . The control parameter is initialized as .

The SPD parameters which differ for the narrowband and the wideband implementation are summarized in Tab 1.

| Parameter |  |  |  |
| --- | --- | --- | --- |
| kHz | 6 |  | 1.085 |
| kHz | 7 |  | 1.055 |

Tab 1 Parameters of the SPD

# Measurement Setup

At a sampling frequency of 8 kHz (16 kHz), the noisy speech signal   has to be segmented by a Hann window, an analysis frame length of 256 (512) samples, and an analysis frame shift of 50% (50%) to obtain . The raw SNR value can be calculated as follows

(13)

using (2), the noise variance tracker, and the VAD as described in Section 3, as well as (3), the noise variance tracker from Section 2, and the SPD as introduced in Section 4. This raw SNR value has to be corrected by the following mapping function in order to obtain an unbiased SNR measurement

,	(14)

with the scaling parameters  and , as well as the polynomial coefficients  as shown in Tab 2.

| 8 kHz | 16.043 | 11.252 | -2.1312 | 6.4129 | 2.0957 | -19.199 | 5.0992 | 19.709 | -7.7268 | -6.6348 | 2.0857 | 13.066 | 11.555 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 kHz | 15.461 | 11.798 | -0.8082 | 2.8537 | -0.3609 | -7.5337 | 4.3304 | 7.2828 | -5.0623 | -1.8734 | 0.8842 | 13.06 | 11.48 |

Tab 2 Scaling parameters  and  as well as the polynomial coefficients  of the mapping function

The approach is summarized by means of a pseudocode in Tab 3.

... % parameter initialization for signal segmentation, FFT, VAD, SPD, etc. for l=1:L, % main loop over all frames ... % signal analysis: segmentation, windowing, FFT, absolute square (4), (7), (6), (5) % 3-state noise variance tracking, VAD (4), (8), (9), (12), (11), (10) % SPD end (2) % speech power estimation (3) % noise power estimation (13) % raw SNR (14) % corrected SNR

Tab 3 Overview of the approach by means of pseudocode

# References

[1]	Fodor, B.; Fingscheidt, T., “Reference-free SNR Measurement for Narrowband and Wideband Speech Signals in Car Noise”, In Proc. 10th ITG Conference on Speech Communication, Braunschweig, Germany, Sep. 26-28, 2012, available online at IEEE Xplore

[2]	ITU-T, “Objective Measurement of Active Speech Level”, ITU-T Recommendation P.56,        Mar. 1993

Annex G

Frame process and Delay

## General

This Annex provides definition and handling of delay for frame process subsystems.

Typical speech enhancement subsystems have spectral analysis/synthesis processes to realize good noise reduction or echo cancellation. Time domain input signal in a fixed length of window is applied, analyzed, enhanced, then synthesized by overlap-add method. The windowing period is often called “frame shift”.

Typical data transport subsystems also take frame processes for efficient data compression or error correction. A unit of the data transferred is called a “packet”.

When two subsystems, sender and receiver, having different frame data interval interfaces to each other, a special data buffer must be inserted to arbitrate the difference of intervals. Several definitions about delay and framing process should be given to such processing. This Annex provides the definitions for such parameters.

## Frame interval

Frame interval is defined as a logical time interval of input or output frame data. Frame interval of time domain data is regarded as 1 sample period. Subsystem having framed input or output signals shall specify the frame interval.

Frame rate is the reciprocal of frame interval.

## Delay

Subsystem having framed input or output signals may have two types of delays, buffer delay and input/output delay. If the subsystem is composed of software, algorithmic delay is defined, and computational delay is also defined after the software implementation. Table X.1 provides the definition of these delays. A subsystem supplier shall specify the correspondent delay time.

Table X.1 – Definition of delays

| Category |  | Definition |
| --- | --- | --- |
| Buffer delay |  | Additional delay time when a buffer is introduced at interface to arbitrate frame size difference. |
| Input/output delay |  | Subsystem’s physical delay time from data input to output. |
| Software | Algorithmic delay | Subsystem’s logical delay time in case the subsystem is realized by software. |
|  | Computational delay | Additional computational delay time from data input to output caused by a processor. It is applied in case the software subsystem is implemented onto the processor. |

Figure X.1 shows a system integrating two framed software subsystems on a processor. Input buffer A is placed before subsystem A to convert input sampled data to framed data. If Subsystems A and B has the same frame interval, a buffer between these subsystems should not be necessary. After subsystem B, the output framed data is converted to sampled data by output buffer B which can be regarded as no delay.

The overall delay of this system is calculated by summing buffer delay A, algorithmic delay A, algorithmic delay B and computational delay.

Overall system delay =

[Processor]

[Software]

Subsystem A

[Software]

Subsystem B

Buffer

delay A

No buffer here

Sampled data

Framed

data

No buffer

delay

Input buffer A

Algorithmic

delay A

Output buffer B

Algorithmic

delay B

Framed

data

Framed

data

Sampled data

Computational

delay

(Buffer delay A) + (Algorithmic delay A) + (Algorithmic delay B) + (Computational delay)

Figure X.1: Framing and Delays

## Arbitration between two subsystems having different frame intervals

When a receiver subsystem interfaces with sender subsystem having different frame interval, a data buffer shall be shall be categorized into following cases.

## Sender’s and receiver’s frame intervals are identical

No additional delay is to be inserted.

## Sender’s frame interval is a multiple of receiver’s frame interval

Sender:

Receiver:

Buffer delay = 15 ms

Buffer delay = 0 ms

Total delay = 15 ms

0 .......... 5 .......... 10 ..........14ms

0 ... 4ms

5 ... 9ms

15 ... 19ms

10 ... 14ms

15 ........ 20 ........ 25 ........ 29ms

30 ........ 35 ....

Sender’s input buffer:

Receiver’s output buffer:

0 ... 14ms

15 ... 29ms

0 ... 4ms

5 ... 9ms

10...14ms

15...19ms

20...24ms

No delay will be added at the receiver. The total delay equals to the sender’s delay. Figure X.2 shows an example that sender’s frame interval is 15 msec and receiver’s is 5 msec. Algorithmic delay and computational delay are omitted. In Figure X.1, subsystem A can be regarded as the sender, subsystem B as the receiver, buffer A as the sender’s input buffer, and buffer B as the receiver’s output buffer.

Figure X.2: Example of Case 2

## Receiver’s frame interval is a multiple of sender’s frame interval

Buffer delay = 5 ms

Sender:

Receiver:

Buffer delay = 10 ms

Total delay = 15 ms

0  ........  5  ........  10  ........ 14ms

15 .......

0 ... 4ms

5 ... 9ms

15 ... 19ms

20 ... 24ms

25 ... 29ms

30 ... 34ms

10 ... 14ms

Sender’s input buffer:

Receiver’s output buffer:

0 ... 14ms

15 ... 29ms

0 ... 4ms

5 ... 9ms

10...14ms

15...19ms

20...24ms

25...29ms

30...34ms

Receiver’s delay buffer size is equal to its frame interval minus sender’s frame interval. However, the total buffer delay is equal to the receiver’s frame interval. Figure X.3 shows an example that sender’s frame interval is 5 msec and receiver’s frame interval is 15 msec.

Figure X.3: Example of Case 3

## Other cases

In general case, an arbitration buffer is inserted between two subsystems. Receiver has to read the buffer with the delay equal to its frame interval. It coincides with the case that the data is transferred with time domain signal. The resultant buffering delay is equal to the sum of frame intervals of two subsystems. Figure X.4 shows an example that sender’s frame interval is 5 msec and receiver’s is 8 msec.

Sender:

Receiver:

Buffer delay = 8 ms

Total delay = 13 ms

0 ... 4ms

5 ... 9ms

15 ... 19ms

20 ... 24ms

25 ... 29ms

30 ... 34ms

10 ... 14ms

Sender’s input buffer:

Receiver’s output buffer:

0

Buffer delay = 5 ms

0 ... 7ms

8 ... 15ms

16 ... 23ms

Arbitration buffer:

1

2

3

4

-5

-4

-3

-2

-1

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

0 ... 4ms

5 ... 9ms

10...14ms

15...19ms

20...24ms

25...29ms

30...34ms

0 ... 7ms

30

8 ... 15ms

16 ... 23ms

Figure X.4: Example of Case 4

In special cases where two subsystems are operating on asynchronous clocks, a drop/insert buffer with a unit or the multiples of frame size should be used to arbitrate them.

## Example

An example of wideband mode SRW/hands-free system is shown. Examples of frame process are shown in Table X.2.

Table X.2 –Frame intervals of subsystems

| Subsystem | Frame interval |
| --- | --- |
| Signal enhancement | 15 ms (to meet SRW frame interval) |
| SRW | 1.25, 2.5, 3.75 ms (CVSD) for Narrow band 7.5, 15 ms (mSBC) for Wideband |
| Cell phone | 20 ms (Speech codec) |

An example of subsystems composing of an automotive hands-free send system is shown in Figure X.5. The upper boxes specify frame intervals and delays of each subsystem. The total send delay is calculated by summing all delays surrounded by red circles excepted for cell phone factors which are out of send delay specified by P.1100 or P.1110. According to Case 2 in clause X.4, buffer delay at input of SRW is omitted.

Cell phone

Frame interval: 1 sample period

Delay:0.5 ms

Frame interval: 15ms

Delay:25 ms

Frame interval:7.5ms

Delay:15? ms

Processor

MIC

Signal enhancement

SRW

Speech CODEC

Buffer delay: 15 ms

Buffer delay: 20 ms

Acoustic delay: 2 ms

MRP

Frame interval: 20ms

Delay:don’t care

Computational delay: 7.5 ms

2 + 0.5 + 15 + 25 + 7.5 + 15? = 65 msec

Figure X.5: An example of automotive hands-free send system

In the same way, the subsystems composing of the receive system is shown in Figure X.6. The total receive delay is calculated by summing all delays surrounded by red circles excepted for cell phone factors. According to Case 3 in clause X.4, buffer delay at input of SRW is omitted.

15? + 15 +15 + 7.5 + 0.5 + 2 = 55 msec

Cell phone

Frame interval: 1 sample period

Delay:0.5 ms

Frame interval: 15ms

Delay:15 ms

Frame interval:7.5ms

Delay:15? ms

Processor

Audio

Signal enhancement

SRW

Speech CODEC

Buffer delay: 7.5 ms

Frame interval: 15ms

Delay:25 ms

SRW

Speech CODEC

Buffer delay: 15 ms

Computational delay: 7.5 ms

Acoustic delay: 2 ms

Frame interval: 20ms

Delay:don’t care

DRP

Frame interval: 20ms

Delay:don’t care

Buffer delay: 15 ms

Figure X.6: An example of automotive hands-free receive system

In this scenario, the round trip delay is equal to 120 msec, which meets P.1110.

The next examples are the typical cases that the signal enhancement subsystems of automotive hands-free send system and receive system are implemented without special care for delay or realized as an independant chip from SRW and having 16 msec frame interval. Since the interval is not simple multiple of SRW frame interval, those processes are implemented as asynchronous tasks. Figure X.7 shows the send system. The total delay of the send system is calculated as below. According to Case 4 in clause X.4, buffer delay at input of SRW is also added.

Cell phone

Frame interval: 1 sample period

Delay:0.5 ms

Frame interval: 16ms

Delay:26 ms

Frame interval:7.5ms

Delay:15? ms

Processor

MIC

Signal enhancement

SRW

Speech CODEC

Buffer delay: 16 ms

Buffer delay: 20 ms

Computational delay: 7.5 ms

Acoustic delay: 2 ms

MRP

Frame interval: 20ms

Delay:don’t care

Buffer delay: 7.5 ms

Computational delay: 16 ms

2 + 0.5 + 16 + 16 + 26 + 7.5 + 7.5 + 15? = 90.5 msec

Figure X.7: A bad example of automotive hands-free send system

The receive system is shown in Figure X.8. The total receive delay is calculated as below. According to Case 4 in clause X.4, buffer delay at input of SRW is also added.

7.5 + 15? + 7.5 + 16 + 16 + 16 + 0.5 + 2 = 80.5 msec

Cell phone

Frame interval: 1 sample period

Delay:0.5 ms

Frame interval: 16ms

Delay:16 ms

Frame interval:7.5ms

Delay:15? ms

Processor

Audio

Signal enhancement

SRW

Speech CODEC

Buffer delay: 7.5 ms

Frame interval: 15ms

Delay:25 ms

SRW

Speech CODEC

Buffer delay: 15 ms

Computational delay: 16 ms

Acoustic delay: 2 ms

Frame interval: 20ms

Delay:don’t care

DRP

Frame interval: 20ms

Delay:don’t care

Buffer delay: 16 ms

Computational delay: 7.5 ms

Figure X.8: A bad example of automotive hands-free receive system

In this scenario, the round trip delay is equal to 171 msec, which cannot meet P.1110.

_______________

The near field is characterized by the distance (measured from the sound source) where the direct sound and the reflected sound are of equal intensity. In acoustics, this distance is often referred to as critical distance.