| Bureau de la normalisation des télécommunications |  |
| --- | --- |

Genève, le 1er avril 2009

| Réf.: Tél.: Fax: E-mail: | Circulaire TSB 32 COM 12/JKK +41 22 730 5780 +41 22 730 5853 tsbsg12@itu.int | - Aux administrations des Etats Membres de l'Union |
| --- | --- | --- |
|  |  | Copie: - Aux Membres du Secteur UIT-T; - Aux Associés de l'UIT-T; - Aux Président et Vice-Présidents de la Commission d'études 12; - Au Directeur du Bureau de développement des télécommunications; - Au Directeur du Bureau des radiocommunications |

| Objet: | Appel à propositions de la Commission d'études 12 concernant un modèle d'opinion pour les applications de diffusion vidéo et audio en continu (G.OMVAS) |
| --- | --- |
| Suite à donner: Prière de répondre le 31 mai 2009 au plus tard |  |

Madame, Monsieur,

1	La Question 13/12 de la Commission d'études 12 de l'UIT-T vise à normaliser un nouveau modèle d'opinion pour la planification de la qualité des applications de diffusion audio et vidéo en continu, intitulé provisoirement G.OMVAS.

2	L'objectif du projet G.OMVAS et le programme de travail correspondant figurent respectivement dans les Annexes A et B.

3	Ce projet repose essentiellement sur un mécanisme de collaboration entre les auteurs de propositions en vue de la conception d'un modèle. Les auteurs de propositions sont encouragés à offrir des bases de données de formation et validation, qui devraient contenir des conditions expérimentales ainsi que leur qualité subjective, par exemple des notes moyennes d'opinion (MOS).

4	Les organisations qui ont l'intention de soumettre des propositions de modèle sont priées d'en informer le secrétariat de la Commission d'études 12 de l'UIT-T (tsbsg12@itu.int), avec copie aux Corapporteurs pour la Question 13/12 (takahashi.akira@lab.ntt.co.jp; yi.chen@huawei.com), avant le 31 mai 2009.

5	Le domaine d'application de la Recommandation G.OMVAS est le suivant:

La présente Recommandation décrit un modèle de calcul destiné aux applications de diffusion audio et vidéo en continu et de télévision linéaire sur réseaux IP, y compris la TVIP, qui pourra servir d'outil de planification de la qualité d'expérience et de la qualité de service pour analyser les effets conjugués des variations de plusieurs paramètres audio et

vidéo influant sur la qualité d'expérience (QoE). La Recommandation G.1070 vise à être utilisée expressément pour la visiophonie bidirectionnelle en mode duplex intégral, alors que la présente Recommandation fournit un modèle d'opinion destiné aux applications de diffusion audio et vidéo unidirectionnelles en continu, par exemple la TVIP.

Ce modèle peut être utilisé par les responsables de la planification de la qualité d'expérience (QoE) et de la qualité de service (QoS), pour contribuer à garantir que les utilisateurs seront satisfaits de la qualité de service de bout en bout et éviter une conception technique insuffisante. Il intègre des paramètres de qualité des réseaux, des applications et des terminaux très importants pour les responsables de la planification de la qualité d'expérience et de la qualité de service.

Le modèle décrit dans la présente Recommandation doit être un outil offrant la souplesse nécessaire et pouvant fournir des informations en retour sur les différentes caractéristiques de qualité ainsi que sur la qualité globale.

La présente Recommandation n'utilise pas les mêmes paramètres d'entrée que la Recommandation UIT-T J.148. Dans cette Recommandation, la qualité multimédia est calculée au moyen des paramètres des réseaux, des applications et des équipements terminaux.

La présente Recommandation prend pour hypothèse des applications de diffusion vidéo et audio en continu et de télévision linéaire utilisant la télévision à haute définition et la télévision à définition normale, des ordinateurs de bureau, des ordinateurs portables, des assistants numériques personnels et des téléphones mobiles occupant l'intégralité de la largeur de bande audio.

L'application de la présente Recommandation est limitée à la planification de la qualité d'expérience et de la qualité de service. Les autres applications telles que l'analyse comparative et le contrôle de la qualité ne relèvent pas de la présente Recommandation.

6	Pour obtenir des précisions ou des éclaircissements complémentaires sur le présent appel à propositions, nous vous prions de bien vouloir vous adresser au secrétariat de la Commission d'études 12, à l'adresse suivante: tsbsg12@itu.int.

7	Je tiens par ailleurs à souligner l'importance de votre participation à la normalisation de ce nouveau modèle. Vous aiderez ainsi la Commission d'études 12 à élaborer une nouvelle Recommandation décrivant un modèle de calcul destiné aux applications de diffusion vidéo et audio en continu et de télévision linéaire sur réseaux IP, y compris la TVIP, qui servira d'outil de planification de la qualité d'expérience et de la qualité de service pour évaluer les effets conjugués des variations de plusieurs paramètres vidéo et audio ayant une incidence sur la qualité d'expérience (QoE).

Veuillez agréer, Madame, Monsieur, l'assurance de ma haute considération.

Malcolm Johnson
Directeur du Bureau de la normalisation 
des télécommunications

Annexes: 2

annex A

(to TSB Circular 32)

Terms of Reference (ToR) of G.OMVAS

# G.OMVAS model

A block diagram of G.OMVAS model is shown in Figure 1. The model is developed for each assumed service condition. Input, output, and assumed service condition of G.OMVAS are defined as follows.

Figure 1 - G.OMVAS model

# Input parameters for quality-estimation module

| Input parameters | Value range and unit |
| --- | --- |
| Bit rate | Total bit rate: 32 kbps to 22 Mbps Video bit rate: HD - MPEG2: 6 to 22 Mbps - H.264: 3 to 15 Mbps SD - MPEG2: 3 to 15 Mbps - H.264: 1.5 to 8 Mbps QVGA - MPEG4/H.264: 80 to 800 kbps QCIF - MPEG4/H.264: 32 to 256 kbps Audio bit rate: High bit rates: 64 to 384 kbps Low bit rates: 12 to 128 kbps |
| Packet-loss ratio | 0 to 20 % (See Table 4/G.1050) |
| Averaged burst packet-loss length | 0 to 10 packets |
| Jitter (Delay variation) | 0 to 500 ms (See Table 4/G.1050) |

Input parameters for coefficient database

| Input parameters | Value range and unit |
| --- | --- |
| Video codec type | For low bitrate mode, one of the following: - MPEG4 Visual Simple Profile - H.264 baseline profile For high bitrate mode, one of the following: - MPEG2 - H.264 baseline profile - H.264 main profile - H.264 high profile |
| Audio codec type | AMR-NB, AMR-WB+, AAC-LC, HE-AAC (V1 and V2) To be completed with the list from http://wiki.videolan.org/VLC_Features_Formats |
| Product type and implementation | Various types and implementations of: - Encoder - Decoder/buffer/PLC e.g., set-top boxes |
| Video format | QCIF, QVGA, SD: - PAL - NTSC HD: - 720P - 1080P - 1080I |
| Group of pictures (GoP) | e.g., M = 3, N = 15, M = 1, N = 15 Length: fixed, variable, adaptive Structure (e.g.: IBBPBB, IPPPPP, etc.) Note: The value of GoP is determined by the system. |
| Frame rate | High bit rate: 24, 25, 30, 50 (interlaced), 60 (interlaced) fps Low bit rate: 5, 8.33, 12.5, 15, 20, 25, 30 fps Note: The value of frame rate is determined by the system. |
| Number of slices | 1, 2, 3, 6, … Note: The number of slice is determined by the system. |
| Audio sampling rate | 48, 44.1, 22.05, 16, 8 kHz Note: The value of audio sampling rate is determined by the system. |
| Audio channel number | 1 (mono), 2 (stereo) Note: The value of audio channel is determined by the system. |
| Audio frame length | e.g., 20, 64 ms, … Note: The value of audio frame length is determined by the system. |
| Jitter | SD and HD: Jitter is absorbed by jitter buffer because the client terminal has a large amount of capacity. QCIF and QVGA: Packets discarded by jitter buffer translate into packet loss or re-buffering time. |

G.OMVAS output parameters

The G.OMVAS model’s outputs are multimedia ACR MOS, with separate video and audio ACR MOS. G.OMVAS should have different ACR MOS scales for different video formats.

Assumptions of G.OMVAS

| General assumptions | Value range and unit |
| --- | --- |
| Packet loss characteristics | Packet loss characteristics include random and burst packet loss. Packet loss model is based on G.1050 Appendix II. |
| Jitter characteristics | Jitter model is based G.1050 Appendix II. |
| Terminal characteristics | Video: display size and resolution Audio: frequency characteristics, signal-to-distortion ratio, and loudness rating |
| Environmental characteristics | Ambient illuminance and noise |
| Type of content | Should include audio (spoken voices, background noises, music) Different levels of complexity and motion for both video and audio |

Requirements of G.OMVAS in quality estimation accuracy (PROVISIONAL)

Minimum requirements of quality estimation accuracy based on condition-base analysis: R > 0.85, RMSE < 0.5, OR < 0.6.

The R, RMSE, and OR, which are averaged over all experiments, needs to satisfy the minimum requirements.

These requirements are the same for all codecs and video formats.

Note 1: The “condition-base analysis” means that all the evaluation scores for the same coding/transmission-error condition are averaged over different contents.

Note 2: The calculation procedures of the R, RMSE, and OR can be found in Recommendation ITU-T J.247 Appendix 2.

annex B

(to TSB Circular 32)

Time schedule of G.OMVAS project

| Item no. | Description | Remark | Schedule |
| --- | --- | --- | --- |
| 1 | Drafting the scope | Done | 13 March 2008 |
| 2 | Evaluating the so-called “core model approach” in conjunction with P.NAMS under Q.14/12 | Done | 13 March 2008 |
| 3 | Agreement on the scope | Done | SG12 meeting in May 2008 |
| 4 | Deciding whether to take the so-called “core model approach” in conjunction with P.NAMS under Q.14/12 | Done (No “core model” for the time being. Revisit this issue after candidates are available.) | SG12 meeting in May 2008 |
| 5 | ToR | Done | March 2009 |
| 6 | Initial agreement on the model development procedure (collaboration/competition) | Done (Collaboration) | March 2009 |
| 7 | Call for Proposals | Done | March 2009 |
| 8 | Initial indication of participation |  | June 2009 |
| 9 | Test plan for model validation |  | interim meeting in June/July 2009 |
| 10 | Declaration of participation in model development |  | August 2009 |
| 11 | Collection of training databases |  | August 2009 |
| 12 | Submission of candidate model |  | October 2009 |
| 13 | Development of validation databases |  | December 2009 |
| 14 | Evaluation of candidate model(s) |  | January 2010 |
| 15 | Initial draft of G.OMVAS (possible Q.13/12 Rapporteur meeting) |  | March 2010 |
| 16 | Consent of G.OMVAS |  | June 2010 |

_________________