|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  |  | SG9-TD483-R1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 9 |  |  |
|  |  |  |  | Original: English |  |  |
| Question(s): |  |  | 10/9 | Bogota, Colombia, 21-28 November 2018 |  |  |
| TD |  |  |  |  |  |  |
| Source: |  |  | Editors of J.tda |  |  |  |
| Title: |  |  | Consent - Draft new Recommendation ITU-T J.1 (ex. J.tda) " Terms, definitions and acronyms for television and sound transmission and integrated broadband cable networks " |  |  |  |
| Purpose: |  |  | Discussion |  |  |  |
| Contact: |  |  | Hongjun Jia ABP,SAPPRFT China |  |  | Tel: +86 10 86092923 Fax: +86 10 86093711 E-mail: jiahjcn@163.com |
| Contact: |  |  | Zhongzhao Li ABP,SAPPRFT China |  |  | Tel: +86 10 86093737 Fax: +86 10 86093658 E-mail: lizhongzhao@abp2003.cn |

Draft new Recommendation ITU-T J.1 (ex. J.tda)

Terms, definitions and acronyms for television and sound transmission and integrated broadband cable networks

Summary

This draft Recommendation compiles all the definitions related to television and sound transmission, and integrated broadband cable networks, and which are in force in J-series and N-series Recommendations developed under the responsibility of SG9. The Recommendation is regularly updated to reflect newly-approved terms and definitions.

Keywords

Terms, definitions, abbreviations, acronyms,J-series, N-series.

Introduction

In the pursuit of its work aimed towards the standardization of the transmission of television and sound transmission and integrated broadband cable networks,  ITU-T has often found it necessary to develop new terminology, or to adapt it from current technical parlance.

This continuing work has resulted in a small glossary of terms, acronyms and definitions that is specific to that area of activity, and that should desirably find wide recognition and application.

The purpose of this Recommendation is to formalize this small glossary of terms, acronyms and definitions, and to recommend its use in all the texts that address the services and technologies listed above.

It is expected that the glossary will continue to gradually grow over the years, as new terms are developed or adapted.

The ITU also maintains a terms and definitions database which is available online, see [b-ITU DB].

NOTE: This Version 1 of the J.1, approved on xxx includes the terms and definitions related to SG9 scope. Those terms and definitions are found in all Recommednations approved until November 2018.

# 1	Scope

This Recommendation provides a collection of terms, definitions and acronyms compiled from in forced ITU-T Recommendations related to television and sound transmission, and integrated broadband cable networks.

# 2	References

None.

# 3	Definitions

## 3.1	Terms defined elsewhere

Clause 6 contains the terms and definitions compiled by this Recommendation.

## 3.2	Terms defined in this Recommendation

None.

# 4	Abbreviations and acronyms

Clause 7 contains the abbreviations and acronyms compiled by this Recommendation.

# 5	Conventions

None.

# 6	Terms for television and sound transmission, and integrated broadband cable networks

The following definitions are contained in in-force ITU-T Recommendations related to television transmission, sound transmission and integrated broadband cable networks.

6.1	2-way authenticated communication channels [b-ITU-T J.292]: These channels are used for renewable conditional access key management, remote management of SVD, downloadable firmware updates, private interactive application data, and reconfiguration of encryption algorithms between the headend and an SVD.

6.2	2-way communication channel [b-ITU-T J.193]: An IP-based communications link between a NG-STB and the cable network service management system, for the purpose of service management communications.

6.3	  2T pulse/bar ratio error [b-ITU-T J.64]: The 2T sine-squared pulse/bar ratio error is defined as the difference between the amplitudes of the 2T pulse (element B1) and the luminance bar (element B2), expressed as a percentage of the luminance bar amplitude. The peak amplitude of the 2T pulse is referred to a reference point b1 (see Note) (Figs. 1 and 2 of [b-ITU-T J.64]) before the first riser of the staircase. The sign of the difference is positive if the 2T pulse amplitude is greater than the luminance bar amplitude.

NOTE: To avoid error due to line tilt, it may be preferable to use a reference point exclusively for the measurement of 2T pulse/bar ratio error, which is defined to be the linear mean level of the insertion test signal during the periods: 2 to 1 µs before, and 1 to 2 µs after the 2T pulse.

6.4	4K UHDTV [b-ITU-T J.297]: supports 3,840 x 2,160 resolution and 60p frame frequency specified in [b-ITU-R BT.2020].

6.5	A/V [b-ITU-T J.117]: Audio and Video.

6.6	access control [b-ITU-T J.170]: Limiting the flow of information from the resources of a system only to authorized persons, programs, processes or other system resources on a network.

6.6bis	access control [b-ITU-T J.223.1]: Access control is used to control the cable modems (CMs) defined in this document to access networks. It is a process for connecting cable media converters (CMCs) and controlling data communication.

6.7	access network interface [b-ITU-T J.294]: The logical interface between the wide area network (WAN) (i.e., access network) and the residential gateway (RG).

6.8	access node [b-ITU-T J.190]: As used in this Recommendation, an access node is a termination device that terminates the network end of an access network connection. The access node is technology specific; for example, in Annex A of [b-ITU-T J.112] it is called the INA while in Annexes B and C it is the CMTS.

6.9	access unit [b-ITU-T J.89], [b-ITU-T J.187]: A coded representation of a presentation unit. In the case of audio, an access unit is the coded representation of an audio frame. In the case of video, an access unit includes all the coded data for a picture, and any stuffing that follows it, up to, but not including, the start of the next access unit. If a picture is not proceeded by a group_start_code or a sequence_header_code, the access unit begins with the picture_start_code. If a picture is preceded by a group_start_code and/or a sequence_header_code, the access unit begins with the first byte of the first of these start codes. If it is the last picture preceding a sequence_end_code in the bitstream, all bytes between the last byte of the coded picture and the sequence_end_code (including the sequence_end_code) belong to the access unit.

6.9bis 	access unit [b-ITU-T J.181 Amendment 1], [b-ITU-T J.189]: The coded representation of a video picture or an audio frame [b-ITU-T H.222.0].

6.10	accounting [b-ITU-T J.363], [b-ITU-T J.460.3]: The process of collecting usage data.

6.11	acknowledge data [b-ITU-T J.285]: Transfer control data transmitted by a receiver as an affirmative response to the sender. The acknowledge data is included in the synchronization packet.

6.12	active [b-ITU-T J.177], [b-ITU-T J.178], [b-ITU-T J.361]: A service flow is said to be "active" when it is permitted to forward data packets. A service flow must first be admitted before it is active.

6.13	active call/active session [b-ITU-T J.460.1]: A call state where the called party has answered the call and two-way media is being exchanged.

6.14	Active Codes [b-ITU-T J.222.1], [b-ITU-T J.222.2]: The set of spreading codes which carry information in an S-CDMA upstream. The complementary set, the unused codes, are idle and are not transmitted. Reducing the number of active codes below the maximum value of 128 may provide advantages including more robust operation in the presence of coloured noise.

6.15	active service flow [b- ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: An admitted service flow from the CM to the CMTS which is available for packet transmission.

6.16	adaptive quantizer [b-ITU-T J.88]: A quantizer in which the step size is controlled by the chosen slice type, the buffer occupancy and a model of human vision.

6.17	adaptive scanning [b-ITU-T J.88]: A approach that selects the optimal pattern to scan the two-dimensional array of transform coefficient, in order to minimize the number of coefficients scanned up to the end of the block.

6.18	address learning [b-ITU-T J.195.1]: A process that establishes the mapping between higher layer protocol data unit (PDU) addresses and high performance network over coax (HiNoC) node addresses.

6.19	Address Resolution Protocol (ARP) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A protocol of the IETF for converting network addresses to 48-bit Ethernet addresses.

6.20	ADM interface [b-ITU-T J.380.3]: The ADM to ADS message communication link defined by [b-ITU-T J.380.3].

6.21	admitted  [b-ITU-T J.178]: A service flow is said to be "admitted" when the CMTS has reserved resources (e.g., bandwidth) for it on the DOCSIS network.

6.22	admitted service flow [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: A service flow, either provisioned or dynamically signalled, which is authorized and for which resources have been reserved but is not active.

6.23	Advanced Audio Coding (AAC) (based on [ISO/IEC 13818-7]) [b-ITU-T J.296]: An audio coding system standardized by the International Organization for Standardization.

6.24	advanced digital cable transmission technology (ACTT) [b-ITU-T J.381]: Advanced digital cable transmission technology on the physical layer, which includes modulation, channel coding, transmission schemes and frame structure to provide higher spectral efficiency.

6.25	advanced query [b-ITU-T J.380.8]: The "Advanced Query" interface defined by Recommendation ITU-T J.380.8 permits the consumer of a logical service implementation derived from J.380.8 to use an advanced query language to formulate queries against a logical service's data model.

6.26	advanced security [b-ITU-T J.1011]: Function of an ECI compliant CPE which provides enhanced security functions (hardware and software) for an ECI client. Note that the details are specified in [b-ETSI GS ECI 001-5].

6.27	Advanced Query Filter [b-ITU-T J.380.8]: An "Advanced Query Filter" is a collection of free form data items that individually represent complete query terms for a given query language. The individual terms are additively applied (ANDed) together during a query operation against a specific service data model, which results in the identification of a collection of objects contained within the data store.

6.28	Advanced Query Language [b-ITU-T J.380.8]: As used in this document, the term "Advanced Query Language" refers to any language used to formulate queries against a logical service's data model using the advanced query interface. XPath and XQuery are both examples of an advanced query language.

6.29	advertisement (also called "ad") [b-ITU-T J.181]: An inducement to buy or patronize. As used in the cable industry, usually with a duration under 2 minutes (sometimes called "short-form" content).

6.30	aggregation and forwarding [b-ITU-T J.223.1]: An aggregation network device, such as a passive optical network (PON) optical line terminal (OLT), a router, or a switch, receives data from CMCs and forwards the data to different uplinks for transmission based on the preset QoS priorities.

6.31	alarm indication signal [b-ITU-T J.214]: Also known as the blue alarm. When no incoming signal is detected, a CSU/DSU transmits an unframed all-ones pattern to the network to maintain synchronization and announce its presence to the network..

6.32	algorithm [b-ITU-T J.93], [b-ITU-T J.95]: A mathematical process which can be used for the scrambling and descrambling of a data stream.

6.33	Alignment signal (AS) [b-ITU-T J.27]: Sine-wave signal at 1 kHz (See Note) at a level of 0 dBm0s, which is used to align the international sound programme connection.

NOTE – This frequency is nominal, and 1020 Hz recommended by CCITT Recommendation O.33 may be used.

6.34	allocate [b-ITU-T J.117]: The process of acquiring the resources, the address and other parameters of a plug for the purpose of establishing an asynchronous connection data transfer capability.

6.35	allocation [b-ITU-T J.222.1], [b-ITU-T J.122]: A group of contiguous mini-slots in a MAP which constitutes a single transmit opportunity.

6.36	All-ones level [b-ITU-T J.101]: The all-ones level is the level resulting from a continuous stream of “one” pulses. For measuring purposes the all-ones level is defined as twice the mean value of clock run-in minus the all-zeros level.

6.37	All-zeros level [b-ITU-T J.101]: The all-zeros level is the level resulting from a continuous stream of “zero” pulses. For measuring purposes the all-zeros level is defined as the mean level of the back porch within the nominal duration of the colour burst.

6.38	  Amplitude error of the burst at n MHz (see Note) [b-ITU-T J.64]: This quantity is defined as the difference in terms of magnitude and sign between the peak-to-peak amplitude of the burst at n MHz and the reference quantity A0 (defined as above), expressed as a percentage of A0.

NOTE: n is the designation of the frequency of the burst taken into account. Note 1 of § 2.20.1 also applies here.

6.39	analogue cue tone [b-ITU-T J.181 Amendment 1]: In an analogue system, a signal which is usually either a sequence of DTMF tones or a contact closure that denotes to ad insertion equipment that an advertisement avail is about to begin or end.

6.40	analogue protection system bits (APS bits) [b-ITU-T J.197]: Bits 3 and 2 of the CCI, designating the state of analogue protection for a set top box.

6.41	announcement server [b-ITU-T J.178]: An announcement server plays informational announcements in the IPCablecom network. Announcements are needed for communications that do not complete and to provide enhanced information services to the user.

6.42	announcement servers [b-ITU-T J.175]: Also known as Audio Servers, Announcement Servers are network components that manage and play informational tones and messages in response to events that occur in the network. Most announcements are media streams that originate from servers in the network. Some simple tones and short announcements can also reside at the MTA and in the MG.

6.43	anomalous frame repetition [b-ITU-T J.246], [b-ITU-T J.247]: An event where the HRC outputs a single frame repeatedly in response to an unusual or out of the ordinary event. Anomalous frame repetition includes but is not limited to the following types of events: an error in the transmission channel, a change in the delay through the transmission channel, limited computer resources impacting the decoder's performance, and limited computer resources impacting the display of the video signal.

6.44	API connection [b-ITU-T J.280]: A TCP/IP socket connection between a server and a splicer for transferring API messages.

6.44bis	API connection [b-ITU-T J.287]: A communications connection between an automation system and an injector for transferring API messages.

6.45	application [b-ITU-T J.205]: Any active DTV service content aimed at end-user interaction.

6.45bis	application [b-ITU-T J.200]: Information that expresses a specific set of observable behaviour.

6.45ter	application [b-ITU-T J.215]: An application is a functional implementation realized as software running in one or spread over several interplaying hardware entities.

6.46	application catalogue [b-ITU-T J.205]: Data structure listing available applications (interactive content). This data structure is local to the IBB DTV receiver, and describes the IBB applications installed on the IBB DTV receiver.

6.47	application catalogue user interface [b-ITU-T J.205]: A user interface functionality over the IBB DTV receiver, intended to allow the end user to browse the available applications in the application catalogue or to query application catalogues exposed by application repositories. Also, it lists applications that are currently available in the selected IBB DTV service or broadcast DTV service.

6.48	application component [b-ITU-T J.205]: A block of data that forms part of an application. Application component types are: code, resources, meta-data, control and user settings.

6.49	application component delivery mechanism [b-ITU-T J.205]: A mechanism, channel or medium used to deliver application components to an IBB DTV receiver.

6.50	application control data structure [b-ITU-T J.205 Cor. 2]: Data structures transmitted within the integrated broadcast and broadband (IBB) DTV service to control IBB applications (e.g., AIT in [b-GINGA Data Transmission], [b-MHP]).

6.51	application descriptor file [b-ITU-T J.206]: A file containing metadata information for an application.

6.52	application entity [b-ITU-T J.200]: A unit of information that expresses some portion of an application.

6.53	application environment (environment) [b-ITU-T J.200]: The context or software environment in which an application is processed.

6.54	application ID [b-ITU-T J.128]: This is a 16-bit field indicating a numeric ID for an application running on the Set-top Device. The Application ID is typically assigned through a Source Name Sub-table (SNS) from [ITU-T J.94] carried in the Broadcast DSG Tunnel.

6.54bis	application ID [b-ITU-T J.290]: This is a field indicating a numeric ID for an application running on the set-top device.

6.55	application install package [b-ITU-T J.205]: Application delivery mechanism in which several application components are bundled together in a single file.

6.56	application manager [b-ITU-T J.365]: A system that interfaces to Policy Server(s) for requesting QoS based service on behalf of an end-user or network management system.

6.57	application program interface (API) [b-ITU-T J.215]: An application program interface is the software interface to system services or software libraries. An API can consist of classes, function calls, subroutine calls, descriptive tags, etc.

6.58	application programming interface (API) [b-ITU-T J.200]: Software libraries that provide uniform access to system services.

6.59	application repository [b-ITU-T J.205]: Entity, reachable through the broadband channel, that provides access to integrated broadcast and broadband (IBB) applications contained in it. These IBB applications can be downloaded and installed on the IBB DTV receiver, manually, by the end user, or, in case of being signalled within an IBB DTV service, launched or installed automatically under the IBB DTV service's control using the IBB application control mechanism. An application repository can be managed by IBB service providers, IBB DTV receiver manufacturers, or other third party entities.

6.60	application resource [b-ITU-T J.200]: A bit-stream serialization (a physical embodiment) of an application entity.

6.61	application resource collection [b-ITU-T J.200]: The set of application resources that embody an application entity collection.

6.62	application usage [b-ITU-T J.367]: Detailed information on the interaction of an application with the XCAP server.

6.63	application-free EPG [b-ITU-T J.90]: An EPG in which the provider can freely select the content and the layout of the presentation, in a way that is implemented by the consumer television/multimedia display.

6.64	Aq [b-ITU-T J.148]: Objective measurement of audio quality.

6.65	Aq(Vq) [b-ITU-T J.148]: Objective measurement of audio quality, accounting for the influence of video quality.

6.66	area availability code [b-ITU-T J.90]: A code used to denote that part of the area covered by a programme distribution service, to which a specific programme should be distributed.

6.67	assured capabilities [b-ITU-T J.260]: Capabilities providing high confidence or certainty that critical telecommunications are available and perform reliably.

6.68	asynchronous connection [b-ITU-T J.117]: A point-to-point communication path established between a producer node and a consumer node, that supports robust high-bandwidth flow-controlled transfers of one or more data frames.

6.69	asynchronous push [b-ITU-T J.117]: A method of data delivery in which the node producing the data uses 1394 write transactions to deposit data into the address space of a consumer node.

6.70	asynchronous time division multiple access (A-TDMA) [b-ITU-T J.291]: A variant on the protocol for wireless communication, used in DOCSIS 2.0.

6.71	asynchronous transfer mode (ATM) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A protocol for the transmission of a variety of digital signals using uniform 53-byte cells.

6.72	A-TDMA [b-ITU-T J.122]: DOCS 2.0 TDMA mode (as distinguished from DOCS 1.x TDMA).

6.73	attach [b-ITU-T J.117]: The process of communicating the address and other parameters of a plug to another plug for the purpose of establishing data transfer capability.

6.74	attribute [b-ITU-T J.164]: An Event Message Attribute is a predefined data element described by an attribute definition and attribute type.

6.75	audience [b-ITU-T J.380.6]: The term "audience" is used to refer to a collection of one or more subscribers. A logical service that implements the SIS interface described in this document may often provide profile information about an audience. For example, a logical service may provide information about one or more audiences within a linear advertising zone rather than information about individual subscribers.

6.76	audience measurement [b-ITU-T J.296]: The measurement of user viewing conditions, records of viewing, recording, playing, and manipulation, based on data collected with the permission of users for improvement of personalized services.

6.77	audience measurement functions [b-ITU-T J.296]: The functions that, when given permission, measure end-user behaviour by processing events or samples from cable TV services. Such functions may request and collect end-user information. They transfer processed events, samples, and end-user information to aggregation functions.

6.78	Audio Interchange File Format (AIFF) (based on [b-DAVIC 1.4.1]) [b-ITU-T J.296]: The Audio Interchange File Format is defined in [b-DAVIC 1.4.1] Specification Part 9 Annex B; it is the file format for audio encoding of pulse code modulation (PCM).

6.79	audio server [b-ITU-T J.170], [b-ITU-T J.161]: An Audio Server plays informational announcements in IPCablecom network. Media announcements are needed for communications that do not complete and to provide enhanced information services to the user. The component parts of Audio Server services are Media Players and Media Player Controllers.

6.80	augmentation region [b-ITU-T J.301]: The targeted area to augment any object or region in a TV scene. It provides the parameters of a static or animated 2D region related to natural media.

6.81	augmentation time [b-ITU-T J.301]: The appointed time to augment any object or region in a TV scene. It provides the parameters of start presentation time and time duration for augmented content.

6.82	augmented broadcast provider [b-ITU-T J.301]: A broadcaster who provides augmented broadcasting services, or an entity allowed to develop and provide augmented broadcasting service as a broadcast provider.

6.83	augmented broadcasting [b-ITU-T J.301]: Broadcasting service or programme to realize augmented reality smart television (AR-STV).

6.84	augmented broadcasting metadata [b-ITU-T J.301]: Formatted data describing augmented content.

6.85	augmented content [b-ITU-T J.301]: A binary object, such as 2D images, 3D animated models or audio/video streaming files, to be augmented into a predefined augmentation region.

6.86	augmented content provider [b-ITU-T J.301]: An entity allowed to develop and provide augmented content.

6.87	augmented reality (AR) [b-ITU-T J.301]: A type of mixed reality where graphical elements are integrated into the real world in order to enhance user experience and enrich information.

6.88	augmented reality smart television (AR-STV) [b-ITU-T J.301]: AR-based TV broadcasting service to implement the mixed content of a broadcast programme and augmented object at a targeted position and time in real time on a receiving terminal such as a TV or set-top box.

6.89	authentication [b-ITU-T J.260]: The act or method used to verify a claimed identity.

6.89bis	authentication [b-ITU-T J.93], [b-ITU-T J.95]: The process intended to allow the system to check with certainty the identification of a party.

6.89ter	authentication [b-ITU-T J.361]: The act of giving access to a service or device if one has permission to have the access.

6.89quater	authentication [b-ITU-T J.170], [b-ITU-T J.178]: The process of verifying the claimed identity of an entity to another entity.

6.90	authenticity [b-ITU-T J.170]: The ability to ensure that the given information is without modification or forgery and was in fact produced by the entity that claims to have given the \on.

6.91	authorization [b-ITU-T J.170], [b-ITU-T J.178]: The act of giving access to a service or device if one has the permission to have the access.

6.91bis	authorization [b-ITU-T J.260]: The act of determining if a particular privilege, such as access to telecommunications resources, can be granted to the presenter of a particular credential.

6.92	authorization centre (AC) [b-ITU-T J.1001]: An entity which issues identification information of the conditional access module (CAM) and performs the authentication process when the CAM requests renewal of the conditional access client software.

6.92bis authorization centre (AC) [b-ITU-T J.1002]: An entity which issues identification information of CAM and performs authentication process when CAM requests renewing of CACS.

6.93	authorization coding [b-ITU-T J.93], [b-ITU-T J.95]: A digital word that describes the personality or service access capability of the subscriber decoder unit.

NOTE: This code word, which is based on the service access authorized by the billing system, determines which keys are distributed to each customer, and is required at the subscriber decoder to authorize the descrambling of any specific program.

6.94	authorization module [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: An abstract module that the CMTS can contact to authorize service flows and classifiers. The authorization module tells the CMTS whether the requesting CM is authorized for the resources it is requesting.

6.95	authorized collector [b-ITU-T J.204]: An Event Tracking API-compliant server that implements the receiving side of the IPDR Streaming Protocol, and which has been authorized to participate in the overall Collection System.

6.96	authorized IBB application provider [b-ITU-T J.205]: An entity allowed to develop and provide integrated broadcast and broadband (IBB) applications within a given implementation of an IBB digital television (DTV) system.

6.97	authorized output domain (AOD) [b-ITU-T J.290]: The devices in this domain are connected to the ASD using operator-approved output interfaces.

6.98	authorized service domain (ASD) [b-ITU-T J.290]: The devices in this domain are able to authenticate themselves and support content usage rights as defined by the network operator.

6.99	automatic location identification (ALI) [b-ITU-T J.460.1]: The database that maps telephone number to location in the current 9-1-1 system.

6.100	automatic number identification (ANI) [b-ITU-T J.460.1]: The mechanism used to determine the telephone number of the caller.

6.101	automation system [b-ITU-T J.287]: A control system for a program origination facility which controls operation of the production facilities and devices.

6.102	avail [b-ITU-T J.181 Amendment 1], [b-ITU-T J.287]: Time space provided to cable operators by cable programming services during a program for use by the community antenna television (CATV) operator; the time is usually sold to local advertisers or used for channel self-promotion.

6.103	availability [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: In cable television systems, availability is the long-term ratio of the actual RF channel operation time to scheduled RF channel operation time (expressed as a percent value) and is based on a bit error rate (BER) assumption.

6.104	available sub-carrier [b-ITU-T J.195.2]: Sub-carriers of OFDM symbol for data bearing.

6.105	AVC [b-ITU-T J.181 Amendment 1]: Abbreviation for "Advanced Video Coding" and refers specifically to video compression standardized in [b-ITU-T H.264].

6.106	backoff [b-ITU-T J.287]: A mechanism, commonly used in data communications, to randomize the interval between retries.

6.107	back-to-back insertion [b-ITU-T J.280]: Insertion of two or more temporally contiguous sessions without returning to the primary channel between sessions.

6.108	 bandwidth allocation map [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.222.1]: The MAC management message that the CMTS uses to allocate transmission opportunities to cable modems.

6.109	Bandwidth Efficiency (BWE) [b-ITU-T J.141]: The data capacity that can be transmitted through the channel. It is expressed in terms of the amount of data transmitted through the unit of bandwidth per unit of time (bits/s/Hz).

6.110	Bar tilt [b-ITU-T J.64]: The luminance bar tilt is defined as the difference between the level of the luminance bar one microsecond after the half amplitude point of its leading edge, and the level one microsecond before the half amplitude point of its trailing edgeexpressed as a percentage of the luminance bar amplitude. The sign of the difference is positive if b4 is higher than b3. (See more details in Figs. 1 and 2 of J.64).

NOTE: The parameter bar tilt as defined above is a unique measurement by automatic devices of a specific form of line time waveform distortion, i.e. the difference in the level of the line bar at two specific reference points. This measurement is different to the measurements of line time waveform distortion described in Recommendation 567 (§ C.3.5.1.3 and Annex III to Part C, § 2.1) where the maximum difference in level at any point between defined reference. points is measured

6.111	  Base-line distortion	[b-ITU-T J.64]: The base-line distortion is defined as the difference between the levels of the signal at point b7, which is located after the mid-amplitude point of the trailing edge of the bar (element B2) at a distance of 400 ns for 625-line systems and 500 ns for 525-line systems (see Figs. 1 and 2 of J.64), and at a reference point b1 located before the beginning of the staircase in line 17. (See more details in Figs. 1 and 2 of J.64).
	The base-line distortion is expressed as a percentage of the luminance bar amplitude. It is to be measured after the bandwidth of the signal has been limited (see Note). The sign of the difference is positive if the signal level at point b7 is higher than the level of reference point b1.

NOTE: Limitation may be achieved by the use of a network, the design of which is based on “Solution 3” [b-Thomson, 1952], having its first zero at 3.3 MHz, or by an equivalent technique.

6.112	basic [b-ITU-T J.287]: A category of request or response operation supported by this API.

6.113	basic amplitude [b-ITU-T J.101]: The basic amplitude is the difference between the all-ones level and the all-zeros level.

6.114	Basic amplitude error [b-ITU-T J.101]: This parameter is defined as the difference between the basic amplitude and the nominal teletext signal amplitude expressed as a percentage of the latter. For mathematical notation, see [b-ITU-T J.101].

6.115	basic query [b-ITU-T J.380.8]: To obtain information from a logical service implementation derived from Recommendation ITU-T J.380.8, a logical service consumer issues a "query" against the data. The "Basic Query" interface is based on an exchange of name/value pairs, referred to as qualifiers, and requires no specialized knowledge of advanced query languages such as XQuery.

6.116	Basic Query Filter [b-ITU-T J.380.8]: A "Basic Query Filter" is a collection of name and value pairs additively applied (ANDed) together during a basic query operation against a specific service data model, which results in the identification of a collection of objects contained within the data store.

6.117	BC-TD [b-ITU-T J.293]: A logical interface defined in 6.1.1 for content reception through a non-IP (RF-based) network.

6.118	best effort domain (BED) [b-ITU-T J.290]: Devices and physical layer segments not conforming to the requirements of ASD, AOD, GSD. The devices in this domain do not require content protection or guaranteed quality of service.

6.119	billing correlation ID (BCID) [b-ITU-T J.363]: A Billing Correlation ID (BCID) is an IPCablecom-defined term created for the multimedia session, which uniquely identifies the session within the IPCablecom Multimedia billing domain.

6.120	bit rate [b-ITU-T J.88], [b-ITU-T J.89]: The rate at which the compressed bit stream is delivered from the channel to the input of a decoder.

6.121	bit stream [b-ITU-T J.189]: MPEG-2 transport stream defined in ITU-T Rec. H.222.0 | ISO/IEC 13818-1.

6.122	bit stream format [b-ITU-T J.181]: An encoding of information resulting in a compliant MPEG-2 transport stream.

6.123	Bit-error ratio [b-ITU-T J.67]: The bit-error ratio (BER) is defined as the ratio of the number of detected bit errors to the number of transmitted bits over a given period of time.

6.124	block [b-ITU-T J.88]: A unit of 8 pixels by 8 lines size for application of the Walsh Hadamart Transform.

6.125	Bonded Channel Set [b-ITU-T J.222.2]: An identified set of upstream or downstream channels among which a stream of packets is distributed.

6.126	Bonded Channel(s) [b-ITU-T J.222.2]: One or more independent RF channels whose data packets are logically combined into one higher-speed data stream.

6.126bis bonded channels[b-ITU-T J.212]: A logical channel comprising multiple individual channels.

6.127	Bonding Group [b-ITU-T J.222.2]: A list of channels providing a means to identify the specific channels bonded together. Sometimes referred to as a "Bonded Channel Group".

6.128	bouquet [b-ITU-T J.180]: A collection of services multiplexed in a single data stream.

6.128bis	 bouquet [b-ITU-T J.94]: A collection of services marketed as a single entity.

6.129	box [b-ITU-T J.123], [b-ITU-T J.124], [b-ITU-T J.94]: An object-oriented building block defined by a unique type identifier and length.

6.130	break [b-ITU-T J.181], [b-ITU-T J.181 Amendment 1]: Avail or an actual insertion in progress.

6.131	bridge protocol data unit (BDU) [b-ITU-T J.112], [b-ITU-T J.116], [b-ITU-T J.122]: Spanning tree protocol messages as defined in [b-ISO/IEC 15802-3].

6.132 	bridge protocol data unit (BPDU) [b-ITU-T J.112 Annex B]: Spanning tree protocol messages as defined in [b-ISO/IEC 10038].

6.132bis	Bridge Protocol Data Unit (BPDU) [b-ITU-T J.112 Annex C]: Spanning tree protocol messages as defined in [b-RFC 1350].

6.133	bridged network [b-ITU-T J.213]: A set of IEEE 802 LANs interconnected by IEEE 802.1D MAC bridges.

6.134	bridging mode [b-ITU-T J.211]: A short-term operating condition of the DTI clock where the DTI client has recently lost its controlling input and is using stored data, acquired while in normal or fast mode operation, to control its output. While in bridging, the degree of deviation of the output is deemed to be such that DTI client clock is still performing within normal or acceptable limits. If an outage period persists, the DTI client clock will transition to the holdover mode indicating that the DTI client clock output may be degraded.

6.135	broadband cable modem [b-ITU-T J.193]: A cable modem built into the STB, which provides full Broadband access to the Internet, and is intended for customer use. In addition, cable services can be delivered via this broadband IP connection.

6.136	broadband channel [b-ITU-T J.205]: A medium used to deliver interactive content. Usually this medium is based on the Internet and allows the delivery of non-linear and on-demand content. A broadband channel allows access to servers that may be located in the Internet.

6.137	broadband IP network [b-ITU-T J.241]: Access IP telecommunications network offered by ADSL, ADSL2+, VDSL, Optical Access Network, etc.

6.138	broadcast [b-ITU-T J.215]: A broadcast is a service that is delivered to all customers. Each customer may select a particular broadcast channel out of many.

6.139	broadcast address [b-ITU-T J.112], [b-ITU-T J.116], [b-ITU-T J.122]: A predefined destination address that denotes the set of all data network service access points.

6.140	broadcast addresses [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C]: Predefined destination address that denotes the set of all data network service access points.

6.141	broadcast application [b-ITU-T J.215]: A broadcast application is an application running on the set-top converter that is loaded through in-band information, inserted either at the head-end or by a content provider farther upstream.

6.142	broadcast channel [b-ITU-T J.110]: A unidirectional, broadband, point-to-multipoint channel, which may include video, audio and data. The broadcast channel is established from the services provider to the users. It may include the forward interaction path.

6.142bis broadcast channel [b-ITU-T J.205]: A medium used to deliver digital television (DTV) services. Examples: free to air, satellite and cable.

6.143	broadcast DTV service [b-ITU-T J.205]: Any digital television (DTV) service, delivered through a broadcast channel.

6.144	broadcast markup language (BML) standard [b-ITU-T J.200]: An XML [b-W3C XML] application language [b-ARIB STD B-24] that deals with tags and attributes for multimedia representation exclusively.

6.145	broadcast transport stream [b-ITU-T J.296]: The broadcast transport stream is composed of MPEG-2 transport stream (TS) signals, which have multi-frame architecture using the same frame length as the orthogonal frequency-division multiplexing (OFDM) frame.

6.146	broadcaster (service provider) [b-ITU-T J.94]: An organization which assembles a sequence of events or programmes to be delivered to the viewer based upon a schedule.

6.147	broadcasting organization [b-ITU-T N.1], [b-ITU-T N.51]: A broadcasting organization is an organization which is concerned with either or both sound and television broadcasting. Most of the customers ordering facilities for sound-programme and television transmission are broadcasting organizations. For convenience, the term broadcasting organization is used to denote the activity of any user or customer and, where so used, it is equally applicable to any other customer requiring sound-programme or television transmissions.

6.148	broadcasting organization (receive) [b-ITU-T N.1], [b-ITU-T J.13], [b-ITU-T N.51]: The broadcasting organization at the receiving end of the sound programme being transmitted over the international sound-programme connection.

6.149	broadcasting organization (send) [b-ITU-T N.1], [b-ITU-T J.13], [b-ITU-T N.51]: The broadcasting organization at the sending end of the sound programme being transmitted over the international sound-programme connection.

6.150	Bslbf [b-ITU-T J.287]: Bit string, left bit first, where "left" is the order in which bit strings are written in the Recommendation. Bit strings are written as a string of 1s and 0s within single quote marks, e.g., '1000 0001'. Blanks within a bit string are for ease of reading and have no significance. (See [b-ITU-T H.222.0].)

6.151	burst [b-ITU-T J.122]: A single continuous RF signal from the upstream transmitter, from transmitter on to transmitter off.

6.152	burst error second [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: Any Errored Second containing at least 100 errors.

6.153	BWA BTS modem [b-ITU-T J.116]: Broadband Wireless Access Base Transceiver Station modem. One or more downstream demodulators and their corresponding upstream modulators.

6.154	BWA CPE modem [b-ITU-T J.116]: Broadband Wireless Access Customer Premises Equipment Modem.

6.155	byte [b-ITU-T J.117]: 8 bits of data.

6.155bis	 byte [b-ITU-T J.150]: A group of eight bits.

6.156	C3 [b-ITU-T J.181 Amendment 1]: An audience measurement that is specified to include live viewing as well as DVR viewing up to 75 hours from the ad minute original broadcast time.

6.157	CA_system_ID [b-ITU-T J.128], [b-ITU-T J.290]: This is a 16-bit field indicating the type of CA system applicable for either the associated ECM and/or EMM streams. The CA_system_ID may be used as a DSG Client ID in DSG Advanced Mode.

6.157bis CA_system_id [b-ITU-T J.296]: The conditional access system identifier having a 16-bit field, which can identify the conditional access (CA) system.

6.158	cable integrated broadcast and broadband DTV service [b-ITU-T J.205]: An integrated broadcast and broadband digital television (DTV) service managed by cable operators.

6.159	cable modem [b-ITU-T J.160]: A cable modem is a layer two termination device that terminates the customer end of the DOCSIS connection.

6.159bis cable modem [b-ITU-T J.162], [b-ITU-T J.163], [b-ITU-T J.164], [b-ITU-T J.166], [b-ITU-T J.167]: A cable modem is a layer two termination device that terminates the customer end of the J.112 (or J.122) connection.

6.159ter cable modem [b-ITU-T J.171.1]: The delivery of high-speed data access to customer locations using equipment built in conformance with ITU-T Recs J.83 and J.112.

6.159quater cable modem (CM) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.210], [b-ITU-T J.212]: A modulator-demodulator at subscriber locations intended for use in conveying data communications on a cable television system.

6.160	Cable Modem Service Group [b-ITU-T J.222.2]: In the HFC plant topology, the complete set of downstream and upstream channels within a single CMTS that a single Cable Modem could potentially receive or transmit on. In most HFC deployments, a CM-SG corresponds to a single Fibre Node. Usually, a CM-SG serves multiple CMs.

6.161	cable modem termination system (CMTS) [b-ITU-T J.161], [b-ITU-T J.365]: The device at a cable head-end which implements the DOCSIS RFI MAC protocol and connects to CMs over an HFC network.

6.161bis	 cable modem termination system (CMTS) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: Cable modem termination system, located at the cable television system headend or distribution hub, which provides complementary functionality to the cable modems to enable data connectivity to a wide-area network.

6.161ter	 cable modem termination system (CMTS) [b-ITU-T J.291]: Located at the cable television system headend or distribution hub, which provides complementary functionality to the cable modems to enable data connectivity to a wide-area network.

6.162		cable modem termination system; Network Side Interface (CMTS-NSI) [b-ITU-T J.112 Annex B], [b-ITU-T J.122]: The interface, defined in [b-DOCSIS3], between a CMTS and the equipment on its network side.

6.162bis 	cable modem termination system; Network Side Interface (CMTS-NSI) [b-ITU-T J.112 Annex C], The interface, defined in "DataOver-Cable Service Interface Specifications, Cable Modem Termination System Network Side Interface Specification, SP-CMTS-NSI-I01-960702", between a CMTS and the equipment on its network side.

6.162ter 	cable modem termination system; Network Side Interface (CMTS-NSI) [b-ITU-T J.112]: The interface, defined in [MCNS3], between a CMTS and the equipment on its network side (see Annex B). Called INA in Annex A and MC in Annex C.

6.163	Cable Modem to CPE Interface (CMCI) [b-ITU-T J.112]: The interface, defined in [MCNS4], between a CM and CPE.

6.163bis cable modem to CPE interface (CMCI) [b-ITU-T J.122]: The interface, defined in [DOCS4], between a CM and CPE.

6.163ter cable modem to CPE interface (CMCI) [b-ITU-T J.112 Annex B]: The interface, defined in [DOCSIS4], between a CM and CPE.

6.163quater cable modem to CPE interface (CMCI) [b-ITU-T J.112 Annex C]: The interface between a CM and CPE.

6.164	cable platform [b-ITU-T J.295]: This refers to the entire provision infrastructure of the cable business, including the service delivery platform (SDP), cable network, hybrid cable set-top box, home network and others.

6.165	Cable Security Portal (CSP) [b-ITU-T J.191]: A functional element that provides security management and translation functions between the HFC and the Home.

6.166	cable television [b-ITU-T J.142]: Communications systems distributes broadcast and non-broadcast signals, as well as a multiplicity of satellite signals originating programming and other signals by means of coaxial cable and/or optical fibre.

6.167	cable television operator [b-ITU-T J.295]: A service provider and/or network provider providing television programmes and other interactive services, via a cable television network with radio frequency (RF) signals through coaxial cables or digital light pulses through fixed optical fibres.

6.168	CableHome [b-ITU-T J.126]: This is a Cable Television Laboratories, Inc. ("CableLabs") specification (see http://www.cablelabs.com/projects/cablehome/) for the interfaces necessary to extend high-quality cable-based services to network devices within the home. The CableHome project addresses issues such as device interoperability, QoS (Quality of Service), and network management. This term is also used for a system or device that is compliant with the CableHome specifications [b-ITU-T J.191] or [b-ITU-T J.192].

6.169	CableModem base specifications [b-ITU-T J.126]: There are currently four versions of what are in this Recommendation referred to as the CableModem Base Specifications. The original CableModem is specified in SCTE 22-1 2002: DOCSIS 1.0 Radio Frequency Interface and SCTE 22-3. SCTE 22-1 is J.112 (1998) with some corrections and minor additions. The second specification is J.112 with SCTE 23-3, the third specification is J.122 with SCTE 79-2, and the fourth specification is J.222.

6.170	call [b-ITU-T J.164]: A call is an instance of user-initiated voice communication capabilities. In traditional telephony, a call is generally considered as the establishment of connectivity directly between two points: originating party and terminating party. In the IPCablecom context, as noted above, the communication between the parties is "connectionless" in the traditional sense.

6.171	call management server (CMS) [b-ITU-T J.161]: Controls the audio connections. Also called a call agent in MGCP/SGCP terminology. This is one example of an application server.

6.171bis Call Management Server (CMS) [b-ITU-T J.191]: [IPCablecom] Controls the audio connections. Also called a Call Agent in MGCP/SGCP terminology.

6.172	callback [b-ITU-T J.460.1]: When a PSAP initiates a new call back to a caller. This is not the same as ringback. See below.

6.173	CAM_ID [b-ITU-T J.1004]: The identification value of the conditional access module (CAM) having a size of 8 bytes.

6.174	capacity provider [b-ITU-T J.90]: The entity that provides the technical facilities needed to deliver a programme schedule (e.g. the common carrier).

6.175	capture bandwidth (CBW) [b-ITU-T J.222.1]: The sum of the tuning bands in the TB list in MHz.

6.176	carrier hum modulation [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: The peak-to-peak magnitude of the amplitude distortion relative to the RF carrier signal level due to the fundamental and low-order harmonics of the power-supply frequency.

6.177	carrier related band [b-ITU-T J.112]: A frequency bandwidth spacing of television channels on a cable television system in exact frequency increments.

6.178	Carrier-to-Noise Ratio (C/N or CNR) [b-ITU-T J.112], [b-ITU-T J.116]: The square of the ratio of the root mean square (rms) of the voltage of the digitally-modulated RF carrier to the rms of the continuous random noise voltage in the defined measurement bandwidth (if not specified explicitly, the measurement bandwidth is the symbol rate of the digital modulation).

6.178bis Carrier-to-Noise Ratio (C/N or CNR) [b-ITU-T J.122 Annex B], [b-ITU-T J.112 Annex C]: The square of the ratio of the root mean square (rms) of the voltage of the digitally-modulated RF carrier to the rms of the continuous random noise voltage in the defined measurement bandwidth. (If not specified explicitly, the measurement bandwidth is the symbol rate of the digital modulation; for video it is 4 MHz.)

6.178ter	Carrier-to-Noise Ratio (C/N or CNR) [b-ITU-T J.122]: The ratio of signal power to noise power in the defined measurement bandwidth. For digital modulation, CNR = Es/N0, the energy-per-symbol to noise-density ratio; the signal power is measured in the occupied bandwidth, and the noise power is normalized to the modulation-rate bandwidth. For video, the measurement bandwidth is 4 MHz.

6.178quater Carrier-to-Noise Ratio (C/N or CNR) [b-ITU-T J.210]: The ratio of signal power to noise power in a defined measurement bandwidth. For digital modulation, CNR = Es/N0, the energy-per symbol to noise-density ratio; the signal power is measured in the occupied bandwidth, and the noise power is normalized to the modulation-rate bandwidth. For analog NTSC video modulation, the noise measurement bandwidth is 4 MHz.

6.179	cascading style sheets (CSS) [b-ITU-T J.200]: Standard for the style sheet for a markup language document.

6.180	CDCS_ISS [b-ITU-T J.1020]: One of the roles for DM system. It has the role of personalization of CDCS installed in a user secondary device.

6.181	CDCS_MSS [b-ITU-T J.1020]: One of the roles for DM system. It has the role of establishment of secure channel between CDCS_MSS and DM agent, and managing CDCS download.

6.182	CDCS_PSS [b-ITU-T J.1020]: One of the roles for DM system. It has the role of managing content access entitlement policy, based on access rights, which is different per entitlement levels and hardware capability of the secondary device considered.

6.183	C-DOCSIS CM [b-ITU-T J.223.2]: A cable modem (CM) that complies with the CM requirements of DOCSIS 3.0.

6.184	C-DOCSIS CMTS [b-ITU-T J.223.2]: A cable modem termination system (CMTS) that complies with the CMTS requirements of DOCSIS 3.0. In the context of this specification, it consists of a coax media converter (CMC) controller and a CMC or multiple CMCs operating together.

6.185	C-DOCSIS data tag (CDT) [b-ITU-T J.223.2]: As defined in the C-DOCSIS system, C-DOCSIS data tags (CDTs) are used to identify a service flow to which each data packet belongs.

6.186	C-DOCSIS management messages (CDMM) [b-ITU-T J.223.2]: The C-DOCSIS management messages (CDMMs) are used for exchanging configurations, status, and management information between the system control module and the radio frequency interface (RFI) module.

6.187	C-DOCSIS system [b-ITU-T J.223.1]: The C-DOCSIS system consists of the CMC controller, CMC, and C-DOCSIS CM. It implements broadband data access and forwarding, service configuration, as well as management and maintenance of coaxial cable networks.

6.188	ceiling (ceil) [b-ITU-T J.210]: The ceiling function rounds a number up to the nearest integer or nearest multiple of significance. Use: Ceiling (number, significance).

6.189	centralized distribution structure [b-ITU-T J.195.1]: A physical structure of a passive coaxial access network and a multiple stage power distribution network realized by splitters only.

6.190	channel [b-ITU-T J.122]: The frequency spectrum occupied by a signal. Usually specified by centre frequency and bandwidth parameters.

6.190bis channel [b-ITU-T J.280]: A channel is a synonym for a "Service" in DVB terminology, or a "Program" in MPEG terminology.

6.191	channel bonding [b-ITU-T J.222.1]: A logical process that combines the data packets received on multiple independent channels into one higher-speed data stream. Channel bonding can be implemented independently on upstream channels or downstream channels.

6.192	channel number [b-ITU-T J.296]: The channel number refers to the numbers specified by broadcasting stations to label them (1 to 12). These numbers should correspond to the number of one-touch buttons on a remote controller unit.

6.193	channel scan (scan) [b-ITU-T J.296]: The channel scan function searches all channels to determine the presence or absence of broadcast signals. It is used by viewers when it is unknown which terrestrial broadcasting programmes can be received in the area of residence. When a broadcast signal is detected, the information on the station will be registered.

6.194	channel service unit [b-ITU-T J.214]: The piece of a CSU/DSU that talks to the telco network, understands framing and line coding, and provides electrical isolation of the network from the telco network.

6.195	chapter [b-ITU-T J.181]: A short section of a longer program, usually situated to permit a viewer to easily locate a scene or section of the program.

6.196	character DATA (CDATA) [b-ITU-T J.380.4], [b-ITU-T J.380.7], [b-ITU-T J.380.8]: XML data that is not parsed. CDATA carries markup examples that would otherwise be interpreted as XML because of the tags.

6.197	charging [b-ITU-T J.363], [b-ITU-T J.460.3]: The process of applying rating to usage data for a given session for the generation of a subscriber's bill.

6.198	chip [b-ITU-T J.122]: Each of the 128 bits comprising the S-CDMA spreading codes.

6.199	chip duration [b-ITU-T J.122]: The time to transmit one chip of the S-CDMA spreading code. The inverse of the chip rate.

6.200	chip rate [b-ITU-T J.122]: The rate at which individual chips of the S-CDMA spreading codes are transmitted (1280 to 5120 kHz).

6.201	   Chrominance-luminance delay inequality [b-ITU-T J.64]: The chrominance-luminance delay inequality is defined as the time difference (expressed in ns) between the luminance and the chrominance component of the composite pulse (element F). This difference is positive, if the symmetry axis of the demodulated chrominance component lags behind the symmetry axis of the luminance component.

6.202	   Chrominance-luminance gain inequality [b-ITU-T J.64]: The chrominance-luminance gain inequality is defined as the difference between the peak-to-peak amplitude of the chrominance component of the element G, G1, G2 and the amplitude of the luminance bar (element B2) expressed as a percentage of the luminance bar amplitude. The sign of the difference is positive if the amplitude of the chrominance component is greater than that of the luminance bar. Note that in the 525-line case the nominal amplitude of element G is 80 IRE units. This factor must be taken into account when normalizing results.

If for any reason signal elements G, G1 or G2 are not available, the measurement can be made with the chrominance component of element F.

6.203		Chrominance-luminance intermodulation [b-ITU-T J.64]: The chrominance-luminance intermodulation is measured on element G, G1 or G2, after suppressing the incoming colour sub-carrier. It is defined as the difference between the luminance amplitude in element G1, or in the last section of element G or G2 (b5 in Figs. 3 and 4 of ITU-T J.64) and the amplitude of the succeeding section (b6 in Figs. 3 and 4 of ITU-T J.64) in which the test signal has no sub-carrier, expressed as a percentage of the amplitude of the luminance bar (element B2). The sign of the difference is positive if the luminance amplitude b5 is greater than the luminance amplitude of the succeeding section b6.

NOTE: Some administrations use element F instead of G, G1 or G2 for measurement of this parameter. In this case measurement of the amplitude of the luminance component of the composite pulse (element F) is made after suppressing the incoming colour sub-carrier. The result will be given by the difference between the composite pulse luminance amplitude and half the luminance bar amplitude, expressed as a percentage of the luminance bar amplitude. The sign of the difference is positive if the amplitude of the composite pulse component is greater than half the luminance bar amplitude. In some cases the result may differ from that given by the preferred method, since the signal element F is not so well suited as element G to the measurement of this distortion.

6.204	  Chrominance reference amplitude error [b-ITU-T J.64]: This parameter relates to the variation in amplitude of the colour sub-carrier occurring in the region of blanking level. It is defined as the difference between the peak-to-peak amplitude of the colour sub-carrier on the blanking level tread of element D2 and its normalized value, (i.e. 4/10 luminance bar amplitude) (see § 2.18 of J.64, Note 1), expressed as a percentage. The sign of the difference is positive if the amplitude of the colour sub-carrier on the blanking level tread is larger than the normalized value.

6.205	chunk [b-ITU-T J.123], [b-ITU-T J.124]: A contiguous set of samples for one track.

6.206	cipher [b-ITU-T J.170]: An algorithm that transforms data between plaintext and ciphertext.

6.207	cipher block chaining (CBC) [b-ITU-T J.181 Amendment 1]: This is a specific method of encryption. It is one of the methods used in DES.

6.208	ciphersuite [b-ITU-T J.170]: A set, which must contain both an encryption algorithm and a message authentication algorithm (e.g., a MAC or an HMAC). In general, it may also contain a key management algorithm, which does not apply in the context of IPCablecom.

6.209	classifier [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: A set of criteria used for packet matching according to TCP, UDP, IP, LLC, and/or 802.1P/Q packet fields. A classifier maps each packet to a service flow. A downstream classifier is used by the CMTS to assign packets to downstream service flows. An upstream classifier is used by the CM to assign packets to upstream service flows.

6.210	client device [b-ITU-T J.215]: The CPE that is connected to the cable network in the consumer's home, receives the television signals at the client's premises, and presents it for display on a display device.

6.211	client DVR (cDVR) [b-ITU-T J.700]: An instance of a DVR where the end-user terminal device contains the recording capability that can be solicited and operated by end users to record and store video, audio and other associated content locally for subsequent playback.

6.212	client type 1 [b-ITU-T J.179]: Client type 1 represents existing "legacy" endpoints (e.g., PC applications, gaming consoles) which lack specific QoS awareness or signalling capabilities. This Client knows nothing about CableModem, IPCable2Home, or IPCablecom messaging, and hence no related requirements can be placed upon it. Such clients may range from simple analog audio and video presentation devices to complex networked peripherals and consumer electronics, such as set-top boxes or gaming consoles. This Client communicates with an Application Manager to request service, and does not request QoS resources directly from the operator access network. This Recommendation supports only client type 1.

6.213	client type 2 [b-ITU-T J.179]: Client type 2 is similar to an IPCablecom-T telephony MTA in that it supports QoS signalling based on IPCablecom DQoS. This Client is aware of IPCablecom Multimedia QoS, and communicates with an Application Manager to request service and obtain a token for access-network resources. The client then presents this token when requesting QoS resources from the access network (pkt-mm-1, pkt-mm-6). This Recommendation's support for client type 2 remains for further study.

6.214	client type 3 [b-ITU-T J.179]: Client type 3 requests QoS based on RSVP without Application Manager interaction. This Client is aware of IETF standards-based RSVP and uses this protocol to request QoS resources from the access network directly from the CMTS. This Recommendation's support for client type 3 remains for further study.

6.215	clock recovery unit (CRU) [b-ITU-T J.214]: A clock recovery unit exists in the IWF and is responsible for regenerating the circuits clock based on the average inter-arrival time of the packets in the adaptive clocking mode, or on the time stamp differentials received when operating in the differential clock mode. The output clock is provided to the CU.

6.216	clock unit (CU) [b-ITU-T J.214]: A clock unit performs translations and distribution of TDM clocking information across (and between) physical, data and network layers.

6.217	closed captioning [b-ITU-T J.193]: Text scrolling on a television display that represents the audio portion of the program, typically provided for the hearing impaired.

6.218	cloud terminal [b-ITU-T J.295]: A terminal that has at least a client function, browser and Internet connectivity for cloud network connection.

6.219	CMC [b-ITU-T J.223.1]: The cable media converter (CMC) converts data from a coaxial cable network to a packet digital optical network (such as PON or Ethernet). The CMC connects to a cable modem (CM) through the coaxial cable network in the downstream direction and to the CMC controller through the packet digital optical network in the upstream direction.

6.220	CMC controller [b-ITU-T J.223.1]: The CMC controller forwards upstream and downstream service data and manages the configuration of the CMC.

6.221	code-hopping matrix [b-ITU-T J.122]: A shifted version of the reference code matrix (see below) that is used when code hopping is employed to vary the codes used by each CM. The code-hopping matrix is either 128 rows by 128 columns (when all 128 codes are active) or is 127 rows by 128 columns (when less than 128 codes are active in the S-CDMA spreader-on frame). When less than 128 codes are active, code 0 (all ones) is deleted from the matrix, but all remaining codes are still cycled through even if less than 127 codes are active in a frame.

6.222	coding mode [b-ITU-T J.88]: Mix A, Mix B and refresh (all intra) mode

6.223	colour sub-carrier phase shift [b-ITU-T J.88]: Phase shift of 3.58 MHz colour sub-carrier of the coding block from the motion compensated reference block in the previous frame

6.224	command [b-ITU-T J.287]: A single directive from the automation system to the compression system. A command is always carried within a multiple_operation message. This term is also used to specify specific ITU-T J.181 commands.

6.225	command and control messaging [b-ITU-T J.193]: Messaging between devices which request particular actions associated with video service such as Play, Rewind, and Pause.

6.226	common agent service [b-ITU-T J.230]: One of the particular resident applications which is specific to the mobile-set top box (STB) cooperation. Common agent service equips the common application program interfaces (APIs) to enable each application to utilize its mobile-STB cooperation function. Therefore, many application developers can realize the mobile-STB cooperation application easily and quickly.

6.227	CompareSwap4 [b-ITU-T J.117]: A bus transaction that stores, at the specified address, a provided data value when the contents of the specified address is equal to a provided argument value. This operation is performed indivisibly on the addressed quadlet.

6.228	compliance rules [b-ITU-T J.197]: The rules which apply to set top boxes for the purpose of preventing the unauthorized copying of controlled content.

6.229	compliant CM [b-ITU-T J.213]: A CM that implements this DOCSIS L2VPN Recommendation.

6.230	component (ELEMENTARY Stream) [b-ITU-T J.94]: One or more entities which together make up an event, e.g., video, audio, teletext.

6.231	component splice mode [b-ITU-T J.181 Amendment 1]: A mode of the Cueing Message whereby the program_splice_flag is set to '0' and indicates that each PID/component that is intended to be spliced will be listed separately by the syntax that follows. Components not listed in the Message are not be spliced.

6.231bis 	component splice mode [b-ITU-T J.287]:A mode of the splice_info_section whereby the program_splice_flag is set to '0' and indicates that each PID/component that is intended to be spliced will be listed separately by the syntax that follows. Components not listed in the splice_info_section are not to be spliced.

6.232	composite motion compensation [b-ITU-T J.88]: Motion Compensation (MC) is conducted on WHT domain not on pixel domain to solve colour sub-carrier phase shift

6.233	Composite Second Order beat (CSO) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: The peak of the average level of distortion products due to second-order non-linearities in cable system equipment.

6.234	Composite Triple Beat (CTB) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: The peak of the average level of distortion components due to third-order non-linearities in cable system equipment.

6.235	conditional access [b-ITU-T J.193]: The conditional granting of access to cable services and content based upon what service suite has been purchased by the customer.

6.236	conditional access (CA) system [b-ITU-T J.94]: A system to control subscriber access to services, programmes and events e.g., Videoguard, Eurocrypt.

6.236bis 	Conditional Access system (CA) [b-ITU-T J.93], [b-ITU-T J.95]: The complete system for ensuring that cable services are accessible only to those who are entitled to receive them, and that the ordering of such services is not subject to modification or repudiation.

6.237	conditional access client software (CACS) [b-ITU-T J.1001]: An image of conditional access client software code downloaded onto the CACS remotely renewable security system.

6.237bis conditional access client software (CACS) [b-ITU-T J.1002] An image of conditional access client software code downloaded onto the CRS CAM.

6.238	conditional access module (CAM) [b-ITU-T J.1001]: A cryptographic functional module which is located in a set-top box with the main functions of entitlement validation, key management, and authentication. A set-top box can have one chip of secure hardware that includes the functions of CAM and descrambler, or a physically separated CAM in the form of a secure hardware IC or smart-card. The form of CAM can be determined by the policy of the multiple system operator or the conditional access client software vendor.

6.238bis 	conditional access module (CAM) [b-ITU-T J.1002]: A cryptographic functional module which is located in set-top boxes, whose main function is entitlement validation, key management and authentication. Set-top boxes can have one chip of secure hardware that includes the functions of CAM and descrambler, or physically separated CAM in the form of a secure hardware IC or smart-card. The form of CAM can be determined by the policy of the MSO or CAS vendor.

6.239	confidentiality [b-ITU-T J.170]: A way to ensure that information is not disclosed to any one other than the intended parties. Information is encrypted to provide confidentiality. Also known as privacy.

6.240	configuration [b-ITU-T J.369], [b-ITU-T J.460.4]: Configuration is the process of defining and propagating data to network elements for providing services.

6.241	configuration server (Config. Server) [b-ITU-T J.460.1]: The logical network element responsible for UE provisioning, configuration and management.

6.242	consensus watermark [b-ITU-T J.197]: A standard watermark that has been developed for use in DRM.

6.243	constant frame skipping [b-ITU-T J.246], [b-ITU-T J.247]: An event where the HRC outputs frames with updated content at an effective frame rate that is fixed and less than the source frame rate.

6.244	constellation mapping [b-ITU-T J.195.1]: The process of mapping data bits to the constellation symbol.

6.245	constellation scrambler [b-ITU-T J.196.2]: The process that takes phase rotation of the constellation symbols in 4 quadrants by using binary pseudo random sequence.

6.246	constrained image [b-ITU-T J.197]: The visual equivalent of not more than 520 000 pixels per frame (e.g., an image with a resolution of 540 vertical lines by 960 horizontal lines for a 16:9 aspect ratio). A constrained image can be output or displayed using video processing techniques such as line doubling or sharpening to improve the perceived quality of the image.

6.247	constrained image trigger (CIT) [b-ITU-T J.197]: The field or bits used to trigger the output of a "constrained image" in the high definition analogue output of set top boxes.

6.248	consumer [b-ITU-T J.117]: A device that accepts OSD data.

6.249	consumer port [b-ITU-T J.117]: A port that is the sink of data frames and is flow controlled by updates of its externally visible iAPR control register.

6.250	consumer premises equipment provider [b-ITU-T J.90]: The entity that supplies the television/multimedia equipment at the consumer premises (e.g. the equipment manufacturer).

6.251	contact address [b-ITU-T J.360]: The URI of a user agent on the network. Contact addresses, in the context of IPCablecom are often, but not always, addresses used to deliver requests to a specific user agent.

6.252	container box [b-ITU-T J.123], [b-ITU-T J.124]: A box whose sole purpose is to contain and group a set of related boxes.

6.253	content [b-ITU-T J.181]: Generic term for television material, either advertisements or programs.

6.253bis content [b-ITU-T J.200]: A general term that refers to any of the following: application, application resource collection, or application resource.

6.253ter content [b-ITU-T J.295]: Information content transmitted via various media through the communication network, including the broadcasting network and IP.

6.253quater content [b-ITU-T J.380.2]: The video, audio, and data streams taken together as a single identifiable unit. Content may refer to the original entertainment (programming) content, an ad spot, an interactive or enhanced application asset, or any other similar asset.

6.254	content distribution network (CDN) [b-ITU-T J.292]: CDN contains a Core network and Access network where content delivery is controlled by identified packet routing and a QoS oriented mechanism.

6.255	content key [b-ITU-T J.1005]: A key used to protect the cable content data stream(s).

6.256	content protection [b-ITU-T J.197]: The application of technical safeguards that prevent the unauthorized replication and/or redistribution of network delivered content.

6.256bis		content protection [b-ITU-T J.702]: Ensuring that end-users can only use the content they have already acquired in accordance with the rights that they have been granted by the rights holder. Content protection includes protecting contents from illegal copying and distribution, interception, tampering, unauthorized use, etc.

6.257	content provider [b-ITU-T J.90]: The entity that provides the creative content of a programme (e.g. the programme producer or the owner of its rights).

6.257bis content provider [b-ITU-T J.702]: The entity that owns or is licensed to sell content or content assets.

6.258	content tracing [b-ITU-T J.702]: A process to enable the identification of the (arbitrary) origin of content, and/or the responsible party (e.g., the end-user), to facilitate subsequent investigation in the event of unauthorized use of content, for example, content copying or redistribution.

NOTE: Content tracing information may be attached to content either as metadata, or as a forensic watermark.

6.259	Continuous random noise [b-ITU-T J.67]: The signal-to-noise ratio for continuous random noise is defined as the ratio, expressed in decibels, of the nominal amplitude of the luminance signal (1 V) to the r.m.s. amplitude of the noise measured after band limiting. A signal-to-weighted-noise ratio is defined as a ratio, expressed in decibels, of the nominal amplitude of the luminance signal, to the r.m.s. amplitude of the noise measured after band limiting and weighting with a specified network.

One possibility is that wideband random noise should be measured in a bandwidth of 8.4 MHz using a constant impedance noise-weighting network with a time constant of 90 ns. Such a network is based partly on the assumption that with the trend towards larger picture displays and with the improved picture quality available from the MAC/packet television standard, future subjective tests will more commonly employ a viewing distance of four times the picture height, rather than six times, as at present.

The second possibility uses the existing unified weighting network, scaled according to the 3:2 compression ratio, as a common weighting network for all MAC systems. This filter gives the same results as would be obtained from a signal in decompressed form with the unified weighting filter described in ITU T J.612. It also takes account of the noise carried in the more-compressed colour-difference signals. The possibly greater noise sensitivity due to the higher bandwidth HD MAC signals when these use the same networks that are designed for present day MAC signals is also considered. The definition of this network and its amplitude/frequency response are given in Figure 1 of J.67.

6.260	control [b-ITU-T J.287]: A category of request operation supported by this API. See clause 8.3 of J.287.

6.261	control frame [b-ITU-T J.195.2]: Frame of the MAC layer used for access control and channel allocation.

6.262	control point [b-ITU-T J.362]: Within the context of this Recommendation, control point refers to a point in the network that can be used to apply a function for a media flow that flows through that point. Functions described here are: QoS (IPCablecom multimedia [b-ITU-T J.179] or IPCablecom DQoS [b-ITU-T J.163]). Replication, encapsulation and transmission for the purposes of LI content tapping.

6.263	control point discovery [b-ITU-T J.362]: The act of discovering information (IP address, protocol) concerning a control point in order to allow a requestor to apply a specific controlling function.

6.264	control word [b-ITU-T J.287]: A multiple key value used by the encryption mechanisms specified in [ITU-T J.181].

6.264bis control word (CW) [b-ITU-T J.1001]: The value which is used to scramble and descramble transport streams.

NOTE: The control word should be refreshed frequently during service operation to enhance security.

6.264ter control word (CW) [b-ITU-T J.1002]: The value which is used to scramble and descramble transport streams; it is refreshed frequently during the service operation to enhance security.

6.265	controlled content [b-ITU-T J.197]: Content that has been transmitted from a video service provider's network with the encryption mode indicator (EMI) bits set to a value other than zero, zero (0,0) ("copying not restricted").

6.266	converged interconnect network [b-ITU-T J.212]: The network (generally gigabit Ethernet) that connects an M-CMTS Core to an EQAM.

6.267	conversational television programme [b-ITU-T J.146]: A television programme that contains live conversation contributions shot at different locations, linked together at a base location.

6.268	copy control [b-ITU-T J.296]: A control copy generation that limits copying when the programme and other copyright objects are copied by the recording equipment connected with the broadcasting receiver.

6.269	copy control information (CCI) [b-ITU-T J.197]: A one-byte field that contains information that set top boxes use to control copying of content. See Annex A of [b-ITU-T J.197] for further details.

6.270	core services [b-ITU-T J.193]: The set of services that MUST, at a minimum, be supported by the NG-STB.

6.271	cost [b-ITU-T J.283]: A cost is a parameter configured by an operator in order to make a network resource utilization effective. Example definition is described in [b-IETF RFC 2328].

6.272	CPE controlled cable modem (CCCM) [b-ITU-T J.122]: Refer to the DOCS cable modem to customer premises equipment interface (CMCI) specification.

6.272bis CPE controlled cable modem (CCCM) [b-ITU-T J.112 Annex B]: Refer to the DOCSIS Cable Modem to Customer Premises Equipment Interface (CMCI) specification.

6.273	criticality [b-ITU-T J.88]: The reference for the difficulty of the picture judged in macro block units

6.274	cross platform [b-ITU-T J.295]: A coordination service between platforms.

6.275	cross-modulation [b-ITU-T J.112], [b- ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A form of television signal distortion where modulation from one or more television channels is imposed on another channel or channels.

6.276	cryptanalysis [b-ITU-T J.93], [b-ITU-T J.95]: The science of recovering the plaintext of a message without access to the key (to the electronic key in electronic cryptographic systems).

6.276bis cryptanalysis [b-ITU-T J.170]: The process of recovering the plaintext of a message or the encryption key without access to the key.

6.277	cryptographic duty cycle [b-ITU-T J.93], [b-ITU-T J.95]: The maximum secure capacity of a cryptographic process, based on the total number of bits that can be securely encrypted before it becomes advisable to change the key.

6.278	cueing message [b-ITU-T J.181 Amendment 1], [b-ITU-T J.287]: In the context of this appendix, a message is the contents of any splice_info_section.

6.279	cursor [b-ITU-T J.380.8]: A temporary construct containing static data. Consumers of logical services implementing the GIS interface may create and access cursor information using the standard query mechanisms described in this document.

6.280	customer [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C]: see definition for end user (clause 6.437).

6.281	customer premises equipment [b-ITU-T J.460.2]: Usage of CPE within this Recommendation generically refers to the cable modem and E-DVA device that resides at the subscriber home, as well as any customer telephony equipment (telephones, answering machines, fax machines, etc.). Typically, CPE would refer to equipment that is beyond the service provider network interface, such as a telephone or personal computer. However, since the cable modem and E-DVA represent the service provider network interface device at the subscriber home, it is commonly referred to as CPE.

6.281bis Customer Premises Equipment (CPE) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.210], [b-ITU-T J.212]: Equipment at the end user's premises; it may be provided by the end user or by the service provider.

6.281ter customer premises equipment (CPE) [b-ITU-T J.292]: CPE covers subscriber video devices (SVD), residential gateway (RGW), and optional in home networking devices.

6.282	customer-facing interface [b-ITU-T J.218]: An eRouter interface used for connecting CPE devices.

6.283	customer-facing IP interface [b-ITU-T J.218]: An IP interface connected to the eRouter which is not necessarily mapped one-to-one with the number of customer-facing ports on the eRouter.

6.284	customer-facing logical IP interface [b-ITU-T J.218]: A logical interface connected to the eRouter which is not necessarily mapped one-to-one with the number of customer-facing ports on the eRouter.

6.285	cyclic prefix [b-ITU-T J.195.2]: Data located at the front of an OFDM symbol, which is a copy of the data from the end of the OFDM symbol.

6.286	cyclic redundancy check [b-ITU-T J.112]: A method of error detection using cyclic code.

6.286bis cyclic redundancy check (CRC) [b-ITU-T J.181 Amendment 1]: A method to verify the integrity of a transmitted Message.

6.287	data frame (frame) [b-ITU-T J.117]:A contiguous group of data bytes sent between producer and consumer nodes.

6.287bis data frame [b-ITU-T J.195.2]: Frame of the MAC layer used to carry data of the upper layer.

6.288	data link layer [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: Layer 2 in the Open Systems Interconnection (OSI) architecture; the layer that provides services to transfer data over the transmission link between open systems.

6.289	data message  [b-ITU-T J.204]: A message transmitted between an IPDR Exporter and Collector across the Streaming Protocol, containing a common Streaming Protocol header and an optional payload consisting of control and Data Records.

6.290	data model [b-ITU-T J.369], [b-ITU-T J.460.4]: An abstract model that describes representation of data in a system.

6.290bis	 data model [b-ITU-T J.380.8]: A data model is a formal view of the data items contained in an information store to which an information service implementing this standard will provide access and is specified for purposes of formulating and executing queries against the information store's data. This standard specifies one data model that may be used for querying a logical service's information store with this standard's "Basic Query" interface. More complex data models may be specified independently of this standard and may be queried with this standard's "Advanced Query" interface. In this latter case, the mechanisms by which a consumer incorporates a data model specification so that meaningful queries may be issued against it are outside the scope of this standard.

6.291	data rate [b-ITU-T J.212]: Throughput, data transmitted in units of time usually in bits per second (bit/s).

6.292	data record  [b-ITU-T J.204]: The binary encoding of an IPDR record.

6.293	data segment (segment) [b-ITU-T J.117]: A largest portion of a data frame that can be written into the segment buffer before updating the consumer's iAPR register.

6.294	data service unit [b-ITU-T J.214]: The part of the CSU/DSU that interfaces with routers, switches and packets. It has a serial port to interface with compatible data equipment.

6.295	Data signal waveform [b-ITU-T J.67]: The data signals have very different characteristics within the family of MAC systems. They are defined in the former CCIR special publication "Specifications of Transmission Systems for the Broadcasting-Satellite Service".

6.296	Data timing [b-ITU-T J.101]: In teletext System B the data timing is defined as the time difference between the peak of the penultimate “1” run-in bit and the line time datum (see Report 624). In teletex System A the data timing is defined as the time difference between the leading edge of the data signal and the line time datum (see more details in Fig. 2 of J.101).

6.297	Data-Over-Cable Service Interface Specification (DOCSIS) [b-ITU-T J.296]: The cable modem standard that was established by the Multimedia Cable Network System Partners, Ltd. (MCNS) in 1997. This term is generally used as a generic term for a cable modem and is implemented in [ITU-T J.122] or [ITU-T J.112].

6.298	dBm0 [b-ITU-T J.14]: The absolute signal power level, in decibels, referred to a point of zero relative level. [This symbol traditionally relates to telephony relative levels.]

6.299	dBm0s [b-ITU-T J.14]: The absolute signal power level, in decibels, referred to a point of zero relative sound-programme level.

6.300	dBq0ps [b-ITU-T J.16]: Weighted noise level, measured with a quasi-peak measuring instrument complying with CCIR Recommendation 468  and referred to a point of zero relative sound-programme level

6.301	dBq0s [b-ITU-T J.16]: Unweighted noise level, measured with a quasi-peak measuring instrument complying with CCIR Recommendation 468 and referred to a point of zero relative sound-programme level

6.302	dBr [b-ITU-T J.14]: The relative power level, in decibels.[These symbols traditionally relate to telephony relative levels.]

6.303	dBrs [b-ITU-T J.14]: The relative (power) level, in decibels, with respect to sound-programme signals. (This abbreviation is only applicable at points in a sound-programme circuit where the signals can nominally be related to the input by a simple scaling factor.)

NOTE: The use of level definitions is given in CCIR Recommendation 574.

6.304	decibel-microvolt (dBμV) [b-ITU-T J.210]: Unit of RF power expressed in decibels relative to 1 microvolt over 75 ohms, where dBμV = 20log10 (value in μV/1 μV).

6.305	decibel-millivolt (dBmV) [b-ITU-T J.210], [b-ITU-T J.212]: Unit of RF power expressed in decibels relative to 1 millivolt, where dBmV = 20log10(value in mV/1 mV).

6.306	decibels (dB) [b-ITU-T J.210], [b-ITU-T J.212]: Ratio of two power levels expressed mathematically as dB = 10log10(POUT/PIN).

6.307	declarative application [b-ITU-T J.200]: An application which primarily makes use of declarative information to express its behaviour; an XML document instance is an example of a declarative application.

6.308	declarative application environment [b-ITU-T J.200]: An environment that supports the processing of declarative applications; an XML user agent (browser) is an example of a declarative application environment.

6.308bis	 declarative application environment [b-ITU-T J.296]: An application environment using HTML.

6.309	Decoding margin [b-ITU-T J.67]: Another method to evaluate the data signal quality by adding a Gaussian noise is to measure the level of added noise to obtain a given bit-error ratio. This is, by definition, the "decoding margin".

6.309bis Decoding margin [b-ITU-T J.101]: The decoding margin is defined as the difference between the highest “0” bit level and the lowest “1” bit level measured at the sampling instants for a bit error ratio of 10–3. The difference is expressed as percentage of the basic amplitude.

6.310	decoding time-stamp [b-ITU-T J.187], [b-ITU-T J.89]: A field that may be present in a PES packet header that indicates the time that an access unit is decoded in the system target decoder.

6.311	decryption function [b-ITU-T J.96]: refers to a logical function used to decrypt the Encrypted Session Words, with the help of a key.

6.312	default data model [b-ITU-T J.380.8]: The data model that is used when no specific data model is provided. The default data model typically occupies the first location in a data model sequence (i.e., a list of data models).

6.313	default endpoint [b-ITU-T J.380.2]: The endpoint where messages are delivered in the absence of a message specific endpoint designation.

6.314	deferred processing mode [b-ITU-T J.287]: Processing of a multiple operation message(), when the value of time type within timestamp() is non-zero.

6.315	delay [b-ITU-T J.161]: The absolute time required for a signal to transit from source to receiver.

6.316	delivery network gateway functions (DNGF) [b-ITU-T J.702]: Set of functions that mediate between the network and service provider domains and the IPTV terminal function (ITF).

NOTE: A device implementing the DNGF is commonly referred to as the residential gateway (RG) or delivery network gateway (DNG).

6.317	delivery system [b-ITU-T J.94]: The physical medium by which one or more signal multiplexes are transmitted, e.g. satellite transponder, wide-band coaxial cable, fibre optics.

6.318	demodulator module [b-ITU-T J.222.1]: A physical entity in the CM that demodulates a block of one or more contiguous channels of a single bandwidth (6 MHz or 8 MHz) within the output from a single tuner.

6.319	depth [b-ITU-T J.902]: Distance from the capturing camera to a surface of an object in the scene.

6.320	depth map [b-ITU-T J.901]: Distance from the capturing camera to a surface of an object in the scene measured per each pixel on the captured image.

6.321	DER Encoded [b-ITU-T J.222.3]: Refers to a value which is encoded using the ASN.1 Distinguished Encoding Rules [b-ITU-T X.690].

6.322	descrambling [b-ITU-T J.91]: is defined as the restoration of the characteristics of a vision/sound/data signal in order to allow reception in a clear form. This restoration is a specified process under the control of the conditional access system (receiving end).

6.322bis descrambling [b-ITU-T J.93], [b-ITU-T J.95]: The process of reversing the scrambling function (see Scrambling) to yield usable pictures, sound, and data services.

6.323	descriptor [b-ITU-T J.94]: A data structure of the format: descriptor_tag, descriptor_length, and a variable amount of data. The tag and length fields are each 8 bits. The length specifies the length of data that begins immediately following the descriptor_length field itself. A descriptor whose descriptor_tag identifies a type not recognized by a particular decoder shall be ignored by that decoder. Descriptors can be included in certain specified places within PSIP tables, subject to certain restrictions. Descriptors may be used to extend data represented as fixed fields within the tables. They make the protocol very flexible since they can be included only as needed. New descriptor types can be standardized and included without affecting receivers that have not been designed to recognize and process the new types.

6.324	detach [b-ITU-T J.117]: The process of removing access to an asynchronous connection plug's address space.

6.325	DIAMETER [b-ITU-T J.363], [b-ITU-T J.460.3]: The DIAMETER protocol provides an Authentication, Authorization and Accounting (AAA) framework for applications such as network access or IP mobility.

6.326  Differential phase [b-ITU-T J.64]: Differential phase is determined by evaluating the phase modulation of the colour sub-carrier superimposed on the staircase in element D2.

Recommendation 567 defines differential phase in terms of two parameters x and –y which represent the maximum (peak) differences in phase between the sub-carrier on the treads of the received test signal and the sub-carrier on its blanking level, expressed in degrees difference from the latter. In the case of a monotonic characteristic either x or y will be zero.

x and y can be found from the expressions below:

where:

0 :	phase of sub-carrier on the blanking level tread of element D2.

max :	highest value of sub-carrier phase on any tread.

min :	lowest value of sub-carrier phase on any tread.

Two alternative methods of expressing the results are acceptable for automatic measurement. These are:

(a)	“peak differential phase”, which is defined by either x or –y depending upon which of these parameters has the larger magnitude.

(b)	“peak-to-peak differential phase”, which is defined as x    y.

6.327	  Differential gain [b-ITU-T J.64]: Differential gain is determined by evaluating the amplitude modulation of the colour sub-carrier superimposed on the staircase in element D2.

Recommendation 567 defines differential gain in terms of two parameters x and –y which represent the maximum (peak) differences in amplitude between the sub-carrier on the treads of the received test signal and the sub-carrier on its blanking level, expressed as a percentage of the latter. In the case of a monotonic characteristic, either x or y will be zero.

x and y can be found from the expressions below:

where:

A0 :	amplitude of the received sub-carrier on the blanking level tread of element D2.

Amax :	highest value of sub-carrier on any tread.

Amin :	lowest value of sub-carrier on any tread.

Two alternative methods of expressing the results are acceptable for automatic measurement. These are:

(a)	“peak differential gain”, which is defined by either x or –y, depending upon which of these parameters has the larger magnitude.

(b)	“peak-to-peak differential gain”, which is defined as x    y.

NOTE: For the measurement of peak-to-peak differential gain some administrations use Amax rather than A0. The formula used then is:

Results obtained by this method will differ only slightly from those defined above if the magnitude of the distortion is not excessive.

6.328	digital channel [b-ITU-T J.94]: A set of one or more digital elementary streams. See virtual channel.

6.329	digital cue tone [b-ITU-T J.287]: Widely used term to refer to an ITU-T J.181 splice_info_section().

6.330	digital program insertion (DPI) [b-ITU-T J.215]: Insertion of alternative content into digitally encoded content in response to messaging in the stream.

6.331	digital rights management (DRM) [b-ITU-T J.197], [b-ITU-T J.294]: The definition, management, and enforcement of a set of content usage rules. These usage rules will indicate things such as the right to copy, view, or distribute a particular piece of content.

6.331bis 	digital rights management (DRM) [b-ITU-T J.295]:Digital rights management is for the protection of digital content.

NOTE: In these requirements, use within a home network is assumed.

6.332	digital signal 0 (DS0) [b-ITU-T J.214]: Digital signal 0 (DS0) is a basic digital signalling rate of 64 kbit/s, corresponding to the capacity of one voice-frequency-equivalent channel.

6.333	digital storage media command and control (DSM-CC) [b-ITU-T J.200]: A control method defined in [b-ISO/IEC 13818-6], which provides access to files and streams for digital interactive services.

6.333bis Digital storage media command and control (DSM-CC) [b-ITU-T J.291]: DSM-CC is for developing control channels associated with MPEG-2 streams.

6.334	Digital Television (DTV) [b-ITU-T J.117]: A device that receives, decodes and presents audio and video material that has been transmitted in a compressed form.  The device can be a single unit or it can be constructed from a number of individual components (e.g. a digital terrestrial set-top box and an analog television).

6.335	digital transmission content protection (DTCP) [b-ITU-T J.197]: The method of encryption, decryption, key exchange and renewability that is described in the specification entitled "5C digital transmission content protection release 1.0".

6.336	digital video recorder (DVR) [b-ITU-T J.700]: A recording capability that can be solicited and operated by end users to record and store video, audio and other associated content for subsequent playback.

6.337	Distortions due to echoes [b-ITU-T J.67]: This distortion is that caused by the superposition of the direct signal in the RF paths and an attenuated version of that signal delayed in time and shifted in phase relative to the direct signal.

6.338	distributed DVR (dDVR) [b-ITU-T J.700]: Multiple instances of a DVR where a combination of cDVRs and nDVRs can be used to record and store video, audio and other associated content for subsequent playback. For example, this usually occurs within a home network containing multiple cDVRs in order to distribute storage of video, audio and other content.

6.339	distribution hub [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: A location in a cable television network which performs the functions of a Headend for customers in its immediate area, and which receives some or all of its television program material from a Master Headend in the same metropolitan or regional area.

6.340	DM_ISS [b-ITU-T J.1020]: One of the roles for DM system. It has the role of personalization of DM agent installed in a user secondary device.

6.341	DM_MSS [b-ITU-T J.1020]: One of roles for DM system, and has a role of managing on-line or off-line download.

6.342	DOCS 1.0 [b-ITU-T J.122]: Data-over-cable system defined by [b-SCTE 22-1].

6.343	DOCS 1.1 [b-ITU-T J.122]: Data-over cable system defined by [b-ITU-T J.112 Annex B], (also see 6.346bis).

6.344	DOCS 2.0 [b-ITU-T J.122]: Data-over-cable system as defined in this Recommendation.

6.345	DOCSIS [b-ITU-T J.112 Annex B]: A generic term for a system or device compliant with any one of the Data Over Cable Service Interface Specification versions, namely DOCSIS 1.0, DOCSIS 1.1, DOCSIS 2.0.

6.345bis	 DOCSIS [b-ITU-T J.125], [b-ITU-T J.126]: The term for a system or device compliant with any one of the Cable Television Laboratories, Inc. ("CableLabs") series of specifications located at: http://www.cablemodem.com/specifications/.

6.345ter	 DOCSIS [b-ITU-T J.179]: Describes a specific CableModem technology as developed by Cable Television Laboratories, Inc. ("CableLabs") located at: http://www.cablemodem.com/specifications/. The international version is defined in Annex B/J.112.

6.345quater DOCSIS [b-ITU-T J.368]: A set of interface specifications for transmitting data over cable television systems in a standard fashion.

6.346	DOCSIS 1.0 [b-ITU-T J.112 Annex B], [b-ITU-T J.125]: A system or device compliant with the following Data Over Cable Service Interface Specifications [SCTE22-1], [SCTE22-2], [SCTE22-3], [DOCSIS4].

6. 347  DOCSIS 1.1 [b-ITU-T J.125]: A system or device compliant with the following Data Over Cable Service Interface Specifications: [ITU-T J.112-B], [SCTE23-3], [DOCSIS4] and this Recommendation.

6.347bis	DOCSIS 1.1 [b- ITU-T J.112 Annex B]: A system or device compliant with the following Data Over Cable Service Interface Specifications [J.125], [SCTE4], [DOCSIS4], and Annex B.

6.348	  DOCSIS 2.0 [b-ITU-T J.112 Annex B]: A system or device compliant with the following Data Over Cable Service Interface Specifications [J.122], [J.125], [SCTE5], [DOCSIS4].

6.348bis DOCSIS 2.0 [b-ITU-T J.125]: A system or device compliant with the following Data Over Cable Service Interface Specifications [ITU-T J.122], [SCTE79-2], [DOCSIS4] and this Recommendation.

6.349	DOCSIS flow [b-ITU-T J.163]: A unidirectional or bidirectional flow of data packets that is subject to MAC layer signalling and QoS assignment compliant to [ITU-T J.112] (or ITU-T Rec. J.122).

6.350	DOCSIS L2PDU [b-ITU-T J.213]: A Packet PDU of a DOCSIS MAC Frame, i.e., the L2PDU following a MAC Header with FC_TYPE=00. This definition means that a MAC Management message with FC_TYPE=11 is not considered to be a DOCSIS L2PDU, even though the form of a MAC Management Message Header is the same form as an L2PDU.

6.351	DOCSIS MAC frame [b-ITU-T J.213]: The unit of transmission on the DOCSIS cable RF interface, consisting of a MAC Header and a (possibly null) Data PDU. The FC_TYPE field of MAC Header identifies the Data PDU as either a Packet PDU (FC_TYPE=00), or a MAC-specific PDU (FC_TYPE=11).

6.352	DOCSIS Set-Top Gateway (DSG) [b-ITU-T J.128]: The DOCSIS Set-top Gateway (DSG) defines functionality on a DOCSIS CMTS and DOCSIS CM to support the configuration and transport of a class of service known as "Out-Of-Band (OOB) messaging" between a Set-top Controller (or application servers) and the customer premises equipment (CPE). The DSG is not intended for the delivery of programming content.

6.352bis 	DSG  [b-ITU-T J.291]: The DOCSIS Set-top Gateway is an interface defining signalling to and from a DOCSIS modem embedded in a cable set-top device.

6.353	DOCSIS signalling [b-ITU-T J.291]: DOCSIS signalling as defined in ITU-T Recs J.125 and J.126. DOCSIS is the term for a system or device compliant with any one of the Cable Television Laboratories, Inc. ("CableLabs") series of specifications located at: http://www.cablemodem.com/specifications/.

6.354	DOCSIS-based CPE [b-ITU-T J.700]: A terminal device that contains an embedded DOCSIS cable modem. Hybrid CPEs and IP-only CPEs may be DOCSIS based.

6.355	document object model (DOM) [b-ITU-T J.200]: An API that defines the logical structure of XML [b-W3C XML] and HTML [b-W3C HTML] documents and the way a document is accessed and manipulated. It is also called DOM-API. It is an interface independent from platforms and languages.

6.355bis 	document object model (DOM) [b-ITU-T J.380.7]: A specification for a programming interface (API) from the W3C that allows programs and scripts to update the content, structure and style of HTML and XML documents.

6.356	document object model (DOM) object [b-ITU-T J.200]: An object generated by a HTML [b-W3C HTML] document.

6.357	domain [b-ITU-T J.178]: Any number of IPCablecom networks controlled by a single cable network operator.

6.357bis domain [b-ITU-T J.294]: The extent of home-network-compliant direct influence.

6.357ter domain [b-ITU-T J.1005]: A group of devices defined by a rights issuer such that the rights issuer can issue rights to objects for the group that can be processed by all devices within the group and only by those devices.

6.358	domain name system (DNS) (based on [IETF RFC 1034]) [b-ITU-T J.296]: A protocol used by the service that maps a host name on a network onto its IP address.

6.359	domains [b-ITU-T J.190]: The extent of home-network-compliant direct influence.

6.360	downlink [b-ITU-T J.195.2]: Link from HiNoC bridge (HB) to HiNoC modem (HM).

6.361	downloadable conditional access system (DCAS) [b-ITU-T J.291]: A technology for downloading conditional access into software on a set-top box; this is an effort under way at CableLabs and due for publication in 2006.

6.362	downstream [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.170], [b-ITU-T J.184], [b-ITU-T J.361]: In cable television, the direction of transmission from the headend to the subscriber.

6.362bis downstream (DS) [b-ITU-T J.212]: 1)Transmissions from CMTS to CM. This includes transmission from the M-CMTS Core to the EQAM as well as the RF transmissions from the EQAM to the CM. 2)RF spectrum used to transmit signals from a cable operator's headend or hub site to subscriber locations.

6.362ter Downstream [b-ITU-T J.222.3]: Flow of signals from the cable system control center through the distribution network to the customer. For communication purposes, associated with transmission (down) to the end-user.

6.363	Downstream Bonded Service Flow [b-ITU-T J.222.2]: A downstream Service Flow for which DOCSIS MAC Frames are transmitted on one or more Downstream Channels.

6.364	Downstream Bonding Group [b-ITU-T J.222.2]: A subcomponent object of a MAC Domain that distributes packets from an assigned set of Downstream Bonding Service Flows to an associated set of Downstream Channels of that MAC Domain.

6.365	Downstream Service Extended Header [b-ITU-T J.222.2]: A DOCSIS extended header that contains a Downstream Service ID (DSID).

6.366	Downstream Service Group [b-ITU-T J.222.2]: The complete set of Downstream Channels (DCs) from a single CMTS that could potentially reach a single Cable Modem. A DS-SG corresponds to a broadband forward carrier path signal from one CMTS. In an HFC deployment, a DS-SG corresponds to the downstream fibre transmission from one CMTS to one or more Fibre Nodes.

6.367	DPI cue message [b-ITU-T J.287]: See splice_info_section. A term used in [ITU-T J.181]; a "DPI Cue Message" is a splice_info_section in this document.

6.368	DPI PID [b-ITU-T J.287]: A single PID carrying ITU-T J.181 splice_info_sections.

6.369	DRM bridge [b-ITU-T J.197]: The distribution and home network infrastructure and technologies put in place to enable content protection and Digital Rights Management for network delivered content being stored and distributed on a home network.

6.370	drop cable [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122] Coaxial cable that connects to a residence or service location from a directional coupler (tap) placed on the nearest coaxial feeder cable.

6.371	DSC_ID [b-ITU-T J.1004]: The identification value of the descrambler (DSC) having a size of 40 bytes.

6.372	DSG address table [b-ITU-T J.128]: The collection of DSG Rules and DSG Classifiers contained within the DCD message. The DSG Client uses its DSG Client ID as an index into the DSG Address Table to determine what DSG Tunnel Address to receive.

6.373	DSG advanced mode [b-ITU-T J.128]: Operation with the DCD message. Address assignment is dynamic. The DSG Tunnel Address is determined by the DSG Agent and learned by the DSG Client through the DSG Address Table in the DCD message.

6.374	DSG agent [b-ITU-T J.128]: The DSG Agent is the implementation of the DSG protocol within the CMTS. The DSG Agent creates the DSG Tunnel, places content from the DSG Server into the DSG Tunnel, and sends the DSG Tunnel to the DSG Client.

6.375	DSG basic mode [b-ITU-T J.128]: Operation without the DCD message. Address assignment is static. The DSG Tunnel Address is determined by the DSG Client and learned by the DSG Agent through configuration. This mode provides backwards compatibility with earlier versions of the DSG specification.

6.376	DSG channel [b-ITU-T J.128]: Any DOCSIS downstream channel that contains one or more DSG Tunnels.

6.377	DSG classifier [b-ITU-T J.128]: A description of layer 3 and layer 4 filtering applied to DSG Tunnel traffic. DSG Classifiers may be specified in the DSG Agent and sent as a component of the DSG Address Table in the DCD Message.

6.378	DSG client [b-ITU-T J.128], [b-ITU-T J.290]: The DSG Client terminates the DSG Tunnel and receives content from the DSG Server. There may be more than one DSG Client within a Set-top Device.

6.379	DSG client controller [b-ITU-T J.128]: The portion of the Set-top Device that handles the processing of DCD messages and makes decisions regarding the forwarding of DSG Tunnels within the Set-top Device.

6.380	DSG client ID [b-ITU-T J.128]: This is an identifier that uniquely identifies a DSG Client. The DSG Client ID is unique per DSG Client, but is not unique per Set-top Device as the same DSG Client which provides the same function may exist in multiple Set-top Devices. In DSG Basic Mode, the DSG Client ID is a 6-byte MAC address. In DSG Advanced Mode, the DSG Client ID may additionally be a 2-byte Application ID, a 2-byte CA_system_ID, or a broadcast ID.

6.381	DSG eCM [b-ITU-T J.128]: A DOCSIS Cable Modem that has been embedded into a Set-top Device and includes DSG functionality.

6.382	DSG rule [b-ITU-T J.128]: A row entry within the DSG Address Table that assigns a DSG Client ID to a DSG Tunnel Address.

6.383	DSG server [b-ITU-T J.128]: The DSG Server refers to any server such as an Application Server or other network attached device that provides content that is transported through the DSG Tunnel to the DSG Client.

6.384	DSG tunnel [b-ITU-T J.128]: A stream of packets sent from the CMTS to the Set-top Terminal. In DSG Basic Mode, a DSG Tunnel is identified solely by its DSG Tunnel Address; all of the DSG Tunnel's packets use the same DSG Tunnel Address and different DSG Tunnels use different DSG Tunnel Addresses. In DSG Advanced Mode, a DSG Tunnel might be identified solely by its DSG Tunnel Address, or it might be identified by a combination of the DSG Tunnel Address along with other DSG Rule parameters: UCID List, Classifier IP addresses, and UDP port numbers.

6.385	DSG tunnel address [b-ITU-T J.128]: This specifically refers to the destination MAC address of the DSG Tunnel. If the source MAC address, the destination IP address, or the source IP address is to be referenced, then that reference must be explicitly stated.

6.386	DTI minimum clock oscillator [b-ITU-T J.211]: An oscillator that supports all the client clock performance requirements with holdover limited to the minimum bridging time. A non-ovenized oscillator can be used to support this oscillator category.

6.387	DTV service [b-ITU-T J.205]: This is the unit for delivering audio-visual content to the end users. In a more extensive definition (the one adopted in this Recommendation), this unit also comprehends the delivery of interactive content. It constitutes an editorially consistent whole and it is an aggregation of different kinds of service components.

6.388	DTV service component [b-ITU-T J.205]: Each one of the audio, visual or interactive elements that compose a digital television (DTV) service.

6.389	DTV service component delivery mechanism [b-ITU-T J.205]: Used to refer to any mechanism, channel or medium to deliver digital television (DTV) service components to an integrated broadcast and broadband (IBB) DTV receiver.

6.390	DTV service content [b-ITU-T J.205]: Audio/visual/interactive components delivered within a digital television (DTV) service as single editorially consistent whole.

6.391	Dynamic Host Configuration Protocol (DHCP) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: An Internet protocol used for assigning network-layer (IP) addresses.

6.391bis Dynamic Host Configuration Protocol (DHCP) (based on [IETF RFC 2131]) [b-ITU-T J.296]: A protocol used to automatically configure terminals on a TCP/IP network. For example, this protocol allows IP addresses to be assigned dynamically.

6.391ter dynamic host configuration protocol (DHCP) [b-ITU-T J.460.1]: The protocol used to configure an endpoint on an IP network commonly used to assign it an IP address, and recently extended to configure the location of the endpoint.

6.392	dynamic invocation interface (DII) [b-ITU-T J.380]: A method of accessing web service resources through low level application programming interface (API) functions.

6.393	dynamic quality of service (DQoS) [b-ITU-T J.161]: Assigned on the fly for each communication depending on the QoS requested.

6.393bis dynamic Quality of Service (DQoS) [b-ITU-T J.191]:[IPCablecom] Assigned on the fly for each communication depending on the QoS requested.

6.393ter dynamic quality of service [b-ITU-T J.361]: A quality of service assigned on the fly for each communication depending on the QoS requested.

6.394	dynamic range [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: The ratio between the greatest signal power that can be transmitted over a multichannel analogue transmission system without exceeding distortion or other performance limits, and the least signal power that can be utilized without exceeding noise, error rate or other performance limits.

6.395	Dynamically-joined Multicast Sessions [b-ITU-T J.222.3]: Multicast sessions joined after cable modem registration.

6.396	E.164 [b-ITU-T J.360]: E.164 is an ITU-T Recommendation which defines the international public telecommunication numbering plan used in the PSTN and other data networks.

6.397	E1 [b-ITU-T J.214]: E1 is a physical layer telephony protocol carrying data at 2048 Mbit/s. It can carry up to 32 DS0s, each of which can carry a telephone conversation.

6.398	embedded common interface (ECI) [b-ITU-T J.1010], [b-ITU-T J.1011]: Architecture and system to be specified in the ETSI ISG "Embedded CI", which allows the development and implementation of software-based swappable ECI clients in customer premises equipment (CPE) and thus provides interoperability of CPE devices with respect to ECI.

6.399	embedded common interface client (ECI client) [b-ITU-T J.1010]: Implementation of a CA/DRM client which is compliant with the planned Embedded CI specifications. Note that it is the software module in a CPE which provides all means to receive, in a protected manner, a consumer's entitlements and rights concerning the content that is distributed by a content distributor or operator. It also receives the conditions under which a right or an entitlement can be used by the consumer, and the keys to decrypt the various messages and content. An Embedded CI client may have an associated smart card.

6.399bis ECI client (embedded CI client) [b-ITU-T J.1011]: Implementation of a CA/DRM client which is compliant with the embedded CI specifications. Note that it is the software module in a CPE which provides all means to receive, in a protected manner and to control execution of a consumer's entitlements and rights concerning the content that is distributed by a content distributor or operator. It also receives the conditions under which a right or an entitlement can be used by the consumer and the keys to decrypt the various messages and content.

6.400	ECI client loader [b-ITU-T J.1011]: Software module part of the ECI host which allows downloading, verification and installation of new ECI client software in an ECI container of the ECI host.

6.401	ECI container (embedded CI container) [b-ITU-T J.1011]: Abstract concept which provides an isolated environment comprised of a virtual machine and a single ECI client.

6.402	embedded common interface (ECI) host [b-ITU-T J.1010]: Hardware and software system of a CPE, which covers ECI related functionalities and has interfaces to an ECI Client. Note that the ECI host is one part of the CPE firmware.

6.402bis ECI host [b-ITU-T J.1011]: Hardware and software system of a CPE, which covers ECI related functionalities and has interfaces to an ECI client. Note that the ECI host is one part of the CPE firmware. The ECI host is responsible for ensuring the isolation of each ECI container and provides authenticated loading of ECI clients.

6.403	ECI host loader [b-ITU-T J.1011]: Software module which allows downloading, verification and installation of (new) ECI host software into a CPE. Note that in a multi-stage loading configuration this term is used to refer to all security critical loading functions involved in loading the ECI host.

6.404	eCM [b-ITU-T J.126]: An eCM is an embedded Cable Modem, i.e., one that has been enhanced with the features of this Recommendation.

6.404bis eCM[b-ITU-T J.369], [b-ITU-T J.370], [b-ITU-T J.460.4] The logical DOCSIS CM component of a E-UE, complies with DOCSIS, eDOCSIS and IPCablecom requirements.

6.405	ECMAScript [b-ITU-T J.200]: Programming language defined by [b-ECMAScript].

6.406	Edge QAM [b-ITU-T J.291]: A device that provides QAM modulation to increase capacity in existing legacy cable networks.

6.407	edge QAM modulator (EQAM) [b-ITU-T J.212, [b-ITU-T J.210]: A headend or hub device that receives packets of digital video or data. It re-packetizes the video or data into an MPEG transport stream and digitally modulates the digital transport stream onto a downstream RF carrier using quadrature amplitude modulation (QAM).

6.408	eDOCSIS [b-ITU-T J.218]: Embedding another functional device within a cable modem.

6.408bis eDOCSIS [b-ITU-T J.126]: eDOCSIS is a CableLabs specification that defines the interface between the eCM and an eSAFE. The international version of the specification is this Recommendation.

6.409	eDOCSIS device [b-ITU-T J.126]: An eDOCSIS device is one that includes an eCM entity, one or more eSAFEs and supports a single software image using a CableModem secured software download mechanism.

6.410	EDTV-II sampling [b-ITU-T J.88]: 14.3 MHz with 8-bit precision

6.411	EDTV-II signals [b-ITU-T J.88]: EDTV-II signals comprise NTSC-compatible components transmitted in the centre part of the picture and helper components located in the upper and lower parts of the picture.

6.412	effective frame rate [b-ITU-T J.246], [b-ITU-T J.247]: The number of unique frames (i.e., total frames-repeated frames) per second.

6.413	effectively transmitted signals in sound-programme transmission [b-ITU-T N.1], [b-ITU-T J.13]: For sound-programme transmission, a signal at a particular frequency is said to be effectively transmitted if the nominal overall loss at that frequency does not exceed the nominal overall loss at 800 Hz by more than 4.3 dB. This should not be confused with the analogous definition concerning telephony circuits given in [1].[CCITT Recommendation General performance objectives applicable to all modern international circuits and national extension circuits, Vol. Ill, Rec. G.151, § 1, Note 1.]

For sound-programme circuits, the overall loss (relative to that at 800 Hz) defining effectively transmitted frequency is 1.4 dB, i.e. about one-third of the allowance.

6.414	electronic code book (ECB) [b-ITU-T J.181 Amendment 1]: This is a specific method of encryption. It is one of the methods used in DES.

6.415	electronic key [b-ITU-T J.93], [b-ITU-T J.95]: The term for data signals that are used to control the descrambling process in subscriber decoders.

NOTE: There are at least three types of electronic keys: those used for television signal streams, those used for protecting control system operations, and those used for the distribution of electronic keys on the cable system.See also "authorization coding" which is also effectively a key.

6.416	electronic programme guide [b-ITU-T J.90]: A structured multimedia database, intended to provide information on programmes to be broadcast or cablecast.

6.417	element uniqueness [b-ITU-T J.380.2]: Generally, XML elements shall be unique according to existing XML compliance where the element's distinctiveness is unambiguous and unique relative to its immediate spatial relationship to other elements.

6.418	elementary stream (ES) (based on [ITU-T J.200]) [b-ITU-T J.296]: A basic stream that contains video data, audio data, or private data. A single elementary stream is carried in a sequence of PES packets with one and only one stream.

6.419	embedded MTA device (e-MTA) [b-ITU-T J.126]: An eDOCSIS device that contains both an eMTA and an eCM.

6.420 embedded multimedia terminal adapter (eMTA) [b-ITU-T J.126]: An embedded version of an MTA.

6.420bis Embedded Multimedia Terminal Adapter (E-MTA) [b-ITU-T J.191]: [IPCablecom] A single node that contains both an MTA and a cable modem.

6.420ter E-MTA [b-ITU-T J.173]: Term used in this Recommendation generically representing the CM and MTA combination. This could be an embedded MTA or a stand-alone MTA.

6.421	embedded portal service element (ePS) [b-ITU-T J.126]: An IPCable2Home-compliant eSAFE that provides management and network address translation functions between the cable data network and the home network.

6.421bis embedded PS [b-ITU-T J.192]: A Portal Services element that does not use a standalone interface to connect to a CM.

6.421ter embedded PS [b-ITU-T J.290] A portal services element that does not use a stand-alone interface to connect to a set-top box device.

6.422	embedded security eSTB [b-ITU-T J.126]: An eSTB with integrated security functions.

6.423	embedded service/application functional entity (eSAFE) [b-ITU-T J.126]: An embedded version of a specified application, such as an IPCablecom multimedia terminal adapter (MTA), that provides a service using the CableModem IP platform, or a function or set of functions, such as the IPCable2Home Portal Service logical element, that supports the delivery of one or more services over an IP platform.

6.424	embedded set-top box (eSTB) [b-ITU-T J.126]: An eSAFE that is compliant with [b-ITU-T J.128], providing video, audio and data services. An example OpenCable-compliant eSTB is further specified in [b- HOST2.0].

6.424bis Embedded set-top box [b-ITU-T J.128]: An embedded Set-top Box is an embedded Service Application Functional Entity (eSAFE) defined in [b-ITU-T J.126]. It includes the DSG Client(s), a DSG Client Controller, an embedded processor for an application environment, and either an embedded or removable module for Conditional Access.

6.424ter embedded STB [b-ITU-T J.193]: A Set-Top-Box that does not use a stand-alone interface to connect to a RG.

6.424quater embedded set-top box [b-ITU-T J.290]: An embedded set-top box is an embedded service application functional entity. It includes the DSG client(s), a DSG client controller, an embedded processor for an application environment, and either an embedded or removable module for conditional access.

6.425	embedded TDM emulator adapter (eTEA) [b-ITU-T J.126]: An eSAFE that is compliant with [TEI], providing T1 and E1 Circuit transport over IP.

6.426	embedded user equipment (E-UE) [b-ITU-T J.369], [b-ITU-T J.370], [b-ITU-T J.460.4]: A single physical device embedded with an eDOCSIS-compliant DOCSIS Cable Modem and an IPCablecom eUE.

6.427	emergency alert system [b-ITU-T J.193]: A system, within which the NG-STB participates, that allows a service provider to distribute public emergency alarms and information about the public emergency to all of the customers attached to the cable network.

6.427bis emergency alert system (EAS) [b-ITU-T J.294]: A system, within which the H-STB participates, that allows a service provider to distribute public emergency alarms and information about the public emergency to all of the customers attached to the cable network.

6.428	emergency services gateway (ESGW) [b-ITU-T J.460.1]: A device which bridges the VoIP network to the Selective Router. In IPCablecom2 this is a function of the MGC, SG and MG.

6.429	emergency services query key (ESQK) [b-ITU-T J.460.1]: A code that looks like a telephone number that is temporarily assigned to a VoIP 9-1-1 call by the VPC and is used as the key to the ALI database to retrieve location information for that call.

6.430	emergency services routing number (ESRN) [b-ITU-T J.460.1]: A code that is used to route a VoIP 9-1-1 call to the correct ESGW and also to choose an appropriate trunk group on that ESGW that connects to a specific Selective Router.

6.431	emergency situation [b-ITU-T J.260]: A situation, of serious nature, that develops suddenly and unexpectedly. Extensive immediate important efforts, facilitated by telecommunications, may be required to restore a state of normality to avoid further risk to people or property. If this situation escalates, it may become a crisis and/or disaster.

6.432	emergency warning system [b-ITU-T J.296]: An emergency warning system is used for disaster broadcasts. Its startcontrol signal forces receiver units to receive the disaster broadcast programme.

6.433	enabler [b-ITU-T J.367]: A term used by OMA to mean "a technology intended for use in the development, deployment or operation of a service". Examples of OMA Enablers are Device Management, Push to-Talk over Cellular (PoC), Presence SIMPLE, and XML Document Management (XDM).

6.434	encryption [b-ITU-T J.170], [b-ITU-T J.178], [b-ITU-T J.361]: A method used to translate information in plaintext into ciphertext.

6.434bis encryption [b-ITU-T J.93], [b-ITU-T J.95]: The process of scrambling signals to avoid unauthorized access.

6.435	encryption key [b-ITU-T J.178]: The key used in a cryptographic algorithm to translate the plaintext to ciphertext.

6.436	encryption mode indicator bits (EMI Bits) [b-ITU-T J.197]: Two bits, associated with protected content, that specify the copy operations that are permissible for the associated content.

6.437	end user [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A human being, organization, or telecommunications system that accesses the network in order to communicate via the services provided by the network.

6.438	endpoint [b-ITU-T J.170], [b-ITU-T J.177], [b-ITU-T J.178], [b-ITU-T J.361]: A Terminal, Gateway or Multipoint Conference Unit (MCU).

6.438bis	endpoint [b-ITU-T J.380.1]: An address, a uniform resource identifier (URI), or a specific location where a logical service may be found and consumed.

6.438ter endpoint [b-ITU-T J.380.2]: An address, a Uniform Resource Identifier (URI), or a specific location where a logical service function or functions shall be found and consumed via message exchange.

6.439  engineering change notice (ECN) [b-ITU-T J.112 Annex B]: The final step in the procedure to change specifications.

6.440  engineering change order (ECO) [b-ITU-T J.112 Annex B]: The second step in the procedure to change specifications. DOCSIS posts ECO to website EC table and ECO page (with indication of ECO Comment Deadline). DOCSIS issues ECO announcement to DOCSIS-announce and working group mail lists (with indication of ECO Comment Deadline).

6.441  engineering change request (ECR) [b-ITU-T J.112 Annex B]: The first step in the procedure to change specifications. DOCSIS issues ECR number, posts to website EC table and ECR page. DOCSIS sends ECR to subject area working group mail list (and author).

6.442	engineering TS [b-ITU-T J.296]: The engineering transport stream (TS) is a kind of TS signal that transmits downloaded software contents or watching control information on a satellite broadcast network.

6.443	enhanced broadcasting [b-ITU-T J.700]: A system that is capable of delivering broadcast programmes over existing secondary distribution networks composed of HFC or FTTx with enhancements by applications and/or services transferred over IP-enabled networks.

6.444	entitlement control message (ECM) [b-ITU-T J.181 Amendment 1]:  These are private conditional access information messages which specify control words and possibly other, typically stream-specific, scrambling and/or control parameters.

6.444bis entitlement control messages (ECMs) [b-ITU-T J.290]: An ECM is an encrypted message that contains access criteria to various service tiers and a control word (CW).

6.445	Entitlement Management Messages (EMM)[b-ITU-T J.94]: Private Conditional Access information which specifies the authorization levels or the services of specific decoders; they may be addressed to individual decoder or groups of decoders.

6.445bis entitlement management message (EMM) [b-ITU-T J.181 Amendment 1]: These are private conditional access information messages which specify the authorization levels or the services of specific decoders. They may be addressed to single decoders or groups of decoders.

6.446 entitlement management messages (EMMs)[b-ITU-T J.290]: The EMM contains the actual authorization data and shall be sent in a secure method to each CPE device.

6.447	entitlements [b-ITU-T J.702]: Refer to authorization level(s) including conditional access information that a subscriber can use to access certain IPTV services in his/her IPTV TD.

6.448	EPG provider [b-ITU-T J.90]: The entity that collects, collates and assembles the elements of information that constitute the EPG database.

6.449	Equivalent impairment [b-ITU-T J.67]: The data signal quality is evaluated by adding a Gaussian noise signal to the received signal and plotting the bit-error ratio versus the noise level. For a given bit-error ratio, the difference in dB between the measured noise level and the theoretical level produces, by definition, the "equivalent impairment".

6.450	eRouter [b-ITU-T J.126]: DOCSIS Embedded Router. An eSAFE that is compliant with eRouter in [b-ITU-T J.218], providing version 4 and/or version 6 Internet Protocol-formatted data forwarding, address configuration, and Domain Name services to Internet Protocol host devices connected to the cable modem in a customer's premises.

6.450bis		eRouter [b-ITU-T J.218]: An eSAFE device that is implemented in conjunction with the DOCSIS embedded cable modem.

6.451	errored second [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: Any one second interval containing at least one bit error.

6.452	eTEA [b-ITU-T J.214]:	An eTEA is an eDOCSIS eSAFE; an embedded version of a TEA.

6.453	eUE [b-ITU-T J.369], [b-ITU-T J.370], [b-ITU-T J.460.4]: The logical IPCablecom UE component of a E-UE, complies with eSAFE and IPCablecom requirements.

6.454	event [b-ITU-T J.94]: A grouping of elementary broadcast data streams with a defined start and end time belonging to a common service, e.g. first half of a football match, News Flash, first part of an entertainment show.

6.454bis event [b-ITU-T J.181], [b-ITU-T J.181 Amendment 1], [b-ITU-T J.287]:	A splice event or a viewing event.

6.454ter event [b-ITU-T J.380.2]: A general term indicating something has happened or occurred.

6.455	event message [b-ITU-T J.164]: An Event Message is a set of data, representative of an event in the IPCablecom architecture that could be indicative of usage of one or more billable IPCablecom capabilities. An Event Message by itself may not be fully indicative of a customer's billable activities, but an Event Message correlated with other Event Messages builds the basis of a billable Usage Detail Record.

6.455bis 	event message [b-ITU-T J.170], [b-ITU-T J.178]: Message capturing a single portion of a connection.

6.456	execution engine [b-ITU-T J.200]:	A subsystem in a receiver that evaluates and executes procedural applications consisting of computer language instructions and associated data and media content. An execution engine may be implemented with an operating system, computer language compilers, interpreters and application programming interfaces (APIs), which a procedural application may use to present audiovisual content, interact with a user, or execute other tasks that are not evident to the user. A common example of an execution engine is the JavaTV software environment, using the Java programming language and byte code interpreter, JavaTV APIs, and a Java virtual machine for program execution.

6.457	extended subsplit [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.122]: A frequency division scheme that allows bidirectional traffic on a single coaxial cable; in North America, reverse path signals come to the Headend from 5 to 42 MHz, and forward path signals go from the Headend from 50 or 54 MHz to the upper frequency limit.

6.458	extended upstream frequency range [b-ITU-T J.222.1]:	An optional upstream frequency range over which a CM may be capable of transmitting. In the technology option that uses 6 MHz downstream channelization, this is 5-85 MHz. In the technology option that uses 8 MHz downstream channelization, no extended upstream frequency range is defined.

6.459	extensible HTML (XHTML) 	[b-ITU-T J.200]: Reformulation of HTML [b-W3C HTML] as an application language of XML [b-W3C XML].

6.460	Eye diagram [b-ITU-T J.67]: The eye diagram is defined as the superposition of all the configurations of the data signals.

6.461	factor [b-ITU-T J.262]: A factor, as used in the process of authentication, represents either something known (such as a PIN, password or passphrase), something possessed (such as a card with a magnetic stripe or a security token) or something unique (such as a finger or voice print) about the individual whose identity is be authenticated.

6.462	fast mode [b-ITU-T J.211]: An operating condition of a clock in which it is locked to an external reference and is using time constants, which are reduced to quickly bring the local oscillator's frequency into approximate agreement with the synchronization reference frequency.

6.463	fault reporting centre [b-ITU-T N.51]: A centre at a receiving country dealing with enquiries and fault reports concerning transmission to TVROs not related to an ITC.

6.464	feeder cable [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: Coaxial cables that run along streets within the served area and connect between the individual taps which serve the customer drops.

6.465	fetcher [b-ITU-T J.367]: A form of Watcher that has asked the Presence Service for the Presence Information of one or more Presentities, but is not requesting a Notification from the Presence Service of (future) changes in a Presentity's Presence Information [b-OMA RD-PRS].

6.466	Fiber Distributed Data Interface (FDDI) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A fiber-based LAN standard.

6.467	fibre node [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A point of interface between a fibre trunk and the coaxial distribution.

6.468	Field-time waveform distortion [b-ITU-T J.67]: If a square-wave signal with a period of the same order as one field and of nominal luminance amplitude is applied to the input of the circuit, the field-time waveform distortion is defined as the change in shape of the square wave at the output. A period at the beginning and end of the square wave, equivalent to the duration of a few lines, is excluded from the measurement.

6.469	file downloading [b-ITU-T J.127]: Program transmission method whereby the program starts playing after the entire data has been downloaded.

6.470	final edited master [b-ITU-T J.248]: The final edited master is the final instance of a television program as it is provided at the end of the program production chain, ready to be dispatched to the distributors and the end users.

6.471	fixed stuff [b-ITU-T J.131], [b-ITU-T J.132]: Bytes that are used to fill up unused data positions.

6.472	flooding	[b-ITU-T J.213]: An operation of an L2 bridge in which it replicates an L2PDU addressed to a group MAC or unlearned individual MAC address to all Bridge Ports other than the L2PDU's ingress port.

6.473	flow [b-ITU-T J.212]: A stream of packets in DEPI used to transport data of a certain priority from the M-CMTS Core to a particular QAM channel of the EQAM. In PSP operation, there can exist several flows per QAM channel.

6.474	forbidden [b-ITU-T J.94]: The term "forbidden" when used in the clauses defining the coded bitstream, indicates that the value shall never be used.

6.475	formatter [b-ITU-T J.200]: A subsystem in a receiver that evaluates and presents declarative applications consisting of content in multiple formats. A formatter also responds to formatting information associated with the content, to user inputs, and to script statements that control presentation behaviour and initiate other processes in response to user input and other events. An example of a formatter is a nested context language (NCL) [b-NCL] formatter engine.

6.476	forward channel [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: The direction of RF signal flow away from the headend toward the end user; equivalent to Downstream.

6.476bis forward channel [b-ITU-T J.116]: The direction of RF signal flow away from the BTS toward the end user; synonymous to Downstream.

6.477	forward data channel [b-ITU-T J.184]: A data channel carried from the headend to the terminal device in a modulated channel at a rate of 1.544 to 3.088 Mbit/s. The FDC carries IP traffic only for:

• Conditional access for analogue signals.

• Entitlement management messages for digital signals.

• General messaging.

• Application download.

• PC data services.

• Variable bit rate (VBR) download.

• Broadcast data.

• Network management.

6.478	forward error correction (FEC) [b-ITU-T J.210]: A class of methods for controlling errors in a communication system. FEC sends parity information with the data which can be used by the receiver to check and correct the data.

6.479	fractional [b-ITU-T J.214]: A fractional T1 or fractional E1 carries only a portion of the total number of DS0s that a T1 or E1 carries.

6.480	fragmented TLV packet	[b-ITU-T J.183], [b-ITU-T J.288]: A fixed-length packet that has fragmented type-length-value (TLV).

6.481	frame [b-ITU-T J.122]: See MAC frame, S-CDMA frame and MPEG frame.

6.482	frame check sequence [b-ITU-T J.195.2]: A redundant sequence that is used for verifying the correctness of the received data.

6.483	Frame Loss Ratio (FLR)	[b-ITU-T J.141]: The ratio of errored data frames with respect to total number of frames transmitted, when the data frames are transmitted over a noisy channel.

6.484	frame rate [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342], [b-ITU-T J.246],[b-ITU-T J.247]: The number of (progressive) frames displayed per second (fps).

6.485	frame synchronizer [b-ITU-T J.248]: A device that receives a video signal from a remote source, and synchronizes it to the local video synchronization pulses, in order that it may be seamlessly mixed with locally generated video signals.

6.486	free-run mode [b-ITU-T J.211]: An operating condition of a DTI clock whose output signals are internally controlled by the DTI server. The clock has never had, or has lost, external reference input and has no access to stored data that was acquired from a previously connected external reference during the time after the last power cycle. Free-run ends when the clock output is influenced by an external reference or the process to achieve lock to an external reference. Free-run may provide needed stability when external reference has been lost or not equipped.

6.487	FTV [b-ITU-T J.901]: Video media system that can provide audiences of freedom to choice their viewpoint.

6.488	fukinuki hole	[b-ITU-T J.88]: Frequency regions around the colour sub-carrier of the compatible centre part of EDTV-II images, where normal NTSC signals have lower spectral density.

6.489	full period terminated service	[b-ITU-T J.93], [b-ITU-T J.95]:	A subscription service that is always available to subscribers during the operating hours of the delivery system.

NOTE: By contrast, other services, such as a pay-per-view feature film, are only available for a specific period of time.

6.490	function [b-ITU-T J.190]: Capabilities that compromise logical elements.

6.490bis		function [b-ITU-T J.200]: A process which conveys or transforms data in a predictable way. It may be effected by hardware, software or a combination of both.

6.491	Gain/frequency response [b-ITU-T J.67]: The gain/frequency characteristic of the circuit is defined as the variation in gain between the input and the output of the circuit over the frequency band extending from the field repetition frequency to the nominal cut-off frequency of the MAC signal, relative to the gain at a suitable reference frequency.

6.492	gateway	[b-ITU-T J.170], [b-ITU-T J.178], [b-ITU-T J.361]:Devices bridging between the IPCablecom IP voice communication world and the PSTN. Examples are the media gateway, which provides the bearer circuit interfaces to the PSTN and transcodes the media stream, and the signalling gateway, which sends and receives circuit switched network signalling to the edge of the IPCablecom network.

6.493	generic format of transport stream [b-ITU-T J.183]: A data stream other than Motion Picture Experts Group version 2 (MPEG-2) transport stream (TS), e.g., type-length-value (TLV). For transmitting a TLV stream in an existing ITU-T J.83 system, packetization is carried out in accordance with [ITU-T J.288].

6.494	GigE (GE) [b-ITU-T J.212]:	Gigabit Ethernet (1 Gbit/s).

6.495	GINGA [b-ITU-T J.200]: The middleware specification for the Brazilian digital TV system. It comprises two execution environments, for both declarative and imperative applications.

6.496	GINGA-J [b-ITU-T J.200]: Ginga's execution environment for imperative applications written in Java. It also comprises a set of APIs for the development of interactive digital TV applications.

6.497	global uniqueness [b-ITU-T J.380.2]: Global or universally unique and at no other time shall the item be compromised, reused, or otherwise taken to have more than one meaning. The enforcement of uniqueness as well as the creation of globally unique identifiers is outside the scope of this Recommendation and [b-IETF RFC 4122] is recommended.

6.498	GPI [b-ITU-T J.287]: A general purpose interface which is commonly used to source or sink contact closures in video facilities.

6.499	gpssec [b-ITU-T J.211]: The gpssec is a 32-bit timestamp counter that is incremented every second. GPS system time began on January 6, 1980. The gpssec value was set to zero at the January 6, 1980 start epoch.

6.500	graphic interchange format (GIF) [b-ITU-T J.296]: A bitmap image format that was introduced by CompuServe in 1987 and has since come into widespread usage on the World Wide Web due to its wide support and portability. GIF images are compressed using the Lempel-Ziv-Welch (LZW) lossless data compression technique to reduce the file size without degrading the visual quality.

6.501	graphical user interface [b-ITU-T J.296]: A graphical user interface is equipped with graphical input and output on the computer monitor.

6.502	group delay	[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: The difference in transmission time between the highest and lowest of several frequencies through a device, circuit or system.

6.503	group MAC (GMAC) address	[b-ITU-T J.213]: An IEEE 6-byte MAC address with the first transmitted bit (the group bit) set to "1", indicating that the address refers to a group of MAC hosts. In the canonical representation of MAC addresses used for Ethernet transmission, the group bit is the least significant bit of the first byte. The all-1s broadcast MAC address is considered to be a GMAC address.

6.504	Group-delay distortion [b-ITU-T J.67]: The group-delay distortion, expressed in ns, is defined by the difference between the group delay for each measured frequency and the group delay for a given reference frequency.

6.505	Grouping-broadcast [b-ITU-T J.196.3]:	A transmission method for a HiNoC bridge (HB) to broadcast frames to the HiNoC modems (HMs) joined in a certain group with the group ID indicated in the frame headers, while the HMs that are not part of the group cannot receive the frames.

6.506	guaranteed service domain (GSD) [b-ITU-T J.290]: Devices in the GSD will be able to receive QoS sensitive content services such as VoIP, multiplayer interactive gaming, and IP video-phone.

6.507  guardband [b-ITU-T J.112 Annex B]: Minimum time allocated between bursts in the upstream referenced from the symbol centre of the last symbol of a burst to the symbol centre of the first symbol of the following burst. The guardband should be at least the duration of five symbols plus the maximum system timing error.

6.508	guard time [b-ITU-T J.112], [b-ITU-T J.116]: Minimum time allocated between bursts in the upstream, referenced from the symbol centre of the last symbol of a burst to the symbol centre of the first symbol of the following burst.

6.508bis		guard time [b-ITU-T J.112 Annex B]: The term guard time is similar to guardband, except that it is measured from the end of the last symbol of one burst to the beginning of the first symbol of the preamble of an immediately following burst. Thus, the guard time is equal to the guardband- 1.

6.508ter		guard time [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: Minimum time allocated between bursts in the upstream referenced from the symbol centre of the last symbol of a burst to the symbol centre of the first symbol of the following burst. The guard time should be at least the duration of five symbols plus the maximum system timing error.

6.509	HANC [b-ITU-T J.287]: Horizontal ancillary data space in digital video streams.

6.510	Harmonic Related Carrier (HRC) [b-ITU-T J.112]: A method of spacing television channels on a cable television system in exact increments, with all carrier frequencies harmonically related to a common reference.

6.510bis	harmonic related carrier (HRC) [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: A method of spacing television channels on a cable television system in exact 6-MHz increments, with all carrier frequencies harmonically related to a common reference.

6.510ter harmonic related carriers (HRC) [b-ITU-T J.210]: A method of spacing channels on a cable television system with all carriers related to a common reference.

6.511	HD-SDI	[b-ITU-T J.287]: Abbreviation for high definition serial digital interface (see [b-SMPTE ST 292-1]).

6.512	headend [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.360] The central location on the cable network that is responsible for injecting broadcast video and other signals in the downstream direction. See also Master Headend, Distribution Hub.

6.513	header	[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.170], [b-ITU-T J.178], [b-ITU-T J.361]: Protocol control information located at the beginning of a protocol data unit.

6.514	helper signals	[b-ITU-T J.88]: Spatial-temporal video enhancement signals, e.g. Horizontal High frequency helper signal (HH), Vertical High frequency helper signal (VH) Vertical Temporal helper signal (VT).

6.515	HFC access network [b-ITU-T J.363], [b-ITU-T J.460.3]: The Hybrid-Fibre Coax Network, which provides physical transport of video and high speed data services via DOCSIS.

6.516	HFC-based networks [b-ITU-T J.381]: HFC-based networks include legacy cable networks such as hybrid fibre coax; recent technology deployments such as radio frequency over glass (RFoG); and cable network technologies that may be deployed in the near future.

6.517	high definition analogue form or output	[b-ITU-T J.197]: A format or output that is not digital, and has a resolution higher than standard definition analogue form or output.

6.518	High Frequency (HF) [b-ITU-T J. 112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: Used in this Recommendation to refer to the entire subsplit (5-30 MHz) and extended subsplit (5-42 MHz) band used in reverse channel communications over the cable television network.

6.519	high return [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: A frequency division scheme that allows bidirectional traffic on a single coaxial cable; reverse channel signals propagate to the headend above the downstream passband.

6.520	high-bandwidth digital content protection (HDCP) [b-ITU-T J.197]: The method of authentication, encryption, decryption, and renewability that is described in the specification entitled "High bandwidth digital content protection system, rev. 1.1".

6.521	HN-TD1 [b-ITU-T J.293]: A logical interface defined in 6.1.4 for content reception through an IP home network.

6.522	HN-TD2 [b-ITU-T J.293]: A logical interface defined in 6.1.5 for interactive communications to entities within an IP home network.

6.523	holdover mode [b-ITU-T J.211]: An operating condition of a DTI clock that has lost its controlling input and is using stored data, acquired while in normal or fast mode operation, to control its output. The stored data is filtered to minimize the effects of short-term variations and to establish a predictor of oscillator behaviour during the reference outage. This permits the output deviation from normal operation to be minimized.

6.524	home access (HA)	[b-ITU-T J.190]: A device class that connects access network with home bridge.

6.525	home access (HA) device [b-ITU-T J.192]: A grouping of logical elements used to achieve HFC access for IPCable2Home network(s), referred to as a Residential Gateway in this Recommendation.

6.525bis	 home access (HA) device [b-ITU-T J.290]: A grouping of logical elements used to achieve HFC access for IPCable2Home network(s).

6.526	home bridge (HB)	[b-ITU-T J.190]: A device class that connects home access with home client.

6.527	home bridge (HB) device [b-ITU-T J.290]: A group of logical elements used to bridge IPCable2Home networks together.

6.528	home client (HC) [b-ITU-T J.190]: A device class that connects home bridge with home decoder.

6.529	home client (HC) device [b-ITU-T J.192]: A group of logical elements used to provide functionality to client applications, referred to as an IPCable2Home Host in this Recommendation.

6.529bis 	home client (HC) device [b-ITU-T J.290]: A group of logical elements used to provide functionality to client applications.

6.530	home decoder (HD) [b-ITU-T J.190]: A device class that terminates home network.

6.531	home network [b-ITU-T J.295]: A network within a home compliant with Recommendation ITU-T J.190, consisting of IP-based transport including a wired LAN (Ethernet: IEEE 802.3) and wireless LAN (WiFi: IEEE 802.11a/b/g/n) and non-IP-based communication methods such as ZigBee (IEEE 802.15.4), where the hybrid cable set-top box is equipped with all of the HA, HB, HC and HD functionalities defined in Recommendation ITU-T J.190

6.532	home network (HN) type remote control unit (RCU)	[b-ITU-T J.295]: An RCU connected to the set-top box via an IP home network. Interactive operation is supported. In addition to communicating and controlling the set-top box, the HN type RCU can also access directly services provided by cable operators or other content and/or service providers.

6.533	home network coaxial modem [b-ITU-T J.293]:A modem for IP communications modulated over Coaxial cable for communications within the home, e.g., MOCA, HPNA over Coax.

6.534	home network interface [b-ITU-T J.294]: The logical interface between the RG and the local area network (LAN) (i.e., home network).

6.535	home network planes [b-ITU-T J.190]: User interfaces sharing the same Layer 1/Layer 2 or internal link.

6.536	home terminal [b-ITU-T J.193]: The device attached to the cable network that receives and renders services for the customer. Also known as a set top box.

6.537	host [b-ITU-T J.93], [b-ITU-T J.95]: A device with generalized functionality where modules containing specialized functionality can be connected.

6.538	HTTP Digest over TLS-based GBA	[b-ITU-T J.366.9]: This is a GBA that uses HTTP Digest over TLS.

6.539	HTTP over SSL or HTTP secure (HTTPS) [b-ITU-T J.380.7]: This is the use of Secure Socket Layer (SSL) or Transport Layer Security (TLS) as a sub-layer under regular HTTP application layering.

6.540	hum modulation [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: Undesired modulation of the television visual carrier by the fundamental or low-order harmonics of the power supply frequency, or other low-frequency disturbances.

6.541	hybrid cable set-box top (STB) [b-ITU-T J.296]: An STB that satisfies the requirements defined in this Recommendation. Unless specifically noted otherwise, in this Recommendation the term STB shall mean the hybrid cable STB.

6.541bis	 hybrid cable set-top box (STB) [b-ITU-T J.295]: An STB that satisfies the requirements defined in this Recommendation. Unless specifically noted, STB shall mean the hybrid cable STB in this Recommendation.

6.542	hybrid CPE 	[b-ITU-T J.700]: A terminal device that is capable of receiving content services over MPEG transport streams and IP.

6.543	hybrid fibre coax [b-ITU-T J.460.2]: Access network architecture consisting of fibre-optic feeders from the head end to nodes, at which point a coaxial cable is used for the final distribution to the subscribers.

6.544	hybrid fibre/coax (HFC) system [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.161], [b-ITU-T J.210], [b-ITU-T J.212]: A broadband bidirectional shared-media transmission system using fibre trunks between the headend and the fibre nodes, and coaxial distribution from the fibre nodes to the customer locations.

6.545	Hybrid Fibre-and-Coaxial (HFC) network [b-ITU-T J.112]: A broadband bidirectional shared-media transmission system using fibre trunks between the headend and the fibre nodes, and coaxial distribution from the fibre nodes to the customer locations.

6.546	hybrid full reference encrypted model [b-ITU-T J.343]: An objective video quality model that predicts subjective quality using the reference video, the decoded video frames, and packet headers. Such models are suitable for use with encrypted video.

6.547	hybrid full reference model [b-ITU-T J.343]: An objective video quality model that predicts subjective quality using the reference video, the decoded video frames, packet headers, and the video payload. Such models cannot analyse encrypted video.

6.548	hybrid no reference encrypted model [b-ITU-T J.343]: An objective video quality model that predicts subjective quality using the decoded video frames and packet headers. Such models can be deployed in-service and are suitable for use with encrypted video.

6.549	hybrid no reference model [b-ITU-T J.343]: An objective video quality model that predicts subjective quality using the decoded video frames, packet headers, and video payload. Such models can be deployed in-service but cannot analyse encrypted video.

6.550	hybrid reduced reference encrypted model [b-ITU-T J.343]: An objective video quality model that predicts subjective quality using the decoded video frames, packet headers, and features extracted from the reference video. These models can be deployed in-service and are suitable for use with encrypted video.

6.551	hybrid reduced reference model [b-ITU-T J.343]: An objective video quality model that predicts subjective quality using the decoded video frames, packet headers, video payload and features extracted from the reference video. Such models can be deployed in-service but cannot analyse encrypted video.

6.552	hybrid set-top-box (H-STB) [b-ITU-T J.294]:	A compilation of hardware and software functional entities contained within one or more physical devices, that, at a baseline level, provides the receiving functions for cable broadcast services. In addition, the hybrid set-top-box should support the interactive functions of IP-based services, additional time critical services between the access and the home network as well as extension and supplemental services.

6.553	hypertext transfer protocol (HTTP)	[b-ITU-T J.380.7]:	The underlying protocol used by the World Wide Web. HTTP defines how messages are formatted and transmitted, and what actions Web servers and browsers should take in response to various commands.

6.554	hypothetical reference circuit (in the fixed satellite service) [b-ITU-T J.61]: A reference circuit for a system in the fixed-satellite service which may form part of an international television circuit and is defined as follows:

–it consists of one Earth station satellite-Earth station system;

–it includes one pair of modulation and demodulation equipment for translation from the baseband to the radio-frequency carrier, and from the radio-frequency carrier to the baseband, respectively;

–it does not include a standards converter or a synchronizing-pulse regenerator, or equipment for the insertion of signals in the line/field blanking interval.

6.555	hypothetical reference circuit (terrestrial) [b-ITU-T J.61]: A reference circuit, used as an example of an international television circuit. It may be of either radio or cable type and it has the following characteristics:

– the overall length between video terminal points is 2500 km;

– two intermediate video points divide the circuit into three sections of equal length;

– the three sections are lined up individually and then interconnected without any form of overall adjustment or correction;

– the circuit does not contain a standards converter or a synchronizing pulse regenerator, or equipment for the insertion of signals in the line/field blanking interval.

6.556	i2	[b-ITU-T J.460.1]: The second 911 VoIP migration phase, called "i2" as defined by NENA providing a viable solution for VoIP carriers. There are two specialized "service operators" introduced in this migration phase the VPC Service Operator and the Emergency Services Gateway Operator.

6.557	i3	[b-ITU-T J.460.1]: The third and final migration phase planned by NENA, a more long-term approach to address the needs of having IP-enabled emergency centres.

6.558	iAPR [b-ITU-T J.117]:	A register affiliated with an asynchronous connection, that indicates how much of data has been produced. This register also has other bits that are used for demarcation of variable-length frames, and to support the connection disconnection sequence.

6.559	IBB application [b-ITU-T J.205]: An Application that is meant to be handled and executed within an integrated broadcast and broadband (IBB) application control framework defined by this Recommendation. Such applications can have their application contents delivered using different application component delivery mechanisms.

6.560	IBB application control mechanism [b-ITU-T J.205]: A mechanism used by integrated broadcast and broadband (IBB) digital television (DTV) service providers to signal and control IBB applications associated to their respective IBB DTV services. The mechanism contemplates the transmission of an IBB DTV service's application control data structure within the IBB DTV service. Such data structure can support the signalling of IBB applications that are retrievable from the broadcast channel or through the broadband channel from HTTP servers or application repositories.

6.561	IBB application provider [b-ITU-T J.206]: An organization that makes available an IBB application; in the most common case, it is the application developer.

6.562	IBB DTV receiver [b-ITU-T J.205]: A device capable of receiving and displaying DTV Services as well as integrated broadcast and broadband (IBB) DTV services.

6.563	IBB DTV service provider [b-ITU-T J.205]: An entity (i.e., broadcasters) making available and delivering IBB DTV services.

6.564	IBB installable application [b-ITU-T J.205]: An integrated broadcast and broadband (IBB) application that can be downloaded and saved onto the IBB DTV receiver's persistent storage for later usage.

6.565	IBB resident application	[b-ITU-T J.205]: An integrated broadcast and broadband (IBB) application embedded into the IBB DTV receiver by the device manufacturer.

6.566	immediate mode [b-ITU-T J.287]: Processing of a multiple_operation_message() when the value of time_type within timestamp() is 0.

6.567	impulse noise [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: Noise characterized by non-overlapping transient disturbances.

6.568	IMS delta specifications	[b-ITU-T J.360]: Suite of 3GPP IMS specifications modified to reflect cable specific deltas necessary to comply with IPCablecom.

6.569	in point	[b-ITU-T J.181 Amendment 1]: A point in the stream, suitable for entry.

6.569bis in point	[b-ITU-T J.189]: A point in the bit stream, suitable for entry, that lies on an elementary access unit boundary.

6.569ter in point	[b-ITU-T J.287]: A point in the stream, suitable for entry, that lies on an access unit boundary.

6.570	in point packet [b-ITU-T J.189]: A transport stream packet which corresponds to the first packet following the In Point.

6.571	in stream device [b-ITU-T J.181]:	A device that receives the transport stream directly and is able to derive timing information directly from the transport stream.

6.572	Incremental Related Carriers (IRC) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: A method of spacing NTSC television channels on a cable television system in which all channels except 5 and 6 correspond to the standard channel plan, used to reduce composite triple beat distortions.

6.572bis incremental related carriers (IRC) [b-ITU-T J.210]: A method of spacing NTSC television channels on a cable television system in which all channels are offset up 12.5 kHz with respect to the [CEA 542 B] standard channel plan except for channels 5 and 6.

6.573	individual MAC address	[b-ITU-T J.213]: An IEEE 6-byte MAC address with the first transmitted bit (the group bit) set to "0", indicating that the address refers to a single MAC host. For the Ethernet MAC addresses of DOCSIS, the group bit is the least significant bit of the first byte of the MAC address.

6.574	information element (IE) [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: The fields that make up a MAP and define individual grants, deferred grants, etc.

6.575	infrared rays (IR) type RCU [b-ITU-T J.295]: An RCU operating on infrared rays. Only uni-directional operation is supported.

6.576	injector	[b-ITU-T J.287]: A device or combination of devices within the DCS capable of converting ITU-T J.287 message data into an ITU-T J.181 splice_info_section(), including a program-specific PCR splice time value, if necessary, and multiplexing the resulting section data along with the other program components into the eventual MPEG SPTS or MPTS.

6.577	injector instance [b-ITU-T J.287]:	A specific instance of an injector, constrained to place a single DPI PID into a single MPEG program in a single transport stream.

6.578	insertion channel [b-ITU-T J.280]: The insertion multiplex channel(s) that replaces the primary channel in whole or in part of the duration for a splice event.

6.579	insertion gain	[b-ITU-T J.61]: The ratio, expressed in decibels, of the peak-to-peak amplitude of a specified test signal at the receiving end to the nominal amplitude of that signal at the sending end, the peak-to-peak amplitude being defined as the difference between the amplitudes measured at defined points of the signal used.

6.580	insertion multiplex	[b-ITU-T J.280]: This is the source of the insertion channel. A multiplex produced by a server may under some circumstances exclude program specific information (PSI), thus it is understood that this multiplex may be a non-compliant MPEG-2 transport stream.

6.581	instance	[b-ITU-T J.94]: See table instance. (See 6.1206).

6.582	integrated broadcast and broadband (IBB) DTV service [b-ITU-T J.205]: A service that simultaneously provides an integrated experience of broadcasting and interactivity relating to media content, data and applications from multiple sources, where the interactivity is sometimes associated with broadcasting programmes.

6.583	integrated services digital network (ISDN) [b-ITU-T J.460.1]:	User Part (part of the SS7 signaling stack).

6.584	integrity [b-ITU-T J.93], [b-ITU-T J.95]: The ability of a function to withstand being usurped for unauthorized use, or being modified to yield unauthorized results.

6.584bis integrity [b-ITU-T J.170]:A way to ensure that information is not modified except by those who are authorized to do so.

6.585	intended frame rate [b-ITU-T J.246], [b-ITU-T J.247]: The Number of video frames per second physically stored for some representation of a video sequence. The intended frame rate may be constant or may change with time. Two examples of constant intended frame rates are a BetacamSP tape containing 25 fps and a VQEG FR-TV Phase I compliant 625-line YUV file containing 25 fps; these both have an intended frame rate of 25 fps. One example of a variable intended frame rate is a computer file containing only new frames; in this case the intended frame rate exactly matches the effective frame rate. The content of video frames is not considered when determining intended frame rate.

6.586	interaction channel [b-ITU-T J.110]: A bidirectional channel between the user and the service provider for interaction purposes. In certain implementations the interaction channel could also carry a user selected broadcast service. In general, the interaction channel is formed by:

–return interaction path (return channel): A communication channel from the user to the service provider. It is multiple point-to-point.

–forward interaction path: An individual communications channel from the service provider to the user. It may be embedded into the broadcast channel. This channel may not be present in all implementations.

6.587	interactive interface module (see Annex A) [b-ITU-T J.112]: Called CM in Annex B and MH in Annex C.

6.588	Interference [b-ITU-T J.67]: The signal-to-interference ratio is defined as the ratio, expressed in decibels, of the nominal amplitude of the luminance signal (1 V) to the peak-to-peak amplitude of the interfering signal.

6.589	interframe coding [b-ITU-T J.248]: Bit rate reduction video signal encoding that exploits the video signal redundancy over several pictures.

6.590	interleave [b-ITU-T J.112], [b-ITU-T J.116]: An error correction method that enables the correction of burst noise induced errors.

6.591	interleave frame allocation [b-ITU-T J.296]: A method of 3D video data for both-eye viewing.

6.592	Intermediate audio quality [b-ITU-T J.145]: Audio quality lower than defined in ITU-R BS.1116, if acceptable or unavoidable. Rapid developments in the use of the Internet for distribution and broadcast of audio material, where the data rate is limited, have led to a compromise in audio quality. Other applications for such lower audio quality are digital AM, digital satellite broadcasting, commentary circuits in radio and TV, audio on demand services and audio on dial-up lines.

6.593	international emergency situation [b-ITU-T J.260]: An emergency situation, across international boundaries, that affects more than one country.

6.594	international multiple destination sound-programme circuit section [b-ITU-T N.1]: The unidirectional sound-programme transmission path from one frontier station to two or more of the frontier stations at which interconnection is made at audio frequencies.

6.595	international multiple destination sound-programme circuit [b-ITU-T N.1]: The unidirectional transmission path from one ISPC to two or more other ISPCs comprising sound-programme circuit sections (national or international) one of which is an international multiple destination circuit section, together with any necessary audio equipment.

6.596	international multiple destination sound-programme connection [b-ITU-T N.1]: The unidirectional transmission path between the broadcasting organization (send) and two or more broadcasting organizations (receive) comprising the international multiple destination sound-programme link extended at its ends over national sound-programme circuits to the broadcasting organizations.

6.597	international multiple destination sound-programme link [b-ITU-T N.1]: The unidirectional transmission path between the ISPCs of the terminal countries involved in an international multiple destination sound-programme transmission. The international multiple destination sound-programme link comprises international sound-programme circuits, one of which is an international multiple destination sound-programme circuit.

6.598	international multiple destination television circuit [b-ITU-T N.51]: The unidirectional transmission path from one ITC to two or more other ITCs comprising television circuit sections (national or international) one of which is an international multiple destination circuit section, together with any necessary video equipment.

6.599	international multiple destination television circuit section [b-ITU-T N.51]: The unidirectional television transmission path from one frontier station to two or more of the frontier stations at which interconnection is made at video frequencies.

The transmission path between two ITCs which comprises one or more television circuit sections (national or international) together with any necessary video equipment. The transmission path may be established via terrestrial or single destination satellite routing.

6.600	international multiple destination television connection [b-ITU-T N.51]: The unidirectional transmission path between the broadcasting organization (send) and two or more broadcasting organizations (receive) comprising the international multiple destination television link extended at its end over national television circuits to the broadcasting organizations.

6.601	international multiple destination television link [b-ITU-T N.51]: The unidirectional transmission path between the ITCs of the terminal countries involved in an international multiple destination television transmission. The international multiple destination television link comprises international television circuits, one of which is an international multiple destination television circuit.

6.602	international satellite transmission centre (ISTC) [b-ITU-T N.51]: A centre at a transmitting country responsible for the national extension and up-link to satellite. This term is applicable only for transmission to TVROs not related to an ITC.

6.603	international sound-programme centre (ISPC)	[b-ITU-T N.1], [b-ITU-T J.13]: "A centre at which at least one international sound-programme circuit terminates and in which international sound-programme connections can be made by the interconnection of international and national sound-programme circuits.

The ISPC is responsible for setting up and maintaining international sound-programme links and for the supervision of the transmissions made on them."

6.604	international sound-programme circuit [b-ITU-T J.13]: The unidirectional transmission path between two ISPCs and comprising one or more sound-programme circuit sections (national or international), together with any necessary audio equipment (amplifiers, compandors, etc.).

6.605	international sound-programme circuit [b-ITU-T N.1]: The transmission path between two ISPCs which comprises one or more sound-programme circuit sections (national or international), together with any necessary audio equipment. The transmission path may be established via terrestrial or single destination satellite routing.

6.606	international sound-programme connection [b-ITU-T N.1], [b-ITU-T J.13]: "The unidirectional path between the broadcasting organization (send) and the broadcasting organization (receive) comprising the international sound-programme link extended at its two ends over national sound-programme circuits to the broadcasting organizations (see Figure 2/J.13).

The assembly of the “international sound-programme link” and the national circuits between the broadcasting organizations, constitutes the “international sound-programme connection”. Figure 3/J.13 illustrates, by way of example, an international sound-programme connection as it might be encountered in practice."

6.607	international sound-programme link [b-ITU-T N.1], [b-ITU-T J.13]: The unidirectional path for sound-programme transmissions between the ISPCs of the two terminal countries involved in an international sound-programme transmission. The international sound-programme link comprises one or more international sound-programme circuits interconnected at intermediate ISPCs. It can also include national sound-programme circuits in transit countries.

6.608	international sound-programme transmission	[b-ITU-T N.1], [b-ITU-T J.13]: The transmission of sound over the international telecommunication network for the purpose of inter-changing sound-programme material between broadcasting organizations in different countries. Such a transmission includes all types of programme material normally transmitted by a sound broadcasting service, for example, speech, music, sound accompanying a television programme. etc.

6.609	international television-sound transmission [b-ITU-T N.1]: The transmission of television-sound signals over the international telecommunications network for the purpose of interchanging television-sound material between broadcasting organizations in different countries.

6.610	international television centre (ITC) [b-ITU-T N.51]: A centre at which at least one international television circuit terminates and in which international television connections can be made up by the interconnection of international and national television circuits.

6.611	international television connection [b-ITU-T N.51]: The unidirectional transmission path between the broadcasting organization (send) and the broadcasting organization (receive) comprising the international television link extended at its two ends over national television circuits to the broadcasting organization.

6.612	international television link [b-ITU-T N.51]: The unidirectional transmission path between the ITCs of the two terminal countries involved in an international television transmission. The international television link comprises one or more international television circuits interconnected at intermediate ITCs. It can also include national television circuits in transit countries.

6.613	international television transmission [b-ITU-T N.51]: transmission of video signals over the international telecommunication network for the purpose of interchanging television material between broadcasting organizations in different countries.

6.614	Internet Control Message Protocol (ICMP) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: An Internet network-layer protocol.

6.614bis 	Internet control message protocol (ICMP) [b-ITU-T J.161]: An extension to the Internet Protocol, ICMP supports packets containing error, control and information messages.

6.615	Internet group management protocol (IGMP)	[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: A network-layer protocol for managing multicast groups on the Internet.

6.616	Internet key exchange [b-ITU-T J.177]: A key-management mechanism used to negotiate and derive keys for SAs in IPSec.

6.617	Internet Protocol (IP) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.120], [b-ITU-T J.122], [b-ITU-T J.212], [b-ITU-T J.296]: An Internet network-layer protocol that defines the addressing mechanism on the Internet to allow data to be transmitted, based on [b-IETF RFC 791].

6.617bis	Internet protocol (IP) [b-ITU-T J.380.7]: A protocol by which data is sent from one computer to another computer over a network.

6.618	Internet Protocol Detail Record (IPDR) [b-ITU-T J.204]: The fundamental unit of data transferred between an Exporter and a Collector. It is defined by a Template and contains Fields.

6.619	Internet Protocol Detail Record/Streaming Protocol (IPDR/SP) [b-ITU-T J.204]: The protocol used to transfer Data Messages and Data Records between Exporter and Collectors.

6.620	Internet type remote control unit (RCU) [b-ITU-T J.295]: A terminal with RCU functionalities that allow users to control the set-top box from a remote location via the Internet, e.g., remote setting of schedule recording. This type of RCU may be mobile phones (smart phones, feature phones, etc.), tablet devices, PCs, etc. In addition to communicating and controlling the set-top box, the Internet type RCU can also access directly services provided by cable operators or other content and/or service providers.

6.621	interoperability [b-ITU-T J.200]: The reception and presentation of applications in a vendor-, author- and broadcaster-neutral framework.

6.622	interoperable function [b-ITU-T J.96]: refers to a decryption function that shall be embedded in all units.

The bits in binary numbers or sequences are numbered from the left, according to engineering notation. Bit 0 is on the right and is the least significant one; the bit on the left is the most significant one.

6.623	interstitial [b-ITU-T J.380.3]: A placement opportunity occurring during the play out of an entertainment asset (also commonly referred to as a mid-roll).

6.624	interval usage code (IUC) [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.222.1]: A field in MAPs and UCDs to link burst profiles to grants.

6.625	interworking function (IWF)	[b-ITU-T J.214]: An interworking function (IWF) is a logical entity. It consists of a TSP data interface on one side, an IP packet interface on the other side, and the functionalities to encode TDM data into a pseudo wire in one direction and decode TDM data from a pseudo wire in the other direction.

6.626	intrusion resistance [b-ITU-T J.93], [b-ITU-T J.95]: The ability of a hardware object to deny physical, electrical, or irradiation-_x005f based access to internal functionality by unauthorized parties.

6.627	IP address [b-ITU-T J.120]: The network layer address defined by the Internet Protocol. This address is mapped onto the layer one address of the respective system.

6.628	IP enhanced cable modem [b-ITU-T J.191]: A cable modem that has been enhanced by the addition of the IP features of this Recommendation.

6.629	IP interactive service [b-ITU-T J.295]: A type of service provided by the cable operator or service provider where the subscriber requests and/or responds to services.

6.630	IP multicast [b-ITU-T J.292]: It is used for IP broadcasting in terms of bandwidth use efficiency of headend, CDN (Content Distribution Network), and CPE segment.

6.631	IP stream (IPS) [b-ITU-T J.282]:	A flow of IP packets that is identified by destination/source IP address and port number.

6.632	IP VoD	[b-ITU-T J.1005]: A service to deliver video content following a request from a user. IP-VOD supplies each video content on an on-demand basis.

6.633	IP-based services [b-ITU-T J.193]: A generic term that includes QoS-controllable both-way and one-way IP-type services rendered over a CATV network on which a high-speed packet-based IP communication system is available. Examples include IP telephony or VoIP, videoconferencing, streaming video feeds, competitive games, and other similar services.

6.634	IP-based switched digital video (SDV) [b-ITU-T J.1101], [b-ITU-T J.1102]:	A service mechanism which provides interfaces and functionalities to enable cable television system operators to offer QoS-guaranteed broadcasting and multicasting.

6.635	IPCable2Home [b-ITU-T J.291]: The domain in MediaHomeNet that is well bounded and specified based on Internet Protocol Layer 3 interoperability, as opposed to other domains that can be independently, arbitrarily, or privately designed to an individual manufacturer's specification.

6.636	IPCable2Home security portal (CSP) [b-ITU-T J.192]: A functional element that provides security management and translation functions between the HFC and Home network.

6.637	IPCablecom	[b-ITU-T J.160], [b-ITU-T J.162],[b-ITU-T J.163], [b-ITU-T J.164], [b-ITU-T J.166], [b-ITU-T J.167], [b-ITU-T J.171.1], [b-ITU-T J.291]: An ITU-T project that includes an architecture and a series of Recommendations that enable the delivery of real-time services over the cable television networks using cable modems.

6.638	IPCablecom multimedia	[b-ITU-T J.360]: An application agnostic QoS architecture for services delivered over DOCSIS networks.

6.639	IPCablecom transaction [b-ITU-T J.164]: An IPCablecom transaction is a collection of events on the IPCablecom network when delivering a service to a subscriber. Event Messages for the same transaction are identified by one unique billing correlation ID (as described in Table 39). For some services, multiple transactions may be required to provide information that is necessary to collect the total usage for the service. Multiple Event Messages may be required to track resources for each individual service used. A transaction may persist over time.

6.640	IPCablecom2 application manager (PAM) [b-ITU-T J.368]: A specialized Application Manager defined in IPCablecom2, primarily responsible for determining QoS resources needed for a session.

6.641	IPCablecom-T [b-ITU-T J.179]: The suite of IPCablecom ITU-T Recommendations that support telephone service.

6.642	IPDR/SP collector functionality [b-ITU-T J.204]:	An implementation on the data-receiving side of the IPDR/SP. It enables the reception and collection of Data Records from IPDR/SP Exporters. It is typically part of an OSS/BSS, or a mediation system.

6.643	IPDR/SP exporter functionality [b-ITU-T J.204]:	An implementation on the data-producing side of the IPDR/SP. It enables formatting and sending of Data Records to an interested consumer system, e.g., at a cable headend using the IPDR/SP.

6.644	IPDR-Type [b-ITU-T J.204]: A constraint on the value and format of an individual Field within a Data Record; e.g., dateTime.

6.645	IPNet2Home [b-ITU-T J.190]: The domain in MediaHomeNet that is well bounded and specified based on Internet protocol layer 3 interoperability, as opposed to other domains that can be independently, arbitrarily, or privately designed to an individual manufacturer's specification.

6.646	IP-only CPE	[b-ITU-T J.700]: A terminal device that is capable of receiving content services over IP only.

6.647	IPTV TD [b-ITU-T J.702]: A terminal device which has ITF functionality, e.g., a set-top box (STB).

6.648	IPTV terminal function (ITF)	[b-ITU-T J.702]: The client-side function(s) associated with a) receiving and responding to network control channel messages regarding session set-up, maintenance, and tear down, and b) receiving the content of an IP transport from the network and rendering.

6.649	Japan Cable Television Engineering Association (JCTEA) [b-ITU-T J.112]: A body responsible for developing standards concerning cable television systems in Japan.

6.650	JavaScript [b-ITU-T J.296]:	JavaScript is a scripting language for a Web browser. JavaScript is an implementation of the ECMAScript language standard and is primarily used in the form of client-side JavaScript, implemented as part of a Web browser in order to provide enhanced-user interfaces and dynamic websites.

6.651	jitter [b-ITU-T J.161], [b-ITU-T J.361], [b-ITU-T J.460.0]: Variability in the delay of a stream of incoming packets making up a flow such as a voice communication.

6.652	Joint Photographic Experts Group (JPEG) (based on [ISO/IEC 10918]) [b-ITU-T J.296]:	JPEG is a standard format for compressing pictures. The degree of compression can be adjusted, allowing a selectable tradeoff between storage size and image quality.

6.653	Kerberos [b-ITU-T J.170]: A secret-key network authentication protocol that uses a choice of cryptographic algorithms for encryption and a centralized key database for authentication.

6.653bis	Kerberos [b-ITU-T J.369]: Authentication protocol allowing one network entity (Client) to be mutually authenticated to another one (Application Server) using the "Kerberos ticket" retrieved by the Client from a dedicated Authentication Server (KDC).

6.654	key [b-ITU-T J.170], [b-ITU-T J.178], [b-ITU-T J.361]: A mathematical value input into the selected cryptographic algorithm.

6.655	key exchange [b-ITU-T J.170]: The swapping of public keys between entities to be used to encrypt communication between the entities.

6.656	key features [b-ITU-T J.193]: Features that MUST be included in the NG-STB in order to support the Core Services identified in this Recommendation.

6.657	key management [b-ITU-T J.170]: The process of distributing shared symmetric keys needed to run a security protocol.

6.658	Key transition period [b-ITU-T J.222.3]: The time period in which an Authentication Key that is near its expiration is replaced by a new Authentication Key through a negotiated update process between the CMTS and the CM.

6.659	KeyPairingID [b-ITU-T J.1004]: The value of the concatenation of CAM_ID and DSC_ID, i.e., CAM_ID||DSC_ID.

6.660	Ki	[b-ITU-T J.1004]: The pre-shared key having a size of 128 bits. The AC uniquely assigns three Ki to each CAM. Ki should be a generated random generation function.

6.661	KPK [b-ITU-T J.1004]: Key pairing key (KPK). The authorization centre (AC) generates the KPK if the KeyPairingID is valid.

6.662	L2 forwarder [b-ITU-T J.213]: A network element that forwards layer-2 packets from one L2 interface to another L2 interface. A layer-2 forwarder may operate in point-to-point or multipoint forwarding mode, i.e., forwarding between only two interfaces without learning; or multipoint, forwarding unicast-destined packets only to the interface from which a MAC address was learned.

6.663	L2 interface [b-ITU-T J.213]: A physical interface port or virtual circuit on which an L2PDU is transmitted. Physical L2 interface ports include an Ethernet NSI at a CMTS or the CMCI port at a CM. Virtual circuit L2 interfaces include a CMTS network system interface (NSI) pseudowire (PW) and a CMTS single-CM BPI security association. An L2 interface may or may not have an ifIndex assigned to it.

6.664	L2 protocol data unit (L2PDU) [b-ITU-T J.213]: A sequence of bytes consisting of a destination MAC address (DMAC), source MAC address (SMAC), (optional) tag header(s), EtherType/Length, L2 payload, and CRC.

6.665	L2 virtual private network (L2VPN) [b-ITU-T J.213]: A set of LANs and the L2 forwarders between them that enable hosts attached to the LANs to communicate with layer-2 protocol data units (L2PDUs). A single L2VPN forwards L2PDUs based only on the destination MAC (DMAC) address of the L2PDU, transparent to any IP or other layer-3 address. A cable operator administrative domain supports multiple L2VPNs, one for each subscriber enterprise to which transparent LAN service is offered.

6.666	L2TP access concentrator (LAC) [b-ITU-T J.212]: If an L2TP Control Connection Endpoint (LCCE) is being used to cross connect an L2TP session directly to a data link, we refer to it as an L2TP Access Concentrator (LAC). An LCCE may act as both an L2TP Network Server (LNS) for some sessions and LAC for others, so these terms must only be used within the context of a given set of sessions unless the LCCE is in fact single purpose for a given topology.

6.667	L2TP attribute value pair (AVP) [b-ITU-T J.212]: The L2TP variable-length concatenation of a unique Attribute (represented by an integer), a length field, and a Value containing the actual value identified by the attribute.

6.668	L2TP control connection	[b-ITU-T J.212]: An L2TP control connection is a reliable control channel that is used to establish, maintain, and release individual L2TP sessions as well as the control connection itself.

6.669	L2TP control connection endpoint (LCCE) [b-ITU-T J.212]: An L2TP node that exists at either end of an L2TP control connection. May also be referred to as LAC or LNS, depending on whether tunnelled frames are processed at the data link (LAC) or network layer (LNS).

6.670	L2TP control connection ID [b-ITU-T J.212]: The Control Connection ID field contains the identifier for the control connection, a 32-bit value. The Assigned Control Connection ID AVP, Attribute Type 61, contains the ID being assigned to this control connection by the sender. The Control Connection ID specified in the AVP must be included in the Control Connection ID field of all control packets sent to the peer for the lifetime of the control connection. Because a Control Connection ID value of 0 is used in this special manner, the zero value must not be sent as an Assigned Control Connection ID value.

6.671	L2TP control message [b-ITU-T J.212]: An L2TP message used by the control connection.

6.672	L2TP data message [b-ITU-T J.212]: An L2TP message used by the data channel.

6.673	L2TP endpoint [b-ITU-T J.212]: A node that acts as one side of an L2TP tunnel.

6.674	L2TP network server (LNS) [b-ITU-T J.212]: If a given L2TP session is terminated at the L2TP node and the encapsulated network layer (L3) packet processed on a virtual interface, we refer to this L2TP node as an L2TP Network Server (LNS). A given LCCE may act as both an LNS for some sessions and LAC for others, so these terms must only be used within the context of a given set of sessions unless the LCCE is in fact single purpose for a given topology.

6.675	L2TP pseudowire (PW) [b-ITU-T J.212]: An emulated circuit as it traverses a packet-switched network. There is one Pseudowire per L2TP Session.

6.676	L2TP pseudowire type [b-ITU-T J.212]: The payload type being carried within an L2TP session. Examples include PPP, Ethernet, and Frame Relay.

6.677	L2TP session [b-ITU-T J.212]: An L2TP session is the entity that is created between two LCCEs in order to exchange parameters for and maintain an emulated L2 connection. Multiple sessions may be associated with a single Control Connection.

6.678	L2TP session ID [b-ITU-T J.212]:	A 32-bit field containing a non-zero identifier for a session. L2TP sessions are named by identifiers that have local significance only. That is, the same logical session will be given different Session IDs by each end of the control connection for the life of the session. When the L2TP control connection is used for session establishment, session IDs are selected and exchanged as Local Session ID AVPs during the creation of a session. The Session ID alone provides the necessary context for all further packet processing, including the presence, size, and value of the Cookie, the type of L2-Specific Sublayer, and the type of payload being tunnelled.

6.679	L2VPN identifier [b-ITU-T J.213]: An octet string that uniquely identifies an L2VPN within a cable operator administrative domain, corresponding to a single subscriber enterprise.

6.680	L3 forwarder	[b-ITU-T J.213]: A network element that forwards a layer-3 PDU from an ingress interface to one or more egress interfaces. Also called a "router".

6.681	label [b-ITU-T J.260]: An identifier occurring within or attached to data elements. In the context of preferential telecommunications it is an indication of priority. This identifier can be used as a mapping mechanism between different network priority levels.

6.682	LAN IP device [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.193], [b-ITU-T J.290]:A LAN IP device is representative of a typical IP device expected to reside on home networks, and is assumed to contain a TCP/IP stack as well as a DHCP client

6.682bis	LAN IP device [b-ITU-T J.190]: A component using the Internet protocols on a local area network.

6.683	latency	[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.161]: The time, expressed in quantity of symbols, taken for a signal element to pass through a device.

6.683bis	 latency [b-ITU-T J.361]: The time taken for a signal to pass through a device or network.

6.683ter	 latency [b-ITU-T J.460.0]: The time taken for a signal element to pass through a device or circuit.

6.684	Latency time [b-ITU-T J.145]: The absolute delay of a signal between the signal source and the signal destination. The latency time can be divided into codec latency time, i.e. the latency ascribed to the codec equipment in the connection, and network latency time dependent on the type and length of the transmission channel.

6.685	layer [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A subdivision of the Open System Interconnection (OSI) architecture, constituted by subsystems of the same rank.

6.686	learning [b-ITU-T J.213]: An operation of a layer-2 bridge by which it associates the source MAC (SMAC) address of an incoming L2PDU with the bridge port from which it arrived.

6.687	leg [b-ITU-T J.365]: A single segment of a session associated with a UE (e.g., calling UE or called UE).

6.688	legacy application 	[b-ITU-T J.296]: An application or applications to support legacy services if the cable operator wishes.

6.689	linear TV [b-ITU-T J.702]: A television service in which a continuous stream flows in real time from the service provider to the terminal device and where the user cannot control the temporal order in which contents are viewed.

6.690	line-up period [b-ITU-T N.54]: The period during which the telecommunication Administrations line up the international television link before handing it over to the broadcasting organizations.

6.691	Line-time waveform distortion [b-ITU-T J.67]: If a square-wave signal with a period of the same order as one line and of nominal luminance amplitude is applied to the input of the circuit, the line-time waveform distortion is defined as the change in shape of the square wave at the output. A period at the beginning and end of the square wave, equivalent to a few picture elements, is excluded from the measurement.

6.692	link maintenance [b-ITU-T J.195.1]: The function of estimating and exchanging parameters of links between a master node and client nodes to adapt to the variation of channel characteristics and maintain the steady working of the system.

6.693	lip synchronization (lip-sync)	[b-ITU-T J.248]: Operation to provide the feeling that the speaking motion of the displayed person is synchronized with that person's voice, or other sounds are synchronized to their visually displayed source. Alternatively, the minimization of the relative delay between the visual display of a person speaking and the audio of the voice of the person speaking. The objective is to achieve a natural relationship between the visual image and the aural message for the viewer/listener.

6.694	live network conditions [b-ITU-T J.246], [b-ITU-T J.247]: Errors imposed upon the digital video bit stream as a result of live network conditions. Examples of error sources include packet loss due to heavy network traffic, increased delay due to transmission route changes, multi-path on a broadcast signal, and fingerprints on a DVD. Live network conditions tend to be unpredictable and unrepeatable.

6.695	live transmission [b-ITU-T J.127]: Program transmission method whereby the program starts playing after a certain amount of data has been buffered while receiving subsequent data in the background, where the program is fed in real time by the content provider.

6.696	Load Balancing Group [b-ITU-T J.222.2]: A full or partial subset of a MAC Domain Cable Modem Service Group (MD-CM-SG) to which a CM is administratively assigned. LBGs contain at least one upstream channel and at least one downstream channel.

6.697	local application service delivery platform (LASDP)	[b-ITU-T J.294]: An application and service execution environment that resides within the home network for the purpose of delivering advanced applications and services developed by either a service provider or by a third party to the cable service subscribers.

6.698	Local Area Network (LAN) [b-ITU-T J.112]:	A data network in which serial transmission is used for direct data communication among data stations located on the user's premises.

6.698bis local area network (LAN) [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A non-public data network in which serial transmission is used for direct data communication among data stations located on the user's premises.

6.699	local number portability	[b-ITU-T J.177]: Allows a customer to retain the same number when switching from one local service provider to another.

6.700	logical (upstream) channel [b-ITU-T J.122]: A MAC entity identified by a unique channel ID and for which bandwidth is allocated by an associated MAP message. A physical upstream channel may support multiple logical upstream channels. The associated UCD and MAP messages completely describe the logical channel.

6.701	logical CPE interface [b-ITU-T J.126]: A bidirectional, data-only 802.3/Ethernet MAC frame interface between eCM and an eSAFE.

6.702	logical CPE interface (LCI) [b-ITU-T J.214]:	A logical CPE interface (LCI) is a logical 802.3/Ethernet MAC frame data interface.

6.703	logical element [b-ITU-T J.190]: A collection of one or more functions.

6.704	Logical Link Control (LLC) procedure	[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: In a local area network (LAN) or a Metropolitan Area Network (MAN), that part of the protocol that governs the assembling of data link layer frames and their exchange between data stations, independent of how the transmission medium is shared.

6.705	logical service [b-ITU-T J.380.1]:	A well-defined, self-contained set of functions which is the endpoint of a connection. The logical service has some type of underlying computer system that supports message communication.

6.705bis	logical service [b-ITU-T J.380.2]:	A well-defined, self-contained set of functions accessible via one or more endpoints. The logical service has some type of underlying computer system that supports message communication.

6.706	long form insertion [b-ITU-T J.287]: Refers to insertions of material with a duration generally greater than 10 minutes, i.e., program length material.

6.707	Long-time waveform distortion [b-ITU-T J.67]: If a test signal, simulating a sudden change of the luminance from a black level to a white level or vice versa, is applied to the input of a circuit, a long-time waveform distortion is present if the variations of the clamp level (medium grey) of the output signal do not precisely follow those of the clamp level of the input signal. This failure may be either in exponential form, or more frequently in the form of damped very low frequency oscillations.

6.708	loop latency [b-ITU-T J.146]: The total transit time of signals in a transmission circuit arranged in a loop configuration.

6.709	loss of frame [b-ITU-T J.214]: Also called the red alarm. When a T1 CSU/DSU is unable to synchronize framing patterns with the remote end for 2.5 seconds, LoF is declared.

6.710	loss of signal 	[b-ITU-T J.214]: When no incoming pulses are received by a T1 CSU/DSU for a prescribed number of bit times, LoS is declared. Even if only zeros were transmitted as data, some framing bits should result in pulses on the line during that time. If LoS persists, LoF will eventually be declared because there is no incoming signal with which to synchronize.

6.711   Low-frequency errors [b-ITU-T J.64]: This parameter is defined as the peak-to-peak amplitude of the fluctuations of the blanking level, measured in a frequency band from 10 Hz to 2 kHz, and expressed as a percentage of the amplitude of the luminance bar (element B2). Further information is given in [CCIR, 1974-78].

6.712	Low frequency noise [b-ITU-T J.67]: The signal-to-noise ratio for low frequency noise is defined as the ratio, expressed in decibels, of the nominal amplitude of the luminance signal (1 V) to the mean square value of the noise.

6.713	Low frequency non-linear distortion [b-ITU-T J.67]: For a particular value of average picture level, the low frequency non-linear distortion is defined as the departure from proportionality between the amplitude of the input signal and the output signal, when the input signal is shifted from the black level to the white level within the duration of a line period.

6.714	LSDI application	[b-ITU-T J.600]: An instancing of the LSDI service, designed to meet a specific set of user requirements.

6.715	LSDI presentation venue [b-ITU-T J.600]: The venue (a theatre, an auditorium or another venue for group viewing) where LSDI programs are presented to a collective audience.

6.716	LSDI service	[b-ITU-T J.600]: A service whereby programs are distributed in the form of digital signals, in real-time or non-real-time, for collective viewing in theatres or other group venues equipped with appropriate electronic projectors, to provide excellent presentation in terms of picture and sound quality, size of the presentation screen, and presentation environment.

6.717	Lua [b-ITU-T J.200]: Lightweight embeddable scripting language that combines simple procedural syntax with data description constructs, based on associative arrays and extensible semantics.

6.718	Luminance bar amplitude [b- ITU-T J.64]: The luminance bar amplitude is defined as the difference between the level corresponding to the mid-point of the bar (element B2) and the level corresponding to a point immediately following the composite pulse (element F). These points are shown as b2 and b1 respectively in Figs. 1 and 2. It is to be expressed as a percentage of the nominal bar amplitude (0.7 V for 625-line signals, 0.714 V for 525-line signals).

6.719	Luminance bar amplitude error [b- ITU-T J.64]: The luminance bar amplitude error is defined as the difference between the actual luminance bar amplitude and the nominal value expressed as a percentage of the nominal value (0.7 V for 625-line signals, 0.714 V for 525-line signals).

6.720	Luminance non-linearity [b- ITU-T J.64]: The luminance non-linearity is to be measured with the staircase signal in line 17 (element D1 for 625-lines, D2 for 525-lines). It is defined as the difference between the largest and the smallest step amplitudes, expressed as a percentage of the amplitude of the largest step. As the sign of the difference is not significant it is taken to be positive.

6.721	M/N [b-ITU-T J.210]: Relationship of integer numbers M, N that represents the ratio of the downstream symbol clock rate to the DOCSIS master clock rate.

6.722	MAC domain [b-ITU-T J.212]: A grouping of layer 2 devices that can communicate with each other without using bridging or routing. In DOCSIS, it is the group of CMs that are using upstream and downstream channels linked together through a MAC forwarding entity.

6.722bis	 MAC domain [b-ITU-T J.222.3]: A logical link layer network consisting of a common address scheme (such as IEEE 802.3 Ethernet) in which elements may send and receive OSI layer 2 messages between and among one another. MAC domain boundaries may be established through both physical and logical means; separate channels or subchannels utilizing differing frequency and/or encoding methods, or assigning separate bundles/bridge groups or subinterfaces to common frequency-domain channels or subchannels.

6.723	MAC frame [b-ITU-T J.122]: MAC header plus optional PDU.

6.724	MAC service access point (MSAP) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.116], [b-ITU-T J.122]: An attachment to a MAC-sublayer domain.

6.725	macro block [b-ITU-T J.88]: A size of 16 pixels ´ 16 lines composed of four 8 ´ 8 Walsh Hadamard Transform.

6.726	managed IP backbone [b-ITU-T J.160]:	A Managed IP network that is used for interconnecting IPCablecom domains.

6.727	managed IP network [b-ITU-T J.160]: An IP network, managed by a single entity for the purpose of transporting IPCablecom signalling and media packets.

6.728	management	[b-ITU-T J.460.4]: Management refers to the protocols, methodologies and interfaces that enable oversight services in a Service Provider Network.

6.729	management centre [b-ITU-T J.96]: refers to an organization controlling or managing the conditional access system.

6.730	management information base [b-ITU-T J.166]: The specification of information in a manner that allows standard access through a network management protocol.

6.730bis	 management information base [ [b-ITU-T J.296]: A virtual database used for managing the entities in a communications network. Most often associated with the Simple Network Management Protocol (SNMP), the term is also used more generically in contexts such as in the OSI/ISO network management model.

6.730ter management information base [b-ITU-T J.370]: The description of the data items used by the Network Management for management and configuration of the IPCablecom compliant E-UE. Such description is done based on the formal meta-language SMI defined by the corresponding IETF standards.

6.730quater  management information base (MIB) [b-ITU-T J.460.4]: The description of the data items used by the Network Management for management and configuration of the IPCablecom compliant E-UE. Such description is done based on the formal meta-language SMI defined by the corresponding IETF standards.

6.731	management L2VPN [b-ITU-T J.213]: An L2VPN for the post-registration SNMP traffic to eCM or eSAFE devices. May be combined with a provisioning L2VPN.

6.732	MAP cycle [b-ITU-T J.195.3]: A period of time planned by a MAP frame.

6.733	markup language [b-ITU-T J.200]: A formalism that describes a document's structure, appearance, or other aspects. An example of a markup language is XHTML [b-W3C HTML1].

6.734	master headend [b-ITU-T J.112], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: A headend which collects television program material from various sources by satellite, microwave, fibre and other means, and distributes this material to Distribution Hubs in the same metropolitan or regional area. A Master Headend may also perform the functions of a Distribution Hub for customers in its own immediate area.

6.734bis	 master headend [b-ITU-T J.112 Annex B], [b-ITU-T J.116]:	A headend which collects television program material from various sources by satellite, microwave, fibre and other means, and distributes this material to Distribution Hubs in the same metropolitan or regional area.

6.735	maximum downstream bonded channels (MDBC) [b-ITU-T J.222.1]: Maximum number of downstream bonded channels supported by the cable modem. (See 6.191: channel bonding.)

6.736	maximum time interval error (MTIE) [b-ITU-T J.211]: For a sequence of time delay samples xi, MTIE at observation time (S) is:

MTIE measurement:

where:

τo = sample period

N = number of samples in the sequence

n = [S/τo] + 1

S = observation time

xi = time delay sample

6.737	maximum transmission unit (MTU)	[b-ITU-T J.212]: The layer 3 payload of a layer 2 frame.

6.738	Mean Time to Repair (MTTR) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: In cable television systems, the MTTR is the average elapsed time from the moment a loss of RF channel operation is detected up to the moment the RF channel operation is fully restored.

6.739	Mean value of clock run-in [b-ITU-T J.101]: The mean value of clock run-in is defined as the mean level of the clock run-in waveform excluding the first two bits.

6.740	Measurement signal (MS) [b-ITU-T J.27]: Sine-wave signal at 1 kHzNOTE at a level 12 dB below the alignment signal level, which should be used for long-term measurements and measurements at all frequencies.

NOTE: This frequency is nominal, and 1020 Hz recommended by Recommendation ITU-T O.33 may be used.

6.741	media access control (MAC) [b-ITU-T J.210], [b-ITU-T J.212]: Used to refer to the layer 2 element of the system which would include DOCSIS framing and signalling.

6.742	Media Access Control (MAC) address [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: The "built-in" hardware address of a device connected to a shared medium.

6.743 media access control (MAC) procedure [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]:	In a subnetwork, that part of the protocol that governs access to the transmission medium independent of the physical characteristics of the medium, but taking into account the topological aspects of the subnetworks, in order to enable the exchange of data between nodes. MAC procedures include framing, error protection, and acquiring the right to use the underlying transmission medium.

6.744	Media Access Control (MAC) sublayer [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: The part of the data link layer that supports topology-dependent functions and uses the services of the Physical Layer to provide services to the Logical Link Control (LLC) sublayer.

6.745	media data box [b-ITU-T J.123], [b-ITU-T J.124]: A container box which can hold the actual media data for a presentation ('mdat').

6.746	media gateway [b-ITU-T J.460.0], [b-ITU-T J.460.1]: Devices bridging between the IPCablecom2 IP Voice Communication network and the PSTN. A Media Gateway provides the bearer circuit interfaces to the PSTN and transcodes the media stream.

6.746bis	 media gateway (MG) [b-ITU-T J.161], [b-ITU-T J.177], [b-ITU-T J.361]: Provides the bearer circuit interfaces to the PSTN and transcodes the media stream.

6.747	media gateway control protocol (MGCP) [b-ITU-T J.291]: A protocol used within a Voice over IP system.

6.748	media gateway controller (MGC) [b-ITU-T J.161]:	The overall controller function of the PSTN gateway. Receives, controls and mediates call-signalling information between the IPCablecom and PSTN.

6.749	media presentation description (MPD) [b-ITU-T J.181 Amendment 1]:	A formalized description for a DASH Media Presentation for the purpose of providing a streaming service,

6.750	media terminal adapter (MTA) [b-ITU-T J.166]: Contains the interface to a physical voice device, a network interface, CODECs, and all signalling and encapsulation functions required for VoIP transport, class features signalling and QoS signalling.

6.750bis	 Media Terminal Adapter (MTA) [b-ITU-T J.173]:	An MTA is an IPCablecom client that can be attached to a CM (stand-alone) or integrated with a CM (embedded) that supports POTS.

6.751	MediaHomeNet [b-ITU-T J.190]:	An ITU-T project that includes an architecture and a series of Recommendations that support the delivery of services over home networks. A network that connects multiple elements in a home environment to allow delivery of multi-purpose, multimedia services.

6.751bis	 MediaHomeNet [b-ITU-T J.294]: A network that connects multiple elements in a home environment to allow delivery of multi-purpose, multimedia services.

6.752	message	[b-ITU-T J.94]: The more general term message is used interchangeably with section, especially to refer to non-table-oriented data structures such as, for example, the SYSTEM TIME message. Likewise, the term message is used to refer to a data structure that may deliver portions of various types of tables. The NETWORK INFORMATION message, for example, defines portions of several types of network tables.

6.752bis	 message[b-ITU-T J.287]: In the context of this document a message is a single communication between the automation system and the compression system or between the automation system and the PAMS. A message may contain one or more operations.

6.752ter	 message [b-ITU-T J.380.1], [b-ITU-T J.380.2]:	The unit of communication between two logical service s.

6.753	metadata [b-ITU-T J.98]: Metadata is descriptive data associated with a content asset package or file. It may vary in depth from merely identifying the content package title or information to populate an EPG to providing a complete index of different scenes in a movie or providing business rules detailing how the content package may be displayed, copied, or sold. Separate uses for metadata have originated from the studios, distribution networks (Cable, Satellite), down to the CPE (STBs, PVRs).

6.753bis	 metadata [b-ITU-T J.702]: Structured, encoded data that describe characteristics of information-bearing entities to aid in the identification, discovery, assessment, and management of the described entities.

NOTE: EPG metadata has many applications and may vary in depth from merely identifying the content package title or information to populate an EPG to providing a complete index of different scenes in a movie or providing business rules detailing how the content package may be displayed, copied, or sold.

6.754	micro-reflections	[b-ITU-T J.112]: Echoes in the forward transmission path due to departures from ideal amplitude and phase characteristics of the path.

6.754bis	 micro-reflections [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: Echoes in the forward transmission path due to departures from ideal amplitude and phase characteristics.

6.755	mid split [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.122]: A frequency division scheme that allows bidirectional traffic on a single coaxial cable; e.g. in North America, reverse channel signals propagate to the headend from 5 to 108 MHz, the forward path signals go from the headend from 162 MHz to the upper frequency limit, and the diplex crossover band is located from 108 to 162 MHz.

6.755bis  mid split [b-ITU-T J.112 Annex C]: Frequency division scheme that allows bidirectional traffic on a single coaxial cable. Reverse channel signals propagate to the headend. Forward path signals go from the headend.

6.756	middleware	[b-ITU-T J.193]: Software within the NG-STB which provides a set of APIs that against which applications can be developed, and that provide access to the resources and services of the NG-STB.

6.756bis	 middleware [b-ITU-T J.294]: Software within the H-STB which provides a set of APIs that against which applications can be developed, and that provide access to the resources and services of the H-STB.

6.756ter	 middleware [b-ITU-T J.296]: Software within the STB that provides a set of application programming interfaces (APIs) against which applications can be developed, and that provide access to the resources and services of the STB.

6.757	mini-slot [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.116], [b-ITU-T J.122]: A mini-slot is an integer multiple of 6.25-microsecond increments.

6.757bis mini-slot [b-ITU-T J.112 Annex C]: "Mini-slot" is an integer multiple of 64/9.216 microsecond increments.

6.758	mix mode [b-ITU-T J.88]: A mode of operation in which inter-frame and intra-frame modes are mixed in the same macro block to enhance coding efficiency.

6.759	mobile DVR (mDVR) [b-ITU-T J.700]:	A mobile instance of a DVR where a mobile terminal device can be a cDVR (having the capability to store video, audio and other associated content locally) or contain a means of accessing an nDVR or other cDVR.

6.760	Modular Cable Modem Termination System [b-ITU-T J.222.2]: A CMTS composed of discrete functional blocks linked together using Gigabit Ethernet links.

6.761	modulation error ratio (MER)	[b-ITU-T J.210], [b-ITU-T J.212]: The ratio of the average symbol power to average error power.

6.762	modulation rate [b-ITU-T J.122]: The signalling rate of the upstream modulator (1280 to 5120 kHz). In S-CDMA, the chip rate. In TDMA, the channel symbol rate.

6.763	module	[b-ITU-T J.93], [b-ITU-T J.95]: A small device, not working by itself, designed to run specialized tasks in association with a host.

6.764	monolithic firmware image [b-ITU-T J.126]: A single firmware image containing one or more code images for the entire eDOCSIS device. For eDOCSIS devices, the Monolithic Firmware Image contains both the eCM code image as well as the applicable eSAFE code image. As an example for an eDOCSIS device containing an eSTB, the Monolithic Firmware Image contains the eCM code image as well as the eSTB code image (which may also be composed of multiple eSTB code images).

6.765	movie box [b-ITU-T J.123], [b-ITU-T J.124]: A container box whose sub-boxes define the metadata for a presentation ('moov').

6.766	Moving Pictures Expert Group-1 (MPEG-1) (based on [ISO/IEC 11172]) [b-ITU-T J.296]: MPEG-1 is a data compression coding technology including video and audio, which is standardized by the International Organization for Standardization.

6.767	Moving Pictures Expert Group-2 (MPEG-2) (based on [ITU-T H.262] and [ITU-T J.94]) [b-ITU-T J.296]:	 MPEG-2 is a compression and coding technology for data (such as moving images and audio data) specified by the International Organization for Standardization.

6.768	MPEG transport CPE [b-ITU-T J.700]: A terminal device that is capable of receiving content services over MPEG-2 transport streams only.

6.769	MPEG-2 [b-ITU-T J.142]: Coding system of video and audio signals defined in ISO/IEC Standard 13818, ITU-T H.222 and H.262.

6.769bis	 MPEG-2 [b-ITU-T J.180]: Refer to ISO/IEC 13818, where System is defined in [ITU-T H.222.0], Video coding is defined in [ITU-T H.262] and Audio coding is defined in 13818-3.

6.770	MPEG-2 Transport Stream (TS) packet	[b-ITU-T J.131], [b-ITU-T J.132]: A data packet possessing a length of 188 bytes including 4 bytes of header information. The header contains MPEG related data.

6.771	MPEG-2 video [b-ITU-T J.286]: The video coding format defined in [ITU-T H.222.0]. The transport syntax of a spliceable bit stream is recommended in [ITU-T H.222.0], so called MPEG-2 Transport Stream or MPEG-2 TS.

6.772	multi program transport stream (MPTS) [b-ITU-T J.181], [b-ITU-T J.291]: A transport stream with multiple programs.

6.773	multicast [b-ITU-T J.283]: A packet delivery mechanism from one source to many clients supported by IP routers.

6.774	multicast subscription database [b-ITU-T J.218]: A simple table of entries for the IPv4 or IPv6 multicast group membership information maintained by the eRouter on respective interfaces. Implementation details for storage of records is completely vendor-defined.

6.775	multichannel video distribution [b-ITU-T J.295]: A digital television distribution service over cable networks that provides a wide range of multichannel television programmes to subscribers.

6.776	multi-DRM [b-ITU-T J.1005]: A system which can select suitable DRM from two or more DRMs based on a special service feature, the load of a system, a network situation, etc.

6.777	multimedia [b-ITU-T J.148]: The combination of multiple forms of media such as audio, video, text, graphics, fax, and telephony in the communication of information.

6.778	Multimedia Cable Network System (MCNS) partners [b-ITU-T J.112]: A consortium of several cable television operators interested in deploying high-speed data communications systems on cable television systems.

6.779	multimedia center equipment [b-ITU-T J.112]: Equipment located at cable television headend, which provides complementary functionality to the Multimedia Home Equipment to enable data connectivity to a wide-area network.

6.780	multimedia home equipment [b-ITU-T J.112]: A modulator-demodulator at subscriber locations intended for use in conveying data communications on a cable television system.

6.781	multimedia hypermedia experts group-5 (MHEG-5)	[b-ITU-T J.200]: A specification [b-ISO/IEC 13522-5] for presentation engine applications designed for decoding in interactive television receivers using modest resources. The UK profile [b-MHEG Profile], which is recognized within the ISO standard, extends the specification. It shares common text and graphics formats, and carousel mechanism with multimedia home platform (MHP), thus allowing the use of common data between MHEG-5 and MHP applications, with only a small overhead.

6.782	multimedia session	[b-ITU-T J.360]: A set of multimedia senders and receivers and the data streams flowing from senders to receivers. A multimedia conference is an example of a multimedia session.

6.783	multimedia terminal adapter (MTA) [b-ITU-T J.126]: An IPCablecom device that contains the interface to a physical voice device, a network interface, CODECs, and all signalling and encapsulation functions required for VoIP transport, class features signalling and QoS signalling.

6.783bis	 multimedia terminal adapter (MTA) [b-ITU-T J.161]: Contains the interface to a physical voice device, a network interface, codecs, and all signalling and encapsulation functions required for VoIP transport, class features signalling and QoS signalling.

6.783ter	 multimedia terminal adapter (MTA) [b-ITU-T J.190]: Defined by IPCablecom as an element that provides IP packetized multimedia services.

6.784	Multiple Outstanding Requests [b-ITU-T J.222.2]: The ability of the cable modem to make additional bandwidth requests for new packets for a service flow while one or more previous requests for older packets remain unfulfilled.

6.785	multiple system operator (MSO) [b-ITU-T J.210]: A corporate entity that owns and/or operates more than one cable system.

6.786	multiplex [b-ITU-T J.94], [b-ITU-T J.142]: A stream of all the digital data carrying one or more services within a single physical channel.

6.786bis	 multiplex [b-ITU-T J.280]: A multiplex is a collection of one or more channel(s) that may include the associated service information. A multiplex is an MPEG-2 transport stream with the possible exception of an insertion multiplex.

6.787	multipoint access [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: User access in which more than one terminal equipment is supported by a single network termination.

6.788	multipoint connection [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A connection among more than two data network terminations.

6.789	multipoint L2 forwarding [b-ITU-T J.213]: Operation of an L2 forwarder among multiple L2 networks that forwards individual MAC destined packets only to the interface from which a source MAC address was learned and that floods group MAC destined packets to all interfaces.

6.790	multi-task [b-ITU-T J.295]: A simultaneous process of live viewing or playback and one or multiple recordings of broadcasting content; the viewing of IP streaming content; and other applications such as games.

6.791	mutual isolation [b-ITU-T J.142]: The attenuation between specified system outlets at any frequency within the range of the system under investigation. It is always specified, for any particular installation, as the minimum value obtained within specified frequency limits.

6.792	national circuit [b-ITU-T J.13]: A circuit that connects the ISPC to the broadcasting authority; this applies both at the sending and at the receiving ends. A national circuit may also interconnect two ISPCs within the same country.

6.793	national sound-programme centre (NSPC) [b-ITU-T N.1]: A centre at which two or more national sound-programme circuits terminate and at which national sound-programme circuits may be interconnected.

6.794	national television centre (NTC) [b-ITU-T N.51]: A centre at which two or more national television circuits terminate and at which national television circuits may be interconnected.

6.795	nested context language (NCL) [b-ITU-T J.200]: The nested context language [b-NCL] is a declarative language that is used to describe the temporal behaviour of a multimedia presentation to associate hyperlinks (user interaction) with media objects, to define alternatives for presentation (adaptation), and to describe the layout of the presentation on multiple devices.

6.796	network [b-ITU-T J.94], [b-ITU-T J.183]: A collection of Motion Picture Experts Group version 2 (MPEG-2) transport stream (TS) multiplexes transmitted on a single delivery system, e.g., all digital channels on a specific cable system.

6.796bis	 network [b-ITU-T J.295]: A wired or wireless communication network for the provision of broadcasting and communication services. In this Recommendation, this shall include the in-house communications network, in addition to the access network operated by the cable operator.

6.797	network address translation (based on [ITU-T Y.2111]) [b-ITU-T J.296]: The operation by which IP addresses are translated (mapped) from one address domain to another address domain.

6.798	network DVR (nDVR) [b-ITU-T J.700]:	An instance of a DVR where a network element contains the recording capability that can be solicited and operated by end users to record and store video, audio and other associated content in the network for subsequent playback.

6.799	network enforced preconditions [b-ITU-T J.368]: A method of ensuring that necessary QoS resources are available before alerting the called user by withholding SIP messages within the network until the results of necessary policy interactions are known. This capability is independent from the use by the UE of the similarly named QoS Preconditions capability defined by RFCs 3312 and 3313.

6.800	network information table [b-ITU-T J.296]: The table that carries information to relate transmission path information, such as frequencies to channels, and that lists ID numbers for all the service channels contained in a distribution system.

6.801	network layer [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: Layer 3 in the Open Systems Interconnection (OSI) architecture; the layer that provides services to establish a path between open systems.

6.801bis	 network layer [b-ITU-T J.178]: Layer 3 in the Open System Interconnection (OSI) architecture that provides network information that is independent from the lower layers.

6.802	network management [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]:	The functions related to the management of data link layer and physical layer resources and their stations across the data network supported by the hybrid fibre/coax system.

6.802bis	 network management [b-ITU-T J.361], [b-ITU-T J.370], [b-ITU-T J.460.4]: The functions related to the management of data across the network.

6.803	network termination (NT) [b-ITU-T J.282]: Generic term for equipment that terminates an access network at the user side. NT includes ONU, cable modem and xDSL modem.

6.804	network virtual terminal (NVT) [b-ITU-T J.126]: As defined in the Telnet Protocol. NVT was a bidirectional character device, representing characters as 7-bit ASCII codes, using an 8-bit field.

6.805	Network_id [b-ITU-T J.296]: The identifier assigned to each master transmitter.

6.806	Next Generation Set-Top-Box (NG-STB)	[b-ITU-T J.193]: A compilation of hardware and software functional entities contained within one or more physical devices, that at a baseline level provides the receiving functions for cable Broadcast services. In addition, the NG-STB MAY support the interactive functions of IP-based services, additional time-critical services between the HFC and the Home Network as well as extension and supplemental services.

6.807	NGNA LLC [b-ITU-T J.210]: Company formed by cable operators to define a next-generation network architecture for future cable industry market and business requirements.

6.808	node [b-ITU-T J.240]: Link point in the transmission chain.

6.809	node quitting/deletion [b-ITU-T J.195.1]: The function to support a client node quitting a high performance network over coax (HiNoC) system or a master node deleting a client node from a HiNoC system.

6.810	Nominal signal amplitude [b-ITU-T J.67]: The nominal amplitude of a MAC signal is 1 V. It is defined as the difference between the white level and the black level of the reference signal of line 624.

6.811	Nominal teletext signal amplitude [b-ITU-T J.101]: The nominal teletext signal amplitude is defined as a fixed percentage of the luminance bar amplitude and represents the ideal binary “1” amplitude in any teletext system (See more details in Fig. 1 of J.101). If no luminance bar signal is present, the nominal value of the luminance bar signal is used.

NOTE: The luminance bar amplitude is defined in Recommendation 569. The relationship of the nominal teletext signal amplitude to the luminance bar amplitude is defined in Recommendation 653.

6.812	nominal video signal amplitude [b-ITU-T J.61]: The peak-to-peak amplitude of the monochrome video signal that includes the synchronizing signal and luminance signal component set to peak-white.

6.813	non-compliant CM	[b-ITU-T J.213]: A CM that does not implement this DOCSIS L2VPN Recommendation.

6.814	non-repudiation	[b-ITU-T J.93], [b-ITU-T J.95]:	A process by which the sender of a message (e.g. a request on a pay-per-view) cannot deny having sent the message.

6.814bis non-repudiation [b-ITU-T J.170]: The ability to prevent a sender from denying later that he or she sent a message or performed an action.

6.815	non-STB [b-ITU-T J.1005]: Personal computers (PCs), tablets and smartphone devices other than set-top boxes (STBs) which are capable of handling IP based interactive services.

6.816	normal [b-ITU-T J.287]: A category of request operation supported by this API. See clause 8.3.

6.817	normal mode	[b-ITU-T J.211]: An operating condition of a clock in which the output signals are controlled by an external input reference. The expected mode and state permits each clock within a distribution to have the same long-term average frequency and time. Clocks in this mode are referred to as locked meaning that they are in tight relationship with the DTI root clock. A DTI server clock in a fault-free free-run mode will be considered in normal mode.

6.818	notification [b-ITU-T J.367]: A message sent from the Presence Service to a Subscriber when there is a change in the Presence Information of some Presentity of interest, as recorded in one or more Subscriptions [b-OMA RD-PRS].

6.819	number of allocated codes [b-ITU-T J.122], [b-ITU-T J.222.1]: The total number of codes which a single CM uses in a single S-CDMA frame. This number is determined by the size of the grants in mini-slots and the mapping of these mini-slots to S-CDMA frames (note that a CM may receive multiple grants which are mapped to a single S-CDMA frame). The number of allocated codes can be in the range of the number of codes per mini-slot to the number of active codes, and may vary from frame to frame, but is constant within an S-CDMA frame.

6.820	Number of run-in bits [b-ITU-T J.101]: This parameter counts the number of the “1” and “0” run-in bits present at the start of the teletext waveform prior to the framing code. The result will be always an even number because a “0” bit follows every “1” run-in bit. The counting starts with the first bit with amplitude exceeding the mean value of the clock run-in.

6.821	NW-TD1 [b-ITU-T J.293]: A logical interface defined in 6.1.2 for content reception through an IP network.

6.822	NW-TD2 [b-ITU-T J.293]: A logical interface defined in 6.1.3 for interactive communications to outside of a home network through an IP network.

6.823	oAPR [b-ITU-T J.117]:	A producer-resident register affiliated with a segment buffer that is updated by the consumer to indicate how much data has been consumed. This register also has other bits that are used for demarcation of variable-length frames, and to support the connection disconnection sequence.

6.824	oAPR.count [b-ITU-T J.117]: An internal consumer-local register affiliated with a segment buffer, that indicates how much data has been consumed.

6.825	object [b-ITU-T J.200]: An identifiable entity consisting of data and/or computer code.

6.826	objective perceptual measurement (picture) [b-ITU-T J.144]:	The measurement of the performance of a programme chain by the use of programme-like pictures and objective (instrumental) measurement methods to obtain an indication that approximates the rating that would be obtained from a subjective assessment test.

6.827	off hook	[b-ITU-T J.460.0], [b-ITU-T J.460.1]:	The active state of a traditional telephone, while a call is in progress or being attempted, and the telephone handset is out of its cradle.

6.828	off-net [b-ITU-T J.260]: Not on an IPCablecom network.

6.829	off-net call [b-ITU-T J.161], [b-ITU-T J.178]: A communication connecting an IPCablecom subscriber out to a user on the PSTN.

6.829bis	 off-net call [b-ITU-T J.361]: A communication connecting an IPCablecom subscriber out to a user on other networks such as PSTN or cellular.

6.830	on hook	[b-ITU-T J.460.0], [b-ITU-T J.460.1]:	The idle state of a traditional telephone, while no call is in progress and the telephone handset is sitting in its cradle.

6.831	one-way [b-ITU-T J.128], [b-ITU-T J.290]: This expression infers that the downstream path (from the network to the subscriber) is operational, and that the upstream path (from the subscriber to the network) is not operational. This may occur because the upstream path is not available, the Set-top Device is not registered, or the Set-top Device does not support a two-way mode of operation.

6.832	one-way hash	[b-ITU-T J.93], [b-ITU-T J.95]: A mathematical process or algorithm whereby a variable length message is changed into a fixed length digital word, such that it is very difficult to calculate the original message from the word, and also very difficult to find a second message with the same word.

6.832bis	 one-way hash [b-ITU-T J.361]: A hash function that has an insignificant number of collisions upon output.

6.833	on-net [b-ITU-T J.260]: On an IPCablecom network.

6.834	on-net call [b-ITU-T J.161], [b-ITU-T J.178], [b-ITU-T J.361]:A communication placed by one customer to another customer entirely on the IPCablecom Network.

6.835	open application [b-ITU-T J.296]:	An application or applications that cable operators or subscribers can select and install, assuming downloadable applications in market.

6.836	Open Shortest Path First (OSPF) [b-ITU-T J.283]:	A unicast routing protocol for large-scale intra-domain networks. OSPF is a link state based routing protocol specified according to the ISO IS-IS routing protocol.

6.837	Open Systems Interconnection (OSI) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A framework of ISO Standards for communication between different systems made by different vendors, in which the communications process is organized into seven different categories that are placed in a layered sequence based on their relationship to the user; each layer uses the layer immediately below it and provides a service to the layer above. Layers 7 through 4 deal with end-to-end communication between the message source and destination, and layers 3 through 1 deal with network functions.

6.838		organizationally unique identifier (OUI) b-ITU-T J.112 Annex C]: 3-octet IEEE assigned identifier that can be used to generate Universal LAN MAC addresses and Protocol Identifiers per [b-IEEE 802] for use in Local and Metropolitan Area Network applications.

6.839		organizationally unique identifier (OUI) [b-ITU-T J.112 Annex B]: 3-octet IEEE assigned identifier that can be used to generate Universal LAN MAC addresses and Protocol Identifiers per [b-IEEE 802Q] for use in Local and Metropolitan Area Network applications.

6.840	OpenCable Applications Platform (OCAP) [b-ITU-T J.291]: The middleware standard for United States cable set-tops; J.200 Worldwide common core is a part of OCAP.

6.841	OpenCable host eSTB [b-ITU-T J.126]: An eSTB device built to CableLabs OpenCable Host specifications.

6.842	operator [b-ITU-T J.295]: The cable TV service operator.

6.843	operator-facing interface [b-ITU-T J.218]: The eRouter interface which is connected to the embedded cable modem.

6.844	operator-facing IP interface [b-ITU-T J.218]:	IP interface that is connected to the embedded cable modem and is provisioning with an IP address provided by the operator.

6.845	optional features [b-ITU-T J.193]:	Features that can be added to the NG-STB in order to support the Optional Services identified in this Recommendation.

6.846	optional services [b-ITU-T J.193]:	Services, in addition to the Core Services, that can be supported by the NG STB.

6.847	organisationally unique identifier (OUI) [b-ITU-T J.112], [b-ITU-T J.116], [b-ITU-T J.122]: A three-octet IEEE assigned identifier that OUI can be used to generate Universal LAN MAC addresses and Protocol Identifiers per ANSI/IEEE Std 802 for use in Local and Metropolitan Area Network applications.

6.848	original_network_id [b-ITU-T J.94]: A unique identifier of a network.

6.848bis	 original_network_id [b-ITU-T J.183]: A label identifying the network_id of the originating delivery system.

6.849	orthogonal frequency division multiplexing (OFDM) (based on [ITU-R BT.1306-6]) [b-ITU-T J.296]:	A digital multi-carrier modulation scheme, which uses a large number of closely-spaced orthogonal sub-carriers.

6.850	OSD consumer [b-ITU-T J.117]: A device that receives an OSD bitmap for the purpose of presenting the information on a display device or storing the information for future use.

6.851	OSD producer [b-ITU-T J.117]: A device that is the source of an OSD bitmap.

6.852	out of frame [b-ITU-T J.214]: When frame synchronization is lost, an OoF event is recorded. If OoF persists, LoF is declared. OoF is cleared when frame synchronization is regained.

6.853	out of stream device [b-ITU-T J.181]: A device that receives the cue message from an in stream device over a separate connection from the transport stream. An out of stream device does not receive or pass the transport stream directly.

6.854	out point [b-ITU-T J.181]: A point in the stream, suitable for exit, that lies on an elementary presentation unit boundary. An out point is actually between two presentation units rather than being a presentation unit itself.

6.854bis		out point [b-ITU-T J.189]: A point in the bit stream, suitable for exit, that lies on an elementary access unit boundary.

6.854ter		out point [b-ITU-T J.287]: A point in the stream, suitable for exit, that lies on an access unit boundary.

6.855	out point packet [b-ITU-T J.189]: A transport stream packet which corresponds to the last packet prior to the Out Point.

6.856	(OOB) Out-of-band  [b-ITU-T J.184]: Outside of the programming channels band. The OOB channels provide communication channels between the network and the terminal.

6.857	out-of-band messaging [b-ITU-T J.128]: The control and information messages sent from the Set-top Controller (or Application Server or similar device for legacy Out-Of-Band (OOB) messaging) to one or more Set-top Devices. Specifically, OOB infers the use of a dedicated channel for signalling which is separate from the video channels. This includes the following types of messages:

Conditional Access (CA) messages including entitlements;

Service Information (SI) messages;

Electronic Program Guide (EPG) messages;

Emergency Alert System (EAS) messages;

Other control or information messages.

6.858	output channel [b-ITU-T J.280]: The channel that is produced at the output of the splicer.

6.859	output multiplex [b-ITU-T J.280]: The MPEG-2 transport stream produced by multiplexing one or more output channels.

6.860	package filter [b-ITU-T J.195.1]: The process of analysing and selecting a data packet according to the established rule set.

6.861	packet [b-ITU-T J.200]: A packet is a set of contiguous bytes consisting of a header followed by its payload.

6.862	packet identifier [b-ITU-T J.89, [b-ITU-T J.187]: A unique integer value used to identify elementary streams of a program in a single or multi-program transport stream as described in 2.4.3 of ITU-T Rec. H.222.0| ISO/IEC 13818-1.

6.863	Packet Identifier (PID)	[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.291]: A unique integer value used to identify elementary streams of a programme in a single- or multi-programme MPEG-2 stream.

6.863bis	 packet identifier (PID) [b-ITU-T J.181 Amendment 1]: A unique 13-bit value used to identify elementary streams of a program in a single or multi-program Transport Stream [ITU-T H.222.0].

6.863ter	 packet identifier (PID) [b-ITU-T J.215]: MPEG-2 assigns a PID to each data packet. Packets with the same PID belong to the same logical channel.

6.864	PacketCable [b-ITU-T J.126]: PacketCable is a CableLabs specification located at: http://www.packetcable.com/specifications/. The PacketCable specifications are interoperable interface specifications for delivering advanced, real-time multimedia services over a two-way cable plant. Built on top of the industry's highly successful cable modem infrastructure, PacketCable networks uses Internet Protocol (IP) technology to enable a wide range of multimedia services, such as IP telephony, multimedia conferencing, interactive gaming, and general multimedia applications. This term is also applicable to a system or device that is compliant to the PacketCable specifications. The international version of PacketCable is standardized in ITU-T J series Recommendations J.160 to J.179.

6.865	packing [b-ITU-T J.195.1]: A procedure of combining multiple Ethernet media access control (MAC) frames with the same destination and priority to form a high performance network over coax (HiNoC) MAC frame.

6.866	parental control (viewer age restriction) [b-ITU-T J.296]: A system to restrict programme viewing using a combination of an age restriction listed as a programme attribute and a parental level (minimum age for viewing) in the receiver, set by the user himself using a password.

6.867	partial grant	[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: A grant that is smaller than the corresponding bandwidth request from the CM.

6.868	passive [b-ITU-T J.117]: The consumer plug is in this state when it accepts transactions directed at the plug's address space but does not respond with updates to the producer's registers.

6.869	passive coaxial access network [b-ITU-T J.195.1]: An access network composed of a coaxial network and passive power distribution components such as splitters and taps.

6.870	pass-through [b-ITU-T J.191]: A sub-function of the CAP, the Pass-through function bridges packets on the WAN-Data side of the CAP to the LAN-Pass side unchanged.

6.871	pausing with skipping [b-ITU-T J.246], [b-ITU-T J.247]:Events where the video pauses for some period of time and then restarts with some loss of video information. In pausing with skipping, the temporal delay through the system will vary about an average system delay, sometimes increasing and sometimes decreasing. One example of pausing with skipping is a pair of IP Videophones, where heavy network traffic causes the IP Videophone display to freeze briefly; when the IP Videophone display continues, some content has been lost. Another example is a videoconferencing system that performs constant frame skipping or variable frame skipping. Constant frame skipping and variable frame skipping are subsets of pausing with skipping. A processed video sequence containing pausing with skipping will be approximately the same duration as the associated original video sequence.

6.872	pausing without skipping [b-ITU-T J.246], [b-ITU-T J.247]: Any event where the video pauses for some period of time and then restarts without losing any video information. Hence, the temporal delay through the system must increase. One example of pausing without skipping is a computer simultaneously downloading and playing an AVI file, where heavy network traffic causes the player to pause briefly and then continue playing. A processed video sequence containing pausing without skipping events will always be longer in duration than the associated original video sequence.

6.873	payload [b-ITU-T J.89], [b-ITU-T J.187]: Payload refers to the bytes which follow the header bytes in a packet. For example, the payload of some transport stream packets includes a PES_packet_header and its PES_packet_data_bytes, or pointer_field and PSI sections, or private data: but a PES_packet_payload consists of only PES_packet_data_bytes. The transport stream packet header and adaptation fields are not payload.

6.873bis	 payload [b-ITU-T J.200]: The bytes following the header byte in a packet.

6.874	payload header suppression (PHS)	[b-ITU-T J.122 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: The suppression of the header in a payload packet (e.g., the suppression of the Ethernet header in forwarded packets).

6.875	payload unit start indicator (PUSI)	[b-ITU-T J.122 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: A flag in an MPEG header. A value of 1 indicates the presence of a pointer field as the first byte of the payload.

6.876	payload_unit_start_indicator	[b-ITU-T J.181 Amendment 1]:A bit in the transport packet header that signals, among other things, that a section begins in the payload that follows [ITU-T H.222.0].

6.877	pay-per-view	[b-ITU-T J.93], [b-ITU-T J.95]: A payment system whereby the subscriber can pay for an individual program or specified period of time.

6.878	Pd cycle	[b-ITU-T J.195.2]: A time interval between two adjacent downlink probe frames.

6.879	peak-to-peak amplitude	[b-ITU-T J.101]:The peak-to-peak amplitude is defined as the sum of basic amplitude zero overshoots and ones overshoots. It is expressed as a percentage of the basic amplitude (see Fig. 1 of J.101).

6.880		Peak ripple of the multi-burst signal [b-ITU-T J.64]: This quantity is defined on the basis of two numbers x and y, which represent the maximum (peak) differences between the amplitudes of the bursts of the test signal C (see Note 1) and a reference quantity A0, the two numbers x and y being expressed as a percentage of A0.

For 625-line signals, A0 is the peak-to-peak amplitude of element C1.

For 525-line signals, A0 is equal to half the amplitude of the luminance bar, as defined in § 2.1 above (see Note 2).

x and y can be found from the following expressions:

where Amax and Amin are respectively the highest and the lowest value of the peak-to-peak amplitude of the relevant bursts (see Note 3) measured at their half duration point.

The peak ripple of the multi-burst signal is defined by either x or –y depending upon which of these parameters has the larger magnitude.

NOTE 1: For 625-line signals, the last burst (having a frequency of 5.8 MHz), is not taken into account in this measurement.

NOTE 2: Further study is required to check if, as an alternative, A0 may also be derived from test element C1.

NOTE 3: For 625-line signals, the last burst (having a frequency of 5.8 MHz), is not taken into account in this measurement.

6.881	Permitted maximum signal (PMS) [b-ITU-T J.27]: Sine-wave signal at 1 kHzNOTE, 9 dB above the alignment signal level, equivalent to the permitted maximum programme-signal level.

NOTE: This frequency is nominal, and 1020 Hz recommended by CCITT Recommendation O.33 may be used.

6.882	personal information [b-ITU-T J.295]: Recorded information about an identifiable individual.

NOTE: This may include (1) name, address, email address, (2) race, nationality, ethnicity, origin, colour, religious or political beliefs or associations, (3) age, sex, sexual orientation, marital status, family status, (4) identifying number, code, symbol, (5) finger prints, blood type, inherited characteristics, (6) health care history including information on physical/mental disability, (7) educational, financial, criminal, employment history, (8) others' opinion about the individual, and (9) personal views except those about other individuals.

6.883	PES packet [b-ITU-T J.88]: The data structure used to carry elementary stream data. It is a layer in the system coding syntax described in 2.4.3.6/H.222.0.

6.883bis	 PES packet [b-ITU-T J.89], [b-ITU-T J.187]: The data structure used to carry elementary stream data. A PES packet consists of a PES packet header followed by a number of contiguous bytes from an elementary data stream. It is a layer in the system coding syntax described in 2.4.3.6 of ITU-T Rec. H.222.0 | ISO/IEC 13818-1.

6.884	PES packet header	[b-ITU-T J.89], [b-ITU-T J.187]: The leading fields in a PES packet up to and not including the PES_packet_data_byte fields, where the stream is not a padding stream. In the case of a padding stream the PES packet header is similarly defined as the leading fields in a PES packet up to and not including padding_byte fields.

6.885	PES Stream [b-ITU-T J.89], [b-ITU-T J.187]:A PES stream consists of PES packets, all of whose payloads consist of data from a single elementary stream, and all of which have the same stream_id. Specific semantic constraints apply. Refer to Intro. 4 of ITU-T Rec. H.222.0 | ISO/IEC 13818-1.

6.886	Phase distortion [b-ITU-T J.67]: The phase-frequency distortion is defined as the difference in degrees relative to a linear phase characteristic over a frequency band extending from, ideally, 0 Hz to a defined upper frequency.

6.887	Physical (PHY) layer [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116] Layer 1 in the Open Systems Interconnection (OSI) architecture; the layer that provides services to transmit bits or groups of bits over a transmission link between open systems, and which entails electrical, mechanical and handshaking procedures.

6.888	physical channel [b-ITU-T J.94]: A generic term to refer to the each of the 6-8 MHz frequency bands where television signals are embedded for transmission. Also known as the Physical Transmission Channel (PTC). One analog virtual channel fits in one PTC but multiple digital virtual channels typically coexist in one PTC.

6.889	physical interface [b-ITU-T J.183]: The interface on a physical layer equipment for transmission.

6.890	physical layer (PHY) [b-ITU-T J.122]: Layer 1 in the Open System Interconnection (OSI) architecture; the layer that provides services to transmit bits or groups of bits over a transmission link between open systems and which entails electrical, mechanical and handshaking procedures.

6.891	Physical Media Dependent (PMD) sublayer [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.210], [b-ITU-T J.212]: A sublayer of the Physical Layer which is concerned with transmitting bits or groups of bits over particular types of transmission link between open systems, and which entails electrical, mechanical and handshaking procedures.

6.892	picture layer [b-ITU-T J.88]: A repeated structure composed of a I-picture followed by several P_x005f pictures.

6.893	PID stream [b-ITU-T J.189]: All the packets with the same PID within a transport stream.

6.983bis PID strem [b-ITU-T J.181 Amendment 1], [b-ITU-T J.287]: A stream of packets with the same PID within a transport stream.

6.894	pilot sub-carrier [b-ITU-T J.196.2]: Sub-carriers for transmission of specific symbols in an orthogonal frequency division multiplexing (OFDM) symbol.

6.895	piracy [b-ITU-T J.93], [b-ITU-T J.95]: The act of acquiring unauthorized access to programs, usually for the purpose of reselling such access for unauthorized reception.

6.896	placement [b-ITU-T J.380.1], [b-ITU-T J.380.3]: The decision resulting from a placement opportunity which may include a content binding and a set of constraints.

6.897	placement management [b-ITU-T J.380.3]: The aggregated operational sequence of placement messaging and placement update messaging.

6.898	placement messaging [b-ITU-T J.380.3]: The PlacementRequest/PlacementResponse message exchange sequence.

6.899	placement operations [b-ITU-T J.380.3]: Placement management aggregated with placement status messaging.

6.900	placement opportunity [b-ITU-T J.380.1]: A potentially constrained location relative to digital content where advertisement insertion or content alterations can occur. The alterations may include insertions, replacements, or deletions of content in whole or in part. These locations which contain the opportunity for content insertion have traditionally been referred to as Avails [b-SCTE 35] for linear video content; however, placement opportunity refers to address and time locations where content may be placed, regardless of platform (i.e., Video in VOD, Banner images on menus and ITV channels, etc).

6.900bis	 Placement opportunity [b-ITU-T J.380.3]: A potentially constrained location relative to digital content where ad insertion or content alterations may occur. The alterations may include insertions, replacements, or deletions of content in whole or in part. These locations, which contain the opportunity for content insertion, have traditionally been referred to as avails [b-SCTE 35] for linear video content; however, placement opportunity refers to address and time locations where content may be placed, regardless of platform.

6.901	placement service [b-ITU-T J.380.3]: The distinctive information (i.e., characteristic set) identifying an entertainment content flow where advanced advertising techniques may apply.

6.902	placement status messaging [b-ITU-T J.380.3]: The Placement Status Notification/Placement Status Acknowledgement message exchange sequence.

6.903	placement update messaging [b-ITU-T J.380.3]: The Placement Update Notification/Placement UpdateAcknowledgement message exchange sequence.

6.904	platform [b-ITU-T J.296]: A business entity that manages and operates a collection service on a network of digital broadcasting.

6.905	plug [b-ITU-T J.117]: A collection of externally visible components (called ports) that can be connected to a sub-unit for the purposes of sending sequences of variable-length frames. There are three types of plugs: those associated with asynchronous connections, those associated with AV/C Isochronous Channels, and those associated with IEC 61883 Isochronous Channels.

6.906	pluge [b-ITU-T J.140]: Test signal consisting of a peak white level patch and several dark level patches/stripes used for the setting of brightness and contrast of the display. For details, see Recommendation ITU-R BT.814.

6.907	plug-in [b-ITU-T J.295]: Software for displaying, installed independently from the browser.

6.908	POD [b-ITU-T J.128]: A detachable device distributed by cable providers that connects to the cable receiver and manages Conditional Access.

6.909	point of deployment (POD) module	[b-ITU-T J.126]: A removable conditional access module which, when inserted into certain eSTB implementations, enables delivery of digital video programming and other services. POD module functionality includes copy protection and signal demodulation. The interface between the POD module and the eSTB is specified by [b-CCIF 2.0].

6.910	point-to-point L2 forwarding [b-ITU-T J.213]: Operation of an L2 forwarder between only two L2 networks with no source MAC address learning.

6.911	policy [b-ITU-T J.260]: Rules (or methods) for allocating telecommunications network resources among types of traffic that may be differentiated by labels.

6.912	policy server [b-ITU-T J.365], [b-ITU-T J.368]: A system that primarily acts as an intermediary between Application Manager(s) and CMTS(s). It applies network policies to Application Manager requests and proxies messages between the Application Manager and CMTS.

6.913	port [b-ITU-T J.117]: A sub-component of an asynchronous connection plug that supports unidirectional asynchronous connection data transfers.

6.913bis Port [b-ITU-T J.120]: The abstraction that transport protocols use to distinguish among multiple destinations within a given host computer. The transport selectors used by the OSI transport layers are equivalent to ports.

6.913ter port [b-ITU-T J.287]: See "socket". Refers to a bit-field defined in a TCP header. May also refer to a specific physical connector mounted on a device.

6.914	portable network graphics [b-ITU-T J.296]: A graphics file format succeeding GIF. It is pronounced "PING" and is capable of lossless compression. The file format is comprised of an 8-byte signature followed by a series of "chunks".

6.915	portal functions [b-ITU-T J.193]: Functionality that resides within a Residential Gateway that provides connectivity between the cable IP network, and the home network. See ITU-T Recs J.190, J.191 and J.192 for more details.

6.916	Portal Service (PS) [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.193], [b-ITU-T J.290], [b-ITU-T J.292]: A functional element that provides management and translation functions between the HFC and Home.

6.917	portal site [b-ITU-T J.296]: An entrance web site of Internet, providing links, search engines, news, Web mail services, and electrical bulletin board, etc.

6.918	post-roll [b-ITU-T J.380.3]: A placement opportunity following the play out of an entertainment asset.

6.919	Pre-3.0 DOCSIS [b-ITU-T J.222.2]: Versions of Data-Over-Cable-Service-Interface-Specifications (DOCSIS) specifications prior to the DOCSIS 3.0 suite of specifications.

6.920	preferential [b-ITU-T J.260]: A capability offering advantage over regular capabilities.

6.921	pre-roll	[b-ITU-T J.380.3]: A placement opportunity preceding an entertainment asset.

NOTE: Definition differs from [b-SCTE 35].

6.922	preparatory period [b-ITU-T N.54]: The period during which the broadcasting organizations carry out their own adjustments, tests, etc., before the television transmission itself commences.

The exact time at which the preparatory period begins (point H on Figure 1/N.54) is determined by the broadcasting organizations.

6.923	presence information [b-ITU-T J.367]: Dynamic set of information pertaining to a Presentity that may include Presence Information Elements such as the status, reachability, willingness, and capabilities of that Presentity [b-OMA RD-PRS].

6.924	presence information element	[b-ITU-T J.367]: A basic unit of Presence Information [b-OMA RD-PRS].

6.925	presence list	[b-ITU-T J.367]: A list of presentities that can have their individual states subscribed to with a single subscription request (e.g., a subscription list).

6.926	presence server [b-ITU-T J.367]: A logical entity that receives Presence Information from a multitude of Presence Sources pertaining to the Presentities it serves and makes this information available to Watchers according to the rules associated with those Presentities [b-OMA RD-PRS].

6.927	presence service [b-ITU-T J.367]: The capability to support management of Presence Information between Watchers and Presentities, in order to enable applications and services to make use of Presence Information [b-OMA RD-PRS].

6.928	presence source [b-ITU-T J.367]: A logical entity that provides Presence Information pertaining to exactly one or more Presentities to the Presence Server. 3GPP Presence User Agents, Presence Network Agents, and Presence External Agents are examples of Presence Sources [b-OMA RD-PRS].

6.929	presentation [b-ITU-T J.123], [b-ITU-T J.124]: One or more motion sequences, possibly combined with audio.

6.930	presentation engine [b-ITU-T J.200]: A subsystem in a receiver that evaluates and presents declarative applications consisting of content, such as audio, video, graphics, and text, primarily based on presentation rules defined in the presentation engine. A presentation engine also responds to formatting information, or "markup", associated with the content, to user inputs, and to script statements, which control presentation behaviour and initiate other processes in response to user input and other events. A common example of a presentation engine is an HTML browser, capable of displaying text and graphic content formatted in HTML [b-W3C HTML], with interactive behaviour programmed in ECMAScript [b-ECMAScript].

6.931	presentation time [b-ITU-T J.181 Amendment 1], [b-ITU-T J.287]: The time that a presentation unit is presented in the system target decoder [ITU-T H.222.0]..

6.932	presentation time-stamp (PTS) [b-ITU-T J.89], [b-ITU-T J.187], [b-ITU-T J.189]: A field that may be present in a PES packet header that indicates the time that a presentation unit is presented in the system target decoder.

6.933	presentation unit (PU) [b-ITU-T J.89], [b-ITU-T J.181], [b-ITU-T J.187], [b-ITU-T J.189]: A decoded Audio Access Unit or a decoded picture (see ITU-T Rec. H.262 | ISO/IEC 13818-2).

6.934	presentation-free [b-ITU-T J.90]: A service for which the information content is specified, but not the way in which the information must be presented on reception.

6.935	presentation-free EPG [b-ITU-T J.90]: An EPG for which the information content is specified, but the operation of the consumer television/multimedia display is not.

6.936	presentity [b-ITU-T J.367]: A logical entity that has Presence Information associated with it. This Presence Information may be composed of a multitude of Presence Sources. A Presentity is most commonly a reference for a person, although it may represent a role such as "help desk" or a resource such as "conference room #27". Presentities are generally referenced by distinguished names, such as "john.smith@example.com" or by phone numbers like "+19724735455". In SIMPLE, presentities are generally referenced using a sip:, pres: or tel: URL [b-OMA RD-PRS].

6.937	pre-shared key [b-ITU-T J.177]: A shared secret key passed to both parties in a communication flow, using an unspecified manual or out-of-band mechanism.

6.938	primary channel [b-ITU-T J.280]: The primary multiplex channel that is replaced in whole or in part. A single primary channel may result in multiple output channels.

6.939	primary distribution [b-ITU-T J.248]: Use of a transmission channel for transferring audio and/or video information to one or several destination points without a view to further post-processing on reception (e.g., from a continuity studio to a transmitter network).

6.940	Primary Downstream Channel [b-ITU-T J.222.2]: The downstream channel from which a CM derives CMTS master clock timing for upstream transmission. All other concurrently received channels are called "secondary downstream channels".

6.941	primary multiplex [b-ITU-T J.280]: This is the source of the primary channel(s).

6.942	primary service flow [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: All CMs have a primary upstream service flow and a primary downstream service flow. They ensure that the CM is always manageable and they provide a default path for forwarded packets that are not classified to any other service flow.

6.942bis Primary Service Flow [b-ITU-T J.222.2]: The first service flow, in each direction, defined in the CM configuration file.

6.943	Primary-Capable Downstream Channel [b-ITU-T J.222.2]: A Downstream Channel that can be used by a DOCSIS 3.0 CM as its Primary Downstream Channel, or by a DOCSIS 1.x/2.0 CM as its Downstream Channel.

6.944	priority treatment capabilities	[b-ITU-T J.260]: Capabilities that provide premium access to, and/or use of telecommunications network resources.

6.945	privacy	[b-ITU-T J.170], [b-ITU-T J.178], [b-ITU-T J.361]:: A way to ensure that information is not disclosed to any one other than the intended parties. Information is usually encrypted to provide confidentiality. Also known as confidentiality.

6.946	private key [b-ITU-T J.170]: The key used in public key cryptography that belongs to an individual entity and must be kept secret.

6.947	private user identity [b-ITU-T J.360]: Used, for example, for registration, authorization, administration and accounting purposes. A private user identity is associated with one or more public user identities.

6.948	probe frame [b-ITU-T J.195.2]: Frame of the physical layer used for carrying signalling frames of the MAC layer.

6.949	procedural application [b-ITU-T J.200]: An application which primarily makes use of procedural information to express its behaviour. A Java program is an example of a procedural application.

6.950	procedural application environment [b-ITU-T J.296]: An environment for necessary libraries for Java standard applications and broadcast co-ordinated Java applications.

6.951	procedural information [b-ITU-T J.200]: Information expressed in the form of procedures, e.g., do F or F( ).

6.952	product	[b-ITU-T J.197]: A device and/or technology that receives and possibly distributes content with redistribution control and/or copy control.

6.953	profile [b-ITU-T J.388]: A specified subset of the functionalities of the terminal defined in this Recommendation, or a specified subset of the bitstream syntax of video codecs.

6.954	programme [b-ITU-T J.94], [b-ITU-T J.183]: A concatenation of one or more events under the control of a broadcaster, e.g., news show, entertainment show.

6.954bis	 program [b-ITU-T J.117]: In MPEG-2 terminology, a collection of related elementary stream components making up a television service.

6.954ter	 program [b-ITU-T J.181 Amendment 1]: A collection of video, audio, and data PID streams which share a common program number within an MPTS [ITU-T H.222.0].

6.954quater program [b-ITU-T J.287]: A collection of video, audio and data PID streams which share a common program number within a SPTS or MPTS.

6.954quintic program[b-ITU-T J.380.2]: A time-bounded collection of video, audio, and data streams.

6.955	program clock reference	[b-ITU-T J.89], [b-ITU-T J.187]: A time-stamp in the transport stream from which decoder timing is derived.

6.956	program element [b-ITU-T J.94]: A generic term for one of the elementary streams or other data streams that may be included in a program. For example: audio, video, data, etc.

6.957	program in point [b-ITU-T J.181], [b-ITU-T J.181 Amendment 1], [b-ITU-T J.189]: A group of PID stream in points that correspond in presentation time.

6.958	program map table (PMT) [b-ITU-T J.215]: This is a MPEG-2 entity that contains all of the PIDs that make up a program.

6.959	program out point [b-ITU-T J.181 Amendment 1], [b-ITU-T J.189]: A group of PID stream out points that correspond in presentation time.

6.960	programme originator [b-ITU-T N.51]: A customer at a transmitting country needing up-linking of a transmission to television receive-only stations (TVROs) not related to an ITC.

6.961	program specific information [b-ITU-T J.89]: PSI consists of normative data which is necessary for the demultiplexing of transport streams and the successful regeneration of programs and is described in 2.4.4 of ITU-T Rec. H.222.0 | ISO/IEC 13818-1. An example of privately defined PSI data is the non-mandatory network information table.

6.961bis program specific information (PSI)	[b-ITU-T J.116]: In MPEG-2, normative data necessary for the demultiplexing of Transport Streams and the successful regeneration of programs.

6.961ter programme-specific information (PSI) [b-ITU-T J.112 Annex B]: In MPEG-2, normative data necessary for the demultiplexing of Transport Streams and the successful regeneration of programmes.

6.961quater Program-Specific Information (PSI) [b-ITU-T J.112 Annex C]: In MPEG-2, normative data necessary for the demultiplexing of Transport Streams and the successful regeneration of programs.

6.962	program splice mode [b-ITU-T J.181], [b-ITU-T J.181 Amendment 1]: A mode of the cueing message whereby the program_splice_flag is set to '1' and indicates that the message refers to a Program Splice Point and that all PIDs/components of the program are to be spliced.

6.962	program splice point [b-ITU-T J.181], [b-ITU-T J.181 Amendment 1], [b-ITU-T J.189]: A program in point or a program out point.

6.963	programme stream [b-ITU-T J.112 Annex B], [b-ITU-T J.122]: In MPEG-2, a multiplex of variable-length digital video and audio packets from one or more program sources having a common time-base.

6.963bis program stream [b-ITU-T J.112], [b-ITU-T J.112 Annex C], [b-ITU-T J.116],: In MPEG-2, a multiplex of variable-length digital video and audio packets from one or more program sources having a common time-base.

6.964	Programme Specific Information (PSI) [b-ITU-T J.112], [b-ITU-T J.122]: In MPEG-2, normative data necessary for the demultiplexing of Transport Streams and the successful regeneration of programmes.

6.965	progressive download [b-ITU-T J.124]: Streaming by download-based protocol over TCP/IP without any session control protocols. Client can start playing the media before the full file is downloaded.

6.966	proponent [b-ITU-T J.144]: An organization or company that proposes a video quality model for validation testing and possible inclusion in an ITU Recommendation.

6.967	protected content [b-ITU-T J.1010]: All kinds of protected media, in particular A/V and associated metadata, delivered to the customer application either via linear or non-linear delivery means.

6.968	protocol [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A set of rules and formats that determines the communication behaviour of layer entities in the performance of the layer functions.

6.969	Protocol Independent Multicast-Sparse Mode (PIM-SM) [b-ITU-T J.283]: A multicast routing protocol based on an explicit join model for multicast groups that may span a wide area.

6.970	provisioned service flow	[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: A service flow that has been provisioned as part of the registration process, but has not yet been activated or admitted. It may still require an authorization exchange with a policy module or external policy server prior to admission.

6.971	provisioning	[b-ITU-T J.295]: Automated initial setting activity.

6.971bis	 provisioning [b-ITU-T J.369], [b-ITU-T J.460.4]: Provisioning refers to the processes involved in the initialization of user attributes and resources to provide services to a User. This involves protocols, methodologies, and interfaces to network elements such as: Order Entry and Workflow Systems that carry out business processes, Operational Support Elements that handle network resources, Application Servers that offer services and User Equipment that offer services, among others.

6.972	provisioning L2VPN [b-ITU-T J.213]: An L2VPN for the pre-registration traffic of DHCP, TOD, and TFTP that provisions eCMs and eSAFE hosts. May be combined with a management L2VPN.

6.973	proxy [b-ITU-T J.170], [b-ITU-T J.178], [b-ITU-T J.361]: A facility that indirectly provides some service or acts as a representative in delivering information, thereby eliminating the need for a host to support the service.

6.974	proxy server [b-ITU-T J.460.1]: An intermediary entity that acts as both a server and a client for the purpose of making requests on behalf of other clients.

6.975	pseudo wire [b-ITU-T J.214]: A pseudo wire (PW) is a logical entity capable of the emulation of a native service (i.e., T1) over a packet switched network (like DOCSIS IP).

6.976	public key [b-ITU-T J.170], [b-ITU-T J.178]: The key used in public key cryptography that belongs to an individual entity and is distributed publicly. Other entities use this key to encrypt data to be sent to the owner of the key.

6.977	public key certificate [b-ITU-T J.170]: A binding between an entity's public key and one or more attributes relating to its identity, also known as a digital certificate.

6.978	public key cryptography	[b-ITU-T J.170], [b-ITU-T J.178]: A procedure that uses a pair of keys, a public key and a private key for encryption and decryption, also known as asymmetric algorithm. A user's public key is publicly available for others to use to send a message to the owner of the key. A user's private key is kept secret and is the only key which can decrypt messages sent encrypted by the user's public key.

6.978bis	 public key cryptography [b-ITU-T J.93], [b-ITU-T J.95]: A cryptographic technique based upon a two-key algorithm, private and public, wherein a message is encrypted with the public key but can only be decrypted with the private key. Also known as a Private-Public Key (PPK) system.

NOTE: Knowing the public key does not reveal the private key.

6.979	public user identity [b-ITU-T J.360]: Used by any user for requesting communications to other users.

6.980	pulse code modulation (PCM)	[b-ITU-T J.161]: A commonly employed algorithm to digitize an analog signal (such as a human voice) into a digital bit stream using simple analog to digital conversion techniques.

6.980bis pulse code modulation [b-ITU-T J.361]: A common method of digitizing an analogue signal (such as a human voice) into a bit stream using simple analogue to digital conversion techniques. [ITU T G.711] defines its use in the PSTN with two encoding laws, μ-law, used in North America, and A-law, used elsewhere.

6.981	QAM channel (QAM ch) [b-ITU-T J.210], [b-ITU-T J.212]: Analog RF channel that uses quadrature amplitude modulation (QAM) to convey information.

6.982	QoS parameter set	[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.128], [b-ITU-T J.290]: The set of service flow encodings that describe the quality of service attributes of a service flow or a service class.

6.983	QPSK/differential coding [b-ITU-T J.184]: A special QPSK system that uses differential encoding scheme to resolve the 90° ambiguity in the detection of the QPSK signal at the demodulator.

6.984	quadlet	[b-ITU-T J.117]: Four bytes of data.

6.985	Quadrature Amplitude Modulation (QAM) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A method of modulating digital signals onto a radio frequency carrier signal involving both amplitude and phase coding.

6.985bis quadrature amplitude modulation (QAM) [b-ITU-T J.210], [b-ITU-T J.212]: A modulation technique in which an analog signal's amplitude and phase vary to convey information, such as digital data.

6.986	Quadrature Phase Shift Keying (QPSK) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116],[b-ITU-T J.122]: A method of modulating digital signals onto a radio-frequency carrier signal using four phase states to code two digital bits.

6.987	qualifier [b-ITU-T J.380.8]: A "Qualifier" is a name/value pair used to describe one characteristic of an object in a logical service's basic query data model. For instance, <Qualifier name="Age" value="30to40"/> is an example of a qualifier where "Age" is the characteristic's name and "30to40" is the characteristic's value.

6.988	QualifierSet [b-ITU-T J.380.8]: A "QualifierSet" is a complete set of Qualifier elements that describe an object in a logical service's basic query data model.

6.989	quality of service [b-ITU-T J.361]: Guarantees network bandwidth and availability for applications.

6.989bis	 quality of service [b-ITU-T J.365]: Method used to reserve network resources and guarantee availability for applications.

6.989ter	Quality of Service (QoS)	[b-ITU-T J.145]: The collective effect of service performances which determine the degree of satisfaction of a user of the service.

NOTE: The Quality of Service is characterized by the combined aspects of service support performance, service operability performance, service integrity and other factors specific to each service.

6.989quater quality of service (QoS)	[b-ITU-T J.161], [b-ITU-T J.166]: Guarantees network bandwidth and availability for applications.

6.990	Queue-Depth Based Request [b-ITU-T J.222.2]: Request in multiples of bytes based on the CM's queue depth and QoS parameters for a specific service flow. This request does not include any estimation of physical layer overhead.

6.991	Radio Frequency (RF) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.212]: In cable television systems, this refers to electromagnetic signals typically in the range 5 to 1000 MHz.

6.991bis radio frequency (RF) [b-ITU-T J.116]: Refers to electromagnetic signals typically in the range 5 to 40 000 MHz.

6.991ter radio frequency (RF) [b-ITU-T J.210]: A portion of the electromagnetic spectrum from a few kilohertz to just below the frequency of infrared light.

6.992	radio frequency (RF) type remote control unit (RCU) [b-ITU-T J.295]: An RCU using wireless technology such as RF4CE.

6.993	radio frequency interface (RFI) [b-ITU-T J.210], [b-ITU-T J.212]: Term encompassing the downstream and the upstream radio frequency interfaces.

6.994	RAND [b-ITU-T J.1004]: A random number with 320 bits.

6.995	RCU [b-ITU-T J.297]: The equipment used by users when they operate STB remotely.

6.996	real time transport protocol (RTP) [b-ITU-T J.291]: A transport protocol for real-time applications defined in ITU-T Rec. H.225.0. It is designed for real-time transmission of audio and video data.

6.997	realm [b-ITU-T J.178]: A single instance of an IPCablecom network.

6.998	Real-time Streaming Protocol (RTSP) [b-ITU-T J.120]: RTSP is a protocol defined in 6.3 and 6.4. RTSP specifies session control method between server and receiver. Its syntax is based on HTTP.

6.999	real-time text conversation [b-ITU-T J.361]: Text conversation sessions as specified in [ITU-T T.140] and [IETF RFC 4103].

6.1000	real-time transport protocol [b-ITU-T J.361]: A protocol for encapsulating encoded voice and video streams. Refer to [b-IETF RFC 3550].

6.1001	Real-time Transport Protocol (RTP) [b-ITU-T J.120], [b-ITU-T J.121]: A transport protocol for real-time applications defined in ITU-T H.225.0. It is designed for real-time transmission of audio and video data.

6.1001bis real-time transport protocol (RTP) [b-ITU-T J.161]: A protocol for encapsulating encoded voice and video streams.

6.1002	Receive Channel Configuration [b-ITU-T J.222.2]: The CMTS sends the RCC encoding in the REG-RSP message. The RCC contains TLVs to initially configure CM's Receive Channels (RCs) and Receive Modules (RMs).

6.1003	Receive Channel Profile	[b-ITU-T J.222.2]: The RCP describes a logical representation of the CM's downstream physical layer in terms of Receive Channels (RCs) and Receive Modules (RMs). A Cable Modem reports its ability to receive multiple channels with one or more RCP Encodings in a REG-REQ message.

6.1004	Receive Channel Set [b-ITU-T J.222.2]: The set of downstream channels assigned to an individual CM is called its Receive Channel Set, and is explicitly configured by the CMTS using the RCC encodings.

6.1005	Receive Module [b-ITU-T J.222.2]: A component in the CM physical layer implementation shared by multiple Receive Channels.

6.1006	receiver platform (platform) [b-ITU-T J.200]: The receiver's hardware, operating system and native software libraries of the manufacturer's choice.

6.1007	receiving device [b-ITU-T J.181]: A device that receives or interprets sections conforming to this Recommendation. Examples of these devices include splicers, ad servers, segmenters and satellite receivers.

6.1008	Recovery time [b-ITU-T J.145]: Delay from the end of an interruption (or after an impairment or disturbance) until the start of the normal operation, i.e. up to the time when the audio quality comes back to the normative quality.

6.1009	Reed Solomon code [b-ITU-T J.112], [b-ITU-T J.116]: A forward error correction code located before interleaving that enables correction of errors induced by burst noise.

6.1010	reference code matrix [b-ITU-T J.122]: A 128-by-128 element matrix formed by stacking successive spreading codes on top of each other, i.e., the bottom row of the reference code matrix is code 0 (all ones) and the top row is code 127. The code elements are placed in the matrix from right to left, i.e., the right-most column of the code matrix is the first element of each code, and the left-most column is the last element of each code.

6.1011  reference image [b-ITU-T J.302]: A resource to be referenced in the tracking process of acquiring coordinates generated by a particular region's movement in the video scene.

6.1012	refresh rate [b-ITU-T J.246], [b-ITU-T J.247]: The rate at which the computer monitor is updated.

6.1013	registered Jack-11 (RJ-11) [b-ITU-T J.161]: A standard 4-pin modular connector commonly used for connecting a phone unit into a wall jack.

6.1014	registration descriptor [b-ITU-T J.181 Amendment 1]: Carried in the PMT of a program to indicate that, when signalling Splice Events, splice_info_sections is carried in a PID stream within this program. The presence of the Registration Descriptor signifies a program's compliance with ITU-T J.181.

6.1014bis	registration descriptor [b-ITU-T J.287]: An MPEG-2 ([ITU-T H.222.0]) construct to uniquely and unambiguously identify formats of private data. As used in this context, it is carried in the PMT of a program to indicate the program's compliance with [ITU-T J.181]. (See clause 2.6.8 of [ITU-T H.222.0].)

6.1015	registration, admissions and status [b-ITU-T J.177]: RAS Channel is an unreliable channel used to convey the RAS messages and bandwidth changes between two H.323 entities.

6.1016	registration-established service channel [b-ITU-T J.380.2]: A service channel duration commencing with a successful registration and continuing until termination through deregistration.

6.1016bis	registration-established service channel [b-ITU-T J.380.3]: A service channel duration commencing with a successful placement service registration and continuing until either participating logical service completes terminating an active service channel.

6.1017	remote alarm indication [b-ITU-T J.214]: Also known as yellow alarm. When a CSU/DSU enters the red alarm state, an RAI is transmitted in the outgoing direction. The RAI signals to the remote end that the local end is unable to synchronize framing patterns.

6.1018	remote application catalogue [b-ITU-T J.205]: Application catalogue exported from an application repository. This differs from the normal application catalogue in that, if the remote application catalogue is very large, there must be a mechanism for querying it and retrieving it in small parts that can be handled in an efficient way by the integrated broadcast and broadband (IBB) DTV receiver.

6.1019	remote procedure call (RPC) [b-ITU-T J.380.7]: A protocol that one program can use to request a service from a program located in another computer in a network without having to understand network details.

6.1020	remote terminal [b-ITU-T J.284]: Video terminal at a remote site.

6.1021	remux [b-ITU-T J.296]: A method of transmission of TS created by the cable operator to cable a subscriber's STB over a cable TV network.

6.1022	request	[b-ITU-T J.287]: A single directive, from either the automation system, the injector or the PAMS, to another portion of the overall system. "Request" and "Command" are used interchangeably. A request is always carried within a message. A request is normally answered by a response message.

6.1023	Request For Comments (RFC) [b-ITU-T J.112], [b-ITU-T J.112 Annex B]: Technical policy document of the IETF; these documents can be accessed on the World Wide Web at http://www.rfc-editor.org/rfc-index.html.

6.1023bis	Request For Comments (RFC)  [b-ITU-T J.122], [b-ITU-T J.212]: A technical policy document of the IETF; these documents can be accessed on the worldwide web at http://www.rfc-editor.org/.

6.1023ter 	Request For Comments (RFC) [b-ITU-T J.112 Annex C]: Technical policy document of the IETF; these documents can be accessed on the World Wide Web at http://ds.internic.net/ds/rfcindex.html.

6.1023quater request for comments [b-ITU-T J.365]: Technical policy documents approved by the IETF which are available at http://www.ietf.org/rfc.html.

6.1024	requestor [b-ITU-T J.362]: The requestor in this context is the controller that wishes to control the control point and hence needs to discover the necessary information to do so.

6.1025	Resequencing Channel List [b-ITU-T J.222.2]: This is a list of channels on which the CM receives packets labelled with that DSID.

6.1026	Resequencing Context [b-ITU-T J.222.2]: A CM Resequencing Context, identified by a Resequencing DSID, is the set of Downstream Resequencing Channel List, Sequence Change Count, and DSID Resequencing Wait time. Downstream packets containing a Resequencing DSID and a sequence number are delivered, resequenced and forwarded according to the attributes of the Resequencing Context.

6.1027	Resequencing Downstream Service Identifier [b-ITU-T J.222.2]: A downstream service identifier for which the CMTS signals packet resequencing attributes.

6.1028	reserved [b-ITU-T J.89], [b-ITU-T J.187]: The term "reserved", when used in the clauses defining the coded bit stream, indicates that the value may be used in the future for ISO-defined extensions. Unless otherwise specified within ITU-T Rec. H.222.0 | ISO/IEC 13818-1, all reserved bits shall be set to "1".

6.1028bis reserved [b-ITU-T J.94], [b-ITU-T J.287]: The term "reserved" when used in the clause defining the coded bitstream, indicates that the value may be used in the future for ISO defined extensions. Unless otherwise specified within this Recommendation, all "reserved" bits shall be set to "1".

6.1028ter reserved [b-ITU-T J.181 Amendment 1]: The term "reserved", when used in the clauses defining the coded bit stream, indicates that the value may be used in the future for extensions to the standard. Unless otherwise specified in the core part of ITU-T J.181, all reserved bits are set to '1'.

6.1029	reserved_future_use [b-ITU-T J.94]: The term "reserved_future_use", when used in the clause defining the coded bitstream, indicates that the value may be used in the future for ETSI defined extensions. Unless otherwise specified within this Recommendation all "reserved_future_use" bits shall be set to "1".

6.1030	resident application [b-ITU-T J.230]: Application that is stored in the storage area of a terminal device, and is not tied to any broadcast programme. For example, a pre-installed application is classified as a resident application.

6.1031	residential gateway	[b-ITU-T J.190], [b-ITU-T J.290], [b-ITU-T J.292]: A logical element that provides in-premise and aggregated security, management, provisioning, and addressing services for logical elements within a compliant IPNet2Home network. In this Recommendation, it is also referred to as portal services (PS).

6.1031bis	residential gateway[b-ITU-T J.294]: A grouping of logical elements used to achieve access for home network(s).

6.1031ter Residential Gateway (RG) [b-ITU-T J.193]: A grouping of logical elements used to achieve HFC access for Home Network(s).

6.1032	resource (system) [b-ITU-T J.200]: A well-defined capability or asset of a receiver, which can be used by the application environment. Examples: MPEG decoder, graphics system.

6.1033	resource abstraction/middleware interface [b-ITU-T J.701]: An interface between the resource abstraction layer and the middleware service that encapsulates OS operations and the resources layer, and provides an abstract view of the resource layer.

6.1034	resources [b-ITU-T J.701]: The native hardware and software entities that provide certain functionality to the middleware services component through the resource abstraction layer. The middleware service functions are provided to the application layer through the API of the middleware services.

6.1035	response [b-ITU-T J.287]: A reply message to a request directive from the other portion of the system. Responses are made by the automation system, the compression system, and the PAMS in reply to requests. A response is always carried within a single_operation message.

6.1036	Response time [b-ITU-T J.145]: Delay in applications of interactive services between the start of a user command and the reaction of the whole system reaching the user. It is combined from both the latency time of the interaction channel, the latency time of the server and the combined network and codec latency time of the forward channel.

6.1037	return channel [b-ITU-T J.200]: The communication mechanism which provides connection between the receiver and a remote server.

6.1038	return loss [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: The parameter describing the attenuation of a guided wave signal (e.g. via a coaxial cable) returned to a source by a device or medium resulting from reflections of the signal generated by the source.

6.1039	reverse channel [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: The direction of signal flow towards the headend, away from the subscriber; equivalent to Upstream.

6.1039bis	reverse channel [b-ITU-T J.116]: The direction of signal flow towards the BTS, away from the subscriber; equivalent to Upstream.

6.1040	reverse data channel [b-ITU-T J.184]: A data channel transmitted from the terminal device to the headend in a modulated channel at a rate of 0.256 to 3.088 Mbit/s. The RDC carries IP traffic only for:

• Messaging.

• Personal computer data services.

• Network management.

6.1041	rights [b-ITU-T J.702]: Refer to the ability to perform a pre-defined set of utilization functions on a content item. These utilization functions include the permissions (e.g., to view/hear, copy, modify, record, excerpt, sample, keep for a certain period, distribute), constraints (e.g., play/view/hear multiple times, play/view/hear certain number of hours) and obligations (e.g., payment, content tracing) that apply to the content and provide liberty of use granted to the end-user.

6.1042	ringback [b-ITU-T J.460.1]: A function applied by a PSAP that causes an offhook phone to get ROH tone and an on hook phone to ring. This is not a new call. Ringback is not the same as Callback. See above.

6.1043	robustness rules [b-ITU-T J.197]: The rules described in clause 6, which apply to set top boxes, and are for the purpose of resisting attempts to modify set top boxes to defeat the functions of the compliance rules.

6.1044	roll off	[b-ITU-T J.112], [b-ITU-T J.116]: A coefficient of the cosine roll-off function that determines the frequency characteristics of the filter.

6.1045	root DTI server [b-ITU-T J.211]: The DTI server that is the source of traceable time and frequency for all subtending DTI servers and clients in a building.

6.1046	root mean square (RMS）[b-ITU-T J.210]: Square root of the mean value squared a function.

6.1047	root private key [b-ITU-T J.170]: The private signing key of the highest-level Certification Authority. It is normally used to sign public key certificates for lower-level Certification Authorities or other entities.

6.1048	Routing Information Protocol (RIP) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A protocol of the IETF for exchanging routing information about IP networks and subnets.

6.1049	Routing CMTS [b-ITU-T J.222.2]: A CMTS that makes traffic forwarding decisions between its Network System Interfaces and MAC Domain Interfaces based upon the Layer 3 (network) address of a packet.

6.1050	RP [b-ITU-T J.283]: A Rendez-vous Point among multicast sources and group members. Packets transmitted from multicast sources are distributed via an RP router at the beginning of multicast transmission.

6.1051	RS-coded MPEG-2 Transport Stream (TS) packet [b-ITU-T J.131], [b-ITU-T J.132]: A data packet possessing a length of 204 bytes. Bytes 1 to 188 contain an MPEG-2 transport stream packet. Bytes 189 to 204 contain the parity-check bytes for the error correction of the preceding bytes of this packet. These parity-check bytes are generated using a shortened Reed Solomon Code RS (204, 188), as specified in Annex A/J.83.

6.1052	RTP control protocol (RTCP)	[b-ITU-T J.121]: A control protocol for RTP packets defined in ITU T Rec. H.225.0.

6.1053	sample	[b-ITU-T J.123], [b-ITU-T J.124]: An individual frame of video, or a time-contiguous compressed section of audio.

6.1054	sample description	[b-ITU-T J.123], [b-ITU-T J.124]: A structure which defines and describes the format of some number of samples in a track.

6.1055	sample table	[b-ITU-T J.123], [b-ITU-T J.124]: A packed directory for the timing and physical layout of the samples in a track.

6.1056	Sampling instants for decoding margin [b-ITU-T J.101]: The sampling instants for decoding margin are half way between the timing instants defined in § 2.6.

6.1057	satellite master antenna television (SMATV) network [b-ITU-T J.84]: A broadband network intended for the distribution of television, sound and data signals received directly from one or more satellites, possibly in frequency division multiplex with similar terrestrial UHF or VHF signals, to households located in one or more adjacent buildings. Where intended also for the distribution of new digital multi-programme television, sound and data services, such networks are known as "Digital SMATV networks", and the digital configuration made for this purpose is known as a "Digital multi‑programme SMATV system".

6.1058	S-CDMA Frame [b-ITU-T J.122], [b-ITU-T J.222.1], [b-ITU-T J.222.2]: A two dimensional representation of mini-slots, where the dimensions are codes and time. An S-CDMA frame is composed of p active codes in the code dimension and K spreading intervals in the time dimension. Within the S-CDMA frame, the number of mini-slots is determined by the number of codes per mini-slot (c) and p, the number of active codes in the S-CDMA frame. Each S-CDMA frame thus contains s mini-slots, where s = p/c, and each mini-slot contains c*K information (QAM) symbols.

6.1059	S-CDMA subframe [b-ITU-T J.122], [b-ITU-T J.222.1]: A subframe is a vertically-smaller subset of an S-CDMA frame over which interleaving is performed, where the vertical dimension is R' codes, where R' ≤ p (the number of active codes). A subframe is generally used to constrain the interleaving region to be of a similar size to the Reed-Solomon codeword in order to provide protection from impulse noise.

6.1060	schedule provider [b-ITU-T J.90]: The entity that decides the schedule in which programmes are sequenced on a delivery channel (e.g. the broadcaster).

6.1061	scope of uniqueness [b-ITU-T J.380.2]: Uniqueness is context relative and for this specification's purpose shall be defined by one of the following: global, service channel or element.

6.1062	scrambler [b-ITU-T J.96]: relates to the overall mechanisms required to meet the DVB-CSA specification.

6.1062bis	scrambler [b-ITU-T J.195.2]: Process that randomizes data using a pseudo-random binary sequence.

6.1063	scrambling [b-ITU-T J.91]: is defined as the alteration of the characteristics of a vision/sound/data signal in order to prevent unauthorized reception in a clear form. This alteration is a specified process under the control of the conditional access system (sending end)

6.1063bis scrambling [b-ITU-T J.93], [b-ITU-T J.95]: The process of using an encryption function to render television and data signals unusable to unauthorized parties.

6.1064	scripting language [b-ITU-T J.200]: A language to describe the program process, which is embedded in markup documents.

6.1065	second screen	[b-ITU-T J.295]: This refers to a display screen of mobile phones or other network-enabled devices that show services associated with the television screen.

6.1066	secondary distribution 	[b-ITU-T J.248]: Use of a transmission channel for distribution of programs to viewers at large.

6.1067	section [b-ITU-T J.94]: A section is a syntactic structure that shall be used for mapping each [ITU T H.222.0] defined PSI table or private data table into transport stream packets. Private data tables include service information (SI) except for PSI.

6.1067bis	section [b-ITU-T J.200]:A syntactic structure specified in [b-ITU-T H.222.0] for the embedding of data in the transport stream. A data structure comprising a portion of a [b-ITU-T H.222.0] (or a [b-ISO/IEC 13818-6]) defined table, such as the program association table (PAT), conditional access table (CAT), program map table (PMT) or DSM-CC section.

6.1067ter	section [b-ITU-T J.287]: A private_section structure as defined by [ITU-T H.222.0] and (in this case) [ITU-T J.181]. As used here, the term is usually "splice_info_section". See clause 6.2 of [ITU-T J.181] and clause 2.4.4.10 of [ITU-T H.222.0].

6.1068	secure microprocessor [b-ITU-T J.126]: The security element in a device that supports downloadable conditional access.

6.1069	secure signature [b-ITU-T J.93], [b-ITU-T J.95]: A mathematical process by which the origin and integrity of a transmitted message can be ascertained.

NOTE: If a secure signature system is used, the originator cannot deny having sent the message, and the receiver can determine if the message has been modified.

6.1070	secure socket layer (SSL) (based on [ITU-R BT.1699]) [b-ITU-T J.296]: A security protocol that works at a socket level. This layer exists between the TCP layer and the application layer to encrypt/decode data and authenticate concerned entities.

6.1071	security association (SA) [b-ITU-T J.213]: An association between the CMTS and a set of CMs in a MAC domain that enables encrypted communication between the CMTS and the CM set. A single CM SA is one with a single CM, and enables a private point-to-point L2 Network connection between the CMTS and the CPE LAN of that CM. A security association descriptor (SA-Descriptor) is a multiple-part message element defined in the DOCSIS baseline privacy [ITU-T J.125] that includes a security association ID (SAID).

6.1072	security association identifier (SAID) [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: A baseline privacy security identifier between a CMTS and a CM.

6.1072bis security association ID (SAID) [b-ITU-T J.213]: A 14-bit identifier that appears in a BPI extended header (BPI-EH) of a DOCSIS PDU packet to identify the key used to encrypt the packet.

6.1073	segment [b-ITU-T J.181]: Either a Program, a Chapter, a Provider Advertisement, a Distributor Advertisement, or an Unscheduled Event as listed in Table 10‑8, segmentation_type_id.

6.1074	segment buffer [b-ITU-T J.117]: An externally visible address space on a consumer into which data is written by the connected producer.

6.1075	Segment Header OFF [b-ITU-T J.222.2]: Mode of Upstream DOCSIS3.0 Operation where segment headers are not used for any segment. This mode is provisioned per upstream service flow and prohibits fragmenting a packet across segment boundaries.

6.1076	Segment Header ON [b-ITU-T J.222.2]: Mode of Upstream DOCSIS3.0 Operation where segment headers are used for each segment. This mode is provisioned per upstream service flow.

6.1077	Selectable Active Codes [b-ITU-T J.222.2]: A methodology to determine the set of active codes and its complement, the set of unused codes. In SAC mode 1, a consecutive set of codes starting with code 0 are unused. In SAC mode 2, the active codes are selectable via a 128-bit string.

6.1078	selectable active codes (SAC) [b-ITU-T J.222.1]: A methodology to determine the set of active codes and its complement, the set of unused codes. In SAC mode 1, a consecutive set of codes starting with code 0 are unused. In SAC mode 2, the active codes are selectable via a 128-bit string.

6.1079	self-aggregation [b-ITU-T J.210]: Method used to compute the headend noise floor by summing measured noise from a single device over a specified output frequency range.

6.1080 	send reference station [b-ITU-T N.1], [b-ITU-T N.51]: The transmit sub-control station of an international multiple destination sound-programme circuit section, circuit or link.

6.1081	sequence layer [b-ITU-T J.88]: Uppermost layer of a coding bit stream which coordinates coding and decoding parameters.

6.1082	server [b-ITU-T J.280]: The device that originates the insertion channel(s) to be spliced into the primary channel(s). This device communicates with the splicer about when and what to splice.

6.1082bis	server [b-ITU-T J.360]: A network element that receives requests in order to service them and sends back responses to those requests. Examples of servers are proxies, user agent servers, redirect servers, and registrars.

6.1083	service	[b-ITU-T J.94]: A sequence of programmes under the control of a broadcaster which can be broadcast as part of a schedule.

6.1083bis	service [b-ITU-T J.164]: A service is an individual or package of communications features a subscriber may select. A service is identified by a set of one or more "calls" or transactions that deliver the desired functionality to the subscriber. Examples of a service include: a voice communication between two local IPCablecom subscribers, a 3-way call, pay-per-view movie, and a web surfing session. A service may be instantaneous or persist over time.

6.1083ter	service [b-ITU-T J.197]: The video, audio, or data signals, whether in analogue or digital format, transmitted over the video service provider network to (or from) the set top box, for the purposes of effectuating the reception or transmission of information, entertainment, or communications content.

6.1083quater service [b-ITU-T J.200]: Content and applications provided by network operators and broadcasters.

6.1083quintic service [b-ITU-T J.296], [b-ITU-T J.702]: A set of functionalities, enabled by a provider for end users that provides, for example, a television programme delivery service, a content-on-demand service, IP connectivity, next generation services where the hybrid cable STB is associated with mobile devices, etc.

6.1084	Service Access Point (SAP) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: The point at which services are provided by one layer, or sublayer, to the layer immediately above it.

6.1085	service and content protection	[b-ITU-T J.702]: A combination of service protection and content protection, or a system or implementation thereof.

6.1086	service associated IBB application [b-ITU-T J.205 Cor. 2]: An application that is part of the integrated broadcast and broadband (IBB) DTV service tuned to by the user at a given time.

6.1087	service bound application [b-ITU-T J.200]: An application delivered as part of a broadcast stream.

6.1088	service channel [b-ITU-T J.380.1], [b-ITU-T J.380.2]: A message communication path between two logical service endpoints.

6.1089	service channel uniqueness [b-ITU-T J.380.2]: Uniqueness scoped by the @identity attribute and the service channel and at no other time shall the item be compromised, reused, or otherwise taken to have more than one meaning. XML messages shall be service channel unique and a message shall not be compromised or reused for the duration of the service channel. Service channel uniqueness is relative only to the endpoints where the message exchange is occurring and within the identity domain of the two endpoints involved in the exchange. Enforcement of uniqueness as well as the creation of identity unique identifiers is outside the scope of this Recommendation and [b IETF RFC 4122] is recommended.

6.1090	service class	[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.128], [b-ITU-T J.290] A set of queueing and scheduling attributes that is named and that is configured at the CMTS. A service class is identified by a service class name. A service class has an associated QoS parameter set.

6.1091	service class name [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: An ASCII string by which a service class may be referenced in modem configuration files and protocol exchanges.

6.1092	service components [b-ITU-T J.701]: The components that offer functionalities to any upper layers, regardless of the type of software and hardware in the resource layer.

6.1093	Service Data Unit (SDU) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: Information that is delivered as a unit between peer service access points.

6.1094	service exclusive IBB application [b-ITU-T J.205 Cor. 2]: An application that is listed as a component of an integrated broadcast and broadband (IBB) DTV service; its life cycle is strictly tied to the exhibition of such an IBB DTV service.

6.1095	service flow [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: A MAC-layer transport service which: provides unidirectional transport of packets from the upper layer service entity to the RF; shapes, polices and prioritizes traffic according to QoS traffic parameters defined for the flow.

6.1096	service flow identifier (SFID)	[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: An identifier assigned to a service flow by the CMTS (32 bits).

6.1097	service flow reference [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: A message parameter in configuration files and dynamic service MAC messages used to associate classifiers and other objects in the message with the service flow encodings of a requested service flow.

6.1098	service flow tagging [b-ITU-T J.223.1]: The service flow tagging is a method of using tags to indicate the service flow to which a packet has been classified.

6.1099	Service Group [b-ITU-T J.222.2]: A SG is formally defined as the complete set of upstream and downstream channels that can provide service to a single subscriber device. This includes channels from different DOCSIS MAC Domains and even different CMTSs as well as video EQAMs.

6.1100	Service Identifier (SID) [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.222.2]: A Service Flow Identifier assigned by the CMTS (in addition to a Service Flow Identifier) to an Active or Admitted Upstream Service Flow. (14 bits].

6.1100bis service identifier (SID) [b-ITU-T J.122]: An identifier assigned by the CMTS (in addition to a service flow identifier) to an active or admitted upstream service flow (14 bits).

6.1101	service information (SI)	[b-ITU-T J.94], [b-ITU-T J.142]: Digital data describing the delivery system, content and scheduling/timing to broadcast data streams etc. It includes MPEG-2 Program Specific Information (PSI) together with independently defined extensions.

6.1101bis	service information (SI)	[b-ITU-T J.200]: Data that describes programs and services.

6.1102	service priority mapping [b-ITU-T J.195.1]: A function whereby the media access control (MAC) layer maps the service stream matching the message features into the assignment priority queue of the quality of service (QoS) level and simultaneously re‑labels the priority level of the service stream.

6.1103	service protection [b-ITU-T J.702]: Ensuring that an end-user can only acquire a service, and, by extension, the content contained therein, that they are entitled to receive. Service protection includes protecting service from unauthorized access as IPTV contents traverse through the IPTV service connections.

6.1104	service provider interface (SPI) [b-ITU-T J.294]: A service provider interface residing in the network side for the purpose of delivering advanced applications and services to the user. Applications and services can be developed either by the service provider or by a third party developer.

6.1105	service shared IBB application [b-ITU-T J.205 Cor. 2]: An application that is listed as a component of several different integrated broadcast and broadband (IBB) DTV services; its life cycle is for as long as any of those IBB DTV services are being exhibited.

6.1106	service_id [b-ITU-T J.94]: A unique identifier of a service within a TS.

6.1107	services [b-ITU-T J.294]: The set of services that are required and to be supported by the RG.

6.1108	session [b-ITU-T J.212]: An L2TP data plane connection from the M-CMTS Core to the QAM channel. There must be one session per QAM Channel. There is one DEPI pseudowire type per session. There may be one MPT flow or one or more PSP flows per session. Multiple sessions may be bound to a single control connection.

6.1108bis	session [b-ITU-T J.280]: A session is the insertion of content (such as spot advertisements of various lengths, program substitution, public service announcements, or program material created by splicing portions of the program from a server). Each session is identified by a unique SessionID.

6.1109	session client [b-ITU-T J.284]: One end that sets up a connection.

6.1110	session initiation protocol (SIP) [b-ITU-T J.291], [b-ITU-T J.361]: An application-layer control (signalling) protocol for creating, modifying, and terminating sessions with one or more participants. These sessions include Internet telephone calls, multimedia distribution, and multimedia conferences.

6.1111	session initiation protocol plus	[b-ITU-T J.361]: An extension to SIP.

6.1112	session server [b-ITU-T J.284]: One end that accepts a connection request.

6.1113	session word [b-ITU-T J.96]: relates to the word assigned during a transmission by the Management Centre.

6.1114	set top box (STB)	[b-ITU-T J.197]: Any device that receives content directly from a video service provider, this includes both devices that are separate from the display device, and display devices that have the proper embedded functionality. The STB functions as the service gateway for the home network and includes the conditional access (CA) system and a Digital Rights Management (DRM) system.

6.1114bis set-top box [b-ITU-T J.183]: A hardware box that contains a digital signal demodulator, de-multiplexer, Motion Picture Experts Group version 2 (MPEG-2) decoder, other functionalities and interfaces related to digital signal reception and presentation of the distributed programme at the subscriber's site.

6.1114ter set-top box [b-ITU-T J.296]: A hardware box that contains a digital signal demodulator, de-multiplexer, decoder, and other functionalities and interfaces related to digital signal reception and presentation of the distributed programme at the subscriber's site. The set-top box that satisfies the requirements defined in this Recommendation is called a hybrid cable set-top box.

6.1115	set-top controller [b-ITU-T J.128], [b-ITU-T J.290]: This is the computer system responsible for managing the Set-top Devices within a cable system. It manages Set-top Devices through control and information messages sent via the Out-Of-Band channel.

6.1116	set-top device	[b-ITU-T J.126]: An eDOCSIS device that contains an eSTB.

6.1116bis set-top device [b-ITU-T J.128]: A cable receiver that contains an embedded Cable Modem for DOCSIS connectivity and an embedded Set-top Box.

6.1116ter set-top device [b-ITU-T J.290]:A receiver that contains an embedded PS function for home network connectivity and an embedded set-top box.

6.1117	short form insertion [b-ITU-T J.287]: Refers to insertions of material with a duration of generally less than 10 minutes, i.e., advertising or promotional material. At the time of writing, it is the primary use of DPI technology.

6.1118	Short-time waveform distortion [b-ITU-T J.67]: If a short pulse (or a rapid step-function) of nominal luminance amplitude and defined shape is applied to the input of the circuit, the short-time waveform distortion is defined as the departure of the output pulse (or step) from its original shape.

6.1119	SID Cluster	[b-ITU-T J.222.2]: A group of SIDs containing one and only one SID for each upstream channel within an upstream bonding group and treated the same from a request/grant perspective.

6.1120	SID Cluster Group	[b-ITU-T J.222.2]: The set of all SID Clusters associated with a specific service flow.

6.1121	Signal-to-chrominance periodic noise ratio [b-ITU-T J.64]: This parameter is to be measured on the part of the signal used in § 2.15 above. It is defined as the ratio of the amplitude of the luminance bar (element B2) to the peak-to-peak amplitude of spurious signals in a total 3 dB bandwidth of 0.2 MHz centred on the appropriate colour sub-carrier frequency as in § 2.15 above. The result of the measurement is to be given in dB.

6.1122	Signal-to-quasi peak-to-peak noise ratio [b-ITU-T J.64]: The signal-to-quasi peak-to-peak noise ratio is defined as the ratio of the amplitude of the luminance bar (element B2) to the value exceeded by the noise voltage deviation for a specified measurement time percentage (see Notes 1 and 2). It may be measured both under weighted or unweighted conditions. The comparison between these parameters and that defined in § 2.15.1 and 2.15.2 is intended to confirm the Gaussian nature of the noise. They are to be given in dB.
    NOTE 1: The upper limit of the noise bandwidth is selected so as to eliminate noise which occurs outside of the wanted band of the video signal. The high pass filter and the notch filter are used to minimize the effects of periodic noise at low frequencies and at the sub-carrier frequency, respectively. The high pass filter has also been specified to minimize the measurement errors caused by residual wave-form distortion in the measurement period.
    Attention is directed to the fact that the high pass filter and the notch filter modify the spectral composition of the random noise and therefore alter its r.m.s. or quasi peak-to-peak value. The conversion factors in dB established for noise with a spectrum ideally limited to 5 MHz are given in Table I (see also [CCIR, 1978-82b]).
 	NOTE 2: Further study is required to specify this percentage.

6.1123	Signal-to-unweighted random noise ratio [b-ITU-T J.64]: The signal-to-unweighted random noise ratio is defined as the ratio of the amplitude of the luminance bar (element B2) to the r.m.s. value of the noise measured on a specified line, or part of this line, (line 22, or optionally both lines 22 and 335, in the case of 625-line signals). It is to be given in dB. The noise bandwidth is assumed to be limited by the low pass filter defined in Recommendation 567 Annex II to Part C. Lower frequency limiting shall be done by a 200 kHz high pass filter with a slope of 20 dB per decade (see Note).
    To suppress any periodic noise at sub-carrier frequency, a notch filter should be used (see Note).
    For 625-line signals, the amplitude/frequency response of the filter should be as in Fig. 8 and a possible implementation of the filter as a constant impedance network is given in [CCIR, 1978-82a].

NOTE: The upper limit of the noise bandwidth is selected so as to eliminate noise which occurs outside of the wanted band of the video signal. The high pass filter and the notch filter are used to minimize the effects of periodic noise at low frequencies and at the sub-carrier frequency, respectively. The high pass filter has also been specified to minimize the measurement errors caused by residual wave-form distortion in the measurement period.
Attention is directed to the fact that the high pass filter and the notch filter modify the spectral composition of the random noise and therefore alter its r.m.s. or quasi peak-to-peak value. The conversion factors in dB established for noise with a spectrum ideally limited to 5 MHz are given in Table I (see also [CCIR, 1978-82b]).
6.1124	Signal-to-weighted random noise ratio [b-ITU-T J.64]:The signal-to-weighted random noise ratio is defined as in § 2.15.1 above, save for the addition of the unified weighting network specified by the CCIR in Recommendation 567.

6.1125	signal measurement (picture) [b-ITU-T J.144]: The measurement of the performance of a programme chain by the use of test signals and objective (instrumental) measurement methods.

6.1126	signalling frame [b-ITU-T J.195.2]: Frame of the MAC layer used for node admission, node quitting/deletion and link maintenance.

6.1127	Simple Network Management Protocol (SNMP) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A network management protocol of the IETF.

6.1127bis	Simple Network Management Protocol (SNMP) [b-ITU-T J.296]: An Internet-standard protocol for managing devices on IP networks. Essentially, SNMP agents expose management data on the managed systems as variables. The Protocol also permits active management tasks, such as modifying and applying a new configuration through remote modification of these variables. The variables that are accessible via SNMP are organized in hierarchies. These hierarchies, and other metadata (such as type and description of the variable), are described by management information bases (MIBs).

6.1128	simple object access protocol (SOAP) [b-ITU-T J.291]: A lightweight, XML-based protocol for exchange of information in a decentralized, distributed environment.

6.1128bis	simple object access protocol/service oriented architecture protocol (SOAP) [b-ITU-T J.380.7]: A way for a program executing in one kind of operating system to communicate with a program executing in the same or another kind of operating system by using the World Wide Web's Hypertext Transfer Protocol (HTTP) and its Extensible Markup Language (XML) as the mechanisms for information exchange.

6.1129	simple profile [b-ITU-T J.287]: A defined subset of the automation to injector messages in this API which supports all basic splicing functionality while excluding schedules, encryption and component mode. An implementer may choose to support only the "Simple Profile" or features beyond it. The implementer can then describe their implementation in common terms (for example "Simple Profile plus encryption").

6.1130	simulated transmission errors	[b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342]: Errors imposed upon the digital video bit stream in a highly controlled environment. Examples include simulated packet loss rates and simulated bit errors. Parameters used to control simulated transmission errors are well defined.

6.1131	single program transport stream [b-ITU-T J.181]: A transport stream containing a single MPEG program.

6.1131bis single program transport stream (SPTS) [b-ITU-T J.291]: A transport stream in which there is a single program.

6.1132	single-frequency network (SFN) (based on [ITU-R BT.1306-6]) [b-ITU-T J.296]: A broadcast network where several transmitters simultaneously send the same signal over the same frequency channel.

6.1133	SIP Client [b-ITU-T J.460.0], [b-ITU-T J.460.1]: The functional element subscribers use to attach to the IPCablecom2 network.

6.1134	slice layer [b-ITU-T J.88]: Composed of 768 pixels 16 lines active pixels, and comprises a horizontal arrangement of macro blocks. The slice is categorized into four types of ID and control signal area, boundary area, block area and active image area

6.1135	SMATV grouping terminal or interactive head-end [b-ITU-T J.118]: The Grouping Terminal at the SMATV performs the collection and the distribution of information coming from and going to the user terminals. It is composed by the Grouping Terminal Interactive Network Adapter (INA), the MAC functions and the interfaces with the other networks in order to collect/distribute the information in the SMATV coax network related to each user terminal (Interactive Interface Module IIM).

6.1136	SMATV network [b-ITU-T J.84]: A Satellite Master Antenna TV network intended for the broadband distribution of television, sound and data signals received directly from one or more satellites, possibly in frequency-division multiplex with similar terrestrial VHF/UHF signals, to households located in one or more adjacent buildings. Where intended also for the distribution of new digital multiprogramme television, sound and data services, such networks are known as "Digital SMATV networks" and the digital configuration for this purpose is known as "Digital multiprogramme SMATV System".

6.1137	SMATV system A (SMATV-DTM)	[b-ITU-T J.118]: "This system approach consists of the transmodulation from satellite Quaternary Phase Keying (QPSK) signals as defined in ITU-R BO.1516 to a Quadrature Amplitude Modulation (QAM) scheme as defined in ITU-T J.83. The process of transmodulation without baseband interfacing is also known as Transparent Transmodulation and the head-end unit performing this function is known as Transparent Digital Transmodulator (TDT)."

6.1138	SMATV system B [b-ITU-T J.118]: "This system is based on the use of QPSK modulation. The SMATV system B concept allows a direct reception of digital satellite signals using frequency conversion of the received satellite signal to a frequency band appropriate to the characteristics of the SMATV distribution network. The functional elements of this system are given in the baseline satellite specification provided in ITU-R BO.1516. Two configurations of SMATV system B are considered as follows:

SMATV-IF: which uses the Intermediate Frequency (IF) as delivered by the Low Noise Block (LNB) (e.g. 950-2150 MHz)

SMATV-S: which uses a conversion to the extended S band (e.g. 230-470 MHz).

6.1139	SMATV-D system	[b-ITU-T J.84]: A SMATV system equipped to receive QPSK-modulated digital television satellite signals and to distribute them, still in digital form, using QAM or QPSK modulation (the term "D" refers, in a generic way, to the ability to process digital television signals).

6.1140	SMATV-DTM system [b-ITU-T J.84]: A SMATV-D system based on digital transmodulation performed at headend from a QPSK-modulated satellite signal to a QAM-modulated cable signal, which is then distributed to users in the VHF/UHF bands (The approach is called "transparent", since the satellite carrier content is transferred to the cable carrier without demultiplexing or other baseband processing.)

6.1141	SMATV-IF [b-ITU-T J.84]: A SMATV-D system based on the direct distribution of the QPSK_x005f modulated television satellite signal, taken from the LNB and distributed in the extended IF band (e.g. 950-2050 MHz in Europe) without further processing apart from a possible frequency conversion within the IF band.

6.1142	SMATV-S [b-ITU-T J.84]: A SMATV-D system based on the direct distribution of the QPSK_x005f modulated television satellite signal, taken from the LNB and distributed in the "Extended Superband" (e.g. 230-470 MHz in Europe) without any further processing apart from frequency conversion.

6.1143	social television [b-ITU-T J.295]: This is a general term for technology that supports communication and social interaction in either the context of watching television, or related to TV content. It includes the study of television-related social behaviour, devices and networks. Social television systems can for example integrate voice communication, text chat, presence and context awareness, TV recommendations, ratings, or video-conferencing with the TV content, either directly on the screen or by using ancillary devices

6.1144	socket [b-ITU-T J.287]: A TCP/IP mechanism used for connection-oriented communications. Sometimes it is also called "port" in an interchangeable manner.

6.1145	software container	[b-ITU-T J.1010]: Set of software interfaces to the host and to the client, which strictly separates the CA/DRM client from the host. The provisioning of the interfaces enables the exchangeability of the CA/DRM clients.

6.1146	sound-programme circuit-section [b-ITU-T J.13]: "Part of an international sound-programme circuit between two stations at which the programme is transmitted at audio frequencies.

The normal method of providing a sound-programme circuit section in the international network will be by the use of carrier sound-programme equipment. Exceptionally sound-programme circuit sections will be provided by other means, for example, by using amplified unloaded or lightly loaded screened-pair cables or by using the phantoms of symmetric-pair carrier cables."

6.1147	sound-programme circuit section [b-ITU-T N.1]: The unidirectional national or international sound-programme transmission path between two stations at which the programme is accessible at audio frequencies. The transmission path may be established via terrestrial or single destination satellite routing.

6.1148	Source	[b-ITU-T J.117]: A device that produces or passes on OSD data.

6.1149	source coding (bit-rate reduction) [b-ITU-T J.248]: The encoding of the original digital signal (video, audio or data) in bit-rate reduction (BRR) representation before protection is applied against bit errors in the channel.

6.1150	source frame rate (SFR)	[b-ITU-T J.246], [b-ITU-T J.247]: The intended frame rate of the original source video sequences. The source frame rate is constant. For the VQEG MM Phase I test the SFR was either 25 fps or 30 fps.

6.1151	source identification [b-ITU-T J.27], [b-ITU-T N.13]: An announcement used to identify the originating point of the test signals. It should be as short as possible, and it should contain at least the following information:

– name of originating organization;

– location;

– country.

The sound-programme signal should be controlled by the sending broadcaster so that the amplitudes of the peaks only rarely exceed the peak amplitude of the permitted maximum (sine-wave test) signal.

6.1152	Spectrum Management System (SMS) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C]: A system, defined in [b-SMS], for managing the RF cable spectrum.

6.1152bis	spectrum management system (SMS) [b-ITU-T J.122]: A system, defined in [b-SMS], for managing the RF cable spectrum.

6.1153	splice event [b-ITU-T J.181], [b-ITU-T J.181 Amendment 1], [b-ITU-T J.287]: An opportunity to splice one or more PID streams.

6.1154	splice immediate mode [b-ITU-T J.181 Amendment 1], [b-ITU-T J.287]: A mode of the Cueing Message whereby the splicing device chooses the nearest opportunity in the stream, relative to the splice_info_table, to splice. When not in this mode, the Message gives a "pts_time", which is a Presentation Time, for the intended splicing moment.

6.1155	splice point	[b-ITU-T J.181], [b-ITU-T J.181 Amendment 1], [b-ITU-T J.189], [b-ITU-T J.287]: A point in a PID stream that is either an out point or an in point.

6.1156	splice time [b-ITU-T J.189]: A presentation time of the intended splice point, which is equivalent to the presentation time of the access unit following the intended splice point.

6.1157	splice_info_section [b-ITU-T J.287]: Basic ITU-T J.181 structure for carrying DPI commands in a TS to downstream equipment.

6.1158	spliceable stream [b-ITU-T J.189]: A PID stream whose discontinuity in a time-stamp or a time base can be processed seamlessly by the MPEG-2 bit stream splicing device. The basic syntax is defined in ITU-T Rec. H.222.0 | ISO/IEC 13818-1.

6.1159	splice-in [b-ITU-T J.280]: The splice at the start of the insertion. This happens at the time specified in the Splice_Request message.

6.1160	splice-out [b-ITU-T J.280]: The splice at the end of the insertion. The expected insertion end time is calculated by adding the start time and the duration specified in the Splice_Request message; however, this may occur earlier due to error conditions.

6.1161	splicer [b-ITU-T J.280]: The device that splices the insertion channel(s) into the primary channel(s). It may receive ANSI/SCTE 35 cue messages. This device also communicates with the server about when and what to splice.

6.1162	spot [b-ITU-T J.287]: Term for the content of an advertisement, and sometimes also used to refer to an avail.

6.1163	spread symbol [b-ITU-T J.222.1]: At the output of the spreader, a group of 128 chips which comprises a single S-CDMA spreading code, and is the result of spreading a single information (QAM constellation) symbol, which is referred to as a "spread symbol".

6.1164	spread symbol or spreading interval [b-ITU-T J.122]: At the output of the spreader, a group of 128 chips which comprise a single S-CDMA spreading code, and are the result of spreading a single information (QAM) symbol. One spread symbol = one spreading interval = 128 chips = one information (QAM) symbol.

6.1165	spreader-off S-CDMA burst [b-ITU-T J.122], [b-ITU-T J.222.1]: A transmission from a single CM in a spreader-off frame on an S-CDMA channel defined by the time in which the cable modem's transmitter turns on to the time it turns off. There will generally be several spreader off bursts in a spreader-off frame.

6.1166	spreader-off S-CDMA frame	[b-ITU-T J.122], [b-ITU-T J.222.1]: TDMA mini-slots on an S-CDMA channel in which the spreader is turned off. These are differentiated from TDMA bursts on a TDMA channel in that, for example, the number of mini-slots per spreader-off S-CDMA burst frame is constrained to be the same as the number of mini-slots in a spreader-on S-CDMA frame(s). This number of mini-slots will be less than the number of TDMA mini-slots in a TDMA channel over the same time interval if the number of active codes is significantly less than 128.

6.1167	spreading codes [b-ITU-T J.222.1]: A family of orthogonal digital codewords used in S-CDMA direct-sequence spread-spectrum modulation.

6.1167bis Spreading Codes [b-ITU-T J.222.2]: The set of 128 binary sequences of 128 bits each which may be used to carry information in the S-CDMA upstream. The spreading codes are orthogonal, meaning their cross-correlation is zero. Each code carries a single QAM symbol of information when the code's amplitude and phase are modulated.

6.1168	spreading interval	[b-ITU-T J.222.1]: The period of a spread symbol (128 chips) is called a "spreading interval".

6.1168bis	spreading interval	[b-ITU-T J.122], [b-ITU-T J.222.2]: Time to transmit a single complete S-CDMA spreading code, equal to the time to transmit 128 chips. Also, time to transmit a single information (QAM) symbol on an S-CDMA channel. See also spread symbol.

6.1169	stand-alone IBB application [b-ITU-T J.205 Cor. 2]: Resident or downloaded integrated broadcast and broadband (IBB) application that is not part of an IBB DTV service. Such an application can be created by an authorized IBB application provider, and typically delivered through the application repository.

6.1170	Stand-alone Multimedia Terminal Adapter (S-MTA) [b-ITU-T J.191]: A single node that contains an MTA and a non-DOCSIS MAC (e.g., Ethernet).

6.1171	standalone PS [b-ITU-T J.192]: A Portal Services element that connects to the CM using only a standalone interface.

6.1172	standalone RG [b-ITU-T J.193]: A RG Services element that connects to the CM using only a standalone interface.

6.1173	standard application [b-ITU-T J.296]: An application which a cable operator uses for service delivery, to be installed before or after shipment on decision by the cable operator.

6.1174	standard channel plan (STD) 	[b-ITU-T J.210]: Method of spacing NTSC television channels on a cable television system defined in [CEA-542-B].

6.1175	standard definition analogue form or output [b-ITU-T J.197]: A format or output that is not digital (e.g., PAL RF, NTSC RF, Composite, S-Video, YUV, Y, R-Y, B-Y or RGB) and has no more than 483 interlace or progressive active scan lines.

6.1176	standard upstream frequency range [b-ITU-T J.222.1]: The required upstream frequency range over which a CM is to be capable of transmitting. In the technology option that uses 6 MHz downstream channelization, this is 5-42 MHz. In the technology option that uses 8 MHz downstream channelization, this is 5-65 MHz.

6.1177	Static Multicast Sessions	[b-ITU-T J.222.3]: Multicast sessions joined during cable modem registration.

6.1178	StopCCN [b-ITU-T J.212]: L2TPv3 Stop-Control-Connection-Notification message.

6.1179	stream [b-ITU-T J.94]: An ordered series of bytes. The usual context for the term stream is the series of bytes extracted from Transport Stream packet payloads which have a common unique PID value (e.g., video PES packets or Program Map Table sections).

6.1180	stream classification [b-ITU-T J.195.1]: The process of distinguishing higher layer protocol data units (PDUs) in order to apply appropriate traffic and congestion control mechanisms.

6.1181	stream_id [b-ITU-T J.183]: A unique identifier of a transport stream (TS) or other generic stream within an original network.

6.1182	sub_table [b-ITU-T J.94]: A sub_table is collection of sections with the same value of table_id and:

– for a NIT: the same table_id_extension (network_id) and version_number;

– for a BAT: the same table_id_extension (bouquet_id) and version_number;

–for a SDT: the same table_id_extension (transport_stream_id), the same original_network_id and version_number;

– for a EIT: the same table_id_extension (service_id), the same transport_stream_id, the same original_network_id and version_number.

The table_id_extension field is equivalent to the fourth and fifth byte of a section when the section_syntax_indicator is set to a value of "1".

6.1183	sub-channel [b-ITU-T J.122]: A logical channel sharing the same upstream spectrum (RF centre frequency and RF channel) with other logical channels.

6.1184	subjective assessment (picture) [b-ITU-T J.144]: The determination of the quality or impairment of programme-like pictures presented to a panel of human assessors in viewing sessions.

6.1185	sublayer [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A subdivision of a layer in the Open Systems Interconnection (OSI) reference model.

6.1186	subnetwork [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: Sub-networks are physically formed by connecting adjacent nodes with transmission links.

6.1187	Subnetwork Access Protocol (SNAP) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: An extension of the LLC header to accommodate the use of IEEE 802 type networks as IP networks.

6.1188	subscriber [b-ITU-T J.295]: The registered subscriber (representative) to the cable TV service; shall include all persons in the household.

6.1188bis subscriber [b-ITU-T J.360]: An entity (comprising one or more users) that is engaged in a subscription with a service provider.

6.1188ter subscriber [b-ITU-T J.367]: A form of watcher that has asked the Presence service to notify it immediately of changes in the Presence Information of one or more presentities [b-OMA RD-PRS].

6.1188quater subscriber [b-ITU-T J.380.6]: The term subscriber is used to refer to one or more members of a viewing audience to whom advertising messages may be addressed. Depending on the transmission method and receiver technology employed, it may be possible to address an individual viewer, an entire household, or all households in a broadcast area such as a cable head-end, metropolitan market or some other aggregation.

6.1188 quinties subscriber [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C]: See definition for end user (clause 6.437).

6.1189	subscription [b-ITU-T J.360]: A contract for service(s) between a user and a service provider.

6.1189bis	subscription [b-ITU-T J.367]: The information kept by the presence service about a subscriber's request to be notified of changes in the Presence Information of one or more presentities [b-OMA RD-PRS].

6.1190	subsplit	[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.122]: A frequency-division scheme that allows bidirectional traffic on a single cable; in North America, reverse path signals come to the headend from 5 to 30 (up to 42 on Extended Subsplit systems) MHz, and forward path signals go from the headend from 50 or 54 MHz to the upper frequency limit of the cable network.

6.1191	subsystem [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: An element in a hierarchical division of an open system that interacts directly with elements in the next higher division or the next lower division of that open system.

6.1192	subtitle	[b-ITU-T J.296]: A service of superimposing related text on a TV video broadcast.

6.1193	sub-unit [b-ITU-T J.117]: A uniquely identifiable and addressable entity contained within a unit.

6.1194	superimpose [b-ITU-T J.296]: A subtitle provided asynchronously to the main video, audio, and data. It is used for up-to-the-minute news, changes in air times, and time signals.

6.1195	supplemental [b-ITU-T J.287]: A category of request operation supported by this API. See clause 8.3.

6.1196	switch engine	[b-ITU-T J.215]: This term refers to the functionality that executes switching in the host device. This switching can either be seamless, or non-seamless. Because this functionality needs to be implemented in a real-time way, with predictable timing behaviour, the switching engine is implemented as part of an OpenCable Applications Platform (OCAP) implementation ("below the line"). It exposes an API to OCAP applications (targeting engines) to select which commercials are switched to/from. A switching engine can be implemented in hardware, software, or a combination of both.

6.1197	Sync. amplitude error [b-ITU-T J.64]: Sync. amplitude error is defined as the difference between sync. amplitude and its normalized value (i.e. 3/7 luminance bar amplitude for 625-lines, 4/10 luminance bar amplitude for 525-lines) (see Note 1) expressed as a percentage of the normalized value. The sign of the difference is positive if sync. pulses are larger than the normalized value.

To provide a measurement result in the presence of sound-in-syncs signals, sync. amplitude must be measured at the mid-point of the last broad pulse of each field (point b8 in Fig. 6) (see Note 2).

NOTE 1: The luminance bar amplitude is defined in § 2.1 of [b-ITU-T J.64].

NOTE 2: To avoid error due to field tilt it may be preferable to use a reference point exclusively for the measurement of sync. amplitude error which is placed at point b9 in Fig. 6 of each field.

6.1198	synchronization packet [b-ITU-T J.285]: A TCP packet that controls a sender's transfer rate in the application layer. The packet is transmitted from a receiver to a sender.

NOTE: The packet is not the TCP SYN segment.

6.1199	Synchronous Code Division Multiple Access (S-CDMA) [b-ITU-T J.291]: A variant of the protocol used for wireless communication, used in DOCSIS 2.0.

6.1199bis synchronous-code division multiple access (S-CDMA) [b-ITU-T J.222.1]: A multiple access physical layer technology in which different transmitters can share a channel simultaneously. The individual transmissions are kept distinct by assigning each transmission an orthogonal "code". Orthogonality is maintained by all transmitters being precisely synchronized with one another.

6.1200	synthesized timeline [b-ITU-T J.203]: A timeline for a piece of content which was synthesized by the GEM recording terminal (as opposed to being included as part of the piece of content when it was transmitted).

6.1201	system and service information [b-ITU-T J.193]: Information about the video service, for example channel maps that map the programs within an MPEG transport stream, electronic programming guide data, and conditional access information.

6.1202	system clock period [b-ITU-T J.122]: The period of the 10.24 MHz system clock, nominally 97.65625 ns.

6.1203	systems management [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: Functions in the application layer related to the management of various Open Systems Interconnection (OSI) resources and their status across all layers of the OSI architecture.

6.1204	T1 [b-ITU-T J.214]: T1 is a physical layer telephony protocol carrying data at 1544 Mbit/s. It can carry up to 24 DS0s, each of which can carry a telephone conversation. It is used mainly in North America.

6.1205	table [b-ITU-T J.94]: A table is comprised of a number of sub_tables with the same value of table_id.

6.1206	table instance	[b-ITU-T J.94]: Tables are identified by the table_id field. However, in cases such as the RRT and EIT, several instances of a table are defined simultaneously. All instances have the same PID and table_id but different table_id_extension.

6.1207	taboo channel [b-ITU-T J.87]: A channel which coincides with the frequency of the local oscillator in the single super heterodyne receiver which is tuned to an analogue channel.

6.1208	tag header [b-ITU-T J.213]: A 16-bit tag protocol ID (0x8100) followed by a 16-bit tag control field. The tag control field consists of a 3-bit User Priority field, a 1-bit Canonical Format Indicator, and a 12 bit VLAN ID [b-IEEE 802.1Q].

6.1209	target advertising 	[b-ITU-T J.295]: An advertising service from the operator that targets an individual, a viewer segment, specific markets, specific areas, or facilities, etc.

6.1210	TDM emulation adapter (TEA) [b-ITU-T J.214]: A logical entity containing various functions to provide a TDM-E circuit emulation service.

6.1211	TDM emulation cable modem (TE-CM) [b-ITU-T J.214]: A special purpose cable modem that integrates a DOCSIS eCM with an eTEA.

6.1212	TDM interface [b-ITU-T J.214]: A physical time domain multiplex (TDM) telephony interface such as T1 or E1. Time division multiplexing (TDM) is the means by which multiple digital signals (DS0s) can be carried on a single transmission path by interleaving portions of each signal in time.

6.1213	TDM service processor (TSP)	[b-ITU-T J.214]: Consists of a TSP data interface on one side, a TDM interface on the other side, and the functionalities to encapsulate TSP data into TDM interface signals in one direction and extract TSP data from TDM interface signals in the other direction.

6.1214	TD-OD	[b-ITU-T J.293]: A logical interface defined in 6.1.7 between an output device and the STB.

6.1215	TD-PD	[b-ITU-T J.293]: A logical interface defined in 6.1.6 for connection between a peripheral device and the STB.

6.1216	TE-CMTS [b-ITU-T J.214]: A TE-CMTS is a CMTS or M-CMTS that can support a TE-CM and can meet the minimum clocking and performance goals to support TDM-E traffic. A TE-CMTS may or may not support integrated TEAs.

6.1217	telemetry service [b-ITU-T J.291]: Remote monitoring of data for purposes such as energy management or security in the home.

6.1218	television circuit section [b-ITU-T N.51]: The unidirectional national or international television transmission path between two stations at which the programme is accessible at video frequencies. The transmission path may be established via terrestrial or single satellite routing.

6.1219	television receive-only station (TVRO) [b-ITU-T N.51]: An earth station which is used only for reception. In this respect the term is used to denote any TVRO whose owner is authorized to receive the programme material.

6.1220	terminal [b-ITU-T J.295]: Equipment consisting of hardware and software with communication means, and equipped with a user interface to enable the user to connect to the network.

6.1221	terminal adapter [b-ITU-T J.361]: A device that converts an analogue tip and ring interface into a digital signal; it includes a hybrid to convert the interface from 2-wire to 4-wire.

6.1222	terminal device (TD) [b-ITU-T J.702]: An end-user device which typically presents and/or processes content, such as a personal computer, a computer peripheral, a mobile device, a TV set, a monitor, a VoIP terminal or an audiovisual media player.

6.1223	terrestrial digital TV broadcast [b-ITU-T J.296]: A mode of digital television broadcasting which does not involve satellite transmission.

6.1224	tick [b-ITU-T J.112], [b-ITU-T J.116]: Time intervals that are the reference for upstream mini-slot definition and upstream transmission times.

6.1224bis	tick [b-ITU-T J.112 Annex B], [b-ITU-T J.122], [b-ITU-T J.222.1]: 6.25-microsecond time intervals that are the reference for upstream mini-slot definition and upstream transmission times.

6.1224ter tick [b-ITU-T J.112 Annex C]: 6.9444…. microsecond time intervals that are the reference for upstream mini-slot definition and upstream transmission times.

6.1225	tilt [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: Maximum difference in transmission gain of a cable television system over a given bandwidth (typically over the entire forward operating frequency range).

6.1226	time division multiplexing (TDM) [b-ITU-T J.126]: The means by which multiple digital signals can be carried on a single transmission path by interleaving portions of each signal in time.

6.1227	Timebase Tick [b-ITU-T J.222.2]: 6.25-microsecond or 6.94-microsecond time intervals that are the reference for upstream mini-slot definition and upstream transmission times for TDMA channels.

6.1228	time-stamp [b-ITU-T J.89, b-ITU-T J.187]: A term that indicates the time of a specific action such as the arrival of a byte or the presentation of a presentation unit.

6.1229	Timing reference [b-ITU-T J.101]: The timing reference for each line is a uniform sequence of timing instants whose timing is derived only from the clock run-in of that line excluding the first two bits.

The timing of these instants is such that they coincide with the average timing of the points where the clock run-in crosses the mean value defined in § 2.5.

6.1230	track [b-ITU-T J.123], [b-ITU-T J.124]: A collection of related samples, which corresponds to a sequence of images or sampled audio.

6.1231	transaction initiator [b-ITU-T J.284]: One end that initiates a video transmission transaction operated by a human.

6.1232	transaction recipient [b-ITU-T J.284]: One end that accepts a video transmission transaction automatically.

6.1233	transcoder [b-ITU-T J.295]: An image conversion process to convert images to match the display capability of the terminal.

6.1234	transit delay	[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: The time difference between the instant at which the first bit of a PDU crosses one designated boundary, and the instant at which the last bit of the same PDU crosses a second designated boundary.

6.1235	transit delays	[b-ITU-T J.161]: The time difference between the instant at which the first bit of a PDU crosses one designated boundary, and the instant at which the last bit of the same PDU crosses a second designated boundary.

6.1236	Transmission Control Protocol (TCP) [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C]: A transport-layer Internet protocol which ensures successful end-to-end delivery of data packets without error, as defined by the IETF.

6.1237	transmission control protocol (TCP) [b-ITU-T J.380.7]: A set of rules used along with IP to send data in the form of message units between computers over the Internet.

6.1237bis	transmission control protocol (TCP) [b-ITU-T J.116], [b-ITU-T J.122]: A transport-layer Internet protocol which ensures successful end-to-end delivery of data packets without error, as defined by the IETF.

6.1237ter	Transmission Control Protocol (TCP) (based on [IETF RFC 793]) [b-ITU-T J.296]: A transport layer protocol that provides highly reliable end-to-end, connection-oriented data delivery using an error detection and correction mechanism.

6.1238	transmission convergence sublayer	[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: A sublayer of the Physical Layer that provides an interface between the Data Link Layer and the PMD Sublayer.

6.1239	transmission errors [b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342]: Any error imposed on the video transmission. Example types of errors include simulated transmission errors and live network conditions.

6.1240  transmission link [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: The physical unit of a subnetwork that provides the transmission connection between adjacent nodes.

6.1241	transmission medium [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122] The material on which information signals may be carried; e.g. optical fibre, coaxial cable, and twisted wire pairs.

6.1242	transmission system [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: The interface and transmission medium through which peer physical layer entities transfer bits.

6.1243	transmit on/off ratio [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: In multiple-access systems, the ratio between the signal powers sent to line when transmitting and when not transmitting.

6.1244	trans-modulation [b-ITU-T J.296]: A transmission method of a transport stream (TS). It is created and operated by other media and broadcasted to cable subscribers without changing parameters in the TS.

6.1245	Transparent Digital Transmodulator (TDT) [b-ITU-T J.84]: A headend device for SMATV_x005f DTM systems that transparently processes the QPSK-modulated television satellite signal, just adapting its modulation and coding so that it can be fed through the SMATV system using QAM modulation.

6.1246	transparent LAN service (TLS) [b-ITU-T J.213]: A service offering of a cable operator that implements a private L2VPN among the CPE networks of the CMs of a single subscriber enterprise.

6.1247	transport stream (TS) (based on [ITU-T H.262] [b-ITU-T J.93], [b-ITU-T J.95], [b-ITU-T J.200],[b-ITU-T J.296]: The transport stream defined by the MPEG-2 system standard (in digital terrestrial television broadcasting, one TS is assigned to a master transmitter).

6.1247bis transport stream [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: In MPEG-2, a packet-based method of multiplexing one or more digital video and audio streams having one or more independent time bases into a single stream.

6.1247ter Transport Stream (TS) [b-ITU-T J.120]: A data packet possessing a length of 188 bytes including 4 bytes of header information. The header contains MPEG related data.

6.1247quater transport stream (TS) [b-ITU-T J.281], [b-ITU-T J.282]: A data structure defined in ITU-T Rec. H.222.0 | ISO/IEC 13818-1.

6.1247quintic transport stream (TS)  [b-ITU-T J.292]: The transport stream described in this Recommendation is delivered using a certain multicast flow, which is identified by multicast group address, UDP port number, etc.

6.1248	transport stream packet header [b-ITU-T J.89], [b-ITU-T J.187]:: The leading fields in a transport stream packet, up to and including the continuity_counter field

6.1249	transport_stream_id [b-ITU-T J.94]: A unique identifier of a TS within an original network.

6.1250	tree distribution structure [b-ITU-T J.195.1]: A physical structure of a passive coaxial access network and a multiple stage power distribution network realized by splitters and taps or by taps only.

6.1251	trick mode functionality	[b-ITU-T J.702]: The ability to pause, rewind or forward stored content.

6.1252	trigger [b-ITU-T J.200]: An event that may cause a change in the behaviour of the application that registers interest in such events. Triggers may come from many sources, e.g., the broadcast stream, or may be generated from other data (such as the system clock). It also can carry some semantically significant payload in order to affect changes in an application based on information not available at the time the application was written.

6.1253	Trivial File Transfer Protocol (TFTP) or Trivial File-Transfer Protocol (TFTP)  [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]: An Internet protocol for transferring files without the requirement for user names and passwords that is typically used for automatic downloads of data and software.

6.1254	trunk [b-ITU-T J.161], [b-ITU-T J.178], [b-ITU-T J.460.0], [b-ITU-T J.460.1]: An analog or digital connection from a circuit switch that carries user media content and may carry voice signaling (MF, R2, etc.).

6.1255	trunk cable [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]: Cables that carry the signal from the headend to groups of subscribers. The cables can be either coaxial or fibre depending on the design of the system.

6.1256	trust authority (TA) [b-ITU-T J.1011]: Organization governing all rules and regulations that apply to implementations of ECI. Note that the trust authority has to be a legal entity to be able to achieve legal claims. The trust authority needs to be impartial to all players in the downloadable CA/DRM ecosystem.

6.1257	trusted third party (TTP) [b-ITU-T J.1011]: Technical service provider which issues certificates and keys to compliant manufacturers of the relevant components of an ECI-system under control of the trust authority (TA).

6.1258  type/length/value (TLV) [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C]: Encoding of three fields, in which the first field indicates the type of element, the second the length of the element, and the third value.

6.1259 	types of sound-programme circuit [b-ITU-T N.1]: The various types of international sound-programme circuit or sections of such circuits should be referred to by quoting the top nominal frequency, in kHz, effectively transmitted.

Example: 10-kHz sound-programme circuit.

6.1260	TVE [b-ITU-T J.181 Amendment 1]: Acronym for "TV Everywhere", after being authenticated as a subscriber to an operator, it is the ability to view TV content on the internet in addition to on one's television.

6.1261	Two-dimensional VLC [b-ITU-T J.88]: Huffman variable-length code having the combination of zero-run length and quantization output level as a symbol.

6.1262  Two-level chrominance amplitude non-linearity [b-ITU-T J.64]: This parameter is to be measured with element G or G2. Its value, expressed in per cent, and with a sign, is defined by:

where V1 and V3 are respectively the peak-to-peak amplitudes of the first and last sections of element G or G2.

6.1263	Two-level chrominance phase non-linearity [b-ITU-T J.64]: 	This parameter is to be measured with element G or G2. Its value, expressed in degrees, and with a sign, is defined by:

3  –  1

where 3 and 1 are respectively the phases of the last and first sections of element G or G2.

6.1264	two-way [b-ITU-T J.128], [b-ITU-T J.290]: This expression infers that the downstream path and the upstream path are operational.

6.1265	type/length/value (TLV) [b-ITU-T J.112], [b-ITU-T J.116], [b-ITU-T J.122]: An encoding of three fields in which the first field indicates the type of element, the second the length of the element, and the third field the value.

6.1266	unavailable sub-carrier [b-ITU-T J.195.2]: Sub-carriers of OFDM symbol for adjacent channel protection and zero frequency sub-carrier.

6.1267	unicast advertising [b-ITU-T J.215]: Advertising content directed at a single target or small group of targets.

6.1268	uninterruptible power supply [b-ITU-T J.460.2]: A power supply including a battery for backup power when AC input power fails.

6.1269	UniqueQualifier [b-ITU-T J.380.8]: A "Unique Qualifier" is a set of one or more Qualifier elements that – taken together – uniquely identify an object in a logical service's basic query data model. To "uniquely identify" an object means that no other object in the data store has the same UniqueQualifier. However, an object may have more than one UniqueQualifier.

6.1270	UniqueQualifierDeclaration [b-ITU-T J.380.8]: The "Unique Qualifier Declaration" defines the set of Qualifier element characteristic name identifiers comprising a unique qualifier. When each specified Qualifier element's named characteristic identifier is paired with a value (i.e., a name/value pair which is a Qualifier element), the result is a service data model UniqueQualifier.

6.1271	unit [b-ITU-T J.117]: The instantiation of an AV/C device. A unit is addressable in a specific way using AV/C commands. A unit may contain zero or more sub-units.

6.1271bis	unit [b-ITU-T J.96]: relates to a device for which this Recommendation might apply.

6.1272	universal plug and play (UPnP) [b-ITU-T J.291]: Set of standards for device and content discovery on home networks, promulgated by the UPnP Forum.

6.1273	unpacking [b-ITU-T J.195.1]: A procedure of dividing a high performance network over coax (HiNoC) media access control (MAC) frame into individual Ethernet MAC frames; the opposite of packing.

6.1274	upconverter [b-ITU-T J.212]: A device used to change the frequency range of an analog signal, usually converting from a local oscillator frequency to an RF transmission frequency.

6.1275	uplink [b-ITU-T J.195.2]: Link from HiNoC modem (HM) to HiNoC bridge (HB).

6.1276	upstream [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.361]: The direction from the subscriber location toward the headend.

6.1276bis upstream [b-ITU-T J.116]: The direction from the subscriber location toward the BTS.

6.1276ter upstream [b-ITU-T J.184]: Transmission from terminal device to Headend.

6.1276quater upstream (US) [b-ITU-T J.212]: 1)Transmissions from CM to CMTS. This includes transmission from the EQAM to M-CMTS Core as well as the RF transmissions from the CM to the EQAM. 2)RF spectrum used to transmit signals from a subscriber location to a cable operator's headend or hub site.

6.1276quintic Upstream [b-ITU-T J.222.3]: The term used to describe traffic and paths that go from the subscriber to the headend.

6.1277	Upstream Bonded Channel [b-ITU-T J.222.2]: One of a group of independent upstream RF channels whose data packets are logically combined into one higher-speed data stream.

6.1278	Upstream Bonded Service Flow [b-ITU-T J.222.2]: An upstream Service Flow for which Upstream MAC Frames or Segments are transmitted on one or more Upstream Channels.

6.1279	Upstream Bonding Group [b-ITU-T J.222.2]: A subcomponent object of a MAC Domain that collects and resequences/reassembles Upstream Segments from a UBSF from an administered set of UCs.

6.1280	upstream channel descriptor (UCD) [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.210], [b-ITU-T J.212], [b-ITU-T J.222.1]: The MAC management message used to communicate the characteristics of the upstream physical layer to the cable modems.

6.1281	Upstream Service Group [b-ITU-T J.222.2]: The complete set of Upstream Channels (UCs) within a single CMTS potentially reachable by the transmission of a single Cable Modem. In an HFC deployment, a US SG corresponds to the physical combining of the upstream reverse carrier path signal from one or more Fibre Nodes reaching a single CMTS. [Changes to definitions made per ECN MULPIv3.0-N-06.0371-4 by kn on 1/26/07.]

6.1282	usage data [b-ITU-T J.363], [b-ITU-T J.460.3]: A collection of data representing the usage of network resources for a given session.

6.1283	user [b-ITU-T J.295]: The user of a device connected to a set-top box and a home network. This refers not only to human beings, but may also include logical elements for the communication and usage of applications between devices.

6.1283bis user [b-ITU-T J.360]: A person who, in the context of this Recommendation, uses a defined service or invokes a feature on a UE.

6.1284	user agent [b-ITU-T J.200]: An embodiment of a declarative application environment.

6.1284bis	user agent (UA) [b-ITU-T J.360]: A SIP user agent as defined by [b-IETF RFC 3261].

6.1285	user datagram protocol [b-ITU-T J.161], [b-ITU-T J.361]: A connectionless protocol built upon Internet Protocol (IP).

NOTE: Delay and latency are similar concepts and frequently used interchangeably. However, delay focuses on the time to transit from transmitter (such as a speaker's mouth) to a receiver (such as a listener's ear), while latency focuses on the time to transit from a receiver to a transmitter, as would be the case for a signal going through a piece of equipment.

6.1285bis User Datagram Protocol (UDP) (based on [b-IETF RFC 768]) [b-ITU-T J.296]: A transport layer protocol between two hosts without a confirmation function but which minimizes protocol overhead and is a connectionless type of communication suitable for services with high transmission efficiency.

6.1286	user equipment [b-ITU-T J.261]: Any device used directly by an end user to communicate.

6.1287	user interface (UI) [b-ITU-T J.295]: The basic UI is provided by the operating system and defines the display format of information to the user and the data input and operation methods by the user. These requirements assume the use of both character based UIs (CUI) and graphics based UIs (GUI).

6.1288	variable frame skipping [b-ITU-T J.246], [b-ITU-T J.247]: An event where the HRC outputs frames with updated content at an effective frame rate that changes with time. The temporal delay through the system will increase and decrease with time, varying about an average system delay. A processed video sequence containing variable frame skipping will be approximately the same duration as the associated original video sequence.

6.1289	video content protection system (VCPS) [b-ITU-T J.197]: For recording encrypted content on DVD+RW and DVD+R optical digital media protected by VCPS technology.

6.1290	video distribution [b-ITU-T J.283]: Digital video services for an unspecified number of clients.

6.1291	video on demand (VoD) [b-ITU-T J.702]: A service in which the subscriber can view and/or select a stored video content whenever desired.

6.1292	video-on-demand (VoD) system [b-ITU-T J.210], [b-ITU-T J.212]: System that enables individuals to select and watch video content over a network through an interactive television system.

6.1293	video server [b-ITU-T J.284]: Video server at a broadcaster's station.

6.1294	video service provider (VSP) [b-ITU-T J.197]: A service provider offering a "service" as defined in this Recommendation.

6.1295	view log [b-ITU-T J.296]: Information history of pay-per-view content viewing.

6.1296	view range [b-ITU-T J.902]: The position and direction of a viewpoint in a three-dimensional (3D) scene, where a virtual view can be generated.

6.1297	viewer [b-ITU-T J.295]: A subscriber who is receiving video, audio and/or other information services.

6.1298	viewing event [b-ITU-T J.181], [b-ITU-T J.181 Amendment 1]: A television program or a span of compressed material within a service; as opposed to a splice event, which is a point in time.

6.1299	virtual channel [b-ITU-T J.94]: A virtual channel is the designation, usually a number, that is recognized by the user as the single entity that will provide access to an analog TV program or a set of one or more digital elementary streams. It is called "virtual" because its identification (name and number) may be defined independently from its physical location. Examples of virtual channels are: digital radio (audio only), a typical analog TV channel, a typical digital TV channel (composed of one audio and one video stream), multi-visual digital channels (composed of several video streams and one or more audio tracks), or a data broadcast channel (composed of one or more data streams). In the case of an analog TV channel, the virtual channel designation will link to a specific physical transmission channel. In the case of a digital TV channel, the virtual channel designation will link both to the physical transmission channel and to the particular video and audio streams within that physical transmission channel.

6.1300	virtual LAN (VLAN) [b-ITU-T J.213]: A subset of the LANs of an IEEE 802.1 bridged network to which a VLAN identifier (VLAN ID) is assigned. An L2VPN may consist of several VLANs, each with different VLAN IDs, and even of VLANs on different IEEE 802.1 bridged networks with the same VLAN ID.

6.1301	virtual LAN identifier (VLAN ID) [b-ITU-T J.213]: An IEEE 802.1Q VLAN ID is a 12-bit number that identifies a VLAN within an IEEE 802.1 bridged network. An IEEE 802.1ah stacked VLAN ID consists of an outer service 12-bit VLAN ID and an inner customer 12-bit VLAN ID.

6.1302	virtual machine (based on [ITU-T Q.1741.1]) [b-ITU-T J.296]: A software program that simulates a hypothetical computer central processing unit. The programs executed by a virtual machine are represented as byte codes, which are primitive operations for this hypothetical computer.

6.1303	VoD transmission [b-ITU-T J.127]: Program transmission method whereby the program starts playing after a certain amount of data has been buffered while receiving subsequent data in the background, where the program is completely created by the content provider.

6.1304	VoIP Positioning Centre [b-ITU-T J.460.1]: A specialized service operator who determines which PSAP should get a VoIP 9-1-1 call given the reported location of the caller and supplies that location to the PSAP when the PSAP consults the ALI.

6.1305	Vq [b-ITU-T J.148]: Objective measurement of video quality

6.1306	Vq(Aq) [b-ITU-T J.148]: Objective measurement of video quality, accounting for the influence of audio quality.

6.1307	watcher [b-ITU-T J.367]: Any uniquely identifiable entity that requests Presence Information about a presentity, or watcher information about a watcher, from the Presence service. Special types of watchers are fetchers and subscribers [b-OMA RD-PRS].

6.1308	watcher Information [b-ITU-T J.367]: Information about watchers that have received or may receive Presence Information about a particular presentity within a particular recent span of time [b-OMA RD-PRS].

6.1309	Waveforms and line allocations [b-ITU-T J.67]: The MAC analogue waveform is directly derived from the standard 4:2:2 sampling ratio used for digital television (ITU-R BT.601). MAC coding produces a sequential transmission of a chrominance signal, compressed in a 3:1 ratio, and the luminance signal, compressed in a 3:2 ratio.

Given the sampling frequencies defined for the digital television standard (13.5 MHz for luminance and 6.75 MHz for chrominance), the consequent MAC sampling frequency is 20.25 MHz. The resulting nominal bandwidth required for the coded MAC signal is 8.4 MHz. After decompression the luminance bandwidth is 5.6 MHz.

It is important to note that, even though the MAC signal is derived through a sampling process, the resulting signal has an analogue form for transmission. A remarkable feature of the MAC coding system is that there is no absolute limit for the bandwidth. This characteristic can be used to broadcast the MAC signal in a narrow-band channel.

6.1310	Web services description language (WSDL) [b-ITU-T J.380.7]: An XML based general purpose language for describing interfaces, protocol bindings, and deployment details of network services.

6.1311	Webcasting [b-ITU-T J.120]: Distribution of sound and television programs over the Internet.

6.1312	well-known MAC address [b-ITU-T J.290]: This refers to the MAC address of the client within the set-top device. This MAC address has been assigned by the manufacturer of the conditional access system within the set-top device.

6.1312bis well-known MAC address (in the context of DOCSIS Set-top Gateway) [b-ITU-T J.128]: This refers to the MAC address of the DSG Client within the Set-top Device. This MAC address has been assigned by the manufacturer of the POD and/or Conditional Access system within the Set-top Device, and has been made known to the operator for use in configuring the DSG Agent.

6.1313	X.509 certificate [b-ITU-T J.170]: A public key certificate specification developed as part of the ITU-T Rec. X.500 standards directory.

6.1314	zapping control [b-ITU-T J.292]: The control mechanism for multicast group join and leave considering the IGMP/MLD protocol sequence is required in NG-STB compliant CPE devices.

6.1315	(ENUM) Telephone Number Mapping [b-ITU-T J.460.1]: In [b-IETF-RFC 3761], the E.164 to Uniform Resource Identifiers (URI) Dynamic Delegation Discover System (DDDS) Application (ENUM).

# 7	Abbreviations and acronyms for television and sound transmission, and integrated broadband cable networks

The following abbreviations are contained in in-force ITU-T Recommendations related to television transmission, sound transmission and integrated broadband cable networks.

1024-QAM 	1024-ary Quadrature Amplitude Modulation        [b-ITU-T J.382]

16-QAM		16-ary Quadrature Amplitude Modulation	      [b-ITU-T J.382]

256-QAM		256-ary Quadrature Amplitude Modulation	      [b-ITU-T J.382]

2D			Two Dimension							 [b-ITU-T J.900]

3D	Three-Dimensional						 [b-ITU-T J.900], [b-ITU-T J.902]

3DES	Triple DES – a block cipher formed from the Data Encryption Standard (DES) cipher 								[b-ITU-T J.366.8]

3DES 		Triple encryption with the Data Encryption Standard	[b-ITU-T J.222.3]

3DTV	Three Dimensional Television				[b-ITU-T J.195.1], [b-ITU-T J.196.1], [b-ITU-T J.381], [b-ITU-T J.900]

3G			third Generation							[b-ITU-T J.195.1]

3WC 	Three Way Calling						[b-ITU-T J.460.0], [b-ITU-T J.460.1]

4096-QAM		4096-ary Quadrature Amplitude Modulation		[b-ITU-T J.382]

4G			fourth Generation						[b-ITU-T J.195.1]

64-QAM		64-ary Quadrature Amplitude Modulation		[b-ITU-T J.382]

8-VSB		Eight Vestigial SideBand					[b-ITU-T J.142]

A/D	Analog to Digital converter					[b-ITU-T J.173], [b-ITU-T J.460.2]

A/D			Analogue to Digital Conversion				[b-ITU-T J.95]

A/V	Audio/Video							[b-ITU-T J.114], [b-ITU-T J.117], [b-ITU-T J.151], [b-ITU-T J.192], [b-ITU-T J.700]

AAA			Authentication, Authorization and Accounting		[b-ITU-T J.177]

AAC	Advanced Audio Coding					[b-ITU-T J.89], [b-ITU-T J.388], [b-ITU-T J.296], [b-ITU-T J.700]

AAC-LC		Advanced Audio Coding – Low Complexity		[b-ITU-T J.700]

AAL	ATM Adaptation Layer					[b-ITU-T J.82], [b-ITU-T J.111], [b-ITU-T J.131], [b-ITU-T J.132]

AAL5	ATM Adaptation Layer 5					[b-ITU-T J.116]

ABM			Augmented Broadcasting Metadata				[b-ITU-T J.302]

ABR			Adaptive Bit Rate						[b-ITU-T J.1005]

AC	Authorization Centre						[b-ITU-T J.1001], [b-ITU-T J.1002], [b-ITU-T J.1003], [b-ITU-T J.1004]

AC 			Alternating Current	 					[b-ITU-T J.185]

AC 	Automatic Callback						[b-ITU-T J.460.3]

AC 	Auto Callback or Alternating Current			[b-ITU-T J.460.0], [b-ITU-T J.460.1]

AC-3			Arc Consistency algorithm #3 (Dolby)			[b-ITU-T J.700]

ACA  		Accounting-Answer						[b-ITU-T J.460.3]

ACAP-X		ACAP declarative (XHTML)					[b-ITU-T J.201]

ACD			Application Control Data					[b-ITU-T J.111]

ACD			Application Communication Data				[b-ITU-T J.111]

ACF	Application Control Framework				[b-ITU-T J.206], [b-ITU-T J.205]

ACK			L2TPv3 Explicit Acknowledgement message		[b-ITU-T J.212]

ACM	Adaptive Coding and Modulation				[b-ITU-T J.382], [b-ITU-T J.381]

ACR	Absolute Category Rating					[b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342], [b-ITU-T J.343]

ACR			Adaptive Clock Recovery  					[b-ITU-T J.214]

ACR 	Anonymous Call Rejection					[b-ITU-T J.460.0], [b-ITU-T J.460.1]

ACR 		Automatic Content Recognition				[b-ITU-T J.230]

ACR 		Accounting-Request						[b-ITU-T J.460.3]

ACR-HR	Absolute Category Rating with Hidden Reference	[b-ITU-T J.246], [b-ITU-T J.249], [b-ITU-T J.341]

ACR-HR		Absolute Category Rating with Hidden Reference (see [b-ITU-T P.910])

[b-ITU-T J.247], [b-ITU-T J.342]

ACS	Auto Configuration Server					[b-ITU-T J.296], [b-ITU-T J.705]

ACS			Access Control System						[b-ITU-T J.91]

ACTT		Advanced digital Cable Transmission Technology	[b-ITU-T J.381]

Ad	Advertisement							 [b-ITU-T J.706], [b-ITU-T J.707]

ad	advertisement							[b-ITU-T J.181], [b-ITU-T J.380.1]

ADC			Analogue-to-Digital Converter				[b-ITU-T J.382]

ADI 			Asset Distribution Interface					[b-ITU-T J.98], [b-ITU-T 			J.181]

Ad-ID 		Advertisement Identifier					[b-ITU-T J.181]

ADM	Ad Management Service					[b-ITU-T J.380.2], [b-ITU-T J.380.1], [b-ITU-T J.706]

ADS	Ad Decision Service						[b-ITU-T J.380.1], [b-ITU-T J.380.2], [b-ITU-T J.706]

ADSL		Asymmetric Digital Subscriber Line			[b-ITU-T J.110]

AES	Advanced Encryption Standard				[b-ITU-T J.170], [b-ITU-T J.197], [b-ITU-T J.222.3], [b-ITU-T J.290], [b-ITU-T J.1002], [b-ITU-T J.1006], [b-ITU-T J.1010]

AF			Application Function						[b-ITU-T J.368]

AGC			Automatic Gain Controller					[b-ITU-T J.186]

AH	Authentication Header is an IPsec security protocol that provides message integrity for complete IP packets, including the IP header.			[b-ITU-T J.170]

AIFF			Audio Interchange File Format				[b-ITU-T J.296]

AIP	Application Install Package					[b-ITU-T J.205], [b-ITU-T J.206]

AIS	Alarm Indication Signal					[b-ITU-T J.132], [b-ITU-T J.214]

AIT	Application Information Table				[b-ITU-T J.205], [b-ITU-T J.206], [b-ITU-T J.700]

AK			Authorization Key						[b-ITU-T J.222.3]

AKA	Authentication and Key Agreement			[b-ITU-T J.261], [b-ITU-T J.360 Amendment 1]

ALC			Automatic Level Controller					[b-ITU-T J.186]

AL-FEC		Application Level FEC						[b-ITU-T J.702]

ALG			Application Level Gateway					[b-ITU-T J.218]

ALG 	Application Layer Gateway					[b-ITU-T J.192], [b-ITU-T J.360]

AM	Application Manager						[b-ITU-T J.360], [b-ITU-T J.363], [b-ITU-T J.365]

AM 	Application Manager (a system that interfaces to Policy Server(s) for requesting QoS‑based service on behalf of an end-user or network management system)									[b-ITU-T J.179]

AMA			Automated Message Accounting				[b-ITU-T J.164]

AMF	Action Message Format					[b-ITU-T J.181 Amendment 1]

AMF 		Audience Measurement Function				[b-ITU-T J.296]

AMID		Application Manager Identifier				[b-ITU-T J.368]

AMP/BRC-U	Amplifier and Branch Unit					[b-ITU-T J.185], [b-ITU-T J.186]

AMR	Adaptive Multi-Rate						[b-ITU-T J.388], [b-ITU-T J.296]

AM-SCM 		Amplitude Modulation Sub-Carrier Multiplexing 	[b-ITU-T J.185]

AM-VSB	Amplitude Modulation Vestigial Sideband		[b-ITU-T J.185], [b-ITU-T J.186]

AN	Access Node							[b-ITU-T J.173], [b-ITU-T J.190]

ANC			Ancillary								[b-ITU-T J.89]

ANC			Announcement Controller					[b-ITU-T J.160]

ANOVA		ANalysis Of Variance						[b-ITU-T J.144]

ANP			Announcement Player						[b-ITU-T J.160]

ANS			Announcement Server						[b-ITU-T J.160]

AOD	Approved Output Domain					[b-ITU-T J.293], [b-ITU-T J.294]

AP	Access Point							[b-ITU-T J.195.1], [b-ITU-T J.296]

AP 			Aggregation Proxy						[b-ITU-T J.367]

APD			Avalanche Photo Diode					[b-ITU-T J.185]

API	Application Programming/Program Interface 			[b-ITU-T J.111], [b-ITU-T J.117], [b-ITU-T J.120], [b-ITU-T J.162], [b-ITU-T J.200], [b-ITU-T J.201], [b-ITU-T J.206], [b-ITU-T J.207], [b-ITU-T J.215], [b-ITU-T J.230], [b-ITU-T J.280], [b-ITU-T J.287], [b-ITU-T J.290], [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.700], [b-ITU-T J.701], [b-ITU-T J.704], [b-ITU-T J.1011]

APL	Average picture level						[b-ITU-T N.62]

APM			Alternate Provisioning Mode					[b-ITU-T J.222.2]

APP			Application								[b-ITU-T J.192]

AppCatUI		Application Catalogue User Interface			[b-ITU-T J.206]

APS	Analogue Protection System					[b-ITU-T J.95], [b-ITU-T J.197]

AR	Augmented Reality						[b-ITU-T J.301], [b-ITU-T J.302]

AR 	Auto Recall							[b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.460.3]

A-RA		Receiver Amplifier for Analogue video transmission	[b-ITU-T J.186]

ARP	Address Resolution Protocol					[b-ITU-T J.111], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.218], [b-ITU-T J.222.2], [b-ITU-T J.222.3]

ARQ	Automatic Repeat Request					[b-ITU-T J.196.1], [b-ITU-T J.196.3], [b-ITU-T J.292], [b-ITU-T J.702]

AR-STV	Augmented Reality Smart Television			[b-ITU-T J.301], [b-ITU-T J.302]

AS	Application Server						[b-ITU-T J.262], [b-ITU-T J.360], [b-ITU-T J.365], [b-ITU-T J.367], [b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.460.3]

AS 			Automation System						[b-ITU-T J.287]

ASCII	American Standard Code for Information Interchange [b-ITU-T J.126], [b-ITU-T J.181], [b-ITU-T J.222.2], [b-ITU-T J.171.2]

ASD	Application-Specific Data. An application-specific field in the IPsec header that along with the destination IP address provides a unique number for each SA.

[b-ITU-T J.170]

ASD 	Authorized Service Domain					[b-ITU-T J.290], [b-ITU-T J.293], [b-ITU-T J.294], [b-ITU-T J.700]

ASI	Asynchronous Serial Interface				[b-ITU-T J.131], [b-ITU-T J.132]

ASM			Any Source Multicast						[b-ITU-T J.222.2]

ASN	Abstract Syntax Notation					[b-ITU-T J.1003], [b-ITU-T J.1004]

ASN.1	Abstract Syntax Notation 1					[b-ITU-T J.111], [b-ITU-T J.218], [b-ITU-T J.222.3], [b-ITU-T J.222.2]

ASO			Arbitrary Slice Ordering					[b-ITU-T J.361]

ASP	Application Specific Proxy					[b-ITU-T J.191], [b-ITU-T J.192]

ASP 			Audio Server Protocol						[b-ITU-T J.175]

A-TA		Transmitter Amplifier for Analogue video transmission	[b-ITU-T J.186]

A-TDMA	Advanced Time Division Multiple Access		[b-ITU-T J.214], [b-ITU-T J.222.2]

ATI			Absolute Temporal Information				[b-ITU-T J.249]

ATM	Asynchronous Transfer Mode				[b-ITU-T J.81], [b-ITU-T J.82], [b-ITU-T J.83], [b-ITU-T J.111], [b-ITU-T J.112 Annex A], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.131], [b-ITU-T J.132], [b-ITU-T J.222.2]

ATM			Automatic Teller Machine					[b-ITU-T J.261]

AU			Administrative Unit						[b-ITU-T J.132]

AUG			Administrative Unit Group					[b-ITU-T J.132]

AUID		Application Unique ID						[b-ITU-T J.367]

AV			Audio-Visual							[b-ITU-T J.190]

AV 			Audio/Video							[b-ITU-T J.388]

AV/C		Audio/Video Control						[b-ITU-T J.117]

AVC	Advanced Video Coding					[b-ITU-T J.181 Amendment 1], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.361], [b-ITU-T J.388], [b-ITU-T J.604], [b-ITU-T J.1006]

AVC			Advanced Video Codec					[b-ITU-T J.700]

AVI	Audio Video Interleave						[b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342]

AVP	   		Audio Visual Profile	   					[b-ITU-T J.171.2]

AVP 		L2TPv3 Attribute Value Pair					[b-ITU-T J.212]

AVP 	Attribute Value Pair						[b-ITU-T J.261], [b-ITU-T J.460.3]

AVP 	Attribute-Value Pair						[b-ITU-T J.367], [b-ITU-T J.360 Amendment 1],

AVP	Attribute-Value Pairs						[b-ITU-T J.800.0]

AVT			Audio/Video Transport						[b-ITU-T J.700]

AWGN	Additive White Gaussian Noise				[b-ITU-T J.195.1], [b-ITU-T J.222.1], [b-ITU-T J.382]

B2BUA 		Back-to-Back User Agent  					[b-ITU-T J.460.3]

BAQ			Basic Audio Quality						[b-ITU-T J.145]

BAT			Bouquet Association Table					[b-ITU-T J.94]

BB	BaseBand	 							[b-ITU-T J.83], [b-ITU-T J.150], [b-ITU-T J.382]

BBF			Broadband Forum						[b-ITU-T J.296]

BC	Broadcast Channel						 [b-ITU-T J.112], [b-ITU-T J.112 Annex A], [b-ITU-T J.114], [b-ITU-T J.115], [b-ITU-T J.116], [b-ITU-T J.118]

BC 	 		BroadCast network						[b-ITU-T J.702]

BCAST  		Broadcast								[b-ITU-T J.702]

BCD			Binary Coded Decimal						[b-ITU-T J.94]

BCH 		Bose-Chaudhuri-Hocquenghem				[b-ITU-T J.195.2]

BCH			Bose-Chaudhuri-Hocquenghem (code)			[b-ITU-T J.196.2]

BCH 		Bose-Chaudhuri-Hocquenghem multiple error correction binary block code													[b-ITU-T J.382]

BCID 	Billing Correlation ID						[b-ITU-T J.178], [b-ITU-T J.363], [b-ITU-T J.368]

BCID 	Billing Correlation ID (defined in the IPCablecom Event Messaging Recommendation)								[b-ITU-T J.179]

BCP			Best Current Practice						[b-ITU-T J.222.3]

BCT  		Blind Call Transfer						[b-ITU-T J.460.0]

BE			Best Effort								[b-ITU-T J.292]

BED	Best Effort Domain						[b-ITU-T J.290], [b-ITU-T J.293], [b-ITU-T J.294]

BER	Bit Error Rate or	Bit Error Ratio				[b-ITU-T J.83], [b-ITU-T J.131], [b-ITU-T J.132], [b-ITU-T J.142], [b-ITU-T J.145], [b-ITU-T J.185], [b-ITU-T J.195.1], [b-ITU-T J.222.1], [b-ITU-T J.241], [b-ITU-T J.287], [b-ITU-T J.296]

BER	Basic Encoding Rules						[b-ITU-T J.111], [b-ITU-T J.1003], [b-ITU-T J.1004]

BGCF		Breakout Gateway Control Function			[b-ITU-T J.360]

BGP			Border Gateway Protocol					[b-ITU-T J.292]

BIM	Broadcast Interface Module					[b-ITU-T J.112 Annex A], [b-ITU-T J.115], [b-ITU-T J.116], [b-ITU-T J.118]

BISS	Basic Interoperable Scrambling System			[b-ITU-T J.96]

B-ISDN		Broadband Integrated Services Digital Network	[b-ITU-T J.82]

bit/s			Bits per second							[b-ITU-T J.91]

BLT			Bit-block Transfer						[b-ITU-T J.700]

BLV	Busy Line Verification						[b-ITU-T J.178], [b-ITU-T J.460.0], [b-ITU-T J.460.1]

BML	Broadcast Markup Language					[b-ITU-T J.296], [b-ITU-T J.200], [b-ITU-T J.201]

Bp			Maximum Buffer Capacity					[b-ITU-T J.88]

BP 	Boundary Point							[b-ITU-T J.190], [b-ITU-T J.192]

BP 			(IPCable2Home) Boundary Point				[b-ITU-T J.290]

BP 			Baseline Profile							[b-ITU-T J.388]

BPDU	Bridge Protocol Data Unit					[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]

BPI	Baseline Privacy Interface					[b-ITU-T J.213], [b-ITU-T J.222.2]

BPI+	Baseline Privacy Interface Plus				[b-ITU-T J.125], [b-ITU-T J.222.2], [b-ITU-T J.222.3]

BPI+	Baseline Privacy Interface Plus is the security portion of ITU-T Rec. J.112 that runs on the MAC layer.					[b-ITU-T J.170]

BPKM	Baseline Privacy Key Management				[b-ITU-T J.112 Annex C], [b-ITU-T J.125], [b-ITU-T J.222.2], [b-ITU-T J.222.3]

BPL 			Broadcast over Power Line					[b-ITU-T J.700]

B-PON		Broadband Passive Optical Network 			[b-ITU-T J.185]

BPP			Basic POTS Provisioning					[b-ITU-T J.177]

bps			Bits per second							[b-ITU-T J.83]

BPSC 		Bulk Portal Services Configuration				[b-ITU-T J.192]

BPSK		Binary Phase Shift Keying					[b-ITU-T J.142]

BRA	Basic Rate Access						[b-ITU-T J.112], [b-ITU-T J.112 Annex A], [b-ITU-T J.116]

BS			Broadcasting Satellite						[b-ITU-T J.296]

BS 	Broadcast Satellite television					[b-ITU-T J.297], [b-ITU-T J.1005]

BSC	 		Base Station Controller						[b-ITU-T J.115]

BSF	Bootstrapping Server Function				[b-ITU-T J.360 Amendment 1], [b-ITU-T J.367]

bslbf	bit string, left bit first						[b-ITU-T J.96], [b-ITU-T J.183]

bslbf			Bit string, left bit first, where left is the order in which bit strings are written.												[b-ITU-T J.181]

BSoD	Business Services over DOCSIS				[b-ITU-T J.126], [b-ITU-T J.213], [b-ITU-T J.800.2]

BSS	Business Support System(s)					[b-ITU-T J.363], [b-ITU-T J.700], [b-ITU-T J.705]

BTS			Base Transceiver Station					[b-ITU-T J.115]

BTS 			Base Transceiver Station. A BTS could contain multiple BTS modems

[b-ITU-T J.116]

BUFP		Buffer Pointer							[b-ITU-T J.88]

BW	BandWidth								[b-ITU-T J.84], [b-ITU-T J.116], [b-ITU-T J.142]

BWA	Broadband Wireless Access					[b-ITU-T J.116]

BWE			Bandwidth Efficiency						[b-ITU-T J.141]

BWMF		Bandwidth Management Function				[b-ITU-T J.190]

C/N or CNR	Carrier-to-Noise Ratio						[b-ITU-T J.83], [b-ITU-T J.112 Annex B], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.141], [b-ITU-T J.142], [b-ITU-T J.150], [b-ITU-T J.222.1]

CA	Conditional Access						[b-ITU-T J.96], [b-ITU-T J.98], [b-ITU-T J.128], [b-ITU-T J.181], [b-ITU-T J.287], [b-ITU-T J.290], [b-ITU-T J.292], [b-ITU-T J.293], [b-ITU-T J.294], [b-ITU-T J.296], [b-ITU-T J.700], [b-ITU-T J.705], [b-ITU-T J.1001], [b-ITU-T J.1010], [b-ITU-T J.1011], [b-ITU-T J.1020]

CA 			Customer Address (for conditional access)		[b-ITU-T J.91]

CA 			Call Agent								[b-ITU-T J.178]

CA 	Call Agent. The part of the CMS that maintains the communication state, and controls the line side of the communication.					[b-ITU-T J.170]

CA 	Certification/Certificate Authority				[b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.3], [b-ITU-T J.366.8], [b-ITU-T J.800.2]

CA 	Certification Authority. A trusted organization that accepts certificate applications from entities, authenticates applications, issues certificates and maintains status information about certificates.				[b-ITU-T J.170]

CA/DRM 		Conditional Access/Digital Rights Management	[b-ITU-T J.1010]

CABAC		Context-based Adaptive Binary Arithmetic Coding	[b-ITU-T J.361]

CACS	Conditional Access Client Software			[b-ITU-T J.1001], [b-ITU-T J.1002], [b-ITU-T J.1003], [b-ITU-T J.1004]

CAD			Conditional Access Device					[b-ITU-T J.91]

CAM	Conditional Access Module					J.[b-ITU-T J.1001], [b-ITU-T J.1002], [b-ITU-T J.1003], [b-ITU-T J.1004]

CAP			IPNet2Home Address Portal					[b-ITU-T J.190]

CAP 		Cable Address Portal						[b-ITU-T J.191]

CAP 		IPCable2Home Address Portal				[b-ITU-T J.192]

CAS	Conditional Access System					[b-ITU-T J.282], [b-ITU-T J.290], [b-ITU-T J.292], [b-ITU-T J.293], [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.700], [b-ITU-T J.1001], [b-ITU-T J.1005],

CAS   		Central Authentication Service				[b-ITU-T J.705]

CAS/DRM  	Conditional Access Solution/Digital Rights Management	     [b-ITU-T J.703]

CASS	CAM Authentication Sub-System				[b-ITU-T J.1002], [b-ITU-T J.1003], [b-ITU-T J.1004]

CAT	Conditional Access Table					[b-ITU-T J.94], [b-ITU-T J.96], [b-ITU-T J.200]

CAT 		IPNet2Home Address Translation				[b-ITU-T J.190]

CAT 		Cable Address Translation					[b-ITU-T J.191]

CAT 		IPCable2Home Address Translation			[b-ITU-T J.192]

CATV 	Community Antenna Television				[b-ITU-T J.111], [b-ITU-T J.181], [b-ITU-T J.296]

CATV	Community Antenna TeleVision (System) 		[b-ITU-T J.83], [b-ITU-T J.116]

CATV 	Community Access Television, Cable Television	[b-ITU-T J.126]

CATV 	Cable Television							[b-ITU-T J.112 Annex A], [b-ITU-T J.116], [b-ITU-T J.142], [b-ITU-T J.150], [b-ITU-T J.185], [b-ITU-T J.230], [b-ITU-T J.281], [b-ITU-T J.282]

CATV 		Cable TV distribution system					[b-ITU-T J.118]

CAVLC		Context-Based Adaptive Variable Length Coding	[b-ITU-T J.361]

CBC	Cipher Block Chaining						[b-ITU-T J.112 Annex A], [b-ITU-T J.125], [b-ITU-T J.181], [b-ITU-T J.222.3], [b-ITU-T J.290], [b-ITU-T J.366.8], [b-ITU-T J.1006]

CBC 	Cipher-block Chaining mode is an option in block ciphers that combine (XOR) the previous block of ciphertext with the current block of plaintext before encrypting that block of the message.			[b-ITU-T J.170]

CBI  		Common Billing Interface					[b-ITU-T J.703]

CBR	Constant Bit Rate						[b-ITU-T J.82], [b-ITU-T J.98], [b-ITU-T J.181 Amendment 1], [b-ITU-T J.214], [b-ITU-T J.222.2]

CBW			Capture Bandwidth						[b-ITU-T J.222.1]

CCCIEK		Common CAM Client Image Encryption Key		[b-ITU-T J.1003]

CCCM		Customer premises equipment-Controlled Cable Modem	[b-ITU-T J.122], [b-			ITU-T J.222.2]

CCF			Continuous Concatenation and Fragmentation		[b-ITU-T J.222.2]

CCF 		Charging Collection Function				[b-ITU-T J.363]

CCI	Copy Control Information					[b-ITU-T J.95], [b-ITU-T J.197]

CCI 			Control and Classifier Interface				[b-ITU-T J.223.2]

CCI 			CAM Client Image						[b-ITU-T J.1003]

CCM			Constant Coding and Modulation				[b-ITU-T J.382]

CCM			CPE Controlled Cable Modem				[b-ITU-T J.112 Annex B]

CCP	Channel Charge Protocol					[b-ITU-T J.1101], [b-ITU-T J.1102], [b-ITU-T J.1103]

CCS 		One Hundred Call Seconds					[b-ITU-T J.460.2]

CCT  		Consultative Call Transfer					[b-ITU-T J.460.0]

CD 			Controller Device (for conditional access)		[b-ITU-T J.91]

Cd 	 		Control down							[b-ITU-T J.196.2]

Cd 			downlink Control							[b-ITU-T J.196.3]

CDATA	Character Data							[b-ITU-T J.380.7], [b-ITU-T J.380.8]

CDC			Cable DHCP Client						[b-ITU-T J.191]

CDC 		IPCable2Home DHCP Client					[b-ITU-T J.192]

CDCS		CA/DRM Client Software					[b-ITU-T J.1020]

CDF	Charging Data Function					[b-ITU-T J.367], [b-ITU-T J.363], [b-ITU-T J.360], [b-ITU-T J.460.3]

CDM			Code Division Multiplex 					[b-ITU-T J.118]

CDMM		C-DOCSIS Management Message				[b-ITU-T J.223.2]

CDN	Content Delivery Network					[b-ITU-T J.181 Amendment 1], [b-ITU-T J.282], [b-ITU-T J.292], [b-ITU-T J.296] [b-ITU-T J.297], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.1005], [b-ITU-T J.1006]

CDN 		L2TPv3 Call-Disconnect-Notify message 		[b-ITU-T J.212]

C-DOCSIS	Cabinet DOCSIS							[b-ITU-T J.223.2], [b-ITU-T J.223.1]

CDP			IPNet2Home DHCP Portal					[b-ITU-T J.190]

CDP 		Cable DHCP Portal						[b-ITU-T J.191]

CDP 		IPCable2Home DHCP Portal					[b-ITU-T J.192]

CDR	Call Detail Record						[b-ITU-T J.164], [b-ITU-T J.363], [b-ITU-T J.360]

CDRP		Call Data Rating Point						[b-ITU-T J.703]

CDS 		IPCable2Home DHCP Server				[b-ITU-T J.192]

CDT			Carrier Definition Table					[b-ITU-T J.94]

CDT 		C-DOCSIS Data Tag						[b-ITU-T J.223.2]

CDV			Cell Delay Variation						[b-ITU-T J.82]

cDVR		client Digital Video Recorder				[b-ITU-T J.700]

CE	Consumer Electronics						[b-ITU-T J.290], [b-ITU-T J.1010]

CEA			Consumer Electronics Association				[b-ITU-T J.151]

CEC	Consumer Electronics Control				[b-ITU-T J.230], [b-ITU-T J.296]

CENC	Common Encryption						[b-ITU-T J.297], [b-ITU-T J.1011]

CENC  		Common Encryption scheme					[b-ITU-T J.1005]

CENC  		Common Encryption in ISO Base Media File Format	 [b-ITU-T J.1006]

CF			Colour Frame							[b-ITU-T J.88]

CF 	Call Forwarding							[b-ITU-T J.460.1], [b-ITU-T J.460.3]

CFB			Cipher Feedback							[b-ITU-T J.222.3]

CFBL	Call Forwarding Busy Line					[b-ITU-T J.460.0], [b-ITU-T J.460.1]

CFDA 	Call Forwarding Don't Answer				[b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.460.3]

CFNA		Call Forwarding No Answer					[b-ITU-T J.178]

CFP			Call for Proposal							[b-ITU-T J.95]

CFP 			Call Feature Provisioning					[b-ITU-T J.177]

CFV 	Call Forwarding Variable					[b-ITU-T J.460.3], [b-ITU-T J.460.1], [b-ITU-T J.460.0]

CGF	Charging Gateway Function					[b-ITU-T J.367], [b-ITU-T J.363]

CGMS-A	Copy Generation Management System Analogue	[b-ITU-T J.197], [b-ITU-T J.296]

CH			IPCable2Home Host						[b-ITU-T J.192]

CHAP		Challenge Handshake Authentication Protocol		[b-ITU-T J.111]

CHK			Common Hash Key						[b-ITU-T J.1003]

CI			Command Identifier (for conditional access)		[b-ITU-T J.91]

CI 	Common Interface						[b-ITU-T J.114], [b-ITU-T J.1011]

CID			Circuit ID								[b-ITU-T J.177]

CID  	Caller Identity Delivery					[b-ITU-T J.460.0], [b-ITU-T J.460.1]

CID 			Caller Identity Display						[b-ITU-T J.700]

CIDB 	Caller Identity Delivery Blocking				[b-ITU-T J.460.1], [b-ITU-T J.460.0]

CIDS 	Caller Identity Delivery (and) Suppression		[b-ITU-T J.460.0], [b-ITU-T J.460.1]

CIF	Common Intermediate Format				[b-ITU-T J.161], [b-ITU-T J.361]

CIF	Common Intermediate Format (352 x 288 pixels)	[b-ITU-T J.246], [b-ITU-T J.247]

CIN			Composite Intermodulation Noise				[b-ITU-T J.87]

CIN 	Converged Interconnect Network				[b-ITU-T J.212], [b-ITU-T J.222.2]

CIP			Common Isochronous Packet					[b-ITU-T J.117]

CIS	Content Information Service					[b-ITU-T J.380.1], [b-ITU-T J.380.2], [b-ITU-T J.706]

CIT			Constrained Image Trigger					[b-ITU-T J.197]

CIW			Container identification word				[b-ITU-T J.81]

CL			CableLabs								[b-ITU-T J.222.1]

CLASS		Custom Local Area Signalling Services			[b-ITU-T J.178]

CLI	Command Line Interface					[b-ITU-T J.212], [b-ITU-T J.223.2]

CLP			Cell Loss Priority						[b-ITU-T J.82]

CLUT	Colour Look-Up Table						[b-ITU-T J.117], [b-ITU-T J.151]

CM			Copy Mark								[b-ITU-T J.95]

CM	Cable Modem							[b-ITU-T J.112 Annex A], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.125], [b-ITU-T J.126], [b-ITU-T J.128], [b-ITU-T J.160], [b-ITU-T J.163], [b-ITU-T J.164], [b-ITU-T J.166], [b-ITU-T J.167], [b-ITU-T J.170], [b-ITU-T J.173], [b-ITU-T J.179], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.211], [b-ITU-T J.212], [b-ITU-T J.214], [b-ITU-T J.218], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.223.1], [b-ITU-T J.223.2], [b-ITU-T J.260], [b-ITU-T J.261], [b-ITU-T J.262], [b-ITU-T J.263], [b-ITU-T J.293], [b-ITU-T J.294], [b-ITU-T J.296], [b-ITU-T J.360], [b-ITU-T J.363], [b-ITU-T J.460.2], [b-ITU-T J.700], [b-ITU-T J.704], [b-ITU-T J.800.2], [b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1104], [b-ITU-T J.1105], [b-ITU-T J.1106], [b-ITU-T J.1107]

CM	Cable Modem, IIM, MH					[b-ITU-T J.112], [b-ITU-T J.116]

CM 	DOCSIS Cable Modem – a DOCSIS-compliant devise which provides data transport connectivity from RFI to IP networks		[b-ITU-T J.161], [b-ITU-T J.177], [b-ITU-T J.368], [b-ITU-T J.370], [b-ITU-T J.460.4]

CMC			Cable Media Converter						[b-ITU-T J.223.1]

CMC			Coax Media Converter						[b-ITU-T J.223.2]

CMCI		Cable Modem to Customer Premises Equipment(CPE) Interface

[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.126], [b-ITU-T J.210], [b-ITU-T J.212], [b-ITU-T J.222.2], [b-ITU-T J.222.3]

CMIM	Cable Modem(CM) Interface Mask			[b-ITU-T J.213], [b-ITU-T J.222.2]

CMP			Connection Management Procedures			[b-ITU-T J.117]

CMP 		IPNet2Home Management Portal				[b-ITU-T J.190]

CMP 		Cable Management Portal					[b-ITU-T J.191]

CMP 		IPCable2Home Management Portal			[b-ITU-T J.192]

CMS	Call Management Server					[b-ITU-T J.160], [b-ITU-T J.161], [b-ITU-T J.164], [b-ITU-T J.167], [b-ITU-T J.172], [b-ITU-T J.173], [b-ITU-T J.175], [b-ITU-T J.177], [b-ITU-T J.179], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.260], [b-ITU-T J.261], [b-ITU-T J.360], [b-ITU-T J.362], [b-ITU-T J.363], [b-ITU-T J.460.2], [b-ITU-T J.800.2]

CMS 	Call Management Server. Controls the audio connections. Also called a Call Agent in MGCP/SGCP terminology. This is one example of an Application Server.									[b-ITU-T J.170]

CMS 	Cryptographic Message Syntax				[b-ITU-T J.170], [b-ITU-T J.177]

CM-SG		Cable Modem Service Group					[b-ITU-T J.222.2]

CMTS	Cable Modem Termination System				[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.125], [b-ITU-T J.126], [b-ITU-T J.128], [b-ITU-T J.161], [b-ITU-T J.163], [b-ITU-T J.170], [b-ITU-T J.172], [b-ITU-T J.175], [b-ITU-T J.177], [b-ITU-T J.178], [b-ITU-T J.179], [b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.210], [b-ITU-T J.211], [b-ITU-T J.212], [b-ITU-T J.214], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.223.1], [b-ITU-T J.223.2], [b-ITU-T J.261], [b-ITU-T J.263], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.293], [b-ITU-T J.294], [b-ITU-T J.360], [b-ITU-T J.362], [b-ITU-T J.363], [b-ITU-T J.365], [b-ITU-T J.368], [b-ITU-T J.381], [b-ITU-T J.460.2], [b-ITU-T J.700], [b-ITU-T J.704], [b-ITU-T J.800.0], [b-ITU-T J.800.2], [b-ITU-T J.1101], [b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1104], [b-ITU-T J.1105], [b-ITU-T J.1106], [b-ITU-T J.1107]

CMTS-NSI	Cable Modem Termination System-Network Side Interface 												[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.222.3]

C-n			Container-n							[b-ITU-T J.132]

CNAM	Calling NAMe							[b-ITU-T J.178], [b-ITU-T J.360 Amendment 1], [b-ITU-T J.460.0], [b-ITU-T J.460.1]

CNAME		Canonical Name							[b-ITU-T J.170]

C-NAPT		Cable Network Address and Portal Translation		[b-ITU-T J.191]

C-NAPT 		IPCable2Home Network Address and Port Translation	[b-ITU-T J.192]

C-NAT		Cable Network Address Translation			[b-ITU-T J.191]

C-NAT 		IPCable2Home Network Address Translation		[b-ITU-T J.192]

CND	Calling Number Delivery					[b-ITU-T J.460.0], [b-ITU-T J.460.1]

CNG			Comfort Noise Generation					[b-ITU-T J.361]

CNP			IPNet2Home Naming Portal					[b-ITU-T J.190]

CNP 		Cable Naming Portal						[b-ITU-T J.191]

CNP 		IPCable2Home Name Portal					[b-ITU-T J.192]

CNR	Carrier-to-Noise Ratio						[b-ITU-T J.112], [b-ITU-T J.382], [b-ITU-T J.186], [b-ITU-T J.185]

Codec	Coder-DECoder	or Coder/Decoder			[b-ITU-T J.161], [b-ITU-T J.177], [b-ITU-T J.290], [b-ITU-T J.361],

CODEC	Coder-DECoder							[b-ITU-T J.244], [b-ITU-T J.343], [b-ITU-T J.343.1], [b-ITU-T J.343.2], [b-ITU-T J.343.3], [b-ITU-T J.343.4], [b-ITU-T J.343.5], [b-ITU-T J.343.6], [b-ITU-T J.700], [b-ITU-T J.702]

CODEC 	COding-DECoding algorithms used to compress/decompress the data representing the Voice (or Video) media traffic	 			[b-ITU-T J.369]

COFDM		Coded Orthogonal Frequency Division Multiplex	[b-ITU-T J.142]

Connection Id	Connection Identifier						[b-ITU-T J.112 Annex A]

COPS	Common Open Policy Service				[b-ITU-T J.163], [b-ITU-T J.170], [b-ITU-T J.362]

COPS	Common Open Policy Service (b-IETF RFC 2748)		[b-ITU-T J.179], [b-ITU-T J.263]

CORBA  		Common Object Request Broker Architecture		[b-ITU-T J.703]

CoS	Class of Service							[b-ITU-T J.223.2], [b-ITU-T J.292]

COT 	Customer Originated Trace					[b-ITU-T J.460.0], [b-ITU-T J.460.1]

CP	Cyclic Prefix							[b-ITU-T J.196.3], [b-ITU-T J.195.2], [b-ITU-T J.196.2]

CP 	Content Provider							[b-ITU-T J.297], [b-ITU-T J.1005]

CP 	Content Protection						[b-ITU-T J.702], [b-ITU-T J.1006]

CPAC		Copy Protection Advisory Committee			[b-ITU-T J.95]

CPB			Coded Picture Buffer						[b-ITU-T J.286]

CPCM 		Content Protection and Copy Management		[b-ITU-T J.1005]

CPD			Control Point Discovery					[b-ITU-T J.362]

CPE	Customer Premise/Premises Equipment 				 [b-ITU-T J.97], [b-ITU-T J.98], [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.128], [b-ITU-T J.160], [b-ITU-T J.162], [b-ITU-T J.163], [b-ITU-T J.164], [b-ITU-T J.167], [b-ITU-T J.173], [b-ITU-T J.204], [b-ITU-T J.212], [b-ITU-T J.218], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.223.1], [b-ITU-T J.223.2], [b-ITU-T J.241], [b-ITU-T J.282], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.292], [b-ITU-T J.293], [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.360], [b-ITU-T J.361], [b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.460.2], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.703], [b-ITU-T J.705], [b-ITU-T J.800.2], [b-ITU-T J.1003], [b-ITU-T J.1004], [b-ITU-T J.1010], [b-ITU-T J.1011], [b-ITU-T J.1020]

CPS	Common Part Sublayer						[b-ITU-T J.195.1], [b-ITU-T J.195.3], [b-ITU-T J.196.1], [b-ITU-T J.196.3]

CPT			IPNet2Home Address Passthrough				[b-ITU-T J.190]

CPTWG		Copy Protection Technical Working Group		[b-ITU-T J.117]

CPU	Central Processing Unit					[b-ITU-T J.111], [b-ITU-T J.117], [b-ITU-T J.296], [b-ITU-T J.603], [b-ITU-T J.151], [b-ITU-T J.192], [b-ITU-T J.295]

CQoS		IPNet2Home Quality of Service				[b-ITU-T J.190]

CQoS 		Cable Quality of Service					[b-ITU-T J.191]

CQoS 		IPCable2Home Quality of Service				[b-ITU-T J.192]

CQP			IPNet2Home QoS Portal					[b-ITU-T J.190]

CQP 		Cable QoS Portal							[b-ITU-T J.191]

CQP 		IPCable2Home QoS Portal					[b-ITU-T J.192]

CR			Control Relationship						[b-ITU-T J.362]

CRC	Cyclic Redundancy Check					[b-ITU-T J.82], [b-ITU-T J.112 Annex A], [b-ITU-T J.116], [b-ITU-T J.131], [b-ITU-T J.132], [b-ITU-T J.181], [b-ITU-T J.183], [b-ITU-T J.195.2], [b-ITU-T J.195.3], [b-ITU-T J.196.3], [b-ITU-T J.200], [b-ITU-T J.211], [b-ITU-T J.212], [b-ITU-T J.213], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.287], [b-ITU-T J.382], [b-ITU-T J.1102], [b-ITU-T J.1103],

CRC-16		CRC of length 16							[b-ITU-T J.212]

CRG			IPCable2Home Residential Gateway			[b-ITU-T J.192]

CRL	Certificate Revocation List					[b-ITU-T J.170], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.3], [b-ITU-T J.366.8]

CRS	 		CACS remote by Renewable Security System		[b-ITU-T J.1001]

CRT			Chinese Remainder Theorem					[b-ITU-T J.222.3]

CS	Convergence Sublayer						[b-ITU-T J.82], [b-ITU-T J.131], [b-ITU-T J.132], [b-ITU-T J.196.3], [b-ITU-T J.195.1], [b-ITU-T J.195.3], [b-ITU-T J.196.1]

CS 			Communications Satellite					[b-ITU-T J.296]

CSA	(DVB) Common Scrambling Algorithm			[b-ITU-T J.96], [b-ITU-T J.290], [b-ITU-T J.1010],

CSCF	Call Session Control Function				[b-ITU-T J.263], [b-ITU-T J.360], [b-ITU-T J.363], [b-ITU-T J.365], [b-ITU-T J.367], [b-ITU-T J.368], [b-ITU-T J.388], [b-ITU-T J.460.0], [b-ITU-T J.460.2], [b-ITU-T J.460.3], [b-ITU-T J.700]

CSI			Convergence Sublayer Indication				[b-ITU-T J.82]

CSMA		Carrier Sense Multiple Access				[b-ITU-T J.212]

CSO	Composite Second Order beat				[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.142], [b-ITU-T J.222.1]

CSO 	Composite Second Order distortion			[b-ITU-T J.185], [b-ITU-T J.186]

CSP	Configurable Security Processor				[b-ITU-T J.290], [b-ITU-T J.293]

CSP 			IPNet2Home Security Portal					[b-ITU-T J.190]

CSP 			Cable Security Portal						[b-ITU-T J.191]

CSP			IPCable2Home Security Portal				[b-ITU-T J.192]

CSR			Control Status Register						[b-ITU-T J.117]

CSR 		Customer Service Record					[b-ITU-T J.170]

CSR 		Customer Service Representative				[b-ITU-T J.177]

CSRC		Contributing Source						[b-ITU-T J.361]

CSS			Contents Scramble System					[b-ITU-T J.95]

CSS 	Cascading Style Sheet						[b-ITU-T J.201], [b-ITU-T J.296]

CSU			Channel Service Unit						[b-ITU-T J.214]

CT 	Call Transfer							[b-ITU-T J.460.0], [b-ITU-T J.460.1]

CTA			Cordless Terminal Adapter					[b-ITU-T J.114]

CTB	Composite Triple Beat						[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.142], [b-ITU-T J.222.1]

CTB 	Composite Triple Beat distortion				[b-ITU-T J.185], [b-ITU-T J.186]

CTL			Certification Testing Laboratory				[b-ITU-T J.192]

CTP			IPNet2Home Testing Portal					[b-ITU-T J.190]

CTP 			Cable Testing Portal						[b-ITU-T J.191]

CTP 			IPCable2Home Test Portal					[b-ITU-T J.192]

CTR			Counter: the counter mode of a block cipher		[b-ITU-T J.222.3]

CTR  		Counter								[b-ITU-T J.1006]

CTS			Clear To Send							[b-ITU-T J.118]

CVC	Code Verification Certificate					[b-ITU-T J.126], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.800.2]

CVCT	Cable Virtual Channel Table					[b-ITU-T J.117], [b-ITU-T J.151]

CVS	Code Verification Signature					[b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.3]

CVT  	Code Version Table						[b-ITU-T J.128], [b-ITU-T J.705]

CW	Control Word							[b-ITU-T J.81], [b-ITU-T J.287], [b-ITU-T J.290], [b-ITU-T J.1001], [b-ITU-T J.1002]

CW 			Control Word (for conditional access)			[b-ITU-T J.91]

CW 	Continuous Wave						[b-ITU-T J.96], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.210], [b-ITU-T J.142]

CWEK	Control Words Encryption Key				[b-ITU-T J.1002], [b-ITU-T J.1003]

CWMP		CPE WAN Management Protocol				[b-ITU-T J.296]

CWT 		Call Waiting							[b-ITU-T J.460.1]

CxP			Cable PS Sub-function						[b-ITU-T J.191]

CxP 			IPCable2Home Portal Services Sub-function		[b-ITU-T J.192]

D/A			Digital to Analogue Conversion				[b-ITU-T J.95]

D/A 			Digital-to-Analogue						[b-ITU-T J.151]

D/A 			Digital to Analog converter					[b-ITU-T J.460.2]

D/U			Desired-to-Undesired signal ratio				[b-ITU-T J.185]

D/U 			Desired-to-Undesired distortion ratio			[b-ITU-T J.186]

DA	Destination Address						[b-ITU-T J.112], [b-ITU-T J.116], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.1102], [b-ITU-T J.1103]

DAC	 		Distributed Authorization Centre				[b-ITU-T J.1004]

DAD	Duplicate Address Detection					[b-ITU-T J.218], [b-ITU-T J.222.2]

DAE			Declarative Application Environment			[b-ITU-T J.296]

DAM			DECT Authentication Module				[b-ITU-T J.114]

DASH	Dynamic Adaptive Streaming over HTTP		[b-ITU-T J.181 Amendment 1], [b-ITU-T J.1005], [b-ITU-T J.1006]

DAV	Distributed Authoring and Versioning			[b-ITU-T J.205], [b-ITU-T J.206]

dB	decibel								[b-ITU-T J.142], [b-ITU-T J.287], [b-ITU-T J.382]

DB 			Database								[b-ITU-T J.292]

DBA	Dynamic Bandwidth Allocation				[b-ITU-T J.195.1], [b-ITU-T J.196.1]

dBc	Decibels relative to carrier power				[b-ITU-T J.210], [b-ITU-T J.222.1]

dBc			Decibels below a Carrier used as reference		[b-ITU-T J.142]

DBC 		Dynamic Bonding Change					[b-ITU-T J.222.2]

DBC-REQ		Dynamic Bonding Change Request MAC Message	[b-ITU-T J.222.1]

DBG			Downstream Bonding Group					[b-ITU-T J.222.2]

dBm			Decibels referred to a 1 mW power				[b-ITU-T J.142]

dBmV		Decibels referred to a 1 milliVolt rms signal level	[b-ITU-T J.142]

dBµV		Decibels referred to a 1 microVolt rms signal level	[b-ITU-T J.142]

DBS			Direct Broadcast Satellite					[b-ITU-T J.117]

DC	Direct Current 							[b-ITU-T J.185], [b-ITU-T J.460.0]

DC bis	Downstream Channel						[b-ITU-T J.222.2], [b-ITU-T J.1102], [b-ITU-T J.1103]

DCAS	Downloadable Conditional Access System		[b-ITU-T J.293],	[b-ITU-T J.702]

DCC	Dynamic Channel Change					[b-ITU-T J.122], [b-ITU-T J.128], [b-ITU-T J.222.2]

DCD	Downstream Channel Descriptor				[b-ITU-T J.128]

DCE	Data Communication Equipment				[b-ITU-T J.112], [b-ITU-T J.112 Annex A], [b-ITU-T J.116], [b-ITU-T J.118]

DCE 		Data Circuit-terminating Equipment			[b-ITU-T J.115]

DCF			Dispersion Compensation Fibre				[b-ITU-T J.186]

DCI			Device Class Identifier						[b-ITU-T J.222.2]

DCID	Downstream Channel Identifier				[b-ITU-T J.222.2], [b-ITU-T J.1102], [b-ITU-T J.1103]

DCS	Distributed Call Signalling					[b-ITU-T J.163], [b-ITU-T J.178]

DCS 		Downstream Channel Set					[b-ITU-T J.222.2]

DCS 		Digital Compression System					[b-ITU-T J.287]

DCT	Discrete Cosine Transform					[b-ITU-T J.92], [b-ITU-T J.603]

Dd	downlink Data							[b-ITU-T J.195.2], [b-ITU-T J.195.3], [b-ITU-T J.196.2], [b-ITU-T J.196.3]

dDVR		distributed Digital Video Recorder				[b-ITU-T J.700]

DDC			Data Download Control 					[b-ITU-T J.111]

DEC	Decoder 								[b-ITU-T J.147], [b-ITU-T J.240]

DECE	Digital Entertainment Content Ecosystem		[b-ITU-T J.1005], [b-ITU-T J.1010]

DECT		Digital Enhanced Cordless Telecommunications	[b-ITU-T J.114]

DEG			DEGraded								[b-ITU-T J.132]

DEMUX		De-Multiplexer							[b-ITU-T J.382]

DEMUX 		Demultiplex							[b-ITU-T J.702]

DEPI	Downstream External (-) PHY Interface			[b-ITU-T J.210], [b-ITU-T J.211], [b-ITU-T J.212], [b-ITU-T J.222.2], [b-ITU-T J.800.2], [b-ITU-T J.1101], [b-ITU-T J.1102], [b-ITU-T J.1103]

DEPI 	DOCSIS External PHY Interface				[b-ITU-T J.214], [b-ITU-T J.700]

DER	Distinguished Encoding Rules				[b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.800.2]

DES	Data Encryption Standard					[b-ITU-T J.96], [b-ITU-T J.112 Annex A], [b-ITU-T J.170], [b-ITU-T J.181], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.287], [b-ITU-T J.290]

DES 		US Data Encryption Standard				[b-ITU-T J.125]

DF	Delivery Function						[b-ITU-T J.170], [b-ITU-T J.362]

D-H	Diffie-Hellman							[b-ITU-T J.112 Annex A]

DH	Diffie-Hellman							[b-ITU-T J.170], [b-ITU-T J.366.8]

DHCP	Dynamic Host Configuration Protocol [b-IETF RFC 2131]

[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.126], [b-ITU-T J.160], [b-ITU-T J.167], [b-ITU-T J.170], [b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.223.1], [b-ITU-T J.223.2], [b-ITU-T J.282], [b-ITU-T J.290], [b-ITU-T J.292], [b-ITU-T J.295], [b-ITU-T J.296],  [b-ITU-T J.360], [b-ITU-T J.369],[b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.703], [b-ITU-T J.705], [b-ITU-T J.800.0], [b-ITU-T J.800.1]

DHCPv4		IPv4 version of the Dynamic Host Configuration Protocol	   [b-ITU-T J.222.2]

DHCPv6		Version of DHCP for IPv6					[b-ITU-T J.222.3]

DHCPv6 		IPv6 version of the Dynamic Host Configuration Protocol	   [b-ITU-T J.222.2]

DHSG		Data Hiding Sub Group					[b-ITU-T J.95]

DI	Discrete Interference						[b-ITU-T J.185], [b-ITU-T J.186]

DiffServ		Differentiated Services Architecture for Network Traffic

[b-ITU-T J.290], [b-ITU-T J.292]

DII			Dynamic Invocation Interface				[b-ITU-T J.380.7]

DIME		Downstream IP Multicast Encryption			[b-ITU-T J.213]

DIX	Digital Intel Xerox						[b-ITU-T J.126], [b-ITU-T J.222.2]

DL	Data Link								[b-ITU-T J.112 Annex A], [b-ITU-T J.116]

DLNA	Digital Living Network Alliance				[b-ITU-T J.290], [b-ITU-T J.294], [b-ITU-T J.296], [b-ITU-T J.700]

DM	Device Management						[b-ITU-T J.367], [b-ITU-T J.700], [b-ITU-T J.705]

DM 	Downloadable Mobile multi-CA/DRM			[b-ITU-T J.1020]

DMAC		Destination MAC						[b-ITU-T J.213]

DMAC 		Destination Media Access Control address		[b-ITU-T J.222.2]

DMC			Digital Media Controller					[b-ITU-T J.296]

DMC 		Device Management Client					[b-ITU-T J.367]

DMIPS		Dhrystone Million Instructions Per Second		[b-ITU-T J.296]

DMOS	Difference Mean Opinion Score				[b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342], [b-ITU-T J.343.3], [b-ITU-T J.343.4], [b-ITU-T J.343.5], [b-ITU-T J.343.6]

DMP	Digital Media Player						[b-ITU-T J.296], [b-ITU-T J.700]

DMPI	DOCSIS MAC-PHY Interface				[b-ITU-T J.222.2], [b-ITU-T J.1102], [b-ITU-T J.1103]

DMR	Digital Media Renderer					[b-ITU-T J.296], [b-ITU-T J.700]

DMS			Digital Media Server						[b-ITU-T J.296]

DMS 		Device Management Server					[b-ITU-T J.367]

DN 			Directory Number						[b-ITU-T J.460.0]

DND	Do Not Disturb							[b-ITU-T J.460.0], [b-ITU-T J.460.1]

DNG 		Delivery Network Gateway					[b-ITU-T J.702]

DNS	Domain Name Server						[b-ITU-T J.126], [b-ITU-T J.170], [b-ITU-T J.192]

DNS	Domain Name System	[b-IETF RFC 1034]		[b-ITU-T J.160], [b-ITU-T J.167], [b-ITU-T J.171.1], [b-ITU-T J.171.2], [b-ITU-T J.175], [b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.200], [b-ITU-T J.218], [b-ITU-T J.296], [b-ITU-T J.360], [b-ITU-T J.369], [b-ITU-T J.700], [b-ITU-T J.703]

DNS 		Domain Name Service						[b-ITU-T J.362]

DNS SRV		A DNS RR for specifying the location of services	[b-ITU-T J.170]

DOBSS		Data over BWA Security System				[b-ITU-T J.116]

DOC	Data over Cable							[b-ITU-T J.112], [b-ITU-T J.116]

DOCS		Data-Over-Cable System					[b-ITU-T J.122]

DOCS 1.x 		DOCS 1.0 or 1.1							[b-ITU-T J.122]

DOCSIS	Data Over Cable Service Interface Specifications	[b-ITU-T J.126], [b-ITU-T J.128], [b-ITU-T J.161], [b-ITU-T J.170], [b-ITU-T J.177], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.197], [b-ITU-T J.199], [b-ITU-T J.210], [b-ITU-T J.211], [b-ITU-T J.212], [b-ITU-T J.214], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.223.1], [b-ITU-T J.223.2], [b-ITU-T J.292], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.360], [b-ITU-T J.368], [b-ITU-T J.370], [b-ITU-T J.381], [b-ITU-T J.460.4], [b-ITU-T J.604], [b-ITU-T J.700], [b-ITU-T J.703], [b-ITU-T J.704], [b-ITU-T J.800.0], [b-ITU-T J.800.2], [b-ITU-T J.1003], [b-ITU-T J.1004], [b-ITU-T J.1005], [b-ITU-T J.1101], [b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1104], [b-ITU-T J.1105], [b-ITU-T J.1106], [b-ITU-T J.1107]

DOCSIS 	Data Over Cable System Interface Specification	 [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.460.2]

DOCSIS 1.x	Data-Over-Cable Service Interface Specification version 1.0 or 1.1

[b-ITU-T J.222.1]

DOCSIS-MPT(D-MPT)	DOCSIS MPT Mode				[b-ITU-T J.212]

DOM	Document Object Model					[b-ITU-T J.200], [b-ITU-T J.201], [b-ITU-T J.380.7]

DoS	Denial of Service							[b-ITU-T J.192], [b-ITU-T J.222.3], [b-ITU-T J.296]

DPI	Digital Program(me) Insertion					[b-ITU-T J.215], [b-ITU-T J.287], [b-ITU-T J.294], [b-ITU-T J.700]

DPV			DOCSIS Path Verify						[b-ITU-T J.222.2]

DQoS	Dynamic Quality of Service					[b-ITU-T J.161], [b-ITU-T J.170], [b-ITU-T J.177], [b-ITU-T J.179], [b-ITU-T J.361], [b-ITU-T J.190], [b-ITU-T J.261], [b-ITU-T J.362]

DQoS 		Dynamic Quality of Service (IPCablecom)		[b-ITU-T J.191]

DQoS 		Dynamic Quality-of-Service (PacketCable)		[b-ITU-T J.192]

DQPSK	Differential Quadrature Phase-Shift Keying		[b-ITU-T J.195.2], [b-ITU-T J.196.2]

D-RA		Receiver Amplifier for Digital video transmission	[b-ITU-T J.186]

DRFI	Downstream Radio Frequency Interface			[b-ITU-T J.210], [b-ITU-T J.212], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.1102], [b-ITU-T J.1103]

DRM	Digital Right/Rights Management					[b-ITU-T J.124], [b-ITU-T J.190], [b-ITU-T J.197], [b-ITU-T J.206], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.294], [b-ITU-T J.297], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.705], [b-ITU-T J.1005], [b-ITU-T J.1006], [b-ITU-T J.1010], [b-ITU-T J.1011], [b-ITU-T J.1020]

DRRP		Dynamic Resource Registration Protocol			[b-ITU-T J.700]

DRW	  		Dynamic Range Window					[b-ITU-T J.222.1]

DS	Down Stream  							[b-ITU-T J.128], [b-ITU-T J.211], [b-ITU-T J.212], [b-ITU-T J.214], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.296], [b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1104], [b-ITU-T J.1106], [b-ITU-T J.1107]

DS 			Data Slice								[b-ITU-T J.382]

DSA	Dynamic Service Addition					[b-ITU-T J.112 Annex C], [b-ITU-T J.163]

DSA 		Dynamic Service Add						[b-ITU-T J.263]

DSC	Dynamic Service Change					[b-ITU-T J.112 Annex C], [b-ITU-T J.163]

DSC  	Descrambler							[b-ITU-T J.1002], [b-ITU-T J.1004]

DSD	Dynamic Service Deletion					[b-ITU-T J.112 Annex C]

DSCP	Differentiated Services Code Point				[b-ITU-T J.212], [b-ITU-T J.222.2], [b-ITU-T J.263], [b-ITU-T J.368]

DSCP 	DiffServ Code Point						[b-ITU-T J.290], [b-ITU-T J.292]

DSCP 	DiffServ Code Point. A field in every IP packet that identifies the DiffServ Per‑Hop Behavior. In IP version 4, the TOS byte is redefined to be the DSCP. In IP version 6, the Traffic Class octet is used as the DSCP. See IETF RFC 4556.										[b-ITU-T J.170]

DSCQS	Double Stimulus Continuous Quality Scale		[b-ITU-T J.144]

DS-EH/DS EHDR	Downstream Service Extended Header		[b-ITU-T J.222.2]

DSG	DOCSIS Set Top Gateway					[b-ITU-T J.126], [b-ITU-T J.128], [b-ITU-T J.222.2], [b-ITU-T J.291], [b-ITU-T J.294]

DSG 		DOCSIS Set-top box Gateway				[b-ITU-T J.700]

DSID	Downstream Service Identifier				[b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.800.2], [b-ITU-T J.1102], [b-ITU-T J.1103]

DSL	Digital Subscriber Line						[b-ITU-T J.290], [b-ITU-T J.700]

DSLAM		Digital Subscriber Line Access Multiplexer		[b-ITU-T J.292]

DSM-CC	Digital Storage Media – Command and Control	[b-ITU-T J.111], [b-ITU-T J.200]

DSMCC 		Digital Storage Media Command and Control		[b-ITU-T J.230]

DSM-CC 	Digital Storage Media Command and Control		[b-ITU-T J.291], [b-ITU-T J.705], [b-ITU-T J.700], [b-ITU-T J.703]

DSM-CC-U-N	DSM-CC User-to-Network					[b-ITU-T J.111]

DSM-CC-U-U	DSM-CC User-to-User						[b-ITU-T J.111]

DSMCC DC	Digital Storage Media Command and Control – Data Carousel

[b-ITU-T J.205], [b-ITU-T J.206]

DSMCC OC	Digital Storage Media Command and Control – Object Carousel

[b-ITU-T J.205], [b-ITU-T J.206]

DSNG		Digital Satellite News Gathering			[b-ITU-T J.96]

DSP			Digital Signal Processor				[b-ITU-T J.95], [b-ITU-T J.603]

DSP 			Data Services Profile						[b-ITU-T J.114]

DS-SG		Downstream Service Group				[b-ITU-T J.222.2]

DSU			Data Service Unit					[b-ITU-T J.214]

DSx (Messaging)		J.112 Annex B QoS signalling mechanism providing Dynamic Service Add, Change and Delete semantics						[b-ITU-T J.179]

D-TA		Transmitter Amplifier for Digital video transmission	 [b-ITU-T J.186]

DTCP	Digital Transmission Content Protection			[b-ITU-T J.197], [b-ITU-T J.290], [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.700], [b-ITU-T J.1005]

DTCP-IP 		Digital Transmission Content Protection over Internet Protocol	[b-ITU-T J.296]

DTD	Document Type Definition				[b-ITU-T J.200], [b-ITU-T J.800.0]

DTE	Data Terminal Equipment				[b-ITU-T J.112], [b-ITU-T J.112 Annex A], [b-ITU-T J.115], [b-ITU-T J.116], [b-ITU-T J.118]

DTI	DOCSIS Timing Interface				[b-ITU-T J.210], [b-ITU-T J.211], [b-ITU-T J.212], [b-ITU-T J.214], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.800.2]

DTMF	Dual Tone Multi Frequency				[b-ITU-T J.160], [b-ITU-T J.162], [b-ITU-T J.175], [b-ITU-T J.181], [b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.460.2]

DTMF	Dual Tone Multi Frequency (tones)		[b-ITU-T J.161], [b-ITU-T J.170], [b-ITU-T J.361]

DTMF	Dual Tone Multifrequency (dialling mode)	[b-ITU-T J.112], [b-ITU-T J.112 Annex A], [b-ITU-T J.116]

DTS			DOCSIS Time Stamp, 32 bits			[b-ITU-T J.212]

DTS 		32-bit DOCSIS Time Stamp				[b-ITU-T J.211]

DTS 	Decoding Time Stamp					[b-ITU-T J.89], [b-ITU-T J.181], [b-ITU-T J.343.5], [b-ITU-T J.343.6]

DTV	Digital Television					[b-ITU-T J.117], [b-ITU-T J.201], [b-ITU-T J.206], [b-ITU-T J.223.1], [b-ITU-T J.290], [b-ITU-T J.292], [b-ITU-T J.205], [b-ITU-T J.230]

DTV 		Digital Television: ATSC compliant receiving device	[b-ITU-T J.151]

DTVC		Digital Television by Cable				[b-ITU-T J.83]

DTX			Discontinuous Transmission				[b-ITU-T J.361]

Du	uplink Data							[b-ITU-T J.195.2], [b-ITU-T J.195.3], [b-ITU-T J.196.2], [b-ITU-T J.196.3]

DUID	DHCP Unique Identifier				[b-ITU-T J.218], [b-ITU-T J.222.2]

DUT	Downstream Unencrypted Traffic			[b-ITU-T J.213], [b-ITU-T J.222.2]

DVA	Digital Voice Adaptor or Adapter			[b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.460.2]

DVB	Digital Video Broadcasting				[b-ITU-T J.96], [b-ITU-T J.111], [b-ITU-T J.112 Annex A], [b-ITU-T J.115], [b-ITU-T J.117], [b-ITU-T J.118], [b-ITU-T J.131], [b-ITU-T J.142], [b-ITU-T J.200], [b-ITU-T J.700], [b-ITU-T J.705], [b-ITU-T J.1010], [b-ITU-T J.1011]

DVB 		Digital Video Broadcast				[b-ITU-T J.181], [b-ITU-T J.290]

DVB-ASI 		Digital Video Broadcast – Asynchronous Serial Interface		[b-ITU-T J.280]

DVB-C		DVB system for cable transmission		[b-ITU-T J.382]

DVB-C2		DVB system for second generation cable transmission		[b-ITU-T J.382]

DVB-MS		DVB-Microwave Satellite				[b-ITU-T J.116]

DVB-SI		Digital Video Broadcasting – Service Information			[b-ITU-T J.142]

DVCR		Digital Video Cassette Recorder			[b-ITU-T J.151]

DVD	Digital Versatile Disk/Disc				[b-ITU-T J.117], [b-ITU-T J.190], [b-ITU-T J.296], [b-ITU-T J.700]

DVD 		Digital Video Disk					[b-ITU-T J.151]

DVD+R		Digital Versatile Disk + Recordable		[b-ITU-T J.197]

DVD-ROM	Digital Versatile Disc-Read Only Memory	[b-ITU-T J.95]

DVD-RW		Digital Versatile Disk – Re-Writable		[b-ITU-T J.197]

DVI			Digital Visual Interface					[b-ITU-T J.197]

DVI 			Digital Video Interface					[b-ITU-T J.290]

DVNR		Digital Video Noise Reduction			[b-ITU-T J.95]

DVR			Digital Video Recording				[b-ITU-T J.290]

DVR 	Digital Video Recorder					[b-ITU-T J.293], [b-ITU-T J.296], [b-ITU-T J.700], [b-ITU-T J.703], [b-ITU-T J.704]

DVS			Digital Video Service					[b-ITU-T J.700]

E.164			Telephone number standard of ITU			[b-ITU-T J.178]

E/O			Electrical to Optical converter 			[b-ITU-T J.185]

E/O 			Electrical to Optical					[b-ITU-T J.186]

E/O			Electrical to Optic					[b-ITU-T J.1107]

EAC 	 	Emergency Alert Controller				[b-ITU-T J.703]

EAE	Early Authentication and Encryption		[b-ITU-T J.218], [b-ITU-T J.222.2], [b-ITU-T J.222.3]

EAM 		Emergency Alert Message				[b-ITU-T J.703]

EAS	Emergency Alert System				[b-ITU-T J.294], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.703], [b-ITU-T J.705]

Eb			Energy per bit						[b-ITU-T J.142]

EB 			Errored Block						[b-ITU-T J.142]

EBC			Error Block Count					[b-ITU-T J.132]

EC			Errors Corrected						[b-ITU-T J.222.1]

ECB	Electronic Code Book					[b-ITU-T J.96], [b-ITU-T J.181], [b-ITU-T J.222.3], [b-ITU-T J.290]

ECB  		Electric Code Block					[b-ITU-T J.1002]

ECG	Electronic Content Guide				[b-ITU-T J.294], [b-ITU-T J.296], [b-ITU-T J.702]

ECI 	Embedded Common Interface			[b-ITU-T J.1010], [b-ITU-T J.1011]

ECL 		Entitlement Control License				[b-ITU-T J.1006]

ECM	Entitlement Control Message				[b-ITU-T J.81], [b-ITU-T J.91], [b-ITU-T J.96], [b-ITU-T J.181 Amendment 1], [b-ITU-T J.281], [b-ITU-T J.282], [b-ITU-T J.290], [b-ITU-T J.293], [b-ITU-T J.297], [b-ITU-T J.700], [b-ITU-T J.703], [b-ITU-T J.1001]

eCM	embedded Cable Modem    			[b-ITU-T J.126], [b-ITU-T J.128],  [b-ITU-T J.213], [b-ITU-T J.222.2], [b-ITU-T J.293], [b-ITU-T J.460.2], [b-ITU-T J.800.1]

eCM  		embedded cable modem entity (e.g., STB, router, MTA, etc.)	[b-ITU-T J.700]

ECN			Engineering Change Notice				[b-ITU-T J.112 Annex B]

ECO			Engineering Change Order				[b-ITU-T J.112 Annex B]

ECR			Engineering Change Request				[b-ITU-T J.112 Annex B]

ECW			Even control word					[b-ITU-T J.81]

E-CSCF 		Emergency-Call Session Control Function	[b-ITU-T J.460.1]

EDE	Encrypt-Decrypt-Encrypt				[b-ITU-T J.181], [b-ITU-T J.222.3]

EDE 		A 3DES mode where the data is encrypted, decrypted and encrypted

[b-ITU-T J.366.8]

EDH			Error Detection and Handling			[b-ITU-T J.89]

EDID		Extended Display Identification Data		[b-ITU-T J.296]

EDL			Edit Decision List					[b-ITU-T J.285]

eDOCSIS	Embedded DOCSIS					[b-ITU-T J.126], [b-ITU-T J.199], [b-ITU-T J.460.2], [b-ITU-T J.800.2]

E-DVA	Embedded Digital Voice Adapter or Adaptor	[b-ITU-T J.261], [b-ITU-T J.460.0], [b-ITU-T J.460.2], [b-ITU-T J.460.4]

EEPROM		Electrically Erasable Programmable Read-Only Memory

[b-ITU-T J.91], [b-ITU-T J.197]

EH 	Extended Header						[b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.1102], [b-ITU-T J.1103]

EHDR or EH	Extended Header						[b-ITU-T J.112], [b-ITU-T J.116]

EHDR 	Extended MAC Header					[b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.1102], [b-ITU-T J.1103]

EI	Emergency Interrupt					[b-ITU-T J.178], [b-ITU-T J.460.0]

EIDR			Entertainment Identifier Registry association	[b-ITU-T J.181]

EISF			Extended Information Subframe			[b-ITU-T J.196.3]

EIT	Event Information Table				[b-ITU-T J.94], [b-ITU-T J.151], [b-ITU-T J.302]

EKE			Explicit Key Exchange					[b-ITU-T J.112 Annex A]

EM			Event Messages						[b-ITU-T J.363]

EMAC	Ethernet MAC						[b-ITU-T J.195.3], [b-ITU-T J.196.3]

EMC	Electromagnetic Compatibility			[b-ITU-T J.114], [b-ITU-T J.195.1], [b-ITU-T J.196.2], [b-ITU-T J.222.1]

EMF			Equipment Management Function			[b-ITU-T J.131], [b-ITU-T J.132]

EMI			Encryption Mode Indicator				[b-ITU-T J.197]

EML 		Entitlement Management License			[b-ITU-T J.1006]

EMM	Entitlement Management Message			[b-ITU-T J.81], [b-ITU-T J.91], [b-ITU-T J.96], [b-ITU-T J.181 Amendment 1], [b-ITU-T J.297], [b-ITU-T J.281], [b-ITU-T J.282], [b-ITU-T J.290], [b-ITU-T J.292], [b-ITU-T J.293], [b-ITU-T J.296], [b-ITU-T J.700], [b-ITU-T J.1001]

EMS			Element Management System			[b-ITU-T J.360]

eMTA  		embedded Media Terminal Adapter   		 [b-ITU-T J.213]

eMTA		embedded Multimedia Terminal Adapter		[b-ITU-T J.126], [b-ITU-T J.700]

eMTA		Embedded Media Transport Agent	   		[b-ITU-T J.222.2]

E-MTA		Embedded Media Terminal Adapter		[b-ITU-T J.261]

E-MTA		Embedded Multimedia Terminal Adapter	[b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.363], [b-ITU-T J.360], [b-ITU-T J.460.0]

E-MTA		Embedded MTA 					[b-ITU-T J.173], [b-ITU-T J.175]

EMTA  		Embedded Multimedia Terminal Adaptor	[b-ITU-T J.704]

EN			European Norm						[b-ITU-T J.115]

ENC			Encoder 							[b-ITU-T J.147], [b-ITU-T J.240]

ENG 		Electronic News Gathering				[b-ITU-T J.388]

ENUM		E.164 Number Mapping				[b-ITU-T J.360]

ENUM 		Telephone Number Mapping				[b-ITU-T J.460.1]

EP			Endpoint							[b-ITU-T J.190]

EPG	Electronic Program(me) Guide				[b-ITU-T J.90], [b-ITU-T J.97], [b-ITU-T J.98], [b-ITU-T J.117], [b-ITU-T J.128], [b-ITU-T J.205], [b-ITU-T J.206], [b-ITU-T J.230], [b-ITU-T J.281], [b-ITU-T J.282], [b-ITU-T J.290], [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.700], [b-ITU-T J.701], [b-ITU-T J.702], [b-ITU-T J.703], [b-ITU-T J.704], [b-ITU-T J.707], [b-ITU-T J.900]

EPON	Ethernet Passive Optical Network			[b-ITU-T J.185], [b-ITU-T J.223.2]

ePS			embedded Portal Services Element			[b-ITU-T J.126]

ePS 	embedded Portal Services 				[b-ITU-T J.192],	[b-ITU-T J.213] ,[b-ITU-T J.222.2]

EPSNR	Edge Peak Signal-to-Noise Ratio 			[b-ITU-T J.249], [b-ITU-T J.343.4]

EQAM	Edge QAM (A network element which receives MPEG-TS frames over a network interface such as Ethernet, and modulates them onto QAM carriers for use on a HFC plant)							[b-ITU-T J.211]

EQAM 	Edge QAM							[b-ITU-T J.212], [b-ITU-T J.222.2]

ERM			Edge Resource Manager				[b-ITU-T J.212], [b-ITU-T J.700]

ERMI	Edge Resource Manager Interface	[b-ITU-T J.210], [b-ITU-T J.211], [b-ITU-T J.212], [b-ITU-T J.222.2]

eRouter		Embedded Router					[b-ITU-T J.222.2]

ES			Errored Second						[b-ITU-T J.132], [b-ITU-T J.142]

ES 	Elementary Stream					[b-ITU-T J.296], [b-ITU-T J.700], [b-ITU-T J.900]

ES 	Elementary bitStream					[b-ITU-T J.343], [b-ITU-T J.343.4]

eSAFE		embedded Service/Application Functional Entity

[b-ITU-T J.126], [b-ITU-T J.213], [b-ITU-T J.218], [b-ITU-T J.222.2], [b-ITU-T J.800.1], [b-ITU-T J.800.2]

ESG			Electronic Service Guide	   			[b-ITU-T J.294], [b-ITU-T J.702]

ESP			IPsec Encapsulating Security				[b-ITU-T J.170]

ESP 			IPSec Encapsulating Security Payload		[b-ITU-T J.177]

ESP 			Encapsulating Security Payload			[b-ITU-T J.360]

ESSID		Extended Service Set Identifier	 		[b-ITU-T J.296]

eSTB			embedded Set-Top Box				[b-ITU-T J.126], [b-ITU-T J.128]

ESW			Encrypted Session Word				[b-ITU-T J.96]

eTEA		embedded T1/E1 TDM Emulation Adapter (TEA)			[b-ITU-T J.126]

ETS	European Telecommunications Standard  	[b-ITU-T J.84], [b-ITU-T J.115], [b-ITU-T J.118]

ETV			Enhanced TV						[b-ITU-T J.181 Amendment 1]

EU			Errors Uncorrectable					[b-ITU-T J.222.1]

EUI			Extended Unique Identifier				[b-ITU-T J.117], [b-ITU-T J.218]

EUI-64		64-bit Extended Unique Identifier			[b-ITU-T J.222.2]

EVM			Error Vector Magnitude				[b-ITU-T J.151]

exLSDI 		expanded hierarchy of Large Screen Digital Imagery			[b-ITU-T J.603]

FAS	Frame Alignment Signal				[b-ITU-T J.112 Annex A], [b-ITU-T J.116], [b-ITU-T J.131], [b-ITU-T J.132]

FAT			File Allocation Table					[b-ITU-T J.296]

FAX			Facsimile (ITU-T Rec. T.30)				[b-ITU-T J.190]

FC	Frame Control   					[b-ITU-T J.116], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.1102], [b-ITU-T J.1103]

FC 			Frame Compatible					[b-ITU-T J.900]

FCAPS		Fault, Configuration, Accounting, Performance and Security

[b-ITU-T J.700]

FCRC	Fragment Cyclic Redundancy Check		[b-ITU-T J.222.2], [b-ITU-T J.222.3]

FCS	Frame Check Sequence					[b-ITU-T J.126], [b-ITU-T J.195.2]

FDDI	Fibre Distributed Data Interface			[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]

FDM	Frequency Division Multiplexing			[b-ITU-T J.87], [b-ITU-T J.116], [b-ITU-T J.185], [b-ITU-T J.186]

FDMA	Frequency Division Multiple Access		[b-ITU-T J.112], [b-ITU-T J.116], [b-ITU-T J.222.1]

FDR			False Detection Rate					[b-ITU-T J.147]

FEC	Forward Error Correction				[b-ITU-T J.81], [b-ITU-T J.82], [b-ITU-T J.83], [b-ITU-T J.131], [b-ITU-T J.132], [b-ITU-T J.141], [b-ITU-T J.116], [b-ITU-T J.142], [b-ITU-T J.150], [b-ITU-T J.161], [b-ITU-T J.185], [b-ITU-T J.195.1], [b-ITU-T J.195.2], [b-ITU-T J.195.3], [b-ITU-T J.196.1], [b-ITU-T J.196.2], [b-ITU-T J.196.3], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.241], [b-ITU-T J.282], [b-ITU-T J.292], [b-ITU-T J.295], [b-ITU-T J.297], [b-ITU-T J.381], [b-ITU-T J.382], [b-ITU-T J.702], [b-ITU-T J.704], [b-ITU-T J.1102], [b-ITU-T J.1103]

FFT			Fast Fourier Transform					[b-ITU-T J.382]

FG IPTV		ITU-T IPTV Focus Group				[b-ITU-T J.700]

FHCS		Fragment Header Checksum				[b-ITU-T J.222.2]

FID			Flow Identifier						[b-ITU-T J.164]

FIFO	 		First In, First Out 					[b-ITU-T J.112 Annex A]

FIFO			First in First out						[b-ITU-T J.116]

FIFO			First in First out (shift register)			[b-ITU-T J.83]

FIP			Forward Interaction Path				[b-ITU-T J.116]

FIPS	Federal Information Processing Standard		[b-ITU-T J.181], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.290]

FITL	Fibre In The Loop. A PSTN architecture consisting of a fibre-optic access network							[b-ITU-T J.460.2]

FLR			Frame Loss Ratio					[b-ITU-T J.141]

FLUTE		File Delivery over Unidirectional Transport	[b-ITU-T J.296]

FM	Frequency Modulation					[b-ITU-T J.185], [b-ITU-T J.222.1]

FMO			Flexible Macroblock Ordering	 		[b-ITU-T J.361]

FN			Fibre Node	  						[b-ITU-T J.222.2]

Fn 			The nth Fermat number	  			[b-ITU-T J.222.3]

FoV			Field of View	  					[b-ITU-T J.302]

FP			Fixed Part	  						[b-ITU-T J.114]

FPGA		Field Programmable Gate Array			[b-ITU-T J.95], [b-ITU-T J.603]

FQDN	Fully Qualified Domain Name (Refer to IETF RFC 821 for details)										[b-ITU-T J.126], [b-ITU-T J.160], [b-ITU-T J.167], [b-ITU-T J.170], [b-ITU-T J.172], [b-ITU-T J.177], [b-ITU-T J.178], [b-ITU-T J.179], [b-ITU-T J.212], [b-ITU-T J.360]

FR 	Full Reference 						[b-ITU-T J.244], [b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.340], [b-ITU-T J.341], [b-ITU-T J.342], [b-ITU-T J.343], [b-ITU-T J.343.5], [b-ITU-T J.343.6]

FRTV/FR-TV	Full Reference TeleVision 				[b-ITU-T J.144], [b-ITU-T J.149], [b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342]

FS			File Server							[b-ITU-T J.285]

FSK  		Frequency Shift Key 					[b-ITU-T J.460.0]

FSM			Finite State Machine					[b-ITU-T J.222.3]

FSR			Linear Feedback Shift Register			[b-ITU-T J.116]

FSW			Frame Synchronization Word			[b-ITU-T J.81], [b-ITU-T J.88]

FT			Fixed Termination					[b-ITU-T J.114]

FTP	File Transfer Protocol					[b-ITU-T J.120], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.218], [b-ITU-T J.222.2], [b-ITU-T J.703], [b-ITU-T J.1102], [b-ITU-T J.1103]

FTTB	Fibre To The Building					[b-ITU-T J.195.1], [b-ITU-T J.195.2], [b-ITU-T J.196.1], [b-ITU-T J.196.2], [b-ITU-T J.281], [b-ITU-T J.282],

FTTH	Fibre To The Home					[b-ITU-T J.281], [b-ITU-T J.282], [b-ITU-T J.290], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.1005], [b-ITU-T J.1106], [b-ITU-T J.1107]

FTTx	Fibre To The ''x'', where ''x'' indicates the final location on the user side of any one of a variety of optical fibre architectures, e.g., fibre to the building (FTTB), fibre to the curb (FTTC), fibre to the premises (FTTP)		[b-ITU-T J.223.2], [b-ITU-T J.700]

FTV			Free Viewpoint Television				[b-ITU-T J.902]

FUMO  		Firmware Update Management Object		[b-ITU-T J.705]

FW	FireWall							[b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.294], [b-ITU-T J.360], [b-ITU-T J.700]

FWA			Fixed Wireless Access					[b-ITU-T J.116]

GA			Grand Alliance						[b-ITU-T J.94]

GAA			Generic Authentication Architecture		[b-ITU-T J.367]

GAP			Generic Access Profile					[b-ITU-T J.114]

GARP		Generic Attribute Registration Protocol		[b-ITU-T J.222.2]

GB			Gigabyte							[b-ITU-T J.296]

GBA	Generic Bootstrapping Architecture		[b-ITU-T J.360 Amendment 1], [b-ITU-T J.367]

GBA_H 		GBA with HTTP Digest over TLS enhancements	[b-ITU-T J.366.9]

GBE 	Giga Bit Ethernet						[b-ITU-T J.1101], [b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1104]

GC			Gate Controller						[b-ITU-T J.160], [b-ITU-T J.178]

GCR			Group Classifier Rule					[b-ITU-T J.222.2]

GE			Grant Element  						[b-ITU-T J.195.3]

GE 			Gigabit Ethernet						[b-ITU-T J.223.2]

GE 			Gigabit Ethernet (1 Gbit/s)				[b-ITU-T J.211]

GE 	Gigabit Ethernet (GigE)				[b-ITU-T J.212], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.292]

GEM	Globally Executable Multimedia home platform (MHP)	[b-ITU-T J.294], [b-ITU-T J.200], [b-ITU-T J.700]

GE-PON		Gigabit Ethernet – Passive Optical Network	[b-ITU-T J.700]

GF			Galois Field						[b-ITU-T J.222.1]

GFC			Generic Flow Control					[b-ITU-T J.132]

GHz			Giga Hertz (109 Hertzs)				[b-ITU-T J.142]

GI			Guard Interval  						[b-ITU-T J.382]

GIF			Graphic(s) Interchange Format  			[b-ITU-T J.296], [b-ITU-T J.700]

GIS 			Generalized Information Service			[b-ITU-T J.380.6]

GMAC		Group MAC address					[b-ITU-T J.213]

GMAC 		Group Media Access Control				[b-ITU-T J.222.2]

GMSK		Gaussian Minimum Shift Keying			[b-ITU-T J.115]

GMT	Greenwich Mean Time					[b-ITU-T J.94], [b-ITU-T J.191], [b-ITU-T J.192]

GOB	Group of Blocks						[b-ITU-T J.120]

GOP	Group of Pictures					[b-ITU-T J.88], [b-ITU-T J.181 Amendment 1], [b-ITU-T J.244]

GPI			General Purpose Interface				[b-ITU-T J.287]

G-PON	Gigabit Passive Optical Network  		[b-ITU-T J.185], [b-ITU-T J.703]

GPON 	Gigabit-capable Passive Optical Network	 [b-ITU-T J.223.2], [b-ITU-T J.700]

GPRS		General Packet Radio Service			[b-ITU-T J.363]

GPS			Global Positioning System				[b-ITU-T J.94], [b-ITU-T J.211]

GPU			Graphics Processing Unit				[b-ITU-T J.293], [b-ITU-T J.296]

GQC			Group QoS Configuration				[b-ITU-T J.222.2]

GRUU	Globally Routable User (-) Agent URI		[b-ITU-T J.360], [b-ITU-T J.366.2], [b-ITU-T J.460.1]

GSD	Guaranteed Service Domain				[b-ITU-T J.290], [b-ITU-T J.293], [b-ITU-T J.294]

GSE			Generic Stream Encapsulation			[b-ITU-T J.382]

GSF			Group Service Flow					[b-ITU-T J.222.2]

GSM	Global System for Mobile communications	[b-ITU-T J.111], [b-ITU-T J.115], [b-ITU-T J.195.1], [b-ITU-T J.196.2], [b-ITU-T J.366.0]

GSTN	General Switched Telephone Network		[b-ITU-T J.112], [b-ITU-T J.112 Annex A], [b-ITU-T J.116]

GT			Global Time						[b-ITU-T J.112], [b-ITU-T J.116]

GUI	Graphic(al) User Interface				[b-ITU-T J.117], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.700], [b-ITU-T J.701], [b-ITU-T J.702]

GUSS		GBA User Security Setting				[b-ITU-T J.360 Amendment 1]

GW			Gateway							[b-ITU-T J.170], [b-ITU-T J.700]

H.248	An ITU-T/IETF protocol for media gateway control. Also known as MEGACO. See www.itu.int.									[b-ITU-T J.175]

H/E			cable television Head-End				[b-ITU-T J.297]

HA	 		Home Access						[b-ITU-T J.190], [b-ITU-T J.192]

HA 			High Availability						[b-ITU-T J.380.2]

HAN	Home Area Network					[b-ITU-T J.205], [b-ITU-T J.206], [b-ITU-T J.230]

HANC		Horizontal Ancillary data space in digital video streams	[b-ITU-T J.287]

HAVi		Home Audio Video Interoperability		[b-ITU-T J.117]

HAVi UI		Home Audio Video Interoperability, User Interface specification														[b-ITU-T J.200]

HB			Home Bridge						[b-ITU-T J.190]

HB 	HiNoC Bridge						[b-ITU-T J.195.1], [b-ITU-T J.195.2], [b-ITU-T J.195.3], [b-ITU-T J.196.2], [b-ITU-T J.196.3]

HC			Home Client						[b-ITU-T J.190]

HCS	Header Check Sequence				[b-ITU-T J.112], [b-ITU-T J.116], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.1102], [b-ITU-T J.1103]

HD	High Definition						[b-ITU-T J.181 Amendment 1], [b-ITU-T J.183], [b-ITU-T J.196.1], [b-ITU-T J.291], [b-ITU-T J.296], [b-ITU-T J.381], [b-ITU-T J.341], [b-ITU-T J.361], [b-ITU-T J.604], [b-ITU-T J.700], [b-ITU-T J.703], [b-ITU-T J.900], [b-ITU-T J.1005], [b-ITU-T J.1006], [b-ITU-T J.1011]

HD 			Home Decoder						[b-ITU-T J.190]

HDCP		High-bandwidth Digital Content Protection system																[b-ITU-T J.296], [b-ITU-T J.297]

HDCP 	High-bandwidth Digital Content Protection	[b-ITU-T J.197], [b-ITU-T J.290], [b-ITU-T J.700]

HDD	Hard Disk Drive						[b-ITU-T J.230], [b-ITU-T J.293], [b-ITU-T J.296], [b-ITU-T J.700]

HDLC		High-level Data Link Control			[b-ITU-T J.111]

HDMI	High-Definition Multimedia Interface		[b-ITU-T J.197], [b-ITU-T J.290], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.700], [b-ITU-T J.702]

HDND		Home Digital Network Device			[b-ITU-T J.117]

HDNI		Home Digital Network Interface			[b-ITU-T J.700]

HDR			High Dynamic Range					[b-ITU-T J.297]

HDS 		HTTP Dynamic Streaming				[b-ITU-T J.1005]

HD-SDI		High Definition – Serial Digital Interface		[b-ITU-T J.287]

HDTV	High Definition Television				[b-ITU-T J.151], [b-ITU-T J.183],  [b-ITU-T J.281], [b-ITU-T J.282], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.292], [b-ITU-T J.341], [b-ITU-T J.604], [b-ITU-T J.900], [b-ITU-T J.381], [b-ITU-T J.388], [b-ITU-T J.700]

HE	Head End 							[b-ITU-T J.186], [b-ITU-T J.190], [b-ITU-T J.192], [b-ITU-T J.292], [b-ITU-T J.296]

HE-AAC		High-Efficiency Advanced Audio Coding	[b-ITU-T J.296]

HEC	Header Error Control					[b-ITU-T J.83], [b-ITU-T J.112 Annex A], [b-ITU-T J.132]

HELLO		L2TPv3 Hello message					[b-ITU-T J.212]

HEVC	High Efficiency Video Coding			[b-ITU-T J.297], [b-ITU-T J.381], [b-ITU-T J.604], [b-ITU-T J.1006]

HEX	Hexadecimal						[b-ITU-T J.83], [b-ITU-T J.191], [b-ITU-T J.192]

HF	High Frequency						[b-ITU-T J.116], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]

HFC	Hybrid Fibre/Coax  	  				[b-ITU-T J.160], [b-ITU-T J.361], [b-ITU-T J.700], [b-ITU-T J.179], [b-ITU-T J.190], [b-ITU-T J.212], [b-ITU-T J.222.3], [b-ITU-T J.295], [b-ITU-T J.381]

HFC	Hybrid Fibre Coax					[b-ITU-T J.164], [b-ITU-T J.173], [b-ITU-T J.186], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.214], [b-ITU-T J.382], [b-ITU-T J.460.2]

HFC	Hybrid Fibre Coaxial 	  				[b-ITU-T J.128], [b-ITU-T J.290], [b-ITU-T J.1105], [b-ITU-T J.1106], [b-ITU-T J.1107]

HFC	Hybrid Fibre-Coaxial					[b-ITU-T J.222.2], [b-ITU-T J.296]

HFC	Hybrid Fibre/Coax System				[b-ITU-T J.116], [b-ITU-T J.222.1]

HFC			Hybrid Fibre/Coax (HFC) System			[b-ITU-T J.122]

HFC	Hybrid-Fibre/Coax					[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.170], [b-ITU-T J.193], [b-ITU-T J.294]

HFC 	Hybrid Fibre/Coaxial					 [b-ITU-T J.1101], [[b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1104], [b-ITU-T J.1005]

HFC			Hybrid Fibre/Coaxial cable				[b-ITU-T J.161]

HFC	Hybrid Fibre and Coaxial 				[b-ITU-T J.293], [b-ITU-T J.223.2], [b-ITU-T J.900]

HFC			Hybrid Fibre and Coaxial network			[b-ITU-T J.110]

HFC			Hybrid of Fibre-Coaxial	  			[b-ITU-T J.297]

HGW		Home Gateway						[b-ITU-T J.292]

HH			Horizontal High frequency component		[b-ITU-T J.88]

HiMAC		HiNoC MAC 						[b-ITU-T J.195.3]

HiMAC 		HiNoC 2.0 MAC						[b-ITU-T J.196.3]

HiNoC	High performance Network over Coax		[b-ITU-T J.195.1], [b-ITU-T J.195.2], [b-ITU-T J.195.3], [b-ITU-T J.196.1], [b-ITU-T J.196.2], [b-ITU-T J.196.3]

HL			High Level							[b-ITU-T J.388]

HLS 	HTTP Live Streaming					[b-ITU-T J.181 Amendment 1], [b-ITU-T J.296], [b-ITU-T J.1005]

HM	HiNoC Modem   					[b-ITU-T J.195.1], [b-ITU-T J.195.2], [b-ITU-T J.195.3], [b-ITU-T J.196.2], [b-ITU-T J.196.3]

HMAC		Keyed-Hashing for Message Authentication	[b-ITU-T J.125]

HMAC 	Keyed-Hash Message Authentication Code	[b-ITU-T J.222.2], [b-ITU-T J.222.3]

HMAC 		Hashed Message Authentication Code		[b-ITU-T J.1002]

HMAC  	Hash-based Message Authentication Code	[b-ITU-T J.112 Annex A], [b-ITU-T J.1003], [b-ITU-T J.1004]

HMAC 	Hashed Message Authentication Code. A message authentication algorithm, based on either SHA-1 or MD5 hash and defined in IETF RFC 2104.	[b-ITU-T J.170]

HN	Home Network						[b-ITU-T J.193], [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.702]

HOVC		Higher Order Virtual Container			[b-ITU-T J.132]

HP			High Profile						[b-ITU-T J.296], [b-ITU-T J.388]

HPNA		Home Phoneline Networking Alliance		[b-ITU-T J.293], [b-ITU-T J.294]

HRC 	Harmonic Related Carriers				[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.222.1], [b-ITU-T J.700]

HRC 	Hypothetical Reference Circuit			[b-ITU-T J.149], [b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.343.1], [b-ITU-T J.343.2], [b-ITU-T J.343.3], [b-ITU-T J.343.4], [b-ITU-T J.343.5], [b-ITU-T J.343.6], [b-ITU-T J.244], [b-ITU-T J.342]

HRC			Hypothetical Reference Circuit or connection	[b-ITU-T J.21]

HSD			High Speed Data						[b-ITU-T J.700]

HSP			Home Security Portal					[b-ITU-T J.294]

HSS	Home Subscriber Server				[b-ITU-T J.262], [b-ITU-T J.360], [b-ITU-T J.460.1], [b-ITU-T J.700]

H-STB		Hybrid Set-Top-Box					[b-ITU-T J.294], [b-ITU-T J.295]

HT			Home Terminal						[b-ITU-T J.193]

HTML	Hyper Text Mark-up Language			[b-ITU-T J.117], [b-ITU-T J.200], [b-ITU-T J.201], [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.701], [b-ITU-T J.702]

HTTP	Hyper Text Transport Protocol			[b-ITU-T J.120], [b-ITU-T J.124], [b-ITU-T J.190], [b-ITU-T J.380.1]

HTTP 	Hyper Text Transfer Protocol			[b-ITU-T J.127], [b-ITU-T J.160], [b-ITU-T J.167], [b-ITU-T J.177], [b-ITU-T J.200], [b-ITU-T J.205], [b-ITU-T J.206], [b-ITU-T J.296], [b-ITU-T J.360],[b-ITU-T J.365], [b-ITU-T J.367], [b-ITU-T J.369], [b-ITU-T J.380.7], [b-ITU-T J.700], [b-ITU-T J.701], [b-ITU-T J.702], [b-ITU-T J.703], [b-ITU-T J.705], [b-ITU-T J.707], [b-ITU-T J.1006], [b-ITU-T J.1011]

HTTPS	Secure Hyper Text Transfer Protocol		[b-ITU-T J.205], [b-ITU-T J.206], [b-ITU-T J.700], [b-ITU-T J.707]

HTTPS 		Hyper Text Transfer Protocol over Secure Socket Layer	[b-ITU-T J.296]

HTTPS 		HTTP over SSL or HTTP Secure			[b-ITU-T J.380.7]

HTTPS 		Hyper Text Transfer Protocol Security		[b-ITU-T J.702]

HV			Horizontal and Vertical					[b-ITU-T J.249]

HVS	Human Visual System					[b-ITU-T J.343.5], [b-ITU-T J.343.6]

HW			Hardware							[b-ITU-T J.296], [b-ITU-T J.701]

Hybrid-FR	Hybrid Full Reference 				[b-ITU-T J.343], [b-ITU-T J.343.5], [b-ITU-T J.343.6]

Hybrid-FRe	Hybrid Full Reference encrypted			[b-ITU-T J.343], [b-ITU-T J.343.5]

Hybrid-NR	Hybrid No Reference					[b-ITU-T J.343], [b-ITU-T J.343.1], [b-ITU-T J.343.2]

Hybrid-NRe	Hybrid No Reference encrypted			[b-ITU-T J.343], [b-ITU-T J.343.1]

Hybrid-RR	Hybrid Reduced Reference				[b-ITU-T J.343], [b-ITU-T J.343.3]

Hybrid-RR		Hybrid Reduced Reference model 			[b-ITU-T J.343.4]

Hybrid-RRe	Hybrid Reduced Reference encrypted		[b-ITU-T J.343], [b-ITU-T J.343.3]

Hz			Hertz (1 cycle per second)				[b-ITU-T J.142]

I			In-phase modulation component			[b-ITU-T J.222.1]

I, Q			In-phase, Quadrature phase components of the modulated signal 	[b-ITU-T J.150]

I/F			Interface							[b-ITU-T J.296]

I/Q			In-Phase/Quadrature-Phase				[b-ITU-T J.142]

iAPR	A register affiliated with an asynchronous connection, that indicates how much of data has been produced								[b-ITU-T J.117]

IB	In-Band							[b-ITU-T J.112], [b-ITU-T J.112 Annex A], [b-ITU-T J.116], [b-ITU-T J.118]

IBB	Integrated Broadcast-Broadband			[b-ITU-T J.201], [b-ITU-T J.207], [b-ITU-T J.302]

IBB 			Integrated Broadcast and Broadband		[b-ITU-T J.205], [b-ITU-T J.206]

IC 	Interaction Channel 					[b-ITU-T J.112], [b-ITU-J.114], [b-ITU-T J.112 Annex A], [b-ITU-T J.116], [b-ITU-T J.118]

IC	Interactive Channel 			 		[b-ITU-T J.115]

IC 			Inter-exchange Carrier					[b-ITU-T J.177]

IC 			Integrated Circuit						[b-ITU-T J.296]

ICCIEK 		Individual CAM Client Image Encryption Key [b-ITU-T J.1003]

ICCN		L2TPv3 Incoming-Call-Connected message  	[b-ITU-T J.212]

ICE	Interactive Connectivity Establishment	  	[b-ITU-T J.360], [b-ITU-T J.362], [b-ITU-T J.460.1], [b-ITU-T J.700]

ICID	IMS Charging Identity (ID)	  		     [b-ITU-T J.363], [b-ITU-T J.460.3]

ICMP	Internet Control Message Protocol [b-IETF RFC 792]

[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.161], [b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.1], [b-ITU-T J.222.2]

ICMPv4		IPv4 version of the Internet Control Message Protocol	[b-ITU-T J.222.2]

ICMPv6		IPv6 version of the Internet Control Message Protocol	[b-ITU-T J.222.2]

I-CMTS		Integrated Cable Modem Termination System			[b-ITU-T J.222.2]

ICPAC		Interim CPAC	[b-ITU-T J.95]

ICRP 		L2TPv3 Incoming-Call-Reply message		[b-ITU-T J.212]

ICRQ 		L2TPv3 Incoming-Call-Request message	[b-ITU-T J.212]

I-CSCF	Interrogating-Call Session Control Function (CSCF)	[b-ITU-T J.360], [b-ITU-T J.388], [b-ITU-T J.700]

ICT			Information and Communication Technologies [b-ITU-T J.296]

ID	IDentifier or Identifier					[b-ITU-T J.116], [b-ITU-T J.280], [b-ITU-T J.290]

Id	Identifier 							[b-ITU-T J.181], [b-ITU-T J.218],

IDR	Instantaneous Decoding Refresh			[b-ITU-T J.181 Amendment 1], [b-ITU-T J.286]

IDS			Insertion Data Signal					[b-ITU-T J.92]

iDTV			integrated Digital TV					[b-ITU-T J.1011]

IDU			Interface Data Unit					[b-ITU-T J.82]

IE 	Information Element					[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.222.1], [b-ITU-T J.222.2]

IE			Information Element (An element of a MAP message)		[b-ITU-T J.211]

IEV			International Electrotechnical Vocabulary	[b-ITU-T J.61]

IF	Intermediate Frequency				[b-ITU-T J.83], [b-ITU-T J.116], [b-ITU-T J.151], [b-ITU-T J.185]

IFFT	Inverse Fast Fourier Transform			[b-ITU-T J.195.2], [b-ITU-T J.196.2], [b-ITU-T J.382]

IFG 	Inter-Frame Gap						[b-ITU-T J.195.3], [b-ITU-T J.196.3]

IGD			Internet Gateway Device				[b-ITU-T J.296]

IGMP	Internet Group Management Protocol		[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.282], [b-ITU-T J.292], [b-ITU-T J.294], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.1101], [b-ITU-T J.1102], [b-ITU-T J.1103]

IGMPv3 	 	Internet Group Management Protocol, version 3			[b-ITU-T J.703]

IHK 			Individual Hash Key	     			[b-ITU-T J.1003]

IIM	Interactive Interface Module	   			[b-ITU-T J.112], [b-ITU-T J.112 Annex A], [b-ITU-T J.114], [b-ITU-T J.115], [b-ITU-T J.116], [b-ITU-T J.118]

IIOP			Internet Inter-ORB Protocol				[b-ITU-T J.111]

IJ			Injector	 						[b-ITU-T J.287]

IKE 	A notation defined to refer to the use of IKE with pre-shared keys for authentication.						[b-ITU-T J.170]

IKE 			Internet Key Exchange					[b-ITU-T J.177]

IKE 	Internet Key Exchange is a key management mechanism used to negotiate and derive keys for SAs in IPsec.				[b-ITU-T J.170]

IM			Intensity Modulated					[b-ITU-T J.185]

IM 			Intensity Modulation					[b-ITU-T J.186]

IM 			Instant Messaging					[b-ITU-T J.700]

IM CN		IP Multimedia Core Network				[b-ITU-T J.360 Amendment 1]

IMDTC		International multiple destination television connection		[b-ITU-T N.62]

IMPI			IM Private Identity					[b-ITU-T J.360 Amendment 1]

IMPU		IM Public Identity					[b-ITU-T J.360 Amendment 1]

IMPU 		IMS Public Identity					[b-ITU-T J.460.3]

IMS	IP Multimedia Subsystem				[b-ITU-T J.360], [b-ITU-T J.366.0], [b-ITU-T J.363], [b-ITU-T J.367], [b-ITU-T J.388], [b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.460.3], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.704]

IN			Interactive Network					[b-ITU-T J.114]

INA	Interactive Network Adapter				[b-ITU-T J.112], [b-ITU-T J.112 Annex A], [b-ITU-T J.114], [b-ITU-T J.115], [b-ITU-T J.116], [b-ITU-T J.118], [b-ITU-T J.163], [b-ITU-T J.170]

InfoSet		Information Set						[b-ITU-T J.181]

IOI	Inter-Operator Identifier				[b-ITU-T J.363], [b-ITU-T J.460.3]

IOR	Interoperable Object Reference			[b-ITU-T J.111]

IP	Internet Protocol						[b-ITU-T J.111], [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.120], [b-ITU-T J.121], [b-ITU-T J.122], [b-ITU-T J.124], [b-ITU-T J.126], [b-ITU-T J.128] [b-ITU-T J.160], [b-ITU-T J.161], [b-ITU-T J.162], [b-ITU-T J.163], [b-ITU-T J.164], [b-ITU-T J.167], [b-ITU-T J.171.1], [b-ITU-T J.171.2], [b-ITU-T J.177], [b-ITU-T J.178], [b-ITU-T J.179], [b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.193], [b-ITU-T J.195.1], [b-ITU-T J.197], [b-ITU-T J.211], [b-ITU-T J.212], [b-ITU-T J.214], [b-ITU-T J.218], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.223.2], [b-ITU-T J.230], [b-ITU-T J.241], [b-ITU-T J.245], [b-ITU-T J.281], [b-ITU-T J.282],[b-ITU-T J.288], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.292], [b-ITU-T J.293], [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.360], [b-ITU-T J.361], [b-ITU-T J.362], [b-ITU-T J.363], [b-ITU-T J.365], [b-ITU-T J.367], [b-ITU-T J.380.7], [b-ITU-T J.382], [b-ITU-T J.388], [b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.1010], [b-ITU-T J.1011], [b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1104], [b-ITU-T J.1106]

IP   		Internet Protocol. A network layer protocol	[b-ITU-T J.460.2]

IPAM		IPCablecom Application Manager (sometimes abbreviated as PAM)

[b-ITU-T J.365]

IPC 	Inter Process Communications			[b-ITU-T J.1101], [b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1104]

IP-CAN		IP Connectivity Access Network			[b-ITU-T J.363], [b-ITU-T J.368]

IPCDN		IP over Cable Data Network – a working group of the IETF	[b-ITU-T J.192]

IPCMM		IPCablecom Multimedia				[b-ITU-T J.291]

IPCP			Internet Protocol Control Protocol			[b-ITU-T J.111]

IPDR 	Internet Protocol Detail Record			[b-ITU-T J.204], [b-ITU-T J.222.1], [b-ITU-T J.222.2]

IPDR/SP		Internet Protocol Detail Record Streaming Protocol			[b-ITU-T J.204]

IPDV 		IP Packet Delay Variation				[b-ITU-T J.294]

IPER 		IP Packet Error Ratio					[b-ITU-T J.241], [b-ITU-T J.294]

IPF	    		Inbound Packet Filter					[b-ITU-T J.192]

IPG	   	 	Interactive Program Guide				[b-ITU-T J.294], [b-ITU-T J.702]

IPLR	    		IP Packet Loss Ratio					[b-ITU-T J.241], [b-ITU-T J.294]

IPPV  		Impulse Pay Per View					[b-ITU-T J.94], [b-ITU-T J.700]

IPR  		Intellectual Property Rights				[b-ITU-T J.222.3]

IPS	    		Internet Protocol Stream				[b-ITU-T J.282]

IPSec	Internet Protocol Security				[b-ITU-T J.160], [b-ITU-T J.167], [b-ITU-T J.170], [b-ITU-T J.171.1], [b-ITU-T J.171.2], [b-ITU-T J.177], [b-ITU-T J.192], [b-ITU-T J.261], [b-ITU-T J.360]

IPT 			IP Telephone						[b-ITU-T J.703]

IPTD			IP Packet Transfer Delay				[b-ITU-T J.294]

IPTV	Internet Protocol Television				[b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.703], [b-ITU-T J.1011]

IPTV 		IP Packet Television					[b-ITU-T J.381]

IPTV  		TV using the Internet Protocol			[b-ITU-T J.1010]

IPv4 	Internet Protocol version 4	   			[b-ITU-T J.120], [b-ITU-T J.195.1], [b-ITU-T J.196.1], [b-ITU-T J.212], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.296]

IPv4   		Version 4 of the Internet Protocol			[b-ITU-T J.222.3]

IPv6	Internet Protocol version 6				[b-ITU-T J.120], [b-ITU-T J.195.1], [b-ITU-T J.196.1], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.296], [b-ITU-T J.700]

IPv6   		Version 6 of the Internet Protocol	  		[b-ITU-T J.222.3]

IQ	In-phase and Quadrature Components		[b-ITU-T J.112 Annex A], [b-ITU-T J.116]

IR			Infrared							[b-ITU-T J.296]

IRC			Incrementally-Related Carriers			[b-ITU-T J.116], [b-ITU-T J.700]

IRC 	Incremental Related Carriers				[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.222.1]

IRC 			Interface Reference Clock				[b-ITU-T J.151]

IRD	Integrated Receiver Decoder				[b-ITU-T J.83], [b-ITU-T J.112 Annex A], [b-ITU-T J.114], [b-ITU-T J.116], [b-ITU-T J.118]

IRT	Initial Retransmission Time				[b-ITU-T J.218], [b-ITU-T J.222.2]

IRT    	 	Integrated Receiver Transcoder			[b-ITU-T J.700]

ISA  	 	Instruction Set Architecture				[b-ITU-T J.703]

ISAKMP 		Internet Security Association and Key Management Protocol	[b-ITU-T J.170]

ISAN 		International Standard Audiovisual Number 	[b-ITU-T J.181]

ISCI 		Industry Standard Commercial Identifier		[b-ITU-T J.181]

ISCM		Interactive Services Commercial Module (DVB)			[b-ITU-T J.118]

ISDB-T	Integrated Services Digital Broadcasting–Terrestrial

[b-ITU-T J.185], [b-ITU-T J.186]

ISDN	Integrated Services Digital Network		[b-ITU-T J.110], [b-ITU-T J.111], [b-ITU-T J.112 Annex A], [b-ITU-T J.114], [b-ITU-T J.161], [b-ITU-T J.700]

ISDN 	Integrated Switched Digital Network		[ITU-T J.115], [b-ITU-T J.116]

ISF	  	 	Individual Service Flow				[b-ITU-T J.222.2]

ISIM	   		IMS Subscriber Identity Module			[b-ITU-T J.360 Amendment 1]

ISMS			Information Security Management System	[b-ITU-T J.700]

ISP			Internet Service Provider				[b-ITU-T J.161], [b-ITU-T J.296]

ISS			Initialization Personalization Sub-System	[b-ITU-T J.1020]

ISTP	Internet Signalling Transport Protocol		[b-ITU-T J.160], [b-ITU-T J.170], [b-ITU-T J.262]

ISUP 	Integrated Services Digital Network (ISDN) User Part	[b-ITU-T J.160], [b-ITU-T J.171.1], [b-ITU-T J.171.2], [b-ITU-T J.460.1]

ITC			International Television Centre			[b-ITU-T N.62]

ITS			Insertion Test Signal					[b-ITU-T J.92]

ITV			Interactive Television					[b-ITU-T J.700]

iTV			interactive TV						[b-ITU-T J.703]

IUC	Interval Usage Code					[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.222.1], [b-ITU-T J.222.2]

IV	Initialization Vector 					[b-ITU-T J.112 Annex A], [b-ITU-T J.222.3], [b-ITU-T J.1003]

IVR			Interactive Voice Response System 		[b-ITU-T J.161]

IVR 			Interactive Voice Response				[b-ITU-T J.170], [b-ITU-T J.175]

IVR  	Interactive Voice Responder	 			[b-ITU-T J.460.1], [b-ITU-T J.460.3]

IW	Initialization word					[b-ITU-T J.81]

IWU			Inter-Working Unit	 				[b-ITU-T J.114]

JAR			Java Archive 						[b-ITU-T J.700]

JCE			Java Cryptography Extension			[b-ITU-T J.700]

JMF	 		Java Media Framework					[b-ITU-T J.200]

JPG			JPEG image format					[b-ITU-T J.200]

KDC	Key Distribution Centre				[b-ITU-T J.170], [b-ITU-T J.179], [b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.261], [b-ITU-T J.800.2]

KDC 	Key Distribution Center: the Authentication Server which implements the Kerberos PKINIT Authentication Protocol		[b-ITU-T J.369]

KEK			Key Encryption Key					[b-ITU-T J.222.3]

kHz			kilo Hertz (1000 Hz)					[b-ITU-T J.142]

KID	 		Key Identification					[b-ITU-T J.1006]

KLV	 		Key-Length-Value coding 				[b-ITU-T J.285]

KPK	Key Pairing Key						[b-ITU-T J.1002], [b-ITU-T J.1003], [b-ITU-T J.1004]

L			Level								[b-ITU-T J.296]

L*			Level* (e.g., L0 means Level Zero) 		[b-ITU-T J.388]

L2	Layer 2							[b-ITU-T J.213], [b-ITU-T J.222.2], [b-ITU-T J.282]

L2PDU		Layer 2 Protocol Data Unit				[b-ITU-T J.222.2]

L2TP	    		Layer 2 Transport Protocol				[b-ITU-T J.212]

L2TPv3		Layer 2 Transport Protocol – Version 3		[b-ITU-T J.212]

L2VPN	Layer 2 Virtual Private Network			[b-ITU-T J.213], [b-ITU-T J.222.2], [b-ITU-T J.800.2]

L3			Layer 3							[b-ITU-T J.212]

LA			License Agreement					[b-ITU-T J.1011]

LAC			L2TP Access Concentrator				[b-ITU-T J.212]

LAN			Local Access Network					[b-ITU-T J.296]

LAN 	Local Area Network					[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.117], [b-ITU-T J.122], [b-ITU-T J.190], [b-ITU-T J.192], [b-ITU-T J.218], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.297], [b-ITU-T J.700], [b-ITU-T J.1102]

LAN-Pass		LAN Passthrough Address Realm			[b-ITU-T J.190]

LAN-Pass 		Pass-through LAN address				[b-ITU-T J.191]

LAN-Pass 		Pass-through Local Area Network address	[b-ITU-T J.192]

LAN-Trans		LAN Translated Address Realm			[b-ITU-T J.190]

LAN-Trans		Translated LAN address				[b-ITU-T J.191]

LAN-Trans		Translated Local Area Network address		[b-ITU-T J.192]

LAP			Link Access Protocol					[b-ITU-T J.114]

LASDP		Local Application Service Delivery Platform	[b-ITU-T J.294]

Layer 3 		Network layer 3 in OSI stack				[b-ITU-T J.292]

Layer 3	Network layer in OSI stack; Layer in firewall in which routing is based on IP address							[b-ITU-T J.290]

LBG			Load Balancing Group					[b-ITU-T J.222.2]

LC			Layer-1/2 Converter					[b-ITU-T J.190]

LCCE		L2TP Control Connection Endpoint 		[b-ITU-T J.212]

LCD			Loss of Cell Delineation				[b-ITU-T J.132]

LCD 	Liquid Crystal Display					[b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.460.0]

LCI			Logical CPE Interface					[b-ITU-T J.126]

LCP			Link Control Protocol					[b-ITU-T J.111]

LDPC	Low Density Parity Check				[b-ITU-T J.196.1], [b-ITU-T J.292]

LDPC 	Low Density Parity Check (codes)			[b-ITU-T J.196.2], [b-ITU-T J.382]

LE			Line Extender						[b-ITU-T J.186]

LEC 		Local Exchange Carrier				[b-ITU-T J.460.2]

LED			Light Emitting Diode					[b-ITU-T J.199], [b-ITU-T J.296]

LEN			Length							[b-ITU-T J.112]

LEN 		Length (in bytes unless otherwise stated)		[b-ITU-T J.116]

LFSR	Linear Feedback Shift Register			[b-ITU-T J.112 Annex A], [b-ITU-T J.116], [b-ITU-T J.222.1]

LI			Lawful Intercept						[b-ITU-T J.362]

LIDB  		Line Identification Database				[b-ITU-T J.460.3]

LLC			Link Layer Control					[b-ITU-T J.111]

LLC 	Logical Link Control  				[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.126] [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.223.2], [b-ITU-T J.700]

LLC 	Logical Link Control (LLC) procedure		[b-ITU-T J.116], [b-ITU-T J.122]

LLME		Lower Layer Management Entity			[b-ITU-T J.114]

LMC			Lost and Misinserted Cells	 			[b-ITU-T J.132]

LMCS		Local Multipoint Communication System	[b-ITU-T J.116]

LMDS		Local Multipoint Distribution System		[b-ITU-T J.110], [b-ITU-T J.116]

LNB			Low Noise Block						[b-ITU-T J.84]

LNP			Local Number Portability				[b-ITU-T J.177], [b-ITU-T J.178]

LNS			L2TP Network Server    				[b-ITU-T J.212]

LoF	Loss of Frame     					[b-ITU-T J.131], [b-ITU-T J.132], [b-ITU-T J.214]

LOM 		Loss of Multiframe  					[b-ITU-T J.132]

LOP			Loss of Pointer						[b-ITU-T J.132]

LoS	Loss of Signal						[b-ITU-T J.131], [b-ITU-T J.132],  [b-ITU-T J.214]

LOVC		Lower Order Virtual Container			[b-ITU-T J.132]

LPCM		Linear Pulse Code Modulation			[b-ITU-T J.700]

LPF			Low Pass Filter   					[b-ITU-T J.249]

LRN			Local Routing Number					[b-ITU-T J.178]

LSB			Least Significant Byte					[b-ITU-T J.96]

LSB 	Least Significant Bit  					[b-ITU-T J.83], [b-ITU-T J.112 Annex A], [b-ITU-T J.116], [b-ITU-T J.117], [b-ITU-T J.195.3], [b-ITU-T J.197], [b-ITU-T J.212], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3]

lsb 	Least Significant Bit					[b-ITU-T J.96]

LSDI 		Large Screen Digital Imagery 			[b-ITU-T J.601], [b-ITU-T J.603]

LSSGR		LATA Switching System Generic Requirements			[b-ITU-T J.161]

LT			Local Time							[b-ITU-T J.112], [b-ITU-T J.116]

LTC			Longitudinal Time Code				[b-ITU-T J.89]

LTE	 		Long Term Evolution					[b-ITU-T J.1005]

LTI			Loss of Timing Inputs					[b-ITU-T J.132]

LUT	Look-Up Table						[b-ITU-T J.343.1], [b-ITU-T J.343.2], [b-ITU-T J.343.3], [b-ITU-T J.343.4], [b-ITU-T J.343.5], [b-ITU-T J.343.6]

LWUIT		Light Weight User Interface Toolkit 		[b-ITU-T J.200]

M/N	Relationship of integer numbers M,N that represents the ratio of the downstream symbol clock rate to the DOCSIS master clock rate			[b-ITU-T J.212], [b-ITU-T J.222.1], [b-ITU-T J.222.2]

MAA			MPEG ATM Adaptation				[b-ITU-T J.131], [b-ITU-T J.132]

MAA	 		Multimedia-Auth-Answer				[b-ITU-T J.360 Amendment 1]

MAC 	Multiplexed Analogue Component	  		[b-ITU-T N.62], [b-ITU-T J.80], [b-ITU-T J.81]

MAC 	Message Authentication Code. A fixed-length data item that is sent together with a message to ensure integrity; also known as a MIC.			[b-ITU-T J.170]

MAC  	Medium Access Control				[b-ITU-T J.111], [b-ITU-T J.114], [b-ITU-T J.388]

MAC 	Media Access Control					[b-ITU-T J.112 Annex A], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.118], [b-ITU-T J.128], [b-ITU-T J.160], [b-ITU-T J.167], [b-ITU-T J.172], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.195.1], [b-ITU-T J.195.2], [b-ITU-T J.195.3], [b-ITU-T J.196.1], [b-ITU-T J.196.2], [b-ITU-T J.196.3], [b-ITU-T J.212], [b-ITU-T J.213], [b-ITU-T J.214], [b-ITU-T J.218], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.223.1], [b-ITU-T J.223.2], [b-ITU-T J.281], [b-ITU-T J.282], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.292], [b-ITU-T J.296], [b-ITU-T J.365], [b-ITU-T J.700], [b-ITU-T J.703], [b-ITU-T J.1101], [b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1104], [b-ITU-T J.1105]

MAC	Media Access Control. It is a sublayer of the Data Link Layer. It normally runs directly over the physical layer.						[b-ITU-T J.170]

MAC	Media Access Control (Used to refer to the layer-2 element of the system, which would include DOCSIS framing and signalling)			[b-ITU-T J.126], [b-ITU-T J.211]

MAC			Media Access Control (MAC) procedure		[b-ITU-T J.122]

MAD			Mean Absolute Difference				[b-ITU-T J.249]

MAN			Metropolitan Area Network				[b-ITU-T J.223.2]

MAP	Bandwidth Allocation Map				[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]

MAP 	Media Access Plan					[b-ITU-T J.195.3], [b-ITU-T J.196.2], [b-ITU-T J.196.3]

MAR			Multimedia-Auth-Request				[b-ITU-T J.360 Amendment 1]

MATV	Master Antenna Television				[b-ITU-T J.111], [b-ITU-T J.114], [b-ITU-T J.118],	[b-ITU-T J.142], [b-ITU-T J.382]

MB			Macro-Block						[b-ITU-T J.249]

MB 			Megabyte							[b-ITU-T J.296]

mbps	Megabits per second					[b-ITU-T J.117]

Mbit			220 = 1,048,576 bits	[b-ITU-T J.382]

Mbit/s	Mbit per second						[b-ITU-T J.382]

MBP 		Management Boundary Point				[b-ITU-T J.192]

MC			Management Centre					[b-ITU-T J.96]

MC 		    	Multimedia Centre equipment			[b-ITU-T J.112], [b-ITU-T J.116]

MC    		Multipoint Controller					[b-ITU-T J.177]

MC 		 	Motion Compensation					[b-ITU-T J.603]

MCF 		Management Client Function				[b-ITU-T J.190], [b-ITU-T J.192]

M-CMTS	Modular Cable Modem Termination System	[b-ITU-T J.210], [b-ITU-T J.211], [b-ITU-T J.212], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.700], [b-ITU-T J.800.2], [b-ITU-T J.1102], [b-ITU-T J.1103]

MCNS		Multimedia Cable Network System		[b-ITU-T J.112], [b-ITU-T J.116]

MCPT		Multiple Carriers per Transponder			[b-ITU-T J.94]

MD	    		Media Access Control Domain			[b-ITU-T J.222.2]

MD5	    		Message Digest 5					[b-ITU-T J.170]

MDBC		Maximum Downstream Bonded Channels	[b-ITU-T J.222.1]

MD-CM-SG	Media Access Control Domain Cable Modem Service Group

[b-ITU-T J.222.2]

MDD	MAC Domain Descriptor	 			[b-ITU-T J.222.3], [b-ITU-T J.1101], [b-ITU-T J.1102], [b-ITU-T J.1103]

MD-DS-SG	Media Access Control Domain Downstream Service Group														[b-ITU-T J.222.2]

MD-DS-SG-ID	Media Access Control Domain Downstream Service Group Identifier													[b-ITU-T J.222.2]

MDF	        	Multicast DSID Forwarding	 			[b-ITU-T J.222.2]

mDNS		multicast Domain Name System			[b-ITU-T J.230]

MDU			Multi-Dwelling Unit	      	 		[b-ITU-T J.700]

MD-US-SG	Media Access Control Domain Upstream Service Group	    [b-ITU-T J.222.2]

MD-US-SG-ID	Media Access Control Domain Upstream Service Group Identifier													[b-ITU-T J.222.2]

mDVR		mobile Digital Video Recorder			[b-ITU-T J.700]

ME 			Motion Estimation	   			 	[b-ITU-T J.603]

MEGACO	An IETF/ITU-T protocol for media gateway control. Also known as ITU‑T Rec. H.248. See www.ietf.org for details.	[b-ITU-T J.175]

MEK 		Message Encryption Key   				[b-ITU-T J.1003]

MER	Modulation Error Ratio  				[b-ITU-T J.142], [b-ITU-T J.212], [b-ITU-T J.222.1]

MER  		Modulation Error Rate  				[b-ITU-T J.185]

MF	    		Multi-Frequency						[b-ITU-T J.160], [b-ITU-T J.178]

MG	Media Gateway						[b-ITU-T J.160], [b-ITU-T J.161], [b-ITU-T J.170], [b-ITU-T J.171.2], [b-ITU-T J.177], [b-ITU-T J.178], [b-ITU-T J.179], [b-ITU-T J.360], [b-ITU-T J.361], [b-ITU-T J.460.1]

MGC	Media Gateway Controller				[b-ITU-T J.160], [b-ITU-T J.161], [b-ITU-T J.164], [b-ITU-T J.170], [b-ITU-T J.171.1], [b-ITU-T J.171.2], [b-ITU-T J.172], [b-ITU-T J.177], [b-ITU-T J.178], [b-ITU-T J.179], [b-ITU-T J.261], [b-ITU-T J.360], [b-ITU-T J.460.0], [b-ITU-T J.460.1]

MGCI		Media Gateway Controller Interface		[b-ITU-T J.162]

MGCP	Media Gateway Control Protocol(refer to [b-IETF RFC 3435])

[b-ITU-T J.162], [b-ITU-T J.170], [b-ITU-T J.171.1], [b-ITU-T J.171.2], [b-ITU-T J.175], [b-ITU-T J.177], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.361]

MGF			Mask Generation Function				[b-ITU-T J.222.3]

MH			Multimedia Home equipment			[b-ITU-T J.112], [b-ITU-T J.116]

MHP	Multimedia Home Platform				[b-ITU-T J.117], [b-ITU-T J.200], [b-ITU-T J.700]

MHz	Mega Hertz or106 Hertz					[b-ITU-T J.142], [b-ITU-T J.382]

MIB	Management Information Base			[b-ITU-T J.111], [b-ITU-T J.160], [b-ITU-T J.162], [b-ITU-T J.166], [b-ITU-T J.170], [b-ITU-T J.171.1], [b-ITU-T J.171.2], [b-ITU-T J.172], [b-ITU-T J.175], [b-ITU-T J.177], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.199], [b-ITU-T J.212], [b-ITU-T J.218], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.290], [b-ITU-T J.295], [b-ITU-T J.362], [b-ITU-T J.369], [b-ITU-T J.370], [b-ITU-T J.460.4], [b-ITU-T J.702], [b-ITU-T J.800.2], [b-ITU-T J.1102], [b-ITU-T J.1103]

MIC	Message Integrity Check				[b-ITU-T J.126], [b-ITU-T J.222.2], [b-ITU-T J.222.3]

MID			Multiple UPID						[b-ITU-T J.181]

MIME	Multipurpose Internet Mail Extensions		[b-ITU-T J.127], [b-ITU-T J.200], [b-ITU-T J.367]

MIPS			Million Instructions Per Second			[b-ITU-T J.296]

MJD			Modified Julian Date					[b-ITU-T J.94]

MK 			Master Key							[b-ITU-T J.1003]

MKE			Main Key Exchange					[b-ITU-T J.112 Annex A]

ML			Main Level							[b-ITU-T J.388]

MLD	Multicast Listener Discovery				[b-ITU-T J.218], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.280], [b-ITU-T J.282], [b-ITU-T J.292], [b-ITU-T J.294], [b-ITU-T J.700], [b-ITU-T J.702]

MM			Multimedia							[b-ITU-T J.246], [b-ITU-T J.247]

MM 5b		MultiMedia video class 5b				[b-ITU-T J.249]

MMD		Multimedia Domain					[b-ITU-T J.361], [b-ITU-T J.367]

MMDS	Multichannel Multipoint Distribution System 	[b-ITU-T J.83], [b-ITU-T J.110], [b-ITU-T J.116], [b-ITU-T J.142], [b-ITU-T J.150]

MMDS	Multi-channel Multi-point Distribution Systems	  [b-ITU-T J.112 Annex A]

MMDS 		Microwave Multipoint Distribution System	[b-ITU-T J.111], [b-ITU-T J.118]

MMH	Multilinear Modular Hash				[b-ITU-T J.160], [b-ITU-T J.170], [b-ITU-T J.222.3]

MMT		Modulation Mode Table				[b-ITU-T J.94]

MMT 		MPEG Media Transport				[b-ITU-T J.297]

MO			Management Object					[b-ITU-T J.367]

MOCA		Multimedia Over Coax Alliance 			[b-ITU-T J.294]

MON			Monitoring							[b-ITU-T J.131]

MOS  	Mean Opinion Score					[b-ITU-T J.244], [b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342], [b-ITU-T J.343], [b-ITU-T J.343.1], [b-ITU-T J.343.2], [b-ITU-T J.343.5], [b-ITU-T J.343.6]

MOSp	Mean Opinion Score, predicted			[b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342]

MOV			Model Output Variable					[b-ITU-T J.145]

MP			Multilink Point-to-Point Protocol (PPP)		[b-ITU-T J.111]

MP 			Management Point					[b-ITU-T J.132]

MP 			Media Player						[b-ITU-T J.175]

MP 			Main Profile						[b-ITU-T J.296], [b-ITU-T J.388]

MP3			MPEG-1 audio layer 3					[b-ITU-T J.700]

MP4			MPEG-4 File Format					[b-ITU-T J.124]

MPAC		Media Access Control layer				[b-ITU-T J.190]

MPC			Media Player Controller				[b-ITU-T J.175]

MPD 	Media Presentation Description			[b-ITU-T J.181 Amendment 1], [b-ITU-T J.1006]

MPEG-2	Motion Picture Experts Group version 2		[b-ITU-T J.183], [b-ITU-T J.288], [b-ITU-T J.900]

MPEG-2 TS	MPEG-2 Transport Stream				[b-ITU-T J.131]

MPEG-ES	 	Moving Picture Experts Group Elementary Stream			[b-ITU-T J.700]

MPEG-TS	 	MPEG Transport Stream				[b-ITU-T J.111], [b-ITU-T J.700]

MPEG-TS  	Motion Picture Experts Group Transport Stream			[b-ITU-T J.211]

MPEG-TS  	Moving Picture Experts Group Transport Stream			[b-ITU-T J.212]

MPF	     	Management Portal Function 			[b-ITU-T J.190]

MPI	     	MPEG Physical Interface				[b-ITU-T J.131], [b-ITU-T J.132]

MPI      	Main Path Interface					[b-ITU-T J.223.2]

MPLS		Multiprotocol Label Switching			[b-ITU-T J.192]

MPT	     	MPEG-TS mode of DEPI				[b-ITU-T J.212]

MPTS	Multiple Program(me) Transport Stream		[b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.700]

MPTS 	Multi (-) Program Transport Stream			[b-ITU-T J.181], [b-ITU-T J.212], [b-ITU-T J.280], [b-ITU-T J.287]

MPU	 		Managed Private UPID					[b-ITU-T J.181]

MRC	Maximum Retransmission Count			[b-ITU-T J.218], [b-ITU-T J.222.2]

MRD	Maximum Retransmission Duration		[b-ITU-T J.218], [b-ITU-T J.222.2]

MRT	Maximum Retransmission Time			[b-ITU-T J.218], [b-ITU-T J.222.2]

MS	    		Mobile Station						[b-ITU-T J.115]

MS 	Microwave Satellite					[b-ITU-T J.116]

MSA	   	 	Multiplex Section Adaptation			[b-ITU-T J.132]

MSAP	Media Access Control Service Access Point	[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.222.2]

MSB			Most significant Byte					[b-ITU-T J.96]

MSB 	Most Significant Bit					[b-ITU-T J.83], [b0-ITU-T J.1122 Annex A], [b-ITU-T J.116], [b-ITU-T J.117], [b-ITU-T J.170], [b-ITU-T J.195.3], [b-ITU-T J.212], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.287], [b-ITU-T J.1004], [b-ITU-T J.1102], [b-ITU-T J.1103]

msb 			Most Significant Bit					[b-ITU-T J.96]

MSC 		Mobile Switching Centre	   			[b-ITU-T J.115]

MSC   	Maximum Scheduled Codes				[b-ITU-T J.222.1], [b-ITU-T J.222.2]

MSE	    		Mean Squared Error					[b-ITU-T J.340]

MSE 	Mean Square Error	 	 			[b-ITU-T J.343.5], [b-ITU-T J.343.6]

MSF	    		Management Server Function			[b-ITU-T J.190], [b-ITU-T J.192]

MSO	Multiple System Operator	    			[b-ITU-T J.380.6], [b-ITU-T J.700], [b-ITU-T J.1001]

MSO			Multiple Service Operator, a Cable Network Operator		 [b-ITU-T J.97]

MSO	    		Multiple System Operator (A network operator)			[b-ITU-T J.369]

MSO  		Multiple System Operators				[b-ITU-T J.381]

MSO	Multiple Systems Operator	    			[b-ITU-T J.195.1], [b-ITU-T J.196.2], [b-ITU-T J.222.2], [b-ITU-T J.1002]

MSOH		Multiplex Section Overhead  			[b-ITU-T J.132]

MSS 		Microsoft Smooth Streaming 			[b-ITU-T J.1005]

MSS 		Management Sub-System				[b-ITU-T J.1020]

MST 		Multiplex Section Termination			[b-ITU-T J.132]

MST   		Minimum Spanning Tree    			[b-ITU-T J.703]

MT	    		Mobile Termination     				[b-ITU-T J.115]

MTA	Multimedia Terminal Adapter			[b-ITU-T J.126], [b-ITU-T J.128], [b-ITU-T J.161], [b-ITU-T J.163], [b-ITU-T J.171.2], [b-ITU-T J.177], [b-ITU-T J.178], [b-ITU-T J.179], [b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.2], [b-ITU-T J.290], [b-ITU-T J.460.0], [b-ITU-T J.700]

MTA 	Media Terminal Adapter				[b-ITU-T J.160], [b-ITU-T J.162], [b-ITU-T J.164], [b-ITU-T J.166], [b-ITU-T J.167], [b-ITU-T J.170], [b-ITU-T J.171.1], [b-ITU-T J.172], [b-ITU-T J.173], [b-ITU-T J.175], [b-ITU-T J.260], [b-ITU-T J.261], [b-ITU-T J.262], [b-ITU-T J.263]

MTC	Multiple Transmit Channel	    			[b-ITU-T J.128], [b-ITU-T J.222.1]

MTIE		Maximum Time Interval Error	    		[b-ITU-T J.211]

MTP	    		Message Transfer Part   				[b-ITU-T J.160]

MTTR	Mean Time to Repair					[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122]

MTU			Multipoint Transceiver Unit				[b-ITU-T J.111]

MTU			Maximum Transfer Unit				[b-ITU-T J.118]

MTU	Maximum Transmission Unit 			[b-ITU-T J.112 Annex A], [b-ITU-T J.128], [b-ITU-T J.212]

MULPI	MAC and Upper Layer Protocols Interface	[b-ITU-T J.222.1], [b-ITU-T J.222.2]

MUX	Multiplex 							[b-ITU-T J.83], [b-ITU-T J.150], [b-ITU-T J.702]

MVC	    		Multiview Video Coding  				[b-ITU-T J.296]

MVDS		Multichannel Video Distribution System		[b-ITU-T J.142]

MVPD		Multichannel Video Programming			[b-ITU-T J.181 Amendment 1]

MVPD 		Multichannel Video Programming Distributor	[b-ITU-T J.295], [b-ITU-T J.1020]

MW			Middleware						[b-ITU-T J.294]

mW			Milliwatt							[b-ITU-T J.142]

MWD	Maximum Waiting Delay				[b-ITU-T J.162], [b-ITU-T J.171.1], [b-ITU-T J.171.2]

MXF	    		Material Exchange Format				[b-ITU-T J.285]

MXF-GC		Material Exchange Format Generic Container	[b-ITU-T J.285]

NA 			Not Applicable 						[b-ITU-T J.382]

Na			Number of active codes				[b-ITU-T J.222.1]

NA(P)T		Network Address and Port Translation; used interchangeably with NAT

[b-ITU-T J.360]

NACF	Network Attachment Control Function(s)	[b-ITU-T J.700], [b-ITU-T J.702]

NACO	Network Access Control Object			[b-ITU-T J.222.1], [b-ITU-T J.222.2]

NAL	    		Network Adaptation Layer				[b-ITU-T J.286]

NAL		    	Network Abstraction Layer				[b-ITU-T J.1006]

NAPT	Network Address Port Translation	   		[b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.218], [b-ITU-T J.294], [b-ITU-T J.296]

NAS	   	 	Network Attached Storage  				[b-ITU-T J.296]

NAT 		Network Address Translator				[b-ITU-T J.160]

NAT	Network Address Translation			[b-ITU-T J.179], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.218], [b-ITU-T J.290], [b-ITU-T J.292], [b-ITU-T J.294], [b-ITU-T J.360], [b-ITU-T J.362], [b-ITU-T J.368], [b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.460.2], [b-ITU-T J.700], [b-ITU-T J.703], [b-ITU-T J.705]

NAT	Network Address Translation [b-IETF RFC 1631], [b-IETF RFC 2663] and [b‑IETF RFC 3022]					[b-ITU-T J.190]

NAT	(IP) Network Address Translation			[b-ITU-T J.361]

NB			Narrow-Band						[b-ITU-T J.388]

NCL			Nested Context Language				[b-ITU-T J.200], [b-ITU-T J.201]

NCO			Network Controlled Oscillator			[b-ITU-T J.211]

NCS	Network Call Signalling				[b-ITU-T J.161], [b-ITU-T J.162], [b-ITU-T J.166], [b-ITU-T J.170], [b-ITU-T J.177], [b-ITU-T J.178], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.361], [b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.460.2]

NCS  	Network-based Call Signalling			[b-ITU-T J.160], [b-ITU-T J.163], [b-ITU-T J.171.1], [b-ITU-T J.171.2], [b-ITU-T J.175], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.360]

NCS    	Network Call Signalling (the IPCablecom MGCP profile used for controlling calls)								[b-ITU-T J.173]

ND	Neighbour Discovery					[b-ITU-T J.218], [b-ITU-T J.222.2]

nDVR		network Digital Video Recorder			[b-ITU-T J.700]

NE	Network Element						[b-ITU-T J.131], [b-ITU-T J.132], [b-ITU-T J.290], [b-ITU-T J.362]

NFD			Normalized Frame Difference 			[b-ITU-T J.249]

NFS			Network File System					[b-ITU-T J.205], [b-ITU-T J.206]

NGHE		Next Generation Headend				[b-ITU-T J.291]

NGN	Next Generation Network				[b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.703]

NGNA	     Next Generation Network Architecture, see NGNA LLC	  	 [b-ITU-T J.210]

NG-STB	     Next Generation Set-Top Box			[b-ITU-T J.292]

NG-STB-A	     Next Generation STB Architecture			[b-ITU-T J.290]

NG-STB-MI-A	Next Generation Set-Top Box Media-Independent Architecture	[b-ITU-T J.292]

NHFE		Normalized High Frequency Energy		[b-ITU-T J.249]

NHM 	New HiNoC Modem					[b-ITU-T J.195.3], [b-ITU-T J.196.3]

NIT	Network Information Table				[b-ITU-T J.94], [b-ITU-T J.183], [b-ITU-T J.281], [b-ITU-T J.282], [b-ITU-T J.292], [b-ITU-T J.296], [b-ITU-T J.700]

NIU  	Network Interface Unit					[b-ITU-T J.112], [b-ITU-T J.112 Annex A], [b-ITU-T J.114], [b-ITU-T J.115], [b-ITU-T J.116], [b-ITU-T J.118], [b-ITU-T J.290]

NLS	    		Network Layer Signalling 				[b-ITU-T J.362]

NLS-TL		Network Layer Signalling Transport Layer 	[b-ITU-T J.362]

NM	    		Noise Margin						[b-ITU-T J.142]

NMC			Network Management Centre (for conditional access)		[b-ITU-T J.91]

NMS  	Network Management System			[b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.1], [b-ITU-T J.222.3], [b-ITU-T J.223.1], [b-ITU-T J.223.2], [b-ITU-T J.360], [b-ITU-T J.703]

No			Noise power normalized to 1 Hz 			[b-ITU-T J.142]

NPT			Normal Play Time					[b-ITU-T J.120], [b-ITU-T J.700]

NR	No (or zero) Reference					[b-ITU-T J.244], [b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342], [b-ITU-T J.343], [b-ITU-T J.343.1], [b-ITU-T J.343.2]

NS			Authoritative Name Server				[b-ITU-T J.192]

NSAP	Network Service Access Point			[b-ITU-T J.111], [b-ITU-T J.112], [b-ITU-T J.112 Annex A], [b-ITU-T J.116]

NSI	Network Service Interface				[b-ITU-T J.1101], [b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1104]

NSI	Network Side Interface					[b-ITU-T J.222.2], [b-ITU-T J.223.2]

NT			Network Termination					[b-ITU-T J.282]

NTP	Network Time Protocol				[b-ITU-T J.171.1], [b-ITU-T J.171.2], [b-ITU-T J.280], [b-ITU-T J.287], [b-ITU-T J.702]

NTSC RF		National Television System Committee Radio Frequency		[b-ITU-T J.197]

NTU			Network Termination Unit				[b-ITU-T J.114]

NVOD		Near Video on Demand				[b-ITU-T J.700]

NVRAM		Non Volatile Random Access Memory		[b-ITU-T J.170], [b-ITU-T J.296]

NW 	 		Network							[b-ITU-T J.702]

NWK		Network							[b-ITU-T J.114]

º	    		Degrees Celsius	    					[b-ITU-T J.142]

O/E	    		Optical to Electrical 					[b-ITU-T J.186]

OAM	Operation, Administration and Maintenance  	[b-ITU-T J.196.1], [b-ITU-T J.223.2]

OATC	Open Authentication Technology Committee	[b-ITU-T J.181 Amendment 1]

OCAP	Open Cable Applications Platform	         [b-ITU-T J.126], [b-ITU-T J.128], [b-ITU-T J.181 Amendment 1], [b-ITU-T J.200], [b-ITU-T J.215], [b-ITU-T J.222.1], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.294], [b-ITU-T J.700]

OCB 	Outbound Call Blocking	   			[b-ITU-T J.460.1], [b-ITU-T J.460.3]

OCSP		Online Certificate Status Protocol	  		[b-ITU-T J.222.3]

OCW			Odd control word					[b-ITU-T J.81]

OD  		Output Device  						[b-ITU-T J.702]

ODG  		Objective Difference Grade				[b-ITU-T J.145]

OEM			Original Equipment Manufacturer	  		[b-ITU-T J.290]

OFDM		Orthogonal Frequency Division Multiplex	[b-ITU-T J.142], [b-ITU-T J.382]

OFDM 	Orthogonal Frequency Division Multiplexing	[b-ITU-T J.185], [b-ITU-T J.195.1], [b-ITU-T J.195.2], [b-ITU-T J.196.1], [b-ITU-T J.196.2], [b-ITU-T J.293], [b-ITU-T J.296]

OFDMA	Orthogonal Frequency Division Multiple Access		[b-ITU-T J.196.1], [b-ITU-T J.196.2], [b-ITU-T J.196.3]

OH	OverHead							[b-ITU-T J.112 Annex A], [b-ITU-T J.116]

OID	    		Object Identification					[b-ITU-T J.170]

OID 	Object Identifier 					[b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.800.2]

OID 	Object ID							[b-ITU-T J.218]

OID 	Object Identifier. The sequence of integer positive numbers uniquely identifying the position of each MIB Object in the MIB Hierarchy		[b-ITU-T J.370]

OIPF	    		Open IPTV Forum  					[b-ITU-T J.296]

OLT	Optical Line Terminal					[b-ITU-T J.195.1], [b-ITU-T J.223.1], [b-ITU-T J.223.2], [b-ITU-T J.281], [b-ITU-T J.292], [b-ITU-T J.1106], [b-ITU-T J.1107]

OMA			Open Mobile Alliance					[b-ITU-T J.367], [b-ITU-T J.700]

OMA	 		Open Mobile Access					[b-ITU-T J.1010]

OMI	  		Optical Modulation Index  				[b-ITU-T J.185]

OMI   		Operation and Management Interface		[b-ITU-T J.223.2]

OMUX	Optical Multiplexer	        			[b-ITU-T J.1106], [b-ITU-T J.1107]

ONT	    		Optical Network Terminal  				[b-ITU-T J.186]

ONT   		Optical Network Termination   			[b-ITU-T J.190]

ONU	Optical Network Unit					[b-ITU-T J.195.1], [b-ITU-T J.223.2], [b-ITU-T J.281], [b-ITU-T J.282], [b-ITU-T J.294], [b-ITU-T J.700]

OOB	Out of Band 						[b-ITU-T J.112], [b-ITU-T J.116], [b-ITU-T J.118], [b-ITU-T J.128], [b-ITU-T J.197]

OOB	Out-of-Band						[b-ITU-T J.112 Annex A], [b-ITU-T J.700]

OoF   	  	Out of Frame						[b-ITU-T J.214]

OpenGL		Open Graphics Library					[b-ITU-T J.296]

OpenGL ES	Open Graphics Library for Embedded Systems	[b-ITU-T J.296]

OPF	  		Outbound Packet Filter					[b-ITU-T J.192]

OR	  		Outlier Ratio						[b-ITU-T J.249]

ORO	  		Option Request Option					[b-ITU-T J.222.2]

OS	Operating System					[b-ITU-T J.200], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.701], [b-ITU-T J.1011]

OS	Operational System					[b-ITU-T J.230]

OSD	On-Screen Display					[b-ITU-T J.117], [b-ITU-T J.151], [b-ITU-T J.206], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.1011]

OSGi			Open Services Gateway Initiative			[b-ITU-T J.294]

OSI	Open Systems Interconnection			[b-ITU-T J.111], [b-ITU-T J.112], [b-ITU-T J.112 Annex A], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.114], [b-ITU-T J.115], [b-ITU-T J.116], [b-ITU-T J.118], [b-ITU-T J.122], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.287]

OSPF		Open Shortest Path First				[b-ITU-T J.292]

OSPS		Operator Services Positioning System		[b-ITU-T J.178]

OSS	Operation Support System				[b-ITU-T J.160], [b-ITU-T J.164], [b-ITU-T J.172], [b-ITU-T J.177], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.193], [b-ITU-T J.222.3], [b-ITU-T J.290], [b-ITU-T J.294], [b-ITU-T J.700], [b-ITU-T J.704], [b-ITU-T J.705]

OSS	Operations Support Systems. The back-office software used for configuration, performance, fault, accounting, and security management.		[b-ITU-T J.170]

OSSI	Operations Support System Interface		[b-ITU-T J.210], [b-ITU-T J.212], [b-ITU-T J.460.2], [b-ITU-T J.800.2]

OSSI	   		Operations System Support Interface		[b-ITU-T J.222.2]

OT	   		Orthogonal Transform 					[b-ITU-T J.240]

OTT 		Over-The-Top						[b-ITU-T J.1011]

OTT 		Over The Top (over the open Internet)		[b-ITU-T J.1010]

OUI	Organizationally Unique Identifier 		[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.296], [b-ITU-T J.800.1]

OUI 	Organization Unique Identifier			[b-ITU-T J.116], [b-ITU-T J.117], [b-ITU-T J.122], [b-ITU-T J.218]

P0			Profile Zero						[b-ITU-T J.388]

PAC			Provisioning, Activation and Configuration element (PAC element)

[b-ITU-T J.360]

PACM		Provisioning, Activation, Configuration, and Maintenance		[b-ITU-T J.293]

PACM 	Provisioning, Activation, Configuration, and Management		[b-ITU-T J.360], [b-ITU-T J.705]

PAL	Phase Alternating Line					[b-ITU-T J.185], [b-ITU-T J.222.1]

PAL			Phase Alternating Line standard (50 Hz TV)	[b-ITU-T J.244]

PAL	Phase Alternate Line					[b-ITU-T J.81], [b-ITU-T J.197], [b-ITU-T J.702]

PAL	Phase Alternate Line. (The European colour television format that evolved from the American NTSC standard.)			 [b-ITU-T J.161]

PAM			IPCablecom Application Manager			[b-ITU-T J.360]

PAM			IPCablecom2 Application Manager		[b-ITU-T J.368]

PAMS		Provisioning and Alarm Management System	[b-ITU-T J.287]

PAP			Password Authentication Protocol			[b-ITU-T J.111]

PAPR		Peak to Average Power Ratio			[b-ITU-T J.382]

PAR	  		Peak to Average Ratio					[b-ITU-T J.222.1]

PAT	Program Association Table				[b-ITU-T J.94], [b-ITU-T J.151], [b-ITU-T J.200], [b-ITU-T J.280],

PAT	Programme Association Table			[b-ITU-T J.117], [b-ITU-T J.700]

PBP	  		Personal Basic Profile					[b-ITU-T J.700]

PC	  		Phase Compensation					[b-ITU-T J.88]

PC 	Personal Computer					[b-ITU-T J.190], [b-ITU-T J.282], [b-ITU-T J.290], [b-ITU-T J.296]

PCC	  		Policy and Charging Control				[b-ITU-T J.368]

PCEF	  	Policy and Charging Enforcement Function	[b-ITU-T J.368]

PCI	  		Peripheral Component Interface			[b-ITU-T J.197]

PCM	Pulse Code Modulation	  			[b-ITU-T J.21], [b-ITU-T J.161], [b-ITU-T J.177], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.361], [b-ITU-T J.700]

PCMM		PacketCable Multimedia				[b-ITU-T J.700]

PCR	Programme Clock Reference				[b-ITU-T J.82]

PCR	Program Clock Reference				[b-ITU-T J.133], [b-ITU-T J.181], [b-ITU-T J.200], [b-ITU-T J.211], [b-ITU-T J.280], [b-ITU-T J.292]

PCR 	Program Clock Reference. A time stamp in the Video Transport Stream from which decoder timing is derived.			[b-ITU-T J.212]

PCR 	    	Plug Control Register	    				[b-ITU-T J.117]

PCRF		Policy Control and Charging Rules Function	[b-ITU-T J.263]

PCRF 		Policy and Charging Rules Function		[b-ITU-T J.368]

P-CSCF	Proxy- Call Session Control Function (CSCF)	[b-ITU-T J.261], [b-ITU-T J.262], [b-ITU-T J.263], [b-ITU-T J.360], [b-ITU-T J.363], [b-ITU-T J.365], [b-ITU-T J.367], [b-ITU-T J.368], [b-ITU-T J.388], [b-ITU-T J.460.1], [b-ITU-T J.460.3], [b-ITU-T J.700]

PCSP			IPCablecom CMS Subscriber Provisioning	[b-ITU-T J.177]

PD			Photo Detector						[b-ITU-T J.186]

Pd 	downlink Probe						[b-ITU-T J.195.2], [b-ITU-T J.195.3], [b-ITU-T J.196.2], [b-ITU-T J.196.3]

PD 			Peripheral Device					[b-ITU-T J.702]

PDA			Personal Digital Assistant				[b-ITU-T J.246], [b-ITU-T J.247]

PDCR		Prime Differential Clock Recovery			[b-ITU-T J.214]

PDH	Plesiochronous Digital Hierarchy			[b-ITU-T J.81], [b-ITU-T J.83], [b-ITU-T J.131]

PDP			Policy Decision Point (defined in RFC 2753)	[b-ITU-T J.179]

PDS			Profile Delivery Server					 [b-ITU-T J.360]

PDU	Protocol Data Unit					[b-ITU-T J.82], [b-ITU-T J.131], [b-ITU-T J.132], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.195.1], [b-ITU-T J.195.3], [b-ITU-T J.196.3], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.1102], [b-ITU-T J.1103]

PDU 		Packet Data Unit						[b-ITU-T J.700]

PE			Parameter Element					[b-ITU-T J.196.3]

PEAQ		Perceived Audio Quality				[b-ITU-T J.145]

PEG			Public, Education and Government access	[b-ITU-T J.700]

PEP			Policy Enforcement Point (defined in RFC 2753)		[b-ITU-T J.179]

PER			Packet Error Rate					[b-ITU-T J.222.2]

PES	Packetized Elementary bitStream			[b-ITU-T J.343], [b-ITU-T J.343.1], [b-ITU-T J.343.2], [b-ITU-T J.343.3], [b-ITU-T J.343.4], [b-ITU-T J.343.5], [b-ITU-T J.343.6]

PES 	Packetized Elementary Stream			[b-ITU-T J.94], [b-ITU-T J.181], [b-ITU-T J.200], [b-ITU-T J.281], [b-ITU-T J.282], [b-ITU-T J.286], [b-ITU-T J.293], [b-ITU-T J.603], [b-ITU-T J.700]

PES 			Program Elementary Stream				[b-ITU-T J.151]

PEVQ-S		Perceptual Evaluation of Video Quality for Streaming

[b-ITU-T J.343.5], [b-ITU-T J.343.6]

PF 			Platform						[b-ITU-T J.1005], [b-ITU-T J.1006]

PF 			Packet Filter					[b-ITU-T J.192]

PHL			Physical						[b-ITU-T J.114]

PHS	Payload Header Suppression			[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.163], [b-ITU-T J.222.2], [b-ITU-T J.222.3]

PHSF		Payload Header Suppression Field		[b-ITU-T J.222.2]

PHSI			Payload Header Suppression Index		[b-ITU-T J.222.2]

PHSM		Payload Header Suppression Mask		[b-ITU-T J.222.2]

PHSR		Payload Header Suppression Rule		[b-ITU-T J.222.2]

PHSS		Payload Header Suppression Size		[b-ITU-T J.222.2]

PHSV		Payload Header Suppression Verify	[b-ITU-T J.222.2]

PHY			Physical						[b-ITU-T J.196.1]

PHY 	Physical layer					[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.190], [b-ITU-T J.195.1], [b-ITU-T J.210], [b-ITU-T J.212], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.223.1], [b-ITU-T J.223.2], [b-ITU-T J.281], [b-ITU-T J.290], [b-ITU-T J.381], [b-ITU-T J.382], [b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1104]

PHY 	Physical Layer (Used to refer to the downstream QAM transmitters and the upstream burst demodulators (receiver)	[b-ITU-T J.211]

PID	Packet Identification number			[b-ITU-T J.96]

PID 	Packet Identifier					[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.181], [b-ITU-T J.183], [b-ITU-T J.200], [b-ITU-T J.222.2], [b-ITU-T J.280], [b-ITU-T J.286], [b-ITU-T J.287], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.292], [b-ITU-T J.702], [b-ITU-T J.1102], [b-ITU-T J.1103]

PID 	Packet Identifier, defined by ISO/IEC 13818 (MPEG-2) [b-ITU-T J.112 Annex A]

PID 			Packet Identifier used in MPEG-TS	[b-ITU-T J.211]

PID 	Packet Identifier; PID (system): A unique integer value used to identify elementary streams of a program in a single or multi-program Transport Stream as described in 2.4.3 of [H.222.0 | ISO 13818-1] 				[b-ITU-T J.212]

PID			Packet Identifier Field				[b-ITU-T J.343.6]

PID 			Program Identifier				[b-ITU-T J.117], [b-ITU-T J.151]

PIDF			Presence Information Data Format		[b-ITU-T J.367]

PIDF-LO 		Presence Information Data Format Location Object		[b-ITU-T J.460.1]

PIM			Protocol Independent Multicast		[b-ITU-T J.222.2]

PIMSM		Protocol Independent Multicast Sparse Mode				[b-ITU-T J.292]

PIN	Personal Identification Number		[b-ITU-T J.260], [b-ITU-T J.261], [b-ITU-T J.262], [b-ITU-T J.263], [b-ITU-T J.296], [b-ITU-T J.702], [b-ITU-T J.1011]

PING			Packet Inter-Network Grouper		[b-ITU-T J.191], [b-ITU-T J.192]

PIN-PD		p-i-n Photo Diode				[b-ITU-T J.185]

PIP			Picture-In-Picture				[b-ITU-T J.117], [b-ITU-T J.700]

PKCROSS	Utilizes PKINIT for establishing the inter-realm keys and associated inter-realm policies to be applied in issuing cross-realm service tickets between realms and domains in support of Intradomain and Interdomain CMS-to-CMS signalling (CMSS).						[b-ITU-T J.170]

PKCS	Public Key Cryptography Standards	 	[b-ITU-T J.170]

PKI	Public Key Infrastructure	 		[b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.3], [b-ITU-T J.261]

PKI 	Public Key Infrastructure. A process for issuing public key certificates, which includes standards, Certification Authorities, communication between authorities and protocols for managing certification processes. 			[b-ITU-T J.170]

PKINIT	Public Key Cryptography for Initial Authentication	   	 	[b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.261]

PL			Path Layer						[b-ITU-T J.131]

PLC			Power Line Communication			[b-ITU-T J.294]

PLC 			Packet Loss Concealment			[b-ITU-T J.361]

PLL	Phase Locked Loop				[b-ITU-T J.81], [b-ITU-T J.211], [b-ITU-T J.222.1]

PLM			Payload Label Mismatch			[b-ITU-T J.132]

PLP			Physical Layer Pipe				[b-ITU-T J.382]

PLR			Packet Loss Ratio				[b-ITU-T J.241]

PM			Primary Mark					[b-ITU-T J.95]

PM 	Pulse Modulation					[b-ITU-T J.112], [b-ITU-T J.112 Annex A], [b-ITU-T J.116]

PM 			Phase Modulation				[b-ITU-T J.116]

PM 			Programme Material				[b-ITU-T J.285]

PMD	Physical Media Dependent sublayer	[b-ITU-T J.112], [b-ITU-T J.116], [b-ITU-T J.212], [b-ITU-T J.222.1], [b-ITU-T J.222.2]

PMD	Physical Media Dependent			[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]

PMT	Program Map Table				[b-ITU-T J.94], [b-ITU-T J.117], [b-ITU-T J.151], [b-ITU-T J.200], [b-ITU-T J.215], [b-ITU-T J.280], [b-ITU-T J.287], [b-ITU-T J.302]

PMT	Program Map Table (see [ITU-T H.222.0])				[b-ITU-T J.181]

PMT	Programme Map Table				[b-ITU-T J.96], [b-ITU-T J.700]

PN 			Pseudo random Noise 				[b-ITU-T J.83]

PNG			Portable Network Graphics			[b-ITU-T J.200], [b-ITU-T J.700]

PoC			Push-to-Talk over Cellular			[b-ITU-T J.367]

PODM 		Placement Opportunity Data Model	[b-ITU-T J.380.3]

POH 		Path Overhead 					[b-ITU-T J.132]

POIS	Opportunity Information Service		[b-ITU-T J.380.1], [b-ITU-T J.380.2], [b-ITU-T J.380.5], [b-ITU-T J.706]

PON	Passive Optical Network			[b-ITU-T J.185], [b-ITU-T J.195.1], [b-ITU-T J.196.1], [b-ITU-T J.223.1], [b-ITU-T J.223.2], [b-ITU-T J.297], [b-ITU-T J.700], [b-ITU-T J.1107]

POTS		Plain Old Telephone Service			[b-ITU-T J.173], [b-ITU-T J.460.2]

PP 	    		Portable Part 					[b-ITU-T J.114]

PPD	    		Pay Per Day					[b-ITU-T J.296]

PPI			Phase parity identifier				[b-ITU-T J.81]

PPI 		    	PDH Physical Interface 			[b-ITU-T J.131]

ppm	    		Parts per million					[b-ITU-T J.83], [b-ITU-T J.210]

PPP	Point-to-Point Protocol				[b-ITU-T J.111], [b-ITU-T J.114], [b-ITU-T J.200], [b-ITU-T J.212], [b-ITU-T J.287]

PPS	    		Picture Parameter Set				[b-ITU-T J.286]

PPT 			PDH Path Termination 			[b-ITU-T J.131]

PPV	Pay Per View					[b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.704], [b-ITU-T J.705]

PRACK		Provisional Response ACKnowledgement			  	[b-ITU-T J.263]

PRACK 		Provisional Acknowledgement		[b-ITU-T J.460.1]

PRBS		Pseudo Random Binary Sequence		[b-ITU-T J.83], [b-ITU-T J.142]

PRF	    		Pseudo Random number generation Function

[b-ITU-T J.1003], [b-ITU-T J.1004]

PRG			Pseudo-random (sequence) generator	[b-ITU-T J.81]

PRG 		Pseudo-Random (digital sequence) Generator 				[b-ITU-T J.91]

PRNG	Pseudo Random Number Generator	[b-ITU-T J.1003], [b-ITU-T J.1004]

PRNG 	Pseudo-Random Number Generator	[b-ITU-T J.112 Annex A]

Prop Trans		Proprietary Translated Address Realm	[b-ITU-T J.190]

PRS	    		Primary Reference Source			[b-ITU-T J.222.1]

PS	Portal Service					[b-ITU-T J.126], [b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.290], [b-ITU-T J.800.2]

PS 			Program(me) Segment				[b-ITU-T J.140], [b-ITU-T J.245]

PS 			Provisioning Server				[b-ITU-T J.177]

PS 	Policy Server					[b-ITU-T J.179], [b-ITU-T J.362], [b-ITU-T J.363], [b-ITU-T J.365], [b-ITU-T J.368]

PS 	 		Program Stream					[b-ITU-T J.296]

PS WAN-Data	Portal Service element WAN data interface				[b-ITU-T J.191]

PS WAN-Data 	IPCable2Home Portal Services element WAN data interface

[b-ITU-T J.192]

PS WAN-Man	Portal Service element WAN management interface			[b-ITU-T J.191]

PS WAN-Man  	IPCable2Home Portal Services element WAN management interface

[b-ITU-T J.192]

PSAP 		Public Safety Answering Point – the entity that answers 9-1-1 calls

[b-ITU-T J.460.1]

PSD			Power Spectral Density				[b-ITU-T J.195.2], [b-ITU-T J.196.2]

PSI 	Program/program-/proramme Specific Information 	[b-ITU-T J.94], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.142], [b-ITU-T J.212], [b-ITU-T J.280], [b-ITU-T J.281] [b-ITU-T J.282], [b-ITU-T J.293], [b-ITU-T J.297], [b-ITU-T J.302], [b-ITU-T J.700], [b-ITU-T J.900]

PSI 			Public Service Identity				[b-ITU-T J.360]

PSI 			Pairing Status Information			[b-ITU-T J.1002]

PSIP	Program and System Information Protocol	[b-ITU-T J.117], [b-ITU-T J.151], [b-ITU-T J.200], [b-ITU-T J.302]

PSK	Phase Shift Keying				[b-ITU-T J.116], [b-ITU-T J.142], [b-ITU-T J.185]

PSL 			Physical Section Layer 			[b-ITU-T J.131]

PSNR	Peak Signal-to-Noise Ratio			[b-ITU-T J.147], [b-ITU-T J.240], [b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.340], [b-ITU-T J.341], [b-ITU-T J.342], [b-ITU-T J.343]

PSNRconst 	Peak Signal to Noise Ratio with Compensation for Constant Spatial Shifts, Constant Temporal Shift, and Constant Luminance Gain and Offset

[b-ITU-T J.340]

PSP			Packet Streaming Protocol			[b-ITU-T J.212]

PSPN 		Public Switched Packet Network 		[b-ITU-T J.91]

PSS			Policy Sub-System				[b-ITU-T J.1020]

PSTN	Public Switched Telephone Network	[b-ITU-T J.111], [b-ITU-T J.112 Annex A], [b-ITU-T J.114], [b-ITU-T J.115], [b-ITU-T J.116], [b-ITU-T J118], [b-ITU-T J.160], [b-ITU-T J.161], [b-ITU-T J.162], [b-ITU-T J.163], [b-ITU-T J.164], [b-ITU-T J.167], [b-ITU-T J.170], [b-ITU-T J.171.2], [b-ITU-T J.173], [b-ITU-T J.175], [b-ITU-T J.177], [b-ITU-T J.179], [b-ITU-T J.260], [b-ITU-T J.261], [b-ITU-T J.262], [b-ITU-T J.263], [b-ITU-T J.360], [b-ITU-T J.361], [b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.460.2], [b-ITU-T J.700]

PT 	    		Payload Type 					[b-ITU-T J.132]

PTC	   		Physical Transmission Channel		[b-ITU-T J.200]

PTS	Presentation Time Stamp (see [ITU-T H.222.0])	 [b-ITU-T J.94], [b-ITU-T J.181], [b-ITU-T J.200], [b-ITU-T J.287], [b-ITU-T J.343.5], [b-ITU-T J.343.6]

Pu	uplink Probe					[b-ITU-T J.195.2], [b-ITU-T J.195.3], [b-ITU-T J.196.2], [b-ITU-T J.196.3]

PUSI	Payload Unit Start Indicator			[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.211], [b-ITU-T J.222.2], [b-ITU-T J.1103]

PVD 		Preferred Viewing Distance 			[b-ITU-T J.140]

PVR	Personal Video Recorder			[b-ITU-T J.181 Amendment 1], [b-ITU-T J.205], [b-ITU-T J.206], [b-ITU-T J.296], [b-ITU-T J.700], [b-ITU-T J.701], [b-ITU-T J.702], [b-ITU-T J.703], [b-ITU-T J.1010], [b-ITU-T J.1011]

PVR 		Personal Video Recording			[b-ITU-T J.193]

PVS	Processed Video Sequence			[b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342], [b-ITU-T J.343], [b-ITU-T J.343.1], [b-ITU-T J.343.2], [b-ITU-T J.343.3], [b-ITU-T J.343.4], [b-ITU-T J.343.5], [b-ITU-T J.343.6]

PW			Pseudowire						[b-ITU-T J.212]

PW 			Pseudo Wire					[b-ITU-T J.214], [b-ITU-T J.800.2]

Q			Quadrature modulation component		[b-ITU-T J.210], [b-ITU-T J.222.1]

QAM	Quadrature Amplitude Modulation		[b-ITU-T J.83], [b-ITU-T J.112 Annex A], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.118], [b-ITU-T J.122], [b-ITU-T J.142], [b-ITU-T J.150], [b-ITU-T J.183], [b-ITU-T J.185], [b-ITU-T J.186], [b-ITU-T J.195.2], [b-ITU-T J.196.2], [b-ITU-T J.211], [b-ITU-T J.212], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.293], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.381], [b-ITU-T J.382], [b-ITU-T J.604], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1104], [b-ITU-T J.1105]

QBP	   		Quality of Service Boundary Point		[b-ITU-T J.192]

QCC	   		Quality of Service Characteristics Client	[b-ITU-T J.192]

QCF	  		QoS Client Function				[b-ITU-T J.190]

QCIF  	Quarter Common Intermediate Format	[b-ITU-T J.161], [b-ITU-T J.177], [b-ITU-T J.361]

QCIF  		Quarter Common Intermediate Format (176 x 144 pixels)

[b-ITU-T J.246], [b-ITU-T J.247]

QCS	   		Quality of Service Characteristics Server				[b-ITU-T J.192]

QEF	   		Quasi Error Free					[b-ITU-T J.142], [b-ITU-T J.83]

QFM  		Quality of Service Forwarding & Media Access			[b-ITU-T J.192]

QKE			Quick Key Exchange				[b-ITU-T J.112 Annex A]

QoE			Quality of Experience				[b-ITU-T J.702]

QoS	Quality of Service				[b-ITU-T J.82], [b-ITU-T J.112 Annex A], [b-ITU-T J.116], [b-ITU-T J.125], [b-ITU-T J.145], [b-ITU-T J.160], [b-ITU-T J.161], [b-ITU-T J.163], [b-ITU-T J.166], [b-ITU-T J.170], [b-ITU-T J.171.1], [b-ITU-T J.171.2], [b-ITU-T J.179], [b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.193], [b-ITU-T J.195.1], [b-ITU-T J.196.1], [b-ITU-T J.197], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.223.1], [b-ITU-T J.223.2], [b-ITU-T J.241], [b-ITU-T J.261], [b-ITU-T J.281], [b-ITU-T J.282], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.292], [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.340], [b-ITU-T J.360], [b-ITU-T J.361], [b-ITU-T J.362], [b-ITU-T J.365], [b-ITU-T J.367], [b-ITU-T J.382], [b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.703], [b-ITU-T J.704], [b-ITU-T J.705], [b-ITU-T J.800.2], [b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1104]

QP			Quality Parameter				[b-ITU-T J.140], [b-ITU-T J.245]

QP 			Quantization Parameter			[b-ITU-T J.343.2], [b-ITU-T J.343.4]

QPF	  		QoS Portal Function				[b-ITU-T J.190]

QPSK	Quadrature Phase-Shift Keying		[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.185], [b-ITU-T J.186], [b-ITU-T J.195.2], [b-ITU-T J.196.2], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.290], [b-ITU-T J.700]

QPSK	Quaternary Phase Shift Keying		[b-ITU-T J.84], [b-ITU-T J.112 Annex A], [b-ITU-T J.116], [b-ITU-T J.118], [b-ITU-T J.142], [b-ITU-T J.150],

QR 			Quick Response					[b-ITU-T J.230]

QSF			QoS Server Function				[b-ITU-T J.190]

RA			Radio Adaptation					[b-ITU-T J.11]

RA 			Repeater Amplifier				[b-ITU-T J.185]

RA 			Receiver Amplifier				[b-ITU-T J.186]

RA 	Router Advertisement				[b-ITU-T J.218], [b-ITU-T J.222.2], [b-ITU-T J.282]

RACF  	Remote Activation of Call Forwarding	[b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.460.3]

RACF 		Resource Admission Control Functions	[b-ITU-T J.700], [b-ITU-T J.702]

RADIUS	Remote Authentication Dial-In User Service				[b-ITU-T J.170], [b-ITU-T J.363]

RADIUS	Remote Authentication Dial-In User Service (defined in RFC 2138 and RFC 2139) 					[b-ITU-T J.179]

RAI			Remote Alarm Indication			[b-ITU-T J.214]

RAL 		Resource Abstraction Layer			[b-ITU-T J.701], [b-ITU-T J.702]

RAM	Random Access Memory			[b-ITU-T J.192], [b-ITU-T J.296], [b-ITU-T J.700], [b-ITU-T J.701], [b-ITU-T J.702]

RAN			Regional Area Network			[b-ITU-T J.290]

RAP 		Radio in the local loop Access Profile 	[b-ITU-T J.114]

RAP 		Resource Allocation Protocol		[b-ITU-T J.163]

RAP 	Resource Allocation Protocol (Working Group in the IETF – responsible for the definition and maintenance of the COPS protocol)			[b-ITU-T J.179]

RAS			Registration, Admissions and Status	[b-ITU-T J.177]

RBSP		Raw Byte Sequence Payload			[b-ITU-T J.286]

RC 			Reason Code 					[b-ITU-T J.296]

RC4	A variable key length stream cipher offered in the ciphersuite, used to encrypt the media traffic in IPCablecom.			[b-ITU-T J.170]

RCA			Radio Corporation of America		[b-ITU-T J.296]

RCAS	Renewable Conditional Access System	[b-ITU-T J.1002], [b-ITU-T J.1003], [b-ITU-T J.1004], [b-ITU-T J.1020]

RCC  	Receive Channel Configuration(s)		[b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.800.0]

RCD			Resource Control Domain			[b-ITU-T J.179]

RCID		Receive Channel Identifier			[b-ITU-T J.222.2]

RCP	Receive Channel Profile			[b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.800.0]

RCP-ID		Receive Channel Profile Identifier		[b-ITU-T J.222.2]

RCS			Receive Channel Set				[b-ITU-T J.222.2]

RCU	Remote Control Unit				[b-ITU-T J.230], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.297]

RD			Router Discovery					[b-ITU-T J.218]

RDA			Rate-Determination Algorithm		[b-ITU-T J.361]

RDI 			Remote Defect Indication 			[b-ITU-T J.131], [b-ITU-T J.132]

RDN			Relative Distinguished Name			[b-ITU-T J.192]

RE 			Reservation Element				[b-ITU-T J.195.3]

REG-REQ		Registration Request MAC Message	[b-ITU-T J.222.1]

REI 			Remote Error Indication 			[b-ITU-T J.132]

REQ 		Request indicator used in Annex B 	[b-ITU-T J.112], [b-ITU-T J.116]

Reservation Id	Reservation Identifier				[b-ITU-T J.112 Annex A]

RF	Radio Frequency					[b-ITU-T J.83], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.114], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.126], [b-ITU-T J.142], [b-ITU-T J.150], [b-ITU-T J.151], [b-ITU-T J.185], [b-ITU-T J.195.2], [b-ITU-T J.196.2], [b-ITU-T J.197], [b-ITU-T J.212], [b-ITU-T J.214], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.223.1], [b-ITU-T J.223.2], [b-ITU-T J.293], [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.381], [b-ITU-T J.604], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.1106], [b-ITU-T J.1107]

RF4CE		Radio Frequency for Consumer Electronics				[b-ITU-T J.296]

RFC	Request for Comments 			[b-ITU-T J.111], [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.177], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.193], [b-ITU-T J.212], [b-IU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.294], [b-ITU-T J.296], [b-ITU-T J.361], [b-ITU-T J.365]

RFC	Request for Comments (IETF standard)	 [b-ITU-T J.178]

RFC 	Request for Comments (technical policy documents approved by the IETF which are available at http://www.ietf.org/rfc.html) [b-ITU-T J.179]

RFC	Request for Comments. Technical policy documents approved by the IETF which are available on the World Wide Web at http://www.ietf.cnri.reston.va.us/rfc.html

[b-ITU-T J.370], [b-ITU-T J.460.4]

RFI	Remote Failure Indication			[b-ITU-T J.132]

RFI 	Radio Frequency Interface			[b-ITU-T J.170], [b-ITU-T J.212], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.223.1], [b-ITU-T J.223.2], [b-ITU-T J.460.4], [b-ITU-T J.800.2]

RFI 	Radio Frequency Interface (specification defining MAC and Physical layer interfaces between CMTS and CM network elements)		[b-ITU-T J.179]

RFoG	Radio Frequency(RF) over Glass		[b-ITU-T J.294], [b-ITU-T J.381], [b-ITU-T J.700]

RF(oG) 		Radio Frequency (over Glass)		[b-ITU-T J.382]

RG	Residential Gateway				[b-ITU-T J.192], [b-ITU-T J.193], [b-ITU-T J.293], [b-ITU-T J.294], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.703], [b-ITU-T J.705]

RGB			Red Green Blue					[b-ITU-T J.197], [b-ITU-T J.700]

RGW			Residential Gateway				[b-ITU-T J.292]

RIN			Relative Intensity Noise			[b-ITU-T J.185], [b-ITU-T J.186]

RIP			Relative Interaction Path			[b-ITU-T J.116]

RIP 	Routing Information Protocol		[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]

RJ			Registered Jack					[b-ITU-T J.296]

RJ-11		Registered Jack-11				[b-ITU-T J.161]

RKS	Record Keeping Server				[b-ITU-T J.160], [b-ITU-T J.164], [b-ITU-T J.179], [b-ITU-T J.360], [b-ITU-T J.363]

RKS 	Record Keeping Server. The device which collects and correlates the various Event Messages.					[b-ITU-T J.170]

RLL 		Radio in the Local Loop 			[b-ITU-T J.114]

RLS			Resource List Server				[b-ITU-T J.367]

RM			Receive Module					[b-ITU-T J.222.1], [b-ITU-T J.222.2]

RMS	Root Mean Square				[b-ITU-T J.116], [b-ITU-T J.142], [b-ITU-T J.185], [b-ITU-T J.222.1]

RMS 		Rights Management System			[b-ITU-T J.290]

RMSE	Root Mean Square Error			[b-ITU-T J.149], [b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342], [b-ITU-T J.343]

RMS-FUS 	Remote Management and Firmware Update Services		[b-ITU-T J.705]

RNG 		Ranging 						[b-ITU-T J.112], [b-ITU-T J.116]

RNG-RSP		Ranging Response MAC Message		[b-ITU-T J.222.1]

ROH	Receiver-Off-Hook, also known as "howler tone"		[b-ITU-T J.460.0], [b-ITU-T J.460.1]

ROI			Region Of Interest				[b-ITU-T J.343.5], [b-ITU-T J.343.6]

RoIP 		Radio over Internet Protocol			[b-ITU-T J.1106]

RoIP			Radio over IP					[b-ITU-T J.1107]

ROM	Read-Only Memory				[b-ITU-T J.126], [b-ITU-T J.192], [b-ITU-T J.206], [b-ITU-T J.296], [b-ITU-T J.1011]

R-P			Resource Priority					[b-ITU-T J.263]

RPC	Remote Procedure Call				[b-ITU-T J.111], [b-ITU-T J.296], [b-ITU-T J.380.7], [b-ITU-T J.1101], [b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1104]

rpchof	Remainder polynomial coefficients, highest order first		[b-ITU-T J.181], [b-ITU-T J.183]

RR 			Radio Relay 					[b-ITU-T J.132]

RR 	Reduced Reference				[b-ITU-T J.244], [b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342], [b-ITU-T J.343], [b-ITU-T J.343.3], [b-ITU-T J.343.4],

RRNR-TV	Reduced Reference and No Reference objective models for standard definition TeleVision						[b-ITU-T J.249]

RS or R/S 	Reed-Solomon/Reed Solomon(coding)	[b-ITU-T J.82], [b-ITU-T J.83], [b-ITU-T J.116], [b-ITU-T J.131], [b-ITU-T J.132], [b-ITU-T J.142], [b-ITU-T J.222.1]

RS 			Router Solicitation				[b-ITU-T J.222.2]

RSA	An algorithm for public-key encryption invented by Ron Rivest, Adi Shamir and Len Adleman 					[b-ITU-T J.366.8]

RSA 	Public key cryptosystem developed by Rivest, Shamir, Adleman; also company by same name marketing public key technology			[b-ITU-T J.290]

RSA 	Rivest, Shamir, Adleman(a public key cryptographic algorithm)

[b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.2], [b-ITU-T J.222.3]

RSIP			Realm Specific IP				[b-ITU-T J.170]

RSOH 		Regenerator Section Overhead 		[b-ITU-T J.132]

RST 			Regenerator Section Termination 		[b-ITU-T J.132]

RST 	Residential SIP Telephony			[b-ITU-T J.460.0], [b-ITU-T J.460.1], b-ITU-T J.460.2], [b-ITU-T J.460.3], [b-ITU-T J.460.4], [b-ITU-T J.704], [b-ITU-T J.800.0]

RSVP	Resource reSerVation Protocol 		[b-ITU-T J.161], [b-ITU-T J.163], [b-ITU-T J.170], [b-ITU-T J.192], [b-ITU-T J.222.2], [b-ITU-T J.290], [b-ITU-T J.361], [b-ITU-T J.700]

RSVP	Resource ReSerVation Protocol (defined in RFC 2205)		[b-ITU-T J.179]

RSVP	Resource ReSerVation Protocol [b-IETF RFC 2210]			[b-ITU-T J.190]

RSVP 		Resource reSerVation setup Protocol	[b-ITU-T J.703]

RSVP+	IPCablecom profile and extension of RSVP (defined in the IPCablecom DQoS Recommendation)				[b-ITU-T J.179]

RTCP	Real Time Control Protocol			[b-ITU-T J.170], [b-ITU-T J.171.1], [b-ITU-T J.192], [b-ITU-T J.241], [b-ITU-T J.361], [b-ITU-T J.460.2]

RTCP	Real-time Transport Control Protocol	 [b-ITU-T J.120], [b-ITU-T J.161], [b-ITU-T J.460.1], [b-ITU-T J.700]

RTCP	RTP Control Protocol				[b-ITU-T J.121], [b-ITU-T J.171.2], [b-ITU-T J.360]

RTCP XR 		Real Time Control Protocol Extended Reports			[b-ITU-T J.460.2]

RTD 		Round Trip Delay 				[b-ITU-T J.112], [b-ITU-T J.116]

RTE			Real-time Encoder				[b-ITU-T J.700]

RTMP		Real Time Messaging Protocol		[b-ITU-T J.181 Amendment 1]

RTO	Retransmission Timeout			[b-ITU-T J.170], [b-ITU-T J.171.1], [b-ITU-T J.171.2]

RTP	Real-time Transport Protocol			 [b-ITU-T J.120], [b-ITU-T J.121], [b-ITU-T J.161], [b-ITU-T J.171.2], [b-ITU-T J.192], [b-ITU-T J.193], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.261], [b-ITU-T J.263], [b-ITU-T J.281], [b-ITU-T J.282], [b-ITU-T J.293], [b-ITU-T J.294], [b-ITU-T J.360], [b-ITU-T J.361], [b-ITU-T J.388],[b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.700], [b-ITU-T J.702]

RTP 	Real Time Protocol				[b-ITU-T J.111], [b-ITU-T J.162], [b-ITU-T J.170], [b-ITU-T J.171.1], [b-ITU-T J.175], [b-ITU-T J.205], [b-ITU-T J.206], [b-ITU-T J.241], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.292], [b-ITU-T J.703]

RTP 			Real-Time Transfer Protocol			[b-ITU-T J.160]

RTS			Request to Send					[b-ITU-T J.118]

RTSP	Real Time Streaming Protocol		[b-ITU-T J.120], [b-ITU-T J.205], [b-ITU-T J.206], [b-ITU-T J.290], [b-ITU-T J.292], [b-ITU-T J.294], [b-ITU-T J.700], [b-ITU-T J.701], [b-ITU-T J.702], [b-ITU-T J.703]

RTT 		Rating Text Table 				[b-ITU-T J.94]

Ru			Report up						[b-ITU-T J.196.2]

Ru 			uplink Report					[b-ITU-T J.196.3]

Rx			Receiver						[b-ITU-T J.381]

S/N			Signal to Noise ratio				[b-ITU-T J.142]

SA	Security Association				[b-ITU-T J.125], [b-ITU-T J.170], [b-ITU-T J.222.2], [b-ITU-T J.222.3]

SA 	Source Address					[b-ITU-T J.160], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.1102], [b-ITU-T J.1103]

SAA			Server-Assignment-Answer			[b-ITU-T J.360 Amendment 1]

SAC			Secure Authenticated Channel		[b-ITU-T J.293]

SAC			Selectable Active Codes			[b-ITU-T J.222.1], [b-ITU-T J.222.2]

SADS 		Service and Application Discovery and Selection			[b-ITU-T J.702]

SAFE		Service/Application Functional Entity	[b-ITU-T J.126]

SAID	Security Association Identifier		[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.125], [b-ITU-T J.213], [b-ITU-T J.222.2], [b-ITU-T J.222.3]

SAP	Service Access Point				[b-ITU-T J.82], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.131], [b-ITU-T J.132], [b-ITU-T J.195.1]

SAR 	Segmentation and Reassembly sublayer 	[b-ITU-T J.82], [b-ITU-T J.131], [b-ITU-T J.132]

SAR			Server-Assignment-Request			[b-ITU-T J.360 Amendment 1]

SAV			Source Address Verification			[b-ITU-T J.222.2], [b-ITU-T J.222.3]

SBM			Subnet Bandwidth Manager			[b-ITU-T J.190]

SBR			Spectral Band Replication			[b-ITU-T J.388]

SBS			Stimulated Brillouin Scattering		[b-ITU-T J.185]

SC 			Sequence Count 					[b-ITU-T J.82], [b-ITU-T J.132]

SC 			Sub-Channel					[b-ITU-T J.196.2], [b-ITU-T J.196.3]

SC 			SID_Cluster					[b-ITU-T J.222.2]

SC 			Service Compatible				[b-ITU-T J.900]

SCA 		Selective Call Acceptance			[b-ITU-T J.460.0]

SCB 		Solicitor Call Blocking				[b-ITU-T J.460.3]

SCCCN		L2TPv3 Start-Control-Connection-Connected message		[b-ITU-T J.212]

SCCP		Signalling Connection Control Part		[b-ITU-T J.160]

SCCRP 		L2TPv3 Start-Control-Connection-Reply message			[b-ITU-T J.212]

SCCRQ		L2TPv3 Start-Control-Connection-Request message			[b-ITU-T J.212]

SCD			Session Control Domain			[b-ITU-T J.179]

S-CDMA	Synchronous Code Division Multiple Access	[b-ITU-T J.210], [b-ITU-T J.211], [b-ITU-T J.212], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.223.1]

SCF			Security Client Function			[b-ITU-T J.190]

SCF 			Selective Call Forwarding			[b-ITU-T J.460.0], [b-ITU-T J.460.1]

SCF 			Service Control Function			[b-ITU-T J.700]

SCG_Ru		Sub-Carrier Group for Ru frame		[b-ITU-T J.196.2], [b-ITU-T J.196.3]

SCI 			IP-based SDV Control Interface		[b-ITU-T J.1102]

SCM			Sub-Carrier Multiplexing			[b-ITU-T J.185], [b-ITU-T J.186]

SCMS		Serial Copy Management System		[b-ITU-T J.296]

SCP	Service and Content Protection		[b-ITU-T J.296], [b-ITU-T J.701], [b-ITU-T J.702], [b-ITU-T J.704], [b-ITU-T J.705]

SCR 		Selective Call Rejection			[b-ITU-T J.460.0]

S-CSCF	Serving- Call Session Control Function	[b-ITU-T J.262], [b-ITU-T J.360], [b-ITU-T J.363], [b-ITU-T J.388], [b-ITU-T J.460.1], [b-ITU-T J.460.3], [b-ITU-T J.700]

SD	Standard Definition				[b-ITU-T J.181 Amendment 1], [b-ITU-T J.196.1], [b-ITU-T J.291], [b-ITU-T J.296], [b-ITU-T J.361], [b-ITU-T J.381], [b-ITU-T J.700], [b-ITU-T J.1006]

SD 			Secure Digital					[b-ITU-T J.290]

SD 			Service Discovery				[b-ITU-T J.702]

SD 	Secondary Device				[ITU-T J.1020]

SD&S		Service Discovery and Selection		[b-ITU-T J.700]

SD/HD 		TV Signal Degrade/High Definition Television		[b-ITU-T J.195.1]

SDH 		Synchronous Digital Hierarchy 		[b-ITU-T J.81], [b-ITU-T J.132]

SDI			Serial Digital Interface				[b-ITU-T J.287]

SDI 			IP-based SDV Data Interface			[b-ITU-T J.1102]

SDK			Software Development Kit			[b-ITU-T J.295], [b-ITU-T J.1005]

SDL	Specification and Description Language	[b-ITU-T J.195.3], [b-ITU-T J.196.3], [b-ITU-T J.222.2]

SDP	Session Description Protocol	defined by [RFC 4566]		[b-ITU-T J.120], [b-ITU-T J.161], [b-ITU-T J.162], [b-ITU-T J.170], [b-ITU-T J.171.1], [b-ITU-T J.171.2], [b-ITU-T J.175], [b-ITU-T J.177], [b-ITU-T J.178], [b-ITU-T J.192], [b-ITU-T J.360], [b-ITU-T J.361], [b-ITU-T J.362], [b-ITU-T J.368], [b-ITU-T J.388], [b-ITU-T J.460.1], [b-ITU-T J.460.2], [b-ITU-T J.700], [b-ITU-T J.703]

SDP 			Severely Disturbed Period			[b-ITU-T J.142]

SDP 	Service Delivery Platform			[b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.704]

SDT 		Service Description Table 			[b-ITU-T J.94]

SDTV	Standard Definition Television		[b-ITU-T J.140], [b-ITU-T J.151], [b-ITU-T J.381], [b-ITU-T J.388]

SDU 	Service Data Unit				[b-ITU-T J.82], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.195.3], [b-ITU-T J.196.3]

SDU 		Single-Dwelling Unit				[b-ITU-T J.700]

SDV	Switched Digital Video				[b-ITU-T J.381], [b-ITU-T J.700], [b-ITU-T J.704], [b-ITU-T J.1101], [b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1104]

SECAM 		Séquentiel couleur à mémoire (Sequential colour with memory)

[b-ITU-T J.81], [b-ITU-T J.94], [b-ITU-T J.185], [b-ITU-T J.222.1], [b-ITU-T J.702]

SEI			Supplemental Enhancement Information	[b-ITU-T J.286], [b-ITU-T J.296]

SEQ 		Sequences         	              [b-ITU-T J.89]

SEQ 		Sequence index					[b-ITU-T J.343.5], [b-ITU-T J.343.6]

SES			Severely Errored Second			[b-ITU-T J.132], [b-ITU-T J.142]

SET			Secure Electronic Transaction		[b-ITU-T J.222.3]

SETPI 		Synchronous Equipment Timing Physical Interface 			[b-ITU-T J.132]

SETS 		Synchronous Equipment Timing Source [b-ITU-T J.132]

SF	Service Flow					[b-ITU-T J.222.2], [b-ITU-T J.1102], [b-ITU-T J.1103]

SFID	Service Flow Identifier				[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122], [b-ITU-T J.222.2], [b-ITU-T J.1102], [b-ITU-T J.1103]

SFN			Single Frequency Network			[b-ITU-T J.296]

SFR	Source Frame Rate				[b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342]

SG	Signalling Gateway. A SG is a signalling agent that receives/sends SCN native signalling at the edge of the IP network. In particular, the SS7 SG function translates variants ISUP and TCAP in an SS7-Internet Gateway to a common version of ISUP and TCAP.			[b-ITU-T J.170]

SG 	Signalling Gateway				[b-ITU-T J.160], [b-ITU-T J.171.1], [b-ITU-T J.171.2], [b-ITU-T J.360], [b-ITU-T J.460.1]

SG 			Service Group					[b-ITU-T J.222.2]

SHA			Secure Hash Algorithm				[b-ITU-T J.222.2], [b-ITU-T J.366.8]

SHA-1	Secure Hash Algorithm 1			[b-ITU-T J.112 Annex A], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.3], [b-ITU-T J.290]

SHF			Super High Frequency				[b-ITU-T J.142]

SHP			Stereo High Profile				[b-ITU-T J.296]

SI	Service Information				[b-ITU-T J.94], [b-ITU-T J.111], [b-ITU-T J.117], [b-ITU-T J.128], [b-ITU-T J.142], [b-ITU-T J.183], [b-ITU-T J.200], [b-ITU-T J.282], [b-ITU-T J.292], [b-ITU-T J.293], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.900], [b-ITU-T J.1011]

SI 			Spatial Information				[b-ITU-T J.249]

SI   		System Information or Service Information				[b-ITU-T J.703]

SID	Service Identifier					[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.125], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.1102], [b-ITU-T J.1103]

SID 			System IDentification number		[b-ITU-T J.160]

SID 	Service ID. A 14-bit number assigned by a CMTS to identify an upstream virtual circuit. Each SID separately requests and is granted the right to use upstream bandwidth.						[b-ITU-T J.175]

SID 			(Upstream) Service Identifier			[b-ITU-T J.213]

SID			Silence Insertion Descriptor			[b-ITU-T J.361]

SIM			Subscriber Identity Module			[b-ITU-T J.360 Amendment 1]

SIMPLE		SIP for Instant Message and Presence Leveraging Extensions	[b-ITU-T J.367]

SIP	Session Initiation Protocol			[b-ITU-T J.218], [b-ITU-T J.261], [b-ITU-T J.360], [b-ITU-T J.361], [b-ITU-T J.262], [b-ITU-T J.365], [b-ITU-T J.366.0], [b-ITU-T J.367], [b-ITU-T J.368], [b-ITU-T J.369], [b-ITU-T J.388], [b-ITU-T J.460.0], [b-ITU-T J.460.2], [b-ITU-T J.460.3], [b-ITU-T J.700], [b-ITU-T J.701], [b-ITU-T J.705], [b-ITU-T J.1102], [b-ITU-T J.1101], [b-ITU-T J.1103]

SIP 	Session Initiation Protocol. An application-layer control (signalling) protocol for creating, modifying, and terminating sessions with one or more participants.								[b-ITU-T J.170]

SIP 	Session Initiation Protocol, VoIP signaling protocol, defined by [b-IETF RFC 3261]							[b-ITU-T J.460.1]

SIP+	Session Initiation Protocol Plus. An extension to SIP.		[b-ITU-T J.170], [b-ITU-T J.361]

SIS			Systems for Interactive Services		[b-ITU-T J.111]

SIS 	Subscriber Information Service		[b-ITU-T J.380.1], [b-ITU-T J.380.2], [b-ITU-T J.380.6], [b-ITU-T J.706]

SIT 			Satellite Information Table 			[b-ITU-T J.94]

SL-ESF	Signalling Link Extended SuperFrame	[b-ITU-T J.112 Annex A], [b-ITU-T J.116]

SLA	Service Level Agreement			[b-ITU-T J.241], [b-ITU-T J.282], [b-ITU-T J.705]

SLAAC		Stateless Address Autoconfiguration	[b-ITU-T J.218], [b-ITU-T J.222.2]

SLE			Screening List Editing				[b-ITU-T J.460.0], [b-ITU-T J.460.1]

SLED  		Software Loopback for eDOCSIS		[b-ITU-T J.126], [b-ITU-T J.800.2]

SLF			Subscription Locator Function		[b-ITU-T J.360]

SLI			L2TPv3 Set Link Info message		[b-ITU-T J.212]

SLIP			Serial Line Interconnection Protocol	[b-ITU-T J.118]

SM			Station Maintenance				[b-ITU-T J.222.2]

SM  		Security Module					[b-ITU-T J.702]

SMAC		Source MAC					[b-ITU-T J.213]

SMATV	Satellite Master Antenna Television	[b-ITU-T J.84], [b-ITU-T J.84], [b-ITU-T J.110], [b-ITU-T J.111], [b-ITU-T J.112 Annex A], [b-ITU-T J.114], [ITU-T J.116], [b-ITU-T J.118], [b-ITU-T J.142], [b-ITU-T J.183], [b-ITU-T J.382]

SMATV-DTM 	SMATV system based on Digital TransModulation 			[b-ITU-T J.84]

SMATV-IF 	SMATV system based on distribution at IF 				[b-ITU-T J.84]

SMATV-S 	SMATV system based on distribution at extended Superband 	[b-ITU-T J.84]

SMI  		Structure of Management Information	[b-ITU-T J.800.0]

SMIL		Synchronized Multimedia Integration Language			[b-ITU-T J.124]

SMS	Subscriber Management System		[b-ITU-T J.116], [b-ITU-T J.380.6], [b-ITU-T J.1005]

SMS 	Spectrum Management System 		[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122]

SMS 		Short Message Service 			[b-ITU-T J.115]

S-MTA	Standalone Multimedia Terminal Adapter [b-ITU-T J.191], [b-ITU-T J.192]

S-MTA 	Stand-alone MTA – a single node which contains an MTA and a non J.112 MAC (e.g., Ethernet)					[b-ITU-T J.175]

S-MTA 	Standalone MTA (a single node that contains an MTA and a non-DOCSIS MAC (e.g., Ethernet))					[b-ITU-T J.179]

SN 			Sequence Number 				[b-ITU-T J.131], [b-ITU-T J.132]

SNAP	Sub-network Access Protocol		[b-ITU-T J.111], [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-	ITU-T J.222.2]

SNF 			Service Navigation Function			[b-ITU-T J.702]

SNFD		Source Normalized Frame Difference	[b-ITU-T J.249]

SNG 		Satellite News Gathering 			[b-ITU-T J.92]

SNI 			Sequence Number Invalid 			[b-ITU-T J.132]

SNMP	Simple Network Management Protocol	[b-ITU-T J.111], [b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.126], [b-ITU-T J.160], [b-ITU-T J.167], [b-ITU-T J.170], [b-ITU-T J.172], [b-ITU-T J.173], [b-ITU-T J.175], [b-ITU-T J.177], [b-ITU-T J.186], [b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.223.1], [b-ITU-T J.223.2], [b-ITU-T J.290], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.360], [b-ITU-T J.369], [b-ITU-T J.370], [b-ITU-T J.460.4], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.705]

SNMPE		Source Normalized High Frequency Energy				[b-ITU-T J.249]

SNR	Signal-to-Noise Ratio				[b-ITU-T J.83], [b-ITU-T J.223.2], [b-ITU-T J.287], [b-ITU-T J.296], [b-ITU-T J.382]

SNS			Source Name Sub-Table			[b-ITU-T J.128]

SNS 			Social Network Service			[b-ITU-T J.230], [b-ITU-T J.295]

SNTP		Simple Network Time Protocol		[b-ITU-T J.287], [b-ITU-T J.360]

SO			Service Operator					[b-ITU-T J.1106], [b-ITU-T J.1107]

SoA			Start of Authority				[b-ITU-T J.191], [b-ITU-T J.192]

SOAP	Simple Object Access Protocol		[b-ITU-T J.177], [b-ITU-T J.291], [b-ITU-T J.700]

SOAP 		Simple Object Access Protocol / Service Oriented Architecture Protocol

[b-ITU-T J.380.7]

SOC			System-on-Chip					[b-ITU-T J.290]

SOH 		Section Overhead 				[b-ITU-T J.132]

SOHO		Small Office, Home Office			[b-ITU-T J.195.1]

SP			Simple Profile					[b-ITU-T J.296], [b-ITU-T J.388]

SP 	Service Provider					[b-ITU-T J.700], [b-ITU-T J.1005], [b-ITU-T J.1006]

SP   		Service Protection				[b-ITU-T J.702]

SPF			Security Portal Function			[b-ITU-T J.190]

SPF 			Stateful Packet Filtering			[b-ITU-T J.191], [b-ITU-T J.192]

SPI 	Synchronous Parallel (or Physical) Interface 				[b-ITU-T J.131], [b-ITU-T J.132]

SPI 			Security Parameters Index			[b-ITU-T J.171.1], [b-ITU-T J.171.2]

SPI 			Serial Peripheral Interface			[b-ITU-T J.222.2]

SPI 			Service Provider Interface			[b-ITU-T J.294], [b-ITU-T J.700]

SPP 			Subscriber Programmable PIN		[b-ITU-T J.460.0], [b-ITU-T J.460.1]

SPR			Subscription Profile Repository		[b-ITU-T J.368]

sps 			Symbols per second 				[b-ITU-T J.83]

SPS 			Sequence Parameter Set			[b-ITU-T J.286]

SPTS	Single Program(me) Transport Stream		[b-ITU-T J.117], [b-ITU-T J.181], [b-ITU-T J.181 Amendment 1], [b-ITU-T J.212], [b-ITU-T J.280], [b-ITU-T J.287], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.700]

SRC			Source						[b-ITU-T J.149]

SRC 	Source Reference Channel or Circuit	[b-ITU-T J.244], [b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342], [b-ITU-T J.343], [b-ITU-T J.343.1], [b-ITU-T J.343.2], [b-ITU-T J.343.3], [b-ITU-T J.343.4], [b-ITU-T J.343.5], [b-ITU-T J.343.6]

SRM			Session and Resource Manager		[b-ITU-T J.111], [b-ITU-T J.700]

SRM 		System Renewability Message		[b-ITU-T J.197]

SRM 	   	Service Reference Model			[b-ITU-T J.703]

SROI			Spatial Region of Interest			[b-ITU-T J.340]

SRV			Server						[b-ITU-T J.170]

SS	Security Sublayer				[b-ITU-T J.195.1], [b-ITU-T J.195.3], [b-ITU-T J.196.1], [b-ITU-T J.196.3]

SS 			Spread Spectrum					[b-ITU-T J.240]

SS7 	Signalling System No. 7			[b-ITU-T J.164], [b-ITU-T J.360], [b-ITU-T J.460.1]

SS7	Signalling System No. 7. An architecture and set of protocols for performing out-of-band call signalling with a telephone network.			[b-ITU-T J.170]

SSC			Symbol Sub-Cell					[b-ITU-T J.196.2], [b-ITU-T J.196.3]

SSCQE		Single Stimulus Continuous Quality Evaluation

[b-ITU-T J.140],[b-ITU-T J.244], [b-ITU-T J.245]

SSCS 		Service Specific Convergence Sublayer 	[b-ITU-T J.82]

SSD	Secure Software Download			[b-ITU-T J.126], b-ITU-T J.128], [b-ITU-T J.222.3]

SSDP		Simple Service Discovery Protocol		[b-ITU-T J.296]

SSF 			Server Signal Fail 				[b-ITU-T J.132]

SSF 			Security Server Function			[b-ITU-T J.190]

SSI 			Synchronous Serial Interface 		[b-ITU-T J.131], [b-ITU-T J.132]

SSID			Service Set Identifier				[b-ITU-T J.296]

SSL	Secure Socket(s) Layer				[b-ITU-T J.170], [b-ITU-T J.296], [b-ITU-T J.365], [b-ITU-T J.380.7], [b-ITU-T J.700]

SSM	Source (-) Specific Multicast			[b-ITU-T J.222.2], [b-ITU-T J.702], [b-ITU-T J.703], [b-ITU-T J.700]

SSRC		Synchronizing Source				[b-ITU-T J.361]

SSW 		Sequence Synchronizing Word 		[b-ITU-T J.88]

ST 			Stuffing Table 					[b-ITU-T J.94]

ST 			Spatial-Temporal 				[b-ITU-T J.340]

STB	Set Top Box					[b-ITU-T J.111], [b-ITU-T J.116], [b-ITU-T J.118], [b-ITU-T J.197], [b-ITU-T J.241], [b-ITU-T J.604], [b-ITU-T J.703], [b-ITU-T J.1004],

STB	Set-Top Box					[b-ITU-T J.112 Annex A], [b-ITU-T J.114], [b-ITU-T J.115], [b-ITU-T J.117], [b-ITU-T J.151], [b-ITU-T J.185], [b-ITU-T J.186], [b-ITU-T J.190], [b-ITU-T J.195.1], [b-ITU-T J.230], [b-ITU-T J.281], [b-ITU-T J.282], [b-ITU-T J.290], [b-ITU-T J.292], [b-ITU-T J.293], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.301], [b-ITU-T J.343.4], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.707], [b-ITU-T J.900], [b-ITU-T J.1001], [b-ITU-T J.1005], [b-ITU-T J.1011], [b-ITU-T J.1101], [b-ITU-T J.1102], [b-ITU-T J.1103], [b-ITU-T J.1106], [b-ITU-T J.1107]

STB	Set-Top-Box					[b-ITU-T J.193], [b-ITU-T J.294], [b-ITU-T J.302]

STC			System Time Clock				[b-ITU-T J.181]

STD			Set Top Device					[b-ITU-T J.128]

STD 		Standard Channel Plan				[b-ITU-T J.222.1]

STM 		Synchronous Transport Module 		[b-ITU-T J.132]

StopCCN		L2TPv3 Stop-Control-Connection-Notification message		[b-ITU-T J.212]

STP			Signalling Transfer Point			[b-ITU-T J.177]

STT			System Time Table				[b-ITU-T J.700]

STU 	Set Top Unit 					[b-ITU-T J.111], [b-ITU-T J.116],[b-ITU-T J.118]

STU	Set-Top Unit					[b-ITU-T J.112 Annex A], [b-ITU-T J.114], [b-ITU-T J.115]

STUN 	Simple Traversal of User Datagram Protocol (UDP) Through Network Address Translators						[b-ITU-T J.296], [b-ITU-T J.360], [b-ITU-T J.362], [b-ITU-T J.460.1], [b-ITU-T J.460.2], [b-ITU-T J.700], [b-ITU-T J.703]

SVC  		Scalable Video Coding				[b-ITU-T J.604]

SVD	Subscriber Video Device			[b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.292], [b-ITU-T J.702]

SVG			Scalable Vector Graphics			[b-ITU-T J.294]

- Video		Super-Video					[b-ITU-T J.197]

SVoD		Subscription Video-on-Demand		[b-ITU-T J.98]

SW			Session Word					[b-ITU-T J.96]

SW 	Software						[b-ITU-T J.126], [b-ITU-T J.296], [b-ITU-T J.701]

sync 			synchronizing signal 				[b-ITU-T J.83]

SYNC 		Synchronisation 					[b-ITU-T J.112], [b-ITU-T J.116]

SYSLOG		System Logging					[b-ITU-T J.190]

SYSLOG 		System Log					[b-ITU-T J.191], [b-ITU-T J.192]

SYSLOG 	System Logging Protocol – a protocol which defines the transport mechanism for the messages carrying various logging information			[b-ITU-T J.369]

T-MAC		Transmission-MAC				[b-ITU-T N.62]

TA			Transmitter Amplifier				[b-ITU-T J.186]

TA 			Trust Authority					[b-ITU-T J.1011]

TAI 			International Atomic Time 			[b-ITU-T J.94]

TB			Tuning Band					[b-ITU-T J.222.1]

TBD 		To Be Determined 				[b-ITU-T J.83]

TC 			Transmission Convergence sublayer 	[b-ITU-T J.112], [b-ITU-T J.116]

TC 			Time Code						[b-ITU-T J.285]

TC8PSK	Trellis Coded 8-Phase Shift Keying	[b-ITU-T J.142], [b-ITU-T J.185], [b-ITU-T J.186]

TCA 	 	Threshold Crossing Alerts			[b-ITU-T J.705]

TCAP	Transaction Capabilities Application Protocol. A protocol within the SS7 stack that is used for performing remote database transactions with a Signalling Control Point.							[b-ITU-T J.170]

TCAP 		Transaction Capabilities Application Part				[b-ITU-T J.160]

TCAP 	Transaction Capabilities Application Part (part of the SS7 signalling stack)

[b-ITU-T J.460.1]

TCC			Transmit Channel Configuration		[b-ITU-T J.222.2]

tcimsbf		twos complement integer, most significant bit first			[b-ITU-T J.280]

TCM			Trellis Code Modulation			[b-ITU-T J.142], [b-ITU-T J.222.1]

TCP	Transmission Control Protocol		[b-ITU-T J.111], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.120], [b-ITU-T J.121], [b-ITU-T J.122], [b-ITU-T J.124], [b-ITU-T J.128], [b-ITU-T J.161], [b-ITU-T J.179], [b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.218], [b-ITU-T J.222.2], [b-ITU-T J.223.2], [b-ITU-T J.230], [b-ITU-T J.241], [b-ITU-T J.290], [b-ITU-T J.292], [b-ITU-T J.293], [b-ITU-T J.296], [b-ITU-T J.360], [b-ITU-T J.380.7], [b-ITU-T J.388], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.705]

TCP/IP		Transport Control Protocol/Internet Protocol				[b-ITU-T J.280]

TCS	Transmit Channel Set				[b-ITU-T J.128], [b-ITU-T J.222.1], [b-ITU-T J.222.2]

TD 			Transmit Degrade 				[b-ITU-T J.132]

TD 			Timeout for Disconnect			[b-ITU-T J.170], [b-ITU-T J.177]

TD 			Terminal Device					[b-ITU-T J.702], [b-ITU-T J.705]

TDD			Time Division Duplexing			[b-ITU-T J.195.1]

TDD  		Telecommunications Device for the Deaf			[b-ITU-T J.460.0]

TDL 		Tapped Delay Line 				[b-ITU-T J.84]

TDM	Time Division Multiplex			[b-ITU-T J.83], [b-ITU-T J.84], [b-ITU-T J.118], [b-ITU-T J.382]

TDM 	Time Division Multiplexing			[b-ITU-T J.126], [b-ITU-T J.183], [b-ITU-T J.211], [b-ITU-T J.222.1], [b-ITU-T J.361], [b-ITU-T J.800.2]

TDM Services	Legacy T1/E1 or T3/E3 voice and/or data transport	[b-ITU-T J.211]

TDMA		Time Division Multiplex Access		[b-ITU-T J.112], [b-ITU-T J.118],

TDMA	Time-Division Multiplex Access		[b-ITU-T J.112 Annex A]

TDMA	Time Division Multiple Access		[b-ITU-T J.115], [b-ITU-T J.116], [b-ITU-T J.196.2], [b-ITU-T J.196.3], [b-ITU-T J.211], [b-ITU-T J.222.1], [b-ITU-T J.222.2]

TE	Terminating Equipment			[b-ITU-T J.115]

TEA  		TDM Emulation Adapter 			[b-ITU-T J.800.2]

TE-CMTS		Time Division Multiplexing Emulation – Cable Modem Termination System

[b-ITU-T J.222.1]

TEI			TDM Emulation Interface			[b-ITU-T J.222.1], [b-ITU-T J.222.2]

TEK	Traffic Encryption Key				[b-ITU-T J.125], [b-ITU-T J.222.2], [b-ITU-T J.222.3]

TEL			Telephone						[b-ITU-T J.190]

TEV			Target Error Vector				[b-ITU-T J.142]

TF 			Transmit Fail 					[b-ITU-T J.132]

TFTP	Trivial File Transfer Protocol			[b-ITU-T J.112], [b-ITU-T J.116], [b-ITU-T J.126], [b-ITU-T J.160], [b-ITU-T J.167], [b-ITU-T J.170], [b-ITU-T J.177], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.369], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.705]

TFTP	Trivial File-Transfer Protocol		[b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.122],

TFTP	Trivial File Transfer Protocol	 [b-IETF RFC 1350]		 	[b-ITU-T J.190]

TGCP	Trunking Gateway Control Protocol	[b-ITU-T J.160], [b-ITU-T J.171.2], [b-ITU-T J.175], [b-ITU-T J.360]

TGS			Ticket Granting Server				[b-ITU-T J.160], [b-ITU-T J.167]

TGS 		Ticket Granting Server. A sub-system of the KDC used to grant Kerberos tickets.			[b-ITU-T J.170]

TGT			Ticket Granting Ticket				[b-ITU-T J.261], [b-ITU-T J.369]

TI			Turner Identifier					[b-ITU-T J.181]

TI 			Time Interleaver					[b-ITU-T J.382]

TID			Tribune Identifier				[b-ITU-T J.181]

TIM 			Trace Identifier Mismatch 			[b-ITU-T J.132]

TLS	Transport Layer Security			[b-ITU-T J.170], [b-ITU-T J.192], [b-ITU-T J.261], [b-ITU-T J.292], [b-ITU-T J.296], [b-ITU-T J.360], [b-ITU-T J.366.8], [b-ITU-T J.367], [b-ITU-T J.380.7], [b-ITU-T J.702], [b-ITU-T J.705]

TLS 			Transparent LAN Service			[b-ITU-T J.213]

TLV	Type Length Value				[b-ITU-T J.128], [b-ITU-T J.288],	[b-ITU-T J.297]

TLV	Type/Length/Value				[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.218], [b-ITU-T J.222.1], [b-ITU-T J.222.2], [b-ITU-T J.222.3]

TLV	Type-Length-Value				[b-ITU-T J.126], [b-ITU-T J.163], [b-ITU-T J.177], [b-ITU-T J.183], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.196.3]

TLV			Type-Length-Value (technique used in formatting protocol elements)

[b-ITU-T J.179]

TMN			Telecommunications Management Network

[b-ITU-T J.110],	[b-ITU-T J.700]

TN			Telephone Number				[b-ITU-T J.175]

TNT 			Transponder Name Table 			[b-ITU-T J.94]

ToD	Time of Day					[b-ITU-T J.192], [b-ITU-T J.213], [b-ITU-T J.222.2], [b-ITU-T J.222.3]

ToD 	Time of the Day – the network protocol which delivers the time of the day to a network client from the Time Of the Day from the network server

[b-ITU-T J.369]

ToS	Type of Service (also DiffServ Code Point, DSCP)		[b-ITU-T J.160], [b-ITU-T J.222.2], [b-ITU-T J.282], [b-ITU-T J.290], [b-ITU-T J.292]

TOT 			Time Offset Table 				[b-ITU-T J.94]

TP			Test Presentation					[b-ITU-T J.140], [b-ITU-T J.245]

TR			Technical Report					[b-ITU-T J.296], [b-ITU-T J.360]

TROI			Temporal Region of Interest			[b-ITU-T J.340]

TS	Transport Stream					[b-ITU-T J.82], [b-ITU-T J.83], [b-ITU-T J.112 Annex A], [b-ITU-T J.116], [b-ITU-T J.118], [b-ITU-T J.120], [b-ITU-T J.131], [b-ITU-T J.132], [b-ITU-T J.142], [b-ITU-T J.181 Amendment 1], [b-ITU-T J.183], [b-ITU-T J.193], [b-ITU-T J.281], [b-ITU-T J.282], [b-ITU-T J.286], [b-ITU-T J.287], [b-ITU-T J.288], [b-ITU-T J.290], [b-ITU-T J.292], [b-ITU-T J.293], [b-ITU-T J.294], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.302], [b-ITU-T J.343.5], [b-ITU-T J.343.6], [b-ITU-T J.382], [b-ITU-T J.388], [b-ITU-T J.702], [b-ITU-T J.900], [b-ITU-T J.1006]

TS 			Test Session					[b-ITU-T J.140], [b-ITU-T J.245]

TS 			Technical Specification			[b-ITU-T J.366.0]

TSID 		Transport Stream ID 				[b-ITU-T J.117]

TSID			Transport Stream Identification		[b-ITU-T J.151]

TSID			MPEG2 Transport Stream ID			[b-ITU-T J.212]

TSLE 		Transport Stream synchronisation Loss Error 				[b-ITU-T J.132]

TSMF		Transport Stream Multiplexing Frame	[b-ITU-T J.183], [b-ITU-T J.282]

TSP			Time Division Multiplexing Service Processor			[b-ITU-T J.222.1]

TSP  		Telephony Service Provider			[b-ITU-T J.800.1]

T-STD		Transport Stream System Target Decoder[b-ITU-T J.181 Amendment 1]

TTL			Time To Live					[b-ITU-T J.218], [b-ITU-T J.282]

TTP 			Trusted Third Party				[b-ITU-T J.1010], [b-ITU-T J.1011]

TTS			Timestamped TS					[b-ITU-T J.297]

TU 			Tributary Unit 					[b-ITU-T J.132]

TURN		Transversal Using Relay Network address translation (NAT)

[b-ITU-T J.360], [b-ITU-T J.362], [b-ITU-T J.460.1], [b-ITU-T J.700]

TV	Television						[b-ITU-T J.84], [b-ITU-T J.183], [b-ITU-T J.195.1], [b-ITU-T J.196.1], [b-ITU-T J.1011], [b-ITU-T J.1106]

TV 3			TeleVision class 3				[b-ITU-T J.249]

TVCT		Terrestrial Virtual Channel Table		[b-ITU-T J.117], [b-ITU-T J.151]

TVE			TV Everywhere					[b-ITU-T J.181 Amendment 1]

Tx or TX	Transmitter						[b-ITU-T J.185], [b-ITU-T J.186], [b-ITU-T J.381]

UA	User Agent						[b-ITU-T J.262], [b-ITU-T J.263], [b-ITU-T J.360], [b-ITU-T J.365]

UAR			User-Authorization-Request			[b-ITU-T J.360 Amendment 1]

UBG			Upstream Bonding Group			[b-ITU-T J.222.2]

UCC 	Upstream Channel Change 			[b-ITU-T J.112], [b-ITU-T J.116], [b-ITU-T J.122]

UCD	Upstream Channel Descriptor		[b-ITU-T J.112], [b-ITU-T J.112 Annex B], [b-ITU-T J.112 Annex C], [b-ITU-T J.116], [b-ITU-T J.122], [b-ITU-T J.212], [b-ITU-T J.222.1], [b-ITU-T J.222.2]

UCID		Upstream Channel Identifier			[b-ITU-T J.128], [b-ITU-T J.222.2]

UDC			Upstream Drop Classifier			[b-ITU-T J.222.2]

UDP	User Datagram Protocol			[b-ITU-T J.111], [b-ITU-T J.120], [b-ITU-T J.121], [b-ITU-T J.126], [b-ITU-T J.128], [b-ITU-T J.160], [b-ITU-T J.161], [b-ITU-T J.162], [b-ITU-T J.170], [b-ITU-T J.171.2], [b-ITU-T J.172], [b-ITU-T J.177], [b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.211], [b-ITU-T J.212], [b-ITU-T J.214], [b-ITU-T J.218], [b-ITU-T J.222.2], [b-ITU-T J.223.2], [b-ITU-T J.241], [b-ITU-T J.281], [b-ITU-T J.282], [b-ITU-T J.290], [b-ITU-T J.293], [b-ITU-T J.296], [b-ITU-T J.360], [b-ITU-T J.361], [b-ITU-T J.362], [b-ITU-T J.388], [b-ITU-T J.460.2], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.703]

UDP 	User Datagram Protocol. A connectionless protocol built upon Internet Protocol (IP)							[b-ITU-T J.179], [b-ITU-T J.370]

UE	User Equipment					[b-ITU-T J.261], [b-ITU-T J.360], [b-ITU-T J.363], [b-ITU-T J.367], [b-ITU-T J.368], [b-ITU-T J.369], [b-ITU-T J.388], [b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.460.3]

UGS 	Unsolicited Grant Service			[b-ITU-T J.214], [b-ITU-T J.222.1], [b-ITU-T J.222.2]

UGS	Unsolicited Grant Service (Annex B/ J.112 QoS scheduling type used for constant bit rate services (e.g., voice codecs))	[b-ITU-T J.179]

UGS/AD	Unsolicited Grant Service with Activity Detection			[b-ITU-T J.179]

UHD	  		Ultra High Definition 				[b-ITU-T J.381], [b-ITU-T J.1006]

UHDTV	Ultra High Definition Television		[b-ITU-T J.183], [b-ITU-T J.195.1], [b-ITU-T J.196.1], [b-ITU-T J.207], [b-ITU-T J.297], [b-ITU-T J.381]

UHF			Ultra High Frequency				[b-ITU-T J.84], [b-ITU-T J.151]

UI 			Unit Interval 					[b-ITU-T J.132]

UI 	User Interface					[b-ITU-T J.200], [b-ITU-T J.205], [b-ITU-T J.230], [b-ITU-T J.290], [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.1011]

UICC		Universal Integrated Circuit Card		[b-ITU-T J.360 Amendment 1]

uimsbf	unsigned integer, most significant bit first	[b-ITU-T J.96], [b-ITU-T J.181], [b-ITU-T J.183], [b-ITU-T J.280], [b-ITU-T J.287]

uinshf		unsigned integer, most significant first	[b-ITU-T J.128]

UMID		Unique Material Identifier			[b-ITU-T J.181]

UNEQ 		UNEQuipped 					[b-ITU-T J.132]

UNI			User-Network Interface			[b-ITU-T J.170]

UNO-CDR		Universal Networked Object – Common Data Representation													[b-ITU-T J.111]

UNO-RPC		Universal Networked Object – Remote Procedure Call

[b-ITU-T J.111]

UPID			Unique Program Identifier 			[b-ITU-T J.380.2]

UPnP	Universal Plug and Play			[b-ITU-T J.190], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.292], [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.705]

UPS			Uninterrupted Power Supply			[b-ITU-T J.126], [b-ITU-T J.199]

UPS 			Uninterruptible Power Supply		[b-ITU-T J.173], [b-ITU-T J.460.2]

URI	Uniform Resource Identifier			[b-ITU-T J.171.2], [b-ITU-T J.200], [b-ITU-T J.360], [b-ITU-T J.367], [b-ITU-T J.380.1], [b-ITU-T J.380.2], [b-ITU-T J.388], [b-ITU-T J.460.0], [b-ITU-T J.460.1],

URI	Uniform Resource Identifier (see [IETF RFC 3986])			[b-ITU-T J.181]

URI 			Universal Resource Identifier		[b-ITU-T J.175]

URI 			Usage Rights Information			[b-ITU-T J.1010]

URL	Uniform Resource Locator			[b-ITU-T J.120], [b-ITU-T J.192], [b-ITU-T J.230], [b-ITU-T J.296], [b-ITU-T J.367], [b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.800.0]

URL			Universal Resource Locator			 [b-ITU-T J.205], [b-ITU-T J.206]

URN 		Uniform Resource Name			[b-ITU-T J.460.1]

US	Upstream						[b-ITU-T J.128], [b-ITU-T J.211], [b-ITU-T J.212], [b-ITU-T J.222.2], [b-ITU-T J.296], [b-ITU-T J.1106], [b-ITU-T J.1107]

USB	Universal Serial Bus				[b-ITU-T J.126], [b-ITU-T J.190], [b-ITU-T J.290], [b-ITU-T J.291], [b-ITU-T J.296], [b-ITU-T J.702]

USFS	Upstream Selective Forwarding Switch	[b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192]

USM			User Security Model				[b-ITU-T J.191], [b-ITU-T J.192]

US-SG		Upstream Service Group			[b-ITU-T J.222.2]

Ut			Unavailable time					[b-ITU-T J.142]

UTC 			Universal Time Coordinated 			[b-ITU-T J.94]

UTC 	Coordinated Universal Time			[b-ITU-T J.181], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.222.2], [b-ITU-T J.222.3], [b-ITU-T J.280], [b-ITU-T J.287], [b-ITU-T J.460.1]

UTC 	Coordinated Universal Time (Also known as Greenwich Mean Time (GMT) or Zulu time)						[b-ITU-T J.211]

UTF-8		Unicode Transformation Format (the 8-bit form)			[b-ITU-T J.124]

UTF-16		Unicode Transformation Format (the 16-bit form)			[b-ITU-T J.124]

UUID		Universally Unique Identifier		[b-ITU-T J.124], [b-ITU-T J.380.2]

VACM	View-based Access Control Model		[b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.370]

VAD	Voice Activity Detection			[b-ITU-T J.161], [b-ITU-T J.163], [b-ITU-T J.164], [b-ITU-T J.361]

VANC		Vertical Ancillary data space in digital video streams			[b-ITU-T J.287]

VBI	Vertical Blanking Interval			[b-ITU-T J.89], [b-ITU-T J.700], [b-ITU-T J.702]

VBR	Variable Bit Rate					[b-ITU-T J.161], [b-ITU-T J.181 Amendment 1], [b-ITU-T J.361]

VBV	Video Buffer Verifier				[b-ITU-T J.181 Amendment 1], [b-ITU-T J.286]

VC 			Virtual Container or Virtual Channel 	[b-ITU-T J.132]

VC-1			Video Codec 1					[b-ITU-T J.700]

VCI	Virtual Channel Identifier			[b-ITU-T J.116], [b-ITU-T J.132], [b-ITU-T J.170]

VCI			ATM Virtual Channel Identification, defined by ITU-T I.363

[b-ITU-T J.112 Annex A]

VCM			Variable Coding and Modulation		[b-ITU-T J.382]

VCN 			Virtual Channel Number 			[b-ITU-T J.94]

VCO			Voltage-Controlled Oscillator		[b-ITU-T J.292]

VCPS		Video Content Protection System		[b-ITU-T J.197]

VCR	Video Cassette Recorder			[b-ITU-T J.151], [b-ITU-T J.700], [b-ITU-T J.702]

VCT	Virtual Channel Table				[b-ITU-T J.94], [b-ITU-T J.151], [b-ITU-T J.302], [b-ITU-T J.700]

VGA			Video Graphics Array				[b-ITU-T J.343.4]

VGA			Video Graphics Array (640 x 480 pixels)	[b-ITU-T J.246], [b-ITU-T J.247]

VH 			Vertical High frequency component 	[b-ITU-T J.88]

VHF			Very High Frequency				[b-ITU-T J.84], [b-ITU-T J.151]

V-ISAN		Version-ISAN 					[b-ITU-T J.181]

VITC			Vertical Interval Time Code			[b-ITU-T J.89], [b-ITU-T J.287]

VITS 		Vertical Interval Test Signal 			[b-ITU-T J.88]

VLAN	Virtual Local Area Network			[b-ITU-T J.222.2], [b-ITU-T J.223.2], [b-ITU-T J.290]

VLC 	Variable Length Coding			[b-ITU-T J.81], [b-ITU-T J.88], [b-ITU-T J.603]

VLD 			Variable Length Decoding 			[b-ITU-T J.88]

VLSI 		Very Large Scale Integration 		[b-ITU-T J.83]

VM	Virtual Machine					[b-ITU-T J.294], [b-ITU-T J.296], [b-ITU-T J.1010], [b-ITU-T J.1011]

VM 			Voice Mail						[b-ITU-T J.460.1]

VMWI 		Visual Message Waiting Indicator		[b-ITU-T J.460.0]

VoD	Video on Demand				[b-ITU-T J.97], [b-ITU-T J.181 Amendment 1], [b-ITU-T J.201], [b-ITU-T J.207], [b-ITU-T J.230], [b-ITU-T J.241], [b-ITU-T J.281], [b-ITU-T J.292], [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.297], [b-ITU-T J.380.3], [b-ITU-T J.381], [b-ITU-T J.700], [b-ITU-T J.701], [b-ITU-T J.703], [b-ITU-T J.704], [b-ITU-T J.707], [b-ITU-T J.1005], [b-ITU-T J.1006]

VoD	Video-on-Demand				[b-ITU-T J.98], [b-ITU-T J.127], [b-ITU-T J.193], [b-ITU-T J.212], [b-ITU-T J.290], [b-ITU-T J.900]

VoIP	Voice over IP					[b-ITU-T J.161], [b-ITU-T J.179], [b-ITU-T J.193], [b-ITU-T J.222.2], [b-ITU-T J.223.1], [b-ITU-T J.290], [b-ITU-T J.294], [b-ITU-T J.361], [b-ITU-T J.704]

VoIP	Voice-over-IP					[b-ITU-T J.175]

VoIP	Voice over Internet Protocol 		[b-ITU-T J.177], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.195.1], [b-ITU-T J.196.1], [b-ITU-T J.292], [b-ITU-T J.367], [b-ITU-T J.460.0], [b-ITU-T J.460.1], [b-ITU-T J.700]

V-OLT		Video-Optical Line Terminal 		[b-ITU-T J.185]

V-OLT 		Optical Line Terminal for Video signals	[b-ITU-T J.186]

V-ONT		Video-Optical Network Terminal 		[b-ITU-T J.185]

V-ONT 		Optical Network Terminal for Video signals				[b-ITU-T J.186]

V-ONU		Video distribution Optical Network Unit	[b-ITU-T J.294]

V-ONU 		Video Optical Network Unit			[b-ITU-T J.700]

VP 			Virtual Path 					[b-ITU-T J.131], [b-ITU-T J.132]

VPC 		VoIP Positioning Centre			[b-ITU-T J.460.1]

VPE 			Virtual Path Entity 				[b-ITU-T J.131], [b-ITU-T J.132]

VPI			Virtual Path Identifier				[b-ITU-T J.132], [b-ITU-T J.170]

VPI			ATM Virtual Path Identification, defined by ITU-T I.363

[b-ITU-T J.112 Annex A]

VPME 		Virtual Path Multiplexing Entity 		[b-ITU-T J.131], [b-ITU-T J.132]

VPN	Virtual Private Network			[b-ITU-T J.179], [b-ITU-T J.190], [b-ITU-T J.213], [b-ITU-T J.700], [b-ITU-T J.704]

VQE  		Visual Quality Experience			[b-ITU-T J.705]

VQEG	Video Quality Experts Group			[b-ITU-T J.149], [b-ITU-T J.244], [b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.249], [b-ITU-T J.341], [b-ITU-T J.342],[b-ITU-T J.343], [b-ITU-T J.343.1], [b-ITU-T J.343.2], [b-ITU-T J.343.3], [b-ITU-T J.343.4], [b-ITU-T J.343.5], [b-ITU-T J.343.6],

VQM	Video Quality Metric(s)				[b-ITU-T J.149], [b-ITU-T J.246], [b-ITU-T J.249], [b-ITU-T J.343.1], [b-ITU-T J.343.2], [b-ITU-T J.343.3], [b-ITU-T J.343.4], [b-ITU-T J.343.5], [b-ITU-T J.343.6]

VRN			Video Rich Navigation				[b-ITU-T J.700]

VSB	Vestigial SideBand				[b-ITU-T J.83], [b-ITU-T J.150], [b-ITU-T J.151]

VSC 		Vertical Service Code				[b-ITU-T J.460.1]

VSI	Video Service Interface			[b-ITU-T J.1101], [b-ITU-T J.1102], [b-ITU-T J.1104], [b-ITU-T J.1103]

VSP			Vendor Specific Parameter			[b-ITU-T J.128]

VSP 			Video Service Provider				[b-ITU-T J.197]

VSWR		Voltage Standing Wave Ratio		[b-ITU-T J.185], [b-ITU-T J.186]

VT 			Vertical Temporal frequency component [b-ITU-T J.88]

VT 	Video Telephony (bidirectional conversational communication with video and audio)						[b-ITU-T J.388]

VTR 			Video Tape Recorder 				[b-ITU-T J.92]

W3C	World Wide Web Consortium		[b-ITU-T J.296], [b-ITU-T J.365], [b-ITU-T J.1006]

WAN	Wide Area Network				[b-ITU-T J.126], [b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192], [b-ITU-T J.218], [b-ITU-T J.294], [b-ITU-T J.295], [b-ITU-T J.296], [b-ITU-T J.700]

WAN-Data		Wide Area Network(WAN) Data Address Realm

[b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192]

WAN-Man		Wide Area Network (WAN) Management Address Realm

[b-ITU-T J.190], [b-ITU-T J.191], [b-ITU-T J.192]

WDM	Wavelength Division Multiplexing		[b-ITU-T J.185], [b-ITU-T J.186], [b-ITU-T J.293], [b-ITU-T J.700]

WEP			Wired Equivalent Privacy			[b-ITU-T J.296]

WHT	Walsh-Hadamard Transform			[b-ITU-T J.88], [b-ITU-T J.147], [b-ITU-T J.240]

Wi-Fi		Wireless Fidelity					[b-ITU-T J.195.1]

WM 			Water Mark 					[b-ITU-T J.95]

WMA		Windows Media Audio				[b-ITU-T J.700]

WPA			Wi-Fi Protected Access			[b-ITU-T J.296]

WPS			Wi-Fi Protected Setup				[b-ITU-T J.296]

WRS 		Wireless Relay Station 				[b-ITU-T J.114]

WS			Web Service					[b-ITU-T J.365]

WSDL	Web Services Description Language	[b-ITU-T J.365], [b-ITU-T J.380.7], [b-ITU-T J.700]

WST			World System Teletext				[b-ITU-T J.287]

WTR 		Wait to Restore 					[b-ITU-T J.132]

WTSA		World Telecommunication Standardization Assembly		[b-ITU-T J.197]

WTSC 		World Telecommunication Standardization Conference 		[b-ITU-T J.90]

wTVML		worldwide Television Markup Language	[b-ITU-T J.201]

WVGA		Wide Video Graphics Array			[b-ITU-T J.343.4]

WWW		World Wide Web				[b-ITU-T J.282]

X509 	Recommendation ITU-T X.509: Information Technology – Open Systems Interconnection – The Directory: Authentication Framework

[b-ITU-T J.800.2]

XAIT		Extended Application Information Table (OCAP)			[b-ITU-T J.128]

XCAP	XML Configuration Access Protocol	[b-ITU-T J.360], [b-ITU-T J.367], [b-ITU-T J.460.1], [b-ITU-T J.705]

XCAP 		eXtensible markup language Configuration Access Protocol	[b-ITU-T J.700]

XDM	XML		Document Management			[b-ITU-T J.367]

XDMC		XML Document Management Client	[b-ITU-T J.367]

XDMS		XML Document Management Server	[b-ITU-T J.367]

XDS			XCAP Data Server				[b-ITU-T J.360]

xDSL		x Digital Subscriber Line			[b-ITU-T J.282]

xDSL 		(symmetric, asymmetric, high bit-rate, very high speed) Digital Subscriber Line										[b-ITU-T J.700]

XG-PON		10-Gigabit Passive Optical Network 	[b-ITU-T J.185]

XHTML	extensible Hyper Text Markup Language	[b-ITU-T J.127], [b-ITU-T J.200], [b-ITU-T J.201]

XM			Cross Modulation				[b-ITU-T J.142]

XM 			Cross Modulation distortion			[b-ITU-T J.185], [b-ITU-T J.186]

XML	eXtensible Markup Language		[b-ITU-T J.181], [b-ITU-T J.200], [b-ITU-T J.201], [b-ITU-T J.290], [b-ITU-T J.360 Amendment 1], [b-ITU-T J.365], [b-ITU-T J.367], [b-ITU-T J.380.1], [b-ITU-T J.380.2], [b-ITU-T J.700], [b-ITU-T J.702], [b-ITU-T J.703], [b-ITU-T J.705], [b-ITU-T J.800.0]

XOR	Exclusive						[b-ITU-T J.83], [b-ITU-T J.147], [b-ITU-T J.222.1], [b-ITU-T J.222.3]

XSD			XML Schema Definition			[b-ITU-T J.365], [b-ITU-T J.800.0]

XSL			eXtensible Stylesheet Language		[b-ITU-T J.200]

XSLT		XSL Transformations				[b-ITU-T J.200]

YUV	Color/Colour Space and file format		[b-ITU-T J.246], [b-ITU-T J.247], [b-ITU-T J.342]

Appendix I

<Appendix Title>

(This appendix does not form an integral part of this Recommendation.)

<Body of appendix I>

Bibliography

[b-ITU DB]         ITU terminology database 	http://itu.int/go/terms

[b-ARIB STD B-24]	ARIB standard STD-B24 (1999), Data Coding and Transmission Specification for Digital Broadcasting.

[b-CCIF 2.0]		CableCARDTM Interface 2.0 Specification, OC-SP-CCIF2.0-I09-070105, January 5, 2007, Cable Television Laboratories, Inc.

[b-CEA-542-B]		CEA Standard (2003),  Cable Television Channel Identification Plan.

[b-DAVIC 1.4]		Digital Audio-Visual Council, (DAVIC) 1.4 Part 9 (1998), Annex B, the file format of audio encoding of PCM.

[b-DOCS3] 		Data-Over-Cable Service Interface Specifications, Cable Modem Termination System - Network Side Interface Specification, SP-CMTS-NSII01-960702.

[b-DOCSIS3]		Data-Over-Cable Service Interface Specifications, Cable Modem Termination System  Network Side Interface Specification, SP-CMTS-NSII01-960702.

[b-DOCSIS4]	 	Data-Over-Cable Service Interface Specifications, Cable Modem to Customer Premises Equipment Interface Specification, SP-CMCI-I04-000714.

[b-DOCS4]			Data-Over-Cable Service Interface Specifications, Cable Modem to Customer Premise Equipment Interface Specification, SP-CMCI-I04-000714.

[b-ECMAScript]		Standard ECMA-262 (2000), 3rd Edition, ECMAScript Language Specification.

[b-ETSI GS ECI 001-5]	ETSI GS ECI 001-5 (2017), Embedded Common Interface (ECI) for exchangeable CA/DRM solutions; Part 5: The Advanced Security System.

[b-GINGA Data Transmission]	ABNT-NBR 15606-3 (2007), Digital terrestrial television – Data coding and transmission specification for digital broadcasting, Part 3:Data transmission specification.

[b-ISO/IEC 10038]	ISO/IEC 10038 (1993)| ANSI/IEEE Std 802.1D (1993), Information technology – Telecommunications and information exchange between systems – Local area networks – Media access control (MAC) bridges.

[b-ISO/IEC 11172]	ISO/IEC 11172 (1993), Information technology – Coding of moving pictures and associated audio for digital storage media at up to about 1,5 Mbit/s – Part 1: Systems.

[b-ISO/IEC 13522-5]	ISO/IEC 13522-5 (1997), Information technology – Coding of multimedia and hypermedia information – Part 5: Support for base-level interactive applications.

[b-ISO/IEC 13818-7]	ISO/IEC 13818-7 (2010), Corrigendum 2 – Information technology – Generic coding of moving pictures and associated audio information – Part 7: Advanced Audio Coding (AAC).

[b-ISO/IEC 15802-3]	ISO/IEC 15802-3 (1998), Information technology. Telecommunications and information exchange between systems. Local and metropolitan area networks - common specifications. Remote media access control (MAC) bridging.

[b-IEEE 802]		IEEE 802 (1990), Local and Metropolitan Area Networks: Overview and Architecture.

[b-IEEE 802.1Q]		IEEE 802.1Q (2003), Local and Metropolitan Area Networks: Virtual Bridged Local Area Networks.

[b-IETF RFC 768]	IETF RFC 768 (1980), User Datagram Protocol.

[b-IETF RFC 791]	IETF RFC 791 (1981), Internet Protocol.

[b-IETF RFC 792]	IETF RFC 792 (1981), Internet Control Message Protocol – DARPA Internet Program – Protocol Specification.

[b-IETF RFC 793]		IETF RFC 793 (1981), Transmission Control Protocol.

[b-IETF RFC 1034]	IETF RFC 1034 (1987), Domain names – Concepts and facilities.

[b-IETF RFC 1350]	IETF RFC 1350 (1992), The TFTP Protocol (Revision 2).

[b-IETF RFC 1631]	IETF RFC 1631 (1994), The IP Network Address Translator (NAT).

[b-IETF RFC 2131]	IETF RFC 2131 (1997), Dynamic Host Configuration Protocol.

[b-IETF RFC 2138]	IETF RFC 2138 (1997), Remote Authentication Dial In User Service (RADIUS).

[b-IETF RFC 2139]	IEFT RFC 2139 (1997), RADIUS Accounting.

[b-IETF RFC 2205] 	IETF RFC 2205 (1997), Resource ReSerVation Protocol (RSVP) – Version 1 Functional Specification.

[b-IETF RFC 2210]	IETF RFC 2210 (1997), The Use of RSVP with IETF Integrated Services.

[b-IETF RFC 2328]	IETF RFC 2328 (1998), OSPF Version 2.

[b-IETF RFC 2663] 	. IETF RFC 2663 (1999), IP Network Address Translator (NAT) Terminology and Considerations.

[b-IETF RFC 2748]	IETF RFC 2748 (2000), The COPS (Common Open Policy Service) Protocol.

[b-IETF RFC 2753]	IETF RFC 2753 (2000), A Framework for Policy-based Admission Control.

[b‑IETF RFC 3022]	IETF RFC 3022 (2001), Traditional IP Network Address Translator (Traditional NAT).

[b-IETF RFC 3261]	IETF RFC 3261 (2002), SIP: Session Initiation Protocol.

[b-IETF RFC 3312]	IETF RFC 3312 (2002), Integration of Resource Management and Session Initiation Protocol (SIP).

[b-IETF RFC 3313]	IETF RFC 3313 (2003), Private Session Initiation Protocol (SIP) Extensions for Media Authorization.

[b-IETF RFC 3435]	IETF RFC 3435 (2003), Media Gateway Control Protocol (MGCP) Version 1.0.

[b-IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[b-IETF RFC 3761]	IETF RFC 3761 (2004), The E.164 to Uniform Resource Identifiers (URI) Dynamic Delegation Discovery System (DDDS) Application (ENUM).

[b-ITU-R BT.601-7]	Recommendation ITU-R BT.601-7 (2011), Studio encoding parameters of digital television for standard 4:3 and wide screen 16:9 aspect ratios.

[b-ITU-R BT.1306-6]	Recommendation ITU-R BT.1306-6 (2011), Error correction, data framing, modulation and emission methods for digital terrestrial television broadcasting.

[b-ITU-R BT.1699-2]	Recommendation ITU-R BT.1699-2 (2013), Harmonization of declarative application formats for interactive TV.

[b-ITU-R BT.2020-2]	Recommendation ITU-R BT.2020-2 (2015), Parameter values for ultra-high definition television systems for production and international programme exchange.

[ITU-R BO.1516-1]	Recommendation ITU-R BT.1516-1 (2012), Digital multiprogramme television systems for use by satellites operating in the 11/12 GHz frequency range.

[b-ITU-T H.222.0]	Recommendation ITU-T H.222.0 (2006)| ISO/IEC 13818-1 (2007), Information technology – Generic coding of moving pictures and associated audio information: Systems.

[b-ITU-T H.225.0]	Recommendation ITU-T H.225.0 (2000), Call signaling protocols and media stream packetization for packet-based multimedia communication systems.

[b-ITU-T H.262]		Recommendation ITU-T H.262 (2012) | ISO/IEC 13818-2 (2013), Information technology – Generic coding of moving pictures and associated audio information – Part 2: Video.

[b-ITU-T I.363]		Recommendation ITU-T I.363 (1993), B-ISDN ATM Adaptation Layer (AAL) specification.

[b-ITU-T J.13]		Recommendation ITU-T J.13 (1988), Definitions for international sound-programme circuits.

[b-ITU-T J.14]		Recommendation ITU-T J.14 (1988), Relative levels and impedances on an international sound-programme connection.

[b-ITU-T J.16]		Recommendation ITU-T J.16 (1988), Measurement of weighted noise in sound-programme circuits.

[b-ITU-T J.27]		Recommendation ITU-T J.27 (1990), Signals for the alignment of international sound-programme connections.

[b-ITU-T J.64] 		Recommendation ITU-T J.64 (1986), Definitions of parameters for simplified automatic measurement of television insertion test signals.

[b-ITU-T J.67]		Recommendation ITU-T J.67 (2001), Test signals and measurement techniques for transmission circuits carrying MAC/packet signals. Formerly ITU-R Rec. CMTT 772-1

[b-ITU-T J.81]		Recommendation ITU-T J.81 (1993), Transmission of component-coded digital television signals for contribution-quality applications at the third hierarchical level of ITU-T Recommendation G.702.

[b-ITU-T J.82]		Recommendation ITU-T J.82 (1996), Transport of MPEG-2 constant bit rate television signals in B-ISDN.

[b-ITU-T J.83]		Recommendation ITU-T J.83 (2007), Digital multi-programme systems for television, sound and data services for cable distribution.

[b-ITU-T J.84]		Recommendation ITU-T J.84 (2001), Distribution of digital multi-programme signals for television, sound and data services through SMATV networks.

[b-ITU-T J.87]		Recommendation ITU-T J.87 (2001), Use of hybrid cable television links for the secondary distribution of television into the user's premises.

[b-ITU-T J.88]		Recommendation ITU-T J.88 (2001), Transmission of enhanced definition television signals over digital links.

[b-ITU-T J.89]		Recommendation ITU-T J.89 (1999), Transport Mechanism for component-coded digital television signals using MPEG-2 4:2:2 P@ML including all service elements for contribution and pr imary distribution.

[b-ITU-T J.90]		Recommendation ITU-T J.90 (2001), Electronic programme guides for delivery by digital cable television and similar methods - Reference operating scenario and requirements.

[b-ITU-T J.91]		Recommendation ITU-T J.91 (1994), Technical methods for ensuring privacy in long-distance international television transmission.

[b-ITU-T J.92]		Recommendation ITU-T J.92 (1997), Recommended operating guidelines for point-to-point transmission of television programmes.

[b-ITU-T J.93]		Recommendation ITU-T J.93 (1997), Requirements for conditional access in the secondary distribution of digital television on cable television systems.

[b-ITU-T J.94]		Recommendation ITU-T J.94 (2016), Service information for digital broadcasting in cable television systems.

[b-ITU-T J.95]		Recommendation ITU-T J.95 (1999), Copy protection of intellectual property for content delivered on cable television systems.

[b-ITU-T J.96]		Recommendation ITU-T J.96 (2002), Technical method for ensuring privacy in long-distance international MPEG-2 television transmission conforming to ITU-T Recommendation J.89.

[b-ITU-T J.97]		Recommendation ITU-T J.97 (2002), Metadata on cable networks.

[b-ITU-T J.98]		Recommendation ITU-T J.98 (2003), Metadata requirements for video-on-demand in cable networks.

[b-ITU-T J.101]		Recommendation ITU-T J.101 (1990), Measurement methods and test procedures for teletext signals.

[b-ITU-T J.110]		Recommendation ITU-T J.110 (1997), Basic principles for a worldwide common family of systems for the provision of interactive television services.

[b-ITU-T J.111]		Recommendation ITU-T J.111 (1998), Network independent protocols for interactive systems.

[b-ITU-T J.112]		Recommendation ITU-T J.112 (1998), Transmission systems for interactive cable television services.

[b-ITU-T J.112 Annex A]	Recommendation ITU-T J.112 Annex A (2001), Transmission systems for interactive cable television services. Annex A: Digital Video Broadcasting: DVB interaction channel for Cable TV (CATV) distribution systems.

[b-ITU-T J.112 Annex B]	Recommendation ITU-T J.112 Annex B (2004), Transmission systems for interactive cable television services. Annex B: Data-over-cable service interface specifications: Radio-frequency interface specification.

[b-ITU-T J.112 Annex C]	Recommendation ITU-T J.112 Annex B (2004), Transmission systems for interactive cable television services. Annex C: Data-over-cable service interface specifications: Radio-frequency interface specification using QAM technique.

[b-ITU-T J.114]		Recommendation ITU-T J.114 (1999), Interaction channel using digital enhanced cordless telecommunications.

[b-ITU-T J.115]		Recommendation ITU-T J.115 (1999), Interaction channel using the global system for mobile communications.

[b-ITU-T J.116]		Recommendation ITU-T J.116 (2000), Interaction channel for local multipoint distribution systems.

[b-ITU-T J.117]		Recommendation ITU-T J.117 (1999), Home digital network interface specification.

[b-ITU-T J.118]		Recommendation ITU-T J.118 (2000), Access systems for interactive services on SMATV/MATV networks.

[b-ITU-T J.120]		Recommendation ITU-T J.120 (2000), Distribution of sound and television programs over the IP network.

[b-ITU-T J.121]		Recommendation ITU-T J.121 (2002), Quality control protocol for webcasting.

[b-ITU-T J.122]		Recommendation ITU-T J.122 (2002), Second-generation transmission systems for interactive cable television services - IP cable modems.

[b-ITU-T J.123]		Recommendation ITU-T J.123 (2002), Multiplexing format for webcasting on the TCP/IP network.

[b-ITU-T J.124]		Recommendation ITU-T J.124 (2004), Multiplexing format for multimedia webcasting over TCP/IP networks.

[b-ITU-T J.125]		Recommendation ITU-T J.125 (2007), Link privacy for cable modem implementations.

[b-ITU-T J.126]		Recommendation ITU-T J.126 (2007), Embedded Cable Modem device specification.

[b-ITU-T J.127]		Recommendation ITU-T J.127 (2004), Transmission protocol for multimedia webcasting over TCP/IP networks.

[b-ITU-T J.128]		Recommendation ITU-T J.128 (2008), Set-top gateway specification for transmission systems for interactive cable television services.

[b-ITU-T J.131]		Recommendation ITU-T J.131 (1998), Transport of MPEG-2 signals in PDH networks.

[b-ITU-T J.132]		Recommendation ITU-T J.132 (1998), Transport of MPEG-2 signals in SDH networks.

[b-ITU-T J.133]		Recommendation ITU-T J.133 (2002), Measurement of MPEG-2 transport streams in networks.

[b-ITU-T J.140]		Recommendation ITU-T J.140 (1998), Subjective picture quality assessment for digital cable television systems.

[b-ITU-T J.141]		Recommendation ITU-T J.141 (1999), Performance indicators

[b-ITU-T J.142]		Recommendation ITU-T J.142(2000), Methods for the measurement of parameters in the transmission of digital cable television signals.

[b-ITU-T J.144]		Recommendation ITU-T J.144 (2004), Objective perceptual video quality measurement techniques for digital cable television in the presence of a full reference.

[b-ITU-T J.145]		Recommendation ITU-T J.145 (2001), Measurement and control of the quality of service for sound transmission over contribution and distribution networks.

[b-ITU-T J.146]		Recommendation ITU-T J.146 (2002), Loop latency issues in contribution circuits for conversational TV programmes.

[b-ITU-T J.147]		Recommendation ITU-T J.147 (2002), Objective picture quality measurement method by use of in-service test signals.

[b-ITU-T J.148]		Recommendation ITU-T J.148 (2003), Requirements for an objective perceptual multimedia quality model.

[b-ITU-T J.149]		Recommendation ITU-T J.149 (2004), Method for specifying accuracy and cross-calibration of Video Quality Metrics (VQM).

[b-ITU-T J.150]		Recommendation ITU-T J.150 (1998), Operational functionalities for the delivery of digital multiprogramme television, sound and data services through multichannel, multipoint distribution systems (MMDS).

[b-ITU-T J.151]		Recommendation ITU-T J.151 (2000), RF remodulator interface for digital television.

[b-ITU-T J.160]		Recommendation ITU-T J.160 (2005), Architectural framework for the delivery of time-critical services over cable television networks using cable modems.

[b-ITU-T J.161]		Recommendation ITU-T J.161 (2007), Audio and video codec requirements and usage for the provision of bidirectional audio services over cable television networks using cable modems.

[b-ITU-T J.162]		Recommendation ITU-T J.162 (2007), Network call signalling protocol for the delivery of time-critical services over cable television networks using cable modems.

[b-ITU-T J.163]		Recommendation ITU-T J.163 (2007), Dynamic quality of service for the provision of real-time services over cable television networks using cable modems.

[b-ITU-T J.164]		Recommendation ITU-T J.164 (2007), Event message requirements for the support of real-time services over cable television networks using cable modems.

[b-ITU-T J.166]		Recommendation ITU-T J.166 (2007), IPCablecom Management Information Base (MIB) framework.

[b-ITU-T J.167]		Recommendation ITU-T J.167 (2007), Media terminal adapter (MTA) device provisioning requirements for the delivery of real-time services over cable television networks using cable modems.

[b-ITU-T J.170]		Recommendation ITU-T J.170 (2005), IPCablecom security specification.

[b-ITU-T J.171.1]	Recommendation ITU-T J.171.1 (2005), IPCablecom trunking gateway control protocol (TGCP): Profile 1.

[b-ITU-T J.171.2]	Recommendation ITU-T J.171.2 (2005), IPCablecom trunking gateway control protocol (TGCP): Profile 2.

[b-ITU-T J.172]		Recommendation ITU-T J.172 (2005), IPCablecom management event mechanism.

[b-ITU-T J.173]		Recommendation ITU-T J.173 (2005), IPCablecom embedded MTA primary line support.

[b-ITU-T J.175]		Recommendation ITU-T J.175 (2005), Audio server protocol.

[b-ITU-T J.177]		Recommendation ITU-T J.177 (2005), IPCablecom CMS subscriber provisioning specification.

[b-ITU-T J.178]		Recommendation ITU-T J.178 (2005), IPCablecom CMS to CMS signalling.

[b-ITU-T J.179]		Recommendation ITU-T J.179 (2005), IPCablecom support for multimedia.

[b-ITU-T J.180] 		Recommendation ITU-T J.180 (2000), User requirements for statistical multiplexing of several programmes on a transmission channel.

[b-ITU-T J.181] 		Recommendation ITU-T J.181 (2014), Digital program insertion cueing message for cable television systems.

[b-ITU-T J.181 Amendment 1] 	Recommendation ITU-T J.181 Amendment 1 (2014), Digital program insertion cueing message for cable television systems. Amendment 1: New Appendix II: Recommended practices for the implementation of ITU-T J.181.

[b-ITU-T J.183] 		Recommendation ITU-T J.183 (2016), Time-division multiplexing of multiple MPEG-2 transport streams and generic formats of transport streams over cable television systems.

[b-ITU-T J.184] 		Recommendation ITU-T J.184 (2001), Digital broadband delivery system: Out-of-band transport.

[b-ITU-T J.185] 		Recommendation ITU-T J.185 (2012), Transmission equipment for transferring multi-channel television signals over optical access networks by frequency modulation conversion.

[b-ITU-T J.186] 		Recommendation ITU-T J.186 (2008), Transmission equipment for multi-channel television signals over optical access networks by sub-carrier multiplexing (SCM).

[b-ITU-T J.187] 		Recommendation ITU-T J.187 (2002), Transport mechanism for component-coded digital high-definition television signals using MPEG-2 video coding including all service elements for contribution and primary distribution.

[b-ITU-T J.189] 		Recommendation ITU-T J.189 (2002), Seamless splicing for MPEG-2 bit streams.

[b-ITU-T J.190] 		Recommendation ITU-T J.190 (2007), Architecture of MediaHomeNet.

[b-ITU-T J.191] 		Recommendation ITU-T J.191 (2004), IP feature package to enhance cable modems.

[b-ITU-T J.192] 		Recommendation ITU-T J.192 (2005), A residential gateway to support the delivery of cable data services.

[b-ITU-T J.193] 		Recommendation ITU-T J.193 (2004), Requirements for the next generation of set-top-boxes.

[b-ITU-T J.195.1] 	Recommendation ITU-T J.195.1 (2016), Functional requirements for high speed transmission over coaxial networks connected with fibre to the building.

[b-ITU-T J.195.2] 	Recommendation ITU-T J.195.2 (2014), Physical layer specification for high speed transmission over coaxial networks.

[b-ITU-T J.195.3]	Recommendation ITU-T J.195.3 (2014), Medium Access Control layer specification for high speed transmission over coaxial networks.

[b-ITU-T J.196.1] 	Recommendation ITU-T J.196.1 (2016), Functional requirements for second-generation HiNoC.

[b-ITU-T J.196.2] 	Recommendation ITU-T J.196.2 (2016), Physical layer specification of second generation HiNoC.

[b-ITU-T J.196.3] 	Recommendation ITU-T J.196.3 (2016), Media access control layer specification of second generation HiNoC.

[b-ITU-T J.197] 		Recommendation ITU-T J.197 (2005), High level requirements for a Digital Rights Management (DRM) bridge from a cable access network to a home network.

[b-ITU-T J.199] 		Recommendation ITU-T J.199 (2006), Battery backup for cable-based devices.

[b-ITU-T J.200] 		Recommendation ITU-T J.200 (2010), Worldwide common core - Application environment for digital interactive television services.

[b-ITU-T J.201] 		Recommendation ITU-T J.201 (2014), Harmonization of declarative content format for interactive television applications.

[b-ITU-T J.203] 		Recommendation ITU-T J.203 (2008), Common core for digital video recorder platform.

[b-ITU-T J.204] 		Recommendation ITU-T J.204 (2008), Metrics gathering specification.

[b-ITU-T J.205] 		Recommendation ITU-T J.205 (2012), Requirements for an application control framework using integrated broadcast and broadband digital television.

[b-ITU-T J.205 Cor. 2]	Corrigendum 2 (2014) to Recommendation ITU-T J.205 (2012).

[b-ITU-T J.206] 		Recommendation ITU-T J.206 (2013), Architecture for an application control framework using integrated broadcast and broadband digital television.

[b-ITU-T J.207] 		Recommendation ITU-T J.207 (2018), Specification for integrated broadcast and broadband digital television application control framework.

[b-ITU-T J.210] 		Recommendation ITU-T J.210 (2006), Downstream RF interface for cable modem termination systems.

[b-ITU-T J.211] 		Recommendation ITU-T J.211 (2006), Timing interface for cable modem termination systems.

[b-ITU-T J.212] 		Recommendation ITU-T J.212 (2006), Downstream external Physical layer interface for modular cable modem termination systems.

[b-ITU-T J.213] 		Recommendation ITU-T J.213 (2006), Layer 2 virtual private networks for IP cable modem systems.

[b-ITU-T J.214] 		Recommendation ITU-T J.214 (2007), Cable modem TDM emulation interface.

[b-ITU-T J.215] 		Recommendation ITU-T J.215 (2007), Client digital program insertion API.

[b-ITU-T J.218] 		Recommendation ITU-T J.218 (2007), Cable modem IPv4 and IPv6 eRouter specification.

[b-ITU-T J.222.1] 	Recommendation ITU-T J.222.1 (2007), Third-generation transmission systems for interactive cable television services - IP cable modems: Physical layer specification.

[b-ITU-T J.222.2] 	Recommendation ITU-T J.222.2 (2007), Third-generation transmission systems for interactive cable television services - IP cable modems: MAC and Upper Layer protocols.

[b-ITU-T J.222.3] 	Recommendation ITU-T J.222.3 (2007), Third-generation transmission systems for interactive cable television services - IP cable modems: Security services.

[b-ITU-T J.223.1] 	Recommendation ITU-T J.223.1 (2016), Functional requirements for Cabinet DOCSIS (C-DOCSIS).

[b-ITU-T J.223.2] 	Recommendation ITU-T J.223.2 (2016), Cabinet DOCSIS (C-DOCSIS) system specification.

[b-ITU-T J.230] 		Recommendation ITU-T J.230 (2016), Requirements for platform functionalities on the integration of cable STB and companion devices including mobile second screen devices.

[b-ITU-T J.240] 		Recommendation ITU-T J.240(2004), Framework for remote monitoring of transmitted picture signal-to-noise ratio using spread-spectrum and orthogonal transform.

[b-ITU-T J.241] 		Recommendation ITU-T J.241 (2005), Quality of service ranking and measurement methods for digital video services delivered over broadband IP. Networks.

[b-ITU-T J.244] 		Recommendation ITU-T J.244 (2008), Full reference and reduced reference calibration methods for video transmission systems with constant misalignment of spatial and temporal domains with constant gain and offset.

[b-ITU-T J.245] 		Recommendation ITU-T J.245 (2008), Subjective assessment method of picture quality assessed in the home.

[b-ITU-T J.246] 		Recommendation ITU-T J.246 (2008), Perceptual visual quality measurement techniques for multimedia services over digital cable television networks in the presence of a reduced bandwidth reference.

[b-ITU-T J.247] 		Recommendation ITU-T J.247 (2008), Objective perceptual multimedia video quality measurement in the presence of a full reference.

[b-ITU-T J.248] 		Recommendation ITU-T J.248 (2008), Requirements for operational monitoring of video-to-audio delay in the distribution of television programs.

[b-ITU-T J.249] 		Recommendation ITU-T J.249 (2010), Perceptual video quality measurement techniques for digital cable television in the presence of a reduced reference.

[b-ITU-T J.260] 		Recommendation ITU-T J.260 (2005), Requirements for preferential telecommunications over IPCablecom networks.

[b-ITU-T J.261] 		Recommendation ITU-T J.261 (2009), Framework for implementing preferential telecommunications in IPCablecom and IPCablecom2 networks.

[b-ITU-T J.262] 		Recommendation ITU-T J.262 (2009), Specifications for authentication in preferential telecommunications over IPCablecom2 networks.

[b-ITU-T J.263] 		Recommendation ITU-T J.263 (2009), Specification for priority in preferential telecommunications over IPCablecom2 networks.

[b-ITU-T J.280] 		Recommendation ITU-T J.280 (2013), Digital program insertion: Splicing application program interface.

[b-ITU-T J.281] 		Recommendation ITU-T J.281 (2005), Requirements for multichannel video signal transmission over IP-based fibre network.

[b-ITU-T J.282] 		Recommendation ITU-T J.282 (2006), Architecture of multi-channel video signal distribution over IP-based networks.

[b-ITU-T J.283] 		Recommendation ITU-T J.283 (2006), IP network architecture with network layer route diversity providing resilient IP multicast video distribution.

[b-ITU-T J.284] 		Recommendation ITU-T J.284 (2007), Requirements and framework for gathering electronic content over IP-based network.

[b-ITU-T J.285] 		Recommendation ITU-T J.285 (2007), Architecture for synchronised programme transfer with pull operation over IP-based networks.

[b-ITU-T J.286] 		Recommendation ITU-T J.286 (2009), Seamless splicing for heterogeneous ITU-T H.262 | ISO/IEC 13818-2 (MPEG-2 video) and ITU-T H.264 | ISO/IEC 14496-10 bitstreams.

[b-ITU-T J.287] 		Recommendation ITU-T J.287 (2014), Automation system to compression system communications application program interface.

[b-ITU-T J.288] 		Recommendation ITU-T J.288 (2016), Encapsulation of type length value (TLV) packet for cable transmission systems.

[b-ITU-T J.290] 		Recommendation ITU-T J.290 (2006), Next generation set-top box core architecture.

[b-ITU-T J.291] 		Recommendation ITU-T J.291 (2006), Next generation set-top-box cable architecture.

[b-ITU-T J.292] 		Recommendation ITU-T J.292 (2006), Next generation set-top box media-independent architecture.

[b-ITU-T J.293] 		Recommendation ITU-T J.293 (2008), Component definition and interface specification for the next generation set-top box.

[b-ITU-T J.294] 		Recommendation ITU-T J.294 (2010), Residential gateway requirements for the support of broadcast and IP-based interactive services over cable television networks.

[b-ITU-T J.295] 		Recommendation ITU-T J.295 (2012), Functional requirements for a hybrid cable set-top box.

[b-ITU-T J.296] 		Recommendation ITU-T J.296 (2012), Specifications for a hybrid cable set-top box.

[b-ITU-T J.297] 		Recommendation ITU-T J.297 (2018), Requirements and functional specification of cable set-top box for 4K ultra high definition television.

[b-ITU-T J.301] 		Recommendation ITU-T J.301 (2014), Requirements for augmented reality smart television systems.

[b-ITU-T J.302]	 	Recommendation ITU-T J.302 (2016), System specifications of augmented reality smart television service.

[b-ITU-T J.340] 		Recommendation ITU-T J.340 (2010), Reference algorithm for computing peak signal to noise ratio of a processed video sequence with compensation for constant spatial shifts, constant temporal shift, and constant luminance gain and offset.

[b-ITU-T J.341] 		Recommendation ITU-T J.341 (2016), Objective perceptual multimedia video quality measurement of HDTV for digital cable television in the presence of a full reference.

[b-ITU-T J.342] 		Recommendation ITU-T J.342 (2011), Objective multimedia video quality measurement of HDTV for digital cable television in the presence of a reduced reference signal.

[b-ITU-T J.343] 		Recommendation ITU-T J.343 (2014), Hybrid perceptual bitstream models for objective video quality measurements.

[b-ITU-T J.343.1] 	Recommendation ITU-T J.343.1 (2014), Hybrid-NRe objective perceptual video quality measurement for HDTV and multimedia IP-based video services in the presence of encrypted bitstream data.

[b-ITU-T J.343.2] 	Recommendation ITU-T J.343.2 (2014), Hybrid-NR objective perceptual video quality measurement for HDTV and multimedia IP-based video services in the presence of non-encrypted bitstream data.

[b-ITU-T J.343.3] 	Recommendation ITU-T J.343.3 (2014), Hybrid-RRe objective perceptual video quality measurement for HDTV and multimedia IP-based video services in the presence of a reduced reference signal and encrypted bitstream data.

[b-ITU-T J.343.4] 	Recommendation ITU-T J.343.4 (2014), Hybrid-RR objective perceptual video quality measurement for HDTV and multimedia IP-based video services in the presence of a reduced reference signal and non-encrypted bitstream data.

[b-ITU-T J.343.5] 	Recommendation ITU-T J.343.5 (2014), Hybrid-FRe objective perceptual video quality measurement for HDTV and multimedia IP-based video services in the presence of a full reference signal and encrypted bitstream data.

[b-ITU-T J.343.6] 	Recommendation ITU-T J.343.6 (2014), Hybrid-FR objective perceptual video quality measurement for HDTV and multimedia IP-based video services in the presence of a full reference signal and non-encrypted bitstream data.

[b-ITU-T J.360] 		Recommendation ITU-T J.360 (2006), IPCablecom2 architecture framework.

[b-ITU-T J.360 Amendment 1] 	Recommendation ITU-T J.360 Amendment 1 (2007), IPCablecom2 architecture framework. Amendment 1: New Appendix IV - Home subscriber server (HSS).

[b-ITU-T J.361] 		Recommendation ITU-T J.361 (2006), IPCablecom2 codec and media.

[b-ITU-T J.362] 		Recommendation ITU-T J.362 (2006), IPCablecom2 control point discovery.

[b-ITU-T J.363] 		Recommendation ITU-T J.363 (2006), IPCablecom2 data collection to support accounting.

[b-ITU-T J.365] 		Recommendation ITU-T J.365 (2006), IPCablecom2 application manager interface.

[b-ITU-T J.366.0] 	Recommendation ITU-T J.366.0 (2006), IPCablecom2 IP Multimedia Subsystem (IMS): Delta Recommendations overview.

[b-ITU-T J.366.2] 	Recommendation ITU-T J.366.2 (2010), IPCablecom2 IP Multimedia Subsystem (IMS): Session handling - IM call model - Stage 2. specification

[b-ITU-T J.366.8] 	Recommendation ITU-T J.366.8 (2006), IPCablecom2 IP Multimedia Subsystem (IMS): Network domain security specification.

[b-ITU-T J.366.9] 	Recommendation ITU-T J.366.9 (2006), IPCablecom2 IP Multimedia Subsystem (IMS): Generic authentication architecture specification (3GPP TS 33.220) .

[b-ITU-T J.367] 		Recommendation ITU-T J.367 (2008), IPCablecom2 presence specification.

[b-ITU-T J.368] 		Recommendation ITU-T J.368 (2008), IPCablecom2 quality of service specification.

[b-ITU-T J.369] 		Recommendation ITU-T J.369 (2008), IPCablecom2 E-UE provisioning framework specification.

[b-ITU-T J.370] 		Recommendation ITU-T J.370 (2008), IPCablecom2 embedded user equipment provisioning data model specification.

[b-ITU-T J.380.1] 	Recommendation ITU-T J.380.1 (2011), Digital program insertion - Advertising systems interfaces - Advertising systems overview.

[b-ITU-T J.380.2] 	Recommendation ITU-T J.380.2 (2011), Digital program insertion - Advertising systems interfaces - Core data elements.

[b-ITU-T J.380.3] 	Recommendation ITU-T J.380.3 (2011), Digital program insertion - Advertising systems interfaces - Management interface.

[b-ITU-T J.380.4] 	Recommendation ITU-T J.380.4 (2011), Digital program insertion - Advertising systems interfaces - Content information service.

[b-ITU-T J.380.5] 	Recommendation ITU-T J.380.5 (2011), Digital program insertion - Advertising systems interfaces - Placement opportunity information service.

[b-ITU-T J.380.6] 	Recommendation ITU-T J.380.6 (2011), Digital program insertion - Advertising systems interfaces - Subscriber information service.

[b-ITU-T J.380.7] 	Recommendation ITU-T J.380.7 (2011), Digital program insertion - Advertising systems interfaces - Message transport.

[b-ITU-T J.380.8] 	Recommendation ITU-T J.380.8 (2011), Digital program insertion - Advertising systems interfaces - General information service.

[b-ITU-T J.381] 		Recommendation ITU-T J.381 (2012), Requirements for advanced digital cable transmission technologies.

[b-ITU-T J.382] 		Recommendation ITU-T J.382 (2018), Advanced digital downstream transmission systems for television, sound and data services for cable distribution.

[b-ITU-T J.388] 		Recommendation ITU-T J.388 (2010), Real-time video and audio transmission system over IP networks.

[b-ITU-T J.460.0] 	Recommendation ITU-T J.460.0 (2008), IPCablecom2 residential SIP telephony: Feature definition.

[b-ITU-T J.460.1] 	Recommendation ITU-T J.460.1 (2008), IPCablecom2 residential SIP telephony: Feature specification.

[b-ITU-T J.460.2] 	Recommendation ITU-T J.460.2 (2008), IPCablecom2 residential SIP telephony: embedded digital voice adapter (E-DVA) specification.

[b-ITU-T J.460.3] 	Recommendation ITU-T J.460.3 (2008), IPCablecom2 residential SIP telephony: Usage data recording.

[b-ITU-T J.460.4] 	Recommendation ITU-T J.460.4 (2008), IPCablecom2 residential SIP telephony: Embedded-user equipment provisioning specification.

[b-ITU-T J.600] 		Recommendation ITU-T J.600 (2004), Transport of Large Screen Digital Imagery (LSDI) applications that employ MPEG-2 encoded HDTV signals.

[b-ITU-T J.601] 		Recommendation ITU-T J.601 (2005), Transport of Large Screen Digital Imagery (LSDI) applications for its expanded hierarchy.

[b-ITU-T J.603] 		Recommendation ITU-T J.603 (2011), Real-time transmission system of exLSDI signals under spatial image segmentation for parallel processing.

[b-ITU-T J.604] 		Recommendation ITU-T J.604 (2014), Requirements for a scalable video transmission system over cable networks.

[b-ITU-T J.700] 		Recommendation ITU-T J.700 (2009), IPTV service requirements and framework for secondary distribution.

[b-ITU-T J.701] 		Recommendation ITU-T J.701 (2008), Broadcast-centric IPTV terminal middleware.

[b-ITU-T J.702] 		Recommendation ITU-T J.702 (2008), Enablement of current terminal devices for the support of IPTV services.

[b-ITU-T J.703] 		Recommendation ITU-T J.703 (2010), IPTV client control interface definition.

[b-ITU-T J.704] 		Recommendation ITU-T J.704 (2009), Functional requirements of the service provider interface for television primary and secondary distribution and associated interactive services.

[b-ITU-T J.705] 		Recommendation ITU-T J.705 (2011), IPTV client provisioning, activation, configuration and management interface definition.

[b-ITU-T J.706] 		Recommendation ITU-T J.706 (2012), Overview of the distribution of target-specific content.

[b-ITU-T J.707] 		Recommendation ITU-T J.707 (2012), Messages and protocols enabling the distribution of target-specific content within integrated broadband cable networks.

[b-ITU-T J.800.0] 	Recommendation ITU-T J.800.0 (2008), Cable assigned names and numbers.

[b-ITU-T J.800.1] 	Recommendation ITU-T J.800.1 (2008), Cable DHCP Registry.

[b-ITU-T J.800.2] 	Recommendation ITU-T J.800.2 (2008), Cable definition MIB specification.

[b-ITU-T J.900] 		Recommendation ITU-T J.900 (2014), Requirements for stereoscopic three-dimensional television service over hybrid fibre and coaxial based networks.

[b-ITU-T J.901] 		Recommendation ITU-T J.901 (2008), Requirements for the free viewpoint television (FTV) video transmission system.

[b-ITU-T J.902] 		Recommendation ITU-T J.902 (2012), Multilayered data structure for scalable view-range representation.

[b-ITU-T J.1001] 	Recommendation ITU-T J.1001 (2012), Requirements for renewable conditional access system.

[b-ITU-T J.1002] 	Recommendation ITU-T J.1002 (2013), Pairing protocol specification for renewable conditional access system.

[b-ITU-T J.1003] 	Recommendation ITU-T J.1003 (2014), Specifications of network protocol for renewable conditional access system.

[b-ITU-T J.1004] 	Recommendation ITU-T J.1004 (2015), Specifications of authorization centre interfaces for renewable conditional access system.

[b-ITU-T J.1005] 	Recommendation ITU-T J.1005 (2015), Architecture and requirements of digital rights management (DRM) for cable television multiscreen

[b-ITU-T J.1006] 	Recommendation ITU-T J.1006 (2016), Specification of IP-VoD DRM for cable television multiscreen system in multi-DRM environment.

[b-ITU-T J.1010] 	Recommendation ITU-T J.1010 (2016), Embedded common interface for exchangeable CA/DRM solutions; Use cases and requirements.

[b-ITU-T J.1011] 	Recommendation ITU-T J.1011 (2016), Embedded common interface for exchangeable CA/DRM solutions; Architecture, definitions and overview.

[b-ITU-T J.1020] 	Recommendation ITU-T J.1020 (2017), Service model and architecture of downloadable mobile multi-CA/DRM solutions for delivering CA/DRM client software to secondary device.

[b-ITU-T J.1101] 	Recommendation ITU-T J.1101 (2012), Functional requirements for IP-based switched digital video using data over cable service interface specifications.

[b-ITU-T J.1102] 	Recommendation ITU-T J.1102 (2015), Interface specifications for IP-based switched digital video using DOCSIS.

[b-ITU-T J.1103] 	Recommendation ITU-T J.1103 (2015), Transmission specification for IP-based switched digital video using data over cable service interface specifications.

[b-ITU-T J.1104] 	Recommendation ITU-T J.1104 (2016), Control specification for IP-based switched digital video using data over cable service interface specifications.

[b-ITU-T J.1105] 	Recommendation ITU-T J.1105 (2016), Requirement of channel switching service over hybrid fibre and coaxial based network.

[b-ITU-T J.1106] 	Recommendation ITU-T J.1106 (2017), Requirement for radio over IP transmission system.

[b-ITU-T J.1107] 	Recommendation ITU-T J.1107 (2018), Architecture and specification for Radio over IP transmission systems.

[b-ITU-T N.1]		Recommendation ITU-T N.1 (1993), Definitions for application to international sound-programme and television-sound transmission.

[b-ITU-T N.13]		Recommendation ITU-T N.13 (1988), Measurements to be made by the broadcasting organizations during the preparatory period.

[b-ITU-T N.51]		Recommendation ITU-T N.51 (1988), Definitions for application to international television transmissions.

[b-ITU-T N.54]		Recommendation ITU-T N.54 (1988), Definition and duration of the line-up period and the preparatory period

[b-ITU-T N.62]		Recommendation ITU-T N.62 (1993), Tests to be made during the line-up period that precedes a television transmission

[b-ITU-T P.910]		Recommendation ITU-T P.910 (2008), Subjective video quality assessment methods for multimedia applications.

[ITU-T Q.1741.1]	Recommendation ITU-T Q.1741.1 (2002), IMT-2000 references to release 1999 of GSM evolved UMTS core network with UTRAN access network.

[b-ITU-T X.500]		Recommendation ITU-T X.500 (2016)| ISO/IEC 9594-1 (2008), Information technology – Open Systems Interconnection – The Directory: Overview of concepts, models and services.

[b-ITU-T X.509]		Recommendation ITU-T X.509 (2005) | ISO/IEC 9594-8V (2005), Information Technology – Open Systems Inter-connection – The Directory: Authentication Framework.

[b-ITU-T X.690]		Recommendation ITU-T X.690 (2002) | ISO/IEC 8825-1 (2002), Information Technology - ASN.1 Encoding Rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER).

[b-ITU-T Y.2111]	Recommendation ITU-T Y.2111 (2011), Resource and admission control functions in next generation networks.

[b-MCNS3]  	DataOver-Cable Service Interface Specifications, Cable Modem Termination System Network Side Interface Specification, SP-CMTS-NSI-I01-960702

[b-MCNS4]	Data-Over-Cable Service Interface Specifications, Cable Modem to Customer Premises Equipment Interface Specification, SP-CMCI-I01-960702.

[b-MHEG Profile]	Digital Terrestrial Television MHEG-5 Specification (1998).
<http://www.mheg.org/users/mheg/archives/doc/MHEG-5_Profile_Issue_1.pdf>

[b-MHP]		ETSI TS 102 727 v1.1.1, Digital Video Broadcasting (DVB); Multimedia Home Platform (MHP) Specification 1.2.2.

[b-NCL]		Nested Context Language 3.0, Part 8 – NCL Digital TV Profiles, L.F. Gomes Soares and R. Ferreira Rodrigues (2006), ISSN 0103-9741. <http://www.ncl.org.br/documentos/NCL3.0-DTV.pdf>

[b- HOST2.0]	OC-SP-HOST2.0-CFR-I14-070615 (2007):OpenCable Host Device 2.0 Core Functional Requirements, Cable Television Laboratories, Inc.

[b-OMA RD-PRS]	OMA-RD-Presence_SIMPLE-V1_0-20060725-A, Presence SIMPLE Requirements, July 2006, Open Mobile Alliance.

[b-SCTE 5]			ANSI/SCTE 79-2 (2002), DOCSIS 2.0, Operations Support System Interface.

[b-SCTE 22-1]		ANSI/SCTE 22-1 (2002), Data-Over-Cable Service Interface Specification, DOCSIS 1.0 Radio Frequency Interface (RFI).

[b-SCTE 22-2] 		ANSI/SCTE 22-2 (2002), DOCSIS 1.0, Baseline Privacy Interface

[b-SCTE 22-3]		ANSI/SCTE 22-3 (2002), DOCSIS 1.0 Part 3: Operations Support System Interface.

[b-SCTE 23-3] 		ANSI/SCTE 23-3 (2003), DOCSIS 1.1 Part 3: Operations Support System Interface.

[b-SCTE 35] 		ANSI/SCTE 35 (2007), Digital Program Insertion Cueing Message for Cable.

[b-SCTE 79-2]		ANSI/SCTE 79-2 2002, DOCS 2.0 Operations Support System Interface.

[b-SMS] 			The Spectrum Management Application (SMA) and the Common Spectrum Management Interface (CSMI), Time Warner Cable, December 24, 1995.

[b-Thomson, 1952]	THOMSON, W. E. [1952] “Solution 3”. Proc. IEE, Part III, 99, 373.

[b-W3C HTML]		W3C Recommendation (1999), HTML 4.01 Specification . <http://www.w3.org/TR/1999/REC-html401-19991224>

[b-W3C XML]		W3C Recommendation (1998), Extensible Markup Language (XML), Version 1.0. <http://www.w3.org/TR/1998/REC-xml-19980210>

_______________________