|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  | SG12-C230R1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 12 |  |  |
|  |  |  | Original: French |  |  |
| Question(s): |  | All/12 |  | Geneva, 27 November–6 December 2018 |  |
| CONTRIBUTION |  |  |  |  |  |
| Source: |  | Comoros |  |  |  |
| Title: |  | Homologation of mobile phones as means of improving QoE |  |  |  |
| Purpose: |  | Proposal |  |  |  |
| Contact: |  | Djinti Ahamada Autorité Nationale de Régulation des Technologies de l’Information et de la Communication (ANRTIC) Comoros |  |  | Tel: +2693370892 Fax: +2697738762 E-mail: djintia@yahoo.fr |

| Keywords: | QoE; mobile phones; homologation; approval |
| --- | --- |
| Abstract: | The purpose of this contribution is to show how the certification (“homologation”) of mobile phones in the Union of the Comoros can help to improve the QoE of telecommunication services. |

Background

The telecommunication sector of the Union of the Comoros was a State monopoly for a number of years. The incumbent operator, Comores Télécom, was solely responsible for the provision and operation of telecommunication services. Only in December 2015 was the sector liberalized. Once the market was opened to competition, the Union of the Comoros saw a dramatic increase in rates of penetration and network coverage. Despite the impact of competition, however, quality of experience (QoE) was deemed substandard. Field research by the regulator ANRTIC revealed that many consumers use counterfeit mobile phones that do not comply with international standards, having never been subject to certification.

Issue

One of the primary missions of the Comorian Autorité Nationale de Régulation des TIC (ANRTIC), as guardian of consumer rights, is to monitor operators’ quality of service (QoS) and QoE. Surveys are carried out annually to determine levels of consumer satisfaction with services provided by operators.

Under their respective terms of reference, the operators Comores Télécom and Telma Comores have no obligations as regards the sale of mobile phones, and indeed in 2013 Comores Télécom launched a 3G network without having any 3G-friendly phones on offer. When it launched its 4.5G network in August 2018, it adopted the same strategy, considering it unnecessary to sell phones adapted to the new service.

Its competitor, Telma Comores, opted to include the sale of 3G and 4G mobile phones in its business strategy when it began operating in 2016. A large proportion of consumers consider the phones to be somewhat overpriced in terms of value for money.

Consequently, a large proportion of Comorian consumers acquire phones either from the French diaspora or at very low prices in the streets of Moroni.

The latter route usually involves counterfeit phones coming, for the most part, from neighbouring countries and entering the domestic market without any form of certification or approval check. As they can be sold very cheaply and easily, vendors prefer to buy this type of terminal for sale on the national market.

Methodology adopted

The methodology used to identify counterfeit mobile phones comprises two steps: first, awareness raising; and second, testing.

–	Awareness raising phase: the methods used involved traditional hands-on inspection of phones to determine whether they are counterfeit. This meant convincing consumers of the importance of the initiative. Most consumers were in any case curious to know the status of their phones and were reassured that, whatever the outcome, we did not have the right to confiscate their devices.

–	Testing phase: if the owner consents, a two-check test is carried out, involving verification of model and IMEI numbers.

Verification of model number: if a phone being checked (e.g. an iPhone 5) exists in the ANRTIC database as an approved model, its model number is checked against the approved model number. Any inconsistency means that the phone is counterfeit.

Verification of IMEI number: this involves checking the validity of the IMEI number by entering *#06# on the keypad. Once the IMEI number is displayed, it is entered and checked on the www.imei.info website. If the phone is counterfeit, the phone’s IMEI will not correspond to the phone listed on the website.

The use of counterfeit phones on operators’ networks will inevitably have a negative impact on the assessment of QoE, given the large numbers of consumers using counterfeit phones that are non-compliant with international standards.

Solution

To counter this scourge, which stunts the development of the telecommunication sector and represents a loss of earnings for operators, the Minister of Posts and Telecommunications and of Promotion of New Information and Communication Technologies signed a decree on conditions and procedures for the approval of terminal equipment in the Union of the Comoros. It requires the approval of every mobile phone (except for a single unit of quantity) prior to entry into the country. The regulator is responsible for approving these terminals and, working in collaboration with the customs services at borders, ensures that a certificate of approval or “homologation” is presented before the terminals clear customs. This measure is helping today to limit the entry of a large number of counterfeit phones into the Comoros and prevent distortions of the balance between QoS and QoE. In principle, this should be an efficient way of improving QoE. A study is required to confirm this improvement.

Proposals

We propose that ITU-T SG 12 introduce this restrictive measure as a means of improving QoE.

We propose that ITU-T SG 12 consider this contribution with a view to accelerating work on the development of an ITU-T Recommendation.

______________

____________________