|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  |  |  | SG12-LS70 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | STUDY GROUP 12 |  |
|  |  |  |  |  | Original: English |  |
| Question(s): |  |  | 17/12 |  | Geneva, 27 November - 6 December 2018 |  |
| Ref.: SG12-TD719 |  |  |  |  |  |  |
| Source: |  |  | ITU-T Study Group 12 |  |  |  |
| Title: |  |  | LS on harmonization of IP Capacity and Latency Parameters: Revision of Draft Rec. Y.1540 on IP packet transfer performance parameters and New Annex A with Lab Evaluation Plan |  |  |  |
| LIAISON STATEMENT |  |  |  |  |  |  |
| For action to: |  |  |  | ITU-T SG11; ETSI INT |  |  |
| For comment to: |  |  |  | TSAG; ETSI STQ; IETF IPPM WG; BBF |  |  |
| For information to: |  |  |  |  |  |  |
| Approval: |  |  |  | ITU-T SG12 meeting (Geneva, 6 December 2018) |  |  |
| Deadline: |  |  |  | March 2019 |  |  |
| Contact: |  | A.C. Morton AT&T USA |  |  |  | Tel: +1 723 420 1571 E-mail: acmorton@att.com |

| Keywords: | IP Packet Transfer Performance |
| --- | --- |
| Abstract: | Further Progress on Evaluating IP Capacity Metrics and Methods for Revised Recommendation Y.1540, “Internet protocol data communication service – IP packet transfer and availability performance parameters” |

As an update to our communications from the Q17/12 Interim meeting (Darmstadt, 16-17 October 2018) we now share further results from our Geneva, 27 November – 6 December 2018 meeting with the full SG 12. We have made operational further progress with our test bed that supports our Y.1540/Annex A Evaluation Plan for IP Capacity and Latency methods of measurement.

The first test bed was demonstrated for the Q17 meeting. The T-rex measurement tool and iPerf 2 test tool are now running reliably, and this allows us to test with both TCP connections and UDP streams with iPerf 2.

Calibrated rates of 100, 200, 300, and 500 Mbps were tested. The UDP benchmark accurately assessed the case with a 500 Mbps traffic shaper when accounting for the fact that iPerf2 reports UDP payload in the results. The headers of Ethernet, IP, and UDP (18+20+8 octets) add another 3.0% on 1470 byte packet payloads, yielding 500.5 Mbps. The tests with 3 TCP connections (iPerf2) produced measured rates that were nominally 5% lower than the 200/300/500 Mbps traffic shaper settings and the UDP benchmark; they have not been corrected for the Ethernet, IP and TCP header overhead, but this is the typical report displayed to users. Even with correction for the header overhead, the combined TCP connections appear to underestimate the calibrated rate targets.

Q17/12 has successfully recruited new participants to develop our evaluation plan and Lab work, and the invitation remains open in the near-term.

Attachment:

SG12-TD619R1: Revisions to Y.1540 and text of new Annex A with Initial Evaluation Plan

_______________________