|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  |  | SG2-TD1452R2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 2 |  |
|  |  |  |  | Original: English |  |
| Question(s): |  |  | 5/2 | Virtual, 31 May - 11 June 2021 |  |
| TD |  |  |  |  |  |
| Source: |  |  | Rapporteur, Q5/2 |  |  |
| Title: |  |  | Telecommunications management and OAM project plan |  |  |
| Purpose: |  |  | Information |  |  |
| Contact: |  | Dmitry Cherkesov Russian Federation |  |  | Tel: +7 985 239 06 00 E-mail: dcherkesov@gmail.com |

| Keywords: | Telecommunications management OAM terms definitions |
| --- | --- |
| Abstract: | This TD contains the last June 2020 version of the Telecommunications management and OAM Project Plan |

Telecommunications management and OAM project plan

(June 2021)

1.	Scope	3

2.	References	3

3.	Terms and Definitions	3

4.	Abbreviations	4

5.	Overview	4

6.	Telecommunication Management and OAM Target Domains	5

7.	Telecommunication Management and OAM Business Processes and Functional Requirements	5

8.	Telecommunication Management and OAM Principles, Frameworks, and Architecture	7

9.	Telecommunication Management Interfaces	8

10.	OAM Interfaces	9

11.	Hot topics for current and nearest future activities within TM&OAM PP	9

12.	Maintenance of document information	9

13.	Notes for completing the table in Annex 1	10

13.1.	Document status column	10

13.2.	Document classification column	10

13.3.	Publication date or planned date for SG consent column	10

13.4.	Dependency column	11

14.	Contact Information	11

Annex 1 – List of Documents within scope of the Telecommunications management and OAM project	16

Annex 2 – List of Terms and Definitions within scope of the Telecommunications management and OAM project	16

Annex 3 – List of Abbreviations within scope of the Telecommunications management and OAM project	16

Annex 4 – Tasks	17

Annex 5 – Authors Team	19

Introduction

- Scope

Telecommunication management and OAM activities include the management of telecommunication services, networks, and equipment, by means of designing generic and technology-specific principles, frameworks, (methodology), architectures, functional requirements, information models, and protocols for the specification of management and OAM interfaces between network elements, between network elements and management systems, and also between management systems.

A coordination and communication activity among the involved Study Groups and Questions is fundamental in order to facilitate the most efficient completion of the work. This activity includes the identification of development directions, missing work areas and those that are overlapping, or potentially so, between multiple Study Groups, Questions and Recommendations. It also includes encouraging the relevant work items in the most appropriate Questions, helping to define a suitable time schedule, and monitoring its development in a consistent way. This should allow a proper integration of the project information coming from telecommunications management and OAM projects, and eventually should ensure the mutual consistency of the standards prepared by the various ITU bodies, on this subject.

The scope of this Project is to define - what aspects of telecommunication management and OAM are under development or need to be assigned to what Study Groups or Questions for timely development?

The fulfilment of this scope includes, but is not limited to perform following tasks:

Develop, maintain, and regularly distribute a Telecommunication Management and OAM Project Plan that:

- Documents the work and time schedules of all major stable and new telecommunications management and OAM activities;

- Identifies the directions for development, missing work areas and those that are overlapping, or potentially so, between multiple Study Groups, Questions and Recommendations;

- Encouraging the relevant work items in the most appropriate ITU Questions and monitoring its development in a consistent way.

- References

Telecommunication Management and OAM Project Plan is a living document. It may be updated between SG meetings as result of Q5 interim meetings. The latest agreed version can be found in

https://www.itu.int/en/ITU-T/studygroups/2017-2020/02/Pages/default.aspx – Telecommunication Management and OAM Project Plan.

- Terms and Definitions

Telecommunications Management - tbd

Operation, administration and maintenance – tbd

- Abbreviations

FCAPS		Fault, configuration, accounting, performance and security management

NGN			Next Generation Network

OAM			Operation, administration and maintenance

TM			Telecommunication management

TMN			Telecommunication management network

TM&OAM PP		Telecommunication Management and OAM Project Plan

WTSA			World Telecommunication Standardization Assembly

- Overview

At WTSA-04, SG4 was designated the Lead Study Group for Telecommunication Management. This role is a carryover and an expansion of its role as Lead Study Group for TMN held during the previous Study Period which was supported by the TMN Documentation Plan.

Concurrently, WTSA-04 also approved Q12/4 which is responsible for developing, maintaining, and regularly distributing an overview and/or work plan for the Telecommunication Management and OAM Project [this document - hereafter called the Telecommunication Management and OAM Project Plan (TM&OAM PP)].

SG 4 subsequently assigned its Lead SG responsibility to Q12/4 to be supported by the TM&OAMPP which has replaced the TMN Documentation Plan and its successor, the Telecommunication Management Documentation Plan.

At WTSA-08 SG4 was merged into SG2 and SG2 continued this project in Q12/2.

At WTSA-12 Q12/2 was merged into Q5/2 and now SG2 continues this project in Q5/2.

At WTSA-16 has been decided that Q5/2 will continue to maintain TM&OAM PP.

As noted in Resolution 2 WTSA-16, some aspects of telecommunications management and OAM are being developed in other Study Groups. Telecommunication management and OAM standardization activities include generic and technology-specific architectures, functional requirements, information models, and protocols for the specification of management and OAM interfaces between network elements, between network elements and management systems, and also between management systems. These interfaces may be divided into two categories:

- Telecommunication Management interfaces: between network elements and management systems; and also between management systems. [also known as FCAPS (fault, configuration, accounting, performance, and security management) interfaces]

- OAM interfaces: between network elements [also known as transmission interfaces]

These activities also include the standardization of OAM functionality within network elements.

More specifically, these ITU-T activities include the management of telecommunication services, networks, and equipment, including support for next generation networks (NGN) and the application and evolution of the telecommunication management network (TMN) framework. Also included are other telecommunication management studies relating to designations, transport-related operations procedures, and test and measurement techniques and instrumentation.

All types of telecommunication networks and network elements - such as components of the NGN service and transport strata, analogue networks, digital networks, public networks, private networks, switching systems, transmission systems, telecommunication software, and logical resources of the network (such as a circuit, path, or telecommunication services supported by these resources) - are subject to management and OAM and such activities should be identified and tracked.

- Telecommunication Management and OAM Target Domains

This clause summarizes the telecommunication equipment, network, and service domains that are the target of ITU-T Telecommunication Management and OAM.

Telecommunications services are provided by telecommunications networks which are built from telecommunications equipment. The target domains of the ITU-T Telecommunications and OAM project comprise these three main categories: equipment, networks and services. The most important characteristics of these domains are:

Equipment:

Equipment forms the physical layer of a telecommunications network. Equipment consists of hardware, called network elements, which are connected by cables which carry the telecommunication signals in a way appropriate to the physical medium employed between the network elements. Network elements are usually highly sophisticated and versatile products which need to be properly configured in order to provide their designated role in a telecommunications network. The main management areas for equipment are fault management and configuration management.

Networks:

The transport capabilities provided by the interconnected equipment are usually described in terms of networks. Broadly speaking, a network consists of access points, fixed connections, and flexible connections. The payload signal to be transported is offered by the client at one access point where it is prepared for the transport over the intermediate links and connections to the other access point. At this point, the payload signal becomes available again to the client. Networks usually provide means to monitor the integrity and the quality of the transport the provide. The management of the network is mainly concerned with the configuration of the flexible connections and the processing of the information about the integrity and the quality of the transport. Therefore fault management, configuration management and performance management are relevant management areas for the operation of networks.

Services:

The term service represents the telecommunication capabilities that the customer buys or leases from a service provider. It is an abstraction of the equipment-oriented or network-oriented views. Identical services can be provided by different networks, and different services can be provided by the same network. Management of services implies: setting up and tearing down services, assuring the agreed service level to the customers, and keeping track of the use of the services by the customers. Hence, the management areas which are important for service management are: fault management, configuration management, accounting management, performance management, and security management.

- Telecommunication Management and OAM Business Processes and Functional Requirements

This clause describes the Recommendations that specify the business processes and functional requirements that relate to Telecommunication Management and OAM.

ITU-T SG2 describes booth the Business processes and the Functional Requirements for Telecommunications management.

In order to support the wide variety of management activities including planning, installation, operations, administration, maintenance, and provisioning of telecommunications networks and services, the concepts of Telecommunications Managed Areas and TMN Management Services were developed and are described in Recommendation M.3200.

The Telecommunications Managed Areas relate to the grouping of telecommunications resources being managed and the Management Services relate to the set of processes needed to achieve business objectives (i.e., Management Goals). It has been accepted that flexibility is required in the definition of TMN Management Services to enable additional requirements to be accommodated as they are identified (see M.3200).

The TMN Management Services (M.3200) are described from the TMN users' perspective and are independent of the protocols, messages, and information models chosen. The management services are comprised of management functions and specify offerings fulfilling specific Telecommunications Management needs.

Management Functions are grouped together and referred to as a Management Function Set. The library of general Management Function Sets and their Management Functions members can be found in Recommendation M.3400, categorized according to their FCAPS application

The business process approach has built on the concepts of management services and functions in order to develop a reference framework for categorizing all the business activities that a service provider will use. This is done through a business-oriented definition of each area of business activity, in the form of a process view that describes the service provider's enterprise in a top-down, structured way with progressive decomposition to expose increasing detail. The individual process elements that are identified can then be positioned within a model for analysis of organizational, functional and other relationships, and can be combined within process flows that trace activity paths through the business.

The vehicle for expressing this process view of the service provider is the Enhanced Telecom Operations Map® (eTOM) Business Process Framework, developed by the TeleManagement Forum. In the context of TMN, the eTOM framework provides the business-oriented view of service provider requirements that management services and functions need to support, and the mapping from individual eTOM processes to management functions, and vice versa, is documented to assist and support the application of both these processes and functions within management solutions.

The TMN management services/functions can be related to the eTOM business processes, but they provide a different perspective on the management environment.

Figure 1 shows the relationship between the Management Service/Function approach from M.3200 / M.3400 and the Business Process approach documented in M.3050.

Figure 7-1 – Relationship between the Management Service/Function and Business Process approaches

- Telecommunication Management and OAM Principles, Frameworks, and Architecture

This clause describes the Recommendations that specify Telecommunication Management and OAM principles, frameworks, and architectures.

Telecommunication Management Principles, Frameworks, and Architectures are described in M.3010 series documents and M.3060 (for NGN Management).

The TMN architecture is described in Recommendation M.3010, "Principles for a Telecommunications Management Network". Three basic aspects are included in the TMN architecture.

These are:

–	the TMN functional architecture;

–	the TMN information architecture; and

–	the TMN physical architecture.

The TMN functional architecture describes the appropriate distribution of functionality within the TMN, appropriate in the sense of allowing for the creation of function blocks from which a TMN of any complexity can be implemented. The definition of function blocks and reference points between them leads to the requirements for the TMN-recommended interface specifications.

The TMN information architecture is based on standardised open management paradigms that support the standardised modelling of the information to be communicated. TMN standardisation activities will not develop a specific management paradigm but build upon industry recognised solutions, focusing primarily on object-oriented techniques. Specific management paradigms may be used in TMN standards when judged to be adequate.

The TMN physical architecture describes interfaces that can actually be implemented and examples of physical components that make up the TMN.

M.3060 presents the management requirements, general principles and architectural requirements for managing Next Generation Networks (NGN) to support business processes to plan, provision, install, maintain, operate and administer NGN resources and services.

This Recommendation defines concepts of Next Generation Networks Management (NGNM) architectures (business process architecture, functional architecture, information architecture, and physical architectures) and their fundamental elements.

This Recommendation also describes the relationships among the architectures and provides a framework to derive the requirements for the specification of management physical architectures from the management functional and information architectures. A logical reference model for partitioning of management functionality, the Logical Layered Architecture (LLA), is provided

- Telecommunication Management Interfaces

This clause describes the Recommendations for specifying specific implementable Telecommunication Management interfaces in M.3020 terms: requirements, analysis (protocol-neutral information models), and design (protocols and protocol-specific information models). The focus is on interfaces between network elements and management systems and also between management systems.

Telecommunication Management Interfaces are designed following a management interface methodology found in M.3020. The methodology consists of three phases, requirements, analysis and design. After identifying management interfaces subject to design, requirements on the management interface are described by using a requirement template. Unified Modelling Language (UML) technique is used in the next analysis phase to identify managed entity and associated characteristics. Entities associated with attributes and behaviour composes a protocol neutral information model. The protocol neutral information model does not rely on the management protocol used on the real management interfaces and is the foundation to be used for various management protocol specific management information design. In the design phase, the protocol neutral information is converted to a management protocol specific information model by deeply considering capabilities of the management protocols. Specific management protocols are CMIP, CORBA/IIOP, SNMP, etc. Management protocol applied for Q and X interfaces of management systems are described in Q.811 and Q.812. To develop revised M.3020, extensive harmonization work on methodology were took place between ITU-T SG4 and 3GPP SA5. The revised M.3020 is now technically common both in ITU-T and 3GPP SA5.

Management information models developed in analysis phase and design phase are mostly reusable. So as to increase the reusability of management information definition, there are several common management information models which are widely used among application specific management interfaces. The application specific management interface is used to manage access and transport networks of specific systems, such as BPON, Ether over transport, etc. While, common management interfaces are commonly used among these application specific interfaces, such as log, event, alarm record, etc.

For management interface using CMIP, management information model are developed by GDMO, Guidelines for Definition of Managed Objects, X.722. And M.3100, Generic Network Information Model is referenced to develop further specific managed object classes. X.730 series Recommendations provide common management services can be used on the management information.

For management interface using CORBA/IIOP, there are three design patterns, fine grain approach, coarse grain approach and service oriented approach, following guiding Recommendations X.780, X.780.1 and X.780.2, respectively. In this environment, Q.816, Q816.1 and Q.816.2 indicate relevant applicable CORBA services in corresponding design pattern. M.3120 is a generic information model for CORBA/IIOP that corresponds to M.3100.

For management interface using XML/SOAP, there are not yet concrete design patterns. Currently, other forums and consortia works may be referenced. The guide of XML use is described in M.3030 Telecommunication Markup Language (tML) framework.

There are not a few management interface Recommendations under ITU-T SG2 responsibility. Those are identified through Recommendation section of ITU-T SG2 homepage of the following:

https://www.itu.int/en/ITU-T/studygroups/2017-2020/02/Pages/q7.aspx

- OAM Interfaces

[Editor's Note: This clause will describe the Recommendations for specifying specific OAM implementable interfaces between network elements.] [SG15 + SG13](?)

- Hot topics for current and nearest future activities within TM&OAM PP

- M.resm-AI – Requirements for energy saving management of 5G RAN system with AI (TD1413) in SG2

- X.785 (X.rest) – Guidelines for the definition of REST-based managed objects and management interface (TD1435r1) in SG2

- Q.rest – REST-based management services (C308) in SG2

- M.3365 (ex M.rvqms) – Requirements for QoE management of video in visual surveillance (TD1427R1) in SG2

- X.rest-ics  –  Guidelines for implementation conformance statement proformas associated with REST-based management systems (TD1436) in SG2

- M.rwop-AI  –  Requirements for work orders processing in telecom management with AI (TD1426) in SG2

- M.rla-AI  –  Requirements for Log Analysis with AI-enhanced Management System (TD1418r3) in SG2

- M.rmnoc-AI  –  Requirements for the management of network operation cost within AITOM in telecom operational aspects (TD1433r1) in SG2

- M.rfmls  –  Management Requirements for Federated Machine Learning Systems (TD1434r1) in SG2

- M.il-AITOM  –  Intelligence Levels of AI enhanced Telecom Operation and Management (TD1420r1) in SG2

- Maintenance of document information

Review comments and additions to this project should be sent to the project coordination team. Changes and enhancements should be shown in a colour on a copy of the appropriate page(s) in the change tracking mode (MS Word). Where a new document is to be added to the plan the full text to complete the table row should, as far as possible, be provided.

- Notes for completing the table in Annex 1

The Annex 1 lists all the identified documents (Recommendation/Standards), including those in force (as documented on ITUDOC) and those under development or revision, applicable to the TM and OAM project plan. Each document is described and classified according to one or more generic classifications that are related to the TMN methodology. Where a document is dependent on the completion or at least significant progress of another document within the plan, then this is also indicated. Dates shown are planned dates or approved and published dates.

  - Document status column

The Document status column is used to identify how advanced the current study is. It captures the current state of completion of the document being described. States are represented as follows:

0	Document planned - an actual document does not exist.

1	Work underway; a paper available; but full draft text is not yet available.

2	Working Party draft text is available (i.e. delayed or temporary document status).

3	Draft Recommendation or Standard is available.

4	Standard or Recommendation is available (if a future date for publication is shown then it should be taken as an estimated one).

5	Standard or Recommendation is available, but a revised version is planned or in progress. In this case the revised version will be separately classified as either 0, 1, 2 or 3 if a document exists, otherwise left blank.

77	Standard or Recommendation is ready for (or under) AAP or TAP procedure

  - Document classification column

Each Recommendation/Standard is classified by the subject area of contribution it makes. Some documents will provide contributions to multiple areas. For these, all significant areas of contribution are shown in order of the magnitude of that contribution.

The areas are defined and denoted as follows:

A	Architecture and principles;

C	Conformance requirements;

E	Management services;

F	Functional requirements (protocol independent);

PN	Protocol-neutral Management information models;

P	Protocols;

PS	Protocol specific Management information models;

S	Specification methodologies;

H	Other

  - Publication date or planned date for SG consent column

A "planned date for SG consent" is the date for consent on the SG level. The plan date is an estimate although in the case of status "3" may generally represent an accurate position.

  - Dependency column

This lists all the documents, generally draft Recommendations or Recommendations, that the subject document (i.e. the document in column 1) is dependent on or influent to for some aspect of specification. For example all documents that specify management information are dependent on X.722 (GDMO). Information in the "Dependency" column has not been included in this issue, except for some M-series Recommendations.

- Contact Information

# Project coordination team

| TSB Secretariat Jie Zhang SG2 Adviser ITU/TSB Place des Nations 1211 Geneva 20 Switzerland Tel: +41 22 730 5855 Fax: +41 22 730 5853 E-mail: jie.zhang@itu.int | Working Party 2 Chairman Wang Zhili, MIIT/BUPT China Tel: +86 10 61198090 ext 8726 Fax: +86 10 6228 3119 Email: zlwang@bupt.edu.cn | Q.5/2 Rapporteur Zhao Ping, China Telecom Cor. China Tel: +86 20 3863 9086 Email:zhaop@gsta.com Acting Editor Mr. Dmitry V. Cherkesov Russian Federation Tel: +7 985 239 06 00 Email: dcherkesov@gmail.com |
| --- | --- | --- |

Contact information in appropriate ITU-T Study Groups

| Coordination body | Question | Contact information | Area of interests |
| --- | --- | --- | --- |
| SG2 | Q.3 (Service and Operational Aspects of Telecommunication, including Service Definitions) | Hossam Abd El Maoula Saker National Telecommunication Regulatory Authority (NTRA) Smart Village, Building No. 4 Alex Desert Road, Kilo 28 6th of October Governorate CAIRO Egypt Fax: +20235344155 Email: HabdElmaoula@tra.gov.eg |  |
|  | Q.5 (Requirements, priorities and planning for telecommunication management and OAM Recommendations) | Ping Zhao (Rapporteur) China Telecom 109 West Zhongshan Ave, Tianhe District Guangzhou China Tel: +86 20 3863 9086 Fax: +86 20 3863 9277 Email: zhaop@gsta.com |  |
|  | Q.6 (Management Architecture and Security) | Chen Qiaogang (Rapporteur) ZTE China Tel: +86 755 2677 3000 ext 7313 Fax: +86 755 2573 7515 Email: chen.qiaogang@zte.com.cn |  |
|  | Q.7 (Interface specifications and specification methodology) | Zhili Wang (Rapporteur), BUPT - National Laboratory of Switching and Networks, Mail Box 198 Beijing University of Posts and Telecommunications 10 Xi Tu Cheng Road BEIJING 100876 China Tel: +86 10 6228 3119 ext. 8516 Fax: +86 106228 3412 Email: zlwang@bupt.edu.cn |  |
| SG9 | Q.1(Transmission and delivery control of television and sound programme signal for contribution, primary distribution and secondary distribution) | Kei Kawamura KDDI Corporation 3-10-10 Iidabashi, Chiyoda-ku, Tokyo 102-8460 Japan Email: ki-kawamura@kddi.com |  |
| SG9 | Q.6 (Functional requirements for residential gateway and set-top box for the reception of advanced content distribution services) | Mr SHIZHU LONG SHENZHEN Skyworth Digital Technology Limited China Tel: +86-13602517949 Email: longshizhu@skyworth.com | content mngmt capability for tomorrows STB, tracking MIBs from J.191 |
| SG9 | Q.7 (Cable television delivery of digital services and applications that use Internet protocol (IP) and/or packet-based data over cable networks) | Tae Kyoon Kim ETRI, Korea (Rep.of) Broadcasting and Telecommunications Convergence Research Lab. Broadcasting System Research Dept./Digital CATV System Research Team 138 Gajeongno, Yuseong-gu, Daejeon 305-700 Korea (Rep. of) Tel: +82 42 860 6917 Fax: +82 42 860 6465 Email: tkkim@etri.re.kr |  |
| SG11 |  |  |  |
| SG12 |  | Feng QI (BUPT, China) qifeng@bupt.edu.cn | Relationship with SG2 on management aspect |
|  | Q12 (Operational aspects of telecommunication network service quality) | Mrs. Yvonne Umutoni (RURA, Rwanda, yvonne.umutoni@rura.rw) and Mr. Tiago Sousa PRADO (ANATEL, Brazil, tiago.prado@anatel.gov.br). | Relationship with SG2 on management aspect |
| SG13 | Q.3 (Functional architecture for NGN evolution (NGN-e) including support of IoT and use of software-defined networking) |  | General Reference Model of the NGN, Functional Requirements and Architecture of the NGN |
|  | Q.9 (Mobility management (including support for software-defined networking) |  | IMT2020 Management |
| SG15 | Q.2 (Optical systems for fibre access networks) | Frank Effenberger Futurewei Technologies, USA frank.effenberger@futurewei.com | relationship with SG2 on management aspect |
|  | Q.4 (Broadband access over metallic conductors) | Frank Van der Putten Nokia, Finland frank.van_der_putten@nokia.com | relationship with SG2 on management aspect |
|  | Q.12 (Transport network architectures) | Stephen Shew Ciena, Canada sshew@ciena.com | relationship with SG2 on management aspect |
|  | Q.14 (Management and control of transport systems and equipment) | Hing-Kam Lam China Information Communication Technologies, P.R. China kamlam@fiberhome.com | relationship with SG2 on management aspect |
|  | Q18 (Broadband in-premises networking) | Les Brown Huawei, P.R. China lesbrown@sympatico.ca | relationship with SG2 on management aspect |
| SG16 | Q.2 |  | SG16 would welcome input from SG2 to improve management functionality for H.323, H.325, H.341 |
|  | Q.3 |  | H.248 and H.248.x sub-series - control of gateway functionality, the collection and reporting of statistics. |
|  | Q.5 |  | Definitions for Telepresence Systems |
|  | Q.12 |  |  |
|  | Q.15 (Voice gateway signal processing functions and circuit multiplication equipment/systems) | Dominic Ho Ericsson Canada Inc. 8400, Decarie Blvd. Town of Mount Royal QC H4P 2N2, Canada Tel: +1 573 823 6188 E-mail: hodmissouri@yahoo.com |  |
|  | Q.16 (Speech enhancement in signal processing network equipment) | Bob Reeves, BT, Adastral Park MLB 3, Room 7 pp1 Martlesham Heath Ipswich, Suffolk IP5 3RE United Kingdom Tel: +44 1473 644654 Fax: +44 1473 649777 E-mail: bob.reeves@bt.com |  |
|  | Q.18 (Interaction aspects of signal processing network equipment) | Harald Kullmann, Deutsche Telekom Am Kavalleriesand 3 D-64307 Darmstadt, Germany Tel: +49 61 51 628 2296 Fax: +49 391 580285694 E-mail: Harald. Kullmann@telekom.de |  |
|  | Q21 |  | Visual surveillance may require management of a huge number of camera nodes. It requires a management protocols to have that kind capability to deal with huge number of distributed nodes. |
|  | Q22 |  | Same as Q.21 |
|  | Q.25 |  | H.SNMF (SNMP-based sensor network management framework) |
| SG17 | All | Yutaka Miyake (WP 1/17 Chairman) Tel: +81 49 278 7367 Email: yu-miyake@kddi.com |  |
|  | Q.1 (Telecommunication/ICT security coordination) | Mohamed M. K. Elhaj Tel: +249 91215 2424 Email: mohamed.elhaj@tpra.org.sd |  |
|  | Q.3 (Telecommunications information security management) | Miho Naganuma Tel: +81 70 1000 7370 Fax: +81 3 5425 3182 Email: mnaganuma@bx.jp.nec.com |  |
|  | Q.11 (Directory, public key infrastructure (PKI), privilege management infrastructure (PMI), Abstract Syntax Notation One (ASN.1), object identifiers (OIDs)) to support secure applications) | Jean-Paul Lemaire Tel: +33 672 197 819 Email: paul.lemaire@univ-paris-diderot.fr |  |
|  | Q.12 (Formal languages for telecommunication software and testing) | Dieter Hogrefe Tel : +49 551 39172001 Email : hogrefe@informatik.uni-goettingen.de |  |
| SG20 |  |  | IoT Management |

Annex 1 – List of Documents within scope of the Telecommunications management and OAM project

Please see attached.

Annex 2 – List of Terms and Definitions within scope of the Telecommunications management and OAM project

Please see attached.

Annex 3 – List of Abbreviations within scope of the Telecommunications management and OAM project

Please see attached.

Annex 4 – Tasks

- September 2005. Beginning of a Telecommunications management and OAM project on the base of Telecommunication management documentation plan. Definition of a new structure, checking SG4 Recommendations information. Updated on information available from SG Work Programmes and liaisons (from SG15 and SG19) to the September 2005 meeting of ITU-T SG4. Waiting for initial and highly qualified assistance.

- October 2005. First version issued. Happy birthday. 

- June 2006. Sections 7 (Telecommunication Management and OAM Business Processes and Functional Requirements) and 8 (Telecommunication Management and OAM Principles, Frameworks, and Architecture) have got the text.

- February 2007. Section 9 (Telecommunication Management Interfaces) has got the text.

- August 2007. Section 6 (Telecommunication Management and OAM Target Domains), Section 1 (Scope), Section 2 (References), Section 3 (Definitions) have got initial text. Changing the scope of Annex 4 from the "History" to "Tasks"

- Section 11 (Hot topics for current and nearest future activities within TM&OAM PP) added

| # | Issue | Source | Date of origin (reopen) | Priority (H, M, L) | Remarks | Date of closing |
| --- | --- | --- | --- | --- | --- | --- |
|  | Obsolescence in M.60 definitions with existing Recommendations | D.Cherkesov | 2007.08.29 | H | Begin joint resolution with Q1/4 |  |
|  | Fulfilment of Contact data (Section 13 TM&OAM PP) | D.Cherkesov | 2007.08.29 | M | Attention to more precise reason for collaboration in Area column. Align permanently at each meeting, special case – after WTSAs | 2013 |
|  | Clause 3 - definitions | D.Cherkesov | 2007.08.29 | H |  |  |
|  | In addition put on the web all parts of PP separately |  | 2008.05.21 |  |  | 2012 |
|  | Examined duplicated terms and recommend preferred one |  | 2008.05.21 |  | Possible only in a new Recs. Not possible for old stable Recs. |  |
|  | ISO ¦ IEC standards – keep it? | D.Cherkesov | 2009.04.02 | M | Deleted | 2010 |
|  | Documents that are not finished and has no Contributions for more than 4 years | D.Cherkesov | 2009.04.02 | M | Deleted | 2010 |
|  | Fulfil the column “Dependencies within ITU doc” in Annex 1 |  | 2011.06 | M |  |  |
|  | Results for the last period |  | 2013.01 | H |  | 2013.01 |
|  | Results of WTSA-12 |  | 2013.01 | H | Sent LS to all SGs. | 2013.01 |
|  | Results of 2013 ITU-T SGs meetings |  | 2013.09 | H |  |  |
|  | Results of 2014 ITU-T SGs meetings |  | 2014.06 | H | Sent LS to all SGs |  |
|  | Results of 2015 ITU-T SGs meetings |  | 2015.06 | H | Sent LS to all SGs |  |
|  | Results of 2016 ITU-T SGs |  | 2016.09 | H | Sent LS to all SGs |  |
|  | Results of the SGs meetings from the Sept 2016 SG2 meeting |  | 2017.04 | H | Sent LS to all SGs IoT mngmnt issue IMT 2020 mapping |  |
|  | Results of the SGs meeting from the April 2017 SG2 |  | 2017.12 | H | LS, IoT mngmnt issue, Media, Sync, Anti-Fraud management |  |
|  | Results of the SGs meeting from the Dec 2017 SG2 |  | 2018.07 | H | LS, SOMM, SG20 |  |
|  | Results of the SGs meeting from the July 2018 SG2 |  | 2019.02 | H | LS, SG9, SG15, SG16, SG17, SG20 |  |
|  | Results of the SGs meeting from the June 2020 SG2 |  | 2020.06 | H | LS SG9, 15, 17 |  |

Annex 5 – Authors Team

D.Sidor, C.Smith, G.Carrier, D.Cherkesov, H-K.Lam, A.Korneychuk, A.jr.Korneychuk, V.Rabinovich, N.Fujii, L.Mak…

_______________________