|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  |  | SG13-TD681/WP3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 13 |  |
|  |  |  |  | Original: English |  |
| Question(s): |  |  | 16/13 | Virtual, 29 November - 10 December 2021 |  |
| TD |  |  |  |  |  |
| Source: |  |  | Editors |  |  |
| Title: |  |  | Draft new Recommendation ITU-T Y.energy-brokerage: "Framework of trusted electricity brokerage for distributed energy resources” |  |  |
| Purpose: |  |  | Proposal |  |  |
| Contact: |  | Taein Hwang ETRI Korea (Republic of) |  |  | Tel: +82-42-860-4875 Fax: +82-42-860-5218 E-mail: tihwang@etri.re.kr |
| Contact: |  | Il Woo Lee ETRI Korea (Republic of) |  |  | Tel: +82-42-860-5411 Fax: +82-42-860-5218 E-mail: ilwoo@etri.re.kr |

| Keywords: | Output document; Y.energy-brokerage; Q16/13, November 2021 |
| --- | --- |
| Abstract: | This is the output document for draft Recommendation ITU-T Y.energy-brokerage “Framework of trusted electricity brokerage for distributed energy resources” based on the results of Q16/13 virtual meeting held on 29 November – 10 December 2021. |

This document is the output document for draft Recommendation ITU-T Y.energy-brokerage “Framework of trusted electricity brokerage for distributed energy resources” based on the previous output document TD555/WP3, the following contribution and the meeting discussion.

Base document: TD555/WP3

| C-1351 | ETRI | Y.energy-brokerage: Proposal to update references, abbreviations, and security considerations | Q16/13 |
| --- | --- | --- | --- |

- Proposal of contribution

- This contribution proposes to update references, abbreviations, and security considerations.

- Meeting result

- The meeting agreed to accept the updated references, abbreviations, and security considerations.

Draft new Recommendation ITU-T Y.energy-brokerage

Framework of trusted electricity brokerage for distributed energy resources

Summary

Due to the rapid spread of distributed energy resources, the demand for intermediary trading (i.e., brokerage) of surplus electricity for energy prosumers in electricity markets is significantly increasing. To support transparency of brokerage transactions in the trading process, various technologies such as blockchain can be applied to applications that require mutual trust between users in a trustless environment. Thus, this draft Recommendation provides a framework of trusted electricity brokerage for distributed energy resources taking into account the blockchain technology for trust provisioning in electricity markets. After introducing key characteristics, core technologies and service scenarios for trusted electricity brokerage as well as the necessity of the blockchain technology to ensure trust, this draft Recommendation mainly presents requirements, architecture overview specifying related interfaces and functional blocks, and detailed mechanisms and service operations for the blockchain enabled trusted electricity brokerage.

Contents

1.	Scope	4

2.	References	4

3.	Definitions	4

3.1	Terms defined elsewhere	4

3.2	Terms defined in this Recommendation	5

4.	Abbreviations and acronyms	5

5.	Conventions	5

6.	Introduction to trusted electricity brokerage	5

6.1	Trusted electricity brokerage	5

6.2	Stakeholders related to the trusted electricity brokerage platform	7

6.3	General service scenarios for trusted electricity brokerage	8

7.	Requirements for trusted electricity brokerage in trustless environments	9

7.1	General requirements	9

7.2	Requirements for each stakeholder	9

8.	Architecture overview of a trusted electricity brokerage platform	10

9.	Mechanisms and service operations for the blockchain enabled trusted electricity brokerage	12

10.	Security considerations	12

Appendix I. Related use cases analysis	13

Bibliography	16

Draft new Recommendation ITU-T Y.energy-brokerage

Framework of trusted electricity brokerage for distributed energy resources

1.	Scope

Due to the rapid spread of distributed energy resources, trusted intermediary trading (i.e., brokerage) of surplus electricity for energy prosumers in electricity markets is very essential to support transparency of brokerage transactions in the trading process. Thus, this Recommendation provides a framework of trusted electricity brokerage for distributed energy resources with the blockchain technology for trust provisioning in electricity markets.

Therefore, this draft Recommendation covers the followings:

- Key characteristics, core technologies and service scenarios for electricity brokerage;

- Necessity of the blockchain technology to ensure trust in the trading process;

- Requirements for trusted electricity brokerage in trustless environments;

- Architecture overview specifying related interfaces and functional blocks;

- Mechanisms and service operations for the blockchain enabled trusted electricity brokerage.

2.	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T Y.2072]	Recommendation ITU-T Y.2072 (05/2018), Framework for an energy-sharing and trading platform.

[ITU-T X.1111] 	Recommendation ITU-T X.1111 (2007), Framework of security technologies for home network.

[ITU-T Y.2701]	Recommendation ITU-T Y.2701 (2007), Security requirements for NGN release 1.

3.	Definitions

3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	distributed energy resource (DER) [ITU-T Y.2072]: An energy generation and energy storage facility located at the customer premises or power transmission and distribution systems.

3.1.2	energy storage [ITU-T Y.2072]: Energy storage provides storage functions of electricity using various types of batteries.

3.1.3	renewable energy certificate (REC) [ITU-T Y.2072]: A tradable, non-tangible energy commodity that represents proof that energy was generated from an eligible renewable energy resource (renewable electricity) and was fed into the shared system of power lines that transport energy.

NOTE – An REC is also known as a “green tag”, “renewable energy credit”, “renewable electricity certificate”, or “tradable renewable certificate”.

3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

TBD.

4.	Abbreviations and acronyms

DER		Distributed Energy Resource

ESS		Energy Storage System

REC		Renewable Energy Certificate

ADER		Aggregate Distributed Energy Resource

RPS		Renewable Portfolio Standard

FIT		Feed in Tariff

5.	Conventions

TBD.

6.	Introduction to trusted electricity brokerage

(Editor’s Note) This section describes key characteristics, core technologies and service scenarios for trusted electricity brokerage as well as the necessity of the blockchain technology for ensuring trust in the trading process.

6.1	Trusted electricity brokerage

Due to the rapid spread of Distributed Energy Resources (DERs), it is important to support an open service platform with the capacity for more effective electricity markets and electricity trading in order to increase the efficiency of the electricity grid and reduce electricity grid wastage [ITU-T Y.2072]. With an energy sharing and trading platform, the demand for intermediary trading (i.e., brokerage) of surplus electricity for energy prosumers in electricity markets is significantly increasing.

NOTE 1 – An energy prosumer refers to an energy user who generates renewable energy in his/her domestic environment and either store the surplus energy for future use or trades to interested energy customers.

Energy prosumers with small-scale DERs have difficulty in effectively maintaining DER facilities at a low cost and in selling surplus energy and Renewable Energy Certificates (RECs) at a good price in electricity markets due to lack of expertise on energy resources. In addition, the intermittent nature of DERs leads to unpredictability in grid operation and reduction in power supply flexibility.

NOTE 2 – DERs are typically in the range of 3KW ~ 50MW, however small-scale DERs are in the range of less than 1MW.

To solve the above problems, electricity brokerage is very essential. The small-scale electricity brokerage combines various DERs (e.g., renewable energy like wind power and solar power, demand response resources, and electric storage devices, etc.) into a portfolio, thereby enhancing predictability of electric power generation. In addition, it provides a variety of information and supplementary services to support the decision making of energy prosumers, and services for assuring the reliability of the consumers for licensing, construction, maintenance and safety management, etc.

Figure 1 – The concept of small-scale electricity brokerage

Figure 1 shows the concept of small-scale electricity brokerage. The brokerage agency (i.e., broker) provides an intermediary service for trading the aggregated electricity and RECs in the electricity markets based on brokerage contracts with small electricity suppliers (or the prosumers). Such an intermediary service needs a sort of common brokerage platform for small-scale DERs integration and their surplus energy trading in order to maximize its customer’s profit through optimized DER operation and electricity sales bidding.

With the brokerage platform, various supplementary services can be provided to their customers. Examples of the services include DER maintenance services, DER data analysis (e.g., power generation forecasting, Energy Storage System (ESS) optimal scheduling, photovoltaic (PV) fault diagnosis), and DER business feasibility assessment services. The brokerage services deal with very sensitive information such as meter data for customer profit settlement. This could make some disputes such as a contractual default, data inconsistency, and so on. The cost of solution for the disputes may be excessive. In order to solve the problem, data integrity and transparency of the brokerage services should be ensured. For this reason, it is essential to apply a blockchain to the brokerage service platform. Brokerage contracts or transaction data can be secure on the blockchain. In addition, blockchain-based brokerage platform automatically processes customer profit settlement in real-time by transparently sharing the brokerage transactions and related processes with prosumers through smart contracts.

Figure 2 – Blockchain based trusted electricity brokerage platform

The above-mentioned blockchain-based trusted electricity brokerage platform requires the following key technologies:

- Group identification for authentication and access control of prosumers participating in electricity brokerage transactions;

- IoT-based meter data management for data collecting, analysing and controlling DERs to support electricity brokerage transactions;

- Distributed ledger to prevent forgery and alteration of data in power generation and electricity brokerage transactions;

- Smart contract to guarantee transparency of brokerage contract and automate transactions;

- Decentralized Application (dApp) to provide smart contract-based supplementary services in electricity brokerage;

- Generation, distribution and transaction management of energy tokens for automating the amount of electric power generation and the settlement of electricity brokerage transactions.

6.2	Stakeholders related to the trusted electricity brokerage platform

The following key stakeholders are involved in energy trading through the trusted electricity brokerage platform.

Electricity Broker

- Market participants who aggregate distributed energy resources owned by prosumers and sell their surplus energy to electricity markets on behalf of them.

Electricity Prosumer

- Market participants who have distributed energy resources that are available to supply their surplus energy to power grids.

Electricity Brokerage Market Operator

- Market operators who monitor and control electricity brokerage and trading between electricity brokers and prosumers.

6.3	General service scenarios for trusted electricity brokerage

This sub-section describes a general service scenario for the blockchain-based trusted electricity brokerage as follows.

1)  Resource registration

An electricity prosumer requests a power generation resource to be registered in an electricity brokerage market. After the request is approved by the brokerage market, the transaction on the resource approval is generated and stored on a blockchain. Then, the transaction is shared with market participants who have appropriate access rights to the blockchain.

2) Brokerage contract

The electricity prosumer requests the registered resource to be delegated to one of the electricity brokers participating in the brokerage market. Once a broker signs a contract on the delegation of the resource with the prosumer, the brokerage contract transaction is stored on the blockchain. After that, the transaction is securely shared between the contractors through the blockchain.

3) Resource aggregation

The broker creates a virtual large-scale power generator by grouping small-scale resources delegated from electricity prosumers and requests the newly created virtual power generator to be registered on the brokerage market. Upon the registration approval of the virtual power generator, the transaction on the registration approval is stored on the blockchain. The energy produced by the generator becomes reliably tradable on the brokerage market through the blockchain.

4) Energy trading

The broker predicts the amount of energy produced by the registered virtual power generator and places a sales order for the predicted power generation in the brokerage market. The trade for the sales order is transparently processed through the smart contracts deployed on the blockchain. Then, the transaction on the energy trade is stored on the blockchain.

5) Metering and settlement

Electric power meters measure how much electricity is generated by the power generation resources involved in the energy trade. The measured meter data or their hash values are stored on the blockchain. Based on the data and electricity price information, the settlement of the trade is processed through the smart contracts deployed on the blockchain.

7.	Requirements for trusted electricity brokerage in trustless environments

(Editor’s Note) This clause describes several types of stakeholders associated to electricity brokerage and specifies requirements for ensuring trust of brokerage transactions in the trading process taking into account various stakeholders and the blockchain technologies.

7.1	General requirements

Blockchain allows prosumers to sell and purchase energy in a manner that has greater transparency, enhanced security, improved traceability, increased efficiency and speed of transactions, with reduced costs. To support reliable and secure energy trading in trustless environments, this draft Recommendation considers blockchain-based trusted electricity brokerage platform with the following general requirements.

- It is required to ensure the transparency of electricity brokerage processes among a prosumer, a broker, and a brokerage market;

- It is required to process electricity brokerage transactions in real time;

- It is required to keep private data secure;

NOTE – 1: Private data includes electricity brokerage contract data, electric power metering data, etc.;

- It is required to automate electricity brokerage processes through smart contracts;

- It is required to store electricity brokerage transaction data in a distributed, immutable, and shared ledger.

7.2	Requirements for each stakeholder

Requirements for electricity brokers

- It is required to create electricity brokerage contracts and electronically sign the contracts on the brokerage platform;

- It is required to create an Aggregate DER (ADER) as a portfolio of DERs on the platform;

- It is required to create bids for the sale of the power and energy of ADERs on the platform.

Requirement for electricity prosumers

- It is required to register DERs available for electricity brokerage on the platform;

- It is required to retrieve electricity brokerage contracts stored on the platform and electronically sign the contracts.

Requirements for electricity brokerage market operators

- It is required to approve the registration of DERs and ADERs on the platform;

- It is required to automate bidding for the sale of the power and energy of ADERs on the platform;

- It is required to store on the platform the meter data on energy consumed or produced from DERs;

- It is recommended to store electricity price information on the platform;

- It is recommended to automate profit settlement based on the meter data and electricity price information stored on the platform.

Requirements for all stakeholders

- It is required to retrieve DERs, ADERs, bids, meter data stored on the platform.

8.	Architecture overview of a trusted electricity brokerage platform

(Editor’s Note) This clause provides architectural framework specifying related interfaces and key functionalities. Based on the high-level functional model, this section provides detailed functional architecture of a trusted electricity brokerage platform.

Trusted electricity brokerage markets ensure the reliability and transparency of their operation process by sharing brokerage transactions with market participants based on a blockchain. The followings describe the electricity brokerage procedure for DERs in electricity markets.

- Resource registration: Energy prosumers (e.g., DER energy producers) register their DERs as a power generation resource in an electricity market.

- Brokerage contract: Energy prosumers delegate the registered DERs to electricity brokers through brokerage contracts.

- Resource aggregation: Electricity brokers group small-scale DERs into an Aggregated DER (ADER) so that its energy can be tradable in an electricity market.

- Brokerage trading: Electricity brokers sell the generation power and energy of ADERs as a form of energy product to energy consumers through an electricity market.

- Profit settlement: Electricity brokers settle sales revenue and commission after electric power and energy delivery according to the contracted energy supply schedule.

Figure 3 – Electricity brokerage Procedure for DERs

Regarding the electricity brokerage procedure shown in Figure 3, Figure 4 shows the conceptual diagram of blockchain-based electricity brokerage as a trusted electricity brokerage platform.

Figure 4 – Conceptual Model of blockchain-based electricity brokerage

The followings describe the functional components for blockchain-based electricity brokerage.

- Electricity Brokerage Client is a decentralized application that enables market participants to take part in a reliable and transparent electricity brokerage market through Electricity Brokerage Platform. It also provides functions for market participants to realize their own business logics.

- Electricity Brokerage Platform provides electricity brokerage clients with common energy brokerage market functions (e.g., identification and access control of market participants, DER/ADER registration, brokerage contract, and bidding) through the interaction with other functional components.

- Electricity Brokerage Blockchain Node validates the transactions sent from electricity brokerage clients, generates the data blocks that contain the validated transactions based on a consensus mechanism. It also shares the generated blocks with other blockchain nodes on the same blockchain network.

- Metering Node generates the energy metering data of DERs periodically, and requests to process the data to blockchain nodes through Electricity Brokerage Platform so that original or hashed value of the metering data can be stored in a blockchain.

- Certificate Authority is responsible for issuing and validating the digital certificates for the identification and access control of market participants in order to protect the confidentiality and privacy of brokerage transactions in a blockchain.

9.	Mechanisms and service operations for the blockchain enabled trusted electricity brokerage

(Editor’s Note) This clause provides detailed mechanisms and service operations using the blockchain technology in a trusted electricity brokerage platform.

TBD.

10.	Security considerations

The security considerations in a trusted electricity brokerage platform should be in accordance with the security requirements in [ITU-T X.1111] and [ITU-T Y.2701].  The general security requirements and technologies are described in [ITU-T X.1111]. This Recommendation applies the technologies in [ITU-T X.1111] to the functional components of trusted electricity brokerage platform.

A trusted electricity brokerage platform should consider various security aspects in terms of physical security, system security, and operation security as follows:

•	Verifying the identity of an entity (e.g., user, process, or device) for accessing to system resources for trusted electricity brokerage platform;

•	Determining the adequacy of security requirements and ensuring compliance with the established security policy and procedures for a trusted electricity brokerage platform;

•	Authorization to use various services based on a trusted electricity brokerage platform;

•	Supporting cryptography and validation mechanisms for data integrity in a trusted electricity brokerage platform;

•	Providing the privacy considerations with respect to a trusted electricity brokerage platform.

Appendix I. Related use cases analysis

(Editor’s Note) This Appendix provides various use cases for the blockchain enabled trusted electricity brokerage.

I.1	Blockchain based renewable energy certificate trade platform

I.1.1	Overview

With the widespread of the renewable energy supplement, many countries tries to encourage the deployment of renewable energy sources to overcome climate change issues, and the concept of renewable portfolio standard (RPS) is introduced. The RPS is a regulation that the energy generation company must follow; that is, power generation companies produces a certain portion of their entire power generation by only using the renewable energy sources. The main purpose of the RPS is that to increase the usage of renewable energy sources. However, the power generation companies have difficulty to achieve mandatory renewable power generation with its own renewable generator because the renewable energy sources has uncertainty and risk to achieve the target electricity supply amount due to many external causes such as weather, natural environment, etc. Therefore, the concept of renewable energy certificate (REC) is introduced that makes possible to trade the quota for the RPS, which generated by renewable energy source, between energy generation companies. To satisfy the required energy generation from the RPS, individual generation company is able to purchase REC from other renewable energy generation companies.

Figure I.1 – blockchain based REC trading model

There are two main ways to deal with RECs trading. One is a fixed pricing scheme based transaction with a long-term period contract. As an example, “feed in tariff (FIT)” is one of possible tariff policies for the fixed pricing scheme that the energy generation company purchases the entire electricity generated from renewable energy facilities under the standard price announced by the government. The other is a dynamic pricing scheme based transaction through a spot market.

In the former case, there is no problem because it keeps the fixed transaction price constantly. However, in the spot market (the latter case), transaction price is changed dynamically by sensitively reflecting the daily trading volume and/or desired price of the market participants. In addition, in case of trading in the spot market, the transaction results and the retained amount of REC should be timely recorded and updated based on the real-time trading results. In order to efficiently solve various consideration factors due to the expansion of the spot market, a REC transaction model based on blockchain technology is introduced as shown in Figure I.1. In this model, transaction records between market participants are automatically recorded in blocks. In addition, other market participants are allowed to confirm the block that contains information on previous transactions with the bidding amount and transaction prices. With this approach, it is possible to keep a reasonable price fortransaction and to induce user’s participation in the market.

I.1.2 Blockchain-based certification trading platform

As mentioned above, transaction of REC in the market is essential to meet the requirements of energy generation facilities for RPS. The blockchain technology in the market not only ensures the automation of transaction and the anonymity of each participant but also makes possible to store transaction records safely in distributed storage.

Figure I.2 – A REC transaction scenario based on blockchain technology

Figure I.2 shows a REC transaction scenario with three major market participants: broker, buyer, and seller. The broker announces to the participants (i.e., buyers and sellers) that the REC spot market take places. When buyers and sellers informed about the holding of the REC market, they will decide the amount of REC for transaction and the desired price they need within a certain time period. In the case of the broker, the market is terminated by notifying the sellers and the buyers about the amount of transaction quantity and profit after the bidding result based on the desired decision-making process with the consideration of transaction information collected from the participants. The results and progress of such transactions are handled with blockchain technology. Therefore, information on the number of RECs held by each participant is also recorded in a block.

I.1.3 Description of market participants

In a particular scenario, the role of the market participants could be described as follow:

- Information announcement task: Broker decides specific time to announce the market operation for the REC trading.

- Sales information: Sellers participate the market with retained REC quantity and desired price.

- Bidding information: Buyers participate the market submit desired transaction quantity and price to maximize their own profit.

- Requirements for gathering information:

- Whole of data is periodically transmitted to the server within certain time period.

- It is essential for the participants to submit their desired bidding price and quantity.

- User should be informed previous transaction prices and penalty charge in the market

- Expectation

- Transaction result could be stored in the distributed storage safely

- Whole of transactions are operated anonymously which do not exposure user’s personal data

- Sellers could maximize their profit according to participate in the market

- Buyers ensure profit by avoiding penalties

1)	Broker

Since transaction process is automatic based on real-time pricing scheme in the market rather than the individual decision of each participant, a broker is necessary in the market to determine the trading object and transaction price.

- Transaction price

In the market, there are multiple sellers and buyers to maximize their own profit according to the transaction of REC. The most typical pricing decision method is an auction theory (e.g., Double auction, Naive auction) which derives proper transaction price to maximize overall social welfare or individual market participant’s profit. Especially, most of energy markets use the double auction method, which leads reasonable pricing decision in an environment where market participants of various attributes coexist.

- Revenue distribution and records

In the REC trading market based on blockchain technology, the broker has two specific roles in the market. One is to distribute the profit for the sellers and REC bid amount for the buyers who succeeded in the bid. The other is to record the amount of individual participant’s transaction quantity and manage the additional required REC amount of buyer in the market.

2)	Seller

A seller has renewable generation facilities with enough capacity. They are given different weights according to the characteristics of the generators (e.g., photovoltaic energy, other renewable energies), not just the capacity of the equipment they have. In addition, they are weighted depending on the location of the plants and the linkage of energy storage devices.

- Sales price

It is necessary to determine a reasonable selling price considering the previous transaction records and forecasting the price changes in future.  In addition, the sellers need to minimize the risk of failing to sell REC in the market by considering the price of others competitors in the spot market. In general, REC price tends to rise over time, but not always, so sellers need to determine reasonable selling price based on future expected price and their current sales volume.

- Sales quantity

Sales quantity is an important factor in determining the income of the sellers. They can sell their own RECs through either the general trading market or the spot market to maximize their profit. When the sellers participate in the general trading market, they can trade their REC at a stable price; however, they could miss the opportunity to sell in the spot market with a relatively higher price. On the other hand, it is possible to sell the electricity with a relatively high price in the spot market; however, the price of electricity in the market could decline due to the huge volume of supply provided from other sellers. Moreover, a loss could be occurred because a bidding process is not made on time. With the consideration of these cases, the sellers should determine their sales volume reasonably.

3)	 Buyer

Buyers are market participants who generally get the amount of REC that they have to meet in a certain period of time by participating in the market. Generally, it refers to power generation companies, power generation subsidiaries, public institutions, and many private operators in the system. To obtain REC at a higher rate and a stable low price, various approaches are needed.

- Bidding price

Bidding price must be made by simultaneously considering all of the expected value that the buyers will have.  They should take a reasonable strategy based on the penalty that is incurred when they fail to meet the RTP requirements within a given period is failed, the average bidding price of other buyers, and the previous transaction records. In addition, they are necessary to determine reasonable price considering the expected effect of participating in the general market rather than the spot market.

- Bidding quantity

The amount of bidding quantity should also be taken into account both the expected profit from the buyer’s choice and the prior forecast of other buyers’ strategies. In addition, buyers also need to consider the current portion of their REC holdings and remaining time

Bibliography

TBD.

____________________