|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  |  | SG13-TD680/WP3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 13 |  |
|  |  |  |  | Original: English |  |
| Question(s): |  |  | 16/13 | Virtual, 29 November - 10 December 2021 |  |
| TD |  |  |  |  |  |
| Source: |  |  | Editors |  |  |
| Title: |  |  | Draft new Recommendation ITU- T Y.dv-ess: "Framework of distributed and virtualized energy storage systems" |  |  |
| Purpose: |  |  | Proposal |  |  |
| Contact: |  | Taein Hwang ETRI Korea (Republic of) |  |  | Tel: +82-42-860-4875 Fax: +82-42-860-5218 E-mail: tihwang@etri.re.kr |
| Contact: |  | Il Woo Lee ETRI Korea (Republic of) |  |  | Tel: +82-42-860-5411 Fax: +82-42-860-5218 E-mail: ilwoo@etri.re.kr |

| Keywords: | Output document; Y.dv-ess; Q16/13; November 2021 |
| --- | --- |
| Abstract: | This is the output document for draft Recommendation ITU-T Y.dv-ess “Framework of distributed and virtualized energy storage systems” based on the results of Q16/13 virtual meeting held on 29 November - 10 December 2021. |

This document is the output document for draft Recommendation ITU-T Y.dv-ess “Framework of distributed and virtualized energy storage systems” based on the previous output document TD554/WP3, the following contribution and the meeting discussion.

Base document: TD554/WP3

| C-1350 | ETRI | Y.dv-ess: Proposal for security consideration | Q16/13 |
| --- | --- | --- | --- |

- Proposal of contribution

- This contribution proposes to add new texts regarding security consideration in Clause 11.

- Meeting result

- The meeting agreed to accept the proposed security considerations.

Draft new Recommendation ITU-T Y.dv-ess

Framework of Distributed and Virtualized Energy Storage Systems

Summary

Energy storage systems aim to store electricity when the demand is low and provide stored electricity when the demand is high. For efficient grid operations and satisfying increasing customers’ demands, these systems should be distributed, and virtualized storages are essential to be dynamically allocated and operated for individual customers. Thus, this draft Recommendation provides the framework of distributed and virtualized energy storage systems. After identifying key characteristics and core technologies of energy storage systems, this draft Recommendation specifies requirements taking into account the energy value chain comprising various stakeholders. Then, this draft Recommendation provides architectural overview specifying related interfaces and functional blocks. Finally mechanisms for virtualization of energy storage systems and detailed service operations using energy storage systems are described.

Contents

1.	Scope	4

2.	References	4

3.	Definitions	4

3.1	Terms defined elsewhere	4

3.2	Terms defined in this Recommendation	5

4.	Abbreviations and acronyms	5

5.	Conventions	5

6.	Introduction to energy storage system	5

6.1	Virtual energy storage	6

6.2	General service scenario with distributed and virtualized energy storage systems	7

7.	Requirements for distributed and virtualized energy storage systems	8

7.1	Stakeholders associated to Virtual Energy Storage Systems	8

7.2	Requirements of each stakeholder for virtual energy storage systems	8

8.	Architecture overview of energy storage systems	9

8.1	Generic architectural model of distributed platforms for virtualized energy storage systems	9

8.2.	Functional architecture for distributed and virtualized energy storage systems	12

8.2.1	Application Service Functions	13

8.2.2	VES Management Functions	13

8.2.3	ESS Operation Functions	14

9.	Mechanisms for virtualization of energy storage systems	14

10.	Service operations using energy storage systems	17

11.	Security considerations	17

Appendix I. Use cases and service scenarios	18

Bibliography.	20

Draft new Recommendation ITU-T Y.dv-ess

Framework of Distributed and Virtualized Energy Storage Systems

1.	Scope

Energy storage systems aim to store electricity when the demand is low and provide stored electricity when the demand is high. For efficient grid operations and satisfying increasing customers’ demands, these systems should be distributed, and virtualized storages are essential to be dynamically allocated and operated for individual customers. Thus, this draft Recommendation provides the framework of distributed and virtualized energy storage systems. After introducing the overview of energy storage systems, this draft Recommendation mainly specifies requirements, architectural overview, detailed mechanisms and operations.

Therefore, this draft Recommendation covers the followings:

- Key characteristics and core technologies of distributed and virtualized energy storage systems;

- Requirements taking into account the energy value chain comprising various stakeholders;

- Architectural overview specifying related interfaces and functional blocks;

- Mechanisms for virtualization of energy storage systems;

- Service operations using energy storage systems.

2.	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T X.1111] 	Recommendation ITU-T X.1111 (2007), Framework of security technologies for home network.

[ITU-T Y.2701]	Recommendation ITU-T Y.2701 (2007), Security requirements for NGN release 1.

3.	Definitions

3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	energy storage [b-ITU-T FG-Smart]: Energy storage provides storage functions of electricity using various types of batteries.

NOTE – One example usage of energy storage is used to respond effectively to a dynamic price mechanism from a utility network. The electric energy is stored in the storage during a relatively lower price period, while the stored electric energy may replace the higher price of electric power from the utility network.

3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

TBD

4.	Abbreviations and acronyms

TBD

5.	Conventions

TBD

6.	Introduction to energy storage system

(Editor’s Note) This section describes the concepts and characteristics of energy storage system, and also explains key technologies and considerations.

Energy storage is the capture of energy produced at one time for use at a later time with storage functions of electricity. Energy storage systems basically aim to store electricity when the demand is low and provide stored electricity when the demand is high. Energy storage systems are becoming more economically important as they can reduce costs. Furthermore, energy storage systems have quite significant role in energy trading through the stabilized market prices for electricity.

In particular, energy storage systems have the great potential in order to improve the efficiency and reliability of electric delivery. Energy storage systems can balance peak loads, control the increasing costs of grid infrastructure equipment, and mitigate the intermittent nature of various types of renewable generation.

The goals of energy storage systems [1][2] can be summarized as follows:

- To reduce electricity costs by storing electricity obtained at off-peak times;

- To improve power grid optimization for bulk power production;

- To balance the power system operations with renewable generation options;

- To help to defer capital-intensive upgrades in the transmission and distribution grids;

- To provide ancillary services to grid operations.

With the help of energy storage systems, various types of applications (such as for supplying electricity during peak hours, improving power quality) can be provided.

Distributed energy storage systems can improve the overall efficiency of the grid operations while minimizing operational cost through the integration of these systems. Furthermore, with the help of virtualization technologies, the storage can be used to modify load shapes dynamically based on customers’ demands. Multiple virtual energy storages from a single storage are allocated and operated for customers.

NOTE – ESS(Energy Storage System), VES(Virtual Energy Storage)

Figure 1 – The concept of virtual energy storage systems

Figure 1 illustrates the concept of virtual energy storage systems. The total four energy storage systems with 10KW output, 50KWh charge/discharge capacity (i.e., ESS-1, ESS-2, ESS-3 and ESS-4) are virtualized as the integrated virtual energy storage with 200KWh capacity. The virtualized energy storage with different outputs and capacity (i.e., VES-1, VES-2, VES-3 and VES-4) is dynamically allocated into individual customers. Each VES has its own electricity measuring process for customized operations and adjustment.

6.1	Virtual energy storage

Virtual energy storage can be integrated with HEMS, BEMS, and FEMS user applications to reduce power bills and be used in a variety of smart grid consumer domains (e.g., homes, buildings, and factories). In addition, it can contribute to stabilization of power grid supply and demand by linking with applications of DRMS (Demand Response Management System), DERMS (Distributed Energy Resource Management System) and VPP (Virtual Power Plant).

Virtual energy storage provides on-demand energy storage capacity to energy consumers who cannot afford the direct investment of distributed generation resources (e.g., energy storage devices and solar power generation) due to high initial investment cost.

Energy prosumers can use virtual energy storage-based applications to distribute power generation capacity and energy storage capacity according to buyer's desired size.

A variety of applications for energy consumers and prosumers need a standardized platform for ESS integrated management and virtualization to take advantage of virtual energy storage. The VESS (Virtual Energy Storage System) integrates and manages physical ESSs, and provides virtual energy storage capacity requested by the above mentioned user applications.

To this end, it is necessary to:

- periodically manage and control the status of various ESSs that are managed in an integrated manner (ESS Interface);

- provide a standardized interface (Metering Interface) to meter the amount of power charged and discharged on each physical or logical ESS;

- provide a standard interface for various applications to monitor and control the VES.

Figure X illustrates the relationship between the VESS and user applications, ESSs, and metering devices.

NOTE - MDMS(Meter Data Management System), VESS(Virtual Energy Storage System), VES(Virtual Energy Storage), AMI(Advanced Metering Infrastructure)

Figure 2 – VESS and related interfaces

6.2	General service scenario with distributed and virtualized energy storage systems

This sub-section describes a general service scenario with distributed and virtualized energy storage systems as follows.

Stage 1: Virtual Energy Storage Mapping and Allocation

Energy storage virtualization allows a large number of energy storage devices to be divided into logical energy storage capacity divisions without any physical separation. This stage allocates the space of the energy storage device and conducts the mapping of virtual energy storage and the allocated storage space.

Stage 2: Virtual Energy Storage Operation Setup

This stage configures virtual energy storage operations such as maximum demand power reduction operation, participation in demand resource trading market, participation in small-scale distributed resource trading market, participation in REC market, etc. with respect to the divided virtual energy storage.

Stage 3: Virtual Energy Storage Charge/Discharge Control

This stage controls the charging and discharging of the energy storage device according to the configured virtual energy storage operation mode.

Stage 4: Metering and Settlement

This stage conducts the settlement of virtual energy storage service revenue based on power generation and consumption measurement information collected from individual logical energy storages.

7.	Requirements for distributed and virtualized energy storage systems

This clause describes several types of stakeholders associated to Virtual Energy Storage Systems (VESS) and specific requirements of each stakeholder.

7.1	Stakeholders associated to Virtual Energy Storage Systems

VES User

- Energy consumers who lease Virtual Energy Storage (VES) for the purpose of reducing electricity charges and reducing peak load.

- Energy producers who want to maximize power sales revenue by charging and discharging electricity generated by solar power and wind power generation facilities.

VESS Operator

- Operators who support real-time monitoring of the operation status of Energy Storage System (ESS) and related facilities, as well as administrating VESS to supply VES users with customized power and energy capacity meeting the requirements of VES users.

ESS Owner

- Holders of ESS equipment entrusted to VESS Operator to operate ESS.

7.2	Requirements of each stakeholder for virtual energy storage systems

Requirements of VES User

- It is required to create and lease a VES with customized energy storage capacity.

- It is required to register and update VES charge and discharge schedules.

- It is required to monitor VES operations and notify their faults in real-time.

- It is required to provide information on energy generation and consumption of VES.

- It is required to settle VES operation profits and losses based on the amount of energy produced and consumed by VES-mapped ESSs and energy price information.

Requirements of VESS Operator

- It is required to logically allocate partial or full energy storage capacity of ESS to VES.

- It is required to discover available energy storage capacities and assign them to VES.

- It is required to schedule ESS operations according to VES charge and discharge schedules.

- It is required to monitor VES-mapped ESS operations and their faults in real-time.

- It is required to provide a standardized communication interface for interworking with various types of ESSs.

- It is recommended to provide a function to optimize ESS charge and discharge schedules in consideration of ESS characteristics and predicted energy demand.

Requirements of ESS Owner

- It is required to monitor ESS operation status (e.g., State of Charge (SOC), charge/discharge schedule, etc.) in real-time.

- It is required to notify information on ESS operation performance and fault in real-time.

8.	Architecture overview of energy storage systems

(Editor’s Note) This section provides architectural framework specifying related interfaces and key functionalities. Based on the high-level functional model, this section provides detailed functional architecture.

8.1	Generic architectural model of distributed platforms for virtualized energy storage systems

Energy storage virtualization enables logical integration and separation of one or more ESSs. For this, VESS needs to support open service interfaces for the interactions with both various types of ESSs and smart grid applications. Figure 3 shows the reference architecture of VESS to describe the relations between VESS and the external systems such as ESSs, metering systems, and EMSs.

Figure 3. Reference architecture and reference points

- VESS

- VESS has key functions as a common service platform for delivering VES to a variety of smart grid applications through the interoperation with various types of ESSs.

- VESS allows smart grid applications to monitor and control the allocated VES according to the VES operating policies and schedules set by its users.

- Optionally, VESS provides data analytics and prediction services (e.g., user behavior analysis and electric power demand forecasting for VES operating policy decisions).

- For the interoperation with ESSs, some of the VESS functions (e.g., ESS monitoring and control) are implemented on ESS controllers (e.g., ESS inverters and RTUs) in remote. For this reason, VESS is logically separated into two modules VES Manager and ESS Controller.

- VES Manager provides the following functions as a VES management server platform:

    - VES account management: User registration and access control, VES contract management, VES meter data management, and user profit settlement;

    - VES lifecycle management: VES creation, allocation, and destruction;

    - VES operation management: VES configuration, status monitoring, and operation control.

- ESS Controller provides the following functions as a VES management agent platform:

    - ESS configuration management;

    - ESS status monitoring;

    - ESS charge and discharge control.

The VESS-related applications and the external systems are described as follows

- Application

- VESS enables smart grid applications to lease or buy VES with the desired storage capacity and to monitor and control VES. It also manages the access rights of VES users and applications, and provides VES lease contract and profit settlement services.

- The following smart grid applications can use VES in order to minimize electricity costs and maximize electricity sales revenue through peak load curtailment and surplus energy trading:

    - Customer EMS: Electricity usage reduction and peak load curtailment of customer domains (e.g., residential, commercial, and industrial buildings) by utilizing VES;

    - Demand Response Management System (DRMS): Contribution to stabilization of electric power grids at peak load time by joining incentivized DR programs with VES;

    - Electricity Brokerage System:  Aggregating small-scale energy generated by customer VESs into a large-scale energy product in order to sell it to an electric power wholesale market.

- ESS

- ESSs interworking with VESS needs to provide VESS with their status information (e.g., battery charge status) in real-time.

NOTE: Various ESSs include Li-ion, flow battery, Compressed Air Energy Storage (CAES), etc.

- VESS provides ESSs with common data models and communication interfaces in order to support vendor-specific data model and communication protocols.

- A suitable ESS should be selected in order to create and allocate VES to meet customer requirements considering battery characteristics, and so on.

- Metering System

- Metering systems collect the meter data from smart meters installed on utility grids and deliver the data to VESS.

- Meter data can be directly collected from ESS Controller without the help of utility provider’s metering system. In that case, the data is mainly used for ESS status monitoring rather than customer profit settlement.

- VESS utilizes the meter data for more accurate profit settlement and dispute resolution between stakeholders.

The following reference points indicate the relations between VESS functional components and external systems.

- V-A reference point

- The V-A reference point allows energy service applications to utilize VESs through the online lease contract with VESS in order to maximize the profit of application users and to minimize the cost of energy consumption.

- Through this reference point, a variety of energy service applications can access VES lease contract, VES operation, and VES information service provided by VES Manager.

- V-C reference point

- The V-C reference point is an interface between VESS internal functional components (e.g., VES Manager and ESS Controller).

- Through this interface, VES Manager collects ESS status information from ESS Controller in real time and transmits ESS control messages to ESS Controller according to the operation schedule set by VES users.

- V-E reference point

- The V-E reference point provides communication paths between ESS Controller and various types of ESSs.

- Through the communication paths, ESS Controller keeps monitoring the status of the managed ESSs and control the ESSs according to the ESS operation schedules sent from VES Manager. It also can receive various types of event messages such as fault notification from the ESSs.

- V-M reference point

- The V-M reference point is an interface for collecting the electric power meter data of the managed ESSs from Metering System.

- Through this interface, the meter data for profit settlement are securely collected from metering systems and smart meters that are installed by utility providers.

8.2.	Functional architecture for distributed and virtualized energy storage systems

VESS helps VES users to optimize electric power demand profile or to reduce electricity bills by saving energy when energy prices are low and releasing stored energy when energy prices are high. Instead of buying ESSs, VES users can lease customized electric power and energy capacity through VES application services. VESS controls energy storage devices to maximize the utilization of ESSs considering VES operation schedules set by VES users. The VES management functions of VES Manager enable available energy storage capacity of ESSs to be dynamically allocated based on VES operation schedules. The ESS operation functions of ESS Controller perform the configuration, control, and monitoring of ESSs associated with VESs. The following describes VESS key functions from the perspective of VES application service, VES management, and ESS operation.

Figure 4. Key functions of VESS

8.2.1	Application Service Functions

- VES Account Service

- VES user authentication, authorization, and accounting management.

- VES user profit settlement based on VES metering data and electricity prices.

- Profit prediction based on VES operation schedules and forecasted electricity prices.

- VES Market Service

- VES service advertisement (e.g., VES power generation capabilities, available storage capacity range, and VES lease price).

- VES service recommendation based the forecasted electricity demand of VES users.

- VES purchase and lease service contract management.

- VES Operation Service

- VES charge and discharge schedule management.

- VES operation status and fault event messages reporting.

    - VES Management Functions

- Energy Storage Allocation

- Discovery of the available ESSs to meet VES user requirements such as VES capability, storage capacity, and lease period.

- Dynamic energy storage allocation considering ESS characteristics, status, and operation schedules.

- VES-to-ESS mapping information management.

- VES Operation Scheduling

- VES charge and discharge schedule optimization.

- VES operation policy configuration.

- VES Monitoring

- ESS meter data management and VES metering.

- VES status monitoring and operation fault handling.

- VES performance analysis and fault diagnosis.

8.2.3	ESS Operation Functions

- ESS Configuration

- Energy storage facility information registration.

- ESS communication interface configuration.

- ESS operation policy configuration.

- ESS Control

- ESS operation schedule optimization.

- ESS charge and discharge control.

- ESS Monitoring

- ESS status monitoring (e.g., State of Charge (SoC), ESS operation state, meter data, operation faults).

- ESS performance analysis and fault diagnosis.

9.	Mechanisms for virtualization of energy storage systems

A method of controlling an energy storage and an energy storage controlling system are described. The method includes dividing an energy storage of large capacity into virtual energy storages corresponding to respective users based on a required energy amount and a period of use of an energy storage of each of the users, and integrally controlling, for each time, the virtual energy storages corresponding to the respective users based on a management schedule of the energy storage of each of the users.

Figure 1. System configuration for controlling energy storage

In Figure 1, an energy storage controlling system includes a user device, an energy storage controlling system, and an energy storage of large capacity. The user device may communicate with the energy storage controlling system.

- The user device is used by each user.

  - It transmits a user response signal to the energy storage based on a response of each user.

  - It displays control information on the energy storage of large capacity transmitted from the energy storage controlling system.

  - It is an interface connecting each of the users and the energy storage controlling system.

- The energy storage controlling system controls the energy storage of large capacity based on the user response signal transmitted from the user device of each of the users.

  - It integrally controls, for each time, a charge and a discharge of the virtual energy storages corresponding to the respective users based on the management schedule of the energy storage of each of the users.

  - It rents  and integrally controls the virtual energy storages corresponding to the respective users to allow a user who has not built an energy storage yet to reduce initial investment costs for installing the energy storage, to provide an energy storage that is used more effectively, to reduce a cost of managing the energy storage, to reduce an effort required to manage the energy storage, to  reduce a maximum power requirement due to an energy storage rent, and to enable participation in a demand resource trading market.

  - It allows a businessman who builds an energy storage to obtain initial investment costs for installing the energy storage and minimize a risk to business promotion.

- The energy storage of large capacity includes a plurality of virtual energy storages. The energy storage of large capacity may be implemented as a single physical energy storage or a plurality of physical energy storages.

NOTE - The energy storage (or energy) of small capacity may be required by various structures such as homes, buildings, and factories. A consumer may require an energy storage (or energy) during at least one of a predetermined period or a predetermined season.

Figure 2. Components of an energy storage controlling system

Figure 2 shows main components of an energy storage controlling system. This system includes an interface, a processor, and a memory.

- Interface: The interface receives a user response signal transmitted from the user device of each user. It transmits control information on the energy storage of large capacity to the user device.

- Processor: The processor controls an overall operation of the energy storage controlling system. It divides the energy storage of large capacity into virtual energy storages corresponding to respective users based on a required energy amount (or required energy storage space) and a period of use (or rental period) of an energy storage of each of the users. In addition, it integrally controls the virtual energy storages corresponding to the respective users for each time based on a management schedule of the energy storage of each of the users.

The processor includes a divider, a register, a calculator, and a controller.

- Divider: The divider divides the energy storage of large capacity into multiple virtual energy storages.

  - It divides the energy storage of large capacity into a plurality of virtual logical energy storages.

  - It allocates energy storage areas corresponding to the respective users to the virtual logical energy storages based on the required energy amount (or required energy storage space) and the period of use (or rental period) of the energy storage of each of the users.

  - It generates the virtual energy storages corresponding to the respective users based on the invariably allocated energy storage area or the variably allocated energy storage area.

- Register: The register registers the management schedule of the energy storage of each of the users.

- Calculator: The calculator calculates an integrated management schedule of the virtual energy storages corresponding to the respective users based on the management schedule of the energy storage of each of the users.

  - It may calculate a time-based first management schedule of the virtual invariable energy storage corresponding to the long-term renter based on the management schedule of the energy storage of the long-term renter.

  - Subsequently, it may calculate a time-based second management schedule of the virtual variable energy storage corresponding to the short-term renter based on the management schedule of the energy storage of the short-term renter.

  - It may calculate the integrated management schedule for each time based on the first management schedule and the second management schedule.

  - It may calculate the management schedule based on a discharge loss rate of a physical energy storage linked to the virtual energy storages.

  - In response to a discharge of the virtual energy storages, it may calculate the management schedule based on the discharge loss rate of the physical energy storage linked to the virtual energy  storages  such  that  the  virtual  energy  storages  discharge  90%  of  total  generation capacity.

- Controller: The controller integrally controls the virtual energy storages corresponding to the respective users for each time based on the integrated management schedule. It transmits information on the energy storage of large capacity to the user device via the interface.

- Memory: The memory stores information on the energy storage of large capacity and the user response signal.

10.	Service operations using energy storage systems

(Editor’s Note) This section provides detailed service operations using energy storage systems from use cases and service scenarios analysis.

TBD

11.	Security considerations

The security considerations in a virtual energy storage system should be in accordance with the security requirements in [ITU-T X.1111] and [ITU-T Y.2701].  The general security requirements and technologies are described in [ITU-T X.1111]. This Recommendation applies the technologies in [ITU-T X.1111] to the key system components and functions of virtual energy systems.

A virtual energy storage system should consider various security aspects in terms of physical security, system security, and operation security as follows:

•	Verifying the identity of an entity (e.g., user, process, or device) for accessing to resources in a virtual energy storage system;

•	Determining the adequacy of security requirements and ensuring compliance with the established security policy and procedures for a virtual energy storage system;

•	Authorization to use various services in a virtual energy storage system;

•	Supporting cryptography and validation mechanisms for data integrity in a virtual energy storage system;

•	Providing the privacy considerations with respect to a virtual energy storage system.

Appendix I. Use cases and service scenarios

(Editor’s Note) This Appendix provides various use cases and service scenarios considering distributed and virtualized energy storage systems.

I.1       VESS based energy management system in power system

I.1.1	Overview

Depending on the development of IT technology, it is expected that overall power system will be changed as data based infrastructure. This change will be arisen due to the development of technology in power transaction and data accumulation process depending on sensors in power usage environment. This kind of development of ICT infrastructure in the power system has brought more data movement which make it possible to implement various services for power consumers. For example, the emergence of smart meter makes possible to impose charges according to power usage at time horizon and seasonal period. In addition, the emergence of ancillary services enables the operation of the power market through control the amount of electricity used, rather than the existing power system which manage the system depending on various kinds of generators [Fig. I.1].

Fig. I.1. Changes of power systems and the necessity of VESS

One of the factors should be considered in the power system is the charging tariff policy for participants. Since power data are collected from heterogeneous environments, it is hard to decide a proper charging tariff to control overall power system. VESS now becomes an essential element for system operators to manage and plan the power generation schedule by considering the relationship between power charging tariff and power management.

I.1.2    Types and characteristics of general demand response market

The DR market that we considered one of use cases of VPP could be classified according to the operating method and the incentive system as follow:

  - Price based demand response (PBDR) :

    - Based on the pricing factors, the system operator analyzes the supply and demand of overall power system according to time and environment, and it imposes a proper charge at each time. According to electricity charging rate, power consumers adjust their power usage voluntarily

    - This method analyzes overall power usage of all power consumers on an annual basis; therefore, the system operator determines and announces proper power charges by taking into the weight of power usage consideration (e.g., seasonal, peak time etc.).

  - Incentive based demand response (IBDR) :

    - In a specific situation (e.g., unbalanced power supply, peak power load, frequency instability), the system operator sends signals to the demand response market participants to manage the power usage to solve the problem. At this time, incentives/penalties are provided/imposed to the participants based on the contract. In this way, the system operator forces consumers to manage their power usage based on the contract. Table I.1 lists various demand response programs.

| Program | Explanation | Purpose | Participants | Type |
| --- | --- | --- | --- | --- |
| Utility program |  |  |  |  |
| Time of use | Differential rates according to time period | Minimize peak power | Whole of participants | PBDR |
| Curtailable load programs | Charging discount according to load control | Emergency Reliability of system | Type of energy usage | IBDR |
| Emergency DR | Incentive for load control | Emergency Reliability of system | Large scale power user | IBDR |
| Aggregator programs | Contract between aggregator and system operator | Minimize peak power Emergency | Aggregator | IBDR |
| Peak time rebates | Give incentive according to estimated peak baseline | Minimize peak power | Small scale power user | IBDR |
| Direct load control | Control customers’ electricity | Minimize peak power | Small scale power user | IBDR |
| Critical peak pricing | Based on ToU, high rates are applied under certain conditions | Power system stability | Whole of power consumer Industry | PBDR |
| Real time pricing | Price changes based on whole sale prices | Minimize peak power Power system stability | Type of energy usage | PBDR |

Table. I.1. Types and characteristics of demand response programs

    - As Table I.1 mentioned, most IBDR programs consider an urgent situation in the system. In other words, if the pre-contracted power operation of IBDR participant is not achieved, the system operator has to activate an alternative plan, which will incur additional costs. This is the reason that the penalty is imposed if the DR market participant who fails to achieve the actual pre-engaged contract, and it indicates that the VESS system is needed for the practical power environment.

I.1.3    The use of VESS from the perspective of power consumers and its direction

The main issue from the perspective of power consumers is the minimization of electricity tariffs through participating in the demand response market. As mentioned in Table I.1, consumers in the electricity market are classified according to the amount or type of electricity they used. This classification also effects on the decision of demand response market that they could participate. Therefore, it is possible that VESS can be applied  to the DR market proposed in the table based on the integrated operation of various algorithms such as HEMS, BEMS, and FEMS. In other words, the application of VESS not only enables more small scale power consumers to participate in the DR market but also has the advantage of enabling more efficient power system operation by configuring more controllable demand power resources from the system operator point of view.

I.1.4    The use of VESS from the perspective of system operator to maintain power system

The VESS system encourages more power consumers to participate in the demand resource market and makes that various  power consumers are able to stabilize their power consumption rates. In addition, from the system operator’s perspective, the VESS enables that the system operator operates the power system more efficiently based on the diversification of controllable power resources. In now a day, the widespread of renewable energy and electric vehicle cause many uncertain factors in the system. To minimize the risk of the system, the operator needs to reserve more baseline power supply in the system or schedules rapid generators to maintain the system. However, the broadening of the DR market with the VESS enables that system operators make possible to minimize the financial losses and maintain system reliability more efficiently in an environment where there are increment of undetermined power resources.

Bibliography.

[1] 	Safak bayram, et. al., “A survey on energy trading in smart grid,” IEEE Globalsip
            Conference, 2014.

[2]	IEC white paper, “Electrical energy storage,” 2011.

_____________________