|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  | SG11-TD182/GEN |  |
| --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 11 |  |
|  |  |  | Original: English |  |
| Question(s): |  | 2/11 |  | Geneva, 6-15 July 2022 |
| TD |  |  |  |  |
| Source: |  | Editors |  |  |
| Title: |  | Output – draft new Recommendation Q.QKDN_profr “Quantum key distribution networks - Protocol framework” (Geneva, 6-15 July 2022) |  |  |
| Contact: |  | Kaoru Kenyoshi NICT Japan | E-mail: kaoru.kenyoshi@nict.go.jp |  |
| Contact: |  | Taesang Choi ETRI Korea (Rep. of) | E-mail: choits@etri.re.kr |  |
| Contact: |  | Hongyu Wu QuantumCTek Co., Ltd. China | E-mail: hongyu.wu@quantum-info.com |  |

| Abstract: | This TD is the revised draft of Recommendation ITU-T Q.QKDN_profr “Quantum key distribution networks - Protocol framework”. |
| --- | --- |

Summary

This TD is the outcome of revised draft Recommendation ITU-T Q.QKDN_profr “Quantum key distribution networks - Protocol framework” based on the discussion results on contribution C75 with modifications at the Q2/11 meetings (Geneva, 6-15 July 2022).

Attachments:

Annex A Draft Recommendation ITU T Y.QKDN_profr “QKDN - Protocol framework”

ANNEX A

Draft new Recommendation ITU-T Q.QKDN_profr

Quantum key distribution networks – Protocol framework

Summary

Recommendation ITU-T Q.QKDN_profr specifies a framework for signalling requirements and protocols for quantum key distribution networks (QKDN).

Keywords

Protocol, QKD (quantum key distribution), QKDN (QKD network), signalling requirement

Table of Contents

1	Scope	5

2	References	5

3	Definitions	5

3.1	Terms defined elsewhere	5

3.2	Terms defined in this Recommendation	6

4	Abbreviations and acronyms	6

5	Conventions	6

6	Overview	6

7	Signalling requirements	8

8	Protocol suites and stacks	8

Appendix I  Signalling procedures	10

Appendix II  Possible grouping of interfaces in QKDN	12

Bibliography	14

Draft new Recommendation ITU-T Q.QKDN_profr

Quantum key distribution networks - Protocol framework

# Scope

This Recommendation specifies a framework for signalling aspects of a quantum key distribution network (QKDN), especially the following areas:

- Overview of signalling and protocols for QKDN;

- Signalling requirements for QKDN;

- Protocol suites for QKDN.

NOTE – QKD protocols which perform between a pair of QKD modules through QKD links are outside the scope of this Recommendation.

# References

[ITU-T X.1710] Recommendation ITU-T X.1710 (2020), Security framework for quantum key distribution networks

[ITU-T X.1712] Recommendation ITU-T X.1712, Security requirements and measures for QKD networks - key management

[ITU-T Y.3800] 	Recommendation ITU-T Y.3800 (2019)/Cor.1 (2020), Overview on networks supporting quantum key distribution.

[ITU-T Y.3801] 	Recommendation ITU-T 3801 (2020) Functional requirements for quantum key distribution network.

[ITU-T Y.3802] 	Recommendation ITU-T Y.3802 (2020), Functional architecture of the quantum key distribution network.

[ITU-T Y.3803] 	Recommendation ITU-T Y.3803 (2020), Key management for quantum key distribution Networks

[ITU-T Y.3804] 	Recommendation ITU-T Y.3804 (2020), Control and Management for Quantum Key Distribution Networks.

# Definitions

  - Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

    - information theoretically secure (IT-secure) [ITU-T Y.3800]: Secure against any deciphering attack with unbounded computational resources.

    - key management [ITU-T Y.3800]: All activities performed on keys during their life cycle starting from their reception from the quantum layer, the storage, formatting, relay, synchronization, authentication, to supply to cryptographic application and delete or preserve depending on the key management policy.

    - key manager (KM) [ITU-T Y.3800]: A functional module located in a quantum key distribution (QKD) node to perform key management in the key management layer.

    - key manager link (KM link) [ITU-T Y.3800]: A communication link connecting key managers (KMs) to perform key management.

    - key relay [ITU-T Y.3800]: A method to share keys between arbitrary quantum key distribution (QKD) nodes via intermediate QKD node(s).

    - quantum key distribution (QKD) [b-ETSI GR QKD 007]: Procedure or method for generating and distributing symmetrical cryptographic keys with information theoretical security based on quantum information theory.

    - quantum key distribution link (QKD link) [ITU-T Y.3800]: A communication link between two quantum key distribution (QKD) modules to operate the QKD.

NOTE – A QKD link consists of a quantum channel for the transmission of quantum signals, and a classical channel used to exchange information for synchronization and key distillation.

    - quantum key distribution module (QKD module) [ITU-T Y.3800]: A set of hardware and software components that implements cryptographic functions and quantum optical processes, including quantum key distribution (QKD) protocols, synchronization, distillation for key generation, and is contained within a defined cryptographic boundary.

NOTE – A QKD module is connected to a QKD link, acting as an endpoint module in which a key is generated. These are two types of QKD modules, namely, the transmitters (QKD-Tx) and the receivers (QKD-Rx).

    - quantum key distribution network (QKDN) [ITU-T Y.3800]: A network comprised of two or more quantum key distribution (QKD) nodes connected through QKD links.

NOTE – A QKDN allows sharing keys between the QKD nodes by key relay when they are not directly connected by a QKD link.

    - quantum key distribution network controller (QKDN controller) [ITU-T Y.3800]: A functional module, which is located in a quantum key distribution (QKD) network control layer to control a QKD network.

    - quantum key distribution network manager (QKDN manager) [ITU-T Y.3800]: A functional module, which is located in a quantum key distribution (QKD) network management layer to monitor and manage a QKD network.

    - quantum key distribution node (QKD node) [ITU-T Y.3800]: A node that contains one or more quantum key distribution (QKD) modules protected against intrusion and attacks by unauthorized parties.

NOTE – A QKD node can contain a key manager (KM).

  - Terms defined in this Recommendation

None.

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

IT-secure	Information-theoretically secure

KM	Key manager

QKD	Quantum Key Distribution

QKDN	QKD Network

# Conventions

None.

# Overview

Basic functions and layered structures of the QKDN are defined in [ITU-T Y.3800]. Functional requirements and architectures are specified in [ITU-T Y.3801] and [ITU-T Y.3802], respectively. A security framework for the QKDN is specified in [ITU-T X.1710], by addressing the security threats against the QKDN, and deriving the general security requirements and the security measures for the QKDN.

This Recommendation describes a framework of signalling requirements and protocols for QKDN. Various kinds of protocols can be used in a QKDN. This Recommendation specifies a framework of signalling requirements and protocols for key management layer, QKDN control layer and QKDN management layer. Protocols for quantum layer which are performed between two QKD modules are outside the scope of this Recommendation.

Figure 1 shows the functional architecture of QKDN which is defined in [ITU-T Y.3802].

Figure 1 – A functional architecture model of QKDN defined in [ITU-T Y.3802]

The following reference points are defined in [ITU-T Y.3802].

- reference points of QKD modules: Qqc, Qsync, Qdist

- reference points of KMs: Kq-1, Kq-2, Kx-1, Kx-2

- reference points of QKDN controllers: Ck, Cq, Cops, Cqrp, Cx

- reference points of QKDN managers: Mq, Mops, Mqrp, Mk, Mc, Mu

- reference points of user network managers: Ma

- reference points of cryptographic applications: Ak, Ax

The functional architecture model in figure 1 and the above reference points defined in [ITU-T Y.3802] are the references of this Recommendation.

Reference points of Qqc, Qsync, Qdist in the quantum layer, and reference points of Ma, and Ax in the user network are outside the scope of this Recommendation.

# Possible grouping of interfaces in QKDN

The scheme of grouping may affect the structure of series of the interface Recommendations in QKDN. There are several possible schemes for grouping. Note that Ax and Mu are outside of scope of this Recommendation.

- Grouped from reference points perspective

QKDN functional architecture model, shown in Figure 1, describes reference points of QKD modules, key management, QKDN control, QKDN management, user network management, and cryptographic application. Interfaces can be grouped from reference point perspective as follows:

- Interface group of KMs: Kq-1, Kq-2, Kx-1, Kx-2

- Interface group of QKDN controllers: Ck, Cq, Cops, Cqrp, Cx

- Interface group of QKDN managers: Mq, Mops, Mqrp, Mk, Mc, Mu

- Interface group of cryptographic applications: Ak

Figure 2 illustrates this grouping scheme.

Figure 2 – Grouping by reference points

- Grouped from functional unit perspective (classified into several units according to functions)

Key supply and key relay can be achieved by defining Ak, Kx and Ck. This grouping might be the functional unit #1. Then, other interfaces to realize functionalities such as key generation might be the functional unit #2 and QKDN management might be the functional unit #3. Figure 3 illustrates an example of grouping by functional units. The functional unit #1 consists of Ak, Kx, and Ck. The functional unit #2 consists of Cq, Kq-1, Kq-2, Cops and Cqrp. The functional unit #3 covers QKDN management. This kind of grouping need to be considered for OKDN implementation phases.

Figure 3 – Grouping by functional units

- Grouped from node acting role perspective (origination, transfer, termination)

This grouping scheme is based on node acting roles, such as origination, transfer, and termination nodes. Each node would have several implementation ways in general, thus all reference points need to be considered in each node to cover all implementation ways. In other words, it is difficult to make groping by node acting role.

- Grouped from layer perspective (key management layer, QKDN control, QKDN management).

Considering layers such as key management, QKDN control and QKDN management, interfaces related to each layer could be grouped. Figure 4 shows an example of grouping by layers. In this figure, the QKDN management layer is not shown because the layer is orthogonal from other layers. The key management layer includes Kx, Kq and Ak interfaces, and the control layer consists of Ck, Cq, Cops and Cqrp. This grouping is very similar with grouping by reference points, described in 1), and the difference is only the grouping of Ak, where the Ak interface is considered to relate key management.

Figure 4 – Grouping by layers

# Signalling requirements

This clause specifies signalling requirements of each reference points. Key file format and metadata are defined in [ITU-T Y.3803]. Key data, metadata and control and management information are specified in [ITU-T Y.3804] and [ITU-T X.1712].

Table 1 summarises information which is transferred at each reference point.

Table 1 – transferred information at reference points

| reference points | Transferred information |  |  | Note |
| --- | --- | --- | --- | --- |
|  | key data | metadata | Control and management information |  |
| Kq-1 | ✓ | ✓ | ✓ |  |
| Kq-2 |  | ✓ | ✓ |  |
| Kx-1 | ✓ | ✓ | ✓ | IT-secure encryption for key relay such as OTP is highly recommended. |
| Kx-2 |  | ✓ | ✓ |  |
| Ck |  | ✓ | ✓ |  |
| Cq |  | ✓ | ✓ |  |
| Cops |  |  | ✓ |  |
| Cqrp |  |  | ✓ |  |
| Cx |  | ✓ | ✓ |  |
| Mq |  |  | ✓ |  |
| Mops |  |  | ✓ |  |
| Mqrp |  |  | ✓ |  |
| Mk |  |  | ✓ |  |
| Mc |  |  | ✓ |  |
| Mu |  |  | ✓ |  |
| Ak | ✓ | ✓ |  |  |

# Protocol suites and stacks

This clause specifies protocol suites in QKDN. Appropriate protocols can be selected for each reference points and network interfaces.

Table 2 includes list of protocols which can be applied at each reference point.

Table 2 – protocol suites

|  |  | reference | Note |
| --- | --- | --- | --- |
| High layer protocols | RPC HTTP/HTTPS | RFC 5531 [b-IETF RFC 5531] RFC 7231 [b-IETF RFC 7231: |  |
| L4 protocols | TLS TCP UDP | RFC 5246 [b-IETF RFC 5246] RFC 793 [b-IETF RFC793] RFC 768 [b-IETF 768] |  |
| L3 protocols | IP | RFC 791 [b-IETF 791] RFC2460 [b-IETF 2460] |  |
| L2 protocols | Ethernet | IEEE 802.3 [b-IEEE 802.3] |  |

Figure 2 illustrates protocol stacks between cryptographic applications and KMs and between QKDN controllers in QKDN.

Figure 2 - Protocol stacks between cryptographic applications and KMs and between QKDN controllers in QKDN

Figure 3 illustrates protocol stacks between QKDN controller, KM, QKD module, QKD link and QKDN manager in QKDN.

Figure 3 – Protocol stacks between QKDN controller, KM, QKD module, QKD link and QKDN manager in QKDN.

Appendix I

Signalling procedures

(This appendix does not form an integral part of this Recommendation.)

- Key relay

Figure I-1 shows typical signalling procedures for key request, key relay, and key supply for a distributed QKDN which is defined in [ITU-T Y.3802].

At first, the source cryptographic application sends “Key request” message to the KM1 in the QKD node 1, where is the nearest node of the source cryptographic application. Then, the KM1 sends “Key replay next hop request” message to the QKDN controller in the QKD node 1, and the QKDN controller responds “Response to key relay next hop request” message with the next hop destination for key relay, then the KM1 relays key along with the response. The KM2 and the QKDN controller in the QKD node 2 performs the same procedures as the KM1 of the QKDN node 1. If the key reaches the destination QKD node which is the nearest node to the destination cryptographic application, the KM (shown as KM3 in the figure) sends “Notify completion of key relay” message to the source KM (shown as the KM1 in the figure), then the KM 1 responds “Response to key request” message to the source cryptographic application with keys. The source cryptographic application sends “Notify key ID” message to the destination cryptographic application with key ID. The destination cryptographic application sends “Key request with ID” message with key ID which is received from the source cryptographic application, then the nearest KM (shown as KM3 in the figure) responds “Response to key request” message with keys.

Figure I-1 – Typical signalling procedures for key relay for distributed QKDNs

Figure I-2 shows typical signalling procedures for key request, key relay, and key relay for a centralized QKDN which is defined in [ITU-T Y.3802].

At first, the source cryptographic application sends “Key request” message to the KM1 in the QKD node 1, where is the nearest node of the source cryptographic application. Then, the KM1 sends “Notify key replay request” message to the QKDN controller, and the QKDN controller responds “Key relay request” message with the full key relay route to the destination node, then the KM1 starts key relay along with the key relay route. The KM2 in QKD node 2 performs key relay according with the key relay route. If the key reaches the destination QKD node which is the nearest node to the destination cryptographic application, the KM (shown as KM3 in the figure) sends “Notify completion of key relay” message to the source KM (shown as KM1 in the figure), then the KM 1 sends “Response to key relay request” message to the QKDN controller, and also responds “Response to key request” message to the source cryptographic application with keys. The source cryptographic application sends “Notify key ID” message to the destination cryptographic application with key ID. The destination cryptographic application sends “Key request with ID” message with key ID which is received from the source cryptographic application, then the nearest KM (shown as KM3 in the figure) responds “Response to key relay” message with keys.

Figure I-2 – Typical signalling procedures for key relay for centralized QKDNs

NOTE 1 – Notify key ID message is transmitted through the Ax reference point between two cryptographic applications. This signalling message is outside the scope of this Recommendation.

NOTE 2 – Figure A-1 and A-2 illustrate signalling procedures for the case 2 of key relay which is specified in [ITU-T Y.3800].

Editor’s note – The wording of proactive key supply will be discussed at the next meeting.

- Key request and key supply

There are two modes that can be applied for the key request and key supply at the Ak interface. One mode is key supply upon request, wherein both KMs at the source and destination QKD nodes initiate key supplies after receiving key requests from the corresponding cryptographic applications. The other mode is proactive key supply, wherein the KM at the source QKD node initiates a key supply upon request, and then instructs the KM at the destination QKD node to make a proactive key supply. The proactive key supply can be adopted in scenarios with much security concerns, wherein no direct communication is allowed for the cryptographic applications at both sides before they have KSA-keys.

- Key supply upon request

Figure I-3 shows typical signalling procedures for key supply upon request mode implemented by two QKD nodes.

The typical signalling procedures shown in Figure I-3 are briefly described as follows:

1) The source cryptographic application sends “Key request” message to the source KM at the source QKD node.

2) The source KM responds “Response to key request” message to the source cryptographic application with keys requested and the corresponding key IDs.

3) The source cryptographic application sends “Notify key ID” message to the destination cryptographic application with the key IDs.

4) The destination cryptographic application sends “Key request with ID” message with the received key IDs to the destination KM at the destination QKD node.

5) The destination KM responds “Response to key request” message to the destination cryptographic application with keys requested.

Figure I-3 – Typical signalling procedures for key supply upon request mode implemented by two QKD nodes

NOTE 1 – Notify key ID message is transmitted through the Ax reference point between two cryptographic applications. This signalling message is outside the scope of this Recommendation.

- Proactive key supply

Figure I-4 shows typical signalling procedures for proactive key supply mode implemented by two QKD nodes.

The typical signalling procedures shown in Figure I-4 are briefly described as follows:

1) The source cryptographic application sends “Session creation request” message to the source KM at the source QKD node.

2) The source KM sends “Session creation request” message to the corresponding QKDN controller.

3) The QKDN controller generates a session ID and sends “Session creation notification” message with the session ID to the destination KM at the destination QKD node. If there are distributed QKDN controllers, the “Session creation notification” message will be sent from the QKDN controller at the source QKD node to the QKDN controller at the destination QKD node, and then relayed to the destination KM.

4) The destination KM sends “Session creation notification” message with the received session ID to the destination cryptographic application.

5) The destination cryptographic application responds “Response to session creation notification” message to the destination KM with the session creation result.

6) The destination KM responds “Response to session creation notification” message to the corresponding QKDN controller with the received session creation result. If there are distributed QKDN controllers, the “Response to session creation notification” message will be sent from the destination KM to the QKDN controller at the destination QKD node, and then relayed to the QKDN controller at the source QKD node.

7) As the session is successfully created, the QKDN controller responds “Response to session creation request” message to the source KM with the session ID.

8) The source KM responds “Response to session creation request” message to the source cryptographic application with the received session ID.

9) The source cryptographic application sends “Key request with session ID” message to the source KM with the received session ID.

10) The source KM sends “Key supply notification” message to the destination KM with the number of keys to be supplied.

11) The destination KM sends “Proactive key supply” message to the destination cryptographic application with the notified number of keys.

12) The destination cryptographic application responds “Response to proactive key supply” message to the destination KM with key IDs of the received keys.

13) The destination KM responds “Response to key supply notification” message to the source KM with the received key IDs.

14) The source KM responds “Response to key request with session ID” message to the source cryptographic application with keys corresponding to the received key IDs.

Figure I-4 – Typical signalling procedures for proactive key supply mode implemented by two QKD nodes

Appendix II

Possible grouping of interfaces in QKDN

(This appendix does not form an integral part of this Recommendation.)

The scheme of grouping may affect the structure of series of the interface Recommendations in QKDN. There are several possible schemes for grouping. Note that Ax and Mu are outside of scope of this Recommendation.

- Grouped from reference points perspective

QKDN functional architecture model, shown in Figure 1, describes reference points of QKD modules, key management, QKDN control, QKDN management, user network management, and cryptographic application. Interfaces can be grouped from reference point perspective as follows:

- Interface group of KMs: Kq-1, Kq-2, Kx-1, Kx-2

- Interface group of QKDN controllers: Ck, Cq, Cops, Cqrp, Cx

- Interface group of QKDN managers: Mq, Mops, Mqrp, Mk, Mc, Mu

- Interface group of cryptographic applications: Ak

Figure II-1 illustrates this grouping scheme.

Figure II-1 – Grouping by reference points

- Grouped from functional unit perspective (classified into several units according to functions)

Key supply and key relay can be achieved by defining Ak, Kx and Ck. This grouping might be the functional unit #1. Then, other interfaces to realize functionalities such as key generation might be the functional unit #2 and QKDN management might be the functional unit #3. Figure II-2 illustrates an example of grouping by functional units. The functional unit #1 consists of Ak, Kx, and Ck. The functional unit #2 consists of Cq, Kq-1, Kq-2, Cops and Cqrp. The functional unit #3 covers QKDN management. This kind of grouping need to be considered for OKDN implementation phases.

Figure II-2 – Grouping by functional units

- Grouped from node acting role perspective (origination, transfer, termination)

This grouping scheme is based on node acting roles, such as origination, transfer, and termination nodes. Each node would have several implementation ways in general, thus all reference points need to be considered in each node to cover all implementation ways. In other words, it is difficult to make groping by node acting role.

- Grouped from layer perspective (key management layer, QKDN control, QKDN management).

Considering layers such as key management, QKDN control and QKDN management, interfaces related to each layer could be grouped. Figure II-3 shows an example of grouping by layers. In this figure, the QKDN management layer is not shown because the layer is orthogonal from other layers. The key management layer includes Kx, Kq and Ak interfaces, and the control layer consists of Ck, Cq, Cops and Cqrp. This grouping is very similar with grouping by reference points, described in 1), and the difference is only the grouping of Ak, where the Ak interface is considered to relate key management.

Figure II-3 – Grouping by layers

# Bibliography

[b-ETSI GR QKD 007] 	Group Report ETSI GS QKD 007 (2018), Quantum Key Distribution (QKD); Vocabulary.

[b-ETSI GS QKD 014] 	ETSI GS QKD 014 V1.1.1 (2019-02), Protocol and data format of REST-based key delivery API.

[b-ITU-T FG-QIT4N D2.3] 	D2.3 Technical Report on quantum key distribution network (QKDN) protocols part 2: Key management layer, QKDN control layer, and QKDN management layer.

[b-IETF RFC 5531]	IETF RFC 5531, RPC: Remote Procedure Call Protocol Specification Version 2.

[b-IETF RFC 7231]	IETF RFC 7321, Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content.

[b-IETF RFC 5246]	IETF RFC 5246, The Transport Layer Security (TLS) Protocol                              Version 1.2.

[b-IETF RFC 793]	IETF RFC 793, TRANSMISSION CONTROL PROTOCOL.

[b-IETF RFC 768]	IETF RFC 768, User Datagram Protocol.

[b-IETF RFC 791]	IETF RFC 791, INTERNET PROTOCOL.

[b-IETF RFC 2460]	IETF RFC 2460, Internet Protocol, Version 6 (IPv6) Specification.

[b-IEEE 802.3]	IEEE 802.3-2018, IEEE Standard for Ethernet.

_____________________