|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  |  | SG11-TD183/GEN |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 11 |  |
|  |  |  |  | Original: English |  |
| Question(s): |  |  | 2/11 | Geneva, 6-15 July 2022 |  |
| TD |  |  |  |  |  |
| Source: |  |  | Editors |  |  |  |  |
| Title: |  |  | Output – draft new Recommendation Q.QKDN_Kx “Protocols for Kx interface for QKDN” (Geneva, 6-15 July 2022) |  |  |  |  |
| Contact: |  |  | Kaoru KENYOSHI NICT Japan |  |  | Tel: +81 50 3566 5852 E-mail: kaoru.kenyoshi@nict.go.jp |  |
| Contact: |  |  | Mariko Honda NICT Japan |  |  | E-mail: mariko.honda@ntt-at.co.jp |  |
| Contact: |  |  | Hongyu Wu QuantumCTek Co., Ltd. China |  |  | Email: hongyu.wu@quantum-info.com |  |
| Contact: |  |  | Zhangchao Ma CAS Quantum Network Co., Ltd. China |  |  | Email: mazhangchao@qtict.com |  |
| Contact: |  |  | Junsen Lai CAICT, Ministry of Industry and Information Technology (MIIT) China |  |  | E-mail: laijunsen@caict.ac.cn |  |

| Abstract: | This is the output of draft Recommendation Q.QKDN_Kx “Protocols for Kx interface for QKDN” based on the result of the SG11 meeting Geneva, 6-15 July 2022. |
| --- | --- |

Summary

This is the output draft of Q.QKDN_Kx “Protocols for Kx interface for QKDN” based on the result of  the SG11 meeting Geneva, 6-15 July 2022. The text proposed in C76 Rev.1 is agreed as it is in the meeting.

Attachment:

Annex A: Draft Recommendation ITU T Q.QKDN_Kx “Protocols for Kx interface for QKDN”

Annex A

Draft new Recommendation ITU-T Q.QKDN_Kx

Protocols for Kx interface for QKDN

Summary

Recommendation ITU-T Q.QKDN_Kx specifies protocols for Kx interface in quantum key distribution networks (QKDN).

Keywords

Protocol, QKD (quantum key distribution), QKDN (QKD network), signalling procedure, message parameters
Table of Contents

1.	Scope	5

2.	References	5

3.	Definitions	5

3.1.	Terms defined elsewhere	5

3.2.	Terms defined in this Recommendation	7

4.	Abbreviations and acronyms	7

5.	Conventions	7

6.	Kx interface	7

7.	Signalling procedure	7

8.	Message format and parameters	8

8.1. Key relay	8

8.2. Notify completion of key relay	9

9.	Security considerations	9

Bibliography	12

# Scope

This Recommendation specifies protocols at Kx interface for quantum key distribution network (QKDN) especially the following areas.

- signalling procedures for Kx interface for QKDN;

- message format and parameters for Kx interface for QKDN;

- security considerations.

# References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T Y.3800] 	Recommendation ITU-T Y.3800 (2019)/Cor.1 (2020), Overview on networks supporting quantum key distribution.

[ITU-T Y.3802] 	Recommendation ITU-T Y.3802 (2020), Functional architecture of the quantum key distribution network.

[ITU-T Y.3803] 	Recommendation ITU-T Y.3803 (2020), Key management for quantum key distribution Networks

[ITU-T Q.QKDN_profr] draft Recommendation Q.QKDN_profr, QKDN - Protocol framework.

# Definitions

# Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

    - key management [ITU-T Y.3800]: All activities performed on keys during their life cycle starting from their reception from the quantum layer, the storage, formatting, relay, synchronization, authentication, to supply to cryptographic application and delete or preserve depending on the key management policy.

    - key management agent (KMA) [ITU-T Y.3802]: A functional element to manage keys generated by a quantum key distribution (QKD) module/QKD modules in a QKD node (trusted node).

NOTE - KMA acquires keys from a QKD module/QKD modules, synchronizes, resize, formats, and stores them. It also relays keys through key management agent (KMA) links.

    - key management agent-key (KMA-key) [ITU-T Y.3803]: Key data stored and processed in a key management agent (KMA), and securely shared between a KMA and a matching KMA.

    - key management agent link (KMA link) [ITU-T Y.3802]: A communication link connecting KMAs to perform key relay and communications for key management.

    - key manager (KM) [ITU-T Y.3800]: A functional module located in a quantum key distribution (QKD) node to perform key management in the key management layer.

    - key manager link (KM link) [ITU-T Y.3800]: A communication link connecting key managers (KMs) to perform key management.

    - key relay [ITU-T Y.3800]: A method to share keys between arbitrary quantum key distribution (QKD) nodes via intermediate QKD node(s).

    - quantum key distribution (QKD) [b-ETSI GR QKD 007]: Procedure or method for generating and distributing symmetrical cryptographic keys with information theoretical security based on quantum information theory.

    - quantum key distribution link (QKD link) [ITU-T Y.3800]: A communication link between two quantum key distribution (QKD) modules to operate the QKD.

NOTE – A QKD link consists of a quantum channel for the transmission of quantum signals, and a classical channel used to exchange information for synchronization and key distillation.

    - quantum key distribution module (QKD module) [ITU-T Y.3800]: A set of hardware and software components that implements cryptographic functions and quantum optical processes, including quantum key distribution (QKD) protocols, synchronization, distillation for key generation, and is contained within a defined cryptographic boundary.

NOTE – A QKD module is connected to a QKD link, acting as an endpoint module in which a key is generated. These are two types of QKD modules, namely, the transmitters (QKD-Tx) and the receivers (QKD-Rx).

    - quantum key distribution network (QKDN) [ITU-T Y.3800]: A network comprised of two or more quantum key distribution (QKD) nodes connected through QKD links.

NOTE – A QKDN allows sharing keys between the QKD nodes by key relay when they are not directly connected by a QKD link.

    - quantum key distribution network controller (QKDN controller) [ITU-T Y.3800]: A functional module, which is located in a quantum key distribution (QKD) network control layer to control a QKD network.

    - quantum key distribution node (QKD node) [ITU-T Y.3800]: A node that contains one or more quantum key distribution (QKD) modules protected against intrusion and attacks by unauthorized parties.

NOTE – A QKD node can contain a key manager (KM).

This Recommendation uses the following terms defined elsewhere:

# Terms defined in this Recommendation

None

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

ID	Identifier

KM	Key manager

KMA	Key Management Agent

QKD	Quantum Key Distribution

QKDN		QKD Network

# Conventions

None.

# Kx interface

Kx interface is at a reference point connecting two key managers (KMs) in each quantum key distribution (QKD) node via a KM link. It is responsible for exchanging information and operations required for key relay, key synchronization and authentication between KMs.

# Signalling procedure

The whole signalling procedure of Key request, key relay, and key supply in QKDN is described in the Appendix I of [ITU-T Q.QKDN_profr]. The protocol suites applied for the signalling described in Clause 6 and clause 7 are specified in Clause 8 of [ITU-T Q.QKDN_profr].

The key is relayed in Kx interface from the source to the destination. A KM sends the key to the KM which is specified in the message from a QKDN controller.

Figure 1 shows signalling procedures for key relay at Kx reference point.

Figure 1 - Signalling procedures for key relay at the Kx reference point

When the key reaches at the destination KM, the destination KM notifies the completion to the source KM. The notification is sent with the information to specify the transaction linked to the key relay which has been completed.

Figure 2 shows signalling procedures for Notification of key relay completion at Kx reference point.

Figure 2 - Signalling procedures for Notification of key relay completion at the Kx reference point

# Message format and parameters

This clause specifies message format and parameters for the Kx reference point.

M indicates that this parameter is mandatory to a message, and O indicates that this parameter is optional to a message in tables in this clause.

# 8.1. Key relay

Key relay message conveys the key from the source KM to the destination KM.

Table 1 shows message format and parameters of Key relay.

Table 1 - Message format and parameters of Key relay

| parameter |  | description | format | M/O | remarks |
| --- | --- | --- | --- | --- | --- |
| Source KMA ID |  | ID of KMA that is the source in the entire key relay route | string | M |  |
| Destination KMA ID |  | ID of KMA that is the destination in the entire key relay route | string | M |  |
| Transit KMA IDs |  | List of IDs of KMAs that are the transition node of key relay route | string | O |  |
| Keys |  | Key file | Array of objects | M |  |
|  | KMA-key IDs | IDs of KMA-keys relayed | string | M |  |
|  | KMA-keys | KMA-keys relayed | string | M |  |
|  | Key extension | Extensions to key file | object | O | Hash value etc. |
| Key relay request ID |  |  |  | O |  |
| Extension |  | Array of extension parameters | Array of objects | O |  |

# 8.2. Notify completion of key relay

When the key reaches the destination KM, the KM notifies the completion of the key relay to the source KM. The notification is sent with the information to specify the transaction linked to the key relay which has been completed.

Table 2 shows message format and parameters of Notify completion of key relay.

Table 2 - Message format and parameters of Notify completion of key relay

| parameter | description | format | M/O | remarks |
| --- | --- | --- | --- | --- |
| Key relay request ID |  |  | O |  |

# Security considerations

Key data, metadata and control and management information are transferred through Kx reference point. Security requirements and measures to protect them are specified in [ITU-T X.1712].

Bibliography

[b-ETSI GR QKD 007] 	Group Report ETSI GS QKD 007 (2018), Quantum Key Distribution (QKD); Vocabulary.

[b-ETSI GS QKD 014] 	ETSI GS QKD 014 V1.1.1 (2019-02), Protocol and data format of REST-based key delivery API.

[b-ITU-T FG-QIT4N D2.3] 	D2.3 Technical Report on quantum key distribution network (QKDN) protocols part 2: Key management layer, QKDN control layer, and QKDN management layer.

[b-ITU-T Y.3801] 	Recommendation ITU-T 3801 (2020) Functional requirements for quantum key distribution network.

[b-ITU-T Y.3804] 	Recommendation ITU-T Y.3804 (2020), Control and Management for Quantum Key Distribution Networks.

_______________________