|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  |  | SG11-TD184/GEN |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 11 |  |
|  |  |  |  | Original: English |  |
| Question(s): |  |  | 2/11 | Geneva, 6-15 July 2022 |  |
| TD |  |  |  |  |  |
| Source: |  |  | Editors |  |  |  |  |
| Title: |  |  | Output – draft new Recommendation Q.QKDN_Ck “Protocols for Ck interface for QKDN” ( Geneva, 6-15 July 2022) |  |  |  |  |
| Contact: |  |  | Kaoru KENYOSHI NICT Japan |  |  | Tel: +81 50 3566 5852 E-mail: kaoru.kenyoshi@nict.go.jp |  |
| Contact: |  |  | Mariko Honda NICT Japan |  |  | E-mail: mariko.honda@ntt-at.co.jp |  |
| Contact: |  |  | Hongyu Wu QuantumCTek Co., Ltd. China |  |  | E-mail: hongyu.wu@quantum-info.com |  |
| Contact: |  |  | Zhangchao Ma CAS Quantum Network Co., Ltd. China |  |  | E-mail: mazhangchao@qtict.com |  |
| Contact: |  |  | Junsen Lai CAICT, Ministry of Industry and Information Technology (MIIT) China |  |  | E-mail: laijunsen@caict.ac.cn |  |

| Abstract: | This is the output of draft new Recommendation Q.QKDN_Ck “Protocols for Ck interface for QKDN” based on the result of the SG11 meeting (Geneva, 6-15 July 2022). |
| --- | --- |

Summary

This is the output draft of Q.QKDN_Ck “Protocols for Ck interface for QKDN” based on the result of  the SG11 meeting  Geneva, 6-15 July 2022. The text proposed in C77 Rev.1 is agreed as it is in the meeting.

Attachment:

Annex A: Draft Recommendation ITU T Q.QKDN_Ck “Protocols for Ck interface for QKDN”

Annex A

Draft new Recommendation ITU-T Q.QKDN_Ck

Protocols for Ck interface for QKDN

Summary

Recommendation ITU-T Q.QKDN_Ck specifies protocols for Ck interface in quantum key distribution networks (QKDN).

Keywords

Protocol, QKD (quantum key distribution), QKDN (QKD network), signalling procedure, message parameters
Table of Contents

1.	Scope	5

2.	References	5

3.	Definitions	5

3.1.	Terms defined elsewhere	5

3.2.	Terms defined in this Recommendation	7

4.	Abbreviations and acronyms	7

5.	Conventions	7

6.	Ck Interface	7

7.	Signalling procedure	7

7.1.	Signalling procedures at CK reference point for a distributed QKDN	8

7.2.	Signalling procedures at CK reference point for a centralized QKDN	8

8.	Message format and parameters	9

8.1.	Key relay next hop request	9

8.2.	Response to key relay next hop request	9

8.3.	Notify key relay request	10

8.4.	Key relay request	11

8.5.	Response to key relay request	11

9.	Security considerations	11

Bibliography	14

# Scope

This Recommendation specifies protocols at Ck interface for quantum key distribution network (QKDN) especially the following areas.

- signalling procedures for Ck interface for QKDN;

- message format and parameters for Ck interface for QKDN;

- security considerations

# References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T Y.3800] 	Recommendation ITU-T Y.3800 (2019)/Cor.1 (2020), Overview on networks supporting quantum key distribution.

[ITU-T Y.3802] 	Recommendation ITU-T Y.3802 (2020), Functional architecture of the quantum key distribution network.

[ITU-T Q.QKDN_profr] draft Recommendation Q.QKDN_profr, QKDN - Protocol framework.

# Definitions

# Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

    - key management [ITU-T Y.3800]: All activities performed on keys during their life cycle starting from their reception from the quantum layer, the storage, formatting, relay, synchronization, authentication, to supply to cryptographic application and delete or preserve depending on the key management policy.

    - key management agent (KMA) [ITU-T Y.3802]: A functional element to manage keys generated by a quantum key distribution (QKD) module/QKD modules in a QKD node (trusted node).

NOTE - KMA acquires keys from a QKD module/QKD modules, synchronizes, resize, formats, and stores them. It also relays keys through key management agent (KMA) links.

    - key management agent link (KMA link) [ITU-T Y.3802]: A communication link connecting KMAs to perform key relay and communications for key management.

    - key manager (KM) [ITU-T Y.3800]: A functional module located in a quantum key distribution (QKD) node to perform key management in the key management layer.

    - key manager link (KM link) [ITU-T Y.3800]: A communication link connecting key managers (KMs) to perform key management.

    - key relay [ITU-T Y.3800]: A method to share keys between arbitrary quantum key distribution (QKD) nodes via intermediate QKD node(s).

    - key supply agent (KSA) [ITU-T Y.3802]: A functional element to supply keys to a cryptographic application, being located between a key management agent (KMA) and the client.

NOTE - Application interfaces for cryptographic applications are installed into the key supply agent (KSA). The KSA synchronizes keys, and verifies their integrity via a KSA link before supplying them to the client.

    - key supply agent link (KSA link) [ITU-T Y.3802]: A communication link connecting KSAs to perform key synchronization and integrity verification.

    - quantum key distribution (QKD) [b-ETSI GR QKD 007]: Procedure or method for generating and distributing symmetrical cryptographic keys with information theoretical security based on quantum information theory.

    - quantum key distribution link (QKD link) [ITU-T Y.3800]: A communication link between two quantum key distribution (QKD) modules to operate the QKD.

NOTE – A QKD link consists of a quantum channel for the transmission of quantum signals, and a classical channel used to exchange information for synchronization and key distillation.

    - quantum key distribution module (QKD module) [ITU-T Y.3800]: A set of hardware and software components that implements cryptographic functions and quantum optical processes, including quantum key distribution (QKD) protocols, synchronization, distillation for key generation, and is contained within a defined cryptographic boundary.

NOTE – A QKD module is connected to a QKD link, acting as an endpoint module in which a key is generated. These are two types of QKD modules, namely, the transmitters (QKD-Tx) and the receivers (QKD-Rx).

    - quantum key distribution network (QKDN) [ITU-T Y.3800]: A network comprised of two or more quantum key distribution (QKD) nodes connected through QKD links.

NOTE – A QKDN allows sharing keys between the QKD nodes by key relay when they are not directly connected by a QKD link.

    - quantum key distribution network controller (QKDN controller) [ITU-T Y.3800]: A functional module, which is located in a quantum key distribution (QKD) network control layer to control a QKD network.

    - quantum key distribution network manager (QKDN manager) [ITU-T Y.3800]: A functional module, which is located in a quantum key distribution (QKD) network management layer to monitor and manage a QKD network.

    - quantum key distribution node (QKD node) [ITU-T Y.3800]: A node that contains one or more quantum key distribution (QKD) modules protected against intrusion and attacks by unauthorized parties.

NOTE – A QKD node can contain a key manager (KM).

This Recommendation uses the following terms defined elsewhere:

# Terms defined in this Recommendation

None

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

ID	Identifier

KM	Key manager

KMA	Key Management Agent

KSA	Key Supply Agent

QKD	Quantum Key Distribution

QKDN		QKD Network

# Conventions

None.

# Ck Interface

Ck is a reference point connecting a QKDN controller control and management function in a QKDN controller and a KM control and management function in a key manager (KM). It is responsible for the QKDN controller to communicate control information with a key management agent (KMA) and a key supply agent (KSA).

# Signalling procedure

The whole signalling procedure of Key request, key relay, and key supply in QKDN is described in the Appendix I of [ITU-T Q.QKDN_profr]. The protocol suites applied for the signalling described in Clause 6 and clause 7 are specified in Clause 8 of [ITU-T Q.QKDN_profr]. Two kinds of signalling procedures are defined depending on the network architecture of distributed QKDN and centralized QKDN.

# Signalling procedures at CK reference point for a distributed QKDN

The distributed QKDN performs key relay with a series of hops between KMs to the destination KM. At the Ck interface of the distributed QKDN, a KM requests a QKDN controller for the information of the KM of neighbers for the next hop for key relay and the KM relays the key to the next KM based on the controller’s response. Then the next KM requests again to the controller for the next hop. This request and hop procedure repeats until the key relay is completed to the destination.

Figures 1 shows signalling procedures at the Ck reference point for key request in a distributed QKDN.

The KM sends Key relay next hop request to the QKDN controller in order to request the KM identifier (ID) of the next hop. The QKDN controller responds possible KMs with the KM IDs to hop to the next KM.

Figure 1 - Signalling procedures at Ck reference point in a distributed QKDN

# Signalling procedures at CK reference point for a centralized QKDN

In centralized QKDN, the request from the cryptographic application to the KM is transmitted to the QKDN controller, and the QKDN controller returns the whole route to the destination KM (list of KMs to be passed) to the requesting KM. When all the key relays are completed, the destination KM (KM that passes the key to the destination cryptographic application) returns notification that the key relay is completed.

Figures 2 shows signalling procedures at Ck reference point for key request in a centralized QKDN.

After the cryptographic application sends a key request to the KM, the KM sends notification of receipt of a key request to the QKDN controller with Notify key relay request. The QKDN controller specifies the whole route for the key transfer to the destination and notifies the requesting KM of the list of transit KMs by a Key relay request. The source KM starts the key relay according to the list of transit KMs. When the key relay completes at the KM to which the destination cryptographic application is connected, the source KM notifies the QKDN controller of completion of key relay by Response to key replay request.

Figure 2 - Signalling procedures at the Ck reference point in a centralised QKDN

# Message format and parameters

This clause specifies message format and parameters for the Ck reference point.

M indicates that this parameter is mandatory to a message, and O indicates that this parameter is optional to a message in tables in this clause.

# Key relay next hop request

Table 1 shows message format and parameters of Key relay next hop request. Either Destination KMA ID or Application destination ID is mandatory to specify the destination.

Table 1 - Message format and parameters of Key relay next hop request

| parameter | description | format | M/O | remarks |
| --- | --- | --- | --- | --- |
| Source KMA ID | ID of KMA that is the source in the entire key relay route | string | O |  |
| Destination KMA ID | ID of KMA that is the destination in the entire key relay route | string | Either Destination KMA ID or Application destination ID is mandatory |  |
| Application destination ID | ID of the cryptographic application with which the source cryptographic application (i.e. source application) requests to communicate | string | Either Destination KMA ID or Application destination ID is mandatory |  |
| Extension | Array of extension parameters | Array of objects | O |  |

# Response to key relay next hop request

Table 2 shows message format and parameters of Response to key relay next hop request. For distributed QKDN, the QKDN controller returns the IDs of the possible KMs to reach the destination cryptographic application or the destination KMA ID.

Table 2 - Message format and parameters of Response to key relay next hop request

| parameter | description | format | M/O | remarks |
| --- | --- | --- | --- | --- |
| Source KMA ID | ID of KMA that is the source in the entire key relay route | string | O |  |
| Destination KMA ID | ID of KMA that is the destination in the entire key relay route | string | Mandatory if application destination ID is contained in the key relay next hop request message. |  |
| Next KMA IDs | IDs of KMAs available as a next relay hop to relay the keys to the destination KMA | Array of string | M |  |
| Extension | Array of extension parameters | Array of objects | O |  |

# Notify key relay request

Table 3 message format and parameters of Notify key relay request. In a centralized QKDN, when a key relay is requested by a cryptographic application, the KM notifies the QKDN controller the receipt of a request and requests the whole route of the key relay. At this time, as same in the case of the distributed QKDN, information is required for either the Destination KMA ID or the Application destination ID in order to specify the destination of the key relay.

Table 3 - Message format and parameters of Notify key relay request

| parameter | description | format | M/O | remarks |
| --- | --- | --- | --- | --- |
| Source KMA ID | ID of KMA that is the source in the entire key relay route | string | O |  |
| Destination KMA ID | ID of KMA that is the destination in the entire key relay route | string | Either Destination KMA ID or Application destination ID is mandatory |  |
| Application destination ID | ID of the cryptographic application with which the source cryptographic application (i.e. source application) requests to communicate | string | Either Destination KMA ID or Application destination ID is mandatory |  |
| Extension | Array of extension parameters | Array of objects | O |  |

# Key relay request

Table 4 shows message format and parameters of Key relay request. The QKDN controller returns all KMs in the route (Transit KMA IDs) to reach the destination cryptographic application or the destination KMA ID.

Table 4 - Message format and parameters of Key relay request

| parameter | description | format | M/O | remarks |
| --- | --- | --- | --- | --- |
| Source KMA ID | ID of KMA that is the source in the entire key relay route | string | O |  |
| Destination KMA ID | ID of KMA that is the destination in the entire key relay route | string | M |  |
| Transit KMA IDs | List of IDs of KMAs that are the transition node of key relay route | string | M |  |
| Key relay request ID |  |  | O |  |
| Extension | Array of extension parameters | Array of objects | O |  |

# Response to key relay request

Table 5 shows message format and parameters of Response to key relay request.

Table 5 - Message format and parameters of Response to key relay request

| parameter | description | format | M/O | remarks |
| --- | --- | --- | --- | --- |
| Key relay request ID |  |  | O |  |

# Security considerations

Control and management information is transferred through Ck reference point. Security requirements and measures to protect it are specified in [ITU-T X.1712].

Bibliography

[b-ETSI GR QKD 007] 	Group Report ETSI GS QKD 007 (2018), Quantum Key Distribution (QKD); Vocabulary.

[b-ETSI GS QKD 014] 	ETSI GS QKD 014 V1.1.1 (2019-02), Protocol and data format of REST-based key delivery API.

[b-ITU-T FG-QIT4N D2.3] 	D2.3 Technical Report on quantum key distribution network (QKDN) protocols part 2: Key management layer, QKDN control layer, and QKDN management layer.

[b-ITU-T Y.3801] 	Recommendation ITU-T 3801 (2020) Functional requirements for quantum key distribution network.

[b-ITU-T Y.3803] 	Recommendation ITU-T Y.3803 (2020), Key management for quantum key distribution Networks

[b-ITU-T Y.3804] 	Recommendation ITU-T Y.3804 (2020), Control and Management for Quantum Key Distribution Networks.

___________________