|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  |  | SG11-TD185/GEN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 11 |  |  |
|  |  |  |  | Original: English |  |  |
| Question(s): |  | 2/11 |  |  | Geneva, 6-15 July 2022 |  |
| TD |  |  |  |  |  |  |  |
| Source: |  |  | Editors |  |  |  |  |
| Title: |  |  | Output – draft new Recommendation Q.QKDN_Ak “Protocols for Ak interface for QKDN” (Geneva, 6-15 July 2022) |  |  |  |  |
| Contact: |  |  | Hongyu Wu QuantumCTek Co., Ltd. China |  |  | E-mail: hongyu.wu@quantum-info.com |  |
| Contact: |  |  | Zhangchao Ma CAS Quantum Network Co., Ltd. China |  |  | E-mail: mazhangchao@qtict.com |  |
| Contact: |  |  | Kaoru KENYOSHI NICT Japan |  |  | Tel: +81 50 3566 5852 E-mail: kaoru.kenyoshi@nict.go.jp |  |
| Contact: |  |  | Mariko Honda NICT Japan |  |  | E-mail: mariko.honda@ntt-at.co.jp |  |
| Contact: |  |  | Junsen Lai CAICT, Ministry of Industry and Information Technology (MIIT) China |  |  | E-mail: laijunsen@caict.ac.cn |  |

| Abstract: | This TD is the output of draft new RecommendationQ.QKDN_Ak “Protocols for Ak interface for QKDN” (Geneva, 6-15 July 2022). |
| --- | --- |

Summary

This is the output draft of Q.QKDN_Ak “Protocols for Ak interface for QKDN” based on the result of  the SG11 meeting Geneva, 6-15 July 2022. The text proposed in C78 R1 and C83 is agreed as it is in the meeting.

Attachment:

Annex A: Draft Recommendation ITU T Q.QKDN_Ak “Protocols for Ak interface for QKDN”

Annex I

Draft new Recommendation ITU-T Q.QKDN_Ak

Protocols for Ak interface for QKDN

Summary

Recommendation ITU-T Q.QKDN_Ak specifies protocols for Ak interface in quantum key distribution networks (QKDN).

Keywords

Protocol, QKD (quantum key distribution), QKDN (QKD network), signalling procedure, message parameters
Table of Contents

1.	Scope	5

2.	References	5

3.	Definitions	5

3.1.	Terms defined elsewhere	5

3.2.	Terms defined in this Recommendation	6

4.	Abbreviations and acronyms	6

5.	Conventions	6

6.	Ak interface	6

7.	Signalling procedure	7

7.1.	Signalling procedure for key supply upon request	7

7.2.	Signalling procedure for proactive key supply	8

8.	Signalling parameters	10

8.1.	Key request	10

8.2.	Key request with ID	11

8.3.	Response to key request	11

8.4.	Session creation request	12

8.5.	Response to session creation request	12

8.6.	Session creation notification	13

8.7.	Response to session creation notification	13

8.8.	Key request with session ID	14

8.9.	Response to key request with session ID	14

8.10.	Proactive key supply	15

8.11.	Response to proactive key supply	15

9.	Protocol stack bindings	16

9.1.	TCP binding	16

9.2.	HTTPS binding	17

10.	Security considerations	18

# Scope

This Recommendation specifies protocols at Ak interface for QKDN especially the following areas.

- signalling procedures for Ak interface for QKDN;

- message format and parameters for Ak interface for QKDN;

- security considerations.

# References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T Y.3800] 	Recommendation ITU-T Y.3800 (2019)/Cor.1 (04/2020), Overview on networks supporting quantum key distribution.

[ITU-T Y.3802] 	Recommendation ITU-T Y.3802 (2020)/Cor.1 (04/2021), Quantum key distribution networks - Functional architecture.

[ITU-T Y.3803] 	Recommendation ITU-T Y.3803 (2020), Quantum key distribution networks - Key management.

[ITU-T Q.QKDN_profr] draft Recommendation Q.QKDN_profr, Quantum key distribution networks - Protocol framework.

# Definitions

# Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

    - key management [ITU-T Y.3800]: All activities performed on keys during their life cycle starting from their reception from the quantum layer, the storage, formatting, relay, synchronization, authentication, to supply to cryptographic application and delete or preserve depending on the key management policy.

    - key manager (KM) [ITU-T Y.3800]: A functional module located in a quantum key distribution (QKD) node to perform key management in the key management layer.

    - key relay [ITU-T Y.3800]: A method to share keys between arbitrary quantum key distribution (QKD) nodes via intermediate QKD node(s).

    - key supply agent (KSA) [ITU-T Y.3802]: A functional element to supply keys to a cryptographic application, being located between a key management agent (KMA) and the client.

NOTE - Application interfaces for cryptographic applications are installed into the key supply agent (KSA). The KSA synchronizes keys, and verifies their integrity via a KSA link before supplying them to the client.

    - key supply agent-key (KSA-key) [ITU-T Y.3803]: Key data stored and processed in a key supply agent (KSA), and securely shared between a KSA and a matching KSA.

    - quantum key distribution (QKD) [b-ETSI GR QKD 007]: Procedure or method for generating and distributing symmetrical cryptographic keys with information theoretical security based on quantum information theory.

    - quantum key distribution link (QKD link) [ITU-T Y.3800]: A communication link between two quantum key distribution (QKD) modules to operate the QKD.

NOTE – A QKD link consists of a quantum channel for the transmission of quantum signals, and a classical channel used to exchange information for synchronization and key distillation.

    - quantum key distribution module (QKD module) [ITU-T Y.3800]: A set of hardware and software components that implements cryptographic functions and quantum optical processes, including quantum key distribution (QKD) protocols, synchronization, distillation for key generation, and is contained within a defined cryptographic boundary.

NOTE – A QKD module is connected to a QKD link, acting as an endpoint module in which a key is generated. These are two types of QKD modules, namely, the transmitters (QKD-Tx) and the receivers (QKD-Rx).

    - quantum key distribution network (QKDN) [ITU-T Y.3800]: A network comprised of two or more quantum key distribution (QKD) nodes connected through QKD links.

NOTE – A QKDN allows sharing keys between the QKD nodes by key relay when they are not directly connected by a QKD link.

    - quantum key distribution node (QKD node) [ITU-T Y.3800]: A node that contains one or more quantum key distribution (QKD) modules protected against intrusion and attacks by unauthorized parties.

NOTE – A QKD node can contain a key manager (KM).

This Recommendation uses the following terms defined elsewhere:

# Terms defined in this Recommendation

None

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

HTTPS	HyperText Transfer Protocol Secure

ID	Identifier

KM	Key manager

KSA	Key Supply Agent

QKD	Quantum Key Distribution

QKDN		QKD Network

TCP		Transmission Control Protocol

# Conventions

None.

# Ak interface

Ak is a reference point connecting a cryptographic application and a key supply function in a KSA. It is responsible for sending key requests from the cryptographic application to the KSA, performing authentication between the cryptographic application and the KSA, and supplying keys from the KSA to the cryptographic application.

# Signalling procedure

The whole signalling procedure of key request, key relay, and key supply in QKDN is described in the Appendix I of [ITU-T Q.QKDN_profr]. The protocol suites applied for the signalling are specified in clause 8 of [ITU-T Q.QKDN_profr].

At Ak interface, signalling procedures for key request and key supply are different at the source and destination sides. Two modes can be applied for the key request and key supply at the Ak interface. One mode is key supply upon request, wherein both KMs at the source and destination sides initiate key supplies after receiving key requests from the corresponding cryptographic applications. The other mode is proactive key supply, wherein the KM at the source side initiates a key supply upon request, and then instructs the KM at the destination side to make a proactive key supply. The proactive key supply can be adopted in scenarios with much security concerns, wherein no direct communication is allowed for the cryptographic applications at both sides before they have KSA-keys.

Editor’s note – This sentence will be discussed at the next meeting.

Editor’s note – The wording of proactive key supply will be discussed at the next meeting.

# Signalling procedure for key supply upon request

(1) Key request at the source side

When cryptographic application needs keys for encryption, the cryptographic application sends a key request to the KM, and the KM supplies keys to the cryptographic application. If the KM doesn’t have enough number of keys in key storage, the KM initiates key generation or key relay to share the necessary number of keys, and supply them to the cryptographic applications when the key generation or key relay is completed.

Figure 1 shows signalling procedures for key request at the Ak interface at the source side.

Figure 1 – Signalling procedures for key request at the Ak interface at the source side

(2) Key request with ID at the destination side

The destination cryptographic application requests the key to the KM which has the connection with itself. The destination cryptographic application sends a request with the key identifier (ID) which is received from the source cryptographic application in order to specify the key.

Figure 2 shows signalling procedures for key request with ID at the Ak interface at the destination side.

Figure 2 – Signalling procedures for key request with ID at the Ak interface at the destination side

# Signalling procedure for proactive key supply

(1) Session creation at the source side

When cryptographic application needs keys for encryption, the cryptographic application first sends a session creation request to the KM at the source side. Then the source KM notifies the KM at the destination side to create a session and responses with a session ID to the source cryptographic application when the session is successfully created. Based on the created session, the source cryptographic application can request keys to the source KM.

Figure 3 shows signalling procedures for session creation at the Ak interface at the source side.

Figure 3 – Signalling procedures for session creation at the Ak interface at the source side

(2) Session creation at the destination side

The destination cryptographic application receives a session creation notification from the KM which has the connection with itself. The destination KM sends the notification with the session ID which is received from the source KM in order to specify the session.

Figure 4 shows signalling procedures for session creation at the Ak interface at the destination side.

Figure 4 – Signalling procedures for session creation at the Ak interface at the destination side

(3) Key request with session ID at the source side

With a created session, the KM at the source side supplies KSA-keys upon key request from the source cryptographic application.

Figure 5 shows signalling procedure for key request with session ID at the Ak interface at the source side.

Figure 5 – Signalling procedures for key request with session ID at the Ak interface at the source side

(4) Proactive key supply at the destination side

The KM at the destination side proactively supplies KSA-keys to the destination cryptographic application which has the connection with it. This scheme is applicable when the key request from the source cryptographic application is received by the source KM, which then instructs the destination KM to make a proactive key supply.

Figure 6 shows signalling procedures for proactive key supply at the Ak interface at the destination side.

Figure 6 – Signalling procedures for proactive key supply at the Ak interface at the destination side

# Signalling parameters

This clause specifies message format and parameters for the Ak interface.

In the M/O column of the tables in this clause, M indicates that the parameter is mandatory for signalling, and O indicates that the parameter is optional for signalling in tables in this clause.

NOTE: The parameters defined in this clause is independent of specific protocol stacks. The binding of the parameters to some protocol stacks are described in clause 9. In such bindings the signalling parameters are not necessarily mapped to a field in the message payload and might be a part of control parameters of the protocol.

Editor’s note: Further study will be needed whether the Data Type should be included in the tables in this clause as it might be rather related to the protocol binding.

# Key request

Key request message is sent from the cryptographic application to the KM at the source side for requesting of keys.

Table 2 shows message parameters of key request.

Table 2 – Message parameters of key request

| Parameter | Description | Data type | M/O | Remarks |
| --- | --- | --- | --- | --- |
| Application source ID | ID of the source cryptographic application (i.e. source application) | string | M |  |
| Application destination ID | ID of the cryptographic application with which the source cryptographic application (i.e. source application) requests to communicate | string | M |  |
| Application name | Name of the cryptographic application | string | O |  |
| Number of KSA-keys | Number of KSA-keys requested | integer | O | A default value is applied if omitted. |
| Size of KSA-key | Length of each KSA-key requested | integer | O | A default value is applied if omitted. |
| Extension | Array of extension parameters | Array of objects | O |  |

# Key request with ID

When receiving the KSA-key, the source cryptographic application sends the corresponding key ID to the destination cryptographic application. The destination cryptographic application sends a request to the destination KM with the key ID. Then the destination cryptographic application receives the key which has been shared between the source and destination KMs.

Table 3 shows message parameters of key request with ID

Table 3 – Message parameters of key request with ID

| Parameter | Description | Data type | M/O | Remarks |
| --- | --- | --- | --- | --- |
| Application source ID | ID of the source cryptographic application (i.e. source application) | string | M |  |
| Application destination ID | ID of the cryptographic application with which the source cryptographic application (i.e. source application) requests to communicate | string | M |  |
| Application name | Name of the cryptographic application | string | O |  |
| KSA-key ID | ID of the KSA-key requested | string | M | This ID is notified from the source application. |
| Extension | Array of extension parameters | Array of objects | O |  |

# Response to key request

Response to key request message is sent from the KM to the cryptographic application in response to the key request from the cryptographic application. The KM supplies the requested KSA-keys to the cryptographic application. There is no difference between the source side and the destination side for the response to key request.

Table 4 shows message parameters of response to key request.

Table 4 – Message parameters of response to key request

| Parameter |  | Description | Data type | M/O | Remarks |
| --- | --- | --- | --- | --- | --- |
| Key |  | Key file consists of key data and metadata. | Array of objects | M |  |
|  | KSA-key | KSA-key data provided for the request | string | M |  |
|  | KSA-key ID | ID of the KSA-key provided | string | M |  |
|  | Key extension | Extensions to key file | object | O | Hash value etc. |
| Response |  | Result of key supply | string | M | Success or failure reason |
| Extension |  | Array of extension parameters | Array of objects | O |  |

# Session creation request

Session creation request message is sent from the cryptographic application to the KM at the source side. A session will be created to facilitate the key supply between the cryptographic applications and the KMs at both sides.

Table 5 shows message parameters of session creation request.

Table 5 – Message parameters of session creation request

| Parameter | Description | Data type | M/O | Remarks |
| --- | --- | --- | --- | --- |
| Application source ID | ID of the source cryptographic application (i.e. source application) | string | M |  |
| Application destination ID | ID of the cryptographic application with which the source cryptographic application (i.e. source application) requests to communicate | string | M |  |
| Application name | Name of the source cryptographic application | string | O |  |
| Maximum number | Maximum number of KSA-keys requested during one session | integer | O |  |
| Extension | Array of extension parameters | Array of objects | O |  |

# Response to session creation request

Response to session creation request message is sent from the KM to the cryptographic application at the source side. After receiving a session creation request, the source KM notifies the KM at the destination side to create a session and responses with a session ID to the source cryptographic application when the session is successfully created.

Table 6 shows message parameters of response to session creation request.

Table 6 – Message parameters of response to session creation request

| Parameter | Description | Data type | M/O | Remarks |
| --- | --- | --- | --- | --- |
| Session ID | ID of the session created | string | M |  |
| Response | Result of the creation of the session | string | M | Success, failure reason, or status table of key supply. |
| Source KM ID | ID of the source KM | string | O |  |
| Extension | Array of extension parameters | Array of objects | O |  |

# Session creation notification

Session creation notification message is sent from the KM to the cryptographic application at the destination side. The destination KM proactively sends the session ID to the destination cryptographic application, and notifies it with the ID of the source cryptographic application that requests to communicate with it.

Table 7 shows message parameters of session creation notification.

Table 7 – Message parameters of session creation notification

| Parameter | Description | Data type | M/O | Remarks |
| --- | --- | --- | --- | --- |
| Application source ID | ID of the source cryptographic application (i.e. source application) | string | M |  |
| Application destination ID | ID of the cryptographic application with which the source cryptographic application (i.e. source application) requests to communicate | string | M |  |
| Application name | Name of the source cryptographic application | string | O |  |
| Session ID | ID of the session created | string | M |  |
| Maximum number | Maximum number of KSA-keys requested during one session | integer | O |  |
| Extension | Array of extension parameters | Array of objects | O |  |

# Response to session creation notification

Response to session creation notification message is sent from the cryptographic application to the KM in response to the session creation notification at the destination side. The destination cryptographic application notifies the result of the creation of the session to the destination KM.

Table 8 shows message parameters of response to session creation notification.

Table 8 – Message parameters of response to session creation notification

| Parameter | Description | Data type | M/O | Remarks |
| --- | --- | --- | --- | --- |
| Session ID | ID of the session created | string | M |  |
| Response | Result of the creation of the session | string | M | Success or failure reason |
| Extension | Array of extension parameters | Array of objects | O |  |

# Key request with session ID

With a created session, the cryptographic application sends a key request with session ID message to the KM at the source side. Then the source KM supplies the requested KSA-keys to the source cryptographic application during the session.

Table 9 shows message parameters of key request with session ID

Table 9 – Message parameters of key request with session ID

| Parameter | Description | Data type | M/O | Remarks |
| --- | --- | --- | --- | --- |
| Session ID | ID of the session created | string | M |  |
| Number of KSA-keys | Number of KSA-keys requested | integer | O | A default value is applied if omitted. |
| Size of KSA-key | Length of each KSA-key requested | integer | O | A default value is applied if omitted. |
| Extension | Array of extension parameters | Array of objects | O |  |

# Response to key request with session ID

Response to key request with session ID message is sent from the KM to the cryptographic application at the source side. The source KM supplies the requested KSA-keys to the source cryptographic application during the created session.

Table 10 shows message parameters of response to key request with session ID.

Table 10 – Message parameters of response to key request with session ID

| Parameter |  | Description | Data type | M/O | Remarks |
| --- | --- | --- | --- | --- | --- |
| Session ID |  | ID of the session created | string | M |  |
| Key |  | Key file consists of key data and metadata. | Array of objects | M |  |
|  | KSA-key | KSA-key data provided for the request | string | M |  |
|  | KSA-key ID | ID of the KSA-key provided | string | M |  |
|  | Key extension | Extensions to key file | object | O | Hash value etc. |
| Response |  | Result of key supply | string | M | Success or failure reason |
| Extension |  | Array of extension parameters | Array of objects | O |  |

# Proactive key supply

Proactive key supply message is sent from the KM to the cryptographic application at the destination side. The destination KM proactively supplies the KSA-key to the destination cryptographic application during the created session.

Table 11 shows message parameters of proactive key supply.

Table 11 – Message parameters of proactive key supply

| Parameter |  | Description | Data type | M/O | Remarks |
| --- | --- | --- | --- | --- | --- |
| Session ID |  | ID of the session created | string | M |  |
| Key |  | Key file consists of key data and metadata. | Array of objects | M |  |
|  | KSA-key | KSA-key data supplied | string | M |  |
|  | KSA-key ID | ID of the KSA-key supplied | string | M |  |
|  | Key extension | Extensions to key file | object | O | Hash value etc. |
| Extension |  | Array of extension parameters | Array of objects | O |  |

# Response to proactive key supply

Response to proactive key supply message is sent from the cryptographic application to the KM in response to the proactive key supply at the destination side. The destination cryptographic application notifies the result of the receipt of the KSA-key to the destination KM.

Table 12 shows message parameters of response to proactive key supply.

Table 12 – Message parameters of response to proactive key supply

| Parameter | Description | Data type | M/O | Remarks |
| --- | --- | --- | --- | --- |
| Session ID | ID of the session created | string | M |  |
| KSA-key ID | ID of the KSA-key received | string | M |  |
| Response | Result of the receipt of the KSA-key | string | M | Success or failure reason |
| Extension | Array of extension parameters | Array of objects | O |  |

# Protocol stack bindings

# TCP binding

The cryptographic application shall connect to the KM using TCP protocols [b-IETF RFC 793]. The corresponding message format for the Ak interface is as follows.

Figure 7 – Message format for the Ak interface using TCP protocols

Version: the current version of the message format adopted, 2 bytes;

MessageID: the unique identifier of each message, 4 bytes;

CommandCode: a unique code that denotes different command/response messages transferred at the Ak interface, 2 bytes;

Length: the length of the message payload, 2 bytes;

Payload: the message parameters of a specific command/response message, JSON data format [b-IETF RFC 8259].

At the connection establishment, mutual authentication between the cryptographic application and the KM shall be performed. After the mutual authentication, a command/response message can be transferred at the Ak interface for key request and key supply.

Table 1 shows a list of CommandCode vs. command/response message name.

Table 1 – CommandCode vs. command/response message name

| CommandCode | Command/Response message name |
| --- | --- |
| 0x01 | Key request |
| 0x02 | Key request with ID |
| 0x03 | Response to key request |
| 0x04 | Session creation request |
| 0x05 | Response to session creation request |
| 0x06 | Session creation notification |
| 0x07 | Response to session creation notification |
| 0x08 | Key request with session ID |
| 0x09 | Response to key request with session ID |
| 0x0A | Proactive key supply |
| 0x0B | Response to proactive key supply |

# HTTPS binding

Editor’s note: HTTPS binding described in this clause is intended to be equivalent to the protocol defined in [b-ETSI GS QKD 014].

Key request

| Parameter | Mapped to | Data type |
| --- | --- | --- |
| Application source ID | Implicit (Source of HTTPS connection) |  |
| Application destination ID | Part of the URL to which the source Application connects (‘target_SAE_ID’ part) | string |
| Number of KSA-keys | HTTPS body (‘number’ in key request container) | JSON (‘number’: value) |
| Size of KSA-key | HTTPS body (‘size’ in key request container) | JSON (‘size’: size) |
| Extension | HTTPS body (‘extension_mandatory’ or ‘extension_optional’ in key request container) | JSON array |

Key request with ID

| Parameter | Description | Data type |
| --- | --- | --- |
| Application source ID | Part of the URL to which the source Application connects (‘initiator SAE_ID’ part) | string |
| Application destination ID | Implicit (Source of HTTPS connection) |  |
| KSA-key ID | HTTPS body | JSON array ( of ‘key_ID’:key_ID ) |
| Extension | HTTPS body | JSON array |

Response to  key request

| Parameter |  | Description | Data type |
| --- | --- | --- | --- |
| Key |  | HTTPS body | Array of objects |
|  | KSA-key | HTTPS body | JSON (‘key’: base64string) |
|  | KSA-key ID | HTTPS body | JSON (‘key_ID’: uuid) |
|  | Key extension | HTTPS body | object |
| Response |  | HTTPS response header | String(integer) |
| Extension |  | HTTPS body | object |

# Security considerations

Key data and associated metadata are transferred through Ak reference point. Security requirements and measures to protect them are specified in [ITU-T X.1712].

Annex A

<Annex Title>

(This annex forms an integral part of this Recommendation.)

<Body of annex A>

Appendix I

<Appendix Title>

(This appendix does not form an integral part of this Recommendation.)

Bibliography

[b-ETSI GR QKD 007] 	Group Report ETSI GR QKD 007 (2018), Quantum Key Distribution (QKD); Vocabulary.

[b-ETSI GS QKD 014] 	Group Report ETSI GR QKD 007 (2018), Quantum Key Distribution (QKD); Protocol and data format of REST-based key delivery API

[b-IETF RFC 793] 	IETF RFC 793, TRANSMISSION CONTROL PROTOCOL.

[b-IETF RFC 8259] 	IETF RFC 8259, The JavaScript Object Notation (JSON) Data Interchange Format.

___________________