|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  | SG11-TD186/GEN |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 11 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | 2/11 |  | Geneva, 6-15 July 2022 |  |
| TD |  |  |  |  |  |
| Source: |  | Editors |  |  |  |
| Title: |  | Output – draft new Recommendation Q.QKDN_Kq-1 “Protocols for Kq-1 interface for QKDN” (Geneva, 6-15 July 2022) |  |  |  |
| Contact: |  | Hongyu Wu QuantumCTek Co., Ltd. China |  |  | Email: hongyu.wu@quantum-info.com |
| Contact: |  | Zhangchao Ma CAS Quantum Network Co., Ltd. China |  |  | Email: mazhangchao@qtict.com |
| Contact: |  | Junsen Lai CAICT, Ministry of Industry and Information Technology (MIIT) China |  |  | E-mail: laijunsen@caict.ac.cn |
| Contact: |  | Kaoru KENYOSHI NICT Japan |  |  | Tel: +81 50 3566 5852 E-mail: kaoru.kenyoshi@nict.go.jp |
| Contact: |  | Mariko Honda NICT Japan |  |  | E-mail: mariko.honda@ntt-at.co.jp |

| Abstract: | This is the output of draft Recommendation Q.QKDN_Kq-1 “Protocols for Kq-1 interface for QKDN”, based on the result of the SG11 meeting in Geneva, 6-15 July 2022. |
| --- | --- |

Summary

This is the output draft of Q.QKDN_Kq-1 “Protocols for Kq-1 interface for QKDN” based on the result of  the SG11 meeting Geneva, 6-15 July 2022. The text proposed in C80 and C81 is agreed as it is in the meeting.

Attachment:

Annex A: Draft Recommendation ITU T Y.QKDN_Kq-1 “Protocols for Kq-1 interface for QKDN”

Annex I

Draft Recommendation ITU-T Q.QKDN_Kq-1

Protocols for Kq-1 interface for QKDN

Summary

Recommendation ITU-T Q.QKDN_Kq-1 specifies protocols for Kq-1 interface in quantum key distribution networks (QKDN).

Keywords

Protocol, QKD (quantum key distribution), QKDN (QKD network), signalling procedure, message parameters
Table of Contents

1.	Scope	4

2.	References	4

3.	Definitions	4

3.1.	Terms defined elsewhere	4

3.2.	Terms defined in this Recommendation	6

4.	Abbreviations and acronyms	6

5.	Conventions	6

6.	Kq-1 interface	6

7.	Signalling procedure	6

8.	Signalling parameters	7

8.1.	Key supply	7

8.2.	Response to key supply	8

9.	Protocol stack bindings	8

9.1.	TCP binding	8

9.2.	HTTPS binding	9

10.	Security considerations	10

# Scope

This Recommendation specifies protocols at Kq-1 interface for quantum key distribution network (QKDN) especially the following areas.

- signalling procedures for Kq-1 interface for QKDN;

- message format and parameters for Kq-1 interface for QKDN;

- security considerations.

# References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T X.1710] 	Recommendation ITU-T X.1710 (2020), Security framework for quantum key distribution networks

[ITU-T X.1712] 	Recommendation ITU-T X.1712 (2021)/Cor.1 (02/2022), Security requirements and measures for quantum key distribution networks - key management.

[ITU-T Y.3800] 	Recommendation ITU-T Y.3800 (2019)/Cor.1 (04/2020), Overview on networks supporting quantum key distribution.

[ITU-T Y.3801] 	Recommendation ITU-T Y.3801 (2020), Functional requirements for quantum key distribution networks.

[ITU-T Y.3802] 	Recommendation ITU-T Y.3802 (2020)/Cor.1 (04/2021), Quantum key distribution networks - Functional architecture.

[ITU-T Y.3803] 	Recommendation ITU-T Y.3803 (2020), Quantum key distribution networks - Key management.

[ITU-T Q.QKDN_profr] draft Recommendation Q.QKDN_profr, Quantum key distribution networks - Protocol framework.

# Definitions

# Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

    - information theoretically secure (IT-secure) [ITU-T Y.3800]: Secure against any deciphering attack with unbounded computational resources.

    - key life cycle [ITU-T Y.3800]: A sequence of steps that a key undergoes from its reception by a key manager (KM) through its use in a cryptographic application and until deletion or preservation depending on the key management policy.

    - key management [ITU-T Y.3800]: All activities performed on keys during their life cycle starting from their reception from the quantum layer, the storage, formatting, relay, synchronization, authentication, to supply to cryptographic application and delete or preserve depending on the key management policy.

    - key management agent (KMA) [ITU-T Y.3802]: A functional element to manage keys generated by a quantum key distribution (QKD) module/QKD modules in a QKD node (trusted node).

NOTE - KMA acquires keys from a QKD module/QKD modules, synchronizes, resize, formats, and stores them. It also relays keys through key management agent (KMA) links.

    - key management agent-key (KMA-key) [ITU-T Y.3803]: Key data stored and processed in a key management agent (KMA), and securely shared between a KMA and a matching KMA.

    - key management agent link (KMA link) [ITU-T Y.3802]: A communication link connecting KMAs to perform key relay and communications for key management.

    - key manager (KM) [ITU-T Y.3800]: A functional module located in a quantum key distribution (QKD) node to perform key management in the key management layer.

    - key manager link (KM link) [ITU-T Y.3800]: A communication link connecting key managers (KMs) to perform key management.

    - key relay [ITU-T Y.3800]: A method to share keys between arbitrary quantum key distribution (QKD) nodes via intermediate QKD node(s).

    - key supply agent (KSA) [ITU-T Y.3802]: A functional element to supply keys to a cryptographic application, being located between a key management agent (KMA) and the client.

NOTE - Application interfaces for cryptographic applications are installed into the key supply agent (KSA). The KSA synchronizes keys, and verifies their integrity via a KSA link before supplying them to the client.

    - key supply agent-key (KSA-key) [ITU-T Y.3803]: Key data stored and processed in a key supply agent (KSA), and securely shared between a KSA and a matching KSA.

    - key supply agent link (KSA link) [ITU-T Y.3802]: A communication link connecting KSAs to perform key synchronization and integrity verification.

    - quantum key distribution (QKD) [b-ETSI GR QKD 007]: Procedure or method for generating and distributing symmetrical cryptographic keys with information theoretical security based on quantum information theory.

    - quantum key distribution link (QKD link) [ITU-T Y.3800]: A communication link between two quantum key distribution (QKD) modules to operate the QKD.

NOTE – A QKD link consists of a quantum channel for the transmission of quantum signals, and a classical channel used to exchange information for synchronization and key distillation.

    - quantum key distribution module (QKD module) [ITU-T Y.3800]: A set of hardware and software components that implements cryptographic functions and quantum optical processes, including quantum key distribution (QKD) protocols, synchronization, distillation for key generation, and is contained within a defined cryptographic boundary.

NOTE – A QKD module is connected to a QKD link, acting as an endpoint module in which a key is generated. These are two types of QKD modules, namely, the transmitters (QKD-Tx) and the receivers (QKD-Rx).

    - quantum key distribution network (QKDN) [ITU-T Y.3800]: A network comprised of two or more quantum key distribution (QKD) nodes connected through QKD links.

NOTE – A QKDN allows sharing keys between the QKD nodes by key relay when they are not directly connected by a QKD link.

    - quantum key distribution network controller (QKDN controller) [ITU-T Y.3800]: A functional module, which is located in a quantum key distribution (QKD) network control layer to control a QKD network.

    - quantum key distribution network manager (QKDN manager) [ITU-T Y.3800]: A functional module, which is located in a quantum key distribution (QKD) network management layer to monitor and manage a QKD network.

    - quantum key distribution node (QKD node) [ITU-T Y.3800]: A node that contains one or more quantum key distribution (QKD) modules protected against intrusion and attacks by unauthorized parties.

NOTE – A QKD node can contain a key manager (KM).

This Recommendation uses the following terms defined elsewhere:

# Terms defined in this Recommendation

None

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

HTTPS	HyperText Transfer Protocol Secure

KM	Key manager

KSA	Key Supply Agent

QKD	Quantum Key Distribution

QKDN		QKD Network

TCP		Transmission Control Protocol

# Conventions

None.

# Kq-1 interface

The Kq-1 interface is defined between the key manager (KM) and the QKD module. The Kq-1 interface is used for key acquisition between the key storage function in the KM and the QKD-key supply function in the QKD module.

# Signalling procedure

At Kq-1 interface, signalling procedures for key supply are the same for each QKD node. The QKD module supplies QKD-keys to the KM which has the connection with it. This process is initiated when the QKD-keys are generated in the QKD module. The amount of the QKD-keys supplied mainly depends on the key generation request from the QKDN controller.

Figure 1 shows signalling procedures for key supply at the Kq-1 interface.

Figure 1 – Signalling procedures for key supply at the Kq-1 interface

# Signalling parameters

This clause specifies signalling parameters for the Kq-1 interface.

In the M/O column of the tables in this clause, M indicates that the parameter is mandatory for signalling, and O indicates that the parameter is optional to signalling.

NOTE: The parameters defined in this clause is independent of specific protocol stacks. The binding of the parameters to some protocol stacks are described in clause 9. In such bindings the signalling parameters are not necessarily mapped to a field in the message payload and might be a part of control parameters of the protocol.

Editor’s note: Further study will be needed whether the Data Type column should be included in the tables in this clause as it might be rather related to the protocol binding.

# Key supply

Key supply message is sent from the QKD module to the KM at the same QKD node. The QKD module supplies the QKD-key with a unique QKD-key ID to the KM.

Table 2 shows message parameters of key supply.

Table 2 – Message parameters of key supply

| Parameter | Description | Data type | M/O | Remarks |
| --- | --- | --- | --- | --- |
| QKD-key | QKD-key data supplied | string | M |  |
| QKD-key ID | ID of the QKD-key supplied | string | M |  |
| QKD module ID | ID of the QKD module (Alice or Bob) that supplies the QKD-key | string | M |  |
| Matching QKD module ID | ID to identify the matching QKD module that constitutes the pair of Alice and Bob | string | O |  |
| Key length | Length of each QKD-key supplied | integer | O | A default value is applied if omitted. |
| Generation time stamp | Time stamp of QKD-key generation at the pair of QKD modules | string | O |  |
| Hash value | Hash value of the QKD-key data. | string | O |  |
| Extension | Array of extension parameters | Array of objects | O | For future use |

# Response to key supply

Response to key supply message is sent from the KM to the QKD module in response to the key supply. The KM notifies the results of the receipt of the QKD-keys to the QKD module.

Table 3 shows message parameters of response to key supply.

Table 3 – Message parameters of response to key supply

| Parameter | Description | Data type | M/O | Remarks |
| --- | --- | --- | --- | --- |
| QKD-key ID | ID of the QKD-key received. | string | M |  |
| QKD module ID | ID of the QKD module (Alice or Bob) that supplies the QKD-key | string | O |  |
| Matching QKD module ID | ID to identify the matching QKD module that constitutes the pair of Alice and Bob | string | O |  |
| Response | Result of the receipt of the QKD-key | string | M | Success or failure reason |
| Extension | Array of extension parameters | Array of objects | O | For future use |

# Protocol stack bindings

# TCP binding

With the TCP binding, the QKD module connects to the KM using TCP protocols [b-IETF RFC 793]. The corresponding message format for the Kq-1 interface is as follows.

Figure 2 – Message format for the Kq-1 interface using TCP protocols

Version: the current version of the message format adopted, 2 bytes;

MessageID: the unique identifier of each message, 4 bytes;

CommandCode: a unique code that denotes different command/response messages transferred at the Kq-1 interface, 2 bytes;

Length: the length of the message payload, 2 bytes;

Payload: the message parameters of a specific command/response message, JSON data format [b-IETF RFC 8259].

At the connection establishment, mutual authentication between the QKD module and the KM shall be performed. After the mutual authentication, a command/response message can be transferred at the Kq-1 interface for key supply from the QKD module to the KM.

Table 1 shows a list of CommandCode vs. command/response message name.

Table 1 – CommandCode vs. command/response message name

| CommandCode | Command/Response message name |
| --- | --- |
| 0x01 | Key supply |
| 0x02 | Response to key supply |

# HTTPS binding

(Texts to be added)

Editor’s note: With HTTPS binding, a pair of Key supply and Response to key supply can be mapped to a single HTTPS transaction. The tables below describe an example mapping of signalling parameters to HTTPS transaction.

Key supply

| Parameter | Mapped to | Data type |
| --- | --- | --- |
| QKD-key | HTTPS request body | JSON key:value pair |
| QKD-key ID | HTTPS request body | JSON key:value pair |
| QKD module ID | Implicit (source address of HTTPS connection) |  |
| Matching QKD module ID | HTTPS request body | JSON key:value pair |
| Key length | HTTPS request body | JSON key:value pair |
| Generation time stamp | HTTPS request body | JSON key:value pair |
| Hash value | HTTPS request body | JSON key:value pair |
| Extension | HTTPS request body | JSON Array |

Response to Key supply

| Parameter | Mapped to | Data type |
| --- | --- | --- |
| QKD-key ID | HTTPS response body | JSON key:value pair |
| QKD module ID | Implicit (source address of HTTPS connection) |  |
| Matching QKD module ID | HTTPS response body | JSON key:value pair |
| Response | Status code in HTTP response header | String(integer) |
| Extension | HTTP response body | JSON object |

# Security considerations

Key data and associated metadata are transferred through Kq-1 reference point. Security requirements and measures to protect them are specified in [ITU-T X.1712].

Annex A

<Annex Title>

(This annex forms an integral part of this Recommendation.)

<Body of annex A>

Appendix I

<Appendix Title>

(This appendix does not form an integral part of this Recommendation.)

<Body of appendix I>

Bibliography

[b-ETSI GR QKD 007] 	Group Report ETSI GR QKD 007 (2018), Quantum Key Distribution (QKD); Vocabulary.

[b-IETF RFC 793] 	IETF RFC 793, TRANSMISSION CONTROL PROTOCOL.

[b-IETF RFC 8259] 	IETF RFC 8259, The JavaScript Object Notation (JSON) Data Interchange Format.

___________________