|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  |  | SG16-TD149/WP1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 16 |  |
|  |  |  |  | Original: English |  |
| Question(s): |  | 13/16 | Geneva, 10-21 July 2023 |  |  |
| TD |  |  |  |  |  |
| Source: |  | Editor F.IPTV-VRSReqs |  |  |  |
| Title: |  | F.IPTV-VRSReqs "Requirements of enabling VR service based on IPTV architecture" (New): Output draft (Geneva, 10-21 July 2023) |  |  |  |
| Contact: |  | Xiufang Zhong China Unicom P.R.China |  |  | Tel: +86-10-68799999-7266 E-mail: zhongxf1@chinaunicom.cn |
| Contact: |  | Xin Shen China Mobile P.R.China |  |  | Tel: +86 21 51856688-81147 E-mail: shenxin_sx@migu.cn |
| Contact: |  | Chuanyang Miao ZTE P.R.China |  |  | Tel: +86-25-88014611 E-mail: miao.chuanyang@zte.com.cn |
| Contact: |  | Jiaqi Xu China Unicom P.R.China |  |  | Tel: +86-10-68799999-7426 E-mail: xujq25@chinaunicom.cn |

| Abstract: | This TD contains the updated text for the draft Recommendation F.IPTV-VRSReqs "Requirements of enabling VR service based on IPTV architecture" according to the result of the discussion at the SG16 Q13 meeting (Geneva, 10-21 July 2023). |
| --- | --- |

This updated draft Recommendation uses the baseline of Q13/16-DOC11-R1 (230424) from Q13/16 RGM meeting, E-meeting, 24-26 April 2023, and is based on the discussions of the following contributions at this SG16 meeting (Geneva, 10-21 July 2023).

| Document | Source | Title | Agreements |
| --- | --- | --- | --- |
| SG16-C337-R1 | China Mobile (China) | F.IPTV-VRSReqs: Proposal on clause 6.4 - Enabling VR Service base on IPTV functional architecture" | There was suggestion that content process functional block within CDF might be considered to support VR stream rendering while the cloud rendering or edge rendering is performed. It is suggested that the requirement study should include the consideration on implementation of IPTV VR service relied on open IPTV architecture (LSFA). Proposal was accepted |
| SG16-C338 | China Mobile (China) | F.IPTV-VRSReqs: Proposal on requirements | Proposal was accepted |

CONTENTS

Page

1. Scope 5 2. References 5 3. Definitions 5 3.1 Terms defined elsewhere 5 3.2 Terms Defined within this document 6 4. Abbreviations 6 5. Convention 6 6. Overview 7 6.1 VR Service 7 6.2 Relationship between VR service with IPTV system 7 6.3 VR enabled IPTV service 8 6.4 Enabling VR Service base on IPTV functional architecture 9 7. Requirements 11 7.1 General aspects 11 7.1.1 Service offerings 11 7.1.2 Accounting and charging 12 7.1.3 Service consumption 12 7.1.4 Miscellaneous 12 7.2 QoS and performance aspects 12 7.3 Security aspects including service and content protection 13 7.4 Network related aspects 13 7.5 End systems and interoperability aspects 13 7.6 Middleware, application and content aspects 13 7.6.1 Middleware 13 7.6.2 Metadata 14 8 Functions of VR service based on IPTV architecture 14 8.1 Video transcoding function 14 8.2 Rendering function 14 8.3 Multi-screen function 14 8.4 VR content delivery function 14 8.5 Other enhanced functions 14 9 Security considerations 14 I.1 Use case 1: VR video services 15 I.2 Use case 2: VR interactive services 16

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 6-1 – IPTV domains supporting VR service 8 Figure 6-2 – The relationship between VR service and current IPTV related Recommendations 9 Figure 6-3 – Functional architecture for VR service based on IPTV architecture 10 Figure I.1-1 – Single-user use case 15 Figure I.1-2 – Multi-device use case 16 Figure I.2-1 – Multiple users and terminals use case 17

Draft new Recommendation F.IPTV-VRSReqs

Requirements of enabling VR service based on IPTV architecture

Summary

[ADD TEXT]

Keywords

[ADD TEXT]

# Scope

Virtual reality (VR) is a virtual environment rendered with the delivered visual and audio scene, which gives simulated experience to the user as an immersive feel of a virtual world. This draft Recommendation identifies the requirements and functions of enabling VR service base on IPTV architecture. This draft Recommendation discussed the VR services based on IPTV terminal devices. This draft Recommendation also describes the related use cases of VR service based on IPTV architecture.

Editor Note: we need more explanation of “functions” in the future contributions.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.

[TBD]

# Definitions

## 3.1	Terms defined elsewhere

This document uses the following terms defined elsewhere

3.1.1	content provider [ITU-T Y.1910]: The entity that owns or is licensed to sell content or content assets.

3.1.2	delivery [ITU-T Y.1910]: In the context of IPTV architecture, "delivery" is defined as sending contents to the end user.

3.1.3	end user [ITU-T Y.1910]: end user: The actual user of the products or services.

NOTE – The end-user consumes the product or service. An end-user can optionally be a subscriber (see definition of "subscriber").

3.1.4	home network [b-ITU-T H.622]: Home network is the collection of elements that process, manage, transport, and store information, thus enabling the connection and integration of multiple computing, control, monitoring, communication and entertainment devices in the home.

3.1.5	linear television (linear TV) [ITU-T Y.1910]: A television service in which a continuous stream flows in real time from the service provider to the terminal device and where the user cannot control the temporal order in which contents are viewed.

3.1.6	Internet Protocol Television (IPTV) [ITU-T Y.1901]: Multimedia services such as television/video/audio/text/graphics/data delivered over IP-based networks managed to support the required level of QoS/QoE, security, interactivity and reliability.

3.1.7	IPTV terminal function (ITF) [ITU-T Y.1901]: The end-user function(s) associated with a) receiving and responding to network control channel messages regarding session set-up, maintenance, and tear-down, and b) receiving the content of an IP transport from the network and rendering.

3.1.8	IPTV terminal device [ITU-T Y.1901]: A terminal device which has ITF functionality, e.g., an STB.

3.1.9	network provider [ITU-T Y.1910]: The organization that maintains and operates the network components required for IPTV functionality.

NOTE 1 – A network provider can optionally also act as service provider.

NOTE 2 – Although considered as two separate entities, the service provider and the network provider can optionally be one organizational entity.

3.1.10	service provider [ITU-T M.1400]: A general reference to an operator that provides telecommunication services to customers and other users either on a tariff or contract basis. A service provider can optionally operate a network. A service provider can optionally be a customer of another service provider.

3.1.11	terminal device (TD) [ITU-T Y.1901]: An end-user device which typically presents and/or processes the content, such as a personal computer, a computer peripheral, a mobile device, a TV set, a monitor, a VoIP terminal or an audiovisual media player.

3.1.12	video on demand (VoD) [ITU-T Y.1910]: A service in which the end user can, on demand, select and view video content and where the end user can control the temporal order in which the video content is viewed (e.g., the ability to start the viewing, pause, fast forward, rewind, etc.).

NOTE – The viewing may occur some time after the selection of the video content.

## 3.2	Terms Defined within this document

This document defines the following terms

3.2.1	VR services: A rendered version of either reality-recorded or reality-simulated audio-visual scenes, offering a 360° immersive experience and real-time interaction with end users through IPTV system.

NOTE – The 360° immersive experience is considered as Basic VR Service, while the addition of real-time interaction is referred to as Enhanced VR Service.

[Editor’s Note]: Detailed descriptions and definitions will be discussed in the main text. More contributions are expected.

3.2.2	VR terminal device: A device or software entity supports VR services through IPTV system.

# Abbreviations

| [TBD] |  |
| --- | --- |

# Convention

In this Recommendation:

- The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted, if conformance to this Recommendation is to be claimed.

- The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required. Thus, this requirement need not be present to claim conformance.

- The keywords "can optionally" indicate an optional requirement which is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor's implementation must provide the option, and the feature can be optionally enabled by the network operator/service provider. Rather, it means the vendor may optionally provide the feature and still claim conformance with this Recommendation.

Requirements are identified using the following conventions:

- Requirement number xx in subclause n.m is of the form R n.m-xx;

- Recommended requirement number yy in subclause n.m is of the form RR n.m-yy;

- Optional requirement number zz in subclause n.m is of the form OR n.m-zz.

NOTE – Future issues of this Recommendation should never reuse previously assigned numbers.

# Overview

VR service, as a new form of multimedia service, is fundamentally a user-oriented IP-based multimedia system which involving end-users, networks providers, service providers, and content providers. The IPTV system, which has been proved to be a mature commercialized end-to-end system, can be used to quickly support VR service and enable its rapid deployment.

This Recommendation will discuss the implementation of VR services using the IPTV system, including user cases, service requirements, and functional architecture, based on the relevant ITU-T IPTV Recommendations.

## 6.1	VR Service

VR services provide  realistic images, sounds and other sensations that simulate users’ physical presence in virtual environment.

Recommendation ITU-T Y Suppl. 5 presents IPTV interactive services such as educational applications, entertainment applications (such as games), communications services and information services. The VR usage scenarios based on IPTV architecture would also cover above interactive services.

Considering the IPTV services mainly offer the video and audio services, both the video and audio services requirements and functions are discussed in this recommendation.

From the user experience, the VR services could be divided by the freedom or opportunity with which users can interact within the virtual environment.

At least, VR users should be able to look around from a single observation point in 3D virtual environment, by either a producer or the position of a capturing devices. 360-degree video content captured in this way and combined with captured audio is available to enable the VR service.

Furthermore, the virtual 3D space which the users can move within it under their own control with almost full freedom is appeared. With the virtual reality equipment, the user would be able to look around the artificial world, move around in it, and interact with virtual features or items.

## 6.2	Relationship between VR service with IPTV system

According to ITU-T Recommendation [ITU-T Y.1910], IPTV is defined as "multimedia services such as television/video/audio/text/graphics/data delivered over IP based networks managed to provide the required level of quality of service and experience, security, interactivity and reliability".

In Recommendation [ITU-T H.700], an IPTV terminal device as "a device or software entity that receives and presents media content, associated metadata, and interactive elements for the end user in an IPTV system." It includes set-top boxes, mobile devices, personal computers, and other devices that can be used to access IPTV services.

As a multimedia service providing various forms of content such as video, audio, images, and digital scenes, VR service can be carried by the mature IPTV system, which provides high-quality multimedia services via IP networks. The IPTV system is a suitable carrier that can support various service modes and achieve end-to-end carrying, from content creation to user reception. Additionally, the IPTV system's wide coverage area offers convenience for the promotion and popularization of VR business.

According to the description of IPTV Domains in [ITU-T Y.1901], Figure 6-1 explains the relationship between VR service and IPTV service.It is provided in this Recommendation to help the understanding the relationship which in some way relate to these domains. These relationship in IPTV domains do not define a business model. Also, one provider may play in multiple domains in the provision of an actual service.

Figure 6-1 – IPTV domains supporting VR service

The four IPTV domains supporting VR service, the definitions of which are provided in clause 3, are:

•	content provider: With VR content provided.

•	service provider: With VR service provided.

•	network provider: With VR service enabled.

•	end user: With VR terminal device used.

The functional elements constituting these above domains are described in more detail in clause 8.

## 6.3	VR enabled IPTV service

In Recommendation [ITU-T Y.Sup5] "Supplement on IPTV service use cases", various types of services supported by IPTV system have been listed, including distributed content, interactive services, communication services, hosting services, presence services, etc.

By offering VR service in IPTV system, IPTV service providers may provide VR-enhanced versions of immersive TV programming, such as watching a live VR sports event delivered by IPTV system, which would provide a more immersive and interactive experience than watching it on a traditional screen. Interactive services are also a good example of the combination of IPTV and VR services. Through the functions provided by the IPTV system, users can experience a better immersive interactive experience.

By enabling VR services, the IPTV system can enhance the various services it offers with more immersive experiences and real-time interactions than those currently supported. The user cases listed in Recommendation [ITU-T Y.Sup5] should be updated in this Recommendation to account for VR-supported scenarios. Additionally, this Recommendation will also discuss IPTV services requirements for the support of VR services based on Recommendation [ITU-T Y.1901].

The relationship between VR service and Recommendation [ITU-T Y.1901] and Recommendation [ITU-T Y.1910] is shown in Figure 6-2.

Figure 6-2 – The relationship between VR service and current IPTV related Recommendations

## 6.4	Enabling VR Service base on IPTV functional architecture

[Editor’s Note]: During Q13/16 meeting (Geneva, 10-21 July 2023), it is suggested that the requirement study should include the consideration on implementation of IPTV VR service relied on open IPTV architecture (LSFA). More contributions are expected.

As a video streaming service similar to Linear TV and VoD, VR service involves the addition of VR service functional blocks based on the IPTV functional architecture defined in [ITU-T Rec Y.1910]. Support requirements for the existing functional blocks are also required. As shown in Figure 6-3 , the dark green blocks represent the newly added VR service modules, while the yellow blocks indicate the blocks that need to be enhanced for VR service.

Figure 6-3 – Functional architecture for VR service based on IPTV architecture

[Editor’s Note]: VR service could be both rendered locally by IPTV TD or remotely by cloud computing platforms. The relationship between IPTV system and cloud computing platform should be discussed in this Draft Recommendation in the future. More contributions are expected.

IPTV terminal functions

- The VR service client functional block interacts with the VR service application functional block to perform session management, service authorization, presentation of the content metadata and execution of the service logic for the VR service applications.

- The SADS client functional block provides for the end user's discovery and selection of VR applications.

IPTV application functions

- The VR service application functional block performs session management, service authorization, presentation of the content metadata and execution of the service logic for VR service  applications.

- The application profile functional block for VR service includes VR service settings.

Content preparation functions

- The content preparation functions are comprised of content management, metadata processing, content processing control and content pre-processing functional blocks. In VR service, these functional blocks are used to control the preparation and/or combination of VR content, as delivered by the VR content owner(s), into the required delivery format.

Service control functions

- The IPTV service control functional block provides the functions to handle service initiation, modification and termination requests, perform service access control, establish and maintain the network and system resources required to support the IPTV services requested by the IPTV terminal functions necessary for launching VR service application.

Content delivery client functions

- The content delivery client functions are responsible for VR content reception in the IPTV terminal functions.

Content delivery functions

- The content delivery functions (CDF) perform cache and storage functionalities and deliver VR content according to the request from the end-user functions with low latency.

# Requirements

## 7.1	General aspects

General requirements

- R 7.1-01: It is required to support convenient service switching between VR, linear TV, VoD and other services for the end-user.

- R 7.1-02: It is required to support service navigation in VR service.

- R 7.1-03: It is required to support convenient service control such as view angle adjustment, virtual environment exploration.

•	R 7.1-04: It is required to support playback of 360-degree VR audio-visual content.

General recommendations

- RR 7.1-01: It is recommended to support real-time interaction capability to enable the engagement with virtual objects in real-time.

- RR 7.1-02: It is recommended to support connecting other devices to the IPTV TD for enhanced VR service experience.

- RR 7.1-03: It is recommended to collect TD information to provide customized VR service based on different TD capabilities.

General options

- OR 7.1-01: It can optionally support adaptive playback functionality based on user preferences and usage conditions.

### 7.1.1	Service offerings

For general service offering of IPTV service, please see Clause 6.1.1 in [ITU-T Y.1901].

Extended requirements and recommendations on VR service over IPTV system are as the following:

Requirements

- R 7.1.1-01: It is required to support the VR content provision and processing.

- R 7.1.1-02: It is required to support stable and high-speed transmission of VR audio-visual content to ensure immersive experience.

- R 7.1.1-03: It is required to support rendering of VR contents to ensure high-quality representation of VR content.

- R 7.1.1-04: It is required to support navigation interface for convenient service exploration and interaction.

Recommendations

- RR 7.1.1-01: It is recommended to support the provision of real-time interaction for realistic experience.

- RR 7.1.1-02: It is recommended to support service settings, such as audio and video quality adjustment, control sensitivity, etc.

Options

- OR 7.1-01: It can optionally support gesture control or/and voice control to enhance user interactivity and provide alternative means of service navigation.

### 7.1.2	Accounting and charging

For Accounting and charging service requirements and recommendations of VR service, please see Clause 6.1.2 in [ITU-T Y.1901].

There is no extended accounting and charging requirements on VR service over IPTV system.

### 7.1.3	Service consumption

For general service consumption requirements and recommendations of IPTV service, please see Clause 6.1.3 in  [ITU-T Y.1901].

Extended requirements and recommendations on VR service over  IPTV system are as the following:

Requirements

- R 7.1.3-01: It is required to support t seamless navigation and switching between VR content and traditional 2D content ensuring a smooth transition for users.

- R 7.1.3-02: It is required to support smooth and high-quality playback of VR audio and video content delivering an immersive experience.

Recommendations

- RR 7.1.3-01: It is recommended to support social features that allow users to interact and communicate with others within VR services to enhance users’ engagement.

### 7.1.4	Miscellaneous

For general miscellaneous requirements and recommendations of IPTV service, please see clause 6.1.4 in [ITU-T Y.1901].

There is no extended miscellaneous requirements on VR service over IPTV system.

## 7.2	QoS and performance aspects

For general QoS and performance requirements and recommendations of IPTV service, please see clause 7.2 in  [ITU-T Y.1901].

Extended requirements and recommendations on VR service over IPTV system are as the following:

Requirements

- R 7.2-01: It is required to support continuous, reliable, and stable service quality for immersive VR service experience.

Recommendations

- RR 7.2-01: It is recommended to support high visual quality VR content for immersive and realistic visual experience.

- RR 7.2-02: It is recommended to support high audio quality VR content for immersive and realistic audio experience.

- RR 7.2-03: It is recommended to support low latency interactive response for real-time user responsiveness.

## 7.3	Security aspects including service and content protection

For general security requirements and recommendations of IPTV service, please see clause 7.3 in  [ITU-T Y.1901].

There is no extended security requirements on VR service over IPTV system.

## 7.4	Network related aspects

For general security  requirements and recommendations of IPTV service, please see clause 7.4 in  [ITU-T Y.1901].

There is no network requirements on VR service over IPTV system.

## 7.5	End systems and interoperability aspects

For general end system and interoperability requirements and recommendations of IPTV service, please see clause 7.5 in  [ITU-T Y.1901].

Extended requirements and recommendations on VR service over IPTV system are as the following:

Requirements

- R 7.5-01: It is required to support fundamental playback of VR audio-visual content.

- R 7.5-02: It is required to support essential audio-visual content control functions, such as pause, FFW, FRW, etc.

Recommendations

- RR 7.5-01: It is recommended to support interactive functionality for virtual objects engagement.

- RR 7.5-02: It is recommended to support connecting to other devices supporting VR service, such as helmet-mounted displays, controllers, etc.

Options

- OR 7.5-01: It can optionally support additional interaction methods and control options, such as gesture recognition and voice recognition to provide end-user with diverse and flexible ways for service interaction.

## 7.6	Middleware, application and content aspects

For general end system and interoperability requirements and recommendations of IPTV service, please see clause 7.6 in  [ITU-T Y.1901].

Extended requirements on VR service over IPTV system are as the following:

### 7.6.1	Middleware

Requirements

- R 7.6.1-01: It is required to support VR service client start-up and initialization function.

- R 7.6.2-02: It is required to support VR service application start-up and initialization function.

### 7.6.2	Metadata

Requirements

R 7.6.2-01: The IPTV metadata is required to support carrying information on VR services facilitating the identification and accessibility of VR contents.

# 8	Functions of VR service based on IPTV architecture

## 8.1	Video transcoding function

[TBD]

## 8.2	Rendering function

[TBD]

## 8.3	Multi-screen function

[TBD]

## 8.4	VR content delivery function

[TBD]

## 8.5	Other enhanced functions

[TBD]

VR service authentication, VR service management, VR EPG, etc.

# 9	Security considerations

Appendix I 
Use cases for VR service based on IPTV architecture

(This appendix does not form an integral part of this Recommendation)

The usage scenarios about VR services could be classified by the freedom or opportunity with which users can interact within the virtual environment from user experience aspect.

Firstly, VR service based on IPTV architecture should include the VR video services. VR users should be able to look around from a single observation point in 3D virtual environment, by either a producer or the position of a capturing devices.

Secondly, it would also provide interactive services. The virtual 3D space which the users can move within it under their own control with almost full freedom is appeared. With the virtual reality equipment, the user would be able to look around the artificial world, move around in it, and interact with virtual features or items.

[Editor’s notes (2023/01/19) ]:

- Contributions are expected to propose new names for these use cases that better differentiate them.

- Terminal devices in these use cases are related to H.722, H.723, TDES.7 (in this case – home gateway integrated model - we have VR service support defined). References and contributions to those texts may be considered as we progress in this draft recommendation.

- IPTV network should be added to figures I.1-1 and I.1-2

- The intention of presenting these use cases is to better identify the requirements that will be added to the body of the text. These new requirements may be considered for future contributions on terminal devices and IPTV platform functions.

## I.1	Use case 1: VR video services

In a single-user use case, the user could watch the 360-degree video and audio non-interactive content with using the remote controller or by gesture control in the virtual environment. The content could be VR films, sport games or live shows, etc.

Figure I.1-1 – Single-user use case

Multi-devices use case, one user employs the VR terminal equipment connected with the IPTV terminal device and IPTV network, and another user watches the traditional television. End user 1 who wears the VR terminal device could interact with the elements created by the VR application in the virtual space, and end user 2 as the spectator shares the experience. The different experience provided in this use case would like the first-person game and the third-person game.

Figure I.1-2 – Multi-device use case

## I.2	Use case 2: VR interactive services

Multiple users and terminals use case, by means of the IPTV terminals and IPTV network, the IPTV VR end users could participate in the interactive virtual space with other similar users. The IPTV VR application could create a virtual 3D space, in which the users could move around and interact with virtual persons or other virtual items. Just like the reality space, the users could interact with real people to take lessons, play games, have a party, attend a conference, even do business and so on.

Figure I.2-1 – Multiple users and terminals use case

Bibliography

[TBD]

_________________

Note from TSB: Translated terminology based on the discussion at https://en.wikipedia.org/wiki/‌Asynchronous_reprojection.