|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  | SG13-TD111-R1/WP1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 13 |  |
|  |  |  | Original: English |  |
| Question(s): |  | 20/13 | Geneva, 3-14 March 2025 |  |
| TD |  |  |  |  |
| Source: |  | Editors |  |  |
| Title: |  | Draft new Technical Report ITU-T TR.IMT2030-req: “IMT-2030 networks – Considerations on requirements” |  |  |
| Contact: |  | Lu Lu China Mobile China |  | Tel: +86 13910556279 E-mail: lulu@chinamobile.com |
| Contact: |  | Chutian Wang China Mobile China |  | Tel: +86 15010392951 E-mail: wangchutian@chinamobile.com |
| Contact: |  | Nanxiang Shi China Mobile China |  | Tel: +86 13699291776 E-mail: shinanxiang@chinamobile.com |
| Contact: |  | Luhan Wang Beijing University of Posts and Telecommunications China |  | Tel: +86 18810608898 E-mail: wluhan@bupt.edu.cn |
| Contact: |  | Menghan Yu China Telecom China |  | Tel: +86 13716166389 E-mail: yumh1@chinatelecom.cn |
| Contact: |  | Amirhossein Jafari Pozveh MCI Iran |  | Tel: +98 021 81712988 E-mail: am.jafari@mci.ir |

| Abstract: | This document is the initial output of draft new Technical Report ITU-T TR.IMT2030-req: “IMT-2030 networks – Considerations on requirements”, resulting from discussion of Q20/13 meeting on 3-14 March 2025. |
| --- | --- |

This document is the initial output of draft new Technical Report ITU-T TR.IMT2030-req: “IMT-2030 networks – Considerations on requirements”, resulting from discussion of Q20/13 meeting on 3-14 March 2025.

This document is based on this meeting's discussion and results on the following contribution:

| No. | Title | Source | Discussion |
| --- | --- | --- | --- |
| SG13-C202 | Proposal to initiate a new Technical Report on “IMT-2030 networks – Considerations on requirements” | China Mobile Communications Co. Ltd., Beijing University of Posts and Telecommunications, China Telecommunications Corporation, Mobile Telecommunication Company of Iran | Accepted with modifications Key proposals: This contribution proposes to initiate a new Technical Report on “IMT-2030 networks – Considerations on requirements”. Main changes: It was agreed to add works as references from SG11 and SG17; update the introduction to the IMT-2030 network; add more considerations on requirements from the network operation point of view; change the wording "potential" to "considerations on"; and make editorial improvements. |

Draft new Technical Report ITU-T TR.IMT2030-req

IMT-2030 networks – Considerations on requirements

Summary

This Technical Report describes the considerations of the requirements of the International Mobile Telecommunications 2030 (IMT-2030) networks. It can be used as a guideline for the further development of IMT-2030 networks related documents.

The Technical Report first provides an introduction to the IMT-2030 network, then specifies the considerations on general principles of the IMT-2030 network and considerations on requirements for overall non-radio aspects of the IMT-2030 network from both the service and network operation points of view.

Keywords

Capabilities, IMT-2030, requirements.

Table of Contents

Page

1. Scope	6

2. References	6

3. Definitions	7

3.1. Terms defined elsewhere	7

3.2. Terms defined in this Technical Report	8

4. Abbreviations and acronyms	8

5. Conventions	9

6. Introduction to the IMT-2030 network	9

7. Considerations on general principles of the IMT-2030 network	10

7.1. Sustainability	10

7.2. Connecting the unconnected	10

7.3. Security and resilience	11

7.4. Ubiquitous intelligence	11

7.5. Service diversity	11

7.6. Distributed network architecture	11

8. Considerations on requirements from the service point of view	12

8.1. Immersive communication services	12

8.2. Massive communication services	12

8.3. Hyper reliable and low-latency communication services	12

8.4. AI and communication services	13

8.5. Ubiquitous connectivity-based services	13

8.6. Integrated sensing and communication services	13

9. Considerations on requirements from the network operation point of view	13

9.1. Network flexibility and programmability	14

9.2. Fixed, mobile and satellite convergence	14

9.3. Enhanced mobility management	14

9.4. Network capability exposure	14

9.5. Identification and authentication	14

9.6. Security and personal data protection	14

9.7. More efficient signalling	14

9.8. Quality of service control	15

9.9. Network management	15

9.10. Charging	15

9.11. Interworking with non-IMT-2030 networks	15

9.12. IMT-2030 network deployment and migration	15

9.13. Network simplification	15

Bibliography	16

Annex A A.13 justification for proposed draft new Technical Report ITU-T TR.IMT2030-req “IMT-2030 networks – Considerations on requirements”	18

Draft new Technical Report ITU-T TR.IMT2030-req

IMT-2030 networks – Considerations on requirements

- Scope

This Technical Report studies the considerations on requirements of IMT-2030 networks, aiming to provide a general understanding and a guideline for further standardization works related to IMT-2030 networks.

This Technical Report addresses the following aspects of IMT-2030 networks:

- Introduction to the IMT-2030 network;

- Considerations on general principles of the IMT-2030 network;

- Considerations on requirements from the service point of view;

- Considerations on requirements from the network operation point of view.

NOTE – The radio aspects are out of the scope of this Technical Report.

- References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Technical Report. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Technical Report are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Technical Report does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T Y.3100]	Recommendation ITU-T Y.3100 (2017), Terms and definitions for IMT-2020 network.

[ITU-T Y.3101]	Recommendation ITU-T Y.3101 (2018), Requirements of the IMT-2020 network.

[ITU-T TR.IMT2030-terms]	Technical Report ITU-T TR.IMT2030-terms (2024), IMT-2030 networks – Terms and definitions.

[ITU-T TR.IMT2030-sec-con]	Technical Report ITU-T TR.IMT2030-sec-con (2024), Security Consideration for IMT-2030 Networks.

[ITU-T QSTR.SP-IMT2030]		Report ITU-T QSTR.SP-IMT2030, Signalling and Protocol Consideration on IMT-2030 networks.

[ITU-R M.2160-0]	Recommendation ITU-R M.2160-0 (2023), Framework and overall objectives of the future development of IMT for 2030 and beyond.

[ITU-R M.2516-0]	Recommendation ITU-R M.2516-0 (2022), Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond.

- Definitions

  - Terms defined elsewhere

[Editor's Note] The list of terms defined elsewhere is for further consideration.

This Technical Report uses the following terms defined elsewhere:

3.1.1	backhaul [ITU-T Y.3100]: A network path between base station systems and a core network.

3.1.2	capability level [b-ITU-T Y.3119]: The level of availability of capabilities in a network.

3.1.3	dedicated network [b-ITU-T Y.3114]: A network designed for application domains with common requirements.

3.1.4	digital twin network [b-ITU-T Y.3090]: A virtual representation of a physical network. It is useful for analysing, diagnosing, emulating and controlling the physical network based on data, model and interface, to achieve the real-time interactive mapping between the physical network and virtual twin network.

3.1.5	energy efficiency (EE) [b-ITU-T L.1330]: The relation between the useful output and energy/power consumption.

3.1.6	fixed mobile convergence (FMC) [ITU-T Y.3100]: In the context of IMT-2020, the capabilities that provide services and applications to end users regardless of the fixed or mobile access technologies being used and independently of the users' location.

3.1.7	fixed, mobile and satellite convergence (FMSC) [b-ITU-T Y.3200]: The capabilities that provide services and applications to end users regardless of the fixed, mobile or satellite access technologies being used independently of the users' location.

3.1.8	fronthaul [ITU-T Y.3100]: A network path between centralized radio controllers and remote radio units of a base station function.

3.1.9	IMT-2020 [ITU-T Y.3100]: (Based on [b-ITU-R M.2083-0]) Systems, system components, and related technologies that provide far more enhanced capabilities than those described in [b-ITU-R M.1645].

3.1.10	machine learning (ML) [b-ITU-T Y.3172]: Processes that enable computational systems to understand data and gain knowledge from it without necessarily being explicitly programmed.

3.1.11	machine learning overlay [b-ITU-T Y.3172]: A loosely coupled deployment model of machine learning functionalities whose integration and management with network functions are standardized.

3.1.12	management [ITU-T Y.3100]: In the context of IMT-2020, the processes aiming at fulfilment, assurance, and billing of services, network functions, and resources in both physical and virtual infrastructure including compute, storage, and network resources.

3.1.13	management plane [b-ITU-T Y.2011]: The set of functions used to manage entities in the stratum or layer under consideration, plus the functions required to support this management.

3.1.14	network functions virtualization (NFV) [b-ETSI GS NFV 003]: Principle of separating network functions from the hardware they run on by using virtual hardware abstraction.

3.1.15	protocol data unit (PDU) session [ITU-T Y.3100]: In the context of IMT-2020, an association between a user equipment (UE) and a data network that provides a protocol data unit (PDU) connectivity service.

3.1.16	quality of service (QoS) [b-ITU-T Q.1743]: The collective effect of service performances, which determine the degree of satisfaction of a user of a service. It is characterized by the combined aspects of performance factors applicable to all services, such as:

–	service operability performance;

–	service accessibility performance;

–	service retainability performance;

–	service integrity performance; and

–	 other factors specific to each service.

3.1.17	service continuity [b-ITU-T Y.3204]: The ability for a moving object to maintain ongoing service, including maintaining current states such as the user network environment and session.

3.1.18	service level agreement (SLA) [b-ITU-T Y.3106]: A formal agreement between two or more entities reached after a negotiating activity with the scope to assess service characteristics, responsibilities and priorities of every part. A SLA may include statements about performance, billing, service delivery but also legal and economic issues.

3.1.19	service level objective (SLO) [b-ETSI GS ZSM 007]: Element in a service level specification that is defined in terms of parameters, and related metrics, thresholds and tolerances associated with the parameters.

3.1.20	software-defined networking [b-ITU-T Y.3300]: A set of techniques that enables to directly program, orchestrate, control and manage network resources, which facilitates the design, delivery and operation of network services in a dynamic and scalable manner.

3.1.21	sustainability [b-ITU-T Y.4051]: State of the global system, including environmental, social and economic aspects, in which the needs of the present are met without compromising the ability of future generations to meet their own needs.

3.1.22	third party (3rd party) [ITU-T Y.3100]: In the context of IMT-2020, with respect to a given network operator and network end-users, an entity which consumes network capabilities and/or provides applications and/or services.

3.1.23	user equipment (UE) [b-ITU-T H.360]: Equipment under the control of an End User.

  - Terms defined in this Technical Report

[Editor's Note] The list of terms defined in this Technical Report is for further consideration.

This Technical Report defines the following terms:

None.

- Abbreviations and acronyms

This Technical Report uses the following abbreviations and acronyms:

AR		Augmented Reality

CP		Control Plane

DP		Data Plane

EE		Energy Efficiency

FMC		Fixed Mobile Convergence

FMSC		Fixed, Mobile and Satellite Convergence

IMT		International Mobile Telecommunications

ML		Machine Learning

NF		Network Function

NFV		Network Functions Virtualization

NS		Network Service

PDU		Protocol Data Unit

QoS		Quality of Service

SLA		Service Level Agreement

SLO		Service Level Objective

SDN		Software-Defined Networking

UE		User Equipment

UHD		Ultra-High Definition

UP		User Plane

XR		Extended Reality

- Conventions

In this Technical Report:

The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted, if conformance to this Technical Report is to be claimed.

The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required. Thus, this requirement need not be present to claim conformance.

The keywords "can optionally" indicate an optional requirement which is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor's implementation must provide the option, and the feature can be optionally enabled by the network operator or service provider. Rather, it means the vendor may optionally provide the feature and still claim conformance with this Technical Report.

- Introduction to the IMT-2030 network

[Editor's Note] This clause studies the definition, usage scenario, framework, and objectives of IMT-2030 network given by ITU-T Recommendations and other references.

According to [ITU-T TR.IMT2030-terms], IMT-2030 has been defined as "systems, networks, services and related technologies that support the coordination of communications, sensing, computing, artificial intelligence (AI) and security, and provide the capabilities as described in [ITU-R M.2160-0]". [b-ITU-R M.2160] defines the framework and overall objectives of the future development of IMT for 2030 and beyond. Besides, [ITU-T QSTR.SP-IMT2030] discusses the considerations on the signalling and protocol of the IMT-2030 network; [ITU-T TR.IMT2030-sec-con] discusses the security considerations of the IMT-2030 network.

The services expected to be supported by the IMT-2030 network can be classified into six categories [ITU-R M.2160-0]: Immersive communication, hyper reliable and low-latency communication, massive communication, ubiquitous connectivity, artificial intelligence and communication, and integrated sensing and communication.

The considerations on general principles and requirements of the IMT-2030 network are based on relevant studies of different expert groups. The first main source is the agreed Recommendations and Reports regarding the expectations of the IMT-2030 network, including, but not limited to, [ITU-R M.2160-0] (corresponding to the considerations on the general principles Clause 7.1 to 7.4, and requirements from the service point of view in Clause 8), [b-NGMN-6GRDC] (corresponding to Clause 9.13), and [b-3GPP TR 22.870] (corresponding to Clause 8 and Clause 9.6). Another main source is the inherited requirements from the IMT-2020 network and beyond, such as those specified in [ITU-T Y.3101] (corresponding to Clause 7.5, 7.6 and Clause 9.1 to 9.12) and [b-ITU-T Y.3200] (corresponding to Clause 9.2).

- Considerations on general principles of the IMT-2030 network

[Editor's Note] This clause studies the considerations on the general principles of the IMT-2030 network, including the four overarching aspects proposed by [ITU-R M.2160-0] and the inherited principles from the IMT-2020 network in [ITU-T Y.3101] (some of them are overlapping). The details are for further discussion. More general principles are to be added.

  - Sustainability

[Editor's Note] This subclause studies one of the overarching aspects proposed by ITU-R M.2160-0. The details are for further discussion.

IMT-2030 is expected to help address the need for increased environmental, social, and economic sustainability, and also support the goals of the Paris Agreement of the United Nations Framework Convention on Climate Change.

IMT-2030 implementations are expected to be designed to achieve the least possible environmental impact and to use resources efficiently by minimizing power consumption, using energy efficiently and reducing greenhouse gas emissions. Leveraging circular economy principles helps retain and recover value from resources and extend lifetime through such important considerations as reusing, repairing, repurposing or recycling.

Moreover, IMT-2030 is expected to allow for efficient deployments and operation, thereby improving both environmental sustainability and the affordability necessary to support social sustainability.

Beyond its own environmental impact, IMT-2030 is expected to contribute towards empowering other industries/sectors to reduce their environmental impacts by promoting digital transformation.

  - Connecting the unconnected

[Editor's Note] This subclause studies one of the overarching aspects proposed by ITU-R M.2160-0. The details are for further discussion.

IMT-2030 is expected to contribute to achieving the United Nations Sustainable Development Goals (UN SDGs), bridging digital divide, by connecting the unconnected and under-connected areas in an efficient manner, by addressing the challenges of connectivity, coverage, capacity, data rate and the mobility of terminals.

IMT-2030 is therefore expected to continue to support further development of ubiquitous connectivity that would provide digital inclusion for all by meaningfully connecting the rural and remote communities, further extending into sparsely populated areas, and maintaining the consistency of user experience between different locations including deep indoor coverage.

  - Security and resilience

[Editor's Note] This subclause studies one of the overarching aspects proposed by ITU-R M.2160-0 and general principles in ITU-T Y.3101. The details are for further discussion.

In the context of IMT-2030:

−	Security refers to preservation of confidentiality, integrity, and availability of information, such as user data and signalling, and protection of networks, devices and systems against cyberattacks such as hacking, distributed denial of service, man in the middle attacks, etc.

−	Resilience refers to capabilities of the networks and systems to continue operating correctly during and after a natural or man-made disturbance, such as the loss of primary source of power, etc.

IMT-2030 is expected to support security and personal data protection, and is expected to be designed and operated with resilience.

  - Ubiquitous intelligence

[Editor's Note] This subclause studies one of the overarching aspects proposed by ITU-R M.2160-0. The details are for further discussion.

With the steady progress and fast spread of technologies in AI and particularly machine learning, it is expected that intelligence would be present in every part of the communication system to support the building of smart cities and communities.

IMT-2030 is expected to support the autonomous management of networks by AI/ML, such as performing self-monitoring, self-organization, self-optimization, and self-healing without human intervention.

IMT-2030 could serve as an AI-enabling infrastructure capable of providing services for intelligent applications. IMT-2030 systems is expected to have functions of inferences, model training, model deployment, as well as computing distributed across networks and devices.

  - Service diversity

[Editor's Note] This subclause studies one of the inherited principles from the requirement of the IMT-2020 network ITU-T Y.3101. The details are for further discussion.

The IMT-2030 network is expected to support diversified services accommodating a wide variety of traffic characteristics and behaviours. This includes, but not limited to, diversity of quality of service requirements, diversity of mobility levels, diversity of user data types, and diversity of traffic patterns.

  - Distributed network architecture

[Editor's Note] This subclause studies one of the inherited principles from the requirement of the IMT-2020 network ITU-T Y.3101. The details are for further discussion.

The IMT-2030 network is expected be flexible enough to handle the explosive increase of traffic from emerging bandwidth-hungry services such as Ultra-High Definition (UHD) television, AR, video conferencing and remote medical surgery.

A heavily centralized architecture on to an anchor node of legacy IMT networks is expected not to satisfy the requirements associated with the explosion in mobile data traffic. This implies that the gateways to the core network are expected to be located closer to the network edges, resulting in a distributed network architecture.

- Considerations on requirements from the service point of view

[Editor's Note] This clause studies the considerations on the requirements from the service point of view of the IMT-2030 network, including the six usage scenarios proposed by [ITU-R M.2160-0]. The details are for further discussion.

  - Immersive communication services

[Editor's Note] This subclause studies one of the usage scenarios proposed by ITU-R M.2160-0, regarding the services IMT-2030 is expected to provide. The details are for further discussion.

### 8.1.1	Description

This usage scenario extends the enhanced Mobile Broadband (eMBB) of IMT-2020 and covers use cases which provide a rich and interactive video (immersive) experience to users, including the interactions with machine interfaces.

### 8.1.2	Consideration on requirements

TBD

  - Massive communication services

[Editor's Note] This subclause studies one of the usage scenarios proposed by ITU-R M.2160-0, regarding the services IMT-2030 is expected to provide. The details are for further discussion.

### 8.2.1	Description

This usage scenario extends massive Machine Type Communication (mMTC) of IMT-2020 and involves connection of massive number of devices or sensors for a wide range of use cases and applications.

### 8.2.2	Consideration on requirements

TBD

  - Hyper reliable and low-latency communication services

[Editor's Note] This subclause studies one of the usage scenarios proposed by ITU-R M.2160-0, regarding the services IMT-2030 is expected to provide. The details are for further discussion.

### 8.3.1	Description

This usage scenario extends the Ultra-Reliable and Low-Latency Communication (URLLC) of IMT-2020 and covers specialized use cases that are expected to have more stringent requirements on reliability and latency. This is typically for time-synchronized operations, where failure to meet these requirements could lead to severe consequences for the applications.

### 8.3.2	Consideration on requirements

TBD

  - AI and communication services

[Editor's Note] This subclause studies one of the usage scenarios proposed by ITU-R M.2160-0, regarding the services IMT-2030 is expected to provide. The details are for further discussion.

### 8.4.1	Description

This usage scenario would support distributed computing and AI applications.

### 8.4.2	Consideration on requirements

TBD

  - Ubiquitous connectivity-based services

[Editor's Note] This subclause studies one of the usage scenarios proposed by ITU-R M.2160-0, regarding the services IMT-2030 is expected to provide. The details are for further discussion.

### 8.5.1	Description

This usage scenario is intended to enhance connectivity with the aim to bridge the digital divide. Connectivity could be enhanced, inter alia, through interworking with other systems.

### 8.5.2	Consideration on requirements

TBD

  - Integrated sensing and communication services

[Editor's Note] This subclause studies one of the usage scenarios proposed by ITU-R M.2160-0, regarding the services IMT-2030 is expected to provide. The details are for further discussion.

### 8.6.1	Description

This usage scenario facilitates new applications and services that require sensing capabilities. It makes use of IMT-2030 to offer wide area multi-dimensional sensing that provides spatial information about unconnected objects as well as connected devices and their movements and surroundings.

### 8.6.2	Consideration on requirements

TBD

- Considerations on requirements from the network operation point of view

[Editor's Note] This clause studies the considerations on the requirements from the network operation point of view of the IMT-2030 network, including the inherited requirements from the IMT-2020 network in [ITU-T Y.3101] and requirements from relevant studies of different expert groups. The details are for further discussion.

  - Network flexibility and programmability

[Editor's Note] This subclause is one of the inherited requirements from IMT-2020 [ITU-T Y.3101]. The details are for further discussion.

TBD

  - Fixed, mobile and satellite convergence

[Editor's Note] This subclause is one of the inherited requirements from IMT-2020 [ITU-T Y.3101] and [b-ITU-T Y.3200]. The details are for further discussion.

TBD

  - Enhanced mobility management

[Editor's Note] This subclause is one of the inherited requirements from IMT-2020 [ITU-T Y.3101]. The details are for further discussion.

TBD

  - Network capability exposure

[Editor's Note] This subclause is one of the inherited requirements from IMT-2020 [ITU-T Y.3101]. The details are for further discussion.

TBD

  - Identification and authentication

[Editor's Note] This subclause is one of the inherited requirements from IMT-2020 [ITU-T Y.3101]. The details are for further discussion.

TBD

  - Security and personal data protection

[Editor's Note] This subclause is one of the inherited requirements from IMT-2020 [ITU-T Y.3101]. The details are for further discussion.

TBD

  - More efficient signalling

[Editor's Note] This subclause is one of the inherited requirements from IMT-2020 [ITU-T Y.3101] and [ITU-T QSTR.SP-IMT2030]. The details are for further discussion.

TBD

  - Quality of service control

[Editor's Note] This subclause is one of the inherited requirements from IMT-2020 [ITU-T Y.3101]. The details are for further discussion.

TBD

  - Network management

[Editor's Note] This subclause is one of the inherited requirements from IMT-2020 [ITU-T Y.3101]. The details are for further discussion.

TBD

  - Charging

[Editor's Note] This subclause is one of the inherited requirements from IMT-2020 [ITU-T Y.3101]. The details are for further discussion.

TBD

  - Interworking with non-IMT-2030 networks

[Editor's Note] This subclause is one of the inherited requirements from IMT-2020 [ITU-T Y.3101]. The details are for further discussion.

TBD

  - IMT-2030 network deployment and migration

[Editor's Note] This subclause is one of the inherited requirements from IMT-2020 [ITU-T Y.3101]. The details are for further discussion.

TBD

  - Network simplification

[Editor's Note] This subclause is one of the requirements promoted by NGMN [b-NGMN-6GRDC]. The details are for further discussion.

TBD

# Bibliography

[b-ITU-T Y.2011]	Recommendation ITU-T Y.2011 (2004), General principles and general reference model for Next Generation Networks.

[b-ITU-T Y.3090]	Recommendation ITU-T Y.3090 (2022), Digital twin network – Requirements and architecture.

[b-ITU-T Y.3102]	Recommendation ITU-T Y.3102 (2018), Framework of the IMT-2020 network.

[b-ITU-T Y.3104]	Recommendation ITU-T Y.3104 (2018), Architecture of the IMT-2020 network.

[b-ITU-T Y.3106]	Recommendation ITU-T Y.3106 (2019), Quality of service functional requirements for the IMT-2020 network.

[b-ITU-T Y.3114]	Recommendation ITU-T Y.3114 (2022), Future networks including IMT-2020: requirements and functional architecture of lightweight core for dedicated networks.

[b-ITU-T Y.3119]	Recommendation ITU-T Y.3119 (2023), Future networks including IMT-2020 – Capability classification framework for dedicated networks.

[b-ITU-T Y.3172]	Recommendation ITU-T Y.3172 (2019), Architectural framework for machine learning in future networks including IMT-2020.

[b-ITU-T Y.3200]	Recommendation ITU-T Y.3200 (2022), Fixed, mobile and satellite convergence – Requirements for IMT-2020 networks and beyond.

[b-ITU-T Y.3204]	Recommendation ITU-T Y.3204 (2023), Fixed, mobile and satellite convergence – Service continuity for IMT-2020 networks and beyond.

[b-ITU-T Y.3300]	Recommendation ITU-T Y.3300 (2014), Framework of software-defined networking.

[b-ITU-T Y.4051]	Recommendation ITU-T Y.4051 (2019), Vocabulary for smart cities and communities.

[b-ITU-T H.360]	Recommendation ITU-T H.360 (2004), An architecture for end-to-end QoS control and signalling.

[b-ITU-T L.1330]	Recommendation ITU-T L.1330 (2015), Energy efficiency measurement and metrics for telecommunication networks.

[b-ITU-T Q.1743]	Recommendation ITU-T Q.1743 (2016), IMT-Advanced references to Release 11 of LTE-Advanced evolved packet core network.

[b-ITU-T X.1500]	Recommendation ITU-T X.1500 (2011), Overview of cybersecurity information exchange.

[b-ITU-R M.1645]	Recommendation ITU-R M.1645 (2003), Framework and overall objectives of the future development of IMT-2000 and systems beyond IMT-2000.

[b-ITU-R M.2083-0]	Recommendation ITU-R M.2083-0 (2015), IMT Vision – Framework and overall objectives of the future development of IMT for 2020 and beyond.

[b-ETSI GS NFV 003]	ETSI GS NFV 003 V1.2.1 (2014), Network functions virtualisation (NFV); Terminology for main concepts in NFV.

[b-ETSI ZSM 007]	ETSI GS ZSM 007 V1.1.1 (2019), Zero-touch network and Service Management (ZSM); Terminology for concepts in ZSM.

[b-3GPP TR 22.870]	3GPP TR 22.870 (2024), Study on 6G Use Cases and Service Requirements; Stage 1 (Release 20).

[b-NGMN-6GRDC]	NGMN Alliance (2023), 6G Requirements and Design Considerations, version 1.0

Annex A
A.13 justification for proposed draft new Technical Report ITU-T TR.IMT2030-req “IMT-2030 networks – Considerations on requirements”

| Question: | Q20/13 | Proposed new ITU-T Technical report | Geneva, 3-14 March 2025 |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference and title: | ITU-T TR.IMT2030-req “IMT-2030 networks – Considerations on requirements” |  |  |  |  |
| Base text: | SG13-TD111/WP1 |  |  | Timing: | 2027-03 |
| Editor(s): | Lu Lu, China Mobile, e-mail: lulu@chinamobile.com Chutian Wang, China Mobile, e-mail: wangchutian@chinamobile.com Nanxiang Shi, China Mobile, e-mail: shinanxiang@chinamobile.com Luhan Wang, Beijing University of Posts and Telecommunications, e-mail: wluhan@bupt.edu.cn Menghan Yu, China Telecom, email: yumh1@chinatelecom.cn Amirhossein Jafari Pozveh, MCI, e-mail: am.jafari@mci.ir |  |  | Approval process: | Agreement |
| Purpose and scope (Define what this document will address and its intent or objectives in order to indicate the limits of its applicability): |  |  |  |  |  |
| On the basis of the progress made by ITU-R and ITU-T on IMT-2030, including future technology trends [ITU-R M.2516-0], framework and overall objectives [ITU-R M.2160-0], considerations on terms and definitions [ITU-T TR.IMT2030-terms], considerations on security [ITU-T TR.IMT2030-sec-con], and considerations on signalling and protocol [ITU-T QSTR.SP-IMT2030], this Technical Report aims to study the considerations on requirements of IMT-2030 networks, focusing on the following aspects: - Introduction to the IMT-2030 network; - Considerations on general principles of the IMT-2030 network, including sustainability, connecting the unconnected, security and resilience, ubiquitous intelligence, service diversity, and distributed network architecture; - Considerations on requirements from the service point of view, including immersive communication services, massive communication services, hyper reliable and low-latency communication [ITU-R M.2160-0] services, AI and communication services, ubiquitous connectivity-based services, and integrated sensing and communication services; - Considerations on requirements from the network operation point of view. |  |  |  |  |  |
| Summary (provides a brief overview of the proposal): |  |  |  |  |  |
| IMT-2030 is a key area for study and standardization within ITU, and significant progress on IMT-2030 have already been made in ITU-R and ITU-T. As a starting point, it is necessary to study the considerations on the requirements of IMT-2030 networks. This work contributes to the development of the standardization series of IMT-2030 networks, provides a general understanding and a guideline for further standardization works related to IMT-2030 networks, and specifies the considerations on requirements for network operators, service providers, and end users for practical use. This Technical Report studies the considerations on requirements of IMT-2030 networks, in the aspects of considerations on general principles, considerations on service requirements, and considerations on network operation requirements. |  |  |  |  |  |
| Relations to ITU-T Recommendations or other documents (approved or under development): |  |  |  |  |  |
| ITU-T Y.3100, ITU-T Y.3101, ITU-T TR.IMT2030-terms, ITU-T TR.IMT2030-sec-con, ITU-T QSTR.SP-IMT2030, ITU-R M.2160-0, ITU-R M.2516-0 |  |  |  |  |  |
| Liaisons with other study groups or with other standards bodies: |  |  |  |  |  |
| All ITU-T SGs, TSAG, SCV, ITU-R WP 4B, ITU-R WP 5D, 3GPP SA1, ETSI, NGMN, GSMA |  |  |  |  |  |
| Supporting members that are committing to contributing actively to the work item: |  |  |  |  |  |
| China Mobile, BUPT, China Telecom, MCI, IUST |  |  |  |  |  |

_______________________