|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  | SG13-TD101/WP1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 13 |  |
|  |  |  | Original: English |  |
| Question(s): |  | 21/13 | Geneva, 3-14 March 2025 |  |
| TD |  |  |  |  |
| Source: |  | Editors |  |  |
| Title: |  | Draft new Technical Report ITU-T TR.NS-IMT2030: “Network softwarization– Considerations on technologies for IMT-2030 networks” |  |  |
| Contact: |  | Yushuang Hu China Mobile China |  | Tel: +86 13810622705 E-mail: huyushuang@chinamobile.com |
| Contact: |  | Lu Lu China Mobile China |  | Tel: +86 13910556279 E-mail: lulu@chinamobile.com |
| Contact: |  | Huiqing Sun China Mobile China |  | Tel: +86 18701380263 E-mail: sunhuiqing@chinamobile.com |
| Contact: |  | Jinyan Li, China Telecom China |  | Tel: +86 10-50902527 E-mail: lijinyan@chinatelecom.cn |

| Abstract: | This document is the initial output of draft new Technical Report ITU-T TR.NS: “Network softwarization– Considerations on technologies for IMT-2030 networks”, resulting from discussion of Q21/13 meeting on 3-14 March 2025. |
| --- | --- |

This document is the initial output of draft new Technical Report ITU-T TR.NS: “Network softwarization– Considerations on technologies for IMT-2030 networks”, resulting from discussion of Q21/13 meeting on 3-14 March 2025.

This document is based on this meeting's discussion and results on the following contribution:

| No. | Title | Source | Discussion |
| --- | --- | --- | --- |
| SG13-C256 | Proposal to initiate a new Technical Report on "Network softwarization– Considerations on technologies for IMT-2030 networks" | China Mobile | Accepted with modifications Main changes: change the wording of "potential" to "considerations on"; and make editorial improvements. |

Draft new Technical Report ITU-T TR.NS-IMT2030

Network softwarization – Considerations on technologies for IMT-2030 networks

Summary

Network softwarization is an overall approach for designing, implementing, deploying, managing and maintaining network equipment and network components by software. It exploits the natures of software such as flexibility and rapidity all along the life-cycle of network equipment/components, for the sake of creating conditions enabling the re-design of network and services architectures, optimizing costs and processes, enabling self-management and bringing added values in network infrastructures.

This Technical Report studies the general considerations, potential use cases, potential requirements, potential framework, potential network function enhancements, potential procedures and potential security considerations of network softwarization, in the context of IMT-2030 networks. This Technical Report provides a general understanding and a guideline for network softwarization related work items for IMT-2030 networks.

Keywords

IMT-2030, Network softwarization, M&O

Table of Contents

Page

1. Scope	10

2. References	10

3. Definitions	11

3.1. Terms defined elsewhere	11

3.2. Terms considered in this Technical Report	13

4. Abbreviations and acronyms	13

5. Conventions	14

6. General considerations on Network softwarization for IMT-2030 networks	14

6.1. Overview on Network softwarization for IMT-2030 networks	14

6.2. Network softwarization related mechanisms for IMT-2030 networks	15

7. Potential use cases of Network softwarization for IMT-2030 networks	16

8. Potential requirements of Network softwarization for IMT-2030 networks	16

9. Potential framework of Network softwarization for IMT-2030 networks	16

9.1. Potential Network softwarization framework	16

9.2. Potential reference points	16

10. Potential network function enhancements of Network softwarization for IMT-2030 networks	16

11. Potential procedures of network softwarization for IMT-2030 networks	16

11.1. Potential procedures of network softwarization related mechanisms	16

11.2. Potential service procedures	17

12. Potential security considerations of network softwarization for IMT-2030 networks	17

Bibliography	18

Annex A A.13 justification for proposed draft new Technical Report ITU-T TR.NS-IMT2030: “Network softwarization – Considerations on technologies for IMT-2030 networks”	19

Draft new Technical Report ITU-T TR.NS-IMT2030

Network softwarization – Considerations on technologies for IMT-2030 networks

- Scope

This Technical Report studies the considerations on technologies of network softwarization for IMT-2030 networks. Network softwarization is an overall approach for designing, implementing, deploying, managing and maintaining network equipment and network components by software.  [ITU-T Y.3150]. This Technical Report provides a general understanding and a guideline for network softwarization related work items for IMT-2030 networks.

This Technical Report addresses the following aspects of Network softwarization for IMT-2030 networks:

- General considerations of the Network softwarization for IMT-2030 network;

- Considerations on requirements of Network softwarization for IMT-2030 network;

- Considerations on enhancements of Network softwarization for IMT-2030 network.

- References

[ITU-T Y.3101]	Recommendation ITU-T Y.3101 (2018), Requirements of the IMT-2020 network.

[ITU-T Y.3102]	Recommendation ITU-T Y.3102 (2018), Framework of the IMT-2020 network.

[ITU-T Y.3104]	Recommendation ITU-T Y.3104 (2018), Architecture of the IMT-2020 network.

[ITU-T Y.3110]	Recommendation ITU-T Y.3110 (2017), IMT-2020 network management and orchestration requirements.

[ITU-T Y.3111]	Recommendation ITU-T Y.3111 (2017), IMT-2020 network management and orchestration framework.

[ITU-T Y.3112]	Recommendation ITU-T Y.3112 (2018), Framework for the support of network slicing in the IMT-2020 network.

[ITU-T Y.3151]	Recommendation ITU-T Y.3151 (2019), High-level technical characteristics of network softwarization for IMT-2020 - Part: SDN.

[ITU-T Y.3152]	Recommendation ITU-T Y.3152 (2019), Advanced data plane programmability for IMT-2020.

[ITU-T Y.3153]	Recommendation ITU-T Y.3153 (2020), Network slice orchestration and management for providing network services to 3rd party in the IMT-2020 network.

[ITU-T Y.3154]	Recommendation ITU-T Y.3154 (2020), Resource pooling for scalable network slice service management and orchestration in the IMT-2020 network.

[ITU-T Y.3300]	Recommendation ITU-T Y.3300 (2014), Framework of software-defined networking.

[ITU-T Y.3502]	Recommendation ITU-T Y.3502 (2014), Information technology – Cloud computing – Reference architecture

[ITU-T TR.IMT2030-terms]		Report ITU-T TR.IMT2030-terms, IMT-2030 networks – Considerations on terms and definitions.

[ITU-T TR.IMT2030-sec-con]	Report ITU-T TR.IMT2030-sec-con, Security Consideration for IMT-2030 Networks.

[ITU-R M.2160-0]	Recommendation ITU-R M.2160-0 (2023), Framework and overall objectives of the future development of IMT for 2030 and beyond.

[ITU-R M.2460-0]	Report ITU-R M.2460-0 (2019), Key elements for integration of satellite systems into Next Generation Access Technologies.

[ITU-R M.2516-0]	Report ITU-R M.2516-0 (2022), Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond.

- Definitions

  - Terms defined elsewhere

This Technical Report uses the following terms defined elsewhere:

TBD

  - Terms considered in this Technical Report

This Technical Report considers the following terms:

TBD

- Abbreviations and acronyms

This Technical Report uses the following abbreviations and acronyms:

AI		Artificial Intelligence

CNC		Coordination of Networking and Computing

CP		Control Plane

DP		Data Plane

EE		Energy Efficiency

E2E		End-to-End

IMT		International Mobile Telecommunications

ML		Machine Learning

NF		Network Function

NFV		Network Functions Virtualization

NSSF		Network Slice Selection Function

QoS		Quality of Service

SDN		Software-Defined Networking

SLA		Service Level Agreement

SLO		Service Level Objective

SMF		Session Management Function

UE		User Equipment

UPF		User Plane Function

- Conventions

In this Technical Report:

The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted, if conformance to this Technical Report is to be claimed.

The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required. Thus, this requirement need not be present to claim conformance.

The keywords "can optionally" indicate an optional requirement which is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor's implementation must provide the option, and the feature can be optionally enabled by the network operator/service provider. Rather, it means the vendor may optionally provide the feature and still claim conformance with this Technical Report.

- General considerations on Network softwarization for IMT-2030 networks

[Editor's Note] This clause presents the overview on Network softwarization related mechanisms, in the context of IMT-2030 networks.

  - Overview on Network softwarization for IMT-2030 networks

Nowadays, the IMT systems have evolved from IMT-2020 to IMT-2030, the Report ITU-R M.2516-0 “Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond” provides a broad view of future technical aspects of terrestrial IMT systems considering the time frame up to 2030 and beyond, characterized with respect to key emerging services, applications trends and relevant driving factors and is applicable to radio interfaces, mobile terminals, and radio access networks by considering the time frame up to 2030 and beyond. It also emphases the roles of modularity and complementarity of new technological solutions become increasingly important in the development of increasingly complex systems. The use of data and algorithms such as artificial intelligence (AI) will play an important role, and technological complementarities are required to ensure that the technology innovations complement each other. This is particularly important as the role of IMT-2030 and beyond can be seen as a pervasive general-purpose system, instead of simply an enabling technology, resulting in complex technical dependencies.

The another Recommendation ITU-R M.2160-0 “Framework and overall objectives of the future development of IMT for 2030 and beyond” provides the trends of IMT-2030 framework and overall objectives, in which the user and application trends include ubiquitous intelligence, ubiquitous computing, immersive multimedia and multi-sensory interactions, digital twin and virtual world, smart industrial applications, digital health and well-being, ubiquitous connectivity, integration of sensing and communication, and sustainability. Six main usage scenarios of IMT-2030 have been defined in this Recommendation, which include immersive communication, hyper reliable and low-latency communication, massive communication, ubiquitous connectivity, artificial intelligence and communication, and integrated sensing and communication.

Based on the above work progress made by ITU-R, it’s a time window for ITU-T to study and standard the work on non-radio aspects of IMT-2030, on the standardization of network softwarization in the fields of requirements, frameworks, network capabilities, enabling technologies, network function enhancements, and service enhancements. The whole study is specifically to support the non-radio aspects of 2020 network evolution and IMT-2030 networks.

In order to smoothly continue the evolution of the existing study and to address the new network capabilities for IMT-2030, it is essential to begin studying the considerations of network softwarization for 2030 now, which will provide a baseline for network softwarization related work items for IMT-2030 networks. It is suggested that the considerations on network softwarization technologies for IMT-2030 networks should be taken into further study. This proposed Technical Report contributes to the development of the standardization series of network softwarization and IMT-2030 networks.

The further study area can divided into the following areas, which can be the technique guidance for this draft Technical Report:

‒	Enhancing the standardization work for network softwarization, including generic SDN and their profiles for intent-based networking, network virtualization, network slicing, NFV, network platformized, DevOps of networking software.

‒	Extending the management and orchestration related work, which covers aspects such as operation, assurance, fulfillment and automation, including management interaction with entities external to the network operator (e.g. service providers and verticals).

‒	Strengthening the study related to network autonomous to enhance the operation and maintenance of the network, corresponding technologies can include the ALML management, management data analytics, network digital twin, zero outage network.

‒	Supporting the new services including but not limited to computing, sensing, energy efficiency, management capability exposure, AI agent communication network.

This draft Technical Report will focus on the following aspects:

- General considerations of the Network softwarization for IMT-2030 network;

- Considerations on requirements of Network softwarization for IMT-2030 network;

- Considerations on enhancements of Network softwarization for IMT-2030 network.

  - Network softwarization related mechanisms for IMT-2030 networks

- Enhancing the mechanisms for network softwarization

- Extending the management and orchestration related mechanisms

- Strengthening the mechanisms related to network autonomous

- Supporting the new services

- Potential use cases of Network softwarization for IMT-2030 networks

[Editor's Note] This clause studies the potential use cases of Network softwarization, in the context of IMT-2030 networks.

The potential use cases of Network softwarization for IMT-2030 networks include, but are not limited to, the follows:

- Potential requirements of Network softwarization for IMT-2030 networks

[Editor's Note] This clause studies the potential requirements of Network softwarization, in the context of IMT-2030 networks, which include the potential general requirements, potential requirements on framework, potential requirements on network functions, and potential requirements on network performance.

TBD

- Potential enhancementsof Network softwarization for IMT-2030 networks

[Editor's Note] This clause studies the potential enhancements of Network softwarization, in the context of IMT-2030 networks, which include potential Network softwarization framework and potential reference points.

- Potential security considerations of network softwarization for IMT-2030 networks

[Editor's Note] This clause presents the potential security and privacy considerations of Network softwarization, in the context of IMT-2030 networks, which include network security, service security, user privacy and service privacy.

The potential security and privacy considerations of Network softwarization for IMT-2030 networks based on the consideration described in [ITU-T Y.3150].

Firstly, general security requirements for IP-based networks in [ITU-T Y.2701] and for IMT-2020 systems in [ITU-T Y.3101] are fundamentals for network softwarization.

Secondly, network softwarization consist of SDN, NFV and cloud computing techniques; therefore, security threats in each technical field should be taken care.

Security and privacy issues related to cloud computing services and some of the identified issues may occur in network softwarization environments using network virtualization techniques.  The issues described in [ITU-T X.1601], which introduces general security framework for cloud computing, should be considered during planning and designing networks for mitigating the issues.

In addition, network softwarization usually applies a centralized control manner based on the separation of control plane and data plane.  With a centralized SDN controller, the impact of a denial-of-service (DoS)/distributed denial-of-service (DDoS) attack, security issues on scalability and resilience of the control plane can be higher than that directed against a single router.  [ITU-T X.1038] identifies security threats for SDN application layer, SDN control layer, SDN resource layer, application-control interface, and resource-control interface according to the framework of SDN [ITU-T Y.3300].  Possible security countermeasures against the issues should be examined.

# Bibliography

[b-ITU-T Y.2011]	Recommendation ITU-T Y.2011 (2004), General principles and general reference model for Next Generation Networks.

[b-ITU-T Y.2701]	Recommendation ITU-T Y.2701 (2007), Security requirements for NGN release 1.

[b-ITU-T Y.3061]	Recommendation ITU-T Y.3061 (2023), Autonomous Networks - Architecture framework.

[b-ITU-T Y.3090]	Recommendation ITU-T Y.3090 (2022), Digital twin network – Requirements and architecture.

[b-ITU-T Y.3100]	Recommendation ITU-T Y.3100 (2017), Terms and definitions for IMT-2020 network.

[b-ITU-T Y.3106]	Recommendation ITU-T Y.3106 (2019), Quality of service functional requirements for the IMT-2020 network.

[b-ITU-T Y.3144]	Recommendation ITU-T Y.3144 (2024), Future networks including IMT-2020 – Requirements and functional architecture of distributed core network.

[b-ITU-T Y.3300]	Recommendation ITU-T Y.3300 (2014), Framework of software-defined networking.

[b-ITU-T H.360]	Recommendation ITU-T H.360 (2004), An architecture for end-to-end QoS control and signalling.

[b-ITU-T L.1330]	Recommendation ITU-T L.1330 (2015), Energy efficiency measurement and metrics for telecommunication networks.

[b-ITU-T M.3080]	Recommendation ITU-T M.3080 (2021), Framework of artificial intelligence enhanced telecom operation and management (AITOM).

[b-ITU-R M.1645]	Recommendation ITU-R M.1645 (2003), Framework and overall objectives of the future development of IMT-2000 and systems beyond IMT-2000.

[b-3GPP TS 28.100]	3GPP TS 28.100 (2024), Management and orchestration; Levels of autonomous network (Release 18).

[b-3GPP TR 22.870]	3GPP TR 22.870 (2024), Study on 6G Use Cases and Service Requirements; Stage 1 (Release 20).

[b-ETSI GS NFV 003]	ETSI GS NFV 003 V1.4.1 (2018), Network Functions Virtualisation (NFV); Terminology for main concepts in NFV.

[b-ETSI GS ZSM 007]	ETSI GS ZSM 007 V2.1.1 (2023), Zero-touch network and Service Management (ZSM); Terminology for concepts in ZSM.

_______________________

Annex A
A.13 justification for proposed draft new Technical Report ITU-T TR.NS-IMT2030: “Network softwarization – Considerations on technologies for IMT-2030 networks”

| Question: | Q21/13 | Proposed new ITU-T Technical report | Geneva, 3-14 March 2025 |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference and title: | Technical Report ITU-T TR.NS-IMT2030 “Network softwarization – Considerations on technologies for IMT-2030 networks” |  |  |  |  |
| Base text: | SG13-TD101/WP1 |  |  | Timing: | 2027-03 |
| Editor(s): | Yushuang Hu, China Mobile, e-mail: huyushuang@chinamobile.com Lu Lu, China Mobile, e-mail: lulu@chinamobile.com Huiqing Sun, China Mobile, e-mail: sunhuiqing@chinamobile.com Jinyan Li, China Telecom, e-mail: lijinyan@chinatelecom.cn |  |  | Approval process: | Agreement |
| Purpose and scope (Define what this document will address and its intent or objectives in order to indicate the limits of its applicability): |  |  |  |  |  |
| This Technical Report studies the considerations on technologies of network softwarization for IMT-2030 networks. Network softwarization is an overall approach for designing, implementing, deploying, managing and maintaining network equipment and network components by software. [ITU-T Y.3150]. This Technical Report provides a general understanding and a guideline for network softwarization related work items for IMT-2030 networks. This draft Technical Report will focus on the following aspects: - General considerations of the Network softwarization for IMT-2030 network; - Considerations on requirements of the Network softwarization for IMT-2030 network; - Considerations on enhancements of the Network softwarization for IMT-2030 network. |  |  |  |  |  |
| Summary (provides a brief overview of the proposal): |  |  |  |  |  |
| Based on the above work progress made by ITU-R, it’s a time window for ITU-T to study and standard the work on non-radio aspects of IMT-2030, on the standardization of network softwarization in the fields of requirements, frameworks, network capabilities, enabling technologies, network function enhancements, and service enhancements. The whole study is specifically to support the non-radio aspects of 2020 network evolution and IMT-2030 networks. In order to smoothly continue the evolution of the existing study and to address the new network capabilities for IMT-2030, it is essential to begin studying the considerations of network softwarization for 2030 now, which will provide a baseline for network softwarization related work items for IMT-2030 networks. It is suggested that the considerations on network softwarization technologies for IMT-2030 networks should be taken into further study. This proposed Technical Report contributes to the development of the standardization series of network softwarization and IMT-2030 networks. |  |  |  |  |  |
| Relations to ITU-T Recommendations or other documents (approved or under development): |  |  |  |  |  |
| ITU-T Y.3101, ITU-T Y.3102, ITU-T Y.3110, ITU-T Y.3111, ITU-T Y.3150, ITU-T Y.3112, ITU-T Y.3151, ITU-T Y.3152, ITU-T Y.3153, ITU-T Y.3154, ITU-T Y.3156, ITU-T Y.3157, ITU-T Y.3158, ITU-T Y.3159, ITU-T Y.3160, ITU-T Y.3161, ITU-T TR.IMT2030-terms, ITU-T TR.IMT2030-sec-con |  |  |  |  |  |
| Liaisons with other study groups or with other standards bodies: |  |  |  |  |  |
| ITU-T SG11, ITU-T SG2, ITU-T SG17 |  |  |  |  |  |
| Supporting members that are committing to contributing actively to the work item: |  |  |  |  |  |
| China Mobile, China Unicom, China Telecom, CICT |  |  |  |  |  |

_______________________