|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  | SG13-TD77/WP1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 13 |  |
|  |  |  | Original: English |  |
| Question(s): |  | 23/13 | Geneva, 3-14 March 2025 |  |
| TD |  |  |  |  |
| Source: |  | Editors |  |  |
| Title: |  | Draft new Technical Report ITU-T TR.FMSC-IMT2030: “Fixed, mobile and satellite convergence – Considerations on enhancements to support IMT-2030 networks” |  |  |
| Contact: |  | Nanxiang Shi China Mobile China |  | Tel: +86 13699291776 E-mail: shinanxiang@chinamobile.com |
| Contact: |  | Lu Lu China Mobile China |  | Tel: +86 13910556279 E-mail: lulu@chinamobile.com |
| Contact: |  | Yufeng He China Telecom China |  | Tel: +86 18925052573 E-mail: heyf@chinatelecom.cn |
| Contact: |  | Lin Lin China Unicom China |  | Tel: +86 18610352240 E-mail: linl12@chinaunicom.cn |
| Contact: |  | Hui Xu CICT China |  | Tel: +86 10 58832000 E-mail: xuhui1@cictmobile.com |
| Contact: |  | Amirhossein Jafari Pozveh MCI Iran |  | Tel: +98 021 81712988 E-mail: am.jafari@mci.ir |

| Abstract: | This document is the initial output of draft new Technical Report ITU-T TR.FMSC-IMT2030: “Fixed, mobile and satellite convergence – Considerations on enhancements to support IMT-2030 networks”, resulting from discussion of Q23/13 meeting on 3-14 March 2025. |
| --- | --- |

This document is the initial output of draft new Technical Report ITU-T TR.FMSC-IMT2030: “Fixed, mobile and satellite convergence – Considerations on enhancements to support IMT-2030 networks”, resulting from discussion of Q23/13 meeting on 3-14 March 2025.

This document is based on this meeting's discussion and results on the following contribution:

| No. | Title | Source | Discussion |
| --- | --- | --- | --- |
| SG13-C154 | Proposal to initiate a new Technical Report on “Fixed, mobile and satellite convergence – Considerations on enhancements to support IMT-2030 networks” | China Mobile Communications Co. Ltd. | Accepted with modifications Key proposals: This contribution proposes to initiate a new Technical Report on “Fixed, mobile and satellite convergence – Considerations on technologies for IMT-2030 networks”. Main changes: It was agreed to change the title to “Fixed, mobile and satellite convergence – Considerations on enhancements to support IMT-2030 networks”; change the wording of “potential” to “considerations on”; and make editorial improvements. |

Draft new Technical Report ITU-T TR.FMSC-IMT2030

Fixed, mobile and satellite convergence – Considerations on enhancements to support IMT-2030 networks

Summary

Fixed, mobile and satellite convergence (FMSC) is the capability that provides services and applications to end users regardless of the fixed, mobile or satellite access technologies being used. This Technical Report studies the general considerations, considerations on use cases, considerations on requirements, considerations on framework, considerations on network function enhancements, considerations on procedures and considerations on security of FMSC, in the context of IMT-2030 networks. This Technical Report provides a general understanding and a guideline for FMSC related work items for IMT-2030 networks.

Keywords

FMSC, IMT-2030, satellite network

Table of Contents

Page

1. Scope	5

2. References	5

3. Definitions	6

3.1. Terms defined elsewhere	6

3.2. Terms considered in this Technical Report	8

4. Abbreviations and acronyms	8

5. Conventions	9

6. General considerations on FMSC for IMT-2030 networks	10

6.1. Overview on FMSC for IMT-2030 networks	10

6.2. FMSC related mechanisms for IMT-2030 networks	12

7. Considerations on use cases of FMSC for IMT-2030 networks	13

8. Considerations on requirements of FMSC for IMT-2030 networks	14

8.1. Considerations on general requirements	14

8.2. Considerations on requirements of framework	15

8.3. Considerations on requirements of network functions	15

8.4. Considerations on requirements of network performance	15

9. Considerations on framework of FMSC for IMT-2030 networks	15

9.1. Considerations on FMSC framework	15

9.2. Considerations on reference points	15

10. Considerations on network function enhancements of FMSC for IMT-2030 networks	16

10.1. Considerations on network function enhancements to access network	16

10.2. Considerations on network function enhancements to core network	16

10.3. Considerations on network function enhancements to service plane	17

10.4. Considerations on network function enhancements to management plane	17

11. Considerations on procedures of FMSC for IMT-2030 networks	17

11.1. Considerations on procedures of FMSC related mechanisms	17

11.2. Considerations on service procedures	17

12. Considerations on security of FMSC for IMT-2030 networks	17

Bibliography	19

Annex A A.13 justification for proposed draft new ITU-T TR.FMSC-IMT2030: “Fixed, mobile and satellite convergence – Considerations on enhancements to support IMT-2030 networks”	21

Draft new Technical Report ITU-T TR.FMSC-IMT2030

Fixed, mobile and satellite convergence – Considerations on enhancements to support IMT-2030 networks

- Scope

This Technical Report studies the considerations on enhancements to support IMT-2030 networks in FMSC network. FMSC is the capability that provides services and applications to end users regardless of the fixed, mobile or satellite access technologies being used [ITU-T Y.3200]. This Technical Report provides a general understanding and a guideline for FMSC related work items for IMT-2030 networks.

This Technical Report addresses the following aspects of FMSC for IMT-2030 networks:

- General considerations;

- Considerations on use cases;

- Considerations on requirements;

- Considerations on framework;

- Considerations on network function enhancements;

- Considerations on procedures;

- Considerations on security.

NOTE – The radio aspects are out of the scope of this Technical Report.

- References

[ITU-T Y.3101]	Recommendation ITU-T Y.3101 (2018), Requirements of the IMT-2020 network.

[ITU-T Y.3102]	Recommendation ITU-T Y.3102 (2018), Framework of the IMT-2020 network.

[ITU-T Y.3104]	Recommendation ITU-T Y.3104 (2018), Architecture of the IMT-2020 network.

[ITU-T Y.3114]	Recommendation ITU-T Y.3114 (2022), Future networks including IMT-2020: requirements and functional architecture of lightweight core for dedicated networks.

[ITU-T Y.3172]	Recommendation ITU-T Y.3172 (2019), Architectural framework for machine learning in future networks including IMT-2020.

[ITU-T Y.3200]	Recommendation ITU-T Y.3200 (2022), Fixed, mobile and satellite convergence – Requirements for IMT-2020 networks and beyond.

[ITU-T Y.3201]	Recommendation ITU-T Y.3201 (2023), Fixed, mobile and satellite convergence – Framework for IMT-2020 networks and beyond.

[ITU-T Y.3216]	Recommendation ITU-T Y.3216 (2024), Fixed, mobile and satellite convergence – Distributed core network for IMT-2020 networks and beyond.

[ITU-T TR.IMT2030-terms]		Report ITU-T TR.IMT2030-terms, IMT-2030 networks – Considerations on terms and definitions.

[ITU-T TR.IMT2030-sec-con]	Report ITU-T TR.IMT2030-sec-con, Security Consideration for IMT-2030 Networks.

[ITU-T QSTR.SP-IMT2030]		Report ITU-T QSTR.SP-IMT2030, Signalling and Protocol Consideration on IMT-2030 networks.

[ITU-T Y.supp.fmsc-roadmap]	Supplement ITU-T Y.supp.fmsc-roadmap, Fixed, mobile and satellite convergence – Standardization roadmap.

[ITU-R M.2160-0]	Recommendation ITU-R M.2160-0 (2023), Framework and overall objectives of the future development of IMT for 2030 and beyond.

[ITU-R M.2460-0]	Report ITU-R M.2460-0 (2019), Key elements for integration of satellite systems into Next Generation Access Technologies.

[ITU-R M.2516-0]	Report ITU-R M.2516-0 (2022), Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond.

- Definitions

  - Terms defined elsewhere

This Technical Report uses the following terms defined elsewhere:

3.1.1	access network [ITU-T TR.IMT2030-terms]: A network that connects user equipment (UE) to the communications network, and provides network services.

NOTE – It includes, but is not limited to, fixed access network, mobile access network, and satellite access network.

3.1.2	artificial intelligence (AI) [b-ITU-T M.3080]: Computerized system that uses cognition to understand information and solve problems.

3.1.3	autonomous network [b-3GPP TS 28.100]: Telecommunication system (including management system and network) with autonomy capabilities which is able to be governed by itself with minimal to no human intervention.

3.1.4	backhaul [b-ITU-T Y.3100]: A network path between base station systems and a core network.

3.1.5	computing plane [ITU-T TR.IMT2030-terms]: A pool of computing related network functionalities which enable computing functions such as computing enforcement, computing resource management, and computing task management.

3.1.6	control plane (CP) [ITU-T TR.IMT2030-terms]: A pool of control related network functionalities which enable control functions such as communications control, sensing control, computing control, artificial intelligence (AI) / machine learning (ML) control, security control, energy efficiency control, and communications services.

3.1.7	core network [ITU-T TR.IMT2030-terms]: A network that aggregates access networks, connects to the data network, and provides network services.

3.1.8	data network [ITU-T TR.IMT2030-terms]: A network that connects communications network to the Internet and/or third party services.

3.1.9	data plane (DP) [ITU-T TR.IMT2030-terms]: A pool of data related network functionalities which enable data functions such as data storage management, data agent, data orchestration, data services, and sensing services.

3.1.10	dedicated network [ITU-T Y.3114]: A network designed for application domains with common requirements.

3.1.11	digital twin network [b-ITU-T Y.3090]: A virtual representation of a physical network. It is useful for analysing, diagnosing, emulating and controlling the physical network based on data, model and interface, to achieve the real-time interactive mapping between the physical network and virtual twin network.

3.1.12	energy efficiency (EE) [b-ITU-T L.1330]: The relation between the useful output and energy/power consumption.

3.1.13	fixed mobile convergence (FMC) [b-ITU-T Y.3100]: In the context of IMT-2020, the capabilities that provide services and applications to end users regardless of the fixed or mobile access technologies being used and independently of the users' location.

3.1.14	fixed, mobile and satellite convergence (FMSC) [ITU-T Y.3200]: The capabilities that provide services and applications to end users regardless of the fixed, mobile or satellite access technologies being used independently of the users' location.

3.1.15	fronthaul [b-ITU-T Y.3100]: A network path between centralized radio controllers and remote radio units of a base station function.

3.1.16	IMT-2020 [b-ITU-T Y.3100]: Systems, system components, and related technologies that provide far more enhanced capabilities than those described in [b-ITU-R M.1645].

NOTE – [b-ITU-R M.1645] defines the framework and overall objectives of the future development of IMT-2000 and systems beyond IMT-2000 for the radio access network.

3.1.17	IMT-2030 [ITU-T TR.IMT2030-terms]: Systems, networks, services and related technologies that support the coordination of communications, sensing, computing, artificial intelligence (AI) and security, and provide the capabilities as described in [ITU-R M.2160-0].

NOTE – [ITU-R M.2160-0] defines the framework and overall objectives of the future development of IMT for 2030 and beyond.

3.1.18	machine learning (ML) [ITU-T Y.3172]: Processes that enable computational systems to understand data and gain knowledge from it without necessarily being explicitly programmed.

3.1.19	machine learning overlay [ITU-T Y.3172]: A loosely coupled deployment model of machine learning functionalities whose integration and management with network functions are standardized.

3.1.20	management plane [b-ITU-T Y.2011]: The set of functions used to manage entities in the stratum or layer under consideration, plus the functions required to support this management.

3.1.21	multi-access user equipment (MUE) [b-ITU-T Y.FMSC-SS]: A user equipment which can support accessing two or more access networks simultaneously under the control of a network enhanced for multi-access capability.

3.1.22	network function (NF) [ITU-T TR.IMT2030-terms]: A processing function in a network, which has defined functional behaviour and defined interfaces.

3.1.23	network functions virtualization (NFV) [b-ETSI GS NFV 003]: Principle of separating network functions from the hardware they run on by using virtual hardware abstraction.

3.1.24	network service (NS) [ITU-T TR.IMT2030-terms]: A communication service or an information service, which is provided by network functions or a network.

3.1.25	security plane [ITU-T TR.IMT2030-terms]: A pool of security related network functionalities which enable security functions such as security and trust capabilities, security and trust policies, and security and trust services.

3.1.26	service continuity [b-ITU-T Y.3204]: The ability for a moving object to maintain ongoing service, including maintaining current states such as the user network environment and session.

3.1.27	service level agreement (SLA) [b-ITU-T Y.3106]: A formal agreement between two or more entities reached after a negotiating activity with the scope to assess service characteristics, responsibilities and priorities of every part. A SLA may include statements about performance, billing, service delivery but also legal and economic issues.

3.1.28	service level objective (SLO) [b-ETSI GS ZSM 007]: Element in a service level specification that is defined in terms of parameters, and related metrics, thresholds and tolerances associated with the parameters.

3.1.29	service plane [b-ITU-T Q.1290]: The plane in the unified functional model that describes services.

3.1.30	software-defined networking (SDN) [b-ITU-T Y.3300]: A set of techniques that enables to directly program, orchestrate, control and manage network resources, which facilitates the design, delivery and operation of network services in a dynamic and scalable manner.

3.1.31	sustainability [b-ITU-T Y.4051]: State of the global system, including environmental, social and economic aspects, in which the needs of the present are met without compromising the ability of future generations to meet their own needs.

3.1.32	third party (3rd party) [b-ITU-T Y.3100]: In the context of IMT-2020, with respect to a given network operator and network end-users, an entity which consumes network capabilities and/or provides applications and/or services.

3.1.33	user equipment (UE) [b-ITU-T H.360]: Equipment under the control of an End User.

3.1.34	user plane (UP) [ITU-T TR.IMT2030-terms]: A pool of user related network functionalities which enable user functions such as traffic processing, service processing, and policy enforcement.

  - Terms considered in this Technical Report

This Technical Report considers the following terms:

None.

- Abbreviations and acronyms

This Technical Report uses the following abbreviations and acronyms:

AF		Application Function

AI		Artificial Intelligence

ASF		Authentication Server Function

CEF		Capability Exposure Function

CNC		Coordination of Networking and Computing

CP		Control Plane

DP		Data Plane

EE		Energy Efficiency

E2E		End-to-End

FMC		Fixed Mobile Convergence

FMSC		Fixed, Mobile and Satellite Convergence

GSO		Geostationary-Satellite Orbit

IMT		International Mobile Telecommunications

ML		Machine Learning

MUE		Multi-access User Equipment

NACF		Network Access Control Function

NF		Network Function

NFR		Network Function Registry function

NFV		Network Functions Virtualization

NGSO		Non-Geostationary Satellite Orbit

NS		Network Service

NSSF		Network Slice Selection Function

PCF		Policy Control Function

QoS		Quality of Service

SDN		Software-Defined Networking

SLA		Service Level Agreement

SLO		Service Level Objective

SMF		Session Management Function

UE		User Equipment

UP		User Plane

UPF		User Plane Function

USM		Unified Subscription Management function

- Conventions

In this Technical Report:

The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted, if conformance to this Technical Report is to be claimed.

The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required. Thus, this requirement need not be present to claim conformance.

The keywords "can optionally" indicate an optional requirement which is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor's implementation must provide the option, and the feature can be optionally enabled by the network operator/service provider. Rather, it means the vendor may optionally provide the feature and still claim conformance with this Technical Report.

- General considerations on FMSC for IMT-2030 networks

[Editor's Note] This clause presents the overview on FMSC and FMSC related mechanisms, in the context of IMT-2030 networks. The text is for further discussion.

  - Overview on FMSC for IMT-2030 networks

Fixed, mobile and satellite convergence (FMSC) is the capability that provides services and applications to end users regardless of the fixed, mobile or satellite access technologies being used. The FMSC network consists of multi-access user equipment (MUE), fixed/mobile/satellite access network, land-based/satellite-based core network, interworking function, land-based/satellite-based service platform, and land-based/satellite-based data network. The FMSC network is typically a large scale end-to-end (E2E) network covering a large geographic area. With the introduction of geostationary-satellite orbit (GSO) satellite and non-geostationary satellite orbit (NGSO) satellite, the topology and organization of FMSC network is dynamic; the network entities, resources, capabilities, capacity, interfaces and procedures in FMSC network are heterogeneous. Figure 6-1 depicts the overall framework of FMSC for IMT-2020 networks and beyond [ITU-T Y.3201].

Figure 6-1 – Overall framework of FMSC for IMT-2020 networks and beyond [ITU-T Y.3201]

IMT-2030 is a key area for study and standardization within ITU. ITU-R WP 5D has started the work on “IMT towards 2030 and beyond” by applying the well-proven IMT-process since 2021. Recommendation ITU-R M.2160-0 provides the usage scenarios of IMT-2030, which include immersive communication, hyper reliable and low-latency communication, massive communication, ubiquitous connectivity, artificial intelligence and communication, and integrated sensing and communication. Figure 6-2 depicts the usage scenarios and overarching aspects of IMT-2030 [ITU-R M.2160-0].

Figure 6-2 – Usage scenarios and overarching aspects of IMT-2030 [ITU-R M.2160-0]

ITU-T has also initiated the study and standardization work on non-radio aspects of IMT-2030. ITU-T TR.IMT2030-terms addresses the common terms and definitions for IMT-2030 networks to provide a general understanding and a guideline for the further development of IMT-2030 networks related documents. ITU-T TR.IMT2030-sec-con provides the analysis on the key driving force of security of IMT-2030; provides the analysis on the security challenges and issues based on the use cases and capabilities of IMT-2030; and provides the analysis on considerations of standardization work for security of IMT-2030.

With regard to FMSC, as a starting point, it is necessary to study the considerations on enhancements to support IMT-2030 networks in FMSC network, which will provide a general understanding and a guideline for FMSC related work items for IMT-2030 networks. This Technical Report studies the considerations on enhancements to support IMT-2030 networks in FMSC network, in the aspects of general considerations, considerations on use cases, considerations on requirements, considerations on framework, considerations on network function enhancements, considerations on procedures and considerations on security.

Figure 6-3 depicts the reference model of FMSC for IMT-2030 networks. In the reference model, the FMSC enhancements are made to the MUE, fixed/mobile/satellite access network, land-based/satellite-based core network, interworking function, land-based/satellite-based service plane and land-based/satellite-based management plane. The land-based/satellite-based core network consists of control plane, user plane, data plane, computing plane, security plane and AI/ML agent.

Figure 6-3 – Reference model of FMSC for IMT-2030 networks

[Editor's Note] The reference model of FMSC for IMT-2030 networks is for further discussion.

[Editor's Note] The fixed part of FMSC network needs further elaboration.

  - FMSC related mechanisms for IMT-2030 networks

### 6.2.1	Heterogeneous and lightweight network organization for FMSC

Given the usage scenarios of IMT-2030 and FMSC network environment, the heterogeneous and lightweight network organization is required for FMSC for IMT-2030 networks. On the basis of the FMSC framework specified in [ITU-T Y.3201] [ITU-T Y.3216], the mechanism to support the heterogeneous and lightweight network organization is needed by taking into account the characteristics of FMSC network, including the heterogeneous network entities, resources, capabilities, capacity, interfaces and procedures [ITU-T Y.3201].

### 6.2.2	Distributed network functionalities and networking for FMSC

Given the usage scenarios of IMT-2030 and FMSC network environment, the distributed network functionalities and networking are required for FMSC for IMT-2030 networks. On the basis of the FMSC network functions and networking methods specified in [ITU-T Y.3201] [ITU-T Y.3216], the mechanism to support the distributed network functionalities and networking is needed by taking into account the characteristics of FMSC network, including the large-scale and dynamic topology and organization [ITU-T Y.3201].

### 6.2.3	Network autonomy and robustness for FMSC

Given the usage scenarios of IMT-2030 and FMSC network environment, the network autonomy and robustness are required for FMSC for IMT-2030 networks. On the basis of the FMSC network organization methods and management and orchestration methods specified in [ITU-T Y.3201] [ITU-T Y.3216], the mechanism to support the network autonomy and robustness is needed by taking into account the characteristics of FMSC network, including the large-scale and dynamic topology and organization [ITU-T Y.3201].

### 6.2.4	Coordination of communications, sensing, computing and AI/ML for FMSC

Given the usage scenarios of IMT-2030 and FMSC network environment, the coordination of communications, sensing, computing and AI/ML is required for FMSC for IMT-2030 networks. On the basis of the FMSC framework specified in [ITU-T Y.3201] [ITU-T Y.3216], the mechanism to support the coordination of communications, sensing, computing and AI/ML is needed by taking into account the characteristics of FMSC network, including the heterogeneous network entities, resources, capabilities, capacity, interfaces and procedures [ITU-T Y.3201].

### 6.2.5	Efficient service provision for FMSC

Given the usage scenarios of IMT-2030 and FMSC network environment, the efficient service provision is required for FMSC for IMT-2030 networks. On the basis of the FMSC network functions and network services specified in [ITU-T Y.3201] [ITU-T Y.3216], the mechanism to support the efficient service provision is needed by taking into account the characteristics of FMSC network, including the limited networking, computing and storage resources [ITU-T Y.3201].

- Considerations on use cases of FMSC for IMT-2030 networks

[Editor's Note] This clause studies the considerations on use cases of FMSC, in the context of IMT-2030 networks. The text is for further discussion.

The considerations on use cases of FMSC for IMT-2030 networks include, but are not limited to, the follows:

Use case 1: Ubiquitous connectivity for IMT-2030 networks

In FMSC network for IMT-2030, the use case of ubiquitous connectivity for various types of UE is to be supported with the considerations on FMSC technologies. This use case is related to the IMT-2030 usage scenario "ubiquitous connectivity".

Use case 2: Multi-access convergence for IMT-2030 networks

In FMSC network for IMT-2030, the use case of multi-access convergence for fixed access, mobile access and satellite access is to be supported with the considerations on FMSC technologies. This use case is related to the IMT-2030 usage scenario "immersive communication" and "ubiquitous connectivity".

Use case 3: Space-air-ground integration for IMT-2030 networks

In FMSC network for IMT-2030, the use case of space-air-ground integration for land-based network, airborne network and satellite-based network is to be supported with the considerations on FMSC technologies. This use case is related to the IMT-2030 usage scenario "ubiquitous connectivity".

Use case 4: Distributed collaboration for IMT-2030 networks

In FMSC network for IMT-2030, the use case of distributed collaboration between land-based network and satellite-based network is to be supported with the considerations on FMSC technologies. This use case is related to the IMT-2030 usage scenario "immersive communication" and "hyper reliable and low-latency communication".

Use case 5: Introduction of sensing, computing and AI/ML for IMT-2030 networks

In FMSC network for IMT-2030, the use case of introduction of sensing, computing and AI/ML in support of usage scenarios of IMT-2030 is to be supported with the considerations on FMSC technologies. This use case is related to the IMT-2030 usage scenario "artificial intelligence and communication" and "integrated sensing and communication".

- Considerations on requirements of FMSC for IMT-2030 networks

[Editor's Note] This clause studies the considerations on requirements of FMSC, in the context of IMT-2030 networks, which include the considerations on general requirements, considerations on requirements of framework, considerations on requirements of network functions, and considerations on requirements of network performance.

  - Considerations on general requirements

[Editor's Note] The considerations on general requirements of FMSC for IMT-2030 networks are for further discussion.

The considerations on general requirements of FMSC for IMT-2030 networks are as follows:

- It is required for FMSC for IMT-2030 networks to support the usage scenarios of IMT-2030, including the ubiquitous connectivity [ITU-R M.2160-0].

- It is required for FMSC for IMT-2030 networks to support the general requirements of IMT-2030 [ITU-R M.2160-0].

- It is required for FMSC for IMT-2030 networks to support the general service requirements and general network capability requirements of FMSC for IMT-2020 networks and beyond [ITU-T Y.3200].

- It is required for FMSC for IMT-2030 networks to be compatible with UEs, access networks and core networks of IMT-2020 and IMT-2030.

- It is recommended for FMSC for IMT-2030 networks to support the FMSC related mechanisms, including the heterogeneous and lightweight network organization, distributed network functionalities and networking, network autonomy and robustness, coordination of communications, sensing, computing and AI/ML, and efficient service provision.

- It is required for FMSC for IMT-2030 networks to facilitate the introduction of new services and new enabling technologies.

- It is required for FMSC for IMT-2030 networks to optimize the use of networking, computing and storage resources.

- It is required for FMSC for IMT-2030 networks to optimize the energy efficiency of E2E communications network.

- It is required for FMSC for IMT-2030 networks to optimize the service continuity and quality of service (QoS).

  - Considerations on requirements of framework

[Editor's Note] The considerations on requirements of framework of FMSC for IMT-2030 networks are for further discussion.

The considerations on requirements of framework of FMSC for IMT-2030 networks are as follows:

- It is required for FMSC for IMT-2030 networks to make enhancements on the basis of FMSC framework specified in [ITU-T Y.3201].

- It is required for FMSC for IMT-2030 networks to make enhancements to the MUE, fixed/mobile/satellite access network, land-based/satellite-based core network, interworking function, land-based/satellite-based service plane and land-based/satellite-based management plane.

- It is required for FMSC for IMT-2030 networks to support the control plane, user plane, data plane, computing plane, security plane and AI/ML agent in core network.

- It is required for FMSC for IMT-2030 networks to enhance the coordination mechanism between land-based core network and satellite-based core network.

  - Considerations on requirements of network functions

The considerations on requirements of network functions of FMSC for IMT-2030 networks are as follows:

  - Considerations on requirements of network performance

The considerations on requirements of network performance of FMSC for IMT-2030 networks are as follows:

- Considerations on framework of FMSC for IMT-2030 networks

[Editor's Note] This clause studies the considerations on framework of FMSC, in the context of IMT-2030 networks, which include considerations on FMSC framework and considerations on reference points.

  - Considerations on FMSC framework

The considerations on FMSC framework for IMT-2030 networks is based on the reference model of FMSC for IMT-2030 networks, and specifies the FMSC network functions and reference points.

  - Considerations on reference points

[Editor's Note] The considerations on reference points of FMSC for IMT-2030 networks are for further discussion.

The considerations on reference points of FMSC for IMT-2030 networks are as follows:

- RP-cn1: The reference point for control plane of core network. RP-cn1 enables the use of control plane network capabilities.

- RP-cn2: The reference point for user plane of core network. RP-cn2 enables the use of user plane network capabilities.

- RP-cn3: The reference point for data plane of core network. RP-cn3 enables the use of data plane network capabilities.

- RP-cn4: The reference point for computing plane of core network. RP-cn4 enables the use of computing plane network capabilities.

- RP-cn5: The reference point for security plane of core network. RP-cn5 enables the use of security plane network capabilities.

- RP-cn6: The reference point for AI/ML agent of core network. RP-cn6 enables the use of AI/ML related capabilities.

- RP-ex1: The reference point exists between the MUE and the core network. RP-ex1 corresponds to the reference point RP-tn as defined in [ITU-T Y.3104].

- RP-ex2-1: The reference point exists between the fixed access network and the core network. RP-ex2-1 corresponds to the reference point RP-an as defined in [ITU-T Y.3104].

- RP-ex2-2: The reference point exists between the mobile access network and the core network. RP-ex2-2 corresponds to the reference point RP-an as defined in [ITU-T Y.3104].

- RP-ex2-3: The reference point exists between the satellite access network and the core network. RP-ex2-3 corresponds to the reference point RP-an as defined in [ITU-T Y.3104].

- RP-ex3: The reference point exists between the core network and the IMT-2020 core network. RP-ex3 corresponds to the interoperability interfaces as defined in [b-3GPP TS 23.501].

- RP-ex4: The reference point exists between the core network and the data network. RP-ex4 corresponds to the reference point RP-ud as defined in [ITU-T Y.3104].

- RP-ex5: The reference point exists between the core network and the service plane, including service invoking interfaces and capability exposure interfaces.

- RP-ex6: The reference point exists between the core network and the management plane, including, but not limited to, network function management interfaces, network connection management interfaces, resource management interfaces, capability management interfaces, and user management interfaces.

- Considerations on network function enhancements of FMSC for IMT-2030 networks

[Editor's Note] The considerations on network function enhancements to access network, core network, service plane and management plane are to be addressed. With regard to core network, the considerations on network function enhancements to control plane, user plane, data plane, computing plane, security plane and AI/ML agent are to be addressed. The text is for further discussion.

  - Considerations on network function enhancements to access network

The considerations on network function enhancements to access network of FMSC network for IMT-2030 are as follow:

  - Considerations on network function enhancements to core network

The considerations on network function enhancements to core network of FMSC network for IMT-2030 include the enhancements to control plane, user plane, data plane, computing plane, security plane and AI/ML agent, which are as follow:

  - Considerations on network function enhancements to service plane

The considerations on network function enhancements to service plane of FMSC network for IMT-2030 are as follow:

  - Considerations on network function enhancements to management plane

The considerations on network function enhancements to management plane of FMSC network for IMT-2030 are as follow:

- Considerations on procedures of FMSC for IMT-2030 networks

[Editor's Note] This clause studies the considerations on procedures of FMSC, in the context of IMT-2030 networks, which include considerations on procedures of FMSC related mechanisms and considerations on service procedures. The text is for further discussion.

  - Considerations on procedures of FMSC related mechanisms

The considerations on procedures of FMSC related mechanisms for IMT-2030 networks include the following:

- Procedure of heterogeneous and lightweight network organization for FMSC;

- Procedure of distributed network functionalities and networking for FMSC;

- Procedure of network autonomy and robustness for FMSC;

- Procedure of coordination of communications, sensing, computing and AI/ML for FMSC;

- Procedure of efficient service provision for FMSC.

  - Considerations on service procedures

The considerations on service procedures of FMSC for IMT-2030 networks include the registration management procedure, connection management procedure, session management procedure, handover procedure, capability exposure procedure, network slicing procedure, and multi-access convergence procedure. The above service procedures are based on the procedures specified in [ITU-T Y.3104], [b-ITU-T Y.3108], [b-ITU-T Y.3153], and [ITU-T Y.3201]; and they support the FMSC related mechanisms for IMT-2030 networks.

- Considerations on security of FMSC for IMT-2030 networks

[Editor's Note] This clause presents the considerations on security and privacy of FMSC, in the context of IMT-2030 networks, which include network security, service security, user privacy and service privacy. The text is for further discussion.

The considerations on security and privacy of FMSC for IMT-2030 networks include the following aspects:

- Network security, including the security considerations on the access network, core network, service plane and management plane of FMSC network, in which the core network consists of control plane, user plane, data plane, computing plane, security plane and AI/ML agent.

- Service security, including the security considerations on the communications services and third party services provided by FMSC network.

- User privacy, including the privacy considerations on the data plane, which is responsible for collecting, storing, caching and processing the privacy sensitive user data.

- Service privacy, including the privacy considerations on the service plane, which is responsible for collecting, storing, caching and processing the privacy sensitive service data.

In addition, the considerations on security and privacy of FMSC for IMT-2030 networks should be aligned with the requirements specified in [ITU-T TR.IMT2030-sec-con], [ITU-T Y.3200], [ITU-T Y.3101], and [b-ITU-T Y.2701].

# Bibliography

[b-ITU-T Y.2011]	Recommendation ITU-T Y.2011 (2004), General principles and general reference model for Next Generation Networks.

[b-ITU-T Y.2701]	Recommendation ITU-T Y.2701 (2007), Security requirements for NGN release 1.

[b-ITU-T Y.3061]	Recommendation ITU-T Y.3061 (2023), Autonomous Networks - Architecture framework.

[b-ITU-T Y.3090]	Recommendation ITU-T Y.3090 (2022), Digital twin network – Requirements and architecture.

[b-ITU-T Y.3100]	Recommendation ITU-T Y.3100 (2017), Terms and definitions for IMT-2020 network.

[b-ITU-T Y.3106]	Recommendation ITU-T Y.3106 (2019), Quality of service functional requirements for the IMT-2020 network.

[b-ITU-T Y.3108]	Recommendation ITU-T Y.3108 (2019), Capability exposure function in IMT-2020 networks.

[b-ITU-T Y.3144]	Recommendation ITU-T Y.3144 (2024), Future networks including IMT-2020 – Requirements and functional architecture of distributed core network.

[b-ITU-T Y.3153]	Recommendation ITU-T Y.3153 (2019), Network slice orchestration and management for providing network services to 3rd party in the IMT-2020 network.

[b-ITU-T Y.3202]	Recommendation ITU-T Y.3202 (2023), Fixed, mobile and satellite convergence – Mobility management for IMT-2020 networks and beyond.

[b-ITU-T Y.3203]	Recommendation ITU-T Y.3203 (2023), Fixed, mobile and satellite convergence – Connection management for IMT-2020 networks and beyond.

[b-ITU-T Y.3204]	Recommendation ITU-T Y.3204 (2023), Fixed, mobile and satellite convergence – Service continuity for IMT-2020 networks and beyond.

[b-ITU-T Y.3206]	Recommendation ITU-T Y.3206 (2023), Fixed, mobile and satellite convergence – Capability exposure for IMT-2020 networks and beyond.

[b-ITU-T Y.3300]	Recommendation ITU-T Y.3300 (2014), Framework of software-defined networking.

[b-ITU-T Y.4051]	Recommendation ITU-T Y.4051 (2019), Vocabulary for smart cities and communities.

[b-ITU-T Y.FMSC-SS]	Recommendation ITU-T Y.FMSC-SS, Fixed, mobile and satellite convergence – Service scheduling for IMT-2020 networks and beyond.

[b-ITU-T H.360]	Recommendation ITU-T H.360 (2004), An architecture for end-to-end QoS control and signalling.

[b-ITU-T L.1330]	Recommendation ITU-T L.1330 (2015), Energy efficiency measurement and metrics for telecommunication networks.

[b-ITU-T M.3080]	Recommendation ITU-T M.3080 (2021), Framework of artificial intelligence enhanced telecom operation and management (AITOM).

[b-ITU-T Q.1290]	Recommendation ITU-T Q.1290 (1998), Glossary of terms used in the definition of intelligent networks.

[b-ITU-R M.1645]	Recommendation ITU-R M.1645 (2003), Framework and overall objectives of the future development of IMT-2000 and systems beyond IMT-2000.

[b-3GPP TS 28.100]	3GPP TS 28.100 (2024), Management and orchestration; Levels of autonomous network (Release 18).

[b-3GPP TR 22.870]	3GPP TR 22.870 (2024), Study on 6G Use Cases and Service Requirements; Stage 1 (Release 20).

[b-3GPP TR 23.737]	3GPP TR 23.737 (2021), Study on architecture aspects for using satellite access in 5G (Release 17).

[b-ETSI GS NFV 003]	ETSI GS NFV 003 V1.4.1 (2018), Network Functions Virtualisation (NFV); Terminology for main concepts in NFV.

[b-ETSI GS ZSM 007]	ETSI GS ZSM 007 V2.1.1 (2023), Zero-touch network and Service Management (ZSM); Terminology for concepts in ZSM.

Annex A
A.13 justification for proposed draft new ITU-T TR.FMSC-IMT2030: “Fixed, mobile and satellite convergence – Considerations on enhancements to support IMT-2030 networks”

| Question: | Q23/13 | Proposed new ITU-T Technical report | Geneva, 3-14 March 2025 |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference and title: | ITU-T TR.FMSC-IMT2030 “Fixed, mobile and satellite convergence – Considerations on enhancements to support IMT-2030 networks” |  |  |  |  |
| Base text: | SG13-TD77/WP1 |  |  | Timing: | 2027-03 |
| Editor(s): | Nanxiang Shi, China Mobile, e-mail: shinanxiang@chinamobile.com Lu Lu, China Mobile, e-mail: lulu@chinamobile.com Yufeng He, China Telecom, e-mail: heyf@chinatelecom.cn Lin Lin, China Unicom, e-mail: linl12@chinaunicom.cn Hui Xu, CICT, e-mail: xuhui1@cictmobile.com Amirhossein Jafari Pozveh, MCI, e-mail: am.jafari@mci.ir |  |  | Approval process: | Agreement |
| Purpose and scope (Define what this document will address and its intent or objectives in order to indicate the limits of its applicability): |  |  |  |  |  |
| On the basis of the progress made by ITU-R and ITU-T on IMT-2030, including future technology trends [ITU-R M.2516-0], framework and overall objectives [ITU-R M.2160-0], considerations on terms and definitions [ITU-T TR.IMT2030-terms], considerations on security [ITU-T TR.IMT2030-sec-con], and considerations on signalling and protocol [ITU-T QSTR.SP-IMT2030], this Technical Report aims to study the considerations on enhancements to support IMT-2030 networks in fixed, mobile and satellite convergence (FMSC) network, focusing on the following aspects: - General considerations on FMSC for IMT-2030 networks, including the overview and FMSC related mechanisms. - Considerations on use cases of FMSC for IMT-2030 networks. - Considerations on requirements of FMSC for IMT-2030 networks, including the considerations on general requirements, considerations on requirements of framework, considerations on requirements of network functions, and considerations on requirements of network performance. - Considerations on framework of FMSC for IMT-2030 networks, including the considerations on FMSC framework and considerations on reference points. - Considerations on network function enhancements of FMSC for IMT-2030 networks, including the considerations on network function enhancements to access network, core network, service plane and management plane. - Considerations on procedures of FMSC for IMT-2030 networks, including the considerations on procedures of FMSC related mechanisms and considerations on service procedures. - Considerations on security of FMSC for IMT-2030 networks. |  |  |  |  |  |
| Summary (provides a brief overview of the proposal): |  |  |  |  |  |
| IMT-2030 is a key area for study and standardization within ITU, and significant progress on IMT-2030 have already been made in ITU-R and ITU-T. FMSC is the capability that provides services and applications to end users regardless of the fixed, mobile or satellite access technologies being used. As a starting point, it is necessary to study the considerations on enhancements to support IMT-2030 networks in FMSC network, which will provide a general understanding and a guideline for FMSC related work items for IMT-2030 networks. This work contributes to the development of the standardization series of FMSC and IMT-2030 networks; and is market valuable for network operators, service providers, and end users, since ubiquitous connectivity is one of the key usage scenarios of IMT-2030. This Technical Report studies the considerations on enhancements to support IMT-2030 networks in FMSC network, in the aspects of general considerations, considerations on use cases, considerations on requirements, considerations on framework, considerations on network function enhancements, considerations on procedures and considerations on security. |  |  |  |  |  |
| Relations to ITU-T Recommendations or other documents (approved or under development): |  |  |  |  |  |
| ITU-T Y.3101, ITU-T Y.3102, ITU-T Y.3104, ITU-T Y.3114, ITU-T Y.3172, ITU-T Y.3200, ITU-T Y.3201, ITU-T Y.3216, ITU-T TR.IMT2030-terms, ITU-T TR.IMT2030-sec-con, ITU-T QSTR.SP-IMT2030, ITU-T Y.supp.fmsc-roadmap, ITU-R M.2160-0, ITU-R M.2460-0, ITU-R M.2516-0 |  |  |  |  |  |
| Liaisons with other study groups or with other standards bodies: |  |  |  |  |  |
| ITU-T SG2, ITU-T SG11, ITU-T SG17, ITU-R WP4B, ITU-R WP5D, 3GPP SA1, 3GPP SA2, ETSI SES |  |  |  |  |  |
| Supporting members that are committing to contributing actively to the work item: |  |  |  |  |  |
| China Mobile, China Telecom, China Unicom, CICT, BUPT, MCI, IUST, Tunisie Telecom |  |  |  |  |  |

_______________________