|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  | SG13-TD54/WP1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 13 |  |
|  |  |  | Original: English |  |
| Question(s): |  | 20/13 | Geneva, 3-14 March 2025 |  |
| TD |  |  |  |  |
| Source: |  | Editors |  |  |
| Title: |  | Draft new Technical Report ITU-T TR.IMT2030-terms: “IMT-2030 networks – Considerations on terms and definitions” |  |  |
| Contact: |  | Nanxiang Shi China Mobile China |  | Tel: +86 13699291776 E-mail: shinanxiang@chinamobile.com |
| Contact: |  | Lu Lu China Mobile China |  | Tel: +86 13910556279 E-mail: lulu@chinamobile.com |
| Contact: |  | Huan Deng China Telecom China |  | Tel: +86 10 50902867 E-mail: denghuan@chinatelecom.cn |
| Contact: |  | Xueqin Jia China Unicom China |  | Tel: +86 1068799999 E-mail: jiaxq21@chinaunicom.cn |
| Contact: |  | Silva Humberto ANATEL Brazil |  | E-mail: humbertobruno@anatel.gov.br |

| Abstract: | This document is the output of draft new Technical Report ITU-T TR.IMT2030-terms: “IMT-2030 networks – Considerations on terms and definitions”, resulting from discussion of Q20/13 meeting on 3-14 March 2025. |
| --- | --- |

This document is the output of draft new Technical Report ITU-T TR.IMT2030-terms: “IMT-2030 networks – Considerations on terms and definitions”, resulting from discussion of Q20/13 meeting on 3-14 March 2025.

This document is based on SG13-TD14/WP1, the output of Q20/13 meeting on 9-12 December 2024, and according to this meeting's discussion and results on the following contribution:

| No. | Title | Source | Discussion |
| --- | --- | --- | --- |
| SG13-C146 | TR.IMT2030-terms: Proposal for updating definitions and abbreviations of IMT-2030 networks | China Mobile Communications Co. Ltd. | Accepted with modifications |

Draft new Technical Report ITU-T TR.IMT2030-terms

IMT-2030 networks – Considerations on terms and definitions

Summary

This Technical Report aims to study the common terms and definitions for IMT-2030 networks to provide a general understanding for IMT-2030 networks related documents. It can be used as a guideline for the further development of IMT-2030 networks related documents.

This Technical Report considers the terms already been defined and the new terms, which are considered to have some relationship with IMT-2030 networks.

Keywords

Definitions, IMT-2030, terms

Table of Contents

Page

1. Scope	4

2. Criteria for terms considered in this Technical Report	4

3. References	4

4. Definitions	5

4.1. Terms defined elsewhere	5

4.2. Terms considered in this Technical Report	8

5. Abbreviations and acronyms	12

6. Conventions	13

Bibliography	14

Draft new Technical Report ITU-T TR.IMT2030-terms

IMT-2030 networks – Considerations on terms and definitions

- Scope

This Technical Report aims to study the common terms and definitions for IMT-2030 networks to provide a general understanding for IMT-2030 networks related documents and a guideline for the further development of IMT-2030 networks related documents.

This Technical Report provides terms and definitions, which are considered particularly suitable and applicable to the development of IMT-2030 documents, and have already been defined in published ITU-T Recommendations and other relevant specifications. New terms are also considered as required according to the identified criteria.

- Criteria for terms considered in this Technical Report

[Editor's Note] The criteria for terms considered in this Technical Report is for further consideration.

The criteria for terms considered in this Technical Report is as follows:

- The term being considered is expected to be a “common” term (i.e., of common (not specific) usage) for IMT-2030 networks related documents (in the context of ITU-T documents, with their usage in at least two ITU-T Study Groups).

NOTE 1 – The radio aspects of IMT-2030 are out of the scope.

NOTE 2 – The term being considered has some relationship with the trends, technologies, enablers, usage scenarios, capabilities, requirements, framework, services and applications, etc. that have been identified in the ITU-R Recommendations and Reports on IMT-2030. By the end of 2023, the ITU-R Recommendations and Reports on IMT-2030 include [ITU-R M.2516-0] and [ITU-R M.2160-0].

NOTE 3 – The term being considered has a value of international standardization.

- References

[Editor's Note] The list of references is for further consideration.

[ITU-T Y.3090]	Recommendation ITU-T Y.3090 (2022), Digital twin network – Requirements and architecture.

[ITU-T Y.3100]	Recommendation ITU-T Y.3100 (2017), Terms and definitions for IMT-2020 network.

[ITU-T Y.3106]	Recommendation ITU-T Y.3106 (2019), Quality of service functional requirements for the IMT-2020 network.

[ITU-T Y.3114]	Recommendation ITU-T Y.3114 (2022), Future networks including IMT-2020: requirements and functional architecture of lightweight core for dedicated networks.

[ITU-T Y.3172]	Recommendation ITU-T Y.3172 (2019), Architectural framework for machine learning in future networks including IMT-2020.

[ITU-T Y.3200]	Recommendation ITU-T Y.3200 (2022), Fixed, mobile and satellite convergence – Requirements for IMT-2020 networks and beyond.

[ITU-T Y.3400]	Recommendation ITU-T Y.3400 (2023), Coordination of networking and computing in IMT-2020 networks and beyond – Requirements.

[ITU-T Y Sup. 48]	Supplement 48 to ITU-T Y-series Recommendations (2018), Proof-of-concept for data service using information centric networking in IMT-2020.

[ITU-R M.2160-0]	Recommendation ITU-R M.2160-0 (2023), Framework and overall objectives of the future development of IMT for 2030 and beyond.

[ITU-R M.2516-0]	Recommendation ITU-R M.2516-0 (2022), Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond.

- Definitions

  - Terms defined elsewhere

[Editor's Note] The list of terms defined elsewhere is for further consideration.

This Technical Report uses the following “common” terms defined elsewhere, which are considered to have some relationship with IMT-2030 networks:

4.1.1	application domain [b-ITU-T Y.4100]: An area of knowledge or activity applied for one specific economic, commercial, social or administrative scope.

4.1.2	artificial intelligence (AI) [b-ITU-T M.3080]: Computerized system that uses cognition to understand information and solve problems.

[Editor's Note] Other definitions of AI exist, see "Glossary of terms and definitions for machine learning" (JCA-ML-Doc-082). The definition is to be revisited.

4.1.3	assurance [b-ITU-T X.1500]: The degree of confidence that the process or deliverable meets defined characteristics or objectives.

4.1.4	autonomous network [b-3GPP TS 28.100]: Telecommunication system (including management system and network) with autonomy capabilities which is able to be governed by itself with minimal to no human intervention.

[Editor's Note] In Y.3060, "autonomous network" is a term defined elsewhere (using the definition in [3GPP TS 28.100]). The definition is to be revisited.

4.1.5	backhaul [ITU-T Y.3100]: A network path between base station systems and a core network.

4.1.6	blockchain [b-ITU-T X.1400]: A type of distributed ledger which is composed of digitally recorded data arranged as a successively growing chain of blocks with each block cryptographically linked and hardened against tampering and revision.

4.1.7	capability level [b-ITU-T Y.3119]: The level of availability of capabilities in a network.

4.1.8	certification authority (CA) [b-ITU-T Y.4500.3]: Authority responsible for establishing and vouching for the authenticity of public keys.

NOTE – This includes binding public keys to distinguished names through signed certificates, managing certificate serial numbers, and certificate revocation.

4.1.9	cloud computing [b-ITU-T Y.3500]: Paradigm for enabling network access to a scalable and elastic pool of shareable physical or virtual resources with self-service provisioning and administration on-demand.

NOTE – Examples of resources include servers, operating systems, networks, software, applications, and storage equipment.

4.1.10	dedicated network [ITU-T Y.3114]: A network designed for application domains with common requirements.

4.1.11	digital twin network [ITU-T Y.3090]: A virtual representation of a physical network. It is useful for analysing, diagnosing, emulating and controlling the physical network based on data, model and interface, to achieve the real-time interactive mapping between the physical network and virtual twin network.

4.1.12	energy efficiency (EE) [b-ITU-T L.1330]: The relation between the useful output and energy/power consumption.

4.1.13	federated machine learning (FML) [b-ITU-T M.3387]: A machine learning framework that facilitates collaborative construction of machine learning models among multiple distributed training nodes without exposing the private data owned by the data owners.

4.1.14	fixed mobile convergence (FMC) [ITU-T Y.3100]: In the context of IMT-2020, the capabilities that provide services and applications to end users regardless of the fixed or mobile access technologies being used and independently of the users' location.

4.1.15	fixed, mobile and satellite convergence (FMSC) [ITU-T Y.3200]: The capabilities that provide services and applications to end users regardless of the fixed, mobile or satellite access technologies being used independently of the users' location.

4.1.16	fronthaul [ITU-T Y.3100]: A network path between centralized radio controllers and remote radio units of a base station function.

4.1.17	information centric networking (ICN) [ITU-T Y Sup. 48]: A new approach to networking where named objects (not only devices) are the principal components for the network. Named data objects can be stored in network nodes (with caching capability) distributed throughout the network. Data objects are transmitted by using names to requesting consumers from any network node that can provide requested data. Locations of the nodes that store data objects in their caches are irrelevant to consumers because they send their requests for data objects by using names (not the data object locations).

4.1.18	integrated network control system [b-ITU-T Y.3207]: A network control system performing the integrated control of multi-technology networks. It is responsible for the design, orchestration, monitoring, and control of network services (NSs) offered combinedly by terrestrial networks (TNs) and non-terrestrial networks (NTNs).

NOTE – It has the capabilities of obtaining monitoring data of NSs from both TNs and NTNs, processing and analysing the data to assess the current status or predict future status of the network resource utilization and service performances, formulating the NS control decisions, and setting parameters values of control actions to be executed on the components of both TNs and NTNs.

4.1.19	IMT-2020 [ITU-T Y.3100]: Systems, system components, and related technologies that provide far more enhanced capabilities than those described in [b-ITU-R M.1645].

NOTE – [b-ITU-R M.1645] defines the framework and overall objectives of the future development of IMT-2000 and systems beyond IMT-2000 for the radio access network.

4.1.20	key performance indicator (KPI) [b-ITU-T Y.4051]: Indicator representing the resource usage effectiveness or efficiency of a given system.

4.1.21	knowledge [b-ETSI GS ENI 005]: Analysis of data and information, resulting in an understanding of what the data and information mean.

NOTE – Knowledge represents a set of patterns that are used to explain, as well as predict, what has happened, is happening, or is possible to happen in the future; it is based on acquisition of data, information, and skills through experience and education.

4.1.22	machine learning (ML) [ITU-T Y.3172]: Processes that enable computational systems to understand data and gain knowledge from it without necessarily being explicitly programmed.

4.1.23	machine learning overlay [ITU-T Y.3172]: A loosely coupled deployment model of machine learning functionalities whose integration and management with network functions are standardized.

4.1.24	management plane [b-ITU-T Y.2011]: The set of functions used to manage entities in the stratum or layer under consideration, plus the functions required to support this management.

4.1.25	multi-access user equipment (MUE) [b-ITU-T Y.FMSC-SS]: A user equipment which can support accessing two or more access networks simultaneously under the control of a network enhanced for multi-access capability.

4.1.26	network functions virtualization (NFV) [b-ETSI GS NFV 003]: Principle of separating network functions from the hardware they run on by using virtual hardware abstraction.

4.1.27	network intelligence level [b-ITU-T Y.3173]: Level of application of automation capabilities including those enabled by the integration of artificial intelligence techniques in the network.

4.1.28	network slice [ITU-T Y.3100]: A logical network that provides specific network capabilities and network characteristics.

4.1.29	network softwarization [ITU-T Y.3100]: An overall approach for designing, implementing, deploying, managing and maintaining network equipment and/or network components by software programming.

4.1.30	protocol data unit (PDU) session [ITU-T Y.3100]: In the context of IMT-2020, an association between a user equipment (UE) and a data network that provides a protocol data unit (PDU) connectivity service.

4.1.31	quality of experience (QoE) [b-ITU-T P.915]: The degree of satisfaction of the user of an application or service. It results from the fulfilment of his or her expectations with respect to the utility or enjoyment of the application or service in the light of the user's personality and current state.

NOTE – Definition based on that appearing in Qualinet-2014.

4.1.32	quality of service (QoS) [b-ITU-T Q.1743]: The collective effect of service performances, which determine the degree of satisfaction of a user of a service. It is characterized by the combined aspects of performance factors applicable to all services, such as:

–	service operability performance;

–	service accessibility performance;

–	service retainability performance;

–	service integrity performance; and

–	 other factors specific to each service.

4.1.33	service continuity [b-ITU-T Y.3204]: The ability for a moving object to maintain ongoing service, including maintaining current states such as the user network environment and session.

4.1.34	service level agreement (SLA) [ITU-T Y.3106]: A formal agreement between two or more entities reached after a negotiating activity with the scope to assess service characteristics, responsibilities and priorities of every part. A SLA may include statements about performance, billing, service delivery but also legal and economic issues.

4.1.35	service level objective (SLO) [b-ETSI GS ZSM 007]: Element in a service level specification that is defined in terms of parameters, and related metrics, thresholds and tolerances associated with the parameters.

4.1.36	self-organizing core network (SOCN) [b-ITU-T Y.3127]: A core network constituted by a group of self-organized network entities cooperating to provide core network functions based on the available network capabilities and resources.

4.1.37	service plane [b-ITU-T Q.1290]: The plane in the unified functional model that describes services.

4.1.38	software-defined networking (SDN) [b-ITU-T Y.3300]: A set of techniques that enables to directly program, orchestrate, control and manage network resources, which facilitates the design, delivery and operation of network services in a dynamic and scalable manner.

4.1.39	sustainability [b-ITU-T Y.4051]: State of the global system, including environmental, social and economic aspects, in which the needs of the present are met without compromising the ability of future generations to meet their own needs.

4.1.40	transport network [b-ITU-T G.805]: The functional resources of the network which conveys user information between locations.

4.1.41	trust execution environment (TEE) [b-ITU-T X.1379]: An environment in which any code execution is trusted in authenticity and integrity, the assets are protected in confidentiality, and both assets and code are protected from unauthorized tracing and control through debug and test features.

NOTE – Adapted from [b-GPD SPE 009].

4.1.42	third party (3rd party) [ITU-T Y.3100]: In the context of IMT-2020, with respect to a given network operator and network end-users, an entity which consumes network capabilities and/or provides applications and/or services.

4.1.43	user equipment (UE) [b-ITU-T H.360]: Equipment under the control of an End User.

  - Terms considered in this Technical Report

[Editor's Note] The list of new terms considered in this Technical Report is for further consideration.

This Technical Report considers the following new “common” terms, which are considered to have some relationship with IMT-2030 networks:

4.2.1	access network (AN): TBD.

[Editor's Note] The current status of discussion on this definition is the following: A network that connects user equipment (UE) to the communications network, and provides network services. NOTE – It includes, but is not limited to, fixed access network, mobile access network, and satellite access network.

4.2.2	AI agent (AIA): TBD.

4.2.3	artificial intelligence (AI) and communication: A usage scenario that supports distributed computing and AI applications.

NOTE – Based on the IMT-2030 usage scenario "Artificial Intelligence and Communication" [ITU-R M.2160-0].

4.2.4	artificial intelligence (AI) native: TBD.

4.2.5	computing plane: TBD.

[Editor's Note] The current status of discussion on this definition is the following: A pool of computing related network functionalities which enable computing functions such as computing enforcement, computing resource management, and computing task management.

4.2.6	computing resources: TBD.

4.2.7	control plane (CP): TBD.

[Editor's Note] The current status of discussion on this definition is the following: A pool of control related network functionalities which enable control functions such as communications control, sensing control, computing control, artificial intelligence (AI) / machine learning (ML) control, security control, energy efficiency control, and communications services.

4.2.8	coordination of networking and computing (CNC): The coordination of utilization, control and management of networking, computing and storage resources.

NOTE 1 – By the application of CNC for the purposes of provisioning and optimization, satisfaction of requirements of resources users and improvement of resource utilization may be achieved.

NOTE 2 – Based on [ITU-T Y.3400].

4.2.9	core network (CN): TBD.

[Editor's Note] The current status of discussion on this definition is the following: A network that aggregates access networks, connects to the data network, and provides network services.

4.2.10	data network (DN): TBD.

[Editor's Note] The current status of discussion on this definition is the following: A network that connects communications network to the Internet and/or third party services.

4.2.11	data plane (DP): TBD.

[Editor's Note] The current status of discussion on this definition is the following: A pool of data related network functionalities which enable data functions such as data storage management, data agent, data orchestration, data services, and sensing services.

4.2.12	deterministic networking: TBD.

4.2.13	distributed autonomous network: TBD.

4.2.14	hyper reliable and low-latency communication: A usage scenario that extends the Ultra-Reliable and Low-Latency Communication (URLLC) of IMT-2020 and covers specialized use cases that are expected to have more stringent requirements on reliability and latency.

NOTE – Based on the IMT-2030 usage scenario "Hyper Reliable and Low-Latency Communication" [ITU-R M.2160-0].

4.2.15	immersive communication: A usage scenario that extends the enhanced Mobile Broadband (eMBB) of IMT-2020 and covers use cases which provide a rich and interactive video (immersive) experience to users, including the interactions with machine interfaces.

NOTE – Based on the IMT-2030 usage scenario "Immersive Communication" [ITU-R M.2160-0].

4.2.16	immersive multimedia: TBD.

4.2.17	IMT-2030: Systems, networks, services and related technologies that support the coordination of communications, sensing, computing, artificial intelligence (AI) and security, and provide the capabilities as described in [ITU-R M.2160-0].

NOTE – [ITU-R M.2160-0] defines the framework and overall objectives of the future development of IMT for 2030 and beyond.

4.2.18	integrated sensing and communication: A usage scenario that facilitates new applications and services that require sensing capabilities, offering wide area multi-dimensional sensing that provides spatial information about unconnected objects as well as connected devices and their movements and surroundings.

NOTE – Based on the IMT-2030 usage scenario "Integrated Sensing and Communication" [ITU-R M.2160-0].

4.2.19	massive communication: A usage scenario that extends massive Machine Type Communication (mMTC) of IMT-2020 and involves connection of massive number of devices or sensors for a wide range of use cases and applications.

NOTE – Based on the IMT-2030 usage scenario "Massive Communication" [ITU-R M.2160-0].

4.2.20	multi-sensory interaction: TBD.

4.2.21	network complexity: TBD.

[Editor's Note] The current status of discussion on this definition is the following: A measurement of the degree of order for the elements of a specific network. NOTE – The elements for measurement may include, but are not limited to, the number of network nodes, number of network connections, number of network functions, number of services, number of interfaces, number of protocols, number of procedures, signalling overhead, type of heterogeneous networks, networking, computing and storage resource attributes, and energy efficiency.

4.2.22	network function (NF): TBD.

[Editor's Note] The current status of discussion on this definition is the following: A processing function in a network, which has defined functional behaviour and defined interfaces.

4.2.23	network service (NS): TBD.

[Editor's Note] The current status of discussion on this definition is the following: A communication service or an information service, which is provided by network functions or a network.

4.2.24	network sharing: TBD.

[Editor's Note] The current status of discussion on this definition is the following: The sharing of part(s) of or all of a network among operators and/or users. NOTE – The access network, core network, service plane and management plane of a network can be used for network sharing.

4.2.25	networking resources: TBD.

4.2.26	security plane: TBD.

[Editor's Note] The current status of discussion on this definition is the following: A pool of security related network functionalities which enable security functions such as security and trust capabilities, security and trust policies, and security and trust services.

4.2.27	service based architecture (SBA): TBD.

[Editor's Note] The current status of discussion on this definition is the following: An architecture in which the interaction between network functions is represented as services, i.e., a network function enables other authorized network functions to access its services.

4.2.28	service based interface (SBI): TBD.

[Editor's Note] The current status of discussion on this definition is the following: An interface that represents how a set of services is provided/exposed by a given network function.

4.2.29	space and terrestrial integration: TBD.

4.2.30	storage resources: TBD.

4.2.31	ubiquitous computing: TBD.

4.2.32	ubiquitous connectivity: A usage scenario that is intended to enhance connectivity with the aim to bridge the digital divide, in which connectivity could be enhanced, inter alia, through interworking with other systems.

NOTE – Based on the IMT-2030 usage scenario "Ubiquitous Connectivity" [ITU-R M.2160-0].

4.2.33	ubiquitous intelligence: TBD.

4.2.34	user plane (UP): TBD.

[Editor's Note] The current status of discussion on this definition is the following: A pool of user related network functionalities which enable user functions such as traffic processing, service processing, and policy enforcement.

- Abbreviations and acronyms

This Technical Report uses the following abbreviations and acronyms:

AI		Artificial Intelligence

AIA		AI Agent

AN		Access Network

CA		Certification Authority

CN		Core Network

CNC		Coordination of Networking and Computing

CP		Control Plane

DN		Data Network

DP		Data Plane

EE		Energy Efficiency

eMBB		enhanced Mobile Broadband

FMC		Fixed Mobile Convergence

FML		Federated Machine Learning

FMSC		Fixed, Mobile and Satellite Convergence

ICN		Information Centric Networking

IMT		International Mobile Telecommunications

KPI		Key Performance Indicator

ML		Machine Learning

mMTC		massive Machine Type Communication

MUE		Multi-access User Equipment

NF		Network Function

NFV		Network Functions Virtualization

NS		Network Service

NTN		Non-Terrestrial Network

PDU		Protocol Data Unit

QoE		Quality of Experience

QoS		Quality of Service

SBA		Service Based Architecture

SBI		Service Based Interface

SDN		Software-Defined Networking

SLA		Service Level Agreement

SLO		Service Level Objective

SOCN		Self-Organizing Core Network

TEE		Trust Execution Environment

TN		Terrestrial Network

UE		User Equipment

UP		User Plane

URLLC	Ultra-Reliable and Low-Latency Communication

- Conventions

None.

# Bibliography

[Editor's Note] The list of bibliography is for further consideration.

[b-ITU-T Y.2011]	Recommendation ITU-T Y.2011 (2004), General principles and general reference model for Next Generation Networks.

[b-ITU-T Y.3061]	Recommendation ITU-T Y.3061 (2023), Autonomous Networks - Architecture framework.

[b-ITU-T Y.3119]	Recommendation ITU-T Y.3119 (2023), Future networks including IMT-2020 – Capability classification framework for dedicated networks.

[b-ITU-T Y.3127]	Recommendation ITU-T Y.3127 (2023), Future networks including IMT-2020 – Requirements and framework for self-organizing core network.

[b-ITU-T Y.3173]	Recommendation ITU-T Y.3173 (2020), Framework for evaluating intelligence levels of future networks including IMT-2020.

[b-ITU-T Y.3204]	Recommendation ITU-T Y.3204 (2023), Fixed, mobile and satellite convergence – Service continuity for IMT-2020 networks and beyond.

[b-ITU-T Y.3207]	Recommendation ITU-T Y.3207 (2024), Fixed, mobile and satellite convergence – Integrated network control architecture framework for IMT-2020 networks and beyond.

[b-ITU-T Y.3300]	Recommendation ITU-T Y.3300 (2014), Framework of software-defined networking.

[b-ITU-T Y.3500]	Recommendation ITU-T Y.3500 (2014), Information technology – Cloud computing – Overview and vocabulary.

[b-ITU-T Y.4051]	Recommendation ITU-T Y.4051 (2019), Vocabulary for smart cities and communities.

[b-ITU-T Y.4100]	Recommendation ITU-T Y.4100/Y.2066 (2014), Common requirements of the Internet of things.

[b-ITU-T Y.4500.3]	Recommendation ITU-T Y.4500.3 (2023), oneM2M – Security solutions.

[b-ITU-T Y.FMSC-SS]	Recommendation ITU-T Y.FMSC-SS, Fixed, mobile and satellite convergence – Service scheduling for IMT-2020 networks and beyond.

[b-ITU-T G.805]	Recommendation ITU-T G.805 (2000), Generic functional architecture of transport networks.

[b-ITU-T H.360]	Recommendation ITU-T H.360 (2004), An architecture for end-to-end QoS control and signalling.

[b-ITU-T L.1330]	Recommendation ITU-T L.1330 (2015), Energy efficiency measurement and metrics for telecommunication networks.

[b-ITU-T M.3080]	Recommendation ITU-T M.3080 (2021), Framework of artificial intelligence enhanced telecom operation and management (AITOM).

[b-ITU-T M.3387]	Recommendation ITU-T M.3387 (2024), Management requirements for federated machine learning systems.

[b-ITU-T P.915]	Recommendation ITU-T P.915 (2016), Subjective assessment methods for 3D video quality.

[b-ITU-T Q.1290]	Recommendation ITU-T Q.1290 (1998), Glossary of terms used in the definition of intelligent networks.

[b-ITU-T Q.1743]	Recommendation ITU-T Q.1743 (2016), IMT-Advanced references to Release 11 of LTE-Advanced evolved packet core network.

[b-ITU-T X.1379]	Recommendation ITU-T X.1379 (2022), Security requirements for roadside units in intelligent transportation systems.

[b-ITU-T X.1400]	Recommendation ITU-T X.1400 (2020), Terms and definitions for distributed ledger technology.

[b-ITU-T X.1500]	Recommendation ITU-T X.1500 (2011), Overview of cybersecurity information exchange.

[b-ITU-R M.1645]	Recommendation ITU-R M.1645 (2003), Framework and overall objectives of the future development of IMT-2000 and systems beyond IMT-2000.

[b-3GPP TS 28.100]	3GPP TS 28.100 (2024), Management and orchestration; Levels of autonomous network (Release 18).

[b-ETSI GS ENI 005]	ETSI GS ENI 005 V3.1.1 (2023), Experiential Networked Intelligence (ENI); System Architecture.

[b-ETSI GS NFV 003]	ETSI GS NFV 003 V1.4.1 (2018), Network Functions Virtualisation (NFV); Terminology for main concepts in NFV.

[b-ETSI GS ZSM 007]	ETSI GS ZSM 007 V2.1.1 (2023), Zero-touch network and Service Management (ZSM); Terminology for concepts in ZSM.

[b-GPD SPE 009]		GPD_SPE_009 (2011), GlobalPlatform Device Technology, TEE System Architecture.

<https://globalplatform.org/specs-library/tee-system-architecture/>

_______________________