|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  | SG11-TD722/GEN |
| --- | --- | --- | --- |
|  |  |  | STUDY GROUP 11 |
|  |  |  | Original: English |
| Question(s): |  | 2/11 | Geneva, 17-26 November 2025 |
| TD |  |  |  |
| Source: |  | Editors |  |
| Title: |  | Output – draft new Recommendation Q.TSCA “Requirements for issuing End-Entity and Certification Authority public-key certificates for enabling trustable signalling interconnection between network entities” (Geneva, 17-26 November 2025) |  |
| Contact: |  | Klinger Assaf Vaulto Communication Technologies Israel | Tel: +972 50 2138133 E-mail: assaf.klinger@gmail.com |
| Contact: |  | Minrui Shi China Telecom China | Tel: +86 189 18588657 E-mail: shimr@chinatelecom.cn |

| Abstract: | This TD is the output of draft Recommendation ITU-T Q.TSCA: Requirements for issuing End-Entity and Certification Authority public-key certificates for enabling trustable signaling interconnection between network entities. (Geneva, 17-26 November 2025). |
| --- | --- |

Summary

This is the output of draft Recommendation ITU-T Q.TSCA: Requirements for issuing End-Entity and Certification Authority public-key certificates for enabling trustable signalling interconnection between network entities, based on the discussion results on C202 and C256 at the Q2/11 meeting (Geneva, 17-26 November 2025).

Draft new Recommendation ITU-T Q.TSCA

Requirements for issuing End-Entity and Certification Authority public-key certificates for enabling trustable signalling interconnection between network entities

Summary

This draft recommendation ITU-T Q.TSCA details the requirements for validation of Certificate Signing Requests (CSR) received by the TSCA from CSPs and the requirements for issuance of end-entity and certification authority public-key certificates (in accordance with [ITU-T X.509]) to prevent access of bad actors to the international signalling exchange networks.

Keywords

Authentication, trust, signalling procedures, certification authority, digital signature, registration authority, data integrity, public key certificate, end entity public key certificate, CA certificate.

Introduction

Communications service providers (CSPs) have traditionally owned network infrastructure, including access networks, core networks and service networks. User equipment (UE) is connected though a user-network interface (UNI) which is regarded as untrusted party by network. Many security requirements have been considered and satisfied on the UNI, such as an authentication and key agreement (AKA) mechanism which provides authentication and authorization. The core network entities of the CSP are connected through a Network-to-Network Interface (NNI) and the relations between the network entities are regarded as trusted based on the closeness and isolation of the network and since they reside within the same security domain managed by the CSP.

Connection between network entities of different CSPs was also considered trusted solely on the premise of a “walled garden”, since the international signalling interconnect networks are segregated from public networks such as the internet and connectivity to the international signalling interconnect networks are controlled by national and/or regional telecom regulators and CSPs are licensed business entities, rather than implementation of security technologies within the international signalling interconnect networks.

Thus, technological security measures and policies on NNI are not implemented (see b-QSTR-SS7-DFS - SS7) due to that premise. Nowadays, the international signalling interconnect networks are much more open, due to the emergence of Over-The-Top (OTT) services such as Digital Financial Services (DFS), eSIM, VoWiFI and virtual CSPs which relay on the underlying global international signalling interconnect networks and the premise of a “walled garden” no longer applies. The following figure illustrates in broad strokes the current connectivity of CSPs to the international signalling exchange networks:

Figure 1 - current connectivity of CSPs to the international signalling exchange networks

The international signalling interconnect networks are accessed through various interfaces and protocols which are used for NNI, e.g., session initiation protocol (SIP) (4G/5G/VoIP), signalling system No. 7 (SS7) (2G/3G/PSTN), GPRS Tunnelling Protocol (GTP) (2G/3G/4G/5G), Diameter (4G/5G-Non-Stand Alone) and Hyper Text Transfer Protocol 2 (HTTP2) (5G Stand Alone). With many unlicensed and unregulated OTT service providers given access via licensed CSPs.

Under such circumstances, signalling for controlling and management is abused, resulting in theft or fraud of subscribers’ sensitive information, robocalls, social engineering using spoofed calls, spoofed short message service (SMS) – calls/SMS from parties pretending to be legitimate business ventures (e.g., representatives of banks, health insurance companies, etc.) in which calling party number is replaced with the number of an official enterprise or anyone of trust, and subscriber location tracking. Furthermore, SMS and calls of users are susceptible to illegal surveillance. These are just a few examples of telecom vulnerably exploitation using signalling protocols which are integral part of international signaling exchange networks. More information on these currently exploited telecom vulnerabilities can be found in [b-QSTR-SS7-DFS - SS7].

To mitigate these threats a framework of recommendations and technical reports was created by ITU-T, with the main objective to adept the existing PKI security controls which were created for the Internet to the international signalling exchange networks, the framework is comprised from:

- [ITU-T Q.3057]: describes the architecture for interconnection between trustable network entities, details the different possible trust models in accordance with [ITU-T X.509] and defines the Trusted Signalling Certification Authority (TSCA).

- [ITU-T Q.3062]: describes the signalling procedures and protocols for enabling interconnection between trustable network entities, details the implementation of end-entity and CA public-key certificates into the different signalling protocols used in the international signalling exchange networks.

- [ITU-T Q.3063]: describes the signalling procedures and protocols for CLI authentication between different CSPs to mitigate spoofed calls and robocalls.

- [b-ITU-T QSTR-SS7-DFS]: a technical report detailing the currently exploited telecom vulnerabilities and surveys the different mitigation measures available for CSPs and telecom regulators.

- [b-ITU-T QSTR-USSD]: a technical report detailing the vulnerabilities of Digital Financial Services (DFS) provided over USSD, which are dominant in developing countries and explores the applicability of implementing modern security controls for this legacy communication protocol.

During the creation of this framework, several workshops and webinars were held with relevant stakeholders to collaborate the work done by ITU-T with requirements from CSPs, regulators, DFS providers and other stakeholders in the telecom sector to ensure the interoperability of the framework with other signalling security initiatives, in particular the adoption of STIR/SHAKEN by the Federal Communication Commission (FCC).

Table of Contents

1.	Scope	6

2.	References	6

3.	Definitions	6

3.1.	Terms defined elsewhere:	6

3.2.	Terms defined in this Recommendation:	8

4.	Abbreviations and acronyms	8

5.	Conventions	9

6.	Stakeholders in the signalling trust chain	9

7.	Requirements for issuing End-Entity and CA public-key certificates	10

7.1.	Requirement for IE verification for the issuance of a PEEC	12

8.	Signalling requirements for retrieval of Assignment Record information	12

8.1.	Assignment Record structure	12

8.2.	Information-exchange overview	13

8.3.	Purpose of Assignment Record retrieval	13

9.	Signalling requirements for revocation updates on the TSr interface	13

9.1.	Revocation-update message structure	13

9.2.	CRL distribution overview	13

Annex A Recommended list of verifiable CSP identifiers to be sent in the PEEC request	15

Bibliography	17

List of figures

Figure 1 - current connectivity of CSPs to the international signalling exchange networks	3

Figure 2 - general representation for a single TSCA trust chain and peer TSCA.	10

Figure 3 - reference architecture for interconnection between trustable network entities	10

Figure 4 - Signalling procedures of a trusted signalling certification authority	11

List of tables

Table ‎8-1 - Assignment Record structure	13

Table ‎10-1 - Revocation-update structure	13

# Scope

This Recommendation specifies the requirements for the for verification of information elements (IEs) in certificate signing requests and the allocation of ITU-T X.509 End-Entity and CA public-key certificates for telecom security within the international signalling exchange networks in support of existing and emerging networks, in continuation of the previous work done in ITU-T Q.3057, ITU-T Q.3062 and ITU-T Q.3063. Assignment of roles to entities, determination of the trust chain structure and the definition of operational procedure are not in the scope of this recommendation. This Recommendation specifies the technical structures and validation mechanisms related to certificate requests and signalling trust.

Operational appointment, governance, and audit of Registration Authorities (including Trusted Signalling Certification Authorities) are defined outside the scope of this Recommendation.

Q.TSCA provides the technical interface and information model supporting systems that implement the arrangements of E.RAA4Q.TSCA.

Editor’s note: E.RAA4Q.TSCA needs to be replaced with the E. series reference once published

# References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T X.509]	Recommendation ITU-T X.509 (2019), Information technology – Open Systems Interconnection – The directory: Public-key and attribute certificate frameworks.

[IETF RFC 4703]	Resolution of Fully Qualified Domain Name (FQDN) Conflicts among Dynamic Host Configuration Protocol (DHCP) Clients

[ITU-T Q.3057]	Recommendation ITU-T Q.3057 (2020) Signalling requirements and architecture for interconnection between trustable network entities.

[ITU-T Q.3062]	Recommendation ITU-T Q.3062 (2022) Signalling procedures and protocols for enabling interconnection between trustable network entities in support of existing and emerging networks.

[ITU-T Q.3063]	Recommendation ITU-T Q.3063 (2022) Signalling procedures of calling line identification authentication.

[ISO/IEC 6523-1]			ISO/IEC 6523-1:1998, Information technology – Structure for the identification of organizations and organization parts – Part 1: Identification of organization identification schemes.

[IETF RFC 2986]        	IETF RFC 2986(2000) PKCS#10 Certification Request Syntax Specification Version 1.7

# Definitions

## Terms defined elsewhere:

This Recommendation uses the following terms defined elsewhere:

Editor’s note: all terms from EV-Cert to be moved to section 5 (conventions) and reworded to avoid IPR issues

3.1.1	authority [ITU-T X.509]: An entity, responsible for the issuance of certificates or of revocation lists.

3.1.2	certification authority (CA) [ITU-T X.509]: An authority trusted by one or more entities to create and digital sign public-key certificates. Optionally the certification authority may create the subjects' keys.

3.1.3	contract signer [b-EV-Cert]: A contract signer is a natural person who is either the Applicant, employed by the Applicant, or an authorized agent who has express authority to represent the Applicant, and who has authority on behalf of the Applicant to sign subscriber agreements.

3.1.4	fully qualified domain name (FQDN) [b-RFC 1983]: Is the full name of a system, rather than just its hostname.

3.1.5	jurisdiction of incorporation [b-EV-Cert]: In the context of a Private Organization, the country and (where Applicable) the state or province or locality where the organization’s legal existence was established by a filing with (or an act of) an appropriate government agency or entity (e.g., where it was incorporated). In the context of a Government Entity, the country and (where Applicable) the state or province where the Entity’s legal existence was created by law.

3.1.6	government agency [b-EV-Cert]: In the context of a Private Organization, the government agency in the Jurisdiction of Incorporation under whose authority the legal existence of Private Organizations is established (e.g., the government agency that issued the Certificate of Incorporation). In the context of Business Entities, the government agency in the jurisdiction of operation that registers business entities. In the case of a Government Entity, the entity that enacts law, regulations, or decrees establishing the legal existence of Government Entities.

3.1.7	government entity [b-EV-Cert]: A government-operated legal entity, agency, department, ministry, or similar element of the government of a country, or political subdivision within such country (such as a state, province, city, county, etc.).

3.1.8 Intermediate CA [ITU-T X.509]: A certification authority (CA) is acting as an intermediate CA within a certification path when it is the issuer of the next public-key certificate on that certification path

3.1.9	place of business [b-EV-Cert]: The location of any facility (such as a factory, retail store, warehouse, etc.) where the Applicant’s business is conducted.

3.1.10	registration agency [b-EV-Cert]: A Governmental Agency that registers business information in connection with an entity’s business formation or authorization to conduct business under a license, charter or other certification.

3.1.11	registration number [b-EV-Cert]: The unique number assigned to a Private Organization by the Incorporating Agency in such entity’s Jurisdiction of Incorporation.

3.1.12	trust anchor [ITU-T X.509]: An entity that is trusted by a relying party and used for validating public-key certificates.

3.1.13	certificate revocation list [ITU-T X.509]: A signed list indicating a set of public-key certificates that are no longer considered valid by the issuing certification authority (CA). In addition to the generic term certificate revocation list (CRL), some specific CRL types are defined for CRLs that cover particular scopes.

3.1.14	trusted signalling certification authority (TSCA) [ITU-T Q.3057]: The trust anchor for verifying digital signatures using the trust anchor certification authority (CA) model.

3.1.15	organization [ISO/IEC 6523-1]: unique framework of authority within which a person or persons act, or are designated to act, towards some purpose

3.1.15 Registration authority [ITU-T X.509]: Those aspects of the responsibilities of a certification authority that are related to identification and authentication of the subject of a public-key certificate to be issued by that certification authority. An RA may either be a separate entity or be an integrated part of the certification authority

3.1.

16 Certificate Signing Request (CSR) [IETF RFC 2986, PKCS #10]: A certificate signing request consists of a distinguished name, a public key, and optionally a set of attributes, collectively signed by the entity requesting certification.

## Terms defined in this Recommendation:

3.2.1 TSCA-RA Identifier: The TSCA-RA Identifier is the unique code or Uniform Resource Identifier (URI) allocated under the Registration Authority to each Trusted Signalling Certification Authority acting as a Registration Authority (TSCA-RA). This identifier enables cross-reference between assignment data and Q.TSCA certificate or request elements.

3.2.2 Assignment Record: The Assignment Record is the structured dataset held within the Registration Authority registry describing a TSCA-RA or Applicant, including eligibility attributes, jurisdictional information, contact references, and status metadata. Technical systems implementing Q.TSCA may reference the Assignment Record to ensure consistency of identifiers and information elements.

3.2.3	Applicant: An entity (organization, individual, etc.) which requests the assignment of an identifier for an object (of interest) from a registration authority.

Note: Examples include general partnerships, unincorporated associations, and sole proprietorships., this term is defined based on [b-EV-Cert].

Editor’s note: add definition for TSCA-RA and TSCA-CA

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

AKA		Authentication and Key Arrangement

CA		Certification Authority

CLI		Calling Line Identification

CSP		Communications Service Provider

CSR		Certificate Signing Request

CRL		Certificate Revocation List

DFS		Digital Financial Services

EEC		End Entity public-key Certificate

eSIM		Electronic Subscriber Identity Module

FCC		Federal Communication Commission

FQDN		Fully Qualified Domain Name

GPRS		General Packet Radio Service

GTP		GPRS Tunnelling Protocol

HTTP		Hyper Text Transfer Protocol

IE		Information Element

IP		Internet Protocol

NNI		Network to Network Interface

OTT		Over the Top

PEEC		Provisional End Entity public-key Certificate

PKI		Public Key Infrastructure

PSTN		Public Switched Telephone Network

RA		Registration Authority

SS7		Signalling System No. 7

SMS		Short Message Service

TSCA		Trusted Signalling Certification Authority

UE		User Equipment

UNI		User Network Interface

SHAKEN	Signature-based Handling of Asserted Information Using toKENs

SIP		Session Initialization Protocol

SSGW        Signalling Security Gateway

STIR		Secure Telephone Identity Revisited

USSD		Unstructured Supplementary Service Data

VEEC		Validated End Entity public-key Certificate

VoIP		Voice Over IP

VoWiFi		Voice Over Wi-Fi

# Conventions

None

# Stakeholders in the signalling trust chain

Following are the key stakeholders in the signalling trust chain:

- TSCA-CA: The trust anchor Certification Authority (CA) and trust anchor for the underlying certification authorities (intermediate or CSP’s) and issue public-key certificates (end-entity or CA) for signaling interconnect.

- TSCA-RA:  The Registration Authority (RA) for certification authorities (intermediate or CSP’s) which determines the requirements for issuing public-key certificates to applicants for use in signaling interconnect.

- CSP CA: The CSP CA is an intermediate CA which governs the CSP security domain, the CSP CA issues end-entity public-key certificates to be used by the CSP Signalling Security Gateway (SSGW) to sign outbound signalling traffic as defined by [ITU-T Q.3062]. the CSP CA is issued a CA public-key certificate by the TSCA or an intermediate CA after registration with the TSCA.

- CSP-SSGW: The end-entity of the reference architecture, the CSP-SSGW resides within the security domain of the CSP and is issued an End-Entity public-key certificate by the CSP-CA, as defined by [ITU-T Q.3062].

- Intermediate CA: The intermediate CA is an optional stakeholder that can be placed in the trust chain between the TSCA and the CSP CA. Since CSPs operate in different regions and/or states, under different legal frameworks there may be a need or requirement an intermediate CA that will act as the certificate issuer and as the Registration Authority (RA) towards the CSP CA, be registered as an intermediate CA in the TSCA RA and be issued a CA public-key certificate signed by the TSCA as the trust anchor.

A more general representation for the trust chain is depicted in figure below, in practice, in the international signaling exchange networks there may be multiple TSCAs, each with their own hierarchy like the one depicted below. In which case, a bridge CA will be required to make the trust chains interoperable, as defined in [ITU-T Q.3057].

Figure 2 - general representation for a single TSCA trust chain and peer TSCA.

Editor’s note: add the TSCA-CA and TSCA-RA to the figure and add TSr interface

[ITU-T Q.3057] defines the following reference architecture, as shown in the figure below, for interconnection between trustable network entities. In the case that an intermediate CA is present, it will replace the TSCA in the reference architecture.

Figure 3 - reference architecture for interconnection between trustable network entities

# Requirements for issuing End-Entity and CA public-key certificates

ITU-T Q.3057 specifies the signalling procedure of the TSCA, shown in the figure below:

Figure 4 - Signalling procedures of a trusted signalling certification authority

The TSCA may issues two types of End-Entity/CA public-key certificates to CSPs:

- PEEC – issued automatically upon verification of IEs sent in the certificate signing request.
Note: PEEC will be granted after automatic verification by TSCA, and it should by valid until the VEEC is issued. It is noted that VEEC will follow and in the event of misalignment, it may result in revocation of PEEC.

- VEEC – issued upon verification of the Applicant’s identity and their eligibility criteria to obtain a CA certificate from the TSCA.
Note: Once the VEEC is issued, it should be valid for a period mandated by the TSCA .

Editor’s note: this section can be defined by E.RAA4Q.TSCA, if defined, it will be removed from this recommendation.

The TSCA, acting as the CSP CA trust anchor, has the responsibility for the allocation and management of both the PEEC and VEEC. Both PEEC and VEEC should be ITU-T X.509 End-Entity/CA public-key certificates either directly or via an intermediate CA.

CSPs who wish to obtain a VEEC should send a certificate request to the TSCA using the TSa interface defined in [ITU-T Q.3062]. the certificate request should contain the list of registration authority defined IEs, in addition to the CSPs public-key and be signed using the CSPs private-key as proof-of-possession.

## Requirement for IE verification for the issuance of a PEEC

The TSCA accepts electronic certificate signing requests from Applicants (CSPs and/or intermediate CAs) over the TSa interface as defined in [ITU-T Q.3062]. Upon receiving a certificate signing request (CSR), from an Applicant, the TSCA-RA, acting as a Registration Authority (RA), validates the IEs from the certificate signing request, using electronic interfaces. The TSCA-RA verifies that the applicant is authorized to operate within the international signalling exchange networks in accordance with the national legal and regulatory framework of the issuing and operational jurisdictions. This is not discussed further in the Recommendation.

Each CSR submitted to the TSCA-CA includes the TSCA-RA Identifier, referencing the registration authority instance responsible for the request. The Assignment Record associated with that TSCA-RA Identifier may be consulted by the TSCA system to obtain registry-maintained metadata such as eligibility attributes, jurisdictional scope, and status information. The Assignment Record information can be accessed through a structured data interface or registry query, ensuring consistency between the technical request and the registration data maintained by the TSCA-RA.

The list of required IEs to be submitted in the CSR is defined in Annex A of this Recommendation. Once all required IEs submitted in the CSR are verified by the TSCA-RA, the TSCA-CA, acting as a Certification Authority (CA), shall issue the Applicant a PEEC over the TSa interface as defined in [ITU-T Q.3062].

# Signalling requirements for retrieval of Assignment Record information

This clause specifies the signalling exchanges and information model used by a TSCA-CA and  a TSCA-RA to retrieve Assignment Record data. The retrieval process operates over a secure management interface, denoted TSr, enabling query and response transactions between the TSCA-CA and TSCA-RA. The Assignment Record is the structured dataset returned by the TSCA-RA in response to a query containing a TSCA-RA Identifier or an Applicant Identifier. Its structure is defined below:

## Assignment Record structure

| Field name | Data type | Description |
| --- | --- | --- |
| recordId | UUID / string | Unique identifier of the Assignment Record in the TSCA-RA registry. |
| tscaRaIdentifier | URI / string | TSCA-RA Identifier and used as the primary lookup key. |
| applicantIdentifier | URI / string | Unique reference of the Applicant or CSP entity associated with the record. |
| eligibilityStatus | Enum (Active / Suspended / Revoked / Pending) | Current operational status of the entity as recorded by the TSCA-RA. |
| contactInformation | Structured object | Includes administrative e-mail, telephone, and postal address of the primary contact for the entity. |
| assignmentTimestamp | UTC datetime | Date and time when the Assignment Record was created or last updated. |
| digitalSignature | Bit string | Digital signature of the TSCA-RA over the serialized Assignment Record for integrity protection. |

Table ‎8-1 - Assignment Record structure

## Information-exchange overview

The TSr query includes either the tscaRaIdentifier or applicantIdentifier as the primary key. The TSCA-RA responds with a message containing a serialized Assignment Record following the structure defined the table above, integrity is protected using [ITU-T X.509]-based digital signatures.

## Purpose of Assignment Record retrieval

The retrieved Assignment Record enables consistency checking of identifiers and attributes included in certificate signing requests processed under Q.TSCA. The information contained in the record is used exclusively for technical validation and does not define or modify any operational authorization or governance procedure covered by [ITU-T E.RAAQ.TSCA].

# Signalling requirements for revocation updates on the TSr interface

This clause specifies the technical signalling exchanges and data structures used by a TSCA-RA to transmit certificate-revocation information to a TSCA-CA through the TSr interface, as defined in Clause 9. The revocation-update process uses the same secure channel defined for Assignment Record retrieval, enabling synchronization of certificate-status information between the TSCA-RA and the TSCA-CA. Each message includes a digital signature generated by the TSCA-RA’s private key corresponding to its registered certificate. The TSCA-CA validates the signature chain using trust anchors defined in [ITU-T Q.3057] and [ITU-T X.509].

## Revocation-update message structure

| Field name | Data type | Description |
| --- | --- | --- |
| updateId | UUID / string | Unique identifier of the revocation transaction |
| tscaRaIdentifier | URI / string | Identifier of the TSCA-RA originating the update |
| certificateSerialNumber | String / hex | Serial number of the affected certificate |
| revocationReasonCode | Enum (KeyCompromise / CessationOfOperation / Superseded / Administrative / AssignmentChange) | Technical reason for the revocation event |
| revocationTimestamp | UTC datetime | Timestamp of revocation or suspension |
| assignmentRecordRef | URI / string | Reference to the Assignment Record associated with the certificate |
| statusIndicator | Enum (Revoked / Suspended / Restored) | Current certificate-status indicator |
| tscaRaSignature | Bit string | Digital signature of the TSCA-RA over the payload for integrity and authenticity |

Table ‎10-1 - Revocation-update structure

## CRL distribution overview

The Certificate Revocation List generated after each revocation update contains the serial numbers and status indicators of revoked or suspended certificates issued under the TSCA hierarchy.

Distribution of the CRL is performed through the TSa interface, following the same authenticated-signalling procedures used for certificate issuance. Peer TSCAs, intermediate CAs, and CSP CAs periodically retrieve or receive notifications of updated CRLs to maintain synchronized revocation information within the international signalling-interconnect environment. The CRL is formatted according to [ITU-T X.509] and includes a digital signature of the TSCA-CA to provide integrity and origin assurance.

# Annex A
Recommended list of verifiable CSP identifiers to be sent in the PEEC request

The IEs to be sent to the TSCA or the intermediate CA in the certificate request, should include but not limited to the following list:

Editor’s note: this list needs to be revised to avoid IPR issues

The applicant information should include, but is not limited to, the following details:

- Organization name. Include the formal legal organization name of the applicant in the EEC, as recorded with the incorporating or registration agency in the applicant’s jurisdiction of incorporation or registration (for private organizations), or as specified in the law of the political subdivision in which the government entity operates (for government entities), or as registered with the government business registration agency (for business entities).

- Assumed name (optional). Include the assumed name (e.g., “doing business as” appellation) of the applicant in the EEC, as recorded in the jurisdiction of Applicant’s Place of the place of business of the applicant, if requested by .the applicant.

- Security domain name (e.g. FQDN). Include the domain name(s) of the applicant in the EEC.

- Jurisdiction of incorporation or registration. Include the jurisdiction of incorporation or registration of the applicant in the EEC, consisting of:

  - city or town (if any).

  - state or province (if any).

  - country.

- Incorporating or registration agency. Include the name of Applicant’s the incorporating or registration agency of the applicant in the EEC.

- Registration number. Include the registration number assigned to applicant by the incorporating or registration agency in the jurisdiction of incorporation or registration of the applicant in the EEC. If the incorporating or registration agency does not issue registration numbers, then the date of incorporation or registration shall be recorded.

- Applicant address. Include the address of the place of business of the Applicant in the EEC, including:

- building number and street.

- city or town.

- state or province (if any)

- country.

- postal code (zip code).

- main telephone number.

- Public-key certificate requester. Include the name and contact information of the public-key certificate requester submitting the EEC request on behalf of the applicant.

The applicant agreement includes the following elements.

- General. Prior to the issuance of the EEC, the TSCA obtains the agreement of the applicant to a legally enforceable applicant agreement with the  registration authority for the express benefit of relying parties. The applicant agreement must be signed by an authorized contract signer acting on behalf of the applicant in accordance with local or peer SSGW law and must apply to the EEC to be issued pursuant to the EEC request. A separate Applicant agreement may be used for each EEC request, or a single applicant agreement may be used to cover multiple future EEC requests and resulting EEC(s), so long as each EEC that the TSCA issues to the applicant is clearly covered by an applicant agreement signed by an authorized contract signer acting on behalf of the applicant.

- Agreement requirements. The applicant agreement must, at a minimum, specifically name both the applicant and the individual contract signer signing the agreement on behalf of the applicant and contain provisions imposing on the applicant the following obligations and warranties.

- Accuracy of information: An obligation and warranty to provide accurate and complete information at all times to the TSCA, both in the EEC request and as otherwise requested by the TSCA in connection with the issuance of the EEC(s) to be supplied by the TSCA.

- Protection of private key. An obligation and warranty by the applicant or a subcontractor (e.g., hosting provider) to take all reasonable measures to maintain sole control of, keep confidential and properly protect at all times the private key that corresponds to the public key to be included in the requested EEC(s) (and any associated access information or device, e.g., password or token).

- Acceptance of EEC. An obligation and warranty that it will not install and use the EEC(s) until it has reviewed and verified the accuracy of the data in each EEC.

- Use of EEC. An obligation and warranty to install the EEC only on a SSGW accessible at the security domain name listed on the EEC and to use the EEC solely in compliance with all applicable laws, solely for signalling and solely in accordance with the Applicant Agreement

  - Reporting and revocation. Upon compromise, an obligation and warranty to promptly cease using an EEC and its associated private key, and promptly request the TSCA to revoke the EEC, if:

- any information in the EEC is or becomes incorrect or inaccurate, or there is any actual or suspected misuse or compromise of the private key of the applicant associated with the public key listed in the EEC.

- Termination of use of EEC. An obligation and warranty to promptly cease all use of the private key corresponding to the public key listed in an EEC upon expiration or revocation of that EEC.

The list above is consistent with lists used elsewhere for example by the CA/Browser Forum in [b-EV-Cert]

# Bibliography

[b-ITU-T QSTR-SS7-DFS]	Technical Report ITU-T QSTR-SS7-DFS (2019) SS7 vulnerabilities and mitigation measures for digital financial services transactions.

[b-ITU-T QSTR-USSD]	Technical Report ITU-T QSTR-USSD (2021) Low resource requirement, quantum resistant, encryption of USSD messages for use in financial services.

[b-EV-Cert]	CA/Browser Forum (2007), Guidelines for the issuance and management of extended validation public-key certificates, version 1.8.0. Available at:

https://cabforum.org/wp-content/uploads/CA-Browser-Forum-EV-Guidelines-1.8.0.pdf

___________________