Draft new ITU-T Recommendation H.264 Amendment 3

Advanced video coding for generic audiovisual services: 
Scalable video coding

AAP Summary

This text contains Amendment 3 to Rec. H.264 to extend its capabilities to enable scalable video coding (SVC). A scalable video bitstream has the property that a subset of the bitstream can be removed and the remaining data will still be decodable to produce video content with a quality that corresponds well to the quantity of remaining data.

Scalable video coding is specified in a new Annex G of the Recommendation. Three SVC profiles (the Scalable Baseline, Scalable High, and Scalable High Intra profiles) are specified therein.

The amendment is provided in integrated form with the complete basis text consisting of Rec. H.264 (2005-09) and its prior amendments Amd.1 (2006-06) and Amd.2 (2007-4).  Some minor corrections and clarifications relative to the prior content of the basis text have also been included.

Rec. H.264 and its amendments have been developed in coordination with ISO/IEC, which publishes the Recommendation as a technically-aligned twin text ISO/IEC 14496-10.

Note to the AAP reviewer:

This document includes the consolidated changes of H.264, its already approved Amendments 1 (change author denoted as “H.264 (2005) Amd.1”) and 2 (change author denoted as “H.264 (2005) Amd.2”), and the changes proposed as Amendment 2 (change author denoted as “H.264 (2005) Amd.3”). They are indicated as follows:

- This marked addition and deletion indicates the items in H.264 Amendment 1 (already approved and not subject of this AAP).

- This marked addition and deletion indicates the items in H.264 Amendment 2 (already approved and not subject of this AAP).

- This marked addition and deletion indicates the items which were Consented by SG 16 on 6 July 2007 and are the subject matter of AAP.

Therefore only the changes indicated as being part of H.264 Amendment 3 are to be considered for this AAP.

_______________