| INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | Joint Coordination Activity on Accessibility and human Factors (JCA-AHF) |
| --- | --- | --- |
|  |  | Doc. 153 |
|  |  | English only Original: English |
| Source: | JCA-AHF Convener |  |
| Title: | Draft New Recommendation J.adcstb-req (as of 25 October 2011) |  |

This document is submitted for information.

Attached is the document ITU-T SG9 placing a reference to ITU.T F.790, in Section 11.

This document is going to be presented for AAP consent on 25 November 2011, during the SG9 plenary.

| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 9 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | TD 653 (GEN/9) |
|  |  | English only Original: English |
| Question(s): | 5/9 | Geneva, 21-25 November 2011 |
| TEMPORARY DOCUMENT |  |  |
| Source: | Rapporteur for Q.5/9 |  |
| Title: | (As of 25 October 2011) Draft New Recommendation J.adcstb-req |  |

Introduction

As an output of teleconferencing held on 25th October 2011, this document provide the latest text of draft new Recommendation J.adcstb-req with change marks from TD633. The meeting agreed that we propose this document for AAP consent at the November meeting..

| Contact: | Mayumi Matsumoto Japan Cable Laboratories | Tel: +81-3-6450-4311 Fax: +81-3-6450-4310 Email: matsumoto@jlabs.or.jp |
| --- | --- | --- |
| Attention: This is not a publication made available to the public, but an internal ITU-T Document intended only for use by the Member States of ITU, by ITU-T Sector Members and Associates, and their respective staff and collaborators in their ITU related work. It shall not be made available to, and used by, any other persons or entities without the prior written consent of ITU-T. |  |  |

Draft New Recommendation J.adcstb-req

Functional Requirements for Advanced Cable Set-Top Box

Table of Contents

1.	Scope	6

2.	References	6

3.	Terms and definitions	7

4.	Abbreviations, acronyms and convention	9

4.1.	Abbreviations and acronyms	9

4.2.	Conventions	10

5.	General Requirements	10

5.1.	Services provided	11

5.2.	Assumed cable network	11

5.3.	Cross platform	12

5.4.	Open software	12

5.5.	Multi-tasking support	12

5.6.	CPU architectures	12

5.7.	Security	12

5.8.	Adaptation for technical evolution and compatibility	12

5.9.	Enhancement of service	12

5.10.	User Interface (UI)	12

5.11.	Cable Portal	13

5.12.	Content protection	13

5.13.	Home network connections	13

6.	Service requirements	13

6.1.	General	13

6.1.1.	Open software, open interface	13

6.1.2.	Compatibility with existing standards	13

6.1.3.	Connection to home terminal device	13

6.1.4.	Backward service compatibility	13

6.2.	Requirements for content reception	14

6.2.1.	Simultaneous viewing and recording	14

6.2.2.	Content quality monitoring	14

6.2.3.	Codec requirements	14

6.2.4.	Multiple content reception and its recording	15

6.2.5.	Content selection	15

6.2.6.	Content sharing	15

6.2.7.	Remote UI	15

6.2.8.	Social Television service	15

6.2.9.	Subscription control	15

6.2.10.	Device authentication and subscriber authentication	15

7.	Broadcasting service	15

7.1.	Navigation of broadcasting service	15

7.2.	Reception of broadcasting content provided by the broadcaster	16

7.3.	Program selection	16

7.4.	Conditional Access	16

7.5.	Schedule Recording	17

7.6.	Program recording	17

7.7.	Notice of additional broadcast service	18

8.	IP interactive services	18

8.1.	Network Connection	18

8.2.	Network Connection Setting	18

8.3.	Delivery of IP interactive services	18

8.4.	Service navigation	19

8.5.	Video and audio content and other services	19

8.6.	IP Video streaming	19

8.7.	Content selection	19

8.8.	Limited reception	19

8.9.	Character input for interactive service	20

8.10.	Communication error and time-out	20

8.11.	Notice of additional IP Interactive service	20

9.	Display functions for web/interactive service	20

9.1.	Web Browsing	20

9.2.	Reception of broadcasting and IP interactive services	20

9.3.	Cable portal	20

9.4.	Customization of cable portal	21

10.	RCU functions	21

10.1.	Conventional remote	21

10.2.	Connection method for RCU	21

10.3.	HN type RCU	21

10.4.	Personalization of HN type RCU screen	21

10.5.	Update of HN type RCU software	22

10.6.	Internet type RCU	22

11.	User Interface (UI)	22

11.1.	General requirements for UI	22

11.2.	Broadcasting content presentation on the UI	22

12.	Connection interface	22

12.1.	Coaxial cable interface	22

12.2.	Wireless LAN (Wi-Fi) interface	22

12.3.	WAN interface	23

12.4.	Wired LAN interface	23

12.5.	Peripheral device interface	23

12.6.	RCU interface	23

12.7.	Output interface	23

12.8.	CA card interface	24

13.	Authentication and authorization	24

13.1.	Device authentication and authorization	24

13.2.	User authentication	24

13.3.	Authorization of application execution	24

14.	Hybrid services of broadcasting and web	25

14.1.	Cable portal screen	25

14.2.	Broadcasting content	25

14.3.	Simultaneous display of IP interactive services and broadcasting content	25

14.4.	Overlaying on the broadcasting content	26

14.5.	Services associated with EPG	26

15.	Download function for system software/firmware update	26

15.1.	Software download method	26

15.2.	Process in case of download failure	27

15.3.	Logging of download schedule and result	27

15.4.	Indication of system software/firmware version	27

16.	Download function for application software	27

16.1.	Push download method	27

16.2.	Pull Download method	27

16.3.	Indication of application software version	27

16.4.	Subscriber authentication and billing for pay software download	28

17.	Home network connection	28

17.1.	Transport for HN	28

17.2.	Routing function	28

17.3.	Protocols	28

18.	Management functions	28

18.1.	Provisioning function	28

18.2.	STB remote management	28

18.3.	Management of Subscriber information	29

18.4.	Monitoring of reception quality	29

19.	Power saving	29

20.	Mechanical requirements	29

Bibliography	41

Draft New Recommendation J.adcstb-req

Functional Requirements for Advanced Cable Set-Top Box

# Scope

This Recommendation defines functional requirements for the advanced cable set-top box to enable cable television operators to provide advanced services to their subscribers. The advanced cable set-top box is intended to apply to cable television operators’ service provision, where many types of access network technologies have been recently introduced, e.g. HFC, PON, RFoG. [Editor’s note: Needs more elaboration.] Cable television operators have a capability of providing both broadcasting and interactivity over its own network originally intended to distribute broadcasting television programs, and the advanced cable set-top box is a core device for the delivery of attractive advanced services.

Functionalities to be implemented in the head-end systems of cable television operators are outside the scope of this Recommendation.

The advanced cable set-top box is primarily equipped with reception functionalities for broadcasting services and IP-based interactive services. In addition, the STB performs as an interface device between cable television access networks and in-home network devices by being equipped with wireless LAN and IP routing capabilities.

In this Recommendation, portal screen support and application software plug-ins capability are also required to the STB to allow cable television operators to provide various types of services to their subscribers not only from their own service delivery platforms but also from other 3rd parties’ platforms interconnected to cable operators’ platforms.

# References

The following ITU-T Recommendations, and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation.  At the time of publication, the editions indicated were valid.  All Recommendations and other references are subject to revision; all users of this Supplement are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T J.190] ITU-T Recommendation J.190 (2007), Architecture of MediaHomeNet that supports cable-based services

[ITU-T J.218] ITU-T Recommendation J.218 (2004), Cable modem IPv4 and IPv6 eRouter specification

[ITU-T J.700] ITU-T Recommendation J.700 (2007), IPTV Framework for Secondary Distribution

[ITU-T J.701] ITU-T Recommendation J.701 (2008), Broadcast Centric Terminal Middleware

[ITU-T X.1191] ITU-T Recommendation X.1191 (2008), IPTV Service Security and Content Protection Aspects

[ITU-T Y.1901] ITU-T Recommendation Y.1901 (2008), IPTV Service Requirements

[ARIB TR-B14] Version 4.3(2010/11), OPERATIONAL GUIDELINES FOR DIGITAL TERRESTRIAL TELEVISION BROADCASTING

[ARIB TR-B15] Version 5.1(2010/11), OPERATIONAL GUIDELINES FOR DIGITAL SATELLITE BROADCASTING

# Terms and definitions

[Editor’s Note: To be aligned with the existing definition in the ITU terminology database.]

| Term | Definition |
| --- | --- |
| Advanced cable STB | A STB that satisfies the requirements defined in this Recommendation. Unless specifically noted, STB shall mean the advanced cable STB in this Recommendation. |
| IR type RCU | RCU operating on infrared rays. Only uni-directional operation is supported. |
| RF type RCU | RCU using wireless technology such as RF4CE. |
| HN type RCU | RCU connected to the STB via IP home network. Interactive operation is supported. In addition to communicating and controlling with the STB, the HN type RCU can be also accessible directly to services provided by cable operators or other content and/or service providers. |
| Internet type RCU | A terminal with RCU functionalities that allows users to control the STB from remote location via the internet, e.g. remote setting of schedule recording. This type of RCU is supposed to be mobile phones (smart phones, feature phones, etc.), tablet devices, PCs, etc. In addition to communicating and controlling with the STB, the Internet type RCU can be also accessible directly to services provided by cable operators or other content and/or service providers. |
| Second screen | It refers to a display screen of mobile phones or other network enabled devices that show services associated with the television screen. |
| Multi-task | Simultaneous process of live viewing or playback and one or multiple recording of broadcasting content; the viewing of IP streaming content; and other applications such as games. |
| User | The user of device connected to the STB and HN. Not only human being, but also includes logical element for the communication and usage of applications between devices. [Editor’s Note: Needs to be found from the existing definition of “user” in the ITU-T database.] |
| Subscriber | The registered subscriber (representative) to the cable TV service; shall include all persons in the household. |
| Viewer | A subscriber who is receiving video, audio and/or other information services. |
| Operator | The cable TV service operator. |
| IP interactive service | A type of service provided by the cable operator or service provider where the subscriber requests and/or responds to services. |
| Target advertising | Advertising service from the operator targeted at an individual or a viewer segment, specific markets, specific areas, facilities, etc. |
| Network | A wired or wireless communication network to provide broadcasting and communication services. In this requirements, it shall include the in-house communication network, in addition to the access network operated by the cable operator. |
| Provisioning | Automated initial setting activity. |
| DRM | Digital Rights Management to protect digital content. In these requirements, use within HN is assumed. |
| Plug-in | Software for displaying, installed independently from the browser. |
| Terminal | Equipment consisting of hardware and software with communication means, equipped with a user interface to enable the user to connect to the network. |
| User Interface (UI) | The basic UI is provided by the OS and defines the display format of information to the user and the data input and operation methods by the user. These requirements assume the use of both character based CUI and graphics based GUI. |
| Cloud terminal | A terminal having at least client function, browser and internet connectivity for cloud network connection. |
| Content | Information content transmitted via various media through the communications network, including the broadcasting network and IP. |
| Transcoder | Image conversion process to convert images to match the display capability of the terminal. |
| Cable platform | Shall mean the entire provision infrastructure of the cable business, including the service distribution platform (SDP), cable network, advanced cable STB, home network and others. |
| Cross platform | Coordination service between platforms. |
| Digital Terrestrial Television Broadcasting | Digital Terrestrial Television Broadcasting defined in ITU-R BT.1306 |
| Satellite Broadcasting | Digital satellite television broadcasting defined in ITU-R BT1306. |
| Multichannel Video Distribution | Digital television distribution service over cable networks that provides wide range of multichannel television programs to subscribers. |
| Emergency Alert Broadcasting | Emergency Alert Broadcasting defined in ITU-R BT.1774 |
| Social Television | General term for technology that supports communication and social interaction in either the context of watching television, or related to TV content. It includes the study of television-related social behavior, devices and networks. Social television systems can for example integrate voice communication, text chat, presence and context awareness, TV recommendations, ratings, or video-conferencing with the TV content either directly on the screen or by using ancillary devices. |
| Personal Information | Recorded information about an identifiable individual that may include his or her (1) name, address, email address, phone number, (2) race, nationality, ethnicity, origin, color, religious or political beliefs or associations, (3) age, sex, sexual orientation, marital status, family status, (4) identifying number, code, symbol, (5) finger prints, blood type, inherited characteristics, (6) health care history including information on physical/mental disability, (7) educational, financial, criminal, employment history, (8) others' opinion about the individual, and (9) personal views except those about other individuals. |
| Home Network | A network within a home compliant with J.190 consisting of IP-based transport including wired LAN (Ethernet: IEEE 802.3) and wireless LAN (WiFi: IEEE 802.11a/b/g/n) and non-IP based communication method such as ZigBee (IEEE 802.15.4), where the advanced cable STB is equipped with all of the HA, HB, HC and HD functionalities defined in J.190. |

3.2 Terminology defined in other Recommendations

3.2.1 Hybrid DTV Service [ITU-T J.acf-req]: Simultaneous provision for integrated experience of broadcasting and interactivity consisting of media content, data and applications from multiple sources, where the interactivity is sometimes associated with broadcasting programs.

3.2.2 Cable Hybrid DTV Service [ITU-T J.acf-req]: Hybrid DTV Service managed by cable operators.

3.2.3 Personal Information [ITU-T H.iptv-am.0]:

# Abbreviations, acronyms and convention

## Abbreviations and acronyms

| Abbreviation/Acronym | Definition |
| --- | --- |
| API | Application Program Interface |
| CAS | Conditional Access System |
| CPE | Customer Premises Equipment |
| DOCSIS | Data over Cable System Interface Specification |
| DTCP | Digital Transmission Content Protection |
| FEC | Forward Error Correction |
| EPG | Electronic Program Guide |
| HFC | Hybrid-Fiber/Coax |
| HN | Home Network |
| H-STB | Hybrid Set-Top-Box |
| HTML | Hyper Text Mark-up Language |
| IP | Internet Protocol |
| LAN | Local Area Network |
| MVPD | Multichannel Video Programming Distributor |
| QoS | Quality of Service |
| RCU | Remote Control Unit |
| RF | Radio Frequency |
| SDP | Service Delivery Platform |
| STB | Set-Top-Box |
| UI | User Interface |
| VOD | Video On Demand |
| WAN | Wide Area Network |

## Conventions

In this Recommendation:

The keywords “is required to” indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

The keywords “is recommended” indicate a requirement which is recommended but which is not absolutely required.  Thus this requirement need not be present to claim conformance.

The keywords “is prohibited from” indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

The keywords “can optionally” indicate an optional requirement which is permissible, without implying any sense of being recommended.  This term is not intended to imply that the vendor’s implementation must provide the option and the feature can be optionally enabled by the network operator/service provider.  Rather, it means the vendor may optionally provide the feature and still claim conformance with the specification.

In the body of this document and its annexes, the words shall, shall not, should, and may sometimes appear, in which case they are to be interpreted, respectively, as is required to, is prohibited from, is recommended, and can optionally. The appearance of such phrases or keywords in an appendix or in material explicitly marked as informative are to be interpreted as having no normative intent.

# General Requirements

[Editor’s note: Figure needs to be inserted.]

Figure 5-1 illustrates overall cable architecture which consists of service delivery platform, cable network, mobile network, home network and the advanced cable STB as a core device for advanced cable services. The figure also shows some service aspect and collaboration of application based on this architecture.

Figure 5-1: Overall cable architecture supported by the advanced cable STB

The advanced cable STB assumes the following conditions in the actual deployment stage.

## Services provided

The advanced cable STB is required to support cable hybrid DTV services.

The STB is required to receive terrestrial and satellite broadcasting re-transmitted (by trans-modulation) over RF (not over IP) on the cable access network. The STB is required to receive RF-based multichannel video programs and can optionally support IP-based ones. The STB is also required to support interactivity such as VOD, Voice over IP, TV telephony and electronic commerce.

## Assumed cable network

The network where the advanced cable STB is to be used shall be the FTTH, HFC or interactive coaxial network, which is currently provided by cable operators. The network is required to be a content delivery network managed by the cable operator. The network assumes interoperability with the networks provided by other operators (broadcasting networks, mobile phone networks, etc.). The network is required to have an Internet connection capability. Furthermore, it is required to connect with the home network inside the subscriber’s residence.

## Cross platform

The advanced cable STB is required to provide services available not only on cable platforms but also on other platforms by coordination between the cable operator and other service providers operating those platforms. .

## Open software

The advanced cable STB is recommended to apply open technologies as much as possible and is recommended to apply software compliant with the ITU-T software license policy. Open-based OS, API, SDK (Software Development Kit), etc. are recommended for the advanced cable STB.

## Multi-tasking support

The advanced cable STB is required to support multi-tasking on CPU kernel level.

## CPU architectures

The advanced cable STB is required to not depend on any specific type of CPU architecture.

## Security

The advanced cable STB is required to be equipped with tampering check functions (including tampering prevention functions for OS, firmware and middleware), application execution permission control, tamper resistance, etc. Enhancement of CPU internal security is also required to prevent from software hacking for open OS.

## Adaptation for technical evolution and compatibility

The advanced cable STB is required to support cable application framework to ensure backward compatibility and also to support open platform to prepare future technical evolution.

## Enhancement of service

The advanced cable STB is required to support service enhancement on application layer.

## User Interface (UI)

The TV screen (first screen) and terminal device screen (second screen) are required to have a capability for supporting various implementation of User Interface (UI). The UI is required to provide intuitive operation for user with minimum delay of the operation. The UI is required to have a capability to promote advanced services. Personalization and customization of the UI are required to follow the intension of the cable operator. The advanced cable STB is required to enable subscribers to customize the UI within the permission of cable operator.

## Cable Portal

The cable portal is the initial interactive presentation for the subscribers to access various services (TV broadcasting, VOD, Web browsing, other interactive services, etc.,) provided by the cable operator. It is required to enable the cable operator to customize the cable portal.

## Content protection

It is required for the advanced cable STB to be equipped with the conventional conditional access system (CAS) to protect broadcasting content. The advanced cable STB is also required to support conventional copy protection system already deployed in cable industry. If content is re-distributed from STB (eg. to Home network) after the conditional access has been removed, the advanced cable STB is required to apply the content protection method (eg. DTCP-IP or higher capability) approved by the broadcasting industry

## Home network connections

The advanced cable STB is required to support IP network connections in the home network. The advanced cable STB is required to support both the DHCP server and client functions, logically. DLNA connection is required for video and audio content delivery within home. The advanced cable STB can optionally support connections to non-IP devices for various home appliances.

# Service requirements

## General

### Open software, open interface

The advanced cable STB is recommended to support open software and open interfaces, especially for OS and middleware.

### Compatibility with existing standards

The advanced cable STB is required to follow existing domestic and international standards when it is applicable.

### Connection to home terminal device

The advanced cable STB is required to support services described in sub-clause 8.3 with home terminal devices connected to the home network.

### Backward service compatibility

The advanced cable STB is required to support limited functionalities of the software for future platform without STB platform upgrade. The full functionalities can be realized by the STB platform upgrading.

## Requirements for content reception

### Simultaneous viewing and recording

The advanced cable STB is required to support simultaneously any combination of viewing including content delivery in-home and recording of at least two (2) programs on the same incoming broadcasting network. It is also required to support simultaneous viewing of IP streaming content and recording of broadcasting program.

### Content quality monitoring

The advanced cable STB is recommended to have a quality monitoring function to provide feedbacks to the cable operator.

### Codec requirements

The advanced cable STB is required to support the following codecs for the broadcasting and the interactive services in accordance with the regional broadcasting system requirements.

- Video decoder

Decode MPEG-2 Video and MPEG-1 Video streams.

Support still-picture decoding on MPEG-1 I-frame

Decode H.264 streams.

- Audio decoder

Decode MPEG-2 Audio AAC stream.

Decode MPEG-1/-2 Audio Layer 2 stream.

Decode AC-3 stream.

Decode DTS stream.

Replay AIFF-C (Audio Interchange File Format - Compression) format, limited to PCM only.

Support audio mixing method.

Support audio mixing of MPEG-2 AAC and AIFF-C code.

- Transcoder

Support conversion of codec, bit rate and file format for above mentioned codecs is required. Real time conversion is required in the reception of broadcasting content, and degradation of picture quality shall be minimum. Post conversion for stored content is also required.

At least two (2) simultaneous transcoding is required for the advanced cable STB.

For post conversion, the advanced cable STB is required to support appropriate transcoding scheduling for minimum delay viewing by smart phone, tablet or PC via DLNA connection.

De-interlacing conversion is also required.

- 3D broadcasting support

Support of full HD quality 3D broadcasting is required.

### Multiple content reception and its recording

The advanced cable STB is required to receive at least two (2) broadcast programs simultaneously regardless whether HDD is embedded in the STB or not.

### Content selection

Regardless of the source or type of the content, the advanced cable STB is required to provide a content selection means so that subscriber can select content easily and intuitively.

### Content sharing

Regardless of the source or type of the content, the advanced cable STB is required to have the function for content delivery among the terminal devices in the home network in accordance with the policy determined by stake holders, i.e., cable operator, content originator, etc.

### Remote UI

The advanced cable STB is required to support a remote UI function on a terminal device connected in home network or via public telecommunication network. For example,  schedule recording from outside the home via the network will be provided by the remote UI.

### Social Television service

The advanced cable STB is required to support the Social Television services such as exchanging social network information, voting for ranking or comment indication from viewers.

### Subscription control

The advanced cable STB is required to have a subscription control function where the cable operator may control the delivery of the services based on the subscription status.

### Device authentication and subscriber authentication

The advanced cable STB is required to support the CAS system for identification of the CAS module of the STB and/or the subscriber.

# Broadcasting service

## Navigation of broadcasting service

The advanced cable STB is required to navigate broadcasting services via the portal screen on either the first screen or the second screen.

## Reception of broadcasting content provided by the broadcaster

The advanced cable STB is required to receive television broadcasting content including related data (e.g. EPG) and 3D content.

The advanced cable STB is required to receive the following servicest.

- Emergency alert information and special programs

- Multi-view (e.g. double pictures on the same screen)

- VOD, PPV

In particular the advanced cable STB is required to receive emergency alert information even if in its stand-by state, then quickly turn to power-on state automatically, and to process the emergency alert signal. The behavior for the emergency alert signal is required pre-configurable by the subscriber.

(Note) Refer to Download Function (Clauses 15 and 16) for downloads.

## Program selection

The advanced cable STB is required to support the following methods of program selection using RCU and other tablet devices.

- Switching broadcasting types (e.g. multi channel video and re-transmission)

- Input of channel number (e.g. three-digit tuning)

- Channel up/down

- Favorite channel list/ Short-cut button to favorite channel

- EPG

- Keyword search (matching of keywords, such as title, cast, genre, etc.).

- return to previous program (previously viewed channel)

It should be noted that the following control on the program will be available after program selection.

- Audio selection (including stereo/mono, main/sub, and language selection)

- Caption on/off, language selection

## Conditional Access

The advanced cable STB is required to support the CA module satisfying the requirements defined by the cable operator.

The following provides examples of the functional requirements to the CA module.

- To support Conditional Access used in each region

- PPV system of each digital broadcasting (including viewing history collection)

- Viewing restriction in each channel using PIN (PPV)

- To cope with copy control using appropriate rights protection method

## Schedule Recording

The advanced cable STB is required to support schedule recording as follows.

- Support for both schedule recording and reminder.

- Support for schedule by date and time, and from the EPG.

- Support for checking or deletion of scheduled programs using screen.

- If a schedule recording has been specified using EPG, and if the start and/or end time happens to change, the advanced cable STB is required to perform the schedule recording as per the updated schedule of the program.

- A lock function is required to prevent the cancellation of recording after the recording is started unless specifically intended to cancel the recording by the subscriber. A warning notification is required for such an action.

## Program recording

The advanced cable STB is required to record broadcasting content onto a storage device as follows:

- Re-compression function of recorded content is recommended.

- Intuitive user interface is recommended for recorded content management.

The advanced cable STB is required to support the following functions when it connects the external storage device.

- Recording onto the external storage device is only allowed if the functions for content rights protection and copy protection are properly provided.

- When the external storage devices such as HDD, DVD connected to the advanced cable STB through LAN, it is required to support DTCP-IP connection.

- When the advanced cable STB recorded the content into USB connected HDD, only that STB is allowed to playback the stored content. If the STB has been replaced due to the machine trouble, it is required to move the stored content from the troubled STB to the new STB with the copy protection linked to the subscriber. USB 2.0 or higher is required.

- The advanced cable STB is required to support copy control function which indicated by the original digital copy control information which is embedded in content, when the STB records the program into external HDD.

## Notice of additional broadcast service

The advanced cable STB is required to support functionality to notify subscribers when a broadcasting service has been added (e.g., addition of a community channel).

# IP interactive services

## Network Connection

The advanced cable STB is required to have a dual stack function to support IPv4 and IPv6 defined in [ITU-T J.218].

## Network Connection Setting

The following setting is required to be configurable.

- LAN communication (wired and wireless LAN, access point, IP address, DNS setting)

- Cable modem or WAN port (IP address, DNS setting)

Also the following setting is recommended.

- Proxy server (address and port number to be set by manual and automatic)

## Delivery of IP interactive services

The advanced cable STB is required to deliver IP interactive services using browser and/or application download functions. It is recommended to, but not limited to, support the following services.

- IP streaming service

- IP based VOD and PPV service

- IP based download service

- Remote access for scheduled recording manager

- Multi device service (TV watching on second screen device, multi-room viewing)

- Video telephony

- TV watching enhancement by social media (Micro-blog, view ranking, etc.)

- E-mail service

- Online municipal or local government information service

- E-commerce

- Local information service

- Targeted advertisements, content recommendations

- Net supermarket

- Electronic flyers

- Online shopping

- Ticket reservations and purchases

- News, weather reports, stock market information

- Remote banking

- Games, network games

- Karaoke

- Internet radio

Additionally the following services can optionally be supported:

- Energy management (real time monitoring, indication, and automatic control of electricity, gas, water, etc.)

- Interphone connection

- Home security

- Infant monitoring, pet monitoring

- Healthcare

- Senior person safety monitoring

- Remote operation of home appliances

## Service navigation

Refer to sub-clause 10.2.

## Video and audio content and other services

If any IP interactive service includes Audio/Video contents, the advanced cable STB is required to  playback/transmit them to other terminal devices. It is also required to add necessary copy control to the output signals, in accordance with the content rights protection method.

## IP Video streaming

The advanced cable STB is required to receive unicast signal and recommended to receive multicast signal. AS the transmission protocol, RTP/UDP is required, HTTP/TCP is optional. In the cable network, QoS control is required to suppress jitter and packet loss in streaming transmission. The advanced cable STB is required to absorb the jitter by buffering in accordance with memory capacity, also clock synchronization mechanism is required for long term video reception. It is also required to compensate packet loss with FEC.

## Content selection

The advanced cable STB is required to provide content selection using RCU and other tablet devices.

## Conditional access for IP interactive services

The advanced cable STB can optionally support the conditional access to the content delivered by IP. When the STB supports conditional access, it is required to support following functionality:

Conditional access for

- Broadcasting IP streaming

- On-demand IP streaming

- Access control by channel basis or the browser using a password

- Copy control in accordance with the content rights protection method.

## Character input for interactive service

The advanced cable STB is required to support the input of characters using the RCU or the soft keyboard of the HN terminal device defined in [ITU-T J.190].

## Communication error and time-out

The advanced cable STB is required to notify communication errors and time-outs to the operator. The advanced cable STB is required to provide a recovery functionality from communication waiting status. Note that application on the STB may notify errors to the users including the communication problems.

## Notice of additional IP Interactive service

The advanced cable STB is required to support functionality to notify subscribers when an IP interactive service has been added.

# Display functions for web/interactive service

## Web Browsing

The advanced cable STB and/or the second screen device are required to have the capability to display web pages. It is required to have a function to receive the URL input and other information from the search site on the web access screen indicated on the screen through the RCU or the software keyboard displayed on screen. It is also required to support password control to restrict the direct input of URLs. The advanced cable STB is required to support latest version of browser and protocols.

## Reception of broadcasting and IP interactive services

Either broadcasting services or IP interactive services are required to be selectable

through the use of the RCU or icons on the second screen.

## Cable portal

The advanced cable STB is required to display the cable portal when STB is turned on or STB receives operational command from RCU. It is required to be selectable by subscriber and/or operator whether the STB primary displays the cable portal or not. The appearance of portal is required to be configurable by the subscriber through the application download.

## Customization of cable portal

It is required to support the customization of cable portal by the operator within the range listed in sub-clause 2.10 (by age, generation, etc.) or by the subscriber.

# RCU functions

Advanced cable STB is required to have a capability to accept controls from the following four types of devices:

- Conventional remote with IR

- Conventional remote with radio access

- Second screen terminals on a home network (HN)

-Tablet and/or smart phone connected to the internet

The RCU described in this Recommendation is required to provide stress-free operation for subscribers with following descriptions for each type of RCU.

## Conventional remote

The conventional remote of the advanced cable STB is required to have the minimum number of buttons to provide simple operation by taking advantage of graphical navigation shown on the screen.

## Connection method for RCU

The advanced cable STB is required to support multiple communication methods for RCU. Multiple connection support is required in each transmission method.

## HN type RCU

The HN type RCU is required to connect to the STB via HN and to display the cable portal screen. Through the portal screen, user can operate the STB with the HN type RCU. In addition, the HN type RCU can access various applications or services provided by cable operators or content providers via HN. A hardware or software based keyboard and graphical UI including desktop icon are required for the RCU. It is recommended to implement microphone and voice recognition function to realize voice input (program selection, etc.).

## Personalization of HN type RCU screen

HN type RCU is required to have a capability to support personalization of the screen in accordance with the preference of subscribers or age groups in their family members.

## Update of HN type RCU software

HN type RCU is required to have a capability of upgrading its functions and application softwares. (Refer to Clause 15).

## Internet type RCU

The internet type RCU is required to connect to the STB via the internet. In addition, the Internet type RCU can access various applications or services provided by cable operators or content providers via the internet. A hardware or software based keyboard and graphical UI including desktop icon are required for the RCU.

# User Interface (UI)

## General requirements for UI

The advanced cable STB is required to equip the UI with the following requirements:

(1) Quick response

(2) Intuitive operation

(2-1) Effortless navigation to new services

(2-2) Literacy level agnostic operation

(3) State-of-the-art graphical appearance

(4) Telecommunications accessibility for older persons and persons with disabilities [ITU-T F.790]

## Broadcasting content presentation on the UI

The advanced cable STB is required to allow cable operators to specify the position and the size of broadcasting content presentation on the UI.

The advanced cable STB is required to have the capability of the broadcasting content presentation compliant with the guideline [b-EBU].

# Connection interface

## Coaxial cable interface

To provide all the services described in this [draft] recommendation for subscribers, the advanced cable STB is required to equip a coaxial cable connector as an input terminal. In the advanced cable STB, RF tuner receives video signals and DOCSIS cable modem sends and receives IP packets through this interface.

## Wireless LAN (Wi-Fi) interface

The advanced cable STB is required to provide wireless LAN (Wi-Fi) functions. It is required to work selectively as access point or client. The advanced cable STB is recommended to embed this functionality, but can optionally support this functionality with an external USB Wi-Fi module.

## WAN interface

The advanced cable STB is required to support Ethernet WAN interface for non-HFC cable networks, where the embedded cable modem will not be not used.

## Wired LAN interface

In addition to the wireless LAN interface described in 12.2, the advanced cable STB is required to support wired LAN interface for the same purpose. This interface can also be used for WAN access when the LAN is accessible to the WAN network.

## Peripheral device interface

The advanced cable STB is required to be equipped with at least two (2) USB terminals for peripheral device support. The STB is required to support the following devices:

Storage (USB Mass Storage Class)

Wi-Fi (USB Wireless Controller Class)

Camera (USB Video Class)

Microphone (USB Audio Class)

Hub (USB HUB Class)

The advanced cable STB is required to allow applications to utilize these peripherals through the APIs.

## RCU interface

The advanced cable STB is required to support the four (4) types of RCU interfaces, i.e., the IR type RCU defined in xx uses Infrared interface (1), the RF type RCU defined in xx uses wireless interface (2) e.g. RF4CE, the HN type RCU uses wireless LAN interface (3) e.g. IEEE 802.11n, and the internet type RCU defined in xx uses IP access interface (4) e.g. WAN or wired/wireless LAN. Interfaces (1), (2) and (3) are used for in-house operation, while interface (4) is used for STB operation from outside the home.

## Output interface

The advanced cable STB is required to be equipped with an HDMI interface as HD digital output. . The HDMI interface is required to be HDMI 1.4a or higher. The STB is required to support power on/off control of the external device (e.g. TV set) by HDMI-CEC. The STB is recommended to be equipped with an Optical Audio interface (IEC 60958). For legacy support, RCA composite with analog audio (yellow, red, white) are required. The STB is required not to equip any of S terminal, analog component or IEEE1394 interface due to copyright concern.

## CA card interface

The advanced cable STB is required to equip CA card interface that comply with the conditional access system used in each region.

# Authentication and authorization

The advanced cable STB is required to support authentication and authorization mechanisms for the device and the user that satisfy the following requirements.

## Device authentication and authorization

The advanced cable STB is required to support the following device authentication and authorization functions.

- CA card or downloadable CA is used to authorize broadcasting service reception by the STB.

- DOCSIS authentication is used to authenticate the STB by the head-end.

- DTCP-IP is used for device authentication between the advanced cable STB and other DLNA capable devices connected to the home network.

- LLC-HDCP is used for device authentication between the advanced cable STB and other HDMI devices such as a TV set.

- The advanced cable STB is required to support authentication mechanisms to authenticate external devices connected to the home network such as a smart phone or a tablet, where the STB is required to support both types of authentication, i.e., direct authentication between devices (e.g., DLNA remote UI) and authentication via server (e.g., id and password controlled). After device authentication, authenticated devices will be linked to the STB.

## User authentication

The advanced cable STB is required to be equipped with the following user authentication functions:

- User authentication using ID and password.

The following authentication methods can optionally be used to provide more advanced services:

- User authentication using image recognition

- User authentication using voice recognition

- User authentication using fingerprint recognition

## Authorization of application execution

Considering the situation that subscribers will be able to obtain both applications developed specifically for cable television services and applications available on the open application markets, it is required to define functions that can be accessible only by the privileged applications from the security point of view such as service and content protection. Thus, the advanced cable STB is required to be equipped with mechanisms to authorize the applications accessible to significant functions including broadcasting-related components.

# Hybrid services of broadcasting and web

## Cable portal screen

The cable portal screen is the initial screen, provided by cable operator, for subscribers to enter TV broadcasting, IP content delivery such as VOD and IP interactive services including web browsing, and hence it is very important for cable television services. The portal screen will also be applicable to various advertising services because it can show banner ads, picture ads, movie ads, etc. It will also provide customization of the portal screen using with personal devices to enable quick service navigation based on viewing history of the subscriber. Refer to Clause 9 for the description of display functionalities and browsers.

Although the cable portal screen is supposed to be initially displayed on the TV screen when the advanced cable STB is turned on, the STB is required to allow the subscriber to choose whether to display the last channel initially or to show the cable portal screen at the time of turn-on of the STB.

The advanced cable STB is required to support the following installation methods of the cable portal function:

Pre-installation of the default portal function to the advanced cable STB before delivery to the user.

Automatic download of the cable portal function from the head-end via the IP network after deployment of the STB.

Download of the cable portal function with user’s intention from the cable operator’s application server.

## Broadcasting content

The advanced cable STB is required to use full-screen mode when the STB only shows one broadcasting content.

## Simultaneous display of IP interactive services and broadcasting content

The advanced cable STB is required to support simultaneous display of broadcasting content and IP interactive services which may contain graphic element such as buttons and icons. Size and position of the broadcasting content may be controlled by the IP interactive services, or by other means to separate their areas for distinction of their own context. However, in any case, the STB is required to comply with regulations and/or guidelines for simultaneous display defined by authorities in each region to the extent defined by such regulations and/or guidelines as their scope.

## Overlaying on the broadcasting content

When any information is overlaid on the broadcasting content, the advanced cable STB is required to comply with regulations and/or guidelines defined by authorities in each region to the extent defined by such regulations and/or guidelines as their scope.

## Services associated with EPG

The advanced cable STB is required to support services associated with EPG. The STB is required to support the application software that search and display related web information based on the attribute information obtained from the EPG.

# Download function for system software/firmware update

The advanced cable STB is required to support the following download methods for the system software/firmware update:

- download via broadcasting RF signals

- download via IP network, i.e., wired LAN or wireless LAN

The STB is required to be capable for the software download regardless of its power on/off state.

## Download methods for system software/firmware

If the software is downloaded by broadcasting RF, the advanced cable STB is required to have the capability to receive Engineering Stream Service defined in [ITU-T H.222.0] while the STB is in power-off state. The STB is required to support both global head-end insertion and local head-end insertion of the engineering stream. If the STB downloads the software in power-on state, the STB is required to show the warning to the user.

If the system software/firmware is downloaded by IP, the STB is required to support the download initiated by the cable operator (push download), and software download initiated by the customer support representative at subscriber’s premises (pull download). The STB is also required to allow the customer support representative to put software image into STB from the external storage (USB, etc.). After the system software/firmware update, the STB is required to show the proper notification for the necessary user action (e.g. power-off and on for reboot).

The advanced cable STB is required to allow the user to choose the behavior of operator initiated download as follows:

- Automatic downloading

- Showing notification for downloading each time

## Process in case of download failure

The advanced cable STB is required to stop the software update and rollback to the previous version when the software download is not completed normally.

## Scheduling of download and report of result

The advanced cable STB is required to provide scheduling of download  and report of the result.

- It is required to schedule the download and report the result as logs of the STB. It is also required to allow user to see these logs, if these logs includes abnormal termination, error message with error code is required to be included.

## Indication of system software/firmware version

The advanced cable STB is required to allow subscriber to see the device manufacturer ID, model name, model number and software version on the TV screen.

# Download function for application software

There are two types of download method for application software. One is the operator initiated application download which is deployed to the multiple addressing by cable operator, the other is the user initiated application download which is downloaded from the download server corresponding to the individual subscriber’s request.

## Operator initiated application download method

The advanced cable STB is required to support the automatic download of the application software without user’s awareness. Typical use case of this functionality is for the deployment of new services. After the download of new service application, the STB is required to notify it on the service portal screen.

## User initiated application download method

When subscriber accesses the application download site and selects the software from the catalogue, the advanced cable STB is required to download the selected application. The STB is required to support the application with expire date and one time application in addition to the normal application.  Regarding the download site, the advanced cable STB is required to support both operators’ download site and download site of third parties.

## Indication of application software version

The advanced cable STB is required to enable subscriber to easily confirm the software version on the TV screen.

## Subscriber authentication and authorization for pay application software download

In case of pay software download, the advanced cable STB is required to authorize  subscriber by coordination with the platform. The subscriber will be authorized based on the individual contract between the cable operator and the subscriber. With the normal completion of the download, billing process is conducted at the cable operator’s platform. The STB is required support the capability to show the billing result to the subscriber clearly.

# Home network connection

The advanced cable STB is required to support various services using HN such as content sharing with terminal devices connected to HN.

## Transport for HN

The advanced cable STB is required to support wired and wireless IP transport methods, e.g., Ethernet, WiFi (IEEE 802.11a/b/g/n). The STB is recommended to support RF-based transport method to communicate with non-IP devices within home, e.g., ZigBee.

## Routing function

The advanced cable STB is required to support IP routing function between WAN and LAN..

## Protocols

The advanced cable STB is required to support UPnP connection among IP devices in HN. The latest version of the DLNA standard is required to transfer audio and video content within HN. It is required to adopt the latest version of DTCP-IP for device authentication and content protection. Proprietary protocol can be used optionally for the transmission of specific information with such devices in HN that only support non-IP protocols.

# Management functions

## Provisioning function

The advanced cable STB is required to provide provisioning function in coordination with the headend system. It is also required to support SNMP and TR-069 standard of the Broadband Forum (BBF).

## STB remote management

The advanced cable STB is required to enable cable operator to manage terminal devices remotely in coordination with the headend system. The STB is required to use BBF TR-069 standard to acquire the STB operation status and control the STB. It is also required to apply SNMP using cable specific MIB to acquire and set parameters to STB specific information for backward compatibility with the existing cable network’s management system.

The advanced cable STB is required to restore to normal operation  to possible extent from head-end by means of BBF TR-069 reset function if the STB falls in improper operation.

The STB is required to maintain broadcasting related services such as watching, recording, scheduled recording regardless of failure of, for example, communication function (DOCSIS, Ethernet and WiFi) and application platform.

## Management of Subscriber information

The advanced cable STB is required to support a gathering function of the subscriber information including viewing history. The STB is also required to protect the collected information. The STB is required to protect transport for sending the collected information to the cable operator.

[Editor’s note: Need to define “Personal Information” (or refer to H.iptv-am.0). The following text should be modified to be aligned with the definition of “Personal Information.”]

It is also required that personal information to be eliminated easily either by the cable operator or the subscriber.

.

## Monitoring of reception quality

The advanced cable STB is required to support function for monitoring the reception signal quality and reporting the result to the head-end system.

# Power saving

The advanced cable STB is recommended to have four (4) power states: Power-on, Function stand-by, Stand-by, and Power-off for energy saving. The STB is required to enable subscriber to set power status. In the Function stand-by state, remote scheduled recording and reception of emergency alert signal shall be supported. EPG information and RCU signal reception are also required in the Stand-by state.

The advanced cable STB is required to have a power ON/OFF communication function with the TV receiver. The STB is required to allow other external device to control the power status of the STB via Wi-Fi/LAN within the HN.

# Mechanical requirements

The advanced cable STB is recommended to have a mechanical structure for space saving, simple operation and cost reduction. The requirements are as listed below:

External adapter type power source is recommended for space saving.

Card slots for conditional access shall be provided.

An LED shall be mounted on the front panel for indication of operation stage. The advanced cable STB is required to have at least one LED to indicate its status. The STB is also required to receive signals from IR type RCU.

The meaning, method and color of the LED shall be left to manufacturer’s product planning.

The following switches shall be provided for direct control of the STB
Power on/off, hardware reset, channel up/down, selection of broadcast types (e.g. multi channel video reception or re-transmission)

The input and output interfaces listed in Clause 12 shall be provided.

It is recommended to support both vertical and horizontal placement.

[Note] The requirement for the content recommendation is expected as the future input and/or future draft new recommendation. Therefore, the deletion of Appendix 1 is proposed.

[Note] Basic requirements for the audience measurement are already defined in H.iptv-am.0 and H.iptv-am.1. Therefore, the deletion of Annex.2 is proposed.

Cable network and service delivery

Cable network architecture

The cable network architecture including the advanced cable STB is shown in Figure Ap.1-1. It is configured to distribute contents (audio and video) and services (broadcast and interactive) to the subscriber through the cable network where the transmission quality is guaranteed. However, if the source of the content is via the Internet, the quality can not be guaranteed due to the best effort connection.

Figure Ap.1-1  Cable network architecture

Type of service delivery

Figure Ap.1-2 shows the type of service delivery using the advanced cable STB.

(1) 	Multi-channel, re-transmission of digital broadcasting and VOD / PPV service

(2) 	Scheduled recording service linked with mobile device

(3) 	Distribution of cable content through the mobile communication network to the terminals of cable subscribers (this service bypasses the advanced cable STB).

(4) 	Distribution of IP services through the Internet

(5) 	Collaboration service with other platforms

Figure Ap.1-2: Type of Service delivery

Use case

Expected use cases for advanced cable STB are described below.

Use Case1: Seamless multi device TV watching experience

- This use case means seamless video content delivery without noticing origin point of content or delivery method between different devices by the collaboration of VOD server, Set-Top Box, smart phone and tablet devices in the cloud network.

- Let suppose that multi-channel broadcast content server, streaming server for multi-device in the network side and Set-Top Box, smart phone, tablet devices in the home network are connected as shown in Figure 2. If user’s information on viewing time, i.e. content resume point, becomes common in the network, it will be able to resume the content on the screen of different device seamlessly without noticing delivery route.

- In case that a user is watching recorded content on TV screen and then moves to user’s room to watch the remaining content in the display of smart phone, the smart phone disseminate request command to Set-Top Box. Upon reception of the command, the Set-Top Box begins trans-coding from the resume point then deliver content toward smart phone, which means a seamless video content delivery service. The resume point is shared in the network as the common information in each content and each user. The content resume service will be available not only in home but also in out-door through the mobile network in which streaming video service is available by smart phones or tablet devices. The associated services such as authentication between devices, parental lock, etc. shall be provided as a set of functions of video delivery server under the viewing situation in which a common Set-Top Box and multiple smart phones existed.

- In addition, a collaboration service will be available for audio content which means music program delivery to the speakers on TV from smart phone by DLNA media transition.

- Figure Ap. 2-1: Seamless multi device TV watching experience

Use Case2: Social media assisted TV watching

- This use case enables value-added services for content viewing by showing viewer’s real-time impression and reputation regarding current program on the screen of user’s device (i.e. tablet device or smart phone) which has authentication-coupling with Set-Top Box. As shown in Figure 3 social media coordination server collects user’ s impression and/or tendency of comments regarding program from Microblog, Blog and SNS, etc, based on Blog topics or hash-tags. In user’s home, tablet device or smart phone catches the present channel information of Set-Top Box and gets comments and statistics data from social media coordination server. In progression of program these real-time comments and/or impressions are collected and displayed on TV screen with statistic data.

- Other usage of social media is a text message transaction through TV screen by sharing information (presence) of watching program among user’s friend and kin. This enables recommend service to the program or to friend of VOD, and sharing impression service on viewing program.

- Figure Ap.2-２: Social media assisted TV watching

Use Case3: Livelihood support service

- This use case describes livelihood support service utilizing the Set-Top Box and cable television network. Figure 4 shows a remote health care service using home health care devices. The data of manometer (brood pressure), bathroom scale (body weighing) and pedometer are collected in Set Top Box through WiFi or Bluetooth system and presented to user on TV screen. At the same time Set-Top Box delivers collected data to Hospital, Pharmacy and Health consultant that relate to the user through the personal data server. Based on the stored data in the server, various services such as health consultancy, remote diagnosis-aid will be available utilizing TV telephony system provided by cable network.

Figure Ap.2-3: Livelihood support service – Health Care

Use case 4: Recommendation service

- This use case assumes that the cable operator will provide a recommendation service to the subscriber by acquiring and utilizing various information on the user to recommend content and goods which are most suited to subscriber (means personalized recommendation). It is also recommended to deliver following services without using subscriber’s personal information, where the subscriber does not explicitly indicate the conditions of recommendation.

- Ranking

- Highly recommended by the shop

- Goods recommendation linked with viewing history

- Meta-data related content (director, artist, cast links, etc.)

- Recommendation by friends

- Recommendation from other users

Based on the watching history obtained from the advanced cable STB which may include subscriber’s tastes, after keyword search and data aggregation by the search engine and weighting by the cable operator’s business rules (e.g. delivery of new titles first), the recommendation service is distributed to the subscriber. Figures Ap.2-4 and Ap.2-5 show sample screen for content recommendation and good recommendation, respectively.

- Figure Ap.2-4: Sample screen of a content recommendation service

- Figure Ap.2-5: Sample of a goods recommendation screen

# Bibliography

[b-TR-069] Broadband Forum TR-069 (2007), CPE WAN Management Protocol v1.1

[b-JCL SPEC-001] Version 1.01（2008/10）, Operational specifications for BS digital Satellite broadcasting trans-modulation

[b-JCL SPEC-003] Version 2.1（2010/3）, Operational specifications for re-max digital broadcasting（Independent broadcasting）

[b-JCL SPEC-004] Version 2.1（2010/3）, Operational specifications for re-max digital broadcasting（i-HITS）

[b-JCL SPEC-005] Version 2.2（2010/6）, Operational specifications for JC-HITS trans-modulation

[b-JCL SPEC-006] Version 2.01（2008/10）, Operational specifications for digital terrestrial television broadcasting pass through

[b-JCL SPEC-007] Version 2.01（2008/10）, Operational specifications for digital terrestrial television broadcasting trans-modulation