| Title*: | DTSEE-00014 ver. 1.1.6 |  |  |
| --- | --- | --- | --- |
| from Source*: | Rapporteaur |  |  |
| Submitted To*: | EE EEPS |  |  |
| Document for*: | Decision |  |  |
|  | Discussion | X |  |
|  | Information |  |  |
| Submission date*: | 2011-02-16 |  |  |
| Agenda Item: |  |  |  |
| Contact: | Anders Andrae |  |  |
| Relevant WI(s), or deliverable(s): |  |  |  |

| Relevant WI(s), or deliverable(s): | DTS/EE-00014 |
| --- | --- |

Draft ETSI DTS/EE 00014 V1.1.6

Environmental Engineering (EE); LCA assessment of ICT equipment, ICT network and service: General definition and common requirement

Reference

DTS/EE–00014

Keywords

Life Cycle Assessment, ICT Equipment, ICT Networks, ICT Services

ETSI

650 Route des Lucioles

F–06921 Sophia Antipolis Cedex – FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

Individual copies of the present document can be downloaded from:
http://www.etsi.org

The present document may be made available in more than one electronic version or in print. In any case of existing or perceived difference in contents between such versions, the reference version is the Portable Document Format (PDF). In case of dispute, the reference should be the printing on ETSI printers of the PDF version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at http://portal.etsi.org/tb/status/status.asp

If you find errors in the present document, please send your comment to one of the following services:
http://portal.etsi.org/chaircor/ETSI_support.asp

Copyright Notification

Reproduction is only permitted for the purpose of standardization work undertaken within ETSI.
The copyright and the foregoing restrictions extend to reproduction in all media.

NOTE:	This paragraph should be used as 1st paragraph within the copyright text for deliverables processed before WG/TB approval and used in meetings.

© European Telecommunications Standards Institute yyyy.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM, TIPHONTM, the TIPHON logo and the ETSI logo are Trade Marks of ETSI registered for the benefit of its Members.
3GPPTM is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners.

# Contents

Contents	4

Intellectual Property Rights	5

Foreword	5

Introduction	6

1	Scope	7

2	References	7

2.1	Normative references	7

2.2	Informative references	7

3	Definitions, Symbols and Abbreviations	7

3.1	Definitions	7

3.2	Symbols	9

3.3	Abbreviations	9

4	General Description	10

5	Methodological framework	10

5.1	General requirements	10

5.2	Goal and Scope definition	11

5.2.1	Functional unit	11

5.2.1.1	General	11

5.2.1.2	ICT equipment	11

5.2.1.3	ICT Networks	12

5.2.1.4	Services	12

5.2.2	System boundaries	12

5.2.2.1	General	12

5.2.2.1.1 The use of unit processes	15

5.2.2.2	ICT equipment	16

5.2.2.2.1	General	16

5.2.2.2.2 Equipment Raw materials acquisition	16

5.2.2.2.3 Production	17

5.2.2.2.4 Use	19

5.2.2.2.5 End–of–life treatment (EoLT)	19

5.2.2.3	ICT Network	20

5.2.2.4	Services	21

5.2.3	Cut–off rules	21

5.2.4	Data Quality Requirements	21

5.3	Life Cycle Inventory (LCI)	23

5.3.1	Data collection	23

5.3.1.1 General	23

5.3.1.1.1 Use of network – data collection	23

5.3.1.2ICT equipment	23

5.3.1.3 Network	23

5.3.1.4 Services	23

5.3.2	Data calculation	23

5.3.2.1 General	23

5.3.2.2 ICT equipment	23

5.3.2.3 Network	23

5.3.2.4 Services	23

5.3.3	Allocation of data	24

5.3.3.1 General	24

5.3.3.2 ICT equipment	24

5.3.3.3 Network	24

5.3.3.4 Services	24

5.4	Life Cycle Impact Assessment (LCIA)	24

5.4.1	Impact categories	24

5.5	Life Cycle Interpretation	24

5.5.1	General	24

5.5.2 	Identification of significant issues	25

5.5.3 	Evaluation	25

5.6	Comparative LCA between ICT and reference technologies	25

6	Reporting	25

6.1	General	25

6.2	ICT equipment	27

6.3	Network	27

6.4	Services	28

7	Critical review	29

Annexes (normative)	30

Annex N1 (Normative)	Generic processes	30

Annex N2 (Normative)	Recommended list of Equipment Raw material acquisition processes	32

Annex N3 (Normative) Part types	35

Annex N4 (Normative)	EoLT processes	37

Annex N5 (Normative) Recommended list of elementary flows in LCA of ICT	38

Appendices (informative)	41

Appendix I1: Life cycle phases overview	41

Appendix I2:  Products and “black box” modules	41

Appendix I3:  Impact assessment method examples	43

Appendix I4:	Examples of Allocation Procedures	43

AI4.1 Examples for Allocation Procedures for Re–use and Recycling of Materials	43

AI4.2 Examples for Allocation Procedures for Transportation	45

AI4.3 Other Examples for Allocation Procedures	45

AI4.3.1 Production process	45

AI4.3.2 Process output allocation	45

AI4.3.3 Waste incineration	45

Appendix I5: Example of use of data quality matrix	45

History	47

# Intellectual Property Rights

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non–members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (http://webapp.etsi.org/IPR/home.asp).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

# Foreword

This Draft TS (DTS) has been produced by ETSI Technical Committee Environmental Engineering (EE).

<Editor’s note:

Color coding used in this draft:

Black text: Agreed standard text

Plum text: Text based on agreed contribution or similar and intended to be Standard text. Proposed by ad hoc group. Not yet reviewed and agreed as text.

Red text: Preliminary text, not based on contribution, not discussed and agreed as text.

Blue text: <Editor’s note: Comment about content or intended content. To be removed before approval of document>

# Introduction

Environmental life cycle assessment (LCA) is a system analytical method and model by which the potential environmental effects related to an ICT Equipment or Network, or Service can be estimated. LCAs have a cradle–to–grave scope where the life cycle phases raw material acquisition, production, use, and end–of–life are included. Transports and energy supply are moreover included in each life–cycle phase.

ISO has standardized the LCA methodology [1], [2]. In this standard, ICT specific additions to the ISO 14040/44 standards will be described. As addition to the ISO 14040/44 standards, the European Commission has published a handbook that gives detailed guidance on all the steps required to conduct an LCA [3]. This handbook will also be referred to with special ICT considerations in mind.

The structure of this LCA methodology standard for ICT Equipment, Networks, and Services shown in Fig. 1Error! Reference source not found..

Fig. 1 Structure of LCA methodology standard for ICT Equipment, Networks and Services

In accordance with ISO 14040/ISO 14044 this standard consists of the following parts

- General requirements: High level requirements of LCA studies

- Goal and scope definition: Requirements on functional unit, system boundaries and data quality

- Life Cycle Inventory (LCI): Requirements on data collection, calculation and allocation

- Life Cycle Impact Assessment (LCIA): Requirements on impact categories

- Life Cycle Interpretation: Requirements on interpretation of results

- Comparative Assertion between ICT and reference technologies

- Reporting: Requirements on reporting

All applicable parts are then divided into the three main applicable product system categories; ICT Equipment, Networks and Services.

The first edition of this standard focuses on the negative and positive impacts of ICT Equipment, networks and services, both. The positive impacts of ICT services are potentially large.

# 1	Scope

The purpose of this standard is to:

-harmonize the LCA studies of ICT Equipment/Networks/Services

-increase the quality of the LCA by adding ICT specific requirements to those of ISO 14040/44

-facilitate communication of LCA studies of ICT Equipment/Networks/Services

-increase the credibility of LCAs of ICT Equipment/Networks/Services

While recognizing ISO 14040/44 as normative references, this standard will give generic requirements for LCA of ICT Equipment/Networks/Services. The standard is valid for all type of Equipment which is/could be part of a Network including end user Equipment. Comparisons of results between studies are beyond the scope of this standard, as such comparisons would require that the assumptions and context of each study are exactly equivalent.

# 2	References

## 2.1	Normative references

- ISO (2006) Environmental Management–Life Cycle Assessment–Principles and Framework. ISO 14040:2006(E), second edition 2006-07-01

- ISO (2006) Environmental Management–Life Cycle Assessment–Requirements and Guidelines. ISO 14040:2006(E), first edition 2006-07-01

## 2.2	Informative references

- European Commission (2010) International Life Cycle Data System Handbook. General guide for life cycle assessment – detailed guidance. First edition. March 12.

- OECD (2009) Guide to measuring the ICT Sector. The 2006-07 OECD ICT sector definition (based on ISIC Rev. 4)

# 3	Definitions, Symbols and Abbreviations

## 3.1	Definitions

<Editor’s note: Available definitions from other standards should be reused by reference as far as possible. >

For the purposes of the present document, the following terms and definitions apply:

“Black box” module: a device, system or object which can be viewed solely in terms of its input, output and transfer characteristics without any knowledge of its internal workings. In this context the “black box” module may consist of several part categories such as integrated circuits, mechanics, cables etc., e. g., a power module on a PCBA

Customer-premises equipment (CPE): any terminal and associated equipment located at a subscriber's premises and connected with a carrier's telecommunication channel(s) at the Network Termination Points (NTPs).

End–user equipment: ICT Equipment and CPE used to access fixed, datacom, broadband, and wireless Network.

Generic data: Data from any relevant data source which need not be specific for ICT applications and processes. Generic data is always secondary.

ICT Equipment: the tangible Equipment deriving from or making use of technologies devoted to or concerned with (a) the acquisition, storage, manipulation (including transformation), management, movement, control, display, switching, interchange, transmission or reception of a diversity of data;  (b) the development and use of the hardware, software, and procedures associated with this delivery; and (c) the representation, transfer, interpretation, and processing of data among persons, places, and machines, noting that the meaning assigned to the data must be preserved during these operations.

ICT specific EoLT: Any disassembly/dismantling/shredding/recycling process which needs special adaptation for handling of ICT equipment

Life cycle phase: one of several consecutive and interlinked stages of a product system

Natural resource: A material source, such as timber, fresh water, or a mineral deposit, that occurs in a natural state and has economic value

Network: a set of nodes and links that provide physical or over the air information and communication connections between two or more defined points.

Network Termination Points (NTP): a point established in a building or complex to separate CPE from telephone company equipment.

Optional activity: a unit process which can be left out of the LCA because of low significance, low precision, general lack of data or other practical reasons

Operator:

Other EoLT: Any disassembly/dismantling/shredding/recycling process which does not need special adaptation for handling of ICT equipment but could be used for any kind of equipment.

Part: Constituent of ICT Equipment and Support Equipment Table A.N 3

Primary data: Data collected/estimated/measured by a researcher, specifically for the LCA project; for example, pilot studies or customer or producer surveys.

Process category:  Table A.N 1, Table A.N 4

Processed materials:

Public data: Data which is available to the public without access being restricted by requirements on membership, none—disclosure agreements or similar.

Raw material: primary or secondary material that is used to produce a Part or Equipment according to ISO14044 Section 3.15

Raw material extraction:

Raw material processing:

Raw material recycling: Production of raw materials from secondary raw materials

Recommended activity: A unit process which is significant to the result and there should be included if the unit process is part of the scope of the LCA study

Secondary data: Data that has been collected for another purpose, but can be analyzed again in a subsequent study; for example, LCA estimates by academia, government, or industry organizations.

Service (Application): the use of ICT Equipment and/or ICT Networks to provide value to one or more users

Specific data: Data emerging from, in this case, ICT specific applications and processes. This data could be either primary or secondary.

Support activity: Activity next to unit processes directly needed to provide the function of the Equipment, Network or Service. Examples are software programming, maintenance, operation, and business travel.

Support Equipment: A device, system or object needed to realize the function of the ICT Equipment. Examples: equipment for power supply and temperature regulation

Traffic intensity:

Unit process: smallest element considered in the life cycle inventory analysis for which input and output data are quantified. Examples: part unit processes in Table A.N 3.

<Editor’s note: Separate contributions, especially from Ericsson, and teleconference on about lifetime definitions.  >

Commercial lifetime: the average time the ICT Equipment is owned by the customer before buying a new one

Aggregated commercial lifetime: commercial lifetime plus secondary re–use.

Lifetime: The length of time the Equipment, Network, Service lasts. Different definitions exist.

Active use period: [Nokia]

Storage time: excluded

Legal lifetime: NTT proposal .accounting law. Depreciation time .

Life time: <Editor’s note: Proposal from Huawei for Effective life of ICT Equipment. Paris Dec. 6-7 2010, could decide this term>

Operating lifetime: Actual time of use. Most users find this lifetime most appropriate for the use phase time. Secondary re–use is excluded.

Service life: the time the equipment is supported by the producer.

Elsewhere as ISO 14040/44 [1][2] or the ILCD Handbook, Table 1 [3].

## 3.2	Symbols

For the purposes of this document, the following symbols apply:

Tbd

## 3.3	Abbreviations

For the purposes of this document, the following abbreviations apply:

3G	Third generation telecom networks

ATM	Asynchronous transfer mode

BHCA	Busy Hour Call Attempt

C2H4e	Ethene equivalent

CAPEX	Capital expenditures

CED	Cumulative Energy Demand

CFC11e	CCl3F equivalent (ChloroFluoroCarbon11)

CO2e	Carbon dioxide equivalent

CN	Core Network

CPE	Customer premises equipment

EHW	Environmentally Hazardous Waste

EIO–LCA	Economic Input Output LCA

EoLT	End of life treatment

ENIG	Electroless Nickel Immersion Gold

FWT

GSM	Global System for Mobile Communications

GPS	Global Positioning System

ICT	Information and Communication Technology

IP	Internet Protocol

IPTV

LAN

LCI	Life Cycle Inventory

LCA	Life Cycle Assessment

LTE	Long Term Evolution

PCB	      Printed Circuit Boards

PCBA		      Printed Circuit Board Assembly

PDA		      Personal Digital Assistant

PO43–e	Phosphate equivalent

PWB		      Printed Wiring Board

RAN		      Radio Access Networks

RBS	Radio Base Station

Sbe	Antimony equivalent

SMT	Surface mounting technology

SO2e	Sulfur dioxide equivalents

TDM	Time Division Multiplex

TOE	Total Oil Equivalent

UE	User Equipment

USB	Universal Serial Bus

UMTS	Universal Mobile Telecommunication Services

# 4	General Description

(According to ISO 14040 Section 4.[1])

This standard will help practitioners to perform and report their LCA studies of ICT equipment, ICT Networks, and Services in a uniform and transparent manner. It is possible to use this document to get guidance on what to consider in an LCA on three levels: ICT Equipment, ICT Network, and Service.

# 5	Methodological framework

## 5.1	General requirements

(According to section 4.1 in ISO 14044 [2])

When performing an ICT related LCA, the requirements of ISO 14040/44 should apply as well as the requirements of this document.

The following life cycle phases (also shown in Fig. 2) in this document apply to ICT equipment/ICT Networks/Services and should be assessed as applicable in LCA studies based on this standard:

A. Equipment Raw material acquisition which is composed by

  - Raw material extraction

  - Raw material processing

B. Production which is composed by

  - ICT Equipment production

  - Support Equipment production

C. Use which is composed by

  - ICT Equipment Use

  - Support Equipment Use

  - Operator activities

D. ICT Equipment End of Life Treatment

  - Re–use

  - ICT specific EoLT

  - Other EoLT

Transports and energy supplies are included in all life cycle phases. It is optional to include the construction of plants in which ICT or Support Equipments are manufactured. The life cycle phases are further described in chapter 5.2.2.

## 5.2	Goal and Scope definition

During the scope phase the building blocks of the ICT Equipment/ICT Network should be identified, preferably from block diagrams provided by system engineers/architects.

Schematically three main levels of targeted product systems exist:

- Equipment (ICT Equipment and Support Equipment)

- Network (ICT Network)

- Services (ICT Service)

All these product systems use ICT Equipment which follows the life cycle phases introduced in 5.1 and further described in this chapter.

Equipment refers to the different physical products constituting the network. A hierarchical view is suitable for describing ICT Networks. At the top level different types of ICT Equipments can be identified, e.g. Network nodes and End–user Equipment. ICT and Support Equipments consist of, e.g., Electronic Parts, Mechanical Parts, Cooling Parts, and Cables.

Routers, soft switches, gateways, and base stations are examples of ICT Equipment. Printed circuit board assemblies (PCBA) and shelves are examples of included Parts. The PCBAs consist of printed circuit boards, integrated circuits, and other Parts.

In summary, any ICT Equipment which can be part of an ICT Network delivering process speech and/or data lies within the scope of this LCA standard.

The following ICT LCA applications are the most frequently used ones, but others may be identified and used as well:

- Benchmarking and comparisons and of specific ICT Equipment, ICT Networks, or Services

- Assessment of product system carbon dioxide impact

- Assessment of primary energy consumption

- Identification of life cycle phases with high significance

### 5.2.1	Functional unit

<Editor’s note: Huawei action: propose appropriate standard text for entire 5.2.1 before March 16, 2011.

#### 5.2.1.1	General

The functional unit requires inclusion of the relevant quantifiable properties and the technical/functional performance of the system. This means that the operational lifetime shall be specified and also the number of users/subscribers and the traffic intensity need to be considered where applicable.

#### 5.2.1.2	ICT equipment

Each LCA study for ICT Equipment could have a different functional unit.

The following basic functional unit should be applied where applicable: Annual equipment use (per one year of typical equipment use).

#### ICT Networks

Each LCA study for ICT Networks could have a different functional unit. The following basic functional unit should be applied where applicable: Annual network use (per one year of network use for a defined purpose)

NSN Annual network use per amount of users

Ericsson: Annual network use per dimension base

The annual environmental load per functional unit for an ICT Network could be expressed as “annual environmental load per phone line, LNW”.

However, the ICT Equipments and Support Equipments have been developed in various forms since the first ICT Network was established. Consequently, the environmental impact assessment is performed separately for each ICT Equipment and Support Equipment composing the ICT Network. The total environmental load of each ICT Equipment and Support Equipment is divided by its operational life time to calculate the annual environmental loadings. Then each of these annual loadings for Equipments is summed up to express the annual environmental load for the entire ICT Network.  If, e.g., the ICT Network requires two phone lines its annual environmental load is LNW×2.

#### 5.2.1.4	Services

Each LCA study for Services could have a different functional unit.  The following basic functional unit should be applied where applicable: Annual average service user (per one year of service use representing a defined usage profile)

Average service user per gigabit…

### 5.2.2	System boundaries

#### 5.2.2.1	General

(This section corresponds to section 4.2.3.3 in ISO 14044).

The system boundaries define the life cycle phases and processes belonging to the analysed product system, i.e., are required for providing its function as defined by its functional unit.

Consequently, the system boundaries here define the life cycle phases and the unit processes that should be taken into account in an LCA study of an ICT product system.

Examples of ICT Equipment product systems are radio base stations and mobile phones etc. On ICT Network level the targeted product system may be, e.g., a certain LTE Network.  On Service level, the targeted product system may be, e.g., a Service for distributing music.

The following basic reference flow units should be used where applicable:

- Raw material acquisition 	one piece of equipment (per one piece of equipment).

- Production:		one piece of equipment (per one piece of equipment).

- Use: 			annual equipment use (per one year of equipment use)

- End of Life Treatment (EoLT): 	one piece of equipment (per one piece of equipment)

Fig. 1 shows the recommended system boundary of an LCA of ICT Equipment/ICT Network/Service which is further specified in Table 1 below. In the figures below dashed lines means that the corresponding life cycle phase is optional.

The boxes for G1–7 in Fig. 1 denote generic processes that reoccur several times during the life cycle. These processes are further defined in Annex N1 (Table A.N 1).

System boundary

Recommended

A: EQUIPMENT RAW MATERIAL ACQUISITION

A1. Raw material extraction

A2. Raw material processing

G1. TRANSPORTS&TRAVEL

B: PRODUCTION

B1. ICT Equipment production

B1.1 Parts production

B1.2 Assembly

B1.3 ICT vendor activities

B2. Support Equipment production

B2.1 Support Equipment manufacturing

B2.2 Site construction

C: USE

C1. ICT Equipment use

C2. Support Equipment use

C3. Operator activities

D: ICT EQUIPMENT END-OF-LIFE TREATMENT

D1. Re–use of ICT Equipment

D2. ICT specific EoLT

D2.1 Disassembly/Dismantling/Shredding

D2.2 Recycling

D3. Other EoLT

G2. ELECTRICITY SUPPLY

G3. FUEL SUPPLY

G4. OTHER ENERGY SUPPLY

G5. RAW MATERIAL AQUISTION

G7. END-OF-LIFE TREATMENT

G7.1 EHW treatment

G7.2 Other waste treatment

G6. SUPPORT ACTIVITIES

Fig. 2 The system boundary of the product system for LCA of ICT equipment/ICT Networks/Services

Table 1 below further details the life cycle phases to be considered in LCA studies of ICT equipment/ICT Networks/Services. The different life cycle phases are further described in chapter 5.2.2.2.2 to 5.2.2.2.5. Recommended in Table 1 means it is recommended to include the life cycle phase. However, this also means that even if, e.g., Parts production (B1.1) is recommended not all Parts given in Annex N3 (Table A.N 3) are applicable to all ICT Equipment. Recommended life cycle phases or unit processes shall not be cut–off before considered.

Table 1 Classification of life cycle phases/unit processes

| Tag | Life cycle phase |  |  | Unit process | Class |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Equipment | Network | Service |
| A | Equipment Raw Material Acquisition |  |  |  |  |  |  |
| A1 |  | Raw material extraction |  |  | Recommended | Recommended | Recommended |
| A2 |  | Raw material processing |  |  | Recommended | Recommended | Recommended |
| B | Production |  |  |  |  |  |  |
| B1 |  |  | ICT equipment production |  |  |  |  |
| B1.1 |  |  |  | Parts production (for further details refer to Annex N3) | Recommended | Recommended | Recommended |
| B1.2 |  |  |  | Assembly | Recommended | Recommended | Recommended |
| B1.3 |  |  |  | ICT vendor activities | Optional | Optional | Optional |
| B2 |  | Support equipment production |  |  |  |  |  |
| B2.1 |  |  |  | Support Equipment manufacturing | Optional (Recommended if Support Equipment is included in the Scope) | Recommended | Recommended i |
| B2.2 |  |  |  | Site construction | Optional | Recommended | Recommended |
| C | Use |  |  |  |  |  |  |
| C1 |  | ICT equipment use |  |  | Recommended | Recommended | Recommended |
| C2 |  | Support equipment use |  |  | Optional (Recommended if Support Equipment is included in the Scope) | Recommended | Recommended |
| C3 |  | Operator activities |  |  | Optional | Optional | Optional |
| D | Equipment End of Life Treatment |  |  |  |  |  |  |
| D1 |  | Re–use of ICT Equipment |  |  | Recommended | Recommended | Recommended |
| D2 |  | ICT specific EoLT |  |  | Recommended | Recommended | Recommended |
| D2.1 |  |  |  | Storage/Disassembly/Dismantling/ Shredding | Recommended | Recommended | Recommended |
| D2.2 |  |  |  | Recycling | Recommended | Recommended | Recommended |
| D3 |  | Other EoLT |  |  | Recommended | Recommended | Recommended |

A more detailed overview (Fig. AI 1), showing the detailed content and connection between all life cycle phases, is shown in Appendix I1.

All phases in the life cycle are associated with different support activities. The term Support activities (G6) refers to, e.g., offices including ICT use for marketing, sales and R&D, and also to business travel, commuting and service vehicles

Support activities (G6) are optional to include but it is recommended to include them where possible.

The extent in which Support activities (G6) and other Optional activities (Table 1) are included for different parts of the life cycle should be clearly described in the study report. For further details refer to chapter 6.

Above in Table 1 “Recommended” means that the life cycle phase, if applicable, should always be taken into account in an LCA study for ICT. The optional life cycle phases, however, may be left out of the study depending on study scope etc. For definition of generic and specific data see Section 3.1.

##### 5.2.2.1.1 The use of unit processes

Each life cycle phase (A–D) is further refined into activities, referred to as unit processes, which represent the basic physical flows (materials and energy) of the life cycle.

A unit process typically represents a site where a product is produced, but it can also represent e.g. an office or a store - or even an activity or a place where a service is produced. A unit process can also be a vehicle or a “mobile site” that transports products. Non–production sites are especially important to the ICT sector [4] as a large part of the total work is related to research and development (including software), operation, maintenance etc.

Any unit process can be modelled as shown in Fig. 3 below. The generic unit process model includes a number of inputs and outputs and can be referred to as a site LCI model or – shorter – a site model.

Fig. 3 The generic unit process model

Emissions to the environment and impact on or use or depletion of resource objects are referred to as elementary flows. All other inputs and outputs are defined as product flows.

Each input of fuel and products, as well as each output of waste, can involve transports which are then part of the input/output data connected to the unit process.

Applicable support activities should also be considered for a production site if applicable.

Emissions (elementary flows)

The following emissions should be considered for ICT:

- Emissions to air

- Emissions to water

- Emissions to ground

Non–material emissions like radiation, odour, and noise need not be considered. Direct impact on health can also be left out of the scope.

Resource objects (elementary flows)

The following resource objects should be considered for LCAs of ICT:

- Land use (or land depletion)

- Material resource use (or material depletion)

- Energy resource use (or energy resources depletion)

- Fresh water use (or fresh water depletion)

Species, biodiversity or eco-system depletion need not be considered as being complex and difficult to directly connect to a unit process.

Esthetical values can also be a resource object.

A list of emissions and resource objects that have been found to be of importance for LCA of ICT products, systems and services can be found in Table A.N 5.

Energy, product and services inputs

Further, the following inputs should also be considered for ICT:

- Electricity

- Other forms of delivered energy (district heating and cooling)

- Fuels (typically indicates the fuels are incinerated on-site or in a vehicle connected to the site)

- Primary products (products that are part of the final product in operation)

- Secondary products (products that are not part of the final product in operation)

- Transports, travel and other services (can be seen as a special non–material secondary product input.)

Product, water and waste output

Finally, the following flows should also be considered:

- Water discharge (to municipal sewage or recipient)

- Waste fractions (residual waste fractions or waste fractions that needs further treatment, also including material recycling and energy recovery)

- Product output (the main purpose with the unit process or activity)

A recommended list of energy, product and service inputs that have been found to be of importance for LCA of ICT products, systems and services can be found in Appendix N2 (Table A.N 2).

An informative list of typical ICT Equipment, Support Equipment and ICT Services can be found in Appendix I2.

#### 5.2.2.2	ICT equipment

##### 5.2.2.2.1	General

##### 5.2.2.2.2 Equipment Raw materials acquisition

Equipment Raw Material Acquisition (A) starts with the extraction of natural resources (e.g., iron ore, crude oil, etc.) and ends with the transport of Raw Materials from Raw materials processing (A2) to Part Production sites. A2 deals with the processing of extracted Raw Materials (e.g., iron ore pellets) into finished Raw Materials (e.g., steel sheet, copper wire, etc.). Raw Material Acquisition is the life cycle phase for ICT Equipment as defined in Fig. 2.

As Raw Materials are used as additives in every life cycle phase, Raw Material acquisition can additionally be a generic process (G5).

Annex N2 (Table A.N 2) provides a recommended set of Raw Materials (both ICT specific and generic) which should be included in the LCA study of ICT equipment.

As shown in Fig. 4,Raw Material Extraction and Raw Material Processing are within the system boundary of Raw Materials Acquisition

System boundary

Optional

Recommended

A1. Raw material extraction

A2. Raw material processing

B. PRODUCTION

G5.1 RAW MATERIAL RECYCLING

Fig. 4 The system boundary of Equipment Raw material acquisition in LCA of ICT equipment

##### 5.2.2.2.3 Production

The Production (B) starts with the Parts Production, and ends with the transport of ICT Equipment and Support Equipment to Use (C). The system boundary for Production is shown in Fig. 5 and within it are ICT Equipment Production and Support Equipment Production.

In case Support equipment is part of the targeted product system, Support Equipment Production (B2) is recommended within the system boundary of the study.

For many studies B2 may not be included as the variance in solution is very large both for B2.1 and B2.2 between markets and operators. However, for studies referring to specific conditions it is encouraged to include also B2 within the system boundary as it can have a significant impact on the use phase.

System boundary

Optional

Recommended

B2.1 Support equipment manufacturing

C.USE

A. EQUIPMENT RAW MATERIAL AQUISITION

B2.2 Site construction

B1.3 ICT vendor activities

B1.2 Assembly

B1.1 Parts production

B1.1.1 Batteries

B1.1.2 Cables

B1.1.3 Electro-mechanics

B1.1.4 Integrated circuits

B1.1.5 Mechanics / Materials

B1.1.6 Displays

B1.1.7 PCBs

B1.1.8 Other PCBA components

B1.1.9 “Black box” modules

Fig. 5The system boundary of Production in LCA of ICT equipment

The ICT Equipment Production (B1) consists of Parts Production (B1.1) and Assembly (B1.2) (both recommended), and ICT Vendor Activities (B1.3) (optional). B1.3 is an application of G6.

Annex N3 lists a recommended set of Parts to be considered where applicable, when performing an LCA of ICT Equipment, as well as examples of unit processes for these. Except for Parts listed in Annex N3, other Parts may be as important and should be considered as well. As an example, if batteries are part of the ICT Equipment they should be included in the study, and for every battery the Battery Cell manufacturing and Battery module manufacturing should be considered.

Note that Parts can be complex modules themselves consisting of several other Part types as building blocks.

The Assembly (B1.2) mainly includes PCBA Module Assembly, Final Assembly, Testing & Repair, Warehousing and Packaging.

Examples of Support Equipment (B2.1) are air conditioners, cables, and power supply systems.

As stated above in Table 1 Site construction (B2.2) is optional even if Support Equipment (B2.1) is included. Depending on the specific case, a site can be pre–produced or constructed on place. Examples of Site building blocks needed for B2.2 are towers, fences, and shelters.

Support activities for ICT vendor (B1.3) are specifically indicated in Fig. 5. Regarding other support activities for Support equipment production and Parts Production, please refer to the general principles in 5.2.2.1

##### 5.2.2.2.4 Use

The Use phase starts with the Installation (optional) of ICT Equipment and Support Equipment and ends with the de–installation (optional) just before the transport to EoLT. As shown in Fig. 6, the use phase includes ICT Equipment Use (recommended) (C1), Support Equipment Use (C2), and Operator activities (optional) (C3).

(C1) and (C2) mainly deal with energy supply during the operating lifetime of the ICT Equipment.

Operator Activities include, e.g., installation and de–installation of ICT Equipment and operation and maintenance of the ICT Equipment and site, including associated transports and travel.

System boundary

Optional

Mandatory

C3. Operator activities

D. EoLT

B.PRODUCTION

C2. Support Equipment Use

C1. ICT Equipment use

Fig. 6 The system boundary of Use in LCA of ICT equipment

##### 5.2.2.2.5 End–of–life treatment (EoLT)

EoLT starts with the transport of de–installed ICT Equipment and Support Equipment to  Storage/Disassembly/Dismantling/Shredding facilities, and ends with the waste treatment of ICT Equipment and Support Equipment.

Raw Material Recycling (G5.1) starts after the EoLT.

As shown in Fig. 7, Re–use of ICT Equipment (D1), ICT specific EoLT (D2) and Other EoLT (D3) are within the recommended system boundary for EoLT.

ICT specific EoLT is applicable to the ICT Equipment itself and may also apply to some Support Equipment which is made of ICT Parts.

Other EoLT mainly deals with Support Equipment not made of ICT Parts.

System boundary

Optional

Recommended

D3. Other EoLT

C. USE

D1 Re–use of ICT Equipment

D2 ICT specific EoLT

D2.1 Storage/Disassembly/Dismantling/Shredding

D2.2 Recycling

D2.2.1 Battery recycling

D2.2.2 PCBA recycling

D2.2.3 Cable recycling

D2.2.4 Mechanics recycling

G5.1 RAW MATERIAL RECYCLING

Fig. 7 The system boundary of ICT Equipment EoLT in LCA of ICT equipment

The ICT specific EoLT (D2) in LCA of ICT Equipment includes Transport from Use to Storage/Disassembly/Dismantling/Shredding processes (D2.1), and Recycling Processes D2.2.1–4 for Batteries, PCBAs, Cables, and Mechanics.

The output from these Recycling Processes is not Raw Materials but rather products which the Raw Material Recycling (G5.1) can use.

It has to be judged from case to case which treatments (PCBA recycling etc.) apply to ICT Equipment and Support Equipment, respectively.

Annex N4 lists a recommended set of EoLT processes to be considered where applicable when performing an LCA of ICT Equipment which includes the EoLT phase

#### 5.2.2.3	ICT Network

<Editor’s note: The system boundary setting for an ICT network is done for each individual ICT Network building block (within the Goal&Scope of the LCA) according to Section 5.2.2.2. The ICT Network usually consists of five groups (ICT Network building blocks, E1–E5) of ICT Equipment: E1. End–user, E2. Home, E3. Access, E4. Data transport and E5. Data centres

An ICT Network is an ICT based infrastructure which offers the possibility to transfer speech and/or data between different access points, usually referred to as nodes, and further on to the end–users (represented, e.g., by a telephone or PC). Ultimately the total ICT Network may be studied, taking into account both wireline and wireless ICT Networks and the connection between them - however a study may also focus on just a part of the ICT Network.  In the goal&scope phase it should be outlined which ICT Network building blocks are covered.

For the ICT Network under study applicable types of nodes and infrastructure, as well as amounts of these, need to be defined. Thus the aggregated impact of an ICT Network usually equals the sum of the impact from the different Equipment constituting the ICT Network. When aggregating results it is important to consider that the data for different nodes is based on equivalent assumptions/use scenarios etc.

As the ICT Network operation depends on the software program needed to run its basic function, as outlined in applicable standards, e.g. 3GPP for LTE, the impact from development of such software needs to be considered.

#### 5.2.2.4	Services

When studying a Service there are both positive and negative environmental aspects to take into account.

Starting by the impact from the Service as such it is associated with using ICT Equipment or ICT Network for a certain purpose. Thus, to calculate the impact, it is necessary to assess the Network or Equipment, as outlined in previous paragraphs, and if necessary (i.e. in a multi–service situation) allocate an appropriate amount of this impact to the service under study.

In addition to the use of ICT Equipment/ICT Network a Service may also have impacts associated with the application software development, use of consumables, sales and logistics infrastructure, associated travels and transports (in addition to those already considered for the ICT Equipment/ICT Network) which should also be considered.

If the Service offers the possibility to replace an already existing service (i.e. an e–health solution replacing hospital visits), a comparative study that considers the reduced impact from this change should also be performed to give a correct level of impact of the service.

### 5.2.3	Cut–off rules

<Editor’s note: Huawei action: propose appropriate standard text for entire 5.2.3 before March 16, 2011.

(This section corresponds to section 4.2.3.3.3 in ISO 14044 [2]).

Internally the cut–off recommendations as described in ISO 14044 Section 4.2.3.3.3 shall be used as closely as possible. The cut–off criteria used within a study should be clearly understood and described. Especially, the cut–off made by excluding unit processes within the Recommended system boundary (Table 1) need to be estimated.

Qualitatively, the cut–off for ICT Equipment/ICT Networks/Services LCAs, is a list of the unit processes within the Recommended system boundary (Table 1) which have been excluded.

### 5.2.4	Data Quality Requirements

< Editor’s note: This is an important part which needs to be agreed upon. The following perspectives need to be considered

Source of data (specific/generic)

Age of data

Uncertainty analysis based on Data Quality Management?

Etc.>

For each unit process (Table 2) it should be stated if generic data or specific data were used.

Table 2 Applicable data types per life cycle phase/unit processes

| Tag | Life cycle phase |  |  | Unit process | Type of data |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Equipment | Network | Service |
| A | Equipment Raw Material Acquisition |  |  |  |  |  |  |
| A1 |  | Raw material extraction |  |  | Generic data | Generic data | Generic data |
| A2 |  | Raw material processing |  |  | Generic data | Generic data | Generic data |
| B | Production |  |  |  |  |  |  |
| B1 |  |  | ICT equipment production |  |  |  |  |
| B1.1 |  |  |  | Parts production (for further details refer to Annex N3) | Specific data | Specific data | Specific data |
| B1.2 |  |  |  | Assembly | Specific data | Specific data | Specific data |
| B1.3 |  |  |  | ICT vendor activities | Specific data | Specific data | Specific data |
| B2 |  | Support equipment production |  |  |  |  |  |
| B2.1 |  |  |  | Support Equipment manufacturing | Specific data: Amounts etc. Generic data: processes | Specific data: Amounts etc. Generic data: processes | Specific data: Amounts etc. Generic data: processes |
| B2.2 |  |  |  | Site construction | Specific data: Amounts etc Generic data: processes | Specific data: Amounts etc Generic data: processes | Specific data: Amounts etc Generic data: processes |
| C | Use |  |  |  |  |  |  |
| C1 |  | ICT equipment use |  |  | Specific data | Specific data | Specific data |
| C2 |  | Support equipment use |  |  | Specific data | Specific data | Specific data |
| C3 |  | Operator activities |  |  | Specific data | Specific data | Specific data |
| D | Equipment End of Life Treatment |  |  |  |  |  |  |
| D1 |  | Re–use of ICT Equipment |  |  | Specific data | Specific data | Specific data |
| D2 |  | ICT specific EoLT |  |  | Specific data | Specific data | Specific data |
| D2.1 |  |  |  | Disassembly/Dismantling/ Shredding | Specific data | Specific data | Specific data |
| D2.2 |  |  |  | Recycling | Specific data | Specific data | Specific data |
| D3 |  | Other EoLT |  |  | Generic data | Generic data | Generic data |

Below in Table 3 is shown an example of a data quality matrix. The matrix should be used for those processes where LCI databases were not used.

Table 3 Matrix for data quality assessment. 5 = maximum quality and 1 = minimum quality.

Source: Toffel MW, Horvath A (2004) Environmental Implications of Wireless Technologies: News Delivery and Business Meetings. Environ Sci Technol 38(11):2961–2970

<Examples of application of Table 2 is found in Appendix I3 as Table A.I 2>

## 5.3	Life Cycle Inventory (LCI)

<Editor’s note: >

(According to ISO 14044 Section 4.3.1 [2])

### 5.3.1	Data collection

<Editor’s note: The data collection should be performed according to ISO 14040/44. Specify mandatory LCI flows, for example CO2, CH4. Specific versus generic data>

Practically, when working with certain LCA software programs, e.g., Transports&Travel (G1) and energy supplies (G2–G4) could be included in larger data sets, whereas other LCA software programs/databases provide transports/energy supply separately. The user should report for which processes transports/energy supplies have been added separately and for which they are “hidden” in “black box”.

#### 5.3.1.1 General

(According to ISO 14044 Section 4.3.2 [2])

See section 5.3.2.

##### 5.3.1.1.1 Use of network – data collection

See section 5.3.2.1.

#### 5.3.1.2ICT equipment

#### 5.3.1.3 Network

#### 5.3.1.4 Services

### 5.3.2	Data calculation

<Editor’s note: The data calculation should be performed according to ISO 14040/44, for relating data to functional unit, refining system boundaries.

It is important to consider and define any use of scenarios>

#### 5.3.2.1 General

#### 5.3.2.2 ICT equipment

#### 5.3.2.3 Network

#### 5.3.2.4 Services

### 5.3.3	Allocation of data

<Editor’s note: This is a major issue especially for services and need to be agreed upon e.g. allocation of common infrastructure etc. to the specific scenario/service. Expected contribution from NTT on network allocation procedure including EoLT>

#### 5.3.3.1 General

(According to ISO 14044 Section 4.3.3 [2])

Allocation of Flows and Releases

(According to ISO 14044 Section 4.3.4 [2])

In Appendix I4 are shown several examples of allocation procedures.<Editor’s note: These examples need modification to illustrate the standard for ICT>

#### 5.3.3.2 ICT equipment

#### 5.3.3.3 Network

#### 5.3.3.4 Services

# 5.4	Life Cycle Impact Assessment (LCIA)

<Editor’s note: This item would be focused on impact categories such like GHG emissions, energy consumption. Other potential areas are:

General

(According to ISO 14044 Section 4.4.1)

Elements of LCIA

(According to ISO 14044 Section 4.4.2)

Below in Table A.1 is shown the LCI flows that should be provided by the external LCA report.

Limitations of LCIA

(According to ISO 14044 Section 4.4.3)

Additional LCIA Data Quality Analysis

(According to ISO 14044 Section 4.4.4)

LCIA Intended to be used in Comparative Assertions Intended to be disclosed to the Public

(According to ISO 14044 Section 4.4.5).>

### 5.4.1	Impact categories

<Editor’s note: Applicable impact categories and sources to be defined>

## 5.5	Life Cycle Interpretation

< Editor’s note: Definition of any ICT specific interpretation rule to be stated>

### 5.5.1	General

(According to ISO 14044 Section 4.5.1)

Conclusions, Limitations and Standards

(According to ISO 14044 Section 4.5.4)

### 5.5.2 	Identification of significant issues

(According to ISO 14044 Section 4.5.2)

### 5.5.3 	Evaluation

(According to ISO 14044 Section 4.5.3)

## 5.6	Comparative LCA between ICT and reference technologies

<Editor’s note: Handling of technology development for ICT and conventional services to be discussed. This chapter is composed by several contributions and need further editing to become proposed standard text>

# 6	Reporting

<Editor’s note: Recommended requirements on reporting format to be agreed upon and described in this chapter. Fig. 2-Fig. 7and text is used as a basis for further work. According to ISO 14044 Section 5.1.   >

## 6.1	General

The type and sources of data for raw materials (examples Table A.N 2) used in the LCA study and given in should be provided in the LCA report.

The type and sources of data for Support Equipment Production should be provided in the LCA report. DATAKAPITLET!

Table 4 below shows an example of reporting the assumptions for LCA of ICT services.  This example shows e.g. for that End—user equipment 2 kg is allocated to service under study and 93 kg to all other services provided by the network.  The total sum is the same as for the Network which delivers the Services. The environmental load matrix can be used to report parameters for ICT services (here an example is given in Table 3 for total bandwidth required (and time used) for all services provided by the network). These parameters are different for different ICT services.

Table 4 Matrix for reporting of assumptions in LCA of ICT services

| Coefficient Matrix |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P1 | P2 | P3 | P4 | P5 | P6 | P7 | P8 | P9 | P10 | P11 | P12 | P13 | Total Sum |
| M1 | 1 | 0 | 0 | 0 | 0 | 0 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| M2 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | -1 | 0 | 0 | 0 | 0 | 0 | 0 |
| M3 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | -1 | 0 | 0 | 0 | 0 | 0 |
| M4 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | -1 | 0 | 0 | 0 | 0 |
| M5 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | -1 | 0 | 0 | 0 |
| M6 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | -1 | 0 | 0 |
| M7 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | -1 | 0 |
| M8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | -1 | 0 |
| M9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | -1 | 0 |
| M10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | -1 | 0 |
| M11 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | -1 | 0 |
| M12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | -1 | 0 |
| M13 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| Environmentalload matrix |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Load (e.g., CO2) | 2 | 3 | 2 | 5 | 6 | 3 | 93 | 32 | 44 | 40 | 10 | 15 | 0 |  |
| Time |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| Band— Width |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| Final environmentalloadvector |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Load (e.g., CO2) | 2 | 3 | 2 | 5 | 6 | 3 | 93 | 32 | 44 | 40 | 10 | 15 | 0 | 255 |
| Time |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |
| Band— Width |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |

Table 4 shows the abbreviations used in Table 3.

Table 4 Explanation of abbreviations in Table 3.

| M1: End—user equipment, service under study | M2: Home equipment, service under study | M3: Access network, service under study |
| --- | --- | --- |
| M4: Service provider activities use, service under study | M5: Data transport/transmission, service under study | M6: Data centers/data room activities, service under study |
| M7: End—user equipment, all other services provided by network | M8: Home equipment, all other services provided by network | M9: Access network, all other services provided by network |
| M10: Service provider activities, all other services provided by network | M11: Data transport/transmission, all other services provided by network | M12: Data centers/data room activities, all other services provided by network |
| M13: Use service |  |  |
| P1–P6: Allocation of network to service under study | P7—P12: Allocation of network to all other services provided by network | P13: Use |

>Format to be agreed upon. Abbrev. M1 and P1 could be used elsewhere.>

Additional Requirements and Guidance for Third—Party Reports

(According to ISO 14044 Section 5.2)

Further Reporting Requirements for Comparative Assertion Intended to be disclosed to the Public

(According to ISO 14044 Section 5.3)

< Editor’s note: FT to review the below proposal w r t tool compatibility>

< Editor’s note:Fig. 8 and Fig. 9 are preliminary and need to be updated according to agreed life cycle phases>

## 6.2	ICT equipment

< Editor’s note: Huawei has a proposal for transparent reporting of LCA for ICT Equipment>

At the highest level the reporting structure for equipment should show impact referred to the life–cycle phases as defined in Table 1.

Each bar should then be divided into appropriate sub—phases according to chapter 5.1 or 5.2.2 (Error! Reference source not found.) as below in Fig. 8 .

Fig. 8Reporting structure for equipment level (example)

The rightmost bar in Fig. 8 represents all the life cycle phases (Error! Reference source not found., Table 1) added together and should be reused when results on network and service level are presented.

## 6.3	Network

At the highest level the reporting structure for networks should show impact referred to the included equipment types, including their respective impact from the life cycle phases defined in Error! Reference source not found..

In addition to the equipment types, the reporting on network level should show the support activities to run the network separately. Fig. 9 illustrates how the network level LCA is built up by the Equipment level LCA data, using a typical wireless network as example.

Fig. 9 Reporting structure for the network level LCA (example)

The same principle applies for other types of Networks, e.g. broadband, traditional fixed voice, IP–voice, LAN and IPTV networks.

The reporting structure defined should also be used when reporting important parameters such as quantities and energy consumption of included equipment (see Table 5).

Table 5 Reporting structure for important parameters (example)

|  | Included in Network LCA | Quantity | Energy consumption |
| --- | --- | --- | --- |
| E1. End–user equipment | Mobile phone (UE) |  |  |
| E2. Home equipment | Fixed Wireless Terminal (FWT) |  |  |
| E3. Access network | RBS sites, control & core network sites |  |  |
| C3 (G6) | Wireless network operator business and O&M activities |  |  |
| E4. Data transport / transmission | Allocation of shared data transport / transmission |  |  |
| E5. Data centers / data rooms | Allocation of shared data centers / data rooms |  |  |

## 6.4	Services

At the highest level the reporting structure for services should show impact referred to the included equipment types, including their respective impact from the life cycle phases defined in  5.1 (Error! Reference source not found.).

Also, the allocation between the service under study and other services should be clearly indicated (see Fig. 10).

Fig. 10 Reporting structure for services level (example)

# 7	Critical review

Any critical review should be performed according to the requirements in ISO and in this Standard.

# Annexes (normative)

## Annex N1 (Normative)	Generic processes

Table A.N 1 defines the generic processes (G1–G7) to be included if applicable in LCA studies for ICT Equipment, Networks, and Services, as well as examples of categories and examples of unit processes to be included.

Table A.N 1Generic processes for LCA of ICT equipment. **=optional

| Generic process | Generic process categories | Unit processes (for each category) | Product flow unit | Important issues |
| --- | --- | --- | --- | --- |
| G1. Transports&Travel | Road Air Ship Train | Direct (during transport) emissions Fuel supply chain Vehicle production** Infrastructure** | ton×km, kg×km, Cton×km | Chargeable weight = Cton×km (function that also considers volume or density) |
| G2. Electricity | Div. country, regional and producer electricity mixes | Fuel supply chain, Direct emissions (during electricity production), power plant construction**, dams, the grid itself**, nuclear waste treatment** | kWh |  |
| G3. Fuels | Oil Diesel Petrol Jet—fuel LPG LNG Coal Gas | Fuel supply chain: Extraction and Production Distribution (transports) and emissions related to the incineration of the fuel is connected to a unit process or "site LCI model". | mass, volume, energy content |  |
| G4. Other energy | District heating (hot water) District heating (steam) District cooling (cold water) as electricity | Fuel supply chain, Direct emissions during energy/electricity production Power plant construction**, Infrastructure** | kWh | Electricity is also a energy source or district heating/cooling production |
| G5. Raw material acquisition G5.1 Raw material recycling |  |  | mass, volume |  |
| G6. Support activities | Offices Travel | ICT use for marketing, sales and R&D, business travel, commuting, service vehicles | person×km (for Travel) |  |
| G7. End–of–life treatment | See Annex N4 |  |  |  |

## Annex N2 (Normative)	Recommended list of Equipment Raw material acquisition processes

Table A.N 2 below lists raw materials (chemicals, fuels, metals, plastics, packaging materials, and additives) which should be taken into account in LCA studies of ICT Equipment. These Raw Materials are either part of the material content of the ICT Equipment/Support Equipment or used as ancillary materials throughout the life cycle. Table A.N 2 is to be regarded as a recommended list and more Raw Materials may be included.

Table A.N 2 Recommended set of cradle–to–gate raw material acquisition processes to be considered in LCA of ICT Equipment

Chemicals

Nitrogen gas (N2)

Oxygen gas (O2)

Hydrogen gas (H2)

Argon gas (Ar)

Acetone

CaO

H2SO4

H2O2

HydroCloric Acid (HCl)

FeCl3

IsoPropyleneAlcohol

Ethylene glycol

HydroFluoric acid

H3PO4

HNO3

NaOH

…

…

Fuels

Heating oil

Bunker oil / ship diesel

Diesel

Petrol

Jet fuel

LPG

LNG

“Biofuels”

…

…

Metals and alloys

Aluminium

Brass

Cadmium

Chromium

Copper

Copper (Brass, Bronze)

Gold

Lead

Lithium

Magnesium

Mercury

Nickel

Palladium

Platinum

Silicon

Silver

Solder – SAC

Solder – Sn/Pb

Solder – SnZn

Stainless steel

Steel – Cr3+ plated

Steel – powder coated

Steel – zinc plated

Steel (Stainless steel, Iron)

Tin

Tin (Div. Solder alloys)

Zinc

Hard metal (W-Co)

….

….

Others

Concrete

Packaging materials

Ceramics

Paper

Cardboard

Wood

Wood board

Glass

Glassfiber

…

….

Plastics

Acrylonitrile butadiene styrene (ABS)

Epoxy

Polycarbonate (PC)

Polyethylene (PE) — HD

Polyethylene (PE) — LD

Polypropylene (PP)

Polystyrene (PS)

Polyurethane (PUR)

Polyester (e.g. PET)

PVC

Silicone rubber

SAN

PA (Nylone)

PTFE (Teflone)

PMMA

…

…

Additives and others

Brominated flame retardants (e.g. TBBP-A)

Sb2O3

Be in alloys ...and similar

Plastic additives

High purity grades of materials/chemicals and gases

Cooling media, fire extinguisher media (High GWPs/ODPs)

…

…

## Annex N3 (Normative) Part types

Table A.N 3 lists parts which should be taken into account (if applicable to the Goal&Scope) when performing an LCA of ICT equipment, as well as the corresponding categories and unit processes.  The list is to be regarded as a recommended list and more parts/part categories/unit processes may be included.

Table A.N 3 Recommended set of parts for LCA of ICT equipment.

| Part | Recommended set of Part categories | Part unit processes (for each category) | Product flow unit | Important issues |
| --- | --- | --- | --- | --- |
| B1.1.1 Batteries | Lead batteries Lithium batteries Nickel—Cadmium batteries | Battery cell Battery module | Piece, mass | Size |
| B1.1.2 Cables | Coaxial cables Fiber cables Power cables Network/signal cables Connectors | Cable production (one unit process per part category) | Piece, mass | Length |
| B1.1.3 Electro—mechanics | Connectors Electric motors Speakers Microphones Camera objectives HDs Lighting (lamps) | Production and assembly |  |  |
| B1.1.4 Integrated circuits (ICs) | Processors, DSPs ASICs Memories Microprocessors Transistors and diodes (T&D) | Special IC materials Wafer Chip IC encapsulation | Piece, die area [cm2] T&D: mass | Yield Business activities Factory and machinery |
| B1.1.5 Mechanics / Materials | Nuts, bolts Fronts Frames Racks Cabinets Towers Containers Packaging Solder *) Metal and/or plastic factory | Production and assembly (one unit process per part category) | Piece, mass |  |
| B1.1.6 Displays | PDP LCD LED | Special display panel materials Display panel Display module | Piece, mass, area | Yield Business activities Factory and machinery |
| B1.1.7 PCB | Plastic Ceramic Flex–film | Special PCB materials PCB semi–produced composite materials PCB production | Piece, mass, cm2×layer | Yield Business activities Factory and machinery |
| B1.1.8 Other PCBA components | Resistors Capacitors Inductors Relays LEDs Potentiometers | Production and assembly (one unit process per part category) | Piece, mass |  |
| B1.1.9 “Black box” modules | Modules bought by ICT equipment producer as complete products (e.g., cameras, modules, memories) | “Cradle–to–gate” LCA from supplier | Piece, mass | Size, mass, technology |

## Annex N4 (Normative)	EoLT processes

Table A.N 4 below defines the different generic and specific EoLT processes to be included (if applicable to the Goal&Scope) in LCA studies for ICT equipment as well as examples of process categories and examples of corresponding EoLT process unit processes. The list is to be regarded as a recommended list and more EoLT processes/process categories/unit processes may be included. Usually D3. Other EoLT (Fig. 7) consists of combinations of G7.1 and G7.2.

Table A.N 4 Examples of EoLT processes for LCA of ICT equipment

| G7. End of Life Treatment | Examples of process categories | EoLT process unit processes (for each category) | Product flow unit | Important issues |
| --- | --- | --- | --- | --- |
| G7.1 EHW treatment | EHW (destruction and energy recovery) Special EHW*** landfill | In general: Recovery/treatment (one unit process or "site LCI model" for the entire EHW category) | mass, (energy content) |  |
| G7.2 Other Waste treatment | Div. material recycling Energy recovery (e.g., incineration) Landfill | In general: Recycling/recovery/treatment (one unit process or "site LCI model" for each material/ waste categories) | mass, (energy content) |  |
| D. EoLT | D1. Re–use of ICT Equipment D2. ICT specific EoLT D2.1 Storage/Disassembly/Dismantling/Shredding D2.2 Recycling D2.2.1 Battery recycling ICT specific metal/mechanical parts/fractions EoLT D2.2.2 PCBA recycling D2.2.3 Cable recycling D2.2.4 Mechanics recycling D3. Other EoLT | Recycling, recovery and treatment (one unit process per material/ waste category) | mass |  |

## Annex N5 (Normative) Recommended list of elementary flows in LCA of ICT

Table A.N 5 below contains elementary flows which should be taken into account in LCA studies for ICT. More flows could be relevant and the list is to be considered to be the recommended LCI flows.  The most commonly used name, chemical name or abbreviation has been put first followed by other common names/abbreviations. It’s recommended to add the term “(water)” to emissions to water and the term “(ground)” to emissions to ground.

The recommended unit is mass (unless stated otherwise): g, kg, ton

.

Table A.N 5 Elementary flows which should be taken into account in LCA studies for ICT

| Inventory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Substance | Measure unit | Quantity | Source | Impact Contributor (Table AI1) |
| Aluminum (resource) | kg |  |  | RD, LU |
| Ammonia (to air) | kg |  |  | AP |
| As (to air) | kg |  |  | HT, ET |
| As (to water) | kg |  |  | HT, ET |
| As (to ground) | kg |  |  | HT, ET |
| Benzene (to air) | kg |  |  | HT, ET |
| BOD (to water) | kg |  |  | EP |
| Cd (to air) | kg |  |  | HT, ET |
| Cd (to water) | kg |  |  | HT, ET |
| Cd (to ground) | kg |  |  | HT, ET |
| Cr (to water) | kg |  |  | HT, ET |
| Cr (to ground) | kg |  |  | HT, ET |
| CClF3, (CFC–13) (to air) | kg |  |  | OD , CC |
| CCl3F, (CFC–11) (to air) | kg |  |  | OD, CC |
| CCl2F2, (CFC–12) (to air) | kg |  |  | OD, CC |
| Cl2FC-CClF2, (CFC–113) (to air) | kg |  |  | OD, CC |
| C2F6 (CFC–116) (to air) | kg |  |  | OD, CC |
| C2H2F4, HFC–134a (to air) | kg |  |  | OD, CC |
| C2H3F3, HFC–143a (to air) | kg |  |  | OD, CC |
| C2H1F5, HFC–125 (to air) | kg |  |  | OD, CC |
| C2H3Cl2F, HCFC–141b (to air) | kg |  |  | OD, CC |
| CF2ClBr, Halon 1211 (to air) | kg |  |  | OD, CC |
| CF3Br, Halon 1301 (to air) | kg |  |  | OD, CC |
| CF4, CFC–14 (to air) | kg |  |  | OD, CC |
| CH3Cl3, HCFC–140 (to air) | kg |  |  | OD, CC |
| CH4, Methane (to air) | kg |  |  | CC |
| CHF2Cl, HCFC–22 (to air) | kg |  |  | OD, CC |
| CHF3, HFC–23 (to air) | kg |  |  | OD, CC |
| Cl- (to water) | kg |  |  | HT, ET, AP |
| Cl- (to ground) | kg |  |  | HT, ET, AP |
| CO, Carbon monoxide (to air) | kg |  |  | HT, POF |
| CO2 (to air) | kg |  |  | CC, AP |
| Coal (resource) | kg, TOE, MJ |  |  | RD, LU |
| Copper (resource) | kg |  |  | RD, LU |
| Copper (to air) | kg |  |  | HT, ET |
| Copper (to water) | kg |  |  | HT, ET |
| COD (to water) | kg |  |  | EP |
| EHW = Environmental hazardous (waste) | kg |  |  | LU |
| EHW Ashes, special EHW landfill (waste) | kg |  |  | LU |
| EHW Metal hydroxides (MeOH), special EHW landfill (waste) | kg |  |  | LU |
| EHW Slag, special EHW landfill (waste) | kg |  |  | LU |
| Ethylene (air) | kg |  |  | POF |
| Ethylene oxide (air) | kg |  |  | HT |
| Formaldehyde, CH2O (to air) | kg |  |  | HT, POF |
| Gas (resource) | kg, TOE, MJ |  |  | RD, LU |
| Gold (resource) | kg |  |  | RD, LU |
| Hg (to air) | kg |  |  | HT, ET |
| Hg (to water) | kg |  |  | HT, ET |
| Hg (to ground) | kg |  |  | HT, ET |
| Hydrogen chloride (to air) | kg |  |  | AP |
| Iron (to air) | kg |  |  | HT |
| Iron (resource) | kg |  |  | RD |
| Iron (to water) | kg |  |  | HT |
| Iron (to ground) | kg |  |  | HT |
| Land occupation, agricultural | m2×year |  |  | LU |
| Land occupation, urban | m2×year |  |  | LU |
| Metals (unspec.) (to water) | kg |  |  |  |
| Metals (unspec.) (to ground) | kg |  |  |  |
| Mineral (waste) | kg |  |  | LU |
| Mo (to water) | kg |  |  | HT |
| Mo (to ground) | kg |  |  | HT |
| N2O (to air) | kg |  |  | CC, OD |
| NF3 (to air) | kg |  |  | CC |
| Nickel (resource) | kg |  |  | RD, LU |
| Nickel (to water) | kg |  |  | HT, ET |
| Nickel (to ground) | kg |  |  | HT, ET |
| N-total (to water) | kg |  |  | EP |
| Nitrate, NO3- (to water) | kg |  |  | EP |
| NMVOC, non-Methane VOC (to air) | kg |  |  | POF |
| NMHC, non-Methane hydrocarbons (to air) | kg |  |  |  |
| NOx (to air) | kg |  |  | POF |
| Oil (to ground) | kg |  |  | HT |
| Oil (resource) | kg, TOE, MJ |  |  | RD, LU |
| Oil (to water) | kg |  |  | HT |
| Other CFCs/HFCs/HCFCs/PFCs (to air) | kg |  |  | OD, CC |
| Other (new) "high GWPs/ODPs" (to air) | kg |  |  | OD, CC |
| PAH, all kinds (to air) | kg |  |  | HT |
| Palladium (resource) | kg |  |  | RD, LU |
| Platinum(resource) | kg |  |  | RD |
| Particulates, all kinds (to water) | kg |  |  | HT |
| Particulates, all kinds (to air) | kg |  |  | HT, POF |
| Pb (resource) | kg |  |  | RD |
| Pb (to air) | kg |  |  | HT, ET |
| Pb (to water) | kg |  |  | HT, ET |
| Pb (to ground) | kg |  |  | HT, ET |
| PF3 (to air) | kg |  |  | CC |
| Phosphate, PO43- (to water) | kg |  |  | EP |
| P-total (to water) | kg |  |  | EP |
| Radioactive (low, volume) (waste) | kg |  |  | IR |
| Radioactive (medium, volume) (waste) | kg |  |  | IR |
| Radioactive (high, volume) (waste) | kg |  |  | IR |
| Selenium (to water) | kg |  |  | HT, Et |
| Selenium (to ground) | kg |  |  | HT, ET |
| Silver (resource) | kg |  |  | RD |
| Solid waste to landfill (waste) | kg |  |  | LU |
| SF6 (to air) | kg |  |  | CC |
| SO2 (to air) | kg |  |  | AP, POF |
| SOx (to air) | kg |  |  | AP, POF |
| TCDDe ("Dioxin" equivalents) (to air) | kg |  |  | HT |
| Tin (resource) | kg |  |  | RD |
| Titanum (to water) | kg |  |  | HT, ET |
| Titanum (to ground) | kg |  |  | HT, ET |
| Toluene (to air) | kg |  |  | POF, HT |
| Uranium (resource) | kg, MJ, TOE |  |  | RD |
| Water, lake (resource) | m3 |  |  | RD |
| Water, river (resource) | m3 |  |  | RD |
| Water, well, in ground (resource) | m3 |  |  | RD |
| Water, unspecified, natural origin (resource) | m3 |  |  | RD |
| Zinc (to water) | kg |  |  | HT, ET |
| Zinc (resource) | kg |  |  | RD |
| Zinc (to ground) | kg |  |  | HT, ET |
| Zinc (to air) | kg |  |  | HT, ET |

# Appendices (informative)

<Editor’s note: Examples and background information should be put here. Once the methodology has been fully defined the examples needs to carefully be reviewed to really illustrate the Standard>

A more detailed overview figure showing the detailed content and connection between all life cycle phases is shown below in Fig. AI 1.

## Appendix I1: Life cycle phases overview

Fig. AI 1 Detailed content and connection between all life cycle phases

## Appendix I2:  Products and “black box” modules

This list summarizes products and modules frequently used in LCA studies. The list shall not be seen as a complete set of ICT Equipment but is rather an example to indicate the broad range of applicable ICT Equipment and Support Equipment to be considered.

Generic products and services

- Electricity [kWh] (G3)

- District heating/cooling [kWh] (G4)

- Transports [Chargeable weight to include both volume and weight aspects] (G1)

- Travel [personal km, pkm] (G1)

- Water treatment [m3] (G7)

- Waste treatment [mass, volume] (G7)

Sub–Parts are important but cannot easily be summarized as the number of  Sub–Parts and related processes (e.g., surface treatment) could be made very long.

ICT specific product types

Each product type may be further divided into more specific product types.

End–user Equipment or User Equipment (UE)

- Mobile phone or standard mobile phone

- Smartphone

- Tablet device (phone/e-reader/PC)

- Net book PC

- Laptop PC

- Desktop PC

- TV

- Any device that can connect to home or Networks

Home (ICT Network) ICT Equipment or customer premises equipment (CPE)

- Fixed wireless terminal (FWT, typically 1 3G+ in and 4 LAN + WLAN out)

- Modem

- Router (Typically 1 WAN in and 4 LAN + WLAN out)

- IPTV box and/or STB for IPTV

- Combo products (e.g. a 3-play or home gateway box: modem/router/IPTV)

- Fiber access (ONU) and combo products including fiber access

ICT Network site Equipment (from base station sites to data centers)

- Base station equipment

- Transmission equipment (e.g. STM-1, radio link)

- RAN control and core equipment (BSC, RNC, SGSN, GGSN, HLR, MSC etc.)

- Fixed access node (FAN) equipment (POTS, xDSL, FTTx/OLT)

- Telecom switches and telecom servers (services)

- Edge/Metro routers/switches

- Core routers/switches

- Optical high capacity transport equipment (WDM)

- Servers

- Data switches

- Storage array network equipment

Site support equipment

- Antenna towers

- Antennas

- Feeders

- Lighting guides

- Buildings, shelters and other mechanical structures

- Diesel tanks

- Diesel generators

- AC units

- HEX units

- Fan units

- Battery units

- UPSs

Module (“black box”) examples (ICT specific):

Cable set/module (cable + connectors)

Memory module

Camera module

Display module

Charger device

AC/DC adapter incl. power cord

Fan unit

HDD

Optical disc player

Transceiver module

PA module

PSU, Power supply unit

## Appendix I3:  Impact assessment method examples

Table A.I 1 below shows examples of impact assessment categories.

Table A.I 1 Examples of environmental impact categories

Impact Assessment Categories (examples)

Climate change (CC)

Ozone depletion (OD)

Human Toxicity (HT)

Respiratory inorganics / Particulate Matter (RI/PM)

Ionizing Radiation (IR)

Eutrophication (E)

Photochemical ozone formation (POF)

Acidification (A)

Ecotoxicity (ET)

Land use (LU)

Resource Depletion (RD)

## Appendix I4:	Examples of Allocation Procedures

### AI4.1 Examples for Allocation Procedures for Re–use and Recycling of Materials

The assumption is that recycling of materials with constant inherent properties into another product system is equivalent to recycling it back into the original product system studied.

Emissions from Raw Materials Recycling (G5.1) of a certain amount of metal can be reduced by the corresponding emissions resulting from producing the same amount of primary metal (A1 and A2). This allocation is only allowed when the recycled metal has the same price as the substituted (primary) metal, and when the metal entering the production is predominately of primary type.

E.g., around 0.85 kt of CO2 is released when 1 ton of gold is obtained from gold containing e–waste (D2.2, D2.2.2, G5.1). Around 19 kt of CO2 is released when 1 ton of gold is obtained from gold containing ores (A1–2). Hence, 0.85 kt can be reduced by 19 kt.

System Expansion and Substitution

Assume that 10 g primary gold is contained in the ICT equipment. 7 g recycled gold ingot are obtained after recovery of e–waste (D2.2, D2.2.2) and smelting (G5.1).

–3 g of gold is lost

–7 g of recycled gold ingots substitute 7 g primary gold ingots

The environmental loadings of the production of lost gold, 3 g primary metal and the recovery operation loadings (D.1.2.1—4 + G.5.1), have to be charged to the ICT Equipment.

For certain metals the recycling processes (D.1.2.1—4 + G.5.1) and the Raw Material Acquisition (A.1—A.2) could have similar emissions and then the discount will not be particularly high. The available discount varies with at least three factors

1) Metal type

2) Scrap type (ICT equipment specific) from which the metal originates

3) Efficiency of recycling technology at the time of LCA

Another simplified approach is to treat the metal going to Raw Material Recycling (G5.1) as an outflow from the product system (Fig. 7).

Value–corrected substitution

For many materials, especially metals, the economic value of secondary metals (output from G5.1) is lower than for primary metals (from A2).

–Assume that 10 g primary gold is contained in the ICT equipment. .

–7 g recycled gold ingots (G5.1), with 95% of the value of primary gold ingots (A2), are obtained after recovery of e–waste (D.1.2. and smelting

–3 g of primary gold is lost

–Additional loss by value correction: 5% of 7 g = 0.35 g

–Total value corrected losses = 3.35 g

–After value correction 6.65 g recycled gold ingot substitute 6.65 g primary gold ingots. The environmental loadings only of the production of lost gold, 3.35 g primary metal, and the recovery operation loadings, have to be charged to the network.

If possible, the economic value (annual average) of different kinds of metals in different kinds of ICT e—waste should be obtained from official e—waste recyclers.

Recycled Metal as Input and Value–Correction

For some materials, especially metals, part of the economic value of secondary metals is lower than for primary metals, and moreover the ICT equipment consist of metals which are blends of primary and secondary.

–Assume that 10 g gold is contained in the ICT Equipment. The input consists of 5 g primary gold and 5 g recycled gold, the latter of 95% of the value of primary gold.

–7 g recycled gold ingot, with 95% of the value of primary gold ingot are obtained after recovery of e–waste and smelting

–3 g of primary gold is lost

–Value corrected mass of input gold: 5 g+(5×0.95) = 9.75 g gold ingot of the value of primary gold ingots

–Value corrected mass of output gold: 7 g×0.95 = 6.65 g gold ingot of the value of primary gold ingots

–Net gold loss: 9.75 g–6.65 g = 3.1 g

–After value correction 6.9 g recycled gold ingots substitute 6.9 g primary gold ingots. The environmental loadings of the production of lost gold, 3.1 g primary metal, and the recovery operation loadings, have to be charged to the ICT Equipment.

If possible, the economic value (annual average) of different kinds of metals in different kinds of ICT e–waste should be obtained from official e–waste recyclers.

### AI4.2 Examples for Allocation Procedures for Transportation

When a lorry is loaded, the maximum load limit can be reached for two reasons: either because the lorry is only permitted to drive with x ton of equipment, or because there is no more space. Transport of equipment (G1) with a high density (e.g., metals) will often reach the weight limit, while transport of equipment with a low density (e.g., new, empty plastic bottles) reaches the volume limit.

When transporting two equipments on the same lorry, allocation of the inputs and outputs (e.g., energy consumption and emissions) between the two pieces of equipment is necessary. Identification of the cause of the limit is necessary: What is the reason for not loading any more equipment on the lorry? Transport of steel and copper together — the reason is probably weight, and the allocation should be based on mass. Transport of different empty packaging — the reason is probably volume, hence the allocation should be based on density of the packaging. In both cases physical allocation is used.

### AI4.3 Other Examples for Allocation Procedures

#### AI4.3.1 Production process

When producing only one Equipment group at a production plant (B1.1.1–9, B2), the resources and the energy consumption as well as emissions and waste should be evenly allocated by mass to equipments or by production volume (amount of equipment) depending on data available and relevance for the type of process. For some processes the use of resources is primarily related to the number of units and for others it is mass related.

When essentially different equipments are being produced at a production plant, the environmental impact related to the processes should firstly be allocated to equipment groups (amount of equipment) and secondly mass flow allocation. Resources and emissions related to processes common for several equipment groups should be allocated by mass of the material treated by the process.

#### AI4.3.2 Process output allocation

If a unit process's output is 40 kg of Product (a), 40 kg of Product (b) and 20 kg of waste, Product (a) and Product (b) each are credited with half of the emissions released from the unit process.

#### AI4.3.3 Waste incineration

ISO 14041 Figure B.2.[2] Energy discount related to incineration (G7.2) of plastics and paper is possible in form of reduced usage of distributed heat.

Emissions from incineration of a certain amount of energy contained in plastic/paper waste can be reduced by the corresponding emissions resulting from extracting the same amount of district heat energy. District heat energy is distributed to, e.g., households in a certain area. This is done in centralized plants in which fossil fuels are burnt to extract heat. Increasingly fossil fuel is replaced by, e.g., biomass, geothermal etc. Waste incinerators could also connect to “heat net” and supply and thereby replace district heat energy.

E.g., around 0.2 kg CO2 is released when 1 MJ of energy is obtained from incinerating plastics waste. Approximately 0.03 kg CO2 is released per produced MJ district heating. That is, 0.2 kg can be reduced by 0.03 kg. The amount of CO2 emitted from district heat extraction is dependent on which primary energies it is based upon.

## Appendix I5: Example of use of data quality matrix

Table A.I 2 below shows how a matrix can be used to give indications of the quality of the data used for a Part (B1.1.7) and for a life cycle phase (C1).

<Editor’s note: The level at which the data quality evaluation should be done is not decided. >

Table A.I 2 Examples of usage of data quality matrix

| indicator | acquisition method | independence of data supplier | Represen- tativeness | data age | Geograp-hical corre-lation | Technological correlation | AVE-RAGE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B1.1.7 | 2 | 5 | 3 | 5 | 5 | 5 | 4.2 |
| C1 | 5 | 5 | 4 | 5 | 3 | 5 | 4.5 |

# History

| Document history |  |  |
| --- | --- | --- |
| 0.0.1 | March 2, 2009 | First Draft |
| 0.0.2 | April 29, 2009 | Second Draft after comments from stakeholders of ETSI |
| 0.0.3 | June 18, 2009 | Third draft after Belgrade EE#34 meeting May 7, 2009 |
| 0.0.4 | July 17, 2009 | Fourth draft after Kista workshop meeting June 25, 2009 |
| 0.0.5 | September 15, 2009 | Fifth draft after Kista workshop meeting August 25, 2009 |
| 0.0.6 | December 4 2009 | Sixth draft after Ovada EE#36 meeting October 14, 2009 |
| 0.0.7 | April 6, 2010 | 7th draft after Paris meeting February 10, 2010 |
| 0.0.8 | June 11, 2010 | 8th draft after Lannion meeting (ad hoc meeting at EE#36, April 2010) Main changes: Restructuring of document to better reflect ISO and the three product system levels (equipment, network and services Functional unit updated acc to decision in Lannion Terminology updated based on agreement in Lannion and comments on chapter 3 |
| 0.0.9 | June 17, 2010 | 9th draft after Rapporteaur check of 8th draft |
| 0.0.9.1 | June 29, 2010 | 9th draft with comments after Kista workshop, June 23, 2010 |
| 0.10 | Sept. 17, 2010 | 10th draft with proposal for Section 5.2.2.2 |
| 0.11 | Sept. 22, 2010 | 11th draft with agreement on compulsory and optional life cycle phases 5.2.2.2 |
| 1.1.4 | Nov. 18, 2010 | 12th draft with new figures and text, especially for 5.2.2.1-4. This draft to be used in Paris workshop Dec. 6-7, 2010 |
| 1.1.5 | Dec. 30, 2010 | 13th draft after Paris workshop Dec. 6-7, 2010 |
| 1.1.6 | Feb.. 16, 2011 | 14th draft after Helsinki workshop Feb. 8-9, 2011 |

Special requirements apply for comparative assertions between ICT and reference technologies. Comparisons are not supported for studies performed and presented by two different users.

By results is meant absolute and relative values.

Studies refer to analyses performed and presented by different individuals. However, comparisons of results obtained by the same individual who uses i) this standard, ii) the same software, and iii) the same LCI databases for all comparables are supported by this standard. A third–party review is needed if the comparison result is to be externally communicated.

See Secton 1. Scope .

The scope for ICT Networks could vary significantly for each LCA study and, within the system boundary, occasionally only specific Parts of an ICT Network might be the target for the LCA.

ICT equipment gives the building blocks also when the target product system is an ICT Network or a service.

It is recommended to consider EoLT but in this standard EoLT is considered as optional as there are contingencies, especially since the EoLT of a specific product always will take place in the future.

The use of the word ”site” in this context may be confusing as the word commonly denotes the location/support equipment needed to operate telecom equipment e g an RBS site. That is not the intended use above.

Detailed flow chart figure s of, e.g., transports are decided/provided in each unique LCA project.

The first destination for the de-installed ICT Equipment depends on the goal&scope of the specific LCA study.

Some of these units are ICT specific, others have a more generic application.

This is only valid for metals which properties are the same after recycling, i.e. when the technical quality of secondary metals is equal to ore metals.

Expanding system boundaries is not suitable within a multi–recycling framework or within the scope of established recycling sectors (i.e., when there is only a very small quantity of primary product).

There are also approaches that take into account the economic value of recycled metal compared to primary metal.