| Title*: | TS 103 199 ver. 1.1.13 |  |  |
| --- | --- | --- | --- |
| from Source*: | Rapporteaur |  |  |
| Submitted To*: | EE EEPS |  |  |
| Document for*: | Decision |  |  |
|  | Discussion | X |  |
|  | Information |  |  |
| Submission date*: | 2011-08-26 |  |  |
| Agenda Item: |  |  |  |
| Contact: | Anders Andrae |  |  |
| Relevant WI(s), or deliverable(s): |  |  |  |

| Relevant WI(s), or deliverable(s): | TS 103 199 |
| --- | --- |

Draft ETSI TS 103 199  V1.1.13 (2011-08)

Environmental Engineering (EE); Life Cycle Assessment (LCA) of ICT equipment, networks and services: General methodology and common requirements

Reference

TS 103 199

Keywords

Life Cycle Assessment, ICT Equipment,  Networks, Services

ETSI

650 Route des Lucioles

F–06921 Sophia Antipolis Cedex – FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

Individual copies of the present document can be downloaded from:
http://www.etsi.org

The present document may be made available in more than one electronic version or in print. In any case of existing or perceived difference in contents between such versions, the reference version is the Portable Document Format (PDF). In case of dispute, the reference should be the printing on ETSI printers of the PDF version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at http://portal.etsi.org/tb/status/status.asp

If you find errors in the present document, please send your comment to one of the following services:
http://portal.etsi.org/chaircor/ETSI_support.asp

Copyright Notification

Reproduction is only permitted for the purpose of standardization work undertaken within ETSI.
The copyright and the foregoing restrictions extend to reproduction in all media.

NOTE:	This paragraph should be used as 1st paragraph within the copyright text for deliverables processed before WG/TB approval and used in meetings.

© European Telecommunications Standards Institute yyyy.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM, TIPHONTM, the TIPHON logo and the ETSI logo are Trade Marks of ETSI registered for the benefit of its Members.
3GPPTM is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners.

# Contents

Contents	5

Intellectual Property Rights	6

Foreword	6

Introduction	7

1	Scope	8

2	References	8

2.1	Normative references	8

2.2	Informative references	8

3	Definitions, Symbols and Abbreviations	9

3.1	Definitions	9

3.2	Symbols	11

3.3	Abbreviations	11

4	General Description	12

5	Methodological framework	12

5.1	General requirements	12

5.1.1 Life cycle stages	12

5.1.2 Lifetime	13

5.2	Goal and Scope definition	13

5.2.1	Functional unit	14

5.2.1.1	General	14

5.2.1.2	ICT equipment	14

5.2.1.3	Networks	14

5.2.1.4	Services	14

5.2.2	System boundaries	15

5.2.2.1	General	15

5.2.2.1.1 The use of unit processes	18

5.2.2.2	ICT equipment	19

5.2.2.2.1	General	19

5.2.2.2.2 Equipment Raw materials acquisition	20

5.2.2.2.3 Production	20

5.2.2.2.4 Use	22

5.2.2.2.5 End–of–life treatment (EoLT)	22

5.2.2.3	Networks	23

5.2.2.4	Services	25

5.2.3	Cut–off rules	25

5.2.4	Data Quality Requirements	26

5.3	Life Cycle Inventory (LCI)	28

5.3.1	Data collection	28

5.3.1.1 General	28

5.3.1.2 ICT equipment	28

5.3.1.3 Network	28

5.3.1.4 Services	29

5.3.2	Data calculation	29

5.3.3	Allocation of data	29

5.3.3.1 General	29

5.3.3.2 ICT equipment	30

5.3.3.3 Network	30

5.3.3.4 Services	30

5.4	Life Cycle Impact Assessment (LCIA)	30

5.5	Life Cycle Interpretation	32

5.6	Comparative LCA between ICT and business as usual systems	32

6	Reporting	33

6.1	General	33

6.2	ICT equipment	34

6.2.1 Total result	34

6.2.2 System boundaries	36

6.2.2.1 Life cycle stages, unit processes and generic processes	36

6.2.2.3 Raw material acquisition	36

6.2.2.4 Production	36

6.2.2.5 Use	36

6.2.2.6 EoLT	36

6.2.3 LCI results	37

6.3	Network	37

6.3.1 Total result	37

6.4	Services	39

6.4.1 Total result	39

7	Critical review	40

Annexes (normative)	41

Annex M1 (Normative)	Generic processes	41

Annex M2 (Normative) Part types	43

Annex M3 (Normative)	EoLT processes	45

Annex M4 (Normative)	Mandatory list of Equipment Raw material acquisition processes	45

Annex M5 (Normative) Mandatory list of elementary flows in LCA of ICT	48

Annex N1 (Normative) Table reporting formats	50

Annex N2 (Normative) Diagram reporting formats	57

Appendices (informative)	58

Appendix I1: Life cycle stages overview	58

Appnedix I2: Generic activities	58

Appendix I3: Examples of Equipment and black box modules	59

Appendix I4 Examples of Network Equipment	60

Appendix I5:	Examples of Allocation Procedures	60

AI5.1 Examples for Allocation Procedures for Recycling of Materials	60

AI5.1.1 Example of the 100/0 and 0/100 methods	61

AI5.1.2 Example of the 50/50 method	61

Appendix I6: Examples of application of the standard	63

AI6.1 ICT Equipment	63

AI6.2 Networks	63

AI6.3 Services	63

History	64

# Intellectual Property Rights

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non–members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (http://webapp.etsi.org/IPR/home.asp).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

# Foreword

This Draft TS (DTS) has been produced by ETSI Technical Committee Environmental Engineering (EE).

<Editor’s note:

Color coding used in this draft:

Black text: Agreed standard text

Plum text: Text based on agreed contribution or similar and intended to be Standard text. Proposed by ad hoc group. Not yet reviewed and agreed as text.

Red text: Preliminary text, not based on contribution, not discussed and agreed as text.

Blue text: <Editor’s note: Comment about content or intended content. To be removed before approval of document>

# Introduction

Environmental life cycle assessment (LCA) is a systematic analytical method and model by which the potential environmental effects related to an ICT Equipment or Network, or Service can be estimated. LCAs have a cradle–to–grave scope where the life cycle stages raw material acquisition, production, use, and end–of–life are included. Transports and energy supply are moreover included in each life–cycle stage.

ISO has standardized the LCA methodology [1], [2]. In this standard, ICT specific additions to the ISO 14040 and ISO 14044 standards will be described. As addition to the ISO 14040 and ISO 14044 standards, the European Commission has published a handbook that gives detailed guidance on all the steps required to conduct an LCA [3]. This handbook will also be referred to with special ICT considerations in mind.

This standard will help LCA practitioners to perform and report their LCAs of ICT Equipment, Networks and Services in a uniform and transparent manner. It is possible to use this document to get guidance on what to consider in an LCA on three levels: ICT Equipment, ICT Network, and ICT Service. MOVED FROM 4.

The following ICT LCA applications are the most frequently used ones, but others may be identified and used as well:

- Assessment of product system carbon dioxide impact

- Assessment of primary energy consumption

- Identification of life cycle stages with high significance

- Comparisons of specific ICT Equipment, Networks, or Services

The structure of this LCA methodology standard for ICT Equipment, Networks, and Services is shown in Fig.1.

Fig.1 Structure of LCA methodology standard for ICT Equipment, Networks and Services

In accordance with ISO 14040 and ISO 14044 this standard consists of the following parts

- General requirements: High level requirements of LCAs

- Goal and scope definition: Requirements on functional unit, system boundaries and data quality

- Life Cycle Inventory (LCI): Requirements on data collection, calculation and allocation

- Life Cycle Impact Assessment (LCIA): Requirements on impact categories

- Life Cycle Interpretation: Requirements on interpretation of results

- Comparative Assertion between ICT and BAU systems

- Reporting: Requirements on reporting

All applicable parts are then divided into the three main applicable product system categories; ICT Equipment, Networks and Services.

The first edition of this standard focuses on both the negative and positive impacts of ICT Equipment, Networks and Services, both. The positive impacts of Services are potentially large under specific circumstances.

# 1	Scope

The purpose of this standard is to:

-harmonize the LCAs of ICT Equipment, Networks and Services

-increase the quality of the LCA by adding ICT specific requirements to those of ISO 14040 and ISO 14044

-facilitate communication of LCAs of ICT Equipment, Networks and Services

-increase the credibility of LCAs of ICT Equipment, Networks and Services

While recognizing ISO 14040 and ISO 14044 as normative references, this standard will give generic requirements for LCA of ICT Equipment, Networks and Services. The standard is valid for all type of Equipment which is/could be part of a Network including end–user Equipment.

Comparisons of results between LCAsare beyond the scope of this standard, as such comparisons would require that the assumptions and context of each LCA are exactly equivalent.

# 2	References

## 2.1	Normative references

- ISO (2006) Environmental Management–Life Cycle Assessment–Principles and Framework. ISO 14040:2006(E), second edition 2006-07-01

- ISO (2006) Environmental Management–Life Cycle Assessment–Requirements and Guidelines. ISO 14040:2006(E), first edition 2006-07-01

## 2.2	Informative references

- European Commission Joint Research Centre (2011) International Reference Life Cycle Data (ILCD) System Handbook: General Guide for Life Cycle Assessment – Detailed Guidance. 1st edn., Dictus Publishing (17 June 2011)

- OECD (2009) Guide to measuring the ICT Sector. The 2006-07 OECD ICT sector definition (based on International Standard Industrial Classification (ISIC) Rev. 4)

- European Commission Joint Research Centre (2011) International Reference Life Cycle Data System (ILCD) Handbook: Framework and Requirements for Life Cycle Impact Assessment Models and Indicators. 1st edn. Dictus Publishing (17 June 2011)

# 3	Definitions, Symbols and Abbreviations

## 3.1	Definitions

<Editor’s note: Available definitions from other standards shall be reused by reference as far as possible. >

For the purposes of the present document, the following terms and definitions apply. Elsewhere as ISO14040 and ISO 14044 [1][2] or the ILCD Handbook, Table 1 [3].

Black box module: A device, system or object which can be viewed solely in terms of its input, output and transfer characteristics without any knowledge of its internal workings. In this context the black box module may consist of several part categories such as integrated circuits, mechanics, cables etc, e.g., a power module on a PCBA

Business as usual system: A (basically non–ICT but can also be ICT) system which is replaced by ICT. E.g. BAU Service which is replaced by an ICT Service.

Commercial lifetime: The time the equipment is owned before a new one is bought to replace it (often used to estimate lifetime for consumer products)

Customer–premises equipment (CPE): Any terminal and associated Equipment located at a subscriber's premises and connected with a carrier's telecommunication channel(s) at the NTPs.

Cut–off: Unit processes excluded from an LCA.

Data gap: LCI flows excluded from a unit process

Depreciation time: The time during which a (new) revenue–generating asset reaches its residual economic value .

End–user equipment: see CPE ICT Equipment used to access fixed, datacom, broadband, and wireless Network. DELETE?

Extended operating lifetime: The aggregated duration of the actual use periods of the first user and consecutive use periods associated with reuse

First order effects: Total (negative) impacts of the ICT System. E.g. GWP100 score for a Service.

Fresh water: Water that does not contain a large amount of salt.

.

Generic data: Data from any relevant data source which need not be specific for ICT applications and processes. Generic data is always secondary.

Home Equipment:

ICT Equipment: The tangible Equipment deriving from or making use of technologies devoted to or concerned with (a) the acquisition, storage, manipulation (including transformation), management, movement, control, display, switching, interchange, transmission or reception of a diversity of data; (b) the development and use of the hardware, software, and procedures associated with this delivery; and (c) the representation, transfer, interpretation, and processing of data among persons, places, and machines, noting that the meaning assigned to the data must be preserved during these operations.

ICT specific EoLT: Any disassembly/dismantling/shredding/recycling process which needs special adaptation for handling of ICT equipment.

Infrastructure: The basic physical and organizational structures needed for the operation of the ICT Equipment, Network or Service. E.g. transportation systems, buildings, power plants. The fundamental facilities and systems serving the ICT Equipment, Network or Service.

Land use: Human exploitation of land for agricultural, industrial, residential, recreational purposes

LCA practitioner: The person(s) or organization(s) performing an LCA.

Life cycle stage: One of several consecutive and interlinked stages of a product system.

Natural resource: A material source, such as wood, water, or a mineral deposit, that occurs in a natural state.

Network: A set of nodes and links that provide physical or over the air information and communication connections between two or more defined points.

Network Termination Points (NTP): A point established in a building or complex to separate CPE from telephone company equipment.

Node: A point in a Network topology at which lines intersect or branch.

Operator: The organization operating Networks and ICT Equipment.

Operating lifetime: The duration of the actual use period (consisting of both active and non–active periods) for the first user. Storage time is not included in operating lifetime

Optional activity: A unit process which can be left out of the LCA because of low significance, low precision, general lack of data or other practical reasons

Organizational data: mentioned in Section 5.3.3.1.2 Allocation rules for allocation of support activities between projects/product systems

Other EoLT: Any disassembly/dismantling/shredding/recycling process which does not need special adaptation for handling of ICT equipment but could be used for any kind of equipment.

Part: Constituent of ICT Equipment and Support Equipment. E.g. Cable

Part category: Classified part type. E.g., Fiber cable

Primary data: Data collected/estimated/measured by a researcher, specifically for the LCA project; for example, pilot studies or customer or producer surveys.

Primary energy: Natural resources which can be used for energy production.

Primary raw material: Material which origins from natural resources.

Process category: Classified process type. E.g., Landfill.

Processed materials: Raw materials which have been physically and/or chemically changed by humans

Public data: Data which is available to the public without access being restricted by requirements on membership, none–disclosure agreements, or similar.

Ratio of recycled Raw material content: The amount of recycled Raw Material in relation to the amount of primary Raw Material used as input to production

Raw material: Primary or secondary raw material that is used to produce a Part or Equipment.

Raw material extraction: Production of extracted raw materials used in raw material processing.

Raw material processing: Production of processed raw materials used in Part Production.

Raw material recycling: Production of raw materials from secondary raw materials.

Mandatory activity: A unit process which is significant to the result and therefore shall be included.

Recycling rate of disposed Raw Material: The rate with which disposed equipment ends up in a recycling process

process is part of the scope of the LCA.

Reuse: The Equipment is used again by a new owner or in a new context.

Secondary raw material: Material which origins from recycled primary raw materials.

Second order effects: The impacts resulting from the change from BAU to ICT system. E.g. Reduction potential per year.

Secondary data: Data that has been collected for another purpose, but can be analyzed again in a subsequent LCA; for example, LCA estimates by academia, government, or industry organizations.

Service (Application): The use of ICT Equipment and/or Networks to provide value to one or more users.

Service provider: Organization operating a Service. Could be the same organization as the operator.

Specific data: In this case, data emerging from, ICT specific applications and processes. This data could be either primary or secondary.

Storage time: The time which the Equipment is stored before and after its use stage

Support activity: An activity supporting the unit processes directly needed to provide the function of the Equipment, Network or Service. Examples are offices for marketing, sales and R&D, and also business travel, commuting, and service vehicles.

Support Equipment: A device, system or object needed to realize the function of the ICT Equipment. Examples: equipment for power supply and temperature regulation.

The 100/0 allocation method: A method that allocates the primary Raw Material Acquisition process fully to the studied product system, i.e., no recycled Raw Material is assumed as input to the studied life cycle. No recycling is assumed to occur at End-of-Life.

The 50/50 allocation method: A method that allocates the primary Raw Material Acquisition process equally to the introducer (initial life cycle that introduces the primary Raw Material) and the “depleter” (the last lifecycle in which the Raw Material is not recycled), i.e., the recycling of Raw Materials is allocated equally to the studied life cycle and the product system losing/introducing the material".

Traffic: The total volume of cells, blocks, frames, packets, calls, messages, or other units of data carried over a circuit or Network, or processed through a switch, router, or other system

Unit process: Smallest element considered in the life cycle inventory analysis for which input and output data are quantified. Examples: part unit process such as IC Encapsulation

## 3.2	Symbols

For the purposes of this document, the following symbols apply:

Tbd

## 3.3	Abbreviations

For the purposes of this document, the following abbreviations apply:

3G	Third generation telecom networks

4G	Fourth generation telecom networks

C&C	Control & Core

CN	Core Network

CPE	Customer premises equipment

DALY	Disability Adjusted Life Years

EHW	Environmentally Hazardous Waste

EIO–LCA	Economic Input Output LCA

EoLT	End of life treatment

ELU	Environmental Load Unit

FWT	Fixed Wireless Terminal

GSM	Global System for Mobile Communications

GPS	Global Positioning System

ICT	Information and Communication Technology

IP	Internet Protocol

IPTV	Internet Protocol Television

LAN	Local Area Network

LCI	Life Cycle Inventory

LCA	Life Cycle Assessment

LTE	Long Term Evolution

PCB	      Printed Circuit Boards

PCBA		      Printed Circuit Board Assembly

PDA		      Personal Digital Assistant

PWB		      Printed Wiring Board

RAN		      Radio Access Networks

RBS	Radio Base Station

SMT	Surface mounting technology

TOE	Total Oil Equivalent

UE	User Equipment

# 4	General Description

(According to ISO 14040 Section 4.[1])

# 5	Methodological framework

## 5.1	General requirements

(According to section 4.1 in ISO 14044 [2])

When performing an ICT related LCA, the requirements of ISO 14040 and ISO 14044 shall apply as well as the requirements of this document.

### 5.1.1 Life cycle stages

The following life cycle stages (also shown in Fig.2) in this document apply to ICT Equipment, Networks and Services and shall be assessed as applicable in LCAs based on this standard:

A. Equipment Raw material acquisition which is composed by

  - Raw material extraction

  - Raw material processing

B. Production which is composed by

  - ICT Equipment production

  - Support Equipment production

C. Use which is composed by

  - ICT Equipment Use

  - Support Equipment Use

  - Operator activities

  - Service Provider Activities

D. Equipment End of Life Treatment

  - Re–use

  - ICT specific EoLT

  - Other EoLT

Transports and energy supplies shall be included in all life cycle stages. It is important that all transports within and between life cycle stages are included in the assessment, especially transports of equipment between production and use stages shall be taken into account. The data collected shall be structured in such a way that transports could be reported transparently as far as possible.

It is optional to include the construction of plants in which ICT or Support Equipment are manufactured. The life cycle stages are further described in chapter 5.2.2.

### 5.1.2 Handling of software

From the perspective of this standard software shall be considered as well as hardware.

Any ICT Equipment or network consists of both hardware and software which both impacts e.g. the production and use stages. For the production stage software development impacts the amount of persons involved in the development work and thus impacts the amount of building and traveling associated with the product or network, in the same way as hardware development does. For the use phase the software impacts e.g. maintenance and energy use. In general it is not relevant to distinguish between software and hardware impact for the use phase but rather to focus on the impact from the product or network as such.

For specific software applications, such as a music distribution application, the software is to be seen as an ICT service and shall be assessed according to the requirements outlined for services. In these cases the hardware needed to operate the software shall be included.

Due to the uncertainties of allocation it is not recommended to consider the embedded impact from the operating system (e.g. windows) when assessing the software impact.

Software is an important

### 5.1.3 Lifetime

Operating lifetime is critical for the interpretation of the results of LCAs and shall therefore always be reported when presenting LCA results. Lifetime estimates and assumptions shall also be clearly described in the reporting. In case of comparisons between ICT Equipment LCAs the operational lifetime shall be set equal. Differences in lifetime could only be accepted if they reflect differences in actual characteristics.

Operating lifetime can only be defined for equipment, i.e., ICT and Support Equipment. The lifetime of a Network cannot be defined as a Network cannot be defined with one start date and one end date, instead the Network is continuously built out, upgraded etc. and the associated lifetime is therefore the lifetimes of the individual nodes. The same is valid for Services.

Operating lifetime should be based on available information on actual product use(e.g. statistics for similar product systems or information on commercial lifetime) and shall model real operating lifetime as closely as possible. If information on actual product system use cannot be found economical statistics may be used to estimate operating lifetime, e.g., depreciation time. However, such estimates are considered as less accurate and shall be avoided.

Storage time is not included in operating lifetime.

When available, results for known extended operating lifetime, taking into account also reuse, should be reported together with corresponding information for first use. Extended operating lifetime is estimated according to the same principles as the (first) operating lifetime.

## 5.2	Goal and Scope definition

During the scope phase the building blocks of the ICT Equipment, Networks or Services shall be identified.

Schematically three main levels of targeted product systems exist:

- Equipment (ICT Equipment and Support Equipment)

- Network (ICT Network)

- Services (ICT Service)

All these product systems use ICT Equipment which follows the life cycle stages introduced in 5.1 and further described in this chapter.

Equipment refers to the different physical products, with associated software, constituting the Network. ICT and Support Equipment consist of, e.g., Electronic Parts, Mechanical Parts, Cooling Parts, and Cables.

Printed circuit board assemblies (PCBA) and shelves are examples of included Parts. The PCBAs consist of printed circuit boards, integrated circuits, and other Parts.

In summary, any ICT Equipment which can be part of a Network delivering voice and/or data lies within the scope of this LCA standard. A hierarchical view is suitable for describing Networks. At the top level different types of ICT Equipment can be identified, e.g., Network nodes, End–user Equipment, and Services such as teleconferencing.

In reality the amount of different ICT product systems which could be studied and analysed with the LCA methodology is unpredictable. The ICT product system to be studied needs to be able to perform a function. After the ICT product system has been defined the LCA practitioner has to follow the requirements in this Standard to be compliant with it.

5.1.1 Compliance to the standard

This standard contains requirements (denoted as mandatory or by the use of the word shall) and optional (denoted as optional or by the use of the word should)

Full compliance towards this standard can be claimed if all mandatory shall requirements are fulfilled and all optional have at least been considered for insertion.

LCAs can also partially comply to this standard by complying to the majority of mandatory shall requirements, but not being able to fulfil all due to data shortage, lack of transparency in data bases and so forth.

In both cases the fundamental LCA principles of relevance, completeness, consistency, accuracy and transparency shall be the guiding start of the practitioner.

The report shall contain a compliance statement saying either that the LCA complies to the standard (in case of full compliance) or that the LCA complies to the standard with the exception of transparently listed and justified requirements (partial compliance).

### 5.2.1	Functional unit

#### 5.2.1.1	General

The functional unit requires inclusion of the relevant quantifiable properties and the technical/functional performance of the system. This means that the operational lifetime shall be specified and also the number of users/subscribers supported and the traffic profile shall be included where applicable.

An ICT Equipment, Network or Service has a number of possible functions and the one(s) selected for an LCA depend(s) on the goal and scope of the specific LCA. E.g., a mobile phone/device may have several functions: phone calls, text messaging, emailing, use the internet, camera, music player etc.

The functional unit defines the quantification of the identified functions (performance characteristics) of the ICT Equipment, Networks and Services. The primary purpose of a functional unit is to provide a reference to which the inputs and outputs are related. This reference is necessary to ensure comparability of LCA results. Comparability of LCA results is particularly critical when different systems are being assessed, to ensure that such comparisons are made on a common basis.

The specification of the functional unit shall be specified even more carefully for comparative LCAs than non–comparative LCAs.

It is important to determine the reference flow in each product system, in order to fulfil the intended function, i.e., the amount of products needed to fulfil the function.

#### 5.2.1.2	ICT equipment

The functional unit shall be chosen in the context of goal and scope of the LCA and shall be further clarified by system boundary and cut–off rules. ICT equipment LCA results may be further used as basis for Networks and Services LCA. Specific attention on selection of the functional unit is needed if the LCA is intended to be communicated to the public in order to help correct understanding of the results.

To comply with this standard, the following basic functional unit shall be applied where applicable:

Annual ICT Equipment use (per one year of ICT Equipment use) or  Total ICT Equipment use per lifetime of ICT Equipment.

For relevant LCA results realistic use scenarios shall be captured.

Additionally, other more specific functional units may be applied as well based on the scope and purpose of the LCA.

#### Networks

For the purpose of this standard, the following basic functional unit shall be applied where applicable:

Annual Network use.

For relevant LCA results realistic use scenarios shall be captured.

Additionally, other more specific functional units may be applied as well based on the scope and purpose of the LCA, for instance: Annual Network use per amount of users, or per transmitted data, or coverage area (if applicable).

#### 5.2.1.4	Services

For the purpose of this standard, the following basic functional unit shall be applied where applicable:

Annual Service use.

For relevant LCA results realistic use scenarios shall be captured.

Additionally, other more specific functional units may be applied as well based on the scope and purpose of the LCA.

### 5.2.2	System boundaries

#### 5.2.2.1	General

(This section corresponds to section 4.2.3.3 in ISO 14044).

The system boundaries define the life cycle stages and processes belonging to the analysed product system, i.e., are required for providing its function as defined by its functional unit.

Consequently, the system boundaries here define the life cycle stages and the unit processes that shall be taken into account in an LCA of an ICT product system.

The following basic reference flow units shall be used where applicable:

- Raw material acquisition 	per one piece of equipment

- Production:		per one piece of equipment

- Use: 			per annual equipment use

- End of Life Treatment (EoLT): 	per one piece of equipment

Fig. 2 below shows the system boundary of an LCA of ICT Equipment, Networks and Services. The boxes for G1–7 in Fig. 2 denote generic processes that reoccur several times during the life cycle. These processes are further defined in Annex N1.

System boundary

A: EQUIPMENT RAW MATERIAL ACQUISITION

A1. Raw material extraction

A2. Raw material processing

G1. TRANSPORTS&TRAVEL

B: PRODUCTION

B1. ICT Equipment production

B1.1 Parts production

B1.2 Assembly

B1.3 ICT manufacturer support activities

B2. Support Equipment production

B2.1 Support Equipment manufacturing

B2.2 Site construction

C: USE

C1. ICT Equipment use

C2. Support Equipment use

C3. Operator activities

C4. Service provider activities

D: EQUIPMENT END-OF-LIFE TREATMENT

D1. Re–use of ICT Equipment

D2. ICT specific EoLT

D2.1 Disassembly/Dismantling/Shredding

D2.2 Recycling

D3. Other EoLT

G2. ELECTRICITY SUPPLY

G3. FUEL SUPPLY

G4. OTHER ENERGY SUPPLY

G5. RAW MATERIAL AQUISTION

G5.1. Raw material extraction

G5.2. Raw material processing

G6. END-OF-LIFE TREATMENT

G6.1 EHW treatment

G6.2 Other waste treatment

G7. RAW MATERIAL RECYCLING

Fig.2 The system boundary of the product system for LCA of ICT Equipment, Networks or Services

Table 1 below further details the life cycle stages to be included in LCAs of ICT equipment, Networks, and Services. The different life cycle stages are further described in chapter 5.2.2.2.2 to 5.2.2.2.5. Mandatory in Table 1 means that the life cycle stage shall be included. However, this also means that even if, e.g., Parts production (B1.1) is mandatory not all Parts given in Annex M2 (Table AM2) are applicable to all ICT Equipment product systems. Mandatory life cycle stages or unit processes shall not be cut–off before considered for inclusion by using alternate data.

Table 1 Classification of life cycle stages/unit processes

| Tag | Life cycle stage |  |  | Unit process | Class |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ICT Equipment | Network | Service |
| A | Equipment Raw Material Acquisition |  |  |  |  |  |  |
| A1 |  | Raw material extraction |  |  | Mandatory | Mandatory | Mandatory |
| A2 |  | Raw material processing |  |  | Mandatory | Mandatory | Mandatory |
| B | Production |  |  |  |  |  |  |
| B1 |  |  | ICT equipment production |  |  |  |  |
| B1.1 |  |  |  | Parts production (for further details refer to Annex N1) | Mandatory | Mandatory | Mandatory |
| B1.2 |  |  |  | Assembly | Mandatory | Mandatory | Mandatory |
| B1.3 |  |  |  | ICT manufacturer support activities | Optional | Optional | Optional |
| B2 |  | Support equipment production |  |  |  |  |  |
| B2.1 |  |  |  | Support Equipment manufacturing* | Optional (Mandatory if Support Equipment is included in the Scope) | Mandatory | Mandatory i |
| B2.2 |  |  |  | Site construction | Optional | Mandatory | Mandatory |
| C | Use |  |  |  |  |  |  |
| C1 |  | ICT equipment use |  |  | Mandatory | Mandatory | Mandatory |
| C2 |  | Support equipment use |  |  | Optional (Mandatory if Support Equipment is included in the Scope) | Mandatory | Mandatory |
| C3 |  | Operator activities* |  |  | Optional | Mandatory | Mandatory |
| C4 |  | Service provider activities |  |  | Not applicable | Optional | Mandatory |
| D | Equipment End of Life Treatment |  |  |  |  |  |  |
| D1 |  | Re–use of ICT Equipment |  |  | Mandatory | Mandatory | Mandatory |
| D2 |  | ICT specific EoLT |  |  | Mandatory | Mandatory | Mandatory |
| D2.1 |  |  |  | Storage/Disassembly/Dismantling/ Shredding | Mandatory | Mandatory | Mandatory |
| D2.2 |  |  |  | Recycling | Mandatory | Mandatory | Mandatory |
| D3 |  | Other EoLT |  |  | Mandatory | Mandatory | Mandatory |

* not applicable for End-user equipment

A more detailed overview (Fig. AI1), showing the detailed content and connection between all life cycle stages, is shown in Appendix I1.

All stages in the life cycle are associated with different support activities. The term Support activities refers to, e.g., offices including ICT use for marketing, sales and R&D, and also to business travel, commuting and service vehicles.

Support activities are optional to include.

The extent in which Support activities and other Optional activities are included for different parts of the life cycle shall be clearly described in the LCA report. For further details refer to chapter 6.

Above in Table 1 “Mandatory” means that the life cycle stage, if applicable, shall always be taken into account in an LCA for ICT. The optional life cycle stages, however, may be left out of the LCA depending on the studied ICT product system etc.

##### 5.2.2.1.1 The use of unit processes

Each life cycle stage (A–D) is further refined into activities, referred to as unit processes, which represent the basic physical flows (materials and energy) of the life cycle.

A unit process typically represents a facility where a product is produced, but it can also represent, e.g., an office or a store - or even an activity or a place where a service is produced. A unit process can also be a vehicle or a “mobile facility” that transports products. Non–production facilities are especially important to the ICT sector [4] as a large part of the total work is related to research and development (including software), operation, maintenance etc.

Any unit process can be modelled as shown in Fig.3 below. The generic unit process model includes a number of inputs and outputs and can be referred to as a facility LCI model or – shorter – a facility model.

In many cases a facility handles not only the product system targeted by the LCA but also other product systems. In this situation the facility data shall be allocated to the targeted product system in a correct way. For allocation rules please refer to chapter 5.3.3.1.3.

Fig.3 The generic unit process model

Emissions to the environment and impact on or use or depletion of resource objects are referred to as elementary flows. All other inputs and outputs are defined as product flows.

Each input of fuel and products, as well as each output of waste, can involve transports which are then part of the input/output data connected to the unit process.

Applicable support activities shall also be included for a production facility.

Emissions (elementary flows)

The following emissions shall be included for ICT:

- Emissions to air

- Emissions to water

- Emissions to ground

Non–material emissions like radiation, odour, and noise are beyond scope of this standard as well as direct impact on health .

Resource objects (elementary flows)

The following resource objects shall be included for LCAs of ICT:

- Land use (or land depletion)

- Material resource use (or material depletion)

- Energy resource use (or energy resources depletion)

- Fresh water use (or fresh water depletion)

Species, biodiversity, and eco-system depletion as well as aesthetical values are beyond the scope of this standard.

A list of emissions and resource objects that have been found to be of importance for LCA of ICT Equipment, Networks, and Services can be found in Annex M5 (Table AM5).

Energy, product and services inputs

Further, the following inputs shall also be included for ICT:

- Electricity

- Other forms of delivered energy (district heating and cooling)

- Fuels (typically indicates the fuels are incinerated on-facility or in a vehicle connected to the facility)

- Primary products (products that are part of the final product in operation)

- Secondary products (products that are not part of the final product in operation)

- Transports, travel and other services (can be seen as a special non–material secondary product input.)

Product, water and waste output

Finally, the following flows shall also be included:

- Water discharge (to municipal sewage or recipient)

- Waste fractions (residual waste fractions or waste fractions that need further treatment, also including material recycling and energy recovery)

- Product output (the main purpose with the unit process or activity)

A mandatory list of generic activities (unit processes) that have been found to be of importance for LCA of ICT Equipment, Networks and Services can be found in Appendix I2.

An informative list of typical ICT Equipment, Support Equipment, Network Equipment can be found in Appendix I3-4.

#### 5.2.2.2	ICT equipment

##### 5.2.2.2.1	General

##### 5.2.2.2.2 Equipment Raw materials acquisition

Equipment Raw Material Acquisition (A) starts with the extraction (A1) of natural resources (e.g., iron ore, crude oil, etc.) and ends with the transport of Raw Materials from Raw materials processing (A2) to Part Production facilities. A2 deals with the processing of extracted Raw Materials (e.g., iron ore pellets) into processed Raw Materials (e.g., steel sheet, copper wire, etc.). Raw Material Acquisition is the life cycle stage for ICT Equipment as defined in Fig.2.

As Raw Materials are used as additives in every life cycle stage, Raw Material acquisition can additionally be regarded as a generic process (G5).

Annex M3 (Table A.M3) provides a mandatory set of Raw Materials (both ICT specific and generic) which shall be included in the LCA of ICT equipment.

As shown in Fig.4, Raw Material Extraction and Raw Material Processing are within the system boundary of Raw Materials Acquisition.

System boundary

A1. Raw material extraction

A2. Raw material processing

B. PRODUCTION

G7. RAW MATERIAL RECYCLING

Fig.4 The system boundary of Equipment Raw material acquisition in LCA of ICT equipment

##### 5.2.2.2.3 Production

The Production (B) starts with the Parts Production, and ends with the transport of ICT Equipment and Support Equipment to Use (C). The system boundary for Production, shown in Fig.5,includes ICT Equipment Production and Support Equipment Production.

It is optional to include the construction of plants in which the ICT Equipment is manufactured.

In case Support equipment is part of the targeted product system, Support Equipment Production (B2) is mandatory within the system boundary of the study. It is optional to include the construction of plants in which the Support Equipment is manufactured.

B2 is optional for ICT Equipment LCAs as the variance in solution is very large both for B2.1 and B2.2 between markets and operators.

System boundary

B2.1 Support equipment manufacturing

C.USE

A. EQUIPMENT RAW MATERIAL AQUISITION

B2.2 Site construction

B1.3 ICT manufacturer support activities

B1.2 Assembly

B1.1 Parts production

B1.1.1 Batteries

B1.1.2 Cables

B1.1.3 Electro-mechanics

B1.1.4 Integrated circuits

B1.1.5 Mechanics / Materials

B1.1.6 Displays

B1.1.7 PCBs

B1.1.8 Other PCBA components

B1.1.9 Packaging materials

B1.1.10 Black box modules

B1.1.11 Software

Fig.5 The system boundary of Production in LCA of ICT equipment

The ICT Equipment Production (B1) consists of Parts Production (B1.1) and Assembly (B1.2), and ICT manufacturer support Activities (B1.3).

Annex N2 lists a mandatory set of Parts to be included where applicable to the studied ICT product system, when performing an LCA of ICT Equipment, as well as mandatory Part unit processes which shall be included for each Part. As an example, if batteries are part of the studied ICT Equipment product system they shall be included in within the system boundary, and for every battery the Battery Cell manufacturing and Battery module manufacturing shall be included. Except for Parts listed in Annex N2, other Parts may be as important and should be considered as well.

Note that Parts can be complex modules themselves consisting of several other Part types as building blocks.

The Assembly (B1.2) shall include PCBA Module Assembly, Final Assembly, Testing & Repair, Warehousing and Packaging.

Support Equipment (B2.1) which shall be included if applicable are air conditioners, cables, and power supply systems.

As stated above in Table 1 Site construction (B2.2) is optional even if Support Equipment (B2.1) is included. Depending on the specific case at hand, a site can be pre–produced or constructed on place. Site building blocks needed for B2.2 which shall be included if applicable are antenna towers, fences, and shelters.

Support activities for ICT manufacturer (B1.3) are specifically indicated in Fig.5. Regarding other support activities for Support equipment production and Parts Production, please refer to the general principles in 5.2.2.1.

##### 5.2.2.2.4 Use

The Use stage starts with the Installation of ICT Equipment and Support Equipment and ends with the de–installation just before the transport to EoLT. As shown in Fig.6, the use stage includes ICT Equipment Use (C1), Support Equipment Use (C2), Operator activities (C3), and Service provider activities (C4).

(C1) and (C2) mainly deal with energy supply during the operating lifetime of the ICT Equipment.

Operator activities include, e.g., installation and de–installation of ICT Equipment and operation and maintenance of the ICT Equipment and site, including associated transports and travel. Service provider activities include, e.g., development of Service applications.

System boundary

C3. Operator activities

D. EoLT

B.PRODUCTION

C2. Support Equipment Use

C1. ICT Equipment use

C4. Service provider activities

Fig. 6 The system boundary of Use in LCA of ICT equipment

##### 5.2.2.2.5 End–of–life treatment (EoLT)

EoLT starts with the transport of de–installed ICT Equipment and Support Equipment to  Storage/Disassembly/Dismantling/Shredding facilities, and ends with the waste treatment of ICT Equipment and Support Equipment.

Raw Material Recycling (G7)starts after the EoLT.

As shown in Fig.7, Re–use of ICT Equipment (D1), ICT specific EoLT (D2) and Other EoLT (D3) are within the mandatory system boundary for EoLT.

ICT specific EoLT is applicable to the ICT Equipment itself and may also apply to  Support Equipment which is made of ICT Parts.

Other EoLT mainly deals with Support Equipment not made of ICT Parts.

System boundary

D3. Other EoLT

C. USE

D1 Re–use of ICT Equipment

D2 ICT specific EoLT

D2.1 Storage/Disassembly/Dismantling/Shredding

D2.2 Recycling

D2.2.1 Battery recycling

ICT specific metal/mechanical parts/fractions EoLT

D2.2.2 PCBA recycling

D2.2.3 Cable recycling

D2.2.4 Mechanics recycling

D2.2.5 Other ICT recycling

G7. RAW MATERIAL RECYCLING

Fig.7 The system boundary of Equipment EoLT in LCA of ICT equipment

The ICT specific EoLT (D2) in LCA of ICT Equipment includes Transport from Use to Storage/Disassembly/Dismantling/Shredding processes (D2.1), and Recycling Processes D2.2.1–5 for Batteries, PCBAs, Cables, Mechanics, and other ICT Parts.

The output from these Recycling Processes is not Raw Materials but rather products which the Raw Material Recycling (G7) can use.

It has to be judged from case to case which treatments (PCBA recycling etc.) apply to ICT Equipment and Support Equipment, respectively.

Annex M3 lists a mandatory set of EoLT processes to be included where applicable when performing an LCA of ICT Equipment which includes the EoLT stage.

#### 5.2.2.3	Networks

An Network is an ICT based infrastructure which offers the possibility to transfer voice and/or data between different access points, usually referred to as nodes, and further on to the end–users (represented, e.g., by a telephone or PC). Ultimately the total Network may be studied, taking into account both wireline and wireless Networks and the connection between them - however a study may also focus on just a part of the Network. In the goal and scope phase it shall be outlined which Network building blocks are covered.

For the Network under study applicable types of nodes and infrastructure, as well as amounts of these, shall be defined.

Thus the aggregated impact of a Network usually equals the sum of the impact from the different Equipment constituting the Network. When aggregating results the data for different nodes shall be based on equivalent assumptions/use scenarios etc.

As the Network operation depends on the software program needed to run its basic primary subscription service, as outlined in applicable standards, e.g., 3GPP for LTE, the impact from development of such software shall be included. This development is either within B1.3 or B1.1.11 depending on whether the software is developed.

The Network shall be defined in terms of ICT equipment, Support Equipment and infrastructure. For each included product types number of units shall be defined as well as corresponding lifetime. For each type of ICT equipment the rules defined for ICT Equipment in this standard applies for the assessment. For the reporting the same reporting rules applies but it is also allowed to aggregate the results to network level.

Typically the network consists of the following ICT equipment:

Access networks:

- Fixed telephony (or POTS, Plain Old Telephony System)

- Fixed broadband access (DSL)

- Mobile broadband access (2G (e.g. GSM), 3G (e.g. WCDMA/HSPA) etc)

- Cable TV (CATV) broadband

- Fiber (FTTx) or City LAN

- Enterprise LAN

Mobile Control and core nodes

- Control and core nodes for mobile

- Cotrol and core nodes for IPTV, VoIP etc

Data transport:

- All other transmission (excluding transmission associated with the access nodes).

- IP edge/metro/core network (switches and routers).

- Submarine optical fiber cables and land terminal stations.

Data centers:

- Servers, storage and network equipment (“switches and routers”)

- Cooling, power and back-up power equipment

As stated before, each of these Networks is associated with Support Equipment for powering and cooling, as well as the Site infrastructure antenna towers, fences, and shelters. These three site equipment shall also to be taken into account.

For assessment of  Networks operator activities shall always be included. Services provider activities and data centers are to be seen as optional.

For examples of Network Equipment refer to Appendix I2.

An LCA may cover an entire Network or a part of a Network, e.g. the access network.

{Editor´s note: Updated Table 1 of draft 1.1.7 accordingly}

#### 5.2.2.4	Services

The operation of a Network could be described as the operation of several services working in parallel, among them the basic primary subscription service allowing transfer of voice and data, but also different applications. Thus, to calculate the impact of a service, it is necessary to assess the Network, as outlined in previous paragraph, and if necessary (i.e., in a multi–service situation) allocate an appropriate amount of this impact to the service under study. For details on allocations refer to 5.3.3.4

The system boundary requirements defined for Networks apply also to Services but with some additions listed here.

In addition to the use of ICT Equipment and Network, a Service may also have additional impacts associated with the application software development, use of consumables, sales and logistics infrastructure, associated travels and transports (in addition to those already included for the ICT Equipment and Network) which shall also be included. Often these activities are part of the overall service provider activities.

The data centers where the service is operating shall be included as well as the activities of the service provider. Service provider activities consist of, e.g., offices and business travel, like operator activities and may also include the activities listed above.

Important data that defines the hardware associated with the Service is number of servers, storage and network equipment units; their energy consumption and the data center overhead energy consumption for cooling and power systems (including back–up power). The data center shall be studied and assessed in the same way as other ICT Equipment.

It is optional to include the data center infrastructure Production, e.g., the construction of the data center building and cooling and power infrastructure.

When assessing the environmental impact of a Service, there are both positive and negative environmental aspects to be taken into account. If the ICT Service offers the possibility to replace an already existing BAU Service (i.e. an e–health solution replacing hospital visits), a comparative study that includes the reduced impact from this change shall also be performed to give a correct level of impact of the service. For further details refer to 5.6.

### 5.2.3	Cut–off rules

(This section corresponds to section 4.2.3.3.3 in ISO 14044 [2]).

Cut–offs shall be avoided as far as possible. An alternative to cut–off is often to model unavailable data based on known data. However, if cut–offs are performed, careful considerations are required.

ISO 14044:2006 4.2.3.3 gives general guidance, especially with regard to mass, energy and environmental significance and cumulative considerations. ISO 14044:2006 4.2.3.3 recommendations shall be used as closely as possible.

The cut–off is strongly connected to chapter 5.2.2 about System boundaries, as system boundary setting can be seen as a qualitative cut–off. LCA modeling of an ICT Equipment, Network or Service involves Mandatory and Optional life cycle stages/unit processes/activities. Obviously 100% of the environmental impacts of a product system are never known a priori. However, the life cycle stages/unit processes/activities of Table 1 together constitute a significant share for typical product systems in ICT LCAs.

All cut–off criteria stated by ISO 14040 and ISO14044 is to be considered before cut–off of a certain process - and the process shall be included if significant to at least one criteria. Moreover, for each criteria the accumulated impacts shall be considered.

The cut–off criteria used within a study shall be clearly understood and described.

For a practical example on cut–off refer to Appendix I6.

The intention of this standard is to include all Mandatory activities of Table 1. If these activities are not included such cut–offs must be clearly motivated. All cut–offs shall be listed and motivated in the final report. See chapter 6 for further details.

It is mandatory to include processes/flows that have been cut–off in the sensitivity analysis.

### 5.2.4	Data Quality Requirements

In LCA context data refers to activity data, emission factors and, in some cases, direct GHG emissions.

In general data used shall reduce bias and uncertainty as far as practicable by using the best quality data achievable. Also data which is more specific with respect to time, geography and technology takes precedence over data which is less specific. Consequently primary data is generally preferred to secondary data.

Also high accuracy and preciseness is preferred.

For all data categories the data quality requirements from ISO 14040 and ISO 14044 4.2.3.6 applies.

A qualitative description on the data quality, and any efforts taken to improve data quality shall be assessed and disclosed. It is appropriate to use quality levels and quality rating for the data quality indicators suggested by ILCD Error! Reference source not found.. For example of data quality indicators please refer to appendix X

Additionally, the percentage of total GHG emissions quantified using the following data types shall be estimated and disclosed:

- ICT specific data

  - Primary data

  - Secondary data

- Generic data

  - Generic process data

  - Generic financial data.

Note! It is not always the case that primary data will have lower uncertainty than secondary data

#### 5.2.4.2 Specific requirements on data and data sources

In general data-age and technology are especially important in LCAs for ICT products, networks and services due to the fast technology evolution and the growth in network traffic. E.g. for data traffic, up–to–date figures shall always be used, e.g., for allocation between Services, as data traffic grows considerably year by year. Older figures tend to give overestimated results for energy use and related emissions per amount of data.

The following requirements on data quality applies for the different life cycle stages and unit processes

Need to clarify generic processes further here?

Table 2 Applicable data types per life cycle stage/unit processes

| Tag | Life cycle stage |  | Unit process | Type of data |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Equipment | Network | Service |
| A | Equipment Raw Material Acquisition |  |  |  |  |  |
| A1 |  | Raw material extraction |  | Generic data | Generic data | Generic data |
| A2 |  | Raw material processing |  | Generic data | Generic data | Generic data |
| B | Production |  |  |  |  |  |
| B1 |  | ICT equipment production |  |  |  |  |
| B1.1 |  |  | Parts production (for further details refer to Annex M2) | Specific data | Specific data | Specific data |
| B1.2 |  |  | Assembly | Specific data | Specific data | Specific data |
| B1.3 |  |  | ICT vendor activities | Specific data | Specific data | Specific data |
| B2 |  | Support equipment production |  |  |  |  |
| B2.1 |  |  | Support Equipment manufacturing | Specific data: Amounts etc Generic data: processes | Specific data:: Amounts etc Generic data: processes | Specific data:: Amounts etc Generic data: processes |
| B2.2 |  |  | Site construction | Specific data: Amounts etc Generic data: processes | Specific data:: Amounts etc Generic data: processes | Specific data:: Amounts etc Generic data: processes |
| C | Use |  |  |  |  |  |
| C1 |  | ICT equipment use |  | Specific data | Specific data | Specific data |
| C2 |  | Support equipment use |  | Specific data | Specific data | Specific data |
| C3 |  | Operator activities |  | Specific data | Specific data | Specific data |
| C4 |  | Service provider activities |  | Not applicable | Specific data | Specific data |
| D | Equipment End of Life Treatment |  |  |  |  |  |
| D1 |  | Re–use of ICT Equipment |  | Specific data | Specific data | Specific data |
| D2 |  | ICT specific EoLT |  | Specific data | Specific data | Specific data |
| D2.1 |  |  | Disassembly/Dismantling/ Shredding | Specific data | Specific data | Specific data |
| D2.2 |  |  | Recycling | Specific data | Specific data | Specific data |
| D3 |  | Other EoLT |  | Generic data | Generic data | Generic data |

For support activities (e.g. ICT vendor activities and operator activities) primary data applies for all individual processes under the financial or operational control of the organization undertaking the LCA, and data shall be representative of the processes for which they are collected.

## 5.3	Life Cycle Inventory (LCI)

(According to ISO 14044 Section 4.3.1 [2])

### 5.3.1	Data collection

<Editor’s note: The data collection shall be performed according to ISO 14040/44. Specify mandatory LCI flows, for example CO2, CH4. Specific versus generic data>

#### 5.3.1.1 General

(According to ISO 14044 Section 4.3.2 [2])

See section 5.3.2.

Practically, when working with certain LCA software programs, e.g., Transports&Travel (G1) and energy supplies (G2–G4) could be included in larger data sets, whereas other LCA software programs/databases provide transports/energy supply separately. The LCA practitioner shall report for which processes transports/energy supplies have been added separately and for which they are “hidden”.

A check of which mandatory elementary LCI flows (Table A.M5), are applicable to which Generic Processes (Table A.M1), Mandatory Parts (Table A.M2), EoLT processes (Table A.M3), and Raw Materials (Table A.M4) shall be perfomed.

#### 5.3.1.2 ICT equipment

5.3.1.2.1 ICT equipment energy consumption during use stage

The best way to determine the energy consumption is to measure a large number of products in real live operating environments over a long period of time (e.g. a year to capture all aspects of different use behavior, climate etc.). Many network products are installed in sites with energy meters.

If data from such measurements are not available the second best alternative would be to estimate energy consumption based on available ETSI standards for laboratory measurements of energy consumption. This method will however only give a snapshot of real energy consumption and is considered as less accurate.

The third alternative would be to use estimated/ measured energy consumption for certain user behaviour.

For many products (especially end–user equipment), periods of idling and power off are important to model the usage profile and shall be included if applicable.

The LCA practitioner is encouraged to use actual electricity mix to calculate the potential environmental impact from the use stage more exactly.

#### 5.3.1.3 Network

<Editor’s note: Contribution needed. Ericsson do this

#### 5.3.1.4 Services

<Editor’s note: Contribution needed. Ericsson do this

### 5.3.2	Data calculation

(According to ISO 14044 Section 4.3.3)

Several operational steps are needed for data calculation. These are described in ISO 14044 [2]sections 4.3.3.2 to 4.3.3.4 and 4.3.4 and ILCD Handbook [3] Section 7.10. All calculation procedures shall be explicitly documented and the assumptions made shall be clearly stated and explained. The same calculation procedures shall be consistently applied throughout the study. Practically, when working with certain LCA software programs, calculation procedure could be included in larger data sets, whereas other LCA software programs/databases provide each procedure separately.

A check on data validity shall be conducted during the process of data collection to confirm that the data quality requirements for the intended application have been fulfilled.

Validation may involve establishing, e.g., mass balances.

### 5.3.3	Allocation of data

#### 5.3.3.1 General

(According to ISO 14044 Section 4.3.3–4)

5.3.3.1.1 Allocation rules for generic processes

Data for generic processes (G1–G7) shall be allocated as a whole (i.e., for the full lifecycle for the generic process) to the associated life cycle stage of the product system.

5.3.3.1.2 Allocation rules for allocation of support activities between projects/product systems

Data for relevant part of the organization/operation shall be allocated to the relevant part of the project/ product system life cycle. If no detailed information on organization/operation is available the allocation shall be based on organizational/economic data.

5.3.3.1.3 Allocation rules for facility data

Facility data for production facilities shall preferably be allocated to product systems based on relevant physical data (i.e., layer×area for printed circuit boards, die area for ICs, weight for other components according to Table A.M2). If information regarding physical parameters is insufficient economical allocation may be used.

For software design the “facility” is usually an office. In that case the allocation rules in 5.3.3.1.2 apply.

5.3.3.1.4 Allocation rules for transports

Transports shall be allocated based on chargeable weight

5.3.3.1.5 Allocation rules for recycling

The impacts of Raw Material Recycling (G7) shall be allocated between life cycles, in practice between Raw Material Acquisition (A1-A2) and EoLT (D), according to the following principles:

- All elementary flows and consequently all environmental impacts of Landfill shall be fully allocated to the life cycle that puts the material on a landfill, or other types of rest waste storage

- The material resource depletion impact and related elementary flow shall be fully allocated to the life cycle that depletes the material resource (e.g., put the material on landfill)

- Energy recovery of incineration processes shall not be taken into account

- The 100/0 allocation method shall be used for calculating primary Raw Material Acquisition impact

- The 50/50 allocation method shall be applied when possible to allocate both the use of recycled input material in the raw material acquisition stage and the recycling of materials in the EoLT stage. USGS yearly mineral report can be used to estimate the ratio of recycled material content for input material if primary data is not accessible.

If available input LCI data does not distinguish between primary Raw Material Acquisition and Raw Material Recycling, the 100/0 method can be used as a fall–back alternative.

#### 5.3.3.2 ICT equipment

See 5.3.3.1

#### 5.3.3.3 Network

Support activities and, when applicable, Support Equipment which is shared between several nodes or all Network equipment need not be allocated to the different ICT Equipment but can be presented without being distributed.

End–user equipment (e.g., PCs, smart phones) which is accessing more than one Network (e.g. 3G, WLAN) shall be allocated to these Networks based on use time. The assumptions regarding use time for access to different Networks and off line work shall be described and motivated.

Impact from shared Network resources (e.g., transmission equipment, core nodes and data centers) shall be allocated to an access Network based on data traffic. The assumptions regarding data traffic shall be described and motivated.

#### 5.3.3.4 Services

The following recommendations for allocation of Network data to a Service shall be used:

- Data for End–users– and home/CPE equipment:

        - to be allocated based on active use time of the service

- Data for access networks, control & core nodes and operator activities:

        - to be allocated based on active use time of the service unless there is a substantial dependency between data traffic and energy consumption. E.g., mobile access networks show a large dependency between data traffic and energy consumption and a traffic model that takes data traffic into account shall be used for allocation purposes. However, also in this case the load independent part of the energy consumption can be allocated based on active use time.

- Data for transport equipment:

        - to be allocated based on data traffic

      - Data for data centers and service provider activities

        - The data center(s) where the service is operated as well as the service provider activities shall be allocated based on number of subscriptions and service users.

Note that average figures for energy use and related emissions per amount of data reflect average traffic. Thus, for low and high data traffic scenarios, average figures may give unrealistic results and results that do not reflect the actual impact of the service data traffic.

## 5.4	Life Cycle Impact Assessment (LCIA)

(According to ISO 14044 Section 4.4 and ILCD Handbook [3] Section 6.7 and 8)

The life cycle impact assessment (LCIA) handles the translation of the environmental loadings of the life cycle inventory (LCI) to potential and real environmental impacts.

In the LCA it shall be ensured that the inventory elementary flows (see Annex M5) are correctly linked with appropriate LCIA characterization factors. The mid–point category Climate change is mandatory whereas the link to end–point categories (e.g., infectious diseases and plant damage) is optional.

For climate change, the recent global warming characterization factors from the Intergovernmental Panel on climate change (IPCC) for each greenhouse gas shall be used as mid–point impact assessment category and the timeframe shall be 100 years.

For other impact categories there is no methodological consensus in the LCA community, thus the practitioner shall decide which impact categories to consider and how to calculate them, based on the studied ICT product system and purpose of the LCA study. In general, a broad approach is recommended to give a broad understanding of the environmental impact of the studied ICT product system.

All impact categories and category indicators included shall be disclosed and justified. Also the characterization factors used shall be reported in accordance with Table AN8.

Section 5.2.3 Cut–off is linked to LCIA as the LCA has to justify possible LCI flow cut–offs which could contribute significantly to any mid–point impact assessment category (especially Climate Change) in Table 4.

Table 4 below shows examples of impact assessment categories.

Table 4 Examples of environmental impact categories and indicators*

| Mid–point Impact Assessment Categories (examples) | Mid–point Category indicator (examples) | End–point Impact Assessment Categories (examples) | End–point Category indicator (examples) | Reference |
| --- | --- | --- | --- | --- |
| Climate change(CC) (mandatory) | Infrared forcing as GWP100 and GWP500 | Infectious diseases, Land loss | DALY, Extinction of species, Resource cost | IPCC for mid–point |
| Ozone depletion(OD) | UV–B radiation as Ozone Depletion Potential | Plant damage, Skin cancer | Net Primary Production, DALY | [5] |
| Human Toxicity (HTC), cancer effects | Concentration at human uptake level as Comparative Toxic Units for humans (CTUh) | Cancer | DALY | [5] |
| Human Toxicity (HTNC), non–cancer effects | Concentration at human uptake level as Comparative Toxic Units for humans (CTUh) | Memory loss | DALY | [5] |
| Respiratory inorganics / Particulate Matter (RI/PM) | Intake fraction for fine particles as ppm | Bronchitis, Asthma attacks | DALY | [5] |
| Ionizing Radiation (IRH), human health | Human exposure efficiency relative to U235 as % | Cancer | DALY | [5] |
| Ionizing Radiation (IRE), ecosystems |  |  |  |  |
| Eutrophication (EA), aquatic | Fraction of nutrients reaching freshwater end compartment (Phosphorous) or marine end compartment (Nitrogen) as PO43--equivalents | Fish population | Resource cost | [5] |
| Eutrophication (ET), terrestrial | Accumulated Exceedence in kiloequivalents nitrogen per year | Herbivore population | Resource cost, Extinction of species | [5] |
| Photochemical ozone formation (POF) | Tropospheric O3 concentration increase as C2H4-equivalents | Asthma, Plant damage | DALY, Net Primary Production | [5] |
| Acidification (A) | Acidification Potential as Accumulated Exceedence in kiloequivalent per year | Plant damage | Net Primary Production | [5] |
| Ecotoxicity (ETFW), freshwater | Concentration at aquatic ecosystem species uptake level as Comparative Toxic Units for ecosystems (CTUe) | Aquatic ecosystem population | Extinction of species | [5] |
| Land use (LU) | Soil Organic Matter as the amount and quality deficit of land occupied or transformed | Land loss | Extinction of species, Resource cost | [5] |
| Resource Depletion (RDW), water | Water amount as Water use related to local scarcity of water | User cost | Resource cost | [5] |
| Resource Depletion (RDMR), mineral, fossil,and renewable | Resource amount as scarcity | User cost | Resource cost | [5] |

*The mid–point impact assessment categories are suggested by the ILCD [5].

´

Section 5.2.3 Cut–off is linked to LCIA as the LCA has to motivate eventual LCI flow (from Table AM5) cut–offs which could contribute significantly to any mid–point impact assessment category (especially Climate Change) in Table 4.

## 5.5	Life Cycle Interpretation

(According to ISO 14044 Section 4.5)

5.5.1 General

In the life cycle interpretation, the results of the LCA are evaluated in order to answer questions raised in the goal definition (Section 5.2). The steps of the interpretation shall ensure the robustness of the conclusions from the LCA.

During the iterative steps of the LCA the interpretation phase serves to improve the LCI model.

In the end the interpretation relates to the intended applications of the LCI/LCA study and is used to draw conclusions, identify limitations and produce recommendations.

The life cycle interpretation shall include an analysis of the results and the consistency, a completeness check, and a sensitivity check of the significant issues and methodological choices as to understand the uncertainty of the results.

The challenge of the completeness check is to overcome the paradox of evaluating the degree of completeness of the product system when not knowing 100% of it.

If two or more ICT Equipment, Networks or Services LCA results do not differ significantly there is a risk of erroneous interpretations. I.e., there is a risk of inappropriately claiming equality/superiority of one or several compared alternatives, based on poor data quality that results in underestimations/overestimations of differences. This risk could lead to bad general conclusions and recommendations.

5.5.2 Uncertainty

The uncertainty of the results of an LCA should be assessed in accordance with ISO 14044:2006 to the extent needed to understand the results.

Also the sources of uncertainty and methodological choices made shall be assessed and disclosed.

For more information regarding uncertainty categories and important uncertainty sources for the different life cycle phases of ICT products, networks and services, refer to Appendix X.

For information regarding opportunities and limitations in the use of LCA for ICT products, networks and services refer to Appendix Y.

5.5.2 Sensitivity analysis

Especially when modelled data is used, different scenarios should be assessed to establish a range of potential outcomes to limit the uncertainty. For requirements on sensitivity analysis refer to ISO 14044 clause 4.5.5.3.

## 5.6	Comparative LCA between ICT and business as usual systems

To be able to quantify the net environmental impact when introducing an ICT based service the environmental impact of both the service itself and of the reference product system shall be assessed from a life cycle perspective. Potentially the reference product system could be any product system including another ICT based system.

The net environmental impact resulting from the introduction of ICTs is calculated as the difference between the environmental load of the reference product system which could be avoided by introducing the ICT based system, and the environmental load of the ICT based system itself.

Comparative LCAs between an ICT based system and a reference product system (e.g., comparison between a face–to–face business meeting including air transport and the ICT Service video conference) aim to compare LCA results for different products, systems, or services that offer the same or similar functions.

To make sure that the comparative assessment is giving a correct result the full life cycle of both systems shall always be included. Cut-off is only allowed if some processes are identical between the systems or are found negligible when applying the cut-off rules of this standard

Correct comparisons also require that the same goal, scope, system boundaries and functional unit are used for both product systems.

From an LCA perspective the reference product system and the ICT Service shall mimic each other as far as possible and the practitioner shall model both systems in an unbiased way. In reality the two product systems may differ with respect to quality, e.g. the experience of a face–to–face meeting is different from that of a video conference meeting.

Usually, the most challenging part for a comparative assessment is to collect real-world data for the use phase both for the reference system and for the ICT based system. Lack of real-world data can be bridged by scenarios. The impact from the scenarios on the results is preferably evaluated by the use of sensitivity analysis.

In general the time perspective applied as well as the scale of introducing the ICT based product system is crucial to the modelling. These perspectives may vary with study scope and purpose, e.g. a small-scale application of a video conferencing system will not in the near future impact the amount of air-planes used, while a large-scale application may have a considerable impact in a medium time frame.

Infrastructure e.g. highways for transportation, is generally assumed to exist independently of introduction of new services and shall be excluded. However in some LCAs focusing on large scale effects of services, infrastructure effects may be applicable to the study scope (i.e. for an LCA trying to examine the effects of large-scale, long-term implications of a wide application of video conferencing). In those cases infrastructure associated impacts should be reported separately

The handling of time-perspective and scale shall be disclosed and motivated in the report.

Considering the complexities associated with comparative assessments,  restrictions as to the interpretation of the results and the equivalence of the ICT and reference product  systems are to be carefully observed, to avoid misinterpretation of results.

A schematic illustration of a comparative assessment is shown in Figure 1. The figure indicates that the reference system and the ICT based system are assessed separately and then compared.

The assessment of the ICT based system shall be performed in accordance with this standard.

For the reference product system applicable requirements in this standard shall be applied, e.g. requirements regarding data quality, cut-off etc. To get further guidance on system boundaries and other product system specific considerations sector specific standards should be used if available

As indicated above the assessment procedure contains several steps

- Definition of goal, functional unit and scenarios

- Definition of system boundaries for each product system

- Life Cycle Inventory including data collection for each product system

- Life Cycle Impact Assessment for each product system

- Life Cycle Interpretation including comparison

.

# 6	Reporting

## 6.1	General

The reporting of ICT product systems shall fulfil the reporting rules as defined by ISO 14040 and ISO 14044.

In addition the rules outlined in this chapter shall also be followed for reporting of studies claiming compliance with this standard.

The intention of having common ways of reporting is to increase transparency and provide the reader with a proper basis for interpretation of results. Optionally other data, graphs, statements etc may be added to the report based on the scope and purpose of the LCA.

For each product system (including ICT Equipment, Network and Service) the following aspects, being of special importance to ICT applications, shall be transparently motivated and described in accordance with the principles defined in this chapter:

- Operating lifetime: All lifetime assumptions shall be stated and motivated

- Cut–off: Any cut–off made shall be clearly stated and motivated

- Allocations: Basis for allocations made shall be described, especially for recycling, use of recycled materials, distribution of facility data and support activities.

- Data sources: Data sources (i.e. ICT specific/generic) shall be clearly stated, and deviations towards Table 2 shall be motivated

## 6.2	ICT equipment

### 6.2.1 Total result

For each impact category studied, a diagram corresponding to below Fig. 8 shall be reported for the corresponding category indicator result. Due to the importance of operating lifetime to results, information regarding this shall always be present in the graph, together with some other basic modelling statements including total result for the indicator, LCA study year operating lifetime etc. as shown below.

Fig. 8 Environmental impact category indicator result diagram example (diagram for Global Warming Potential (GWP100) (CO2e).

For transports, the total result including all transports throughout the life cycle (Annex N1 Table AN2) shall be stated in the immediate proximity of the diagram (Fig. 8). If used data sets do not report transports separately any missing transport shall be listed and motivated.

Optionally, a graph showing distribution between sub–unit processes within each life cycle stage could also be reported (Fig. 9). Such transparency is encouraged.

Fig. 9 Environmental impact category indicator result: distribution between sub–unit processes within each life cycle stage

Additionally a graph summarizing distribution of environmental impact category indicators between life cycle stages according to Fig. 10 below shall also be presented together with absolute figures as shown in the Annex N1, Table AN8.

Fig.10 Environmental impact category indicators overview for ICT Equipment

### 6.2.2 System boundaries

#### 6.2.2.1 Life cycle stages, unit processes and generic processes

Any deviation to Table 1 and 5.2.2 with respect to life cycle stages/activities shall be clearly stated and motivated.

Also handling of optional stages/activities shall be clearly reported as well as electricity mix applied, and handling of support activities and transports. Especially for transports, lack of transparent data is common for many unit processes, which must be considered for the reporting.

Additionally, inclusion of generic processes shall be clearly stated.

For appropriate reporting format refer to Annex N1, Table AN1.

For reporting of transports and travel refer to Annex N1, Table AN2. State if data is missing or is included in, e.g., support activity data but could not be reported separately.

#### 6.2.2.3 Raw material acquisition

The use of raw materials shall be transparently reported as outlined below. The most important metals from recycling point of view shall always be included. Other materials can be shown as well but such reporting is optional.

For appropriate reporting format refer to Annex N1, Table AN3.

#### 6.2.2.4 Production

Parts production

Compliance to Annex M2 Table A.M2 shall be reported according to below and any deviation shall be described and motivated.

For appropriate reporting format refer to Annex N1, Table AN4.

#### 6.2.2.5 Use

ICT equipment use

The basis for the energy consumption figures for the ICT equipment use stage shall be reported together with the annual value of the energy consumption

For many products (especially end–user equipment), periods off idling and power off are important to model the usage profile. It is therefore mandatory to transparently report distribution over time for different usage modes including power off and idle and the basis for those.

For appropriate reporting format refer to Annex N1, Table AN5.

Support equipment use

The basis for the energy consumption figures for the support equipment use shall be transparently described and motivated.

For appropriate reporting format refer to Annex N1, Table AN5.

#### 6.2.2.6 EoLT

If EoLT is included any deviations towards Annex M3 shall be transparently reported and motivated.

For appropriate reporting format refer to Annex N1, Table AN6.

### 6.2.3 LCI results

For LCI the following items shall be reported transparently: total use of primary energy, electricity, land use and fresh water.

Additionally, results for elementary flows according to Annex M5 Table A.M5 could be transparently reported on an optional basis. If such reporting is not made it is mandatory to describe unexpected results, lack of data and other findings associated with the elementary flows.

For appropriate reporting format refer to Annex N1, Table AN7.

## 6.3	Network

### 6.3.1 Total result

For each environmental impact category studied, a diagram corresponding to Fig. 11 below shall be reported for the corresponding category indicator result. Operating lifetime is important also for Networks, but is associated with the lifetime of the different nodes, which shall be reported together in accordance with Annex N1, Table AN9 which also describes the studied Network.

Fig. 11 Environmental impact category indicator result diagram example for Network (diagram for Global Warming Potential (GWP100) (CO2e).

Optionally, a graph showing distribution between sub–activities within each life cycle stage could also be reported in the same way as for ICT equipment (see previous chapter).

Additionally a graph summarizing distribution of environmental impact category indicators between life cycle stages according to below shall also be presented together with absolute figures as shown in the Annex N1, Table AN8.

Fig. 12 Environmental impact category indicators overview for Networks

For reporting of Network energy consumption refers to Annex N1, Table AN10.

## 6.4	Services

### 6.4.1 Total result

For each impact category studied, a diagram corresponding to below shall be reported for the corresponding category indicator result. Operating lifetime is important also for Services, but it is associated with the lifetime of the different nodes, which shall be reported together in accordance with Annex N1, Table AN9 which also describes the studied Network.

Fig. 13 Environmental impact category indicator result diagram example for Services (diagram for Global Warming Potential (GWP100) (CO2e).

Allocation of Network data to the Service shall be reported according to Annex N1, Table AN11.

Additionally a graph summarizing distribution of impact category indicators between life cycle stages according to below (Fig. 14) shall also be presented together with absolute figures as shown in the Annex N1, Table AN8:

Fig. 14 Environmental impact category indicators overview for Services

In addition to the service reporting described above, the general reporting principles for Networks apply.

# 7	Critical review

Any critical review shall be performed according to the requirements in ISO 14040 and ISO 14044 and in this Standard.

# Annexes (normative)

## Annex M1 (Normative)	Generic processes

Table AM1 defines the generic processes (G1–G7) to be included if applicable in LCAs for ICT Equipment, Networks, and Services, as well as examples of categories and examples of unit processes to be included.

Table AM1 Generic processes for LCA of ICT equipment. **=optional

| Generic process | Generic process categories | Unit processes (for each category) | Product flow unit | Important issues |
| --- | --- | --- | --- | --- |
| G1. Transports&Travel | Road Air Ship Train | Mandatory: Direct (during transport) emissions Fuel supply chain Optional: Vehicle production Infrastructure production | ton×km, kg×km, Cton×km | Chargeable weight = Cton×km (function that also considers volume or density) |
| G2. Electricity | Div. country, regional and producer electricity mixes | Mandatory: Fuel supply chain, Direct emissions (during electricity production) Optional: power plant production, dam production, the grid production, nuclear waste treatment | kWh |  |
| G3. Fuels | Oil Diesel Petrol Jet—fuel LPG LNG Coal Gas | Mandatory: Fuel supply chain: Extraction and Production Distribution (transports) and emissions related to the incineration of the fuel is connected to a unit process or "site LCI model". | mass, volume, energy content |  |
| G4. Other energy | District heating (hot water) District heating (steam) District cooling (cold water) as electricity | Mandatory: Fuel supply chain, Direct emissions during energy/electricity production Optional: Power plant construction, Infrastructure production | kWh | Electricity is also a energy source or district heating/cooling production |
| G5. Raw material acquisition |  | Mandatory: Extraction Processing | mass, volume |  |
| G6. End–of–life treatment | See Annex M3 |  |  |  |
| G7. Raw material recycling |  |  |  |  |

## Annex M2 (Normative) Part types

Table AM2 lists parts which shall be taken into account (if applicable to the Goal&Scope) when performing an LCA of ICT equipment, as well as the corresponding categories and unit processes. The list is to be regarded as a mandatory list and more parts/part categories/unit processes may be included.

Table AM2 Mandatory set of parts for LCA of ICT equipment.

| Part | Mandatory set of Part categories | Part unit processes (for each Part category) | Product flow unit | Important issues which influence LCI data |
| --- | --- | --- | --- | --- |
| B1.1.1 Batteries | Lead batteries Lithium batteries Nickel—Cadmium batteries | Raw Material Acquisition, Battery cell assembly, Battery module assembly | Piece, energy content, mass | Size |
| B1.1.2 Cables | Coaxial cables Fiber cables Power cables Network/signal cables Connectors | Raw Material Acquisition, Cable final assembly (one unit process per part category*) | Piece, mass | Length |
| B1.1.3 Electro—mechanics | Connectors Electric motors Speakers Microphones Camera objectives Hard Disc Drives Lighting (lamps) | Raw Material Acquisition, Part final assembly (one unit process per part category*) | Piece, mass |  |
| B1.1.4 Integrated circuits (ICs) | Processors, DSPs ASICs Memories Microprocessors Transistors and diodes | Front–end: Special IC materials production, Wafer production, Chip production (“the wafer fab”) Back–end: IC encapsulation | Piece, front–end: good die area [cm2], back–end: piece package type Example1: A BGA289 package. Good die area 0.166 cm2. Environmental Load per BGA289 = ELU/good die area in cm2 × 0.166 + ELU/piece BGA package type back-end process Example2: A “stacked chip” package. Total Good die area 12 cm2. Environmental Load (ELU) per “stacked chip” package = ELU/good die area in cm2 × 12 + ELU/piece “stacked chip” package type back-end process Transistors and diodes: Piece, mass | Yield in chip production Business activities Factory and machinery |
| B1.1.5 Mechanics / Materials | Nuts, bolts, screws Fronts Frames Racks Cabinets Towers Containers Solder | Raw Material Acquisition, Part final assembly (one unit process per part category*) | Piece, mass |  |
| B1.1.6 Displays | PDP LCD LED | Raw materials Acquisition for special display panel materials, Display panel assembly, Display module assembly | Piece, mass, active area | Yield Business activities, Factory and machinery |
| B1.1.7 PCB | Plastic Ceramic Flex–film | Raw materials Acquisition for special PCB materials, Raw materials Acquisition for PCB semi–produced composite materials, PCB final assembly | Piece, mass, cm2×layer | Yield Business activities, Factory and machinery |
| B1.1.8 Other PCBA components | Resistors Capacitors Inductors Relays LEDs Potentiometers Quartz crystal oscillator | Raw Material Acquisition, Part final assembly (one unit process per part category*) | Piece, mass |  |
| B1.1.9 Packaging materials | Paper Cardboard Plastics Wood Steel | Raw Material Acquisition (one Raw Material per part category*) | Mass, volume | Lifetime, Reuse, energy recovery |
| B1.1.10 Black box modules | Modules bought by ICT equipment producer as complete products (e.g., cameras, modules, memories, printer cartridges) | “Cradle–to–gate” LCA from supplier | Piece, mass | Size, mass, technology |
| B1.1.1 Software | Software development |  |  |  |

*one unit process per part category means that sub–part assembly, and sub–sub part assembly etc. are optional. E.g., for the Cable Part Category Coaxial Cables the Raw Material Acquisition of the material content of the Coaxial Cables, and ancillary materials used in the Coaxial Final Assembly, and the Coaxial Cable Final Assembly are mandatory. The Sub–Part and sub–sub Part assembly factories and Raw Materials used therein are optional.

## Annex M3 (Normative)	EoLT processes

Table AM3 below defines the different generic and specific EoLT processes to be included (if applicable to the Goal&Scope) in LCAs for ICT equipment as well as mandatory process categories and mandatory corresponding EoLT process unit processes. The list is to be regarded as a short mandatory list and more EoLT processes/process categories/unit processes may be included. Usually D3. Other EoLT (Fig.7) consists of combinations of G6.1 and G6.2.

Table AM3 Examples of EoLT processes for LCA of ICT equipment

| G6. End of Life Treatment | Examples of process categories | EoLT process unit processes (for each category) | Product flow unit | Important issues |
| --- | --- | --- | --- | --- |
| G6.1 EHW treatment | EHW (destruction and energy recovery) Special EHW landfill | In general: Recovery/treatment (one unit process or "site LCI model" for the entire EHW category) | mass, (energy content) |  |
| G6.2 Other Waste treatment | Div. material recycling Energy recovery (e.g., incineration) Landfill | In general: Recycling/recovery/treatment (one unit process or "site LCI model" for each material/ waste categories) | mass, (energy content) |  |
| D. EoLT | D1. Re–use of ICT Equipment D2. ICT specific EoLT D2.1 Storage/Disassembly/Dismantling/Shredding D2.2 Recycling D2.2.1 Battery recycling ICT specific metal/mechanical parts/fractions EoLT D2.2.2 PCBA recycling D2.2.3 Cable recycling D2.2.4 Mechanics recycling D2.2.5 Other ICT recycling D3. Other EoLT | Recycling, recovery and treatment (one unit process per material/ waste category) | mass |  |

## Annex M4 (Normative)	Mandatory list of Equipment Raw material acquisition processes

Table AM4 below lists Raw Materials (chemicals, fuels, metals, plastics, packaging materials, and additives) groups which shall be taken into account in LCAs of ICT Equipment. Table AM4 would be very long and unmanageable as there are many variants of each chemical, fuel, metal and alloy, plastic add additive. Therefore each material name in Table AM4 refers to a group of Raw Materials and not specific CAS number materials. These Raw Materials groups are either part of the material content of the ICT Equipment/Support Equipment or used as ancillary materials throughout the life cycle. Table AM4 is to be regarded as a mandatory list and more Raw Materials groups may be included.

Table AM4 Mandatory set of cradle–to–gate raw material acquisition processes to be included in LCA of ICT Equipment if applicable to the studied ICT product system

Chemicals

Nitrogen gas (N2)

Oxygen gas (O2)

Hydrogen gas (H2)

Argon gas (Ar)

Acetone

CaO

H2SO4

H2O2

HydroCloric Acid (HCl)

FeCl3

IsoPropyleneAlcohol

Ethylene glycol

HydroFluoric acid

H3PO4

HNO3

NaOH

…

…

Fuels

Heating oil

Bunker oil / ship diesel

Diesel

Petrol

Jet fuel

LPG

LNG

“Biofuels”

…

…

Metals and alloys

Aluminium

Brass

Cadmium

Chromium

Copper

Copper (Brass, Bronze)

Gold

Lead

Lithium

Magnesium

Mercury

Nickel

Palladium

Platinum

Silicon

Silver

Solder – SAC

Solder – Sn/Pb

Solder – SnZn

Stainless steel

Steel – Cr3+ plated

Steel – powder coated

Steel – zinc plated

Steel (Stainless steel, Iron)

Tin

Tin (Div. Solder alloys)

Zinc

Hard metal (W-Co)

….

….

Others

Concrete

Packaging materials

Ceramics

Paper

Cardboard

Wood

Wood board

Glass

Glassfiber

…

….

Plastics

Acrylonitrile butadiene styrene (ABS)

Epoxy

Polycarbonate (PC)

Polyethylene (PE) — HD

Polyethylene (PE) — LD

Polypropylene (PP)

Polystyrene (PS)

Polyurethane (PUR)

Polyester (e.g. PET)

PVC

Silicone rubber

SAN

PA (Nylone)

PTFE (Teflone)

PMMA

…

…

Additives and others

Brominated flame retardants (e.g. TBBP-A)

Sb2O3

Be in alloys ...and similar

Plastic additives

High purity grades of materials/chemicals and gases

Cooling media, fire extinguisher media (High GWPs/ODPs)

…

## Annex M5 (Normative) Mandatory list of elementary flows in LCA of ICT

Table AM5 below contains elementary flows which shall be taken into account in LCA analyses for ICT. More flows could be relevant and the list refers to the mandatory LCI flows. The most commonly used name, chemical name or abbreviation has been put first followed by other common names/abbreviations. It is mandatory to use the substance names of Table AM5 in reporting.

The mandatory unit is mass (unless stated otherwise): g, kg, ton

.

Table AM5 Elementary flows which shall be taken into account in LCAs of ICT

| Inventory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Substance | Measure unit | Quantity | Source | Mid–point Impact Assessment Categories Contribution (Table 4) |
| Aluminum (resource) | kg |  |  | RDMR, LU |
| Ammonia (to air) | kg |  |  | A |
| As (to air) | kg |  |  | HTC, , ETFW |
| As (to water) | kg |  |  | HTC, ETFW |
| As (to ground) | kg |  |  | HTC,, ETFW |
| Benzene (to air) | kg |  |  | HTC, ETFW |
| BOD (to water) | kg |  |  | EA |
| Cd (to air) | kg |  |  | HTC, ETFW |
| Cd (to water) | kg |  |  | HTC, ETFW |
| Cd (to ground) | kg |  |  | HTC, ETFW |
| Cr (to water) | kg |  |  | HTC, ETFW |
| Cr (to ground) | kg |  |  | HTC, ETFW |
| CClF3, (CFC–13) (to air) | kg |  |  | OD , CC |
| CCl3F, (CFC–11) (to air) | kg |  |  | OD, CC |
| CCl2F2, (CFC–12) (to air) | kg |  |  | OD, CC |
| Cl2FC-CClF2, (CFC–113) (to air) | kg |  |  | OD, CC |
| C2F6 (CFC–116) (to air) | kg |  |  | OD, CC |
| C2H2F4, HFC–134a (to air) | kg |  |  | OD, CC |
| C2H3F3, HFC–143a (to air) | kg |  |  | OD, CC |
| C2H1F5, HFC–125 (to air) | kg |  |  | OD, CC |
| C2H3Cl2F, HCFC–141b (to air) | kg |  |  | OD, CC |
| CF2ClBr, Halon 1211 (to air) | kg |  |  | OD, CC |
| CF3Br, Halon 1301 (to air) | kg |  |  | OD, CC |
| CF4, CFC–14 (to air) | kg |  |  | OD, CC |
| CH3Cl3, HCFC–140 (to air) | kg |  |  | OD, CC |
| CH4, Methane (to air) | kg |  |  | CC |
| CHF2Cl, HCFC–22 (to air) | kg |  |  | OD, CC |
| CHF3, HFC–23 (to air) | kg |  |  | OD, CC |
| Cl- (to water) | kg |  |  | HTNC, ETFW, A |
| Cl- (to ground) | kg |  |  | HTNC, ETFW, A |
| CO, Carbon monoxide (to air) | kg |  |  | HTNC, POF, CC |
| CO2 (to air) | kg |  |  | CC, A |
| Coal (resource) | kg, TOE, MJ |  |  | RDMR, LU |
| Copper (resource) | kg |  |  | RDMR, LU |
| Copper (to air) | kg |  |  | HTNC, ETFW |
| Copper (to water) | kg |  |  | HTNC, ETFW |
| COD (to water) | kg |  |  | EA |
| EHW = Environmental hazardous (waste) | kg |  |  | LU |
| EHW Ashes, special EHW landfill (waste) | kg |  |  | LU |
| EHW Metal hydroxides (MeOH), special EHW landfill (waste) | kg |  |  | LU |
| EHW Slag, special EHW landfill (waste) | kg |  |  | LU |
| Ethylene (air) | kg |  |  | POF |
| Ethylene oxide (air) | kg |  |  | HTNC, ETFW |
| Formaldehyde, CH2O (to air) | kg |  |  | HTC, POF |
| Gas (resource) | kg, TOE, MJ |  |  | RDMR, LU |
| Gold (resource) | kg |  |  | RDMR, LU |
| Hg (to air) | kg |  |  | HTC, ETFW |
| Hg (to water) | kg |  |  | HTC, ETFW |
| Hg (to ground) | kg |  |  | HTC, ETFW |
| Hydrogen chloride (to air) | kg |  |  | A, HTNC |
| Iron (to air) | kg |  |  | HTNC |
| Iron (resource) | kg |  |  | RDMR |
| Iron (to water) | kg |  |  | HTNC |
| Iron (to ground) | kg |  |  | HTNC |
| Land occupation, agricultural | m2×year |  |  | LU |
| Land occupation, urban | m2×year |  |  | LU |
| Metals (unspecified) (to water) | kg |  |  | HTNC, HTC, ETFW |
| Metals (unspecified) (to ground) | kg |  |  | HTNC, HTC, ETFW |
| Mineral (waste) | kg |  |  | LU |
| Mo (to water) | kg |  |  | HTNC, ETFW |
| Mo (to ground) | kg |  |  | HTNC, ETFW |
| N2O (to air) | kg |  |  | CC, OD |
| NF3 (to air) | kg |  |  | CC, OD |
| Nickel (resource) | kg |  |  | RDMR, LU |
| Nickel (to water) | kg |  |  | HTNC, ETFW |
| Nickel (to ground) | kg |  |  | HTNC, ETFW |
| N–total (to water) | kg |  |  | EA, ET |
| Nitrate, NO3- (to water) | kg |  |  | EA |
| NMVOC, non-Methane VOC (to air) | kg |  |  | POF, RI/PM, HTNC |
| NMHC, non-Methane hydrocarbons (to air) | kg |  |  | HTNC, HTC, POF |
| NOx (to air) | kg |  |  | EA,ET, POF, HTNC, ETFW |
| Oil (to ground) | kg |  |  | HTC |
| Oil (resource) | kg, TOE, MJ |  |  | RDMR, LU |
| Oil (to water) | kg |  |  | HTC |
| Other CFCs/HFCs/HCFCs/PFCs (to air) | kg |  |  | OD, CC |
| Other (new) "high GWPs/ODPs" (to air) | kg |  |  | OD, CC |
| PAH, all kinds (to air) | kg |  |  | HTC |
| Palladium (resource) | kg |  |  | RDMR, LU |
| Platinum(resource) | kg |  |  | RDMR |
| Particulates, all kinds (to water) | kg |  |  | HTC, HTN, ETFW |
| Particulates, all kinds (to air) | kg |  |  | HTC,ETFW, POF, CC, RI/PM |
| Pb (resource) | kg |  |  | RDMR |
| Pb (to air) | kg |  |  | HTNC, ETFW |
| Pb (to water) | kg |  |  | HTNC, ETFW |
| Pb (to ground) | kg |  |  | HTNC, ETFW |
| PF3 (to air) | kg |  |  | CC, OD |
| Phosphate, PO43-- (to water) | kg |  |  | EA, ET |
| P–total (to water) | kg |  |  | EA, ET |
| Radioactive (low, volume) (waste) | kg |  |  | IRH, IRE |
| Radioactive (medium, volume) (waste) | kg |  |  | IRH, IRE |
| Radioactive (high, volume) (waste) | kg |  |  | IRH. IRE |
| Selenium (to water) | kg |  |  | HTNC, ETFW |
| Selenium (to ground) | kg |  |  | HTNC, ETFW |
| Silver (resource) | kg |  |  | RDMR, LU |
| Solid waste to landfill (waste) | kg |  |  | LU |
| SF6 (to air) | kg |  |  | CC, OD |
| SO2 (to air) | kg |  |  | A, POF, HTNC, ETFW, RI/PM |
| SOx (to air) | kg |  |  | A, POF, HTNC,,ETFW, RI/PM |
| TCDDe ("Dioxin" equivalents) (to air) | kg |  |  | HTC |
| Tin (resource) | kg |  |  | RDMR |
| Titanum (to water) | kg |  |  | HTNC, ETFW |
| Titanum (to ground) | kg |  |  | HTNC, ETFW |
| Toluene (to air) | kg |  |  | POF, HTC, HTNC |
| Uranium (resource) | kg, MJ, TOE |  |  | RDMR |
| Water, lake (resource) | m3 |  |  | RDW |
| Water, river (resource) | m3 |  |  | RDW |
| Water, well, in ground (resource) | m3 |  |  | RDW |
| Water, unspecified, natural origin (resource) | m3 |  |  | RDW |
| Zinc (to water) | kg |  |  | HTNC, ETFW |
| Zinc (resource) | kg |  |  | RDMR |
| Zinc (to ground) | kg |  |  | HTNC, ETFW |
| Zinc (to air) | kg |  |  | HTNC, ETFW |

## Annex N1 (Normative) Table reporting formats

Table AN1Reporting format for included life cycle stages, activities and generic processes

| Tag | Life cycle stage | Unit process | Included (Yes/No) | Electricity mix (specific/country/world average | Support activities included (Yes/No) | Transport activities included (Yes/No) | Other generic activities included (Yes/No) | Motivation/Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Equipment Raw Material Acquisition |  |  |  |  |  |  |  |
| A1 | Raw material extraction |  |  |  |  |  |  |  |
| A2 | Raw material processing |  |  |  |  |  |  |  |
| B | Production |  |  |  |  |  |  |  |
| B1 | ICT equipment production |  |  |  |  |  |  |  |
| B1.1 |  | Parts production (for further details refer to Annex N1) |  |  |  |  |  |  |
| B1.2 |  | Assembly |  |  |  |  |  |  |
| B1.3 |  | ICT vendor activities |  |  |  |  |  |  |
| B2 | Support equipment production |  |  |  |  |  |  |  |
| B2.1 |  | Support Equipment manufacturing* |  |  |  |  |  |  |
| B2.2 |  | Site construction |  |  |  |  |  |  |
| C | Use |  |  |  |  |  |  |  |
| C1 | ICT equipment use |  |  |  |  |  |  |  |
| C2 | Support equipment use |  |  |  |  |  |  |  |
| C3 | Operator activities |  |  |  |  |  |  |  |
| C4 | Service provider activities |  |  |  |  |  |  |  |
| D | Equipment End of Life Treatment |  |  |  |  |  |  |  |
| D1 | Re–use of ICT Equipment |  |  |  |  |  |  |  |
| D2 | ICT specific EoLT |  |  |  |  |  |  |  |
| D2.1 |  | Storage/Disassembly/Dismantling/ Shredding |  |  |  |  |  |  |
| D2.2 |  | Recycling |  |  |  |  |  |  |
| D3 | Other EoLT |  |  |  |  |  |  |  |

Table AN2 Reporting format for transports/travel

| Mode | CO2 emission factor (fuel consumption only/ /including fuel supply) | Raw material acquisition transports |  | Production stage transports excluding final transport |  | Final transport (Production to use stage) |  | Use stage transports |  | EoLT transports |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Transport work {ton×km} | GWP100 {kg CO2e} | Transport work {ton×km} | GWP100 {kg CO2e} | Transport work {ton×km} | GWP100 {kg CO2e} | Transport work {ton*km} | GWP 100 {kg CO2e} | Transport work {ton×km} | GWP 100 {kg CO2e} |
| Air |  |  |  |  |  |  |  |  |  |  |  |
| Other |  |  |  |  |  |  |  |  |  |  |  |
| Mode | CO2 emission factor (fuel consumption only/ /including fuel supply) | Total travels |  |
|  |  | Transport distance {km} | GWP100 {kg CO2e} |
| Air |  |  |  |
| Other |  |  |  |

Table AN3 Reporting format for raw materials

|  | Total input (g,kg, ton) | Recycled input (%) | Content in product (%) | Recycling of total input* (%) | Reference |
| --- | --- | --- | --- | --- | --- |
| Iron/Steel alloys |  |  |  |  |  |
| Aluminum alloys |  |  |  |  |  |
| Copper alloys |  |  |  |  |  |
| Silver |  |  |  |  |  |
| Gold |  |  |  |  |  |
| Equipment Raw materials (optional) |  |  |  |  |  |
| Raw material 1 |  |  |  |  |  |
| Raw material … |  |  |  |  |  |
| Raw material n |  |  |  |  |  |
| Auxiliary Raw Materials(production materials etc.) (optional) |  |  |  |  |  |
| Auxiliary material 1 |  |  |  |  |  |
| Auxiliary material … |  |  |  |  |  |
| Auxiliary material n |  |  |  |  |  |
| Packaging materials(optional) |  |  |  |  |  |
| Packaging material 1 |  |  |  |  |  |
| Packaging material … |  |  |  |  |  |
| Packaging material n |  |  |  |  |  |

*) Total recycling of all input materials, i.e., recycling of manufacturing waste and recycling of total content in final product.

For a full list of materials that could optionally be reported in Table AN3 refer to Annex M4.

Table AN4 Reporting format for parts production

|  | Part categories included | Part Unit processes included | Handling of special issues |
| --- | --- | --- | --- |
| B1.1.1 Batteries |  |  |  |
| B1.1.2 Cables |  |  |  |
| B1.1.3 Electro–mechanics |  |  |  |
| B1.1.4 Integrated circuits (ICs) |  |  |  |
| B1.1.5 Mechanics / materials |  |  |  |
| B1.1.6 Displays |  |  |  |
| B1.1.7 Printed circuit boards (PCBs) |  |  |  |
| B1.1.8 Other PBA components |  |  |  |
| B1.1.9 Packaging materials |  |  |  |
| B1.1.10 Black box modules |  |  |  |

Table AN5 Reporting format for use stage energy consumption

|  | Energy consumption {kWh/year} | Source {long term average/standardized measurement/ modelled | Motivation/ comment |
| --- | --- | --- | --- |
| ICT equipment |  |  |  |
| Support equipment |  |  |  |

Table AN6Reporting format for EoLT

|  | Process categories included | Process Unit processes included | Handling of special issues |
| --- | --- | --- | --- |
| EHW treatment |  |  |  |
| Other Waste treatment |  |  |  |
| EoLT |  |  |  |

Table AN7 Reporting format for LCI results

|  | TOTAL | Raw materials acquisition and EoLT | Production | Use |
| --- | --- | --- | --- | --- |
| Primary energy use |  |  |  |  |
| Total electricity use |  |  |  |  |
| Land use |  |  |  |  |
| Fresh water use |  |  |  |  |
| LCI data 1(optional) |  |  |  |  |
| LCI data …(optional) |  |  |  |  |
| LCI data n (optional) |  |  |  |  |

Table AN8 Impact category indicators

|  | Impact category indicator value | LCIA methodology reference |
| --- | --- | --- |
| Global warming potential, GWP, GWP100 | # kg CO2e |  |
| Ozone depletion potential, ODP, ODP20 |  |  |
| Etc. |  |  |

Table AN9 Reporting format for Network description

| End–user and home equipment dimension | Included ICT Equipment | Included infrastructure | Quantity [unit] | Lifetime [year] |
| --- | --- | --- | --- | --- |
| End–user equipment |  |  | # [p] |  |
| Home equipment |  |  | # [p] |  |
| Operator Network and activities dimension |  |  |  |  |
| Access network |  |  | # [sites] |  |
| Control & core network |  |  | # [sub] |  |
| Operator activities |  |  | # [empl.] |  |
| Data Services dimension |  |  |  |  |
| Data transport |  |  | # [GB] |  |
| Data center(s)3 |  |  | # [GB] |  |
| Service provider(s) activities |  |  | # [sub] |  |

Table AN10 Reporting format for Network Energy consumption

| End-user and home equipment dimension | ICT equipment energy consumption | Support equipment energy consumption | Source {long term average/ standardized measurement/ modelled} |
| --- | --- | --- | --- |
| End-user equipment |  |  |  |
| Home equipment |  |  |  |
| Operator Network and activities dimension |  |  |  |
| Access network1 |  |  |  |
| Control & core network |  |  |  |
| Operator activities |  |  |  |
| Data Services dimension |  |  |  |
| Data transport2 |  |  |  |
| Data center(s)3 |  |  |  |
| Service provider(s) activities |  |  |  |

Table AN11Reporting format for Service hardware allocation

| End-user and home/ CPE equipment | Allocation method | Allocation of use stage [%] | Allocation of all non–use stages [%] |
| --- | --- | --- | --- |
| End–user equipment |  |  |  |
| Home/CPE equipment |  |  |  |
| Operator Network and activities dimension |  |  |  |
| Access network |  |  |  |
| Control & core network |  |  |  |
| Operator activities |  |  |  |
| Data Services dimension |  |  |  |
| Data transport |  |  |  |
| Data center(s) |  | Specific data center(s) data mandatory |  |
| Service provider(s) activities |  | Specific service provider(s) data mandatory |  |

## Annex N2 (Normative) Diagram reporting formats

Figure AN5 Reporting format for environmental impact category indicators overview for Network

Figure AN6 Reporting format for Service category indicator results

# Appendices (informative)

<Editor’s note: Examples and background information shall be put here. Once the methodology has been fully defined the examples needs to carefully be reviewed to really illustrate the Standard>

A more detailed overview figure showing the detailed contents and connections between all life cycle stages are shown below in Fig. AI1.

## Appendix I1: Life cycle stages overview

Fig. AI1 Detailed content and connection between all life cycle stages

## Appnedix I2: Generic activities

- Electricity [kWh] (G3)

- District heating/cooling [kWh] (G4)

- Transports [Chargeable weight to include both volume and weight aspects] (G1)

- Travel [personal km, pkm] (G1)

- Water treatment [m3] (G6)

- Waste treatment [mass, volume] (G6)

Sub–Parts are important but cannot easily be summarized as the number of Sub–Parts and related processes (e.g., surface treatment) could be made very long.

## Appendix I3: Examples of Equipment and black box modules

This list summarizes entities frequently used in LCAs of ICT. The list shall not be seen as a complete set of ICT Equipment but is rather an example to indicate the broad range of applicable ICT Equipment and Support Equipment to be considered.

Each Equipment type may be further divided into more specific Equipment types.

End–user Equipment or User Equipment (UE)

- Mobile phone or standard mobile phone

- Smartphone

- Tablet device (phone/e-reader/PC)

- Net book PC

- Laptop PC

- Desktop PC

- TV

- Any device that can connect to home or Networks

Home (Network) ICT Equipment or customer premises equipment (CPE)

- Fixed wireless terminal (FWT, typically 1 3G+ in and 4 LAN + WLAN out)

- Modem

- Router (Typically 1 WAN in and 4 LAN + WLAN out)

- IPTV box and/or STB for IPTV

- Combo products (e.g. a 3-play or home gateway box: modem/router/IPTV)

- Fiber access (ONU) and combo products including fiber access

Network site Equipment (from base station sites to data centers)

- Base station equipment

- Transmission equipment (e.g. STM-1, radio link)

- RAN control and core equipment (BSC, RNC, SGSN, GGSN, HLR, MSC etc.)

- Fixed access node (FAN) equipment (POTS, xDSL, FTTx/OLT)

- Telecom switches and telecom servers (services)

- Edge/Metro routers/switches

- Core routers/switches

- Optical high capacity transport equipment (WDM)

- Servers

- Data switches

- Storage array network equipment

Examples of ICT specific black box modules:

- Cable set/module (cable + connectors)

- Memory module

- Camera module

- Display module

- Charger device

- AC/DC adapter incl. power cord

- Fan unit

- Hard Disc Drive

- Optical disc player

- Transceiver module

- Power Amplifier module

- Power Supply Unit

Site support Equipment

- Antenna towers

- Antennas

- Feeders

- Lighting guides

- Buildings, shelters and other mechanical structures

- Diesel tanks

- Diesel generators

- AC units

- HEX units

- Fan units

- Battery units

- UPSs

## Appendix I4 Examples of Network Equipment

This appendix exemplifies Network Equipment.

Table A.I1 Examples of Network Equipment

|  | Access nodes | Infrastructure | Control & core nodes |
| --- | --- | --- | --- |
| POTS network | RSS, remote subscriber switch Subscriber part of local exchanges | “Copper” cable network Telestation building/container | Local and higher order Exchanges Telephony and VoIP C&C nodes |
| Fixed broadband network | DSLAM equipment installed in POTS RSS/Telestations | Reuse of POTS infrastructure | n.a. |
| 2G mobile network | 2G base stations | Antenna towers Site building Container | BSC MSC, HLR, SGSN, GGSN, MGW |
| 3G mobile network | 3G base stations | Same as 2G above | RNC MSC, HLR, SGSN, GGSN, MGW |
| 4G mobile network | 4G base stations | Same as 2G above |  |
| Fiber / City LAN network |  | Fiber network |  |
| CATV broadband network |  | Coax cable network | Fiber nodes |

Services

Services are important but cannot easily be summarized as the number of Services (e.g., e–health, applications for smart phones etc.) could be made very long.

## Appendix I5:	Examples of Allocation Procedures

### AI5.1 Examples for Allocation Procedures for Recycling of Materials

The examples use the same values, here expressed as Environmental Load Units (ELU) representing environmental impact: Raw material acquisition: 80 ELU / kg, Raw Material recycling: 10 ELU / kg
It is also assumed that the total Raw Material input need is1 kg for producing the Equipment and Production waste is not shown separately.

The ratio between Raw Material Recycling ELU and primary Raw Material Acquisition ELU is 1:8 which means that Raw Material Recycling is very favourable.

### AI5.1.1 Example of the 100/0 and 0/100 methods

An example of Raw Material Acquisition with the 100/0 method and Raw Material Recycling with the 0/100 method is given below in Fig. AI2. The methods shall not be used at the same time as it overestimates or overallocates emissions to the studied product/material life cycle, i.e., causes double accounting.

Fig. AI2. The 100/0 method and the 0/100 method, both methods cannot be used at the same time.

### AI5.1.2 Example of the 50/50 method

An example of Raw Material Acquisition and Raw Material Recycling with the 50/50 method is given below in Fig. AI3. Fig. AI4 shows a combination of the 100/0 and 50/50 method, of which the latter is believed to give the best total information. Table AI3 gives an example of the 50/50 method with USGS average numbers applied to recycled contents and 90% recycling of the Raw Material in the studied product.

Table AI3 Example of ELU related to Raw Material Acquisition and Raw Material Recycling of 1 kg material.

|  | Raw Material Acquisition [ELU] | Raw Material Recycling (Reprocessing / Remelting) [ELU] | USGS average recycling [%] | Results with 90% recycling at EoLT and 50/50 method [ELU] |
| --- | --- | --- | --- | --- |
| Steel | 2.5 | 0.5 – 1 | 50 | 1.1 – 1.45 |
| Copper | 7 | 1.5 – 2 | 30 | 3.7 – 4 |
| Aluminium | 12 | 1.5 | 35 | 5.44 – 5.75 |

Fig. AI3. The 50/50 method used strictly with the given input parameters give this result.

Recycling of input material (50/50 split with a prior life cycle) is a small part of Raw Material Acquisition in Fig. AI3. However, Raw Material Acquisition (from primary ore, 50/50 split with a future life cycle) is a substantial part of Raw Material Recycling in Fig. AI3. It is also possible to present results with these two parts changed so that Raw Material Acquisition and Raw Material Recycling are presented together.

The Raw Material Recycling can also be described as a scenario for use of recycled Raw Material as input Raw Material and recycling of the Raw Materials in the product.

In Fig. AI4 below, by subtracting the Raw Material Recycling from the Raw Material Acquisition, the same results as in Fig. AI3 are obtained. However, the importance of Raw Material Recycling is more visible in Fig. AI4 than in Fig. AI3, even with the added Raw Material Recycling figures.

Fig. AI4. A combination of the 100/0 and 50/50 method is believed to give the best total information.

## Appendix I6: Examples of application of the standard

### AI6.1 ICT Equipment

This example shows the implications from the standard on an LCA of Equipment performed by an ICT vendor.

The example is not aiming to outline all details of an LCA but to support the reader in interpretation of some important areas of this standard. For compliance with this standard all body text and annexes needs to be considered when performing an LCA, i.e. the example is not sure as being fully compliant with the standard. More details need to be shown for compliance.

Goal definition:

The goal of the study is to estimate the carbon footprint of a Radio Base Station (RBS)

The purpose of the study was to find a base for internal prioritization.

Scope definition:

The study object is a RBS which consists of building blocks:

pieces

Cabinets 		1

Panels, various 		12

Transceiver Units 	6

Dual Duplexer Units	3

PCBAs, various  		5

Cable Sets		19

Grounding Unit		2

Dust Covers		2

Install kit		2

Combiner Divider	1

Installation Guides and Labels are also part of the study object.

The operating lifetime is estimated to be 5 years based on assumptions of a rather short–lived RBS.

The applicable functional unit is one year of use, where the use is further defined as average and not according to traffic models.

System boundaries

The unit processes considered are according to Table 1 for ICT Equipment:

A1

A2

B1.1

B1.2

B1.3

C1

C3

D2.2

The optional activities B2.1 and B.2.2 were left out as Support Equipment were not part of the study scope.

D1 and D3 are not applicable to the study scope.

Support activities except B1.3 are not considered for any unit processes because of lack of data and models.

C3 activities such as installation and maintenance are included in a simplified way. No operator business travel and officer activities are included.

Processes below marked with yellow were included in the study scope.

Processes below in italic style were not included as they are not part of the study scope.

System boundary

A: EQUIPMENT RAW MATERIAL ACQUISITION

A1. Raw material extraction

A2. Raw material processing

G1. TRANSPORTS&TRAVEL

B: PRODUCTION

B1. ICT Equipment production

B1.1 Parts production

B1.2 Assembly

B1.3 ICT vendor activities

B2. Support Equipment production

B2.1 Support Equipment manufacturing

B2.2 Site construction

C: USE

C1. ICT Equipment use

C2. Support Equipment use

C3. Operator activities

C4. Service provider activities

D: EQUIPMENT END-OF-LIFE TREATMENT

D1. Re–use of ICT Equipment

D2. ICT specific EoLT

D2.1 Disassembly/Dismantling/Shredding

D2.2 Recycling

D3. Other EoLT

G2. ELECTRICITY SUPPLY

G3. FUEL SUPPLY

G4. OTHER ENERGY SUPPLY

G5. RAW MATERIAL AQUISTION

G5.1. Raw material extraction

G5.2. Raw material processing

G6. END-OF-LIFE TREATMENT

G6.1 EHW treatment

G6.2 Other waste treatment

G7. RAW MATERIAL RECYCLING

Data collection

Equipment Raw material acquisition

For raw material extraction and raw material processing databases contained within the LCA software program SimaPro were used.

Production

For unit processes within B1.1 data were mainly collected from literature but also samples from own suppliers of ICs and PCBs.

For B1.2 data were collected as annual figures from the RBS assembly plant.

For data collection from suppliers 5.2.2.1.1 was considered as well as the annexes.

For manufacturing process share of the impact of applicable Parts B1.1.1–10 data were reused from a previous study.

For material content share of the impact, primary data were collected. E.g. amount of steel for a certain part.

Use

Use phase energy consumption (electricity) was estimated to be 12.9 MWh per year based on operators measurement on annual average.

Global average energy mixes (electricity) were applied as the product was operating on a Global market and the purpose of the study was to find design improvements.

Equipment End of Life Treatment

For end of life data were based on assumptions and literature.

Generic processes

For generic processes the databases which come with the LCA software program SimaPro were used.

Other information

For raw material acquisition no transparent data on transports were available, thus impacts from these transports are embedded and could not be reported separately.

Transport of RBS to Use stage was modeled based on assumptions of share of Rail, Air, Truck and Ship Transports.

Data calculation

B1.3

Solder paste used per day divided by an assembly line producing PCBAs  Solder used per PCBA

Power measurements for a number of Assembly steps, hours of usage per day, number of boards per day  electricity per PCBA

G1. for Transports

A = % of logistics from Assembly to World by Air

B = Average air distance from Assembly to World Airport [km]

C= Average emission factor for air freight [kg CO2e/tonkm]

D= Average Distance Assembly to Local Airport [km]

E= Average emission factor for lorry in Assembly Nation [kg CO2e/tonkm]

F = Average Distance Local Airport to warehouse [km]

G = Average emission factor for lorry in World [kg CO2e/tonkm]

H = Average Distance World warehouse to World site [km]

I = Average emission factor for van in World [kg CO2e/tonkm]

Allocation of data

Equipment raw material acquisition

The allocations performed for the database data used were not transparently reported by the database.

Production

For Part Production Facility data were allocated by the practitioner based on the relative proportion of relevant physical characteristics (such as weight and layer×area) compared to totally produced amount as preferred by the standard.

B1.1.1 Batteries was not applicable to the BTS LCA study scope.

For B1.1.2 Cables database process data were combined with material content declaration per piece

For B1.1.3 Electro-mechanics mass based modules were designed and applied to the mass of each piece

For B1.1.4 ICs.

The package type for each IC, transistor and diode inside the RBS was identified.

The mass of each package type was estimated. One package type based LCI module (e.g. BGA packages) was designed and applied to the mass of each IC, transistor and diode .

The good die area inside each IC, transistor and diode was identified. One Si area based LCI module was applied to good die area for each IC.

For B1.1.5 Mechanics. Database modules were used and allocated per mass/area/volume of each piece.

B1.1.6 Displays was not applicable to the BTS LCA study scope

For B1.1.7 PCBs. The [area×layer] value for each PCB piece was identified. An LCI module expressed per area (produced multilayer PCBs) was used for each PCB. The factory for the PCB had allocated the environmental loadings per area multilayer PCB output.

For B1.1.8 Other PCBA Comps. The process environmental data per mass was estimated from databases. Mass based LCI modules were created for a set of Parts in this group. The mass and material content of each Part was identified. The mass based LCI modules were applied to appropriate Parts.

B1.1.9 Packaging. was included in B1.2 and B1.1.5

B1.1.10 Black box modules were modeled per piece based on estimation of mass/pieces of sub–Parts inside the black boxes.

Use

No allocation was done. The operator reported annual average electricity consumption for the BTS. This simplification was judged to be enough for the internal purpose.

Equipment End of Life Treatment

100/0 method was applied for Raw Material Production and Raw Material Recycling applied to Iron/Steel alloys, Aluminum alloys, Copper alloys Silver, Gold.

Generic processes

The transports were allocated based on mass

Support activities:

Only included partly for B1.3. For own operation {project accounting information was used to estimate the number of persons involved in the vendor activities needed for R&D}. These resources were then associated with a proportional amount of the organizational resource need.

E.g. G1. for Travelling by ICT Vendor

Travelling pattern for one person (engineer) per month

Person months per number of RBS developed per year

Travelling impact per RBS per year

….

B1.3 for Office Activity

- Number of personnel (engineers) in office area

- Consumption of water, paper, computer usage, other electricity per month for the office area

Person months per number of RBS developed per year

Office Activity for Engineers per RBS per year

Cut-off

During the data collection it was hard to get hold of data for the specific EoLT transport distances so they were cut-off.

De-installation was also cut-off as being a part of optional C3. Operator’s activities.

G6 and D3 EoLT processes are mandatory and were applicable but no appropriate data could be collected.

A cut off 5% had been defined the excluded. From a CO2 point of view the excluded processes were not associated with any specific concerns, thus cut-off was made.

Common wisdom is that the excluded processes have low CO2 intensities.

LCIA

The following impact categories were included: CC

The impact assessment methods applied were GWP100.

Life cycle interpretation

Conclusions

The assessment showed that the most significant activity w r t GHG emissons is the Use Stage driven by electricity. As far as transports are concerned the impact is limited from a life cycle perspective. The assessment also indicates that Mechanics/Materials dominate both Raw Material Acquisition and Production stages. Support activities were not important.

Uncertainty estimation

The uncertainty associated with each data element (LCI flow) was not changed if the uncertainty numbers were provided by original databases. A Pedigree Uncertainty Calculation as provided by SimaPro was applied to the ten most contributing and driving processes. Uncertainty of LCIA characterization factors was not possible to estimate. The following steps were taken to reduce the uncertainties: verification of the reasonableness of the electricity usage of the RBS.

Sensitivity analyses

The models applied and the data used were assessed with respect to accuracy and a list of candidates bound for sensitivity analysis were identified, e.g.:

- operating life time

- transport distances between final assembly and customer

- production region

- traffic growth assumptions

- 50/50 allocation instead of 100/0 + recycling

- Change to Global electricity mix for Raw Material Acquisition instead of original database electricity mix.

The sensitivity analyses revealed that the conclusions were stable.

Reporting

All omissions and deviations towards the standard were transparently reported and the reporting formats of the standards were followed in detail. Furthermore the reporting formats of the standard were used in the final report.

Fig. 8 GWP100 result diagram example for RBS (diagram for Global Warming Potential (GWP100) (CO2e).

Fig. 9 GWP100 result: distribution between sub–unit processes within each life cycle stage

{To be further discussed how detailed these sections should be in the examples. Probably the best way would be to fill in the tables and diagrams from chapter 6 and he annexes and then remove certain parts if the examples gets to long}

## Annex N1 (Normative) Table reporting formats

Table AN1 Reporting format for included life cycle stages, activities and generic processes

| Tag | Life cycle stage | Unit process | Included (Yes/No) | Electricity mix (specific/country/world average | Support activities included (Yes/No) | Transport activities included (Yes/No) | Other generic activities included (Yes/No) | Motivation/Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Equipment Raw Material Acquisition |  |  |  |  |  |  |  |
| A1 | Raw material extraction |  | Yes | Specific | No | Yes | No | Specific electricity mix might deviate from world average |
| A2 | Raw material processing |  | Yes | Specific | No | Yes | No | Specific electricity mix might deviate from world average |
| B | Production |  |  |  |  |  |  |  |
| B1 | ICT equipment production |  |  |  |  |  |  |  |
| B1.1 |  | Parts production | Yes | World | No | Yes | No |  |
| B1.2 |  | Assembly | Yes | World | No | Yes | No |  |
| B1.3 |  | ICT vendor activities | Yes | World | Yes |  |  | Not all ICT vendor activities included |
| B2 | Support equipment production |  |  |  |  |  |  |  |
| B2.1 |  | Support Equipment manufacturing* | No |  |  |  |  |  |
| B2.2 |  | Site construction | No |  |  |  |  |  |
| C | Use |  |  |  |  |  |  |  |
| C1 | ICT equipment use |  | Yes | World |  |  |  |  |
| C2 | Support equipment use |  | No |  |  |  |  |  |
| C3 | Operator activities |  | Yes |  |  |  |  | Only a few like installation and some maintenance |
| C4 | Service provider activities |  | No |  |  |  |  |  |
| D | Equipment End of Life Treatment |  |  |  |  |  |  |  |
| D1 | Re–use of ICT Equipment |  | No |  |  |  |  |  |
| D2 | ICT specific EoLT |  |  |  |  |  |  |  |
| D2.1 |  | Storage/Disassembly/Dismantling/ Shredding | No |  |  |  |  |  |
| D2.2 |  | Recycling | No |  |  |  |  | G7. Raw Material Recycling included |
| D3 | Other EoLT |  | No |  |  |  |  |  |

We need also a table for generic processes!

Table ANX Reporting format for Generic processes per functional unit

| Generic process | Generic process categories included | Unit processes included (for each process category) | Product flow unit | Important issues |
| --- | --- | --- | --- | --- |
| G1. Transports&Travel | Road Air Ship Train | Direct (during transport) emissions Fuel supply chain Vehicle production** Infrastructure production** | ton×km |  |
| G2. Electricity | World average electricity mixes | Fuel supply chain, Direct emissions (during electricity production), power plant construction**, dams, the grid itself**, nuclear waste treatment** | kWh |  |
| G3. Fuels | Oil Diesel Petrol Jet—fuel Coal Gas | Fuel supply chain: Extraction and Production Distribution (transports) and emissions related to the incineration of the fuel is connected to a unit process or "site LCI model". | mass, volume, energy content |  |
| G4. Other energy | District heating (hot water) | Fuel supply chain, Direct emissions during energy/electricity production | kWh | Electricity is also a energy source or district heating/cooling production |
| G5. Raw material acquisition |  | Extraction Processing | mass, volume |  |
| G6. End–of–life treatment | See Annex M3 |  |  |  |
| G7. Raw material recycling | Amount avoided primary production of Steel, gold, silver, copper, aluminum |  |  |  |

Table AN2 Reporting format for transports/travel per functional unit

| Mode | CO2 emission factor (fuel consumption only/ /including fuel supply) | Raw material acquisition transports |  | Production stage transports excluding final transport |  | Final transport (Production to use stage) |  | Use stage transports |  | EoLT transports |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Transport work {ton×km} | GWP100 {kg CO2e} | Transport work {ton×km} | GWP100 {kg CO2e} | Transport work {ton×km} | GWP100 {kg CO2e} | Transport work {ton*km} | GWP 100 {kg CO2e} | Transport work {ton×km} | GWP 100 {kg CO2e} |
| Air | 1.1 kg CO2e/[ton×km] |  |  |  |  | 120 | 130 |  |  |  |  |
| Other | Road: Lorry 0.25 kg CO2e/[ton×km], Van 1.5 kg CO2e/[ton×km], Rail 0.01 kg CO2e//[ton×km], Ship 0.009 kg CO2e//[ton×km] |  |  |  |  | Road: Lorry 80, Van 10 Rail: 70 Ship: 260 | Road:40 Rail: 1, Ship: 2 |  |  |  |  |
| Mode | CO2 emission factor (fuel consumption only/ /including fuel supply) | Total travels |  |
|  |  | Transport distance {km} | GWP100 {kg CO2e} |
| Air | 0.15 kgCO2/[person×km] | 7 | 1 |
| Other | Car 0.2 kg CO2e/[person×km] | 0.4 | 0.1 |

Table AN3 Reporting format for raw materials per functional unit

|  | Total input (g,kg, ton) | Recycled input (%) | Content in product (%) | Recycling of total input* (%) | Reference |
| --- | --- | --- | --- | --- | --- |
| Iron/Steel alloys | 250 kg | 0 | 0 | 30 | Assumption for recycling of total |
| Aluminum alloys | 80 kg | 0 | 0 | 30 | Assumption |
| Copper alloys | 30 kg | 0 | 0 | 40 | Assumption |
| Silver | 0.2 kg | 0 | 0 | 40 | Assumption |
| Gold | 0.015 kg | 0 | 0 | 35 | Assumption |

*) Total recycling of all input materials, i.e., recycling of manufacturing waste and recycling of total content in final product.

Table AN4 Reporting format for parts production

|  | Part categories included | Part Unit processes included | Handling of special issues |
| --- | --- | --- | --- |
| B1.1.1 Batteries | N.A. | N.A. | N.A. |
| B1.1.2 Cables | Coaxial cables Fiber cables Power cables Network/signal cables | Raw Material Acquisition, Cable final assembly |  |
| B1.1.3 Electro–mechanics | Connectors | Raw Material Acquisition, Part final assembly |  |
| B1.1.4 Integrated circuits (ICs) | Processors, DSPs ASICs Memories Microprocessors Transistors and diodes | Front–end: Special IC materials production, Wafer production, Chip production (“the wafer fab”) Back–end: IC encapsulation | Package type for each Part and Good die area for each Part Load Per B1.1.4 |
| B1.1.5 Mechanics / materials | Fronts Frames Racks Cabinets Solder Nuts, bolts | Raw Material Acquisition, Part final assembly |  |
| B1.1.6 Displays | N.A | N.A. | N.A. |
| B1.1.7 Printed circuit boards (PCBs) | Plastic | Raw materials Acquisition for special PCB materials, Raw materials Acquisition for PCB semi–produced composite materials, PCB final assembly |  |
| B1.1.8 Other PBA components | Resistors Capacitors Inductors Relays LEDs Potentiometers Quartz crystal oscillator | Raw Material Acquisition, Part final assembly |  |
| B1.1.9 Packaging materials | Paper Cardboard Plastics Wood Steel |  |  |
| B1.1.10 Black box modules |  | “Cradle–to–gate” LCA from supplier |  |

Table AN5 Reporting format for use stage energy consumption

|  | Energy consumption {kWh/year} | Source {long term average/standardized measurement/ modelled | Motivation/ comment |
| --- | --- | --- | --- |
| ICT equipment | 12,900 | Modelled | Simplified purpose of LCA study |
| Support equipment | Not included |  |  |

Table AN6 Reporting format for EoLT

|  | Process categories included | Process Unit processes included | Handling of special issues |
| --- | --- | --- | --- |
| EHW treatment | None | None | None |
| Other Waste treatment | None | None | None |
| EoLT | None | None | None |

Table AN7 Reporting format for LCI results per functional unit

|  | TOTAL | Raw materials acquisition and EoLT | Production | Use |
| --- | --- | --- | --- | --- |
| Primary energy use | 130,000 MJ | 4,200 | 5,000 MJ | 120,000 MJ |
| Total electricity use | 13,700 kWh | 380 kWh | 450 kWh | 12,900 kWh |
| Land use* | 340 m2 | 30 m2 | 12 m2 | 300 m2 |
| Fresh water use** | 36 m3 | 4 m3 | 2 m3 | 30 m3 |

*According to ReCiPe MidPoint (H) Land occupation and transformation impact categories

**According to ReCiPe MidPoint (H) Water Depletion impact category

Table AN8 Impact category indicators per functional unit

| Mid–point Impact Assessment Categories included | Impact category indicator value | LCIA methodology reference |
| --- | --- | --- |
| Climate change(CC) | 8,800 kg CO2e | Mid–point Category indicator: Infrared forcing as GWP100, IPCC via SimaPro |

### AI6.2 Networks

3G

### AI6.3 Services

Video conference

Moreover, for the Service (example video conference) the following parameters(if applicable) the assumptions need to be given:

- Duration of typical service (e.g., 2 hours meeting)

- Life span of facility in which Service occurs (e.g. years)

- Use rate (e.g., number of meetings per week)

- Data transfer speed (e.g. Mbit/s)

- Electricity mix used

- Power usage for active and idle mode

- ICT Equipments manufacturing location

-Installation of the facility in which the Service occurs

-Maintenance of the facility in which the Service occurs

-Servers per facility (e.g., dedicated servers per teleconference facility)

# Appendix X Uncertainties of Life Cycle Assessments for ICT Equipment, networks and services

The uncertainty of LCA can be divided into three categories:

- parameter uncertainty

- scenario uncertainty

- model uncertainty

Parameter uncertainty is related to uncertainties in input-data and provides a measure of how close the data and calculated emissions are to the real data and emissions. This includes uncertainties in the inventory analysis and uncertainties when translating inventory flows into environmental impact potentials. The influence of parameter uncertainty on the final result can be assessed analytically or by simulations. One example of parameter uncertainty is the uncertainty associated with the conversion from the emissions of carbon dioxide (CO2) and other Green House Gases (GHGs) into carbon dioxide equivalents (CO2e).

Scenario uncertainty represents variation of results depending on methodological choices, e.g. LCI modeling principles, allocation procedures and cut-off decisions. The scenario uncertainty can be quantified through sensitivity analysis. Sources of scenario uncertainties include e.g. the allocation method for data for production facilities, overhead activities and vehicle use to the product system studied and also use of old data to represent current activities.

Model uncertainty arises from insufficient knowledge of the studied system, leading to omission of data or incorrect assumptions. Model uncertainties are difficult to quantify. Aviation emissions such as NOX and soot and effects such as land use are example of emissions/effects usually left out because of lack of knowledge. One source of model uncertainty much discussed is the possible inclusion of emissions from infrastructure and supply chain for travel and transportation activities.

### Some important uncertainty sources for different life cycle phases

The below table summarizes some important uncertainty sources associated with different life cycle phases.

TABLE 1: Important uncertainty sources of the different life cycle phases

| Life cycle phase | Activities included | Important uncertainty sources |
| --- | --- | --- |
| Raw material acquisition | Raw material extraction Raw material processing | Long supply chain without direct commercial relationship to ICT industry. Variations in geographical location. World market variations beyond control of ICT. |
| Production | ICT Equipment production Support Equipment production | Large supplier base which changes continuously over product system lifetime based on price, availability etc. Allocation of facility data betweens product systems and processes. |
| Use | ICT Equipment use Support Equipment use Support activities | Life time, geographical location, traffic scenario model. Large variations between operators regarding site and network design and energy consumption. Electricity production model and power supply variations. |
| End of Life Treatment | ICT specific EoLT Other EoLT | Future processes principally unknown. Significant variations between suppliers and regions. Allocation of facility data between product systems and processes. |

# Appendix Y : Opportunities and limitations in the use of LCA for ICT Equipment, networks and services

Life Cycle Assessment (LCA) is a powerful, systematic methodology which gives an understanding of the relative importance of the different life cycle stages/activities. LCA assist companies in determining where to put their efforts to improve lifecycle environmental performance and also to monitor how this performance changes over time However it is important to keep in mind that the results of an LCA are always model-based representations of the real environmental impact, and that absolute impact of a certain product, network, service or organization is beyond reach. This is true for all kinds of product systems, but especially so for the complex product systems of the ICT sector.

“ LCA addresses potential environmental impact; LCA does not predict absolute or precise environmental impacts due to the relative expression of potential impacts to a reference unit, the integration of environmental data over space and time, the inherent uncertainty in modeling environmental impacts, and the fact that some possible environmental impacts are clearly future impacts” (Source: ISO 14040:2006 chapter 4.3)

Consequently, any LCA result is only valid under the assumptions of the study and is still associated with uncertainty, which needs to be considered so the outcome of the assessment is interpreted in a correct way. Both uncertainty analysis and sensitivity analysis are important tools to understand the results of a study and what conclusions can be made.

Appropriate use of LCA

LCA should primarily be used for the following purposes:

- Identification of opportunities to improve environmental performance of Equipment, networks, services and organizations

- Information to decisions-makers in industry, government or non-government organizations about typical environmental performance of a product system/organization to assist their policy choices

- Selection of relevant indicators of environmental performance for monitoring

- Understanding of the potential impact of new services and solutions

- Understanding of improvements between generations

On the contrary LCA is less suitable for

- Quantitative benchmarking between different LCAs

- Aggregation of results between different LCAs

- Product system performance legislation (directly measurable parameters more appropriate)

- Labeling of ICT Equipment, networks and services

Appendix Z Example of data quality indicators

Table x below shows an example of data quality indicators.

Table x Matrix for data quality assessment

| Data Quality Indicator | Section in this standard which handles the indicator |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Methodological appropriateness and consistency | Entire standard |  |  |  |  |
| Completeness (total LCA level) | 5.2.3 Cut–off |  |  |  |  |
| Uncertainty (total LCA level) | 5.5 |  |  |  |  |
| Acquisition method | 5.2.4 |  |  |  |  |
| Supplier independence | 5.2.4 |  |  |  |  |
| Data representativeness | 5.2.4 |  |  |  |  |
| Data age | <3 years | <6 years | <10 years | <15 years | Age unknown |
| Geographical correlation | Data from the exact area | Average data from a larger area | Data from an area with similar production conditions | Data from an area with slightly similar production conditions | Unknown area |
| Technological correlation | Data from process studied of the exact company | Data from process studied of company with similar technology | Data from process studied of company with different technology | Data from process related to company with similar technology | Data from process related to company with different technology |
| Rule of inclusion/exclusion (Elements/Flows/Unit process) | 5.2.3 Cut-off |  |  |  |  |

# History

| Document history |  |  |
| --- | --- | --- |
| 0.0.1 | March 2, 2009 | First Draft |
| 0.0.2 | April 29, 2009 | Second Draft after comments from stakeholders of ETSI |
| 0.0.3 | June 18, 2009 | Third draft after Belgrade EE#34 meeting May 7, 2009 |
| 0.0.4 | July 17, 2009 | Fourth draft after Kista workshop meeting June 25, 2009 |
| 0.0.5 | September 15, 2009 | Fifth draft after Kista workshop meeting August 25, 2009 |
| 0.0.6 | December 4 2009 | Sixth draft after Ovada EE#36 meeting October 14, 2009 |
| 0.0.7 | April 6, 2010 | 7th draft after Paris meeting February 10, 2010 |
| 0.0.8 | June 11, 2010 | 8th draft after Lannion meeting (ad hoc meeting at EE#36, April 2010) Main changes: Restructuring of document to better reflect ISO and the three product system levels (equipment, network and services Functional unit updated acc to decision in Lannion Terminology updated based on agreement in Lannion and comments on chapter 3 |
| 0.0.9 | June 17, 2010 | 9th draft after Rapporteaur check of 8th draft |
| 0.0.9.1 | June 29, 2010 | 9th draft with comments after Kista workshop, June 23, 2010 |
| 0.10 | Sept. 17, 2010 | 10th draft with proposal for Section 5.2.2.2 |
| 0.11 | Sept. 22, 2010 | 11th draft with agreement on compulsory and optional life cycle stages 5.2.2.2 |
| 1.1.4 | Nov. 18, 2010 | 12th draft with new figures and text, especially for 5.2.2.1-4. This draft to be used in Paris workshop Dec. 6-7, 2010 |
| 1.1.5 | Dec. 30, 2010 | 13th draft after Paris workshop Dec. 6-7, 2010 |
| 1.1.6 | Feb.. 16, 2011 | 14th draft after Helsinki workshop Feb. 8-9, 2011 |
| 1.1.7 | May, 5, 2011 | 15th draft after revisions in March-April, 2011 |
| 1.1.8 | May, 18, 2011 | 16th draft after revisions in May, 2011 |

See Section 1. Scope .

Special requirements apply for comparative assertions between ICT and business as usual systems. Comparisons are generally not supported for LCAs performed and presented by two different individuals/organisations.

By results is meant absolute and relative values.

LCA can be performed and presented by different individuals/organization or by the same individual/organization. However, comparisons of LCA results obtained by the same individual/organization who uses i) this standard, ii) the same software program, and iii) the same LCI databases for all comparables are supported by this standard. A third–party review is also needed if the comparison result is to be externally communicated.

Legal lifetime

The assessment of raw materials acquisition is generally based on generic databases. At this stage such databases often do not report transports transparently.

If the LCA is used to estimate historic environmental impact, actual use time may be available and can then be used. In most cases actual operating lifetime is not available and estimates are needed.

These building blocks are preferably identified from block diagrams provided, e.g., by system engineers/architects.

The explanation of the common basis could be challenging for comparisons of ICT Services and BAU systems

Generic data such as EIO-LCA

Detailed flow chart figure s of, e.g., transports are decided/provided in each unique LCA project.

However, for LCAs referring to specific conditions it is encouraged to include also B2 within the system boundary as Support equipment can have a significant impact on the use stage for an ICT solution.

The first destination for the de-installed ICT Equipment depends on the goal&scope of the specific LCA study.

Including transmission between the access nodes, which is allocated to the access networks (e.g.  PDH, SDH, WDM and network link   elements like STM/MUX, radio links and WDM elements and repeaters)

The mobile control nodes which are in reality part of the access networks are in this physical view structured together with cthe core nodes as they often share sites.

Data transport is a collective term used for all transmission and IP network equipment that are used

The term data center(s) can include all sizes of server networks, from enterprise data centers down to “a server in a closet”.

The electricity mix shall as closely as possible reflect the region in which the Equipment is used

If possible to derive from the databases, e.g., for Raw Materials

Other relevant physical data is indicated in Annex N2

US geological survey (http://www.usgs.gov/)

Preferably usage studies can be used as a source but if such studies are not available estimates need to be done

Primary energy usage = Cumulative Energy Demand (CED) and Fresh water usage are to be reported as LCI results appropriately according to Table AN7.

See table AM5

The significance is determined for instance by magnitude in difference, modeling assumptions, and LCA software program calculation algorithm.

The Cumulative Energy Demand method is appropriate to express Primary Energy Usage

Relates to transport

The final transport of ICT Equipment from assembly to operator, including pre– and post transports connected to the main transport

average in terms of distance, transport mode, load factor, chargeable weight etc

average in terms of distance, transport mode, load factor, chargeable weight etc

Primary energy usage is appropriate to express as Cumulative Energy Demand

Some of these units are ICT specific, others have a more generic application.

The applicability of USGS average numbers varies case by case

The final transport of ICT Equipment from assembly to operator, including pre– and post transports connected to the main transport

average in terms of distance, transport mode, load factor, chargeable weight etc

average in terms of distance, transport mode, load factor, chargeable weight etc

Primary energy usage is appropriate to express as Cumulative Energy Demand

Often based on economical data

Emission data for a site is typically measured at the site level and not for individual processes and products

Decisions regarding which activities to include in the life cycle is part of the system boundary setting of a study