RECOMMENDATION  ITU-R  BR.1515

International exchange of digital electronic
news gathering recordings

(Questions ITU-R 108/11 and ITU-R 239/11)

(2001)

The ITU Radiocommunication Assembly,

considering

a)	that digital electronic news gathering (ENG) is widely used in broadcasting in several parts of the world;

b)	that videotape recording is a fundamental element in ENG operation;

c)	that many countries retain large analogue archives of valuable, irreplaceable programmes, based on the signal formats in Recommendation ITU-R BR.715;

d)	that there are several types of equipment available for digital ENG recording today, that differ in the recording medium, in the recording format, or in the compression schemes used for signal processing;

e)	that it would be beneficial to broadcasters in their digital ENG recording operation if analogue ENG recordings and digital ENG recordings coexist in an edit environment and possibly if the compatibility of equipment in edit operations is provided;

f)	that the digital broadcasting service for high-definition television (HDTV) programmes is gaining its momentum. There are several services in operation;

g)	that digital ENG recording in HDTV quality is expected to be widely used in the coming HDTV broadcasting age;

h)	that bit-rate compression techniques based on discrete cosinus transform (DCT) and variable-length code (VLC) coding can provide highly efficient methods of recording HDTV programmes, whose source data rate exceeds 1 Gbit/s;

j)	that the operational and handling characteristics of digital HDTV video tape recorders (VTRs) have improved to the point that VTRs can be used not only in studios but also in the field, with a level of convenience similar to that of conventional VTRs;

k)	that HDTV cameras using two-million pixel charge coupled device (CCD) imaging devices and digital signal processing with advanced very large scale integration (VLSI) chip(s) can provide high quality HDTV pictures while offering the operating features and small size of conventional standard definition TV (SDTV) cameras,

recommends

1	that since digital ENG recordings are based on either SDTV or HDTV, an aggregated SDTV and HDTV playback equipment, which can function with both types of recording and/or can facilitate up and/or down converted outputs, should preferably be used. It should also be preferable for the equipment to accept analogue and digital recordings;

2	that, in order to facilitate the use by receiving organizations of programmes originated from digital ENG recordings and relayed over transmission facilities, every effort should be made to minimize the loss of quality in each stage of the broadcast chain;

3	that analogue ENG tape recordings should preferably be transferred to a digital recording format for post-processing and archives;

4	that the 1 080  1 920 common image format specified in Recommendation ITU-R BT.709, Part 2 shall be preferred for image capture in digital HDTV ENG recordings, while the legacy image formats specified in Recommendation ITU-R BT.709, Part 1 will retain their value as an archive format of HDTV material.

NOTE 1 – Analogue ENG tape recordings can be processed to a higher number of generations without significant deterioration in picture quality, if they are first copied on full-broadcast quality digital VTR machines.

NOTE 2 – The Tables in Annex 1 indicate specifications of the available recording devices for digital ENG programmes for SDTV and HDTV scanning systems.

ANNEX  1

(Informative)

Tables 1-6 specify major features and specifications of the commercially available tape based storage devices for SDTV and HDTV acquisition camcorders:

Table 1 – Tape recorder for SDTV 525/59.94 and HDTV 1125/60, (59.94) – Overview

Table 2 – HDTV digital VTR for 1125/60, (59.94) – Details

Table 3 – SDTV digital VTR for 525/59.94 – Details

Table 4 – Digital VTRs for SDTV 625/50 and HDTV 1125/50 – Overview

Table 5 – HDTV digital VTR for 1125/50 – Details

Table 6 – SDTV digital DVR for 625/50 – Details

TABLE  1

Tape recorder for SDTV 525/59.94 and HDTV 1125/60 (59.94) – Overview

| HDTV/SDTV |  |  | HDTV |  |  |  |  |  | SDTV |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Format |  |  | A |  | B |  | C |  | A |  | B |  | C |  | D |  | E |  | F |  | G |
| Commercial name(1) |  |  | HDCAM(2) |  | DVCPRO HD |  | D-9 HD |  | Digital BETACAM |  | DVCPRO |  | DVCAM |  | BETACAM SX |  | DVCPRO 50 |  | Digital-S |  | MPEG IMX |
| Recording/playback time (maximum) |  |  | 40/124 min |  | 46 min |  | 62 min |  | 40/124 min |  | 66/184 min |  | 40/184 min |  | 62/194 min |  | 33/92 min |  | 124 min |  | 60/184 min |
| Video specifi-cation | Luminance (MHz) | 23 |  | 20 |  |  |  | 5.75 |  |  |  |  |  | 4.5 |  | 5.75 |  |  |  | 5.75 |  |
|  | Chrominance (MHz) | 7 |  | 10 |  |  |  | 2.75 |  | 1.5 |  |  |  | 2.0 |  | 2.75 |  |  |  | 2.75 |  |
|  | Number of lines | 1080 (1035)(1) |  | 1080 |  |  |  | 512 |  | 480 (1)(3) |  | 480 |  | 512 |  | 480 (+7.5)(3) |  | 482 |  | 512 |  |
| Audio specifi-cation | Digital or analogue | PCM |  |  |  |  |  | PCM |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Number of channels | 4 |  | 8 |  | 4/8 |  | 4 |  | 2 |  | 2/4 |  | 4 |  |  |  |  |  | 4/8 |  |
|  | Sampling frequency | 48 kHz 20 bits |  | 48 kHz 16 bits |  | 48 kHz 20/16 bits |  | 48 kHz 20 bits |  | 48 kHz 16 bits |  | 48/32 kHz 16/12 bits |  | 48 kHz 16 bits |  |  |  |  |  | 48 kHz 24/16 bits |  |
| Ancillary data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Media | Size | S: 156  96 L: 254  145 |  | L: 125  78 |  | L: 188  104 |  | S: 156  96 L: 254  145 |  | M: 97.5  64.5 L: 125  78 |  | Standard: 125  78 Mini: 66  48 |  | S: 156  96 L: 254  145 |  | M: 97.5  64.5 L: 125  78 |  |  |  | S: 156  96 L: 254  145 |  |
|  | Substance | Metal particle |  |  |  |  |  | Metal particle |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Application example |  |  | Camcorder |  |  |  |  |  | Camcorder |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes |  |  | 1/2 inch Intra field/ frame DCT (1/7) Field editable |  | 6.35 mm Intra frame DCT (1/6.6) |  | 1/2 inch Intra frame DCT (1/10) |  | 1/2 inch Intra field DCT (1/2) |  | 6.35 mm Intra frame DCT (1/5) |  |  |  | 1/2 inch Intra frame DCT (1/10) Frame editable |  | 6.35 mm Intra frame DCT (1/3.3) |  | 12.65 mm Intra frame DCT (1/3.3) |  | 1/2 inch Intra frame DCT (1/3.3) Field editable |
| PCM: pulse code modulation. S: small, M: medium, L: large. (1) Each format as commercially known is listed here. (2) The HDCAM product family provides some product models that can replay all analogue and digital recordings that are made within the Betacam family. (3) Option. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE  2

HDTV digital VTR for 1125/60 (59.94) – Details

| Format |  | A | B | C |
| --- | --- | --- | --- | --- |
| Commercial name(1) |  | HDCAM | DVCPRO HD | D-9 HD |
| Sampling frequency | Video (MHz) | 74.25 |  |  |
|  | Audio (kHz) | 48 |  |  |
| Quantization | Video (bits) | 8 |  |  |
|  | Audio (bits) | 20 | 16 | 20/16 |
| Number of audio channels |  | 4 | 8 | 4 (20 bits) 8 (16 bits) |
| Compression (video) |  | Intra field/ frame DCT 1/7(2) | Intra frame DCT (1/6.6) | Intra frame DCT (1/10) |
| Channel coding |  | S-NRZI | 24-25 I-NRZI |  |
| Total rate (Mbit/s) |  | 185 | 167 | 198 |
| Video rate (Mbit/s) |  | 140 | 100 |  |
| Number of recording RF channels |  | 4 (camcorder)/2 (studio) |  | 4 |
| ECC | Inner | 231, 219 | 85, 77 |  |
|  | Outer | 250, 226 | 149, 138 |  |
| Drum diameter (mm) |  | 81.4 | 21.7 | 62 |
| Drum rotation (rps) |  | 45 (camcorder) /90 (studio) | 149.85 (camcorder) /299.70 (studio) | 75 |
| Number of tracks |  | 6/field | 40/frame | 20/frame |
| Tape speed (mm/s) |  | 96.8 | 135.28 | 115.474 |
| Track pitch (m) |  | 21.7 | 18 | 20 |
| Min. wavelength (m) |  | 0.49 |  | 0.587 |
| Width of tape (mm) |  | 12.65 | 6.35 | 12.65 |
| Media substance |  | Metal particle |  |  |
| Hc (kA/m) |  | 132 | 184 |  |
| Cassette size (mm) |  | S: 156 96 L: 254 145 | L: 125 78 | L: 188 104 |
| Recording time (min) |  | 40/124 | 46 | 62 |
| EEC: Error correction code (1) Each format as commercially known is listed here. (2) 5/8 horizontal prefiltering and 1/4.4 DCT. |  |  |  |  |

TABLE  3

SDTV Digital VTR for 525/59.94 – Details

| Format |  | A |  | B |  | C | D | E | F | G |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Commercial name(1) |  | Digital BETACAM |  | DVCPRO |  | DVCAM | BETACAM SX | DVCPRO 50 | Digital-S | MPEG IMX |
| Sampling frequency | Video (MHz) | 13.5 |  |  |  |  |  |  |  |  |
|  | Audio (kHz) | 48 |  |  | 48/32 |  | 48 |  |  |  |
| Quantization | Video (bits) | 10 | 8 |  |  |  |  |  |  |  |
|  | Audio (bits) | 20 | 16 |  | 16/12 |  | 16 |  |  | 24/16 |
| Number of audio channels |  | 4 | 2 |  | 2/4 |  | 4 |  |  | 4/8 |
| Compression (video) |  | Intra field DCT (1/2) | Intra frame DCT (1/5) |  |  |  | Inter frame DCT (1/10) | Intra frame DCT (1/3.3) | DCT (1/3.3) | Intra frame DCT (1/3.3) |
| Channel coding |  | S-NRZI | 24-25 I-NRZI |  |  |  | S-NRZI | 24-25 I-NRZI |  | S-NRZI |
| Total rate (Mbit/s) |  | 128 | 42 |  |  |  | 44 | 84 | 99 | 97 |
| Video rate (Mbit/s) |  | 90 | 25 |  |  |  | 18 | 50 |  |  |
| Number of recording RF channels |  | 4 (camcorder) /2 (studio) | 1 |  |  |  | 4 (camcorder) /2 (studio) | 2 |  | 8 (camcorder) /2 (studio) |
| ECC | Inner | 178, 164 | 85, 77 |  |  |  | 124, 112 | 85, 77 |  | 162, 150 |
|  | Outer | 106, 96 | 149, 138 |  |  |  | 64, 50 | 149, 138 |  | 60, 46 |
| Drum diameter (mm) |  | 81.8 (camcorder) 81.4 (studio) | 21.7 |  |  |  | 49.6 (camcorder) 81.4 (studio) | 21.7 |  | 54.0 (camcorder) 81.4 (studio) |
| Drum rotation (rps) |  | 44.96 (camcorder) 89.91 (studio) | 149.85 |  |  |  | 37.46 (camcorder) 74.93 (studio) | 149.85 | 75 | 29.97 (camcorder) 59.94 (studio) |
| Number of tracks |  | 6/field | 10/frame |  |  |  | 10/2 frame | 20/frame | 10/frame | 8/frame |
| Tape speed (mm/s) |  | 96.7 | 33.820 |  | 28.193 |  | 59.5 | 67.640 | 57.737 | 64.5 |
| Track pitch (m) |  | 21.7 | 18 |  | 15 |  | 32 | 18 | 20 | 21.7 |
| Min. wavelength (m) |  | 0.69 | 0.49 |  |  |  | 0.74 | 0.49 | 0.587 | 0.56 |
| Width of tape (mm) |  | 12.65 | 6.35 |  |  |  | 12.65 | 6.35 | 12.65 |  |
| Media substance |  | Metal particle |  |  | Metal evaporated |  | Metal particle |  |  |  |
| Hc (kA/m) |  | 125 | 184 |  | 120 |  |  | 184 | 143 | 120 |
| Cassette size (mm) |  | S: 156  96 L: 254  145 | M: 97.5  64.5 L: 125  78 |  | S: 156  96 L: 254  145 |  |  | M: 97.5  64.5 L: 125  78 | L: 188  104 | S: 156  96 L: 254  145 |
| Recording time (min) |  | 40/124 | 66/184 |  | 64/194 |  |  | 33/92 | 124 | 60/184 |
| (1) Each format as commercially known is listed here. |  |  |  |  |  |  |  |  |  |  |

TABLE  4

Digital VTRs for SDTV 625/50 and HDTV 1125/50 – Overview

| HDTV/SDTV |  | HDTV |  | SDTV |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Format |  | A | B | A | B | C | D | E | F | G |
| Commercial name(1) |  | HDCAM(1) | DVCPRO HD | Digital BETACAM | DVCPRO | DVCAM | BETACAM SX | DVCPRO 50 | Digital-S | MPEG IMX |
| Recording/playback time (maximum) |  | 124 min | 46 min | 40/124 min | 66/184 min | 40/184 min | 64/194 min | 33/92 min | 124 min | 72/224 min |
| Video specification | Luminance (MHz) | 23 |  | 5.75 |  |  | 5.5 | 5.75 |  | 5.75 |
|  | Chrominance (MHz) | 7 | 11 | 2.75 | 1.5 | 2 |  | 2.75 |  | 2.75 |
|  | Number of lines | 1080 |  | 608 | 576 (1)(2) | 576 | 608 | 576 (9)(2) | 578 | 608 |
| Audio specification | Digital or analogue | PCM |  | PCM |  |  |  |  |  |  |
|  | Number of channels | 4 | 8 | 4 | 2 | 2/4 | 4 |  |  | 8/4 |
|  | Sampling frequency | 48 kHz 20 bits | 48 kHz 16 bits | 48 kHz 20 bits | 48 kHz 16 bits | 48/32 kHz 16/12 bits | 48 kHz 16 bits |  |  | 48 kHz 16/24 bits |
| Ancillary data |  |  |  |  |  |  |  |  |  |  |
| Media | Size | L: 254  145 | L: 125  78 | S: 156  96 L: 254  145 | M: 97.5  64.5 L: 125  78 | Standard: 125  78 Mini: 66  48 | S: 156  96 L: 254  145 | M: 97.5  64.5 L: 125  78 | L: 188  104 | S: 156  96 L: 254  145 |
|  | Substance | Metal particle |  | Metal particle |  | Metal evaporate | Metal particle |  |  |  |
| Application example |  | Camcorder |  | Camcorder |  |  |  |  |  |  |
| Notes |  | 1/2 inch Intra field/frame Field editable DTC (1/7) | 6.35 mm Intra frame DCT (1/6.6) | 1/2 inch Intra field DCT (1/2) | 6.35 mm Intra frame DCT (1/5) |  | 1/2 inch Inter frame DCT (1/10) Frame editable | 6.35 mm Intra frame DCT (1/3.3) | 12.65 mm Intra frame DCT (1/3.3) | 1/2 inch Intra frame DCT (1/3.3) Field editable |
| (1) Each format as commercially known is listed here. (2) Option. |  |  |  |  |  |  |  |  |  |  |

TABLE  5

HDTV digital VTR for 1125/50 – Details

| Format |  | A | B |
| --- | --- | --- | --- |
| Conventional name(1) |  | HDCAM | DVCPRO HD |
| Sampling frequency | Video (MHz) | 74.25 |  |
|  | Audio (kHz) | 48 |  |
| Quantization | Video (bits) | 8 |  |
|  | Audio (bits) | 20 | 16 |
| Number of audio channels |  | 4 | 8 |
| Compression (video) |  | Intra field/frame DCT 1/7(2) | Intra frame DCT (1/6.6) |
| Channel coding |  | S-NRZI | 24-25 I-NRZI |
| Total rate (Mbit/s) |  | 154 | 167 |
| Video rate (Mbit/s) |  | 117 | 100 |
| Number of recording RF channels |  | 4 (camcorder)/2 (studio) |  |
| ECC | Inner | 231, 219 | 85, 77 |
|  | Outer | 250, 226 | 149, 138 |
| Drum diameter (mm) |  | 81.4 | 21.7 |
| Drum rotation (rps) |  | 37.5/75 | 150 (camcorder) /300 (studio) |
| Number of tracks |  | 6/field | 48/frame |
| Tape speed (mm/s) |  | 80.7 | 135.415 |
| Track pitch (m) |  | 21.7 | 18 |
| Minimum wavelength (m) |  | 0.49 |  |
| Width of tape (mm) |  | 12.65 | 6.35 |
| Media substance |  | Metal particle |  |
| Hc (kA/m) |  | 132 | 184 |
| Cassette size (mm) |  | M: 156  96 L: 254  145 | L: 125  78 |
| Recording time (min) |  | 48/149 | 46 |
| (1) Each format as commercially known is listed here. (2) 5/8 horizontal prefiltering and 1/4.4 DCT. |  |  |  |

TABLE  6

SDTV digital VTR for 625/50 – Details

| Format |  | A | B | C | D | E | F | G |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Commercial name(1) |  | Digital BETACAM | DVCPRO | DVCAM | BETACAM SX | DVCPRO 50 | Digital-S | MPEG IMX |
| Sampling frequency | Video (MHz) | 13.5 |  |  |  |  |  |  |
|  | Audio (kHz) | 48 |  | 48/32 | 48 |  |  |  |
| Quantization | Video (bits) | 10 | 8 |  |  |  |  |  |
|  | Audio (bits) | 20 | 16 | 16/12 | 16 |  |  | 24/16 |
| Number of audio channels |  | 4 | 2 | 2/4 | 4 |  |  | 4/8 |
| Compression (video) |  | Intra field DCT (1/2) | Intra frame DCT (1/5) |  | Inter frame DCT (1/10) | Intra frame DCT (1/3.3) | DCT (1/3.3) | Intra frame DCT (1/3.3) |
| Channel coding |  | S-NRZI | 24-25 I-NRZI |  | S-NRZI | 24-25 I-NRZI |  | S-NRZI |
| Total rate (Mbit/s) |  | 126 | 42 |  | 44 | 84 | 99 | 88 |
| Video rate (Mbit/s) |  | 89 | 25 |  | 18 | 50 |  |  |
| Number of recording RF channels |  | 4 (camcorder) /2 (studio) | 1 |  | 4 (camcorder) /1 (studio) | 2 |  | 8 (camcorder) /2 (studio) |
| ECC | Inner | 178, 164 | 85, 77 |  | 124, 112 | 85, 77 |  | 162, 150 |
|  | Outer | 126, 114 | 149, 138 |  | 64, 50 | 149, 138 |  | 64, 54 |
| Drum diameter (mm) |  | 81.7 (camcorder) /81.4 (studio) | 21.7 |  | 49.6 (camcorder) /81.4 (studio) | 21.7 | 62 | 54.0 (camcorder) /81.4 (studio) |
| Drum rotation (rps) |  | 37.5 (camcorder) /75 (studio) | 150 |  | 37.5 (camcorder) /75 (studio) | 150 | 75 | 25 (camcorder) /50 (studio) |
| Number of tracks |  | 6/field | 12/frame |  | 12/2 frame | 24/frame | 12/frame | 8/frame |
| Tape speed (mm/s) |  | 96.7 | 33.854 | 28.221 | 59.6 | 67.708 | 57.795 | 53.8 |
| Track pitch (m) |  | 26 | 18 | 15 | 32 | 18 | 20 | 21.7 |
| Minimum wavelength (m) |  | 0.59 | 0.49 |  | 0.74 | 0.49 | 0.587 | 0.56 |
| Width of tape (mm) |  | 12.65 | 6.35 |  | 12.65 | 6.35 | 12.65 |  |
| Media substance |  | Metal particle |  | Metal evaporated | Metal particle |  |  |  |
| Hc (kA/m) |  | 125 | 184 | 120 |  | 184 | 143 | 120 |
| Cassette size (mm) |  | S: 156  96 L: 254  145 | M: 97.5  64.5 L: 125  78 | Standard: 125  78 Mini: 66  48 | S: 156  96 L: 254  145 | M: 97.5  64.5 L: 125  78 | L: 188  104 | S: 156  96 L: 254  145 |
| Recording time (min) |  | 40/124 | 66/184 | 40/184 | 64/194 | 33/92 | 124 | 72/224 |
| (1) Each format as commercially known is listed here. |  |  |  |  |  |  |  |  |